3GPP TSG RAN WG1 #120bis		                         R1-2502021

Wuhan, China, April 7th – 11st, 2025

Agenda Item:	9.3.2

Source:	China Telecom

Title:	Discussion on SBFD random access operation

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss SBFD random access operation.

Discussion

Power ramping

In RAN1 #120 meeting [1], the following agreement was achieved with respect to power ramping.

Agreement Consider the following alternatives about power ramping when a SBFD-aware UE switches from additional-ROs to legacy-ROs and from legacy-ROs to additional-ROs (if supported) in PRACH transmission re-attempt in one RACH procedure for RACH configuration Option 1 and Option 2. - Alt-1: Increase the power ramping counter. - FFS whether to introduce a power offset to compensate the power ramping difference for RACH configuration Option 2. - Alt-2: Reset the power ramping. - Note: different alternatives can be applied for different RACH configuration options.

One of the intentions for introducing SBFD operation for PRACH is to reduce the latency. Thus, we don’t think Alt-2 is reasonable, especially when there are already several re-attempts with increased power. In this case, probably the UE is under a poor coverage, no matter the UE transmit on additional-RO or legacy-RO, a large transmit power is anyway needed. If the power ramping is reset when the transmission is switched from additional-ROs to legacy-ROs or vice versa, the UE may never reach the needed transmit power. Thus, we think Alt-1 is the only choice.

For RACH configuration Option 1, the following agreement/conclusion were achieved in previous meetings. It can be known that there will be no separate power ramping related parameters for additional-ROs. Thus, we think the power ramping counter can be directly increased without introducing any power offset.

Agreement (RAN1 #119) For RACH configuration Option 1, for support of separate power control parameters for PRACH transmission in additional-ROs and legacy-ROs, support separate preambleReceivedTargetPower for additional-ROs. - FFS: powerRampingStep, preamble maximum output power PCMAX, preambleTransMax, powerRampingStepHighPriority Conclusion (RAN1 #120) There is no RAN1 consensus on the following proposal: For RACH configuration Option 1, for support of separate power control parameters for PRACH transmission in additional-ROs and legacy-ROs, - also support separate powerRampingStep and powerRampingStepHighPriority for additional-ROs.

For RACH configuration Option 2, although there is no related agreement, from our understanding, the power control related parameters can be separately configured. We think a power offset can be considered to make the ramped power at a reasonable level.

Proposal 1:  When a SBFD-aware UE switches from additional-ROs to legacy-ROs and from legacy-ROs to additional-ROs (if supported) in PRACH transmission re-attempt in one RACH procedure for RACH configuration Option 1 and Option 2, Alt-1 is supported, i.e., increase the power ramping counter.

PRACH mask

Based on current specifications, PRACH mask is utilized for the network to select specific ROs either for CFRA PRACH transmission or for shared RO indication as defined in TS 38.213.

The PRACH occasions are mapped consecutively per corresponding SS/PBCH block index. The indexing of the PRACH occasion indicated by the mask index value is reset per mapping cycle of consecutive PRACH occasions per SS/PBCH block index. The UE selects for a PRACH transmission the PRACH occasion indicated by PRACH mask index value for the indicated SS/PBCH block index in the first available mapping cycle.

For SBFD operation, for both RACH configuration Option 1 and Option 2, separate SSB-RO mapping is expected between additional ROs in SBFD symbols and legacy RO in non-SBFD symbols. If the legacy PRACH mask indication is followed, the network cannot indicate the ROs with enough flexibility, e.g., the network cannot only indicate the additional ROs in SBFD symbols. Thus, we think at least the following aspects needs to be considered:

- RO indexing: we think separate RO indexing is needed for additional ROs and legacy ROs, which is aligned with separate SSB-RO mapping.

- RO type indication: we think network can indicate whether the PRACH mask is indicating additional RO or legacy RO or both additional RO and legacy RO.

For RO indexing, since SSB-RO mapping is separately performed for additional RO and legacy RO, we think the indexing of the PRACH occasion indicated by the mask index value is reset per mapping cycle of consecutive PRACH occasions per SS/PBCH block index for additional ROs and legacy ROs, respectively as illustrated in Fig.1, where one SSB is mapped to 2 ROs for legacy RO, while one SSB is mapped to 4 ROs for additional ROs.

Fig.1. Illustration for indexing of PRACH occasions indicated by PRACH mask index

Proposal 2: SSB-RO mapping cycle is determined separately for legacy RO and additional RO. And indexing of the PRACH occasion is separately indicated by the mask index value for legacy RO and additional RO.

For RACH configuration Option 1, the following agreement was achieved in RAN1 #120 meeting.

Agreement For RACH configuration Option 1, down-select from the following two alternatives: - Alt-1: Shared ssb-SharedRO-MaskIndex configuration for Additional-ROs and Legacy-ROs. - Alt-2: Separate ssb-SharedRO-MaskIndex configurations for Additional-ROs and Legacy-ROs.

For a random access procedure associated with a feature combination, “ssb-SharedRO-MaskIndex” can be used to indicate the set of ROs which can be used for PRACH transmission. Since separate SSB-RO mapping will be used for additional ROs and legacy ROs as agreed in RAN1 #116bis meeting, we think this parameter cannot be shared. Thus, we support Alt-2.

Agreement For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration), - For the legacy-ROs, including the ROs in non-SBFD symbols and the ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon (if any), the legacy SSB-RO mapping is followed. - For the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, separate SSB-RO mapping will be used

Proposal 3: For RACH configuration Option 1, separate ssb-SharedRO-MaskIndex configurations are supported for Additional-ROs and Legacy-ROs, i.e., Alt-2 is supported.

Conclusions

In this contribution, we discuss SBFD random access operation and have following proposals:

Proposal 1:  When a SBFD-aware UE switches from additional-ROs to legacy-ROs and from legacy-ROs to additional-ROs (if supported) in PRACH transmission re-attempt in one RACH procedure for RACH configuration Option 1 and Option 2, Alt-1 is supported, i.e., increase the power ramping counter.

Proposal 2: SSB-RO mapping cycle is determined separately for legacy RO and additional RO. And indexing of the PRACH occasion is separately indicated by the mask index value for legacy RO and additional RO.

Proposal 3: For RACH configuration Option 1, separate ssb-SharedRO-MaskIndex configurations are supported for Additional-ROs and Legacy-ROs, i.e., Alt-2 is supported.

References

- 3GPP RAN1 Chairman’s Notes, RAN1 meeting #120, Athens, Greece, February 17th – 21st, 2025.