3GPP TSG RAN WG1 #120bis			R1-2502030

Wuhan, China, April 7th – 11th, 2025

Agenda item:		9.7.2

Source:	China Telecom

Title:	Discussion on ISAC channel modelling

Document for:		Discussion

Introduction

In RAN# 102 meeting, a new Rel-19 study item on channel modelling for Integrated Sensing And Communication (ISAC) for NR was approved [1]. The objectives are listed as follows.

Objective: The focus of the study is to define channel modelling aspects to support object detection and/or tracking (as per the SA1 meaning in TS 22.137). The study should aim at a common modelling framework capable of detecting and/or tracking the following example objects and to enable them to be distinguished from unintended objects: - UAVs - Humans indoors and outdoors - Automotive vehicles (at least outdoors) - Automated guided vehicles (e.g. in indoor factories) - Objects creating hazards on roads/railways, with a minimum size dependent on frequency All six sensing modes should be considered (i.e. TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, UE monostatic). Frequencies from 0.5 to 52.6 GHz are the primary focus, with the assumption that the modelling approach should scale to 100 GHz. (If significant problems are identified with scaling above 52.6 GHz, the range above 52.6 GHz can be deprioritized.) For the above use cases, sensing modes and frequencies: - Identify details of the deployment scenarios corresponding to the above use cases. - Define channel modelling details for sensing using 38.901 as a starting point, and taking into account relevant measurements, including: - modelling of sensing targets and background environment, including, for example (if needed by the above use cases), radar cross-section (RCS), mobility and clutter/scattering patterns; - spatial consistency.

Based on the scope of the study item and the agreements achieved in the previous meeting [2], we continue to present our views on the issues of ISAC channel modelling in this contribution.

Physical object modelling

  - Bistatic RCS

In the last meeting, a FL proposal [3] regarding the RCS for a scattering point of a target bistatic sensing was discussed and several options were proposed for down-selection.

[H][FL3] Proposal 4.1-2 Study the following options for A*B1 of RCS for a scattering point of a target bistatic sensing: - Option 1: RCSbi=RCSmo∙abscosβ2, β is bistatic angle [0, 360] degree - Option 2: bistatic RCS is constructed into 3 angular regions according to scattered angles - Region 1: derive the RCS based on monostatic RCS - This may or may not be Option 1 - Region 2: derive the RCS around the angles near to the specular reflection angle - Region 3: RCS value including large effect of diffraction - Option 3: bistatic RCS is constructed into 3 angular regions according to scattered angles - Region 1+2: σB(f,θ,θ+β)=4π λ2S2cos2(θ+β)∙sinc22πaλsin(θ+β/2)∙cos(β/2) - Where a is the length of a rectangle (cf R1-2501212) - The formula may be further updated - Region 3: RCS value including large effect of diffraction - Distance dependent RCS is not considered in any incident/scattered angles Which option is selected/updated will be based on validation results

As companies pointed out, there has been a lot of research effects to derive the bistatic RCS based on the backscattering RCS as which is the simplest form of RCS. The monostatic-bistatic equivalence theorem derived in [4] states that the bistatic RCS of a sufficiently smooth, perfectly con-ducting target is equal to the backscattering RCS measured on the bisector of the bistatic angle for electromagnetic waves of high-frequency. Quantitatively, [5] points out the validity of this theorem for bistatic angles less than 50 degrees and the frequency lower by a factor of cos(β/2), where β/2 is the bistatic bisector. Therefore, the following formula, derived from extensive research, should rightly be adopted for static RCS calculation [6].

RCSbi=RCSmo∙abscosβ2

However, this equivalence RCS formula holds in the pseudo-monostatic RSC region only, i.e. within the restricted sector of bistatic angles. As for the bistatic region outside the forward scattering region, the formula no longer applies because the approximation error is too large. Therefore, for regions other than the pseudo-monostatic RSC region, bistatic RCS may need to be described by other formulas.

Proposal 1: For A*B1 of RCS for a scattering point of a target bistatic sensing, support bistatic RCS to be constructed into multiple angular regions according to scattered angles.

Proposal 2: For A*B1 of RCS for a scattering point of a target bistatic sensing, adopt the formula RCSbi=RCSmo∙abscosβ2, at least for the pseudo-monostatic RSC region, where β is bistatic angle.

Target channel modelling

LOS condition of Tx-target/target-Rx link

Agreement regarding LOS conditions with the existence of EO type-2 was made in the previous meeting, providing two options for further down-selection.

#### Agreement When the EO type-2 is modelled in the target channel, down select between the following options to determine the LOS condition of the Tx-target link and target-Rx link - Option A: If type-2 EO is in the LOS ray of one link, the link is determined as NLOS condition, and otherwise use the LOS probability equation to determine the LOS/NLOS condition - FFS changes to the LOS probability defined in existing TRs - FFS details on blockage by EO type-2 Option B: Use the LOS probability equation to determine the LOS/NLOS condition of one link, and then the impacts of type-2 EO is modeled by a blockage model

This issue was discussed in the last meeting, but no consensus was reached. To facilitate the dust-up on this issue, we reiterate our preference for Option A. Since EO type2 represents the wall, which is a stationary object in the environment. It is proper to model the link as a NLOS condition when the EO type2 blocks the direct path between the Tx and the Rx.

Proposal 3: When the EO type-2 is modelled in the target channel, to determine the LOS condition of the Tx-target link and target-Rx link,

- If type-2 EO is in the LOS ray of one link, the link is determined as NLOS condition, otherwise use the LOS probability equation to determine the LOS/NLOS condition.

Power normalization between target channel and background channel

The following agreement related to the topic of the combination of target channel and background channel has been achieved as follows.

#### Agreement - The following options are supported to generate the combined ISAC channel - Option 1: The ISAC channel of a pair of sensing Tx/Rx is obtained by summing the target channel(s) and background channel, i.e., power normalization is not performed. - Option 2: As an additional modelling component, power normalization is performed when summing the target channel(s) and background channel, to keep the same/similar channel power as the background channel without target. Down select between - Alt 1: Power normalization on both target channel and background channel - Alt 2: Power normalization on background channel only - Alt 3: the target channel of a target will replace one cluster in the background channel - FFS Blockage is modelled for the background channel due to sensing target and/or EO type-2 FFS condition to select option, e.g. depending on scenario, sensing mode, number of target/EO type-2

Up to this point, there is still a big disagreement about whether or how to perform power normalization when generating the combined ISAC channel. As some companies pointed out through simulation results, power normalization, if not performed, will distort performance metrics of angle spread distribution, as well as violate the physical law of energy conservation. Hence, we support performing power normalization when summing the target channel and background channel. As for the alternatives down-selection, we prefer Alt 1 to normalize both target and the background channel. However, in order to ensure that the power of the target channel is always significantly smaller than the background channel, a power scaling operation should be conducted before channel summation.

Proposal 4: Support Option 2-Alt 1 to generate the combined ISAC channel.

Conclusions

In this contribution, we discuss ISAC channel modelling related issues and have following proposals:

Proposal 1: For A*B1 of RCS for a scattering point of a target bistatic sensing, support bistatic RCS to be constructed into multiple angular regions according to scattered angles.

Proposal 2: For A*B1 of RCS for a scattering point of a target bistatic sensing, adopt the formula RCSbi=RCSmo∙abscosβ2, at least for the pseudo-monostatic RSC region, where β is bistatic angle.

Proposal 3: When the EO type-2 is modelled in the target channel, to determine the LOS condition of the Tx-target link and target-Rx link

- If type-2 EO is in the LOS ray of one link, the link is determined as NLOS condition, otherwise use the LOS probability equation to determine the LOS/NLOS condition.

Proposal 4: Support Option 2-Alt 1 to generate the combined ISAC channel.

References

- 3GPP RP-234069, “Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR”, Xiaomi, AT&T, Dec. 11 - 15, 2023.

- 3GPP RAN1, Chair’s Notes RAN1#120, RAN1#120, Athens, Greece, February 17th – 21st, 2025.

- 3GPP RAN1, Summary #4 on ISAC channel modelling, Orlando, US, November 18th – 22nd, 2024.

- Schipper, J. (1993). Bistatic Radar Cross Section and -imaging. [Master's thesis, Eindhoven University of Technology].

- R.E. Kell, "On the Derivation of Bistatic RCS from Monostatic Measurements", Proc. IEEE, Vol.53, August 1965, pp.983-988.

- R. L. Eigel, et.al, "Bistatic scattering characterization of complex objects," IEEE Transactions on Geoscience and Remote Sensing, vol. 38, no. 5, pp. 2078-2092, Sept. 2000.