3GPP TSG RAN WG1 #120bis 	                                           R1-2502173

Wuhan, China, April 7th – 11th, 2025

Source: 	CMCC

Title:	Discussion on NR-NTN DL coverage enhancement

Agenda item:	9.11.1

Document for:	Discussion & Decision

- Introduction

During the RAN#102 meeting, a new work item was proposed for further enhancements for NR NTN Phase 3 [1]. One of the objectives is to focus on the downlink coverage enhancements for NR NTN, and based on evaluation results, the objective was updated in the RAN#106 meeting for further clarification on the scope of link-level and system-level enhancements, which is shown as below [2].

Objective The work item aims at specifying further enhancements for NG-RAN based NTN (Non-Terrestrial Networks) with the following assumptions: - GSO (Geo Synchronous Orbit) and NGSO (Non-Geo Synchronous Orbit). NGSO includes Low Earth Orbit (LEO) and Medium Earth Orbit (MEO). - Earth fixed tracking area. Earth fixed & Earth moving cells for NGSO - FDD mode - UEs with GNSS (Global Navigation Satellite Systems) capabilities - In frequency band above 10 GHz, both Terminal Type 1 (Electronic steering antenna) and Type 2 (Mechanical steering antenna) to be considered for GSO and NGSO - Implicit compatibility to support HAPS (High Altitude Platform Station) and ATG (Air To Ground) scenarios, where relevant Note 1: In Rel-19 WID, “VSAT” device with external antenna on moving platform is equivalent to a device that operate on platforms in motion, and this is referred to as ESIM (Earth Station In Motion). The objectives of the work item are the following: - Specify downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Specify solutions, including link level enhancements for FR1-NTN and system level enhancements for FR1-NTN and FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - Link level enhancements are to be specified for the following channels: - PDCCH at least for CSS (except for Type-3) via PDCCH repetition - PDSCH with Msg4 via PDSCH repetition - PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration - System-level enhancements are to be specified for the following: - Support of extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is 160 ms - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km

During last RAN1 meeting, detailed schemes on link-level coverage enhancements for PDCCH and PDSCH have been discussed separately [3]. In this contribution, we will further share our views on detailed link-level enhancement solutions to support repetition of PDCCH CSS (except for Type-3), PDSCH carrying SIB1 and PDSCH with Msg4. And for system-level enhancements, we will continue discuss the impacts of SSB periodicity extension and potential DL coverage enhancements.

- Discussion

  - 2.1 System-level study

For system level study of NTN DL coverage, several key issues, including the reference satellite parameters assumptions, antenna model, evaluation metrics and traffic models have been discussed, some progress on baseline configuration, assumptions and DL coverage ratio evaluation were achieved. For LEO-600 in FR1 band, three sets of reference satellite parameters are assumed for different NTN deployment scenarios, referred to Set1-1 FR1, Set1-2 FR1 and Set1-3 FR1 listed below [4].

RAN1#116 Agreement For DL coverage study, consider the following additional reference satellite parameters scenarios for LEO600km Set1 in FR1 (i.e., S-band), referred to as Set1-1 FR1, Set1-2 FR1 and Set1-3 FR1: LEO600km Set1-1 FR1 (i.e., S-band). Maximum Bandwidth per beam; 5 MHz. SCS; 15 kHz. Beam size(Note 1); 50km. Satellite EIRP density /beam (dBW/MHz); 34. Payload Total DL power level (dBW); 31.24. Aggregated EIRP (Total) (dBW); 61.24*. Satellite Tx max Gain; 30 dBi. Maximum EIRP per Satellite beam (dBW); 41. Total number of beam footprints***; 1058. Total number of simultaneously active beams **; 106. % simultaneously active beams**; 10.02 %. *Note: EIRP limit is 61.24 dBm for the reference configuration. **Assuming 100 % Resource Block utilization within the same beam at max power. Absolute number of simultaneously active beams is up to 212 (due to limitation of RF) *** For a constellation design at 600km with low elevation angle with 30° and selected (i.e Set 1 parameters) beam size Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies. LEO600km Set1-2 FR1 (i.e., S-band). Maximum Bandwidth per beam; 5 MHz. SCS; 15 kHz. Beam size (note 1); 50km. Satellite EIRP density /beam (dBW/MHz); 34. Payload Total DL power level (dBW); 23. Aggregated EIRP (Total) (dBW); 53*. Satellite Tx max Gain; 30 dBi. Maximum EIRP per Satellite beam (dBW); 41. Total number of beam footprints; 1058. Total number of simultaneously active beams**; 16. % simultaneously active beams**; 1.5 %. *Note: EIRP limit is 53 dBm for the reference configuration. **Absolute number of simultaneously active beams is up to 16 (due to limitation of RF) Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies. LEO600km Set 1-3 FR1 (i.e., S-band). Maximum Bandwidth per beam; 5 MHz. SCS; 15 kHz. Beam size (note 1); 50km. Satellite EIRP density /beam (dBW/MHz); 26. Payload Total DL power level (dBW); 23.24. Aggregated EIRP (Total) (dBW); 53.24*. Satellite Tx max Gain; 30 dBi. Maximum EIRP per Satellite beam (dBW); 33. Total number of beam footprints; 1058. Total number of simultaneously active beams**; 106. % simultaneously active beams**; 10.02 %. *Note: EIRP limit is 53.24 dBm for the reference configuration. **Absolute number of simultaneously active beams is up to 212 (due to limitation of RF) Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies. Note: RAN1 will aim to identify necessary enhancements for these scenarios in the study phase. At the end of the study phase, RAN1 will further discuss whether the potential enhancements will be specified within Rel-19 framework.

### 2.1.1 Impact of SSB periodicity extension

Three sets of reference satellite parameters present different satellite capabilities and using scenarios. LEO-600km Set 1-1 FR1 represents the case which has a relatively large number of simultaneously active beams, i.e. 106 beams, to cover 1058 planned beam footprints on the earth. Set 1-2 FR1 has the same DL transmit power as Set 1-1, but with a smaller number of simultaneously active beams, i.e. 16 beams, due to the limitation of satellite payload. The intended coverage area of Set 1-2 is the same as that of Set 1-1, which is also 1058 beam footprints on the earth. Set 1-3 FR1 has the same number of the simultaneously active beams as Set 1-1, but with a lower transmit power at the satellite side, which may cause further link level coverage issue for the DL coverage.

Based on the analytical evaluation results, it can be observed that coverage ratio is improved with longer duration of SSB periodicity from UE side. Extension of the periodicity of SSBs is a prerequisite for reaching almost 100% coverage ratio for all the scenarios using the parameter Set 1-1/1-2/1-3 FR1. Besides, based on the evaluation results of common control channel overhead with different default SSB/SIB1 periodicity summarized in the last RAN1 meeting, it is observed that extending SSB periodicity is also beneficial for reducing common channel overhead, which can be reflected when 160ms or larger SSB periodicity is used.

The candidate values of extended SSB periodicity have been discussed for several meetings, and it is agreed to take 160ms as the default value of extended SSB periodicity in the last RAN1 meeting, which can be used to improve coverage ratio with less specification works.

RAN1#120 Agreement For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The additional default value assumed by UE during initial access (apart from the existing 20ms value) is 160 ms.

In the current specification, legacy UEs assume the 20ms default detection periodicity for SSBs. Considering the impact to backward compatibility when SSB periodicity is extended in NTN scenarios, different SSB periodicities can be supported for covering different beam footprints by dynamic beam scheduling. That is, the legacy UEs will be served by some beam footprints being illustrated with 20ms period, and longer default SSB detection period can be used for other beam footprints serving NTN UEs.

Observation 1: Different SSB periodicities can be used to solve the backward compatibility issue, wherein one set of legacy 20ms SSB periodicity can be reserved for legacy NTN UE in some beam footprints, and other sets of longer SSB periodicities can be used to serve Rel-19 NTN UE in other beam footprints.

As the default SSB periodicity can be extended to 160ms during initial cell research, the periodicity of common control channel, necessary system information and UE measurement which related to the periodicity of SSB also needs to be further discussed. Considering the UE procedure for monitoring Type0-PDCCH CSS in current specification, for operation with shared spectrum channel access and for the SS/PBCH block and CORESET multiplexing pattern 1, UE monitors PDCCH in a frame with odd SFN or even SFN, the periodicity of Type0-PDCCH monitoring occasions aligns with SSB periodicity of 20ms. As a larger default SSB periodicity than 20ms is supported, the periodicity of Type0-PDCCH monitoring needs to be extended correspondingly, to ensure that the initial access process can be completed during the duration when the beam footprint is illuminated. Also, the periodicity UE measurement related to SSB periodicity also needs to be extended correspondingly.

Proposal 1: Considering the extended SSB periodicity of 160ms, the periodicity of Type0-PDCCH monitoring and UE measurement needs to be extended correspondingly.

### 2.1.2 Potential system-level enhancements

As mentioned in the objectives of DL coverage enhancement in NR NTN, system level coverage enhancement via efficient power sharing is proposed for further study. Currently it assumes that a large amount of DL beams from satellites will be configured, but only a limited portion of beams can be activated simultaneously. In order to provide services to all UEs covered by the satellite footprint, TDM beam scheduling/beam hopping can be performed to improve the network throughputs.

Considering system level enhancement with restricted number of simultaneously active beams and power sharing among active beams, the following potential schemes can be further discussed to improve the coverage performance.

- Long revisit period and dwelling window for each beam should be defined to improve the coverage ratio

- Extending the default periodicity for common channels/message (e.g., SSB, SIB1, SIB19)

- Potential enhancements to provide valid ROs.

Combined with long detection period of SSBs and important common control channel for initial access procedure, a revisit period and a dwelling window should be introduced to facilitate the operation under NTN scenarios. It is proposed to support the system level enhancements to improve the coverage ratio in NR-NTN phase 3, at least considering to introduce the revisit periodicity and dwelling window.

From our understanding, the revisit time of the beam to serve each beam footprint is determined by beam hopping pattern and residence time on the beam footprint. Assuming that each footprint is uniformly illuminated based on TDM beam scheduling for common channels, according to the coverage ratio analysis in Table 1, if default 20ms SSB periodicity is assumed, since maximum 40.08% coverage of common control channels (e.g. SSB, CORESET0/SIB1, SIB19) can be satisfied for LEO-600 Set1-1/1-3, longer revisit time than 20ms is needed in order to serve all the 1058 beam footprints.

Observation 2: For LEO-600 Set1-1/1-2/1-3, longer revisit period can be considered to improve the coverage ratio.

Proposal 2: Support the system level enhancements to improve the coverage ratio in NR-NTN phase 3, with the consideration of introducing the revisit periodicity and dwelling window for other channels/signals to adapt to SSB periodicity extension.

Furthermore, the allocation of SSB and necessary system information for each satellite beam or beam footprint need to be further studied. Taking the satellite parameters Set 1-2 as an example, the operation and necessary signals/channels are illustrated in the Figure 2.1-1. 4 SSBs are supported for the S-band. Each SSB can provide DL synchronization over one beam footprint. With one of the 16 simultaneously active beams from Set 1-2, 4 SSBs can be transited at different time instance into 4 different beam footprints. And within one beam footprint, the SIB1 and SIB19 can be transmitted following the same SSB providing the system information and the necessary information for initial access. The SSBs, the related SIB1 and SIB19 can be transmitted in 4 beam footprints in a TDM manner. The transmission of those information over 4 beam footprints will occupy at least 10ms of the 20ms within dwelling window. Then the residual time-domain resources not used for SSB, SIB1 and SIB 19 (e.g., the last 10ms of the dwelling window) can be used for the dynamic scheduling depending on the traffic, such as scheduling for the same beam footprints as SSB or other beam footprints.

For the initial access for the uplink, after UE successfully decoded the information of SIB19, the UE can transmit the PRACH at corresponding RACH occasion associated with the specific SSB index within the dwelling window. Excluding the RACH occasions whose beam direction should be determined according to the associated DL SSB, the other uplink slots can be used dynamically, including the last 10ms of the 20ms dwelling window. Based on the assumption above, the active beam will periodically visit one beam footprint in a 160ms periodicity. The UE can be scheduled every 160ms for at least 10ms for the traffic. If the UEs are not uniformly located in all the beam footprints or triggering any traffic, the gNB can allocate the last 10ms of the 20ms dwelling window to the other beam footprint, or extend the dwelling window for one beam footprint and reduce the dwelling time of other beam footprints.

Figure 2.1-1. An example of 20ms dwelling window and the 160ms revisit period for Set 1-2

Observation 3: Within the dwelling window, the SSB and necessary system information (e.g., SIB1, SIB19) can be broadcasted for the target beam footprints, which can be utilized by UEs to access to the satellite/gNB within the dwelling window. For the residual slots of the dwelling window, it can be used for the dynamic scheduling for the same beam footprints as SSB or other beam footprints.

Observation 4: To provide services to all UEs covered by the satellite footprint, TDM beam scheduling/beam hopping can be performed to illuminate different beam footprint for SSB and necessary system information (e.g., SIB1, SIB19) transmission.

Observation 5: The time span of the dynamic scheduled dwelling window for a given beam footprint can be allocated based on UE distribution and traffic distribution.

In addition to the TDM beam scheduling of DL active beams, it was proposed to discuss whether enhancements on RO configuration are necessary in the last RAN1 meeting. Considering the default SSB periodicity can be extended to 160ms for NR NTN and limited number of simultaneously active beams, when the active beam from the satellite moves to other beam footprints, the RO during these period cannot be used for UE’s initial access covered by the original beam footprint. Therefore, the configuration of RACH occasions needs to be adapted to the beam hopping pattern if TDM scheduling of active beams is supported.

Figure 2.1-2. An example of RO configuration

As depicted in Figure 2.1-2, the current specification mandates that ROs are configured periodically within a 10240 ms interval. However, to align with the beam hopping pattern, only a subset of the configured ROs within this interval is valid for a given SSB index corresponding to a specific beam footprint. Therefore, potential enhancements to associate valid ROs among the entire set of configured ROs with an SSB index corresponding to a beam footprint should be considered.

Opt 1: validity window based										Opt 2: mapping function based

Figure 2.1-3. Potential enhancement for RO validity

Figure 2.1-3 illustrates two potential enhancements for RO validity:

- Option 1 (Validity Window Based): A validity window associated with a specific SSB index is introduced. ROs configured within this validity window are deemed valid.

- Option 2 (Mapping Function Based): A mapping function between the SFN of a specific SSB index and the configured RO index(es) is introduced. The calculated ROs are valid for the corresponding SSB index.

  - Alternatively, a mapping bitmap can also be considered.

Proposal 3: Potential enhancements to associate valid ROs among the entire set of configured ROs with an SSB index corresponding to a beam footprint should be considered.

Proposal 4: To improve the association of valid ROs among the entire set of configured ROs with an SSB index corresponding to a beam footprint, the following enhancements can be considered:

- Option 1 (Validity Window Based): A validity window associated with a specific SSB index is introduced. ROs configured within this validity window are deemed valid.

- Option 2 (Mapping Function Based): A mapping function between the SFN of a specific SSB index and the configured RO index(es) is introduced. The calculated ROs are valid for the corresponding SSB index.

  - Alternatively, a mapping bitmap can also be considered.

  - 2.2 Link-level study

According to the agreements achieved in the RAN1#116 meeting, NGSO at LEO-600 operating in FR1 is considered in priority for the link level evaluation of NTN-specific downlink coverage. Regarding the target services and target data rate, the study in Rel-19 will continue to focus on the handset terminals (including smartphones with -5.5 dBi antenna gain). Also, the target services (i.e., VoIP and low-data rate services for commercial handset terminals) and the target data rate for each service determined in Rel-18 can be re-used to evaluate DL coverage performance in NR NTN.

For link level DL coverage evaluation, the evaluation methodology and performance metrics are determined. Also, the reference parameters and simulation assumptions have been agreed in the RAN1#116bis meeting.

### 2.2.1 Observations on the evaluation results

Considering the link budget calculation of each NTN study case, the two different satellite EIRP density, i.e., 34dBm/MHz for Set1-1/Set1-2 FR1, 26dBm/MHz for Set1-3 FR1, agreed in satellite parameter tables are considered in the link budget calculation. By using the formula specified in Section 6.1.3.1 of TR 38.821, the CNR for each study case in NR NTN are calculated, and it is concluded that the CNRs for the satellite payload parameters Set 1-1, Set 1-2 and Set 1-3 are equal to -1.9 dB, -1.9 dB and -9.9 dB respectively based on the observation by companies.

The coverage performance of following DL channels/signals are evaluated to identify the bottleneck channel(s). And observations on DL coverage performance of each channel/signal have been summarized based on evaluation results provided in the RAN1#117 meeting.

- PDSCH for VoIP

- PDSCH for low data rate service-3kbps

- PDSCH for low data rate service-1Mbps

- PDSCH Msg.2

- PDSCH Msg.4

- PDSCH carry SIB1

- PDSCH carry SIB 19

- PDCCH

According to the evaluation results observed in the previous RAN1 meetings, for LEO-600 Set 1-1/1-2 scenario, since EIRP density is not decreased due to dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint or other additional loss, it is observed that there is no coverage issue for all the evaluated physical channels/signals.

While for LEO-600 Set 1-3 FR1 satellite parameters, EIRP density is 8dB less than that of Set 1-1/1-2 FR1, it can be observed that there are coverage gaps for some evaluated physical channels, including PDCCH, PDSCH carrying SIB1 or SIB19, PDSCH Msg4 and PDSCH for 1Mbps low data rate service, and the coverage gap of each DL channel are summarized in the Table 2. Furthermore, the CNR target for LEO-600 Set 1-3 FR1 was discussed in the last meeting, it is proposed to adjust the CNR target to -8dB based on the coverage performance of PSS detection, as agreed in the last meeting, at least for PDCCH and PDSCH carrying Msg4, it will focus on coverage enhancement with a target CNR of -8 dB for link-level study. Therefore, the coverage gaps of each DL channel/signal compared with CNR target of -8dB are also provide as in Table 2.2-1. The link level analysis and potential coverage enhancements will be discussed by taking the observations on average evaluation results collected in the meeting and our simulation into consideration.

Table 2.2-1. Coverage gap for LEO-600 Set1-3 FR1

| DL Channel/signal | Coverage gap compared with -9.9dB CNR |  | Coverage gap compared with -8dB CNR |  |
| --- | --- | --- | --- | --- |
|  | Average results | CMCC results | Average results | CMCC results |
| PDCCH | 3.9 | 3.52 | 2 | 1.62 |
| PDSCH for low data rate-1Mbps | 5.5 | 7.73 | 3.6 | 5.83 |
| PDSCH Msg4 | 4.7 | 4.49 | 2.8 | 2.59 |
| PDSCH SIB1 (800bits) | 4.1 | 3.76 | 2.2 | 1.86 |
| PDSCH SIB1 (1280bits) | 6.5 | 5.77 | 4.6 | 3.87 |
| PDSCH SIB19(616 bits) | 3.5 | 3.05 | 1.6 | 1.15 |

### 2.2.2 Potential link-level enhancements

As clarified in the scope of WID after RAN#106 meeting, link level enhancements are to be specified for the following channels:

- PDCCH at least for CSS (except for Type-3) via PDCCH repetition

- PDSCH with Msg4 via PDSCH repetition

- PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration

PDCCH

For PDCCH in LEO-600 scenarios with Set1-3 FR1 parameters, it is observed that 1.62dB coverage gap needs to be enhanced based on our simulation results, and the average coverage gap is about 2dB. In order to ensure the PDSCH/PUSCH can be scheduled successfully for both common channels and UE specific channels, coverage enhancements for PDCCH in Set1-3 FR1 needs to be considered. Some potential solutions focusing on PDCCH CSS link level enhancements have been discussed in previous RAN1 meetings, and PDCCH repetition for NTN is agreed as the enhancement scheme for further study. The agreements for PDCCH CSS (except Type-3) achieved in RAN1 meetings are listed below.

RAN1#119 Agreement For PDCCH CSS (except Type-3) link level enhancements, support only PDCCH repetition for NTN. - FFS: intra-slot and/or inter-slot RAN1#120 Agreement Support only inter-slot repetition for Type0 PDCCH CSS. RAN1#120 Agreement At least for enabling PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1, RAN1 to consider the following options - Option 1: Using the spare 1 bit in MIB - Option 2: Using reserved bit(s) in PBCH payload - Option 3: Using codepoint(s) in PBCH payload - Option 4: UE blind decoding without signaling from the network during initial access RAN1#120 Agreement For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1, consider the following: - Option 1: Support repeated PDCCH candidates in the two consecutive slots n0 and n0+1 associated with the same SSB index ( n0 as defined in section 13 of TS 38.213) - Repeated PDCCH candidates share the same aggregation level (AL), coded bits and same candidate index - FFS: Details including how the two PDCCH candidates are counted toward the BD limits - Note: with option 1, if the network repeats the Type 0 PDCCH across two consecutive slots, a legacy UE might decode the PDCCH and associated PDSCH in one slot and skip PDCCH monitoring in the other slot. - FFS: whether/how option 1 can be applicable for M=1 and M= ½ - Option 2: Support repeated PDCCH candidates in the two slots n0 and n0+X [or n0+1 and n0+1+X ] associated with the same SSB index ( n0 as defined in section 13 of TS 38.213) - Value of X>1, predefined or configured - FFS: Value of X - Repeated PDCCH candidates share the same aggregation level (AL), coded bits and same candidate index - FFS: Backward compatibility to legacy UE - Option 3: - The PDCCH candidates in slots n0 associated respectively with different SSB indexes are repetitions of each other and share the same aggregation level, coded bits and same candidate index - For M=1/2 and M=1, the repeated PDCCH candidates in two consecutive slots associated with different SSB indexes; - For M=2, the PDCCH candidates in slots n0+1 associated respectively with different SSB indexes are repetitions of each other and share the same aggregation level, coded bits and same candidate index - Option 4: Option 2 with cross SSB beam repetition support - The PDCCH candidates in slots n0 associated respectively with different SSB indexes are repetitions of each other and share the same aggregation level, coded bits and same candidate index

To improve PDCCH coverage in NTN scenarios, time-domain PDCCH repetition is supported as an effective coverage enhancement scheme to provide coding gain. To eliminate the coverage gap of up to 2dB, supporting 2 PDCCH repetitions is sufficient, which theoretically can provide nearly 3dB of performance gain by additional PDCCH repetition transmission.

As inter-slot repetition for Type-0 PDCCH CSS is supported, one key issue is to discuss the detailed mechanism on how to configure the time/frequency domain resources for PDCCH repetitions, and indicate to UEs to perform PDCCH monitoring in the associated COERSET and search space successfully. Specifically, the resource allocation for Type-0 PDCCH monitoring occasion is indicated through controlResourceSetZero and searchSpaceZero in MIB instead of RRC configuration, according to the operation of UE monitoring Type-0 PDCCH CSS sets specified in TS38.213, for operation without shared spectrum channel access and for the SS/PBCH block and CORESET multiplexing pattern 1, a UE monitors PDCCH in the Type0-PDCCH CSS set over two slots. For SS/PBCH block with index i, the UE determines an index of slot n0 as n0=O∙2μ+i∙MmodNslotframe,μ. Considering the 15kHz SCS is assumed for R19 NTN evaluation, the two slots including the associated Type0-PDCCH monitoring occasions are slots n0 and n0+1. M, O, and the index of the first symbol of the CORESET in slots n0 and n0+1 are provided by Table 13-11 and Table 13-12 configured with set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set.

Considering Option 1 for PDCCH repetition for Type-0 PDCCH CSS, if supporting repeated PDCCH candidates in the two consecutive slots n0 and n0+1 associated with the same SSB index, the PDCCH repetition can be transmitted at the same symbols of each slot. While for the cases when parameter M=1/2 or M=1 is set for PDCCH monitoring occasions, the Type0-PDCCH monitoring occasions corresponding to two adjacent SSBs will overlap in the time slot, which may require different active beams to transmit Type-0 PDCCH associated with different SSBs to avoid collision.

For Option 2, repeated PDCCH candidates in the two n0 and n0+X can effectively avoid the overlapping issue of PDCCH monitoring occasions associated with adjacent SSBs. To avoid additional signalling overhead, the slot offset X between the PDCCH repetitions can be predefined. While for M=1/2, M=1 and M=2, the number of multiplexing slots for adjacent SSBs is different, as shown in Figure 2. To reduce the latency of UE receiving PDCCH repetitions, it is preferred that slot offset X can be predefined separately for each M value. For example, if 4 SSBs per SSB burst is transmitted within SSB periodicity, for M=1/2, the X can be predefined as 2, and for M=1, the X can be predefined as 4. While for M=2, since there is no overlapping issue of PDCCH repetition transmitted in slot n0 and n0+1, the repeated PDCCH candidates in the two consecutive slots n0 and n0+1 associated with the same SSB index can be supported.

And to enable the PDCCH repetition, one simple way is to enable PDCCH repetition via 1 spare bit in MIB, which directly indicates UE to monitor 2 PDCCH repetitions on slot n0 and n0+X.

Figure 2.2-1. An example of PDCCH monitoring occasions with M=1/2, 1 and 2

Proposal 5: For Type-0 PDCCH coverage enhancements, inter-slot PDCCH repetition can be enabled via 1-bit indication in the MIB.

Proposal 6: For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1, support repeated PDCCH candidates in the two slots n0 and n0+noffset, and the slot offset (noffset≥1) between PDCCH monitoring window can be predefined separately for different M values.

Furthermore, for the enhancement of Type0A-PDCCH, Type1-PDCCH and Type2-PDCCH, we think both inter-slot PDCCH repetition and intra-slot PDCCH repetition can work. The intra-slot PDCCH repetition scheme has been supported in multi-TRP scenarios to improve reliability, two linked Search Spaces within the same slot are configured with the same periodicity and offset (monitoringSlotPeriodicityAndOffset), and the same duration. However, as clarified in current specification, the following SS sets cannot be linked with another SS set for PDCCH repetition: SS set 0, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, searchSpaceMCCH, searchSpaceMTCH, peiSearchSpace, and sdt-SearchSpace. PDCCH repetition by search space linking for scheduling system information has not been supported.

From our perspective, intra-slot PDCCH repetition for Type0A-PDCCH, Type1-PDCCH and Type2-PDCCH can be enabled via system information, the two PDCCH repetition can be configured with the same numbers of symbols and RBs within a slot, and there is no need to introduce the linking between the PDCCH candidates. Furthermore, intra-slot PDCCH repetitions will significantly affect PDSCH scheduling and transmission, as they occupy the resources allocated for PDSCH in the same slot. While inter-slot PDCCH repetition can be carried out to solve the payload issue and mitigate monitoring burden of UE within one slot.

Proposal 7: For Type0A-PDCCH, Type1-PDCCH and Type2-PDCCH coverage enhancements, inter-slot PDCCH repetition can be supported.

Figure 2.2-2 provides an illustration of inter-slot PDCCH repetition. In this example, both PDCCH repetitions carry the same payload, such as both DCIs having the same TDRA field with K0=0.

Figure 2.2-2. An example of inter-slot PDCCH repetition

For a R19 UE that supports inter-slot PDCCH repetition capability, it determines the slot (Ks) allocated for the scheduled PDSCH based on the slot n of the first PDCCH repetition, which is referred to as the reference PDCCH, and the indicated parameter K0.

However, concerns regarding backward compatibility were raised in the last RAN1 meeting. For instance, as shown in Figure 3, when a legacy UE decodes the non-reference PDCCH (e.g., the second PDCCH repetition), it may expect PDSCH reception in slot n+1. However, gNB does not schedule PDSCH in slot n+1, which may lead to issues.

To avoid the aforementioned backward compatibility issue for inter-slot PDCCH repetition, potential enhancements to prevent legacy UEs from decoding the non-reference PDCCH can be considered, such as:

- Option 1: The non-reference PDCCH is not located in the CSS of the legacy UE.

- Option 2: Modify the scrambling sequence of the non-reference PDCCH before modulation (as specified in TS 38.211, Clause 7.3.2.3) to ensure that the legacy UE cannot decode it.

  - For example, in the current specification, the scrambling sequence of PDCCH before modulation (as specified in TS 38.211,Clause 7.3.2.3) is initialized with cinit=nRNTI×216+nID mod 231. For CSS, nRNTI=0.

  - To address the backward compatibility issue, nRNTI can be modified, such as setting nRNTI=0 for the reference PDCCH and nRNTI≠0 for the non-reference PDCCH.

Proposal 8: To avoid backward compatibility issues for inter-slot PDCCH repetition, potential enhancements to prevent legacy UEs from decoding the non-reference PDCCH can be considered, such as:

- Option 1: The non-reference PDCCH is not located in the CSS of the legacy UE.

- Option 2: Modify the scrambling sequence of the non-reference PDCCH before modulation (as specified in TS 38.211, Clause 7.3.2.3) to ensure that the legacy UE cannot decode it.

PDSCH with SIB1

For PDSCH in NTN scenarios, when Set1-3 FR1 parameters are assumed, it is agreed to focus on coverage enhancement with the CNR target of -8dB. It is observed that there are 4.1/6.5dB coverage gap for PDSCH carrying SIB1 with different payload sizes, and 3.5dB coverage gap for PDSCH carrying SIB19. PDSCH repetitions for SIB1 transmission has been supported with fixed number of 2 repetitions within a 20 ms duration. The candidate methods to support PDSCH repetition of SIB1 have been summarized during RAN1#120 meeting, which are listed as below.

RAN1#119 Agreement For link level enhancement of PDSCH with SIB1: - Support PDSCH repetitions within 20 ms duration - The number of repetitions is fixed to 2 repetitions - Further discuss the specification impact for at least the following: - Procedure and signaling (enabling repetitions, associated time resource determination, etc.) - Note 1: without the above PDSCH repetitions, the coverage gap is 2.2 dB to 4.6 dB depending on SIB1 size. - Note 2: Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level. - Note 3: the above is not related to multiple SIB1 transmissions across 20 ms periodicities of SSB, which may not be available when the SSB periodicity is 160 ms or larger (if supported) depending on the SSB and CORESET multiplexing pattern. RAN1#120 Agreement For SIB1 link level enhancement, RAN1 to consider the following options: - Option 1: PDSCH repetition of SIB1 is transmitted within the same slot as the type0-CSS PDCCH repetition. - UE supporting SIB1 PDSCH coverage enhancement assumes that the PDCCH and associated PDSCH to be repeated in both slots where the corresponding PDCCHs are transmitted. - Each PDSCH SIB1 repetition is within the same slot of each PDCCH candidate for scheduling DCI - The two associated PDSCHs have the same RV - Option 2: Option 1 and an additional PDSCH with SIB1 repetition can occur after the slot of type0-PDCCH CSS repetition. - FFS: How to schedule the SIB1 repetition - Option 3: The repetition of PDSCH with SIB1 is indicated by the scheduling PDCCH - PDSCH is repeated in two slots Note: Backward compatibility should be maintained

Since the number of repetitions is fixed to 2 repetitions, 1 reserved bit in DCI format 1_0 with CRC scrambled by SI-RNTI can be used to enable the PDSCH repetitions scheduled by corresponding DCI. And the PDSCH repetitions can be transmitted in consecutive two slots or two slots with predefined offset. Although the inter-slot PDCCH repetition for Type-0 PDCCH scheduling SIB1 has been supported, it is preferred the repetition methods can be determined separately for different DL channels/signals.

Proposal 9: Considering DL coverage enhancements for PDSCH carrying SIB1/SIB19 in NTN scenarios, enable PDSCH repetition by 1 reserved bit in DCI format 1_0 with CRC scrambled by SI-RNTI.

PDSCH with Msg4

For PDSCH with Msg4, PDSCH repetition is supported for NTN link-level coverage enhancements. The target coverage enhancement is to bridge the gap of 2.8 dB. And three options for Msg4 PDSCH repetition mechanisms were proposed for further discussion.

RAN1#119 Agreement For PDSCH with Msg4 Link level enhancement: - Support PDSCH repetition - FFS: signalling design including number of repetitions - FFS: impact on UE capability - Note: the target coverage enhancement to bridge the gap with respect to single Msg4 transmission is 2.8 dB - Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level. RAN1#120 Agreement For Msg4 PDSCH repetition support, RAN1 to consider: - Option 1: UE specific repetition indication via DCI - Option 2: Msg4 repetition is configured by SIB1 - Option 3: Msg4 PDSCH repetition is implicitly determined by SIB1 PDSCH repetition

From our perspective, one straightforward way is to carry the PDSCH repetition indication in system information as Option 2, including the signalling to enable PDSCH repetition and repetition factor if multiple repetition factors are supported for PDSCH Msg4 repetition.

To improve scheduling flexibility, UE-specific PDSCH repetition mechanism can also be supported. However, considering there is no reserved bits in current DCI format 1_0 with CRC scrambled by TC-RNTI, if the repetition factor can be identified as one fixed value, then 1-bit new field can be added in DCI scheduling PDSCH Msg4. While if multiple repetition factors are supported, how to indicate the repetition factor needs to be further discussed.

Proposal 10: For PDSCH with Msg4 link-level enhancement, the following solutions can be considered to support PDSCH repetition,

- Enable PDSCH repetition for Msg4 via SIB, and the repetition factor can be indicated if multiple repetition factors are supported.

- DCI scheduling PDSCH Msg4 can be used to enable PDSCH repetition with a fixed number of PDSCH repetitions.

- Conclusions

In this contribution, we share our views on DL coverage enhancement for both system-level study and link-level study in NTN scenarios. The observations and proposals are summarized as follows.

Observation 1: Different SSB periodicities can be used to solve the backward compatibility issue, wherein one set of legacy 20ms SSB periodicity can be reserved for legacy NTN UE in some beam footprints, and other sets of longer SSB periodicities can be used to serve Rel-19 NTN UE in other beam footprints.

Observation 2: For LEO-600 Set1-1/1-2/1-3, longer revisit period can be considered to improve the coverage ratio.

Observation 3: Within the dwelling window, the SSB and necessary system information (e.g., SIB1, SIB19) can be broadcasted for the target beam footprints, which can be utilized by UEs to access to the satellite/gNB within the dwelling window. For the residual slots of the dwelling window, it can be used for the dynamic scheduling for the same beam footprints as SSB or other beam footprints.

Observation 4: To provide services to all UEs covered by the satellite footprint, TDM beam scheduling/beam hopping can be performed to illuminate different beam footprint for SSB and necessary system information (e.g., SIB1, SIB19) transmission.

Observation 5: The time span of the dynamic scheduled dwelling window for a given beam footprint can be allocated based on UE distribution and traffic distribution.

Proposal 1: Considering the extended SSB periodicity of 160ms, the periodicity of Type0-PDCCH monitoring and UE measurement needs to be extended correspondingly.

Proposal 2: Support the system level enhancements to improve the coverage ratio in NR-NTN phase 3, with the consideration of introducing the revisit periodicity and dwelling window for other channels/signals to adapt to SSB periodicity extension.

Proposal 3: Potential enhancements to associate valid ROs among the entire set of configured ROs with an SSB index corresponding to a beam footprint should be considered.

Proposal 4: To improve the association of valid ROs among the entire set of configured ROs with an SSB index corresponding to a beam footprint, the following enhancements can be considered:

- Option 1 (Validity Window Based): A validity window associated with a specific SSB index is introduced. ROs configured within this validity window are deemed valid.

- Option 2 (Mapping Function Based): A mapping function between the SFN of a specific SSB index and the configured RO index(es) is introduced. The calculated ROs are valid for the corresponding SSB index.

  - Alternatively, a mapping bitmap can also be considered.

Proposal 5: For Type-0 PDCCH coverage enhancements, inter-slot PDCCH repetition can be enabled via 1-bit indication in the MIB.

Proposal 6: For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1, support repeated PDCCH candidates in the two slots n0 and n0+noffset, and the slot offset (noffset≥1) between PDCCH monitoring window can be predefined separately for different M values.

Proposal 7: For Type0A-PDCCH, Type1-PDCCH and Type2-PDCCH coverage enhancements, inter-slot PDCCH repetition can be supported.

Proposal 8: To avoid backward compatibility issues for inter-slot PDCCH repetition, potential enhancements to prevent legacy UEs from decoding the non-reference PDCCH can be considered, such as:

- Option 1: The non-reference PDCCH is not located in the CSS of the legacy UE.

- Option 2: Modify the scrambling sequence of the non-reference PDCCH before modulation (as specified in TS 38.211, Clause 7.3.2.3) to ensure that the legacy UE cannot decode it.

Proposal 9: Considering DL coverage enhancements for PDSCH carrying SIB1/SIB19 in NTN scenarios, enable PDSCH repetition by 1 reserved bit in DCI format 1_0 with CRC scrambled by SI-RNTI.

Proposal 10: For PDSCH with Msg4 link-level enhancement, the following solutions can be considered to support PDSCH repetition,

- Enable PDSCH repetition for Msg4 via SIB, and the repetition factor can be indicated if multiple repetition factors are supported.

- DCI scheduling PDSCH Msg4 can be used to enable PDSCH repetition with a fixed number of PDSCH repetitions.

- References

- RP-234078, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3, December 11-15, 2023.

- RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, December 9-12, 2024.

- Chairman's Notes RAN1#120, February 17th – 21st, 2025.

- Chairman's Notes RAN1#116, February 26th - March 1st, 2024.

- Chairman's Notes RAN1#119, November 18th – 22nd, 2024.