3GPP TSG RAN WG1 #120bis		R1-2502266

Wuhan, China, April 7th - 11th, 2025

Source:	OPPO

Title:	Discussion on supporting of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands

Agenda Item:	9.11.2

Document for:	Discussion and Decision

Introduction

In RAN1 meeting #118 and #120, the following agreements/conclusion/working assumption regarding support of RedCap and eRedCap UEs with NR NTN were made.

Agreement To mitigate the collisions of case3 and case4, the following TA reporting enhancements for Rel-19 NTN HD-FDD (e)Redcap UE can be further studied: - Finer TA report granularity - Smaller TA offset threshold - Trigger a TA report when the collision is detected at the UE - TA reporting with a new triggering mechanism from gNB - TA drifting rate reporting Note: The benefit, complexity, power consumption and signaling overhead of each scheme is to be further investigated. Note: other solutions can also be studied. Agreement When network indicates UL overriding DL in RRC-Connected mode for collision case 3, one UE-specific RRC parameter is used to indicate overriding for all the applicable other use cases except for collisions with Type-0/0A/1/2-PDCCH CSS. Conclusion For Rel-19 HD-FDD (e)Redcap UE with CG-SDT procedure ongoing in RRC-inactive mode, for collision case 3, - Handling of collision with PDCCH CSS in RRC-inactive mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note. Note: UE shall comply to the following existing procedure in 38.331: - UEs in RRC_INACTIVE while SDT procedure is not ongoing shall monitor for SI change indication in its own paging occasion(s) that the UE monitors as specified in TS 38.304 [20]. - UEs in RRC_INACTIVE while SDT procedure is ongoing shall monitor for SI change indication in any paging occasion at least once per modification period, if the initial downlink BWP on which the SDT procedure is ongoing is associated with a CD-SSB. - ETWS or CMAS capable UEs in RRC_INACTIVE while SDT procedure is not ongoing shall monitor for indications about PWS notification in its own paging occasion every DRX cycle. - ETWS or CMAS capable UEs in RRC_INACTIVE while SDT procedure is ongoing shall monitor for indication about PWS notification in any paging occasion at least once every defaultPagingCycle. Working assumption For Rel-19 NTN HD-FDD (e)Redcap UE in RRC connected mode, the following handling rule for collision case 4 is supported: - Handling of collision with PDSCH (at least for system information) scheduled by Type-0/0A[/1][/2]-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note. - FFS: handling of PDCCH ordered PRACH transmission - For other use cases, default priority rule for collision case 4 in RRC-Connected mode is that [DL or UL] is prioritized. - Network is allowed to indicate [UL or DL] overriding [DL or UL] for all cases - This is signaled by one UE specific RRC parameter - Note: if DL is prioritized, the DL prioritization applies only if the UL cancellation timeline can be satisfied, otherwise UL is prioritized. Note: UE shall comply to the following existing procedure in 38.331: - UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

In this contribution, we further provide our views on supporting RedCap and eRedCap UEs with NR NTN, including priority rule for case 4 and TA reporting enhancements.

Priority rule between dynamically scheduled DL reception and UL transmission

For UE in RRC-Connected mode, RACH procedure may be triggered due to loss of UE uplink synchronization, or handover, or beam failure, or beam recovery, etc. If RACH procedure is not finished, the reliability of UE’s downlink/uplink transmission may be reduced. For this reason, RACH procedure should have higher priority if msg1/msgA/msg2 are collided with dynamical DL or UL transmissions, i.e., PDSCH scheduled by Type-1-PDCCH CSS should have a higher priority than dynamically scheduled UL transmission, and PDCCH ordered PRACH transmission should have a higher priority than dynamically scheduled DL reception.

Proposal 1: For UE in RRC-Connected mode with collisions between PDSCH scheduled by Type-1-PDCCH CSS and dynamically scheduled UL transmission, PDSCH scheduled by Type-1-PDCCH CSS should have a higher priority.

Proposal 2: For UE in RRC-Connected mode with collisions between PDCCH ordered PRACH transmission and dynamically scheduled DL reception, PDCCH ordered PRACH transmission should have a higher priority.

According to UE’s paging monitoring behaviour in 38.331, UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space... It means UE can flexibly select which PO to monitor. Therefore, handling of collision with PDSCH scheduled by Type-2-PDCCH CSS in RRC-Connected mode can be left to UE implementation.

Proposal 3: For UE in RRC-Connected mode with collisions between PDSCH scheduled by Type-2-PDCCH CSS and dynamically scheduled UL transmission, it is left to UE implementation to prioritize UL or DL.

For other use cases, currently the consensus is to define a default higher priority for DL or UL, and network is allowed to indicate whether the default higher priority being overridden. In our view, we slightly prefer UL as the default higher priority considering PUSCH processing timeline. However, with the note that the DL prioritization applies only if the UL cancellation timeline can be satisfied, we can also accept DL as the default higher priority.

Proposal 4: For other use cases, default priority rule for collision case 4 in RRC-Connected mode is that UL is prioritized, and network is allowed to indicate DL overriding UL for all case.

Study on TA reporting enhancements

The following TA reporting enhancements to mitigate the collisions of case3 and case4 were discussed in previous RAN1 meeting:

- Finer TA report granularity

- Smaller TA offset threshold

- Trigger a TA report when the collision is detected at the UE

- TA reporting with a new triggering mechanism from gNB

- TA drifting rate reporting

According to Rel-17 TA reporting mechanism, the TA report granularity is 1ms, and the minimum value of the TA offset threshold can be configured to 0.5ms. Based on the two parameters, it can be calculated that the maximum error between the reported TA and the actual TA would be ±1.5ms. The transition time from RX to TX or from TX to RX is about 13µs. Therefore, for one allocated target resource, if another resource allocated by gNB in a different direction is 1.513ms earlier or later than the target resource, there would be no collision at UE side. That is to say, with such kind of TA reporting configuration, if the gNB scheduling DL and UL transmissions with an interval larger than or equal to 1.513ms between the two transmissions for a HD-UE, there could be no collisions at UE side.

Observation 1: For TA reporting configuration with 0.5ms TA offset threshold, the collisions at UE side may be avoided if an interval between DL and UL scheduling is considered and the interval value is larger than or equal to 1.513ms.

If we consider finer TA report granularity and smaller TA offset threshold, e.g., the TA report granularity is 0.25ms, and the TA offset threshold is 0.25ms, then the maximum error between the reported TA and the actual TA would be ±0.5ms. The collisions at UE side could be avoided if the gNB scheduling DL and UL transmissions with an interval larger than or equal to 0.513ms between the two transmissions for a HD-UE, as discussed above. However, more TA reporting resources and overhead are required for the enhanced mechanism. Assume the maximum delay variation as seen by the UE is ±40µs/s in LEO-600 transparent payload case according to TR38.821, with 0.5ms TA offset threshold, one UE needs to report TA approximately every 0.5ms/80µs/s ≈ 6 seconds; while with 0.25ms TA offset threshold, the UE needs to report TA approximately every 3 seconds. The reporting resources are doubled compared to legacy mechanism. On the other hand, since the report granularity is reduced from 1ms to 0.25ms, additional 2 bits overhead are needed for each reporting, and a new TA reporting format should be specified. It is not worth making so much effort to just reduce the scheduling interval from 1.513ms to 0.513ms.

Observation 2: For TA reporting enhancements with finer TA report granularity and smaller TA offset threshold, the overhead is huge, the benefit is small, and it is not worth enhancing.

As discussed above, collision avoidance at gNB side depends on the configuration of TA report granularity and TA offset threshold for a HD-UE. Triggering a TA report when the collision is detected at the UE would not impact gNB scheduling, since the gNB is unable to know the reported TA is triggered by a collision detection or other events. The motivation and benefit of triggering a TA report when the collision is detected at the UE is unclear.

Observation 3: The gNB is unable to know the reported TA is triggered by a collision detection or other events, so the motivation and benefit of triggering a TA report when the collision is detected at the UE is unclear.

Regarding TA reporting with a new triggering mechanism from gNB, it is not clear how the new mechanism works. In our view, with current TA triggering and reporting mechanism, it is possible for gNB to mitigate the collisions of case3 and case4 at UE side. There is no need to introduce a new triggering mechanism from gNB.

Observation 4: The gNB may mitigate the collisions of case3 and case4 at UE side with current TA triggering and reporting mechanism, and a new triggering mechanism from gNB is not needed.

From UE perspective, TA drifting rate reporting requires UE to construct a model to calculation TA drifting rate, which causes a lot of implementation complexity and UE cost. Besides, whether gNB can obtain an accurate TA from TA drifting rate is doubtable. This method is not preferred.

Observation 5: TA drifting rate reporting causes a lot of implementation complexity and UE cost, and whether gNB can obtain an accurate TA from TA drifting rate is doubtable.

Based on the above discussion, we prefer to use legacy Rel-17 TA reporting mechanism to mitigate the collisions of case3 and case4.

Proposal 5: Support legacy Rel-17 TA reporting mechanism to mitigate the collisions of case3 and case4.

Conclusion

In this contribution, we further provide our views on supporting RedCap and eRedCap UEs with NR NTN, including priority rule for case 4 and TA reporting enhancements.

The following observations are provided.

Observation 1: For TA reporting configuration with 0.5ms TA offset threshold, the collisions at UE side may be avoided if an interval between DL and UL scheduling is considered and the interval value is larger than or equal to 1.513ms.

Observation 2: For TA reporting enhancements with finer TA report granularity and smaller TA offset threshold, the overhead is huge, the benefit is small, and it is not worth enhancing.

Observation 3: The gNB is unable to know the reported TA is triggered by a collision detection or other events, so the motivation and benefit of triggering a TA report when the collision is detected at the UE is unclear.

Observation 4: The gNB may mitigate the collisions of case3 and case4 at UE side with current TA triggering and reporting mechanism, and a new triggering mechanism from gNB is not needed.

Observation 5: TA drifting rate reporting causes a lot of implementation complexity and UE cost, and whether gNB can obtain an accurate TA from TA drifting rate is doubtable.

The following proposals are provided.

Proposal 1: For UE in RRC-Connected mode with collisions between PDSCH scheduled by Type-1-PDCCH CSS and dynamically scheduled UL transmission, PDSCH scheduled by Type-1-PDCCH CSS should have a higher priority.

Proposal 2: For UE in RRC-Connected mode with collisions between PDCCH ordered PRACH transmission and dynamically scheduled DL reception, PDCCH ordered PRACH transmission should have a higher priority.

Proposal 3: For UE in RRC-Connected mode with collisions between PDSCH scheduled by Type-2-PDCCH CSS and dynamically scheduled UL transmission, it is left to UE implementation to prioritize UL or DL.

Proposal 4: For other use cases, default priority rule for collision case 4 in RRC-Connected mode is that UL is prioritized, and network is allowed to indicate DL overriding UL for all case.

Proposal 5: Support legacy Rel-17 TA reporting mechanism to mitigate the collisions of case3 and case4.

Reference

- R1-2500444, Discussion on supporting of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands, OPPO.