3GPP TSG RAN WG1 #120bis			R1-2502467

Wuhan, China, 7th – 11th April 2025

Source:	Panasonic

Title: 	A-IoT Physical Channels Design on Modulation Aspects

Agenda Item:		9.4.1

Document for:	Discussion, Decision

# Introduction

This contribution discusses the physical channels design on modulation aspects including discussions on target symbol rate, CP handling and CW frequency for R2D.

# Discussion

Target symbol rate (maximum M value)

The following has been agreed in last RAN1#120 meeting.

Agreement For R2D, for the OOK-4 modulation for M-chip per OFDM symbol transmission: - The maximum M value is no less than 16, and to be down-selected from 32, 24 or 16 at RAN1#120bis.

As Rel.19 is only focusing on the device 1, the group should target a competitive bit rate with smaller coverage comparing with device 2a/2b in future Rel.20. Then regarding to the maximum value for M, in order to be competitive UHF RFID, we think 16 is a bit small. The value of 24 or higher should be supported regardless of CP handling.

Proposal 1: M=24 or higher should be target regardless of CP handling in order to be competitive with UHF RFID.

CP Insertion / Handling

The following has been agreed during last RAN1#120 meeting for CP handling.

Agreement For CP handling which retains subcarrier orthogonality, the candidate methods are clarified as follows: - For Method Type 1 (as per TR 38.769) - For purposes of evaluating Method Type 1, it is assumed the device is aware of or determines the boundary of an OFDM symbol using the R2D timing acquisition signal (R-TAS), at least for Alt M1-1 - Detail is up to R2D preamble design - Using Alt M1-1 and/or Alt M1-2 is up to device implementation as per TR 38.769 - FFS: applicable M values - For Method Type 2 Alt M2-1 (as per TR 38.769) - Detail design of Alt M2-1-1 or Alt M2-1-2 from the followings: - Alt M2-1-1 - Candidate 1: - For M less than 6: Use Alt M1-1 - For M >=6: circular shift of the M chips’ samples before CP insertion at the reader side in addition to bit/chip skipping/dropping at the device side after bit decoding - Candidate 2: - Where a known signal shape in the tail-portion of the R2D signal is copied to the CP. - Candidate 3: - Insert padding chips at both start and end OOK chips or only at end OOK chips - Alt M2-1-2 - Candidate 4: - Change a=b≠c case to a=b=c case by changing OOK-4 M value to adjacent M +/- 1 for OFDM symbol n. - where a, b and c is the last chip of symbol n, first chip of symbol n+1 and last chip of symbol n+1. - Candidate 5: - Odd number of chips of PRDCH is transmitted in the first OFDM symbols, while M value is kept unchanged during the remaining OFDM symbols. - FFS: applicable M values with or without combination of Method Type 1 For CP handling which does not retain subcarrier orthogonality, the candidate methods are clarified as follows: - For Method Type 2 Alt M2-2 (as per TR 38.769) - FFS detail design of Alt M2-2 from the following: - Candidate 1: - A duration of OFDM symbol with CP has integer number of OOK symbols. CP is not copied from the end of OFDM symbol. Total 14 OFDM symbols fit in a slot. For further down-selection among candidate methods which retains subcarrier orthogonality, companies are encouraged to provide evaluation (including overhead) among the followings to RAN1#120bis: - Method Type 1 - Method Type 2 Alt M2-1-1 with or without combination of Method Type 1 - Method Type 2 Alt M2-1-2 with or without combination of Method Type 1 - Note: The evaluation for CP handling at least includes a M value from {2,4,6,8}, a M value from {12, 16}, and a M value from {24, 32} - Note: In this evaluation, the transmit power of the ON chips remains constant

The following was agreed in RAN4#114 meeting:

Issue 2-2-2: Deployment scenario Agreement: - To specify RAN4 RF requirements based on the assumption that NR UE is only outdoor in Rel-19. - FFS on whether and how to capture the side condition of outdoor UE assumption in the specifications

In anyway, D2R and CW are not orthogonal to each other. The guard band would be required regardless of orthogonality of R2D even if NR 15 kHz SCS and LTE are located in the same indoor. Also, NR 30 kHz SCS or higher is always not orthogonal as well. Although the orthogonality is essential for device 2a/2b in future Rel.20, the device 1 can be designed to be different based on the deployment scenario of smaller coverage and targeting higher bit rate. Therefore, we think R2D is not necessarily required to be orthogonal to NR or LTE.

Proposal 2: R2D is not required to be orthogonal to NR/LTE.

Since R2D is not required to be orthogonal to NR/LTE, then regarding to the further down-selection among candidate methods, our position is to support Method type 2 Alt M2-2 (as per TR 38.769 which does not retain subcarrier orthogonality) to be used for PRDCH, SIP and CAP as a unified solution. The listed candidate 1 can be used that a duration of OFDM symbol with CP has integer number of OOK symbols. CP is not copied from the end of OFDM symbol. Total 14 OFDM symbols fit in a slot

Proposal 3: A unified CP handling should be considered for PRDCH, SIP and CAP.

Proposal 4: The candidate 1 of Method type 2 Alt M2-2 (as per TR 38.769) can be used for PRDCH, SIP and CAP.

If M2-2 is not agreeable as a unified solution, we think there would be separate solutions for PRDCH, SIP and CAP. As higher M value is not supported for Method type 2 (other than Alt M2-2) for PRDCH, Method type 1 should be used for PRDCH and the device removes the position of CP. Since SIP and CAP are required to operate without knowing the position of CP, it should be Method type 2 with low M value.

Proposal 5: For non-unified CP handling, PRDCH can adopt Method type 1 with removing the position of CP, while SIP and CAP can adopt Method type 2 with low M value.

CW frequency

The following has been concluded in last RAN1#120 meeting.

Conclusion For CW waveform, A-IoT WID RP-243326 and TR 38.769 already give us: It is left up to specification drafting phase how to capture the following. - Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769: - Case 1-4 for D1T1-B. ### 6.8.2 CW characteristics Candidates for the CW for D2R backscattering are waveforms consisting of: Waveform 1: A single-tone unmodulated sinusoid, also referred to as 'a single tone'. Waveform 2: Two single tones..

In addition, WID has following description. CW in the following is for backscatter purpose.

- FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum.

From device perspective, a device needs to have pre-knowledge on CW spectrum for energy harvesting. It is desirable that CW spectrum for energy harvesting is same as the one used for PRDCH. This would influence the AGC settling time assumption on SIP. If CW for backscattering is also used for energy harvesting, how to switch between CW reception for energy harvesting and SIP reception or simultaneous reception should be discussed.

Proposal 6: A device needs to have pre-knowledge on CW spectrum for energy harvesting. It is desirable that CW spectrum for energy harvesting is same as the one used for PRDCH.

# Conclusion

In this contribution, the followings proposals are made:

Proposal 1: M=24 or higher should be target regardless of CP handling in order to be competitive with UHF RFID.

Proposal 2: R2D is not required to be orthogonal to NR/LTE.

Proposal 3: A unified CP handling should be considered for PRDCH, SIP and CAP.

Proposal 4: The candidate 1 of Method type 2 Alt M2-2 (as per TR 38.769) can be used for PRDCH, SIP and CAP.

Proposal 5: For non-unified CP handling, PRDCH can adopt Method type 1 with removing the position of CP, while SIP and CAP can adopt Method type 2 with low M value.

Proposal 6: A device needs to have pre-knowledge on CW spectrum for energy harvesting. It is desirable that CW spectrum for energy harvesting is same as the one used for PRDCH.

# Reference

[1] RP-250796, Revised Work Item: Solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA Inc., RAN#107

[2] TR22.840: Study on Ambient power-enabled Internet of Things

[3] TR38.848: Study on Ambient IoT (Internet of Things) in RAN