3GPP TSG RAN WG1 #120bis                                                                R1- 2502602

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	9.3.1

Source:	Apple Inc.

Title:	Discussion on SBFD TX/RX/measurement procedures

Document for:	Discussion/Decision

# Introduction

In RAN#102, a new work item was endorsed in RP- 234035, later revised in RP-241614, to specify Evolution of NR duplex operation: Sub-band full duplex (SBFD) in Rel-19, where one of the objectives is related SBFD TX/RX/measurement procedures as follows (with some arrangements):

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of time location of SBFD subbands in SIB is not precluded - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of frequency domain location of SBFD subbands in SIB is not precluded - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2] - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2] - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon - UL transmissions within UL subband only - DL receptions within DL subband(s) only, except for CLI measurement by the UE outside of the DL subbands Note: When flexible symbols are used, it is not expected that any legacy Uplink symbol is converted to Downlink/SBFD symbols - Enhancement on resource allocation in frequency domain in SBFD symbols, including - resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols - handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols, including - resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots - CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol

In this contribution, we share our views on objectives related to SBFD TX/RX/measurement procedures.

# Remaining issues for SBFD operation at gNB

## Partial RBG

In RAN1#120, there was some discussion to define partial RBGs for Type 0 PDSCH/PUSCH resource allocation for the RBG(s) that partially overlaps with DL/UL sub-bands.  In RAN1#116b, it was agreed

Agreement

For frequency resource allocation Type 0 for PDSCH or PUSCH in a single slot by DCI based scheduling (without repetition or TBoMS), when an assigned RBG overlaps with the subband boundary, only the PRBs within DL usable PRBs are considered to be valid for PDSCH reception and only the PRBs within UL usable PRBs are considered to be valid for PUSCH transmission.

- SBFD aware UE does not expect to be assigned with a RBG for PDSCH which is fully outside DL usable PRBs or a RBG for PUSCH which is fully outside UL usable PRBs.

Later in RAN1#117, it was agreed that

Agreement

For frequency resource allocation Type 0 for PDSCH or PUSCH in a single slot by DCI based scheduling (without repetition or TBoMS), when an assigned RBG overlaps with the subband boundary, the number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only and assigned PRBs within UL usable PRBs only for PDSCH and PUSCH respectively.

And in RAN1#118b, the following Conclusion was made:

Conclusion

Nominal RBG size for PDSCH/PUSCH in SBFD symbols is determined based on the size of DL/UL BWP as legacy.

In our view, and considering all the above agreements, partial RBG is already defined based on the RBG which is not fully within usable PRBs, where the nominal RBG size is based on active BWP size, not based on sub-band size. The following Conclusion is proposed

Proposal 1: Regarding partial RBG, it is concluded that partial RBGs is already supported where nominal RBG size is based on active BWP size, not based on sub-band size.

## PHR report

In RAN1#120, there was some discussion to support separate PHR report for SBFD and non-SBFD, in a single MAC-CE report. Multiple reports in a single MAC-CE for the case of SBFD and non-SBFD is motivated similar to multiple reports for CA, and mTRP operation, where justification is due to different uplink power control parameters associated to different reports.

Proposal 2: Support separate PHR reports associated to SBFD and non-SBFD, when separate UL power control parameters for SBFD symbols and non-SBFD symbol is indicated through the same unified TCI state.

## More on resource configurations for SBFD operation

For uplink transmission (or downlink reception) with Configuration 2, where different occasions may span different symbol types, some agreements were achieved mainly on how to determine frequency domain resource allocation. In general, for a CG-PUSCH in Configuration 2, where different repetitions span different slot types (non-SBFD vs SBFD), different CG-PUSCH parameters may be desired. This is due to the facts that UE may follow different power control parameters due to different interference profile in SBFD and non-SBFD slots, and/or gNB may experience self-interference, wider beam indication etc for SBFD slots vs non-SBFD slots. In Fig. 2, an example is shown where CG PUSCH is configured with RRC parameter periodicity as 2 slots (sym2x14). In this example, MCS corresponding to those occasions within legacy non-SBFD slots (CG_PUSCH 1 and CG_PUSCH_3) is different from the MCS for occasions that happen in a SBFD slot (CG_PUSCH_2).

Figure 2: CG-PUSCH configuration with different MCS configurations

Proposal 3: For a type-1 CG-PUSCH and Configuration 2, ConfiguredGrantConfig may include a pair of parameters for MCS, (mcsAndTBS, mcsAndTBS2) where the second value indicates the parameter for CG-PUSCH transmission in SBFD slot.

Similarly, for Configuration 2 and type-2 CG PUSCH or SPS-PDSCH, or multi-grant PUSCH/PDSCH, the MCS in the activation DCI can be applied to non-SBFD occasions, while a RRC configured offset MCS in respect to the indicated MCS by the activation DCI can be applied to grants in SBFD occasions.

Proposal 4: For Configuration 2 and type-2 CG PUSCH or SPS-PDSCH, or multi-grant PUSCH/PDSCH, DCI indicates a MCS which is applied to non-SBFD occasions, while a RRC configured offset to the MCS indicated by DCI is applied to SBFD occasions.

In RAN1#119, there has been discussions about interaction between cell-specific indication of SBFD symbol configuration, and legacy TDD slot configuration indications via TDD-UL-DL-ConfigDedicated. More precisely, if a D of F symbol in TDD-UL-DL-ConfigCommon is turned to SBFD symbol, what should be UE behavior on that symbol indication via UE specific TDD-UL-DL-ConfigDedicated.

Proposal 5: A SBFD-aware UE does not expect a SBFD symbol to be configured as UL or DL symbol in TDD slot configuration via TDD-UL-DL-ConfigDedicated.

# Conclusion

In this contribution, we provided our views on remaining issues related to UE TX/RX/measurement procedures for SBFD operation at the gNB. Based on what we discussed, the following proposals are made:

Proposal 1: Regarding partial RBG, it is concluded that partial RBGs is already supported where nominal RBG size is based on active BWP size, not based on sub-band size.

Proposal 2: Support separate PHR reports associated to SBFD and non-SBFD, when separate UL power control parameters for SBFD symbols and non-SBFD symbol is indicated through the same unified TCI state.

Proposal 3: For a type-1 CG-PUSCH and Configuration 2, ConfiguredGrantConfig may include a pair of parameters for MCS, (mcsAndTBS, mcsAndTBS2) where the second value indicates the parameter for CG-PUSCH transmission in SBFD slot.

Proposal 4: For Configuration 2 and type-2 CG PUSCH or SPS-PDSCH, or multi-grant PUSCH/PDSCH, DCI indicates a MCS which is applied to non-SBFD occasions, while a RRC configured offset to the MCS indicated by DCI is applied to SBFD occasions.

Proposal 5: A SBFD-aware UE does not expect a SBFD symbol to be configured as UL or DL symbol in TDD slot configuration via TDD-UL-DL-ConfigDedicated.