3GPP TSG-RAN WG1 Meeting #120-bis                                                                                                            	                                          R1-2502829

Wuhan, China, April 7th-11th, 2025

Agenda item:	9.1.1

Source: 	Qualcomm Incorporated

Title: 	Discussion on AI/ML for beam management

Document for:	Discussion and Decision

# Introduction

The Rel-18 study on AI/ML Air Interface [1] has been concluded. The study has been captured into the Technical Report TR 38.843 [2]. Based on the Rel-18 study outcome, at RAN #102, and a new Work Item on artificial intelligence/machine learning for NR air interface has been approved in [3], with the following objectives briefly summarized for beam management:

Provide specification support for the following aspects:

- Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]:

  - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”)

  - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”)

  - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE

NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2.

In this contribution, we discuss various aspects of the above-mentioned goals for the beam management use case.

# Ensuring consistency between training and inference for UE-side models

Let us consider the following aspects from the Rel-18 SI [1] as well as the WID [3], highlighting the importance of ensuring consistency between training and inference regarding NW-side additional conditions for UE-side AI/ML models.

Observation (RAN1 #113) For BM-Case1 and BM-Case2 with a UE-side AI/ML model, consistency / association of Set B beams and Set A beams across training and inference is beneficial from performance perspective. - Note: Whether specification impact is needed is a separate discussion.

WID for AI/ML for air interface (RAN #102) Provide specification support for the following aspects: - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE

We structure this section in the following manner. First, we identify what the NW-side additional conditions are for the BM use case that need to be consistent, and second, we discuss how we can ensure this consistency. Finally, we discuss the scope of consistency for NW-side additional conditions.

## What needs to be consistent across training and inference?

Let us first review the agreement from RAN1 #116-bis on the consistency of NW-side additional conditions across training and inference.

Agreement (RAN1 #116-bis) Further study, for the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, where the NW-side additional condition may at least impact UE assumption on beams of Set A/Set B: - Opt1: Based on associated ID (Referring to AI 9.1.3.3) - FFS on what can be assumed by UE with the same associated ID across training and inference - FFS on how associated ID is introduced, e.g., within CSI framework, or outside of CSI framework - Opt 2: Performance monitoring based - FFS details - Other options are not precluded.

We structure this section in the following manner. First, we discuss what NW-side additional conditions are for the beam prediction use case, and then, we try to focus on the first FFS of Opt1 in the agreement above, i.e., what UE can assume with the same associated ID across training and inference.

### NW-side additional conditions for beam prediction

As a first step in this direction, we should identify NW-side additional conditions that impact consistency of UE-side AI/ML models across training and inference. These aspects have been summarized in Table 1.

Table 1 Categorization of NW-side additional conditions for UE-side models

| NW-side additional conditions for UE-side models |  |
| --- | --- |
| Logical characteristics of gNB beams: e.g.) Set B and Set A indexing/ordering across training and inference | Physical characteristics of gNB beams: e.g.) beam shapes of Set B beams and Set A beams across training and inference |

In Section 2.3, we discuss the scope of consistency for NW-side additional conditions, and our focus in this section is to identify and elaborate on NW-side additional conditions. Now, we expand what we discussed in Table 1 and discuss what needs to be consistent with regards to NW-side additional conditions, and in particular aspects related to input (Set B) and output (Set A) of AI/ML models. For UE-side AI/ML models, various levels of consistency should be ensured for Set A and Set B beams across training and inference. Here are these various levels explained in more detail:

- Order/indexing consistency: UE shall use L1-RSRP with respect to a resource index (RI) associated with Set B, e.g., SetB-RI =𝑚 to determine input value with respect to the 𝑚th input feature, across training & inference; UE shall use L1-RSRP with respect to a resource index (RI) associated with Set A, e.g., SetA-RI=n as label with respect to the 𝑛th output feature during training, and use value from the 𝑛th output feature to derive prediction results with respect to SetA-RI =𝑛 during inference. In the context of wide-to-narrow beam prediction, SetB-RI and SetA-RI would correspond to SSBRI and CRI, respectively. This is applicable to both sub-use cases of spatial beam prediction, namely wide-to-narrow beam prediction and Set B subset of Set A, as well as temporal beam prediction. Order/indexing consistency is illustrated in Figure 1.

- Beam shape consistency: Relative pointing direction and beamwidth difference between physical beams with respect to two resources should be under predefined tolerances, across two Set B resource sets for training and inference, as well as across two Set A resource sets for training and inference.

Based on the discussion above, the following proposal states what needs to be consistent across training and inference with regards to Set A and Set B beams:

#### Proposal 1

For beam prediction for UE-side AI/ML models, consider the following aspects to ensure consistency between training and inference regarding NW-side additional conditions (with regards to Set A, Set B consistency) for inference at UE

- Order/indexing consistency: consistency in ordering of resources (e.g., resource index consistency) for Set B beams and Set A beams, across training and inference.

- Beam shape consistency: For each Set A resource, the difference between pointing direction and beamwidth of the physical beam associated with that Set A resource during training compared to pointing direction and beamwidth of the physical beam associated with that same Set A resource during inference should be under predefined tolerances. Similarly, for each Set B resource, the difference between pointing direction and beamwidth of the physical beam associated with that Set B resource during training compared to pointing direction and beamwidth of the physical beam associated with that same Set B resource during inference should be under predefined tolerances.

Now that we have identified what needs to be consistent, we investigate the issue of consistency through the lens of what UE can assume with the same associated ID across training and inference, which is the focus of next subsection.

AI/ML Model

(ID#X)

Output features

mth input feature

Ground truth label

L1-RSRP wrt SetB-RI=m

nth output feature

L1-RSRP wrt SetA-RI=n

Input features

Training

Set A consistency

Set B consistency

Set B consistency

AI/ML Model

(ID#X)

Output features

mth input feature

L1-RSRP wrt SetA-RI=n

L1-RSRP wrt SetB-RI=m

nth output feature

Inference

Input features

Figure 1 Consistency of Set A and Set B beams across training and inference

### UE assumptions for the same associated ID across training and inference

Let us consider the following agreement from RAN1 #118:

Agreement For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

We address the above FFS in this section. An important aspect of associated ID is what can be assumed by UE for the same associated ID across training and inference. It is important to highlight the fact that when we say “UE assumes” a certain assumption about NW-side additional conditions, it does NOT impose a mandate on the NW side to satisfy that assumption. It basically means UE side does not need to develop a special feature or software to deal with the case in which what UE assumes does not hold true. Additionally, there is no precedent for using the wording “similar” in the specifications, as it will be hard to interpret what “similar” means. In the Spec, we do have UE assumptions regarding same spatial Tx/Rx filters, but not similar. With that said, in our view, we should be discussing UE assumptions in the context of same properties of a DL Tx beam or beam set/list, rather than similar.

Now, with the above background in mind and given the discussions in Section 2.1.1, we consider two solutions in this section:

First, we consider solution based on spatial Tx filter. For the same associated ID across training and inference, for each Set A resource, UE can assume that the same spatial TX filter has been utilized by gNB, across training and inference. Similarly, for each Set B resource, UE can assume that the same spatial TX filter has been utilized by gNB, across training and inference. From a practical standpoint, a certain tolerance level can be considered for the spatial Tx filter used in inference versus training.

Second, we consider a solution based on QCL relationship. To this end, let us review the existing QCL types in Spec first [4]

Table 2 Different QCL types, as defined in [4]

| QCL type | Description |
| --- | --- |
| QCL-TypeA | Doppler shift, Doppler spread, average delay, delay spread |
| QCL-TypeB | Doppler shift, Doppler spread |
| QCL-TypeC | Average delay, Doppler shift |
| QCL-TypeD | Spatial Rx parameter |

Now, when talking about ensuring consistency via QCL relationship of gBN Tx beams, let us clarify what we mean by QCL relationship of Set B beams and Set A beams in the following. Consider the illustration in Figure 2.

Figure 2 Illustration of Set B and Set A beams across training and inference

- QCL relationship of beams in Set A during inference to beams in Set A during training, and QCL relationship of beams in Set B during inference to beams in Set B during training: For this relationship, we are talking about the relationship of SetA-RI = j during training to SetA-RI = j during inference and also the relationship of SetB-RI = 𝑘 during training to SetB-RI = 𝑘 during inference. This consistency is applicable to both spatial (Set B subset of Set A, and wide-to-narrow beam prediction) and temporal beam prediction. Again, referring to Figure 2, the relationship here is among Aj and Aj', (1≤j≤8) for Set A, and among Bk and Bk', (1≤k≤3).

It is important to highlight the fact that the consistencies defined in Section 2.1.1, i.e., order/indexing consistency and beam shape consistency CANNOT be ensured by relying on each of the existing QCL types, e.g., QCL-TypeD. To enable the above-mentioned consistency across training and inference, let us first review what we have in existing Spec [4], that we can leverage and build on top of, going forward. To this end, and as an example, let us review how the QCL types mentioned in Table 2 are utilized to indicate QCL relationship between DMRS of PDSCH and TRS. Up to two QCL types can be used to indicate QCL relationship for a given QCL relationship and the same RS can be used to indicate QCL relationship for two types [4]:

For the DM-RS of PDSCH, the UE shall expect that a TCI-State indicates one of the following quasi co-location type(s):

- 'QCL-TypeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'QCL-TypeD' with the same CSI-RS resource, or

- 'QCL-TypeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'QCL-TypeD' with a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or

- QCL-TypeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without higher layer parameter repetition and, when applicable, 'QCL-TypeD' with the same CSI-RS resource.

Now, given that we have such QCL relationship mechanisms already defined in Spec, our purpose is to leverage such framework for ensuring consistency across training and inference, for which the consistency needs to be ensured for quite different time stamps, as mentioned above.

To facilitate the discussions on this front, our suggestion is to talk about UE assumptions with the same associated ID AND the same Set A (or Set B) resource. This way, we do not need to talk about beam shape consistency and indexing/ordering consistency separately, as the indexing/ordering consistency is baked in within the above assumption. In other words, we discuss UE assumptions about gNB Tx beams with the same associated ID and the same resource index, so that we avoid separate discussions of UE assumptions about beam shape consistency and beam indexing/ordering consistency.

With that said, we propose the following:

#### Proposal 2

For UE-side beam prediction, for the consistency of NW-side additional conditions across training and inference, with regards to FFS on how to define similar properties of a DL Tx beam or beam set/list for the same associated ID:

- Based on spatial Tx filter: For the same associated ID across training and inference, for each Set A resource, UE can assume that the same spatial TX filter has been utilized by gNB, across training and inference. Similarly, for each Set B resource, UE can assume that the same spatial TX filter has been utilized by gNB, across training and inference.

  - Note: a certain tolerance level can be considered for the spatial TX filter used in inference versus training.

- Based on QCL relationship: For the same associated ID across training and inference, for each Set A resource, UE can assume that the gNB Tx beams used across training and inference have a certain QCL relationship. Similarly, for each Set B resource, UE can assume that the gNB Tx beams used across training and inference have a certain QCL relationship.

  - FFS: definition of such QCL relationships.

## How to ensure consistency across training and inference?

So far, we have discussed what NW-side additional conditions are, that need to be consistent across training and inference, as well as what UE can assume for the same associated ID across training and inference. First, let us review the agreement from RAN1 #116-bis which lists two possible methods addressing how to ensure consistency of NW-side additional conditions across training and inference for UE-side models.

Agreement (RAN1 #116-bis) Further study, for the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, where the NW-side additional condition may at least impact UE assumption on beams of Set A/Set B: - Opt1: Based on associated ID (Referring to AI 9.1.3.3) - FFS on what can be assumed by UE with the same associated ID across training and inference - FFS on how associated ID is introduced, e.g., within CSI framework, or outside of CSI framework - Opt2: Performance monitoring based - FFS details - Other options are not precluded.

Ensuring consistency based on associated ID (Opt1 in the agreement above) was supported in RAN1 #118:

Agreement (RAN1 #118) For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

Later, the following agreements were made in RAN1 #120:

Agreement (RAN1 #120) For UE-sided model, for configuring the resource for data collection purpose, support - CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report. - One CSI-ResourceConfigId is configured for Set A. - One CSI-ResourceConfigId is configured for Set B. - Note: UE performs measurement on all resources - One or two associated IDs can be configured in CSI-ReportConfig - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured, - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B - FFS: whether/how to support 'aperiodic' CSI RS Note: This is not related to whether/how to support delivery/transmission of the collected data for training for UE-sided model.

Agreement (RAN1 #120) For UE-sided model, in CSI-ReportConfig for inference - One or two associated IDs can be configured in CSI-ReportConfig - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured, - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B - FFS: At least BM-Case 1, the applicability for 'aperiodic' CSI RS

First, it is important to highlight the fact that we can always do regular performance monitoring with Opt1 to assess how well the AI/ML performance is during inference, but this is a separate aspect compared to Opt2. What we highlight in the following, is why we believe Opt2 as a standalone method cannot ensure the consistency of NW-side additional conditions across training and inference.

Now, we take a closer look into Opt2, and elaborate on major feasibility challenges it entails, at least due to the following reasons:

- Extra burden at the UE for monitoring before the functionality is activated for inference

  - Note that UE has to monitor the performance of a significantly large number of models to determine the best model for current network-side additional conditions.

  - Monitoring-based model selection needs to be performed every time network side additional condition changes. Furthermore, the network may be required to signal to the UE when network-side additional conditions change.

- Will always be a hit/fail method

  - UE cannot determine with certainty the consistency, as without the information about the network-side additional condition, the UE may not be able to download the relevant models

    - Note that UE may only have limited memory to store the models. It may require frequent download/discard of the model to support monitoring-based consistency.

- UE may need to monitor the performance for a significantly large amount of time before UE can determine a suitable model. This will result in a significant delay in determining applicable functionality at the UE.

Observation 1: For UE-side beam prediction, for the consistency of NW-side additional condition across training and inference, ensuring consistency based on performance monitoring is not feasible, because of the following reasons:

- Extra burden at the UE for monitoring before the functionality is activated for inference

  - Note that UE has to monitor the performance of a significantly large number of models to determine the best model for current network-side additional conditions.

  - Monitoring-based model selection needs to be performed every time network side additional condition changes. Furthermore, the network may be required to signal to the UE when network-side additional conditions change.

- Will always be a hit/fail method

  - UE cannot determine with certainty the consistency, as without the information about the network-side additional condition, the UE may not be able to download the relevant models

    - Note that UE may only have limited memory to store the models. It may require frequent download/discard of the model to support monitoring-based consistency.

- UE may need to monitor the performance for a significantly large amount of time before UE can determine a suitable model. This will result in a significant delay in determining applicable functionality at the UE.

With that said, ensuring consistency based on Opt2 is not a viable option, and should be deprioritized.

#### Proposal 3

For UE-side beam prediction, for the consistency of NW-side additional condition across training and inference, deprioritize Opt2: Performance monitoring based.

## Scope of consistency

In this section, we discuss the dimensions along which the NW-side additional conditions discussed in Section 2.1 should be consistent. To this end, we consider the following:

- Consistency on a per-cell level: This means ensuring consistency across training and inference, in which the training and inference for a UE-side model takes place within the same cell. Through assigning the same associated ID across training and inference, gNB can ensure that the same set of NW-side additional conditions would be utilized during inference, as the ones that were used during training, within the same cell. In addition to the NW-side additional conditions mentioned in Section 2.1.1, another example of NW-side additional conditions that may impact the consistency in this context may be gNB down-tilt angle that may vary over time, and hence alter the effective beam shapes of Set B and Set A beams. The following agreement is meant to ensure consistency at least within a cell:

Agreement (RAN1 #118) For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

- Consistency across different cells: when we consider different cells, then we should factor in aspects such as implementation variation across different cells, different products across cells within an infra vendor or across infra vendors, while preserving NW-side proprietary information. Ensuring consistency across different cells can be very useful from the UE perspective, as it can significantly reduce UE’s efforts in finding a suitable AI/ML model in a target cell. Let us assume, as a toy example, that there are two cells whose NW-side additional conditions (at least with respect to UE assumptions on Set B/Set A beams) is the same. In the absence of associated IDs indicating that the two cells have the same NW-side additional conditions, UE-side will have to rely on trial-and-error based methods to figure out whether one model works for both cells. However, when such information (associated IDs indicating that the two cells have the same NW-side additional conditions) is provided to the UE’s data collection, UE-side would at least know that a single model will suffice, at least as far as the NW-side additional conditions are concerned. While this may not be a big deal for the toy example with two cells, the complexity of UE-side engineering will blow up as the number of cells increases. Moreover, if a given UE moves to a new cell for which no data has been collected yet, in the absence of associated consistency information from the second cell, UE will have to rely on trial-and-error based methods to try to identify an applicable AI/ML model that can be utilized during inference. However, when such information (associated ID) is provided to the UE in the new cell, UE would know which model would be applicable. The consistency in this context needs a closer look and analysis to identify methods that such consistency can be ensured.

In summary, while consistency within a cell is useful for UE-side for model development and runtime LCM (e.g., model selection/switching/activation/deactivation), it is not scalable. If the same associated ID at different cells represents different network-side additional conditions, then it will not be feasible for the UE side to collect enough data and to generalize the model across multiple cells. This poses challenges not only on the UE-side model development but also on the runtime LCM. Without sufficient generalization, the UE must download/get the model frequently causing unnecessary interruptions. Therefore, NW-side additional conditions with the same associated ID should be consistent within a PLMN. How the uniqueness of the associated IDs within a PLMN is achieved can be left up to the OAM implementation.

Given the above explanations, we propose the following:

#### Proposal 4

For UE-side beam prediction, for the consistency of NW-side additional condition across training and inference, the NW-side additional conditions with the same associated ID should be consistent within a PLMN.

# Enhancements related to UE-side AI/ML inference

## Reporting details for temporal beam prediction with multiple future time instances

The following agreement was made in RAN1 #120 on temporal beam prediction with multiple future prediction instances:

Agreement (RAN1 #120) For UE-side AI/ML model inference and BM-Case2, for the quantization of a RSRP value of inference results in a report over multiple future time instances, - the largest RSRP value based on prediction of all time instances is the reference RSRP, and differential RSRPs in the report are computed relative to the reference RSRP. - The time instance information of the beam with the largest RSRP are additionally indicated in the report.

In light of the above agreement, we need to define the reporting format including the content and ordering for the CSI fields for the report that carries the inference results for multiple future time instances. To illustrate this, let us consider Figure 3. The related CSI report configuration is associated with T = 3 future temporal occasions and N = 12 candidate resources (prediction set) and may indicate that the UE is to report predicted L1-RSRPs for K0 = 2 resource IDs in each temporal occasion. As we see in Figure 3,  the CSI payload includes a single field that indicates a “leading” reference resource ID and temporal occasion ID pair (e.g., using log2N+log2T bits.  In the subsequent CSI fields, the resource IDs associated with respective temporal occasions are ordered according to temporal occasion IDs or indices. The set of fields that indicate the resource IDs associated with respective temporal occasions are ordered such that information regarding the resource IDs associated with the first (earliest) temporal occasion appears first, information regarding the resource IDs associated with the second temporal occasion appears second, and information regarding the resource IDs associated with the Tth temporal occasion appears Tth.

Furthermore, the information regarding the resource IDs associated with the tth temporal occasion, where 1≤t≤T, may be explicitly signaled via log2N bits. Once the indication of resource IDs is concluded, following the principle agreed in the agreement cited above, the absolute L1-RSRP is included for the leading {Rsc-ID, Occasion-ID} pair, and for the rest of the resources, the differential predicted L1-RSRP is included in the subsequent CSI fields, and the ordering for the differential predicted L1-RSRPs follow the ordering for the resource IDs in the preceding CSI fields.

Figure 3 Content and ordering of the CSI fields in the CSI report including prediction results for multiple future time instances

#### Proposal 5

For UE-side AI/ML inference and BM-Case2, for inference results in a report for multiple future time instances, support the following format

- Include the {resource-ID, Occasion ID pair} for “leading” reference resource ID and temporal occasion ID pair (that has the largest predicted L1-RSRP value) in the first CSI field location.

- For the subsequent fields, the resource IDs associated with respective temporal occasions should be ordered according to temporal occasion IDs or indexes.

  - This means the resource IDs should be ordered such that information regarding the resource IDs associated with the first (earliest) temporal occasion appears first, information regarding the resource IDs associated with the second temporal occasion appears second, and so on.

- Once the indication of resource IDs is concluded, the absolute predicted L1-RSRP is included for the leading {resource-ID, Occasion-ID} pair.

- For the rest of the resources, the differential predicted L1-RSRP is included in the subsequent CSI fields, and the ordering for the differential L1-RSRPs follow the ordering for the resource IDs in the preceding CSI fields.

## FFS on down-selection among Option 3 and Option 4

Let us consider the following agreement:

Agreement (RAN1 #116) For UE-sided model, at least for BM-Case1, for content in the report of inference results, support - Opt 1: Beam information on predicted Top K beam(s) among a set of beams - Opt 2: Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams - At least K=1 and more, FFS on max value - FFS on beam information - FFS on the definition of predicted Top K beam(s) - FFS on definition of reported RSRP when applicable - FFS on other information in the report with potential down selection among the following options - Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams - FFS on the quantization method of probability information - Probability information is the probability of the beam to be the Top 1 or Top K beam - Opt 4: Beam information on predicted Top K beam(s) among a set of beams, RSRP of predicted Top K beam(s) among a set of beams, and confidence information of the RSRP - FFS on definition of reported RSRP - FFS on the definition and quantization method of confidence information - Other options are not precluded. where the set of beams is Set A, i.e., the beams for UE prediction.

We address the FFS on down-selection among Option 3 and Option 4 in the following:

With regards to Opt 4, as an example of confidence information, we can consider the standard deviation (std) associated with an L1-RSRP prediction. This information can be useful for gNB to evaluate the quality of RSRP prediction and make scheduling decisions accordingly. gNB may be interested in knowing the beams with a small std in prediction, such that gNB can be confident relying on the prediction to make scheduling decision. For instance, for traffic with high reliability requirement, gNB may try to schedule on beams with small std. On the other hand, beam IDs with large std in prediction may also be useful at the gNB side. For instance, gNB may schedule RS(s) to measure the actual RSRP(s) in case the std indicates the prediction(s) of the beams are not reliable.

#### Proposal 6

For UE-side beam prediction, for content in the report of inference results, regarding FFS on potential down-selection among Option 3 and Option 4, support Option 4.

# AI/ML performance monitoring

Three flavours of performance monitoring were agreed in Rel-18 SI [1], with regards to UE-side AI/ML models. The following text from TR38.843 discusses these flavours, and Figure 4 summarizes these approaches.

From TR 38.843: For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Type 1 performance monitoring: - Configuration/Signalling from gNB to UE for measurement and/or reporting - UE may have different operations - Option 1 (NW-side performance monitoring): UE sends reporting to NW (e.g., for the calculation of performance metric at NW) - Option 2 (UE-assisted performance monitoring): UE calculates performance metric(s), either reports it to NW or reports an event to NW based on the performance metric(s) - Indication from NW for UE to do LCM operations - Note: At least the performance and reporting overhead of model monitoring mechanism should be considered - Type 2 performance monitoring: - Indication/request/report from UE to gNB for performance monitoring - Note: The indication/request/report may be not needed in some case(s) - Configuration/Signalling from gNB to UE for performance monitoring measurement and/or reporting - If it is for UE side model monitoring, UE makes decision(s) of model selection/activation/ deactivation/switching/fallback operation - Mechanism that facilitates the UE to detect whether the functionality/model is suitable or no longer suitable

Additionally, in RAN1 #117, the following agreement was made in which Type 1 performance monitoring is supported:

Agreement (RAN1 #117) For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring

Figure 4 Summary of three approaches for monitoring of UE-side AI/ML models agreed in Rel-18

In the remainder of this section, we first talk about the reference signals for performance monitoring in Section 5.1. There are two flavours for reference signals for performance monitoring: the first flavour is when the performance monitoring reference signals comprise entire Set A, per performance monitoring instance. In this case, UE can acquire the measurements from entire Set A, and the monitoring procedure and metrics can be defined accordingly. The second flavour for reference signals for performance monitoring is when performance monitoring reference signals are a subset of Set A, per performance monitoring instance. Here, we can no longer assume that UE will have access to all the beam measurements from Set A, and therefore, the follow-up procedures and metrics should be defined accordingly. Then, in Section 5.2, we introduce metrics for performance monitoring. Finally, we briefly discuss performance monitoring reports at the end of the section.

## Reference signal for performance monitoring

In all the agreed three flavours for performance monitoring we have “configuration/signalling from gNB to UE”. For UE-side performance monitoring we have “configuration/signalling from gNB to UE for performance monitoring” and for NW-side and hybrid performance monitoring, we have “configuration/signalling from gNB to UE for measurement and/or reporting”. As discussed in Rel-18, when UE-side AI/ML models are deployed in a certain environment, we should define a mechanism through which the performance of AI/ML models can be monitored. To achieve this goal, an indispensable part of this mechanism is transmission of some auxiliary reference signals (that span the entire Set A or subset(s) of Set A) that would enable the UE to compare the predictions with regards to Set A (prediction target set) to the actual measurements of beams from Set A. We have the following agreement from RAN1 #119 in which dedicated resource set for monitoring is agreed to be supported:

Agreement At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling: - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report - FFS on the timing related issues - UE measures the dedicated resource set(s) for monitoring.

To facilitate the discussions in the remainder of the document, we introduce the terminology of performance monitoring set. For every performance monitoring instance, we call the set of performance monitoring reference signals as “performance monitoring set”. We illustrate this set in Figure 5 for a given performance monitoring instance.

Figure 5 Illustration of performance monitoring set for a given performance monitoring instance

### Performance monitoring reference signals comprise entire Set A, per performance monitoring instance

Let us consider temporal beam prediction as an example, as depicted in Figure 6. Depending on the size of Set A, it may be reasonable to assume that the reference signals available for performance monitoring (depicted in green in Figure 6) span the entire Set A, and UE can measure all of them. If that is the case, then it is reasonable to assume that UE will know the actual Top-K beam IDs as well as their measured L1-RSRPs, and therefore computing inference error (in terms of performance monitoring metrics) is more straightforward as the UE does know the label measurements and therefore there’s no ambiguity in determining the actual best beams from Set A. In the next subsection, we consider the case in which for each performance monitoring instance, performance monitoring reference signals are only a subset of Set A.

Figure 6 Performance monitoring set equal to Set A for performance monitoring instances

### Performance monitoring reference signals are a subset of Set A, per performance monitoring instance

If the size of Set A is large (e.g., 128 or higher) it may not be reasonable to assume that the performance monitoring reference signals are going to span the entire Set A for all beam prediction monitoring instances. In such cases, only a subset of beams from Set A may be available as performance monitoring set, per performance monitoring instance. Even if the entire Set A is available for measurement, it may be unreasonable to ask UE to measure all of them to derive monitoring KPI due to complexity reason, in which case only a subset of Set A may be configured for performance monitoring. As an illustrative example, consider Figure 7 in which the performance monitoring reference signals (depicted in green) span the entire Set A across multiple consecutive performance monitoring instances in a round robin manner.

Figure 7 performance monitoring sets are subsets of Set A in each performance monitoring instance (span Set A in a round-robin manner)

Now that we have discussed reference signals for performance monitoring, we investigate metrics for performance monitoring in the next subsection.

## Metrics for performance monitoring

At least with regards to UE-assisted performance monitoring, one aspect that needs to be investigated is how to define metrics for inference error, based on different assumptions on the availability of reference signals for performance monitoring. As discussed in 5.1, RS for monitoring may span the entire Set A or only a subset of Set A, per performance monitoring instance. The implications of how to define a metric for monitoring the performance should be discussed for such cases.

Let us consider the following agreement from RAN1 #118:

Agreement (RAN1 #118) For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including: - Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring - Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring - Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring - Note: resources for Set B for monitoring are not precluded and can be study. - Note: this is only applicable when the model can predict RSRP - Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam - Note: this is only applicable when the model can generate probability information - FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference. - For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window)

Further to the agreement above, the following agreement was made in RAN1 #118-bis:

Agreement (RAN1 #118-bis) For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), - At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation - FFS: on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric - FFS other alternatives

In the remainder of this section, we address the FFSs in the above agreement. Let us start with the second FFS on the first bullet:

FFS: on other details including how to configure the resource set/resources for monitoring, including, e.g., whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric

As explained above in 5.1.1, taking practical considerations into account, the performance monitoring set may not span the entire Set A, and the metrics should be defined in such a way that this practical consideration is taken into account.

Observation 2: In practice it may not be reasonable to assume that the measurements from the entire Set A will be available within performance monitoring instances.

#### Proposal 7

For beam prediction for UE-side AI/ML models, performance monitoring metrics should also be defined for scenarios in which the performance monitoring set for each performance monitoring instance is a subset of the prediction set, i.e., Set A.

Now, we consider the first FFS on the first bullet:

FFS on detail definition of the metric, including whether/how to configure or define a window for calculation

We address this question in Section 5.2.2, in which we present a detailed metric definition for Alt. 1.

### Metric based on beam prediction accuracy

We consider Alt.1 in this subsection and provide detailed metrics for definition of Top-K beam prediction accuracy with and without margin. Let us first start with Top-K beam prediction accuracy without L1-RSRP margin.

Definition of Top-K beam prediction accuracy without L1-RSRP margin

Let us assume that the total number of performance monitoring instances is equal to N. Let us further assume that Np is the number of performance monitoring instances (out of N) for which the following statement holds:

- Best measured beam ID from performance monitoring set is among the Top-K predicted beams, for a given performance monitoring instance.

Then the Top-K beam prediction accuracy without L1-RSRP margin is defined as Np/N.

Definition of Top-K beam prediction accuracy with L1-RSRP margin

Let us assume that the total number of performance monitoring instances is equal to N. Let us further assume that Np is the number of performance monitoring instances (out of N) for which the following statement holds:

- The highest measured L1-RSRP of Top-K predicted beams that are within the performance monitoring set is within X dB of measured L1-RSRP of best measured beam ID from performance monitoring set, for a given performance monitoring instance.

  - X can be configured by gNB.

Then the Top-K beam prediction accuracy with L1-RSRP margin is defined as Np/N.

As an illustrative example, let us consider Figure 8, in which we consider five performance monitoring instances. The performance monitoring set is composed of 5 beams (out of 9 beams from Set A). Let us consider monitoring instance t+1 and we describe how the metric should be defined for that instance. The inference outcome for t+1 is composed of Top-4 predicted beams that are denoted by red colour. Let us assume that the Top-3 measured beams from performance monitoring set (from lowest to highest L1-RSRP) are A1, A2, A3. We only consider those among the Top-K predicted beams that are within the performance monitoring set (i.e., A4 and A5) and pick the one that has the highest L1-RSRP and let us assume A4 has the highest L1-RSRP. Now, the time instance t+1 is counted towards Np as a successful instance if L1RSRP of A3- L1RSRP of A4<X, in which X is configured by gNB.

Figure 8 Illustration for definition of Top-K beam prediction accuracy with L1-RSRP margin

A special case of the above metric is Top-1 beam prediction accuracy with L1-RSRP margin which can be a useful metric for assessing how good the Top-1 predicted beam is across N performance monitoring instances, by comparing the measured L1-RSRP of the Top-1 predicted beam to the measured L1-RSRP of the best beam from the performance monitoring set for each performance monitoring instance.

On the necessity of defining Top-K beam prediction accuracy with L1-RSRP margin:

Let us consider the following observations from TR 38.843:

For spatial beam prediction with measurements of fixed Set B of beams that of 1/8 of Set A of beams:

Table 3 Top-1 beam prediction accuracy with and without margin for spatial beam prediction

| Top-1 DL Tx beam prediction accuracy |  | Top-1 DL Tx beam prediction accuracy with 1dB margin |  |
| --- | --- | --- | --- |
| # of sources | Beam prediction accuracy (%) | # of sources | Beam prediction accuracy with 1dB margin (%) |
| 7 | 50 | 7 | 70-80 |
| 4 | 60-70 | 1 | 80-90 |
| 5 | 70-80 | 5 | More than 90 |
| 4 | More than 80 |  |  |

As we see from the above observations, defining margins even as low as 1 dB may have a significant impact on the beam prediction accuracy, while not impacting the overall performance of AI/ML-based beam prediction. With no margin, the reported beam prediction accuracy metrics may be misleading to the NW as the beam prediction may be very reasonable for an instance with very small L1-RSRP delta compared to the best beam from the performance monitoring set. For such a reasonable prediction, Top-K beam prediction accuracy with no L1-RSRP margin counts this as a failure, and hence the reported metric may not be very useful to NW. Additionally, the Top-K beam prediction accuracy with L1-RSRP margin is inclusive of the one with no margin once the margin is set to 0 dB.

During discussions it was argued that if needed, Top-K beam prediction accuracy with L1-RSRP margin can be implemented via NW-side performance monitoring, but there’s a significant difference in terms of the report overhead. As an example, consider 16 performance monitoring instances. Even if UE reports only Top-4 beams from the performance monitoring set to the NW per instance, then the total overhead will be 16×7+3×4=304 bits, whereas the overhead for the Top-K beam prediction accuracy with L1-RSRP margin would be 3 or 4 bits reported at the end of 16 instances. So, the overhead difference is significant, and this clearly motivates definition of Top-K beam prediction accuracy with L1-RSRP margin.

#### Proposal 8

For UE-side beam prediction, with regards to Alt 1 of performance monitoring alternatives for UE-assisted performance monitoring, in which the performance monitoring set may be a subset of Set A or full Set A for each performance monitoring instance, define N as the total number of performance monitoring instances, and define Np as the number of performance monitoring instances (out of N) for which the following statement holds:

- Top-K beam prediction accuracy without L1-RSRP margin: best measured beam ID from performance monitoring set is among the Top-K predicted beams, for a given performance monitoring instance.

- Top-K beam prediction accuracy with L1-RSRP margin: The highest measured L1-RSRP of Top-K predicted beams that are within the performance monitoring set is within X dB of measured L1-RSRP of best measured beam ID from performance monitoring set, for a given performance monitoring instance.

  - X can be configured by gNB.

Define the ratio Np/N as Top-K beam prediction accuracy (without or with margin based on which of the above statements holds).

- Note: “performance monitoring set” is the set of RSs that are to be measured for performance monitoring, per performance monitoring instance.

### Metric based on L1-RSRP difference

Now, we consider Alt 2 from the agreement cited in the beginning of this section. The metric proposed in Section 5.2.1, is computed by counting the number of performance monitoring instances for which a certain condition holds and dividing that to the total number of beam prediction instances. The metric proposed in this section, however, is a per-instance metric that is computed for each performance monitoring instance. In Section 4.3, we discuss the interplay between per-instance metrics and metrics computed over a window including multiple performance monitoring instances, at least from the perspective of performance monitoring reports.

The metric that we consider in this subsection (for each performance monitoring instance) is the L1-RSRP difference between measured L1-RSRP of Top-1 measured beam ID from the performance monitoring set and highest measured L1-RSRP of Top-K predicted beams that are within the performance monitoring set. This metric is demonstrative of how much L1-RSRP degradation the UE will experience if it goes with the best beam from predicted Top-K beams that are within the performance monitoring set versus the actual best beam from Set A, for each performance monitoring instance. This is a useful quantitative metric that indicates how well the AI/ML performance is. Providing such L1-RSRP difference values to NW gives NW more visibility (compared to beam prediction accuracy) into how well the AI/ML model is doing. As an illustrative example, if we consider Top-1 beam prediction accuracy, for quite a few performance monitoring instances, the actual Top-1 beam from performance monitoring set may not be the same as the best predicted beam, whereas, considering this L1-RSRP difference metric, as long as the L1-RSRP difference (of actual Top-1 beam from performance monitoring set compared to L1-RSRP of best beam from Top-K predicted beams within performance monitoring set) is within a threshold, then the performance may be good enough, but this is not reflected through beam prediction accuracy metric.

Let us look back at Figure 10 and consider monitoring instance t+1 and we describe how the metric should be defined for that instance. The inference outcome for t+1 is composed of Top-4 predicted beams that are denoted by red colour. Let us assume that the Top-3 measured beams from performance monitoring set (from lowest to highest L1-RSRP) are A1, A2, A3. We only consider those among the Top-K predicted beams that are within the performance monitoring set (i.e., A4 and A5) and pick the one that has the highest L1-RSRP and let us assume A4 has the highest L1-RSRP. Now, the defined metric for t+1 is L1RSRP of A3- L1RSRP of A4.

With that said, Alt. 2 of performance monitoring should be supported.

#### Proposal 9

For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, at least for Option 2 (UE-assisted performance monitoring), support Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring.

Now, we propose the following metric for Alt. 2.

#### Proposal 10

For UE-side beam prediction, with regards to Alt 2 of performance monitoring alternatives for UE-assisted performance monitoring, in which the performance monitoring set may be a subset of Set A or full Set A for each performance monitoring instance, support the following metric for performance monitoring:

- The L1-RSRP difference between measured L1-RSRP of Top-1 measured beam ID from the performance monitoring set and highest measured L1-RSRP of Top-K predicted beams that are within the performance monitoring set, per performance monitoring instance.

Note: “performance monitoring set” is the set of RSs that are to be measured for performance monitoring, per performance monitoring instance.

## Performance monitoring report

Another important aspect is related to the performance monitoring report. For performance monitoring report, first we should determine what information is reported, which depends on the defined metric. In Section 5.2.1, we proposed a metric that was based on beam prediction accuracy which is computed for a given number of performance monitoring instances over a performance monitoring window, and in Section 5.2.2, we defined a per-instance metric based on L1-RSRP difference. We can define a representative metric over a performance monitoring window as a function of the per-instance metrics, and that representative metric can be at least part of the performance monitoring report content. Another aspect related to the performance monitoring report is how frequent it should be. Unlike UE prediction report carrying the results of UE-side beam prediction during inference, the performance monitoring report does not have as strict latency requirements. After assessing how frequent the performance monitoring report should be, the discussions will set the stage to determining the proper carrier for the performance monitoring reports.

#### Proposal 11

For UE-side beam prediction, and for UE-assisted performance monitoring, study details of performance monitoring reports (contents, frequency of report, carrier), at least as a function of performance monitoring metrics.

Now, let us consider the following agreement again:

Agreement (RAN1 #117) For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring

With regards to the FFS on whether to trigger the performance monitoring report based on events for Option 1 and/or Option 2, if we consider each performance monitoring instance, whether UE will report for that particular instance or not can be determined based on whether a certain event has occurred or not, for that particular performance monitoring instance. The benefit of such an event-triggered approach compared to reporting periodically or in a semi-persistent manner is less UE power consumption (due to opportunistically not reporting when the report is not needed), as well as reducing interference due to skipped uplink transmissions. The events can be defined in such a way, so that only when the UE-side AI/ML model is not doing a good job for that performance monitoring instance, then and only then UE reports to NW. Now, what UE reports depends on whether we consider NW-side performance monitoring (in which measurement results from performance monitoring set are reported), or UE-assisted performance monitoring (in which at least the computed metric/KPI at UE is reported). The exact definition of events will at least partially depend on the agreed metrics for performance monitoring, which is still pending. On the other hand, depending on how the events are defined, the triggering events for performance monitoring reports can be based on number of event occurrences within a window.

It is worth noting that there is an ongoing Rel-19 WI for FeMIMO in which the concept of UE-initiated/event-triggered beam reporting is being discussed. We would of course need relevant definitions for “events” for performance monitoring, but the introduced framework therein, as well as the procedures/mechanisms for the reporting can be leveraged for the purpose of performance monitoring report.

With that said, we propose the following:

#### Proposal 12

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, and for Type 1 performance monitoring, study the following events for event-triggered performance monitoring report for both Option 1 and Option 2.

- Event 1: Top-1 measured beam is NOT among Top-K predicted beams, from performance monitoring set.

- Event 2: Measured L1-RSRP of Top-1 predicted beam is NOT within 𝑋 dB of Top-1 measured L1-RSRP, from performance monitoring set.

Note: FFS on triggering condition: whether based on per-instance event occurrence or based on number of event occurrences within a window.

# AI/ML processing capability

AI/ML features may be implemented using different hardware architecture than legacy non-AI features. Thus, it is essential to revisit the timeline and processing criteria. One important aspect regarding AI/ML functionality is that the AI engine may be shared among different use cases, such as BM-Case1, BM-Case2, CSI prediction and positioning, and potentially CSI compression. Considering that the model size may vary in these use cases, they may consume different memory cost and processing unit occupation. With such consideration, timeline and processing criteria needs to be enhanced.

Observation 3: AIML inference may be performed at a dedicated AI engine (e.g., AI/ML accelerator) different from where non-AIML functionalities are executed.

Observation 4: Different AM/ML functions may share a common AI engine. Timeline and processing criteria may be different for different use cases.

In the subsequent subsections, we discuss processing units, priority rules for AI/ML-enabled CSI reports, and finally timeline considerations for AI/ML enabled CSI reports.

## Processing units

Let us first consider the following agreement:

Agreement (RAN1 #118-bis) For UE-side model, existing CPU mechanism is used as a starting point for AI/ML-based CSI processing. - FFS whether the overall CPU should be shared or separately counted between legacy CSI reporting and AI/ML-based CSI reporting, and among AI/ML features/functionalities. - FFS whether it is fully applicable for BM-Case 1 and/or BM-Case 2

Considering legacy CSI framework, the number of supported simultaneous CSI calculations that a UE can perform is denoted by NCPU, and is counted either per CC (SimultaneousCSI-ReportsPerCC) or for all CCs (SimultaneousCSI-ReportsAllCC). If L CPUs are occupied for calculation of CSI reports in a given OFDM symbol, the UE has NCPU-L unoccupied CPUs. Now, let us address the open issues in the above agreements, in the following.

### Shared versus dedicated CPU pool for AI/ML-based CSI processing

Regarding the cost of processing/calculation unit, the first question is to study whether AI/ML processing should count towards legacy CSI processing units or count towards a new dedicate processing pool, or both. In general, there may exist two types of UEs.

One type is known as low-end UE. These UEs may not a have dedicated hardware for AI/ML processing, they may use the common hardware for both AIML and non-AI/ML operations. For this type of UEs, legacy CPU should be used for AIML operation.

Another type of UE is known as high-end UEs. These UEs may have an independent hardware for AI/ML than the legacy algorithms. In this case, a dedicated processing unit pool is needed for all AI/ML functions, including beam management use cases, CSI prediction and CSI compression, and other potential use cases in the future releases.

Furthermore, based on the study in Rel-18/19, the model size and FLOPs may vary significantly across use cases. Thus, it is necessary to apply different CPU/AIML processing unit counting factor for different use cases based on the requirements of each use case. The counting factor should be reported as UE capability.

Furthermore, different AI/ML implementations across different UEs may have quite different model sizes and complexities for the same use case. So, UEs should be allowed to report an appropriate counting factor that reflect the UE’s AI/ML implementation.

      - Proposal 13

With regards to the CPU pool for AI/ML-based CSI processing, support both of the following, as separate UE capabilities:

- AI/ML shares the same CPU pool as legacy: the CPU pool for AI/ML-based CSI processing is shared with the legacy CPU pool.

- Dedicated AI/ML PU pool separate from legacy: there’s a dedicated CPU pool for AI/ML-based CSI processing (namely AI/ML processing unit-APU), which is separate from legacy CPU pool.

Whether the AI/ML PU should be shared or separately counted among AI/ML related features/functionalities

To address this question, there are two aspects that we need to consider. First, the AI/ML PU is applicable to the use cases that need AI/ML enabled CSI processing including beam prediction, CSI prediction, and CSI compression. Therefore, there would be no AI/ML PU counting for non-CSI-related AI/ML tasks. Second, in order to provide a flexible framework for different AI/ML-enabled CSI processing tasks, the AI/ML PU must be shared among them. Hence, we propose the following:

      - Proposal 14

To address the FFS on whether the AI/ML PU should be shared or separately counted among AI/ML related features/functionalities, support the following:

- The AI/ML PU must be shared among AI/ML-based CSI processing use cases.

  - There will be no AI/ML PU counting for non-CSI-related AI/ML tasks

How should CPU be counted for dedicated AI/ML PU pool?

In light of the above proposal, and considering the description for high-end UEs mentioned above, it is important to note that the existence of a dedicated CPU pool for AI/ML does not imply that for an AI/ML-enabled CSI processing task, UE is only going to count the required #CPUs for that task towards only the AI/ML PU pool budget. The reason for this is that we may need to account for the pre- and post-processing needed to run the AI/ML-enabled CSI processing task, as well as memory costs for buffering/storing, which would consume resources towards legacy CPU pool.

      - Proposal 15

For dedicated AI/ML PU pool, to run an AI/ML-enabled CSI processing task, the #CPUs required for that task should be counted towards both legacy CPU pool and AI/ML PU pool to account for the pre- and post-processing and memory needed to run the task, which would consume resources towards legacy CPU.

### Concurrent processing of different AI/ML-enabled CSI reports

Note that at least for dedicated AI/ML PU pool, the AI/ML enabled CSI processing may run in a hardware that is shared by different AI/ML use cases (BM-Case1/BM-Case2, CSI prediction, [CSI compression]) (Level y and Leve z) and implementation-only (Level x) use cases. Additionally, there could be a large variance in terms of processing needed for running different AI/ML features (different AI/ML model requirements) and different model implementations across devices. With that said, finding a viable solution for defining concurrent processing requirements of different AI/ML-enabled tasks poses some unique and new challenges that need to be carefully addressed. The solution developed in this release may serve as a baseline for potential future AI/ML features, and therefore, it is more desirable to have a scalable solution that could possibly be leveraged in the future. We outline two potential solutions that address the concurrent processing issue mentioned above, in the following (assuming we consider three AI/ML enabled CSI processing features of beam prediction, CSI prediction, and CSI compression):

Solution 1: Use a shared AI/ML processing unit budget and per-use-case processing unit count, reported in UE capability

To explain this solution, let us have an illustrative example. Let us assume that the total AI/ML processing unit (APU) budget is equal to 4 (reported by UE in the capability report). Let us further assume that the required #APUs for CSI compression, CSI prediction and BM-Case2 are 2, 2, and 1, respectively (reported by UE in the capability report, per feature). As the sum of the #APUs for all three features (i.e., 5) is NOT within the APU budget, this would mean that concurrent processing of the above three features is NOT possible. However, concurrent processing of CSI compression and CSI prediction (2+2 = 4), CSI compression and BM-Case2 (2+1 = 3), CSI prediction and BM-Case2 (2+1 = 3) are all possible, as they are all within the APU budget. As an additional example, concurrent processing of two CSI compression tasks (2×2 =4) is possible, and concurrent processing of one CSI prediction task and two BM-Case 2 tasks (2 + 2×1 = 4) is possible.

Observation 5: To enable implementation flexibility, the number of APUs for each AI/ML enabled CSI processing feature can be indicated via UE capability.

Solution 2: Introduce tuples for supported combinations of concurrent AI/ML-enabled CSI processing across different features, as part of UE capability

To explain this solution, let us also use an illustrative example. Let us use the terminology (X, Y, Z) as the tuple that can be used to represent the possible concurrent combinations for CSI compression, CSI prediction, and beam prediction, respectively. In particular, (2X, Y, 0) would mean that concurrent processing of two CSI compression AI/ML-enabled tasks, and one CSI prediction task is possible. As another example, (0, Y, 2Z) would imply that concurrent processing of a CSI prediction feature and two beam prediction features is possible. As one more example, (X, Y, Z) would mean UE is capable of concurrent processing of a CSI compression feature, a CSI prediction feature, and a beam prediction feature. As long as the supported tuples are indicated in the UE capability, the concurrent processing of features can be ensured by relying on that information.

Now, let us compare Solution 1 and Solution 2. Solution 1 primarily relies on a linear equation for determining the concurrent processing capability, whereas Solution 2 provides a more flexible framework that may not be possible via Solution 1. Again, let’s consider an illustrative example here. Let us assume that the total CPU budget for AI/ML-enabled CSI processing is N. Considering UE implementation considerations, UE may support (2X, Y, 0), and (0, Y, 2Z), but not (X, Y, Z). In such a case, if we go with solution 1, we need to ensure that these linear constraints hold: 2X + Y<N, Y + 2Z < N, and X + Y + Z > N, which would not be feasible. In summary, solution 2 gives us more knobs to customize different possible combinations for concurrent AI/ML-enabled CSI processing, accounting for implementation consideration.

### CPU allocation and occupation duration for AI/ML-enabled CSI reports

First, let us review legacy CPU allocation rules and occupation durations.

Overview of legacy CPU allocation rules for CSI reports

In the following, we have a brief overview of legacy CPU allocation rules

- OCPU=0: if reportQuantity of the corresponding CSI report setting is set to none while the CSI Resource Set associated with the CSI report setting is configured with parameter trs-Info. - OCPU=1: if reportQuantity of the corresponding CSI report setting is set to cri-RSRP or ssb-Index-RSRP or cri-SINR or ssb-Index-SINR, or none while the CSI Resource Set associated with the CSI report setting is NOT configured with trs-Info. - OCPU=Ks: If reportQuantity of the corresponding CSI report setting is set to cri-RI-PMI-CQI or cri-RI-i1 or cri-RI-i1-CQI or cri-RI-CQI or cri-RI-LI-PMI-CQI, where Ks is the number of CSI-RS resources in the associated Channel Measurement Resource (CMR) Set.

As we see, the number of CPUs that are occupied for a given CSI report is determined based on the reportQuantity and if the reportQuantity is set to ‘none’, based on whether trs-Info is configured or not.

Overview of legacy CPU occupation durations for CSI reports

We provide an overview of legacy CPU occupation durations in the following:

First, let us consider the scenario in which reportQuantity is NOT set to none.

Duration of CPU occupations (where reportQuantity is NOT set to none). --> 38.214 Sec.-5.2.1.6 - Case#1: P/SP CSI reports (excluding the initial SP CSI report on PUSCH after the PDCCH triggering the report; also excluding R18 typeII-Doppler PMI associated SP CSI reports): from the 1st symbol of the earliest one of each CSI-RS/CSI-IM/SSB resource associated with a CSI-ReportConfig, on the latest CSI-RS/CSI-IM/SSB occasion no later than the corresponding CSI reference resource, until the last symbol of the configured PUSCH/PUCCH carrying the CSI report. - Case#2: AP CSI reports, or an initial SP CSI report on PUSCH triggered by DCI: from the 1st symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report. - Case#3: SP CSI reports (for R18 typeII-Doppler PMI CSI reports (codebookType within a CSI-ReportConfig is set to typeII-Doppler-r18 or typeII-Doppler-PortSelection-r18)): from the 1st symbol of Kp-th latest consecutive P/SP CSI-RS occasions no later than CSI reference resource, until the last symbol of the PUSCH carrying the report, where the value of Kp∈{1,2,4} is indicated by UE capability.

The above cases are illustrated in Figure 9.

Figure 9 CPU occupation durations for cases in which reportQuantity is NOT set to none

Now, let us consider the scenario in which reportQuantity is set to none and trs-info is not configured for the CMR set.

Duration of CPU occupations (reportQuantity is set to none and trs-info not configured for the CMR set). --> 38.214 Sec.-5.2.1.6 - Case#4: SP CSI reports (excluding the initial SP CSI report on PUSCH after the PDCCH triggering the report): from the first symbol of the earliest one of each transmission occasion of periodic or semi-persistent CSI-RS/SSB resource for channel measurement for L1-RSRP computation, until Z3' symbols after the last symbol of the latest one of the CSI-RS/SSB resource for channel measurement for L1-RSRP computation in each transmission occasion. - Periodic CSI reports meeting the conditions are N/A. - Case#5: AP CSI reports: from the first symbol after the PDCCH triggering the CSI report until the last symbol between Z3 symbols after the first symbol after the PDCCH triggering the CSI report and Z3' symbols after the last symbol of the latest one of each CSI-RS/SSB resource for channel measurement for L1- RSRP computation.

We illustrate the above cases in Figure 10.

Figure 10 CPU occupation durations for cases in which reportQuantity is set to none, and trs-info not configured for the CMR set

In the above figure, Z3' and the corresponding value of Z3 are defined by TS38.214-Table-5.4-2, depicted in Figure 11 for reference.

- Z3 is the # of symbols after the end of the last symbol of the PDCCH triggering the AP CSI report.

- Z3' is the # of symbols after the end of the last symbol of the latest of: AP CSI-RS resource for channel measurements, AP CSI-IM used for interference measurements, and AP NZP CSI-RS for interference measurements.

- Here the values of {X1, X2, …, X6} and {KB1, …,  KB4} are UE reported capabilities for beam reporting delay and beam switching delay, wrt different SCSs.

Figure 11 Table 5.4-2 from 38.214

Now that we have reviewed the legacy procedures, let us discuss CPU allocation rules and occupation durations for AI/ML-enabled CSI reports.

#### 5.1.3.1 CPU allocation and occupation duration for AI/ML inference

For AI/ML-enabled CSI reports for inference, the OCPU values for Set B measurements should be set to 1. This follows the same principle as when reportQuantity of the corresponding CSI report setting is set to none while the CSI Resource Set associated with the CSI report setting is NOT configured with trs-Info. On the other hand, the OCPU value for inference operation (which includes generating predictions from Set A) should be determined by accounting for at least the processing requirements needed for the inference operation as well as the pre-and post-processing related to input and out of the AI/ML model, along with other factors. With that said, we propose the following.

#### Proposal 16

For BM-Case1 and BM-Case2, with regards to the OCPU values (number of CPUs) for AI/ML-based CSI reports for inference.

- OCPU value for Set B measurements should be equal to 1 (same as legacy).

- FFS: OCPU value for inference operation (which includes generating predictions from Set A)

Now that we have discussed CPU allocation rules, let us investigate the CPU occupation duration for AI/ML-enabled CSI reports for inference. For this purpose, let us particularly focus on BM-Case2. For this purpose, we consider the following two cases:

- Set B measurement occasions except for the latest one (no later than CSI reference resource): For this, we can follow a similar principle for SP CSI reports (excluding the initial SP CSI report on PUSCH after the PDCCH triggering the report) in legacy when reportQuantity is set to none and trs-info not configured for the CMR set, for which we have reviewed the occupation duration earlier in this section  Case#4 mentioned above.

- Latest Set B measurement occasion (no later than CSI reference resource), and inference report: For this, we can follow a similar principle for P/SP CSI reports (excluding the initial SP CSI report on PUSCH after the PDCCH triggering the report; also excluding R18 typeII-Doppler PMI associated SP CSI reports) in legacy when reportQuantity is NOT set to none  Case#1 mentioned above.

What we mentioned above with regards to occupation durations, is illustrated in Figure 12.

Figure 12 CPU occupation duration for AI/ML-enabled CSI processing for BM-Case2 inference operation

Based on the discussions above, we propose the following:

#### Proposal 17

For BM-Case2, with regards to CPU occupation duration for Set B measurement occasions and AI/ML-based CSI reports for inference

- Set B measurement occasions except for the latest one (no later than CSI reference resource): CPUs are occupied from the first symbol of the earliest one of each transmission occasion of Set B RSs (CSI-RS/SSB resource for channel measurement) until Z3' symbols after the last symbol of the latest Set B RSs (CSI-RS /SSB resource for channel measurement) for L1-RSRP computation, in each transmission occasion (similar to legacy behavior when reportQuantity is set to none and trs-info not configured for the CMR set)

- Latest Set B measurement occasion (no later than CSI reference resource), and inference report: CPUs are occupied from 1st symbol of the earliest one of each Set B (CSI-RS/SSB) resource associated with a CSI-ReportConfig, on the latest Set B occasion no later than the corresponding CSI reference resource, until the last symbol of the configured PUSCH/PUCCH carrying the inference report.

#### 5.1.3.2 CPU allocation and occupation duration for UE-assisted performance monitoring

For performance monitoring, we can follow a similar principle as the one mentioned above for BM-Case2. The difference here is the processing related to performance monitoring metric calculation, and we need to define the corresponding OCPU value accordingly.

#### Proposal 18

For BM-Case1 and BM-Case2, with regards to the OCPU values (number of CPUs) for UE-assisted AI/ML performance monitoring:

- OCPU value for monitoring RS measurements should be equal to 1 (same as legacy).

- FFS: OCPU value related to monitoring report (which includes performance metric calculation)

The CPU occupation duration for monitoring can follow a similar principle as the one explained above for BM-Case2, and therefore we do not repeat the related discussions here, and illustrate the CPU occupation duration for monitoring in Figure 13.

Figure 13 CPU occupation duration for AI/ML-enabled CSI processing for UE-assisted performance monitoring

With the above discussions, we propose the following:

#### Proposal 19

For BM-Case1 and BM-Case2, with regards to CPU occupation duration for monitoring RS measurement occasions and AI/ML-based CSI reports for monitoring

- Monitoring RS measurement occasions except for the latest one: CPUs are occupied from the first symbol of the earliest one of each transmission occasion of monitoring RSs (CSI-RS/SSB resource for channel measurement) until Z3' symbols after the last symbol of the latest monitoring RSs (CSI-RS /SSB resource for channel measurement) for L1-RSRP computation, in each transmission occasion (similar to legacy behavior when reportQuantity is set to none and trs-info not configured for the CMR set).

- Latest monitoring RS measurement occasion (no later than CSI reference resource), and monitoring report: CPUs are occupied from 1st symbol of the earliest one of each monitoring RS (CSI-RS/SSB) resource associated with a CSI-ReportConfig, on the latest monitoring RS occasion no later than the corresponding CSI reference resource, until the last symbol of the configured PUSCH/PUCCH carrying the monitoring report.

- FFS: CPU occupation duration for the case when we have both inference and monitoring (especially for BM-Case2 when the inference results need to be maintained at the UE side to be compared to measurements from monitoring RS in future monitoring instances).

Now that we have identified the CPU occupation duration for monitoring, it is important to also consider the CPU occupation duration for the case when inference and monitoring are being run together. For this case, if the AI/ML model is predicting into one or more future instances, UE needs to buffer inference results until the monitoring occasions to compare inference results to measurements from monitoring RSs. With that in mind, the corresponding CPU occupation duration for this scenario should be further studied.

#### Proposal 20

At least for BM-Case2, further study the CPU occupation duration for the case when we have both inference and monitoring (when the inference results need to be maintained at the UE side to be compared to measurements from monitoring RS in future monitoring instances), and whether/how it impacts CPU occupation duration.

## Priority rules for AI/ML-enabled CSI reports

First, let us review the legacy CSI priority rules in the following:

Legacy (Inter)-CSI Priority Rules (TS38.214-Sec.5.2.5). Priority order of a CSI report (smaller value corresponds to higher priority): PriiCSIy,k,c,s=2NCellsMsy+NcellsMsk+Msc+s Wherein: y={0,1,2,3} for {AP@PUSCH, SP@PUSCH, SP@PUCCH, P@PUCCH} CSI reports. k=0 for CSI reports carrying L1-RSRP or L1-SINR and k=1 for CSI reports not carrying L1-RSRP or L1-SINR. c is the ServCell ID and Ncells is the value of the RRC parameter maxNrofServingCells. s is the reportConfigID associated with the CSI report setting and Ms is the RRC parameter maxNrofCSI-ReportConfigurations.

Additionally, it has been agreed that inference and monitoring report configurations are going to be enabled via separate CSI-ReportConfigs. Now, the question is how the interplay between AI/ML enabled CSI reports and legacy CSI reports would be in terms of inter-CSI priorities. In our view, we can leverage the equation for priority cited above as a starting point and adjust the comparative priority between AI/ML-enabled CSI reports and legacy CSI reports via defining the appropriate k values for AI/ML-enabled CSI reports. With that said, we propose the following.

#### Proposal 21

For BM-Case1 and BM-Case2, with regards to determining the priority rules for AI/ML-enabled CSI reports, use the equation for priority value PriiCSI (as defined in TS38.214-Sec.5.2.5) as a starting point, and further study the value of k for AI/ML-enabled CSI reports.

- FFS: value of k for inference report for spatial and temporal beam prediction.

- FFS: value of k for performance monitoring reports

# RRC parameter list

#### Proposal 22

For BM-Case1 and BM-Case2, consider the following table including RRC parameter list for specification support for beam prediction:

| RAN2 Parent IE | Parameter name in the spec | New or existing? | Description | Value range |
| --- | --- | --- | --- | --- |
| Reporting of inference results |  |  |  |  |
| CSI-ReportConfig | typeOfBeamPrediction | New | Selection among ‘BM-Case1’ and ‘BM-Case2’ | {‘BM-Case1’, ‘BM-Case2’} |
| CSI-ReportConfig | nrofReportedPredictedRSs-r19 | New | Applicable to UE-side models, determining the value of K for number of reported Top-K beams (and their corresponding predicted RSRPs) to be reported per report setting. | K= {1,2,3,4} |
| CSI-ReportConfig | reportQuantity-r19 | New | Related to inference: ‘predicted-TopK-beams-info’, ‘predicted-TopK-beams-and-RSRP-info’ Related to monitoring: ‘monitoring-metric’ Related to data collection: ‘data-collection’ | ‘predicted-TopK-beams-info’, ‘predicted-TopK-beams-and-RSRP-info’ ‘monitoring-metric-BM’ ‘ue-data-collection’ ‘nw-data-collection’ |
| Parameters specific to BM-Case2 |  |  |  |  |
| CSI-ReportConfig | numberFuturePredictionInstancesBMCase2 | New | Applicable to UE-side models and BM-Case2, this parameter N defines the number of future prediction instances | N = {1, 2, …, 8} |
| CSI-ReportConfig | measurementOccasionsTemporalSeparation | New | Applicable to UE-side models and BM-Case2, this parameter defines time interval between consecutive measurement instances (occasions) for Set B. | FFS: {10ms, 20ms, 40ms, 80ms, 160ms} |
| CSI-ReportConfig | predictionOccasionsTemporalSeparation | New | Applicable to UE-side models and BM-Case2, this parameter defines time interval between consecutive prediction instances as well as the time instance between latest Set B measurement and first prediction occasion. | FFS: {10ms, 20ms, 40ms, 80ms, 160ms} |
| CSI-ReportConfig | [numberMeasurementsPerCycle] | New | Applicable to UE-side models and BM-Case2, and for cyclic measurement and prediction patterns, this parameter x determines the number of Set B occasions per cycle. | x = {1,2} |
| CSI-ReportConfig | [numberPredictionsPerCycle] | New | Applicable to UE-side models and BM-Case2, and for cyclic measurement and prediction patterns, this parameter y determines the number of prediction occasions per cycle. | y = {1, 2, …, 8} |
| Configuration for Set A and set B |  |  |  |  |
| CSI-ReportConfig | resourcesForChannelPrediction | New | Prediction resources representing Set A. | {CSI-ResourceConfigId} |
| CSI-ReportConfig | resourcesForChannelMeasurement | Existing | Channel measurement resources for Set B. | {CSI-ResourceConfigId} |
| Associated ID |  |  |  |  |
| CSI-ReportConfig | AssociatedIdForSetA | New | This needs to be configured for all the sub-use cases. | FFS |
| CSI-ReportConfig | AssociatedIdForSetB | New | This only needs to be configured when Set B is composed of wide beams and Set A is composed of narrow beams (wide-to-narrow beam prediction). | FFS |
| Monitoring |  |  |  |  |
| CSI-ReportConfig | linkedInferenceReportConfigId | New | For UE-assisted performance monitoring, the ID of the linked inference report should be included in the CSI report configuration for monitoring. | {CSI-ReportConfigId} |
| CSI-ReportConfig | [beamPredictionAccuracy] | New | This is meant to denote the metric for UE-assisted performance monitoring. Still under discussion. | FFS |
| CSI-ReportConfig | numberOfMonitoringOccasions | New | This is meant to define the number of performance monitoring instances that the UE should use for computing the beam prediction accuracy metric. | FFS e.g., {1, …, 32} |
| New parameters may be added after agreeing on the performance monitoring metric, as well as agreement on linkage between Set A beam IDs and Monitoring RSs. |  |  |  |  |
| NW-sided model |  |  |  |  |
| CSI-ReportConfig | nrofReportedRS-nw-inference-r19 | New | Applicable to NW-side models, for reporting of Top-M measurement results from Set B. We have the following FFS on this: - FFS on the maximum value of M (where M can be larger than 4) based on UE capability (M may or may not be different for different reporting contents) | FFS M values |
| Flag for applicability report |  |  |  |  |
| CSI-ReportConfig | forApplicabilityInquiry | New | This is related to the RNA2 LS discussions. This flag can indicate whether CSI-ReportConfig (e.g., sent in Step 3 in RRCreconfiguration) is for inference purpose (‘0’), or just for applicability inquiry (‘1’). | {‘0’, ‘1’} |

# Conclusions

In this paper, we discussed signalling aspects related to beam prediction use case and made the following proposals and observation:

Proposal 1: For beam prediction for UE-side AI/ML models, consider the following aspects to ensure consistency between training and inference regarding NW-side additional conditions (with regards to Set A, Set B consistency) for inference at UE

- Order/indexing consistency: consistency in ordering of resources (e.g., resource index consistency) for Set B beams and Set A beams, across training and inference.

- Beam shape consistency: For each Set A resource, the difference between pointing direction and beamwidth of the physical beam associated with that Set A resource during training compared to pointing direction and beamwidth of the physical beam associated with that same Set A resource during inference should be under predefined tolerances. Similarly, for each Set B resource, the difference between pointing direction and beamwidth of the physical beam associated with that Set B resource during training compared to pointing direction and beamwidth of the physical beam associated with that same Set B resource during inference should be under predefined tolerances.

Proposal 2: For UE-side beam prediction, for the consistency of NW-side additional conditions across training and inference, with regards to FFS on how to define similar properties of a DL Tx beam or beam set/list for the same associated ID:

- Based on spatial Tx filter: For the same associated ID across training and inference, for each Set A resource, UE can assume that the same spatial TX filter has been utilized by gNB, across training and inference. Similarly, for each Set B resource, UE can assume that the same spatial TX filter has been utilized by gNB, across training and inference.

  - Note: a certain tolerance level can be considered for the spatial TX filter used in inference versus training.

- Based on QCL relationship: For the same associated ID across training and inference, for each Set A resource, UE can assume that the gNB Tx beams used across training and inference have a certain QCL relationship. Similarly, for each Set B resource, UE can assume that the gNB Tx beams used across training and inference have a certain QCL relationship.

  - FFS: definition of such QCL relationships.

Proposal 3: For UE-side beam prediction, for the consistency of NW-side additional condition across training and inference, deprioritize Opt2: Performance monitoring based.

Proposal 4: For UE-side beam prediction, for the consistency of NW-side additional condition across training and inference, the NW-side additional conditions with the same associated ID should be consistent within a PLMN.

Proposal 5: For UE-side AI/ML inference and BM-Case2, for inference results in a report for multiple future time instances, support the following format

- Include the {resource-ID, Occasion ID pair} for “leading” reference resource ID and temporal occasion ID pair (that has the largest predicted L1-RSRP value) in the first CSI field location.

- For the subsequent fields, the resource IDs associated with respective temporal occasions should be ordered according to temporal occasion IDs or indexes.

  - This means the resource IDs should be ordered such that information regarding the resource IDs associated with the first (earliest) temporal occasion appears first, information regarding the resource IDs associated with the second temporal occasion appears second, and so on.

- Once the indication of resource IDs is concluded, the absolute predicted L1-RSRP is included for the leading {resource-ID, Occasion-ID} pair.

- For the rest of the resources, the differential predicted L1-RSRP is included in the subsequent CSI fields, and the ordering for the differential L1-RSRPs follow the ordering for the resource IDs in the preceding CSI fields.

Proposal 6: For UE-side beam prediction, for content in the report of inference results, regarding FFS on potential down-selection among Option 3 and Option 4, support Option 4.

Proposal 7: For beam prediction for UE-side AI/ML models, performance monitoring metrics should also be defined for scenarios in which the performance monitoring set for each performance monitoring instance is a subset of the prediction set, i.e., Set A.

Proposal 8: For UE-side beam prediction, with regards to Alt 1 of performance monitoring alternatives for UE-assisted performance monitoring, in which the performance monitoring set may be a subset of Set A or full Set A for each performance monitoring instance, define N as the total number of performance monitoring instances, and define Np as the number of performance monitoring instances (out of N) for which the following statement holds:

- Top-K beam prediction accuracy without L1-RSRP margin: best measured beam ID from performance monitoring set is among the Top-K predicted beams, for a given performance monitoring instance.

- Top-K beam prediction accuracy with L1-RSRP margin: The highest measured L1-RSRP of Top-K predicted beams that are within the performance monitoring set is within X dB of measured L1-RSRP of best measured beam ID from performance monitoring set, for a given performance monitoring instance.

  - X can be configured by gNB.

Define the ratio Np/N as Top-K beam prediction accuracy (without or with margin based on which of the above statements holds).

- Note: “performance monitoring set” is the set of RSs that are to be measured for performance monitoring, per performance monitoring instance.

Proposal 9: For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, at least for Option 2 (UE-assisted performance monitoring), support Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring.

Proposal 10: For UE-side beam prediction, with regards to Alt 2 of performance monitoring alternatives for UE-assisted performance monitoring, in which the performance monitoring set may be a subset of Set A or full Set A for each performance monitoring instance, support the following metric for performance monitoring:

- The L1-RSRP difference between measured L1-RSRP of Top-1 measured beam ID from the performance monitoring set and highest measured L1-RSRP of Top-K predicted beams that are within the performance monitoring set, per performance monitoring instance.

Note: “performance monitoring set” is the set of RSs that are to be measured for performance monitoring, per performance monitoring instance.

Proposal 11: For UE-side beam prediction, and for UE-assisted performance monitoring, study details of performance monitoring reports (contents, frequency of report, carrier), at least as a function of performance monitoring metrics.

Proposal 12: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, and for Type 1 performance monitoring, study the following events for event-triggered performance monitoring report for both Option 1 and Option 2.

- Event 1: Top-1 measured beam is NOT among Top-K predicted beams, from performance monitoring set.

- Event 2: Measured L1-RSRP of Top-1 predicted beam is NOT within 𝑋 dB of Top-1 measured L1-RSRP, from performance monitoring set.

Note: FFS on triggering condition: whether based on per-instance event occurrence or based on number of event occurrences within a window.

Proposal 13: With regards to the CPU pool for AI/ML-based CSI processing, support both of the following, as separate UE capabilities:

- AI/ML shares the same CPU pool as legacy: the CPU pool for AI/ML-based CSI processing is shared with the legacy CPU pool.

- Dedicated AI/ML PU pool separate from legacy: there’s a dedicated CPU pool for AI/ML-based CSI processing (namely AI/ML processing unit-APU), which is separate from legacy CPU pool.

  - The dedicated CPU pool for AI/ML-based use cases is shared among AI/ML-based CSI processing use cases.

Proposal 14: To address the FFS on whether the AI/ML PU should be shared or separately counted among AI/ML related features/functionalities, support the following:

- The AI/ML PU must be shared among AI/ML-based CSI processing use cases.

  - There will be no AI/ML PU counting for non-CSI-related AI/ML tasks

Proposal 15: For dedicated AI/ML PU pool, to run an AI/ML-enabled CSI processing task, the #CPUs required for that task should be counted towards both legacy CPU pool and AI/ML PU pool to account for the pre- and post-processing and memory needed to run the task, which would consume resources towards legacy CPU.

Proposal 16: For BM-Case1 and BM-Case2, with regards to the OCPU values (number of CPUs) for AI/ML-based CSI reports for inference.

- OCPU value for Set B measurements should be equal to 1 (same as legacy).

- FFS: OCPU value for inference operation (which includes generating predictions from Set A)

Proposal 17: For BM-Case2, with regards to CPU occupation duration for Set B measurement occasions and AI/ML-based CSI reports for inference

- Set B measurement occasions except for the latest one (no later than CSI reference resource): CPUs are occupied from the first symbol of the earliest one of each transmission occasion of Set B RSs (CSI-RS/SSB resource for channel measurement) until Z3' symbols after the last symbol of the latest Set B RSs (CSI-RS /SSB resource for channel measurement) for L1-RSRP computation, in each transmission occasion (similar to legacy behavior when reportQuantity is set to none and trs-info not configured for the CMR set)

- Latest Set B measurement occasion (no later than CSI reference resource), and inference report: CPUs are occupied from 1st symbol of the earliest one of each Set B (CSI-RS/SSB) resource associated with a CSI-ReportConfig, on the latest Set B occasion no later than the corresponding CSI reference resource, until the last symbol of the configured PUSCH/PUCCH carrying the inference report.

Proposal 18: For BM-Case1 and BM-Case2, with regards to the OCPU values (number of CPUs) for UE-assisted AI/ML performance monitoring:

- OCPU value for monitoring RS measurements should be equal to 1 (same as legacy).

- FFS: OCPU value related to monitoring report (which includes performance metric calculation)

Proposal 19: For BM-Case1 and BM-Case2, with regards to CPU occupation duration for monitoring RS measurement occasions and AI/ML-based CSI reports for monitoring

- Monitoring RS measurement occasions except for the latest one: CPUs are occupied from the first symbol of the earliest one of each transmission occasion of monitoring RSs (CSI-RS/SSB resource for channel measurement) until Z3' symbols after the last symbol of the latest monitoring RSs (CSI-RS /SSB resource for channel measurement) for L1-RSRP computation, in each transmission occasion (similar to legacy behavior when reportQuantity is set to none and trs-info not configured for the CMR set).

- Latest monitoring RS measurement occasion (no later than CSI reference resource), and monitoring report: CPUs are occupied from 1st symbol of the earliest one of each monitoring RS (CSI-RS/SSB) resource associated with a CSI-ReportConfig, on the latest monitoring RS occasion no later than the corresponding CSI reference resource, until the last symbol of the configured PUSCH/PUCCH carrying the monitoring report.

- FFS: CPU occupation duration for the case when we have both inference and monitoring (especially for BM-Case2 when the inference results need to be maintained at the UE side to be compared to measurements from monitoring RS in future monitoring instances).

Proposal 20: At least for BM-Case2, further study the CPU occupation duration for the case when we have both inference and monitoring (when the inference results need to be maintained at the UE side to be compared to measurements from monitoring RS in future monitoring instances), and whether/how it impacts CPU occupation duration.

Proposal 21: For BM-Case1 and BM-Case2, with regards to determining the priority rules for AI/ML-enabled CSI reports, use the equation for priority value PriiCSI (as defined in TS38.214-Sec.5.2.5) as a starting point, and further study the value of k for AI/ML-enabled CSI reports.

- FFS: value of k for inference report for spatial and temporal beam prediction.

- FFS: value of k for performance monitoring reports

Proposal 22: For BM-Case1 and BM-Case2, consider the following table including RRC parameter list for specification support for beam prediction:

| RAN2 Parent IE | Parameter name in the spec | New or existing? | Description | Value range |
| --- | --- | --- | --- | --- |
| Reporting of inference results |  |  |  |  |
| CSI-ReportConfig | typeOfBeamPrediction | New | Selection among ‘BM-Case1’ and ‘BM-Case2’ | {‘BM-Case1’, ‘BM-Case2’} |
| CSI-ReportConfig | nrofReportedPredictedRSs-r19 | New | Applicable to UE-side models, determining the value of K for number of reported Top-K beams (and their corresponding predicted RSRPs) to be reported per report setting. | K= {1,2,3,4} |
| CSI-ReportConfig | reportQuantity-r19 | New | Related to inference: ‘predicted-TopK-beams-info’, ‘predicted-TopK-beams-and-RSRP-info’ Related to monitoring: ‘monitoring-metric’ Related to data collection: ‘data-collection’ | ‘predicted-TopK-beams-info’, ‘predicted-TopK-beams-and-RSRP-info’ ‘monitoring-metric-BM’ ‘ue-data-collection’ ‘nw-data-collection’ |
| Parameters specific to BM-Case2 |  |  |  |  |
| CSI-ReportConfig | numberFuturePredictionInstancesBMCase2 | New | Applicable to UE-side models and BM-Case2, this parameter N defines the number of future prediction instances | N = {1, 2, …, 8} |
| CSI-ReportConfig | measurementOccasionsTemporalSeparation | New | Applicable to UE-side models and BM-Case2, this parameter defines time interval between consecutive measurement instances (occasions) for Set B. | FFS: {10ms, 20ms, 40ms, 80ms, 160ms} |
| CSI-ReportConfig | predictionOccasionsTemporalSeparation | New | Applicable to UE-side models and BM-Case2, this parameter defines time interval between consecutive prediction instances as well as the time instance between latest Set B measurement and first prediction occasion. | FFS: {10ms, 20ms, 40ms, 80ms, 160ms} |
| CSI-ReportConfig | [numberMeasurementsPerCycle] | New | Applicable to UE-side models and BM-Case2, and for cyclic measurement and prediction patterns, this parameter x determines the number of Set B occasions per cycle. | x = {1,2} |
| CSI-ReportConfig | [numberPredictionsPerCycle] | New | Applicable to UE-side models and BM-Case2, and for cyclic measurement and prediction patterns, this parameter y determines the number of prediction occasions per cycle. | y = {1, 2, …, 8} |
| Configuration for Set A and set B |  |  |  |  |
| CSI-ReportConfig | resourcesForChannelPrediction | New | Prediction resources representing Set A. | {CSI-ResourceConfigId} |
| CSI-ReportConfig | resourcesForChannelMeasurement | Existing | Channel measurement resources for Set B. | {CSI-ResourceConfigId} |
| Associated ID |  |  |  |  |
| CSI-ReportConfig | AssociatedIdForSetA | New | This needs to be configured for all the sub-use cases. | FFS |
| CSI-ReportConfig | AssociatedIdForSetB | New | This only needs to be configured when Set B is composed of wide beams and Set A is composed of narrow beams (wide-to-narrow beam prediction). | FFS |
| Monitoring |  |  |  |  |
| CSI-ReportConfig | linkedInferenceReportConfigId | New | For UE-assisted performance monitoring, the ID of the linked inference report should be included in the CSI report configuration for monitoring. | {CSI-ReportConfigId} |
| CSI-ReportConfig | [beamPredictionAccuracy] | New | This is meant to denote the metric for UE-assisted performance monitoring. Still under discussion. | FFS |
| CSI-ReportConfig | numberOfMonitoringOccasions | New | This is meant to define the number of performance monitoring instances that the UE should use for computing the beam prediction accuracy metric. | FFS e.g., {1, …, 32} |
| New parameters may be added after agreeing on the performance monitoring metric, as well as agreement on linkage between Set A beam IDs and Monitoring RSs. |  |  |  |  |
| NW-sided model |  |  |  |  |
| CSI-ReportConfig | nrofReportedRS-nw-inference-r19 | New | Applicable to NW-side models, for reporting of Top-M measurement results from Set B. We have the following FFS on this: - FFS on the maximum value of M (where M can be larger than 4) based on UE capability (M may or may not be different for different reporting contents) | FFS M values |
| Flag for applicability report |  |  |  |  |
| CSI-ReportConfig | forApplicabilityInquiry | New | This is related to the RNA2 LS discussions. This flag can indicate whether CSI-ReportConfig (e.g., sent in Step 3 in RRCreconfiguration) is for inference purpose (‘0’), or just for applicability inquiry (‘1’). | {‘0’, ‘1’} |

# References

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #94e, Dec. 2021

- TR 38.843 v18.0.0, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18), Dec. 2023

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #102, Dec. 2023

- TS 38.214, NR Physical layer procedures for data, V18.1.0, Jan. 2024