3GPP TSG RAN WG1 #120bis	                                         	           R1-2503147

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	9.6.2

Source:	Moderator (Apple)

Title:	Summary #6 (Final) on LP-WUS operation in IDLE/INACTIVE mode

Document for:	Discussion/Decision

# Introduction

One of the objectives in Rel-19 WI on LP-WUS/WUR (RP-241824) is to support LP-WUS operation in idle/inactive mode:

- For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

This contribution summarizes the discussions on LP-WUS operation in IDLE/INACTIVE modes in RAN1#120bis.

Section 2 captures the agreements as the outcome of RAN1#120bis discussions. Section 3 captures the proposals for online sessions. Section 4 documents the detailed discussions. Section 5 provides guidance for RAN1#121 discussions. Agreements from previous meetings are captured in the Appendix.

# RAN1#120bis Agreements

Agreement

Confirm the following working assumption with modifications in red:

Working Assumption

If LP-WUS design support 32 subgroups within one MO, do not support Option 3 for LO to PO mapping or Option B for MO configuration.

Agreement

Option A for MO configuration is supported.

Agreement

A LP-WUS MO can span across multiple slots.

- FFS the limitation on the maximum length for an LP-WUS MO

Agreement

Only R = 1 is supported for Option A.

Conclusion

For the offset value(s) between an LO and a reference PO/PF, do not support the configuration of 3 offset values.

Agreement

The maximum number of POs per LO is 4, and the number of POs per LO can be 1, 2 or 4.

Agreement

For Option 2, the maximum value of M (number of LP-WUS MOs per beam) in Option A for MO configuration is 4.

Agreement

For LP-WUS, the N * M LP-WUS MOs in an LO are indexed sequentially in time, from 1 to N*M, where N is the number of beams corresponding to LP-WUS, and M is the number of LP-WUS MOs for each beam.

- The (n*M+m+1)-th LP-WUS MO corresponds to the (n+1)-th beam, where m=0,1,…,M-1, n=0,1,2,…,N-1. (multiple MOs first, beam second)

Note: Above does not change the previous agreement on association between LP-WUS and SSB beams.

Agreement

For OFDM-based LP-WUR, reuse the LP-SS based LP-RSRP/LP-RSRQ definition of OOK-based LP-WUR.

FFS: Whether OFDM receiver can measure LP-SS if overlaid OFDM sequence is not configured (M=1).

Agreement

For LP-SSS-RSRP/RSSI measurement performed by OFDM-based LP-WUR for the serving cell, SMTC window is not applicable.

- Send an LS to RAN4 to confirm the above agreement. Final LS in R1-2503103.

Agreement

LS on the RRM measurement metrics for OFDM-based LP-WUR is agreed. Final LS in R1-2503103.

- Note: RAN1 understanding is existing metrics SS-RSRP and SS-RSRQ are reused for OFDM-based LP-WUR. No separate metrics (LP-SSS-RSRP and LP-SSS-RSRQ) will be introduced in the specifications.

Agreement

For Option 2, at least one codepoint corresponding to each of the subgroups in each PO is supported.

- For codepoint corresponding to more than one subgroups:

  - Alt 2: One codepoint for each PO corresponding to all the subgroups in the PO

Agreement

For Option 2, a common codepoint per PO is always used and the maximum number of subgroups supported per PO is

- 7 for the case where 4 POs are mapped to one LO

- 15 for the case where 2 POs are mapped to one LO

Agreement

Regarding whether there is any restriction on mapping multiple POs to one LO, no additional constraint for mapping multiple POs to one LO

Agreement

UE determines whether a symbol is available for LP-WUS based on:

- Alt 1: Time-domain pattern configured by the gNB

  - Alt 1A: Periodic time-domain pattern

    - E.g. (1) 1-slot periodicity, the pattern indicates the available symbols in each slot; (2) multi-slot or frame-level periodicity with a bitmap indication; (3) Search space set-like pattern; (4) multi-level time-domain patterns; (5) reuse the mechanism of rate matching pattern

  - Alt 1B: Per-MO pattern, applicable for all Mos

- Alt 2: Information from existing configurations available for idle/inactive UEs such as [SSB, CORESET/Type-0 CSS, TDD DL/UL configuration, etc].

- Alt 3: Combination of Alt 1 and Alt 2

- Alt 4: NW ensures LP-WUS configuration without collision with existing signal(s)

Agreement

For the determination of starting time locations of LP-WUS MOs and LP-WUS transmissions in a LO,

- A reference point is the start of a reference frame determined by the frame-level offset from the start of the first PF of the PF(s) associated with the LO.

- The starting time location of the first LP-WUS MO in a LO is indicated by an offset w.r.t. the reference point.

  - FFS slot-level or symbol-level offset

- The starting time locations of the subsequent LP-WUS MOs in a LO are determined based one of the following alternatives:

  - Alt 1: An offset is indicated for each of the subsequent LP-WUS MOs.

    - FFS slot-level or symbol-level offset

  - Alt 2: The start time location of a subsequent LP-WUS MO is determined implicitly at least based on the previous LP-WUS MO.

    - FFS additional configuration to control the subsequent MO locations, e.g.,

      - Alt 2A: configuration of a single gap between the end of the previous MO (or a set of previous MOs) and the start of the next MO

      - Alt 2B: configuration of candidate starting locations for MOs, similar to search space configuration

  - FFS restriction on MO locations, e.g. only on DL slots

  - FFS minimum gap is needed between two MOs to ensure LR processing time

Agreement

Terminology definition

- Nominal MO duration: this includes both the available and unavailable symbols

- Actual LP-WUS duration: the actual number of OFDM symbols used for LP-WUS transmission as assumed by the UE

Nominal MO duration and actual LP-WUS duration, if defined, are determined using one of the following alternatives:

- Alt A: Nominal MO duration is configured. (e.g. in unit of slots)

  - Actual LP-WUS duration is the number of available symbols within the MO.

  - Actual LP-WUS duration can vary from one MO to another MO.

- Alt B: Actual LP-WUS duration is configured. (e.g. in unit of OFDM symbols, or M and L values)

  - From the start of a MO, MO extends until the number of available OFDM symbols reaches the configured actual LP-WUS duration.

    - FFS: Additional termination condition such as time window

- Alt C: Both nominal MO duration and actual LP-WUS duration are configured.

  - If the number of available OFDM symbols within the nominal MO duration is less than the actual LP-WUS duration, the MO is considered as invalid (no LP-WUS monitoring in this MO) and dropped/deferred.

    - FFS UE behavior if there is no valid MO for the beam(s) that the UE monitors

  - Note: the number of available OFDM symbols within a nominal MO duration can be different for different MOs.

- Alt D: Nominal MO duration is configured. The actual LP-WUS duration is determined based on the same pattern for the available symbols for all the MOs (e.g. by using a per-MO pattern), which is the same for all the MOs.

Agreement

On how to handle the UE capability report on the wake-up delay for SSB periodicities other than 20ms, consider the following alternatives for possible down-selection in RAN1#121.

- Alt 1: Do not report for SSB periodicities other than 20ms

  - Note: LP-WUS is not supported for SSB periodicities larger than 20ms

- Alt 2: For UE capability report on the wake-up delay, the UE reports one of the following 3 capabilities (the values in one of the columns):

| SSB periodicity (ms) | Wake-up delay (ms) UE capability 1 | Wake-up delay (ms) UE capability 2 | Wake-up delay (ms) UE capability 3 |
| --- | --- | --- | --- |
| [5] | [x] | [x] | [x] |
| [10] | [x] | [x] | [x] |
| 20 | [70] | [500] | [900] |
| [40] | [x] | [x] | [x] |
| [80] | [x] | [x] | [x] |
| [160] | [x] | [x] | [x] |

- Alt 3: It is up to RAN4

Agreement

Each LP-SS transmission for each beam always occupies consecutive OFDM symbols.

# Proposals for Online Sessions

## Proposals for Monday Online

Proposal 1-1A

Confirm the following working assumption with modifications in red:

Working Assumption

If LP-WUS design support 32 subgroups within one MO, do not support Option 3 for LO to PO mapping or Option B for MO configuration.

Proposal 1-1B

Option A for MO configuration is supported.

Proposal 1-2r1

An LP-WUS MO can span across multiple slots.

- FFS the limitation on the maximum length for an LP-WUS MO

Proposal 1-3

R = 1, i.e., repetition is not supported across multiple LP-WUS MOs.

Proposed conclusion 1-5

For the offset value(s) between an LO and a reference PO/PF, do not support the configuration of 3 offset values.

Proposal 1-4

Regarding the OFDM symbols actually used for LP-WUS transmission in an MO,

- Option 1: these symbols are contiguous in time without any gap.

- Option 2: these symbols can be non-contiguous with gap(s).

Proposal 2-1A

The maximum number of POs per LO is 4, and the number of POs per LO can be 1, 2 or 4.

- Alt 1: no additional constraint for mapping multiple POs to one LO.

- Alt 2: The number of POs per LO is the same as the number of POs per PF Ns.

- Alt 3: The maximum number of PFs associated with one LO is 1.

- Alt 4: The maximum number of PFs associated with one LO is 2.

Proposal 2-1B

For Option 2, the maximum number of subgroups configured per PO is (32/(number of POs per LO) – 1).

Proposal 2-2r1

For Option 2, at least one codepoint corresponding to each of the subgroups in each PO is supported.

- For codepoint corresponding to more than one subgroups:

  - Alt 1: One codepoint corresponding to all the subgroups in all POs

  - Alt 2: One codepoint for each PO corresponding to all the subgroups in the PO

  - FFS other codepoints

Proposal 2-3

For Option 2, the maximum value of M for Option A or Option B with G=1 (if either of them is supported) is 4.

Proposal 4-1

For LP-SS, the LP-SS occasions are indexed sequentially in time, the k-th LP-SS occasion corresponds to the k-th transmitted SSB, k = 1, 2, …, N, where N is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1.

Proposal 4-2

For LP-WUS, the N * K LP-WUS MOs in an LO are indexed sequentially in time, from 1 to N*K, where N is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1, and K is the number of LP-WUS MOs for each beam.

- Option 1: The [k*N+n]th LP-WUS MO corresponds to the n-th transmitted SSB, where k=0,1,…,K-1, n=1,2,…,N. (beam first, multiple MOs second)

- Option 2: The [n*K+k]th LP-WUS MO corresponds to the n-th transmitted SSB, where k=0,1,…,K-1, n=1,2,…,N. (multiple MOs first, beam second)

Proposal 7-2

For LP-SSS-RSRP/RSSI measurement performed by OFDM-based LP-WUR for the serving cell, SMTC window is not applicable.

- FFS: whether the measurement time resource can be configured by the network or not

Proposal 1-7

The starting time location of the first LP-WUS MO in an LO is configured w.r.t. the reference point provided the reference PF minus the frame-level offset, using:

- Option 1: a slot-level offset

- Option 2: a symbol-level offset

## Proposals for Tuesday Online

Proposal 2-3r2

For Option 2, the maximum value of M in Option A for MO configuration is 4.

Proposal 4-2r1

For LP-WUS, the N * K LP-WUS MOs in an LO are indexed sequentially in time, from 1 to N*K, where N is the number of beams corresponding to LP-WUS, and K is the number of LP-WUS MOs for each beam.

- Alt 1: The [k*N+n]th LP-WUS MO corresponds to the n-th beam, where k=0,1,…,K-1, n=1,2,…,N. (beam first, multiple MOs second)

- Alt 2: The [n*K+k]th LP-WUS MO corresponds to the n-th beam, where k=0,1,…,K-1, n=1,2,…,N. (multiple MOs first, beam second)

Proposal 5-1

For the EPRE ratio between LP-WUS/LP-SS and SSB, down-select from the following:

- Option 1: Common configuration is provided for LP-WUS and LP-SS.

- Option 2: Separate configurations are provided for LP-WUS and LP-SS. The difference between the two EPRE ratios should be within [3] dB.

Proposal 7-1

For OFDM-based LP-WUR, reuse the LP-SS based LP-RSRP/LP-RSRQ definition of OOK-based LP-WUR.

- [This is applicable only if LP-SS is configured to use one of the specified overlaid OFDM sequences.]

Proposal 7-2

For LP-SSS-RSRP/RSSI measurement performed by OFDM-based LP-WUR for the serving cell, SMTC window is not applicable.

- FFS: whether the measurement time resource can be configured by the network or not

Proposal 2-2r1

For Option 2, at least one codepoint corresponding to each of the subgroups in each PO is supported.

- For codepoint corresponding to more than one subgroups:

  - Alt 1: One codepoint corresponding to all the subgroups in all POs

  - Alt 2: One codepoint for each PO corresponding to all the subgroups in the PO

  - FFS other codepoints

## Proposals for Thursday morning Online

Proposal 2-2

For Option 2, at least one codepoint corresponding to each of the subgroups in each PO is supported.

- For codepoint corresponding to more than one subgroups:

  - Option 1: One codepoint corresponding to all the subgroups in all POs

  - Option 2: One codepoint for each PO corresponding to all the subgroups in the PO

  - FFS other codepoints

Proposal 2-1B-r1

For Option 2, the maximum number of subgroups supported per PO is

- 7 for the case where 4 POs are mapped to one LO

- 15 for the case where 2 POs are mapped to one LO

Proposal 2-1C

Regarding whether there is any restriction on mapping multiple POs to one LO, down-select one from the following:

- Alt 1: no additional constraint for mapping multiple POs to one LO

- Alt 2: The number of POs per LO is the same as the number of POs per PF Ns.

- Alt 3: The maximum number of PFs associated with one LO is 1.

- Alt 4: The maximum number of PFs associated with one LO is 2.

Proposal 1-9r1

UE determines whether a symbol is available for LP-WUS based on:

- Alt 1: A time-domain pattern configured by the gNB

  - Alt 1A: A periodic time-domain pattern

    - E.g. (1) 1-slot periodicity, the pattern indicates the available symbols in each slot; (2) multi-slot or frame-level periodicity with a bitmap indication; (3) Search space set-like pattern

  - Alt 1B: per-MO pattern, applicable for all MOs

- Alt 2: Information from existing configurations available for idle/inactive UEs such as SSB, CORESET/Type-0 CSS, TDD DL/UL configuration, etc.

- Alt 3: Combination of Alt 1 and Alt 2

- FFS rate matching or puncturing

Proposal 1-10r1

Terminology definition

- Nominal MO duration: this includes both the available and unavailable symbols

- Actual LP-WUS duration: the actual number of OFDM symbols used for LP-WUS transmission

- [Actual MO duration: the actual number of OFDM symbols that the UE monitors for LP-WUS transmission]

Nominal MO duration and actual LP-WUS duration, if defined, are determined using one of the following alternatives:

- Alt A: Nominal MO duration is configured. (e.g. in unit of slots)

  - Actual LP-WUS duration is the number of available symbols within the MO.

  - Actual LP-WUS duration can vary from one MO to another MO.

- Alt B: Actual LP-WUS duration is configured and fixed. (e.g. in unit of OFDM symbols, or M and L values)

  - From the start of a MO, MO extends until the number of available OFDM symbols reaches the configured actual LP-WUS duration.

- Alt C: Both nominal MO duration and actual LP-WUS duration are configured and fixed.

  - If the number of OFDM symbols within a MO is less than the actual LP-WUS duration, the MO is considered as invalid and skipped/dropped.

- Alt D: Nominal MO duration is configured. The OFDM symbols used for LP-WUS transmission is determined based on per-MO pattern, and the UE assumes the actual LP-WUS duration is fixed for all MOs.

Proposal 1-11r1

For the determination of starting time locations of LP-WUS MOs and LP-WUS transmissions in a LO,

- A reference point is the start of a reference frame determined by the frame-level offset from the start of the first PF of the PF(s) associated with the LO.

- The starting time location of the first LP-WUS MO in a LO is indicated by an offset w.r.t. the reference point.

  - FFS slot-level or symbol-level offset

- The starting time locations of the subsequent LP-WUS MOs in a LO are determined based one of the following alternatives:

  - Alt 1: An offset is indicated for each of the subsequent LP-WUS MOs.

    - FFS slot-level or symbol-level offset

  - Alt 2: The start time location of a subsequent LP-WUS MO is determined implicitly based on the previous LP-WUS MO.

Proposal 4-1A

It is supported that gNB transmits LP-WUS/LP-SS associated with SSB indexes, and it is configured by gNB in the SIB for LP-WUS configuration on whether LP-WUS/LP-SS transmission is present or not for each SSB index.

- The detailed signaling design is up to RAN2

- For the MO location determination for LP-WUS/LP-SS, the same LP-WUS/LP-SS is transmitted repeatedly in N MOs via N beams, respectively, where

  - Alt1: N is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1, and gNB can transmits LP-WUS/LP-SS in a subset of the N MOs according to the SIB for LP-WUS configuration.

  - Alt2: N is the number of SSBs associated with LP-WUS/LP-SS, which is determined by the SIB for LP-WUS configuration.

## Proposals for Thursday afternoon Online

Proposal 2-1C

Regarding whether there is any restriction on mapping multiple POs to one LO, down-select one from the following:

- Alt 1: no additional constraint for mapping multiple POs to one LO

- Alt 2: The number of POs per LO is the same as the number of POs per PF Ns.

- Alt 3: The maximum number of PFs associated with one LO is 1.

- Alt 4: The maximum number of PFs associated with one LO is 2.

Proposal 1-9r2

UE determines whether a symbol is available for LP-WUS based on:

- Alt 1: Time-domain pattern configured by the gNB

  - Alt 1A: Periodic time-domain pattern

    - E.g. (1) 1-slot periodicity, the pattern indicates the available symbols in each slot; (2) multi-slot or frame-level periodicity with a bitmap indication; (3) Search space set-like pattern; (4) multi-level time-domain patterns; (5) reuse the mechanism of rate matching pattern

  - Alt 1B: Per-MO pattern, applicable for all MOs

- Alt 2: Information from existing configurations available for idle/inactive UEs such as SSB, CORESET/Type-0 CSS, TDD DL/UL configuration, etc.

- Alt 3: Combination of Alt 1 and Alt 2

FFS rate matching or puncturing

Proposal 1-11r2

For the determination of starting time locations of LP-WUS MOs and LP-WUS transmissions in a LO,

- A reference point is the start of a reference frame determined by the frame-level offset from the start of the first PF of the PF(s) associated with the LO.

- The starting time location of the first LP-WUS MO in a LO is indicated by an offset w.r.t. the reference point.

  - FFS slot-level or symbol-level offset

- The starting time locations of the subsequent LP-WUS MOs in a LO are determined based one of the following alternatives:

  - Alt 1: An offset is indicated for each of the subsequent LP-WUS MOs.

    - FFS slot-level or symbol-level offset

    - (overhead concern)

  - Alt 2: The start time location of a subsequent LP-WUS MO is determined implicitly at least based on the previous LP-WUS MO.

    - FFS additional configuration to control the subsequent MO locations, e.g.,

      - Alt 2A: configuration of a single gap between the end of the previous MO (or a set of previous MOs) and the start of the next MO

      - Alt 2B: configuration of candidate starting locations for MOs, similar to search space configuration

      - …

  - FFS restriction on MO locations, e.g. only on DL slots

  - FFS minimum gap is needed between two MOs to ensure LR processing time

Proposal 1-10r1

Terminology definition

- Nominal MO duration: this includes both the available and unavailable symbols

- Actual LP-WUS duration: the actual number of OFDM symbols used for LP-WUS transmission

- [Actual MO duration: the actual number of OFDM symbols that the UE monitors for LP-WUS transmission]

Nominal MO duration and actual LP-WUS duration, if defined, are determined using one of the following alternatives:

- Alt A: Nominal MO duration is configured. (e.g. in unit of slots)

  - Actual LP-WUS duration is the number of available symbols within the MO.

  - Actual LP-WUS duration can vary from one MO to another MO.

- Alt B: Actual LP-WUS duration is configured and fixed. (e.g. in unit of OFDM symbols, or M and L values)

  - From the start of a MO, MO extends until the number of available OFDM symbols reaches the configured actual LP-WUS duration.

- Alt C: Both nominal MO duration and actual LP-WUS duration are configured and fixed.

  - If the number of OFDM symbols within a MO is less than the actual LP-WUS duration, the MO is considered as invalid and skipped/dropped.

- Alt D: Nominal MO duration is configured. The OFDM symbols used for LP-WUS transmission is determined based on per-MO pattern, and the UE assumes the actual LP-WUS duration is fixed for all MOs.

## Proposals for Friday Online

Proposal 1-10r2

Terminology definition

- Nominal MO duration: this includes both the available and unavailable symbols

- Actual LP-WUS duration: the actual number of OFDM symbols used for LP-WUS transmission as assumed by the UE

Nominal MO duration and actual LP-WUS duration, if defined, are determined using one of the following alternatives:

- Alt A: Nominal MO duration is configured. (e.g. in unit of slots)

  - Actual LP-WUS duration is the number of available symbols within the MO.

  - Actual LP-WUS duration can vary from one MO to another MO.

- Alt B: Actual LP-WUS duration is configured. (e.g. in unit of OFDM symbols, or M and L values)

  - From the start of a MO, MO extends until the number of available OFDM symbols reaches the configured actual LP-WUS duration.

    - FFS: Additional termination condition such as time window

- Alt C: Both nominal MO duration and actual LP-WUS duration are configured.

  - If the number of available OFDM symbols within the nominal MO duration is less than the actual LP-WUS duration, the MO is considered as invalid (no LP-WUS monitoring in this MO) and dropped/deferred.

    - FFS UE behavior if there is no valid MO for the beam(s) that the UE monitors

  - Note: the number of available OFDM symbols within a nominal MO duration can be different for different MOs.

- Alt D: Nominal MO duration is configured. The actual LP-WUS duration is determined based on the same pattern for the available symbols for all the MOs (e.g. by using a per-MO pattern), which is the same for all the MOs.

Proposal 3-1r1

On how to handle the UE capability report on the wake-up delay for SSB periodicities other than 20ms,

- Alt 1: do not report

- Alt 2: For UE capability report on the wake-up delay, the UE reports one of the following 3 capabilities:

| SSB periodicity (ms) | Wake-up delay (ms) UE capability 1 | Wake-up delay (ms) UE capability 2 | Wake-up delay (ms) UE capability 3 |
| --- | --- | --- | --- |
| 5 | [x] | [425] | [825] |
| 10 | [x] | [450] | [850] |
| 20 | [70] | [500] | [900] |
| 40 | [130] | [600] | [1000] |
| 80 | [250] | [800] | [1200] |
| 160 | [490] | [1200] | [1600] |

Proposal 6-4

Each LP-SS transmission occupies consecutive OFDM symbols.

Proposal 4-1A-r1

It is supported that gNB transmits LP-WUS/LP-SS associated with SSB indexes, and it is configured by gNB in the SIB for LP-WUS configuration on whether LP-WUS/LP-SS transmission is present or not for each SSB index.

- The detailed signaling design is up to RAN2

- For the LP-WUS MO and LP-SS occasion determination, the same LP-WUS/LP-SS is transmitted repeatedly in N beams, respectively, where

  - Alt1: S is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1, and LP-WUS MOs and LP-SS occasions are determined assuming S beams. gNB transmits LP-WUS/LP-SS in a subset of the S beams according to the LP-WUS configuration in SIB.

  - Alt2: N is the number of SSBs associated with LP-WUS/LP-SS, which is determined by the SIB for LP-WUS configuration. LP-WUS MOs and LP-SS occasions are determined assuming N beams.

# LP-WUS Operation in Idle/Inactive Mode

## LP-WUS configuration and monitoring

### LP-WUS MO configuration

The following was agreed on the multiple MOs per beam per LO:

Agreement

When K (K>1) LP-WUS MOs are configured for each beam in an LO, down select between

- Option A: K LP-WUS MOs for a beam are divided into M (M >=1) groups of R LP-WUS MOs. A UE monitors all or some of the MO(s) within the K LP-WUS MOs.

  - For each group of R LP-WUS MOs, the same LP-WUS information is transmitted.

    - FFS how the same LP-WUS information is transmitted in the R LP-WUS MOs

  - Different LP-WUS information can be transmitted in different groups of R LP-WUS MOs.

  - M = 1 and M > 1 is supported.

  - FFS: detailed UE monitoring behavior

  - FFS R=1 or R>= 1

- Option B: K LP-WUS MOs for a beam are divided into G (G >= 1) groups of R*M (M >= 1) LP-WUS MOs. A UE monitors all or some of the MO(s) within one group of R*M LP-WUS MOs based on its subgroup ID.

  - Each group of R*M LP-WUS MOs is further divided into M groups of R LP-WUS MOs.

    - For each group of R LP-WUS MOs, the same LP-WUS information is transmitted.

      - FFS how the same LP-WUS information is transmitted in the R LP-WUS MOs

    - Different LP-WUS information can be transmitted in different groups of R LP-WUS MOs.

    - FFS: detailed UE monitoring behavior

  - M = 1 and M > 1 is supported.

  - FFS R=1 or R>=1

  - Note: this achieves the same purpose as “Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO.”

For LO-to-PO mapping, we had the following agreements:

Agreement

It is supported that UEs monitoring the same PO are divided into multiple subgroups, where LP-WUS can provide wake-up indication for each subgroup. Consider the following options:

- Option 1: UEs monitoring the same PO monitor the same LO

- Option 2: UEs corresponding to different POs monitor the same LO

- Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO.

- Combinations of the above options can be considered.

Agreement

For the mapping between LO and PO, supports at least Option 1 (UEs monitoring the same PO monitor the same LO).

Working Assumption

If LP-WUS design support 32 subgroups within one MO, do not support Option 3 for LO to PO mapping or Option B for MO configuration.

There are still a few companies proposing to support Option 3 and/or Option B:

- Option 3: [12] InterDigital, [16] LG (or Option B), [20] Sharp (if the maximum number of subgroups within one LP-WUS MO is less than 32)

- Option B: [1] Futurewei (with R>=1), [16] LG (or Option B), [18] Panasonic (for multiple POs per LO)

Specific MO for all-subgroup codepoint?

- [2] ZTE

Vivo: frame-level offset + slot-level offset -> the start of the first MO

[3] Huawei/HiSi: >4 MOs per PO if one LO maps to multiple POs

Some more detailed proposals are included at the end of this sub-section.

There are only a few companies proposing to consider Option 3 or Option B, and the motivations are not clear to the moderator. There are also many companies proposing to support Option A instead of Option B. The proposal below is to confirm the working assumption of not supporting Option 3 for LO to PO mapping or Option B for MO configuration, with the removal of the condition.

[CLOSED] Proposal 1-1

Confirm the following working assumption with modifications in red:

Working Assumption

If LP-WUS design support 32 subgroups within one MO, do not support Option 3 for LO to PO mapping or Option B for MO configuration.

Note: this means Option A for MO configuration is supported.

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | Separate this working assumption and Note. And this NOTE should be an agreement. |
| CMCC | Support |
| Spreadtrum | Support |
| Xiaomi | Support |
| LG | We have different view with FL’s assessment. The previous agreed working assumption clearly shows the reason to support Option 3 and Option B. We think it is not necessary to discuss what motivation is anymore. We prefer to wait the outcome of 9.6.1 which can confirm the condition is met or not. |
| Huawei, HiSilicon | Support. About ZTE’s question, our understanding is that the note is also agreed. |
| Nokia1 | OK with the proposal. |
| Sharp | Support |
| DCM | Support |
| Samsung | We have one clarification question for the proposal. If it is supported, does it mean the case that multiple LOs to multiple POs mapping is not supported? Since option 2 for LO:PO mapping is supported, which means one LO can be mapped with more than one POs to reduce the resource overhead for LP-WUS, we think multiple LOs to multiple POs mapping should also be supported follow a same logic to support option 2. For example, if more than 16 subgroups are configured, only 1:1 LO:PO mapping can be supported since the total number of subgroups will be larger than 31. But if multiple-to-multiple LO:PO mapping is supported, less LO can be used to indicate the subgroups in these POs compared with one-to-one mapping. The number of subgroups is consecutive for 1:1, 1:X and X:X LO:PO mapping, we suggest to support a full mapping relationship other than only part of mapping relationship. |
| Moderator | @LG, so far I don’t seem to see any companies suggesting that one LP-WUS cannot carry 32 codepoints. Ideally we could wait for 9.6.1 outcome. However, we have very tight schedule now and we need to move things forward. There are still a lot of signaling details to be decided. @Samsung, I think the case you are proposing is an additional optimization. Option 2 was agreed as a sort of compromise, and it is not clear to me if we need further optimization for some cases between Option 1 and 2.some cases between Option 1 and 2. @all, my reason to have it as a note was that we agreed to down-select between Option A and Be in the previous agreement. If we agree not to support B, then A is automatically supported. But if companies want to have it explicitly as an agreement, that is also fine. Proposal 1-1A Confirm the following working assumption with modifications in red: Working Assumption If LP-WUS design support 32 subgroups within one MO, do not support Option 3 for LO to PO mapping or Option B for MO configuration. Proposal 1-1B Option A for MO configuration is supported. |
| Moderator | Both proposals were agreed during online session. Discussion closed. |

One MO confined within one slot

- Yes: [11] OPPO, [12] InterDigital, [17] ETRI, [20] MediaTek

- No: [2] ZTE, [3] vivo, [8] NEC, [9] CMCC, [10] Huawei/HiSi (consecutive slots), [13] Sony, [14] Samsung, [19] Apple

Can be non-contiguous OFDM symbols used for LP-WUS in an MO or not?

- Non-contiguous: [3] vivo, [10] Huawei/HiSi, [19] Apple

- Contiguous: [9] CMCC (?), [12] InterDigital, [13] Sony, [14] Samsung, [17] ETRI

Whether to support repetition across multiple MOs (R=1 or R>=1):

- R>=1: [9] CMCC, [12] InterDigital, [16] LG, [17] ETRI, [20] Sharp, [20] MediaTek (<=2)

- R=1: [3] vivo, [14] Samsung, [19] Apple, [25] Nordic

[25] Nordic: Proposal-1: LP-WUS repetitions (if any) are defined within a MO, i.e. R=1. Each LP-WUS repetition is confined within a slot. Repetitions can be separated by “short”gaps.

With the agreement on the LP-WUS signal design, one full repetition of an LP-WUS goes beyond one slot. With 32 bits after RM coding, it becomes 64 OOK symbols after Manchester coding. The full duration goes beyond one slot for M=1, 2 or 4. From the evaluations, it is also obvious the coverage cannot be satisfied for an LP-WUS within one slot. Unless one LP-WUS is segmented into multiple parts mapping to different MOs (not simple repetition), it is not possible to confine one LP-WUS MO to be within one slot.

[CLOSED] Proposal 1-2

An LP-WUS MO can span across multiple slots.

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | Yes |
| CMCC | Support |
| Spreadtrum | Support |
| Xiaomi | Support |
| LG | Beside the rate matching are under the discussion, we prefer to have any level of limitation in terms of the length of LP-WUS instance. Consider the TDD case and current discussion, without any limitation on LP-WUS instance, we would have 10 slot length of LP-WUS transmission with one or more >10 symbol gaps or empty 6 slot due to TDD handling. We would like to avoid such situation and reduce UE implementation complexity. We prefer that One MO confined within one slot or we want to discuss how may slot/symbol one LP-WUS can span. |
| Huawei, HiSilicon | Support. Especially for M=1 case, confining a LP-WUS within a single MO cannot provide Msg3 coverage. To avoid too many different designs, we suggest to use a unified framework to serve for all M values. |
| Nokia1 | We could wait a bit for decisions in Ag. 9.6.1 for LP-WUS length, but would seem hard to avoid. |
| Sharp | support |
| DCM | Support. |
| Samsung | Support |
| Moderator | @Nokia, I understand the comment, which would be ideal. But we have a tight schedule here and need to move things forward. Proposal 1-2r1 An LP-WUS MO can span across multiple slots. - FFS the limitation on the maximum length for an LP-WUS MO |
| Moderator | The proposal was agreed during online session. Discussion closed. |

If it is agreed that an LP-WUS MO can span over multiple slots, there is not much need to support repetitions across multiple MOs, because any repetition (if needed) can be done within an MO.

[CLOSED] Proposal 1-3

R = 1, i.e., repetition is not supported across multiple LP-WUS MOs.

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | Yes. It can be achieved by rate matching for RM code. |
| Xiaomi | Support |
| Huawei, HiSilicon | Support. Based on our evaluation, considering up to 5 info bits, 32 coded bits are enough to provide a coverage performance better than Msg3 PUSCH. Thus, further repetition is not needed. Instead, rate-matching to less than 32 bits should be further discussed in 9.6.1. |
| Nokia1 | As noted earlier repetitions do not offer significant gain for non-coherent receivers, thus having configurable LP-WUS length (to account the encoding) would seem preferable to repetition. |
| Sharp | Repetition can be configured up to gNB to keep resource allocation flexibility for non-continues resources |
| DCM | Support |
| Samsung | Agree with ZTE. |
| Moderator | The following was agreed during online session. Discussion closed. Agreement Only R = 1 is supported for Option A. |

Regarding whether the OFDM symbols used for a LP-WUS transmission should be contiguous in time or not, there seem to be divergent views. Please share your views on the following two options.

[CLOSED] Proposal 1-4

Regarding the OFDM symbols actually used for LP-WUS transmission in an MO,

- Option 1: these symbols are contiguous in time without any gap.

- Option 2: these symbols can be non-contiguous with gap(s).

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | OK with this proposal. Option2 is better for gNB flexibility for scheduling |
| CMCC | Generally fine with the proposal |
| Spreadtrum | Support |
| Xiaomi | Support Option 1 to reduce UE complexity. |
| LG | Fine with the proposal. We support Option 1. |
| Huawei, HiSilicon | We support option 2. Due to the presence of CORESET, it is difficult to make sure that all OFDM symbols are contiguous if multiple slots are occupied. |
| Nokia1 | I think (as LP-WUS is expected to exceed one slot or more) to consider how certain broadcast transmissions. If LP-WUS overlaps with the initial BWP SSB and common (e.g. Type0) PDCCH needs to be considered. Overlap with SSB is probably avoidable via configuration, but for e.g. Type0 PDCCH some puncturing/rate matching is preferred. UE should be aware of these symbols based on broadcast configuration. Hence, option 2 would be preferable to account this possibility. |
| ETRI | Fine with the proposal. |
| DCM | Support the proposal. But overlapping between legacy signals such as PDCCH and LP-WUS should be discussed later. |
| Samsung | Support. |
| Moderator | Under AI 9.6.1, the following was agreed: [H][FL1] Proposal 3.2-2: Support maximum length for a LP-WUS (actual OFDM symbols used) which is larger than one slot - Non-consecutive OFDM symbols at least across multiple slots can be used for a LP-WUS transmission This means a flavor of Option 2 has been agreed. This discussion is closed. But details will be discussed first. |

For the offset value(s) between an LO and a reference PO/PF, it was agreed that gNB can configure 1 or 2 offset values. The remaining FFS is whether gNB can configure 3 offset values.

- Yes: [1] Futurewei, [4] TCL, [19] Apple, [24] QC

- No: [3] vivo, [5] Spreadtrum, [7] Nokia, [10] Huawei/HiSi, [12] InterDigital, [14] Samsung, [17] ETRI, [21] MediaTek, [22] DCM, [23] Ericsson, [25] Nordic

Majority of the companies prefer not to support 3 offset values.

[CLOSED] Proposal 1-5

For the offset value(s) between an LO and a reference PO/PF, do not support the configuration of 3 offset values.

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | OK with FL proposal. |
| CMCC | Generally fine with the proposal |
| Spreadtrum | Support |
| LG | Support. |
| Huawei, HiSilicon | Support the proposal. BTW, it should be a conclusion, right? |
| Nokia1 | OK with the proposal. |
| ETRI | Support the proposal. |
| DCM | Support the proposal. 3 offset values would cause large impact to NW overhead but make small paging latency benefit compared to 2 offset values. Moreover, larger number of candidate values cause NW/UE complexity. Thus, we do not support 3 offset values. |
| Samsung | Support. |
| Moderator | We will have it as a proposed conclusion. Proposed conclusion 1-5 For the offset value(s) between an LO and a reference PO/PF, do not support the configuration of 3 offset values. |
| OPPO | Support |
| Moderator | The proposed conclusion was agreed. Discussion closed. |

The following was agreed in RAN1#120:

Agreement

For the offset value(s) between an LO and a reference PO/PF, at least a frame-level offset is provided.

- The reference point (reference PO/PF) for the frame-level offset is the start of the PF, or the first PF of the PF(s) (if mapping of POs from multiple PFs to one LO is supported), associated with the LO.

- FFS other offset value(s) to determine the MOs of the LO

When there are multiple POs per PF for Option 1, or the number of POs per PF is larger than the number of POs per LO for Option 2 (if supported), more than one LOs share the same reference PF. There is a need to differentiate the multiple LOs, and a straightforward way is to configure multiple frame-level offset values. Not many companies discussed this issue. The following is proposed to trigger the discussion.

Proposal 1-6

When multiple LOs have the same reference PF, multiple frame-level offset values are configured, with one offset value corresponding to each one of the multiple LOs.

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | If multiple frame level offset is configured, the gNB configuration would be quite limited, since within one PF, different LO may be overlapped as shown in the following figure. We already agreed that, for one UE, it can be configured with two offsets(710ms and 70ms in the figure). If also, multiple frame-level offset values are configured, the situation would be quite complicated for the gNB. As shown in the figure, the smaller offset for PO1/PO2 would be overlapped with the larger offset for PO3/PO4, which would cause UE decoding problem. Given that the precondition for multiple frame offset as mentioned by FL is ‘the number of POs per PF is larger than the number of POs per LO for Option 2 (if supported)’, if we consider the number of POs per PF is the same the number of POs per LO(option2 in Proposal 2-1) for Option 2, then there is no issue. |
| Xiaomi | Generally OK. |
| LG | We are fine with the intention, but the the formulation of proposal is reversed. LO is determined by offset. Thus, the condition “multiple LOs have the same reference PF” is outcome of this proposal. In our view, the proposal should be revised as following: Revised Proposal 1-6 by LG For the case that the multiple PO are in the same PF, multiple frame-level offset values can be configured, with one offset value corresponding to each one of the POs within the PF. |
| Huawei, HiSilicon | We don’t see the need to introduce multiple frame-level offsets. Instead, multiple symbol-level offsets can achieve the same effect, just like the PEI-O design. |
| Samsung | One question for clarification, does “multiple LOs” mentioned in the proposal include the LOs associated with same number of PO(s) but target for different UE capabilities, like one LO target for ultra-deep sleep UEs and another for deep sleep UEs? |
| OPPO | Support multiple symbol-level offsets to address this issue. |
| vivo | We support to configure multiple frame-level offset values for the case that the number of POs per LO is smaller than the number of POs per PF. Besides, we see no benefit but the negative impact on UE power saving gain to limit the number of POs per LO should always be equal to the number of POs per PF, especially consider that LO and PO 1 to 1 mapping is the basic one. For example, when there is 4 POs in a PF, by adopting option2, the UE power saving gain will be largely impacted since the number of subgroups per PO is limited to no more than 7. Hence, the case (the number of POs per LO is smaller than the number of POs per PF) that mentioned by FL is valid and exist. Besides, we agree that introducing multiple symbol-level offsets also be a way to go. But if multiple symbol-level offsets are introduced, there will be a very big symbol level value range e.g., up to 900ms*14symbols*32=403200 symbols for SCS=480KHz, which will bring lots of RRC signalling overhead, so we think multiple frame-level offsets (the value range is in the unit of frame e.g., 90) is more suitable than multiple symbol-level offsets. |
| CATT | Similar views with HW, the first MO in a LO can be determined by a symbol-level offset. There is no need to introduce multiple frame-level offsets. |
| FW | We suggest limiting the discussion to the multiple LOs for the same wake-up delay capability. Then, a slot-level offset along with sufficient MO spacing within the same LO can be used to solve the overlapping problem for the POs within the same PF. |

[CLOSED] Proposal 1-7

The starting time location of the first LP-WUS MO in an LO is configured w.r.t. the reference point provided the reference PF minus the frame-level offset, using:

- Option 1: a slot-level offset

- Option 2: a symbol-level offset

| Company | Comments |  |
| --- | --- | --- |
| ZTE, Sanechips | Option2, same as PEI. |  |
| CMCC | Generally fine with the proposal and prefer option 2. |  |
| Spreadtrum | Prefer Option 2. |  |
| Xiaomi | Support Option 2. |  |
| LG | Fine with the proposal |  |
| Huawei, HiSillicon | Option 2. First, it is the design of PEI, which is justified to work. Second, only with slot-level offset may not be able to indicate the exact starting location of a MO, due to the granularity is not fine enough. Last, I saw some companies argued in their tdoc that slot-level offset can be used together with symbol-level offset to reduce the overhead of configuration. But this is not true, which can be justified by the following example. - “Symbol level offset only” has the same signalling overhead as “symbol level offset + slot level offset”. For example, with symbol level offset only, the value range can be 0~139, corresponding to 8 bits. While with “symbol level offset + slot level offset”, the value range of symbol level offset is 0~13 corresponding to 4 bits, and the value range of slot level offset is 0~9, corresponding to 4 bits, and thus in total 8 bits. |  |
| Nokia1 | Slight preference for option 2 |  |
| Sharp | Prefer Option1,slot-level offset is sufficient for LP-WUS resource allocation |  |
| DCM | Before discussing this, whether overlapping between legacy signals such as PDCCH and LP-WUS should be avoided or not in a MO should be discussed. If overlapping between legacy signals such as PDCCH and LP-WUS in a MO is not permitted, we can not see merit to introduce slot/symbol level offset. |  |
| Samsung | Support in general. |  |
| OPPO | Support option2 |  |
| vivo | Support option 2. |  |
| CATT | Support option 2. |  |
| FW | OK with support of Option 1 as discussed in previous comment. |  |
| Moderator | Proposal 1-7r1 The starting time location of the first LP-WUS MO in an LO is configured w.r.t. the reference point provided the reference PF and the frame-level offset, using: - Alt 1: a slot-level offset - The transmission of LP-WUS starts at the first available symbol in the slot. FFS how available symbols are determined, e.g. via a pattern or legacy configuration such as SSB, CORESET, TDD DL/UL configuration - Alt 2: a symbol-level offset - The first symbol is available for LP-WUS transmission |  |
| DCM | Now we do not see any merit to introduce slot/symbol level offset. The intention to introduce the offset is to avoid overlapping of legacy signals and LP-WUS. But non-consecutive transmission on a MO is already agreed in Monday online session. This way can solve overlapping issue. Thus, unless there is additional merit to introduce such offset, we can not support the proposal. |  |
| LG | Based on DCM’s comment, the slot/symbol level offset may be only useful for the issue which Proposal 1-6 is trying to address. Can we combine Proposal 1-6 and 1-7? |  |
| Moderator | This is reformulated in P1-9/10/11 below. |

[CLOSED] Proposal 1-8

Regarding the number of available OFDM symbols within a MO,

- Alt 1: It can be different for different MOs. FFS any restrictions.

- Alt 2: Same for all the MOs

| Company | Comments |  |
| --- | --- | --- |
| Panasonic | In our understanding, Alt 2 is a good starting point for lower complexity. |  |
| DCM | Support Alt.2. We do not see any merit on Alt.1 to solve any issue. Moreover, we do not see feasibly of Alt.1, since UE/subgroup specific or LO/MO specific configuration can not be done in IDLE/INACTIVE mode. |  |
| Qualcomm | There seems no strong need to mandate the same MO duration (i.e., start to end symbol), but it preferrable to have the same LP-WUS length. Hence, Alt 2 is preferred. |  |
| LG | It would be OK not to have the same MO duration, but prolonged LO duration may bring some issue. To adopt Alt. 2, we think the restriction on LO duration is necessary. |  |
| Moderator | This is reformulated in P1-10 below. |

[CLOSED] Proposal 1-9

UE determines whether a symbol is available for LP-WUS transmission based on:

- Alt 1: A periodic time-domain pattern configured by the gNB

  - E.g. (1) 1-slot periodicity, the pattern indicates the available symbols in each slot; (2) multi-slot or frame-level periodicity with a bitmap indication

- Alt 2: Information from legacy configurations such as SSB, CORESET/Type-0 CSS, TDD DL/UL configuration, etc.

- Alt 3: Combination of Alt 1 and Alt 2

| Company | Comments |
| --- | --- |
| Moderator | Proposal 1-9r1 UE determines whether a symbol is available for LP-WUS based on: - Alt 1: A time-domain pattern configured by the gNB - Alt 1A: A periodic time-domain pattern - E.g. (1) 1-slot periodicity, the pattern indicates the available symbols in each slot; (2) multi-slot or frame-level periodicity with a bitmap indication; (3) Search space set-like pattern - Alt 1B: per-MO pattern, applicable for all MOs - Alt 2: Information from existing configurations available for idle/inactive UEs such as SSB, CORESET/Type-0 CSS, TDD DL/UL configuration, etc. - Alt 3: Combination of Alt 1 and Alt 2 - FFS rate matching or puncturing |
| Moderator | Proposal 1-9r2 UE determines whether a symbol is available for LP-WUS based on: - Alt 1: Time-domain pattern configured by the gNB - Alt 1A: Periodic time-domain pattern - E.g. (1) 1-slot periodicity, the pattern indicates the available symbols in each slot; (2) multi-slot or frame-level periodicity with a bitmap indication; (3) Search space set-like pattern; (4) multi-level time-domain patterns; (5) reuse the mechanism of rate matching pattern - Alt 1B: Per-MO pattern, applicable for all MOs - Alt 2: Information from existing configurations available for idle/inactive UEs such as SSB, CORESET/Type-0 CSS, TDD DL/UL configuration, etc. - Alt 3: Combination of Alt 1 and Alt 2 FFS rate matching or puncturing |
| Moderator | A modified version was agreed. Discussion closed. |

[CLOSED] Proposal 1-10

Terminology definition

- Nominal MO duration: this includes both the available and unavailable symbols

- Actual LP-WUS duration: the actual number of OFDM symbols used for LP-WUS transmission

- [Actual MO duration: the actual number of OFDM symbols that the UE monitors for LP-WUS transmission]

Nominal MO duration and actual LP-WUS duration, if defined, are determined using one of the following alternatives:

- Alt A: Nominal MO duration is configured. (e.g. in unit of slots)

  - Actual LP-WUS duration is the number of available symbols within the MO.

  - Actual LP-WUS duration can vary from one MO to another MO.

- Alt B: Actual LP-WUS duration is configured and fixed. (e.g. in unit of OFDM symbols, or M and L values)

  - From the start of a MO, MO extends until the number of available OFDM symbols reaches the configured actual LP-WUS duration.

- Alt C: Both nominal MO duration and actual LP-WUS duration are configured and fixed.

  - If the number of OFDM symbols within a MO is less than the actual LP-WUS duration, the MO is considered as invalid and skipped.

| Company | Comments |
| --- | --- |
| Moderator | Proposal 1-10r1 Terminology definition - Nominal MO duration: this includes both the available and unavailable symbols - Actual LP-WUS duration: the actual number of OFDM symbols used for LP-WUS transmission - [Actual MO duration: the actual number of OFDM symbols that the UE monitors for LP-WUS transmission] Nominal MO duration and actual LP-WUS duration, if defined, are determined using one of the following alternatives: - Alt A: Nominal MO duration is configured. (e.g. in unit of slots) - Actual LP-WUS duration is the number of available symbols within the MO. - Actual LP-WUS duration can vary from one MO to another MO. - Alt B: Actual LP-WUS duration is configured and fixed. (e.g. in unit of OFDM symbols, or M and L values) - From the start of a MO, MO extends until the number of available OFDM symbols reaches the configured actual LP-WUS duration. - Alt C: Both nominal MO duration and actual LP-WUS duration are configured and fixed. - If the number of OFDM symbols within a MO is less than the actual LP-WUS duration, the MO is considered as invalid and skipped/dropped. - Alt D: Nominal MO duration is configured. The OFDM symbols used for LP-WUS transmission is determined based on per-MO pattern, and the UE assumes the actual LP-WUS duration is fixed for all MOs. |
| Moderator | Proposal 1-10r2 Terminology definition - Nominal MO duration: this includes both the available and unavailable symbols - Actual LP-WUS duration: the actual number of OFDM symbols used for LP-WUS transmission as assumed by the UE Nominal MO duration and actual LP-WUS duration, if defined, are determined using one of the following alternatives: - Alt A: Nominal MO duration is configured. (e.g. in unit of slots) - Actual LP-WUS duration is the number of available symbols within the MO. - Actual LP-WUS duration can vary from one MO to another MO. - Alt B: Actual LP-WUS duration is configured. (e.g. in unit of OFDM symbols, or M and L values) - From the start of a MO, MO extends until the number of available OFDM symbols reaches the configured actual LP-WUS duration. - FFS: Additional termination condition such as time window - Alt C: Both nominal MO duration and actual LP-WUS duration are configured. - If the number of available OFDM symbols within the nominal MO duration is less than the actual LP-WUS duration, the MO is considered as invalid (no LP-WUS monitoring in this MO) and dropped/deferred. - FFS UE behavior if there is no valid MO for the beam(s) that the UE monitors - Note: the number of available OFDM symbols within a nominal MO duration can be different for different MOs. - Alt D: Nominal MO duration is configured. The actual LP-WUS duration is determined based on the same pattern for the available symbols for all the MOs (e.g. by using a per-MO pattern), which is the same for all the MOs. |
| Moderator | This was agreed. Discussion closed. |

[CLOSED] Proposal 1-11

For the determination of starting time locations of LP-WUS MOs and LP-WUS transmissions in a LO,

- A reference point is the start of a reference frame determined by the frame-level offset from the start of the first PF of the PF(s) associated with the LO.

- The starting time location of the first LP-WUS MO in a LO is indicated by an offset w.r.t. the reference point.

  - FFS slot-level or symbol-level offset

- For Alt B, the starting time locations of the subsequent LP-WUS MOs in a LO are determined based one of the following alternatives:

  - Alt 1: An offset is indicated for each of the subsequent LP-WUS MOs.

    - FFS slot-level or symbol-level offset

  - Alt 2: The start time location of a subsequent LP-WUS MO is determined based on the previous LP-WUS MO.

| Company | Comments |
| --- | --- |
| Moderator | Proposal 1-11r1 For the determination of starting time locations of LP-WUS MOs and LP-WUS transmissions in a LO, - A reference point is the start of a reference frame determined by the frame-level offset from the start of the first PF of the PF(s) associated with the LO. - The starting time location of the first LP-WUS MO in a LO is indicated by an offset w.r.t. the reference point. - FFS slot-level or symbol-level offset - For Alt B, tThe starting time locations of the subsequent LP-WUS MOs in a LO are determined based one of the following alternatives: - Alt 1: An offset is indicated for each of the subsequent LP-WUS MOs. - FFS slot-level or symbol-level offset - Alt 2: The start time location of a subsequent LP-WUS MO is determined implicitly based on the previous LP-WUS MO. |
| Moderator | Proposal 1-11r2 For the determination of starting time locations of LP-WUS MOs and LP-WUS transmissions in a LO, - A reference point is the start of a reference frame determined by the frame-level offset from the start of the first PF of the PF(s) associated with the LO. - The starting time location of the first LP-WUS MO in a LO is indicated by an offset w.r.t. the reference point. - FFS slot-level or symbol-level offset - The starting time locations of the subsequent LP-WUS MOs in a LO are determined based one of the following alternatives: - Alt 1: An offset is indicated for each of the subsequent LP-WUS MOs. - FFS slot-level or symbol-level offset - (overhead concern) - Alt 2: The start time location of a subsequent LP-WUS MO is determined implicitly at least based on the previous LP-WUS MO. - FFS additional configuration to control the subsequent MO locations, e.g., - Alt 2A: configuration of a single gap between the end of the previous MO (or a set of previous MOs) and the start of the next MO - Alt 2B: configuration of candidate starting locations for MOs, similar to search space configuration - … - FFS restriction on MO locations, e.g. only on DL slots - FFS minimum gap is needed between two MOs to ensure LR processing time |
| Moderator | A modified version was agreed. Discussion closed. |

Detailed proposals on MO configurations/monitoring:

| [2] ZTE | Proposal 2: LP-WUS MO should be defined based on number of slots and number of symbols. Proposal 3: Symbol level gap between MOs are allowed. FFS slot level gap Proposal 4: Discuss and decide how the MOs organized in time domain • The order for flexible MOs with M>=1, repetition number(if supported), beam index • The gap between MO including beam sweeping |
| --- | --- |
| [3] vivo | Proposal 2: UE is required to monitor each of M groups per beam until its target LP-WUS is detected. Proposal 7: A LP-WUS MO can be configured considering the follows:  The length of a LP-WUS MO is configured contiguous in time by the network. And a LP-WUS MO is a valid MO, if the available symbols within the MO is no smaller than required number of symbols to transmit LP-WUS according to the configured code block length L.  There is no time gap between adjacent LP-WUS MOs within a LO.  The first LP-WUS MO position of a LO is determined by the slot-level offset. Proposal 12: {8,..,154} can be the starting point of the frame-level offset value range for 20ms SSB periodicity. FFS: the frame-level offset value range for other SSB periodicity. Proposal 13: For the case PO-to-LO association is smaller than Ns, a list of frame-level offset needs to be configured, and each frame-level offset determines the start of a frame for each LO. Proposal 14: A UE applies the (flooris/POtoLO association+1) -th value out of Ns/POtoLO association values of the frame-level offset list, wherein is is the associated PO index of the UE, POtoLO association is the configured number of PO per LO. Proposal 15: Support to define a slot-level offset to determine the start of the first MO for a LO which is relative to the start of the LO frame boundary derived by the frame-level offset. - The value range for the slot-level offset can be {0,…,10*2^μ}. |
| [4] TCL | Proposal 4: For the additional offset values between an LO and a reference PO/PF, consider symbol-level or slot-level offsets between the LO and the reference PO/PF. Proposal 7: The maximum number of MOs within an LO that a UE monitors for the same beam is set to at least 44. |
| [6] CATT | Proposal 8: If R>=1 is supported, gNB needs to transmit LP-WUS on every LP-WUS MO to allow UE accumulating the received energy for LP-WUS detection for coverage enhancement. Once the UE detects a LP-WUS to wake up at one of multiple MOs, the UE is not required to monitor the remaining MO(s) associated with the same LO. Proposal 9: The LP-WUS MO can be by configuring a set of locations of LP-WUS MOs. |
| [7] Nokia | Observation: Additional offset(s) are needed to indicate the LP-WUS MO location(s). Proposal: LP-WUS MO should be configured based on starting symbol based on symbol level offset from a reference. This offset could be independent or concatenated so that earlier LP-WUS MO starting symbol is a reference for the latter. Proposal: Define additional common offset for LP-WUS MO(s) of a LO to enable reducing the size of the LP-WUS MO symbol level offset. This offset could be defined in a slot level or ms level. Observation: To reduce the range needed for the PF-LO offset, making the applied value range dependent on the SSB periodicity could be considered, similarly as for the configuration of first paging PDCCH monitoring occasion is dependent on the applied sub-carrier spacing. Observation: Gain from repetitions for non-coherent receivers is very limited. Observation: Enabling codepoint determination via encoding can provide benefit and allow use of repetitions for incremental redundancy Proposal: Consider repetitions only if they can provide significant benefit for the detection performance e.g. via increased redundancy. Observation: It does not seem necessary to support more than two possible LO configurations/offsets per PO. Proposal: Support a maximum of two possible LO configurations/offsets per PO. Observation: To reduce the energy cost of repeated paging monitoring due to MR wake-ups, UE should be provided assistance information whether it should expect further paging in following paging occasion. |
| [8] NEC | Proposal 1: for RRC-IDLE/INACTIVE, the LP-WUS configuration indicates one or more LP-WUS frequency bands in a carrier, and for each LP-WUS frequency band, a periodicity and an offset of LOs are comprised in the configuration. Proposal 2: Support a MO can span multiple slots. Proposal 3: For the locations of MOs in an LO, support both periodic and non-periodic MO locations. For the periodic MO locations, a periodicity and an offset should be configured. For the non-periodic MO locations, an offset value for each MO or each MO group relative to the start time of the LO should be configured. |
| [9] CMCC | Proposal 1. For K (K>1) LP-WUS MOs configured for each beam in an LO, support Option A: K LP-WUS MOs for a beam are divided into M (M >=1) groups of R LP-WUS MOs. Proposal 2. Regarding Option A on LP-WUS MO design: - For detailed UE monitoring behaviour, support: o The UE monitors all M LP-WUS MO group(s) in an LO. o If the UE detects the LP-WUS that corresponds to its subgroup, it may not need to detect the remaining LP-WUS MO(s) in the LO. - For the exact value of R, support R>= 1. Proposal 3. Support a LP-SS occasion/LP-WUS MO can go across multiple slots and contiguous in time. - Also support that a LO/LP-SS burst can be contiguous in time. Proposal 4. RAN1 to discuss the UE behaviour when LP-SS/LP-WUS transmission overlap with existing pattern (e.g. UL slot/symbol, common signal transmission, configuration related to L1/L3 measurement), or when the time interval between LP-SS/LP-WUS transmission and existing pattern is smaller than the RF retuning time (e.g. when LR/MR is deployed in FR2, or when SCS of PRACH is equal or larger than 15kHz). |
| [10] Huawei/HiSi | Proposal 1: For LP-WUS payload size, consider the FAR and MDR requirements, where • For FAR, the requirement is <= 1% per DRX cycle, where the FAR is due to noise • For MDR, the requirement is <= 1% per detection trial Proposal 2: For mapping between LO and PO, Option 3 is not supported. • Note: Option 3 is that UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO. Proposal 3: For option 2 that UEs corresponding to different POs monitor the same LO, max number of POs per LO is 4. Proposal 4: One LO consists of N sets of K LP-WUS MOs, where, • every K consecutive MOs are mapped into one beam, • Within the K consecutive MOs, the same LP-WUS information can be mapped into multiple consecutive MOs for repetition. Proposal 5: When K (K>1) LP-WUS MOs are configured for each beam in an LO, support option A: K LP-WUS MOs for a beam are divided into M (M >=1) groups of R LP-WUS MOs. A UE monitors all or some of the MO(s) within the K LP-WUS MOs. Proposal 6: Support to configure the symbol-level offset to determine the MOs of the LO. Proposal 8: For MO definition, one MO can be across multiple consecutive slots • This means LP-WUS can be mapped to non-contiguous OFDM symbols, which can be realized by ‘available OFDM symbols’ method. Proposal 9: More than 4 MOs per LO are supported for option 2 (one LO mapped to multiple POs). Proposal 10: For LP-WUS in IDLE/inactive mode, to minimize the number of codepoints per MO for a UE to monitor, the codepoint corresponding to all the subgroups is only monitored in some MOs. • In the MOs above, the maximum number of codepoints for a UE to monitor is 1 or no larger than the number of codepoints monitored in the other MOs. Proposal 11: To reduce the signalling/resource overhead, a MO/LO is configured to be monitored by all the UEs to detect the codepoint corresponding to all the subgroups. Proposal 12: Further discuss how to map subgroup ID to codepoint value, especially for Option 2. Proposal 13: For each MO in the LO, support to configure explicitly an SSB index to associate with the MO. |
| [11] OPPO0 | Proposal 6: On the LO configuration for iDRX, if multiple-level offset values between a LO and a reference PO/PF are supported, similar definition as PEI could be considered, e.g., frame-level offset and symbol-level offset. |
| [13] Sony | Proposal 5 – Support time-frequency multiplexing of LP-WUS for different UEs to increase LP-WUS address/identification space. |
| [14] Samsung | Proposal 5: Support UE not monitoring the next X LOs after it detect a LP-WUS in one LO and the LP-WUS provides a wake-up indication. The number of skipped LOs X can be discussed further. Proposal 8: For the mapping relationship between LO and PO, a revised option 3 can be considered to reduce the resource overhead from network perspective, which is • From network perspective, UEs monitoring the multiple POs are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO. Besides, if the revised option 3 can be supported, to simplify the design, the number of LOs should be configured by gNB, and the maximum number of LOs associated with multiple POs should be limited to 2 to avoid excessive increase in paging latency. Proposal 9: To support the revised Option 3, the number of LOs can be determined by configuration, and the maximum number of LOs associated with multiple POs should be limited to 2 to avoid excessive increase in paging latency. Proposal 11: At least a slot-level offset should also be supported for the offset values between an LO and a reference PO/PF since LP-WUS in the MO can be transmitted in a slot-level. Proposal 15: If there is an UL transmission during LP-WUS monitoring, UE should at least suspend LP-WUS monitoring. • FFS: whether LP-WUS monitoring should be stopped or postponed or dropped can be further discussed. |
| [16] LG | Proposal #1: For UEs corresponding to different POs monitor the same LO, following options can be considered: - Option 1: a single offset is configured for a PF and the UE wake-up delay capability. - Option 2: a single offset is configured for one or more PF and the UE wake-up delay capability. Offset is applied based on the first PF Proposal #2: To support one-to-one mapping between LO and PO where multiple POs are configured in a PF, multiple offset can be configured for each PO, for a given PF(s) and UE wake-up capability. Proposal #3: To avoid the overlapping between adjacent two LOs, introduce RRC parameter to configure the duration of a LO. Proposal #4: For LP-WUS operation in TDD configurations, - If multiple LMO (for repetitions) are configured per beam, skip/drop the LP-WUS monitoring opportunity when it overlaps with UL symbols. - Otherwise, postpone the LP-WUS monitoring opportunity for N*K LP-WUS MO when it overlaps with UL symbols. Proposal #5: Consider these options for structuring the LP-WUS payload when multiple POs/PFs corresponds to a single LO: - Option 1: Use the same structure regardless of the number of PO/PF. - All UEs in indicated subgroups across all mapped POs need to wake up - Option 2: Different LP-WUS MOs within a LO correspond to different POs/PFs - Each MO within the LO maps to a specific single PO/PF Proposal #6: Introduce additional symbol or slot offset as well as frame offset for determination of the first LP-WUS monitoring occasion within a LO. Proposal #7: For remaining LP-WUS monitoring occasion within an LO, the sequential allocation methods shall be supported by one of following options: - Option 1: LMOs are allocated in consecutive OFDM symbols - Option 2: LMOs are allocated sequentially within slot boundaries - If an LMO would cross a slot boundary, it starts at the beginning of the next slot - Option 3: Introduce pre-defined or configured LP-WUS starting points within each slot Proposal #8: For sequential allocation of N*K LP-WUS MOs, the following ordering approaches can be considered: - Option 1: - first, in increasing order of R - second, in increasing order of G - third, in increasing order of M - fourth, in increasing order of N - Option 2: - first, in increasing order of R - second, in increasing order of G - third, in increasing order of N - fourth, in increasing order of M - Option 3: - first, in increasing order of R - second, in increasing order of N - third, in increasing order of G - fourth, in increasing order of M Proposal #9: The network shall determine the number of LP-WUS MOs for each beam in an LO using one of the following alternatives: - Alt. 1: Individual configuration of G, R, and M parameters - Network configures G, R, and M parameters - Total number of LMOs is determined as G * R * M - Alt. 2: Hybrid configuration with total LMO constraint - Network configures G and R parameters - Total number of LMOs K is also configured separately - M is derived from K/(G * R) - Alt. 3: Bounded configuration approach - Network configures G, R, and M parameters - Total number of LMOs K_max is determined by LO periodicity and LMO duration. Actual LMOs is bounded by K_max Proposal #10: For repetition and multiple information support, adopt one of the following approaches: - Alt. 1: Network ensures either R>1 with M=1, or M>1 with R=1 - Alt. 2: It is up to UE capability to support values for R and M parameters that can be simultaneously greater than 1 - Alt. 3: UE reports set of supported (R, M) value combinations Proposal #11: Either Option B or Option 3 shall be adopted for LP-WUS MO |
| [18] Panasonic | Proposal 3: Depending on the design of the LP-WUS information bits mapping to OOK chips/symbols after channel coding, if different number of OOK ON symbols in different OFDM symbols is allowed, to keep the OOK ON symbols in the same power level across OFDM symbols, by clarifying that the OOK ON symbols in the last OFDM symbol follow the EPRE configuration of the previous OFDM symbols. Proposal 4: Depending on the supported number of beams, MO groups and repetitions within an LO, an LO is highly possible to span multiple slots. It should be further studied and discussed on whether an LO should always be contiguous or can be non-contiguous, considering the collision/interruption with other channels/signals. Observation 2: Depending on the value of M, number of information bits carried by LP-WUS in one MO and how channel encoding, repetition and CRC is supported, the length of one MO can potentially be longer than one slot. Proposal 5: To settle the LP-WUS design details first, before discussing whether there is any impact if the length of one MO spans multiple slots. |
| [19] Apple | Proposal 5: One MO can span one or multiple slots. Repetition(s) of LP-WUS across multiple MOs are not supported. • One MO does not need to be always contiguous in time. FFS details.Proposal 6: gNB can configure a pattern of invalid symbols for LP-WUS, which is used by the UE to determine the time domain resource for the LP-WUS MOs. Proposal 7: For multi-beam support of LP-WUS monitoring, the LP WUS MO(s) are defined in a similar way as for paging occasions and PEI occasions. |
| [20] Sharp | Proposal 5: Consider the use of a slot-level offset to determine the start point for MOs in an LO. Proposal 6: The gap between offset values should be a multiple of paging cycles. |
| [22] DCM | Proposal 1: • Support FDM of multiple LOs  FFS: How UE determines frequency resource of LO • Do not support FDM of MOs within the same LO Proposal 2: • I-DRX cycle should be larger than wake-up delay values Proposal 3: • If trs-ResourceSetConfig is provided, the UE can receive TRS resource set(s) after wake-up indication Proposal 8: • The start of LO and the start of first MO should be same • Time gap between two consecutive MOs should not be supported Proposal 10:  Support only duty cycle monitoring for LP-WUS Proposal 14: • Before discussing the details on [LP-WUS/LP-SS_startRB_IDLE/INACTIVE] in higher layer parameters list, followings should be firstly discussed  Whether to support FDM of LOs Proposal 15: • The maximum number of [MONumPerLO] in higher layer parameters list should be 4 Proposal 16: • [subgroupNumber PO_LPWUS] should be discussed/determined in RAN1 Proposal 17:  The maximum number of [PO-to-LO association] in higher layer parameters list should be 4 if FDM of LOs is not supported Proposal 18:  For [LO_offset] in higher layer parameters list,  Up to 2 time offset values in frame level between a LO and a reference PO/PF can be configured  do not support symbol level offset values Proposal 19:  [Entry/Exit_condition_OOK/OFDM] should not be captured in RAN1 RRC parameter list |
| [23] Ericsson | Proposal 1 For LP-WUS operation in RRC-IDLE and INACTIVE, LP-WUS is used to trigger paging monitoring of the UE, i.e., monitoring of DCI format 1_0 with CRC scrambled by P-RNTI. Proposal 2 Paging misdetection performance of the UE should not be impacted when LP-WUS is used by the UE for power savings. Proposal 3 LP-WUS and LP-SS configuration is provided to the UE in a cell by system information. Further details of higher layer mechanisms can be discussed in RAN 2. Proposal 4 Type of WUS capability supported in the cell, e.g., support for both OFDM and OOK-based or OFDM-based WUS only, should be configurable by the NW. Proposal 5 When LP-SS is not configured by gNB, LP-WUS operation in the cell is supported only for UEs that support SSB based RRM measurement. Proposal 6 It should be possible for NW to configure flexibly the placement of LP-WUS and LP-SS resources in frequency and time to minimize overhead and NW energy efficiency impact. Proposal 7 LP-WUS and LP-SS frequency domain resource allocations can be separately configured within the initial DL BWP. Proposal 11 UE determines the LP-WUS MO positions it has to monitor in a LO based on a) Starting position of the LO (including any other offsets, if agreed, to determine the first LP-WUS MO of the LO) b) number of LP-WUS MOs configured and LP-WUS MO duration c) position of LP-WUS MO(s) corresponding to the SSB based on which the UE monitors paging PDCCH within the LO. Proposal 13 Parameter related to LP-WUS MOs such as the maximum number of MOs UE should monitor for LP-WUS can be configured as part of the WUR configuration. Proposal 14 Support only configuration of duty-cycled WUR monitoring in Rel-19. |
| [24] QC | Proposal 1: Specify symbol-level offset from the start of the frame determined by the frame level offset to the start of the first LP-WUS MO of the LO. This can follow firstPDCCH-MonitoringOccasionOfPEI-O for PEI. Proposal 3: For the UE to monitor PEI after it receives LP-WUS, the gap between the end of the last LP-WUS MO the UE monitors in the LO and the start of the PEI occasion is configured to be no less than the wake-up delay supported by the UE. Observation 1: There is generally no integer multiple relationship between the number of codepoints per LP-WUS and the number of UE subgroups per PO. Proposal 5: Support sequential mapping of the common codepoint and UE subgroup specific codepoints to codepoints provided by LP-WUS in MOs of the LO. Proposal 6: Support only configuring the common codepoint indicating all UE subgroups associated with the LO in the first LP-WUS MO of the LO. Proposal 7: Support configuring the common codepoint as the last codepoint provided by the LP-WUS in the LP-WUS MO. |
| [25] Nordic | Proposal-5: Consider (at least for overlaid design) one more codepoint/sub-group to be dedicated to early termination of monitoring for the UEs. |
| [26] Lenovo | Proposal 1: Discuss LPWUS sub-band blocks FDMed with multiple MOs and how the LPWUS subgroup IDs in MOs are mapped to the FDMed LPWUS sub-band blocks to reduce network power consumption. Proposal 2: Support UEs corresponding to different POs monitor the different LO, LOs are separated according to the wakeup delay or resource offset from PO. Proposal 3: UEs monitoring the same or different PO are divided into multiple sets of subgroup according to the wakeup delay or resource offset from PO, with UEs within each set of subgroups monitoring the same LO. Proposal 4: Support option 2A - A UE does not expect that the gap between the LO associated with the largest offset and the corresponding PO is less than the wake-up delay the UE supports. The UE monitors the LO associated with one offset that has a gap between the LO and the corresponding PO no less than the wake-up delay. Proposal 5: For gNB can configure UEs of different wake up delays to different LOs. Proposal 6: LO periodicity can be separated based on their wake-up delay offsets. • UEs with shorter wakeup delay monitor LO frequently • UEs with longer wakeup delay monitor LO less frequently |

Details on codepoint determination

| [3] vivo | Proposal 1: - UE identifies its subgroup specific codepoint value as follows: - Let iPO denotes the relative PO index, with starting value of 0, among the POs associated with a LP-WUS: iPO=UE_IDmodN⋅NS+i_smodNPOLP-WUS, where UE_ID, N, NS and i_s are defined in clause 7 of TS 38.304. - NPOLP-WUS denotes the configured number of PO associated with a LP-WUS and 1≤NPOLP-WUS≤4. - A UE determines its subgroup specific codepoint value as “iPO*NSGPO+iSG+1”, where - NSGPO denotes the configured number of LP-WUS subgroup per PO and 1≤NSGPO* NPOLP-WUS≤31; - iSG is the LP-WUS subgroup ID per PO for a UE and 0≤iSG<NSGPO. - The common codepoint value corresponding to all the subgroups needs to be checked by all UEs is “0”. - When a UE detects a codepoint value carried by a LP-WUS which is equal to “0” or “iPO*NSGPO+iSG+1”, the UE monitors its PO; otherwise, the UE is not required to monitor PO. |
| --- | --- |

### Option 2

We agreed to support Option 2 in RAN1#120:

Agreement

For the LO to PO mapping from network perspective, support Option 2 (UEs corresponding to different POs monitor the same LO).

- This should not increase the maximum number of codepoints per LO/LP-WUS compared to Option 1.

- FFS conditions/restrictions for mapping multiple POs to one LO

- Down-select between 2 and 4 for max number of POs per LO.

Max number of POs per LO for Option 2:

- 4

  - [1] Futurewei, [3] vivo, [4] TCL, [5] Spreadtrum, [6] CATT, [7] Nokia, [10] Huawei/HiSi, [11] OPPO, [17] ETRI, [18] Panasonic, [19] Apple, [20] Sharp, [22] DCM (if FDM of multiple LOs is not supported), [24] QC, [25] Nordic

  - {1, 2, 4}: [3] vivo, [4] TCL, [6] CATT, [19] Apple

  - [3] vivo: The POs associated with one LO should be consecutive POs from the network perspective. And the maximum number of PF associated with one LO is up to 2.

  - [4] TCL: The mapping of one LO to 1, 2, or 4 POs is determined by the number of POs configured within a PF.

  - [6] CATT: The maximum number of PF associated with a LO can be 1 for simplicity.

  - [19] Apple: The maximum number of subgroups configured per PO is (32/(number of POs per LO) – 1).

  - [20] Sharp: Proposal 1: A LO can be associated with POs in one PF, and the max number of associated POs per LO can be Ns.

  - [22] DCM: No conditions/restrictions for mapping multiple POs to one LO

  - [25] Nordic: POs are consecutive POs of the same paging frame

- 2

  - [12] InterDigital, [13] Sony, [21] MediaTek (open to consider 4), [23] Ericsson (if the number of configured UE subgroups per PO is less than 16)

- [14] Samsung: the max number of POs per LO can be limited to 2 if 5 information bits are supported. Otherwise, the max number of POs per LO can be limited to 4 if 6 information bits are supported.

  - The condition for mapping multiple POs to one LO can be when the number of subgroups of configured POs is less than or equal to 31 if 5 bits are supported or 63 if 6 bits are supported.

On the maximum number of POs per LO, majority companies support 4, but companies have different conditions/constraints in mind, e.g., the maximum # of subgroups per PO, whether all POs should correspond to the same PF, the maximum number of PFs for one LO.

[CLOSED] Proposal 2-1

The maximum number of POs per LO is 4, and the number of POs per LO can be 1, 2 or 4.

- Option 1: no additional constraint for mapping multiple POs to one LO.

- Option 2: The number of POs per LO is the same as the number of POs per PF Ns.

- Option 3: The maximum number of PFs associated with one LO is 1.

- Option 4: The maximum number of PFs associated with one LO is 2.

- Option 5: The maximum number of subgroups configured per PO is (32/(number of POs per LO) – 1).

- Note: these options are not necessarily mutually exclusive.

| Company | Comments |  |
| --- | --- | --- |
| ZTE, Sanechips | Option2, option3, option5 can be considered |  |
| Spreadtrum | We are fine with the proposal. |  |
| Xiaomi | Option 4 is already agreed in PEI, to restrict the paging latency. But since LP-WUS already cause much longer latency than then paging latency in PEI, so the restriction is not needed. We prefer Option 1 |  |
| Huawei, HiSilicon | Option 4 can be the starting point, which is the same as PEI design. |  |
| Nokia1 | For the number of POs, options 1,2,3,4 can be discussed. For option 5, should this be code points or, if it is sub-groups, should it be 31 (rather than 32)? |  |
| Sharp | We are fine with the proposal. |  |
| ETRI | Fine with the proposal. |  |
| LG | Fine with the proposal. |  |
| DCM | Support the proposal. On top of that, we support option 1. Since LO time duration would consume a lot of time resources, any constrain should not be supported to avoid decrease of resource effectivity. |  |
| Samsung | We have concern to support 4 if 5 bits are supported for the information bits carried by LP-WUS, because 31/4 is less than 8, which is small than the number of subgroups in one PO for PEI. There may have some impact for the power saving gains compared with PEI. |  |
| Moderator | It seems that we should separate out Option 5, as it is rather different from other options in nature. Proposal 2-1A The maximum number of POs per LO is 4, and the number of POs per LO can be 1, 2 or 4. - Alt 1: no additional constraint for mapping multiple POs to one LO. - Alt 2: The number of POs per LO is the same as the number of POs per PF Ns. - Alt 3: The maximum number of PFs associated with one LO is 1. - Alt 4: The maximum number of PFs associated with one LO is 2. Proposal 2-1B For Option 2, the maximum number of subgroups configured per PO is (32/(number of POs per LO) – 1). |  |
| vivo | As we commented for proposal 1-6, the limitation of option 2 that the number of POs per LO should always be equal to the number of POs per PF is not reasonable, especially considering that LO and PO 1 to 1 mapping is the basic one. For example, when there is 4 POs in a PF, by adopting option2, the UE power saving gain will be largely impacted since the number of subgroups per PO is limited to no more than 7. For option 3 or 4, they can be further considered. |  |
| Moderator | The following was agreed during online session: Proposal 2-1A The maximum number of POs per LO is 4, and the number of POs per LO can be 1, 2 or 4. The remaining parts will be further discussed. |  |
| FW | Given that the scaling of the number of POs associated with an LO corresponds to scaling of the number of subgroups supported per PO, then such a configuration might be more suitable for the low paging rate scenario and therefore with a maximum of 4 POs per LO, the LO can be associated with up to 4 PFs if each PF has 1 PO (inline with the low paging rate assumption). So, we prefer option 1. |  |
| Panasonic | We prefer Option/Alt 1 such that the associated number of the POs per LO can be free of constrains of how many POs are configured within a PF. |  |
| DCM | Support option 1. 4POs per 1LO is needed in any cases such as Ns (number of PO per PF)=1 for LO resource configuration effectivity. Since LO consumes a lot of time resources compared to PEI. This is a large issue especially in that PF dense is equal to I-DRX cycle (i.e. 1PF per 1Radio Frame). Thus, we should support Alt.1 and let gNB be able to configure 4POs per 1LO in any cases. |  |
| Moderator |  | The remaining issues are considered in P2-1B and P2-1C below. |

[CLOSED] Proposal 2-1B-r1

For Option 2, the maximum number of subgroups supported per PO is

- 7 for the case where 4 POs are mapped to one LO

- 15 for the case where 2 POs are mapped to one LO

| Company | Comments |
| --- | --- |
| Moderator | A modified version was agreed. Discussion closed. |

[CLOSED] Proposal 2-1C

Regarding whether there is any restriction on mapping multiple POs to one LO, down-select one from the following:

- Alt 1: no additional constraint for mapping multiple POs to one LO

- Alt 2: The number of POs per LO is the same as the number of POs per PF Ns.

- Alt 3: The maximum number of PFs associated with one LO is 1.

- Alt 4: The maximum number of PFs associated with one LO is 2.

| Company | Comments |
| --- | --- |
| Moderator | Alt 1 was agreed. Discussion closed. |

Codepoints for Option 2:

- One codepoint to wake up all the subgroups in the multiple POs

  - [2] ZTE, [3] vivo

- One codepoint for each PO to wake up all the subgroups in the PO

  - [6] CATT, [7] Nokia, [19] Apple

There is also a question of whether subgroups are indexed across POs or within a PO.

[CLOSED] Proposal 2-2

For Option 2, at least one codepoint corresponding to each of the subgroups in each PO is supported.

- For codepoint corresponding to more than one subgroups:

  - Option 1: One codepoint corresponding to all the subgroups in all POs

  - Option 2: One codepoint for each PO corresponding to all the subgroups in the PO

  - FFS other codepoints

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | The major benefit to support option2 is using one codepoint to indicate all the subgroups for multiple POs, which can save NW overhead. If still one codepoint corresponds to one PO subgroups, then there is no need to support option2, which is equivalent to one to one mapping. |
| CMCC | Support the proposal. |
| Spreadtrum | For codepoint corresponding to more than one subgroups, we prefer Option 1. |
| Xiaomi | OK with the proposal. And prefer Option 1. |
| Huawei, HiSilicon | We are OK with the main bullet, and we slightly prefer Option 1. During RAN1#120, the compromise was made to agree on Option 2, where the constraint was added that the max number of codepoints cannot be larger than 32. And the argument at that time is that when gNB configures Option 2, the number of UEs per PO is usually relatively small. Based on this understanding, Option 1 is good enough and will not introduce much false wakeup. If Option 2 in the sub-bullet (can we change it to alt 1/2? To avoid confusion with the main bullet) is considered, we suggest to also revisit the sub-bullet of previous agreement. |
| Nokia1 | My assumption was that if we map more POs to one LO, the number of MOs>1 and therefore Option 2 could be preferred. |
| Sharp | Support option1 |
| LG | Support the proposal. |
| DCM | Support the proposal. |
| Samsung | Support the proposal and prefer option 1 to save the number of bits for LP-WUS. |
| Moderator | To avoid the confusion mentioned in Huawei’s comment, it is updated as follows: Proposal 2-2r1 For Option 2, at least one codepoint corresponding to each of the subgroups in each PO is supported. - For codepoint corresponding to more than one subgroups: - Alt 1: One codepoint corresponding to all the subgroups in all POs - Alt 2: One codepoint for each PO corresponding to all the subgroups in the PO - FFS other codepoints |
| OPPO | For the subgroups within different POs, when UE receive a LP-WUS, it could be aware whether or not the LP-WUS is for its PO. For example, one LP is associated with 4 PO, each PO has 7 subgroups. If the length of codepoint is 3, e.g. 000~111, it needs to indicate the PO. We support 2 bits to indicate the PO, 3bits to indicate the codepoint in this case. If LP-WUS don’t support the PO indication, gNB should ensure the subgroups in different PO has different codepoint, i.e. the codepoint configured for different POs should be not overlapped except for the common codepoint. |
| CATT | Support Option 2. The equal number of subgroups should be configured for multiple POs. One codepoint for each PO corresponding to all the subgroups in the PO will not increase the NW moverhead decrease the FAR. For example, when the number of associated POs is 2, the maximum number of subgroups is 15, in which 30 codepoints corresponds to 30 subgroups and 2 codepoints for each PO corresponding to all the subgroups in the PO. This does not increase the LP-WUS payload. |
| FW | Given our previous comment that the association of multiple POs with one LO is mainly considered for the low paging rate scenarios, then it might be less efficient to dedicate more codepoint resources for the all-subgroup wake-ups and the use of those resources to support more subgroups per PO might be more beneficial. We suggest discussing this proposal along with the expected number of subgroups per PO as they are closely related. For example, if the restriction of an equal number of subgroups per PO is imposed, then Alt2 may be considered, otherwise, Alt1 may be considered. |
| Panasonic | We think Alt 2 is a reasonable approach to support, given the codepoint design is for better granularity to enable network to page the minimum and necessary subgroups for better UE power saving. If Alt 1 leaves more vacant codepoints to have more refined subgrouping for each POs, the design may be more complex as each PO may have different number of subgroups, although we technically agree it is good for power saving gain. But this should be discussed more explicitly whether this is the intention, as pointed out by FW. |
| Qualcomm | Support Alt 1 at least. Alt 2 can be considered additionally. |
| LG | Just for clarficiation 118bis: Agreement At least the following codepoints are supported for LP-WUS: - One codepoint corresponding to each of the subgroups that can be indicated by LP-WUS - One codepoint corresponding to all the subgroups that can be indicated by LP-WUS - FFS: Additional codepoints 119: Conclusion There is no RAN1 consensus to support additional codepoints indicating wake-up at least for 1:1 LO to PO mapping. Based on outcomes from previous meeting, Alt. 1 seems in the light of the agreement of 118bis, and Alt. 2 is an additional codepoint for 1 to multiple mapping. Is it correct understanding? |
| Moderator | Alt 2 was agreed. Discussion closed. |

Maximum number of MOs per beam for Option 2:

- 4: [3] vivo

- >4: [10] Huawei/HiSi

On the maximum number of MOs per beam for Option 2, very few companies provide the views. Given that we agreed that Option 2 “should not increase the maximum number of codepoints per LO/LP-WUS compared to Option 1”, there does not seem to have strong motivation to change the maximum number of MOs per beam compared to Option 1. Therefore, a tentative proposal is presented below for discussion.

[CLOSED] Proposal 2-3

For Option 2, the maximum value of M for Option A or Option B with G=1 (if either of them is supported) is 4.

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | 4 is enough, |
| CMCC | OK |
| Xiaomi | OK |
| Huawei, HiSilicon | Originally, we prefer >4 MOs. However, based on the discussion so far, and with the understanding that Option 2 is usually used for the case that number of UEs per PO is usually relatively small, 4 is also OK for us. |
| Nokia1 | OK with the proposal. |
| LG | Support |
| DCM | Support. |
| Samsung | OK. |
| OPPO | OK |
| vivo | Support and we follow the FL’s explanation also. |
| Moderator | Given that we agreed to support Option A, the proposal is updated to the following: Proposal 2-3r1 For Option 2, the maximum value of M for Option A is 4. |
| FW | OK |
| Moderator | Slight wording change for clarity: Proposal 2-3r2 For Option 2, the maximum value of M for in Option A for MO configuration is 4. |
| Moderator | A slightly modified version was agreed during online session. Discussion closed. |

[16] LG:

Proposal #5: Consider these options for structuring the LP-WUS payload when multiple POs/PFs corresponds to a single LO:

	Option 1: Use the same structure regardless of the number of PO/PF.

	All UEs in indicated subgroups across all mapped POs need to wake up

	Option 2: Different LP-WUS MOs within a LO correspond to different POs/PFs

	Each MO within the LO maps to a specific single PO/PF

Details on codepoint determination

| [3] vivo | Proposal 1: - UE identifies its subgroup specific codepoint value as follows: - Let iPO denotes the relative PO index, with starting value of 0, among the POs associated with a LP-WUS: iPO=UE_IDmodN⋅NS+i_smodNPOLP-WUS, where UE_ID, N, NS and i_s are defined in clause 7 of TS 38.304. - NPOLP-WUS denotes the configured number of PO associated with a LP-WUS and 1≤NPOLP-WUS≤4. - A UE determines its subgroup specific codepoint value as “iPO*NSGPO+iSG+1”, where - NSGPO denotes the configured number of LP-WUS subgroup per PO and 1≤NSGPO* NPOLP-WUS≤31; - iSG is the LP-WUS subgroup ID per PO for a UE and 0≤iSG<NSGPO. - The common codepoint value corresponding to all the subgroups needs to be checked by all UEs is “0”. - When a UE detects a codepoint value carried by a LP-WUS which is equal to “0” or “iPO*NSGPO+iSG+1”, the UE monitors its PO; otherwise, the UE is not required to monitor PO. |
| --- | --- |

### Wake-up delay

The following was agreed for the UE capability report on the wake-up delay:

Agreement

For UE capability report on the wake-up delay:

- Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[70ms], [500ms] and [900ms]}.

  - The reported values assume SSB periodicity of 20ms, where [70ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization.

  - FFS: translation of wake-up delay for different SSB periodicities

  - FFS: different sets of 3 candidate value for different SSB periodicities

- Send an LS to RAN4 after the down-selection to confirm the number of SSBs. Final LS in R1-2501624.

Collection of companies proposals on the FFS bullets:

| [10] Huawei/HiSi | Proposal 16: For UE capability report on the 3 wake-up delays, from RAN1 perspective, the number of SSBs need for synchronization are confirmed to be {3,5,5}, respectively. Proposal 17: 70ms is the minimum value of the wake-up delay assuming UE’s MR wakes up from deep sleep mode in IDLE mode, and for different SSB periodicities other than 20ms: • 70ms is kept for SSB periodicity of 5ms and 10ms; • The wake-up delay is translated for SSB periodicities larger than 20ms, i.e. wake-up delay = 10ms+3*SSB_periodicity |
| --- | --- |
| [15] xiaomi | Proposal 4: For the translation of wake-up delay for different SSB periodicities, the following Table 1 can be considered, Table 1 Mapping between reported wake-up delay and actual wake-up delay Reported wake-up delay; Actual wake-up delay. [70] ms; ms. [500] ms; ms. [900]ms; ms. Note: T is SSB periodicity.. |
| [16] LG | Proposal #12: For UE capability report on the wake-up delay, consider following options to derive sets of 3 candidate value for different SSB periodicities:  Option 1: Reporting alternative wake-up delay assuming larger SSB periodicities:  Support that UEs additionally reports wake-up delays assuming alternative SSB periodicities (e.g., 40ms, 80ms or 160ms). If a non-reported SSB periodicity is configured, use the reported value corresponding to the closest configured SSB period to derive actual wake-up delay.  Option 2: Table-Based Reporting:  This option maintains Alt. 1 structure by utilizing pre-defined table indices linked to SSB periodicity ranges. UEs report an index (Low/Mid/High) only and use the index with the current SSB periodicity to determine the wake-up delay from a predefined table. Proposal #13: UEs with different wake-up delay should be assigned to different UE subgroups Proposal #14: UEs having shorter wake-up delay can be assigned to a specific subgroup(s) separately  The number of those subgroups can be separately configured  Wake-up delays below a certain threshold are considered as shorter wake-up delay |
| [23] Ericsson | Proposal 9 For UE capability report on the wake-up delay, support different sets of 3 candidate values for different SSB periodicities. |

In terms of how to handle the different SSB periodicities, the proposal is to have a table that define values for each SSB periodicities for the 3 UE capabilities. This framework should be able to address the concerns from different companies. The values are filled in tentatively using the formula of ramp-up time + (3 or 5) * SSB periodicity.

[CLOSED] Proposal 3-1

For UE capability report on the wake-up delay, the UE reports one of the following 3 capabilities:

| SSB periodicity (ms) | Wake-up delay (ms) UE capability 1 | Wake-up delay (ms) UE capability 2 | Wake-up delay (ms) UE capability 3 |
| --- | --- | --- | --- |
| 5 | [25] | [425] | [825] |
| 10 | [40] | [450] | [850] |
| 20 | [70] | [500] | [900] |
| 40 | [130] | [600] | [1000] |
| 80 | [250] | [800] | [1200] |
| 160 | [490] | [1200] | [1600] |

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | There is no need to define wake-up delay based on SSB periodicity. For smaller periodicity, there is no much benefits for gNB to reduce the paging delay due to the ramp up time but increase the gNB complexity. For larger periodicity, in TS38.213, the UE would assume the SSB periodicity 20ms, and we do not think we need to spend much time to optimize the SSB periodicity larger than 20ms scenario. |
| CMCC | We can first discuss the values based on 20ms periodicity, and FFS whether other periodicity is needed to be considered. |
| Xiaomi | OK with the proposal. |
| Huawei, HiSilicon | We don’t support this table. First, as pointed in out tdoc, based on different UE implementation, the time needed for sync/re-sync may not change linearly along with the SSB periodicity. Therefore, for the cases with SSB periodicity < 20ms, 70ms is always needed for wakeup delay capability #1. Second, we agree with CMCC that we first agree on the values for 20ms SSB periodicity, and further discuss the scaling cases. |
| LG | Support the proposal. Regarding ZTE’s comment, we don’t think it is not related to gNB complexity. The proposal doesn’t mandate any gNB behavior. gNB always can have their assumption on wake-up delay. |
| Samsung | We can take 20 ms as an example and discuss the other part after RAN4’s response. |
| OPPO | OK |
| CATT | Currently, the periodicity of SSB is assumed as 20ms for initial BWP. Since the SSB periodicity of 20 ms is assumed by legacy UEs in the IDLE/INACTIVE states, different SSB periodicities would not support legacy UEs in IDLE/ INACTIVE states. There is no benefit on the implementation and complexity to consider SSB periodicities other than 20 ms |
| Qualcomm | We prefer to defer this discussion after RAN4 reply our LS. |
| Moderator | Proposal 3-1r1 On how to handle the UE capability report on the wake-up delay for SSB periodicities other than 20ms, - Alt 1: do not report - Alt 2: For UE capability report on the wake-up delay, the UE reports one of the following 3 capabilities: SSB periodicity (ms); Wake-up delay (ms) UE capability 1; Wake-up delay (ms) UE capability 2; Wake-up delay (ms) UE capability 3. 5; [x]; [425]; [825]. 10; [x]; [450]; [850]. 20; [70]; [500]; [900]. 40; [130]; [600]; [1000]. 80; [250]; [800]; [1200]. 160; [490]; [1200]; [1600]. |
| Moderator | 3 alternatives were captured in the chairman’s notes to be considered further. |

### QCL for multi-beam operation

| [7] Nokia | Proposal: Define that there is one-to-one association between LP-SS and LP-WUS MO time locations and SSB beams. This can be done as logical indexing. |
| --- | --- |
| [10] Huawei/HiSi | Proposal 13: For each MO in the LO, support to configure explicitly an SSB index to associate with the MO. |
| [12] InterDigital | Proposal 7: Each LP-WUS is QCLed with LP-SS in QCL Type A and QCL Type D (when applicable). |
| [13] Sony | Proposal 6 – RAN1 should decide on a mechanism for RRC-IDLE and RRC-INACTIVE mode UEs to know the SSB beam to which their LP-WUS will be QCL. |
| [14] Samsung | Proposal 4: The association relationship between LP-WUS/LP-SS beams and SSB beams can be determined by an explicit indication for simplicity. |
| [19] Apple | Proposal 9: The mapping between LP-WUS/LP-SS beams and SSB beams is determined with a 1-to-1 mapping sequentially in time in an implicit way. |
| [20] Sharp | Proposal 7: Consider two options for providing LP-WUS QCL information: - Implicitly, using the beam index within the LP-WUS monitoring occasion - Configured LP-WUS resources with TCI state for each beam. |
| [21] MediaTek | Proposal 7: Fix QCL-Type C and D for stable beam association, use implicit one-to-one LP-WUS/LP-SS to SSB mapping, and add RRC signaling if partial coverage is required. Observation 8: Enforcing one-to-one mapping avoids detection ambiguity, enhances beamforming gains, and ensures consistent beam coverage. Proposal 8: Mandate a strictly one-to-one beam mapping to preserve reliability, streamline beam management, and simplify UE/network implementations. |
| [24] QC | Proposal 11: LP-WUS and LP-SS are transmitted in all beams of the transmitted SSBs in a cell. |

Mapping between LP-WUS/LP-SS and SSB beams

On how the mapping between LP-WUS/LP-SS and SSB beams is determined, there are mainly two approaches, one is implicit mapping (in order mapping), the other is explicit configuration. In RAN1#120, there was a good support of using implicit mapping, but companies prefer to have a more concrete proposal in terms of how implicit mapping is done. Please check the proposals below, which follow the same principle as PO.

Proposal 4-1

For LP-SS, the LP-SS occasions are indexed sequentially in time, the k-th LP-SS occasion corresponds to the k-th transmitted SSB, k = 1, 2, …, N, where N is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1.

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | OK |
| CMCC | Support. |
| Spreadtrum | OK |
| Xiaomi | OK with the proposal. |
| Huawei, HiSilicon | Not support. This is related to the issue whether gNB can transmit LP-WUS/LP-SS using a subset of SSB beams. This issue actually includes two sub-issues: - Whether spec allows that gNB transmits LP-WUS/LP-SS using a subset of SSB beams - Our answer to this question is that it should be allowed. The benefit for this configuration is it enables gNB to control the neigouring cell interference. Especially for OOK based WUS, LP-SS has only 4 binary sequences, and OOK modulation performance is more impacted by interference. Therefore, gNB should have the specification tool to control that some of the directions do not have LP-SS/LP-WUS transmission. - Whether gNB can do it by gNB implementation without letting UE know that. - Our answer to this question is that this cannot be a pure gNB implementation. If ue cannot know which beam shall have LP-WUS/LP-SS transmission, the UE shall scan all SSB beams for any LP-WUS detection. It will kill much power saving benefit. Based on the above discussion, we suggest the following updates to this proposal. Proposal 4-1 For LP-SS, the LP-SS occasions are indexed sequentially in time, the k-th LP-SS occasion corresponds to the k-th transmitted SSB, k = 1, 2, …, N, where N is the number of actual transmitted related SSBs determined according to RRC configuration ssb-PositionsInBurst in SIB1. - FFS how to determine the related SSBs |
| LG | Support |
| Samsung | Explicit indication seems more flexible from gNB’s perspective to support multiple:1 LP-SS:MO mapping without any spec impact. |
| OPPO | Support |
| CATT | OK |
| Huawei, HiSilicon | As the moderator’s suggestion, the mapping between LP-WUS beams and SSB beams will be discussed together with LP-SS later, so I reply it here. From our point of view, allowing gNB to transmit LP-WUS/LP-SS via a subset of SSB beams is beneficial, in terms of reducing the resource overhead. In the following, two scenarios are provided: - The vertical beam scenario, which is shown in the following figure, where some beams are targeted to only cell center while other beams are targeted to only cell edge. Due to the limited coverage of LP-WUS/LP-SS, as well as the sensitivity to interference for OOK receiver, the LP-WUS/LP-SS may not be successfully received by the UEs at cell edge. In this case, transmitting LP-WUS/LP-SS via beams targeting to cell edge is wasting the resources. - The horizon beam scenario, which is shown in the following figure, where two cells are located near to each other, and in some direction (red arrow in the figure) there are strong interference, and in other directions (greed arrow in the figure) the interference is no that strong. Especially for OOK receiver, LP-WUS/LP-SS in the red arrow area may not be successfully received. In this case, gNB may choose not to transmit LP-WUS/LP-SS in some beam directions. To summarize, allowing gNB to transmit LP-WUS/LP-SS via a subset of SSB beams is a powerful tool to reduce the overhead. If the gNB is allowed to transmit LP-WUS/LP-SS via only a subset of SSB beams, the second issue is whether gNB needs to inform UE about this information. Our answer is YES for the following reasons: - If UE cannot know which beam shall have LP-WUS/LP-SS transmission, the most safe way for UE is to scan all SSB beams for any LP-WUS detection. It will kill much power saving benefit. - It will cause the ping-pong issue. Currently, the entry condition for LP-WUS monitoring is mandatorily based on SSB, and optionally based on LP-SS. If gNB only configure the SSB based threshold, UE may detect SSB#3 to be above the threshold, but after starting LP-WUS monitoring, UE may spend several LP-SS periodicities to find that LP-WUS/LP-SS is not transmitted via the beam of SSB#3. Then UE has to go back to MR again. The interruption may cause paging missing. After some offline discussion with companies, most of the companies think the above scenario is reasonable, while the following two comments are provided: - Company#1 thinks it is OK to let gNB inform UE the above information via SIB configuration. For example, gNB can transmit the LP-WUS related SIB via partial of the beams, and it can be based on the signalling design by RAN2. - HW&HiSi: our understanding is that UE may move across beams. For example, gNB transmit LP-WUS/LP-SS via beam#1 but not beam#2. Even gNB transmit LP-WUS related SIB only via beam#1, it may happen that UE receive the SIB when it is under beam#1, but later move to be under beam#2. Therefore, gNB needs to inform UE the whole picture (i.e., by which beams LP-WUS/LP-SS is transmitted) explicitly in SIB. - Company#2 does not want this solution impact the MO locations in a LO. For example, gNB transmits 8 SSBs via beam#1~#8, and transmits LP-WUS/LP-SS via beam#1, #2, #5, #6. In this case, there are two options for the MO locations. Option#1 is that in each LO there are still 8 MOs, but some of them are not used for LP-WUS/LP-SS transmission (i.e., reserved/muted). Option#2 is that in each LO there are only 4 MOs corresponding to beam#1, #2, #5, #6, respectively. The examples are shown in the following figure, and company#2 prefers Option#1. - HW&HiSi: both option#1 and option#2 can reduce the resource overhead, while option#2 can potentially leave a non-fragment resource for gNB. But we are option to these two options. Based on the above analyses and the comments received so far, we suggest the following proposal: Proposal It is supported that gNB transmits LP-WUS/LP-SS via a subset of SSB beams, and it is configured by gNB in the SIB for LP-WUS configuration on which SSB beam(s) are used for LP-WUS/LP-SS transmission. - The detailed signaling design is up to RAN2 - For the MO location determination for LP-WUS/LP-SS, the same LP-WUS/LP-SS is transmitted repeatedly in N MOs via N beams, respectively, where - Alt1: N is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1, and gNB can transmits LP-WUS/LP-SS in a subset of the N MOs according to the SIB for LP-WUS configuration. Alt2: N is the number of SSBs related to LP-WUS/LP-SS, which is determined by the SIB for LP-WUS configuration. |
| Huawei, HiSilicon | After further discussion with companies, the proposal is updated as below. Proposal It is supported that gNB transmits LP-WUS/LP-SS associated with SSB indexes, and it is configured by gNB in the SIB for LP-WUS configuration on whether LP-WUS/LP-SS transmission is present or not for each SSB index. - The detailed signaling design is up to RAN2 - For the MO location determination for LP-WUS/LP-SS, the same LP-WUS/LP-SS is transmitted repeatedly in N MOs via N beams, respectively, where - Alt1: N is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1, and gNB can transmits LP-WUS/LP-SS in a subset of the N MOs according to the SIB for LP-WUS configuration. - Alt2: N is the number of SSBs associated with LP-WUS/LP-SS, which is determined by the SIB for LP-WUS configuration. |

Proposal 4-1A

It is supported that gNB transmits LP-WUS/LP-SS associated with SSB indexes, and it is configured by gNB in the SIB for LP-WUS configuration on whether LP-WUS/LP-SS transmission is present or not for each SSB index.

- The detailed signaling design is up to RAN2

- For the MO location determination for LP-WUS/LP-SS, the same LP-WUS/LP-SS is transmitted repeatedly in N MOs via N beams, respectively, where

  - Alt1: N is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1, and gNB can transmits LP-WUS/LP-SS in a subset of the N MOs according to the SIB for LP-WUS configuration.

  - Alt2: N is the number of SSBs associated with LP-WUS/LP-SS, which is determined by the SIB for LP-WUS configuration.

| Company | Comments |
| --- | --- |
|  | Proposal 4-1A-r1 It is supported that gNB transmits LP-WUS/LP-SS associated with SSB indexes, and it is configured by gNB in the SIB for LP-WUS configuration on whether LP-WUS/LP-SS transmission is present or not for each SSB index. - The detailed signaling design is up to RAN2 - For the LP-WUS MO and LP-SS occasion determination, the same LP-WUS/LP-SS is transmitted repeatedly in N beams, respectively, where - Alt1: S is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1, and LP-WUS MOs and LP-SS occasions are determined assuming S beams. gNB transmits LP-WUS/LP-SS in a subset of the S beams according to the LP-WUS configuration in SIB. - Alt2: N is the number of SSBs associated with LP-WUS/LP-SS, which is determined by the SIB for LP-WUS configuration. LP-WUS MOs and LP-SS occasions are determined assuming N beams. |

[CLOSED] Proposal 4-2

For LP-WUS, the N * K LP-WUS MOs in an LO are indexed sequentially in time, from 1 to N*K, where N is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1, and K is the number of LP-WUS MOs for each beam.

- Option 1: The [k*N+n]th LP-WUS MO corresponds to the n-th transmitted SSB, where k=0,1,…,K-1, n=1,2,…,N. (beam first, multiple MOs second)

- Option 2: The [n*K+k]th LP-WUS MO corresponds to the n-th transmitted SSB, where k=0,1,…,K-1, n=1,2,…,N. (multiple MOs first, beam second)

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | Generally OK. Both options can work and slightly refer option2 . |
| CMCC | Ok |
| Spreadtrum | OK |
| Xiaomi | OK with the proposal. And prefer Option 2. |
| Huawei, HiSilicon | With the same comments as we provided for Proposal 4-1, we do not support this proposal. Instead, we suggest the following changes: Proposal 4-2 For LP-WUS, the N * K LP-WUS MOs in an LO are indexed sequentially in time, from 1 to N*K, where N is the number of actual transmitted related SSBs determined according to RRC configuration ssb-PositionsInBurst in SIB1, and K is the number of LP-WUS MOs for each beam. - Option 1: The [k*N+n]th LP-WUS MO corresponds to the n-th transmitted SSB, where k=0,1,…,K-1, n=1,2,…,N. (beam first, multiple MOs second) - Option 2: The [n*K+k]th LP-WUS MO corresponds to the n-th transmitted SSB, where k=0,1,…,K-1, n=1,2,…,N. (multiple MOs first, beam second) - FFS how to determine the related SSBs |
| Nokia1 | At least for FR1, option 2 could be considered. |
| LG | We prefer to discuss Proposal 4-2 after the decision on proposal 1-1 and 1-3. |
| Samsung | Prefer option 2 and agree with the intention proposed by Huawei. |
| OPPO | Prefer option2. |
| vivo | OK to discuss. Option2 is preferred to make MOs located more concentratedly for a certain beam. |
| CATT | OK with this proposal and we are open for this. |
| Moderator | Let us decouple of the two issues and discuss the ordering of the MOs first. We will discuss the number of beams later. Proposal 4-2r1 For LP-WUS, the N * K LP-WUS MOs in an LO are indexed sequentially in time, from 1 to N*K, where N is the number of beams corresponding to LP-WUS, and K is the number of LP-WUS MOs for each beam. - Alt 1: The [k*N+n]th LP-WUS MO corresponds to the n-th beam, where k=0,1,…,K-1, n=1,2,…,N. (beam first, multiple MOs second) - Alt 2: The [n*K+k]th LP-WUS MO corresponds to the n-th beam, where k=0,1,…,K-1, n=1,2,…,N. (multiple MOs first, beam second) |
| FW | OK with preference to Alt 1. |
| Moderator | Alt 2 was agreed during online session, with some update on the proposal. Discussion closed. The mapping between LP-WUS beams and SSB beams will be discussed together with LP-SS later. |

### Miscellaneous issues

EPRE ratio configuration for LP-WUS and LP-SS

The following was agreed in RAN1#120:

Agreement

The EPRE ratio between LP-WUS/LP-SS and SSB can be configured by the gNB.

- FFS whether there is a common configuration or separate configuration for LP-WUS and LP-SS

- Above is not applicable for the case when all the subcarriers for LP-WUS/LP-SS are transmitted with zero power (from baseband perspective)

- Above does not mandate any UE implementation for different receiver types

- FFS: whether/how to support for the case when there are different number of OOK ON symbols in different OFDM symbols (if supported)

Common or separate EPRE ratio configuration for LP-WUS and LP-SS

- Common: [2] ZTE, [3] vivo, [7] Nokia, [12] InterDigital, [14] Samsung, [19] Apple, [24] QC

  - [3] vivo: FFS: whether gNB is restricted to transmit the LP-WUS based on the configured EPRE ratio to SSB.

  - [24] QC: Do not support different number of OOK ON symbols in different OFDM symbols

- Separate: [15] xiaomi, [16] LG, [18] Panasonic, [21] MediaTek, [23] Ericsson (the ratio is limited to be within a certain range), [25] Nordic

  - [15] xiaomi: Proposal 3: The EPRE ratio between LP-SS/LP-WUS and SSB can be defined as

- [16] LG: Proposal #15: Introduce separated parameters of EPRE ratio to SSS between LP-WUS and LP-SS

  - For both of parameters, the value range is same as existing parameters: ENUMERATED{db-3, db0, db3, db6}

  - For the description of EPRE ratio of LP-WUS RE,

    - Power offset (dB) of LP-WUS RE to SSS RE.

  - For the description of EPRE ratio of LP-SS RE,

    - Power offset (dB) of LP-SS RE to SSS RE.

- [11] OPPO: Proposal 8: The design of LP-SS and LP-WUS would impact the configuration of EPRE ratio.

There is good support for both common and separate configuration.

Proposal 5-1

For the EPRE ratio between LP-WUS/LP-SS and SSB, down-select from the following:

- Option 1: Common configuration is provided for LP-WUS and LP-SS.

- Option 2: Separate configurations are provided for LP-WUS and LP-SS. The difference between the two EPRE ratios should be within [3] dB.

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | If we go with option2, the ratio could be within 6dB considering M=1 and M=4 differently. We are also OK with option1t to save some indication overhead in SIB |
| Spreadtrum | We prefer Option 1. |
| Xiaomi | Prefer Option 2, since LP-WUS is with balanced ‘0/1’ but LP-SS can be with unbalanced ‘0/1’ |
| Huawei, HiSilicon | We suggest that we first discuss the candidate values of the power offset, then discuss whether separate configuration is needed. |
| LG | Fine with the proposal in general. For now, we think the bracket need to cover whole sentence of not only the range but also “limitation to be within a certain range”. |
| DCM | Support the proposal. And support option 1. UE can do the AGC adjustment for LP-WUS based on the reception of LP-SS. Separate EPRE configuration would cause complexity for gNB and UEs. |
| OPPO | If LP-SS is with unbalanced ‘0/1’, we can support option 2. |
| vivo | Option 2 seems give more flexibility configuration on the transmission power of LP-WUS and LP-SS separately, then we prefer option 2, which can also avoid the LP-WUS transmission blocking issue (mentioned in our TDoc [3]) to some extent. |
| CATT | Prefer Option1. |
| Panasonic | We prefer Option 2 considering the M value for LP-WUS and LP-SS is not agreed yet to have any constrain. |
| Qualcomm | The bottom line is relationship between the actual EPREs for LP-SS and LP-WUS is fixed and known to the UE. Otherwise, the UE can not use LP-SS RSRP measurement to properly set the OOK detection threshold for LP-WUS. |

## LP-SS

For LP-SS configuration, a few RRC parameters have already been put on the consideration list in RAN1#120, including [LP-SS_Mvalue], [LP-WUS/LP-SS_startRB_IDLE/INACTIVE], LP-SS_Binary_Seq, LP-SS_periodicityoffset, LP-SS_overlaidSeq.

For LP-SS_periodicityoffset, we agreed to support a periodicity of 320ms, with 160ms, 80ms etc still FFS. For the offset, one question is whether we prefer ms-level offset or slot-level offset. The proposal below is to use slot-level offset, which seems to align with most of the existing RRC parameters.

Proposal 6-1

LP-SS_periodicityoffset provides a slot-level offset at least for SCS of 15kHz and 30kHz.

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | LP-WUS and LP-SS is better to be aligned, i.e., using same level offset. |
| CMCC | Okay |
| Xiaomi | OK |
| Huawei, HiSilicon | If slot-level offset is used for LP-SS, from which symbol the LP-SS will be transmitted? From the first OFDM symbol of a slot? If so, how to handle the case where the first OFDM symbol(s) is occupied by CORESET? Therefore, we tend to agree with ZTE that we first discuss the offset issue of LP-WUS, then use the same mechanism to LP-SS. |
| ETRI | Fine with the proposal. |
| Samsung | Agree with ZTE and HW’s intention. |
| OPPO | OK |
| vivo | OK |
| CATT | Similar views with ZTE and HW, LP-WUS and LP-SS is better to be aligned, i.e., using same level offset. |

To support multi-beam operation, multiple LP-SS occasions corresponding to multiple beams need to be configured. The starting point of each LP-SS occasion should be configured by gNB, which can be defined and configured w.r.t. the reference point provided by LP-SS_periodicityoffset.

Proposal 6-2

The starting time location of the first LP-SS occasion is configured w.r.t. the reference point provided by as LP-SS_periodicityoffset, using:

- Option 1: a slot-level offset

- Option 2: a symbol-level offset

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | Proposal 6-2 seems can be combined with proposal 6-1? |
| Xiaomi | Similar view as ZTE |
| Huawei, HiSilicon | Same question as ZTE. |
| Nokia1 | If the LP-SS design results only one LP-SS in a slot, and fixed starting symbol in a slot is defined the use of slot level offset (same as in 6-1?) could be considered. Otherwise option 2 is preferred. |
| OPPO | If the location of LP-SS within one slot is fixed, we support option 1. |
| vivo | Option 2 is preferred which is similar as the symbol-level offsets for a LO. |
| CATT | Similar view as ZTE. |
| Moderator | As suggested by ZTE, P6-1 and P6-2 are combined into a single proposal for discussion. The exact RRC parameter can be discussed later. Proposal 6-2r1 For LP-SS, a periodicity is configured. The starting time location of the first LP-SS occasion is configured using: - Option 1: a slot-level offset - Option 2: a symbol-level offset |

Proposal 6-3

For the starting time location of the subsequent LP-SS occasion(s),

- Option 1: implicitly derived based on the time location of the first LP-SS occasion

- Option 2: explicitly configured using a slot-level offset or a symbol-level offset

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | Option1 to save NW indication overhead in SIB |
| CMCC | Prefer Option 1 which is similar as current SSB structure |
| Xiaomi | Share similar view as CMCC |
| Huawei, HiSilicon | Similar like our comments to Proposal 6-1, we suggest to discuss LP-WUS first. |
| Nokia1 | I think this would depend bit on the LP-SS length etc. Having fixed pattern can have some drawbacks in TDD operation if LP-SS is long, thus option 2 would be preferred. |
| LG | Support the proposal and prefer Option 2. |
| OPPO | Prefer option1. |
| vivo | Option 1 is preferred. |
| CATT | Share similar view as CMCC. |
| Qualcomm | Option 1 |

[CLOSED] Proposal 6-4

Each LP-SS transmission occupies consecutive OFDM symbols.

| Company | Comments |
| --- | --- |
| Moderator | A modified version was agreed. |

LP-SS configuration related proposals:

| [3] vivo | Proposal 16: For LP-SS occasion, - gNB configures a number of nominal occasions for LP-SS, wherein the number is no smaller than number of beams for LP-SS/LP-WUS. UE assumes LP-SS transmission with beam sweeping in valid occasions within the configured nominal LP-SS occasions, wherein the valid occasion does not collide with UL symbols. - gNB configures periodicty and slot-level offset for the first LP-SS occasion, e.g., - slot80, INTEGER {0..79} - slot160, INTEGER {0..159} - slot320, INTEGER {0..319} - slot640, INTEGER {0..639} - gNB configures symbol-level offset for the first LP-SS occasion, or the symbol-level offset is pre-defined. - Remaining LP-SS occasions within a period is in consecutive slots with same symbol-level offset as the first LP-SS occasion. Proposal 18: It is up to network implementation to transmit smaller number of LP-SS beams compared to SSB beams. |
| --- | --- |
| [7] Nokia | Proposal: For LP-SS a reference time can be configured through (sub-frame or radio frame level) offset and periodicity. If only one periodicity is supported, it can be fixed in specification. Proposal: Support configuration of the start time for each LP-SS (corresponding to each SSB) at symbol level granularity. The reference time for the symbol level offset could be directly the LP-SS reference time. Proposal: Supporting configuration of starting PRB on common resource block grid for LP-SS and LP-WUS. |

## RRM measurement

LP-SS-based LP-RSRP/LP-RSRQ for OOK-based LR has been defined as follows:

Agreement

From RAN1 perspective, at least the following metrics can be supported for RRM serving cell measurement performed by OOK-based receiver based on LP-SS:

- LP-RSRP

  - LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols.

    - FFS: How to determine the received power of LP-SS in OOK ON symbols

- LP-RSRQ

  - LP-RSRQ = LP-RSRP/LP-RSSI

  - For the definition of LP-RSSI for determination of LP-RSRQ, further consider the following options:

    - Option 1: LP-RSSI is the linear average of total received power in all LP-SS OOK symbols.

    - Option 2: LP-RSSI is the linear average of total received power in LP-SS OOK OFF symbols.

    - Option 3: LP-RSSI is the linear average of total received power in LP-SS OOK ON symbols.

- FFS: LP-SINR, Power ratio of OOK-ON symbol and OOK-OFF symbol

Note: RAN1 will send an LS to RAN2 and RAN4 on the measurement metrics that can be supported from RAN1 perspective, to facilitate RAN2/RAN4 discussions. The exact metrics for OOK-based receiver to be used and defined in the specifications depend on the outcome of [RAN1]/RAN2/RAN4 discussions.

Agreement

The definitions of LP-RSRP and LP-RSSI for LP-RSRQ are updated as follows:

LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols over the frequency resources defined by the number of REs that carry LP-SS.

LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols over the frequency resources defined by the number of REs that carry LP-SS.

RAN4 made the following agreement for OFDM-based LR:

Agreement:

Define the requirements for OFDM-based LP-WUR serving cell measurement based only on LP-SS

- Reuse the same requirement for LP-SS OOK based

This means that LP-SS based measurement metrics need to be defined for OFDM-based LR as well.

Some companies proposed to reuse the same definition of OOK-based LR.

[CLOSED] Proposal 7-1

For OFDM-based LP-WUR, reuse the LP-SS based LP-RSRP/LP-RSRQ definition of OOK-based LP-WUR.

- [This is applicable only if LP-SS is configured to use one of the specified overlaid OFDM sequences.]

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | We need more details for OFDM WUR using LP-SS. For example, there are some issues needs to be clarified. - How to switching using SSB to using LP-SS for OFDM WUR? - Whether there are some RAN1 spec impacts or RAN2 spec impacts? |
| Xiaomi | Ok |
| Huawei, HiSilicon | Support. To answer ZTE’s question: - It can be UE implementation. gNB will configure the threshold(s) for both cases, and UE can determine to use one of them, since it is in IDLE mode. - Per our understanding, RAN1 just need to define the definition in 38.215. RAN2 impacts can be discussed in RAN2. |
| LG | Support |
| OPPO | OK |
| CATT | Ok. |
| Moderator | A modified version was agreed. Discussion closed. |

Another open issue that has been discussed in a few meetings is the FFS bullet highlighted in the following agreement:

Agreement

Confirm the following working assumption with the additional text in red:

Working Assumption

From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively.

- Above is applicable for both time-domain processing or frequency-domain processing

- Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications

Existing metrics SS-RSRP and SS-RSRQ are reused for OFDM-based LP-WUR. No separate metrics (LP-SSS-RSRP and LP-SSS-RSRQ) will be introduced in the specifications.

- The metrics SS-RSRP and SS-RSRQ are applicable for OFDM-based LP-WUR for RRC_IDLE and RRC_INACTIVE serving cell measurement.

- FFS: whether to remove or modify the restriction of using SMTC window for LP-SSS-RSRP/RSSI measurement.

This is a summary of companies’ views on this FFS bullet:

- [2] ZTE, [9] CMCC, [22] DCM: do not use SMTC window for OFDM-based and/or OOK-based LR or LP-SSS-RSRP/RSSI for serving cell measurement.

- [3] vivo: network configures the measurement time resource for LP-SSS-RSSI.

- [24] QC: RAN1 asks RAN4 whether to remove or modify the restriction of using SMTC window for LP-SSS-RSRP/RSSI measurement

[CLOSED] Proposal 7-2

For LP-SSS-RSRP/RSSI measurement performed by OFDM-based LP-WUR for the serving cell, SMTC window is not applicable.

- FFS: whether the measurement time resource can be configured by the network or not

Please also provide your views on the FFS bullet.

| Company | Comments |
| --- | --- |
| ZTE, Sanechips | OK with FL proposal. For FFS part, RAN2/RAN4 would define how to measure if needed. |
| CMCC | Support |
| Xiaomi | OK |
| Huawei, HiSilicon | We are OK for this proposal. As for the FFS part, we think it can be up to UE implementation. |
| DCM | Support. |
| Samsung | We would like to know the intention for the proposal because in our understanding, the SMTC window is used in RAN4 which is related with the requirement. |
| OPPO | OK |
| vivo | OK |
| CATT | OK |
| Moderator | A modified version was agreed. Discussion closed. LS to RAN4 will be prepared. |

This is a collection of companies’ proposals on the RRM measurement related issues:

| [2] ZTE | Proposal 1: • SMTC is not needed for OFDM WUR and OOK WUR for serving cell measurement • CSI or DMRS related measurement are not performed by OFDM based WUR. • Serving cell measurement for OFDM-based WUR and OOK-based WUR can be applied for RRC_IDLE intra-frequency and RRC_INACTIVE intra-frequency |
| --- | --- |
| [3] vivo | Proposal 17: The network configures the measurement time resource for LP-SSS-RSSI, which can be different from NR SMTC. |
| [7] Nokia | Proposal: Once LP-SS sequence design has been finalized, verify if the LP-RSRP and LP-RSRQ metric definitions need to be revisited. Proposal: LP-RSRP and LP-RSRQ measurements should be associated to a particular beam. Proposal: For LP-RSRP and LP-RSRQ measurements, it should be possible for theto network to configure a sub-set of LP-SS’s from which LP-RSRP and LP-RSRQ can be measured, similarly as done for SSBs. Observation: As the intent of the LR based measurements is serving cell evaluation, it would not seem necessary to restrict the measurements within SMTC. |
| [9] CMCC | Proposal 5. Do not introduce the SMTC window for LP-SSS-RSRP/RSSI measurement. |
| [10] Huawei/HiSi | Proposal 15: The RRM metrics for OOK-based receiver, including LP-RSRP and LP-RSRQ can be also applied to OFDM-based receiver. |
| [12] InterDigital | Proposal 8: LP-SS based RRM measurement is supported for OFDM based LR. Proposal 9: Introduce definitions of LP-RSRP and LP-RSRQ for OFDM based LR to support LP-SS based RRM measurement. Proposal 10: Introduce the following definitions of LP-RSRP and LP-RSRQ for OFDM based LR. • LP-RSRP is defined as the linear average over the power contributions (in [W]) of the resource elements that carry LP-SSs. • LP-RSRQ is defined as the ratio of N×LP-RSRP / LP-RSSI, where N is the number of resource blocks in the LP-RSSI measurement bandwidth. |
| [13] Sony | Proposal 8 – In order to further avoid ping-pong behaviours, a time window can be used in addition to power level hysteresis. |
| [15] xiaomi | Proposal 5: For OFDM-based LR, reuse the LP-SS based LP-RSRP/LP-RSRQ definition of OOK-based LR. |
| [19] Apple | Proposal 10: For OFDM-based LR, the LP-SS based LP-RSRP/LP-RSRQ are defined in the same way as for OOK-based LP-WUR. |
| [21] MediaTek | Proposal 5: Eliminate SMTC-based constraints, enabling standalone LP-SS timing that is RRC-configurable and optimally aligned with actual LP-SS transmissions for improved power saving and monitoring. Observation 6: Existing LP-SS-based measurements apply directly to OFDM-based WUR; improved processing offers potential accuracy gains without altering fundamental definitions. Proposal 6: Confirm unchanged measurement definitions for OFDM-based WUR; coordinate with RAN4 to address performance-validation criteria instead of redefining measurements. |
| [22] DCM | Proposal 13: • For RRM based on OFDM-based LP-WUR, SMTC window for measurement time resources(s) restriction is not applicable for LP-SSS-RSRP/RSSI measurement. |
| [23] Ericsson | Proposal 16 Entry/exit conditions should be configurable, e.g., as part of the LP-WUS configuration. Proposal 17 It should be possible to configure different entry/exit conditions based on supported WUR types, if any. Proposal 18 If serving cell RRM using LP-SS is supported, only scenarios that are suitable for operation with low LP-SS overhead and NW energy impact should be considered for RRM. |
| [24] QC | Proposal 9: Do not support different number of OOK ON symbols in different OFDM symbols. Proposal 10: RAN1 asks RAN4 whether to remove or modify the restriction of using SMTC window for LP-SSS-RSRP/RSSI measurement. |

## Other

Any topics that are being discussed in AI 9.6.1 will not be discussed here, including but not limited:

- Whether to have additional sync signal for LP-WUS

- Periodicity of LP-SS

- Overlaid OFDM sequence for LP-SS

- The relationship between the frequency resource allocation of LP-WUS/LP-SS and initial/active BWP/SSB

Topics that are deprioritized in this meeting:

- eDRX

- LR supporting less number of bands than MR

#### Question 8-1:

Please provide any other comments/suggestions you may have, e.g., the topics that you think are important to be discussed now.

| Company | Comment |
| --- | --- |
| FW | We would like to suggest the discussion of whether LOs for different wake-up delay capability can be aligned using offset configuration along with UE-specific IDRX configuration for UEs with shorter wake-up delay. This alignment can help reduce the resource overhead that will be associated with LO configuration when multiple wake-up delay capabilities are supported. [Moderator] I think this can be left to gNB implementation. |

# Guidance for RAN1#121 Discussions

RAN1#121 is the last meeting for the WI in RAN1, and it is critical that the remaining issues can be addressed to ensure the WI can be completed in time:

- Detailed MO configuration for LO is the major remaining issue. Companies are highly encouraged to provide all the details needed for a complete design. This can make our discussion much more efficient next meeting.

- LP-SS configuration

- LP-WUS/LP-SS beam association with SSBs

- Common or separate EPRE ratio for LP-WUS and LP-SS w.r.t. SSB

- UE capability report for wake-up delay for SSB periodicities other than 20ms

- Any other issues identified…

# References

- R1-2501711	Discussion on LP-WUS Operation in IDLE/INACTIVE Modes	FUTUREWEI

- R1-2501768	Discussion on LP-WUS operation in IDLE/INACTIVE mode	ZTE Corporation, Sanechips

- R1-2501815	Remaining issues on LP-WUS operation in IDLE/INACTIVE modes	vivo

- R1-2501846	Discussion on LP-WUS Operation in IDLE/INACTIVE state	TCL

- R1-2501876	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Spreadtrum, UNISOC

- R1-2502000	System design and procedure of LP-WUS operation for UE in IDLE/Inactive Modes	CATT

- R1-2502008	LP-WUS operation in IDLE/Inactive mode	Nokia

- R1-2502076	Discussion on LP-WUS operation in RRC IDLE/INACTIVE mode	NEC

- R1-2502168	Discussion on LP-WUS operation in IDLE/INACTIVE modes	CMCC

- R1-2502238	Procedures and functionalities of LP-WUS in IDLE/INACTIVE mode	Huawei, HiSilicon

- R1-2502258	Further consideration on LP-WUS operation in RRC_IDLE/INACTIVE modes	OPPO

- R1-2502303	Discussion on LP-WUS operation in IDLE/INACTIVE modes	InterDigital, Inc.

- R1-2502323	LP-WUS operation in IDLE / INACTIVE modes	Sony

- R1-2502376	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Samsung

- R1-2502449	Discussion on LP-WUS operation in Idle/Inactive modes	Xiaomi

- R1-2502482	Discussion on LP-WUS operation in IDLE/INACTIVE modes	LG Electronics

- R1-2502516	Discussion on LP-WUS operation in IDLE/INACTIVE modes	ETRI

- R1-2502581	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Panasonic

- R1-2502616	LP-WUS operation in IDLE/INACTIVE modes	Apple

- R1-2502676	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Sharp

- R1-2502712	LP-WUS operation in IDLE/INACTIVE modes	MediaTek Inc.

- R1-2502774	Discussion on LP-WUS operation in IDLE/INACTIVE modes	NTT DOCOMO, INC.

- R1-2502806	LP-WUS operation in IDLE and INACTIVE modes	Ericsson

- R1-2502847	LP-WUR operation in idle and inactive modes	Qualcomm Incorporated

- R1-2502879	On LP-WUS operation in IDLE/Inactive	Nordic Semiconductor ASA

- R1-2502907	Discussion on LP-WUS operation in Idle/Inactive modes	Lenovo

# Appendix A: Agreements from previous meetings

## RAN1#116 agreements

Agreement

Multi-beam operations are supported for LP-WUS and LP-SS for idle mode

Agreement

LP-WUS occasions (LOs) are defined for LP-WUS monitoring.

- Each LO has one or more LP-WUS monitoring occasions (MOs), where UE can monitors for LP-WUS transmission in each of the LP-WUS MOs.

  - Different LP-WUS MOs may correspond to different beams in multi-beam operation

  - It is not precluded that FFS whether or not each LO is defined as a time window that covers the corresponding LP-WUS MOs

  - FFS details

- It is at least supported that a UE monitors LOs with a configured periodicity.

- Each UE has a periodicity for LO monitoring, and it is at least supported that a UE monitors one LO per period.

  - FFS: A UE does not expect its LP-WUS monitoring occasions overlapping in time

  - FFS: monitoring of multiple more than one LOs per period e.g. if LP-WUS common to all UEs is supported or in case of eDRX (if supported)

- FFS eDRX, if supported

Agreement

For the case where a UE supports PEI and PEI is configured by the gNB, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not.

Agreement

It is supported that the UE monitors the legacy PO after receiving LP-WUS indicating wake-up.

- FFS: support of UE monitoring dynamic PO

Conclusion

For idle/inactive mode, how to map a UE to a subgroup ID for LP-WUS is left to RAN2 to decide.

## RAN1#116bis agreements

Agreement

For multi-beam operation of LP-WUS, UE assumes the same LP-WUS information payload is repeated in all transmitted beams corresponding to LP-WUS

- the selection of the beam(s) for the reception of the LP-WUS is up to UE implementation

Agreement

Each LO consists of N * K LP-WUS MOs, where N is the number of beams corresponding to LP-WUS, and K is the number of LP-WUS MOs for each beam.

- Option 1: K = 1

- Option 2: K can be larger than or equal to 1

  - FFS if more than 1 LP-WUS is transmitted from the same beam, whether the information in these multiple LP-WUS is always the same or can be different

Agreement

From RAN1 perspective, at least the following metrics can be supported for RRM serving cell measurement performed by OOK-based receiver based on LP-SS:

- LP-RSRP

  - LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols.

    - FFS: How to determine the received power of LP-SS in OOK ON symbols

- LP-RSRQ

  - LP-RSRQ = LP-RSRP/LP-RSSI

  - For the definition of LP-RSSI for determination of LP-RSRQ, further consider the following options:

    - Option 1: LP-RSSI is the linear average of total received power in all LP-SS OOK symbols.

    - Option 2: LP-RSSI is the linear average of total received power in LP-SS OOK OFF symbols.

    - Option 3: LP-RSSI is the linear average of total received power in LP-SS OOK ON symbols.

- FFS: LP-SINR, Power ratio of OOK-ON symbol and OOK-OFF symbol

Note: RAN1 will send an LS to RAN2 and RAN4 on the measurement metrics that can be supported from RAN1 perspective, to facilitate RAN2/RAN4 discussions. The exact metrics for OOK-based receiver to be used and defined in the specifications depend on the outcome of [RAN1]/RAN2/RAN4 discussions.

Working Assumption

From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode,

- The UE may start LP-WUS monitoring if

  - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB, and/or

  - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied

- If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up

- The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if

  - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB, and/or

  - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied

- FFS the serving cell measurement metrics

- The entry/exit thresholds can be configured separately for different types of LR

- It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB.

- Note: This may be revisited based on the RAN2/RAN4 discussion.

Conclusion

LP-SINR is not considered further as a metric for RRM serving cell measurement for OOK-based receiver.

## RAN1#117 agreements

Conclusion

Regarding the “FFS: How to determine the received power of LP-SS in OOK ON symbols” for LP-RSRP, no additional work in RAN1.

Agreement

It is supported that UEs monitoring the same PO are divided into multiple subgroups, where LP-WUS can provide wake-up indication for each subgroup. Consider the following options:

- Option 1: UEs monitoring the same PO monitor the same LO

- Option 2: UEs corresponding to different POs monitor the same LO

- Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO.

- Combinations of the above options can be considered.

Agreement

For LP-SS based LP-RSRQ, LP-RSRP and LP-RSSI are measured within the same bandwidth.

Agreement

For LP-RSSI definition for LP-RSRQ, the following is agreed

- Option 1: LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols.

Note: Above does not constrain LP-SS sequence design for OOK

Working Assumption

From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively.

- Above is applicable for both time-domain processing or frequency-domain processing

- Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications

Agreement

For idle/inactive mode, the maximum number of information bits (excluding CRC) in a LP-WUS is Z, where Z <= [8 or 16].

- FFS the exact value of Z

Agreement

For idle/inactive mode, the maximum number of subgroups per PO is X, where 8 <= X <= 256.

- FFS the exact value of X.

## RAN1#118 agreements

Agreement

The definitions of LP-RSRP and LP-RSSI for LP-RSRQ are updated as follows:

LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols over the frequency resources defined by the number of REs that carry LP-SS.

LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols over the frequency resources defined by the number of REs that carry LP-SS.

Agreement

At least support 1:1 association between LP-WUS MO(s)/LP-SS transmissions and SSB beams.

Agreement

For the wake-up delay, consider the following options:

Option 2: UE reports one value from X candidate values for the wake-up delay via UE capability reporting.

- FFS: X is 2, 3, 4

Above applies for IDLE/INACTIVE mode.

Definition of wake-up delay: Minimum gap time between LP-WUS reception and MR to start PDCCH monitoring

Agreement

For the maximum number of subgroups per PO (X), down-select from the following options in RAN1#118bis:

- Option 1: X = 8

- Option 2: X = 16

- Option 3: X = 32

- Option 4: X = 64

- Option 5: X = 128

- Option 6: X = 256

For decision in RAN1#118bis, companies are encouraged to provide the maximum number of information bits per LP-WUS (Z), the number of OFDM symbols occupied by LP-WUS per MO, the number of MOs for their preferred option.

Conclusion

There is no consensus in RAN1 on the support of dynamic PO.

Agreement

On the LO configuration for iDRX, offset value(s) between a LO and a reference PO/PF is/are configured.

- FFS: one or multiple offset values, and if multiple offset values are supported, how a UE decides which value to use

- FFS: the exact definition of the reference PF/PO and the detailed procedure for UE to determine the LO(s) corresponding to Option 1, 2, or 3 (if supported)

- (Working Assumption) For each UE, the periodicity of LO is the same as its iDRX cycle.

- If a UE receives a wake-up indication in a LP-WUS, consider the following alternatives

  - Alt 1: it monitors the PO associated with the offset.

  - Alt 2: it monitors the first PO after its reported wake-up delay.

  - Other alternatives are not precluded

Note: The PO mentioned above refers to legacy PO configured for the UE.

Agreement

Send an LS to RAN2 and RAN4 to convey the following

In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE. Request RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs.

Final LS in R1-2407559.

Conclusion

RAN1 will not initiate work on entry/exit conditions based on RRM measurement and RRM measurement offloading/relaxation conditions unless triggered by RAN2 and RAN4

## RAN1#118bis agreements

Agreement

Confirm the following working assumption for iDRX: For each UE, the periodicity of LO is the same as its iDRX cycle.

Agreement

For the mapping between LO and PO, supports at least Option 1 (UEs monitoring the same PO monitor the same LO).

Agreement

From RAN1 perspective, when a UE is monitoring LP-WUS (based on the entry/exit condition for LP-WUS), a UE is not required to monitor a PO if

- it does not detect a LP-WUS on the monitored LO

- or the LP-WUS does not indicate a wake-up indication for the UE’s corresponding subgroup

Agreement

For the offset value(s) between an LO and a reference PO/PF, consider the following options:

- Definition: The gap between an LO and a PO is considered to be no less than the wake-up delay a UE supports if the gap between the end of the last LP-WUS MO the UE monitors in the LO and the start of the PO is no less than the wake-up delay.

- Option 1: gNB configures a single offset value.

  - If the gap between an LO and the PO associated with the offset is no less than the wake-up delay a UE supports, the UE monitors the PO associated with the offset after receiving a wake-up indication in a LP-WUS.

  - Otherwise,

    - Option 1-1: the UE follows the legacy paging monitoring procedure.

    - Option 1-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay.

- Option 2: gNB configures one or multiple offset values.

  - For the same PO, each offset corresponds to a LO.

    - This does not preclude the possibility that the same LO may correspond to different POs with different offset values.

  - Option 2A: A UE does not expect that the gap between the LO associated with the largest offset and the corresponding PO is less than the wake-up delay the UE supports. The UE monitors the LO associated with one offset that has a gap between the LO and the corresponding PO no less than the wake-up delay.

    - This implies that the gNB needs to configure at least one offset value that is no less than the largest wake-up delay supported by the UEs.

    - FFS exactly how to choose the offset

  - Option 2B:

    - If the gap between the LO associated with the largest offset and the corresponding PO is no less than the wake-up delay a UE supports, the UE monitors the LO associated with one offset that has a gap between the LO and the PO associated with the offset no less than the wake-up delay.

      - FFS exactly how to choose the offset

    - Otherwise,

      - Option 2B-1: the UE follows the legacy paging monitoring procedure.

      - Option 2B-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay.

        - FFS exactly how to choose the offset

  - FFS the UE shall monitor the LO associated with additional offset(s)

Note: The PO mentioned above refers to legacy PO configured for the UE.

Agreement

At least the following codepoints are supported for LP-WUS:

- One codepoint corresponding to each of the subgroups that can be indicated by LP-WUS

- One codepoint corresponding to all the subgroups that can be indicated by LP-WUS

- FFS: Additional codepoints

Agreement

Support 3 candidate values for the wake-up delay capability a UE reports, two values for ultra-deep sleep state and one value for deep sleep state.

- FFS the values

- Note: Ultra-deep sleep state and deep sleep state are mentioned above solely for the purpose of RAN1 discussions and it is not intended to be captured in specifications from RAN1 perspective

Agreement

Each LP-WUS is QCLed with one SSB. Each LP-SS is QCLed with one SSB.

- FFS QCL Type A or Type C and/or Type D

- FFS implicit QCL determination or some signaling is required

Agreement

The number of beams for LP-SS is the same as the number of beams for the LP-WUS MOs in an LO.

Agreement

When K (K>1) LP-WUS MOs are configured for each beam in an LO, down select between

- Option A: K LP-WUS MOs for a beam are divided into M (M >=1) groups of R LP-WUS MOs. A UE monitors all or some of the MO(s) within the K LP-WUS MOs.

  - For each group of R LP-WUS MOs, the same LP-WUS information is transmitted.

    - FFS how the same LP-WUS information is transmitted in the R LP-WUS MOs

  - Different LP-WUS information can be transmitted in different groups of R LP-WUS MOs.

  - M = 1 and M > 1 is supported.

  - FFS: detailed UE monitoring behavior

  - FFS R=1 or R>= 1

- Option B: K LP-WUS MOs for a beam are divided into G (G >= 1) groups of R*M (M >= 1) LP-WUS MOs. A UE monitors all or some of the MO(s) within one group of R*M LP-WUS MOs based on its subgroup ID.

  - Each group of R*M LP-WUS MOs is further divided into M groups of R LP-WUS MOs.

    - For each group of R LP-WUS MOs, the same LP-WUS information is transmitted.

      - FFS how the same LP-WUS information is transmitted in the R LP-WUS MOs

    - Different LP-WUS information can be transmitted in different groups of R LP-WUS MOs.

    - FFS: detailed UE monitoring behavior

  - M = 1 and M > 1 is supported.

  - FFS R=1 or R>=1

  - Note: this achieves the same purpose as “Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO.”

Working Assumption

The maximum number of subgroups per PO supported in Rel-19 is 32.

## RAN1#119 agreements

Agreement

Confirm the following working assumption with the addition of the note at the bottom:

Working Assumption

From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode,

- The UE may start LP-WUS monitoring if

  - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB

  - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied

- If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up

- The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if

  - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB

  - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied

- FFS the serving cell measurement metrics

- The entry/exit thresholds can be configured separately for different types of LR

- It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB.

- Note: This may be revisited based on the RAN2/RAN4 discussion.

Note: this does not intend to impact any agreements and working assumptions made in RAN1/RAN2 after the working assumption.

Agreement

At least support the case that the number of beams for LP-WUS/LP-SS is the same as the number of SSB beams.

Conclusion

Do not support one-to-multiple mapping between the LP-WUS/LP-SS beams and the SSB beams.

Conclusion

No additional RAN1 specification impact specific to support multiple-to-one mapping between the LP-WUS/LP-SS beams and the SSB beams.

Agreement

For the offset value(s) between an LO and a reference PO/PF, down-select between

- Option 1-1

- Configurability between Option 1-1 and Option 1-2

- Option 2B-1

Agreement

At least for the 1:1 LO to PO mapping,

- The maximum value of M for Option A or Option B with G=1 (if either of them is supported) is 4.

Conclusion

There is no RAN1 consensus to support additional codepoints indicating wake-up at least for 1:1 LO to PO mapping.

For future meetings:

Consider the following alternatives for UE capability report on the wake-up delay:

- Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[80ms], [500ms] and [900ms]}.

  - The reported values assume SSB periodicity of 20ms, where [80ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization.

  - FFS: translation of wake-up delay for different SSB periodicities

- Alt 2: For the wake-up delay capability report, UE reports a pair of values for main radio ramp up and the number of SSBs needed for synchronization (D, NSSB). Support the following 3 candidate capabilities: {[(20ms, 3), (400ms, 5), (800ms, 5)]}.

- Alt 3: Modify the previous agreement on UE capability report on the wake-up delay to UE capability report on the main radio ramp up time. The number of SSBs needed for synchronization is defined in specs, and different number of SSBs can be defined for different UE capability on the main radio ramp up time.

  - 3 candidate values for the main radio ramp up time: {[20ms, 400ms, 800ms]}

  - The number of SSBs needed for synchronization is defined as [3, 5, 5] for [20ms, 400ms, 800ms] main radio ramp up time, respectively.

- Alt 4: Modify the previous agreement on UE capability report on the wake-up delay to UE capability report on the main radio ramp up time. The time needed for synchronization will be defined by RAN4 requirements later.

Agreement

Each LP-WUS or LP-SS is QCLed with an SSB with [select one from ‘typeA’, ‘typeC’], and when applicable, ‘typeD’ with the same SSB.

Working Assumption

If LP-WUS design support 32 subgroups within one MO, do not support Option 3 for LO to PO mapping or Option B for MO configuration.

Conclusion

From RAN1 perspective, there is no consensus on the following proposal:

UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if	the UE has not received LP-WUS for a long time.

For future meetings:

Companies are encouraged to consider section 3.5 of R1-2410862 for further discussions in future meetings.

## RAN1#120 agreements

Agreement

The EPRE ratio between LP-WUS/LP-SS and SSB can be configured by the gNB.

- FFS whether there is a common configuration or separate configuration for LP-WUS and LP-SS

- Above is not applicable for the case when all the subcarriers for LP-WUS/LP-SS are transmitted with zero power (from baseband perspective)

- Above does not mandate any UE implementation for different receiver types

- FFS: whether/how to support for the case when there are different number of OOK ON symbols in different OFDM symbols (if supported)

Agreement

For the offset value(s) between an LO and a reference PO/PF, at least a frame-level offset is provided.

- The reference point (reference PO/PF) for the frame-level offset is the start of the PF, or the first PF of the PF(s) (if mapping of POs from multiple PFs to one LO is supported), associated with the LO.

- FFS other offset value(s) to determine the MOs of the LO

Agreement

The previous agreement in RAN1#119 is updated as follows:

- Each LP-WUS or LP-SS is QCLed with an SSB with [select one from ‘typeA’, ‘typeC’], and when applicable, ‘typeD’ with the same SSB.

Agreement

Confirm the following working assumption with the additional text in red:

Working Assumption

From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively.

- Above is applicable for both time-domain processing or frequency-domain processing

- Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications

Existing metrics SS-RSRP and SS-RSRQ are reused for OFDM-based LP-WUR. No separate metrics (LP-SSS-RSRP and LP-SSS-RSRQ) will be introduced in the specifications.

- The metrics SS-RSRP and SS-RSRQ are applicable for OFDM-based LP-WUR for RRC_IDLE and RRC_INACTIVE serving cell measurement.

- FFS: whether to remove or modify the restriction of using SMTC window for LP-SSS-RSRP/RSSI measurement.

Agreement

For the LO to PO mapping from network perspective, support Option 2 (UEs corresponding to different POs monitor the same LO).

- This should not increase the maximum number of codepoints per LO/LP-WUS compared to Option 1.

- FFS conditions/restrictions for mapping multiple POs to one LO

- Down-select between 2 and 4 for max number of POs per LO.

Agreement

For UE capability report on the wake-up delay:

- Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[70ms], [500ms] and [900ms]}.

  - The reported values assume SSB periodicity of 20ms, where [70ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization.

  - FFS: translation of wake-up delay for different SSB periodicities

  - FFS: different sets of 3 candidate value for different SSB periodicities

- Send an LS to RAN4 after the down-selection to confirm the number of SSBs. Final LS in R1-2501624.

Agreement

For the offset value(s) between an LO and a reference PO/PF, adopt Option 2B-1.

- gNB can configure 1 or 2 offset values.

  - FFS whether gNB can configure 3 offset values

- If multiple offset values are configured and if the gap between the LO associated with the largest offset value and the corresponding PO is no less than the wake-up delay a UE reports, the UE monitors the LO associated with the smallest offset value that has a gap between the LO and the PO no less than the wake-up delay.

  - Note: if a single offset value is configured, UE behaviour is according to Option 1-1.

- All the UEs supporting LP-WUS for idle/inactive mode supports the configuration of 2 offset values (FFS: 3 values).

Agreement

Confirm the following working assumption with the modification:

Working Assumption

The maximum number of subgroups per PO supported in Rel-19 is 31 32.