3GPP TSG RAN WG1 #120bis                                                           R1- 2501751

Wuhan, China, April 7th – 11th, 2025

Source:	Kyocera

Title:	Specification Support for AI/ML for Beam Management

Agenda item:	9.1.1

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss AI/ML applications for Beam Management (BM) in the physical layer, focusing on the objectives outlined in the Rel-19 New Radio (NR) work item on Artificial Intelligence (AI)/Machine Learning (ML) [1]. During RAN#102, the Rel-19 work item titled ‘New Work Item Description (WID) on AI/ML for NR Air Interface’ received endorsement. The primary objective of this work item is as follows:

- Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE - NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

We explore various sub-use cases for an AI/ML-assisted beam management framework, along with an analysis of findings from the Rel-18 study item on AI/ML-based beam management. Additionally, we provide an initial assessment of the potential specification impact required to enable these sub-use cases for 5G NR Rel-19.

# Configuration of Set A/B for NW sided and UE sided AI/ML models

In a NW-sided AI/ML model for BM-Case1 and BM-Case2, the gNB may or may not need to configure Set A for the UE. When Set B is different from Set A, no configuration for Set A is needed. This is because the inference is performed at the gNB side and beam prediction occurs transparently to the UE. The gNB’s responsibilities may include configuring reporting parameters, such as the number of beams to be reported.

When Set B is a subset of or equal to Set A, configuring Set A might only suffice. This is because Set B beams can be interpreted from Set A. In the former case, when Set B is different from Set A, if only Set A is configured, the gNB will need to inform the UE about which beams from Set A is the subset (i.e., Set B) so that the UE does the L1 measurements. However, in the later case, when Set B is the same as Set A, it is sufficient to only configure Set A.

This enables the UE to determine the resources needed to capture and provide the required L1 measurements essential for the NW’s AI/ML model’s operation. The L1 related measurements are reported to the gNB on UL-related data or control channels, which in turns ensures that the UE can effectively gather the necessary data inputs for the NW-sided AI/ML model.

- For a NW-sided AI/ML model for BM-Case1 and BM-Case2, for the configuration of inference results, RAN1 should consider the following options:

- When Set B is different from Set A, no configuration for Set A is required for the UE, as the inference process is conducted transparently from the UE.

- When Set B is a subset of Set A, it might be sufficient for the gNB to only configure Set A.

  - FFS: Determination of Set B out of Set A.

- When Set B is equal to Set A, there is no need to configure Set A and it is sufficient to only configure Set A.

- The gNB should configure reporting-related parameters, including the number of beams that the UE should report.

A similar discussion as above, when Set B is different from Set A, Set B needs to be explicitly configured for the UE to perform measurements and derive its AI/ML model. However, Set A does not need to be configured for measurements. It can optionally be virtually configured for the UE to provide a reference for mapping its AI/ML model output, in the case of classification models, to the corresponding CRIs that the gNB can interpret.

When Set B is a subset of or equal to Set A, it might not be necessary to configure Set B and only Set A configuration is sufficient. In the former case, the gNB should inform the UE which subset beams from Set A are used to represent Set B at the current inference instance. However, in the latter case, Set A beams is fully used to represent Set B and no further gNB-UE signalling is required for the determination of Set B.

Based upon that, we have the following proposal:

- For a UE-side AI/ML model, support configuring Sets A and B for inference results reporting, where:

  - When Set B is different from Set A,

    - Set B is explicitly configured for measurements for the UE. It enables the UE to perform measurements that derive the UE’s AI/ML model.

    - Set A can be optionally virtually configured. It is not intended to provide measurements to the UE. Instead, it serves as a reference to map the UE-side AI/ML model output, in the case of classification models, to the corresponding CRIs that the gNB can interpret.

  - When Set B is a subset of or equal to Set A, it might be sufficient for the gNB to only configure Set A.

    - FFS: Determination of Set B out of Set A.

  - When Set B is equal to Set A, there is no need to configure Set B and it is sufficient to only configure Set A.

# Measurement report for NW sided AI/ML model

In RAN1#117 meeting, the following agreement was made regarding the inference report configuration for a NW side model:

Agreement For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signalling, support - L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB - If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set - FFS: L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, X and M are configured by gNB, and whether/how to report number of reported beams - FFS on the maximum value of M (where M can be larger than 4) based on UE capability (M may or may not be different for different reporting contents) - FFS on beam information Note: Purpose, such as above “For NW-sided model, for inference report, at least for BM-Case 1”, will not be specified in RAN 1 specification

It is reasonable to assume that the NW-side AI/ML model is designed to accommodate full Set B measurements as its input. Therefore, if not all L1-RSRP measurements are reported to the NW as part of the overhead reduction, it is necessary to determine the indices of the reported beams to properly map them to the corresponding pins of the AI/ML model input. Consequently, in the first bullet, if the configured value (M) is smaller than the size of the measurement resource set, it makes sense to report the corresponding SSBRI/CRI of the selected beams that are a subset of Set B. However, regarding the associated sub-bullet, if (M) is equal to the size of the measurement resource set, i.e., all of Set B is configured for reporting, then there may be no need to report the beam-related information. Therefore, we have the following observation:

- For a NW side AI/ML model, for BM Case1 and Case 2, regarding the reporting of Set B measurements for inference purpose, when (M) is equal to the size of the measurement resource set, and Set B is defined solely by the resources of this resource set, it may not be necessary to report the beam index of the largest L1-RSRP value. The measured absolute/differential L1-RSRPs alone may be sufficient for the AI/ML model to operate effectively.

## Beam information

We support using the CRI/SSBRI as the starting point for the beam information. Moreover, we support using a bit map to indicate the measurement resource set. This is because, in legacy L1-RSRP reporting, the maximum number of beam measurements (M) in an L1-RSRP report is limited to 4. However, for NW side AI/ML model inference reporting, the UE may be required to include the measurement report for Set B within a single report, where the number of measurements (M) is likely to exceed 4. In this scenario, the use of a bitmap indication can effectively reduce overhead associated with beam information transmission without causing any loss of information. In addition, we don’t see any benefit from indicating the index of the beam with the largest measured L1-RSRP along with the bit map.

## L1-RSRP omission

In the second bullet of the above agreement from RAN1#117 meeting, the main purpose of enclosed FFS is to introduce variable number of beams reporting when the NW side AI/ML is collecting its model input. In our view, we don’t see the direct benefit of reporting a variable number of beams, i.e., beams that are within X-dB from the largest measured L1-RSRP value. This is for the following reasons:

- If the adaptive number of beams is supported, the number of beams needs to be reported, where, in each inference instance, a variable number of beam indices reporting needs to be supported when CSI/SSBRI reporting is adopted.

- The report configuration for beam measurement report in case of variable number of beams is unclear for us. A potential method is to use CSI partitioning, i.e., CSI part 1 and part 2.

- In some instances, the number of reported beams might not be sufficient to execute the model inference, e.g., no beam is within X-dB of the largest L1-RSRP measured beam.

Based on the discussion above, we propose the following modified version RAN1#117 agreement concerning the configuration of the inference report for the network-side model.

- For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signalling, support

  - L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB

  - If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set.

  - The Following options can be considered for beam information:

    - Option 1: CRI/SSBRI of a measurement resource set

    - Option 2: A bit map to indicate the reported beams of a measurement resource set

# Report for Inference

In this section, we discuss various aspects of the inference results reporting for a NW and UE-sided AI/ML model. This includes considerations for reporting quantities, mechanisms and potential overhead reduction techniques.

## NW Side Model

### The number of reported beams, i.e., the value of M

In BM-Case 2, Set B could be the same as Set A; therefore, the size of Set A should be used as a starting point for the maximum number of reported beams (256 beams). In prior TDocs, companies discussed that with the limit of polar codes (1706 bits), and assuming differential L1-RSRP reporting of the beams of Set B with 4 bits, the maximum number of supported beams can be up to 1706/4 = 426 beams.

- For a NW-side AI/ML model, extend maxNrofNZP-CSI-RS-ResourcesPerSet (i.e., Maximum number of NZP CSI-RS resources per resource set) to 256.

## UE Sided AI/ML Models

For a UE side AI/ML model the following agreement, in RAN1#116 meeting, about the content of the model inference, is made:

Agreement For UE-sided model, at least for BM-Case1, for content in the report of inference results, support - Opt 1: Beam information on predicted Top K beam(s) among a set of beams - Opt 2: Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams - At least K=1 and more, FFS on max value - FFS on beam information - FFS on the definition of predicted Top K beam(s) - FFS on definition of reported RSRP when applicable - FFS on other information in the report with potential down selection among the following options - Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams - FFS on the quantization method of probability information - Probability information is the probability of the beam to be the Top 1 or Top K beam - Opt 4: Beam information on predicted Top K beam(s) among a set of beams, RSRP of predicted Top K beam(s) among a set of beams, and confidence information of the RSRP - FFS on definition of reported RSRP - FFS on the definition and quantization method of confidence information - Other options are not precluded. where the set of beams is Set A, i.e., the beams for UE prediction.

The agreement above provides different options for the UE-side AI/ML model inference output. Options 3 and 4 offer additional probability and confidence information about the Top-K selected beams and predicted RSRP from the AI/ML model. This information could be beneficial for gNB scheduling decisions, as it could allow the gNB to configure the UE with the appropriate value of K measurements and even revert to legacy configurations when necessary. However, a trade-off may exist between the potential benefits of this information in enhancing system performance (after accounting for quantization loss) and the additional overhead incurred due to the reporting of extra probability/confidence information, as well as the increased computational complexity from quantizing this information.

- For the UE-side AI/ML model, regarding the content of the report inference results, support using options 1 through 3. Further study the benefits and gains of adopting option 4, considering the effects of quantization methods on system throughput and considering the additional overhead incurred.

### Definition of the confidence information

Based on our understanding, the confidence information is the statistical confidence interval. A confidence interval represents the range within which an estimate is expected to fall a certain percentage of the time if the experiment is repeated, or the population is resampled in the same manner. By adopting this definition, the confidence information reported by the UE in Option #4 could include a probability value and a range, defined by minimum and maximum values, for each beam in Set A. At each inference instance, the probability value indicates the confidence level of the predicted RSRP by the AI/ML model for the corresponding beam. If a beam in Set A has a narrow range and a high probability, and the predicted RSRP for that beam falls within this narrow range, it implies that the UE is highly confident that this predicted RSRP is accurate, i.e., it is the value that would be measured if this beam were transmitted by the gNB.

Given the definition of confidence information, an additional overhead is incurred as the UE must report a probability value and an interval for each beam. However, if the UE is confident in its reported RSRP value, this can assist the gNB in configuring the performance monitoring set, where RS are transmitted to provide the UE with the ground truth for assessing its AI/ML model performance. This is particularly beneficial when the performance monitoring set size needs to be smaller than Set A, especially in cases where Set A is large. Configuring a performance monitoring set of the same size as a large Set A may not be the most efficient use of DL resources and could also impose a computational burden on the UE by requiring measurements on the entire large set.

Based on the above discussion, we have the following proposals:

- Regarding the definition of the confidence information, for option 4 of a UE side AI/ML model report content for inference, consider the following definition:

  - The confidence information represents a range within which the predicted RSRP of a beam in Set A is expected to fall a certain percentage of the time if the same input sample is reused to derive the AI/ML model.

  - FFS on the details of the calculation of the confidence interval

- For a UE-side AI/ML model report content for inference, further study the applicability of adopting Option 4, including its pros and cons, in terms of the associated UL overhead and its potential to provide the gNB with information that could assist in configuring the performance monitoring set.

### Bit width of the reported probability information

To optimize the quantization of probability information generated by the AI/ML model and reduce uplink overhead, it is proposed that the UE rounds the probability values to one decimal place. This approach would require a maximum of 4 bits to represent the probability information per beam. Therefore, we present the following proposal:

- For a UE side AI/ML model probability information reporting, support one decimal precision probabilities where only 4 bits (bit width) are required to represent the probability information per beam. The choice of rounding up/down the probability values is left up to the UE’s implementation.

### Ranking of the reported Top-K beams

Upon inference, the UE is expected to report the top-K beams based on its AI/ML model predictions. During this process, the UE must inform the gNB about the ranking of the beams within the Top-K set. The method of reporting this ranking depends on how the UE conveys the Top-K beams. If the UE reports the Top-K beams using a CRI vector, it should order the CRIs in descending order. This means the UE will provide a K-dimensional vector of CRIs, where the ith entry corresponds to the ith strongest beam.

Based on the above discussion, we have the following proposal:

- For a UE-side AI/ML model inference report on beam ranking, where the UE is required to report the Top-K beams out of N beams in Set A, consider:

  - CRI-based Reporting: The UE should report the beam information in a ranked manner using a CRI vector. This involves reporting a K-dimensional vector of CRIs, where the ith entry corresponds to the ith strongest beam. The ithstrongest beam can be determined by the beam information.

# Consistency and additional conditions

## UE Sided AI/ML Models

The following two proposals were introduced by FL for discussion:

##### For #120 Proposal G If the associated ID is not configured for data collection/inference, UE may assume the similar properties of a DL Tx beam or beam set/list associated [at least] with the same cell.

##### Proposal 1.4.3 …… - Case 2-1: The associated ID is not configured for inference/step 3. - Alt 1: UE may report it is not applicable - Alt 2: UE may report it is applicable, and UE assume the similar properties of a DL Tx beam or beam set/list for Set A/Set B associated with in the same cell. - Alt 3: UE may report it is applicable, but UE cannot assume anything.

Once the UE starts a data collection session for its model, this session can optionally be assigned an associated ID, which aims to capture the collected data properties and the underlying assumptions beyond it. Since the main purpose of the associated ID is to provide consistency between training and inference, it makes sense to say that each associated ID should have a corresponding trained model for it. In the inference stage, a model should be selected based on the configured associated ID. This in turns helps the UE to select the proper model based on the properties of the collected data.

If the associated ID is configured during training/data collection and inference, the UE can then select the proper trained model based on the configured associated ID. This is equivalent to assuming similar DL Tx beam properties for the same associated ID.

If the associated ID is configured during training/data collection but not configured for inference, the UE may then have multiple different models in its arsenal to use. However, it does not have a way to select any of these models.The UE may optionally report the applicability of the associated ID to the gNB as the gNB might have already categorized its data but is not utilizing this categorization information during inference. Additionally, depending on the UE’s capability, this report could inform the gNB of any untrained AI/ML models on the UE side that could be trained using the gNB's data. It is be left up to implementation the way the UE selects the AI/ML model for inference.

If the associated ID is not configured during training, the UE then will have no way to categorize/split the data and train different models correspondingly. In our view, all what the UE can do is to use the whole data to train one AI/ML model. In other words, the UE assumes the same data distribution for all samples, i.e., similar beam properties.

In the inference stage, if the associated ID is not configured during training, the UE has only one trained AI/ML model that it can use. This is also equivalent to assuming similar properties for the DL transmit beam. Depending on the UE’s capability, the UE may report that the associated ID is applicable. This is in order to let the gNB know that the UE can support more than one AI/ML model and that it could be open for a data collection phase to train any of its available blank/untrained models.

Based on the above discussion, we have the following modified version of the proposals:

- Further study whether to support the following cases for configuration of the associated ID:

- Case 1: The associated ID is configured for data collection.

  - Case 1-0: The associated ID is configured for inference/step 3

    - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID.

  - Case 1-1: The associated ID is not configured for inference/step 3.

    - UE may report the associated ID applicability.

    - AI/ML model selection is left up to the UE’s implementation.

- Case 2: The associated ID is not configured for data collection. Either it is configured or not for inference:

  - Case 2-0: The UE cannot assume anything and only uses its trained AI/ML model.

  - Case 2-1: Depending on the UE’s capability, the UE may report that the associated ID is applicable or not.

## NW Sided AI/ML Models

### How to ensure consistency for NW side AI/ML model

Similar to UE-side AI/ML models, UEs can operate differently between training and inference stages. For a NW-side AI/ML model, the NW relies on the UE to collect the input measurements required for its model. Therefore, it is crucial that UEs behave consistently during training and inference to ensure that the input samples are collected uniformly for the ML model. As we understand, the receive beam at the UE side could potentially impact the collected measurements of Set B. During the Rel 18 evaluations, it was shown that if the received beam changes randomly, the prediction accuracy could degrade severely [1].

One way to mitigate this issue is to provide the NW with information about the receive beam, encapsulated in a beam index without revealing any proprietary information about the beam. To further reduce the overhead associated with reporting the beam index from the UE, the current receive beam can be assumed as the receive beam assumption. When the gNB configures the CSI-RS resource set for channel measurements, it can also define the QCL relationship of the resources included in the configured set. Consequently, the UE can use the most suitable receive beam it has identified for the QCL source for measuring all CSI-RS beams in the configured resource set.

- For a NW-side AI/ML model, to ensure consistency between training and inference, study the feasibility of the following mechanisms:

  - The UE reports the receive beam index to the NW.

  - To eliminate the UL reporting overhead, the best receive beam is assumed by the NW. The NW configures the QCL relationship, and the UE uses the receive beam identified for the QCL source for performing the measurements.

# Performance Monitoring

## UE Sided AI/ML Models

For a UE side AI/ML model, the following agreement was made regarding performance monitoring:

Agreement: For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 FFS Type 2 performance monitoring

### Type 1 option 2 monitoring

In option 1, the UE frequently provides the gNB with inputs to assess the performance of the AI/ML model. While frequent updates of ML model outputs and resource measurements offer the NW flexibility to properly schedule and optimize transmission, this approach may lead to significant UL overhead. Hence, option 2 grants the UE more authority to utilize information from both the ML model and performance resource set measurements to calculate the performance metric/KPI. The UE then reports this metric to the gNB, which takes the monitoring decision, such as model switching or fallback to legacy. Option 2 reduces UL overhead, as the UE reports fewer parameters than in option 1. However, the NW/gNB receives less information about the performance of the UE-side model until the metrics are reported. In option 2, the calculated metric can be reported either statistically (resulting in less UL overhead but leaving the NW unaware of the performance condition for longer periods) or per sample (increasing UL overhead but providing the NW with more frequent updates).

#### The monitoring KPIs

In RAN1#118 meeting, the following agreement was made regarding Type 1 option 2 performance monitoring:

Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including: - Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring - Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring - Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring - Note: resources for Set B for monitoring are not precluded and can be study. - Note: this is only applicable when the model can predict RSRP - Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam - Note: this is only applicable when the model can generate probability information - FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference. For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window)

As a starting point, we support Alt1 through Alt3, and hence, we have the following proposal:

- For type 1, option 2, performance monitoring of a UE side AI/ML model, regarding:

- How to report- assess the impact of adopting the following methods:

  - Sample based performance metric reporting in order to provide the NW with frequent updates about the AI/ML model performance.

  - Statistical based performance metric reporting in order to reduce the UL overhead on the expense of less information being available at the gNB.

- What to report- Support using the following metrics:

  - Alt1: Beam prediction accuracy

  - Alt2: The L1-RSRP difference between the predicted beam and the measured beam from the monitoring resource set.

  - Alt3: The RSRP difference between the predicted RSRP from the UE-side AI/ML model and the measured L1-RSRP from the monitoring resource set.

Alt#4 suggests that the probability information output by the UE-side AI/ML model can be utilized to assess the model’s performance when reported to the gNB. However, we believe that the performance monitoring stage must incorporate a ground truth reference, which in this case would be the RS measurements, to accurately evaluate the AI/ML model’s output. Alt#4, however, does not include any ground truth measurements in its performance monitoring, relying solely on the AI/ML model’s probability information.

Consider the following example illustrated in Figure 1: During the data collection and training phase, a UE moves across Cell A, collecting data and training its model. This UE-side AI/ML model consists of “M” output pins corresponding to “M” beams, with each pin value indicating the probability of the beam being the Top-1 beam. Due to geographical barriers, the UE can only collect data corresponding to beams “m” through “M”, leaving out beams 1 through m-1. Consequently, the UE-side AI/ML model is trained only on these beams.

Now, assume that after training, the UE moves from Cell A to Cell B, which has similar conditions. In Cell B, the UE’s location corresponds to the area where beams were barred in Cell A. In this scenario, the UE’s model, not having been trained on beams 1 through “m” will output low probabilities on the model output pins corresponding to these beams. Based on our understanding, one of the following scenarios could occur:

- The remaining model output pins, ranging from “m” to “M” could also exhibit low probability values. Consequently, when any pin probability value is reported to the gNB, it will determine that the UE-side model lacks confidence in its selection. In response, the gNB will initiate the appropriate procedure, such as model switching or falling to legacy.

- The AI/ML model output pins from “m” to “M” could exhibit high probability values. The AI/ML model will distribute the probabilities among the known labels (from “m” to “M”). Since it does not recognize labels from 1 to “m-1”, it might still assign higher probabilities to labels m, m+1, …, M even though none of these are correct. In this case, the gNB might interpret the high probabilities as an indication that the UE is confident in its model and that its beam selection is suitable for BM operation, when in fact it is not.

Figure 1 A UE is trained in Cell A (with a barrier) and performing inference in Cell B.

Based on the above discussion, we have the following proposal:

- For BM Case 1 and Case 2 in a UE side AI/ML model, support Alt1 through Alt3 and deprioritize Alt 4 as a measure for Type 1 option 2 performance monitoring.

In RAN1#118bis meeting, the following agreement was made regarding Alt1:

Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), - At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation - FFS: on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric FFS other alternatives

To accurately define the metric, we first need to establish the following notations:

- For the UE-side AI/ML model in BM-Case 1, which predicts the top-K beams, the k-th predicted beam out of the K beams in the i-th monitoring instance is denoted as ik.

- Let im* denote the beam with the m-th largest measured L1-RSRP value of the resource set(s) for monitoring.

- 1(.) is an indicator function that evaluates to 1 if the applied argument is true and zero otherwise.

  - The size of the performance monitoring set is the same as the size of Set A

    - Option 1 (Top-1/1) sample accuracy for the i-th monitoring instance

The Top-1/1 beam prediction accuracy for the i-th monitoring instance is the event when the largest measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring is Top-1 predicted beam. It can be represented by:

Ai=1i1=i1*.

    - Option 2 (1/Top-K) sample accuracy for the i-th monitoring instance

The Top-1/K beam prediction accuracy for the i-th monitoring instance is the event when the largest measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring is among the Top-K predicted beams. It can be represented by:

Ai=k=1K1ik=i1*.

    - Option 3 (M/Top-K) sample accuracy for the i-th monitoring instance

The M/Top-K beam prediction accuracy for the i-th monitoring instance is the event when at least one of the Top M beams with the largest M measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring is among the predicted Top-K beams. It can be represented by:

Ai=1m=1Mk=1K1ik=im*>0.

After defining the sample accuracy value Ai for each of the above discussed options. Let the Beam Accuracy Indicator (BAI), be defined as i=1NAi, where N is the number of reported inference instance(s) that are linked with performance monitoring instance(s).

For BM-Case 1

Based on the above discussion, we have the following proposal:

- For a UE-side AI/ML model in BM-Case 1 that predicts K beams, utilizing option 2 (UE-assisted) performance monitoring,

  - Introduce the Beam Accuracy Indicator BAI in the CSI report for monitoring, where BAI is Np (0 ≤ Np≤ N), and the corresponding beam prediction accuracy is NpN, (0≤ NpN≤ 1).

    - The size of CSI field associated with the BAI is log2(N+1)

  - N is implicitly or explicitly configured by NW, as the number of the reported inference result(s) linked with performance monitoring instance(s).

  - Np as the number of the reported inference result(s) linked with performance monitoring instance(s), for which:

    - The largest measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring is Top-1 predicted beam

    - The largest measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring is among the Top-K predicted beams

    - At least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s)

      - Where K is the number of predicted beam(s) in the corresponding inference report

      - Where Top M beam(s) is the beam(s) with largest measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring

      - M is configured by NW in CSI report configuration for monitoring, M= 1, and M is up to 4

For BM-Case 2

For BM-case 2, the above definition for BAI can be extended to be per time instance or per report. In BM-Case 2, an inference report contains the beam predictions for T future time instances, where each instance is composed of the Top-K beam predictions.

Figure 2 BM-Case 2 sample accuracy calculation.

As per our understanding, we have the definition for each as follows:

- Per time instance monitoring

The M/Top-K sample prediction accuracy Ai for the i-th monitoring instance is a value that defines the fraction of time instances associated with monitoring instance i, where at least one of the Top M beams with the largest M measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring is among the predicted Top-K beams. Different from BM-Case 1, the sample accuracy in this case is not a binary value. It is a fraction between 0 and 1. 0 is when none of the predicted beams in the future T time instances achieved the condition of the M/Top-K prediction accuracy and 1 is when all the predictions achieve that condition.

- Per report monitoring

In this case, the M/Top-K sample prediction accuracy for the i-th monitoring instance Ai is more restrictive. Similar to BM-Case 1, it is a binary value of either 1 or 0. Ai is equal to 1 if all the time instances associated with the current monitoring instance i satisfy the M/Top-K prediction accuracy condition. In other words, Ai=1 if at least one of the Top M beams with the largest M measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring is among the predicted Top-K beams for every time instance associated with monitoring instance i.

As can be realized from the definitions above, the per report monitoring sample accuracy definition is more restrictive than that of the per time instance sample accuracy one. It requires all the time instances within a monitoring instance to satisfy the M/Top-K condition, while the per time instance monitoring represents the fraction of the time instance within the monitoring instance where the condition is satisfied.

In our view, each of the two procedures mentioned above has its pros and cons. On one hand, per time instance reporting provides the gNB with a finer granularity information about the performance of the UE-side AI/ML model. However, it comes with the expense of an additional overhead since BAI = i=1NAi is a fraction number between 0 and N. if p decimal precision is used to represent this fraction, the size of the CSI field associated with BAI is log2(10pN+1). For time instance reporting, p=1, i.e., one decimal precision should be enough to provide the BAI information with reasonable overhead.

On the other hand, per report monitoring provide a coarse granularity information about the UE side model with the merit of having less CSI field size compared to the per time instance reporting. The size of the CSI field for the per report monitoring is simply log2(N+1) since BAI is always an integer value between 0 and N.

Based on the above discussion, we have the following proposal:

- For a UE-side AI/ML model in BM-Case 2 that predicts K beams for each of the T future time instances, utilizing option 2 (UE-assisted) performance monitoring,

  - Introduce the Beam Accuracy Indicator (BAI) in the CSI report for monitoring, where BAI is Np (0 ≤ Np≤ N), and the corresponding beam prediction accuracy is NpN, (0≤ NpN≤ 1).

    - N is implicitly or explicitly configured by NW, as the number of the reported inference result(s) linked with performance monitoring instance(s).

  - Further study the following options

    - Per time instance monitoring for the calculation of BAI, where BAI is a rational number Np such that,

      - The number Np ​ is defined as the sum of N fractions. Each fraction represents the proportion of time instances associated with the corresponding monitoring instance where the condition is met that at least one of the Top M beam(s), from the resource set(s) for monitoring, is among the predicted Top K beam(s).

      - The size of CSI field associated with the BAI is log2(10pN+1) if p fractional precision for BAI is reported.

        - p=1, FFS: other values of p

    - Per report monitoring for the calculation of BAI, where BAI is an integer number Np such that,

    - Np is the number of reported inference result(s) linked with performance monitoring instance(s), i.e., the number of inference instances, such that all the time instances within each inference instance satisfy the condition that at least one of the Top M beams(s) of the resource set(s) for monitoring is among Top-K predicted beam(s).

    - The size of CSI field associated with the BAI is log2(N+1)

      - K is the number of predicted beam(s) in the corresponding inference report

      - Top M beam(s) is the beam(s) with largest measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring.

      - M is configured by NW in CSI report configuration for monitoring

  - The size of the performance monitoring set is less than the size of Set A

The following proposal was suggested by the FL for RAN1#120 meeting discussion to handle the case when the performance monitoring set size is less than that of Set A.

At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), for the configuration for monitoring, - The CSI-ReportConfigID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - The resource set(s) for monitoring can be configured for full set of Set A, or a subset of Set A, [or different from Set A]. - For the case that a subset of Set A is configured as the resource set(s) for monitoring, further study the following: - Alt 1: Performance metric is calculated (i.e., as valid instance for metric calculation) only if Top-1 or Top-K (FFS all or at least one of Top-K) predicted beam(s) are included in the subset of Set A - Alt 2: For performance metric calculation, with predefined mapping between the full resources for Set A to the subset of set A (with a multi-to-one mapping so that all resources for Set A has a mapped resource for monitoring) - FFS QCL relationship can be used for the mapping, or based on a down-sampling factor - Alt 3: Performance metric is calculated based on the Top K’ predicted beam(s) among the subset of Set A - Note: Top K’ predicted beam(s) among the subset of Set A may or may not be the same as Top K predicted beam(s) reported in the inference report - Alt 4: If the Top K predicted beam(s) is not covered by the resource set(s) for monitoring, the reported result(s) is counted as inaccurate. - Alt 5: gNB ensures to the Top K predicted beam(s) reported in the inference report is always covered by the resource set(s) for monitoring. - Note1: No spec impact - Note2: This does not preclude to restrict the inference result(s) in a subset of Set A. - Other alternatives are not precluded. - For the case that a different set of Set A is configured as the resource set(s) for monitoring, support: - Alt 2: For performance metric calculation, with predefined mapping between the full resources for Set A to the resources for monitoring (with a multi-to-one mapping so that all resources for Set A has a mapped resource for monitoring) - FFS QCL relationship can be used for the mapping, or based on a down-sampling factor

In the second bullet of the above proposal, it is mentioned that the resource set for monitoring can be configured for full Set A, or for a subset of Set A. The last part [different from Set A] is unclear for us. The UE-side AI/ML model predicts the Top-K beams from Set A, and hence, the resources from the monitoring set need to correspond to beams from Set A, i.e., the RS on those resources should be beam formed using the same spatial filters as Set A, to have an apple-to-apple comparison. However, the resources for the monitoring set can be different from those configured for Set A.

For the five alternatives listed in the proposal we have the following discussion:

- We view Alt1 and Alt4 in a similar way. On one hand, Alt1 states that the accuracy is calculated only if the Top-1 or Top-K predicted beams are included in the subset of A. In our view, at least one of the Top-K predicted beams should be included in the monitoring set for the accuracy to be calculated and there is no need for the more restrictive case of having the whole Top-K predicted beams within the monitoring set for accuracy calculation. Moreover, the UE might not be confident about the accuracy calculation. This is because the optimal beam, which is the beam with the largest measured L1-RSRP if measurements were taken from all Set A beams, might not be among the beams represented by the performance monitoring resources. On the other hand, Alt4 completes Alt1 that when the Top-K beams are not in the monitoring resource set, there is no way to calculate the accuracy, and the model is assumed to mis detect for that instance.

- In Alt2, a predefined multi-to-one mapping could be used in order to map between the full resources for Set A to the subset of set A (the performance monitoring set). This could be done by a down sampling factor for the resources in Set A. For example, consider a Set A of 4 resources that map to a monitoring set of only two resources. Beams 1 and 2 are represented by the first resource while beams 3 and 4 are represented by the second resource in the monitoring resource set. If the UE side AI/ML model detects the Top-2 predicted beams as beams 1 and 2, while the maximum measured L1-RSRP is from the first resource of the monitoring set. The UE will then be confident that the optimal beam is one of its detected beams and feedback the beam prediction accuracy based on that.

- In Alt3, the performance metric is calculated based on the Top K’ predicted beam(s) among the subset of Set A. Those Top-K’ beams might not be the same as the Top-K predicted beams. The Top-K’ beams are a group of beams that the gNB decides to transmit for performance monitoring purpose at a given performance monitoring instance. The gNB should intelligently configure the resources of the K’ beams to provide measurements for the actual top-K beams, which the UE will use as ground truth. The K’ beams should ideally be the Top-K’ genie aided beams. To achieve this, the gNB requires information to select the appropriate monitoring set for configuration for each monitoring instance. As an example, the UE could signal the gNB with information from set B, see Figure 2 below. Based on that information, the gNB can identify the K’ beams for performance monitoring to be included in the performance monitoring set.

Figure 3 Example demonstrating Top-K’ beams generation through beam information from Set B. This information maps to the performance monitoring set, that is a subset of the total resource set configured for monitoring, where each resource corresponds to a beam in Set A

- We don’t support Alt5. First, it is not clear for us how will the gNB ensure that the Top-K predicted beams by the UE are covered within the performance monitoring set. Second, we only see a motivation for Alt5 in case of regression models where the gNB aims to determine how good/bad the UE can predict the RSRP of the beam. However, there is no benefit in classification models because the ground truth in AI/ML model testing/monitoring shall be independent of the AI/ML model’s output.

Based on the above discussion, we have the following proposals:

- At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), for the configuration for monitoring,

- The CSI-ReportConfigID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration

- The resource set(s) for monitoring can be configured for full set of Set A, or a subset of Set A, [or different from Set A].

- For the case that a subset of Set A is configured as the resource set(s) for monitoring, further study the following:

  - Alt 1: Performance metric is calculated (i.e., as valid instance for metric calculation) only if at least one of the Top-K predicted beam(s) are included in the subset of Set A.

  - Alt 2: For performance metric calculation, with predefined mapping between the full resources for Set A to the subset of set A (with a multi-to-one mapping so that all resources for Set A has a mapped resource for monitoring), consider using a down-sampling factor as a starting point.

    - FFS QCL relationship can be used for the mapping.

  - Alt 3: Performance metric is calculated based on the Top K’ predicted beam(s) among the subset of Set A

      - Support using a mapping from beam information to performance monitoring resources.

      - Note: Top K’ predicted beam(s) among the subset of Set A may or may not be the same as Top K predicted beam(s) reported in the inference report

    - Alt 4: If the Top K predicted beam(s) is not covered by the resource set(s) for monitoring, the reported result(s) is counted as inaccurate.

    - Other alternatives are not precluded.

In RAN1#119 meeting, the following agreement was made:

At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signalling: - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report - FFS on the timing related issues - UE measures the dedicated resource set(s) for monitoring.

FFS on timing related issues

Based on the above agreement, since the monitoring resources are configured within a dedicated CSI report configuration that is different from that of inference, there is a need to define a linkage between the monitoring and inference reports to calculate the beam predication accuracy discussed in previous sections.

In case of BM-Case 1, since the CSI reference resource is intended to provide an alignment between the UE and the gNB by configuring the CSI reporting offset and CSI-RS to report association, it is natural to consider linking the transmission occasion of the CSI-RS/SSB resource for monitoring to the CSI reference resource for inference. Moreover, in order to ensure that the monitoring results best represent the beams based on the current channel conditions, it makes sense to link the transmission occasion of the CSI-RS/SSB resource for monitoring to the closest CSI reference resource for inference. It could be before or after the CSI reference resource for inference. As discussed in Section 6.1.2-a, for the calculation of BAI, the value of N should be equal to the number of CSI report configurations for monitoring.

Moreover, a threshold should be defined for the time difference between the CSI reference resource for inference and the CSI-RS/SSB resource for monitoring. This threshold should define the validity of comparing the inference with monitoring results. If the time distance between the CSI reference resource for inference and the CSI-RS/SSB resource for monitoring is greater than that threshold, then this calculation should not be taken into consideration while calculating BAI.

A similar discussion can be made for BM-Case 2, as can be interpreted from Figure 2, the linked transmission occasion for CSI-RS/SSB resource for monitoring should linked to the closest time instance for prediction.

Based on the above discussion, we have the following proposal:

- At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), for the configuration for monitoring,

  - for BM-Case 1, for an inference result with a CSI reference resource for Set B, the linked transmission occasion is a transmission occasion of the CSI-RS/SSB resource for monitoring which is “closest” to the CSI reference resource for Set B.

    - The linked transmission occasion could be before or after the closest CSI reference resource for inference.

  - for BM-Case 2, for an inference result for a time instance for prediction, the linked transmission occasion is a transmission occasion of the CSI-RS/SSB resource for monitoring with is “closest” to the time instance for prediction.

    - The linked transmission occasion could be before or after the closest the time instance for prediction.

  - FFS: the definition of a threshold for the time difference between the transmission occasion of the CSI-RS/SSB resource for monitoring and CSI reference resource/predicted time instance.

FFS on how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams

The configuration of dedicated resources for monitoring that are different from the report configuration for inference necessitates that each resource used for monitoring to be linked to a beam in Set A. This linkage, illustrated in Figure 2, enables the UE to determine which output pins from its AI/ML model should be compared with the monitoring measurements. A straightforward method to establish this linkage is by introducing a new IE “BeamCorrespondence” in the “NZP-CSI-RS-Resource”. This new IE should take an integer value that refers to the linked beam index.

- For a UE-side AI/ML model Type 1, option 2 (UE-assisted) performance monitoring, when the performance monitoring set is configured in a dedicated report separate from the inference report configuration, consider introducing a new IE “BeamCorrespondence” within “NZP-CSI-RS-Resource.” This IE should link a performance monitoring resource to a beam index.

  - FFS: Other alternatives

FFS on whether to support all the combination on time domain behavior of the reportConfigType for inference report and the reportConfigType for monitoring report

For the different time domain behavior combination of the inference and monitoring configuration reports:

| Inference Report | Monitoring Report | Support/No Support | Reason |
| --- | --- | --- | --- |
| Periodic | Periodic | Support | For periodic inference report configuration, periodic and semi persistent performance monitoring reports can be supported to support the statistical based reporting highlighted in Table 1 |
| Periodic | Semi persistent | Support |  |
| Periodic | Aperiodic | Support | For periodic inference report configuration, aperiodic performance monitoring report can be supported to support the sample-based reporting highlighted in Table 1 |
| Aperiodic | Periodic | No Support | We see no benefit in configuring periodic and semi persistent reports for monitoring in case when the report configuration for inference is aperiodic. |
| Aperiodic | Semi persistent | No Support |  |
| Aperiodic | Aperiodic | Support | For aperiodic inference report configuration, aperiodic performance monitoring report can be supported to support the sample-based reporting highlighted in Table 1 |
| Semi persistent | Periodic | No Support | We see no clear benefit in this configuration as there might be performance monitoring instances that are not linked to an inference report instance. |
| Semi persistent | Semi persistent | Support | For Semi persistent inference report configuration, semi persistent performance monitoring report can be supported to support the statistical-based reporting highlighted in Table 1. However, the semi-persistent activation for performance monitoring should occur at least at the activation for inference. |
| Semi persistent | Aperiodic | Support | For semi persistent inference report configuration, aperiodic performance monitoring report can be supported to support the sample-based reporting highlighted in Table 1. |

Table 2 Support for different time behaviour combinations of the inference and monitoring reports.

Based on Table 2, we have the following proposal:

- At least for the monitoring Type 1 Option 2 of UE-side model monitoring, for the time domain behavior of the reportConfigType for inference report and the reportConfigType for monitoring report, support the following:

  - Periodic inference report and either periodic, semi-persistent or aperiodic report for monitoring.

  - Aperiodic inference report and aperiodic report for monitoring.

  - Semi-persistent inference report and either semi-persistent or aperiodic report for monitoring.

### Report triggering based on event(s) for Option 1 and/or Option 2

It is unclear how the UE can trigger a report based on an event in option 1. From our understanding, an event is defined as a scenario where the performance metric (whether sample-based or statistical) exceeds or falls below a specified threshold, prompting a report to be sent to the NW. In option 1, the UE does not calculate the performance metric needed to detect such an event; it simply reports the model output and measurement results to the NW without evaluating performance thresholds. Therefore, it is uncertain how a report can be triggered based on events in option 1. Conversely, this approach appears more applicable to option 2, where the UE calculates the performance metric and can thus determine when an event occurs. Based on this, we propose the following:

- For UE-side AI/ML performance monitoring, RAN1 should further study the following:

- An event is defined as the scenario when the performance metric falls above (or below) a certain threshold.

- The concept of triggering a report based on specific events is not applicable to option 1. It is, however, relevant to option 2, where the UE calculates performance metrics and can detect events.

# CPU for a UE-Side AI/ML Model

Since the AI/ML functionalities require different hardware modules than the non-AI/ML ones, it is natural to consider a dedicated CPU pool as AI/ML based CSI processing unit (ACPU). However, there should be some level coexistence between CPU (for legacy CSI reporting) and ACPU (for inference report calculation in case of BM). This is because a UE may use CPU for reference signals (Set B) measurement and ACPU for model inference. Therefore, in our view, the triggering of an AI/ML model for inference depends jointly on the available number of CPU and ACPU units.

In our view, the CSI report for monitoring, at least for Type 1 option 1 monitoring, is a legacy CSI measurement and reporting procedure. Hence, it should rely on the legacy-based CPU pool.

Additionally, we don’t see a reason to have different ACPUs for different AI/ML functionalities since they share the same HW resources.

Based on the above discussion, we have the following proposal:

- For UE-side model, at least for AI/ML based beam management, for processing of a CSI report at least for inference, introduce a dedicated CPU pool as AI/ML-based CSI processing unit (ACPU)

  - The UE capability for AI/ML inference reporting depends jointly on both CPU and ACPU.

  - At least for Type 1 option 1 monitoring, CSI monitoring report depends on legacy CPU.

  - Study the feasibility of having AI/ML based functionalities (BM and CSI prediction) share the same ACPU pool.

# Conclusion

In the previous sections we made the following observations:

- For a NW side AI/ML model, for BM Case1 and Case 2, regarding the reporting of Set B measurements for inference purpose, when (M) is equal to the size of the measurement resource set, and Set B is defined solely by the resources of this resource set, it may not be necessary to report the beam index of the largest L1-RSRP value. The measured absolute/differential L1-RSRPs alone may be sufficient for the AI/ML model to operate effectively.

Based on the discussion in the previous sections we propose the following:

- For a NW-sided AI/ML model for BM-Case1 and BM-Case2, for the configuration of inference results, RAN1 should consider the following options:

- When Set B is different from Set A, no configuration for Set A is required for the UE, as the inference process is conducted transparently from the UE.

- When Set B is a subset of Set A, it might be sufficient for the gNB to only configure Set A.

  - FFS: Determination of Set B out of Set A.

- When Set B is equal to Set A, there is no need to configure Set A and it is sufficient to only configure Set A.

- The gNB should configure reporting-related parameters, including then number of beams that the UE should report.

- For a UE-side AI/ML model, support configuring Sets A and B for inference results reporting, where:

- When Set B is different from Set A,

  - Set B is explicitly configured for measurements for the UE. It enables the UE to perform measurements that derive the UE’s AI/ML model.

  - Set A can be optionally virtually configured. It is not intended to provide measurements to the UE. Instead, it serves as a reference to map the UE-side AI/ML model output, in the case of classification models, to the corresponding CRIs that the gNB can interpret.

- When Set B is a subset of or equal to Set A, it might be sufficient for the gNB to only configure Set A.

  - FFS: Determination of Set B out of Set A.

- When Set B is equal to Set A, there is no need to configure Set A and it is sufficient to only configure Set A.

- For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signaling, support

  - L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB

  - If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set.

  - The Following options can be considered for beam information when M<the size of the measurement resource set:

    - Option 1: CRI/SSBRI of a measurement resource set

    - Option 2: A bit map to indicate the reported beams of a measurement resource set

- For a NW-side AI/ML model, extend maxNrofNZP-CSI-RS-ResourcesPerSet (i.e., Maximum number of NZP CSI-RS resources per resource set) to 256.

- For the UE-side AI/ML model, regarding the content of the report inference results, support using options 1 through 4. Further study the benefits and gains of adopting options 3 and 4, considering the effects of quantization methods on system throughput and considering the additional overhead incurred.

- Regarding the definition of the confidence information, for option 4 of a UE side AI/ML model report content for inference, consider the following definition:

  - The confidence information represents a range within which the predicted RSRP of a beam in Set A is expected to fall a certain percentage of the time if the same input sample is reused to derive the AI/ML model.

  - FFS on the details of the calculation of the confidence interval.

- For a UE-side AI/ML model report content for inference, further study the applicability of adopting Option 4, including its pros and cons, in terms of the associated UL overhead and its potential to provide the gNB with information that could assist in configuring the performance monitoring set.

- For a UE side AI/ML model probability information reporting, support one decimal precision probabilities where only 4 bits (bit width) are required to represent the probability information per beam. The choice of rounding up/down the probability values is left up to the UE’s implementation.

- For a UE-side AI/ML model inference report on beam ranking, where the UE is required to report the Top-K beams out of N beams in Set A, consider:

  - CRI-based Reporting: The UE should report the beam information in a ranked manner using a CRI vector. This involves reporting a K-dimensional vector of CRIs, where the ith entry corresponds to the ith strongest beam. The ithstrongest beam can be determined by the beam information.

- Further study whether to support the following cases for configuration of the associated ID:

- Case 1: The associated ID is configured for data collection.

  - Case 1-0: The associated ID is configured for inference/step 3

    - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID.

  - Case 1-1: The associated ID is not configured for inference/step 3.

    - UE may report it is not applicable

    - AI/ML model selection is left up to the UE’s implementation.

- Case 2: The associated ID is not configured for data collection. Either it is configured or not for inference:

  - Alt 1: The UE cannot assume anything and only uses its trained AI/ML model.

  - Alt 2: Depending on the UE’s capability, the UE may report that the associated ID is applicable or not.

- For a NW-side AI/ML model, to ensure consistency between training and inference, study the feasibility of the following mechanisms:

  - The UE reports the receive beam index to the NW.

  - To eliminate the UL reporting overhead, the best receive beam is assumed by the NW. The NW configures the QCL relationship, and the UE uses the receive beam identified for the QCL source for performing the measurements.

- For type 1, option 2, performance monitoring of a UE side AI/ML model, regarding the FFS of:

- How to report- assess the impact of adopting the following methods:

  - Sample based performance metric reporting in order to provide the NW with frequent updates about the AI/ML model performance.

  - Statistical based performance metric reporting in order to reduce the UL overhead on the expense of less information being available at the gNB.

- What to report- Support using the following metrics as a starting point:

  - The error between the predicted RSRP and the measured L1-RSRP, where the predicted RSRP is based on AI/ML output.

  - The beam predication accuracy of the top-K beams, where the beam prediction accuracy is the percentage of the beam determined from RS measurements for performance monitoring to be one of the top-K predicted beams by the AI/ML model.

- For BM Case 1 and Case 2 in a UE side AI/ML model, support Alt1 through Alt3 and deprioritize Alt 4 as a measure for Type 1 option 2 performance monitoring.

- For a UE-side AI/ML model in BM-Case 1 that predicts K beams, utilizing option 2 (UE-assisted) performance monitoring,

  - Introduce the Beam Accuracy Indicator BAI in the CSI report for monitoring, where BAI is Np (0 ≤ Np≤ N), and the corresponding beam prediction accuracy is NpN, (0≤ NpN≤ 1).

    - The size of CSI field associated with the BAI is log2(N+1)

  - N is implicitly or explicitly configured by NW, as the number of the reported inference result(s) linked with performance monitoring instance(s).

  - Np as the number of the reported inference result(s) linked with performance monitoring instance(s), for which:

    - The largest measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring is Top-1 predicted beam

    - The largest measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring is among the Top-K predicted beams

    - At least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s)

      - Where K is the number of predicted beam(s) in the corresponding inference report

      - Where Top M beam(s) is the beam(s) with largest measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring

  - M is configured by NW in CSI report configuration for monitoring, M= 1, and M is up to 4

- For a UE-side AI/ML model in BM-Case 2 that predicts K beams for each of the T future time instances, utilizing option 2 (UE-assisted) performance monitoring,

  - Introduce the Beam Accuracy Indicator (BAI) in the CSI report for monitoring, where BAI is Np (0 ≤ Np≤ N), and the corresponding beam prediction accuracy is NpN, (0≤ NpN≤ 1).

    - N is implicitly or explicitly configured by NW, as the number of the reported inference result(s) linked with performance monitoring instance(s).

  - Further study the following options

    - Per time instance monitoring for the calculation of BAI, where BAI is a rational number Np such that,

      - The number Np ​ is defined as the sum of N fractions. Each fraction represents the proportion of time instances associated with the corresponding monitoring instance where the condition is met that at least one of the Top M beam(s), from the resource set(s) for monitoring, is among the predicted Top K beam(s).

      - The size of CSI field associated with the BAI is log2(10pN+1) if p fractional precision for BAI is reported.

        - p=1, FFS: other values of p

    - Per report monitoring for the calculation of BAI, where BAI is an integer number Np such that,

    - Np is the number of the reported inference result(s) linked with performance monitoring instance(s), for which at least one of the Top M beams(s) of the resource set(s) for monitoring is among Top-K predicted beam(s)

    - The size of CSI field associated with the BAI is log2(N+1)

      - K is the number of predicted beam(s) in the corresponding inference report

      - Top M beam(s) is the beam(s) with largest measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring.

      - M is configured by NW in CSI report configuration for monitoring

- At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), for the configuration for monitoring,

- The CSI-ReportConfigID of a inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration

- The resource set(s) for monitoring can be configured for full set of Set A, or a subset of Set A, [or different from Set A].

- For the case that a subset of Set A is configured as the resource set(s) for monitoring, further study the following:

  - Alt 1: Performance metric is calculated (i.e., as valid instance for metric calculation) only if at least one of the Top-K predicted beam(s) are included in the subset of Set A.

  - Alt 2: For performance metric calculation, with predefined mapping between the full resources for Set A to the subset of set A (with a multi-to-one mapping so that all resources for Set A has a mapped resource for monitoring), consider using a down-sampling factor as a starting point.

    - FFS QCL relationship can be used for the mapping.

  - Alt 3: Performance metric is calculated based on the Top K’ predicted beam(s) among the subset of Set A

      - Support using a mapping from beam information to performance monitoring resources.

      - Note: Top K’ predicted beam(s) among the subset of Set A may or may not be the same as Top K predicted beam(s) reported in the inference report

    - Alt 4: If the Top K predicted beam(s) is not covered by the resource set(s) for monitoring, the reported result(s) is counted as inaccurate.

    - Other alternatives are not precluded.

- At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), for the configuration for monitoring,

  - for BM-Case 1, for an inference result with a CSI reference resource for Set B, the linked transmission occasion is a transmission occasion of the CSI-RS/SSB resource for monitoring which is “closest” to the CSI reference resource for Set B.

    - The linked transmission occasion could be before or after the closest CSI reference resource for inference.

  - for BM-Case 2, for an inference result for a time instance for prediction, the linked transmission occasion is a transmission occasion of the CSI-RS/SSB resource for monitoring with is “closest” to the time instance for prediction.

    - The linked transmission occasion could be before or after the closest the time instance for prediction.

  - FFS: the definition of a threshold for the time difference between the transmission occasion of the CSI-RS/SSB resource for monitoring and CSI reference resource/predicted time instance.

- For a UE-side AI/ML model Type 1, option 2 (UE-assisted) performance monitoring, when the performance monitoring set is configured in a dedicated report separate from the inference report configuration, consider introducing a new IE “BeamCorrespondence” within “NZP-CSI-RS-Resource.” This IE should link a performance monitoring resource to a beam index.

  - FFS: Other alternatives

- At least for the monitoring Type 1 Option 2 of UE-side model monitoring, for the time domain behavior of the reportConfigType for inference report and the reportConfigType for monitoring report, support the following:

  - Periodic inference report and either periodic, semi-persistent or aperiodic report for monitoring.

  - Aperiodic inference report and aperiodic report for monitoring.

    - Semi-persistent inference report and either semi-persistent or aperiodic report for monitoring.

- For UE-side AI/ML performance monitoring, RAN1 should further study the following:

- An event is defined as the scenario when the performance metric falls above (or below) a certain threshold.

- The concept of triggering a report based on specific events is not applicable to option 1. It is, however, relevant to option 2, where the UE calculates performance metrics and can detect events.

- For UE-side model, at least for AI/ML based beam management, for processing of a CSI report at least for inference, introduce a dedicated CPU pool as AI/ML-based CSI processing unit (ACPU)

  - The UE capability for AI/ML inference reporting depends jointly on both CPU and ACPU.

  - At least for Type 1 option 1 monitoring, CSI monitoring report depends on legacy CPU.

  - Study the feasibility of having AI/ML based functionalities (BM and CSI prediction) share the same ACPU pool.

# References

| [1] | TR 38.843 v2.0.1, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface (Release 18),” Third Generation Partnership Porject (3GPP), 2023. |
| --- | --- |