3GPP TSG RAN WG1 #120bis		R1-2501804

Wuhan, China, April 7th – 11th, 2025

Source:	vivo

Title:	Discussion on random access for Rel-19 SBFD

Agenda Item:	9.3.2

Document for:	Discussion and Decision

- Background

For SBFD operation, ROs configured within UL subband in DL symbols can provide more PRACH transmission opportunities, which may reduce random access latency and collision probability and improve coverage. In this document, some remaining issues will be discussed:

- RACH configuration

2.1 ssb-SharedRO-MaskIndex

Agreement[1] For RACH configuration Option 1, down-select from the following two alternatives: - Alt-1: Shared ssb-SharedRO-MaskIndex configuration for Additional-ROs and Legacy-ROs. - Alt-2: Separate ssb-SharedRO-MaskIndex configurations for Additional-ROs and Legacy-ROs.

The parameter ssb-SharedRO-MaskIndex is used to indicate a subset of ROs where preambles are allocated for RedCap feature combination. For a RedCap UE supporting SBFD operation, both Additional-ROs and Legacy-ROs can be available for random access. In this case, ssb-SharedRO-MaskIndex is need be determined for Additional-ROs. For a RedCap UE without SBFD capability, only legacy ROs can be available and legacy configuration can be applied.

Since the motivation of RACH configuration option 1 is to reuse the existing parameters based on legacy configuration, the additional parameters should be configured as less as possible. The shared ssb-SharedRO-MaskIndex configuration for Additional-ROs and Legacy-ROs is preferred, i.e. Alt 1.

Proposal 1: For RACH configuration Option 1, shared ssb-SharedRO-MaskIndex configuration for Additional-ROs and Legacy-ROs is preferred.

2.2 Transmission power

Agreement[1] Consider the following alternatives about power ramping when a SBFD-aware UE switches from additional-ROs to legacy-ROs and from legacy-ROs to additional-ROs (if supported) in PRACH transmission re-attempt in one RACH procedure for RACH configuration Option 1 and Option 2. - Alt-1: Increase the power ramping counter. - FFS whether to introduce a power offset to compensate the power ramping difference for RACH configuration Option 2. - Alt-2: Reset the power ramping. - Note: different alternatives can be applied for different RACH configuration options.

For preamble transmission power determination, the initial power level is determined by PathLoss, preambleReceivedTargetPower, preamble format related parameter etc. The power level between legacy-ROs and additional-ROs may have difference. When a UE switches from additional-ROs to legacy-ROs and from legacy-ROs to additional-ROs (if supported) in PRACH transmission re-attempt, the larger power level difference may exist if reset power ramping, as shown in Figure 1. In this case, UE needs more power ramping accumulation to reach the previous power level. This may prolong the initial access latency.

Figure 1 the larger power ramping difference in switching RO type procedure if reset power ramping.

Therefore, Alt-1 is preferred, i.e. increase the power ramping counter in case of switching RO type. A power offset can be introduced to compensate the power ramping difference for RACH configuration Option 2.

In case of switching RO type A to RO type B, the target power level follows the power determination formula for RO type B plus an offset. The offset can be determined according to the power ramping difference between RO type A and Type B, e.g., Power offset = (power ramping counter – 1)* |Power ramping step for RO type A – Power ramping step for RO type B|.

Proposal 2: When a SBFD-aware UE switches from additional-ROs to legacy-ROs and from legacy-ROs to additional-ROs (if supported) in PRACH transmission re-attempt in one RACH procedure for RACH configuration Option 1 and Option 2, the power ramping counter increases. A power offset is introduced to compensate the power ramping difference for RACH configuration Option 2.

- Msg. 3 transmission

3.1 Transmission power

Agreement[1] For determination of the Msg3 PUSCH transmission power and when separate preambleReceivedTargetPower for additional-ROs is configured for RACH configuration Option 1 - preambleReceivedTargetPower configured for legacy-RO is used if Msg3 PUSCH is transmitted in non-SBFD symbols; - preambleReceivedTargetPower configured for additional-RO is used if Msg3 PUSCH is transmitted in SBFD symbols. - FFS: Option 2

According to the cited agreement, transmission power determination for Msg 1 and Msg 3 for a SBFD aware UE can base on power parameters from different RACH configuration. For example, if Msg 1 was transmitted in SBFD symbols, the transmission power is based on preambleReceivedTargetPower configured for SBFD symbols. When Msg 3 is scheduled in non-SBFD symbols, the legacy preambleReceivedTargetPower would be applied for transmission power determination.

Although preambleReceivedTargetPower in SBFD symbols and non-SBFD symbols may have difference due to the separate configuration, the same mechanism can also be extended for RACH configuration option 2. It should be noted that Msg. 3 PUSCH transmission is scheduled by gNB and gNB can adjust the transmission power for Msg. 3 PUSCH based on proper scheduling, e.g., by TPC command. The unified solution between RACH configuration option 1 and option 2 is preferred.

Proposal 3: For determination of the Msg3 PUSCH transmission power and when separate preambleReceivedTargetPower for additional-ROs is configured for RACH configuration Option 2

- preambleReceivedTargetPower configured for legacy-RO is used if Msg3 PUSCH is transmitted in non-SBFD symbols;

- preambleReceivedTargetPower configured for additional-RO is used if Msg3 PUSCH is transmitted in SBFD symbols.

3.2 RB numbering

Agreement (9.3.1) - If UE-specific configuration on frequency locations of SBFD subbands is not supported nor provided, support Option 1 for determining UL/DL usable PRBs. - Option 1: For a UL BWP, UL usable PRBs are determined as intersection between cell-specific UL subband and the UL BWP in SBFD symbols. For a DL BWP, DL usable PRBs are determined as intersection between cell-specific DL subband(s) and the DL BWP in SBFD symbols. 38.213-i40[2] 8.3 PUSCH scheduled by RAR UL grant An active UL BWP, as described in Clause 12 and in [4, TS 38.211], for a PUSCH transmission scheduled by a RAR UL grant is indicated by higher layers. For determining the frequency domain resource allocation for the PUSCH transmission within the active UL BWP - if the active UL BWP and the initial UL BWP have same SCS and same CP length and the active UL BWP includes all RBs of the initial UL BWP, or the active UL BWP is the initial UL BWP, the initial UL BWP is used - else, the RB numbering starts from the first RB of the active UL BWP and the maximum number of RBs for frequency domain resource allocation equals the number of RBs in the initial UL BWP

For RB numbering for Msg.3 transmission in non-SBFD symbols, both initial and active BWP are considered. For an active BWP in ‘else case’, as the above cited paragraph with blue color, RB numbering starts from the first RB of the active UL BWP and the maximum number of RBs for frequency domain resource allocation equals the number of RBs in the initial UL BWP. If the same rule is applied for SBFD symbols, only partial UL frequency resource of UL subband can be available, i.e., frequency range A as shown in Figure 2.

Alternatively, starting RB can be considered from the first RB of UL usable RB for SBFD symbols. The available RB size can take both size of UL usable PRB and initial UL BWP into account. Resource of UL subband can be fully utilized. That is the numbering starting from the first RB of the UL usable PRBs for Msg3 PUSCH transmission and the maximum number of RBs for frequency domain resource allocation equaling to the min {size of UL usable PRBs for Msg3 PUSCH transmission, number of RBs in the initial UL BWP}.

Figure 2 Frequency RA for Msg. 3 PUSCH if follow legacy rule in non-SBFD symbol

Proposal 4: The RB numbering starts from the first RB of the UL usable PRBs for Msg3 PUSCH transmission and the maximum number of RBs for frequency domain resource allocation equals the min {size of UL usable PRBs for Msg3 PUSCH transmission, number of RBs in the initial UL BWP}.

- Msg. 4 transmission

In the previous RAN1 meeting, agreements on PUCCH resource sets and frequency hopping for Msg4 HARQ-ACK PUCCH were achieved. One remaining issue is open-loop power control parameter configuration for Msg4 HARQ-ACK PUCCH. For Msg4 HARQ-ACK PUCCH in SBFD symbols, a separate power control parameter can be configured, i.e., separate p0-nominal.

Proposal 5: For SBFD-aware UEs, support separate configuration of p0-nominal for Msg4 HARQ-ACK PUCCH in SBFD symbols.

- Conclusion

In this contribution, we make discussions on random access for Rel-19 SBFD and have the following observations and proposals.

Proposal 1: For RACH configuration Option 1, shared ssb-SharedRO-MaskIndex configuration for Additional-ROs and Legacy-ROs is preferred.

Proposal 2: When a SBFD-aware UE switches from additional-ROs to legacy-ROs and from legacy-ROs to additional-ROs (if supported) in PRACH transmission re-attempt in one RACH procedure for RACH configuration Option 1 and Option 2, the power ramping counter increases. A power offset is introduced to compensate the power ramping difference for RACH configuration Option 2.

Proposal 3: For determination of the Msg3 PUSCH transmission power and when separate preambleReceivedTargetPower for additional-ROs is configured for RACH configuration Option 2.

- preambleReceivedTargetPower configured for legacy-RO is used if Msg3 PUSCH is transmitted in non-SBFD symbols;

- preambleReceivedTargetPower configured for additional-RO is used if Msg3 PUSCH is transmitted in SBFD symbols.

Proposal 4: The RB numbering starts from the first RB of the UL usable PRBs for Msg3 PUSCH transmission and the maximum number of RBs for frequency domain resource allocation equals the min {size of UL usable PRBs for Msg3 PUSCH transmission, number of RBs in the initial UL BWP}.

Proposal 5: For SBFD-aware UEs, support separate configuration of p0-nominal for Msg4 HARQ-ACK PUCCH in SBFD symbols.

- References

- Chair‘s notes RAN1#120 meeting.

- 38.213-i40, Physical layer procedures for control.