3GPP TSG RAN WG1 #120bis                                                    R1-2501855

# Wuhan, China, April 07th – 11th, 2025

Title:	Draft reply LS on number of beam measurements in the measurement report MAC CE

Response to:	R1-2501686(R2-2501485)

Release:	Rel-19

Work Item:	NR_MIMO_Ph5

Source:                 Spreadtrum, UNISOC, [RAN1]

To:                         RAN2

Cc:

Contact Person:

Name:

E-mail Address:

1. Overall Description:

RAN1 thanks RAN2 for the LS R1-2501686(R2-2501485) and would like to provide the following response.

Question

In the previous RAN2 meetings, RAN2 has agreed that the event-triggered measurement report shall be transmitted via MAC CE. The MAC CE shall include the beam-level measurement results corresponding to the event-triggered report configuration in LTM-CSI-ReportConfig in the RRC spec.

Regarding the format of the MAC CE, we would like to ask the following question:

- What should be the maximum number of beam-level measurement results that can be included in the MAC CE?

Answer

For event-triggered report, the maximum number of beam-level measurement results in the MAC CE is 4 beams per cell and 4 cells for all LTM events, which is the same as the gNB scheduled L1 reporting. The parameters nrOfReportedCells (L, up to four), nrOfReportedRS-PerCell (M, up to four) and spCellInclusion in Rel-18 LTM CSI report configuration can be reused. Even for LTM2, the candidate cell(s) and the corresponding RS resource(s) also can be configured for unified design.

2. Actions:

To RAN2 group:

ACTION: RAN1 respectfully requests RAN2 to take the above information into account.

3. Date of Next RAN1 Meetings:

RAN1#121			May 19 – 23, 2025			Malta, MT