3GPP TSG RAN WG1 #120bis                                                                                       R1-2501910

Wuhan, China, 07th - 11th April 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI

Agenda item:	9.12.1

Document for:	Discussion and Decision

Introduction

In RAN#106 meeting, a revised WID on Multi-carrier enhancements for NR Phase 3 has been approved [1]. The details of the objective are shown below.

1. Specify the support of the following for multi-cell PUSCH/PDSCH scheduling with a single DCI [RAN1] - Different SCS/carrier type among co-scheduled cells by the single DCI. - One or multiple PUSCHs/PDSCHs per scheduled cell by the single DCI. - The maximum number of PUSCHs/PDSCHs for a scheduled cell is 8. - Note: Type-1 HARQ-ACK codebook is not enhanced for Rel-19 multi-cell scheduling. - Note: The maximum number of sub-codebooks for Type-2 HARQ-ACK codebook is not increased for Rel-19 multi-cell scheduling. - Note: UE does not expect to be configured with both single-cell multi-PUSCH/PDSCH scheduling and multi-cell multi-PUSCH/PDSCH scheduling on the same or different cells within a same PUCCH group. - Note: No new DCI format is introduced.

In this contribution, our analyses on the Rel-19 multi-cell PUSCH/PDSCH scheduling with a single DCI are provided, including HARQ timing, Type-1/2 HARQ-ACK codebook, the maximum number of PUSCHs/PDSCHs and RRC parameters.

Scenarios

In RAN1#118bis, 6 cases for scheduled cells in Rel-19 multi-cell scheduling have been agreed as below.

RAN1#118bis Agreement - Consider at least the case that up to two different SCS can be scheduled by a DCI format 0_3/1_3 in Rel-19. - Consider at least the following cases for scheduled cells in Rel-19: - Case 1: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR1 licensed FDD cell(s) with SCS1 and FR1 licensed TDD cell(s) with SCS2. - SCS1 can be same or different to SCS2. - Case 2: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR1 licensed FDD cell(s) with SCS1 and FR2-1 cell(s) with SCS2. - SCS1 can be same or different to SCS2. - Case 3: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR1 licensed TDD cell(s) with SCS1 and FR2-1 cell(s) with SCS2. - SCS1 can be same or different to SCS2. - Case 4: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR1 licensed FDD cell(s) with different SCS. - Case 5: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR1 licensed TDD cell(s) with different SCS. - Case 6: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR2-1 cell(s) with different SCS.

For the 6 cases, the scheduled cells can have at most two different SCS. However, based on the significant commercial demands from operators, it is recommended that the case involving three different SCS can also be supported. This encompasses FR1 licensed FDD cell(s) with SCS1, FR1 licensed TDD cell(s) with SCS2 and FR2-1 cell(s) with SCS3. For instance, an operator in China might deploy one FDD cell at 700 MHz, one TDD cell at 2.6 GHz, and an FR2-1 cell at 26 GHz. Another Chinese operator could utilize a similar deployment with one FDD cell at 2.1 MHz, one TDD cell at 3.5 GHz, and an FR2-1 cell at 26 GHz for multi-cell scheduling. It should be noted there is no extra standardization efforts compared with two SCS cases.

Proposal 1: Consider the case involving three different SCS can be scheduled by a DCI format 0_3/1_3 in Rel-19.

- Case 7: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR1 licensed FDD cell(s) with SCS1, TDD cell(s) with SCS2 and FR2-1 cell(s) with SCS3.

  - SCS1, SCS2 and SCS3 can be same or different.

HARQ-ACK feedback timing

In RAN1#120, the agreement on HARQ-ACK feedback timing was reached as below. The remaining issue is whether there is RAN1 spec impact or not for slot-based HARQ-ACK feedback.

Agreement - For determining the timing of a PUCCH carrying HARQ-ACK information corresponding to a set of co-scheduled PDSCHs by a DCI format 1_3, follow Rel-18 operation. - If the UE is not provided subslotLengthForPUCCH, the DL slot 𝑛𝐷 is the DL slot ending last, amongst the DL slots where the more than one PDSCH are scheduled by the DCI format 1_3. - FFS: RAN1 spec impact till RAN1#120-bis - If the UE is provided subslotLengthForPUCCH, no spec change is necessary. - Note: Specification of this feature shall not impact the existing UE processing PDSCH timeline requirement for any individual PDSCH, as specified in 5.3.1 of TS38.214.

Based on the current spec, if more than one PDSCH end last among the set of co-scheduled PDSCHs, it seems the above agreement has been reflected by the current spec as analyzed below.

- For slot-based PUCCH, the PUCCH slot is determined based on the last UL slot overlapping with the DL slot of the PDSCH in the current spec. The last UL slot overlapping with the DL slot of the PDSCH is exactly overlapping with the DL slot ending last. It means that the DL slot ending last is used for PUCCH slot determination. As a result, the current spec is in line with the agreement. In addition, it should be noted this agreement follows the principle of Rel-18. For the agreement reached in Rel-18, there is no spec impact finally. There should be no spec impact for this similar agreement as well.

Current spec: If the UE is provided subslotLengthForPUCCH, n is the last UL slot for PUCCH transmission that overlaps with a PDSCH reception or with a PDCCH reception providing a DCI format having associated HARQ-ACK information without scheduling a PDSCH reception; otherwise, n is the last UL slot for PUCCH transmission that overlaps with the DL slot nD for the PDSCH reception or with the DL slot nD for the PDCCH reception in case of a DCI format that triggers a HARQ-ACK information report and does not schedule a PDSCH reception.

If the UE detects a DCI format scheduling a number of PDSCH receptions ending in DL slot nD or if the UE detects a DCI format generating a HARQ-ACK information bit and does not schedule a PDSCH reception through a PDCCH reception ending in DL slot nD, the UE provides corresponding HARQ-ACK information in a PUCCH transmission within UL slot n+k, where k is a number of slots and is indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACK-DCI-1-2, or dl-DataToUL-ACK-r17, or dl-DataToUL-ACK-DCI-1-2-r17, or dl-DataToUL-ACK-v1700.

Another issue arises when the PUCCH derived by the reference PDSCH is not available for another PDSCH that does not end last, because Tproc,1 derived by the reference PDSCH is less than that derived by the another PDSCH. Some optimizations can be applicable, i.e., modifying the definition of the reference PDSCH or employing the maximum Tproc,1 derived by each of the PDSCHs among the set of co-scheduled PDSCHs. However, this issue could also be addressed by gNB implementation with a proper K1 indication.

Proposal 2: For determining the timing of a PUCCH carrying HARQ-ACK information corresponding to a set of co-scheduled PDSCHs by a DCI format, there should be no spec impact.

Type-1 HARQ-ACK codebook

Based on the clarification in RAN#105 email reflector, the notes in the WID are applicable to both the 1st and 2nd bullets. However, the K1 extension and time domain HARQ-ACK bundling for Type-1 HARQ-ACK codebook were introduced for multi-PDSCH scheduling on a single cell in Rel-17. To avoid enhancing the Type-1 HARQ-ACK codebook for Rel-19 multi-cell scheduling, the similar restrictions made in Rel-18 multi-cell scheduling should also be applied to Rel-19 multi-cell scheduling. This is achieved by gNB implementation.

RAN1#112 Conclusion Type-1 HARQ-ACK codebook is supported for multi-cell scheduling without K1 extension. - UE expects HARQ-ACK information for all co-scheduled PDSCHs by DCI format 1_X can be mapped in the Type-1 HARQ-ACK codebook. - Type-1 HARQ-ACK codebook is not enhanced for Rel-18 multi-cell scheduling.

In RAN1#119, HARQ-ACK bundling in time domain has been agreed as below. This implies Type-1 HARQ-ACK codebook can also be configured with time domain HARQ-ACK bundling.

Agreement - Time domain HARQ-ACK bundling is configured per cell as Rel-17.

Based on this understanding, Type-1 HARQ-ACK codebook is indeed enhanced compared to that of Rel-18 multi-cell scheduling. As a result, K1 extension for Type-1 HARQ-ACK codebook shall be also supported for Rel-19 multi-cell scheduling with multi-PDSCH on one or more cells.

Proposal 3: Type-1 HARQ-ACK codebook is supported for multi-cell scheduling with K1 extension for Rel-19 multi-cell scheduling.

- UE expects HARQ-ACK information for all co-scheduled PDSCHs by DCI format 1_3 can be mapped in the Type-1 HARQ-ACK codebook.

Type-2 HARQ-ACK codebook

The working assumption on HARQ-ACK information bit for SCell dormancy indication is listed below.

Working Assumption - For a DCI format 1_3 with fields repurposed for SCell dormancy indication and scheduling one or more PDSCHs, if TDRA field indicates multiple SLIVs for the serving cell with smallest serving cell index with invalid FDRA, the HARQ-ACK information bit for SCell dormancy indication is ACK for the first SLIV and followed by NACK bits for the remaining SLIVs.

Time-domain HARQ-ACK bundling was also supported and the following agreements on Type-2 HARQ-ACK codebook are listed below.

RAN1#119 Agreement - Time-domain HARQ-ACK bundling is configured per cell as Rel-17. RAN1#119 Agreement - Type-2 HARQ-ACK codebook is generated by concatenating a first sub-codebook and a second sub-codebook. - The first sub-codebook comprises HARQ-ACK information bits for PDSCH(s) scheduled by DCI(s) with each scheduling a single PDSCH, or each scheduling a single cell with multiple PDSCHs on it and nrofHARQ-BundlingGroups configured as 1, and HARQ-ACK information bit(s) for DCI(s) having associated HARQ-ACK information without scheduling PDSCH reception. - The second sub-codebook comprises HARQ-ACK information bits for PDSCHs scheduled by DCI(s) with each scheduling more than one cell, or each scheduling a single cell with multiple PDSCHs on it without nrofHARQ-BundlingGroups or nrofHARQ-BundlingGroups configured larger than 1. - Separate DAI counting is applied for DCI(s) associated with the first sub-codebook and DCI(s) associated with the second sub-codebook as Rel-18. - Note: For providing HARQ-ACK information corresponding to SCell dormancy indication, the UE assumes that the UE receives a PDSCH on the serving cell associated with fields in DCI format 1_3 used for SCell dormancy indication as Rel-18.

In case Time-domain HARQ-ACK bundling is configured for the serving cell with smallest serving cell index with invalid FDRA, the working assumption is not correct. The HARQ-ACK information bit is NACK after bundling operation if ACK is assumed for the first SLIV and NACK bits are assumed for the remaining SLIVs. It leads to an invalid feedback. As a result, the working assumption should be updated as following to cover the case that time-domain HARQ-ACK bundling is configured.

Working Assumption - For a DCI format 1_3 with fields repurposed for SCell dormancy indication and scheduling one or more PDSCHs, if TDRA field indicates multiple SLIVs for the serving cell with smallest serving cell index with invalid FDRA and without nrofHARQ-BundlingGroups configured, the HARQ-ACK information bit for SCell dormancy indication is ACK for the first SLIV and followed by NACK bits for the remaining SLIVs. - For a DCI format 1_3 with fields repurposed for SCell dormancy indication and scheduling one or more PDSCHs, if TDRA field indicates multiple SLIVs for the serving cell with smallest serving cell index with invalid FDRA and with nrofHARQ-BundlingGroups configured, the HARQ-ACK information bit for SCell dormancy indication is ACK for the first TBG, and followed by NACK bits for the remaining TBGs, if any.

Proposal 4: Confirm the working assumption with the following updates.

Working Assumption - For a DCI format 1_3 with fields repurposed for SCell dormancy indication and scheduling one or more PDSCHs, if TDRA field indicates multiple SLIVs for the serving cell with smallest serving cell index with invalid FDRA and without nrofHARQ-BundlingGroups configured, the HARQ-ACK information bit for SCell dormancy indication is ACK for the first SLIV and followed by NACK bits for the remaining SLIVs. - For a DCI format 1_3 with fields repurposed for SCell dormancy indication and scheduling one or more PDSCHs, if TDRA field indicates multiple SLIVs for the serving cell with smallest serving cell index with invalid FDRA and with nrofHARQ-BundlingGroups configured, the HARQ-ACK information bit for SCell dormancy indication is ACK for the first TBG, and followed by NACK bits for the remaining TBGs, if any.

Maximum number of PUSCHs/PDSCHs

In RAN1#119, the agreements on the maximum number of PUSCHs/PDSCHs are listed below.

RAN1#119 Agreement - Specification supports the maximum number of PUSCHs/PDSCHs for a scheduled cell by a DCI format 0_3/1_3 is 8. - Payload size of a DCI format 0_3/1_3 exceeding 140 is not supported in Rel-19.

Based on the agreement, there could be up to 8*4=32 PUSCHs/PDSCHs scheduled by a DCI format 0_3/1_3. Since the size of second sub-codebook and/or the UE complexity will be affected by the total number of PUSCHs/PDSCHs scheduled by DCI, the maximum number of PUSCHs/PDSCHs scheduled per DCI can be configured or reported if needed. In addition to the maximum number of PUSCHs/PDSCHs scheduled per DCI, other restrictions such as one or multiple combination {X, N} reported by the UE can be also considered, wherein X is the maximum number of PUSCHs/PDSCHs per scheduled cell, N is the maximum number of cells supported in one cell set.

Proposal 5: The maximum number of PUSCHs/PDSCHs per DCI can be configured or reported if needed.

RRC parameters

- TDRA

Agreement - A single TDRA field in DCI format 0_3/1_3 indicates one row from a joint TDRA table. - Each row in the table contains only one TDRA index for each BWP of each cell within the set of cells. Each TDRA index points to one or multiple time domain resource allocations in the TDRA table applicable for multi-PUSCH/PDSCH scheduling by DCI format 0_3/1_3 for the corresponding cell.

Based on the agreement above, a single TDRA field in DCI format 0_3/1_3 indicates one row from a joint TDRA table. Each row in the table contains only one TDRA index for each BWP of each cell within the set of cells. Each TDRA index points to one or multiple time domain resource allocations in the TDRA table applicable for multi-PUSCH/PDSCH scheduling by DCI format 0_3/1_3 for the corresponding cell. As a result, the TDRA table applicable for multi-PUSCH/PDSCH scheduling by DCI format 0_3/1_3 should be introduced.

Proposal 6: Endorse the following RRC parameters for Multi-carrier enhancements for NR Phase 3:

| RAN2 Parent IE | Parameter name in the spec | New or existing? | Default value aspect Description | Value range | Per (UE, cell, TRP, …) | UE-specific or Cell-specific |
| --- | --- | --- | --- | --- | --- | --- |
| ServingCellConfig | TDRA-FieldIndexDCI-1-3 | Existing | Configure each row of the joint TDRA field table for DL scheduling via DCI format 1_3 containing the applicable TDRA field indexes for multiple BWPs/cells, where the TDRA index for a BWP of a cell points to a corresponding TDRA in the TDRA table applicable for DCI format 1-1 if pdsch-TimeDomainAllocationListForMultiPDSCH-1-3 is not configured, or in the TDRA table configured by pdsch-TimeDomainAllocationListForMultiPDSCH-1-3, the order of TDRA index in each row refers the BWP-Id for a cell and the order of cells in scheduledCellListDCI-1-3 (i.e., first TDRA index in a row is for the smallest BWP-Id that can be scheduled by the DCI format 1-3, as specified in TS 38.212 [17], of the first cell in scheduledCellListDCI-1-3, second TDRA index in a row is for the second smallest BWP-Id that can be scheduled by the DCI format 1-3, as specified in TS 38.212 [17], of the first cell and so on ), and the number of TDRA indices in a row of TDRA-FieldIndexDCI-1-3 should be the same as the total number of BWPs that can be scheduled by the DCI format 1-3, as specified in TS 38.212 [17], across cells included in scheduledCellListDCI-1-3. | SEQUENCE (SIZE (2..16)) OF INTEGER (0..maxNrofDL-Allocations-1) | per set of cells (TDRA-FieldIndexListDCI-1-3) | UE-specific |
| PDSCH-Config | pdsch-TimeDomainAllocationListForMultiPDSCH-1-3 | New | Time domain resource allocations table is only used for multi-PDSCH scheduling by DCI format 1_3. The TDRA index indicated by the DCI format 1_3 points to one or multiple time domain resource allocations in this TDRA table | MultiPDSCH-TDRA-r17 | per BWP | UE-specific |
| ServingCellConfig | TDRA-FieldIndexDCI-0-3 | Existing | Configure each row of the joint TDRA field table for UL scheduling via DCI format 0_3 containing the applicable TDRA field indexes for multiple BWPs/cells, where the TDRA index for a BWP of a cell points to a corresponding TDRA in the TDRA table applicable for DCI format 0_1 if pusch-TimeDomainAllocationListForMultiPUSCH-0-3 is not configured, or in the TDRA table configured by pusch-TimeDomainAllocationListForMultiPUSCH-0-3, the order of TDRA index in each row refers the BWP-Id for a cell and the order of cells in scheduledCellListDCI-0-3 (i.e., first TDRA index in a row is for the smallest BWP-Id that can be scheduled by the DCI format 0_3, as specified in TS 38.212 [17], of the first cell in scheduledCellListDCI-0-3, second TDRA index in a row is for the second smallest BWP-Id 1 that can be scheduled by the DCI format 0_3, as specified in TS 38.212 [17], of the first cell and so on), and the number of TDRA indices in a row of TDRA-FieldIndexDCI-0-3 should be the same as the total number of BWPs that can be scheduled by the DCI format 0_3, as specified in TS 38.212 [17], across cells included in scheduledCellListDCI-0-3. | SEQUENCE (SIZE (2..16)) OF INTEGER (0..maxNrofUL-Allocations-r16-1) | per set of cells (TDRA-FieldIndexListDCI-0-3) | UE-specific |
| PUSCH-Config | pusch-TimeDomainAllocationListForMultiPUSCH-0-3 | New | Configuration of the time domain resource allocation (TDRA) table only for multi-PUSCH scheduling by DCI format 0_3. The TDRA index indicated by the DCI format 0_3 points to one or multiple time domain resource allocations in this TDRA table. | PUSCH-TimeDomainResourceAllocationList-r16 | per BWP | UE-specific |

Conclusion

Proposal 1: Consider the case involving three different SCS can be scheduled by a DCI format 0_3/1_3 in Rel-19.

- Case 7: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR1 licensed FDD cell(s) with SCS1, TDD cell(s) with SCS2 and FR2-1 cell(s) with SCS3.

  - SCS1, SCS2 and SCS3 can be same or different.

Proposal 2: For determining the timing of a PUCCH carrying HARQ-ACK information corresponding to a set of co-scheduled PDSCHs by a DCI format, there should be no spec impact.

Proposal 3: Type-1 HARQ-ACK codebook is supported for multi-cell scheduling with K1 extension for Rel-19 multi-cell scheduling.

- UE expects HARQ-ACK information for all co-scheduled PDSCHs by DCI format 1_3 can be mapped in the Type-1 HARQ-ACK codebook.

Proposal 4: Confirm the working assumption with the following updates.

Working Assumption - For a DCI format 1_3 with fields repurposed for SCell dormancy indication and scheduling one or more PDSCHs, if TDRA field indicates multiple SLIVs for the serving cell with smallest serving cell index with invalid FDRA and without nrofHARQ-BundlingGroups configured, the HARQ-ACK information bit for SCell dormancy indication is ACK for the first SLIV and followed by NACK bits for the remaining SLIVs. - For a DCI format 1_3 with fields repurposed for SCell dormancy indication and scheduling one or more PDSCHs, if TDRA field indicates multiple SLIVs for the serving cell with smallest serving cell index with invalid FDRA and with nrofHARQ-BundlingGroups configured, the HARQ-ACK information bit for SCell dormancy indication is ACK for the first TBG, and followed by NACK bits for the remaining TBGs, if any.

Proposal 5: The maximum number of PUSCHs/PDSCHs per DCI can be configured or reported if needed.

Proposal 6: Endorse the following RRC parameters for Multi-carrier enhancements for NR Phase 3:

| RAN2 Parent IE | Parameter name in the spec | New or existing? | Default value aspect Description | Value range | Per (UE, cell, TRP, …) | UE-specific or Cell-specific |
| --- | --- | --- | --- | --- | --- | --- |
| ServingCellConfig | TDRA-FieldIndexDCI-1-3 | Existing | Configure each row of the joint TDRA field table for DL scheduling via DCI format 1_3 containing the applicable TDRA field indexes for multiple BWPs/cells, where the TDRA index for a BWP of a cell points to a corresponding TDRA in the TDRA table applicable for DCI format 1-1 if pdsch-TimeDomainAllocationListForMultiPDSCH-1-3 is not configured, or in the TDRA table configured by pdsch-TimeDomainAllocationListForMultiPDSCH-1-3, the order of TDRA index in each row refers the BWP-Id for a cell and the order of cells in scheduledCellListDCI-1-3 (i.e., first TDRA index in a row is for the smallest BWP-Id that can be scheduled by the DCI format 1-3, as specified in TS 38.212 [17], of the first cell in scheduledCellListDCI-1-3, second TDRA index in a row is for the second smallest BWP-Id that can be scheduled by the DCI format 1-3, as specified in TS 38.212 [17], of the first cell and so on ), and the number of TDRA indices in a row of TDRA-FieldIndexDCI-1-3 should be the same as the total number of BWPs that can be scheduled by the DCI format 1-3, as specified in TS 38.212 [17], across cells included in scheduledCellListDCI-1-3. | SEQUENCE (SIZE (2..16)) OF INTEGER (0..maxNrofDL-Allocations-1) | per set of cells (TDRA-FieldIndexListDCI-1-3) | UE-specific |
| PDSCH-Config | pdsch-TimeDomainAllocationListForMultiPDSCH-1-3 | New | Time domain resource allocations table is only used for multi-PDSCH scheduling by DCI format 1_3. The TDRA index indicated by the DCI format 1_3 points to one or multiple time domain resource allocations in this TDRA table | MultiPDSCH-TDRA-r17 | per BWP | UE-specific |
| ServingCellConfig | TDRA-FieldIndexDCI-0-3 | Existing | Configure each row of the joint TDRA field table for UL scheduling via DCI format 0_3 containing the applicable TDRA field indexes for multiple BWPs/cells, where the TDRA index for a BWP of a cell points to a corresponding TDRA in the TDRA table applicable for DCI format 0_1 if pusch-TimeDomainAllocationListForMultiPUSCH-0-3 is not configured, or in the TDRA table configured by pusch-TimeDomainAllocationListForMultiPUSCH-0-3, the order of TDRA index in each row refers the BWP-Id for a cell and the order of cells in scheduledCellListDCI-0-3 (i.e., first TDRA index in a row is for the smallest BWP-Id that can be scheduled by the DCI format 0_3, as specified in TS 38.212 [17], of the first cell in scheduledCellListDCI-0-3, second TDRA index in a row is for the second smallest BWP-Id 1 that can be scheduled by the DCI format 0_3, as specified in TS 38.212 [17], of the first cell and so on), and the number of TDRA indices in a row of TDRA-FieldIndexDCI-0-3 should be the same as the total number of BWPs that can be scheduled by the DCI format 0_3, as specified in TS 38.212 [17], across cells included in scheduledCellListDCI-0-3. | SEQUENCE (SIZE (2..16)) OF INTEGER (0..maxNrofUL-Allocations-r16-1) | per set of cells (TDRA-FieldIndexListDCI-0-3) | UE-specific |
| PUSCH-Config | pusch-TimeDomainAllocationListForMultiPUSCH-0-3 | New | Configuration of the time domain resource allocation (TDRA) table only for multi-PUSCH scheduling by DCI format 0_3. The TDRA index indicated by the DCI format 0_3 points to one or multiple time domain resource allocations in this TDRA table. | PUSCH-TimeDomainResourceAllocationList-r16 | per BWP | UE-specific |

Reference

[1] RAN#106, RP-242904, Revised WID: Multi-carrier enhancements for NR Phase 3, Lenovo

[2] RAN#105, RP-242379, Way forward on new RAN1/RAN2/RAN3 led REL-19 proposals, RAN chair (Qualcomm)