3GPP TSG RAN WG1 #120bis	R1-2502008

Wuhan, China, 7th- 11th 2025

Agenda item:		9.6.2

Title:					LP-WUS operation in IDLE/Inactive mode

Source:				Nokia

Document for:		Discussion and Decision

# Introduction

In RAN#102 meeting, a new work item was agreed for Low Power Wake Up Signal and Receivers designs. These designs are to be primarily targeted at delay and power-sensitive, small form-factor devices, such as industrial sensors, controllers and wearables. Unlike previous power saving study items, the objectives for this study encompasses new signals and receiver architectures [1].

The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode. - Specify the necessary RAN4 core requirement(s) to support the feature (RAN4). - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance - Define guard RBs for ACS and ASCS cases - Study testability of above requirements - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS. - Current NR BS requirements is baseline - Specify necessary RRM requirements

In this contribution we provide our initial views related to the LP-WUS operation in IDLE/Inactive mode.

# RRM aspects

In following we discuss some aspects related to the LR measurement quantities.

## Consideration on LR measurements quantities

In RAN1#116bis meeting it was agreed to support LP-RSRP and LP-RSRQ for LR-based measurement quantities. Furthermore, following agreements have been made for measurement metrics:

Agreement For LP-SS based LP-RSRQ, LP-RSRP and LP-RSSI are measured within the same bandwidth. Agreement For LP-RSSI definition for LP-RSRQ, the following is agreed - Option 1: LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols. Note: Above does not constrain LP-SS sequence design for OOK Working Assumption From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively. - Above is applicable for both time-domain processing or frequency-domain processing - Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications Agreement The definitions of LP-RSRP and LP-RSSI for LP-RSRQ are updated as follows: LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols over the frequency resources defined by the number of REs that carry LP-SS. LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols over the frequency resources defined by the number of REs that carry LP-SS. Agreement Confirm the following working assumption with the additional text in red: Working Assumption From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively. - Above is applicable for both time-domain processing or frequency-domain processing - Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications Existing metrics SS-RSRP and SS-RSRQ are reused for OFDM-based LP-WUR. No separate metrics (LP-SSS-RSRP and LP-SSS-RSRQ) will be introduced in the specifications. - The metrics SS-RSRP and SS-RSRQ are applicable for OFDM-based LP-WUR for RRC_IDLE and RRC_INACTIVE serving cell measurement. - FFS: whether to remove or modify the restriction of using SMTC window for LP-SSS-RSRP/RSSI measurement.

It was also agreed in last meeting that there should be one to one association between LP-SS and SSB (and LP-WUS) and that there would be 4 OOK sequences for LP-SS.

As discussed on our companion paper [3], the accuracy for the LP-RSRP, may depend on LP-SS design. Firstly, if Manchester encoding is not applied, the power on ON durations may vary (for M>1) as OFDM symbols can have varying number of ONs and OFFs durations. In order to have consistent RSRP reports, this should be accounted in the receiver to have constant RSRP estimates regardless of the applied sequence. Effectively this would mean scaling ON duration energy depending on the number of ON durations in one OFDM symbol. However, this could lead to degraded SNR for the measurement (noise amplification). Thus, how and whether this needs to be accounted would need to be considered after finalization of LP-SS design. Also, depending on the approach and binary sequences selected for the LP-SS, the total number of ONs and OFFs OOK symbols could be unequal. For consistent reports, there could be need to consider this impact also in the measurement metric definition, at least if the difference can be large.

Observation: If Manchester encoding is not applied to the LP-SS and depending on the binary sequence selection for LP-SS, the current definitions might lead to inconsistent results for LP-RSRQ and for LP-RSRQ between different LP-SS sequences

Proposal: Once LP-SS sequence design has been finalized, verify if the LP-RSRP and LP-RSRQ metric definitions need to be revisited.

For NR measurements, e,g For SS-RSRP, it is defined that SS-RSRP is SSB index specific, i.e. measured quantity is associated to a particular beam:

SS-RSRP shall be measured only among the reference signals corresponding to SS/PBCH blocks with the same SS/PBCH block index and the same physical-layer cell identity.

Correspondingly similar approach should be considered also for LP-SS measurements, so that measurement of LP-RSRP including e.g. averaging over multiple LP-SS bursts separated e.g. by 320ms, should be carried over LP-SS signals associated to same beam.

Proposal: LP-RSRP and LP-RSRQ measurements should be associated to a particular beam.

It has been discussed in past meetings, whether LP-SS and LP-WUS need to be present in all the beams, and whether UE can be directly informed about it. In scope of ED-LR that is measuring LP-SS, it could be considered that UE can be configured which LP-SS beams to consider for measurements. This is similar as for SS-RSRP, where based on network configuration select sub-set of SSBs to be used for SS-RSRP measurement:

If SS-RSRP is not used for L1-RSRP and higher-layers indicate certain SS/PBCH blocks for performing SS-RSRP measurements, then SS-RSRP is measured only from the indicated set of SS/PBCH block(s).

Proposal: For LP-RSRP and LP-RSRQ measurements, it should be possible for the network to configure a sub-set of LP-SS’s from which LP-RSRP and LP-RSRQ can be measured, similarly as done for SSBs.

Furthermore, it has been discussed whether for SS-based measurements for LR there is need to limit/restrict the measurements occasions within the SMTC window. Now as these measurements for LR are intended for serving cell evaluation, it would not seem necessary to apply the SMTC restriction to these measurements. The RSSI could be limited to symbols of the SSB.

Observation: As the intent of the LR based measurements is serving cell evaluation, it would not seem necessary to restrict the measurements within SMTC.

Note, that it is also assumed that while LRs processing the signal in time domain can have different sampling rates, the requirements would be defined so that consistent UE reporting is achieved. I.e. UEs correct any bias caused by the applied L1 sampling rate.

Observation: It is expected that performance requirements will ensure consistent reporting from different UEs.

# LP-WUS procedures and content in IDLE/inactive mode

In this section we consider the LP-WUS related configurations and content.

## Aspects related to LP-WUS monitoring configuration

WID objectives determine that duty-cycled LR operation needs to be supported, thus, there would be a need to be able to configure UE with LP-WUS monitoring window. In this section we consider the aspects related to the configuration of LP-WUS monitoring to trigger the paging monitoring in IDLE/Inactive mode.

In RAN1#116 meeting following agreement was made for the definition of LP-WUS monitoring configuration to facilitate the discussion:

Agreement Multi-beam operations are supported for LP-WUS and LP-SS for idle mode Agreement LP-WUS occasions (LOs) are defined for LP-WUS monitoring. - Each LO has one or more LP-WUS monitoring occasions (MOs), where UE can monitors for LP-WUS transmission in each of the LP-WUS MOs. - Different LP-WUS MOs may correspond to different beams in multi-beam operation - It is not precluded that FFS whether or not each LO is defined as a time window that covers the corresponding LP-WUS MOs - FFS details - It is at least supported that a UE monitors LOs with a configured periodicity. - Each UE has a periodicity for LO monitoring, and it is at least supported that a UE monitors one LO per period. - FFS: A UE does not expect its LP-WUS monitoring occasions overlapping in time - FFS: monitoring of multiple more than one LOs per period e.g. if LP-WUS common to all UEs is supported or in case of eDRX (if supported) - FFS eDRX, if supported

Further details were agreed in RAN1#116bis:

Agreement For multi-beam operation of LP-WUS, UE assumes the same LP-WUS information payload is repeated in all transmitted beams corresponding to LP-WUS - the selection of the beam(s) for the reception of the LP-WUS is up to UE implementation Agreement Each LO consists of N * K LP-WUS MOs, where N is the number of beams corresponding to LP-WUS, and K is the number of LP-WUS MOs for each beam. - Option 1: K = 1 - Option 2: K can be larger than or equal to 1 - FFS if more than 1 LP-WUS is transmitted from the same beam, whether the information in these multiple LP-WUS is always the same or can be different

In RAN1#118 it was agreed that:

Agreement On the LO configuration for iDRX, offset value(s) between a LO and a reference PO/PF is/are configured. - FFS: one or multiple offset values, and if multiple offset values are supported, how a UE decides which value to use - FFS: the exact definition of the reference PF/PO and the detailed procedure for UE to determine the LO(s) corresponding to Option 1, 2, or 3 (if supported) - (Working Assumption) For each UE, the periodicity of LO is the same as its iDRX cycle. - If a UE receives a wake-up indication in a LP-WUS, consider the following alternatives - Alt 1: it monitors the PO associated with the offset. - Alt 2: it monitors the first PO after its reported wake-up delay. - Other alternatives are not precluded Note: The PO mentioned above refers to legacy PO configured for the UE.

Agreement For the offset value(s) between an LO and a reference PO/PF, at least a frame-level offset is provided. - The reference point (reference PO/PF) for the frame-level offset is the start of the PF, or the first PF of the PF(s) (if mapping of POs from multiple PFs to one LO is supported), associated with the LO. - FFS other offset value(s) to determine the MOs of the LO Agreement For the LO to PO mapping from network perspective, support Option 2 (UEs corresponding to different POs monitor the same LO). - This should not increase the maximum number of codepoints per LO/LP-WUS compared to Option 1. - FFS conditions/restrictions for mapping multiple POs to one LO - Down-select between 2 and 4 for max number of POs per LO. Agreement For the offset value(s) between an LO and a reference PO/PF, adopt Option 2B-1. - gNB can configure 1 or 2 offset values. - FFS whether gNB can configure 3 offset values - If multiple offset values are configured and if the gap between the LO associated with the largest offset value and the corresponding PO is no less than the wake-up delay a UE reports, the UE monitors the LO associated with the smallest offset value that has a gap between the LO and the PO no less than the wake-up delay. - Note: if a single offset value is configured, UE behaviour is according to Option 1-1. - All the UEs supporting LP-WUS for idle/inactive mode supports the configuration of 2 offset values (FFS: 3 values).

In Rel-17 PEI, the monitoring window is determined in relation to the PF. For Rel-17 UE is configured with ‘pei-FrameOffset’ giving the offset between the paging frame and PEI-frame, and with ‘firstPDCCH-MonitoringOccassionOfPEI-O’ determining the start of the PDCCH monitoring occasions where UE will monitor the DCI format 2_7 in search space defined by pei-SearchSpace. The duration of the PEI monitoring window is set to cover one PDCCH monitoring occasion per SSBs for NR.

Figure 1: Illustration of Rel-17 PEI monitoring configuration.

Hence, for LP-WUS monitoring it would be necessary to be able configure the time location and duration of the LP-WUS monitoring window. The time location of this monitoring window would be affected by the minimum distance from the paging occasion to be monitored by MR, and, possibly, the maximum distance for LP-SS. Furthermore, the duration of the LP-WUS monitoring window would be affected by the number of SSBs, assuming at least one LP-WUS occasion per SSB.

It was agreed in last meeting that a frame level offset from the reference PF is configured. This offset would be determined as a frame level offset. To complete the configuration, additional offsets would be needed to adjust the start of the LO and/or the MO(s) in LO. This would be necessary to account for example TDD UL-DL slot configuration and need to avoid some slots carrying for example SSBs.

Observation: Additional offset(s) are needed to indicate the LP-WUS MO location(s).

In principle there are few approaches how the configuration can be achieved:

- Define for each LP-WUS MO individual offset from directly from reference point given by the PF-LO offset agreed in last meeting.

  - Depending on the selected approach and LP-WUS length, this may need to be symbol or slot level offset

  - Slot level offset may suffice if candidate LP-WUS locations in a slot are predetermined for example similarly as done for SSBs.

  - LP-WUS MO specific individual symbol level offset may need have rather large range to accommodate e.g. 4 MOs that are spread over multiple slots.

  - One approach to limit the required value range of LP-WUS MO specific offset could be to ‘concatenate’ offsets so that reference/starting point for 2nd MO offset is the start of the 1st MO etc. MO specific offsets could be of different size (first being the largest).

- Define an additional LP-WUS MO/LO common offset from reference point given by the PF-LO offset agreed in last meeting, to determine the start reference for the MOs of a LO. From the reference point define a offset(s) for the individual MOs

  - The frame level offset, agreed in last meeting, provides a first reference point.

  - Second offset in slot or frame level, determines a second reference point for the LP-WUS MO(s).

  - From this second reference point, individual MOs are configured.

As noted above, there the offset to LP-WUS MO could be defined in symbol level or in slot level, if LP-WUS candidate occasions inside a slot are predefined defined and indexed (as for SSB). As there would be a need to define this pattern for each M value, it may be simples and most flexible to define this in symbol level. Whether this is done directly from the reference point given by the PF-LO offset agreed in last meeting, or after/from on another LP-WUS common offset, could be considered.

Proposal: LP-WUS MO should be configured based on starting symbol based on symbol level offset from a reference. This offset could be independent or concatenated so that earlier LP-WUS MO starting symbol is a reference for the latter.

To restrict the size of the afore noted symbol level offset for the MO(s), it would be beneficial to define another LP-WUS MO common offset, in addition to the PF-LO offset, that could be on slot level of ms level.

Proposal: Define additional common offset for LP-WUS MO(s) of a LO to enable reducing the size of the LP-WUS MO symbol level offset. This offset could be defined in a slot level or ms level.

RAN1 also send a LS to RAN4 [5] regarding the wake-up delay values and the possible dependency of the values on the applied SSB periodicity. Depending on the RAN4 feedback, the value range for the PF-LO offset may need to be extended or be dependent on the SSB periodicity applied in the cell. E.g. as the first paging PDCCH MO configuration (firstPDCCH-MonitoringOccasionOfPO) value range is dependent on the applied sub-carrier spacing.

Observation: To reduce the range of values needed for the PF-LO offset, making the applied value range dependent on the SSB periodicity could be considered, similarly as for the configuration of first paging PDCCH monitoring occasion is dependent on the applied sub-carrier spacing.

As discussed in our paper [2], for non-coherent receiver the gain from repetitions can be very limited and more benefit can be obtained through channel encoding with increased redundancy. Therefore it would seem best that it is first understood what is the LP-WUS baseline performance, i.e. can codepoint be determined via encoding and thereby UE allow to have an option to use decoding in reception to enhance the detection performance. If encoding based codepoint determination is supported, consider repetitions for increasing the redundancy for the LP-WUS reception.

Observation: Gain from repetitions for non-coherent receivers is very limited.

Observation: Enabling codepoint determination via encoding can provide benefit and allow use of repetitions for incremental redundancy

Proposal: Consider repetitions only if they can provide significant benefit for the detection performance e.g. via increased redundancy.

It was also discussed whether configuring two LOs (offsets) per PO suffices or whether three LOs would be preferable. As these relate to enabling UEs with different wake-up capabilities to use LP-WUS, it would seem that if network can configure offset to support the longest wake-up delay (i.e. [900ms]), there is no implicit need for the middle wake-up delay (i.e. [500ms]), as the UEs supporting the shorter wake-up delay can always monitor the LO with larger offset.

Observation: It does not seem necessary to support more than two possible LO configurations/offsets per PO.

Proposal: Support a maximum of two possible LO configurations/offsets per PO.

## LP-WUS, LP-SS and SSB association

It was agreed in past meeting that there shall be one to one association between LP-SS, LP-WUS and SSB beams. As raised earlier this would be a pre-requisite for the UE to be able to select LP-WUS beams to be monitored, agreed earlier.  It was also agreed that there will be a QCL relation between the LP-WUS/LP-SS and SSB.

Agreement At least support 1:1 association between LP-WUS MO(s)/LP-SS transmissions and SSB beams. Agreement Each LP-WUS is QCLed with one SSB. Each LP-SS is QCLed with one SSB. - FFS QCL Type A or Type C and/or Type D - FFS implicit QCL determination or some signaling is required Agreement The number of beams for LP-SS is the same as the number of beams for the LP-WUS MOs in an LO. Agreement At least support the case that the number of beams for LP-WUS/LP-SS is the same as the number of SSB beams. Conclusion Do not support one-to-multiple mapping between the LP-WUS/LP-SS beams and the SSB beams. Conclusion No additional RAN1 specification impact specific to support multiple-to-one mapping between the LP-WUS/LP-SS beams and the SSB beams. Agreement The previous agreement in RAN1#119 is updated as follows: Each LP-WUS or LP-SS is QCLed with an SSB with [select one from ‘typeA’, ‘typeC’], and when applicable, ‘typeD’ with the same SSB Agreement The EPRE ratio between LP-WUS/LP-SS and SSB can be configured by the gNB. - FFS whether there is a common configuration or separate configuration for LP-WUS and LP-SS - Above is not applicable for the case when all the subcarriers for LP-WUS/LP-SS are transmitted with zero power (from baseband perspective) - Above does not mandate any UE implementation for different receiver types - FFS: whether/how to support for the case when there are different number of OOK ON symbols in different OFDM symbols (if supported)

It was agreed in RAN1#116bis that the LP-SS is a binary sequence that can be configured and/or defined by a rule e.g. based on cell-ID to differentiate different cells (under ag. 9.6.1). In RAN1#118 it was further agreed that the number of binary sequences is 4. This effectively means that LP-SS would not be able to carry any beam indication directly. Hence it would necessary to use time location of LP-SS to associated with specific SSB beam.

Observation: UE should be able to identify the LP-SS beam association with specific SSB index via time location association.

It was agreed in RAN1#116bis that in one LO, there will be one LP-WUS MO for each beam (i.e. ‘N * K LP-WUS MOs, where N is the number of beams corresponding to LP-WUS’). To complete the UE beam selection, also LP-WUS MO association would need to be determined. If the assumption is that UE can obtain power saving from the LP-WUS beam selection, the LP-WUS MOs (in time) would need to be associated in LP-SS/SSB beams.

Observation: To close the loop in UE LP-WUS beam selection, LP-WUS MOs association to LP-SS/SSB beam should be identifiable via time location association.

Proposal: Define that there is one-to-one association between LP-SS and LP-WUS MO time locations and SSB beams. This can be done as logical indexing.

It was also agreed in last meeting that EPRE ratio between LP-WUS/LP-SS and SSB can be configured by network. It was to be further discussed whether separate offset is needed for each, LP-SS and LP-WUS. As LP-SS can provide sufficient synchronisation accuracy for LP-WUS detection in same power levels, it would not seem necessary to have separate offset for LP-SS and LP-WUS.

Observation: As the LP-SS can provide sufficient synchronisation accuracy for LP-WUS detection in equal power levels, it does not seem necessary to have separate EPRE offsets for LP-WUS and LP-SS.

## LP-WUS content

The LP-WUS channel design is discussed under agenda item 9.6.1 and in our contribution [2]. In this section we consider the information content related assumptions and requirements that can be made from IDLE/Inactive mode perspective.

Now the WID objective defines that sub-grouping is to be supported for the IDLE/Inactive mode operation. This results direct question for the number of sub-groups that would need to be supported per PO. In Rel-17 PEI operation the number of sub-groups per PO can be configured (subgroupsNumPerPO) from 1 to 8. It was concluded in RAN1#116bis that the mapping of UE ID to sub-group ID is left to RAN2:

Conclusion For idle/inactive mode, how to map a UE to a subgroup ID for LP-WUS is left to RAN2 to decide.

It was agreed in RAN1#120 that the maximum number of sub-groups per PO is 31:

Agreement Confirm the following working assumption with the modification: Working Assumption The maximum number of subgroups per PO supported in Rel-19 is 31 32.

This enables fitting the needed information (sub-group indication plus ‘wake-up all’ indication) to 5 bits. It was also agreed that at least two POs can map to one LP-WUS, but that the number of sub-groups per LP-WUS should not increase compared to case when only one PO is mapped to LP-WUS. Hence, the information in case of mapping more than one PO to one LP-WUS should be contained in 5 bits. It was not agreed how the information content would be split between PO(s). The simplest way to consider the split would be to have the information content split between POs equal and identical, so that each PO will have same number of sub-groups and separate ‘wake-up all’ indication. This would allow lower FAR than assigning common ‘wake-up all’ indication. Also as UEs would be expected to be equally split over POs, having different number of sub-groups in different POs does not seem necessary.

Proposal: If more than one PO is mapped to LP-WUS, the information content is identical and equal, so that all POs will have same number of sub-group indications and their own individual ‘wake-up all’ indication.

It was also discussed in last meeting, whether to support mapping max 2 or 4 POs to one LO. It is good to note that it has been discussed under Ag 9.6.1 whether the number of LP-WUS information bits is configurable, e.g. {3,4,5}. Thus if LP-WUS information bit size is configurable, the number of codepoints(/sub-groups) per PO could be varied, e.g. between 8 to 32. Hence, from this perspective supporting up to 4 PO per LO does not seem unviable.

Proposal: Support configurable mapping of maximum of 4 POs to one LO.

## Other aspects of LP-SS and LP-WUS configuration

As discussed in section 3.1, LP-WUS monitoring configuration needs to be provided to the UE. Correspondingly the time domain configuration would need to be provided for LP-SS also. For this it can be assumed that UE has knowledge of the cell timing and associated sub-carrier spacing. Effectively , for LP-SS, it would be expected that UE would be provided with a periodicity and starting time location for the LP-SS. The start time should contain at least SFN but depending on the approach also an additional, e.g. slot level offset may be needed. If the time domain pattern of the LP-SS is fixed (as for SSB) it would not need to be separately provided, but if LP-SS time locations can be configured by network, this information would need to be separately provided. As different M values need to be considered, it might be clearest to consider possibility to configure the starting symbol for each LP-SS. In scope of the IDLE/Inactive mode, one option to deliver this information to UE would be via broadcast, but the details can be left for RAN2.

Observation: Both LP-WUS and LP-SS time and frequency resources need to be provided for the IDLE/Inactive mode UE. Delivery method is up to RAN2.

Observation: For LP-SS the time domain configuration would need to contain periodicity and start time (SFN) possibly with additional e.g. slot level offset. Furthermore, fixed time pattern locations for LP-SS corresponding to SSB would be needed.

Proposal: For LP-SS a reference time can be configured through (sub-frame or radio frame level) offset and periodicity. If only one periodicity is supported, it can be fixed in specification.

Proposal: Support configuration of the start time for each LP-SS (corresponding to each SSB) at symbol level granularity. The reference time for the symbol level offset could be directly the LP-SS reference time.

In addition to time domain configuration, UE needs also to be provided information for the frequency domain physical resources for the LP-WUS and LP-SS. As for time location definition, it can be assumed that UE as already acquired necessary information regarding the frequency domain occupancy of the serving cell. Considering the frequency domain resource configuration, as UE as acquired access to the cell, the LP-WUS and LP-SS could be configured on the common resource grid based on sub-carrier spacing applied for the LP-WUS. I.e. the frequency location would be configured based on the first common RB in corresponding scsCarrier. This would follow the configuration done for example for the CSI-RS. Other possible approach would be to configure the location directly based on PointA. Furthermore one approach would be to configure the LP-WUS and LP-SS location in relation to the active BWP e.g. as for CORESET.

Observation: Frequency location of LP-WUS and LP-SS can be determined using existing frequency resource grids/references, such as common resource grid.

Proposal: Supporting configuration of starting PRB on common resource block grid for LP-SS and LP-WUS.

Related to this, it would need to be discussed how the LP-SS/LP-WUS need to be located in relation to the UE active BWP (e.g. initial BWP). Additional aspect is, whether LP-WUS and LP-SS need to be on (approximately) on same frequency resources or whether they can be configured independently of each other. From LR perspective, different resources being in non-overlapping resources can require the UE to change the centre frequency of the LR, thus fully independent and non-overlapping configuration may not be attractive. Furthermore, if LR is able to detect SSB, it would need to be discussed whether there are some restrictions the frequency location relation of LP-WUS and SSB.

Observation: For frequency domain configuration, it would be beneficial to agree whether LP-SS/LP-WUS configuration needs to have some relation to UE active BWP and in relation to each other and/or SSB.

In RAN1#120 it was agreed for CONNECTED mode that LP-WUS needs to be contained within the active BWP. From IDLE/Inactive mode operation, there used active BWP would be the initial BWP, and it could be differently configure for different UE types, i.e. non-RedCap and RedCap UEs. Thus, if the LP-WUS/LP-SS would be required to be contained within the active initial BWP, the LP-WUS/LP-SS would configuration would either need to be duplicated, or the initial BWP configurations would need to be so that they both cover the given LP-WUS/LP-SS configuration. Hence, it may not be preferable to require these to be contained (fully) within the active initial DL BWP. As noted above, it could be reasonable to consider some maximum frequency distance between the LP-WUS and SSB at least for the IQ-LR so that it is able to measure SSB and monitor LP-WUS without requiring to do a large change in the LR centre-frequency. If no change (in LR centre frequency) is assumed, the LP-WUS and SSB would need to be in practices fully overlapping. If SSB and LP-WUS would be too far apart, it could be preferable to assume that in such cases IQ-LR would also use/require LP-SS for tracking and synchronisation.

Observation: For placement of LP-WUS and SSB for IQ-LR, two cases could be considered: if SSB and LP-WUS are sufficiently close, SSB can be used for tracking, ad if SSB is further apart, IQ-LR could use LP-SS.

Proposal: Consider the maximum frequency separation between SSB and LP-WUS that can be supported by IQ-LR.

Also for the ED-LR having LP-SS and SSB not too far apart could help to ensure that the observed radio conditions would be more aligned when considering e.g. entry/exit evaluation. However this is not as stringent requirement as above noted for IQ-LR, and could be left for network configuration.

Observation: For placement of LP-SS and SSB for ED-LR, as ED-LR does not need to consider SSB, there does not appear to be strong reason to restrict the relative placement in frequency domain.

# Other aspects for UE power saving in IDLE/Inactive mode

## False alarm cost reduction

As noted in study item phase, the key aspect driving the UE power consumption for IDLE/Inactive mode operation, is the MR transition energy cost. Hence, if UE is repeatedly woken up by LP-WUS to monitor paging with MR, the power consumption benefits are negatively impacted. While this mostly relates to number of paging occasions (PO) and paging frames and possibility to indicate sub-groups when triggering paging monitoring to reduce the false alarms, this also relates to the paging load. In many scenarios the paging load is not evenly distributed and in certain times of day the paging activity may significantly increase from the normal/average. In such situations the paging message size (due to large number of UE IDs) may result that there will be paging in successive paging occasions, increasing the probability with LP-WUS operation that MR would need to be triggered to monitor paging. This would result increased UE power consumption, due to frequent MR transitions from ultra-deep sleep to normal active mode. Therefore it would be useful to consider methods to make UE aware whether it should expect further paging for the corresponding PO in following paging cycle.

For NR-U, new short message based indication was introduced, as shown in Table 1, for informing the UE that UE may cease paging monitoring. As in shared spectrum operation the number of PDCCH monitoring occasions per SSB can be increased (to overcome LBT), this indication may shorten the PDCCH monitoring time. For operation with LP-WUS operation, especially with eDRX, adopting some method to assist the UE to avoid unnecessary MR transitions could be considered. Using the NR-U based indication or similar indication via short message to inform UE whether paging monitoring can be stopped, could allow the UE to know whether it needs to monitor the next PO. Other alternatives could also be considered, e.g. based on UE implicitly on paging message size or number of paging records whether the same paging message will be repeated, or directly indication that next message will be a repetition.

Table 1: Short Messages [TS38.331]

| Bit | Short Message |
| --- | --- |
| 1 | systemInfoModification If set to 1: indication of a BCCH modification other than SIB6, SIB7, SIB8 and posSIBs. |
| 2 | etwsAndCmasIndication If set to 1: indication of an ETWS primary notification and/or an ETWS secondary notification and/or a CMAS notification. |
| 3 | stopPagingMonitoring This bit can be used for only operation with shared spectrum channel access and if nrofPDCCH-MonitoringOccasionPerSSB-InPO is present. If set to 1: indication that the UE may stop monitoring PDCCH occasion(s) for paging in this Paging Occasion as specified in TS 38.304 [20], clause 7.1. |
| 4 | systemInfoModification-eDRX If set to 1: indication of a BCCH modification other than SIB6, SIB7, SIB8 and posSIBs. This indication applies only to UEs using IDLE eDRX cycle longer than the BCCH modification period. |
| 5 – 8 | Not used in this release of the specification, and shall be ignored by UE if received. |

Observation: To reduce the energy cost of repeated paging monitoring due to MR wake-ups, UE should be provided assistance information whether it should expect further paging in following paging occasion.

## eDRX support for LP-WUS

It has also been discussed in past meetings whether and how the eDRX should be accounted in LP-WUS operation/configuration. In legacy eDRX operation, UE wakes up every eDRX cycle to monitor paging during the PTW. As considered in the evaluations, the paging probability is not even during the PTW, thus depending on the overall paging load and applied eDRX cycle, there can be some congestion at the start of the PTW. Paging message can of course arrive to RAN during the PTW, thus normal IDLE mode type of operation would also need to be supported during PTW. Hence, from system perspective, in minimum it should be possible to page the UE like with normal eDRX operation during the PTW. Therefore it would seem that most straight forward LP-WUS operation with eDRX would be that if UE is configured with LP-WUS, UE can monitor the LP-WUS before each paging occasion for the PTW. This would be aligned with the Rel-17 PEI operation.

Proposal: If LP-WS operation with eDRX is supported and UE is configured with eDRX and LP-WUS, the LP-WUS monitoring is determined based on paging occasions so that UE can monitor LP-WUS associated to the POs that are within the PTW.

# Conclusion

In this contribution we have discussed aspects related to the LP-WUS operation in IDLE/Inactive mode.

In Section 2.1 we discuss the LR measurement quantities, with following observation and proposals:-

Observation: If Manchester encoding is not applied to the LP-SS and depending on the binary sequence selection for LP-SS, the current definitions might lead to inconsistent results for LP-RSRQ and for LP-RSRQ between different LP-SS sequences

Proposal: Once LP-SS sequence design has been finalized, verify if the LP-RSRP and LP-RSRQ metric definitions need to be revisited.

Proposal: LP-RSRP and LP-RSRQ measurements should be associated to a particular beam.

Proposal: For LP-RSRP and LP-RSRQ measurements, it should be possible for theto  network to configure a sub-set of LP-SS’s from which LP-RSRP and LP-RSRQ can be measured, similarly as done for SSBs.

Observation: As the intent of the LR based measurements is serving cell evaluation, it would not seem necessary to restrict the measurements within SMTC.

Observation: It is expected that performance requirements will ensure consistent reporting from different UEs.

In Section 3.1 we consider the aspects related to the LP-WUS monitoring configuration:-

Observation: Additional offset(s) are needed to indicate the LP-WUS MO location(s).

Proposal: LP-WUS MO should be configured based on starting symbol based on symbol level offset from a reference. This offset could be independent or concatenated so that earlier LP-WUS MO starting symbol is a reference for the latter.

Proposal: Define additional common offset for LP-WUS MO(s) of a LO to enable reducing the size of the LP-WUS MO symbol level offset. This offset could be defined in a slot level or ms level.

Observation: To reduce the range needed for the PF-LO offset, making the applied value range dependent on the SSB periodicity could be considered, similarly as for the configuration of first paging PDCCH monitoring occasion is dependent on the applied sub-carrier spacing.

Observation: Gain from repetitions for non-coherent receivers is very limited.

Observation: Enabling codepoint determination via encoding can provide benefit and allow use of repetitions for incremental redundancy

Proposal: Consider repetitions only if they can provide significant benefit for the detection performance e.g. via increased redundancy.

Observation: It does not seem necessary to support more than two possible LO configurations/offsets per PO.

Proposal: Support a maximum of two possible LO configurations/offsets per PO.

Section 3.2 considers the issues related to the determination of beams association:-

Observation: UE should be able to identify the LP-SS beam association with specific SSB index via time location association.

Observation: To close the loop in UE LP-WUS beam selection, LP-WUS MOs association to LP-SS/SSB beam should be identifiable via time location association.

Proposal: Define that there is one-to-one association between LP-SS and LP-WUS MO time locations and SSB beams. This can be done as logical indexing.

Observation: As the LP-SS can provide sufficient synchronisation accuracy for LP-WUS detection in equal power levels, it does not seem necessary to have separate EPRE offsets for LP-WUS and LP-SS.

In Section 3.3 we discuss LP-WUS content presenting some analysis results and make following observations and proposals:-

Proposal: If more than one PO is mapped to LP-WUS, the information content is identical and equal, so that all POs will have same number of sub-group indications and their own individual ‘wake-up all’ indication.

Proposal: Support configurable mapping of maximum of 4 POs to one LO.

Other aspects related to configuration of LP-WUS and LP-SS is discussed in Section 3.4:-

Observation: Both LP-WUS and LP-SS time and frequency resources need to be provided for the IDLE/Inactive mode UE. Delivery method is up to RAN2.

Observation: For LP-SS the time domain configuration would need to contain periodicity and start time (SFN) possibly with additional e.g. slot level offset. Furthermore, fixed time pattern locations for LP-SS corresponding to SSB would be needed.

Proposal: For LP-SS a reference time can be configured through (sub-frame or radio frame level) offset and periodicity. If only one periodicity is supported, it can be fixed in specification.

Proposal: Support configuration of the start time for each LP-SS (corresponding to each SSB) at symbol level granularity. The reference time for the symbol level offset could be directly the LP-SS reference time.

Observation: Frequency location of LP-WUS and LP-SS can be determined using existing frequency resource grids/references, such as common resource grid.

Proposal: Supporting configuration of starting PRB on common resource block grid for LP-SS and LP-WUS.

Observation: For frequency domain configuration, it would be beneficial to agree whether LP-SS/LP-WUS configuration needs to have some relation to UE active BWP and in relation to each other and/or SSB.

Observation: For placement of LP-WUS and SSB for IQ-LR, two cases could be considered: if SSB and LP-WUS are sufficiently close, SSB can be used for tracking, ad if SSB is further apart, IQ-LR could use LP-SS.

Proposal: Consider the maximum frequency separation between SSB and LP-WUS that can be supported by IQ-LR.

Observation: For placement of LP-SS and SSB for ED-LR, as ED-LR does not need to consider SSB, there does not appear to be strong reason to restrict the relative placement in frequency domain.

In section 4.1 we discuss about methods to reduce the false alarms for the IDLE/Inactive mode UEs:-

Observation: To reduce the energy cost of repeated paging monitoring due to MR wake-ups, UE should be provided assistance information whether it should expect further paging in following paging occasion.

Aspects related to eDRX are presented in Section 4.2:-

Proposal: If LP-WS operation with eDRX is supported and UE is configured with eDRX and LP-WUS, the LP-WUS monitoring is determined based on paging occasions so that UE can monitor LP-WUS associated to the POs that are within the PTW.

# References

RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), CMCC

R1-2500573, LP-WUS and LP-SS design, Nokia

R1-2408537, LP-WUS and LP-SS design, Nokia

R1-2501624, LS on the wake-up delay for LP-WUS operation in IDLE/INACTIVE mode, RAN1