3GPP TSG RAN WG1 #120bis                                          R1-2502185

Wuhan, China, April 7th – 11th, 2025

Source: 	CMCC

Title:	Discussion on UE features for NTN for IoT Phase 3

Agenda item:	9.15.9

Document for:	Discussion & Decision

Introduction

This document is to discuss the UE features for NTN for IoT Phase 3.

UE capability for OCC based capacity enhancement

To further enhance the uplink capacity of the IoT NTN system, RAN1 discussed the OCC method for the NPUSCH channel. The UE overlays an OCC sequence on the data symbols of the uplink transmission on NPUSCH format 1, thereby improving the orthogonality of different UEs on the same RU resources and making uplink MU possible. To implement this functionality, UEs that support this feature need to have the capability to overlay OCC sequences in NPUSCH channel encoding. At the same time, the transmission method of DMRS is also different from that of legacy UEs. Therefore, it is necessary to define a new UE capability.

So far, RAN1 has agreed to use symbol-level OCC for NPUSCH with single-tone 3.75 kHz SCS and slot-level OCC for NPUSCH with single-tone 15 kHz SCS. Whether OCC should be supported for multi-tone NPUSCH is still under discussion.

Agreement At least the following schemes are supported for single-tone: - For 3.75kHz SCS OCC for NPUSCH format 1: - OCC length 2, Symbol-level - FFS: DMRS pattern(s) - For 15kHz SCS OCC for NPUSCH format 1: - OCC length 2, Slot-level, CDM DMRS with legacy pattern - FFS: CDM details, e.g. with or without spreading

The first issue to be discussed is whether OCC for single-tone NPUSCH with 3.75 kHz and 15 kHz should be defined as one or two separate features. From our point of view, a single function is beneficial for more flexible scheduling by the eNodeB. For eNodeB and UEs that support the OCC function, the eNodeB can flexibly choose to use OCC on different SCSs according to the needs of capacity and coverage. However, since RAN1 has selected different OCC schemes for different SCSs, some UEs may have difficulties in implementing both OCC schemes simultaneously. At the same time, based on the current progress of the discussion, to distinguish the DMRS of different UEs in OCC group, RAN1 has agreed to adopt the CDMed DMRS multiplexing scheme on the 15kHz SCS, and it has also formed a working assumption to adopt the TDMed DMRS multiplexing scheme on the 3.75kHz SCS. Some UEs may only support one of these DMRS transmission methods. Defining two separate UE capabilities for different SCSs is preferred, considering providing more flexibility for UE implementation.

At the same time, since the UE using the OCC function must use the new DMRS transmission method simultaneously, there is no need to define two separate UE capabilities for OCC and the new DMRS pattern. The UE that supports the OCC function also supports the corresponding DMRS pattern.

Observation 1:

OCC function and its related new DMRS pattern should be defined in a single UE capability.

Proposal 1:

For NPUSCH format 1 OCC based multiplexing, separate UE capabilities for different SCSs can be considered to relief the complexity for UE implementation.

Proposal 2：

Adopt the following UE radio access capability parameters

4.4.XXX ntn-OCC-3.75KHz-singleTone This field indicates whether the UE supports orthogonal cover codes (OCC) based NPUSCH format 1 multiplexing for 2 UEs in 3.75KHz single-tone. A UE supporting this feature shall also support TDMed DMRS as specified in TS 36.211. 4.4.YYY ntn-OCC-15KHz-singleTone This field indicates whether the UE supports orthogonal cover codes (OCC) based NPUSCH format 1 multiplexing for 2 UEs in 15KHz single-tone. A UE supporting this feature shall also support CDMed DMRS as specified in TS 36.211.

Conclusions

In this contribution, we discussed potential UE features for NTN for IoT Phase 3, and the following proposals are made.

Observation 1:

OCC function and its related new DMRS pattern should be defined in a single UE capability.

Proposal 1:

For NPUSCH format 1 OCC based multiplexing, separate UE capabilities for different SCSs can be considered to relief the complexity for UE implementation.

Proposal 2：

Adopt the following UE radio access capability parameters

4.4.XXX ntn-OCC-3.75KHz-singleTone This field indicates whether the UE supports orthogonal cover codes (OCC) based NPUSCH format 1 multiplexing for 2 UEs in 3.75KHz single-tone. A UE supporting this feature shall also support TDMed DMRS as specified in TS 36.211. 4.4.YYY ntn-OCC-15KHz-singleTone This field indicates whether the UE supports orthogonal cover codes (OCC) based NPUSCH format 1 multiplexing for 2 UEs in 15KHz single-tone. A UE supporting this feature shall also support CDMed DMRS as specified in TS 36.211.

References

- RP-250472, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, RAN#107, Incheon, Korea, March 12-14, 2025

- Chairman’s Notes RAN1#120, Athens, Greece, February 17th – 21st, 2025