3GPP TSG RAN WG1 #120bis                                                                                                R1-2502575

Wuhan, China, April 7th – 11th, 2025

Source:               TCL

Title:                    Low band carrier aggregation via switching

Agenda item:      9.12.2

Document for:    Discussion and Decision

- Introduction

In the RAN #107 meeting, an revised WID on Rel-19 low band carrier aggregation via switching was approved [1], the objectives of the WID are shown below:

Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers

In this contribution, further enhancements for low band carrier aggregation via switching are discussed.

- Discussion

Rx Switching pattern for low-low band

To enhance the utilization of low-band spectrum, low band carrier aggregation has been considered by the operators. However, given the current limitations in duplexer and antenna technlogies, aggregating low-band carriers through traditional carrier aggregation architectures, which rely on multiplexers, presents significant technical challenges. In response, operators have formally requested that 3GPP consider standardizing an alternative solution based on a switching mechanism, which could potentially overcome the constraints associated with conventional CA approaches. When Rx switching from one band to another band, simultaneous transmission and reception between the two bands are not supported, as illustrated in Figure 1.

Figure 1. Concept of low band aggregation via switching

Prior to discussing the Rx switching pattern, let us consider n5 and n29 as an example. The switching time between n5 and n29 should to be determined, whether from n5 to n29 or from n29 to n5. Due to different UE may have varying capabilities and/or RF architectures, potentially resulting in different switching times across devices. Consequently, the switching time between n5 and n29 should be reported to the gNB to ensure proper network coordination. Additionally, in ours view, a same rx switching time between transitioning from n5 to n29 or from n29 to n5 should be considered.

- Proposal 1: The switching time between band1 and band2 needs to report by UE, and a value of the switching time between band 1 and band2 should be considered.

Two approaches can be considered for indicating the Rx switching pattern: the semi-static method and dynamic method. The dynamic approach offers greater flexibility, allowing the UE to perform Rx switching at any time, which can better align with varying traffic demands between the UE and gNB across different bands. However, as it is required that  a semi-static switching pattern based on RRC configuration is considered to utilize the switching-based LB CA in Rel-19. Under this semi-static approach, parameters such as the Rx switching periodicity, switching time, and the switching point within a periodicity are configured via RRC signaling, and the switching pattern cannot be altered dynamically. While this reduces flexibility compared to the dynamic method, it significantly lowers implementation complexity. Considering the limited TU budget in Rel-19, we propose that the semi-static method for Rx switching pattern configuration be adopted as the preferred solution.

Proposal 2: Semi-static way for Rx switching pattern configuration low-low band aggregation in Rel-19 can be considered.

In the current specification, the semi-static configuration of switching time is associated with the carrier as indicated in the RRC signaling for downlink carrier switching period location. Similarly, for the Rx switching pattern, the location of the switching time need also be defined. In our view, it is preferable to avoid placing the switching time within the first three symbols of any downlink slot to ensure that critical downlink control resources remain unaffected. Consequently, the switching point can be positioned at the end of a slot, as illustrated in Figure2.

Figure 2. Location of the Rx switching time

- Proposal 3: The switching time would better not to impact on the DL control information of a carrier.

Impact on measurement

- For mobility purposes, RRM measurements on both bands is configured by the gNB. However, when Rx switching is executed, it introduces certain impacts on RRM measurements. Consider a scenario where Band 1 and Band 2 are aggregated, with Band 1 serving as the Pcell and band 2 as the Scell (DL only). According to the described above, the UE cannot receive downlink reference signals from the Pcell in Band 1 while switching form Band 1 to Band 2. If RRM measurements are required during a duration, the UE would be unable to perform them. To address this issue, in our view, the introduction of additional Rx switching point(s) within the semi-statically configured Rx switching pattern for low-low band aggregation could be considered.

- Proposal 4: The impacts on RRM measurement need to study when Rx switching for low-low band aggregation is configured.

- Conclusion

In this contribution, the following proposals have been made:

- Proposal 1: The switching time between band1 and band2 needs to report by UE, and a value of the switching time between band 1 and band2 should be considered.

Proposal 2: Semi-static way for Rx switching pattern configuration low-low band aggregation in Rel-19 can be considered.

- Proposal 3: The switching time would better not to impact on the DL control information of a carrier.

- Proposal 4: The impacts on RRM measurement need to study when Rx switching for low-low band aggregation is configured.

4. References

[1] RP-250247, Revised WID on low band carrier aggregation via switching.