3GPP TSG RAN WG1 #120bis			R1-2502685

Wuhan, China, April 7th – 11th, 2025

Source:	Sharp

Title:	Discussion on specification support for AI/ML based CSI prediction

Agenda Item:	9.1.3

Document for:	Discussion and Decision

Introduction

In RAN#107 meeting, revised WID for Rel-19 AI/ML CSI prediction was made as following [1]:

- CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): - Functionality-based LCM leveraged from other use cases, when necessary and applicable, - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025

For AI/ML CSI prediction, the specification effort should be small leveraging the other use cases, such as AI/ML beam management Case 2 and Rel-18 non-AI/ML based CSI prediction.

In this contribution, we discuss how AI/ML BM and Rel-18 CSI prediction are leveraged, and what the specification impact is.

Discussions

Functionality based LCM

Inference report

In RAN1#120 meeting, the following agreement was made:

Agreement For CSI prediction using UE-side model, at least for inference, Rel-18 CSI framework is reused. - For CSI-RS resource type for CMR, periodic, semi-persistent, and aperiodic CSI-RS are supported. - For inference report, - for N4>=1, support Rel-18 codebook ('typeII-Doppler-r18')

In the RAN1#120 meeting, it was agreed that Rel-18 eType II doppler codebook is reused for inference report of AI/ML based CSI prediction.

For Rel-18 non-AI based CSI prediction, the prediction window is configured with N4 consecutive slot intervals, each with duration of d slots. The first slot interval starts at slot l=n+δ. Prediction window configuration of the Rel-18 non-AI based CSI prediction can be reused. However, for AI/ML based CSI prediction, candidate values of each parameters like N4, δ and d can be longer than non-AI based CSI prediction, which are depending on UE capability. The UE can then report its supported values to network for configuration.

Figure 1: CSI prediction for N>=1 time intervals

Proposal 1: For Rel-19 AI/ML CSI prediction, the parameters δ, d, N for CSI prediction window are configured by gNB via RRC signaling from candidate values reported by UE depending on UE capability where

- slot (n+δ) is location of the first slot of first time interval,

- d is the time duration of time interval,

- N is the number of time intervals for CSI prediction.

Performance monitoring

In TR 38.843[2] and Chair’s note in RAN1#117 meeting [3], three types of performance monitoring were agreed as below:

TR 38.843 For CSI prediction using UE side model use case, at least the following aspects have been proposed by companies on performance monitoring for functionality-based LCM: - Type 1: - UE calculates the performance metric(s) - UE reports performance monitoring output that facilitates functionality fallback decision at the network - Performance monitoring output details can be further defined - NW may configure threshold criterion to facilitate UE side performance monitoring (if needed). - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Type 2: - UE reports predicted CSI and/or the corresponding ground-truth - NW calculates the performance metrics. - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Type 3: - UE calculates the performance metric(s) - UE reports performance metric(s) to the NW - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Functionality selection/activation/deactivation/switching as defined for other UE side use cases can be reused, if applicable. - Configuration and procedure for performance monitoring - CSI-RS configuration for performance monitoring - Performance metric including at least intermediate KPI (e.g., NMSE or SGCS) - UE report, including periodic/semi-persistent/aperiodic reporting, and event driven report - Note: down selection is not precluded. - Note: UE may make decision within the same functionality on model selection, activation, deactivation, switching operation transparent to the NW. Agreement (RAN1#117) For the boundary between Type 3 and Type 1 performance monitoring, the difference is whether UE reports performance metric or performance monitoring output to NW, respectively. - The monitoring output is determined based on performance metric, and additionally, baseline and/or threshold criterion if configured. Observation (RAN1#117) For CSI prediction using UE-sided model, for performance monitoring, at least following specification impacts are additionally identified compared to that has been captured in TR38.843, - Type 1 - Definition/configuration of performance metric - Definition of threshold criterion, if configured - Definition/configuration and report of monitoring output, and corresponding report mechanism - Type 2 - Definition/configuration and report of ground truth CSI, and corresponding report mechanism. - Type 3 - Definition/configuration and report of performance metric, and corresponding report mechanism. - For all types of performance monitoring, NW indication to the UE of the decision regarding the monitoring action

Agreement (RAN1#120) For CSI prediction using UE-side model, if performance monitoring type 1 or 3 is supported, for calculation of monitoring metric, support - Based on intermediate KPI - Down select between SGCS and NMSE by RAN1#120bis - FFS on the definition of SGCS/NMSE and how to calculate monitoring metric

For performance monitoring Type 1, the UE reports a performance monitoring output. The performance monitoring report may simply indicate whether the current model is deteriorated or not by comparing, for example, SGCS or NMSE between predicted CSI and ground-truth CSI. Further if the model is not deteriorated, the UE may not have to report the performance monitoring output. On the other hand, for Type 2 and Type 3, the UE has to report the CSI or performance metric regardless the model is deteriorated. Therefore, performance monitoring type 1 has the least signaling overhead among the three types. However, these behaviors are not specified in the current specification, so additional specification workload may be required, as noted in the agreed observation in RAN1#117.

Observation 1: For AI/ML CSI prediction performance monitoring type 1, signaling overhead is minimum among three types of performance monitoring.

For performance monitoring type 2, the legacy CSI framework can be reused to report predicted CSI and corresponding ground-truth CSI. Thus, the performance monitoring type 2 have no specification impact. Further, NW can make a decision taking into account other UEs or traffic requirements. However, signaling overhead may be the largest among the three monitoring types.

Observation 2: The performance monitoring type 2 have no specification impact.

Observation 3: For performance monitoring type 2, gNB can make a decision taking into account other UEs or traffic requirements.

For performance monitoring type 3, the UE reports performance metric, such as intermediate KPI (e.g., NMSE or SGCS). The signaling overhead of the intermediate KPI will be larger than Type 1 but smaller than Type 2, resulting in a moderate signaling overhead. The specification workload may be also moderate between type 1 and type 2.

Observation 4: The performance monitoring type 3 may require a moderate signaling overhead and moderate specification workload compared to Type 1 and Type 2 performance monitoring.

Since the limited remaining time unit for Rel-19, performance monitoring type 2 should be prioritized, but from the perspective of signaling overhead, performance monitoring type 1 should be also supported to enable better network control of traffic. Therefore, we made the following proposal:

Proposal 2: For Rel-19 AI/ML CSI prediction, performance monitoring type 1 and type 2 can be further considered.

In RAN1#120 meeting, for performance monitoring type 1 and type 3, the intermediate KPI (NMSE or SGCS) was agreed as the performance metric. In this meeting, we should discuss following two remaining issues:

- Whether SGCS or NMSE is used

- What matrix is used for calculating SGCS or NMSE

For issue 1, in our understanding, the SGCS reflects how similar the predicted CSI is to the ground truth CSI on a scale from 0 to 1, while NMSE reflects error between the predicted CSI and the ground truth CSI expressed in dB. Thus, from the perspective of standardization effort, SGCS is preferred to be considered as intermediate KPI.

Proposal 3: For AI/ML based CSI prediction performance monitoring type 1/3, SGCS is preferred for intermediate KPI.

For the issue 2, we think there are mainly two candidate matrixes for intermediated KPI calculation: raw channel matrix, precoding matrix corresponding to a codebook index (PMI) already specified in the specification. Raw channel matrix represents detailed information for the channel across various dimensions (e.g., time domain, frequency domain, Rx port, Tx port and so on). However, standardizing a raw channel matrix in a specification is challenging, and its complexity makes the calculation of the intermediate KPI more difficult. On the other hand, precoding matrix by using TypeII-Doppler-r18 has been already agreed in the last meeting for CSI prediction feedback. Although the precoding matrix may not represent the channel with the same level as the raw channel matrix, it simplifies the standardization process. Moreover, it facilitates the standardization discussion of SGCS calculation according to comparison between predicted PMI and measured PMI.

To reduce the standardization effort for AI/ML based CSI prediction, we prefer a more practical candidate matrix, i.e., to use a precoding matrix represented by the codebook index for SGCS calculation.

Proposal 4: For AI/ML based CSI prediction performance monitoring type 1/3, precoding matrix represented by the codebook is used for SGCS calculation.

For performance monitoring type 1 or 3, deriving the intermediate KPI, which is determined by comparison between the predicted PMI and ground truth CSI, requires a configuration for monitoring that enables UE to obtain the measured CSI for purpose of ground truth CSI. The existing CSI framework can be reused to configure the CSI-RS resource and report for monitoring. Since a UE may be configured with multiple CSI report configurations for different purpose, we can link an inference report configuring for CSI prediction with a CSI configuration for monitoring, by reusing the solution, which is agreed for BM as shown below [5], that is, by including the ID of an inference report configuration in the configuration for monitoring.

Agreement RAN1#119 At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling: - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report - FFS on the timing related issues - UE measures the dedicated resource set(s) for monitoring.

Proposal 5: For CSI prediction using UE-side model, if performance monitoring type 1 or 3 is supported, support to reuse CSI framework for the configuration for monitoring result report in L1 signalling:

- Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring

  - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration

- UE measures the dedicated resource set(s) for monitoring.

Processing capability/priority rules for AI/ML predicted CSI report

In the RAN1#120 meeting [4], following agreement was made for CSI prediction using UE-side model:

Agreement For CSI prediction using UE-side model, for CSI processing criteria and timeline, at least for inference further study on - Whether the CPU should be shared or separately counted between legacy CSI reporting and AI/ML-based CSI reporting - Whether the Processing Unit should be shared or separately counted among AI/ML related features/functionalities. - Whether new timeline is needed/updated for inference, and whether a different timeline is needed when functionality switches/activates. - Whether legacy framework for active CSI-RS resource and port counting can be reused Note: Strive to study CSI processing criteria considering both BM and CSI case, and take the existing solutions as starting point.

For the AI/ML CSI prediction, this kind of discussion is necessary, and this agreement can serve as a starting point of the discussion to reduce the specification effort.

In our understanding, for AI/ML use-case, a dedicated CPU for AI/ML, separate from legacy CSI processing, may be required. The dedicated CPU can be shared with the other AI/ML use cases (e.g. Beam Management). Therefore, we make following proposals:

Proposal 6: For AI/ML based CSI prediction, the overall CPU should be separately counted between legacy CSI reporting and AI/ML based CSI reporting.

Proposal 7: The dedicated CPU for AI/ML can be shared with the other AI/ML use cases (e.g. Beam Management).

Conclusion

In this contribution, we have discussed our views on specification support for AI/ML-based CSI prediction and have the following observations and proposals.

Proposal 1: For Rel-19 AI/ML CSI prediction, the parameters δ, d, N for CSI prediction window are configured by gNB via RRC signaling from candidate values reported by UE depending on UE capability where

- slot (n+δ) is location of the first slot of first time interval,

- d is the time duration of time interval,

- N is the number of time intervals for CSI prediction.

Observation 1: For AI/ML CSI prediction performance monitoring type 1, signaling overhead is minimum among three types of performance monitoring.

Observation 2: The performance monitoring type 2 have no specification impact.

Observation 3: For performance monitoring type 2, gNB can make a decision taking into account other UEs or traffic requirements.

Observation 4: The performance monitoring type 3 may require a moderate signaling overhead and moderate specification workload compared to Type 1 and Type 2 performance monitoring.

Proposal 2: For Rel-19 AI/ML CSI prediction, performance monitoring type 1 and type 2 can be further considered.

Proposal 3: For AI/ML based CSI prediction performance monitoring type 1/3, SGCS is preferred for intermediate KPI.

Proposal 4: For AI/ML based CSI prediction performance monitoring type 1/3, precoding matrix represented by the codebook is used for SGCS calculation.

Proposal 5: For CSI prediction using UE-side model, if performance monitoring type 1 or 3 is supported, support to reuse CSI framework for the configuration for monitoring result report in L1 signalling:

- Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring

  - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration

- UE measures the dedicated resource set(s) for monitoring.

Proposal 6: For AI/ML based CSI prediction, the overall CPU should be separately counted between legacy CSI reporting and AI/ML based CSI reporting.

Proposal 7: The dedicated CPU for AI/ML can be shared with the other AI/ML use cases (e.g. Beam Management).

References

RP-250792 “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN#107, Qualcomm.

3GPP; TSGRAN; Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18) V18.0.0

“RAN1 Chairman’s Notes,” RAN1 #117, May. 2024.

“RAN1 Chairman’s Notes,” RAN1 #118bis, Oct. 2024.

“RAN1 Chairman’s Notes,” RAN1 #119, Nov. 2024.