3GPP TSG RAN WG1 #120bis	R1-2502883

Wuhan, China, April 7th – 11th, 2025

Agenda item:		9.11.2

Source:	Nordic Semiconductor ASA

Title:					Support of RedCap and eRedCap UEs in NR NTN

Document for:		Discussion and Decision

# Introduction

In RAN#104, a revised work item in RP-241789 on NR NTN Phase 3 was approved, including the following objective:

5. Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1] - Enhancements are targeted for the following HD collisions cases (cases 3 and 4): - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded. - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution. - Notes for this objective: GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

In RAN1#118, the following agreements were made under this agenda item:

Agreement

The collision case 3 (Semi-statically configured DL reception collides with semi-statically configured UL transmission) can’t be assumed as error case. When the collision happens at UE side for collision case 3, one of the following options on priority rules is supported.

- Option A: the priority of keeping one direction and overriding another direction is based on different use cases (e.g. the collision happening in different channel types or different scheduling cases) without network signalling

  - It is not precluded that some or all use cases are left to UE implementation regarding whether DL reception or UL transmission is dropped

  - Note: it is not precluded that the same direction is prioritized for all different use cases

- Option B: network indicates the priority configuration to the UE

  - It is not precluded that some use cases are left to UE implementation regarding whether DL reception or UL transmission is dropped

  - It is not precluded to define default rules, if necessary

Agreement

To mitigate the collisions of case3 and case4, the following TA reporting enhancements for Rel-19 NTN HD-FDD (e)Redcap UE can be further studied:

- Finer TA report granularity

- Smaller TA offset threshold

- Trigger a TA report when the collision is detected at the UE

- TA reporting with a new triggering mechanism from gNB

- TA drifting rate reporting

Note: The benefit, complexity, power consumption and signaling overhead of each scheme is to be further investigated.

Note: other solutions can also be studied

In RAN1#118bis, the following conclusion was made:

Conclusion

The different use cases (e.g. the collision happening in different channel types or different scheduling cases) from RAN1#118 agreement for collision case 3 (Semi-statically configured DL reception collides with semi-statically configured UL transmission) consist of pairs of one semi-statically configured DL reception and one semi-statically configured UL transmission, among:

- Semi-statically configured DL reception includes Type-0/0A/0B/1/2-PDCCH CSS and dedicated semi-statically configured PDCCH USS/PDSCH SPS/CSI-RS/PRS/[Type 3 PDCCH CSS]

- Semi-statically configured UL transmission includes semi-statically configured PUSCH/PUCCH/SRS

In RAN1#119, the following agreements were made:

Agreement

For Rel-19 HD-FDD (e)Redcap UE in RRC connected mode, for collision case 3,

- Handling of collision with Type-0/0A/1/2-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note.

- For other use cases, default priority rule for collision case 3 in RRC-Connected mode is that DL is prioritized.

  - Network is allowed to indicate UL overriding DL for all the other use cases above.

    - This is signaled by RRC configuration.

Note: UE shall comply to the following existing procedure in 38.331:

UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

Agreement

The collision case 4 (Dynamically scheduled DL reception collides with dynamic scheduled UL transmission) is not considered as an error case for Rel-19 HD-FDD (e)Redcap UE in RRC connected mode.

- FFS whether to prioritize UL or prioritize DL.

In RAN1#120, the following agreements were made:

Agreement

When network indicates UL overriding DL in RRC-Connected mode for collision case 3, one UE-specific RRC parameter is used to indicate overriding for all the applicable other use cases except for collisions with Type-0/0A/1/2-PDCCH CSS.

Conclusion

For Rel-19 HD-FDD (e)Redcap UE with CG-SDT procedure ongoing in RRC-inactive mode, for collision case 3,

- Handling of collision with PDCCH CSS in RRC-inactive mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note.

Note: UE shall comply to the following existing procedure in 38.331:

- UEs in RRC_INACTIVE while SDT procedure is not ongoing shall monitor for SI change indication in its own paging occasion(s) that the UE monitors as specified in TS 38.304 [20].

- UEs in RRC_INACTIVE while SDT procedure is ongoing shall monitor for SI change indication in any paging occasion at least once per modification period, if the initial downlink BWP on which the SDT procedure is ongoing is associated with a CD-SSB.

- ETWS or CMAS capable UEs in RRC_INACTIVE while SDT procedure is not ongoing shall monitor for indications about PWS notification in its own paging occasion every DRX cycle.

- ETWS or CMAS capable UEs in RRC_INACTIVE while SDT procedure is ongoing shall monitor for indication about PWS notification in any paging occasion at least once every defaultPagingCycle.

Working assumption

For Rel-19 NTN HD-FDD (e)Redcap UE in RRC connected mode, the following handling rule for collision case 4 is supported:

- Handling of collision with PDSCH (at least for system information) scheduled by Type-0/0A[/1][/2]-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note.

- FFS: handling of PDCCH ordered PRACH transmission

- For other use cases, default priority rule for collision case 4 in RRC-Connected mode is that [DL or UL] is prioritized.

  - Network is allowed to indicate [UL or DL] overriding [DL or UL] for all cases

    - This is signaled by one UE specific RRC parameter

  - Note: if DL is prioritized, the DL prioritization applies only if the UL cancellation timeline can be satisfied, otherwise UL is prioritized.

Note: UE shall comply to the following existing procedure in 38.331:

- UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

In the following, we try to address the above issues and all other open issues pertaining to this agenda item.

# Discussion

Two collision cases, case 3 (Semi-statically configured DL reception collides with semi-statically configured UL transmission) and case 4 (Dynamically scheduled DL reception collides with dynamically scheduled UL transmission), have already been discussed in previous meetings. We agree that these collision cases may happen more often in NTN than in TN, due to greater uncertainty in timing advance, and not all such collisions can be regarded as error cases. However, we want to keep changes in specifications and changes in UE implementation in minimum. For collision case 3, except the paging monitoring, a default priority rule has been agreed upon and the rule is such that DL is prioritized but the network is allowed to override DL by UL for all collision cases by RRC signalling. For the collision case 4 we propose a similar approach, and that’s why we strongly support confirmation of the working assumption of the previous meeting with DL selected as a default priority. The only exception to this default prioritization rule is PDSCH scheduled by Type-0/0A-PDCCH of CSS in which case the DL/UL prioritization can be left to UE implementation.

Proposal 1: For Rel-19 NTN HD-FDD (e)Redcap UE in RRC connected mode, the following handling rule for collision case 4 is specified:

- Handling of collision with PDSCH scheduled by Type-0/0A-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note.

- For other use cases, default priority rule for collision case 4 in RRC-Connected mode is that DL is prioritized.

  - Network is allowed to indicate UL overriding DL for all collision cases

    - This is signaled by one UE specific RRC parameter

  - Note: The DL prioritization applies only if the UL cancellation timeline can be satisfied, otherwise UL is prioritized.

Note: UE shall comply to the following existing procedure in 38.331:

- UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

In the previous meeting it was agreed that enhanced TA reporting is further studied. We agree that better TA accuracy and better TA granularity could help to avoid some of the collision cases, but at the same time we believe that an enhanced TA reporting will not significantly increase system efficiency but it will significantly increase UE complexity and UE power consumption. We do not support any chances to the existing TA reporting. On the other hand, it should be studied whether the coarseLocationInfo, which UE can optionally report to the network, could help to solve the problem of system inefficiency.

Proposal 2: Do not specify any changes to the existing TA reporting scheme.

# Conclusions

In this contribution we discussed issues related to RedCap and eRedCap UEs in NR NTN system, and we have the following proposals:

Proposal 1: For Rel-19 NTN HD-FDD (e)Redcap UE in RRC connected mode, the following handling rule for collision case 4 is specified:

- Handling of collision with PDSCH scheduled by Type-0/0A-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note.

- For other use cases, default priority rule for collision case 4 in RRC-Connected mode is that DL is prioritized.

  - Network is allowed to indicate UL overriding DL for all collision cases

    - This is signaled by one UE specific RRC parameter

  - Note: The DL prioritization applies only if the UL cancellation timeline can be satisfied, otherwise UL is prioritized.

Note: UE shall comply to the following existing procedure in 38.331:

- UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

Proposal 2: Do not specify any changes to the existing TA reporting scheme.