3GPP TSG RAN WG1 #120b                                                         R1-2502901

Wuhan, China, April 7th – 11th, 2025

Agenda Item:		9.3.1

Source: 		Google

Title:			Discussion on SBFD TX/RX/measurement procedures

Document for: 	Discussion and Decision

# Introduction

In this document, we provide our views on remaining issues of SBFD TX/RX/measurement procedures.

# Discussion

## PUSCH repetition across SBFD and non-SBFD symbols

Agreement For a physical channel/signal occasion mapped to SBFD and non-SBFD symbols within a slot, an SBFD aware UE does not transmit or receive the physical channel/signal within the slot., except - PRACH transmission in a valid RO across SBFD symbols and non-SBFD symbols is subject to the agreements in AI 9.3.2. - For PUSCH repetition type B, down-select from the following options. - Option 1: A nominal repetition is segmented into actual repetitions around boundary of SBFD symbols and non-SBFD symbols. - Option 2: A nominal repetition is segmented into actual repetitions as in legacy operation. An actual repetition if it is mapped to both SBFD and non-SBFD symbols is dropped.

In the last meeting, RAN1 has discussed the case where a UL nominal repetition is mapped across SBFD and non-SBFD symbols in a slot. Considering repetition type B, it is not clear whether a UE should segment the nominal repetition into 2 actual repetitions according to the boundary of SBFD and non-SBFD symbols (Option-1), or just drop the actual repetition that crosses the boundary of SBFD and non-SBFD symbols (Option-2). With Option-1, the UE can fully utilize the scheduled UL resource, which results in better UL coverage. With Option-2, it is less complex for UE implementation, but the dropped repetition may lead to coverage gap. From our perspective, because one of the major motivation to introduce SBFD is to enhance UL coverage, our preference would be the Option-1 for better coverage.

Proposal 1: For a nominal repetition of PUSCH repetition type B is mapped across SBFD and non-SBFD symbols, UE segments the nominal repetition into actual repetitions around the boundary of SBFD and non-SBFD symbols.

## Collision handling

In the last meeting, the group has discussed whether to apply SBFD specific collision handling rules before or after legacy rules. 2 examples below were given by the feature lead, where a PUCCH (w or w/o repetitions) coincides in time with a DCI scheduled PDSCH reception and a DCI scheduled PUSCH transmission. The PDSCH does not collide with the PUSCH.

| (a) | (b) |
| --- | --- |

Figure 1. Examples for collision handling [2]

In this example, there are two collisions to be resolved. The first is the collision between the PDSCH and the PUCCH, and the second is between the PUCCH and PUSCH. In the legacy TDD system, only flexible symbol can be scheduled with dynamic DL and UL transmission. However, collision between dynamic scheduled DL and UL transmission is considered as an error case. Likewise, higher layer configured DL and UL transmission on the flexible symbol is also considered as an error case. Thus, no legacy rules can resolve the collision case. However, in SBFD, RAN1 has agreed to handle the case based on solutions from HD FDD RedCap, where the UE cancels or partially cancels the UL transmission based on UL preparation time and UE capability. To be noted, similar rules are applied for HD TDD CA. In this regard, the UE behaviour for such case has been there since release 16.

According to the descriptions in clause 9.2.2. in 38.213, if the UE is transmitting a PUSCH and the UE is transmitting a UCI, the UE multiplexes the UCI in the PUSCH, otherwise, the UE transmits the UCI in the associated PUCCH format. From our understanding, before the UE has determined to transmit the UCI on a PUCCH, there are no PUCCH to be considered. Based on this logic, there are no collision occurs in Figure 1. The UE first determines the DG PDSCH and DG PUSCH are valid, and then determines to multiplex the UCI to the PUSCH.

Observation 1: The collision handling between UCI multiplexing and transmission/reception exists in the legacy releases for HD TDD CA and HD FDD Redcap. Although it is not explicit written in the spec, UE with certain feature (e.g., HD FDD RedCap) resolves the collision handling of the feature first, and then applies to legacy rules, if any.

Conclusion 1: UE performs SBFD specific collision handling and physical channel validation first, then applies legacy collision handling rules, if any. UE determines the UCI multiplexing resources after the physical channel validation.

# Conclusion

Proposal 1: For a nominal repetition of PUSCH repetition type B is mapped across SBFD and non-SBFD symbols, UE segments the nominal repetition into actual repetitions around the boundary of SBFD and non-SBFD symbols.

Observation 1: The collision handling between UCI multiplexing and transmission/reception exists in the legacy releases for HD TDD CA and HD FDD Redcap. Although it is not explicit written in the spec, UE with certain feature (e.g., HD FDD RedCap) resolves the collision handling of the feature first, and then applies to legacy rules, if any.

Conclusion 1: UE performs SBFD specific collision handling and physical channel validation first, then applies legacy collision handling rules, if any. UE determines the UCI multiplexing resources after the physical channel validation.

# References

- “Draft Report of 3GPP TSG RAN WG1 #120 v0.2.0,” MCC Support, Athens, Greece, February 17th – 21st, 2025.

- R1-2501340, “ Summary #3 of SBFD TX/RX/measurement procedures,” Moderator (Xiaomi),  Athens, Greece, February 17th – 21st, 2025.