3GPP TSG-WG RAN1 Meeting #120-bis	                                     R1-2503003

Wuhan, China, April 7th – 11th, 2025

Agenda item:	9.13

Source: 	Moderator (EBU)

Title: 	Feature lead summary #3 on TFI for LTE-based 5G Terrestrial Broadcast

Document for:	Discussion and Decision

# Background

RAN#106 (latest WID in [1]) approved a new work item on Introduction of Time and Frequency Interleaving for LTE- based 5G Terrestrial Broadcast with the following objective:

For MBMS-dedicated cells, specify time-frequency interleavers [RAN1] and corresponding signaling [RAN2, RAN3] - For the time-interleaving part of 5G Broadcast TFI, one transport block is mapped to multiple non-consecutive subframes, following the principles of NR TBoMS. The following steps are followed: - Scale the TBS by a factor of N, where N is the number of subframes over which the TB is mapped. - RAN1 to discuss the details of the scaling, the value(s) for N and the set of subframes over which the TB is mapped, including how to schedule the TBs. - Within one subframe, the transmitted rate matched bits are read consecutively from the circular buffer. - In a given subframe, the procedure in TS 36.212 Section 5.1.4.1.2 applies (except for the parameter k0k0 which needs to be modified according to point 3 below) - Before frequency interleaving (if applied), the mapping from coded bits to RE follows the current specification. - Across subframes belonging to the same TB, the starting point for reading from the circular buffer (k0(k0 in TS 36.212 Section 5.1.4.1.2) is to be decided by RAN1 - NOTE 1: The maximum TBS after scaling and soft buffer size should follow limitations imposed by current LTE categories. RAN1 to discuss how to capture this limitation. - NOTE 2: Time interleaving is not applied for PMCH carrying MSI and/or MCCH. - For the frequency-interleaving part of 5G Broadcast TFI, interleaving is performed via a row-column interleaving with the following steps: - Data symbols in an OFDM symbol are written column-wise. - RAN1 to decide what is the unit over which data symbols are written (e.g. a single RE, four REs, etc.) - The rows and elements in each row may be permuted, with details to be decided by RAN1. - Data symbols in an OFDM symbol are read row-wise. - RAN1 to decide the number of {rows, columns} of the interleaver. NOTE: Time-frequency interleaving is not applicable to 0.37kHz SCS.

# Prioritization of proposals for online session #3

The following proposals are prioritized for agreement at the online session #3.

LBRM (details in 4.1)

Proposal

LBRM parameters (e,g, NIR) are consistent between the broadcast network and the (multiple) UEs that are receiving the broadcast transmission.

- These multiple UEs may belong to different UE categories, thus having different values of Nsoft, NIR.

k0 (details in 4.2)

Proposal

For the TB transmitted in N subframes, the starting pointer for the circular buffer is set as  k0n=2RsubblockTC+rvnEmin, where Emin=NLQmG'/C (as per TS 36.212), and rvn∈{0…N-1} is the redundancy version of the n-th subframe and mapped sequentially in the N subframes (i.e., rvn to subframen pattern is [0,1,2,…N-1]).

TI configuration (details in 4.3)

Proposal

The agreement in the previous meeting is updated as follows:

Regarding configuration for time interleaving transmission, the following options can be considered:

- Option 1: Per PMCH configuration, e.g., via pmch-Config

- Option 2: Per MBMS session, e.g., via MBMS-SessionInfo or via MAC CE

- NOTE 1: If per MBMS session configuration is done, a T_proc of 3 ms between the end of the MSI and the first MTCH in the MCH scheduling period is needed for UE de-rate matching.

- NOTE 2: If per MBMS session configuration is done, other options (than NOTE 1 above) are not precluded that allow the UE to know the M, N values of the first MTCH scheduled after the MSI

# Agreements so far

Agreement Support at least the following values of time interleaver depth: - Values M={4,8,16,32} and N={1,2,4,8,16} are supported. - FFS: UE behaviour in case values of M and N exceed UE capabilities in terms of soft buffer size and maximum TBS supportable in a TTI - FFS: larger values, including M=64, N={32,64}. Agreement In case of time interleaving, in order to reduce the resource wastage due to the potential mismatch between MSI periodicity and the (MxN) pattern: - Adopt an extended set of configurable periodicities for MCH scheduling period for R19 PMCHs that includes the intermediate values among the periodicities defined for the legacy configurations. - May include the case of scheduling different M/N within the same MSI periodicity - FFS the values of the new periodicities. - RAN1 to specify UE behavior, if needed, in case of residual mismatch even after new periodicities are adopted, taking into account the amount of subframe available at the end of the periodicity - Option 1: Drop the time-interleaved TBs which do not have all the N RVs transmitted within the MSI periodicity - Option 2: Drop the time-interleaved TBs which could not be successfully decoded. - It is up to network to decide whether to schedule or not schedule MTCH transmission in the residual mismatch part of the MSI periodicity. UE knows this from MSI (legacy behaviour) - Other Options not precluded Agreement The number of columns, C, of the frequency interleaver is C= #CBs Nsymb ×m, where #CBs denotes the number of CBs in the TB (including scaling, if any), Nsymb denotes the number of OFDM symbols of the PMCH, - Working assumption: - m = Nsymbgcd(Nsymb,#CBs), and gcd(.) denotes the operation to compute the greatest common divisor. Agreement The following steps are applied to the rows of the frequency interleaver: - Perform inter-row permutation with a reorder of the row indexes Ri', which is calculated as: Ri'=R'i-1+d########################, R'i-1≤R-d-1 #i×dR#, else. With, 1≤i≤R-1, R'0=0, and d=R/R/RC-C+1. - Perform a cyclic shift to the elements in the j-th row with an offset S(j) defined as: Sj=modj+12×-1j,C, j=0,1,2,…,R-1

# Open Issues: Time Interleaving part of the TFI

### 4.1 Limited buffer rate matching

During RAN1#120 the following agreement was reached:

Agreement

LBRM is applied for MCH (excluding MCCH and MSI) with time interleaving

- FFS spec impact

[ZTE] provides insight on the calculation of the soft buffer size for the transport block with parameters as currently defined in TS 36.212 clause 5.1.

Proposal

For PMCH with time interleaving, the soft buffer size for the transport block is denoted as NIR.

NIR=NsoftKC∙min⁡(M,Nlayer∙Mlimit)

The soft buffer size for the code block is denoted as Ncb=min⁡(NIRC,Kw).

Where Nsoft , KC , Mlimit and Kw are same as defined in TS 36.212 clause 5.1. M is the number of TBs in the basic transmission pattern of PMCH with time interleaving, Nlayer is the number of the supported layers for spatial multiplexing in DL-SCH according to the UE category.

Note that during the off-line discussion it has been clarified that with broadcast there is no handshake between UE and BS (UE cannot communicate UE category) therefore the BS has to make a decision and communicate it to the population of UEs. Therefore the following proposal is raised.

Proposal

LBRM parameters (e,g, NIR) are consistent between the broadcast network and the (multiple) UEs that are receiving the broadcast transmission.

- These multiple UEs may belong to different UE categories, thus having different values of Nsoft, NIR.

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | In general, we tend to think no need to modify such equations for broadcast as this proposal because UE may support unicast and broadcast simultaneously. |
| ZTE | Support. As in exist spec, the and . However, considering the broadcast with time-interleaving should buffer M TBs, the equation of NIR and Ncb should be updated. In addition, the broadcast is one-layer transmission, the maximum parallel processing capability of UE can be restricted to Nlayer∙Mlimit. Therefore, the proposed updates of NIR and Ncb should be considered. |
| Qualcomm | There is an issue, that different UEs cannot use different assumptions on LBRM parameters. While the intent of the proposal is correct, we think there needs to be a “common” understanding for “all” UEs on LBRM parameters, while receiving the MCH with time interleaving. |

### 4.2 k0 and reading bits from the circular buffer

With regards to the value of k0, the following agreement was made in the last meeting.

Agreement The equation of k0 for time interleaving is modified to avoid puncturing of systematic bits. - Further study the exact equation for k0 for the n-th subframe of a transmission belonging to a same TB k0n, n∈{0, 1, 2, …, N-1}), including - Option 1: for each CB of the TB, k0n=2RsubblockTC+rvnEmin, where Emin=NLQmG'/C (as per TS 36.212), and rvn∈{0…N-1} is the redundancy version of the n-th subframe. - Option 2: For each CB of the TB, follow the principle of NR TBoMS to determine k0n - Option 1 and Option 2 assumes all CBs of the TB have the same RV index for all CBs of the TB, indexed by k0n - FFS whether different CBs of the TB mapped to each subframe may have different RV indices (e.g., indexed by different k0n per CB)

[Qualcomm] proposes to use Option 1 and points out that Option 2 requires maintaining multiple values of k0n for circular buffer management. A single pointer across the entire TB is preferred considering implementation complexity.

[ZTE] proposes to reuse the same mechanism in NR TBoMS with redundancy version 0 for each CB of the TB for MBMS transmission with time interleaving.

[Huawei] proposes Option 1 with rvn∈{0…N-1} the redundancy version of the n-th subframe mapped sequentially in the N subframes. Although the basic idea of rate matching for TBoMS in option 2 is similar, option 1 is complete in terms of k0n for each of redundancy version and the redundancy version sequence to be used, i.e., the rvn =0, 1, .., N is used sequentially for the N subframes that the TB is to be transmitted in.

In view of the arguments exposed the following proposal is raised:

Proposal

For the TB transmitted in N subframes, the starting pointer for the circular buffer is set as  k0n=2RsubblockTC+rvnEmin, where Emin=NLQmG'/C (as per TS 36.212), and rvn∈{0…N-1} is the redundancy version of the n-th subframe and mapped sequentially in the N subframes (i.e., rvn to subframen pattern is [0,1,2,…N-1]).

| Company | Comment |
| --- | --- |

Proposal

Proposal: Adopt a cyclic shift to ensure that different RVs of the same codeblock are evenly distributed to fully exploit the frequency diversity.

- Option 1: Circular Shift the RVs of different codeblocks within i -th Subframe in the following codeblock index order:  CB[0+∆ mod C] , CB[1+∆ mod C] , ...,CB[j] , ...,CB[C-1+∆ mod C] , where circular offset ∆=-i, CB[j] denotes the codeblock index, and C is the number of codeblock. Note: the circular granularity is per RV.

| Company | Comment |
| --- | --- |
| Samsung | In order to get better flexibility, we should consider fractional CSs of the length of the RM output as well. The output of codeblock concatenation be CB[0+∆ mod C](P:Q[0+∆ mod C]) , CB[1+∆ mod C] , ...,CB[j] , ...,CB[C-1+∆ mod C] , CB[0+∆ mod C] (1:P-1) where CBf(a:b) denotes elements a to b of the CBfand Q[f] is the number of elements of CBf |
| SJTU | We have discussed carefully within our team and with other companies, and we believe that the RV granularity shift method proposed (option 1) has already achieved the goal of ensuring the whole CB being ‘evenly distributed to fully exploit the frequency diversity’. Further introducing flexibility cannot bring performance gain. |

### 4.3 TI granularity and configuration

During RAN1#120 the following agreement was reached:

Agreement

Specify time interleaving per MBMS session/MTCH without supporting interleaving across different MBMS sessions/MTCH.

Agreement

Regarding configuration for time interleaving transmission, the following options can be considered:

- Option 2: Per PMCH configuration, e.g., via pmch-Config

- Option 3: Per MBMS session, e.g., via MBMS-SessionInfo or via MAC CE

There is a majority of views [ZTE, Huawei, Qualcomm, EBU] proposing the granularity of TI to be defined per MBMS session.

[ZTE] argues that using MAC CE to indicate time-domain interleaving parameters for each MBMS session is complex and incurs high overhead. Therefore, using RRC signaling, such as through MBMS-SessionInfo, is a better approach.

[Samsung] argues that time interleaving is a physical layer attribute/parameter same as MCS, FFT size and is tightly coupled with a physical channel transmission. With the MBMS services (MTCHs) mapped to respective PMCHs, it would seem unreasonable to have MTCHs with different realibity requirements (e.g. TFI, MCS) mapped to a common PMCH. The signalling burden would be excessive if TI configuration parameters are associated with individual MTCH-sessionInfo elements.

In view that the majority of proponents would prefer a PMCH to multiplex different services with different TI parameters, the following proposal is made:

Proposal

The agreement in the previous meeting is updated as follows:

Regarding configuration for time interleaving transmission, the following options can be considered:

- Option 1: Per PMCH configuration, e.g., via pmch-Config

- Option 2: Per MBMS session, e.g., via MBMS-SessionInfo or via MAC CE

- NOTE 1: If per MBMS session configuration is done, a T_proc of 3 ms between the end of the MSI and the first MTCH in the MCH scheduling period is needed for UE de-rate matching.

-NOTE 2:If per MBMS session configuration is done, other options are not precluded that allow the UE to know the M, N values of the first MTCH scheduled after the MSI

| Company | Comment |
| --- | --- |
| Samsung | We would like to emphasize that a MTCH is NOT configured with specific mobility or reliability or coverage parameters, rather it is achieved by mapping the service MTCH to the appropriate MBSFN area (configures SCS, FFT) and PMCH (configures dataMCS). Thus, it is not right to configure diverse TI parameters to individual MTCHs mapped to same PMCH. We would prefer all reliability controlling factors like dataMCS, M and N values should be configured at one place i.e. pmch-Config. We would like companies to re-consider per PMCH based TI configuration If still companies are in favour of per MBMS session configuration, then as an alternative, we would like to propose a generic and signalling efficient approach (in line with Option 1), where pmch-Config provides a list of TI configurations (e.g., TI-ConfigList) and individual MBMS-SessionInfo carries an index to one entry of the TI-ConfigList (e.g., TI-ConfigIndex). Proposal: Configuration for time interleaving transmission is done per MBMS session wherein MBMS-SessionInfo carries an index to one entry of the TI configurations list in pmch-Config. (We think option 2 is an overkill as TI configuration is not expected to be so dynamically changing and it has drastic impact on MSI format and signalling to accommodate TI configuration parameters.) |
| Huawei, HiSilicon | In general, the idea of using MAC CE indicating the values of M,N is to address the mismatch issue with moderate spec impact. |
| ZTE | Support Option 1. Since configuring time-interleaving parameter for each MBMS session in MAC CE costs too much overhead. |
| Qualcomm | Prefer not to change the MSI payload. |

### 4.4 Maximum TBS size per UE category

During RAN1#120 the following agreement was reached:

Agreement

For PMCH with time interleaving, the maximum TBS a UE supports for the scaled TB is derived based on DL-SCH as per TS 36.306 set by UE category.

-	FFS when the UE category has multiple maximum TBS

Agreement

The maximum number of transport block bits received within a TTI defined by the UE category should include the number of bits for unicast and broadcast, where the number of bits for broadcast is the one after being scaled up by N.

[ZTE] argues that if UE's hardware processing capability remains identical for PMCH with time interleaving as for unicast, then the same baseband resources (decoders/buffers) that handle the maximum number of bits in DL-SCH can be supported and therefore proposes to define the maximum TBS a UE supports for the scaled TB is the largest TBS among the multiple maximum TBS of a DL-SCH as per TS 36.306 set by UE category.

[Huawei] argues that the broadcast transmission or the enhancement with time/frequency interleaving should not be affected by UE capability in order to target as many UEs as possible for the reception. Therefore, when the UE category has multiple maximum TBS, the TBS should be set to the smallest one that is supported by the UE category.

[EBU] argues that long time interleaving depths are needed to counteract slow time-varying fading effect and for the 3GPP specifications to allow for at least similar TI depths as in state-of-the-art broadcast systems. The selection of the target UE category is left to the network to decide given the lack of uplink capabilities for the ROM-UE.

[Qualcomm] indicates the need to provision for maximum time-diversity with large interleaving depth. This input also assumes that the TBS is the maximum TBS supportable in a TTI, for a given UE category.

In view of the arguments calling for large interleaving depth, that the selected scaled TBS should in any case be supported by the selected UE category, and the need of flexibility for operators to define their target UE population, the following proposal is made:

Proposal

If the UE category has multiple maximum TBS for unicast, all the maximum TBS sizes are supported for that UE category for broadcast.

- The UE indicates the maximum TBS supported, signaled from among the candidate maximum TBS sizes supported

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | In general, we think the interleaved PMCH should benefit as many UE categories as possible effectively. When UE has multiple Max TBS for unicast, usually which is which should be based on other UE capability reporting, e.g., supported number of aggregated carriers, the support of higher modulation order, etc. Back to broadcast and kept the generality for as many UE categories, we should a minimum value should be specified for such a case. |
| ZTE | We wonder does this proposal mean that the UEs of one category have different supported maximum TBS capabilities according to the supported spatial layers? |
| Qualcomm | This makes sense to us. |

Additionally, [ZTE] provides a solution in case of configuring a large MxN value that cannot be handled by the given UE category.

Proposal

UE to discard the interleaved MTCH when the number of the transmitted TBs of the interleaved MTCH exceed UE capability:

- Option 1: Discard the interleaved MTCH;

- Option 2: Receive a subset of the transmitted TBs of the interleaved MTCH, the number of the received TBs is M', where:

- M' equals the UE's maximum parallel processing capability, OR

- M' is the largest value in the M parameter set that does not exceed UE capabilities

- Option 3: Perform TDMed reception of all M TBs, by receiving a subset of the subframes of each TB for the interleaved MTCH.

| Company | Comment |
| --- | --- |
| Samsung | For option 2, receiving a subset of the transmitted TBs of the interleaved MTCH would result in consistent 50% or higher loss of the TBs (given M and M’ are multiples of 2 and M>M’, e.g. M=16 or 32 and M’=8). We think this is unrecoverable loss and such service reception is not useful from user perspective. For option 3, receiving a subset of the sub-frames of each TB implies not all redundancy versions are received for each TB. We think UE cannot achieve satisfactory decoding performance with such approach, given skipping some sub-frames of TBs carrying certain specific redundancy version (RV0) are drastic. Overall, option 2 and option 3 based UE behaviour with lossy reception and poor decoding respectively, are also against the very objective of this WID targeting for the reliability enhancement. Option 1 seems to be the natural approach for an UE with M’ (< M) as the restricted maximum parallel processing capability. However, we also think the M value should be configured by network so as a majority of UEs can be served with the broadcast transmission in a reliable manner and thereby, avoid such issues. |
| Huawei, HiSilicon | Not sure whether option 3 works actually, I suppose all the subframes should be received to get a decodable TB. For option2, the entire UE soft buffer size pertaining to the UE category determines the capability of the support for M and N values, so I suppose the values of M and N should be discussed together for the capability . |
| ZTE | As we proposed, the UE can select one of the three options as the solution when the M value exceeds its capability, based on channel conditions and actual transmission quality. - Option 1 is the simplest approach. - For Option 2, the value of M' can be chosen according to the UE’s actual capability (e.g., equal to the value of UE capability) rather than half or even 1/4 of M to receive and process as many TBs as possible. - Option 3 ensures that all TBs are received. As for the issue raised by Samsung, the base station can ensure that the redundancy versions (RVs) containing systematic bits are not skipped. |

### 4.5 Signaling

Regarding additional signaling, [Samsung] proposes enhacements for MSI as compared to legacy specifications, to be supported  for scheduling of R19 PMCHs based MTCHs. Such enhancements need to be captured in the (RAN2) MAC specification.

Proposal

Minimal multiplexing enhancements for MCH reception are applied only for scheduling of R19 PMCHs based MTCHs and include:

  - Not allowing multiplexing of two MTCHs in same subframe

  - Not applying Time interleaving to subframe carrying MSI/eMSI/MCCH

  - Not allowing multiplexing of MTCH with MSI/eMSI/MCCH in a sub-frame due to Time interleaving difference

  - Inserting and/ interpreting padding to account for remaining portion of the subframe in above scenarios

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | Is it the common understanding that all these bullets will be reflected in RAN1/RAN2 spec once RAN1 agreed on that? |

Proposal

Coexistance for legacy and R19 transmission and UEs is achieved with defining R19 PMCHs to cater to Time interleaving based configurations, scheduling and transmissions in addition to legacy PMCHs.

| Company | Comment |
| --- | --- |

Proposal

RRC signaling is used for configuring the values M and N

| Company | Comment |
| --- | --- |
| Samsung | This is in line with our previous argument against using MAC CE for configuring M and N values. We think TFI configuration is not expected to be changing dynamically. Time inter-leaver configuration is a typical MBSFN physical channel design attribute such as dataMCS and likewise, is better suited for RRC signaling based configuration |
| ZTE | This is a repeated issue from previous discussions and does not require a separate proposal. |
| Qualcomm | Agree. |

### 4.6 TBS determination

During RAN1#120 the following agreement was reached:

Agreement

For TBS scaling by N, down-select one of the two options below:

- Option 1: Calculate the total number of PRBs NRBtotal as NRBtotal=NRBDL×N, wherein, NRBDL is system bandwidth;

- If NRBtotal≤110, reuse the transport block size table in TS 36.213 and according to the obtained ITBS and NRBtotal, obtain the corresponding TBS;

- Else, reuse the transport block size table in TS 36.213 and according to the obtained ITBS and NRBDL, obtain the corresponding TBS’; and calculate the final TBS as TBS=round{TBS'×N}

The operation “round{x}” is mapping x to the closest TBS from all the existing TBS values in section 7.1.7.2 TS 36.213.

- Option 2: Obtain the initial TBS (TBS') as per current specifications, and calculate the final TBS as TBS=round{TBS'×N}

- The operation “round{x}” is mapping x to the closest TBS from all the existing TBS values in section 7.1.7.2 TS 36.213.

In case two TBSs are at the same distance of TBS'×N, the larger TBS is chosen

Two contributions to RAN1#120bis provide further considerations on the rationale to select Option 1 or Option 2, but both present opposing views.

[ZTE] argues that the WID indicates that the mapping of multiple non-consecutive subframes should follow the principles of NR TBoMS. According to TS 38.214 and TS 36.213, the procedure is aligned with Option 1.

[Huawei] argues that Option 2 is clearer, simpler and also aligned to the existing paragraph in TS 36.213 to determine the TBS scaling for 0,37kHz subcarrier spacing.

In view of the lack of consensus, the two options are open and left for agreement during RAN1#120-bis.

Proposal

RAN1 to decide between:

- Option 1: Calculate the total number of PRBs NRBtotal as NRBtotal=NRBDL×N, wherein, NRBDL is system bandwidth;

- If NRBtotal≤110, reuse the transport block size table in TS 36.213 and according to the obtained ITBS and NRBtotal, obtain the corresponding TBS;

- Else, reuse the transport block size table in TS 36.213 and according to the obtained ITBS and NRBDL, obtain the corresponding TBS’; and calculate the final TBS as TBS=round{TBS'×N}

- Option 2: Obtain the initial TBS (𝑇𝐵𝑆′) as per current specifications, and calculate the final TBS as 𝑇𝐵𝑆=𝑟𝑜𝑢𝑛𝑑{𝑇𝐵𝑆′×𝑁}

- The operation “round{x}” is mapping x to the closest TBS from all the existing TBS values in section 7.1.7.2 TS 36.213.

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | Option 2 is simple and sufficient. Let’s make everything simple. |
| ZTE | We support Option 1. Option 1 is aligned with the design principle of TBoMS. Furthermore, in spec TS 36.213, as for TBs mapped to more than one layers spatial multiplexing, the TBS is determined either through (ITBS, M∙NPRB) or by predefined mapping tables from TBS_L1 to TBS_LM based on the range of NPRB, where M denotes the number of transmission layers. The concept is also aligned with Option 1 for TBS determination for PMCH with time interleaving. By adopting Option 1, the similar approach and table of the TBS determination for four layers can be reused for TBS determination of N=4. |
| SJTU | We give our proposal and simulation results by adapting the method in Option 2. Nonetheless, we note that Option 1 remains a viable alternative. |
| Qualcomm | Don’t care either way. |

# Frequency Interleaving part of the TFI

During RAN1#120 the following agreement was reached:

Agreement The previous agreement is modified as follows: The modulation symbol vector (prior to frequency interleaving) y=y0,y1,…,yZ-1 is interleaved, according to the principles of subclause 5.1.4.1.1 of TS 36.212 as follows - Set the number of columns of the frequency interleaver, C=X, set the number of rows R to be the minimum integer satisfying Z≤R×C, where Z is the number of data REs in one OFDM symbol - FFS: How to determine X - Option 1: X is equal to the number of codeblocks that are mapped to the OFDM symbol - Option 2: X=32 - Other options are not precluded. - Append y with <NULL> elements, as required, to obtain yapp - Write the elements of yapp into the R×C-matrix column-wise, with yapp(0) in column 0 of row 0. - FFS whether the rows and elements in each row will be permuted, with details to be further studied, e.g., intra-row cyclic shifts, inter-row permutations - Obtain the vector yΠ by reading the elements of the above R×C-matrix row-wise, excluding the <NULL>s. - The vector yΠ shall then be mapped to resource elements, according to clause 6.3.5 of TS 36.211. FFS: whether to apply a cyclic shift offset, and the details of the cyclic shift offset for the concatenated frequency interleaver outputs of all OFDM symbols before the mapping to REs of a subframe, where the cyclic shift offset is dependent on the RV index.

### 5.1 Regarding the last FFS

[Samsung] argues that Option 1 may provide robust performance against deep time-domain fading when the number of codeblocks is large. However, its effectiveness diminishes when the number of codeblock is small due to limited frequency-domain interleaving depth, which reduces its resilience to fading-induced performance degradation. The proposal is to make C configurable.

Proposal

RAN1 to study the application of a cyclic shift offset for the concatenated frequency interleaver outputs of all OFDM symbols before the mapping to REs of a subframe, once the rate matching details regarding k0n are agreed.

| Company | Comment |
| --- | --- |
| Samsung | This option is a bit dependent on the RM algorithm and we believe it should be finalized only after RM is finalized. It’s implementation can be part of the frequency interleaver, i.e., for the frequency interleaver, instead of start reading the output from (0,0) element of the row-column interleaver, one can start the read up from some other starting point (a,b) of the row-column interleaver, where at least one of a, b is non-zero. Furthermore, the frequency interleaver distributes the bursty channel errors due to multi-paths (frequency domain) across the various code-blocks. Note that the distribution of these errors across code-blocks is the same for any subframe due to channel bursty errors in the time domain. As regards the distribution of these errors across the code-blocks, if all code-blocks are exposed to same number of errors then there is no problem. But if, the distribution of these errors across code-blocks is not uniform, some code-blocks may experience more errors than other code-blocks. In which case, it becomes necessary to distribute these errors furthermore across code-blocks. The only way to do this is through the above proposal and/or Proposal 1 by SJTU. There are no other proposals to do this. A key difference between this proposal and that of SJTU is that the shifts proposed by SJTU are always multiples of the length of the rate matching output whereas our proposal can even shift by fractional values of multiples of the length of the rate matching output. Furthermore, the benefits of this proposal also are dependent on the finalisation of the frequency interleaver. So, this proposal to be studied after the finalization of Rate matching and frequency interleavers. |
| Qualcomm | We think the SJTU proposal on cyclically shifting the contents across subframes on top of the FI solves this issue. We in principle support that proposal from SJTU, which I think is also the original intent of the Samsung proposal. |

# Summary of proposals to RAN1#120bis

| Company | Proposals |
| --- | --- |
| ZTE Corporation, Sanechips | Proposal 1: Regarding TBS determination for PMCH with time interleaving, Option 1 is supported. That is, For a TB transmitted over N subframes, calculate the total number of PRBs NRBtotal as NRBtotal=NRBDL×N, wherein, NRBDL is system bandwidth; - If NRBtotal≤110, reuse the transport block size table in TS 36.213 and according to the obtained ITBS and NRBtotal, obtain the corresponding TBS; - Else, reuse the transport block size table in TS 36.213 and according to the obtained ITBS and NRBDL, obtain the corresponding TBS’; and calculate the final TBS as TBS=round{TBS'×N} The operation “round{x}” is mapping x to the closest TBS from all the existing TBS values in section 7.1.7.2 TS 36.213. Proposal 2: For PMCH with time interleaving, the maximum TBS a UE supports for the scaled TB is the largest TBS among the multiple maximum TBS of a DL-SCH as per TS 36.306 set by UE category. Proposal 3: For PMCH with time interleaving, the soft buffer size for the transport block is denoted as NIR. NIR=NsoftKC∙min⁡(M,Nlayer∙Mlimit) The soft buffer size for the code block is denoted as Ncb=min⁡(NIRC,Kw). Where Nsoft , KC , Mlimit and Kw are same as defined in TS 36.212 clause 5.1. M is the number of TBs in the basic transmission pattern of PMCH with time interleaving, Nlayer is the number of the supported layers for spatial multiplexing in DL-SCH according to the UE category. Proposal 4: Regarding the issue of mismatch between the allocated subframes for a MBMS session and the basic transmission pattern of (M x N) subframes, the following options can be considered: - Option 1: The transmissions are mapped to the allocated subframes according to the basic transmission pattern of (M x N) time interleaving, the rest of transmissions are dropped. - Option 2: According to the basic transmission pattern of (M*N) time interleaving, some TBs that cannot complete N transmissions are transmitted with a reduced time interleaving depth, the rest of transmissions are dropped. Proposal 5: For each CB of the TB, the starting point kn in the circular buffer for each subframe (subframe#n, n∈{0, 1, 2, ..., N-1}) is determined by kn=2RsubblockTCn=0(kn-1+H)mod Ncbn=1,...,N-1 Where: - RsubblockTC is the number of rows of the sub-block interleaver - N is the number of subframes allocated for a TB - NcbNcb is the length of circular buffer - H is the number of rate matching coded bits available for transmission of the CB in a subframe allocated for a TB. Proposal 6: Regarding configuration for time interleaving transmission, Option 3 is supported via MBMS-SessionInfo. Per the RAN1#120 agreement, the baseline transmission pattern for time-interleaved MTCH consists of (M × N) MBSFN subframes excluding MSI and MCCH. However, when MSI and/or MCCH are present in MBSFN subframes carrying MTCH traffic, the number of transmitted TBs will be less than the configured time-interleaving parameter M. Proposal 7: UE adopt one of the following behaviours when the number of the transmitted TBs of the interleaved MTCH exceed UE capability: - Option 1: Discard the interleaved MTCH; - Option 2: Receive a subset of the transmitted TBs of the interleaved MTCH, the number of the received TBs is M', where: - M' equals the UE's maximum parallel processing capability, OR - M' is the largest value in the M parameter set that does not exceed UE capabilities - Option 3: Perform TDMed reception of all M TBs, by receiving a subset of the subframes of each TB for the interleaved MTCH. Proposal 8: The number of columns of the frequency-domain interleaver is configured by RRC, and each value configured is m*X, where X is the number of codeblocks that are mapped to the OFDM symbol, m is an integer. - FFS the candidate value of m, e.g., m =1,2,3,4. |
| Huawei, HiSilicon | Proposal 1: The maximum number of bits of one transport block for broadcast after scaled up is the same as the one defined within a TTI for unicast. - If the UE category has multiple maximum TBS for unicast, the maximum TBS for the broadcast TB is set to the smallest value defined for unicast pertaining to the UE category. Proposal 2: The scaling factor N = 16 should not be supported. Proposal 3: N=2 can also be considered if helpful to address the mismatch between MSI periodicity and the basic transmission pattern of (M x N) subframes. Proposal 4: The value larger than 16 for M should not be supported. Proposal 5: M=4 can also be considered if helpful to address the mismatch between MSI periodicity and the basic transmission pattern of (M x N) subframes. Proposal 6: With M=4,8,16 and N=2,4,8, the values of (M x N) could be 8,16,32,64,128. Proposal 7: Option 3: Per MBMS session is supported for configuring values of M/N. Proposal 8: If the mismatch between MSI periodicity and the basic transmission pattern of (M x N) subframes has to be resolved, consider to configure values of M/N in MAC CE, or make the last transmission within the MSI periodicity as an exception, i.e., <(M x N) subframes. Proposal 9: Option 2 from the earlier agreement is supported to determine the exact valid TB size after scaling up for the time interleaving. Proposal 10:For the TB transmitted in N subframes, the starting pointer for the circular buffer is set as k0n=2RsubblockTC+rvnEmin, where Emin=NLQmG'/C (as per TS 36.212), and rvn∈{0…N-1} is the redundancy version of the n-th subframe and mapped sequentially in the N subframes. |
| Samsung | Proposal 1: Coexistance for legacy and R19 transmission and UEs is achieved with defining R19 PMCHs to cater to Time interleaving based configurations, scheduling and transmissions in addition to legacy PMCHs. Proposal 2: RRC signaling is used for configuring the values M and N Proposal 3: Configuration for Time interleaving (e.g. M and N values) is per PMCH configuration i.e. via pmch-Config Proposal 4: Include M=4 as supported value to address low speed scenario. Proposal 5: Minimal multiplexing enhancements for MCH reception are applied only for scheduling of R19 PMCHs based MTCHs and include: - Not allowing multiplexing of two MTCHs in same subframe - Not applying Time interleaving to subframe carrying MSI/eMSI/MCCH - Not allowing multiplexing of MTCH with MSI/eMSI/MCCH in a sub-frame due to Time interleaving difference - Inserting and/ interpreting padding to account for remaining portion of the subframe in above scenarios Proposal 6: To address any potential issue of mismatch between MSI periodicity and the basic transmission pattern of (M x N) subframes: - Adopt an extended set of configurable periodicities for MCH scheduling period for R19 PMCHs that includes the intermediate values among the periodicities defined for the legacy configurations. Proposal 7: No non-backward compatible changes are pursued for MSI format and signalling. Proposal 8: As regards the application of a cyclic shift offset for the concatenated frequency interleaver outputs of all OFDM symbols before the mapping to REs of a subframe, we propose that it be deferred until the finalization of the rate matching details regarding k0n. |
| EBU, SWR, BNE, ORS | Proposal 1: Large interleaving depths (large values of M and N) be considered in the design of TFI for 5G Broadcast, in order to provide interleaving depths at least in the range of state-of-the-art terrestrial broadcast standards. Proposal 2: The granularity options for the 5G Broadcast TFI should not be limiting those already offered by state-of-the art broadcast standards. Proposal 3: In order to minimize resource wastage when scheduling time-interleaved MCHs using existing MSI periodicities, a starting pointer to the first subframe (with respect to the MSI) from which the MTCHs are scheduled, has to be signalled via MSI. Proposal 4: The selection of the target UE category is left to the network to decide. Proposal 5: Simulation results for frequency interleaving based on the 0 dB echo profile should be prioritized for the selection of the best frequency interleaving scheme. |
| Shanghai Jiao Tong University, ABS, NERC-DTV, PCL, CUC | Proposal 1: Determination of RV indices and circular shift within each subframe - Determine Redundancy Version Subindices Across Subframes: For each subframe, select RVs with the same RV index rvi from all codeblocks for mapping, where rvi∈{0,1,2,…,n-1}. The RV indices rvi are ordered across n subframes using a specified pattern ΩRV. - e.g n=4, ΩRV can be {0,2,3,1}. - Circular Shift Within i-th Subframe: Map Emin bits from the start pointer k0i of CB[0+∆ mod C] , CB[1+∆ mod C] , ..., CB[C-1+∆ mod C] sequentially, where circular offset ∆=-i, CBj denotes the j-th codeblock, C is the number of codeblock. - Periodic Interleaving of Subframes: Separate the C×n RVs by (M-1) MBSFN subframes (excluding MCCH/MSI), uniformly distributing M TBs across n×M subframes. Proposal 2: The cross row block interleaver with cyclic shift for elements in each row should be specified to enhance the system performance. To interleave modulation symbol vector y=y0,y1,…,yZ-1, the frequency interleaver is operated as the following steps: - Set the number of columns of the frequency interleaver NC be equal to the number of codeblocks that are mapped to the OFDM symbol. Set the number of rows of the frequency interleaver NR as the minimum integer satisfying Z≤NR×NC, where Z is the number of data REs in one OFDM symbol. - Append y with <NULL> elements, as required, to obtain yapp. - Write the elements of yapp into the NR×NC-matrix column-wise, with yapp(0) in column 0 of row 0. - Perform inter-row permutation in accordance with the reordered row index R0,R1,⋯,RNR-1, where Ri can be calculated as Ri=Ri-1+d########################, Ri-1≤NR-d-1 #i×dNR#, else. Here, 1≤i≤NR-1, R0=0, and the interval d=NR/NR/NRNC-NC+1. - Perform cyclic shift to the elements in j-th row with specific offset S(j), which is defined as: Sj=modj+12×-1j,NC, j=0,1,2,…,NR-1 - Obtain the vector yΠ by reading the elements from the above NR×NC-matrix row-wise, excluding the <NULL>s. - Map the vector yΠ to resource elements, according to clause 6.3.5 of TS 36.211. |
| Qualcomm Incorporated | Proposal 1: Configuration of time interleaving parameters shall be done per MBMS session, via MBMS-SessionInfo. Proposal 2: For a MCH scheduling period, signal via the corresponding MSI, a starting pointer to the first subframe (with respect to the MSI) from which the MTCHs are scheduled. Proposal 3: The scheduling delay from the MSI to the first MTCH (Tstart) scheduled by the MSI is given by Tstart, ≥ Tproc , where Tproc=3 ms Proposal 4: For time-interleaved PMCH, specify the starting pointer for each CB of the TB as k0n=2RsubblockTC+rvnEmin, where Emin=NLQmG'/C and rvn∈{0…N-1} is the redundancy version of the n-th subframe. Proposal 5: For time-interleaving parameters N and M, the following additional values are supported: - N=16,32,64 - M=32, 64 Proposal 6: Row-specific cyclic shifts, as described in [1], are specified for the frequency interleaver. Proposal 7: The row permutation operation described in [1] is applied to the frequency interleaver. Proposal 8: Set the number of columns, C, of the frequency interleaver as C=#CBsgcd#CBs, Nsymb, where #CBs denotes the number of CBs in the TB, Nsymb denotes the number of OFDM symbols of the PMCH, and gcd(.) denotes the operation to compute the greatest common divisor. - When Nsymb=1, this is the same as “Option 1” in the RAN1 agreement and in [1] Proposal 9: At least for the case of Nsymb>1, permute the columns of the R×C frequency interleaving matrix, prior to applying row-specific cyclic shifts. |

# References

[1] RP-250794, “Revised WID on LTE-based 5G Broadcast Phase 2”, 3GPP TSG RAN Meeting #107, Incheon, Korea, March 2025