3GPP TSG-RAN WG1 Meeting #121	R1-2503264

St Julian’s, Malta, May 19 – 23, 2025

Agenda Item:	9.15.3

Source:	Huawei, HiSilicon

Title:	UE features for evolution of NR duplex operation

Document for:	Discussion and Decision

Introduction

In this contribution, we provide our views on UE features for evolution of NR duplex operation.

Discussion

Basic UE feature group for SBFD

The following components were agreed for the basic FG for an SBFD-aware UE

1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one [and two] DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling 4. Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1) 5. Support of separate CSI reports for SBFD and non-SBFD symbols FFS: Whether to include two DL subband support in component 2 FFS: other component (which may be separate FG)

One remaining issue is whether to include the support of DL reception across two non-contiguous DL subbands into the basic feature group. Our preference is to define it as a separate FG since it has impact to both PDSCH reception and CSI measurement.

Another remaining issue is whether the following two components are defined as separate UE features.

- Support of separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols

- Support of separate SRS configurations in SBFD symbols and non-SBFD symbols

Our preference is to include both in the basic feature group.

Proposal 1: Include the following additional components into FG 60-1

- Support of separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols

- Support of separate SRS configurations in SBFD symbols and non-SBFD symbols

Tx/Rx related UE features

DL reception in two non-contiguous DL subbands

As discussed in section 2.1, an SBFD-aware UE can optionally support DL reception across two non-contiguous DL subbands. This has an impact on PDSCH reception and CSI processing timeline.

This FG includes the following components

- CSI processing timeline in SBFD symbols for the support of one contiguous CSI-RS resource allocation across two DL subbands

- Maximum number of supported partial PRGs in SBFD symbols for the support of PDSCH reception across two DL subbands

For component 1, UE can indicate whether CSI processing timeline is the same as legacy or the CSI processing timeline Z/Z’ is scaled by factor of 2.

Proposal 2: The support of UL transmission in one UL subband and DL reception in two non-contiguous DL subbands is defined as a separate FG including the following components

- CSI processing timeline in SBFD symbols for the support of one contiguous CSI-RS resource allocation across two DL subbands

- Maximum number of supported partial PRGs in SBFD symbols for the support of PDSCH reception across two DL subbands

Transmissions and receptions across SBFD and non-SBFD symbols (Configuration 2)

In RAN1#120bis, the following was agreed

Agreement: Introduce following FG for [Transmissions and] Receptions across SBFD and non-SBFD symbols (Configuration 2) as follows, and FFS on whether transmission across SBFD and non-SBFD symbols is defined as separate FG or not - Alt.1: Configuration 2 for transmission and reception should be defined in a single FG - Alt.2: Configuration 2 for transmission and reception should be defined in separate FGs

The remaining issue is whether to define transmission and reception in single FG or separate FGs. Alt.2 unnecessarily fragments this feature by separating support of Configuration 2 for DL reception and UL transmission into two FGs. Alt.1 aligns better with the previous agreement. From implementation point of view, Alt-1 is a cleaner approach.

Proposal 3: The support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols (Configuration 2) is defined as a single FG separate from the basic FG.

SBFD random access operation related UE features

According to the RAN1#120 agreement, two separate UE capabilities are introduced for the support of SBFD random access operation based on the two different RACH configuration options

- Support of SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 1 with Alt 1-1)

- Support of SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 2)

In RAN1#120, there was a discussion on whether the support of RACH configuration Option 1 or Option 2 should be included in the basic feature group and the following was agreed

Agreement: Introduce following FGs for SBFD random access, one based on single RACH configuration (Option 1) and the other based on two RACH configurations (Option 2), and FFS on whether UE supporting SBFD random access needs to support at least Option 1 or Option 2. - Alt.1: UE supporting SBFD random access needs to support at least Option 1 - Alt.2: UE supporting SBFD random access needs to support at least Option 2 - Alt.3: UE supporting SBFD random access supports Option 1 or Option 2 (i.e., no default option)

Considering that an SBFD-aware UE can perform random access based on legacy ROs as a legacy UE, it seems not critical to support a default option (Alt.3). One argument for Alt.1 is that UE may not implement either Option 1 or Option 2 if there is no default option. If this is the primary concern, another alternative is to support both Option 1 and Option 2 even through this contradicts with previous agreement. It should be noted that RACH configuration Option 1 and Option 2 have different application scenarios. It does not make sense to prioritize one use case over the other.

Proposal 4: The support of RACH configuration Option 1 and Option 2 are defined as separate FGs, i.e., no default option.

Preamble repetition within additional-ROs

For preamble repetition within additional-ROs, two separate UE capabilities can be defined for RACH configuration Option 1 and RACH configuration Option 2 respectively.

For preamble repetition within additional-ROs based on RACH configuration Option 1, the following components are included

- Determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-ROs

- For RACH configuration Option 1, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.

For preamble repetition within additional-ROs based on RACH configuration Option 2, the following components are included

- Determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-ROs

- For RACH configuration Option 2, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 and msg1-RepetitionNum for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.

Msg3 repetition in SBFD symbols

For Msg3 repetition in SBFD symbols, there was a discussion on whether to introduce to separate FG. Our view is that it can be included into the basic FG. Note that the support of Configuration 1 (UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols) is included is the basic FG. A UE supporting Msg3 repetition (FG 30-6) and the basic SBFD FG should be have the capability to support Msg3 repetition in SBFD symbols without much additional complexity. There is no need to introduce a separate UE capability for this.

Proposal 5: The support of Msg3 repetition in SBFD symbols is not defined as a separate FG.

CLI related UE features

UL resource muting

For the support of UL resource muting, FG 60-7 and FG 60-7a were introduced for CP-OFDM waveform and DFT-S-OFDM waveform respectively. One remaining issue is whether a UE shall support one UL resource muting pattern or multiple UL resource muting patterns for the basic support of UL resource muting. In another words, whether the semi-statically configured time location and frequency location of UL resource muting for DG-PUSCH and Type-2 CG PUSCH and one or multiple Type-1 CG PUSCH configurations should be the same. It should be noted that the following has already been agreed in RAN1#120.

Agreement For the semi-statically configured time location and frequency location of UL muting symbol(s) for PUSCH, - Separate configurations of up to two UL muting symbols can be provided per UE for DG PUSCH/Type 2 CG PUSCH and Type 1 CG PUSCH - Separate configuration for each of multiple Type 1 CG PUSCH configurations is supported

Given that there is not much additional UE implementation effort to support different UL muting patterns for DG-PUSCH and Type-2 CG PUSCH and Type-1 CG PUSCH, we suggest to remove the FFS from FG60-7 and FG60-7a.

Proposal 6: For both FG 60-7 and FG 60-7a, UE supports different UL resource muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH.

L1-CLI-RSSI measurement and reporting

It was agreed that support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting is defined as a separate UE capability. Similarly, the support of semi-persistent L1 L1-RSSI measurement resource and aperiodic reporting should also be defined as a separate UE capability.

Proposal 7: The support of semi-persistent L1 L1-RSSI measurement resource and aperiodic reporting is defined as a separate UE capability.

The second issue is whether to include the support of periodic L1 CLI-RSSI reporting and periodic CLI-RSSI measurement resource in FG 60-8. To align with the existing UE capability definitions, we prefer to include the support of periodic L1 CLI-RSSI reporting and periodic CLI-RSSI measurement resource in FG 60-8.

For L1 CLI-RSSI measurement and reporting, the following components are included (update in red)

- Aperiodic L1 CLI-RSSI reporting on PUSCH and periodic L1 CLI-RSSI reporting on PUCCH

  - Maximum number of periodic CSI report setting per BWP for L1-CLI-RSSI report

  - Maximum number of aperiodic CSI report setting per BWP for L1-CLI-RSSI report

  - Maximum number of configured aperiodic CSI triggering states in CSI-AperiodicTriggerStateList per CC

- Support of CLI-RSSI measurement resource configured in one UL subband only, in one DL subband only, or across two DL subbands only.

- Periodic and aperiodic CLI-RSSI measurement resource

  - Maximum number of CLI-RSSI measurement resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-CLI-RSSI

L1-SRS-RSRP measurement and reporting

For L1-SRS-RSRP measurement and reporting, the remaining issue is whether to include the support of periodic SRS-RSRP measurement resource in FG 60-9. Similar to the discussion above, we prefer to include the support of periodic SRS-RSRP measurement resource in FG 60-9.

In addition, we suggest to separate the component into reporting and measurement resource similar to L1-CLI-RSSI measurement reporting since the reported values are defined separately.

For L1-SRS-RSRP measurement and reporting, the following components are included (update in red)

- Aperiodic L1 SRS-RSRP reporting on PUSCH

  - Maximum number of aperiodic CSI report setting per BWP for L1-SRS-RSRP report

  - Maximum number of configured aperiodic CSI triggering states in CSI-AperiodicTriggerStateList per CC

- Periodic and aperiodic SRS-RSRP measurement resource

  - Maximum number of SRS-RSRP measurement resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SRSR-RSRP within a slot

  - Maximum number of SRS-RSRP measurement resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SRSR-RSRP

  - Maximum number of aperiodic SRS-RSRP measurement resources across all CCs configured to measure L1-SRS-RSRP

Conclusions

In this contribution, the following are proposed:

Proposal 1: Include the following additional components into FG 60-1

- Support of separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols

- Support of separate SRS configurations in SBFD symbols and non-SBFD symbols

Proposal 2: The support of UL transmission in one UL subband and DL reception in two non-contiguous DL subbands is defined as a separate FG including the following components

- CSI processing timeline in SBFD symbols for the support of one contiguous CSI-RS resource allocation across two DL subbands

- Maximum number of supported partial PRGs in SBFD symbols for the support of PDSCH reception across two DL subbands

Proposal 3: The support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols (Configuration 2) is defined as a single FG separate from the basic FG.

Proposal 4: The support of RACH configuration Option 1 and Option 2 are defined as separate FGs, i.e., no default option.

Proposal 5: The support of Msg3 repetition in SBFD symbols is not defined as a separate FG.

Proposal 6: For both FG 60-7 and FG 60-7a, UE supports different UL resource muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH.

Proposal 7: The support of semi-persistent L1 L1-RSSI measurement resource and aperiodic reporting is defined as a separate UE capability.

Proposal 8: The table in Annex 1 is taken as the starting point for further UE feature discussion on Rel-19 SBFD.

# References

- R1-2502965, Session Notes of AI 9.15.3, Ad-Hoc Chair (NTT DOCOMO, INC.)

# Annex 1: UE features for evolution of NR duplex operation

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60. NR_duplex_evo | 60-1 | Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures | - Support of cell-common configuration of time and frequency location of SBFD subbands - Support of UL transmission in one UL subband and DL reception in one DL subband in SBFD symbols - Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling - Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1) - Support of separate CSI reports for SBFD and non-SBFD symbols - Support of separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols - Support of separate SRS configurations in SBFD symbols and non-SBFD symbols - Support of separate PUCCH resources in SBFD symbols and non-SBFD symbols - Support of separate FH offsets for PUSCH transmissions in SBFD symbols and non-SBFD symbols |  | Yes | n/a | Basic feature for SBFD-aware UE TX/RX/ measurement behaviour and procedures is not supported | Per Band | TDD only | Yes | n/a |  | Optional with capability signaling |
| 60. NR_duplex_evo | 60-1a | DL reception in two non-contiguous DL subbands | - CSI processing timeline in SBFD symbols for the support of one contiguous CSI-RS resource allocation across two DL subbands - Maximum number of supported partial PRGs in SBFD symbols for the support of PDSCH reception across two DL subbands | 60-1 | Yes | n/a | DL reception in two non-contiguous DL subbands is not supported | Per Band | TDD only | Yes | n/a | For component 1, ‘Legacy’ means the CSI processing timeline and CPU counting are the same as legacy. ‘Scaled’ means the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy. | Optional with capability signaling Component-1 candidate values: {‘Legacy’, ‘Scaled’} Component-2 candidate values: {2, 4} |
| 60. NR_duplex_evo | 60-2 | Transmission and reception across SBFD symbols in different slots (Configuration 2) | - Support of PDSCH reception across SBFD symbols and non-SBFD symbols (Configuration 2) - Support of UL transmission across SBFD symbols and non-SBFD symbols (Configuration 2) | 60-1 |  | n/a | Transmission and reception across SBFD symbols in different slots (Configuration 2) are not supported | Per Band | TDD only | Yes | n/a |  | Optional with capability signaling |
| 60. NR_duplex_evo | 60-3 | SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode | Support of SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 1) | 60-1 |  | n/a | SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode is not supported | Per Band | TDD only | Yes | n/a |  | Optional with capability signaling |
| 60. NR_duplex_evo | 60-4 | SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode | Support of SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 2) | 60-1 |  | n/a | SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode is not supported | Per Band | TDD only | Yes | n/a |  | Optional with capability signaling |
| 60. NR_duplex_evo | 60-5 | Preamble repetition within additional-ROs based on RACH configuration Option 1 | - Determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-ROs - For RACH configuration Option 1, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs. | 60-4 |  | n/a | Preamble repetition within additional-ROs based on RACH configuration Option 1 is not supported | Per Band | TDD only | Yes | n/a |  | Optional with capability signaling |
| 60. NR_duplex_evo | 60-6 | Preamble repetition within additional-ROs based on RACH configuration Option 2 | - Determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-ROs - For RACH configuration Option 2, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 and msg1-RepetitionNum for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs. | 60-5 |  | n/a | Preamble repetition within additional-ROs based on RACH configuration Option 2 is not supported | Per Band | TDD only | Yes | n/a |  | Optional with capability signaling |
| 60. NR_duplex_evo | 60-7 | UL resource muting for CP-OFDM waveform | - Support semi-static configuration of time location and frequency location of UL resource muting for CP-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol - Support dynamic on/off of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH - Support semi-static determination of UL muting symbols for Type-1 CG PUSCH |  |  | n/a | UL resource muting for CP-OFDM waveform is not supported | Per Band | TDD only | Yes | n/a |  | Optional with capability signaling |
| 60. NR_duplex_evo | 60-7a | UL resource muting for DFTS-OFDM waveform | - Support semi-static configuration of time location and frequency location of UL resource muting for DFTS-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol - Support dynamic on/off of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH - Support semi-static determination of UL muting symbols for Type-1 CG PUSCH |  |  | n/a | UL resource muting for DFT-S-OFDM waveform is not supported | Per Band | TDD only | Yes | n/a |  | Optional with capability signaling |
| 60. NR_duplex_evo | 60-8 | L1 CLI-RSSI measurement and reporting | - Aperiodic L1 CLI-RSSI reporting on PUSCH and periodic L1 CLI-RSSI reporting on PUCCH - Maximum number of periodic CSI report setting per BWP for L1-CLI-RSSI report - Maximum number of aperiodic CSI report setting per BWP for L1-CLI-RSSI report - Maximum number of configured aperiodic CSI triggering states in CSI-AperiodicTriggerStateList per CC - Support of CLI-RSSI measurement resource configured in one UL subband only, in one DL subband only, or across two DL subbands only. - Periodic and aperiodic CLI-RSSI measurement resource - Maximum number of CLI-RSSI measurement resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-CLI-RSSI | 60-1 |  | n/a | L1 CLI-RSSI measurement and reporting is not supported | Per Band | TDD only | Yes | n/a |  | Optional with capability signaling Component-1 candidate values: {8, 16, 32, 64} Component-2 candidate values: {1, 2, 3, 4} Component-2 candidate values: {1, 2, 3, 4} Component-4 candidate values: {Support Measurement resource type#1, Support Measurement resource type#2} Component-5 candidate values: {3, 7, 15, 31, 63, 128} |
| 60. NR_duplex_evo | 60-8a | Support of semi-persistent L1 CLI-RSSI measurement resource and aperiodic reporting | Support of semi-persistent L1 CLI-RSSI measurement resource and aperiodic reporting | 60-8 |  | n/a | Semi-persistent L1 CLI-RSSI measurement resource and aperiodic reporting is not supported | Per Band | TDD only | Yes | n/a |  | Optional with capability signaling |
| 60. NR_duplex_evo | 60-9 | L1 SRS-RSRP measurement and reporting | - Aperiodic L1 SRS-RSRP reporting on PUSCH - Maximum number of aperiodic CSI report setting per BWP for L1-SRS-RSRP report - Maximum number of configured aperiodic CSI triggering states in CSI-AperiodicTriggerStateList per CC - Periodic and aperiodic SRS-RSRP measurement resource - Maximum number of SRS-RSRP measurement resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SRSR-RSRP within a slot - Maximum number of SRS-RSRP measurement resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SRSR-RSRP - Maximum number of aperiodic SRS-RSRP measurement resources across all CCs configured to measure L1-SRS-RSRP | 60-1 |  | n/a | L1 SRS-RSRP measurement and reporting is not supported | Per Band | TDD only | Yes | n/a |  | Optional with capability signaling Component-1 candidate values: {1, 2, 3, 4} Component-2 candidate values: {3, 7, 15, 31, 63, 128} Component-3 candidate values: {4, 8, 16, 32} Component-4 candidate values: {2, 4, 8} Component-5 candidate values: {1, 2, 3, 4} |
| 60. NR_duplex_evo | 60-9a | Simultaneous reception of DL signals/channels and L1 SRS-RSRP measurement resource | Support of FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource | 60-9 |  | n/a | Simultaneous reception of DL signals/channels and L1 SRS-RSRP measurement resource is not supported | Per Band | TDD only | Yes | n/a |  | Optional with capability signaling |
| 60. NR_duplex_evo | 60-9b | Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting | Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting | 60-9 |  | n/a | Semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting is not supported | Per Band | TDD only | Yes | n/a |  | Optional with capability signaling |

# Annex 2: Agreements on UE features for evolution of NR duplex operation

RAN1#120

Agreement:

Introduce following basic FG for SBFD, and FFS on some potential components (that may be separate FGs).

| X-1 | Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures | 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one[/two] DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling FFS: other component (which may be separate FG) |  | YES | n/a | Basic feature for SBFD-aware UE TX/RX/ measurement behaviour and procedures is not supported | [Per Band] | TDD only | n/a | n/a | [Note: Only UL transmissions are allowed within UL subband and only DL receptions (except for CLI measurement) are allowed within DL subband(s)] [Note: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one.] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement:

Add the following component in basic FG for SBFD.

- “Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1)”

Agreement:

Introduce following FGs for SBFD random access, one based on single RACH configuration (Option 1) and the other based on two RACH configurations (Option 2), and FFS on whether UE supporting SBFD random access needs to support at least Option 1 or Option 2.

- Alt.1: UE supporting SBFD random access needs to support at least Option 1

- Alt.2: UE supporting SBFD random access needs to support at least Option 2

- Alt.3: UE supporting SBFD random access supports Option 1 or Option 2 (i.e., no default option)

| X-3 | SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode | Support of SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 1) FFS: Other component(s) which may be separate FG | X-1 | YES | n/a | SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode is not supported | [Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X-4 | SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode | Support of SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 2) FFS: Other component(s) which may be separate FG | [X-1, X-3] | YES | n/a | SBFD random access operation based on two separate RACH configurations in RRC_IDLE/INACTIVE/CONNECTED mode is not supported | [Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |

RAN1#120bis

Agreement:

Introduce following FG for L1 SRS-RSRP measurement/reporting.

| 60-9 | L1 SRS-RSRP measurement and aperiodic reporting | 1. Aperiodic L1 SRS-RSRP reporting on PUSCH, and [periodic and] aperiodic SRS-RSRP measurement resource FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG | FFS | YES | n/a | [Aperiodic] L1 SRS-RSRP measurement and reporting is not supported | [Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement:

Introduce a new FG for FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource as follows

| 60-9a | FDM-ed reception of DL signals/channels and L1 SRS-RSRP measurement resource | 1. Support of FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource | 60-9 | YES | n/a | FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource is not supported | [Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement:

Introduce following FG for L1 CLI-RSSI measurement/reporting.

| 60-8 | L1 CLI-RSSI measurement and [aperiodic] reporting | 1. Aperiodic L1 CLI-RSSI reporting on PUSCH [and periodic L1 CLI-RSSI reporting on PUCCH] 2. Support of CLI-RSSI measurement resource configured in one UL subband only, in one DL subband only, or across two DL subbands only. 3. [Periodic and] aperiodic CLI-RSSI measurement resource FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG | FFS | YES | n/a | [Aperiodic] L1 CLI-RSSI measurement and reporting is not supported | [Per Band] | TDD only | [n/a] | [n/a] |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement:

Define the following FG

| 60-9b | Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting | 1. Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting | 60-9 | YES | n/a | Semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting is not supported | [Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement:

Introduce the following FGs for UL resource muting

| 60-7 | UL resource muting for CP-OFDM waveform | 1. Support semi-static configuration of time location and frequency location of UL resource muting for CP-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol [- FFS: Whether this FG only support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH] 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH FFS: other potential component/FG/additional restriction | [FFS] | YES | n/a | UL resource muting for CP-OFDM waveform is not supported | [Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60-7a | UL resource muting for DFTS-OFDM waveform | 1. Support semi-static configuration of time location and frequency location of UL resource muting for DFTS-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol [- FFS: Whether this FG only support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH] 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH FFS: other potential component/FG/additional restriction | [FFS] | YES | n/a | UL resource muting for DFTS-OFDM waveform is not supported | [Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |

Agreement:

Introduce following FG for [Transmissions and] Receptions across SBFD and non-SBFD symbols (Configuration 2) as follows, and FFS on whether transmission across SBFD and non-SBFD symbols is defined as separate FG or not

- Alt.1: Configuration 2 for transmission and reception should be defined in a single FG

- Alt.2: Configuration 2 for transmission and reception should be defined in separate FGs

| 60-2 | [Transmission and] Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) | 1. Support of PDSCH reception across SBFD symbols and non-SBFD symbols (Configuration 2) [2. Support of UL transmission across SBFD symbols and non-SBFD symbols (Configuration 2)] FFS: Other component(s) FFS: whether component 2 is defined as separate FG |  | YES | n/a | [Transmission and] Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) is not supported | [Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement:

Adopt “Per band” type for FG 60-1.

Agreement:

Adopt “Per band” type for FG 60-3.

Agreement:

Adopt “Per band” type for FG 60-4

Agreement:

Add the following components in FG 60-1 (basic FG for SBFD operation)

- Support of separate PUCCH frequency resource configuration in SBFD symbols and non-SBFD symbols

- [FFS: Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols]

- Support of separate PUSCH frequency hopping offset in SBFD symbols and non-SBFD symbols

- [FFS: Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols]

Agreement:

Update FG 60-1 as follows:

- Confirm “cell-common” in component 1

- Confirm “UL transmission in one UL subband and DL reception in one [and two] DL subband in SBFD symbols”

- Confirm Component 3: “3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling”

- Add “Support of separate CSI reports for SBFD and non-SBFD symbols

| 60-1 | Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures | 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one [and two] DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling 4. Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1) 5. Support of separate CSI reports for SBFD and non-SBFD symbols FFS: Whether to include two DL subband support in component 2 FFS: other component (which may be separate FG) |  | YES | n/a | Basic feature for SBFD-aware UE TX/RX/ measurement behaviour and procedures is not supported | Per band | TDD only | n/a | n/a | [Note: Only UL transmissions are allowed within UL subband and only DL receptions (except for CLI measurement) are allowed within DL subband(s)] [Note: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one.] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |