3GPP TSG-RAN WG1 Meeting #121	R1-2503271

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	9.12.1

Source:	Huawei, HiSilicon

Title:	Discussion on Rel-19 Multi-carrier enhancements

Document for:	Discussion and Decision

Introduction

In the RAN#105 meeting, a WID on multi-carrier enhancements was approved for Release 19 [1], with the following objectives for multi-cell PUSCH/PDSCH scheduling using a single DCI.

1. Specify the support of the following for multi-cell PUSCH/PDSCH scheduling with a single DCI [RAN1] - Different SCS/carrier type among co-scheduled cells by the single DCI. - One or multiple PUSCHs/PDSCHs per scheduled cell by the single DCI. - The maximum number of PUSCHs/PDSCHs per scheduled cell is [4 or 8]. - Note: Type-1 HARQ-ACK codebook is not enhanced for Rel-19 multi-cell scheduling. - Note: The maximum number of sub-codebooks for Type-2 HARQ-ACK codebook is not increased for Rel-19 multi-cell scheduling. - Note: UE does not expect to be configured with both single-cell multi-PUSCH/PDSCH scheduling and multi-cell multi-PUSCH/PDSCH scheduling on the same or different cells within a same PUCCH group. - Note: No new DCI format is introduced.

In the last meeting, RAN1 discussed several issues and reached some agreements in [2]. Further discussion on Rel-19 MC enhancements is provided in this contribution.

Discussion

Supported scenarios

The following agreement was reached at RAN1#118bis [3], considering to further narrow down the supported scenarios for Rel-19 multi-carrier enhancement. However, no related discussion was carried out in the subsequent meetings as the focus was first placed on feature design. Now that the feature design is basically completed, the following agreement can be used as a starting point for further discussion.

Agreement - Consider at least the case that up to two different SCS can be scheduled by a DCI format 0_3/1_3 in Rel-19. - Consider at least the following cases for scheduled cells in Rel-19: - Case 1: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR1 licensed FDD cell(s) with SCS1 and FR1 licensed TDD cell(s) with SCS2. - SCS1 can be same or different to SCS2. - Case 2: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR1 licensed FDD cell(s) with SCS1 and FR2-1 cell(s) with SCS2. - SCS1 can be same or different to SCS2. - Case 3: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR1 licensed TDD cell(s) with SCS1 and FR2-1 cell(s) with SCS2. - SCS1 can be same or different to SCS2. - Case 4: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR1 licensed FDD cell(s) with different SCS. - Case 5: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR1 licensed TDD cell(s) with different SCS. - Case 6: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR2-1 cell(s) with different SCS.

According to the description in WID [1], some scenarios with high commercial demand for operators are listed, including Case 1, Case 2, and Case 3.

Due to quite limited TU for Rel-18 multi-carrier enhancements, some important use cases were excluded from Rel-18, e.g., different SCSs among co-scheduled cells, different carrier types among co-scheduled cells. Co-scheduled carriers with different SCSs have high commercial needs for operators, e.g., 3.5GHz TDD + Sub-3GHz FDD, FR1 + FR2, etc.

Case 1 is a typical complementary deployment of TDD and FDD bands, such as Band n41+ Band n28. In this scenario, the FDD band is leveraged to provide wide-area coverage, while the TDD band is used to deliver high-capacity services. This complementary approach ensures both broad coverage and high data throughput, making it a commercially valuable and widely adopted deployment scenario.

Case 2 and Case 3 are typical multi-frequency deployment scenarios involving FR1 and FR2-1 cells, such as Band n78 + Band n258. These two scenarios contribute to enhanced power saving gains and reduced PDCCH overhead for FR2 cells. Therefore, supporting such configurations with DCI format 0_3/1_3 aligns with the objectives of Rel-19 multi-cell scheduling, allowing the network to leverage both the coverage benefits of FR1 and the capacity advantages of FR2-1.

Therefore, supporting Case 1, Case 2, and Case 3 is in line with the intended scope of the WID.

Proposal 1: At least Case 1, Case 2 and Case 3 are supported for Rel-19 multi-carrier enhancement.

At the previous meeting, a company proposed introducing the following Case 7 to further discuss the potential scenarios that could be supported in Rel-19. While it is currently no commercial deployments requiring more than two different SCSs, future deployment scenarios involving up to three SCSs may arise considering emerging use cases based on e.g., mmWave, and improvement of UE support rate of multiple CCs across FR1 and FR2. Therefore, considering forward compatibility, we support configurations/scheduling with up to three SCSs in Rel-19 multi-carrier enhancement. This does not require additional specification work except for introduction of corresponding UE capabilities.

- Case 7: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR1 licensed FDD cell(s) with SCS1, TDD cell(s) with SCS2 and FR2-1 cell(s) with SCS3. - SCS1, SCS2 and SCS3 can be same or different.

Proposal 2: Support co-scheduled cells with up to three SCSs being scheduled by DCI format 0_3/1_3 in Rel-19.

Joint TDRA table

Based on the discussions at RAN1#120bis and the post-meeting email discussion, a preliminary agreement on the higher layer parameters related to Rel-19 multi-cell scheduling has been reached and included in [4]. The maximum number of rows for the DL joint TDRA table (TDRA-FieldIndexDCI-1-3-r19) and the UL joint TDRA table (TDRA-FieldIndexDCI-0-3-r19) remains to be determined.

It is observed that the TDRA table configured per BWP for multi-PUSCH/PDSCH scheduling by DCI format 0_3/1_3 adopts the same structure and size as the configurable TDRA table defined in Rel-17. Specifically, the maximum number of rows in the TDRA table for Rel-19 multi-PDSCH scheduling, maxNrofDL-AllocationsExt-r19-1, is 63, which is the same as that of maxNrofDL-AllocationsExt-r17 defined for Rel-17 multi-PDSCH scheduling. Similarly, the maximum number of rows in the TDRA table for Rel-19 multi-PUSCH scheduling, maxNrofUL-Allocations-1-r18, is 63, which is the same as maxNrofUL-Allocations-r16 defined for Rel-16/17 multi-PUSCH scheduling.

Compared to Rel-17 multi-PUSCH/PDSCH scheduling, a Rel-19 DCI format 0_3/1_3 extends the scheduling to up to 4 cells with different carrier types and SCSs. This enhancement will increase a great number of schedulable multi-PUSCH/PDSCH combinations. Given that the maximum values of the BWP-specific TDRA tables for multi-PUSCH/PDSCH remain unchanged from Rel-17, it is necessary to expand the maximum number of rows in the joint TDRA table to provide sufficient scheduling flexibility. According to the following specification [7], the maximum size of TDRA field in Rel-18 DCI format 0_3 is 6, while in DCI format 1_3 it is 5 bits. Based on the above discussion, a candidate value for the maximum number of rows in the joint TDRA table for Rel-19 DCI format 0_3/1_3 can be set to double the Rel-18 value as a starting point for further discussion.
For example, the maximum number of rows can be set to 64 for TDRA-FieldIndexListDCI-1-3-r19 and 128 for TDRA-FieldIndexListDCI-0-3-r19.

tdra-FieldIndexListDCI-1-3-r18 SEQUENCE (SIZE (1..32)) OF TDRA-FieldIndexDCI-1-3-r18 OPTIONAL, -- Need R tdra-FieldIndexListDCI-0-3-r18 SEQUENCE (SIZE (1..64)) OF TDRA-FieldIndexDCI-0-3-r18 OPTIONAL, -- Need R

Proposal 3: Support to increase the maximum number of rows in joint TDRA table for DCI format 1_3/0_3 in Rel-19 MCE.

- the maximum number of rows can be set to 64 for TDRA-FieldIndexListDCI-1-3-r19.

- the maximum number of rows can be set to 128 for TDRA-FieldIndexListDCI-0-3-r19.

Feature coexistence

The Draft CR for TS 38.214 [5] has already disallowed the use of DCI format 0_3/1_3 with slot aggregation and repetition simultaneously, which is consistent with the legacy mechanism. On the other hand, according to the following specification [6], TBoMS seems to be supported in Rel-18 DCI format 0_3.

- for PUSCH scheduled by DCI format 0_1, 0_2 or 0_3, if numberOfSlotsTBoMS is present and larger than 1, the UE applies TB processing over multiple slots procedure when determining the time domain resource allocation.

Provided no further modifications are introduced, DCI format 0_3 in Rel-19 multi-cell scheduling may also support TBoMS scheduling. However, as specified in TS 38.331 [7], multi-PUSCH scheduling and TBoMS are not allowed to be configured simultaneously in Rel-17.

pusch-TimeDomainAllocationListForMultiPUSCH Configuration of the time domain resource allocation (TDRA) table for multiple PUSCH (see TS 38.214 [19], clause 6.1.2). The network configures at most 64 rows in this TDRA table in PUSCH-TimeDomainResourceAllocationList-r16 configured by this field. This field is not configured simultaneously with pusch-AggregationFactor. The network does not configure the pusch-TimeDomainAllocationListForMultiPUSCH-r16 simultaneously with the pusch-TimeDomainAllocationListDCI-0-1-r16. The network does not configure the pusch-TimeDomainAllocationListForMultiPUSCH-r16 simultaneously with the numberOfSlotsTBoMS-r17.

TBoMS was introduced in Rel-17, primarily targeting coverage-limited scenarios, similar to the features of slot aggregation and repetition. In contrast, Rel-19 multi-cell scheduling is designed to increase data rates and reduce PDCCH overhead. Given that TBoMS and Rel-19 multi-cell scheduling serve different use cases, and other similar coverage enhancements are not supported either, simultaneous configuration for both features is lack of justification, and also requires more specification work.

Therefore, based on the above discussion, TBoMS and the Rel-19 DCI format 0_3 should not be configured simultaneously in Rel-19 multi-cell scheduling.

Proposal 4: TB processing over multiple slots (TBoMS) is not supported for Rel-19 DCI format 0_3.

The number of HARQ-ACK bits for PUCCH power control

The issue of the number of HARQ-ACK bits for PUCCH power control in Rel-19 multi-carrier enhancement was raised in the previous meeting, but not discussed. In draftCR TS 38.213 [8], the editor also mentioned that this issue might need to be discussed in RAN1.

Rel-19 DCI format 0_3/1_3 is a further enhancement of Rel-18 DCI format 0_3/1_3, and in principle, discussions can be carried out based on the current specification in Rel-18. However, the related text in the current specification seems incorrect.

In Rel-18, due to the introduction of DCI format 1_3, the relevant formula was modified as follows.

If OACK+OSR+OCSI≤11 and NsetsDL>0, for obtaining a PUCCH transmission power as described in clause 7.2.1, the UE determines nHARQ-ACK=nHARQ-ACK,0+nHARQ-ACK,1, where nHARQ-ACK,0 is the value of nHARQ-ACK for the first Type-2 HARQ-ACK sub-codebook and nHARQ-ACK,1 is the value of nHARQ-ACK for the second Type-2 HARQ-ACK sub-codebook that is determined as nHARQ-ACK=VDAI,mlastDL-s=0NsetsDL-1UDAI,smodTDNTB,maxDL,MC+s=0NsetsDL-1m=0M-1Nm,sreceived where - in the following - a DCI format 1_3 schedules more than one PDSCH receptions providing transport blocks with enabled HARQ-ACK information - a dormancy indication is considered as a PDSCH reception providing a single transport block with enabled HARQ-ACK information - VDAI,mlastDL is the value of the total DAI field in a last DCI format 1_3 the UE detects in a last PDCCH monitoring occasion within the M PDCCH monitoring occasions where the UE detects at least one DCI format 1_3. VDAI,mlastDL=0 if the UE does not detect any DCI format 1_3 in any of the M PDCCH monitoring occasions. - UDAI,s is the total number of DCI format 1_3 that the UE detects within the M PDCCH monitoring occasions for the set s of serving cells. UDAI,s=0 if the UE does not detect any DCI format 1_3 associated with scheduling on set s of serving cells in any of the M PDCCH monitoring occasions. - NTB,maxDL,MC=NsetsTB,max if harq-ACK-SpatialBundlingPUCCH is not provided; otherwise, NTB,maxDL,MC=Ncells,setDL,max. - Nm,sreceived is - the number of transport blocks, in PDSCH receptions not overlapping with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated if provided, associated with a DCI format 1_3 that the UE detects in PDCCH monitoring occasion m for set s of serving cells, if harq-ACK-SpatialBundlingPUCCH is not provided - the number of more than one PDSCHs, not overlapping with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated if provided, scheduled by a DCI format 1_3 that the UE detects in PDCCH monitoring occasion m for set s of serving cells, if harq-ACK-SpatialBundlingPUCCH is provided

According to the above specification, nHARQ-ACK is defined as the sum of the corresponding number of HARQ-ACK bits in the first and second HARQ-ACK sub-codebooks. However, the texts in above specification describe the variable Nm,sreceived in the formula only with respect to the second HARQ-ACK sub-codebook. This is inconsistent with the preceding definition in the text of the specification.

Therefore, the issues in the above Rel-18 specification need to be clarified/resolved first. We propose to defer the related discussions for Rel-19 MCE in order to consider a consistent approach based on what Rel-18 will achieve.

Proposal 5: Defer the discussion of PUCCH power control issue for Rel-19 MCE to maintenance phase, till that for Rel-18 MCE is resolved.

Conclusion

According to the above discussions, we have the following observations and proposals:

Proposal 1: At least Case 1, Case 2 and Case 3 are supported for Rel-19 multi-carrier enhancement.

Proposal 2: Support co-scheduled cells with up to three SCSs being scheduled by DCI format 0_3/1_3 in Rel-19.

Proposal 3: Support to increase the maximum number of rows in joint TDRA table for DCI format 1_3/0_3 in Rel-19 MCE.

-	the maximum number of rows can be set to 64 for TDRA-FieldIndexListDCI-1-3-r19.

-	the maximum number of rows can be set to 128 for TDRA-FieldIndexListDCI-0-3-r19.

Proposal 4: TB processing over multiple slots (TBoMS) is not supported for Rel-19 DCI format 0_3.

Proposal 5: Defer the discussion of PUCCH power control issue for Rel-19 MCE to maintenance phase, till that for Rel-18 MCE is resolved.

- References

- RP-242408, “New WID: Multi-carrier enhancements for NR Phase 2”, RAN#105, September 9-12, 2024.

- R1-2503039, “Feature lead summary #3 on multi-cell scheduling with a single DCI”, RAN1#120bis, April 7th – 11th, 2025.

- R1-2409341, “Final Minutes report RAN1#118b, RAN1#118bis”, October 14th – 18th, 2024.

- R1-2503155, “Consolidated Rel-19 higher layers parameters list Post RAN1#120bis”, Moderator (Ericsson), RAN#120bis, April 7th-11th, 2025.

- R1-2503484, Introduction of Multi-carrier enhancements for NR Phase 3.

- TS 38.214, Physical layer procedures for data (Release 18).

- TS 38.331, Radio Resource Control (RRC) protocol specification (Release 18).

- R1-2503173, Introduction of multi-carrier enhancements for NR Phase 3.