3GPP TSG-RAN WG1 Meeting #121	R1-2503293

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	9.15.5

Source:	Huawei, HiSilicon

Title:	UE features for Rel-19 LP-WUS

Document for:	Discussion and Decision

# Introduction

In RAN1#120, UE features for Rel-19 LP-WUS were discussed, and a basic framework was agreed [1]. In this paper, the FFS parts of each row are discussed.

# Discussion

## IDLE/INACTIVE mode (i.e., FG 62-1 and FG 62-1a)

| 62-1 | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OOK [2. The support of LP-SS based RRM measurement and/or SSB-based measurement, corresponding to component 1] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [5. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal is not supported | Per Band | n/a | n/a | n/a | Component 1 candidate values: {OOK-based, OFDM-based, both} Component 5 candidate values: FFS [For UE supporting both X-1 and X-2, UE is not allowed to report different candidate values for component 1] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 62-1a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OFDM overlaid sequence [2. FFS: The support of LP-SS based RRM measurement and/or [X. FFS: The support of SSB-based RRM measurement] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [5. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: {OOK-based, OFDM-based, both} Component 5 candidate values: FFS [For UE supporting both X-1 and X-2, UE is not allowed to report different candidate values for component 1] | Optional with capability signalling |

The above was agreed as the starting point for further discussion. For the existing components marked as yellow (i.e., unstable), we have the following comments:

- For component 2, for a LP-WUR which can monitor LP-WUS via OOK modulation, it is natural to support LP-SS based RRM measurement. For a LP-WUR which can monitor LP-WUS via overlaid sequence, RAN4 has already agreed to define the same requirements for LP-SS based RRM measurement for OFDM based receiver as OOK based receiver [2]. Thus, component 2 should be confirmed for both FG 62-1 and FG 62-1a.

- For component 3 and 4, the RRM relaxation/offloading is mainly discussed in RAN2 and RAN4. It is better to leave it to RAN2 or RAN4 to define. Thus, component 3 and 4 can be removed for both FG 62-1 and FG 62-1a. However, based on the simulation results during the study item phase, the power saving gain provided by LP-WUS itself may be marginal, if RRM measurement is not offloaded/relaxed. Therefore, a note should be added for FG 62-1 and FG 62-1a that ‘a UE reporting support of this FG shall support offloading and relaxation’.

- For component 5, reporting wakeup delay is a key feature for LP-WUS in IDLE/INACTIVE, which significantly impacts the UE timeline. Thus, component 5 should be confirmed for both FG 62-1 and FG 62-1a, and the candidate values can be further added after they are agreed.

- For component X of FG 62-1a, because LP-WUR cannot achieve the same RAN4 accuracy requirement as Rel-15 SSB-based RRM measurement defined for MR and such fact is not clearly reflected in RAN4 progress yet, the component X of FG 62-1a should be pending.

Beside the above existing components, the following components are also necessary:

- The M value (i.e., number of OOK symbols per OFDM symbol) is a key parameter for LP-WUS, which defines the granularity to process received signal. Per our understanding, supporting all M values is not a big burden to UE. Thus, a new component ‘support of M=1, 2 and 4, where M is the number of OOK symbols per OFDM symbol’ should be added for both FG 62-1 and FG 62-1a.

- Beside M value, the SCS is also another key parameter to defines the granularity to process received signal. Also, the SCS impacts the BW of LP-WUS/LP-SS. Therefore, a new component on SCS and BW should be added for both FG 62-1 and FG 62-1a, i.e., ‘support of both SCS 15kHz and 30kHz of CP-OFDM symbol embedded with LP-WUS for FR1 and both SCS 60kHz and 120kHz for FR2. The SCS of CP-OFDM symbol embedded with LP-SS is the same as that of LP-WUS. Both the bandwidths of LP-WUS and LP-SS are 11 PRBs’.

- Exit condition is an important UE procedure, which ensures the UE performance for paging information reception. Thus, a new component on should be added for both FG 62-1 and FG 62-1a, i.e., ‘support of exit operation based on configured channel condition threshold’.

- It was agreed in RAN1#120 that the maximum number of subgroups per PO is 31. Also, while one to multiple mapping between LO and POs was agreed, there was a restriction saying that the maximum number of codepoints would not increase (i.e., up to 32 codepoints per LO) [4]. Therefore, a new component on should be added for both FG 62-1 and FG 62-1a, i.e., ‘Support of up to 31 number of subgroups per PO and up to 32 codepoint values per LO.’

Based on the discussion above, we have the following proposal

- On the UE feature components of LP-WUS in IDLE/INACTIVE mode,

- Component 2 and 5 are confirmed for both FG 62-1 and 62-1a

- Component 3 and 4 are removed for both FG 62-1 and 62-1a

  - A note is added: a UE reporting support of this FG shall support offloading and relaxation

- New components are added for both FG 62-1 and 62-1a, including

  - Support of M=1, 2 and 4, where M is the number of OOK symbols per OFDM symbol.

  - Support of both SCS 15kHz and 30kHz of CP-OFDM symbol embedded with LP-WUS for FR1 and both SCS 60kHz and 120kHz for FR2. The SCS of CP-OFDM symbol embedded with LP-SS is the same as that of LP-WUS. Both the bandwidths of LP-WUS and LP-SS are 11 PRBs.

  - Support of exit operation based on configured channel condition threshold.

  - Support of up to 31 number of subgroups per PO and up to 32 codepoint values per LO.

## CONNECTED mode (i.e., FG 62-2, FG 62-2a and FG 62-3)

| 62-2 | LP-WUS operation in CONNECTED mode based on OOK signal | 1. LP-WUS operation in CONNECTED mode based on OOK signal 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode FFS: M values FFS: SCS FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in CONNECTED mode based on OOK signal is not supported | [Per Band] | n/a | n/a | n/a | Component 2 candidate values: FFS Component 3 candidate values: FFS | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 62-2a | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence | 1. LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode FFS: M values FFS: SCS FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence is not supported | [Per Band] | n/a | n/a | n/a | Component 2 candidate values: FFS Component 3 candidate values: FFS | Optional with capability signalling |
| 62-3 | LP-WUS frequency resource outside active DL BWP in addition to the one within active DL BWP for LP-WUS operation in CONNECTED mode | 1. Support of LP-WUS frequency resource outside active DL BWP in addition to the one within active DL BWP for LP-WUS operation in CONNECTED mode | FG 62-2 or 62-2a | YES | n/a | LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode is not supported | [Per band] | n/a | n/a | n/a |  | Optional with capability signalling |

The above was agreed as the starting point for further discussion. For the existing components marked as yellow (i.e., unstable), we have the following comments:

- For component 2, currently two options of basic UE procedure are agreed, i.e., Option 1-1 (where LP-WUS triggers drx-OnDurationTimer) and Option 1-2 (where LP-WUS triggers a new timer). In our view, there is no different hardware requirement for supporting the two options. And we suggest to have a unified UE feature for the two options on UE procedure in CONNECTED mode. Thus, component 2 should be modified for both FG 62-2 and FG 62-2a as ‘support UE procedure based on both Option 1-1 and Option 1-2’.

- For component 3, Minimum time gap acts as the same role as the wakeup delay, which is a key feature for LP-WUS in CONNECTED mode and significantly impacts the UE timeline. Thus, component 3 should be confirmed for both FG 62-2 and FG 62-2a, and the candidate values can be further added after they are agreed.

Beside the above existing components, the following components are also necessary:

- For the similar analysis as in Section 2.1, a new component ‘support of M=1, 2 and 4, where M is the number of OOK symbols per OFDM symbol’ should be added for both FG 62-2 and FG 62-2a.

- For the similar analysis as in Section 2.1, a new component on SCS and BW should be added for both FG 62-2 and FG 62-2a while LP-SS is not necessarily mentioned, i.e., ‘support of both SCS 15kHz and 30kHz of CP-OFDM symbol embedded with LP-WUS for FR1 and both SCS 60kHz and 120kHz for FR2. The bandwidths of LP-WUS are 11 PRBs’.

- It was agreed in RAN1#120 that the maximum number of codepoints to be checked by UE per MO depends on UE capability [4]. Therefore, a new component should be added for both FG 62-2 and FG 62-2a, i.e., ‘the maximum number of codepoints to be checked by UE per MO’.

- It was agreed in RAN1#120 that a basic UE capability is to receive LP-WUS within active DL BWP  [4]. Therefore, a new component should be added for both FG 62-2 and FG 62-2a, i.e., ‘support of receiving LP-WUS within Active BWP’.

Besides, now columns should be added

- In RAN1 #118bis, it was agreed that UE can indicate its preference for configured time offset between LP-WUS and PDCCH reception using UAI mechanism, where UAI is optional [3]. Therefore, a new row should be added, i.e., ‘support of UAI indication of additional [larger] offset where the offset is between the triggering LP-WUS and triggered PDCCH for LP-WUS in CONNECTED mode’. The Prerequisite feature groups of this newly added row can be 62-2 and/or 62-2a.

Based on the discussion above, we have the following proposal

- On the UE features of LP-WUS in CONNECTED mode,

- Component 2 is updated as ‘support UE procedure based on both Option 1-1 and Option 1-2’.

- Component 3 is confirmed

- New components are added for both FG 62-2 and 62-2a, including

  - Support of M=1, 2 and 4, where M is the number of OOK symbols per OFDM symbol.

  - Support of both SCS 15kHz and 30kHz of CP-OFDM symbol embedded with LP-WUS for FR1 and both SCS 60kHz and 120kHz for FR2. The bandwidths of LP-WUS are 11 PRBs.

  - The maximum number of codepoints to be checked by UE per MO

  - Support of receiving LP-WUS within Active BWP.

- New rows are added, for which the prerequisite feature groups is 62-2 and/or 62-2a

  - Support of UAI indication of additional [larger] offset where the offset is between the triggering LP-WUS and triggered PDCCH for LP-WUS in CONNECTED mode

## Type (granularity) of UE feature

Agreement Send an LS to RAN2 and RAN4 to convey the following In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE. Request RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs. Final LS in R1-2407559. Agreement: Adopt “Per band” for FG 62-1 and 62-1a:

Depending on different UE implementation, LP-WUR may be supported for different bands (see the agreed LS above). Some discussion in RAN1/RAN2/RANP also considers the case that LP-WUS reception is only supported by UE in some of the bands. Therefore, the type (i.e., the granularity) of UE feature for LP-WUS should be per band for single carrier operation. Additionally, in RAN1#120bis, “per band” was adopted for FG 62-1 and FG 62-1a. Thus, for CONNECTED mode, for single carrier operations, the type should be also confirmed as “per Band”.

- The type (i.e., the granularity) of UE feature for LP-WUS is per band for single carrier operation in CONNECTED mode.

## Summary

According to the above discussion, we have the following proposal.

- The table 1 in Appendix is adopted.

# Conclusions

This contribution presents our views on Rel-19 UE features for LP-WUS. Our proposals are summarized in the following.

- On the UE feature components of LP-WUS in IDLE/INACTIVE mode,

- Component 2 and 5 are confirmed for both FG 62-1 and 62-1a

- Component 3 and 4 are removed for both FG 62-1 and 62-1a

  - A note is added: a UE reporting support of this FG shall support offloading and relaxation

- New components are added for both FG 62-1 and 62-1a, including

  - Support of M=1, 2 and 4, where M is the number of OOK symbols per OFDM symbol.

  - Support of both SCS 15kHz and 30kHz of CP-OFDM symbol embedded with LP-WUS for FR1 and both SCS 60kHz and 120kHz for FR2. The SCS of CP-OFDM symbol embedded with LP-SS is the same as that of LP-WUS. Both the bandwidths of LP-WUS and LP-SS are 11 PRBs.

  - Support of exit operation based on configured channel condition threshold.

  - Support of up to 31 number of subgroups per PO and up to 32 codepoint values per LO.

- On the UE features of LP-WUS in CONNECTED mode,

- Component 2 is updated as ‘support UE procedure based on both Option 1-1 and Option 1-2’.

- Component 3 is confirmed

- New components are added for both FG 62-2 and 62-2a, including

  - Support of M=1, 2 and 4, where M is the number of OOK symbols per OFDM symbol.

  - Support of both SCS 15kHz and 30kHz of CP-OFDM symbol embedded with LP-WUS for FR1 and both SCS 60kHz and 120kHz for FR2. The bandwidths of LP-WUS are 11 PRBs.

  - The maximum number of codepoints to be checked by UE per MO

  - Support of receiving LP-WUS within Active BWP.

- New rows are added, for which the prerequisite feature groups is 62-2 and/or 62-2a

  - Support of UAI indication of additional [larger] offset where the offset is between the triggering LP-WUS and triggered PDCCH for LP-WUS in CONNECTED mode

- The type (i.e., the granularity) of UE feature for LP-WUS is per band for single carrier operation in CONNECTED mode.

- The table 1 in Appendix is adopted.

# References

R1-2501394, Session Notes of AI 9.15.5, Ad-Hoc Chair (NTT DOCOMO, INC.)

Chairman’s notes of RAN4 #114.

Chairman’s notes of RAN1 #118bis.

Chairman’s notes of RAN1 #120.

# Appendix

Table 1

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LP-WUS/WUR | 62-1 | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OOK 2. The support of LP-SS based RRM measurement 3. The supported wake-up delay 4. Support of M=1, 2 and 4, where M is the number of OOK symbols per OFDM symbol. 5. Support of both SCS 15kHz and 30kHz of CP-OFDM symbol embedded with LP-WUS for FR1 and both SCS 60kHz and 120kHz for FR2. The SCS of CP-OFDM symbol embedded with LP-SS is the same as that of LP-WUS. Both the bandwidths of LP-WUS and LP-SS are 11 PRBs. 6. Support of exit operation based on configured channel condition threshold. 7. Support of up to 31 number of subgroups per PO and up to 32 codepoint values per LO. |  | YES | n/a | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal is not supported | Per Band | n/a | n/a | n/a | Component 5 candidate values: FFS A UE reporting support of this FG shall support offloading and relaxation. | Optional with capability signalling |
| LP-WUS/WUR | 62-1a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OFDM 2. The support of LP-SS based RRM measurement 3. The supported wake-up delay 4. Support of M=1, 2 and 4, where M is the number of OOK symbols per OFDM symbol. 5. Support of both SCS 15kHz and 30kHz of CP-OFDM symbol embedded with LP-WUS for FR1 and both SCS 60kHz and 120kHz for FR2. The SCS of CP-OFDM symbol embedded with LP-SS is the same as that of LP-WUS. Both the bandwidths of LP-WUS and LP-SS are 11 PRBs. 6. Support of exit operation based on configured channel condition threshold. 7. Support of up to 31 number of subgroups per PO and up to 32 codepoint values per LO. |  | YES | n/a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM is not supported | Per Band | n/a | n/a | n/a | Component 5 candidate values: FFS A UE reporting support of this FG shall support offloading and relaxation. | Optional with capability signalling |
| LP-WUS/WUR | 62-2a | LP-WUS operation in CONNECTED mode based on OOK signal | 1. LP-WUS operation in CONNECTED mode based on OOK signal 2. Support UE procedure based on both Option 1-1 and Option 1-2 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. Support of M=1, 2 and 4, where M is the number of OOK symbols per OFDM symbol. 5. Support of both SCS 15kHz and 30kHz of CP-OFDM symbol embedded with LP-WUS for FR1 and both SCS 60kHz and 120kHz for FR2. The bandwidths of LP-WUS are 11 PRBs. 6. The maximum number of codepoints to be checked by UE per MO 7. Support of receiving LP-WUS within Active BWP. |  | YES | n/a | LP-WUS operation in CONNECTED mode based on OOK signal is not supported | Per Band | n/a | n/a | n/a | Component 2 candidate values: FFS Component 3 candidate values: FFS | Optional with capability signalling |
| LP-WUS/WUR | 62-2b | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence | 1. LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) 2. Support UE procedure based on both Option 1-1 and Option 1-2 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. Support of M=1, 2 and 4, where M is the number of OOK symbols per OFDM symbol. 5. Support of both SCS 15kHz and 30kHz of CP-OFDM symbol embedded with LP-WUS for FR1 and both SCS 60kHz and 120kHz for FR2. The bandwidths of LP-WUS are 11 PRBs. 6. The maximum number of codepoints to be checked by UE per MO 7. Support of receiving LP-WUS within Active BWP. |  | YES | n/a | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence is not supported | Per Band | n/a | n/a | n/a | Component 2 candidate values: FFS Component 3 candidate values: FFS | Optional with capability signalling |
| LP-WUS/WUR | 62-3 | LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode | 1. Support of LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode | FG 62-2 or 62-2a | YES | n/a | LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| LP-WUS/WUR | 62-4 | Offset indication via UAI in CONNECTED mode | 1. Support of UAI indication of additional [larger] offset where the offset is between the triggering LP-WUS and triggered PDCCH for LP-WUS in CONNECTED mode | 62-2 and/or 62-2a | Yes | n/a | UE preferred offset is not supported | Per Band | n/a | Yes | n/a |  | Optional with capability signalling |