3GPP TSG-RAN WG1 Meeting #121	R1-2503294

St Julian’s, Malta, May 19-23, 2025

Agenda Item:	9.4.1

Source:	Huawei, HiSilicon

Title:	Physical channels design on modulation

Document for:	Discussion and Decision

Introduction

In Rel-19, RAN WGs have completed a Rel-19 SI on solutions for Ambient IoT, reported in TR 38.769 [1], which explored more-detailed radio aspects. The Rel-19 WI is now expected to conclude in RAN1#121. Physical channels design on modulation for Ambient IoT needs to be discussed according to the constraints of extremely low device power consumption ~1 μW of device 1. The remaining issues are discussed in this paper.

(Copied from RP-243326 [2]) General Scope The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope: - The overall objective shall be to standardize the following Ambient IoT device: Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Deployment scenario 1 with Topology 1, according to D1T1-B. - FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum. - Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor. - Traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command). - Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769: - Case 1-4 for D1T1-B - Proximity determination via Solution 1 in TR 38.769 only. - Device (un)availability via Direction 1 in TR 38.769 only. … The following objectives are set, within the General Scope: - RAN1 scope: … - PRDCH and PDRCH, which are the only physical channels in R2D and D2R, respectively. … - R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations. …

R2D CP handling

In RAN1#120bis meeting agreement [3], the following agreement has been reached about R2D CP handling.

Agreement For the below agreement, further update on the followings Agreement For further down-selection among CP handing which retains subcarrier orthogonality, at least for PRDCH, at least Method Type 1 is supported - For supported M values <= 12 - RAN1 will not further pursue additional CP handling design - For supported M values > 12 - RAN1 will further down-select one from the followings - Option 1: Candidate 3 of M2-1-1 (as per agreements from RAN1#120) - Insert padding chips only at the end OOK chips of OFDM symbol - The last 2 out of M OOK chips at the end of an OFDM symbol are always ‘ON’ - Option 3: RAN1 will not further pursue additional CP handling design

Overall analysis of Option 1 and Option 3 for M = 24

In RAN1#120bis meeting, it has been agreed that M = {2, 6, 12, 24}. Then for M values > 12, the M value is only M = 24 for PRDCH.

In this Option 1, i.e. Candidate 3 of M2-1-1, the last 2 out of M OOK chips at the end of an OFDM symbol are always ‘ON’ as agreed in RAN1#120bis. This method ensures no false edge occurring in the CP for M = 24. While for Option 3, i.e. no additional CP handling design, the CP includes the false edge since the CP is copied from more than 1 chip from the end of an OFDM symbol. In the following figure, the waveform diagram of Option 1 and Option 3 is shown to illustrate this point.

Figure 1. The waveform diagram of Option 1 and Option 3 for M = 24

For Option 1, the device needs to skip the 2 ‘ON’ chips and the CP part to detect the PRDCH. For Option 3, the device needs to skip only the CP part to detect the PRDCH. The operation of skipping the 2 ‘ON’ chips is similar as the operation of skipping the CP part which is based on the nearest transition edge before and counting the remaining samples.

For M = 24, there are two cases where the performance may be impacted by the last more than 1 PRDCH chips of an OFDM symbol copied to the CP part of this OFDM symbol. Thus, for the choice of Option 1 and Option 3, we give analysis/evaluation for the two cases respectively in the following sections.

- For M = 24

  - Case 1: The OFDM symbol only contains PRDCH

    - PRDCH performance (CP between PRDCH and PRDCH)

  - Case 2: The OFDM symbol contains CAP and PRDCH

    - PRDCH start time detection (CP between SIP and CAP/PRDCH)

Case 1: PRDCH performance for M = 24

- Simulation results

In this section, we provide the evaluation results for M = 24 with the comparison of Option 1 Candidate 3 of M2-1-1 and Option 3 without additional CP handling design. To provide a comprehensive analysis for the two options, we provide the simulations of R2D TBS with 96 bits and 200 bits. Then, the simulation results are shown is in the following figure:

Figure 2. Performance comparison between Option 1 and Option 3 of M = 24

- On one hand, there are 4 M values to schedule from: {2, 6, 12, 24}. This gives the reader flexibility to schedule an appropriate combination of M value and TBS. There is no error floor for {200 bits, M = 24} with M1, so it can be used across the SNR range, or a reader can use a smaller M value if it wants to reach lower SNRs and a lower data rate is suitable.

- On the other hand, there is enhancement potential by adding Candidate 3. As shown by the simulation results, “Candidate 3+M1” has better performance than “M1 only” for TBS 96 bits and 200 bits. Especially, performance gain is observed for TBS 200 bits with “Candidate 3+M1”. The tradeoff is a rather small 2 ON chips out of 24 chips per OFDM symbol.

- Technical Analysis

The technical analysis of Option 1 and Option 3 is as below:

- Even if CP part is removed with M1, the clock inaccuracy at the device due to SFO will result in the residual false edge in the detection of PRDCH. Since the chip length of M = 24 is comparable with ½ CP length, the residual false edge in the CP part will degrade the PRDCH performance.

- A residual false edge in CP removal has a certain probability per OFDM symbol. Hence the more OFDM symbols the block-size requires increases the joint probability that one of them has an error, i.e. larger block sizes are more susceptible to residual edges than smaller block sizes.

- Summary

To allow for wider range of use cases, i.e. data rates, and requirements, we support Option 3 of Candidate 3 of M2-1-1 for M = 24.

Case 2: PRDCH start time detection for M = 24

In 9.4.3 [4], it has been agreed in RAN1#120bis meeting that the duration of SIP is 1 OFDM symbol with CAP pattern ON-OFF-ON-OFF for all values of M corresponding to PRDCH. The agreement is shown in the following.

Agreement For R-TAS, SIP duration of 1 OFDM symbol is adopted with CAP pattern ON-OFF-ON-OFF for all values of M corresponding to PRDCH - Note: device cannot assume the presence/absence of RF transmission prior to the SIP.

The CP between SIP and the following CAP is copied from either CAP or PRDCH and it would be with a single voltage ON or OFF for M = 2, 6 and 12. As analyzed in our paper [4], it is demonstrated that the CP uncertainty of the single voltage with ON or OFF has negligible impact on the SIP detection performance. Furthermore, the CAP detection performance is also not impacted by the CP part by utilizing the two-same direction falling edge to determine the chip length [4]. Thus, no additional CP handling design is needed for M = 2, 6 and 12.

Observation 1: For M = 2, 6 and 12, additional CP handling design is not needed for the OFDM symbol contains CAP. Negligible performance impacts are expected for the SIP and CAP detection.

However, for M = 24, the CAP ON-OFF-ON-OFF pattern occupies the first 4 chips of an OFDM symbol. Then the PRDCH chips follows the CAP chips in the same OFDM symbol. The CP part of this OFDM symbol would include the false edge since the CP part is copied from more than 1 PRDCH chip from the end of the OFDM symbol.

If Option 3 is adopted, the CP between SIP and CAP is copied from the two random PRDCH chips [ON OFF] or [OFF ON] in the end of the OFDM symbol, which is shown in the following figure.

Figure 3. The diagram of SIP/CAP with CP handling Option 3 of M = 24

- For the case CP is copied from PRDCH chips [ON OFF] in the end of the OFDM symbol, the CP part in this case introduces an additional false rising edge and an additional falling edge as the “Purple” part in the CP. For the SIP detection, the false rising edge will terminate the SIP detection and the [ON OFF] CP part could be misunderstood as the first [ON OFF] of the CAP. Consequently, the second [ON OFF] of CAP will be misunderstood as the first bit of the PRDCH which results in a false timing start of the PRDCH and unsuccessful detection of PRDCH.

- For the case CP is copied from PRDCH chips [OFF ON] in the end of the OFDM symbol, the CP part in this case introduces an additional false rising edge as the “Purple” part in the CP. For the SIP detection, when the first rising edge occurs after the last long OFF of SIP, the SIP detection is assumed to be terminated. Since the “Purple” part CP does not introduce a false falling edge, CAP part can use the two same direction falling edges as analysed in [4]. Consequently, the timing start of the PRDCH is correct which does not impact the detection of PRDCH.

Thus, in summary, if Option 3 is adopted for the OFDM symbol contains CAP for M = 24, it would possibly introduce the problem that the CP part will be misunderstood as part of the CAP and cause the false start time of PRDCH receiving.

On the other hand, if Option 1 is adopted, the CP between SIP and CAP is copied from the two fixed padding chips [ON ON] in the end of the OFDM symbol, which is shown in the following figure.

Figure 4. The diagram of SIP/CAP with CP handling Option 1 of M = 24

- For the fixed padding chips [ON ON] in the end of the OFDM symbol, the CP part in this case introduces an additional rising edge as the “Purple” part in the CP. For the SIP detection, when the first rising edge occurs after the last long OFF of SIP, the SIP detection is assumed to be terminated. Since the “Purple” part does not introduce a false falling edge within the CP part, CAP part can use the two same direction falling edges as analysed in [4]. Consequently, the start timing of the PRDCH is correct and does not impact the detection of PRDCH.

Thus, in summary, Option 1 with Candidate 3 of M2-1-1 is needed for this case where the OFDM symbol including the CAP and PRDCH for M = 24.

Observation 2: For M = 24, additional CP handling design Candidate 3 of M2-1-1 is needed for the OFDM symbol containing CAP. It will avoid the problem of misunderstanding the CP part as the CAP and the false detection of the PRDCH start time.

Summary and proposal

As discussed above, the following are our views on the down-selection between Option 1 (Candidate 3 of M2-1-1) and Option 3 (no additional design) for M = 24:

- For PRDCH performance, to allow for wider range of use cases, i.e. data rates, and requirements, we support Option 1 of Candidate 3 of M2-1-1 for M = 24.

- For PRDCH start time detection, Option 1 with Candidate 3 of M2-1-1 is needed for this case where the OFDM symbol including the CAP and PRDCH for M = 24. It will avoid the problem of misunderstanding the CP part as the CAP and the false detection of the PRDCH start time (results in unsuccessful PRDCH receiving).

Proposal 1: For PRDCH CP handling of M = 24, Candidate 3 of M2-1-1 is supported.

R2D chip duration

As agreed, there can be M = 2, 6, 12, 24 chips per OFDM symbol generated by an M-chip OOK waveform.

The chip duration of OOK-4 is a consequence of CP handling design. Either CP handling method Alt M1 or CP handling Candidate 3 of M2-1-1 only regards the chip duration is one OFDM symbol length without CP divided by M, which is calculated as (1/SCS)/M.

Proposal 2: For R2D, the chip duration is equal to one OFDM symbol length without CP divided by M i.e. (1/SCS)/M. For SIP, the number of (effective) chips is used, instead of M (decided in 9.4.3).

Figure 5. Illustration of an OOK chip within an OFDM symbol

R2D padding

In RAN1#120bis meeting agreement [3], the following agreement has been reached about R2D padding.

Agreement When the generated number of chips for the R2D transmission does not fully occupy the last OFDM symbol, padding is used. - Padding is to be down-selected among the following alternatives: - Alt 1a: The content of padding is up to reader implementation and transparent to device. - Note: the timeline determination of any timing relationship refers to the end of padding. - Note: it implies the device should be aware of the duration of padding or the last OFDM symbol boundary by implementation. FFS how accurately the device can be aware. - Alt 1b: The content of padding is up to reader implementation and transparent to device. - Note: the timeline determination of any timing relationship refers to the start of the padding. - Alt 2: Specify the detailed content of padding - Note: the timeline determination of any timing relationship refers to the end of padding. - Note: the end chip(s) of the padding content shall follow the CP handling solution determined in RAN1, and may be affected by other agreements.

- Relation to CP

For the padding content, the end chips of the padding content shall follow the CP handling solution. Thus, for M = 24, since Candidate 3 of M2-1-1 is proposed to be supported, the end 2 chips of the padding content should be two “ON” chips.

Proposal 3: For R2D padding, the end chip(s) of the padding content shall follow the CP handling method determined in section 2.1. For M = 24, the end 2 chips of the padding content shall be two “ON” chips.

- The timeline reference issue

If the timeline determination of any timing relationship refers to the start of the padding, then the Toffset (Toffset1, Toffset2) X μs for the D2R transmission needs to account for the time duration of padding. The following RAN4 agreement means that RAN1’s scheduling and timing relationships need to ensure that D2R transmission cannot begin before the end of R2D transmission.

- Agreement in RAN4#114bis: - CW transmission and A-IoT BS downlink data transmission are non-concurrent. - FFS whether or not introduce the synchronization requirement of 3 us between CW node and A-IoT BS

Then the worst case for the time duration of padding is close to one OFDM symbol in which only 1 chip is valid in one OFDM symbol for M = 24 and the remaining 23 chips in this OFDM symbol is for padding. A timing relationship would have to account for that case to honor the RAN4 agreement. However, for another extreme no-padding case in which the end of R2D transmission is aligned with the boundary of the last OFDM symbol, the start or end of padding is non-existent and the relationships can only be defined with reference to the end of an OFDM symbol. That timing relationship will have to be set long enough for the device, assuming no additional time is added by padding chips. This timing relationship is then suitable for any device, independent of how many padding chips are added.

It is not practical to define a whole range of intermediate timing relationships depending on the value of M and the number of padding chips.

It is therefore convenient to reference a timing relationship to the end of padding if there is any, or equivalently to reference to the end of an OFDM symbol.

Proposal 4: The timeline determination of any relationship refers to the end of padding (if any) or the end of the last OFDM symbol of R2D transmission.

- Time accuracy of the timeline for the device

R2D time tracking for the device is based on transition edges provided in R2D transmissions. Padding occurs in the last OFDM symbol and the largest length of padding is almost one OFDM symbol duration. If there is no transition edge in the last OFDM symbol, the time tracking accuracy might be degraded.

To ease the time error of the ensured timeline, the last OFDM symbol or last 1/2 OFDM symbol including the padding can include at least one transition edge to provide the reference for the timeline determination by the device. The device needs to know where in the padding chips the last transition edge will be, so that it can use it as a latest timing reference for counting to the end of an OFDM symbol.

Then, the most optimized design for the padding content is hence that the last transition edge is the nearest transition edge of the OFDM symbol including the padding occurs with the smallest timing error accumulated.

- For M = 24, the pattern of the end OFDM symbol including the padding is [OFF ON ON], assuming candidate 3 for CP handling is adopted.

- For M = 2, 6, 12, the pattern of the end OFDM symbol including the padding is [OFF ON] or [ON OFF]. Thus, the smallest timing error accumulated for the end of the OFDM symbol can be ensured.

Figure 6. Illustration of the optimized padding chips in the last OFDM symbol for M = 24

Observation 3: At least the last transition edge (one pair of [ON OFF] or [OFF ON]) with a fixed position is required in the last OFDM symbol including the padding content in order to provide a reference transition edge to ensure the OFDM symbol boundary with a limited time error.

Observation 4: At least the last transition edge with a fixed position is provided by the end chips of the last OFDM symbol to enable the smallest timing error accumulated for the timeline.

- The end 3 chips of the last OFDM symbol are [OFF ON ON] for M = 24

- The end 2 chips of the last OFDM symbol are [OFF ON] or [ON OFF] for M = 2, 6, 12

The above solution can deal with an even worse case which is analyzed in the following. In this case, there could be no transition edge for more than 1 OFDM symbol duration (i.e., 2 OFDM symbols).

Since the postamble content is aimed to be a violation of the Manchester encoding rule. Then, following the rule, the all “ON” or “OFF” postamble can be transmitted for the AIoT BS transmission. Then, if the content of the R2D padding is same as postamble without any specification, it will occur one situation that the overall OFDM symbol with all “ON” or “OFF” chips carrying both postamble and padding and none reference transition edge exists within the OFDM symbol. For example, for M = 2 with 3 “ON” chip postamble and 1 “ON” chip padding, it can exist the case that no transition edge is included within the last 2 OFDM symbols. The time error is even larger considering the time error accumulated for the device to detect the end of the last OFDM symbol.

Figure 7. Example of the postamble and padding chips in the last 2 OFDM symbol

Thus, in summary, Alt 1a or Alt 2 in RAN1#120bis for R2D padding is supported.

Proposal 5: R2D padding with Alt 1a or with Alt 2 in RAN1#120bis agreement is supported.

- If Alt 1a is supported, constraints need to be added that at least the provided last transition edge is required with a fixed position which is located in the last 1 or 1/2 OFDM symbol including the padding content, as well as to require the last two chips are [ON ON] for CP handling Option 3 when M = 24.

- If Alt 2 is supported, the end chips of the last OFDM symbol including the padding content is [OFF ON ON] for M = 24 and [OFF ON] or [ON OFF] for M = 2, 6 and 12. Note: The other chips of padding do not require specification.

Conclusions

In this contribution, the physical channels design on modulation for Ambient IoT are discussed and following observations and proposals are made accordingly.

Observation 1: For M = 2, 6 and 12, additional CP handling design is not needed for the OFDM symbol contains CAP. Negligible performance impacts are expected for the SIP and CAP detection.

Observation 2: For M = 24, additional CP handling design Candidate 3 of M2-1-1 is needed for the OFDM symbol containing CAP. It will avoid the problem of misunderstanding the CP part as the CAP and the false detection of the PRDCH start time.

Observation 3: At least the last transition edge (one pair of [ON OFF] or [OFF ON]) with a fixed position is required in the last OFDM symbol including the padding content in order to provide a reference transition edge to ensure the OFDM symbol boundary with a limited time error.

Observation 4: At least the last transition edge with a fixed position is provided by the end chips of the last OFDM symbol to enable the smallest timing error accumulated for the timeline.

- The end 3 chips of the last OFDM symbol are [OFF ON ON] for M = 24

- The end 2 chips of the last OFDM symbol are [OFF ON] or [ON OFF] for M = 2, 6, 12

Proposal 1: For PRDCH CP handling of M = 24, Candidate 3 of M2-1-1 is supported.

Proposal 2: For R2D, the chip duration is equal to one OFDM symbol length without CP divided by M i.e. (1/SCS)/M. For SIP, the number of (effective) chips is used, instead of M (decided in 9.4.3).

Proposal 3: For R2D padding, the end chip(s) of the padding content shall follow the CP handling method determined in section 2.1. For M = 24, the end 2 chips of the padding content shall be two “ON” chips.

Proposal 4: The timeline determination of any relationship refers to the end of padding (if any) or the end of the last OFDM symbol of R2D transmission.

Proposal 5: R2D padding with Alt 1a or with Alt 2 in RAN1#120bis agreement is supported.

- If Alt 1a is supported, constraints need to be added that at least the provided last transition edge is required with a fixed position which is located in the last 1 or 1/2 OFDM symbol including the padding content, as well as to require the last two chips are [ON ON] for CP handling Option 3 when M = 24.

- If Alt 2 is supported, the end chips of the last OFDM symbol including the padding content is [OFF ON ON] for M = 24 and [OFF ON] or [ON OFF] for M = 2, 6 and 12. Note: The other chips of padding do not require specification.

- References

- TR 38.769, “Study on solutions for ambient IoT (Internet of Things)”, December 2024.

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN#106, December 2024, Huawei (moderator).

- R1-2502994, “9.4.1 Final FL summary for Ambient IoT “9.4.1 Physical channels design-modulation aspects”, RAN1#120bis, Wuhan, China, April 2025.

- R1-2503296, “Physical signals design”, RAN1#121, St Julian’s, Malta, May 2025, Huawei, HiSilicon.

- Appendix

Simulation Assumptions of R2D

| Channel model | TDL-A 30ns |
| --- | --- |
| UE velocity | 3 km/h |
| SFO | [104 ppm, 105 ppm] |
| Transmission Bandwidth | 180*3 kHz |
| TBS | 96/200 bits |
| CRC | 6 bits (when TBS ≤ 24 bits) 16 bits (when TBS > 24 bits) |
| Line code | Manchester |
| Waveform | OOK-4 M = 24 |