3GPP TSG RAN WG1 Meeting #120-bis	R1- 2503449

Wuhan, China, April 7th – 11st, 2025

Source: 	Moderator (Intel Corporation)

Title:	Editor’s summary on draft CR 38.215 for NR_MIMO_Ph5

Agenda item:	9

Document for:	Discussion

# Introduction

In this contribution, editor of TS38.215 summarizes email discussions for introduction of Rel-19 MIMO Phase 5 features for TS38.215.

# Summary of Discussion

### Round#1 Discussion

Editor has provided initial version of the draft CR for TS38.215 containing introduction of Rel-19 MIMO Phase 5 features in

https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_120b/Inbox/drafts/9(NR_R19)/38.215%20draft%20CRs/NR_MIMO_Ph5/R1-250xxxx%20TS38215_Rel19_CRdraft_MIMO_v03.docx

Editor has asked companies to provide inputs and comments on initial version.

##### Company Comments for Initial version of draft CR

| Company | Comments |
| --- | --- |
| Google | For phase offset measurement, it is agreed that UE receives the CSI-RS based on the antenna port associated with one of the SRS ports configured by the network. So the receive branch should be based on the one associated with the configured SRS port. Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), - For a given phase offset reporting configuration, the UE can be configured (via higher-layer/RRC signaling) with Q associated SRS resource(s) for antenna switching - FFS: The supported value(s) of Q - The UE antenna port(s) for receiving the CSI-RS configured for phase offset measurement are same as the UE antenna port(s) for transmitting the selected/configured port(s) from the associated SRS resource(s) - For discussion purposes only (not necessarily for specification), the UE antenna port is referred to as the ‘reference UE antenna port’ Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), - Regarding the number of configured associated SRS resource(s) (=Q) for antenna switching xTyR, support at least Q=1 where: - the configured associated SRS resource is selected from all the y/x SRS resources and all the configured resource set(s) - FFS (by RAN1#118): whether Q>1 is also supported - FFS (by RAN1#118): the supported value(s) of x - Regarding how to determine the SRS port corresponding to the ‘reference UE antenna port’, support PSRS =1 SRS port selected from all the ports from the configured Q associated SRS resource(s) - FFS (by RAN1#118): Whether PSRS >1 is also supported Conclusion For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the number of configured associated SRS resource(s) (=Q) for antenna switching xTyR and the number of SRS ports (=PSRS) corresponding to the ‘reference UE antenna port’, there is no consensus to support the following: - Q>1 - PSRS >1 Note: This implies no support for configuring >1 SRS resource sets for associated SRS when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset). Note: There is a source R1-2407184 showing performance benefit of using multiple UE antenna ports non-coherent with each other Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), the selection of PSRS=1 SRS port corresponding to the ‘reference UE antenna port’ (out of available port(s)) is NW-configured via higher-layer (RRC) signalling Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the configured associated SRS resource, - Confirm the following working assumption as agreement: “the configured associated SRS resource can be either periodic, semi-persistent, or aperiodic” - When periodic or semi-persistent associated SRS resource is configured, the SRS transmission occasion for determining the reference UE antenna port corresponds to the latest SRS transmission occasion before the occasions of the NTRP CSI-RS resources used for measuring ‘cjtc-P’ report - FFS: Whether ‘the earliest SRS transmission occasion after the NTRP CSI-RS occasions’ is also supported as an option - FFS: Whether determination of SRS transmission occasion is needed for aperiodic associated SRS resource, and if so, how Proposed change ------------------------ start of TP --------------------------------- ### 5.1.52 Coherent joint transmission calibration phase offset Definition; Coherent joint transmission calibration phase offset is defined as the relative phase offset, Φn or Φn,s , between reference CSI-RS resource, nref, and selected CSI-RS resource over the entire configured reporting band or over the indicated subband, s, respectively. Determination of reference CSI-RS resource, nref, and determination of phase offset, labelled as ‘CJTC-P’, are describe in Clause 5.2.1.4.2 and 5.2.1.4.8 of [6, TS38.214], respectively. For frequency range 1 and 2, the reported coherent joint transmission calibration phase offset shall be measured on the receiver branch associated with the configured port for an SRS resource in the latest SRS transmission occasion before the occasions of the corresponding CSI-RS resources. if receiver diversity is in use by the UE, the reported coherent joint transmission calibration phase offset value shall be no lower than the minimum and no higher than the maximum measured values across the receiver branches. For frequency range 1, the reference point for the coherent joint transmission calibration phase offset shall be the antenna connector of the UE. For frequency range 2, coherent joint transmission calibration phase offset shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch.. Applicable for; RRC_CONNECTED. ------------------------ end of TP --------------------------------- |
| ZTE | Comment #1: coherent joint transmission calibration (CJTC) delay offset The CJTC delay offset and out-of-interval indicator are two separate quantities, and the ‘CJT-Dd’ report includes both the delay offset and the out-of-interval indicator. So, we suggest to define the delay offset and the out-of-interval indicator separately. Proposed change: ### 5.1.50 Coherent joint transmission calibration delay offset Definition; Coherent joint transmission calibration delay offset is defined as the relative receive time difference, Dn,offset, between reference CSI-RS resource set, nref, and selected CSI-RS resource setdn. Out-of-interval indicator, dn, indicates corresponding relative receive time difference plus delay spread is not larger than the duration of the cyclic prefix for symbols other than 0 and 7∙2μ and set to ‘1’, otherwise set to ‘0’, Determination of reference CSI-RS resource set, nref, and determination of delay offset and out-of-interval indicator, labelled as ‘CJTC-Dd’, are describe in Clause 5.2.1.4.2 and 5.2.1.4.6 of [6, TS38.214], respectively. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported coherent joint transmission calibration delay offset value shall be no lower than the minimum and no higher than the maximum measured values across the receiver branches. For frequency range 1, the reference point for the coherent joint transmission calibration delay offset shall be the antenna connector of the UE. For frequency range 2, coherent joint transmission calibration delay offset shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch.. Applicable for; RRC_CONNECTED. Comment #2: CJTC phase offset According to the following agreement, for CJTC phase offset measurement, UE is required to use a same antenna port to receive the configured CSI-RS and transmit a selected/configured SRS port from associated SRS resource(s). In other words, the antenna port for measuring the phase offset is a specific antenna port indicated by NW. So, the description of reference point for measurement for CJTC phase offset may be NOT needed. Agreement: RAN1#117: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), - For a given phase offset reporting configuration, the UE can be configured (via higher-layer/RRC signaling) with Q associated SRS resource(s) for antenna switching - FFS: The supported value(s) of Q - The UE antenna port(s) for receiving the CSI-RS configured for phase offset measurement are same as the UE antenna port(s) for transmitting the selected/configured port(s) from the associated SRS resource(s) - For discussion purposes only (not necessarily for specification), the UE antenna port is referred to as the ‘reference UE antenna port’. Proposed change: ### 5.1.52 Coherent joint transmission calibration phase offset Definition; Coherent joint transmission calibration phase offset is defined as the relative phase offset, Φn or Φn,s , between reference CSI-RS resource, nref, and selected CSI-RS resource over the entire configured reporting band or over the indicated subband, s, respectively. Determination of reference CSI-RS resource, nref, and determination of phase offset, labelled as ‘CJTC-P’, are describe in Clause 5.2.1.4.2 and 5.2.1.4.8 of [6, TS38.214], respectively.. Applicable for; RRC_CONNECTED. |
| Editor | @Google: Editor thinks the text for reference point doesn’t necessarily contradict the agreement on SRS port selection and measurement quantity. The currently description simply states if receiver diversity is used something is done. For the 1 port case, receiver diversity may not be used if there is 1 to 1 mapping between port and receiver branches. However, the specification does not prohibit implementation of 1 logical port to multiple transceiver branch mapping. This is why the language in the measurement doesn’t talk about ports. If companies think further clarification is needed, then we could add the information regarding the agreement on port selection. With this said, for timing related measurement it is odd to have min report across branches, out UE uses the different branches should be left up to implementation as in other TOA like measurements. So this will be deleted and will take Google’s suggested text. @ZTE: For separation of out-of-interval indicator and offset. Editor doesn’t think there is any technical material difference compared to what you are suggesting and what is currently available. Editor can update the third paragraph to include the out-of-interval indicator for reference CSI-RS resource set, as this is technically relevant, but removal of the out-of-interval indicator from the first paragraph or not doesn’t seem to make any technical difference. Also all the agreements seems to hint that the UE makes a measurement to derive two reporting quantity tuple. It is difficult to understand if there is technical justification from agreements to separate them out. For the reference point issue. Completely deleting the reference point doesn’t seem to wise as all other measurement quantities have reference point definitions, it would not be good to start to make exceptions to this. The best method is to clarify anything further that needs to be made. Please see response to google’ comment above. |

### Round#2 Discussion

Editor has provided initial version of the draft CR for TS38.215 containing introduction of Rel-19 MIMO Phase 5 features in

https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_120b/Inbox/drafts/9(NR_R19)/38.215%20draft%20CRs/NR_MIMO_Ph5/R1-250xxxx%20TS38215_Rel19_CRdraft_MIMO_v05.docx

Note in v04 to v05, the reference point for measurement text was moved from delay offset to phase offset. It was added to the wrong section by mistake.

Editor has asked companies to provide inputs and comments on revision. The following are summary of changes made:

- Updated the reference point for measurement (based on Google’s suggestion).

- Updated text to clarify reference CSI-RS resource set for out-of-interval report.

##### Company Comments for Initial version of draft CR

| Company | Comments |
| --- | --- |
| ZTE | Comment #1: Reference for measurement For CJTC-Dd/FO/PO report, since there is no explicit agreement on reference point for measurement, can be put the corresponding descriptions into square brackets for now? Comment #2: Editorial issue for CJTC-PO report In the description of reference point for measurement for CJTC-PO report, the ‘phase offset’ is wrongly written as ‘delay offset’. Proposed change (section 5.1.52): ### 5.1.52 Coherent joint transmission calibration phase offset Definition; Coherent joint transmission calibration phase offset is defined as the relative phase offset, Φn or Φn,s , between reference CSI-RS resource, nref, and selected CSI-RS resource over the entire configured reporting band or over the indicated subband, s, respectively. Determination of reference CSI-RS resource, nref, and determination of phase offset, labelled as ‘CJTC-P’, are describe in Clause 5.2.1.4.2 and 5.2.1.4.8 of [6, TS38.214], respectively. [For frequency range 1 and 2, coherent joint transmission calibration phase offset shall be measured on the receiver branch(es) associated with the configured port for an SRS resource in the latest SRS transmission occasion before the occasions of the corresponding CSI-RS resources. For frequency range 1, the reference point for the coherent joint transmission calibration phase offset shall be the antenna connector of the UE. For frequency range 2, coherent joint transmission calibration phase offset shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch.]. Applicable for; RRC_CONNECTED.. |
| Editor | @ZTE: Will add the brackets. Updated the typo. |

### Round#3 Discussion

Editor has provided initial version of the draft CR for TS38.215 containing introduction of Rel-19 MIMO Phase 5 features in

https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_120b/Inbox/drafts/9(NR_R19)/38.215%20draft%20CRs/NR_MIMO_Ph5/R1-250xxxx%20TS38215_Rel19_CRdraft_MIMO_v06.docx

Editor has asked companies to provide inputs and comments on revision. The following are summary of changes made:

- Added bracket for reference point.

- Corrected typo delay –> phase for phase offset measurement.

##### Company Comments for Initial version of draft CR

| Company | Comments |
| --- | --- |

### Summary of Discussion

Draft CR v06 has been endorsed by the Chairman.