3GPP TSG RAN WG1 #121		                        R1-2503519

Saint Julian, Malta, May 19th – 23th, 2025

Agenda Item:	9.5.1

Source:	Spreadtrum, UNISOC

Title: 	Discussion on on-demand SSB SCell operation

Document for:	Discussion and decision

Introduction

It was agreed in WID [1] to specify OD-SSB for SCell operation.

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

We will share our view on OD-SSB in this contribution.

Frequency relationship of AO-SSB and on-demand SSB

In RAN1#116bis [2], it was agreed that whether AO-SSB is cell-defining SSB or not depending on network implementation. In RAN1 #118 [3], it was agreed that OD-SSB SSB is not located on synchronization raster and is NCD SSB. Whether support of OD-SSB for CD-SSB located on sync raster or not is FFS. In RAN1#119 [4], the frequency location of AO-SSB and OD-SSB had been discussed.

Agreement Down-select at least one of the following alternatives. - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - Alt 2: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB - Alt 3: Do not support the case where always-on SSB is CD-SSB on a synchronization raster. Down-select at least one of the following alternatives. - Alt A: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB can be same or different from the frequency location of always-on SSB, subject to its configuration. - Alt B: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB - Alt C: Do not support the case where always-on SSB is CD-SSB and not on a synchronization raster.

In RAN1#120 [5], the frequency location of AO-SSB and OD-SSB is determined when AO-SSB is CD-SSB on a sync raster.

Agreement Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability - Note: UE is not required to measure both AO-SSB and OD-SSB

For the case when AO-SSB is CD-SSB and not on a sync raster, Alt A is more flexible. We don’t see strong motivation to restrict AO-SSB and OD-SSB’s frequency location. We prefer to leave the decision to gNB implementation.

- For the case when AO-SSB is CD-SSB and not on a synchronization raster, the frequency location of OD-SSB and AO-SSB subject to network configuration.

Mechanism of OD-SSB transmission

In RAN1#120bis [6], it was agreed network can configure the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated and whether support deactivation via MAC CE is FFS when od-ssb-nrofBurst is configured.

Agreement For a cell supporting on-demand SSB SCell operation, for configuring the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofBurst), - Alt 2: The value range of od-ssb-nrofBurst is {N2 integer values} - N2= [8] - If od-ssb-nrofBurst for an on-demand SSB is NOT configured, the on-demand SSB is deactivated via MAC CE. - If od-ssb-nrofBurst for an on-demand SSB is configured, the on-demand SSB is deactivated based on the configured value for od-ssb-nrofBurst [or via MAC CE].

In our view, network can deactivate OD-SSB flexibly via MAC CE to achieve energy saving. Before the last OD-SSB burst is transmitted, network can deactivate OD-SSB via MAC CE and the remaining OD-SSB bursts are not transmitted.

- If od-ssb-nrofBurst for an on-demand SSB is configured, the on-demand SSB is deactivated based on the configured value for od-ssb-nrofBurst or via MAC CE.

CSI measurement

In RAN1#120bis, it was agreed a CSI report configuration can be associated with both AO-SSB and OD-SSB or OD-SSB only.

Agreement For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), - A CSI report configuration can be associated with - both of AO-SSB and OD-SSB - Which SSB to measure is up to UE implementation as long as RAN4 requirements are met - Applies at least for the case where AO-SSB and OD-SSB have the same center frequency and the same positions in the same SSB burst - FFS: It is up to UE to measure both SSBs or one of them if the higher layer timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “Configured”. - OD-SSB only - FFS: Applies at least for the case where AO-SSB and OD-SSB have the different center frequencies

For first FFS, in current spec, when timeRestrictionForChannelMeasurements is set to “configured”, the UE shall derive L1 measurement result based on the most recent SSB. In our view, when timeRestrictionForChannelMeasurements is set to “notConfigured”, it is up to UE to measure both SSBs or one of them.

TS 38.214 If the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to "notConfigured", the UE shall derive the channel measurements for computing L1-RSRP value reported in uplink slot n based on only the SS/PBCH or NZP CSI-RS, no later than the CSI reference resource, (defined in TS 38.211[4]) associated with the CSI resource setting. If the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to "Configured", the UE shall derive the channel measurements for computing L1-RSRP reported in uplink slot n based on only the most recent, no later than the CSI reference resource, occasion of SS/PBCH or NZP CSI-RS (defined in [4, TS 38.211]) associated with the CSI resource setting.

- It is up to UE to measure both SSBs or one of them if the higher layer timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “notConfigured”.

Conclusion

We have the following observations and proposals.

- For the case when AO-SSB is CD-SSB and not on a synchronization raster, the frequency location of OD-SSB and AO-SSB subject to network configuration.

- If od-ssb-nrofBurst for an on-demand SSB is configured, the on-demand SSB is deactivated based on the configured value for od-ssb-nrofBurst or via MAC CE.

- It is up to UE to measure both SSBs or one of them if the higher layer timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “notConfigured”.

Reference

- RP-242354, “Revised WID: Enhancements of network energy savings for NR”, Ericsson, RAN#105, Sep 9-12, 2024.

- 3GPP RAN1#116bis Chair’s Notes.

- 3GPP RAN1#118 Chair’s Notes.

- 3GPP RAN1#119 Chair’s Notes.

- 3GPP RAN1#120 Chair’s Notes.

- 3GPP RAN1#120bis Chair’s Notes.