3GPP TSG RAN WG1 Meeting #121	R1-2503577

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	9.7.2

Title:		Discussion on ISAC channel modelling

Source:	Samsung

Document for:	Discussion and Decision

# Introduction

The Rel-19 study items on “Study on channel modelling for integrated sensing and communications (ISAC) for NR” is endorsed in RAN#103 [1]. The objectives for this SI are shown below:

The focus of the study is to define channel modelling aspects to support object detection and/or tracking (as per the SA1 meaning in TS 22.137). The study should aim at a common modelling framework capable of detecting and/or tracking the following example objects and to enable them to be distinguished from unintended objects: - UAVs - Humans indoors and outdoors - Automotive vehicles (at least outdoors) - Automated guided vehicles (e.g. in indoor factories) - Objects creating hazards on roads/railways, with a minimum size dependent on frequency All six sensing modes should be considered (i.e. TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, UE monostatic). Frequencies from 0.5 to 52.6 GHz are the primary focus, with the assumption that the modelling approach should scale to 100 GHz. (If significant problems are identified with scaling above 52.6 GHz, the range above 52.6 GHz can be deprioritized.) For the above use cases, sensing modes and frequencies: - Identify details of the deployment scenarios corresponding to the above use cases. - Define channel modelling details for sensing using 38.901 as a starting point, and taking into account relevant measurements, including: - modelling of sensing targets and background environment, including, for example (if needed by the above use cases), radar cross-section (RCS), mobility and clutter/scattering patterns; - spatial consistency. It will be discussed at RAN#105 whether to include additional study beyond channel modelling for ISAC.

In this contribution, we will provide our views on ISAC channel modelling based on the agreement in the RAN #120bis meeting.

# Physical object model

  - 2.1 Bistatic RCS for UAV with large size and AGV

In the RAN1#120bis discussion, the bistatic RCS is formulated for vehicles with signle/multiple scattering points.

### [H] Proposal 4.1-1 - The WA on bistatic RCS for vehicle is reused for large size UAV and AGV.

For vehicles, multiple-scattering-point-based modeling captures key RCS features by aggregating contributions from various directions of the vehicles. Similarly, large UAVs and AGVs, despite their different geometries from vehicles are likely to exhibit comparable scattering characteristics that can be modelled by the same modelling methodology applied for vehicles. This allows the adaptation of vehicle-derived parameters—such as angle-dependent values/patterns of A*B1, attenuate factors and scattering point distributions—to UAVs/AGVs with minimal adjustments. Hence, the extension of bistatic RCS modeling to large-size UAVs and AGVs can leverage their inherent similarities to vehicles in terms of scattering mechanisms and parameterization.

Proposal 1: Extend the working assumption on bistatic RCS for vehicle to large size UAV and AGV with necessary parameter adaptations to gain implementation benefit from the unified framework.

In the post-RAN1#120 email discussion, the antenna pattern like approach became a candidate for RCS modeling methodology for the UAV with large size. Since RAN1 has already reached a consensus on this modeling method for automotive targets, it is reasonable to apply the same method to other target (at least UAV and AGV with large size).

#### Proposal 4.1-1-rev1: On the monostatic RCS for UAV with large size and AGV - The monostatic RCS for a scattering point of the target is generated by - The values/pattern A*B1, i.e., rcsdB(θ,φ) is deterministic based on incident/scattered angles rcsdB(θ,φ)=Gmax-min-σVdBθ+σHdB φ,σmax Where, σVdBθ=-min12θ-θcenterθ3dB2, σmax, σHdB φ=-min12φ-φcenterφ3dB2, σmax, - FFS how many rows of the values/pattern A*B1 are defined for the target - Note: each row has a defined applicable range of θ and φ FFS human RCS model 2

As described in our previous contribution [3], we conducted a simulation using a large UAV (1600 mm x 1600 mm x 300 mm @ 3.5 GHz). Figure 1 shows the results obtained from simulation, displaying data distribution over azimuth 0 – 360 degrees and zenith 0 – 180 degrees. Most high RCS values appear when the incident wave faces a flat surface such as the top (roof) or bottom of the UAV, with is accompanied by specular reflection. On the other hand, when the UAV and the incident wave become perpendicular, strong RCS occurs only at certain angles.

Based on these results, dividing each region into Head, Left, Back, Right, Bottom and Top (Roof) would be appropriate classification for deriving an antenna pattern RCS modeling method. Table 1 summarizes the values for each region, and based on this, RAN1 can discuss defining the monostatic RCS values for large size UAV.

Fig. 1 RCS value distribution according to angles

Table 1. monostatic RCS of large size UAV with single scattering point φcenter; φ3dB,k; θcenter; θ3dB,k; Gmax; σmax; Applicable Range of φ; Applicable Range of θ. Head; 0°; 14.37°; 90°; 14.37°; 5.83; 8.2; [315°,360°]∪[0°,45°]; [45°,135°]. Left; 90°; 3.90°; 90°; 3.90°; 6.8; 9.17; [45°,135°]; [45°,135°]. Back; 180°; 7.57°; 90°; 7.57°; 5.83; 8.2; 135°,225°; [45°,135°]. Right; 270°; 3.90°; 90°; 3.90°; 6.8; 9.17; [225°,315°]; [45°,135°]. bottom; /; /; 180°; 3.90°; 20.12; 22.5; 0°,360°; [135°,180°]. Roof; /; /; 0°; 3.90°; 20.12; 22.5; 0°,360°; [0°,45°].

Proposal 2: RAN 1 consider the antenna pattern-like approach for RCS modeling of large size UAV

Proposal 3: RAN 1 discuss the RCS values for large size UAV with table 1

  - 2.2 Bistatic RCS for UAV with small size and Human with RCS model 1

In the post-RAN1#120bis email discussion, the bistatic RCS for UAV with small size and human with RCS model 1 as following proposal is discussed

### Proposal 2.1-1-rev1 - The bistatic RCS of UAV with small size is modelled as - The values/pattern of A*B1 is given by rcsdBθi,ϕi,θs,ϕs=maxσRCS,A-Attenuatefactor,RCSdB,fsθi,ϕi,θs,ϕs - Component A, i.e., σRCS,A: same as component A of mono-static RCS for UAV of small size - Attenuatefactor=3sin(β2) dB, where β is the bi-static angle between incident ray and scattered ray, β is within 0 and 180 degree - Component B2: same as component B2 of mono-static RCS for UAV of small size - The bistatic RCS of Human with RCS model 1 is modelled as - The values/pattern of A*B1 is given by rcsdBθi,ϕi,θs,ϕs=maxA-Attenuatefactor,RCSdB,fsθi,ϕi,θs,ϕs - Component A, i.e., σRCS,A: same as component A of mono-static RCS for Human with RCS model 1 - Attenuatefactor=3sin(β2) dB, where β is the bi-static angle between incident ray and scattered ray, β is within 0 and 180 degree - Component B2: same as component B2 of mono-static RCS for Human with RCS model 1

The proposed formula for modeling the bistatic RCS of small UAVs and human model 1 offers a balanced approach that unifies monostatic and bistatic RCS under a single framework. By defining component A as the same for both monostatic and bistatic RCS, the formula simplifies the model and maintains compatibility with existing monostatic RCS definitions. This consistency is advantageous for practical applications and theoretical analysis. The inclusion of the attenuate factor, 3 sin(β/2) dB, effectively captures the angular dependence of bistatic RCS, aligning with observed trends where RCS decreases with increasing bistatic angle β.

Although the formula makes certain simplifications, such as assuming isotropic scattering for angular-independent RCS and this assumption may not fully capture all scattering phenomena, it provides a practical approximation that balances complexity and accuracy. Additionally, the formula's flexibility to incorporate forward scattering effects enhances its applicability across different scenarios.

Proposal 4: Support the proposal formula considering its consistency, simplicity, and practicality for modelling RCS in both monostatic and bistatic configurations, particularly for the specified target types.

  - 2.3 Bistatic/monostatic RCS for human with RCS model 2

In the post-RAN1#120bis email discussion, there is no agreement on any specific value of component A yet on UAV of large size, human with RCS model 2, vehicle with single/multiple scattering points, and AGV. The following proposal with two options is available to derive the value A.

### Proposal 2.2-1 For UAV of large size, human with RCS model 2, vehicle with single/multiple scattering points, and AGV, please indicate your preference between Option 1 and 2 - Option 1: A is the mean of monostatic RCS values in all 3D incident/scattered angles, and is applied to both monostatic and bistatic RCS - Option 2: A is the mean of bistatic RCS values in all 3D incident/scattered angles, and is applied to both monostatic and bistatic RCS

Option 1 prioritizes monostatic sensing. Since monostatic RCS data is more widely studied and validated in existing literature and practical implementations, so it offers a reliable foundation for modeling. While bistatic RCS values may differ, using the monostatic mean simplifies calculations and maintains compatibility with current frameworks. This approach avoids introducing additional complexity or requiring new bistatic measurements, which are less standardized and computationally intensive. Furthermore, since the same ISAC channel is generated regardless of the chosen A value, Option 1 provides a practical solution that balances accuracy with simplicity. Given the lack of consensus on bistatic RCS values, Option 1 seems to offer a robust, widely applicable solution for diverse targets (UAVs, humans, vehicles, AGVs) without compromising generality.

Proposal 5: Support option 1 for UAV of large size, human with RCS model 2, vehicle with single/multiple scattering points, and AGV.

  - 2.4	Value of component A for bistatic and monostatic sensing modes

Adopting a common component A, i.e., mean monostatic RCS across 3D angles for both monostatic and bistatic modes offers several benefits. It simplifies standardization by eliminating the need for separate parameter definitions. Moreover, this approach leverages existing monostatic data, which is often more accessible and validated, ensuring consistency in target characterization. Although drawbacks including potential inaccuracies in bistatic scenarios, as bistatic RCS can differ significantly from monostatic averages due to angle-dependent scattering behaviors may exist, the simplicity and future refinements (e.g., bistatic-specific corrections) makes this approach, i.e., the common component A attractive due to is simplicity and interoperability.

Proposal 6: Support the value of the RCS component A, i.e., mean of monostatic RCS values in all 3D incident/scattered angles of a target to be applied to both monostatic and bistatic RCS of the target.

  - 2.5	Angular correlation of RCS component B2 and XPR

The B2/XPR values can exhibit significant fluctuations due to changes in the angles of incidence and scattering. While there may be some correlation between adjacent angles, this relationship is not universally consistent. The discontinuities in B2/XPR values arise from various factors, including object shape, material properties, and environmental interference, which cannot be fully captured by angular correlation alone.

Observation 1: Real-world B2/XPR discontinuities are influenced by too many variables to be resolved by angular correlation alone.

Introducing angular correlation into the B2/XPR framework would oversimplify the complex interactions inherent in RCS modeling. Even if angular correlation is incorporated, the model would still fail to account for all real-world variables, such as dynamic environmental interference. Introducing angular correlation into B2/XPR would not only complicate the model but also deviate from the established framework. Therefore, it is reasonable to prioritize maintaining the simplicity and robustness of the current model over introducing additional complexity. Although modeling angular correlation of B2/XPR for moving objects might seem beneficial for achieving a continuous channel when the target moves or rotates, this approach introduces unnecessary complexity into the B2/XPR modeling framework. The current channel model framework already separates angular characteristics (captured by B1) and random fluctuations (handled by B2). The angular relationship can be well-represented by the model, so additional angular correlation seems unnecessary.

Observation 2: Introducing the angular correlation would overcomplicate the model without guaranteeing improved accuracy or practicality.

Proposal 7: Deprioritize the modelling of angular correlation for components B2 and XPR since it is not essential.

During the pre-RAN1#121 email discussion, five options were discussed for modelling angular correlation of B2 and XPR and option 2 among those five options offers a balanced solution by aligning with established principles in existing standards (e.g., 3GPP TR 38.901) while addressing practical implementation constraints. By applying the same B2 value to a path until its update, this option is similar to the handling of large/small-scale parameters in current channel models, where parameters remain static during a certain period, e.g., multiple frames or slots. Although Option 2 may introduce inaccuracies in scenarios with significant angular changes due to mobility, the angular dependence of the A*B1 term ensures dynamic RCS variations even when B2 is temporarily fixed. The same rationale can also be applied to

Proposal 8: Support applying the same value of B2/XPR to a path until the value is updated.

  - 2.6 RCS for other targets/EO type-1

In previous meetings, RAN1 has agreed to model EO type-1 as a target. However, depending on target types, there may be different values for components A/B1/B2. RAN1 needs more discussion what kind of target is applicable to modelling the EO type-1. One possible method is to consider reusing RCS model 1 for small UAV and human with possible modifications. However, whether this simplification can fulfill the needs for various scenarios that require EO type-1 modelling needs further study.

Observation 3: Different target models have different values for components A/B1/B2, which leads to ambiguity in defining components A/B1/B2 for EO type-1.

Proposal 9: RAN1 to further study the types of target applicable to modelling the EO type-1 considering diverse scenarios and necessary modifications.

# ISAC channel model

  - 3.1 Background channel for monostatic

In RAN1 #120bis meeting, it was already agreed that the parameters, i.e., large-sale and small-scale parameters for the Tx-target link and target-Rx link in monostatic sensing should remain the same to ensure reciprocity. This should also apply to initial random phase of the two links to reserve the reciprocity.

Proposal 10: Use same initial random phase for the same ray in Tx-target link and target-Rx link of a target for monostatic sensing.

  - 3.2 Power threshold for path dropping in target channel

In the last meeting, -40dB as path dropping threshold was discussed and not agreed upon due to unclear motivation for such new threshold. The intention of using path dropping is to reduce system complexity while minimizing the impact on the stochastic nature of the channel compared to the non-dropping scenario. According to some companies’ evaluation inputs [6][7], it was shown that for concatenation Option 0, under LOS condition Case 2 and LOS condition Case 3 with a probability of about 90%, when the threshold is set at -40dB, the power drop ratio of indirect paths is up to 5%. This relatively low drop ratio implies that a small number of paths are removed, which can effectively simplify the system without causing a significant change in the channel’s stochastic properties. In contrast, setting the threshold at -25dB leads to a power drop ratio of indirect paths up to 30%, which may have a more substantial impact on the channel’s characteristics. Therefore, -40dB strikes a better balance between complexity reduction and maintaining the integrity of the channel.

In target detection, the near - far effect is a critical problem. Distant targets often have extremely low echo power. Low - power paths in the target channel can play an important role in detecting these far - away targets. By setting the threshold at -40dB, we can retain a certain number of low - power paths. This is beneficial for testing the robustness of sensing algorithms in various scenarios, especially those involving long - range target detection. Even though some paths are dropped, the 5% power drop ratio at -40dB ensures that enough low - power paths are preserved to support the detection of distant targets.

Observation 4: -40dB threshold strikes a fair balance between complexity reduction (5% path drop) and channel modelling accuracy.

Observation 5: Retaining low-power paths via -40dB threshold is beneficial for testing robustness of distant target detection robustness due to near-far effect

Proposal 11: Consider -40dB as threshold for path dropping in target channel generation

  - 3.3 Impact of height of target on LOS probability or PL

In RAN1#120bis meeting, two options as shown below need to be down selected for pathloss calculation for the case when the height of a scattering point of target is less than 1.5m.

### Agreement In sensing scenario UMi, UMa, RMa, if the height of a scattering point of target is less than 1.5m, for pathloss calculation, down-selection one of the options below: - Option 4: use PL1 in Table 7.4.1-1: Pathloss models in TR 38.901 - Option 5: use hUT 1.5 m for pathloss calculation

Some concerns are that breakpoint distance may be negative value when the target scattering point height is below 1 meters (e.g., hUT < 1m). This means PL2 formula will be used according to TR 38.901 (Table 7.4.1-1), which could increase pathloss errors. Two Options, i.e., Option 4 and Option 5 are proposed to solve this issue. Compared to Option 4 which enforces applicability of pathloss model, Option 5 that relies on the fixed height (1.5m) aligns with current TR 38.901 pathloss model that has been well validated. Hence, Option 5 is preferred over Option 4.

Proposal 12: Consider use hUT 1.5 m for pathloss calculation (Option 5) when the height of a scattering point of target is less than 1.5m.

  - 3.4 Absolute delay

In RAN1 #120bis, it was agreed to consider absolute delay in 38.901, i.e., Table 7.6.9-1. However, the values of parameters for Δτ for Urban grid, Highway, HST are not available. The following proposal can be a potential solution.

### [H] Proposal 5.7-1 To generate the absolute delay model for sensing scenarios Urban grid, highway and HST, for both target channel and background channel - For Urban grid, the values of parameters for Δτ of scenarios UMa are reused. - For Highway, the values of parameters for Δτ of scenarios RMa and UMa are reused for FR1 and FR2 respectively. - For HST, the values of parameters for Δτ of scenarios RMa and UMa are reused for FR1 and FR2 respectively. Note: If significant problem is found by validation, we can change the distribution of Δτ

The above proposal reuses the validated parameters for UMa and RMa scenarios considering that these scenarios, i.e., Urban grid, Highway, HST are derived from UMa and RMa models in TRs, which avoids introducing untested variables, ensuring consistency and reducing modeling complexity. However, directly reusing the values with validation results may be not consistent with the reality, which may require unique parameters. Nevertheless, before any empirical data supporting the scenario-specific parameters, this approach seems to maintain technical coherence and aligns with RAN1’s agreement to prioritize absolute delay modeling.

Proposal 13: Consider generating the absolute delay model for sensing scenarios Urban grid, highway and HST based on the values of parameters for UMa and RMa.

  - 3.5 Power normalization

For power normalization between the target and background channels, implementing such normalization risks introducing unnecessary complexity and violating the physical consistency of the channel model. For example, scaling only the indirect paths of the target and background channels while leaving the direct paths unchanged creates an artificial discontinuity in power distribution across multipath components. This could lead to unphysical discrepancies, such as inconsistent direct-path power levels for UAVs in urban macro and micro scenarios. Therefore, omitting power normalization offers a more streamlined and physically consistent approach to combining the channels without artificial adjustments. For instance, the target channel’s power naturally remains significantly lower than the background channel’s power, eliminating the need for normalization while preserving the model’s physical realism. In UAV scenarios, directly superimposing the target and background channels aligns with the sparse scattering characteristics of aerial environments, where target-induced interference with background clusters is negligible. Furthermore, the stochastic framework defined in TR 38.901 inherently accounts for channel variability through ray-tracing and empirical measurements. Treating normalization as an "optional component" fragments the modeling framework, complicating interoperability and convergence across scenarios. By avoiding power normalization, the model ensures robust performance in both simple and complex environments while maintaining theoretical and operational integrity.

Proposal 14: No need to perform power normalization between target channel and background channel.

  - 3.6 Doppler of moving scatters

To model the stochastic clutters, one aspect is its mobility. The mobility of the stochastic clutters can have a significant impact on target channel modeling. Nonstationary clutters can be represented as slow-moving objects, which follow a certain Doppler distribution with an average Doppler of zero. These slow-moving objects caused by non-stationary clutters can greatly influence the detection of slow-moving targets. Thus, capturing the mobility of stochastic clutters in target channel modeling may be crucial to reflect such impact.

Considering that the TR 38.901 provides the modelling method in 7.6.10 dual mobility model to model the movement of stochastic clusters, this model may be reused to reflect the effect of the stochastic clutters in target channel with necessary modifications, e.g., maximum speed and ratio of moving scatters.

Observation 6: The mobility of stochastic clutters can be reflected as slow-moving objects following a specific Doppler distribution with an average Doppler of zero, which has a significant impact on target channel modelling

Proposal 15: RAN1 to consider reusing dual mobility model in TR 38.901 to reflect the effect of the stochastic clutters in target channel with necessary modifications, e.g., maximum speed and ratio of moving scatters.

# Spatial consistency

  - 4.1 Path dropping

In RAN1 #120bis, it was agreed to perform path dropping after concatenation based on a power threshold, e.g., -40dB. Since the power of each direct/indirect path will change during the movement of Tx/target/Rx, whether to redo the path dropping whenever Tx/target/Rx moves is still unclear.

It is needless to point out that redoing path dropping every time Tx/target/Rx moves results in large implementation complexity. Moreover, it requires more discussions on the detailed method that dynamically drops the path based on the movement of Tx/target/Rx, which could be time consuming. Given the limited time, it seems appropriate to perform path dropping only once at the beginning of the simulation and update the parameters (e.g., large scale/small scale parameters) based on spatial consistency without redoing path dropping, which has good computational efficiency. This approach maintains consistency in path behavior while avoiding the overhead of reevaluating paths during movement.

Proposal 16: the path dropping after concatenation of Tx-target and target-Rx link is not reperformed and the large/small scale parameters can be updated according to the procedure of spatial consistency.

  - 4.2	Cases that spatial consistency are supported or not supported

Spatial consistency ensures alignment of parameters across spatially correlated channels, but when links originate from distinct communication scenarios or involve virtual reference points, enforcing such consistency becomes unnecessary and technically impractical. For example, spatial consistency should not be modelled for the cases where two links, i.e., Tx-target link and target-Rx link are generated under different channel model scenarios. In addition, TRP monostatic sensing background channel generation utilizes multiple independent virtual reference points, which need no spatial coherence between them.

Proposal 17: No spatial consistency enabled for the case where the links originate from distinct communication scenarios or involve virtual reference points.

  - 4.3 Spatial consistency between multiple scattering points

For the spatial consistency between multiple scattering points, two options are available as follows and need to be down-selected.

### Proposal - Option 1: When spatial consistency is NOT implemented, channels of the multiple scattering points of a target is independently generated - Option 2: When multiple scattering points of target is modelled, spatial consistency is used

While it was agreed in RAN1#120 that the LoS condition between the Tx/Rx and each scattering point of the same target is generated independently, the close proximity of these scattering points suggests a high likelihood that they share the same LoS condition. This implies that enabling spatial consistency for LoS conditions may help reflect realistic behavior of scattering points considered for scattering points associated with the same target. Extending this logic, spatial consistency may also be applied to large-scale parameters and small-scale parameters, as these parameters are inherently linked to the physical geometry and propagation environment of the target. By incorporating spatial consistency, the model may help reflect the realistic scattering characteristics of scattering points and their associated parameters, ensuring a more accurate and coherent representation of the channel.

Observation 7: Enabling spatial consistency for channels of multiple scattering points may help reflect realistic scattering characteristics of scattering points and their associated parameters.

Although enabling spatial consistency may help reflect the scattering characteristics of targets, especially for large objects modelled by multiple scattering points. However, similar to the spatial consistency as an optional feature in TR 38.901, enabling spatial consistency of channels of multiple scattering points should also be an optional feature. This means that spatial consistency does not have to be enabled when multiple scattering points of a target are modeled. When spatial consistency is NOT implemented, channels of the multiple scattering points of a target are independently generated. This aligns with Option 1, which should be supported to maintain simplicity and flexibility in the channel model. On the other hand, when spatial consistency is not implemented, channels of the multiple scattering points of a target are independently generated. This approach ensures compatibility with existing modeling frameworks and avoids unnecessary complexity.

Proposal 18: When multiple scattering points of target is modelled, enabling spatial consistency is optional.

Proposal 19: When spatial consistency is not implemented, channels of the multiple scattering points of a target is independently generated.

  - 4.4	3D spatial consistency

To enable spatial consistency is quite important feature to simulate 3D correlation when target, e.g., UAV can have a vertical speed. In TR 38.901, the correlation distance is only used to model horizontal correlation distance. The value for the correlation distance has been validated with validation results. However, it is not clear whether horizontal correlation distance can be directly used for vertical correlation distance or 3D correlation distance given the different channel parameters, e.g., LoS probability could be quite different in case of vertical than horizontal. Therefore, RAN1 seems to need more study on this.

To enable spatial consistency is an important feature for simulating 3D correlation, particularly when targets like UAVs exhibit vertical motion. Currently, TR 38.901 defines correlation distance solely for horizontal direction, and these values have been supported with validation results. However, it remains uncertain whether horizontal correlation distances can be directly applied to vertical or 3D correlations, as channel parameters such as LoS probability may differ significantly between vertical and horizontal directions. RAN1 should study on whether existing horizontal correlation distances can be reused for vertical directions. Differences in propagation conditions, such as LoS probability and spatial distribution, may require dedicated vertical correlation distance values. While reusing horizontal parameters simplifies implementation, it risks oversimplifying complex 3D scenarios. Therefore, further study is required to ensure accurate modeling of vertical and 3D correlations in ISAC channels.

Observation 8: The validity of direct application of horizontal correlation distance to vertical or 3D correlation distance is unclear.

Proposal 20: RAN1 to study on whether horizontal correlation can be directly reused to model 3D spatial consistency for ISAC channel.

# EO type-2

  - 5.1 EO type-2 in background channel

It is agreed that EO type-2 can be modelled in target channel. Whether EO type-2 should be modelled in background channel is a long-lasting discussion point. The following two options are on the table to be down-selected.

### Proposal - Option 1: EO type-2 is modelled in background channel if modelled in target channel - Option 2: EO type-2 is not modelled in background channel

When EO type-2 is actually modelled in target channel, it is logically consistent to model its impact to the background channel because the physical presence of EO Type-2 inherently affects both channels in real-world propagation environments. Some concerns raised that adding EO Type-2 to the background channel risks altering power distributions and introducing unvalidated variables, which does not align with existing TR 38.901’s GBSM framework. However, the concerns overlook the fact that TR 38.901 also provides the additional features, e.g., ground reflection model, blockage model, etc. to help accurately certain channel modelling aspects that cannot be addressed by the basic modelling components. Based on above analysis, Option 1 should be supported to realistically and logically to reflect the impact of the EO type-2 to both target channel and background channel.

Proposal 21: EO type-2 should be modelled in background channel if modelled in target channel

  - 5.2 LOS condition considering EO type-2

In the RAN1#120bis email discussion, the antenna pattern like approach became a candidate for RCS modeling methodology for the UAV with large size. Since RAN1 has already reached a consensus on this modeling method for automotive targets, it is reasonable to apply the same method to other target (at least UAV and AGV with large size).

### [H] Proposal 6.2-1 Down-select in RAN1#121 one option from the following options to determine the LOS condition of the Tx-target link and target-Rx link? - Option A: If type-2 EO is in the LOS ray of one link, the link is determined as NLOS condition, and otherwise use the LOS probability equation defined in existing TRs to determine the LOS/NLOS condition - Option B: Use the LOS probability equation to determine the LOS/NLOS condition of one link, and then the impacts of type-2 EO is modelled by a blockage model - Option C: Use the LOS probability equation to determine the LOS/NLOS condition of one link, Note: If EO type-2 is agreed to be modelled in background channel, the agreed option is extended to LOS condition determination for background channel.

Option A reflects the real channel propagation that if a LOS ray is blocked, it is NLOS condition. However, it changes the behavior in current TR. In addition, it requires defining new criteria to determine LOS blockage, which lacks standardization. For example, the exact method or mathematical equations of determining the blockage of LOS ray based on e.g.., environment layout might need to be defined, which requires additional standardization effort.

Option B binds EO type-2 with blockage model B. Compared to option A, it aligns with TR 38.901 by reusing blockage model for EO type-2, ensuring consistency. Moreover, it avoids complex LOS blockage definitions, reducing standardization effort. However, the blockage model in TR 38.901 is particularly designed for human/vehicular blocking, not EO type-2. Its applicability to EO type-2 is unverified and may require more study effort.

Compared to Option A and Option B, Option C ignores the impact of EO type-2 when calculating LOS probability, which may not be realistic.

Observation 9: Option A may require new criteria to determine LOS blockage due to EO type-2.

Observation 10: the blockage model is particularly designed for human/vehicular blocking and its applicability to EO type-2 is unverified and may require more study.

Proposal 22: RAN1 to further address the limitations of Option A and Option B and down-select from the two options.

# Conclusion

This contribution discusses the ISAC channel modelling including the framework, target and background environment modelling, spatial consistency and validation of channel modelling. The observations and proposals are summarized as follows:

Proposal 1: Extend the working assumption on bistatic RCS for vehicle to large size UAV and AGV with necessary parameter adaptations to gain implementation benefit from the unified framework.

Proposal 2: RAN 1 consider the antenna pattern-like approach for RCS modeling of large size UAV

Proposal 3: RAN 1 discuss the RCS values for large size UAV with table 1

Table 1. monostatic RCS of large size UAV with single scattering point

|  | φcenter | φ3dB,k | θcenter | θ3dB,k | Gmax | σmax | Applicable Range of φ | Applicable Range of θ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Head | 0° | 14.37° | 90° | 14.37° | 5.83 | 8.2 | [315°,360°]∪[0°,45°] | [45°,135°] |
| Left | 90° | 3.90° | 90° | 3.90° | 6.8 | 9.17 | [45°,135°] | [45°,135°] |
| Back | 180° | 7.57° | 90° | 7.57° | 5.83 | 8.2 | 135°,225° | [45°,135°] |
| Right | 270° | 3.90° | 90° | 3.90° | 6.8 | 9.17 | [225°,315°] | [45°,135°] |
| bottom | / | / | 180° | 3.90° | 20.12 | 22.5 | 0°,360° | [135°,180°] |
| Roof | / | / | 0° | 3.90° | 20.12 | 22.5 | 0°,360° | [0°,45°] |

Proposal 4: Support the proposal formula considering its consistency, simplicity, and practicality for modelling RCS in both monostatic and bistatic configurations, particularly for the specified target types.

Proposal 5: Support option 1 for UAV of large size, human with RCS model 2, vehicle with single/multiple scattering points, and AGV.

Proposal 6: Support the value of the RCS component A, i.e., mean of monostatic RCS values in all 3D incident/scattered angles of a target to be applied to both monostatic and bistatic RCS of the target.

Observation 1: Real-world B2/XPR discontinuities are influenced by too many variables to be resolved by angular correlation alone.

Observation 2: Introducing the angular correlation would overcomplicate the model without guaranteeing improved accuracy or practicality.

Proposal 7: Deprioritize the modelling of angular correlation for components B2 and XPR since it is not essential.

Proposal 8: Support applying the same value of B2/XPR to a path until the value is updated.

Observation 3: Different target models have different values for components A/B1/B2, which leads to ambiguity in defining components A/B1/B2 for EO type-1.

Proposal 9: RAN1 to further study the types of target applicable to modelling the EO type-1 considering diverse scenarios and necessary modifications.

Proposal 10: Use same initial random phase for the same ray in Tx-target link and target-Rx link of a target for monostatic sensing.

Observation 4: -40dB threshold strikes a fair balance between complexity reduction (5% path drop) and channel modelling accuracy.

Observation 5: Retaining low-power paths via -40dB threshold is beneficial for testing robustness of distant target detection robustness due to near-far effect

Proposal 11: Consider -40dB as threshold for path dropping in target channel generation

Proposal 12: Consider use hUT 1.5 m for pathloss calculation (Option 5) when the height of a scattering point of target is less than 1.5m.

Proposal 13: Consider generating the absolute delay model for sensing scenarios Urban grid, highway and HST based on the values of parameters for UMa and RMa.

Proposal 14: No need to perform power normalization between target channel and background channel.

Observation 6: The mobility of stochastic clutters can be reflected as slow-moving objects following a specific Doppler distribution with an average Doppler of zero, which has a significant impact on target channel modelling

Proposal 15: RAN1 to consider reusing dual mobility model in TR 38.901 to reflect the effect of the stochastic clutters in target channel with necessary modifications, e.g., maximum speed and ratio of moving scatters.

Proposal 16: the path dropping after concatenation of Tx-target and target-Rx link is not reperformed and the large/small scale parameters can be updated according to the procedure of spatial consistency.

Proposal 17: No spatial consistency enabled for the case where the links originate from distinct communication scenarios or involve virtual reference points.

Observation 7: Enabling spatial consistency for channels of multiple scattering points may help reflect realistic scattering characteristics of scattering points and their associated parameters.

Proposal 18: When multiple scattering points of target is modelled, enabling spatial consistency is optional.

Proposal 19: When spatial consistency is not implemented, channels of the multiple scattering points of a target is independently generated.

Observation 8: The validity of direct application of horizontal correlation distance to vertical or 3D correlation distance is unclear.

Proposal 20: RAN1 to study on whether horizontal correlation can be directly reused to model 3D spatial consistency for ISAC channel.

Proposal 21: EO type-2 should be modelled in background channel if modelled in target channel

Observation 9: Option A may require new criteria to determine LOS blockage due to EO type-2.

Observation 10: the blockage model is particularly designed for human/vehicular blocking and its applicability to EO type-2 is unverified and may require more study.

Proposal 22: RAN1 to further address the limitations of Option A and Option B and down-select from the two options.

# References

[1] RP-240799	“New SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR,” RAN#103

[2] R1-2502552	draft CR to TR 38.901 on ISAC	Moderator (xiaomi)

[3] R1-2502378	Discussion on ISAC channel modelling	Samsung

[4] R1-2502379 	Discussion on ISAC channel modelling	Samsung

[5] TR38.901 “Study on channel model for frequencies from 0.5 to 100 GHz (Release 17)”

[6] R1-2502624	Discussion on ISAC channel modelling	Apple

[7] R1-2502452	Discussion on ISAC channel model	Xiaomi, BJTU, BUPT

# Agreement on previous meetings

RAN1#116

Agreement The common framework for ISAC channel model is composed of a component of target channel and a component of background channel, HISAC=Htarget+Hbackground - Target channel Htarget includes all [multipath] components impacted by the sensing target(s). - FFS details of the target channel - Background channel Hbackground includes other [multipath] components not belonging to target channel - FFS details of the background channel - FFS whether/how to model environment object(s), i.e., object(s) with known location, other than sensing target(s) - FFS whether/how to model propagation path(s) between the target(s) and the environment object(s) - FFS whether/how to model propagation path(s) between the target(s) and the stochastic clutter(s) - Note: the notation HISAC can be revised later if needed

RAN1#116 bis

Agreement The following cases of radio propagation in the target channel are considered for the study Case; Tx-target; Target-Rx. 1; LOS condition; LOS condition. 2; LOS condition; NLOS condition. 3; NLOS condition; LOS condition. 4; NLOS condition; NLOS condition. - Case 1/2/3/4 can be considered for bistatic sensing mode - At least Case 1/4 can be considered for monostatic sensing mode - Note: It doesn’t imply the channel response for each link is separately generated then concatenated - FFS how to determine LOS condition and NLOS condition, e.g., based on LOS probability, or determined based on geometrical locations of environment object (EO). - In LOS condition, line of sight ray(s) are present between Tx/Rx and target, and there may or may not exist non-line of sight ray(s) between Tx/Rx and target too - In NLOS condition, there only exist non-line of sight ray(s) between Tx/Rx and targe Agreement - In the target channel between Tx and Rx, scattering of a sensing target can be modelled as single scattering point or multiple scattering points - FFS one or multiple incoming/output rays corresponding to a scattering point - FFS how to select single or multiple scattering points for the target, e.g. depending on the distance between target and Tx/Rx, size/shape of target, etc. - Note: the sensing target can be assumed in far field of sensing Tx/Rx. - FFS details to model the single or multiple scattering points Agreement RCS of a physical object shows dependency to at least the following factors: - Type of the object - The size of the object - The material of the object - The shape of the object - Orientation of the object - FFS: Distance between Tx/Rx and the object - The incident angle and scatter angle - The carrier frequency - polarization of the transmitter and receiver - FFS Temporal or spatial consistency - FFS antenna pattern - FFS whether/how to model the above factors in the CR, e.g. with an RCS model with a scattering point Agreement EO is a non-target object with known location. - FFS other known parameters of the EO - FFS details on EO modeling The following options for EO modeling are considered for further study - Option 1: EO is modelled different from a sensing target - Applicable at least for an EO having extremely large size (referred as EO type-2 for discussion purpose) - FFS modeled similar to section 7.6.8 ground reflection in TR 38.901 - FFS EO modeling impacts the target channel and/or the background channel - Option 2: EO is modeled same/similar as a sensing target - Applicable for an EO having comparable physical characteristics as a sensing target, (referred as EO type-1 for discussion purpose) - FFS Applicable for EO type-2 - FFS EO modeling impacts the target channel and/or the background channel - Option 3: EO is modeled and its location is determined from a stochastic clutter generated following the cluster generation in TR 38.901 - FFS details - Option 4: EO is not modelled - Other options are not precluded - Note: it is not precluded that multiple options can be supported in the channel modelling Agreement The following options are considered for further study to model the target channel for a target - Option 1: modelled by concatenation of path(s) from Tx to target and from target to Rx - Option 2: modelled by Tx-to-Rx path(s) satisfying Tx-target-Rx geometry - Option 3: combination of Option 1 and Option 2 Agreement If a target is modelled with single scattering point, the following options to model RCS of the target are considered for further study. - Option 1: Random RCS value generated by a statistical distribution, depending on the factor(s) having impacts on the RCS modelling. - FFS the distribution. - FFS the factor(s) - Option 2: Deterministic RCS value is defined by a function and/or a table, depending on the factor(s) having impacts on the RCS modelling - Note: Constant RCS for a target type can be a special case of Option 2 - FFS the factor(s) - FFS details of function and/or table - Option 3: combination of Option 1 & 2, e.g., RCS value is generated by combining a deterministic component and a randomly generated component. - FFS application of each option to large scale fading and/or small scale fading - FFS target with multiple scattering points Agreement - Interested companies are encouraged to submit validation results together with their proposal for ISAC channel modeling - Up to each company to select the way for validation - Option 1: Experimental results - Option 2: Experimental results to validate a ray-tracing model, then the ray-tracing based results to validate the ISAC channel model - Note: the layout of the scenario used for validation is up to company choice Agreement ISAC channel model for link level simulation is to be discussed after the system level channel model is sufficiently stable with basic functionalities.

RAN1#117

Agreement Multiple sensing targets can be modelled in the ISAC channel of a pair of sensing Tx and sensing Rx - FFS whether to model a propagation path from Tx to Rx interacting with more than one sensing target The same sensing target can be modelled in the ISAC channels of multiple pairs of sensing Tx and Rx Agreement - For discussion purpose, the propagation paths in the target channel are classified - The direct path, i.e., LOS ray from Tx to target + LOS ray from target to Rx - The indirect paths, i.e., any propagation path other than the direct path, including - LOS ray from Tx to target + NLOS ray from target to Rx - NLOS ray from Tx to target + LOS ray from target to Rx - NLOS ray from Tx to target + NLOS ray from target to Rx - For radio propagation Case 1, - For a direct path, the following parameters are [deterministically] generated at least based on the geometry location of Tx, target and Rx - AoA/ZoA at Rx - AoD/ZoD at Tx - AoA/ZoA/AoD/ZoD at target - delay - FFS initial phase - Doppler - FFS power/polarization including the impact of RCS - FFS the number of direct path(s) for a target - FFS on detailed modelling of indirect path(s) - FFS on details of modelling of indirect paths in radio propagation Case 2/3/4 - To generate the channel coefficients of direct/indirect path(s) in the target channel, the channel coefficient generation function in step 11 in section 7.5 of TR 38.901 (e.g., formula 7.5-22) is used as the start point - Note: modification to step 11 is deemed necessary - FFS adding impact of small scale RCS - FFS Doppler Agreement - Spatial consistency should be supported for ISAC channel - Spatial consistency should be supported based on movement of sensing Tx, sensing target and/or sensing Rx - FFS EO handling Agreement When the stochastic cluster is used to generate the indirect paths in the target channel of a target - The stochastic cluster generation in section 7, TR 38.901 is used as starting point. - FFS a stochastic cluster is generated between Tx and Rx satisfying Tx-target-Rx geometry, or between Tx/Rx and target - FFS modification to stochastic cluster generation in section 7, TR 38.901 - FFS use of sub-cluster to model the indirect paths Note: RAN1 continues studying using EO to generate the indirect paths in the target channel of a target Agreement When the stochastic cluster is used to model indirect path in the target channel - For bistatic, the LOS condition from Tx to target and from target to Rx is determined separately for a target - FFS: The correlation of LOS condition of Tx-target and Rx-target links of a target - For monostatic, a same LOS condition is determined for Tx to target and target to Rx - The LOS condition from Tx to target and/or from target to Rx is determined with the LOS probability - The probability schemes in existing 3GPP TRs, e.g., TR 38.901. TR 36.777, TR 37.885, etc. are considered as start point - FFS: How to consider the impacts of target height on LOS probability. Agreement When stochastic cluster is used to model indirect path in the target channel, down-select between the following options - Option 1: modelled by concatenation of path(s) from Tx to target and from target to Rx - For each of the Tx-target link and target-Rx link, - The parameters delay, power, angle, [initial phase], [Doppler] of NLOS ray(s) in the link Tx-to-Target or Target to RX are generated - FFS following cluster generation in section 7, 38.901 - The target channel is generated by concatenating the parameters of the Tx-target link and target-Rx link. - FFS on Convolutional or 1-by-1 coupling or 1-to-many coupling - FFS how to combine the clusters in target channel and the clusters in background channel - Option 2: modelled by Tx-to-Rx path(s) satisfying Tx-target-Rx geometry of the direct path - The parameters delay, power, angle, initial phase of a stochastic (sub-)cluster between Tx and Rx are generated following cluster generation in section 7, 38.901 - The parameters [delay], [power], [angle], [Doppler] of the (sub-)cluster are updated by the target property - FFS how to combine the clusters in target channel and the clusters in background channel

RAN1#118

Agreement if RCS related coefficient of a scattering point is included in small scale, the RCS related coefficients are separately determined for different pairs of incident/scattered ray(s) at the scattering point. Agreement For radio propagation Case 1, for modelling the target channel of a target with single scattering point, - To model a direct path, a single LOS ray from Tx to target and a single LOS ray from target to Rx are generated - AoA/ZoA of the direct path at Rx, AoD/ZoD of the direct path at target are generated at least based on the 3D location of target and Rx in the global coordinate system - AoD/ZoD of the direct path at Tx, AoA/ZoA of the direct path at target are generated at least based on the 3D location of Tx and target in the global coordinate system - The Delay of the direct path = (d3D_tx_target + d3D_target_rx)/c - The Doppler of the direct path is generated by spherical unit vectors by AoD/ZoD at Tx, by spherical unit vectors by AoA/ZoA at Rx, and velocity of Tx, target and Rx - The power of the direct path is generated as the product of the power of the LOS ray from Tx to target, the power of the LOS ray from target to Rx, and the effect of RCS - FFS initial phase - FFS how to model RCS, polarization of target - FFS number of direct paths - FFS on detailed modelling of indirect path(s) - FFS applicability of direct path generation to each scattering point when the target is modelled as multiple scattering points Agreement For the target channel of a target with single scattering point, when stochastic cluster is used to model an indirect path in the target channel, - An indirect path in small scale is modelled by concatenation of path(s) from Tx to target and from target to Rx, i.e., Option 1 in the agreement of RAN1 #117 - AoD/ZoD/AoA/ZoA from Tx to target or from target to Rx are - generated for a LOS ray at least based on the 3D location of Tx/Rx and target in the global coordinate system - stochastically generated for a NLOS ray using section 7, 38.901 as starting point - Delay is sum of delay of LOS/NLOS ray from Tx to target and the LOS/NLOS ray from target to Rx - Doppler is generated by spherical unit vector by AoD/ZoD at Tx and velocity of Tx, by spherical unit vector by AoA/ZoA at Rx and velocity of Rx, and by spherical unit vectors by AoA/ZoA/AoD/ZoD at target and velocity of target - FFS The mobility of stochastic clutter - The power of the indirect path is generated as the product of the power of the LOS/NLOS ray from Tx to target, the power of the LOS/NLOS ray from target to Rx and the effect of RCS - FFS initial phase - FFS how to model effect of RCS at target - FFS whether/how to model polarization at target - FFS How to reduce complexity - FFS applicability of the indirect path generation to each/a single scattering point when the target is modelled as multiple scattering points Working assumption The RCS related coefficient of a scattering point can be modelled with two components, i.e., linear value RCS = A*B - A first RCS component A (m2) is included in large scale - FFS the first RCS component is deterministic or stochastic - FFS The first component is dependent on incident and scattered directions at target - A second RCS component B (unit ratio) is included in small scale - FFS The second component is dependent on incident and scattered directions at target - FFS the second RCS component is deterministic or stochastic or combination - Note: RCS component A or B can be disabled by setting its linear value to 1 - Whether to disable a component can be discussed per target type - FFS how to determine A and B for each target - FFS whether/how to model polarization impact at target - FFS whether/how to normalize power accounting for target channel and background channel Agreement The impact of a scattering point of the target in the target channel is modelled by a scalar RCS value RCS times a complex-valued 2x2 polarization matrix CPMsp, i.e., RCS∙CPMsp - FFS whether CPMsp is angular/ray-dependent or independent - FFS whether polarization matrix CPMsp is modelled assuming specular reflection or random coefficient for diffraction or scattering - FFS whether polarization matrix CPMsp is explicitly modelled or merged with other polarization matrixes from Tx to target and/or from target to Rx Agreement For modeling stochastic cluster in background channel, in order to define the background channel for TRP-UE and UE-TRP bistatic sensing mode, - The large scale and small scale parameters defined in TR 38.901, TR 37.885, TR 36.777 are used as start point In order to define the background channel for TRP-TRP and UE-UE bistatic sensing mode, - RAN1 to study how to model background channel - Option 1: The large scale and small scale parameters defined in TR 38.901, TR 38.858, 37.885, 38.859 are used as start point - Option 2: New channel model based on measurement results or ray-tracing model validated by experimental results FFS whether/how to do power normalization between target channel and background channel Agreement In order to define the background channel for TRP and UE mono-static sensing mode, - RAN1 to study how to model the background channel - Option 1: randomly drop at least one virtual Rx, and then the background channel is generated based on the channel generated as TR 38.901 between the real Tx and each virtual Rx for a scenario - FFS EO is modelled in the background channel - Option 2: New channel model based on measurement results or ray-tracing model validated by experimental results or radar literatures - FFS EO is modelled in the background channel - Option 3: the locations of clusters in the target channel are deterministically generated, then the background channel is generated using the clusters with the determined locations. - Other options are not precluded Agreement When EO type-2 is modelled, specular reflection is considered to model EO type-2 using section 7.6.8 of TR 38.901 as reference - As starting point, the effect of type-2 EO (i.e., in the path node1-EO-node2) is modelled as bR∥specular00-R⊥specular, b is a scaling factor (e.g., c equals to d3DdGR relative to LOS ray in section 7.6.8 in TR 38.901) - FFS any update to b,R∥specular,R⊥specular - FFS any update taking EO orientation into account - FFS any changes based on section 7.6.8 of TR 38.901 if EO type-2 has finite size - FFS whether diffraction and scattering can be considered in addition to specular reflection - EO type-2 is an optional modelling component if supported in a sensing scenario FFS which deployment scenario(s) EO type-2 will apply

RAN1#118bis

Agreement RAN1 strives to define a single option per target per monostatic/bistatic sensing mode from the following two options to generate RCS values/patterns for a scattering point of a target. - Option 2: The RCS=A*B of a scattering point can be generated by - The component A is commonly applied to any incident/scattered angles at the scattering point - A is [mean] RCS value. FFS value(s) A - Note: Mean RCS value is defined as the mean value of the distribution of RCS - The component B - B is generated by [log-normal] distribution, the related [log-normal] distribution has mean μ=1 and variance V, FFS σ2 - B is separately generated for each direct/indirect path at the scattering point. FFS correlation dependent on the incident/scattered angles of the direct/indirect paths - FFS whether/how power of all generated direct/indirect paths need to be normalized considering impact of RCS - Option 3: The RCS=A*B=A*B1*B2 of a scattering point can be generated by - The component A is commonly applied to any incident/scattered angles at the scattering point - FFS: A = 1 m2 or [mean] RCS value - Note: Mean RCS value is defined as the mean value of the distribution of RCS - The component B is further split into B1, B2, i.e., B=B1*B2 - B1 is deterministic based on incident/scattered angles - FFS: B1 is defined by a function or by a table - B2 is generated by [log-normal] distribution, the related [log-normal] distribution has mean μ=1 and variance V, FFS σ2 - B2 is separately generated for each direct/indirect path at the scattering point. FFS correlation dependent on the incident/scattered angles of the direct/indirect paths - FFS whether/how power of all generated direct/indirect paths need to be normalized considering impact of RCS Agreement RCS Option 3 is selected to model RCS of UAV with single scattering point for monostatic - B2 of UAV is modelled using log-normal distribution for monostatic - Different mean RCS values can be supported for UAV due to different size, shape, frequency, etc. - For UAV of small size (option 2 for UAV size in UAV parameters table) - B1=1 - A is mean RCS value - For UAV of large size (option 1 for UAV size in UAV parameters table) - B1 have dependency on incident/scattered angles - A is mean RCS value Agreement To model the effect of polarization for each direct/indirect path: - Polarization of a direct/indirect path is product of polarization matrix of Tx-target link, the target, and the target-Rx link - Total polarization of a direct/indirect path is CPMtx,sp,rx= CPMsp,rx . CPMsp . CPMtx,sp - For a LOS ray from Tx to target or from target to Rx, CPM=100-1 for CPMtx,sp or CPMsp,rx - For a NLOS ray generated by a stochastic cluster from Tx to target or from target to Rx, CPMtx,sp or CPMsp,rx CPMis generated by XPR ratio  and initial random phases referring to TR 38.901 as start point - FFS how to normalize on CPMtx,sp,rx CPMtx,sp,rx - FFS CPMsp CPMspof a scattering point of the target - FFS: how to model the effect of polarization when EO type-2 is present Agreement A single direct path is modeled for a scattering point of target - In each of the Tx-target and target RX links, the first NLOS cluster is generated with same delay as the LOS ray (when the absolute delay modelling of Δτ as in section 7.6.9, TR 38.901 is not applied) and with the same direction as the LOS ray. - FFS how to generate NLOS cluster when Δτ is applied Agreement In order to generate each of the Tx-target link and target-Rx link in the target channel, the large scale and small scale parameters defined in existing 3GPP TRs, e.g., TR 38.901. TR 36.777, TR 37.885, TR 38.858, TR 38.859, TR 38.802, TR 38.854, etc. are used as starting point Agreement On the background channel for TRP-TRP and UE-UE bistatic sensing mode, the large scale and small scale parameters defined in TR 38.901, TR 38.858, 37.885, 38.859 are used as starting point - Update on values of the LSP/SSP parameters can be discussed based on validation data acquired by measurement or ray-tracing model - FFS The power threshold for removing clusters in step 6, i.e., -25 dB is revised to X<-25 dB. FFS X - FFS whether/how to resolve the inconsistency between TRP-TRP channel according to TR 38.858 and the TRP-target (UAV) channel according to TR 36.777 when UAV and TRP are set to same height Agreement 3D spatial consistency needs to be studied for at least UAV scenario Agreement In LOS condition between sensing Tx/Rx and target, the power of LOS ray is generated following power of LOS ray in TR 38.901. Agreement The following options are to be studied for the concatenation of Tx-target and target-Rx link in the target channel - Direct path (if present) is always kept - Indirect paths of LOS+NLOS, NLOS+LOS (if present) are generated - On other indirect paths of NLOS + NLOS - Option 0: ray level full convolution between Tx-target link and target-Rx link for radio propagation Case 1/2/3/4 - Option 0A: ray level full convolution between Tx-target link and target-Rx link only for radio propagation Case 4 - Option 1: cluster level full convolution between Tx-target link and target-Rx link, then 1-by-1 coupling rays within each pair of clusters for radio propagation Case 1/2/3/4 - Option 1A: cluster level full convolution between Tx-target link and target-Rx link, then 1-by-1 coupling rays within each pair of clusters only for radio propagation Case 4 - Option 2: cluster level 1-by-1 coupling between Tx-target link and target-Rx link, then 1-by-1 coupling rays within each pair of clusters for radio propagation Case 1/2/3/4 - Option 2A: cluster level 1-by-1 coupling between Tx-target link and target-Rx link, then 1-by-1 coupling rays within each pair of clusters only for radio propagation Case 4 - Option 3: ray level 1-by-1 coupling between Tx-target link and target-Rx link for radio propagation Case 1/2/3/4 - Option 3A: ray level 1-by-1 coupling between Tx-target link and target-Rx link only for radio propagation Case 4 - Note: reducing the number of rays per cluster and/or reducing the number of clusters can be considered for the options above - Any indirect path with power metric less than [threshold] is dropped - the power metric of a path is the product of power of a ray in Tx-target link, power of a ray in target-Rx link and RCS of the pair of rays - FFS power normalization of target channel after path dropping - FFS the set of remaining indirect paths can be updated during movement of Tx, target or Rx

RAN1#119

Agreement Bistatic RCS values for a scattering point of a target are obtained by fixing an incident direction in LCS of target and varying the scattered directions in LCS of target; then changing to other incident direction. Agreement - To generate indirect paths of NLOS ray + NLOS ray in the target channel - Option 0 is recommended, i.e., ray level full convolution between Tx-target link and target-Rx link for radio propagation Case 1/2/3/4 - Option 3 to generate a reduced number of indirect paths of NLOS ray + NLOS ray is recommended, i.e., ray level 1-by-1 random coupling between Tx-target link and target-Rx link is supported for radio propagation Case 1/2/3/4 - If number of rays in the two links are different, e.g., M1, M2 respectively for link 1 and link 2, - If M1<M2, randomly M1 rays are selected in link 2, otherwise randomly M2 rays are selected in link 1 - Other methods are up to company choice for complexity reduction - Both option 0 and 3 will be calibrated independently. Company should report which option is used in calibration - The power threshold for path dropping is X=[-25] dB - X is relative to the strongest indirect path in the target channel - FFS: further power normalization of target channel is performed after path dropping, - Note: power normalization when target channel and background channel are combined can be discussed separately - FFS The set of remaining indirect paths can be updated during movement of Tx, target or Rx Agreement The following RCS models are supported when human is modelled with single scattering point for monostatic, where different RCS values and/or models can be supported for human due to different size, shape, frequency, etc. - Model 1 - B1=0 dB - A is mean RCS value - B2 is modelled using log-normal distribution - Model 2 - B1 have dependency on incident/scattered angles, with further down-selection among the alternatives below: - Alt 1: formulated similar as the antenna radiation power pattern in 38.901 - Alt 2: a function - Alt 3: Lookup table - B2 is modelled using log-normal distribution - FFS RCS component A - FFS: conditions for using which model Agreement The following RCS model is supported when vehicle is modelled with single scattering point for monostatic, where different RCS values can be supported for vehicle due to different size, shape, frequency, etc. - B1 have dependency on incident/scattered angles, with further down-selection among the alternatives below: - Alt 1: formulated similar as the antenna radiation power pattern in 38.901 - Alt 2: a function - Alt 3: Lookup table - B2 is modelled using log-normal distribution - FFS RCS component A Agreement When vehicle is modelled with multiple scattering points for monostatic, where different RCS values can be supported for vehicle due to different size, shape, frequency, etc. - the recommended five scattering points are located in front, left, back, right and roof side of the vehicle - the following RCS model is supported for each scattering point - B1 have dependency on incident/scattered angles, with further down-selection among the alternatives below: - Alt 1: formulated similar as the antenna radiation power pattern in 38.901 - Alt 2: a function - Alt 3: Lookup table - B2 is modelled using log-normal distribution - FFS RCS component A Agreement EO type-1 (when modelled) is modelled in the same way as a sensing target in the ISAC channel model. Agreement - If blockage/forward scattering between sensing targets is not considered, a propagation path from Tx to Rx interacting with more than one sensing targets is not modelled - FFS whether/how blockage/forward scattering can be modelled in the target channel. Agreement - Doppler for a target including both macro-Doppler and micro-Doppler can be modeled using a unified formula, rrx,n',m'Tt.vrxt+rp,n',m'Tt∙vspt+rtx,n,mTt.vtxt+rp,n,mTt∙vsptλ0+ft Where, - rrx,n',m'T is the spherical unit vector at receiver for the link from Rx to the scattering point - rtx,n,mT is the spherical unit vector at transmitter for the link from Tx to the scattering point - rp,n',m'T is the spherical unit vector at the scattering point for the link from the scattering point to Rx - rp,n,mT is the spherical unit vector at the scattering point for the link from the scattering point to Tx - Dual mobility model in 7.6.10, TR 38.901 is used as start point to model Doppler effect ft due to movement of stochastic clusters, i.e., 2αn,mDn,mλ0 - ft is only applicable for indirect path - Support one term of 2αn,mDn,mλ0 for indirect path of LOS ray+NLOS ray, NLOS ray+LOS ray - Support two terms of 2αn,mDn,mλ0 for indirect path of NLOS ray+NLOS ray - Doppler is separately determined for each of the multiple scattering points of a target - vspt can include macro-Doppler and/or micro-Doppler motion, vspt=vmacrot+vmicro,pt - FFS: maximum speed of moving scatterers - FFS: ratio of moving scatterers among all scatterers Agreement - The following options are supported to generate the combined ISAC channel - Option 1: The ISAC channel of a pair of sensing Tx/Rx is obtained by summing the target channel(s) and background channel, i.e., power normalization is not performed. - Option 2: As an additional modelling component, power normalization is performed when summing the target channel(s) and background channel, to keep the same/similar channel power as the background channel without target. Down select between - Alt 1: Power normalization on both target channel and background channel - Alt 2: Power normalization on background channel only - Alt 3: the target channel of a target will replace one cluster in the background channel - FFS Blockage is modelled for the background channel due to sensing target and/or EO type-2 - FFS condition to select option, e.g. depending on scenario, sensing mode, number of target/EO type-2 Agreement To model the polarization matrix of a direct/indirect path at a scattering point of an object other than EO type-2, the polarization matrix of the scattering point, i.e., CPMsp,i is modelled by αi,1,αi,2,βi,1,βi,2, and initial random phases Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ, i.e., CPMsp,i=αi,1expjΦsp,iθθβi,1expjΦsp,iθϕβi,2expjΦsp,iϕθαi,2expjΦsp,iϕϕ - The initial random phase Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ is [uniformly distributed within -π,π] - FFS correlation between αi,1,αi,2,βi,1,βi,2 - FFS specular reflection - FFS: CPM normalization The following options are considered for further study, down select one option from the following - Option 1: αi,1=αi,2=1, βi,1=βi,2=κsp,i-1 is generated for path i, where κsp,i is XPR ratio - κsp,i is randomly generated by log-normal distribution. FFS mean/variance of the distribution - Option 2: αi,1=1, αi,2,βi,1,βi,2 are variables generated for path i - Option 3: αi,1,αi,2,βi,1,βi,2 are variables generated for path i - CPMsp,i defined in LCS - Option 4: αi,1=αi,2=1, βi,1=βi,2=0 is generated for path i Agreement The finite size of the EO type-2 affects identification of specular reflection point. In the target channel, EO type-2 is modelled only if the specular reflection point is in the area of the EO type-2. Agreement Component B2 of RCS is upper bounded by kσ dB for the log-normal distribution, where σ is the standard deviation of B2 in dB. FFS the value of k. Agreement When the EO type-2 is modelled in the target channel, down select between the following options to determine the LOS condition of the Tx-target link and target-Rx link - Option A: If type-2 EO is in the LOS ray of one link, the link is determined as NLOS condition, and otherwise use the LOS probability equation to determine the LOS/NLOS condition - FFS changes to the LOS probability defined in existing TRs - FFS details on blockage by EO type-2 - Option B: Use the LOS probability equation to determine the LOS/NLOS condition of one link, and then the impacts of type-2 EO is modeled by a blockage model

RAN1#120

Agreement For bistatic/monostatic RCS - RCS values/pattern for a scattering point of a target for bistatic sensing is generated by A*B1*B2 (i.e., Option 3 from the agreement in RAN1 #118bis) - RCS values/pattern obtained by setting the same incident/scattered angle in the RCS model for bistatic sensing should be aligned with RCS for monostatic sensing Agreement RCS model and application in ISAC channel generation - To define the RCS model (RCS=A*B1*B2) for a scattering point of a target, when the target type is vehicle, large size UAV, human with RCS model 2, AGV - The values/pattern of the product of component A and B1, i.e., A*B1 is given per target type, expressed in dBsm scale - Component A is expressed in dBsm scale. B1 is dependent on A*B1 and value of component A. - A is equal to a single value per target type - FFS: this allows different values for the same target type with different size, if needed - FFS: this allows different values for monostatic and bistatic sensing, if needed - Component B2 follows log-normal distribution. The mean μB2_dB and variance σB2_dB2 used to characterize 10lgB2 satisfied a fixed relation μB2_dB=-ln1020σB2_dB2. - In the procedure of generating ISAC target channel, B1*B2 is applied after coupling of rays for a STX-SPST link and the corresponding SPST-SRX link before path dropping - In the procedure of generating ISAC target channel, the following power scaling factor is applied in the last step in target channel generation (i.e., step 14 in the running CR). LTX-SPST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2 Where, - PLdBd1 is pathloss between Tx and SPST, where d1 is the distance between Tx and SPST - PLdBd2 is pathloss between Rx and SPST, where d2 is the distance between SPST and Rx - σRCS,A is the value of RCS component A - SFdB,1,SFdB,2 are shadow fading respectively generated for the Tx- SPST link and SPST -Rx link referring to step 4 in section 7.5, TR 38.901 - Note: for monostatic sensing, PLdBd1=PLdBd2,SFdB,1=SFdB,2 Agreement RCS upper bound: k equals to 3 is adopted to derive the upper bound of RCS component B2, kσ, where σ is the standard deviation of B2 in dB Agreement For reducing options for reference TRs: for sensing scenario UMi, UMa, RMa, InH, InF, UMi-AV, UMa-AV, and RMa-AV, the reference TR to generate a TRP-TRP channel is: - TRP-TRP link of scenario UMi, UMa, InH, and InF following the option based on TR 38.901 defined in section A.3 of TR 38.858 - For InF, hUE is changed to the same height as the BS - TRP-UE link of scenario RMa defined in section 7 of TR 38.901 by setting hUE=35m - FFS: whether to add very low power clusters R1-2501000 Summary #2 on ISAC channel modelling Moderator (Xiaomi) Agreement For vehicle with single/multiple scattering points: - For mono-static, the RCS=A*B=A*B1*B2 for a scattering point of a vehicle is generated by - The values/pattern A*B1, i.e., rcsdB(θ,φ) is deterministic based on incident/scattered angles rcsdB(θ,φ)=Gmax-min-σVdBθ+σHdB φ,σmax Where, σVdBθ=-min12θ-θcenterθ3dB2, σmax, σHdB φ=-min12φ-φcenterφ3dB2, σmax, For example, in case of vehicle with multiple scattering points: φcenter; φ3dB, n; θcenter; θ3dB,n; Gmax; σmax; Applicable Range of θ; Applicable Range of φ. Left; [ ]; [ ]; [ ]; [ ]; [ ]; [ ]; [ ]; [ ]. Back; [ ]; [ ]; [ ]; [ ]; [ ]; [ ]; [ ]; [ ]. Right; [ ]; [ ]; [ ]; [ ]; [ ]; [ ]; [ ]; [ ]. Front; [ ]; [ ]; [ ]; [ ]; [ ]; [ ]; [ ]; [ ]. Roof; [ ]; [ ]; [ ]; [ ]; [ ]; [ ]; [ ]; [ ]. - Note: the applicable angular range is 360 degrees per row in horizontal domain in case of vehicle with multiple scattering points, and the applicable angular range is < 360 degrees per row in horizontal domain in case of vehicle with a single scattering point. - FFS: angular continuity Working assumption For modelling background channel for monostatic sensing: Solution A: (previous Option 1) - drop N reference point(s) following certain distribution, and then the background channel is generated based on the channel generated as in existing TR between the real Tx and the reference point for a scenario. - The distance between Tx and the reference point, and the height of the reference point follow Gamma distribution, drp_2D_n~Γα1, β1+c1 hrp_n~Γα2, β2+c2 - FFS: value of N (N<4) - FFS: whether to add additional very low power clusters - FFS: any update to parameters e.g. angular distribution and delay spread Agreement The existing spatial consistency model in TR 38.901 is reused to model correlation of links between one TRP and different STs/UEs. Agreement Spatial consistency is not modelled at least for the following links - Case 1: the links from/to two non-co-located TRPs - Link TRP1-X and link TRP2-X, where node X can be a target, a UE or another TRP - Link TRP1-X and link TRP2-Y, where node X, Y can be a target, a UE or another TRP - Case 2: the two links with different LOS/NLOS condition - Case 3: background channel for monostatic vs. any link (Tx-ST, ST-Rx, background channel) for bistatic R1-2501001 Summary #3 on ISAC channel modelling Moderator (Xiaomi) Agreement For sensing scenario UMi, UMa, RMa, InH, InF, UMi-AV, UMa-AV, and RMa-AV, the reference TR to generate a UE-UE channel is - UE-UE link of scenario UMi, UMa, InH, and InF following the option based on TR 38.901 defined in section A.3 of TR 38.858 - TRP-UE link of scenario RMa defined in section 7 of TR 38.901 by setting hBS =1.5m - FFS: whether to add very low power clusters Agreement The reference TR to generate a TRP-UE channel is TRP; normal UE; UMi, UMa, RMa, InH, InF, UMi-AV, UMa-AV, and RMa-AV - Option 1: TRP-UE link of scenario UMi, UMa, RMa, InH, and InF in section 7 of TR 38.901 Highway and Urban grid - Option 1: P2B link of scenario Highway and Urban grid in section 6 of TR 37.885 HST - Option 1: TRP-UE link of scenario RMa in section 7 of TR 38.901 for FR1 and TRP-UE link of scenario UMa in section 7 of TR 38.901 for FR2. TRP; vehicle UE; Highway and Urban grid - Option 1: V2B link of scenario Highway and Urban grid in section 6 of TR 37.885 UMi, UMa, and RMa - Option 1: TRP-UE link of scenario UMi, UMa, and RMa in section 7 of TR 38.901. TRP; aerial UE; UMa-AV, UMi-AV, and RMa-AV - Option 1: - TRP-aerial UE link of scenario UMa-AV, UMi-AV, and RMa-AV in section Annex A and B of TR 36.777 for FR1 - FFS reuse the channel model of scenario UMa-AV, UMi-AV, and RMa-AV of FR1 for FR2. Agreement For mono-static, the following values of component A, B2 are agreed for UAV of small size - Component A: -12.81 dBsm - Component B1: 0 dB (already agreed in RAN1#118bis) - Component B2: - standard deviation: 3.74 dB Agreement For mono-static, the following values of component A, B2 are agreed for RCS model 1 of human - Component A: -1.37 dBsm - Component B1: 0 dB (already agreed in RAN1#118bis) - Component B2: - standard deviation: 3.94 dB Working assumption Absolute delay model (referring to 7.6.9 in TR 38.901 as starting point) is a mandatory feature for both target channel and background channel for ISAC for UMi, UMa, InH, InF - Related model referring to Δτ values from 7-24GHz study item Agreement When absolute delay model Δτ is configured, it applies to all NLOS clusters in each of Tx-target and target-Rx links and background channel. - For bistatic sensing: Different values of Δτ are separately generated for the Tx-target link, target-Rx link and the background channel - For monostatic sensing: the same value of Δτ is used for Tx-target link and target-Rx link, and a different value of Δτ is separately generated for the background channel Agreement To generate the LOS ray and NLOS clusters for the multiple scattering points, each scattering point is separately handled as if a different target with single scattering point. Agreement - The LOS condition between Tx/Rx and each of the multiple scattering points of a same target are individually generated - The pathloss between Tx/Rx and each of the multiple scattering points of a same target are individually generated Agreement For a target with single/multiple scattering points, the 3D location of each scattering point is defined in the evaluation assumptions. Agreement Spatial consistency is needed to model correlation of the following links from ST-UT links and UT-UT links - Case 5: links between same UT and two nodes X/Y, subjected to correlation distance, i.e., link UT1-X and link UT1-Y, where nodes X/Y can be target or UT - Case 6: links between same target and two nodes X/Y, subjected to correlation distance, i.e., link target1-X and link target1-Y, where nodes X, Y are different UTs - Case 7: link X1-Y1 and link X2-Y2, subjected to correlation distance, where X1, X2, Y1, Y2 are 4 different nodes - FFS: Spatial consistency between multiple scattering points of the same target Agreement Correlation type is introduced for large scale parameter, cluster specific parameter and ray specific parameter of ST-UT links and UT-UT links - Definition of link Correlated: parameters for any two links between STs/UTs are correlated, subjected to correlation distance. Table 4: Correlation type for links between STs/UTs Parameters; Correlation type. Delays; link Correlated. Cluster powers; link Correlated. AOA/ZOA/AOD/ZOD offset; link Correlated. AOA/ZOA/AOD/ZOD sign; link Correlated. Random coupling; link Correlated. XPR; link Correlated. Initial random phase; link Correlated. LOS/NLOS states; link Correlated. Blockage (Model A); All-correlated. O2I penetration loss; All-correlated. Indoor distance; All-correlated. Indoor states; All-correlated. - Note: it is not precluded more parameters for spatial consistency can be discussed and added in the table Agreement If a target is modelled with multiple scattering points, - The number of scattering points of the target is generated in the beginning of the simulation and kept unchanged in the whole simulation - The number and locations of the scattering points of the target (if it is a vehicle) are common to each pair of sensing Tx/Rx - RAN1 assumes no ray is scattered from one scattering point to another scattering point of the same target - RCS values of the multiple scattering points are individually determined Agreement To model polarization matrix CPMsp,i=αi,1expjΦsp,iθθβi,1expjΦsp,iθϕβi,2expjΦsp,iϕθαi,2expjΦsp,iϕϕ of a direct/indirect path i of a scattering point of a target - in Rel-19 study item (e.g., UAV, human, vehicle, AGV), αi,1=αi,2=1, βi,1=βi,2=κsp,i-1, i.e., CPMsp,i=expjΦsp,iθθκsp,i-1expjΦsp,iθϕκsp,i-1expjΦsp,iϕθexpjΦsp,iϕϕ Where, - κsp,i is XPR ratio is randomly generated by log-normal distribution per target type - The initial random phase Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ is uniformly distributed within -π,π - FFS: spatial consistency of random phase when a scattering point moves - FFS: whether the base station rotation procedure in 38.901 can be reused to support rotation of the target Agreement - For target channel - The power threshold for removing clusters in step 6 in section 7.5, TR 38.901, i.e., -25 dB is reused to generate Tx-target link and target-Rx link - The power threshold for path dropping after concatenation is relaxed to [X=-40] dB - For background channel - The power threshold for removing clusters in step 6 in section 7.5, TR 38.901, i.e., [-25 dB] is reused to generated the background channel - FFS: whether to add additional very low power clusters - FFS: The reference power for removing cluster is the min (max. Tx-target link cluster power, max. target-Rx link cluster power) Agreement If EO type-2 is modelled in an indirect path, only specular reflection is modeled for EO type-2 - polarization of the indirect path is product of polarization matrix of the target-Rx link, the target, and the Tx-target link, i.e., CPMtx,sp,rx= CPMsp,rx . CPMsp . CPMtx,sp - For the specular reflected ray generated by a EO type-2 in the Tx-target link or the target-Rx link (i.e., Tx-EO type-2-target, or target-EO type-2-Rx), CPMtx,sp or CPMsp,rx is the polarization matrix of EO type-2 - To generate polarization matrix of EO type-2, the procedure in [R1-2409394, R1-2410648] is taken as starting point - FFS applicability if the surface of EO type-2 is tilted