3GPP TSG RAN WG1 #121		                           R1-2503584

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:		9.11.3

Source:			Samsung

Title:			Discussion on uplink capacity/throughput enhancement for NR-NTN

Document for:		Discussion and decision

- Introduction

In RAN1#120bis [1], it was discussed about candidate techniques for improving uplink capacity/throughput, and the following agreements were reached.

Agreement RAN1 assumes that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH. - FFS: under which conditions the above applies, e.g. under the same conditions that phase continuity applies for DMRS bundling Agreement Send LS to RAN4 on requirements for the phase continuity and power consistency: RAN1 has agreed the following: - RAN1 assumes that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH. - FFS: under which conditions the above applies, e.g. under the same conditions that phase continuity applies for DMRS bundling RAN1 ask RAN4 whether the same phase continuity requirements as for DMRS bundling can be applied for OCC length 2 and/or OCC length 4 under the same conditions as for DMRS bundling, or if new requirements are needed. Agreement The draft LS in R1-2503070 is endorsed. Final LS is in R1-2503071. Agreement OCC length and OCC sequence for OCC with CG PUSCH Type 1 is configured by RRC higher-layers. - Up to RAN2 whether to signal this with one or two RRC parameters - Note: OCC lengths and sequences to be provided in the table of RRC parameters to be prepared by the WI rapporteur Agreement For OCC with CG-PUSCH, the RV sequence applied across OCC groups is RRC configured among the RV sequences defined for legacy CG PUSCH, i.e., [0,2,3,1], [0,3,0,3] or [0,0,0,0]. - Note: no new RRC parameter is needed for the above - FFS: if OCC group dropping is later agreed, how to count RVs may need to be discussed for that case Agreement For resolving the overlapping PUSCH repetitions with inter-slot OCC and PUCCH with/without repetitions, the legacy timeline conditions for UCI multiplexing or prioritization for dropping PUSCH [/ PUCCH] applies according with the following update: - “the first PUSCH repetition of an OCC group” is used instead of “PUSCH repetition” for the legacy timeline condition, where the legacy PUSCH repetition that overlaps with a PUCCH belongs to the OCC group Note: how to capture the above agreement is up to the spec editor. Agreement For the OCC length applied for OCC DG PUSCH and CG PUSCH Type 2, consider the following options: - Option 1: Configured by RRC higher-layer parameter. - Note: this does not preclude jointly configuring OCC sequence and OCC length with the same RRC configuration - Option 2: Max value is configured by RRC higher-layer parameter, and the applied value is implicitly determined from the configured repetition number. - Option 3: Candidates values are configured by RRC higher-layer parameter, and the applied value is indicated by scheduling DCI for DG PUSCH or by activation DCI for CG PUSCH Type 2. - Note: this does not preclude jointly configuring OCC sequence and OCC length with the same RRC configuration - Option 4: Single value is indicated by scheduling DCI for DG PUSCH and by activation DCI for CG PUSCH Type 2 (no RRC configuration of candidate values). - Option 5: Max value is configured by RRC higher-layer parameter, and the applied value is indicated by scheduling DCI for DG PUSCH and by activation DCI for CG PUSCH Type 2. Note: it is not precluded to select a different option for DG PUSCH and CG PUSCH Type 2. Agreement If PUCCH without repetitions overlaps with inter-slot OCC with any PUSCH repetitions in an OCC group with a same priority index for PUCCH/PUSCH, the legacy conditions and rules for UCI multiplexing or prioritization for dropping applies with the following updates: - If the UCI is multiplexed on the PUSCH repetition according to legacy rules and the updated timeline conditions for UCI multiplexing are satisfied, UCI is multiplexed on all PUSCH repetitions without A-CSI reports within an OCC group with inter-slot OCC overlaps with the PUCCH. (Option 3-a) - FFS: PUSCH repetition with A-CSI reports - If the PUSCH repetition is dropped according to legacy rules and the updated timeline conditions for PUSCH dropping are satisfied, UCI is transmitted on PUCCH and all PUSCH repetitions within the OCC group that overlaps with the PUCCH are dropped (Option 2) - FFS: if PUCCH is overlap with PUSCH repetition in both time and frequency domain. - UE does not expect there are multiple PUCCHs without repetitions in different PUCCH slots with a same or different UCI types other than SR overlapping with multiple PUSCH repetitions in the same OCC group. - FFS: whether the above applies only when at least one of the overlapping PUCCHs result in a UCI being multiplexed on the PUSCH Note 1: If the UCI on the PUCCH is dropped according to legacy rules and [updated] timeline conditions for UCI dropping are satisfied, there is no [additional] spec impact. (Option 1) Note 2: There can be multiple PUCCHs with same or different UCI types in the same slot (i.e. CSI report and HARQ-ACK) as in the legacy specifications Working assumption 1: The above agreement applies to different priority indexes for PUCCH/PUSCH if no additional specification impact is identified. Working assumption 2: The above agreement applies to PUCCH with repetitions if no additional specification impact is identified. Agreement For the OCC sequence applied for OCC DG PUSCH and CG PUSCH Type 2, the sequence is indicated dynamically in DCI - FFS: with a new field or reusing an existing field

In this contribution, it discusses on remaining issues for PUSCH OCC.

- Discussion

Inter-slot OCC: Intra-UE multiplexing/prioritization

Intra-UE multiplexing/prioritization was discussed in [1] and the following agreement was made.

Agreement If PUCCH without repetitions overlaps with inter-slot OCC with any PUSCH repetitions in an OCC group with a same priority index for PUCCH/PUSCH, the legacy conditions and rules for UCI multiplexing or prioritization for dropping applies with the following updates: - If the UCI is multiplexed on the PUSCH repetition according to legacy rules and the updated timeline conditions for UCI multiplexing are satisfied, UCI is multiplexed on all PUSCH repetitions without A-CSI reports within an OCC group with inter-slot OCC overlaps with the PUCCH. (Option 3-a) - FFS: PUSCH repetition with A-CSI reports - If the PUSCH repetition is dropped according to legacy rules and the updated timeline conditions for PUSCH dropping are satisfied, UCI is transmitted on PUCCH and all PUSCH repetitions within the OCC group that overlaps with the PUCCH are dropped (Option 2) - FFS: if PUCCH is overlap with PUSCH repetition in both time and frequency domain. - UE does not expect there are multiple PUCCHs without repetitions in different PUCCH slots with a same or different UCI types other than SR overlapping with multiple PUSCH repetitions in the same OCC group. - FFS: whether the above applies only when at least one of the overlapping PUCCHs result in a UCI being multiplexed on the PUSCH Note 1: If the UCI on the PUCCH is dropped according to legacy rules and [updated] timeline conditions for UCI dropping are satisfied, there is no [additional] spec impact. (Option 1) Note 2: There can be multiple PUCCHs with same or different UCI types in the same slot (i.e. CSI report and HARQ-ACK) as in the legacy specifications Working assumption 1: The above agreement applies to different priority indexes for PUCCH/PUSCH if no additional specification impact is identified. Working assumption 2: The above agreement applies to PUCCH with repetitions if no additional specification impact is identified.

Regarding the first FFS: PUSCH repetition with A-CSI reports, the current specification supports only the first PUSCH slots used for A-CSI multiplexing. However, it is unclear how to support PUSCH repetition with inter-slot level OCC when A-CSI reports are triggered, especially when there is no UCI to overlap with one of the PUSCH repetitions. The simplest approach would be to not support this case, meaning the UE should not expect to transmit A-CSI reports based on PUSCH repetition if inter-slot level OCC is applied to the PUSCH repetition. Over-optimizing this scenario may not be necessary.

Proposal 1: RAN1 conclude that for PUSCH repetitions with A-CSI reports, inter-slot OCC does not apply .

Regarding the second FFS: If PUCCH overlaps with PUSCH repetition in both the time and frequency domains, there is no need for a special design consideration. Additionally, if PUSCH repetition is dropped according to legacy rules and the updated timeline conditions for PUSCH dropping are met, this scenario applies to both cases: 1) PUCCH overlapping with PUSCH repetition in only the time domain, and 2) PUCCH overlapping with PUSCH repetition in both the time and frequency domains.

Observation 1: No further clarification is needed in case where PUCCH is overlap with PUSCH repetition in both time and frequency domain.

Regarding the third FFS: It is unclear whether the above rule applies only when at least one of the overlapping PUCCHs results in UCI being multiplexed on the PUSCH. There is no clear justification for this additional restriction. The primary reason for prohibiting multiple PUCCHs without repetition across different PUCCH slots is that defining multiplexing UCI over these slots would require significant specification effort. This is because it would involve multiplexing UCI into PUSCH repetitions with identical UCI contents within the same OCC group.

Regarding the two working assumptions, it is preferred to confirm both considering there is no critical issue for supporting the feature.

Proposal 2: Confirm the following two working assumptions made in RAN1#120b meeting.

- Working assumption 1: The above agreement applies to different priority indexes for PUCCH/PUSCH if no additional specification impact is identified.

- Working assumption 2: The above agreement applies to PUCCH with repetitions if no additional specification impact is identified.

The following agreement was made regarding the timeline conditions in RAN1#120b meeting

Agreement For resolving the overlapping PUSCH repetitions with inter-slot OCC and PUCCH with/without repetitions, the legacy timeline conditions for UCI multiplexing or prioritization for dropping PUSCH [/ PUCCH] applies according with the following update: - “the first PUSCH repetition of an OCC group” is used instead of “PUSCH repetition” for the legacy timeline condition, where the legacy PUSCH repetition that overlaps with a PUCCH belongs to the OCC group. Note: how to capture the above agreement is up to the spec editor.

One open issue is whether PUCCH should be included in the timeline conditions, including PUCCH in the timeline conditions would be a simple and unified design and the difference for the timeline conditions with/without considering the PUCCH is just a couple of symbols.

Proposal 3: Update the agreement made in RAN1#120b as following

Agreement For resolving the overlapping PUSCH repetitions with inter-slot OCC and PUCCH with/without repetitions, the legacy timeline conditions for UCI multiplexing or prioritization for dropping PUSCH [/ PUCCH] applies according with the following update: - “the first PUSCH repetition of an OCC group” is used instead of “PUSCH repetition” for the legacy timeline condition, where the legacy PUSCH repetition that overlaps with a PUCCH belongs to the OCC group. Note: how to capture the above agreement is up to the spec editor.

Inter-slot OCC: sequence indication and length indication

Agreement OCC length and OCC sequence for OCC with CG PUSCH Type 1 are configured by RRC higher-layers. - Up to RAN2 whether to signal this with one or two RRC parameters. - Note: OCC lengths and sequences to be provided in the table of RRC parameters to be prepared by the WI rapporteur. Agreement For the OCC length applied for OCC DG PUSCH and CG PUSCH Type 2, consider the following options: - Option 1: Configured by RRC higher-layer parameter. - Note: this does not preclude jointly configuring OCC sequence and OCC length with the same RRC configuration - Option 2: Max value is configured by RRC higher-layer parameter, and the applied value is implicitly determined from the configured repetition number. - Option 3: Candidates values are configured by RRC higher-layer parameter, and the applied value is indicated by scheduling DCI for DG PUSCH or by activation DCI for CG PUSCH Type 2. - Note: this does not preclude jointly configuring OCC sequence and OCC length with the same RRC configuration - Option 4: Single value is indicated by scheduling DCI for DG PUSCH and by activation DCI for CG PUSCH Type 2 (no RRC configuration of candidate values). - Option 5: Max value is configured by RRC higher-layer parameter, and the applied value is indicated by scheduling DCI for DG PUSCH and by activation DCI for CG PUSCH Type 2. Note: it is not precluded to select a different option for DG PUSCH and CG PUSCH Type 2. Agreement For the OCC sequence applied for OCC DG PUSCH and CG PUSCH Type 2, the sequence is indicated dynamically in DCI - FFS: with a new field or reusing an existing field

It was agreed in RAN1#120b meeting that OCC length and OCC sequence for OCC with CG PUSCH Type 1 are configured by RRC higher-layers and it is up to RAN2 whether to signal this with one or two RRC parameters. A unified solution is preferred for CG PUSCH Type 1 and DG PUSCH for simplicity as well as consistency. RRC configures N candidate values of {OCC length and OCC sequence} and a new field in the DCI indicates one of N candidate values in case of N > 1.

Proposal 4: For DG PUSCH and CG PUSCH Type 2,

- RRC configures N candidate values of {OCC length and OCC sequence} and a new field in the DCI indicates one of N candidate values in case of N > 1.

OCC for PUSCH: frequency hopping

For frequency hopping, it is quite straightforward that intra-slot and inter-slot frequency hopping cannot be applicable to inter-slot OCC PUSCH since it degrades performance due to phase shift. However, it is not clear how much performance gain can be achieved if frequency hopping is performed per OCC group (that is, inter-OCC group frequency hopping). Therefore, RAN1 needs to discuss performance benefit before deciding whether to enhance frequency hopping for inter-slot OCC PUSCH.

Proposal 5: RAN1 discusses performance benefit in order to decide whether to enhancement frequency hopping for inter-slot OCC PUSCH.

OCC for PUSCH: TBoMS

TBoMS has been introduced since Rel-17 spec in order to support coverage enhancement. Though the scheme can be considered in Rel-19 NTN with potential OCC scheme, it might be better to consider after finalizing basic procedure(s) of potential OCC scheme first without considering TBoMS. Final decision should be made with the consideration of potential specification impact(s) on top of the basic procedure(s).

Proposal 6: RAN1 defers the decision whether to support TBoMS for PUSCH with OCC in Rel-19 NTN.

OCC for PUSCH: limitation of the PRB

The first issue is whether to stipulate the specific number of RBs that OCC can be applied for PUSCH. In NTN, it is well known that a UE may use smaller RBs, and FDM scheduling can be considered instead of applying OCC over larger RBs for PUSCH transmission. Although there would be no critical reason for not limiting the number of PRBs, it does not need to mandate a specific PRB number applicable to OCC PUSCH in the specification. It would be up to the gNB implementation. If there would be a high burden for some UEs to handle various numbers of PRBs for supporting OCC PUSCH, it could be discussed in the UE feature session.

Proposal 7: RAN1 does not discuss further whether to limit the number of PRBs applicable to OCC PUSCH.

Conclusion

This contribution discussed on remaining issues for uplink capacity/throughput enhancements. Followings are proposals and observations in this contribution.

Proposal 1: RAN1 conclude that for PUSCH repetitions with A-CSI reports, inter-slot OCC does not apply .

Proposal 2: Confirm the following two working assumptions made in RAN1#120b meeting.

- Working assumption 1: The above agreement applies to different priority indexes for PUCCH/PUSCH if no additional specification impact is identified.

- Working assumption 2: The above agreement applies to PUCCH with repetitions if no additional specification impact is identified.

Proposal 3: Update the agreement made in RAN1#120b as following

Agreement For resolving the overlapping PUSCH repetitions with inter-slot OCC and PUCCH with/without repetitions, the legacy timeline conditions for UCI multiplexing or prioritization for dropping PUSCH [/ PUCCH] applies according with the following update: - “the first PUSCH repetition of an OCC group” is used instead of “PUSCH repetition” for the legacy timeline condition, where the legacy PUSCH repetition that overlaps with a PUCCH belongs to the OCC group. Note: how to capture the above agreement is up to the spec editor.

Proposal 4: For DG PUSCH and CG PUSCH Type 2,

- RRC configures N candidate values of {OCC length and OCC sequence} and a new field in the DCI indicates one of N candidate values in case of N > 1.

Proposal 5: RAN1 discusses performance benefit in order to decide whether to enhancement frequency hopping for inter-slot OCC PUSCH.

Proposal 6: RAN1 defers the decision whether to support TBoMS for PUSCH with OCC in Rel-19 NTN.

Proposal 7: RAN1 does not discuss further whether to limit the number of PRBs applicable to OCC PUSCH.

Observation 1: No further clarification is needed in case where PUCCH is overlap with PUSCH repetition in both time and frequency domain.

References

[1] RAN1#120bis Chair note