3GPP TSG RAN WG1 #121				     R1-2503593

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	9.15.3

Source:	Samsung

Title:	UE features for Rel-19 Duplex Evolution for NR

Document for:	Discussion

# 1. Introduction

This contribution provides Samsung’s view regarding UE features for Rel-19 Duplex evolution following the discussions and decisions in RAN1#120 and RAN1#120bis.

# 2. Discussion

  - 2.1. UE features for SBFD operation (excluding Random Access or CLI)

The following agreements on UE features for SBFD operation (excluding Random Access or CLI) were made in RAN1#120bis [2][3].

<FG 60-1> Agreement: Update FG 60-1 as follows: - Confirm “cell-common” in component 1 - Confirm “UL transmission in one UL subband and DL reception in one [and two] DL subband in SBFD symbols” - Confirm Component 3: “3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling” - Add “Support of separate CSI reports for SBFD and non-SBFD symbols 60-1; Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures; 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one [and two] DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling 4. Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1) 5. Support of separate CSI reports for SBFD and non-SBFD symbols FFS: Whether to include two DL subband support in component 2 FFS: other component (which may be separate FG); YES; n/a; Basic feature for SBFD-aware UE TX/RX/ measurement behaviour and procedures is not supported; Per band; TDD only; n/a; n/a; [Note: Only UL transmissions are allowed within UL subband and only DL receptions (except for CLI measurement) are allowed within DL subband(s)] [Note: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one.]; Optional with capability signalling. Agreement: Adopt “Per band” type for FG 60-1. Agreement: Add the following components in FG 60-1 (basic FG for SBFD operation) - Support of separate PUCCH frequency resource configuration in SBFD symbols and non-SBFD symbols - [FFS: Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols] - Support of separate PUSCH frequency hopping offset in SBFD symbols and non-SBFD symbols - [FFS: Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols]

Regarding the basic FG for SBFD (60-1), our view is

- FFS: Whether to include two DL subband support in component 2

  - The basic SBFD feature should be to DU/UD case as it is simpler than DUD case which can be a separate feature

- FFS: Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols]

  - As per symbol type, UE needs to determine UE power control parameter, which requires additional UE complexity. However, the additional complexity is quite marginal since the SBFD symbol determination is semi-static and a UE already have different UL power control parameters associated with TCI state.

- FFS: Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols

  - For periodic and semi-persistent SRS, the additional complexity for separate SRS resource sets could be marginal since gNB will configure a proper periodicity and offset. For aperiodic SRS without available slot counting, again gNB will indicated a proper slot for SRS transmission. For aperiodic SRS with available slot counting, a UE requires to find a valid slot for SRS transmission. This is similar as PUSCH repetition type-A or PUCCH repetition in case of configuration 1. Since configuration 1 is already supported in Basic feature 60-1, the additional complexity could be marginal.

- FFS: other component (which may be separate FG)

  - Link direction determination should be supported as basic FG. Otherwise, UE cannot resolve any collisions on SBFD symbol

Proposal 1. Support the following for basic FG 60-1

| 60-1 | Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures | 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one [and two] DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling 4. Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1) 5. Support of separate CSI reports for SBFD and non-SBFD symbols 6. Support of separate PUCCH frequency resource configuration in SBFD symbols and non-SBFD symbols 7. Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols] 8. Support of separate PUSCH frequency hopping offset in SBFD symbols and non-SBFD symbols 9. Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols 10. Link direction determination in SBFD symbols FFS: Whether to include two DL subband support in component 2 FFS: other component (which may be separate FG) |  | YES | n/a | Basic feature for SBFD-aware UE TX/RX/ measurement behaviour and procedures is not supported | Per band | TDD only | n/a | n/a | [Note: Only UL transmissions are allowed within UL subband and only DL receptions (except for CLI measurement) are allowed within DL subband(s)] [Note: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one.] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

<FG 60-2> Agreement: Introduce following FG for [Transmissions and] Receptions across SBFD and non-SBFD symbols (Configuration 2) as follows, and FFS on whether transmission across SBFD and non-SBFD symbols is defined as separate FG or not - Alt.1: Configuration 2 for transmission and reception should be defined in a single FG - Alt.2: Configuration 2 for transmission and reception should be defined in separate FGs 60-2; [Transmission and] Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2); 1. Support of PDSCH reception across SBFD symbols and non-SBFD symbols (Configuration 2) [2. Support of UL transmission across SBFD symbols and non-SBFD symbols (Configuration 2)] FFS: Other component(s) FFS: whether component 2 is defined as separate FG; YES; n/a; [Transmission and] Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling.

Regarding FG 60-2, our views are

- Configuration 2 can be split into two FGs for UL-Tx and DL-Rx. But, we generally fine to define single FGs for UL-Tx and DL-Rx to avoid lots of FGs (Alt 1)

- It can be per band

Proposal 2. Support the following FG 60-2 for SBFD to support configuration 2.

| 60-2 | [Transmission and] reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) | 1. Support of PDSCH reception across SBFD symbols and non-SBFD symbols (Configuration 2) [2. Support of UL transmission across SBFD symbols and non-SBFD symbols (Configuration 2)] FFS: Other component(s) FFS: whether component 2 is defined as separate FG |  | YES | n/a | [Transmission and] Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) is not supported | [Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For two DL subbands, our views are

- Define separate FG to support DL reception across two DL subbands in SBFD symbols.

- PDSCH reception across two DL subbands in SBFD symbols and CSI-RS reception across two DL subbands in SBFD symbols can be defined in one FG.

Proposal 3. Support the following FG for SBFD to support DL reception across two DL subbands in SBFD symbol

| 60-A | [Support of DL reception in two non-contiguous DL subbands] | 1. Support of DL reception in two non-contiguous DL subbands 2. Support of CSI-RS reception in two non-contiguous DL subbands 3. CSI processing timeline in SBFD symbols for the support of one contiguous CSI-RS resource allocation across two DL subbands FFS: whether each of components within brackets is separate FG or within a combined FG FFS: other component (which may be separate FG) | 60-1 | Yes |  | DL reception in two non-contiguous DL subbands is not supported | Per band |  |  |  | [For component 3, candidate values are {Legacy, Scaled}] [‘Legacy’ means the CSI processing timeline and CPU counting are the same as legacy. ‘Scaled’ means the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy.] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

How to count of non-contiguous CSI-RS towards the maximum number of active CSI-RS and CSI-RS port per CC and across all CCs should be reflected in UE capability. Similar to the CSI processing timeline, the non-contiguous CSI-RS may be counted as 1 (legacy) or 2 (scaled) towards the maximum number of active CSI-RS and CSI-RS port per CC and across all CCs. That is, some UEs may need to relax the counting of non-contiguous CSI-RS towards the limit of the maximum number of active CSI-RS and CSI-RS port.

Proposal 4. Support the following FG for SBFD to take into account counting of non-contiguous CSI-RS across two DL subbands in SBFD symbol

| 60-B | Counting of non-continuous CSI-RS | 1. Counting of non-contiguous CSI-RS towards the maximum number of active CSI-RS and CSI-RS port per CC and across all CCs (maxNumberSimultaneousNZP-CSI-RS-PerCC, totalNumberPortsSimultaneousNZP-CSI-RS-PerCC, maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC) is scaled by R | X-3 | Yes |  | Non-continuous CSI-RS across two DL subbands cannot be configured | Per band |  |  |  | Candidate value for R in component 1 is {1, 2} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Additionally, PDSCH will be rate matched when it crosses the boundaries of DL usable PRBs. In legacy, a UE may be configured with up to 4 RateMatchPattern(s) per BWP and up to 4 RateMatchPattern(s) per serving cell. For example, once a UE supports semi-static rate matching, this UE has to support up to 4 rate matching patters per BWP and per cell where these values are predefined in 38.214. To avoid increasing the UE complexity, such operation should be counted towards the legacy limits on RateMatchPattern(s) per BWP or per cell. It may be counted as a predefined value, e.g., one RateMatchPattern, or it may be counted as β RateMatchPattern(s) where the value of β is indicated via UE capability signaling.

Proposal 5. Support the following FG for SBFD to take into account rate-matching of PDSCH reception within DL subband(s) in SBFD symbol

| 60-C | Rate matching of PDSCH reception within DL subband(s) | 1. Counting of PDSCH rate matching within DL subband(s) toward the maximum number of cell-specific RateMatchingPattern(s) | X-1 | Yes |  | PDSCH cannot be rate-matched within DL subband(s) when 4 cell-specific RateMatchingPattern(s) are configured | Per band |  |  |  | The maximum number of RateMatchPattern(s) that can be configured per BWP or per cell is 3, when rate matching of PDSCH occurs because of cross the boundaries of DL usable PRBs. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - 2.2. UE features for SBFD operation (Random Access related)

The following agreements were made in RAN1#120 [3].

<60-3, 60-4>

Agreement (RAN1#120)

Introduce following FGs for SBFD random access, one based on single RACH configuration (Option 1) and the other based on two RACH configurations (Option 2), and FFS on whether UE supporting SBFD random access needs to support at least Option 1 or Option 2.

- Alt.1: UE supporting SBFD random access needs to support at least Option 1

- Alt.2: UE supporting SBFD random access needs to support at least Option 2

- Alt.3: UE supporting SBFD random access supports Option 1 or Option 2 (i.e., no default option)

| 60-3 | SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode | Support of SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 1) FFS: Other component(s) which may be separate FG | X-1 | YES | n/a | SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode is not supported | [Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60-4 | SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode | Support of SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 2) FFS: Other component(s) which may be separate FG | [X-1, X-3] | YES | n/a | SBFD random access operation based on two separate RACH configurations in RRC_IDLE/INACTIVE/CONNECTED mode is not supported | [Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |

Agreement:

Adopt “Per band” type for FG 60-3.

Agreement:

Adopt “Per band” type for FG 60-4

In RAN1#120, 2 separate FGs were agreed for UE support of RACH configuration Option 1 or 2, respectively. For the gNB, the choice to implement and/or support one of RACH configuration Option 1 or 2 in the serving cell is not restricted. For certain use cases such as extended coverage/cell range using random access on SBFD resources, RACH configuration Option 2 is needed to configure long preamble formats such as B4/F0 when not possible on the legacy ROs. In other cases, PRACH repetition using either RACH configuration Options 1 or 2 can be sufficient. It is also possible, that random access on SBFD resources in the serving cell is enabled by the gNB only for UEs in RRC_CONNECTED mode.

Alt.3, i.e., no default option, has significant risk for feature fragmentation. A UE supporting X-3 entering a cell indicating support for RACH config Option 2 cannot benefit from random access on SBFD resources. Similarly, when the gNB configures use of RACH config Option 1 in SIB1, but the UE is not required to support X-3, there is little incentive to implement such a feature. Our strong preference is therefore at least Alt.1, or alternatively, the new Alt. 4 “UE supporting SBFD random access supports Option 1 and Option 2.

Proposal 6. Support the (new) Alt. 4 “UE supporting SBFD random access supports Option 1 and Option 2”.

Several more FL proposals for random access on SBFD resources were discussed in RAN1#120.

Proposal 3-3 (RAN1#120): Introduce following FG (as FFS) for PRACH repetition in additional ROs, and FFS on whether separate FGs for RACH configuration Option 1 and Option 2 are necessary. - Alt.1: separate FGs for RACH configuration Option 1 and Option 2 are NOT necessary - Alt.2: separate FGs for RACH configuration Option 1 and Option 2 are necessary - Alt.3: If a UE reports legacy Rel-18 FG for PRACH repetition, and also reports support of FG X-3/4, it means the UE supports RACH repetition in SBFD operation for Option 1/2 respectively. X-5; Preamble repetition within additional-ROs; 1. Determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-ROs [2. For RACH configuration Option 1, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.] [3. For RACH configuration Option 2, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 and msg1-RepetitionNum for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.] FFS: separate FGs for RACH configuration Option 1 and Option 2 are necessary; [X-3 or X-4, 54-1]; YES; n/a; Preamble repetition within additional-ROs is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. Proposal 3-4 (RAN1#120): Introduce following FG (as FFS) for Msg.3 repetition in SBFD symbols. X-5; Msg.3 repetition in SBFD symbols; Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI in SBFD symbols; [X-1 or X-3 or X-4, 30-6]; YES; n/a; Msg.3 repetition in SBFD symbols is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling.

Regarding proposal 3-3 (PRACH repetition), we prefer Alt.2 “separate FGs for RACH configuration Option 1 and Option 2” to indicate support for PRACH repetition. PRACH repetition based on Rel-18 principles is already readily supported for the case of RACH configuration Option 2. PRACH repetition with RACH configuration Option 1 is a delta which the UE implementation may or may not choose to support.

We support proposal 3-4 (PUSCH msg.3 repetition”).

Proposal 7. For proposal 3-3 (PRACH repetition) support Alt.2: separate FGs for RACH configuration Option 1 and Option 2 are necessary.

Proposal 8. Support/introduce the FG as by proposal 3-4 (PUSCH msg.3 repetition).

  - 2.3. UE features for CLI handling

    - 2.3.1. UL resource muting for PUSCH in SBFD symbols

The following agreements on UE features for UL muting were made in RAN1#120bis [2][3].

<60-7, 60-7a> Agreement (RAN1#120bis): Introduce the following FGs for UL resource muting 60-7; UL resource muting for CP-OFDM waveform; 1. Support semi-static configuration of time location and frequency location of UL resource muting for CP-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol [- FFS: Whether this FG only support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH] 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH FFS: other potential component/FG/additional restriction; [FFS]; YES; n/a; UL resource muting for CP-OFDM waveform is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. 60-7a; UL resource muting for DFTS-OFDM waveform; 1. Support semi-static configuration of time location and frequency location of UL resource muting for DFTS-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol [- FFS: Whether this FG only support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH] 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH FFS: other potential component/FG/additional restriction; [FFS]; YES; n/a; UL resource muting for DFTS-OFDM waveform is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling.

Regarding the “FFS: Whether this FG only support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH” –

It is our understanding that RRC should restrict the possible configuration of muting symbols to one/same set of either 1 or 2 symbol positions for the DG. However, at least one different/separate (from DG) common pattern of UL muting is required for Type 2 CG PUSCH and Type 1 CG. One reason is that from the gNB perspective, it should be possible to estimate the inter-gNB CLI at least for one set of alternative UL muting positions when compared to the DG based PUSCH allocations for which the UL muting can be dynamically indicated on/off by the TDRA field. We also consider that the support of one different/separate set of UL muting symbols for the CG allocations would not result in undue UE complexity.

Proposal 9: For FGs 60-7/60-7a and FFS: Whether this FG only support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH

Support one common pattern of UL muting for DG and one common pattern for Type 2 CG PUSCH and Type 1 CG PUSCH.

Regarding the FFS: other potential component/FG/additional restriction -

We propose to add the following component to 60-7/60-7A: “Support 1 or 2 UL muting symbols on PUSCH > 7 symbols for PUSCH mapping type A”. Support for UL muting on short PUSCH, e.g., <8 symbols is not justified. The gNB can create symbol level scheduling gaps during back-to-back scheduling for CLI estimation/equalization.

Proposal 10: For FG 60-7/60-7A, add component “Support 1 or 2 UL muting symbols on PUSCH > 7 symbols for PUSCH mapping type A”.

We propose to add the following component to 60-7/60-7A: “For PUSCH mapping type A, support 1 UL muting symbol before the first FL DMRS and 1 UL muting symbol after the first FL DMRS”. The reason is that the original motivation for support of up to 2 UL muting symbols is using one UL muting symbol for CLI equalization in the data region of the slot, i.e., in the middle of the slot with respect to PDSCH-PUSCH, but the other second UL muting symbol for CLI equalization in the control region of the slot, i.e., on the first or second symbol with respect to DL-UL interference incurred from PDCCH. Therefore, for PUSCH mapping type B, the UE only needs to support 1 UL muted symbol.

Proposal 11: For FG 60-7/60-7A, add component “For PUSCH mapping type A, support 1 UL muting symbol before the first FL DMRS and 1 UL muting symbol after the first FL DMRS”

Proposal 12: For FG 60-7/60-7A, add component “For PUSCH mapping type B, support 1 UL muting symbol”.

In our view, mandatory UE side support of UL muting on PUSCH symbols that can carry UCI is not required. For Rel-19 UEs not supporting such a functionality, the gNB can still configure time-domain locations of UL muting symbols based on the maximum UCI payload assumption and accounting for the beta offsets. When the UE not supporting UL muting on PUSCH symbols that can carry UCI determines presence of UCI on such a symbol resulting from the control/data multiplexing for PUSCH, the UE does not apply the UL muting the in the symbol.

It is even more important to exclude specific PUSCH allocation cases such as UCI only payload from UL muting. Even with a combined CSI and A/N payload size of ~2000 bits, the resulting PUSCH allocation would typically not exceed a few symbols over the PUSCH allocation bandwidth. For UCI-only PUSCH, a transparent muting symbol can easily be created by the gNB as scheduling gap following the UCI-only PUSCH similar to the case of a short PUSCH allocations. It should also be considered that in absence of a data payload on PUSCH, the resulting UCI decoding performance is harder to control by the gNB.

Proposal 13: For FG 60-7/60-7A, add the additional restriction that UL resource muting is not applied to a PUSCH which only carries a UCI payload but no data payload.

    - 2.3.2. L1 UE-UE CLI measurement and report

The following agreements on UE features for UE L1-based CLI measurement and reporting were made in RAN1#120bis [2][3].

<60-8>, <60-9a> Agreement (RAN1#120bis): Introduce following FG for L1 SRS-RSRP measurement/reporting. 60-9; L1 SRS-RSRP measurement and aperiodic reporting; 1. Aperiodic L1 SRS-RSRP reporting on PUSCH, and [periodic and] aperiodic SRS-RSRP measurement resource FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG; FFS; YES; n/a; [Aperiodic] L1 SRS-RSRP measurement and reporting is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. Agreement (RAN1#120bis): Introduce a new FG for FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource as follows 60-9a; FDM-ed reception of DL signals/channels and L1 SRS-RSRP measurement resource; 1. Support of FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource; 60-9; YES; n/a; FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. Agreement (RAN1#120bis): Introduce following FG for L1 CLI-RSSI measurement/reporting. 60-8; L1 CLI-RSSI measurement and [aperiodic] reporting; 1. Aperiodic L1 CLI-RSSI reporting on PUSCH [and periodic L1 CLI-RSSI reporting on PUCCH] 2. Support of CLI-RSSI measurement resource configured in one UL subband only, in one DL subband only, or across two DL subbands only. 3. [Periodic and] aperiodic CLI-RSSI measurement resource FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG; FFS; YES; n/a; [Aperiodic] L1 CLI-RSSI measurement and reporting is not supported; [Per Band]; TDD only; [n/a]; [n/a]; Optional with capability signalling. Agreement (RAN1#120bis): Define the following FG 60-9b; Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting; 1. Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting; 60-9; YES; n/a; Semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling.

Regarding the FFS on separate FG/components with respect to aperiodic and periodic measurement and configuration modes in FGs 60-8, 60-9, 60-9a -

In our view, it is important to prevent UE feature fragmentation in Rel-19 for the L1-based CLI reporting. It needs to be considered that based on the RAN1#120 and RAN1#120bis agreement, the additional (to aperiodic) support of periodic L1-CLI RSSI reporting comes at little delta for the UE modem implementation. Therefore, aperiodic L1 SRS-RSRP reporting on PUSCH using periodic or aperiodic SRS-RSRP measurement resources, and aperiodic L1 CLI-RSSI reporting on PUSCH and periodic L1 CLI-RSSI reporting on PUCCH should be supported by the UE

Proposal 14: For FG 60-8, support component 1 “Aperiodic L1 CLI-RSSI reporting on PUSCH and periodic L1 CLI-RSSI reporting on PUCCH, and component 3 Periodic and aperiodic CLI-RSSI measurement resource.

Proposal 15: For FG 60-9, support component 1 “Aperiodic L1 SRS-RSRP reporting on PUSCH, and periodic and aperiodic SRS-RSRP measurement resource.

# 3. Conclusions

In this contribution, we make the following proposals.

(UE features for SBFD operation but excluding Random Access or CLI)

Proposal 1. Support the following for basic FG 60-1

| 60-1 | Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures | 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one [and two] DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling 4. Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1) 5. Support of separate CSI reports for SBFD and non-SBFD symbols 6. Support of separate PUCCH frequency resource configuration in SBFD symbols and non-SBFD symbols 7. Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols] 8. Support of separate PUSCH frequency hopping offset in SBFD symbols and non-SBFD symbols 9. Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols 10. Link direction determination in SBFD symbols FFS: Whether to include two DL subband support in component 2 FFS: other component (which may be separate FG) |  | YES | n/a | Basic feature for SBFD-aware UE TX/RX/ measurement behaviour and procedures is not supported | Per band | TDD only | n/a | n/a | [Note: Only UL transmissions are allowed within UL subband and only DL receptions (except for CLI measurement) are allowed within DL subband(s)] [Note: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one.] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 2. Support the following FG 60-2 for SBFD to support configuration 2.

| 60-2 | [Transmission and] reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) | 1. Support of PDSCH reception across SBFD symbols and non-SBFD symbols (Configuration 2) [2. Support of UL transmission across SBFD symbols and non-SBFD symbols (Configuration 2)] FFS: Other component(s) FFS: whether component 2 is defined as separate FG |  | YES | n/a | [Transmission and] Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) is not supported | [Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 3. Support the following FG for SBFD to support DL reception across two DL subbands in SBFD symbol

| 60-A | [Support of DL reception in two non-contiguous DL subbands] | 1. Support of DL reception in two non-contiguous DL subbands 2. Support of CSI-RS reception in two non-contiguous DL subbands 3. CSI processing timeline in SBFD symbols for the support of one contiguous CSI-RS resource allocation across two DL subbands FFS: whether each of components within brackets is separate FG or within a combined FG FFS: other component (which may be separate FG) | 60-1 | Yes |  | DL reception in two non-contiguous DL subbands is not supported | Per band |  |  |  | [For component 3, candidate values are {Legacy, Scaled}] [‘Legacy’ means the CSI processing timeline and CPU counting are the same as legacy. ‘Scaled’ means the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy.] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4. Support the following FG for SBFD to take into account counting of non-contiguous CSI-RS across two DL subbands in SBFD symbol

| 60-B | Counting of non-continuous CSI-RS | 1. Counting of non-contiguous CSI-RS towards the maximum number of active CSI-RS and CSI-RS port per CC and across all CCs (maxNumberSimultaneousNZP-CSI-RS-PerCC, totalNumberPortsSimultaneousNZP-CSI-RS-PerCC, maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC) is scaled by R | X-3 | Yes |  | Non-continuous CSI-RS across two DL subbands cannot be configured | Per band |  |  |  | Candidate value for R in component 1 is {1, 2} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 5. Support the following FG for SBFD to take into account rate-matching of PDSCH reception within DL subband(s) in SBFD symbol

| 60-C | Rate matching of PDSCH reception within DL subband(s) | 1. Counting of PDSCH rate matching within DL subband(s) toward the maximum number of cell-specific RateMatchingPattern(s) | X-1 | Yes |  | PDSCH cannot be rate-matched within DL subband(s) when 4 cell-specific RateMatchingPattern(s) are configured | Per band |  |  |  | The maximum number of RateMatchPattern(s) that can be configured per BWP or per cell is 3, when rate matching of PDSCH occurs because of cross the boundaries of DL usable PRBs. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Random Access related)

Proposal 6. Support the (new) Alt. 4 “UE supporting SBFD random access supports Option 1 and Option 2”.

Proposal 7. For proposal 3-3 (PRACH repetition) support Alt.2: separate FGs for RACH configuration Option 1 and Option 2 are necessary.

Proposal 8. Support/introduce the FG as by proposal 3-4 (PUSCH msg.3 repetition).

(UL resource muting for PUSCH in SBFD symbols)

Proposal 9: For FGs 60-7/60-7a and FFS: Whether this FG only support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH

Support one common pattern of UL muting for DG and one common pattern for Type 2 CG PUSCH and Type 1 CG PUSCH.

Proposal 10: For FG 60-7/60-7A, add component “Support 1 or 2 UL muting symbols on PUSCH > 7 symbols for PUSCH mapping type A”.

Proposal 11: For FG 60-7/60-7A, add component “For PUSCH mapping type A, support 1 UL muting symbol before the first FL DMRS and 1 UL muting symbol after the first FL DMRS”

Proposal 12: For FG 60-7/60-7A, add component “For PUSCH mapping type B, support 1 UL muting symbol”.

Proposal 13: For FG 60-7/60-7A, add the additional restriction that UL resource muting is not applied to a PUSCH which only carries a UCI payload but no data payload.

(L1 UE-UE CLI measurement and report)

Proposal 14: For FG 60-8, support component 1 “Aperiodic L1 CLI-RSSI reporting on PUSCH and periodic L1 CLI-RSSI reporting on PUCCH, and component 3 Periodic and aperiodic CLI-RSSI measurement resource.

Proposal 15: For FG 60-9, support component 1 “Aperiodic L1 SRS-RSRP reporting on PUSCH, and periodic and aperiodic SRS-RSRP measurement resource.

# Reference

- RP-241614 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”; RAN#104

- R1-2502965 “Session Notes of AI 9.15.3”; Ad-Hoc Chair (DoCoMo)

- R1-2502977 “Updated RAN1 UE features list for Rel-19 NR after RAN1 #120bis”; Ad-Hoc Chair (DoCoMo)

- R1-2503543 “Discussion on RAN4 LS on the transient period for SBFD operation”; Samsung

- R1-2504003 “Discussion on RAN2 LS on simultaneous configuration of SBFD and DC”; Samsung

- R1- 2503563 “SBFD operation and procedures”; Samsung

- R1-2503564 “Random access on SBFD resources”; Samsung

- R1-2503565 “Cross-link interference handling for SBFD”; Samsung