3GPP TSG RAN WG1 #121	R1-2503733

St Julian’s, Malta, May 19th – May 23th, 2025

Agenda item:		9.3.3

Source:		Ofinno

Title:			Discussion on CLI handling

Document for:	Discussion and Decision

Introduction

The WID on Evolution of NR duplex operation: Subband full duplex (SBFD) was revised and approved in RAN#104 [1]. The objectives related to the TX/RX/measurement procedures in SBFD are reproduced from the WID [1]:

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3]: - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for co-channel CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4] - Note: RAN3 will not specify enhancements to network signalling to support inter-operator coordination for CLI handling - Notes: Following are assumed for information exchange among gNBs - No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3 - The information exchanges are subject to RAN3’s assessment of feasibility and specification impact - Notes: Following are assumed for UL resource muting - No new DCI field / MAC CE is introduced for indication/determination of UL resource muting - No impact on data and control multiplexing as defined in section 6.2.7, TS 38.212 - UL resource muting does not apply for Msg A PUSCH and Msg 3 PUSCH - UL resource muting applies only for UEs in RRC_CONNECTED mode - UE assumes that the UL resource muting pattern does not overlap UL DMRS or PT-RS in the same symbol - Power boosting is assumed for REs in the symbol with UL resource muting. PUSCH transmit power does not change across symbols - No changes on TBS determination for PUSCH - Check impact on transmit signal quality/MPR requirement, if any, with RAN4 - This feature is subject to UE capability - Notes: Following are assumed for L1 based UE-to-UE CLI measurement and reporting - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs - Periodic and semi-persistent reporting can be considered in addition to aperiodic reporting - For the new reporting quantities, at least wideband reporting is supported The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point

In the last meeting (RAN1#120bis) the following were the agreements and conclusions related to CLI handling [2-3]:

Agreement For the PUSCH symbol containing PT-RS with UL resource muting when transform precoding is not enabled, 3 dB power boosting for both PUSCH and PTRS REs if present, regardless of the PTRS density K and number of PUSCH layers L. Agreement Update the agreement in RAN1#120 as follows (highlighted in red): Agreement For discussion purpose, SRS-RSRP measurement resource index is referred to as SRS-RSRP-MRI and CLI-RSSI measurement resource index is referred to as SRS-RSRP-MRICLI-RSSI-MRI. The mapping order of CSI fields of one report for SRS-RSRP-MRI/SRS-RSRP or CLI-RSSI-MRI/CLI-RSSI reporting is provided in Table 1 with following modifications based on Table 6.3.1.1.2-8 defined in TS38.212 - CRI and SSBRI is replaced with SRS-RSRP-MRI or CLI-RSSI-MRI. - Capability index is crossed out in the table. - RSRP is replaced with L1-SRS-RSRP - L1-CLI-RSSI is added in the table if the CSI report is associated with CLI-RSSI measurement resource. Table 1: Mapping order of CSI fields of one report for SRS-RSRP-MRI/SRS-RSRP or CLI-RSSI-MRI/CLI-RSSI CSI report number; CSI fields. CSI report #n; SRS-RSRP-MRI or CLI-RSSI-MRI #1 as in Table 2, if reported. SRS-RSRP-MRI or CLI-RSSI-MRI #2 as in Table 2, if reported. …. SRS-RSRP-MRI or CLI-RSSI-MRI #4 as in Table 2, if reported. L1-SRS-RSRP or L1-CLI-RSSI #1 as in Table 2, if reported. Differential L1-SRS-RSRP or L1-CLI-RSSI #2 as in Table 2, if reported. …. Differential L1-SRS-RSRP or L1-CLI-RSSI #4 as in Table 2, if reported. Table 2. Bitwidth for SRS-RSRP-MRI, SRS-RSRP, CLI-RSSI-MRI and CLI-RSSI Field; Bitwidth. SRS-RSRP-MRI; ⌈log2KsSRS-RSRP⌉. CLI-RSSI-MRI; ⌈log2KsCLI-RSSI⌉. L1-SRS-RSRP; 7. L1-CLI-RSSI; 7. Differential RSRP; 4. Differential RSSI; 4. KsSRS-RSRP: Number of SRS-RSRP measurement resources in the corresponding resource set KsCLI-RSSI: Number of CLI-RSSI measurement resources in the corresponding resource set Agreement For periodic L1 CLI-RSSI reporting on PUCCH based on a set of periodic CLI measurement resources, the TCI state(s) with QCL typeD for the CLI measurement resources in the periodic CLI measurement resource set are configured per CLI measurement resource (either for all resources in the set or none) by higher-layer parameter - For each periodic CLI measurement resource, this higher-layer parameter provides a reference to one TCI-State in TCI-States for providing the QCL source and QCL typeD. - If the TCI state is not configured, the UE QCL assumptions are the default rules agreed in RAN1#118bis. Agreement The existing CSI timeline for L1 RSRP reporting is reused for the aperiodic CLI measurement and reporting in Rel-19 SBFD. - FFS: whether the slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured shall not be smaller than 1. Agreement For UL muting applied for UL TBoMS, the index of the starting coded bit in a slot, except the first slot, within the 𝑁𝑠 slots allocated for the transmission of TB processing over multiple slots is determined according to Section 6.2.5, TS38.212 as follows k0=k0'+H+τmodNcb and 𝐻 is the total number of coded bits available for transmission of the transport block in the previous slot within the 𝑁𝑠 slots assuming no UCI multiplexing and considering the muted REs. FFS: Whether additional specification impact is needed to support above. Agreement When transform precoding is enabled and an UL muting symbol overlaps a symbol containing PT-RS, down-select from the following alternatives in RAN1#121 - Alt-3 - For PT-RS insertion in a block of MscPUSCH2 samples prior to transform precoding - NgroupPT-RS and Nsampgroup are determined as legacy from 38.214 Table 6.2.3.2-1 - PT-RS sample positions are determined as legacy, but only for the first NgroupPT-RS2 groups - Note: After PT-RS insertion, all of the mathematically equivalent DFT Options 1, 2A, and 2B from the RAN1#120 agreement are applicable, where the choice is up to UE implementation - Alt-4 - UL resource muting is not applied in the PUSCH symbol containing PT-RS Agreement Configuration of Method 1 or Method 3 are not explicitly provided for L1 CLI-RSSI measurement and reporting. Configuration of Measurement resource type #1 or Measurement resource type #2 are not explicitly provided for L1 CLI-RSSI measurement. - CLI-RSSI measurement resource is configured in one UL subband only, in one DL subband only, or across two DL subbands only. - If CLI-RSSI measurement resource is configured across two DL subbands, the frequency resources of CLI-RSSI measurement resource are derived by excluding the frequency resources outside DL usable PRBs. Conclusion There is no RAN1 consensus to support two new report quantities {‘cli-RSSI-measurement-resource-bitmap’, ‘cli-SRS-RSRP-measurement-resource-bitmap’} to reportQuantity.

In the following sections we provide our views on above the remaining issues identified in the last meeting [2], and other relevant issues related to the physical layer procedures.

UE-to-UE CLI handling

From the beginning of the work item, reusing CSI framework was primarily considered for the L1 CLI measurement and reporting. Following that, CLI measurement resources are configured under the CSI report configuration and UE behavior is very similar to the existing CSI beam reporting. For aperiodic CLI measurement triggered by DCI, the CLI resources (such as L1-SRS-RSRP or L1-CLI-RSSI) are expected to follow a similar timing relationship with the triggering PDCCH as in CSI beam measurement cases.

To ensure that the UE has sufficient time to switch its reception beam, the beamSwitchTiming parameter, which was already introduced for CSI beam measurement, can be reused for AP CLI measurement. This is particularly relevant to ensure consistent UE implementation and avoid tight scheduling constraints between the triggering PDCCH and the CLI measurement resources. Therefore, it is quite natural that beamSwitchTiming is used for the gap between scheduling PDCCH and the gap between the CLI measurement resources.

Proposal 1. For an aperiodic CLI report based on aperiodic CLI measurement resources, a UE does not expect to be configured with a scheduling offset between the last symbol of the PDCCH carrying the triggering DCI for AP CLI and the first symbol of the aperiodic CLI resources is smaller than the UE reported threshold beamSwitchTiming.

For the types of L1-CLI-RSSI measurement resources, the following was agreed.

Agreement For frequency resource allocation of a CLI-RSSI measurement resource, the following are supported: - Measurement resource type#1: The CLI-RSSI measurement resource is configured within a DL subband or a UL subband. - Measurement resource type#2: The CLI-RSSI measurement resource is configured across two DL subbands. - FFS: Number of resources that can be configured. FFS: UE behavior for measurement.

Those measurement resources are assumed to be in the DL subband only or UL subband only. We have not introduced L1-CLI-RSSI measurement resource which is not configured within DL subband or UL subband only. Therefore, at least for L1-CLI-RSSI measurement, it should be within SBFD symbols unless a new type of the L1-CLI-RSSI resource is introduced.

On the other hand, no such restrictions on measurement resource configuration are agreed for the L1-SRS-RSRP measurement resource. However, it is clearly described in the WID that no specification impact for SRS configuration information exchange between gNBs, which implies that intra-cell UE-to-UE CLI measurement and report is targeted. Accounting for that, L1-SRS-RSRP measurement within UL subband only is not restrictive at all from a network perspective since there is no coordination between gNBs are assumed for.

Nevertheless, if L1 CLI measurement were to be extended to both SBFD and non-SBFD symbols, the relevant use case would be when the aggressor UE is SBFD-unaware UE and the victim UE is SBFD-aware UE. However, even in such cases, since SRS transmission can be configured to a subset of the UL BWP, the gNB can sufficiently control the transmission through scheduling. Given that this is the final meeting, it is not desirable to broaden the scope of discussion for such a minor case.

Proposal 2. L1 UE-to-UE CLI measurement is configured within SBFD symbols.

The following is agreed for SBFD UE feature to be aligned with the current specifications. This feature aligns with the principles of L3-based CLI handling, making it a natural extension. However, one remaining issue still needs to be addressed.

| 60-9a | FDM-ed reception of DL signals/channels and L1 SRS-RSRP measurement resource | 1. Support of FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource | 60-9 | YES | n/a | FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource is not supported | [Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

L1-based CLI reporting is based on the CSI framework and can be configured based on aperiodic measurement resources. That is, measurement and reporting can be triggered instantaneous manner but still it is delivered via UCI therefore UE behavior related to it should be crystal clear to ensure consistency and reliability of CLI reports. In this regard, it is important to clarify the scope of downlink signals covered under the corresponding FG. While most DL signals and channels are scheduled by the gNB, one notable exception is the Positioning Reference Signal (PRS), report of which is scheduled by the Location Management Function (LMF). Because PRS scheduling is not fully under the control of the gNB, conflicts may arise when PRS overlaps with CLI measurement resources.

To manage such conflicts, PRS is typically configured with a measurement gap or a PRS Processing Window (PPW), which allows for managing measurement priorities. A practical way to address this issue is to explicitly define that the DL signals/channels referenced by the FG include PRS. Based on this, a UE supporting the FG should behave as follows: when PRS and the CLI measurement resource are frequency-division multiplexed (FDM), the UE measures both. And when they are not FDMed but overlapped in time and frequency, the UE treats the CLI measurement resource as a DL signal in the context of PRS prioritization and applies the existing prioritization rules accordingly. This ensures consistent UE behavior and avoids ambiguity in CLI measurements.

Proposal 3. For the newly introduced UE feature “FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource”, clarify that PRS is included for the “DL signals/channels”.

Conclusion

This contribution has discussed the remaining issues related to the physical layer aspects of CLI handling. The following are our proposals:

Proposal 1. For an aperiodic CLI report based on aperiodic CLI measurement resources, a UE does not expect to be configured with a scheduling offset between the last symbol of the PDCCH carrying the triggering DCI for AP CLI and the first symbol of the aperiodic CLI resources is smaller than the UE reported threshold beamSwitchTiming.

Proposal 2. L1 UE-to-UE CLI measurement is configured within SBFD symbols.

Proposal 3. For the newly introduced UE feature “FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource”, clarify that PRS is included for the “DL signals/channels”.

References

- RP-250328, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei.

- R1-2503036, “Summary #2 of CLI handling”, Moderator (Huawei).

- “Draft Report of 3GPP TSG RAN WG1 #120bis v0.1.0”, MCC Support.