3GPP TSG RAN WG1 Meeting #121	R1-2503866

St Julian’s, Malta, May 19th – 23rd, 2025

Source:		Moderator (Fujitsu)

Title:	Final FL summary of Measurements related enhancements for LTM

Agenda Item:	9.9.1

Document for:	Information

Introduction

This contribution is a Feature Lead (FL) summary for A.I. 9.9.1: Measurements related enhancements for LTM

Plan for Online discussion

[Proposals for Monday Online]

[FL proposal 5-3-v1] Support of intra- and inter-frequency for CSI acquisition (Essential issue)

[FL proposal 5-5-v2]Interference measurement resources for CSI acquisition (RRC impact)

[FL proposal 2-3-v2] SPcell inclusion for CSI-RS based gNB scheduled reporting (Essential issue)

[FL proposal 3-5-v2] Remaining issue on Filtering (RRC impact)

[Proposals for Tuesday Unofficial Offline] 11:00-11:30 outside of Younsun room

[FL proposal 5-7-1-v2] Subset of CSI-RS resources – after CSC (Essential issue)

[FL proposal 5-6-v2] Subset of CSI-RS resources – before CSC (leftover from RAN1#120bis, UE behaviour needs to be clarified for CPU, active CSI-RS discussion)

[Proposals for Tuesday Official Offline] 17:00-17:35

[FL proposal 5-7-1-v3]Subset of CSI-RS resources – after CSC (Essential issue) – just count the number of supporting companies

[FL proposal 5-6-v3]Subset of CSI-RS resources – before CSC (leftover from RAN1#120bis, UE behaviour needs to be clarified for CPU, active CSI-RS discussion)

[FL proposal 5-2-v2] Remaining issues on UL channel to convey UCI report (Essential issue) (10min)

[FL proposal 5-1-1-v2] Definition of Valid CSI (Essential issue) (10min)

[FL proposal 3-6-v2]CSI processing for event triggered reporting (no RAN2 impact, leftover from RAN1#120bis) (5min)

RRC by rapporteur (5~10 min)

[Proposals for Wednesday Unofficial Online] 9:00-9:30 outside of Younsun room

[FL proposal 5-1-1-v2] Definition of Valid CSI (Essential issue)

[Proposals for Wednesday Online]

Draft LS in R1-2504827, formal LS in R1-2504828

[FL proposal 5-7-1-v3]Subset of CSI-RS resources – after CSC (Essential issue)

[FL proposal 5-2-v2] Remaining issues on UL channel to convey UCI report (Essential issue)

[FL proposal 5-6-v1] Subset of CSI-RS resources – before CSC (leftover from RAN1#120bis, UE behaviour needs to be clarified for CPU, active CSI-RS discussion)

[FL proposal 3-6-v2] CSI processing for event triggered reporting (no RAN2 impact, leftover from RAN1#120bis)

[FL proposal 5-1-1-v2] Definition of Valid CSI (Essential issue)

[FL proposal 5-1-2-v1] UE behaviour after invalid CSI report (depending on the offline discussion)

[Proposals for Wednesday Unofficial offline] 16:30 – 17:30 outside Younsun room

[FL proposal 5-1-1-v3]Definition of Valid CSI (Essential issue)

[FL proposal 5-6-v4] Subset of CSI-RS resources – before CSC (leftover from RAN1#120bis, UE behaviour needs to be clarified for CPU, active CSI-RS discussion)

[FL proposal 5-1-2-v2]UE behaviour after invalid CSI report (depending on the offline discussion)

[FL proposal 5-2-v2] Remaining issues on UL channel to convey UCI report (Essential issue)

[Proposals for Thursday 1st Online]

Correction of the agreement (copy & paste error) – yellow part needs to be added

Agreement

When SpCellInclusion is configured for gNB configured reporting for L1-RSRP based on CSI-RS, the following rule is used to determine the CSI-RS for current SpCell:

- NZP-CSI-RS resources in [ltm-CSI-NZP-CSI-RS-ResourceList] associated with the current SpCell are the entries where PCI (given by ltm-CandidatePCI) and frequency information (given by ssb-Frequency for the SSBs QCLed with NZP-CSI-RSs) of the candidate cell associated with the LTM-CandidateId (given by the corresponding entry in ltm-CandidateIdList) is equal to the PCI and center frequency of cell-defining SSB of the current SpCell.

[FL proposal 5-2-v2] Remaining issues on UL channel to convey UCI report (Essential issue)

[FL proposal 5-6-v4] Subset of CSI-RS resources – before CSC (leftover from RAN1#120bis, UE behaviour needs to be clarified for CPU, active CSI-RS discussion)

[FL proposal 5-1-1-v3] Definition of Valid CSI (Essential issue)

[FL proposal 3-6-v2] CSI processing for event triggered reporting (no RAN2 impact, leftover from RAN1#120bis)

[Proposals for Thursday 2nd Online]

[FL proposal 5-10-v2] CSI-RS processing capability after CSC (no RAN2 impact)

[FL proposal 5-12-v2] Active CSI-RS ports/resources after CSC (no RAN2 impact)

[FL proposal 5-9-v2] CSI-RS processing capability before CSC (no RAN2 impact)

[FL proposal 5-11-v2] Active CSI-RS ports/resources before CSC (no RAN2 impact)

[FL proposal 5-1-2-v2]UE behaviour after invalid CSI report

[FL proposal 2-2-v2] Active CSI-RS ports/resources for gNB scheduled reporting

Contact people

The following table is reused from the previous meeting. Please update it if necessary.

| Name | Company | Email address |
| --- | --- | --- |
| Yosuke Akimoto | Fujitsu (FL) | akimoto.yosuke@fujitsu.com |
| Taewoo Lee | Fujitsu | lee.taewoo@fujitsu.com |
| Paul Marinier | InterDigital | paul.marinier@interdigital.com |
| Caroline Liang, Zhen He | NEC | Caroline.Liang@emea.nec.com, he_zhen@nec.cn |
| Yu Yang | Spreadtrum | yu.yang2@unisoc.com |
| Didi Zhang | TCL | didi.zhang@tcl.com |
| Minwoo Song | LG Electronics | minwoo1.song@lge.com |
| Hyunsoo Ko | LG Electronics | hyunsoo.ko@lge.com |
| Sanjay Goyal | Nokia | sanjay.goyal@nokia.com |
| Ling YANG | ZTE Corporation, Sanechips | yang.ling17@zte.com.cn |
| Jiayin Zhang | Huawei, HiSilicon | zhangjiayin@huawei.com |
| Yan LI | CMCC | liyanwx@chinamobile.com |
| Ren Da | CATT | renda@catt.cn |
| Naoki Kusashima | Sony | naoki.kusashima@sony.com |
| Dalin Zhu | Samsung | dalin.zhu@samsung.com |
| Alex Liou | Google | alexliou@google.com |
| Bingchao Liu | Lenovo | liubc2@lenovo.com |
| Yongsun Kim | ETRI | doori@etri.re.kr |
| Kapseok Chang | ETRI | kschang@etri.re.kr |
| Nanxi Li | China Telecom | linanxi@chinatelecom.cn |
| Jerome Vogedes | AT&T | Jerome.vogedes@att.com (CN: jov_travel1024@outlook.com) |
| Hanjun Kim | KT Corp. | hj0704.kim@kt.com |
| Claes Tidestav | Ericsson | claes.tidestav@ericsson.com |
| Gustav Lindmark | Ericsson | gustav.lindmark@ericsson.com |
| Mingju LI | Xiaomi | limingju@xiaomi.com |
| Wooseok Nam | Qualcomm | wnam@qti.qualcomm.com |
| Mamoru Okumura | NTT DOCOMO | mamoru.okumura.nz@nttdocomo.com |
| Jungyeon Hong | KDDI Corp. | xju-hong@kddi.com |

List of Contributions

Contributions under 9.9.1

| R1-2503259 | Measurements related enhancements for LTM | Huawei, HiSilicon |
| --- | --- | --- |
| R1-2503376 | Remaining issues on measurements related enhancements for LTM | vivo |
| R1-2503419 | Discussion on measurements related enhancements for LTM | NEC |
| R1-2503422 | Measurement related enhancements for LTM | Nokia |
| R1-2503428 | Discussion on measurements related enhancements for LTM | LG Electronics |
| R1-2503447 | Measurements related enhancements for LTM | TCL |
| R1-2503526 | Discussion on measurements related enhancements for LTM | Spreadtrum, UNISOC |
| R1-2503580 | Views on Rel-19 measurement related enhancements for LTM | Samsung |
| R1-2503672 | Discussion on measurements related enhancements for LTM | ZTE Corporation, Sanechips |
| R1-2503713 | Discussion on measurement related enhancements for LTM | Lekha Wireless Solutions |
| R1-2503809 | Remaining issues on measurements related enhancements for LTM | CATT |
| R1-2503843 | Discussion on measurements related enhancements for LTM | CMCC |
| R1-2503894 | Discussion on measurements related enhancements for LTM | Xiaomi |
| R1-2503916 | Measurements related enhancements for LTM | Lenovo |
| R1-2503949 | Measurements related enhancements for LTM | InterDigital, Inc. |
| R1-2504004 | Discussion on measurement related enhancements for LTM | Fujitsu Limited |
| R1-2504148 | Discussion on measurements related enhancements for LTM | ETRI |
| R1-2504184 | Measurement related enhancements for LTM | Ericsson |
| R1-2504233 | Discussions on measurement enhancement for LTM | OPPO |
| R1-2504300 | Discussion on measurements related enhancements for LTM | Google |
| R1-2504340 | Measurement related enhancements for LTM | Apple |
| R1-2504408 | Measurements related enhancement for LTM | Qualcomm Incorporated |
| R1-2504450 | Discussion on measurements related enhancements for LTM | KDDI Corporation |
| R1-2504513 | Discussion on measurement related enhancements for LTM | NTT DOCOMO, INC. |
| R1-2504546 | Discussion on measurements related enhancements for LTM | Sharp |
| R1-2504595 | LTM measurements related enhancements | MediaTek Inc. |

| R1-2503862 | FL plan for mobility enhancements in RAN1#121 | Moderator (Fujitsu) |
| --- | --- | --- |
| R1-2503863 | FL summary 1 of Measurements related enhancements for LTM | Moderator (Fujitsu) |
| R1-2503864 | FL summary 2 of Measurements related enhancements for LTM | Moderator (Fujitsu) |
| R1-2503865 | FL summary 3 of Measurements related enhancements for LTM | Moderator (Fujitsu) |
| R1-2503866 | Final FL summary of Measurements related enhancements for LTM | Moderator (Fujitsu) |

Contributions under 9.15.6 – UE feature (For information)

| R1-2503260 | UE features for NR mobility enhancements phase 4 | Huawei, HiSilicon |
| --- | --- | --- |
| R1-2503390 | Discussion on UE features for NR mobility enhancements Phase 4 | vivo |
| R1-2503404 | NR mobility enhancements Phase 4 UE features | Nokia |
| R1-2503596 | UE features for NR mobility enhancements Phase 4 | Samsung |
| R1-2503674 | Discussion on UE features for NR mobility enhancements Phase 4 | ZTE Corporation, Sanechips |
| R1-2503810 | Discussion on UE features for NR mobility enhancements Phase 4 | CATT |
| R1-2503855 | Discussion on UE features for NR mobility enhancements Phase 4 | CMCC |
| R1-2503905 | Discussion on UE features for NR mobility enhancements Phase 4 | Xiaomi |
| R1-2504185 | UE features for NR mobility enhancements phase 4 | Ericsson |
| R1-2504234 | Discussion on UE features for NR mobility enhancements | OPPO |
| R1-2504372 | Summary of UE features for NR mobility enhancements Phase 4 | Moderator (AT&T) |
| R1-2504424 | UE features for NR mobility enhancement Phase 4 | Qualcomm Incorporated |
| R1-2504528 | Discussion on UE features for NR mobility enhancemens Phase4 | NTT DOCOMO, INC. |

Discussion

L1 measurement based on CSI-RS

[No issue] Measurement quantity

[Agreements in previous meetings]

Agreement (RAN1#118)

Support L1-RSRP measurement based on CSI-RS

  - FFS: Support L1-SINR measurement based on CSI-RS

Conclusion (RAN1#118bis)

There is no consensus in RAN1 on the support L1-SINR measurement based on CSI-RS for candidate cells

[Conclusion]

No new issue is identified in this meeting.

[No issue] Support of intra- and inter frequency measurement

[Agreements in previous meetings]

Agreement (RAN1#118bis)

From RAN1 perspective, there is no restriction with regards to the frequency location of CSI-RS used for L1-measurement

[Conclusion]

No new issue is identified in this meeting.

[No issue] Time domain property of CSI-RS for measurement

[Agreements in previous meetings]

Agreement(RAN1#118)

For gNB scheduled reporting and event triggered reporting

At least periodic CSI-RS is supported for L1-RSRP measurement for candidate cell

  - FFS: aperiodic and semi-persistent CSI-RS

At least CSI-RS for beam management is supported for L1-RSRP measurement for candidate cell

  - FFS: CSI-RS for mobility

Working Assumption(RAN1#118bis)

In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective

Send an LS to RAN3 (CC RAN2) to ask for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload).

Support of semi-persistent CSI-RS is subject to UE capability.

Agreement(RAN1#120)

Confirm the following working assumption made in RAN1#118bis

  - Working Assumption(RAN1#118bis)

    - In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective

    - Send an LS to RAN3 (CC RAN2) to ask for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload).

    - Support of semi-persistent CSI-RS is subject to UE capability.

MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2

Send an LS to RAN2 to inform this agreement. Final LS in R1-250XXXX.

Conclusion (RAN1#120)

There is no RAN1 consensus to support SP CSI-RS for event triggered reporting

Conclusion (RAN1#120)

There is no consensus on the support of aperiodic CSI-RS resource for gNB scheduled reporting or event triggered reporting

[Conclusion]

No new issue is identified in this meeting.

[No issue] Type of CSI-RS for L1 measurement

[Agreements in previous meetings]

Agreement

For gNB scheduled reporting and event triggered reporting

At least periodic CSI-RS is supported for L1-RSRP measurement for candidate cell

  - FFS: aperiodic and semi-persistent CSI-RS

At least CSI-RS for beam management is supported for L1-RSRP measurement for candidate cell

  - FFS: CSI-RS for mobility

Conclusion (RAN1#119)

No consensus to support CSI-RS for mobility for L1 measurement in Rel-19 LTM

  - Note: From the actual gNB transmission viewpoint, CSI-RS for mobility and CSI-RS for BM may be the same

[Conclusion]

No new issue is identified in this meeting.

[No issue] Timing reference of CSI-RS

[Agreement in previous meetings]

- Conclusion (RAN1#120bis)

  - RAN1 assumes the SSB QCLed with CSI-RS is used for timing reference of candidate cell CSI-RS measurement in LTM.

[Conclusion]

No new issue is identified in this meeting.

[No issue] High level design of CSI-RS configuration

[Agreements in previous meetings]

Agreement

Explicit configuration of CSI-RS resource(s) for candidate cell(s) for L1-measurement is supported

Agreement(RAN1#120)

Alt.2: The parameters defined in legacy NZP-CSI-RS-resource can be different for CSI-RS resources associated with the resource set

Agreement(RAN1#120)

An LTM report configuration for L1-RSRP is associated with a single resource config that includes:

Alt.A: a single resource set containing CSI-RS resources corresponding to multiple candidate cells. i.e., the same design as that of SSB in Rel-18 LTM.

  - FFS: how to associate between the measurement CSI-RS resources and candidate cells

    - explicit or implicit signaling of candidate cells

Agreement (RAN1#120bis)

In the resource set configured for candidate cell L1-RSRP measurement, a sequence of candidate cell indices is included together with a sequence of CSI-RS resource indices, where each CSI-RS resource index is associated with one candidate cell index.

[Conclusion]

No new issue related to core functionality is identified in this meeting. The details of RRC structures can be discussed under moderator’s session.

gNB scheduled reporting

[No issue] Further details of report framework

[Agreement in previous meetings]

Agreement(RAN1#118)

CSI-RS based L1-RSRP report is supported for gNB scheduled measurement reporting

FFS: CSI-RS based L1-SINR report is supported for gNB scheduled measurement reporting

Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting

Agreement(RAN1#118bis)

The agreement “Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting” made in RAN#118 is further clarified for L1-RSRP as follows:

UCI format defined in Table 6.3.1.1.2-8C of TS38.212 can be used by replacing SSBRI with CRI.

Whether the L1-RSRP(s) of serving cell is always included is configurable (in line with Rel-18)

The quantization method defined in clause 5.2.1.4.3 of TS38.214 and bit width defined in Table 6.3.1.1.2-6 of TS38.212 can be used

No L1 specified filtering for time and spatial domain is introduced

No enhancement on how to report L cells x M beams

Periodic reporting on PUCCH is supported

FFS: semi-persistent reporting on PUCCH/PUSCH, and aperiodic reporting on PUSCH

Agreement(RAN1#118bis)

For CSI-RS based L1-measurement report by gNB scheduled measurement reporting, semi-persistent reporting on PUCCH/PUSCH and aperiodic reporting on PUSCH are supported

[Conclusion]

No new issue is identified in this meeting.

[Closed] Active CSI-RS ports/resources for gNB scheduled reporting

[Agreement in previous meetings]

No discussion so far

[Summary of contributions]

Three companies provided their views in this meeting.

Ericsson

  - It is up to UE to decide which of the configured CSI-RSs for gNB scheduled reporting it should measure on, just as for SSBs in Rel. 18 LTM. The network does not count active CSI-RS resources for LTM.

  - The UE measures L1-RSRP for at least a minimum number of CSI-RS resources configured for LTM beam management. This number is a component of the UE capabilities for gNB scheduled LTM based on CSI-RS.

LG: LTM CSI resources should be counted for the active CSI-RS ports or resources. Down-select from the following options:

  - Option 1: it is counted as conventional active CSI-RS ports or resource in active BWP

  - Option 2: it is newly introduced active CSI-RS ports or resources for LTM

DOCOMO

  - For gNB-scheduled beam reporting, active CSI-RS resource for LTM should be separately counted from serving cell.

[FL observation]

According to the current spec description in section 5.2.1.6 of TS 38.214, FL’s understanding is the following:

Spec description

  - In any slot, the UE is not expected to have more active CSI-RS ports or active CSI-RS resources in active BWPs than reported as capability. NZP CSI-RS resource is active in a duration of time defined as follows. For aperiodic CSI-RS, starting from the end of the PDCCH containing the request and ending at the end of the scheduled PUSCH containing the report associated with this aperiodic CSI-RS. When the PDCCH candidates are associated with a search space set configured with searchSpaceLinkingId, for the purpose of determining the NZP CSI-RS resource active duration, the PDCCH candidate that ends later in time among the two linked PDCCH candidates is used. For semi-persistent CSI-RS, starting from the end of when the activation command is applied, and ending at the end of when the deactivation command is applied. For periodic CSI-RS, starting when the periodic CSI-RS is configured by higher layer signalling, and ending when the periodic CSI-RS configuration is released.

FL understanding

  - The CSI-RS ports and CSI-RS resources are counted if the CSI-RS is in active BWP (maybe this is intra-frequency case)

    - It is treated as error case if more active CSI-RS ports or active CSI-RS resources in active BWPs is configured

  - Otherwise (outside of active BWB, i.e. for inter-frequency, case), the CSI-RS ports and CSI-RS resources are not counted

With this analysis, the following questions can be discussed first to make a progress, it is noted that there is not impact on MAC-CE/RRC is expected.

[FL proposal 2-2-v1]

Discuss the following 3 questions are discussed in RAN1#121, aiming at the consensus in this meeting (if time available)

  - Question 1: For active CSI-RS ports and resources for LTM gNB scheduled L1-RSRP reporting, whether or not the current restriction on BWP (i.e. in active BWPs) should be kept for LTM case?

    - This means that the active CSI-RS ports, and hence resources outside active BWPs are not counted while those in active BWPs are counted

  - Question 2: Differently from legacy active CSI-RS ports/resources, whether or not it is allowed to configure more CSI-RS ports/resources to a UE than its capability?

    - If yes, the UE can measure legacy active CSI-RS ports/resources within its capability, and such a configuration is not handled as an error case.

  - Question 3: Whether or not the capability for LTM active CSI-RS ports/resources is defined separately from legacy (i.e. non-LTM) UE capability

    - Note: this can be discussed under UE capability session

FL proposal 2-2-v2 will be provided based on the answer from companies.

[Comments to 2-2-v1]

| Company | Comment |  |
| --- | --- | --- |
| Fujitsu | Question 1: Our preference is to enhance the current restriction for LTM. For inter-frequency scenario, the active CSI-RS resources/ports to be measured in candidate cells are not counted if we restrict within the active BWP. Accordingly, the gNB cannot manage the number of CSI-RS resources / ports. Question 2: We prefer to allow for gNB to configure more CSI-RS ports / resources than UE capability. Since UE can select the CSI-RS resources to measure in LTM, the gNB needs to provide the flexibility for the selection of CSI-RS resources / ports to UE. Question 3: We think that new feature should not affect to the legacy behavior. In this sense, we prefer to have separate UE capability apart from the legacy UE capability. |  |
| vivo | Question 1: According to RAN4 agreement achieved in the last meeting, RAN4 does not define requirements for L1 measurement with gap for CSI-RS based L1 measurement on neighbor cell. In general, L1 measurement outsides active BWP is only performed within measurement gap. In other word, only L1 measurement based on candidate cell(s) within active BWP of serving cell(s) is supported by RAN4. Hence, it seems no need to discuss this issue in RAN1. Agreement: From RAN4 perspective, - Not to define RAN4 requirements for L1 measurement with gap for CSI-RS based L1 measurement on neighbor cell. - Not to define RAN4 requirements for RTD > CP.. Question 2: According to the agreement achieved before, UE performs beam measurement on periodic CSI-RS resources after RRC configuration or on semi-persistent CSI-RS resources after activation. Therefore, it is unnecessary to require the number of configured CSI-RS resources used for beam measurement on candidate cell(s) larger than UE capability. Question 3: Separate UE capability is clearer. Furthermore, the case that serving cell (e.g., SCell) is the candidate cell should be considered for the determination of the candidate value of the separate capability. |
| Nokia | CPUs were defined for Rel-18 LTM SSB-based measurements. As FL pointed out, for CSI-RS-based measurements, we have additional constraints related to the number of active RSs/ports. Therefore, these constraints should also be applied to LTM CSI-RS-based measurements. Question 1: We agree with vivo that RAN4 currently supports only intra-frequency CSI-RS-based measurements. We can check whether inter-frequency CSI-RS measurements are supported by RAN4, and based on that, we can decide if we should worry about CSI-RSs outside of the active BWP. Question 2: Its Ok to configure more CSI-RSs as in the legacy. The UE can drop some measurements based on the priority. Question 3: We didn’t consider a separate capability for Rel-18 SSB based L1-RSRP measurements, we can follow the same for CSI-RS based L1-RSRP measurements. |  |
| Qualcomm | Question 1: Active CSI-RS counting for the active BWP pertains only to the serving cell, where the CSI resource setting is defined per BWP (by bwp-Id field in CSI-ResourceConfig) – the network ensures the configuration not to violate the UE capability. Conversely, the CSI-RS resources for candidate cell measurement are not part of the serving cell configuration, thus not defined per-BWP. Consequently, the “active BWP” notion does not appear quite relevant to the LTM L1 measurement (regardless of inter- or intra-frequency cases). Question 2: Yes, in general. If a UE capability is defined (whether shared with the serving cell or separate) for candidate cell measurement, it should be more flexible and loose compared to the legacy capability for the serving cell. This consideration is necessary due to the potentially large number of candidate cells and associated CSI-RS resources. For instance, only the maximum number of configured CSI-RS resources (either overall or per candidate cell) could be specified, while the selection of resources for actual measurement may be at the discretion of the UE, as proposed by Ericsson. Question 3: Candidate cell CSI-RS measurement may necessitate distinct processes from those employed by the serving cell, such as time/frequency synchronization. Therefore, a separate UE capability should be defined. |  |
| Ericsson | Question 2: The NW want to use the same resource configuration for all UEs and the configuration is likely so large that the UE cannot measure on all resources simultaneously. This should be allowed and it should be up to UE to decide which of the configured resources/ports that are measured at a specific point in time. If the max-number can anyway be exceeded, it is not clear what the benefit with active resource/port counting is. It is enough that the UE measures a minimum number of resources/ports, by capability which is separate (Question 3) to serving cell capabilities for active resources/ports. Question 1: Our preference is to not differentiate between the inter- resp. intra frequency case for LTM. |  |
| Spreadtrum | Question 1: We think that the current restriction on BWP (i.e. in active BWPs) may not apply to the candidate cell measurement. For LTM case, the active CSI-RS ports/resources for candidate cells can be defined and counted separately. Question 2: The number of the configured active CSI-RS ports/resources may be more, anyway, this is depends on the UE capability. Question 3: Prefer a separate UE capability. |  |
| Lenovo | Q1: We think the current restriction on the active BWP should be kept and then introduce addition restrictions for the case that CSI-RS resource based measurement is configured for LTM. Q2: If separate restriction for LTM is introduced, the UE is not expected to be configured with more active CSI-RS ports or active CSI-RS resources than reported as capability. Q3: Separate restriction on the LTM active CSI-RS ports/resources should be introduced and the UE needs to separately report the capability. |  |
| CATT | For Question 3, we refer to define separate UE capability for LTM. |
| LG | Firstly, the intension of this proposal was, do not make UE to randomly select measurement resources or randomly select whether UE measures those resources. Because there is no way to UE how much the resources are suitable to UE, the randomized procedure doesn’t guarantee the highest value of L1-RSRP result is most favorable to UE. So, whatever, some of the rules from the gNB side should be defined. In that point of view, inter/intra frequency measurement could be the most simplified rule. This can be the answer to Question 2. Question 1: because the active BWP is not exactly same as inter frequency measurement, the wording should be changed from active BWP to inter frequency measurement. Question 3: Considering above points, if the new rule means the number of resources configured for UE can exceed the UE capability, it shouldn’t be adopted. But if the new rule means the number of resources used for intra frequency measurement should follow legacy rule and the number of resources used for inter frequency measurement will be defined as a new rule, it can be supported. |  |
| NTT DOCOMO | Question 1: We share the same view as several companies that the current restriction on BWP may not apply to the candidate cell measurement. Then, the active CSI-RS resources for candidate cells should be defined and counted separately regardless of intar-/inter frequency case. Question 2: Considering the LTM scenario, the number of configured CSI-RS resources should not be restricted by UE capability. Question 3: It should be a separate UE capability. |  |
| ZTE | Question 1: as mentioned in our UE feature contribution in R1-2503674, we can vaguely perceive from the following RAN4’s agreement in RAN4#114bis meeting that RAN4 does not tend to define inter-frequency L1-RSRP measurement for CSI-RS in Rel-19 LTM. This point has also been mentioned in the above comment from vivo. But from our perspective, we think that whether to introduce the definition of inter-frequency L1-RSRP based on CSI-RS should be made decision in RAN4, not RAN1. In such case, we should discuss inter-frequency measurement related issues till RAN4 has a clear conclusion on it. Agreement: - From RAN4 perspective, - Not to define RAN4 requirements for L1 measurement with gap for CSI-RS based L1 measurement on neighbor cell. - Not to define RAN4 requirements for RTD > CP.. Question 2: we don’t identify the motivation to introduce a new rule for active CSI-RS resources/ports for LTM. We tend to follow similar method as legacy, that is, the UE is not expected to be configured with more active CSI-RS resources/ports than a capability UE indicated/supported. Question 3: Yes. We think that the capability for active CSI-RS resources/ports for LTM should be separately defined. But the total number of active CSI-RS resources/ports for serving cell and candidate cell(s) should no larger than UE capability. |

[FL proposal 2-2-v2]

- UE capability for the number of LTM active CSI-RS ports/resources for LTM L1-RSRP measurement is defined separately from that for legacy (i.e. serving cell) operation.

  - FL note: RAN1 will not differentiate intra- and inter-frequency until RAN4 makes such decision

  - FL note: Check if Nokia is OK with this direction (as clear majority companies supports this direction)

- If a UE is configured with more active CSI-RS ports/resources for LTM L1-RSRP measurement than UE capability:

  - Alt 1. It is handled as an error case (i.e. UE is not expected to have more active CSI-RS ports or active CSI-RS resources than reported as capability for candidate cells)

    - Supported by ZTE, vivo, Lenovo, LG

  - Alt 2. UE can be configured with more active CSI-RS ports or active CSI-RS resources than reported as capability for candidate cells (i.e. it is up to UE which CSI resource(s) to measure.)

    - Supported by Fujitsu, Nokia, Qualcomm, Ericsson, DOCOMO, CATT

[Conclusion]

FL proposal 2-2-v2 was not discussed in this meeting due to the lack of time. FL expect that this can be discussed during the maintenance phase, if companies see the necessity. With this, the discussion of this section is closed.

[Closed] SpCell Inclusion for CSI-RS L1-RSRP measurement

[Summary of the contributions]

This discussion is triggered by Nokia’s contribution in RAN1#120bis, and companies views are provided as below. The difference of the potential options is which RRC parameter is used to identify the frequency of CSI-RS.

When SpCellInclusion is configured for gNB configured reporting for L1-RSRP based on CSI-RS, one of the following options can be used to determine the CSI-RSs from the current SpCell:

  - Option 1: Huawei, OPPO, Qualcomm, DOCOMO

    - Justification: the same mechanism for SSB can be reused

    - NZP-CSI-RS resources in [ltm-CSI-NZP-CSI-RS-ResourceList] associated with the current SpCell are the entries where PCI (given by ltm-CandidatePCI) and frequency information (given by ssb-Frequency for the SSBs QCLed with NZP-CSI-RSs) of the candidate cell associated with the LTM-CandidateId (given by the corresponding entry in ltm-CandidateIdList) is equal to the PCI and center frequency of cell-defining SSB of the current SpCell.

  - Option 2: No support

    - Concern by Qualcomm: there is a possibility that different cells may share the same absoluteFrequencyPointA while utilizing different offsetToCarrier values, potentially leading to ambiguity with Option 2.

    - NZP-CSI-RS resources in [ltm-CSI-NZp-CSI-RS-ResourceList] associated with the current SpCell are the entries where PCI (given by ltm-CandidatePCI) and frequency information (absoluteFrequencyPointA) of the candidate cell associated with the LTM-CandidateId (given by the corresponding entry in ltm-CandidateIdList) is equal to the PCI and frequency associated with point A of the current SpCell.

  - Option 3: ZTE

    - FL note: this is a new proposal in this meeting, and refFreqCSI-RS is a parameter used for L3 measurement to refer to ARFCN-ValueNR. If this option is agreed, a new RRC parameter needs to be added.

    - For CSI-RS resources in [ltm-CSI-NZP-CSI-RS-ResourceList] associated with the current SpCell, their PCI (given by ltm-CandidatePCI) and frequency information (given by refFreqCSI-RS) of the candidate cell associated with the LTM-CandidateId (given by the corresponding entry in ltm-CandidateIdList) is determined according to the PCI and center frequency of CSI-RS of the current SpCell.

[FL observation]

Not many companies expressed their opinion on this issue, but Option 1 looks reasonable considering the mechanism for SpCell inclusion for SSB in Rel-18. We can directly go to main session to make the decision between option 1 and 3.

[FL proposal 2-3-v1]

Choose one option from the following:

When SpCellInclusion is configured for gNB configured reporting for L1-RSRP based on CSI-RS, the following rule is used to determine the CSI-RS for current SpCell:

  - Option 1: Huawei, OPPO, Qualcomm, DOCOMO

    - NZP-CSI-RS resources in [ltm-CSI-NZP-CSI-RS-ResourceList] associated with the current SpCell are the entries where PCI (given by ltm-CandidatePCI) and frequency information (given by ssb-Frequency for the SSBs QCLed with NZP-CSI-RSs) of the candidate cell associated with the LTM-CandidateId (given by the corresponding entry in ltm-CandidateIdList) is equal to the PCI and center frequency of cell-defining SSB of the current SpCell.

  - Option 3: ZTE

    - For CSI-RS resources in [ltm-CSI-NZP-CSI-RS-ResourceList] associated with the current SpCell, their PCI (given by ltm-CandidatePCI) and frequency information (given by refFreqCSI-RS) of the candidate cell associated with the LTM-CandidateId (given by the corresponding entry in ltm-CandidateIdList) is determined according to the PCI and center frequency of CSI-RS of the current SpCell.

FL note: Option 1 is aligned with the mechanism for SpCell inclusion for SSB in Rel-18. This has no issue because we have agreed in RAN1#120bis that SSB as QCL source is always configured for timing assumption. For Option 3, refFreqCSI-RS is a parameter used for L3 measurement to refer to ARFCN-ValueNR. If this option is agreed, a new RRC parameter needs to be added.

[To Chair: link to the list of online proposals]

[Comments to 2-3-v1]

| Company | Comment |  |
| --- | --- | --- |
| Fujitsu | Considering the specification impact, we prefer Option1 to reuse existing mechanism for the identification of serving cell CSI-RS. |  |
| vivo | Prefer Option 1. |  |
| Nokia | Support Option 1. |  |
| Qualcomm | We support Option 1. |  |
| Ericsson | Support Opion 1, it is the most straight-forward generalization of the Rel. 18 mechanism. |  |
| Google | Fine with Option 1. |  |
| Samsung | Fine with Option 1 in principle. In addition, the following update in red is needed to make the determination complete – this is because SSBs and NZP CSI-RSs can be explicately associated with candidate cells per RAN1’s agreements for Rel-18 and Rel-19 respectively: given by ssb-Frequency for the SSBs QCLed with NZP-CSI-RSs or associated with a same candidate cell |  |
| Spreadtrum | Prefer Option 1. |  |
| Lenovo | Option 1 based on current spec is enough. |  |
| CATT |  | Support Option 1. |
| LG | Support Option 1. |  |
| NTT DOCOMO | Support Option 1. |  |
| ZTE | From our perspective, either of the two approaches works. If majority companies tend to go for option1, we are also fine. |  |

[FL proposal 2-3-v2]

When SpCellInclusion is configured for gNB configured reporting for L1-RSRP based on CSI-RS, the following rule is used to determine the CSI-RS for current SpCell:

  - NZP-CSI-RS resources in [ltm-CSI-NZP-CSI-RS-ResourceList] associated with the current SpCell are the entries where PCI (given by ltm-CandidatePCI) and frequency information (given by ssb-Frequency for the SSBs QCLed with NZP-CSI-RSs or associated with a same candidate cell) of the candidate cell associated with the LTM-CandidateId (given by the corresponding entry in ltm-CandidateIdList) is equal to the PCI and center frequency of cell-defining SSB of the current SpCell.

FL note: yellow part is the proposal from Samsung. Some discussion is needed for the intention.

[To Chair: link to the list of online proposals]

[Conclusion]

The following agreement was made during the online session on Monday.

Agreement

When SpCellInclusion is configured for gNB configured reporting for L1-RSRP based on CSI-RS, the following rule is used to determine the CSI-RS for current SpCell:

- NZP-CSI-RS resources in [ltm-CSI-NZP-CSI-RS-ResourceList] associated with the current SpCell are the entries where PCI (given by ltm-CandidatePCI) and frequency information (given by ssb-Frequency for the SSBs QCLed with NZP-CSI-RSs) of the candidate cell associated with the LTM-CandidateId (given by the corresponding entry in ltm-CandidateIdList) is equal to the PCI and center frequency of cell-defining SSB of the current SpCell.

With this, the discussion of this section is closed.

Event triggered reporting

[No issue] Report container

[Agreement in previous meetings]

RAN2 agreed to support MAC CE for the container of event triggered reporting. Therefore, RAN1 discussion on this aspect is not necessary anymore.

[Conclusion]

No further discussion is planned unless requested by RAN2

[No issue] Report quantity

[Agreements in previous meetings]

Agreement(RAN1#118)

SSB based L1-RSRP measurements is supported for event triggered reporting

CSI-RS based L1-RSRP measurements is supported for event triggered reporting

FFS: CSI-RS based L1-SINR measurements is supported for event triggered reporting

[Conclusion]

No further discussion is necessary as no consensus was achieved to introduce L1-SINR

[No issue] Report format and contents

[Agreements in previous meetings]

Agreement (RAN1#120bis)

Regarding the LS from RAN2 (R1-2501686) the following contents are included in the reply LS.

  - RAN1 assumes at maximum 16 beams can be reported by a single event triggered reporting

  - Regardless whether or not the report includes the current beam

  - Note: The maximum number of beams that can be reported by a UE subject to UE capability is under discussion in RAN1

[Conclusion]

No new issue is identified in this meeting.

[No issue] RS of serving cell for event evaluation

[Agreement in previous meetings]

Agreement(RAN1#118)

For the identification of the serving cell RS for event evaluation,

  - At least the following options are further studied in RAN1, where different options could apply to different LTM event

    - Option. 1: Derived from QCL (type-D) RS(s) of the indicated joint/DL TCI state for the serving cell

    - Option. 2: Derived from QCL RS(s) or SSB QCLed with the QCL RS of the indicated joint/DL TCI state for the serving cell

      - QCL RS or SSB is configured by the network

    - Option. 3: Measurement RS(s) is/are explicitly configured

    - Option. 4: Derived from QCL RSs of activated TCI states with the best quality, or SSB which is QCLed with the QCL RSs of activated TCI states with the best quality.

    - Option 6: Derived from QCL RSs of activated TCI states, or SSB which is QCLed with the QCL RSs of activated TCI states

The RSs of the candidate cell(s) for event evaluation are explicitly configure

Note: Companies are encouraged to take into account the RAN2 agreement (i.e current beam rather than best beam) for their further study.

Agreement(RAN1#118bis)

The serving cell RS for event evaluation is at least derived from QCL RS or SSB QCLed with the QCL RS of the indicated joint/DL TCI state for the serving cell

  - QCL RS above is the RS w.r.t. QCL-TypeD when the indicated joint/DL TCI state is configured with two QCL RSs

FFS: Details on determination of QCL RS or SSB QCLed with QCL RS

Note: This does not imply the support of mTRP scenarios

Agreement(RAN1#119)

For the identification of the serving cell RS for event evaluation,

If the RS(s) for candidate cell(s) are CSI-RS configured in a CSI-RS resource set configured with repetition, QCL RS of the indicated TCI-state is used for the serving cell; otherwise, SSB QCLed with QCL RS of the indicated TCI-state is used for the serving cell.

  - UE does not expect the following configuration:

  - CSI-RS resource in the indicated TCI state of serving cell is NOT configured in a CSI-RS resource set configured with repetition, and

  - CSI-RS is configured as measurement resource for the candidate cell(s).

Conclusion (RAN1#120)

The following is up to RAN2: Coexistence of LTM event triggered reporting and mTRP operation at serving cells

Agreement(RAN1#120bis)

For the RS type determination for event triggered reporting with event LTM2,

  - Alt 1-2: At least one candidate RS shall be configured

For info: LTM events in RAN2:

Event LTM2: Beam of serving cell becomes worse than absolute threshold;

Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

Event LTM4: Beam of candidate cell becomes better than absolute threshold;

Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

[Conclusion]

The remaining issue is the support of mTRP, which is still under discussion in RAN2. There is nothing to discuss in this meeting.

[Closed] Filtering of measurement results for evaluation and reporting

[Agreement in previous meetings]

Conclusion (RAN1#120)

There is no consensus in RAN1 to support specified filtering of L1-RSRP for LTM.

Send an LS to RAN2 to inform that L1 filtering is up to UE implementation. Final LS in R1-2501577.

[Summary of contributions]

The following proposal was made by Ericsson in RAN1#120bis on top of the conclusion in RAN1#120.

Justification

  - Historically, legacy requirements assumed 3 samples for serving cell L1-RSRP unless timeRestrictionFor-ChannelMeasurements is configured, in which case only one sample is used [1] (Clause 9.5.4.1). However, the LTM L1-RSRP measurement period in Rel-18 is based on single-shot measurements: Primarily due to lack of time, RAN1 missed to define timeRestrictionForChannelMeasurements for LTM CSI framework in Rel-18. Then RAN4 did not define the number of samples needed for the UE implementation filtering as it was linked to configuration of timeRestrictionForChannelMeasurements in the legacy serving cell requirements.

  - For Rel-19 event triggered L1 RSRP measurement reporting it is important that the UE implementation filter can be configured to use either 1 or 3 measurement samples, just like legacy serving cell L1-RSRP measurements.

proposal:

  - Just like L1-RSRP on serving cells, the UE implementation filter for LTM L1-RSRP can use up to M measurement samples, where M can take the values 1 and 3 and is configured by the network.

  - A straight-forward way to achieve this proposal is to configure timeRestrictionForChannelMeasurements for LTM CSI config.

In this meeting, two more companies provided their proposals on this issue.

Fujitsu: Introduce timeRestrictionForChannelMeasurements for L1-RSRP calculation in the event triggered reporting without defining the CSI reference resource.

  - if this parameter is configured, only a single sample can be used

  - Otherwise, multiple samples can be used, and the detail is up to RAN4

DOCOMO: For both event-triggered beam reporting and gNB-scheduled beam reporting, support to introduce configuration of time restriction for channel measurement.

[FL observation]

In the last meeting, this issue is briefly discussed using FL summary and in the offline session, but no conclusion was made. The concerns/questions and FL’s understanding are provided as follows:

What is the L1 behaviour?

  - FL understanding: L1 just provides the L1-RSRP measurement results to upper layer, and the L1 behaviour will not be affected due to the configuration of timeRestrictionForChannelMeasurements.

Is this aligned with our conclusion that filtering is up to UE implementation?

  - FL understanding: It is not easy to answer. Maybe companies have different understanding on this. However according to the proponents’ opinion, this parameter gives more information whether multiple samples can be used to satisfy the requirement.

How and whether the CSI-RS reference resource is defined, which is used for the legacy behaviour related to timeRestrictionForChannelMeasurements?

  - FL understanding: based on Fujitsu’s contribution, CSI-RS reference resource may not be needed because everything is handled in MAC layer.

Based on the above understanding, FL firstly invites companies to provide their views, and the proposal will be updated based on them.

[FL proposal 3-5-v1]

Choose one alternative below:

Alt 1:

  - A RRC parameter timeRestrictionForChannelMeasurements is introduced for CSI-RS based L1-RSRP measurement for both event triggered reporting and gNB scheduled reporting

    - If this parameter is set “configured”, only one sample can be used when RAN4 defines their requirement

    - Otherwise, the number of samples can be up to UE implementation, and the exact number of samples to define requirements is up to RAN4

  - This parameter does not impact the UE behaviour in L1 including the definition of CSI-RS reference resources

Alt 2:

  - RRC configuration for time restriction is NOT introduced for CSI-RS based L1-RSRP measurement for either event triggered reporting or gNB scheduled reporting, i.e. it is up to RAN4 how many samples are used to define their requirement.

[To Chair: link to the list of online proposals]

[Comments to 3-5-v1]

| Company | Comment |  |
| --- | --- | --- |
| Fujitsu | Support Alt.1. The details will be discussed in RAN4, and the purpose of this issue is that RAN1 needs to provide kind of guideline for their discussion. In legacy specification, the parameter needs the CSI reference resource, but it is difficult to define the CSI reference resource for the event evaluation and the L1 measurement after event triggered in the event triggered reporting. Besides, the UE will transfer the measurement results to upper layer as soon as it measures. Considering that the CSI reference resource plays a role to specify and guarantee for UE to measure the CSI-RS in certain time, we do not have to define the CSI reference resource in the event triggered reporting. |  |
| vivo | Support Alt. 2. According to the current specification, if timeRestrictionForChannelMeasurements is configured, the L1-RSRP measurement result is only derived based on the latest RS occasion. However, L1 measurement on candidate cell cannot be performed on the latest RS occasion if it is overlapped with measurement gap. Therefore, it is infeasible to configure this parameter for LTM. |  |
| Nokia | We prefer Alt1. However, it would be better if this can be applied to SSB based measurements as well. |  |
| Ericsson | Prefer Alt1. The Alt2 behavior is the same as Alt1 with timeRestrictionForChannel-Measurements configured. Alt1 further enables the UE to use 3 samples to improve the L1-RSRP accuracy (although it is up to the UE to do that or just use one sample). |  |
| Samsung | We prefer Alt1. |  |
| Lenovo | Support Alt.2. This parameter is not applied to SSB based L1-RSRP report in Rel-18 LTM. We prefer not to introduce for CSI-RS based beam measurement and report in Rel-19. |  |
| CATT | Support Alt 1. Alt 2 seems to be a subset of Alt1 when RRC parameter timeRestrictionForChannelMeasurements is not configured. |
| NTT DOCOMO | Support Alt.1. |  |
| ZTE | Prefer Alt-2. In Rel-18 LTM, whether to introduce this restriction for SSB based L1 measurement has been discussed in RAN4, but eventually it has not been supported in the end of the release. So we don’t think that such restriction needs to be introduced for CSI-RS based L1 measurement in Rel-19. |  |
| Sharp | We prefer Alt-2. Since we don’t have this restriction in Rel-18 SSB based LTM, we don’t see the point to support this in Rel-19 CSI-RS based LTM. |  |

[FL proposal 3-5-v2]

Alt 1: supported by Fujitsu, Nokia, Ericsson, Samsung, CATT, DOCOMO,

  - A RRC parameter timeRestrictionForChannelMeasurements is introduced for CSI-RS and SSB based L1-RSRP measurement for both event triggered reporting and gNB scheduled reporting

    - If this parameter is set “configured”, only one sample can be used when RAN4 defines their requirement

    - Otherwise, the number of samples can be up to UE implementation, and the exact number of samples to define requirements is up to RAN4

  - This parameter does not impact the UE behaviour in L1 including the definition of CSI-RS reference resources

  - FL note: Is an LS to RAN4 necessary?

Alt 2: vivo, Lenovo, ZTE, Sharp

  - RRC configuration for time restriction is NOT introduced for CSI-RS based L1-RSRP measurement for either event triggered reporting or gNB scheduled reporting, i.e. it is up to RAN4 how many samples are used to define their requirement.

[To Chair: link to the list of online proposals]

[Conclusion]

The following conclusion was made during the online discussion on Monday

Conclusion

RRC configuration for time restriction is NOT introduced for CSI-RS based L1-RSRP measurement for either event triggered reporting or gNB scheduled reporting, i.e. it is up to RAN4 how many samples are used to define their requirement.

With this, the discussion of this section is closed.

[Closed] CSI reference resource and processing units for event triggered reporting

[Summary of contributions]

On top of the discussion in RAN1#120bis meeting, the companies views are summarized as below:

Option 1: no CPU is defined for event triggered reporting

  - Supported by Ericsson, vivo, ZTE, Qualcomm, DOCOMO

  - Rationale:

    - As both event evaluation and reporting for event-triggered L1 measurement reporting are managed by the MAC layer, no CPU involvement is necessary.

Option 2: CPU is defined for event triggered reporting, where OCPU = 1

  - Supported by Fujitsu, Nokia, TCL, Google, Lenovo, CATT, LG, Huawei,

  - Rationale:

    - Better handling of CSI-RS processing capability from gNB side

  - CSI reference resource is not defined

  - Start point:

    - Alt 1-1: The first OFDM symbol of the earliest one of each transmission occasion of periodic CSI-RS/SSB resource of all the candidate cells for L1-RSRP measurements

    - Alt 1-2: The first symbol of the earliest one of each CSI-RS resource triggered TTT.

    - Alt 1-3: The activation or (re)configuration of the event-triggered report

  - End point

    - Alt 2-1: the last symbol of the configured PUSCH carrying the MAC-CE report

    - Alt 2-2: Z3' symbols after the last symbol of the latest one of the CSI-RS/SSB resource of all the candidate cells for L1-RSRP measurements in each transmission occasion

    - Alt 2-3: the deactivation or reconfiguration of the event-triggered report

    - Alt 2-4: The symbol for dedicated SR or the last symbol of the slot used for dedicated SR

  - FFS: whether or how to share the UE capability for CPU with legacy operation (i.e. for serving cells)

[FL observation]

As discussed in RAN1#120bis, FL agrees that UE/chipset vendors’ view is important to make the decision. Also, if the CPU processing is not necessary, nothing is required to gNB side in terms of CPU management. This makes the implementation for both gNB and UE easier.

Based on this analysis, FL preference is not to define CPU involvement unless requested by UE/chipset vendors.

[FL proposal 3-6-v1]

FL suggestion is to agree on Option 1 unless UE/chipset vendors strongly request to define CPU for event triggered reporting. If option 2 is taken, the details (i.e. selection of Alt 1-x and Alt 2-x) will be further discussed during RAN1#121.

Option 1: no CPU is defined for event triggered reporting

  - Supported by Ericsson, vivo, ZTE, Qualcomm, DOCOMO

  - Rationale:

    - As both event evaluation and reporting for event-triggered L1 measurement reporting are managed by the MAC layer, no CPU involvement is necessary.

Option 2: CPU is defined for event triggered reporting, where OCPU = 1

  - Supported by Fujitsu, Nokia, TCL, Google, Lenovo, CATT, LG, Huawei,

  - Rationale:

    - Better handling of CSI-RS processing capability from gNB side

  - If Option 2 is taken, further discuss the following during RAN1#121:

    - CSI reference resource is not defined

    - Start point of CPU occupation:

      - Alt 1-1: The first OFDM symbol of the earliest one of each transmission occasion of periodic CSI-RS/SSB resource of all the candidate cells for L1-RSRP measurements

      - Alt 1-2: The first symbol of the earliest one of each CSI-RS resource triggered TTT.

      - Alt 1-3: The activation or (re)configuration of the event-triggered report

    - End point of CPU occupation:

      - Alt 2-1: the last symbol of the configured PUSCH carrying the MAC-CE report

      - Alt 2-2: Z3' symbols after the last symbol of the latest one of the CSI-RS/SSB resource of all the candidate cells for L1-RSRP measurements in each transmission occasion

      - Alt 2-3: the deactivation or reconfiguration of the event-triggered report

      - Alt 2-4: The symbol for dedicated SR or the last symbol of the slot used for dedicated SR

    - Whether or how to share the UE capability for CPU with legacy operation (i.e. for serving cells)

[To Chair: link to the list of online proposals]

[Comments to 3-6-v1]

| Company | Comment |
| --- | --- |
| Fujitsu | If we need to define the CPU, then we prefer the CPU occupation time from the start point of Alt 1-1 to the end point of Alt 2-2. However, we respect the chipset vendors opinion if they don’t require to define the CPU in the event triggered reporting. |
| vivo | In our view, it is unnecessary to define CPU occupation rule for event-triggered report as it is triggered and reported on MAC layer. |
| Nokia | We prefer option 2 to have a clear understanding on the network side. However, we can support option 1 if it is confirmed by all UE vendors that no CPU is required for such measurements. |
| Qualcomm | We support Option 1. From the perspective of PHY layer, defining the CSI processing unit (CPU) is essential to manage the limited processing and memory budgets. However, since both event evaluation and reporting are handled by the MAC layer, the role of the PHY layer could be limited to delivering L1 measurement results to the MAC layer. Consequently, unlike the legacy CSI framework, no CPU (i.e., PHY layer processing and memory budgets) involvement from the PHY layer is required for event-driven L1 measurement reporting. It is also noted that Rel-17 Sidelink CSI reporting is also based on MAC CE, and no CPU occupation rule was defined. |
| Ericsson | Support Option 1. If Option 2 is taken and OCPU = 1 for each resource, which resources should be counted? Is L1-RSRP computed for all configured resources all the time? (This relates to proposal 2-2-v1.) |
| Google | We still prefer Option 2. From QC’s comment, it seems to imply CPU is not needed to be defined in PHY, but in MAC? |
| ITRI | Support Option 1. As both event evaluation and reporting are managed by the MAC layer, defining a CPU occupation rule is not required. |
| Samsung | For event-triggered reporting, though the corresponding beam reports are carried in MAC CE, the event evaluation is still based on L1 measurements. It is better to decouple the discussions of event evaluation and CSI reporting to have a better understanding of whether OCPU is needed or not. |
| Lenovo | Although the CSI is reported by MAC CE, it still be configured by CSI-ReportConfig and the beam measurement is performed in L1, thus the CPU occupation mechanism should be specified. |
| OPPO | Support Option 2 The CPU is defined for measurement and the measurement is conducted in L1, no matter whether the trigger happens in MAC layer. |
| CATT | We support option 2. For option 2, - CSI reference resource is not defined since for LTM event triggered reporting, measurement is performed in the physical layer while evaluation and reporting are performed in the MAC layer. - For CPU occupancy, support Alt 1-1 for the start point of CPU occupancy, Alt1-2 seems to have more than one starting point of CPU occupancy, Alt 1-3 is defines the starting point of CPU occupancy as the gNB’s behavior which seems appropriate. Meanwhile, we support Alt 2-2 for the end point of CPU occupancy as the extension of the legacy mechanism for its simple calculation. For the case of Alt 2-1, the UE doesn’t know if the condition of the LTM event is fulfilled and if the PUSCH carries the MAC CE since the event evaluation are not performed in PHY layer but in upper layers. In order to reuse the existing CPU definition of aperiodic CSI report, it requires additional indication from upper layer whether the condition is met and whether the PUSCH carries MAC CE. Alt 2-4 has similar problem as Alt 2-1. |
| Xiaomi | We are fine to support the direction of Option 2. But the starting point and the ending point needs further discussion. |
| LG | We support Option 2. But if UE and chipset venders all support Option 1, we can go with it. |
| NTT DOCOMO | Support Option-1. |
| Huawei, HiSilicon | We support opiton1. As the evaluation is performed in MAC, no need to define CPU for event triggered report. |
| ZTE | We support Option 1. As mentioned in other proponent of option1, in PHY layer, UE only performs L1 measurement and corresponding measurement results will be quickly transferred to MAC layer for event evaluation and report. During the whole evaluation procedure, the period for L1 measurement in PHY layer is very short. So we don’t think that is is necessary to introduce CPU. In addition, as we known, the evaluation of the whole event and design are already very complex, if CPU is supported, it will further complicate the design of it. |
| Sharp | We support option 1 because Physical layer just handle the L1 measurement and the effort is mainly handled in MAC layer. |

[FL proposal 3-6-v2]

Option 1: no CPU is defined for LTM event triggered reporting

  - Supported by Ericsson, vivo, ZTE, Qualcomm, DOCOMO, ITRI, Huawei, (LG, OK with majority), Nokia(Acceptable), Sharp

  - FL note: Rationale

    - As both event evaluation and reporting for event-triggered L1 measurement reporting are managed by the MAC layer, no CPU involvement is necessary.

Option 2: CPU is defined for LTM event triggered reporting, where OCPU = 1

  - Supported by Fujitsu, Nokia, TCL, Google, Lenovo, CATT, LG, Xiaomi, CATT, LG

  - If Option 2 is taken, further discuss and choose one from the following alternatives:

    - CSI reference resource is not defined

    - Start point of CPU occupation:

      - Alt 1-1: The first OFDM symbol of the earliest one of each transmission occasion of periodic CSI-RS/SSB resource of all the candidate cells for L1-RSRP measurements

      - Alt 1-2: The first symbol of the earliest one of each CSI-RS resource triggered TTT.

      - Alt 1-3: The activation or (re)configuration of the event-triggered report

    - End point of CPU occupation:

      - Alt 2-1: the last symbol of the configured PUSCH carrying the MAC-CE report

      - Alt 2-2: Z3' symbols after the last symbol of the latest one of the CSI-RS/SSB resource of all the candidate cells for L1-RSRP measurements in each transmission occasion

      - Alt 2-3: the deactivation or reconfiguration of the event-triggered report

      - Alt 2-4: The symbol for dedicated SR or the last symbol of the slot used for dedicated SR

    - UE capability for CPU is defined separately from legacy operation (i.e. for serving cells)

[To Chair: link to the list of online proposals]

[Conclusion]

The following conclusion was made during the 1st online discussion on Thursday.

Conclusion

No CPU is defined for LTM event triggered reporting.

With this, the discussion of this section is closed.

[Closed] Other issues

[Summary of contributions]

Fujitsu:

  - The active CSI-RS resources / ports are not counted for the event triggered reporting.

  - FL view: this can be discussed after CPU for event triggered reporting and active CSI-RS resources / ports for gNB scheduled reporting are resolved so that we can save our discussion time.

OPPO

  - When the serving cell is configured with mTRP and has two indicated joint/DL TCI states:

    - The UE derive two RSs for serving cell evaluation and each RS is from the QCL RS or the SSB that the QCL RS is QCLed to of each indicated joint/DL TCI state.

    - The L1-RSRP measurement of serving cell for event evaluation is the minimum value of the L1-RSRP measurement of those two RSs.

  - FL view: RAN1 can wait until RAN2 makes the decision on coexistence issue. This is also captured in the conclusion in section 5.3.4

Qualcomm

  - For event evaluation and measurement reporting for Events LTM2, LTM3, and LTM5, the current SpCell and its corresponding RSs should always be included in the LTM measurement resource configuration associated with the report configuration.

  - FL view: This is a correct understanding and a good proposal. However, FL understands that the same agreement was made in RAN2 for Rel-18 gNB scheduled reporting based on SSB. Therefore, FL suggestion is to wait for the explicit request from RAN2, if RAN1 discussion is necessary .

[Conclusion]

The discussion of this section is closed without any FL proposal.

Beam Management based on CSI-RS

[No issue] Candidate TCI states activation and indication based on CSI-RS

[Agreements in previous meetings]

Agreement

CSI-RS for BM as the referenceSignal with QCL-TypeD is supported for an LTM TCI state, where TRS is configured as referenceSignal with QCL-TypeA

[Conclusion]

No new issue is identified in this meeting.

[No issue] UE Rx beam management

[Agreements in previous meetings]

Agreement (RAN1#120bis)

Repetition=off is supported for candidate cell CSI-RS in Rel-19.

Repetition=on is not supported for candidate cell CSI-RS in Rel-19.

[Conclusion]

No new issue is identified in this meeting.

[Closed] CG resource identification

[Summary of contributions]

Nokia

  - According to the latest draft version of the RAN2 MAC running CR [2], when the CandidateTCI-State or CandidateTCI-UL-State is linked to a CSI-RS, clarification from RAN1 is needed on whether the UE should select a configured grant tied to that specific CSI-RS.

  - For CSI-RS-based TCI states and RACH-less configured grant-based cell switch, allocating separate configured grants for each CSI-RS may increase network overhead.

  - For RACH-less configured grant-based cell switch, when the CandidateTCI-State or CandidateTCI-UL-State indicated by the Cell Switch Command is associated with a CSI-RS, the UE determines the configured grant based on the SS/PBCH block that is QCLed with the CSI-RS, following the same approach as in Rel-18.

- The RAN2 situation can be found in the MAC running CR (R2-2502158), which is quoted below:

- 5.8	Transmission and reception without dynamic scheduling

- 5.8.2	Uplink

For an uplink grant configured for configured grant Type 1 for RACH-less LTM cell switch, when there is an ongoing RACH-less LTM cell switch procedure, for each configured uplink grant valid according to TS 38.214 [7] for which the above formula is satisfied, the MAC entity shall:

1>	if an SSB corresponding to the configured UL grant has the same SSB index as the SSB associated with the TCI state indicated by the TCI state ID field in LTM Cell Switch Command MAC CE, as specified in clause 5.18.35:

2>	select the SSB associated with the TCI state indicated by LTM Cell Switch Command MAC CE.

2>	indicate the SSB index to the lower layer;

2>	consider this configured uplink grant as valid.

[FFS1>	else if a CSI-RS corresponding to the configured UL grant has the same CSI-RS index as the CSI-RS associated with the TCI state indicated by the TCI state ID field in LTM Cell Switch Command MAC CE, as specified in clause 5.18.35:

2>	select the CSI-RS associated with the TCI state indicated by LTM Cell Switch Command MAC CE.

2>	indicate the CSI-RS index to the lower layer;

2>	consider this configured uplink grant as valid.]

Editor’s NOTE: This part will be further updated based on RAN1 progress on CSI-RS. E.g. there is no agreeent on CSI-RS associated with CG.

Comment by Nokia to the yellow part: This has not been agreed, and also may not be needed. The Rel-18 procedure based on SSB can work as it is. Even in Rel-18, the indicated TCI state can have TRS as the configured RS, then the UE finds the SSB associated with that TRS to determine the CG. The same procedure can be used when a CSI-RS if configured as the RS of the indicated TCI state.

Response from the editor: It is different from Rel-18, since Rel-19 introduced CSI-RS based L1 measurement. But anyway, I have put it as FFS and an EN is added

The running CR says that RAN2 will wait for RAN1 progress on CSI-RS.

[FL observation]

Even though FL understand the necessity of technical discussion, FL wonders what is the necessary additional RAN1 agreement to solve this RAN2 issue as RAN2 has not send an explicit question to RAN1 by a formal manner.

Therefore, FL would like to encourage interested companies to provide their views on the issue identified by Nokia. The interested companies can bring the results to RAN2.

[Comments if any]

| Company | Comment |
| --- | --- |
| Fujitsu | As FL mentioned, this is a RAN2 issue and RAN1 has already made sufficient agreements to make the decision in RAN2. |
| vivo | We share the similar view as FL that this issue is a RAN2 issue. If RAN2 spec has supported this feature, including CSI-RS based CG occasion determination in TS38.321 and association between CSI-RS and CG occasion in TS3.331, it can also be supported by RAN1 with necessary RAN1 specification changes in TS38.213. |
| Nokia | As highlighted above in the CRs, RAN2 will wait for RAN1 to reach some agreement on this issue. It would be helpful to achieve a quick consensus in RAN1, especially since this topic falls within RAN1’s scope. From our side, SSB-based CG selection is sufficient. |
| Ericsson | Agree with FL – wait for RAN2 request |
| Samsung | Similar views to the FL, vivo and Ericsson. It is a RAN2 issue; if RAN2 requires inputs/actions from RAN1, we can respond to that. |
| NTT DOCOMO | We share the similar view as FL that we should wait for RAN2 request. |

[Conclusion]

The discussion of this section is closed without any FL proposal. Companies are encouraged to work with their RAN2 colleagues to solve this issue, if necessary.

[Closed] Other issues

[Summary of contributions]

Leonovo

  - Support group based beam report for LTM CSI report to enable the multi-TRP operation after switch to the new serving cell.

  - FL view: at this moment, the interest level of Rx beam is not so high based on the discussing in the past. Considering the fact that this is the last meeting for functional freeze in Rel-19, FL suggestion is not discuss this issue for RAN plenary targeting Rel-20, if necessary.

[Conclusion]

The discussion of this section is closed without any FL proposal.

CSI acquisition for candidate cell(s)

[Closed] CSI acquisition framework – valid CSI

[Agreements in previous meetings]

Agreement (RAN1#118bis)

The following alternatives are further studied:

Alt-1: CSI-RS measurement and CSI reporting operations are performed before reception of LTM Cell Switch Command (CSC) MAC CE.

  - The report is sent to the serving cell and transferred to the candidate/target cell(s)

Alt-2: CSI-RS measurement can start before reception of LTM CSC MAC CE and CSI reporting operation is performed after reception of LTM CSC MAC CE.

  - The report is sent directly to target cell

Alt-3: CSI-RS measurement and CSI reporting operations are performed after reception of LTM CSC MAC CE.

  - The report is sent directly to target cell

Companies are requested to provide the details of exact report timing and triggering mechanism in the next meeting

Working Assumption (RAN1#119)

As baseline, CSI-RS measurement and CSI reporting operations are performed after reception of LTM CSC MAC CE.

  - The report is sent directly to target cell

  - Introduce UE capability for CSI-RS measurement can start before reception of LTM CSC MAC CE

  - Other than UE capability, strive for no additional spec impact compared to the baseline (only one triggering mechanism will be specified)

Agreement(RAN1#120)

For target cell CSI acquisition,

- A UE is provided with RRC configurations for periodic CSI-RS resource(s) and CSI report(s) for one or more candidate cell

  - For a candidate cell,

    - down-select from the following alternatives:

      - Alt 1: A single CSI report configuration is configured

      - Alt 2: Multiple CSI report configurations can be configured

    - down-select from the following alternatives:

      - Alt X: A single CSI-RS resource for CMR is associated with a CSI report configuration

      - Alt Y: Multiple CSI-RS resources for CMR can be associated with a CSI report configuration

  - FFS : Semi-persistent CSI-RS resource

- After the RRC configuration and before the reception of CSC, the UE may measure CSI based on the configured CSI-RS resource(s), which is subject to UE capability

  - FFS: whether or how to select a subset of CSI-RS resources to measure

  - FFS: when the UE may start measuring the configured CSI-RS resources

- UE determines the CSI report configuration based on the CSC

- After the reception of cell switch command, the UE may measure (depending on the timeline) CSI-RS resource(s) associated with determined CSI report configuration

- The latest available measured CSI on target cell resource(s) is conveyed at least by a single report, and the report is sent to the target cell

  - Option 1: to use UCI

  - Option 2: to use MAC CE

Note: with this agreement, the working assumption made in RAN1#119 is automatically confirmed.

Agreement (RAN1#120bis)

For the container of LTM CSI report to a target cell, UCI is used with a solution to avoid blind detection at gNB if valid CSI is not available

  - FFS: the definition of valid CSI

Agreement (RAN1#120bis)

For the solution to avoid blind detection for LTM CSI report if valid CSI is not available,

  - Option 1: CSI is reported when valid CSI is not available

    - Option 1-1: one CQI codepoint is used to indicate that valid CSI is not available

      - Option 1-1-1: Codepoint with the lowest CQI index is reported

[Summary of contributions]

Issue 1: definition of valid CSI

  - Option 1-1: Define a time gap requirement based on Z’, and invalid CSI is reported when the requirement is not satisfied

    - Z’: gap between CSI-RS occasion and report timing

      - CSI-RS occasion is:

        - if measurement isn’t performed before CSC

          - the first CSI-RS transmission occasion after beam application time,

          - the first CSI-RS transmission occasion after LTM CSC MAC CE, or

        - the first CSI-RS transmission occasion after a TCI state in the target cell is activated

      - Report timing is separately discussed in section 5.5.2

  - Option 1-2: Define time gap requirements based on Z and Z’, and invalid CSI is reported when the requirements are not satisfied

    - Z: gap between the measurement trigger and report timing

      - Measurement trigger is:

        - PDSCH carrying CSC MAC CE,

        - HARQ-ACK corresponding to PDSCH carrying CSC MAC CE, or

        - the TCI State activation MAC CE

      - Report timing is separately discussed in section 5.5.2

    - Z’: same as option 1

  - Option 1-3: Up to UE implementation

    - There remains uncertainty regarding the timing of the CSC MAC CE relative to the measurement occasions.

    - Additionally, other factors such as asynchrony and delay difference across cells, along with uncertainties in processing and interruption time for the first UL transmission, complicate the establishment of a clear timeline (including the definition of CSI reference resources), which is essential for determining “valid CSI”

  - Note: one company proposes to define CSI-RS reference resource using Z’ (i.e. Z'Nsymbslot ) to express the requirements above.

    - For Capability #1 UE (measurement only after CSC), the CSI reference resource is defined in time domain as n-nCSI-ref, where nCSI-ref is the smallest value greater than or equal to Z'Nsymbslot and no earlier than the last symbol of CSC MAC-CE command.

    - For Capability #2 UE (measurement before and after CSC) , the CSI reference resource is defined in time domain as n-nCSI-ref, where nCSI-ref is the smallest value greater than or equal to Z'Nsymbslot.

  - Note: Z’ is used to derive Tproc,CSI' as in the current specification

Issue 2: UE behaviour/procedure after invalid CSI report

  - Option 2-1: Aperiodic CSI report associated with LTM CSI report config for CSI acquisition can be triggered by the CSI request filed in DCI

    - A time window for the 2nd transmission may be introduced

  - Option 2-2: After the invalid CSI report, the UE can continue CSI report associated with LTM CSI report config for CSI acquisition using the next PUSCH occasion

    - CG-PUSCH is used even after the completion of LTM cell switch until conveying the valid CSI. The CG-PUSCH resource is released after that, or

    - The number of repetitive CSI report associated with LTM CSI report config is pre-configured in RRC, and the subsequent PUSCH(s) can convey the CSI report

  - Option 2-3: No new UE behaviour to report valid CSI after invalid CSI report is defined

    - Legacy operation (i.e. legacy CSI report config) at the new serving cell can be used after cell switch completion

Issue 3: Further details of invalid CSI reporting

  - ETRI: It is proposed that the specification clarify how to distinguish valid and invalid CSI when CQI=0 is reported. Specifically, if CQI=0 is accompanied by consistent combinations of RI and PMI, the report should be treated as valid. Conversely, CQI=0 with RI=0 or inconsistent combinations of RI and PMI should indicate invalid CSI. This approach avoids implementation ambiguity while requiring no changes to the existing signaling format.

  - ZTE: Report quantity of 'cri-RI-CQI' should be introduced to accommodate the case that a valid CSI is not available. When a valid CSI is not available, a CSI reporting only contains CSI part 1 with fixed size, e.g., CQI, RI and/or CRI.

[FL observation]

For issue 1 (definition of valid CSI), most of the companies think the valid CSI can be defined based on Z and/or Z’ with some modification. Meanwhile, companies think Z might not be needed. On the other hand, two companies think the definition of valid CSI can be left to UE implementation. It might be good to reduce the workload in RAN1 from FL point of view. However, it would make interoperability test difficult because the reason why UE reports invalid CSI will be unclear to the network side, resulting in useless feature.

For issue 2 (procedure after invalid CSI report), FL is not clear if there is any critical issue if we go with option 2-3 (i.e. just reusing the legacy procedure under new serving cell). We can start the discussion by clarifying the essentiality of this issue. Otherwise, the discussion can be deprioritized.

For issue 3 (further details of invalid CSI reporting), FL sees the necessity of more offline discussion as report of lowest CSI brings no big issue. If no valid CSI is available, the gNB will schedule a PDSCH with lowest MCS, which is the same behaviour as lowest CQI report as valid CSI.For cri-RI-CQI report quantity, FL wonders if the quantity will be switched by the UE depending on the validity of CSI. Therefore, issue 3 can be treated after the clarify of the proponents’ intention during offline discussions.

[FL proposal 5-1-1-v1]

FL view: select one option from the following as the first step. After that, the details of the timeline (if necessary) will be discussed further (hopefully in RAN1#121).

  - Option 1-1: Define a time gap requirement based on Z’, and invalid CSI is reported when the requirement is not satisfied

    - Z’ is the gap requirement between CSI-RS occasion and report timing

    - FFS: the details of CSI-RS location and report timing

    - FFS: the necessity to define CSI reference resource

  - Option 1-2: Define time gap requirements based on Z and Z’, and invalid CSI is reported when the requirements are not satisfied

    - Z’ is the gap requirement between CSI-RS occasion and report timing

    - Z is the gap requirement between the CSI measurement trigger and report timing

    - FFS: the details of CSI-RS occasion, CSI measurement trigger and report timing

    - FFS: the necessity to define CSI reference resource

  - Option 1-3: Definition of valid CSI is up to UE implementation

FL note: FFS will be further discussed and resolved in RAN1#121.

[FL proposal 5-1-2-v1]

FL note: For following options on UE behaviour/procedure after invalid CSI report, FL would like to collect companies view on the critical issue when option 2-3 below is taken. If Option 2-3 is the offline consensus, FL will not bring this FL proposal to the main session.

  - Option 2-1: Aperiodic CSI report associated with LTM CSI report config for CSI acquisition can be triggered by the CSI request filed in DCI

    - A time window may be introduced

  - Option 2-2: After the invalid CSI report, the UE continues CSI report associated with LTM CSI report config for CSI acquisition using the next PUSCH occasion(s)

    - CG-PUSCH is used even after the completion of LTM cell switch until conveying the valid CSI. The CG-PUSCH resource is released after that, or

    - The number of repetitive CSI report associated with LTM CSI report config is pre-configured by RRC, and the subsequent PUSCH(s) can convey the CSI report

  - Option 2-3: No new UE behaviour to report valid CSI after invalid CSI report is defined

    - Legacy operation (i.e. legacy CSI report config) at the new serving cell can be used after cell switch completion

[Comments to 5-1-1-v1 and 5-1-2-v2]

| Company | Comment |
| --- | --- |
| Fujitsu | FL proposal 5-1-1-v1: Support Option 1-2. The gNB knows when the periodic or SP CSI-RS is transmitted, the gNB can expect whether the CSI is valid or invalid. Therefore, the definition is needed. Among Option 1-1 and Option 1-2, we think two durations of (Z, Z’)-pair are needed to guarantee the enough time for measurement (Z) and for PUSCH preparation (Z’). FL proposal 5-1-2-v1: Support Option 2-2. We think the CSI to be postponed may be still valid for the DL in new serving cell. Therefore, it requires to re-transmit the CSI as soon as possible. For Option 2-1, if the gNB triggering uses the CSI request field in DCI, it does have impact to the legacy UE since the one entry of aperiodicTriggerStateList should be assigned for the LTM. On the other hand, Option 2-2 can provide the fastest UL occasion for the postponed CSI. The configuration of the number of repetitions can be provided by RRC or fixed value. We think the PUSCH for the postponed CSI can be both CG and DG PUSCH that the earliest PUSCHs can be used. |
| vivo | FL proposal 5-1-1-v1: Support Option 1-1. In our view, the PUSCH carry the CSI report is pre-configured by network or scheduled by the RAR UL grant, rather than CSC MAC CE or measurement trigger signaling before CSC MAC CE, so it is unnecessary to consider the duration of Z to define valid CSI. Only considering the duration of Z’ is sufficient. FL proposal 5-1-2-v1: Support Option 2-3. If no valid CSI report is received, the network can trigger CSI report as legacy to acquire valid CSI. It is unnecessary to introduce additional enhancements. |
| InterDigital | FL proposal 5-1-1-v1: Prefer Option 1-2, UE should be allowed to report invalid if there is not sufficient time from the measurement trigger. FL proposal 5-1-2-v1: Support Option 2-1. At least the UE should be allowed to utilize the CSI-RS resource instance determined from CSC if NW triggers A-CSI report after cell switch completion. Option 2-2 may require additional blind decoding at NW. |
| Nokia | Issue 5-1-1: We support Option 1-2, as it aligns with the current procedure in the serving cell. We agree that Option 1-3 is not feasible, as noted in the FL observation. Issue 5-1-2: We believe Option 2-1 is closest to the legacy operation with minimal changes, while also offering the benefit of early CSI, i.e., the UE can still report available valid measurements compared to the case when legacy operation is triggered to obtain the report. |
| Qualcomm | Proposal 5-1-1-v1: Option 1-3 is our first preference, though we are open for further discussion. - For a concrete definition of valid CSI, Option 1-2 appears to be appropriate. Nevertheless, due to the lack of coordination and synchronization between cells (i.e., the target cell may not know when the UE would receive the CSC MAC CE), defining time gap Z may pose significant challenges from the target cell’s perspective. - As quoted by the FL, other factors such as asynchrony and delay difference across cells, along with uncertainties in processing and interruption time for the first UL transmission (note: only the maximum cell switch delay requirements are defined in the spec) complicate the establishment of a clear timeline. - We should consider the rationale behind defining “invalid CSI”. The purpose may be to facilitate (1) network control to proactively avoid the “invalid CSI” situations, or (2) follow-up actions once “invalid CSI” has been identified. Regarding (1), given the aforementioned issues (e.g., lack of coordination and synchronization), the feasibility of such control is questionable. For (2), if we agree on Option 2-3 in Proposal 5-1-2-v1, distinguishing between “invalid CSI” and “out of range” becomes unnecessary. Proposal 5-1-2-v1: We prefer Option 2-3. It should be noted that the reported CSI would be used for a short period after the cell switch, until another CSI acquisition is accomplished based on the configuration in the new serving cell. Therefore, the benefit of supporting CSI retransmission is limited, and distinguishing “invalid CSI” from “out of range” indication is not essential. |
| Ericsson | FL proposal 5-1-1-v1: Support Option 1-2. If the UE reports out-of-range in the first UL-resource, the NW can use the (Z,Z’) delay requirements to schedule a aperiodic CSI-report at a point in time when CSI is available. FL proposal 5-1-2-v1: Support option 2-1. The new serving cell should be able to trigger aperiodic report for the same LTM-CSI-ReportConfig as for early CSI acquisition. This requires that the UE continues measurement operations for a brief period in the new cell. |
| Google | FL proposal 5-1-1-v1: Support Option 1-2, which is similar to legacy. FL proposal 5-1-2-v1: Prefer Option 2-3. Option 2-1/2-2 seem to be an optimization on this issue. Without them, SPEC is not broken. |
| ITRI | FL Proposal 5-1-1-v1: We prefer Option 1-2, as it aligns with the current procedure. FL proposal 5-1-2-v1: Support option 2-1 is recommended. The new serving cell should address the results following the invalid CSI report. |
| Samsung | FL proposal 5-1-1: We support Option 1-2 based on a pair of (Z, Z’). FL proposal 5-1-2: We support Option 2-3. If the UE cannot send the (early) acquired CSI due to timeline requirements, no further operations regarding the (early) acquired CSI are needed. Sending the early CSI report via the first available UL transmission before cell switch is complete is the essence of the early CSI acquisition framework for LTM – if not possible, legacy behaviors are followed for the “new” serving cell. |
| Spreadtrum | FL proposal 5-1-1-v1: Prefer Option 1-2 where the Z describes how much time to prepare the UL transmission and the Z’ describes how much time for CSI calculation. FL proposal 5-1-2-v1: Prefer Option 2-3. During cell switching, if invalid LTM CSI report is transmitted, the LTM procedure can be prioritized for completion to reduce interruption time. At this point, the LTM CSI measurement results may have become outdated. After cell switching, the lowest MCS can be used for initial transmission and legacy CSI reporting can be scheduled for new serving cell measurement. |
| Lenovo | FL proposal 5-1-1-v1: Support option 1-3. As the UE shall indicate whether a valid CSI is included in the UL transmission, it’s not necessary for such definition. FL proposal 5-1-2-v1: Support option 2-3. Regular aperiodic CSI triggering and report mechanism can be reused in case that an invalid CSI report is indicated. |
| OPPO | Re 5-1-1: we support Option 1-3. Options 1-1/1-2 contradict with the decision of reporting invalid CSI to avoid blind detection. If some specific rule is specified, both gNB and UE would know if one instance has valid or invalid CSI and then the special reporting for invalid CSI to avoid blind detection should not and are needed to support. Re 5-1-2: we support Option 2-3. Now spec change is needed and it can be handled by legacy specification. Introducing new behavior does not have any benefit. |
| ETRI | FL proposal 5-1-1-v1: Support Option 1-2. It provides a clear and specification-aligned way to determine whether a CSI report is valid or invalid, based on predefined timing parameters (Z and Z′). FL proposal 5-1-2-v1: We support Option 2-1 as it allows the network to quickly re-trigger a CSI report in case an invalid CSI is received. In particular, when using CG-based PUSCH, there may be a relatively long delay before the next available PUSCH opportunity. |
| CATT | For proposal 5-1-1-v1, support Option 1-3 since the CSI report instance could not be fully controlled by gNB, which is different to that of legacy CSI. For proposal 5-1-2-v2, support Option 2-2. Although the valid CSI is not reported by the first PUSCH, if reported by the second or third PUSCH, it is still early than the CSI report triggered after cell switch completion. |
| Xiaomi | FL proposal 5-1-1-v1: support option 1-2. Regarding the 1st FFS of Option 1-1, there is a typo need to be revised that: - FFS: the details of CSI-RS occasion and report timing FL proposal 5-1-2-v1: support option 2-3 |
| LG | FL proposal 5-1-1-v1: Support Option 1-3. because it takes place from serving cell to target cell, it is hard to exactly define UE should measure and report in symbol/slot level. So, it is proper way to make UE implementation. FL proposal 5-1-2-v1: Support Option 2-3. If Option 2-2 works, there is no additional indication where UE transmits the CSI. It means UE doesn’t know how long UE should wait for transmitting CSI via next PUSCH and the long term makes the CSI information aging issue. If the UE continuously calculating the CSI, it would be a burden to the UE. If Option 2-1 works, it is unclear what is different from legacy gNB scheduled CSI request. |
| NTT DOCOMO | FL proposal 5-1-1-v1: We slightly prefer Option 1-3 since we have a similar view as QC. FL proposal 5-1-2-v1: Support Option 2-1. |
| Huawei, HiSilicon | FL proposal 5-1-1-v1: Support option 1-1 for measurement before CSC. because the configuration/activation of P/SP CSI-RS is far before the report and Z is not necessary. Option 1-2 may be useful for measurement after CSC. FL proposal 5-1-2-v1: support 2-1. UE reporting invalid CSI can due to lack of time for preparing the CSI report, in such case gNB could schedule a UL resource for the valid CSI reporting without addition delay for the measurement. reusing AP CSI trigger mechanism can minimize the standard impact. |
| ZTE | FL proposal 5-1-1-v1: Firstly, we think that the specific timeline for CSI report should be defined rather than leave it for UE implementation. In our understanding, timeline is a very important indicator in the whole communication system, it can ensure the efficient and stable operation of the entire system. Without clearly timeline relationships, it will not only lead to unnecessary misunderstanding between NW and UE but also cause ambiguity for RAN4 when formulating test cases for CSI acquisition function. With such considerations, we tend to Option1-1 or Option 1-2. compared with option1-1, we slightly prefer option1-1 because triggering mechanism for LTM CSI reporting is different with that of legacy aperiodic CSI reporting, we don’t see the strongly need to introduce the limitation between triggering for LTM CSI reporting and CSI reporting. FL proposal 5-1-2-v1: for this issue, we tend to first discuss and determine whether to support LTM CSI reporting after an invalid CSI is transmitted. If yes, we can further discuss that whether a LTM CSI reporting is transmitted by a triggering signaling or automatically method. In our understanding, if there is no any additional LTM CSI reporting transmission opportunity, it means that UE will suffer a significantly data rate degradation within a period of time after cell switch completion, especially for low latency and high-quality immersive service, where the specific time depends on the time required for CSI measurement and the time required for CSI reporting. In such case, we tend to introduce a middle-method such as option 2-1 (FFS: time window) to avoid prolonged performance degradation as much as possible before suitable MCS/RI/PMI are obtained. |

[FL proposal 5-1-1-v2]

  - Option 1-1: Define a time gap requirement based on Z’, and invalid CSI is reported when the requirement is not satisfied

    - Supported by, vivo, Huawei, ZTE

    - Z’ is the gap requirement between CSI-RS occasion and report timing

    - FFS: the details of CSI-RS occasion and report timing

    - FFS: the necessity to define CSI reference resource

  - Option 1-2: Define a time gap requirements based on Z and Z’, and invalid CSI is reported when the requirements are not satisfied

    - Supported by Fujitsu, IDC, Nokia, Ericsson, Google, ITRI, Samsung, Spreadtrum, ETRI, Xiaomi, ZTE

    - Z’ is the gap requirement between CSI-RS occasion and report timing

    - Z is the gap requirement between the CSI measurement trigger and report timing

    - FFS: the details of CSI-RS occasion, CSI measurement trigger and report timing

    - FFS: the necessity to define CSI reference resource

  - Option 1-3: Definition of valid CSI is up to UE implementation

    - Supported by, Qualcomm, Lenovo, OPPO, CATT, LG, DOCOMO,

FL note: FFS will be further discussed and resolved in RAN1#121.

[FL proposal 5-1-2-v2]

  - Option 2-1: Aperiodic CSI report associated with LTM CSI report config for CSI acquisition can be triggered by the CSI request filed in DCI

    - Supported by, IDC, Nokia, Ericsson, ITRI, DOCOMO, Huawei, ZTE

    - A time window may be introduced

  - Option 2-2: After the invalid CSI report, the UE continues CSI report associated with LTM CSI report config for CSI acquisition using the next PUSCH occasion(s)

    - Supported by Fujitsu, CATT

    - CG-PUSCH is used even after the completion of LTM cell switch until conveying the valid CSI. The CG-PUSCH resource is released after that, or

    - The number of repetitive CSI report associated with LTM CSI report config is pre-configured by RRC, and the subsequent PUSCH(s) can convey the CSI report

  - Option 2-3: No new UE behaviour to report valid CSI after invalid CSI report is defined

    - Supported by, vivo, Qualcomm, Google, Samsung, Spreadtrum, Lenovo, OPPO, Xiaomi, LG

    - Legacy operation (i.e. legacy CSI report config) at the new serving cell can be used after cell switch completion

[To Chair: link to the list of online proposals]

[FL proposal 5-1-1-v3]

Alt :1

- Define delay requirements based on Z and Z’, and UE reports a valid CSI when the delay requirement is satisfied

  - Z’ is the delay requirement between:

    - if the measurement is not performed before CSC / if the UE is not capable of the measurement before CSC

      - the last OFDM symbol of the CSI-RS of the corresponding CSI-RS resource after the PDSCH carrying CSC MAC CE and the first OFDM symbol of the CSI reporting

    - if the measurement is performed before CSC / if the UE is capable of the measurement before CSC

      - For periodic CSI-RS resources, the last OFDM symbol of the CSI-RS corresponding CSI-RS resource after the completion of the RRC configuration for the CSI-RS resource and the first OFDM symbol of the CSI reporting

      - For semi-persistent CSI-RS resources, the last OFDM symbol of the CSI-RS corresponding CSI-RS resource after the activation for the CSI-RS resource and the first OFDM symbol of the CSI reporting

  - Z is the delay requirement between:

    - The last symbol of the PDSCH carrying CSC MAC CE (+3ms?, HARQ-ACK?) and the first OFDM symbol of the CSI reporting

Alt :2

Definition of valid CSI is up to UE implementation

[To Chair: link to the list of online proposals]

[Conclusion]

The following conclusion was made during the 1st online discussion on Thursday, May 22, 2025

Conclusion

There is no RAN1 consensus to introduce delay requirements based on Z and Z’

Also, FL proposal 5-1-2-v2 was presented in the 2nd online session on Thursday, but not discussed due to the lack of time.

With this, the discussion of this section is closed.

[Closed] CSI acquisition framework – UL channel and resource to convey UCI report

[Agreements in previous meetings]

Agreement (RAN1#118bis)

The following alternatives are further studied:

Alt-1: CSI-RS measurement and CSI reporting operations are performed before reception of LTM Cell Switch Command (CSC) MAC CE.

  - The report is sent to the serving cell and transferred to the candidate/target cell(s)

Alt-2: CSI-RS measurement can start before reception of LTM CSC MAC CE and CSI reporting operation is performed after reception of LTM CSC MAC CE.

  - The report is sent directly to target cell

Alt-3: CSI-RS measurement and CSI reporting operations are performed after reception of LTM CSC MAC CE.

  - The report is sent directly to target cell

Companies are requested to provide the details of exact report timing and triggering mechanism in the next meeting

Working Assumption (RAN1#119)

As baseline, CSI-RS measurement and CSI reporting operations are performed after reception of LTM CSC MAC CE.

The report is sent directly to target cell

Introduce UE capability for CSI-RS measurement can start before reception of LTM CSC MAC CE

Other than UE capability, strive for no additional spec impact compared to the baseline (only one triggering mechanism will be specified)

Agreement(RAN1#120)

For target cell CSI acquisition,

- A UE is provided with RRC configurations for periodic CSI-RS resource(s) and CSI report(s) for one or more candidate cell

  - For a candidate cell,

    - down-select from the following alternatives:

      - Alt 1: A single CSI report configuration is configured

      - Alt 2: Multiple CSI report configurations can be configured

    - down-select from the following alternatives:

      - Alt X: A single CSI-RS resource for CMR is associated with a CSI report configuration

      - Alt Y: Multiple CSI-RS resources for CMR can be associated with a CSI report configuration

  - FFS : Semi-persistent CSI-RS resource

- After the RRC configuration and before the reception of CSC, the UE may measure CSI based on the configured CSI-RS resource(s), which is subject to UE capability

  - FFS: whether or how to select a subset of CSI-RS resources to measure

  - FFS: when the UE may start measuring the configured CSI-RS resources

- UE determines the CSI report configuration based on the CSC

- After the reception of cell switch command, the UE may measure (depending on the timeline) CSI-RS resource(s) associated with determined CSI report configuration

- The latest available measured CSI on target cell resource(s) is conveyed at least by a single report, and the report is sent to the target cell

  - Option 1: to use UCI

  - Option 2: to use MAC CE

Note: with this agreement, the working assumption made in RAN1#119 is automatically confirmed.

Agreement (RAN1#120bis)

For the container of LTM CSI report to a target cell, UCI is used with a solution to avoid blind detection at gNB if valid CSI is not available

  - FFS: the definition of valid CSI

[Summary of contributions]

The following issues are discussed in the companies’ contributions

Issue 1: Channel to carry CSI

  - Everyone assumed to use aperiodic PUSCH

  - No proposal to used PUCCH

Issue 2: timing of PUSCH to carry the first CSI report

  - Option 1: Unified PUSCH timing regardless of RACH

    - Supported by CMCC, Fujitsu, MediaTek, Samsung, ZTE, Google, Huawei, Nokia

    - The PUSCH carrying RRC reconfiguration complete message via either the configured grant (CG) or dynamic grant (DG).

      - Note：for CFRA based LTM, FL assumes that PUSCH scheduled by RAR UL grant or PUSCH of MsgA to target cell carries RRC reconfiguration complete message

      - Note: FL understanding is that the first UL carries RRC reconfiguration complete message

  - Option 2: different PUSCH timing depending on RACH

    - Supported by Qualcomm, Spreadtrum, CATT, Lenovo, ETRI, Apple, IDC

    - For RACH based (i.e. both CFRA and CBRA) LTM cell switch

      - The PUSCH scheduled by RAR UL grant or PUSCH of MsgA to target cell.

    - For RACH-less LTM cell switch

      - The PUSCH carrying RRC reconfiguration complete message via either the configured grant (CG) or dynamic grant (DG).

Issue 3: beta_offset

  - Two companies propose the introduction of a separate βoffsetCSI, candidate   configuration for each candidate cell to determine the number of REs used for early CSI report.

Issue 4: reporting format

  - One company proposes to use the UCI report format defined in Table 6.3.1.1.2-7 in TS 38.212 for CSI acquisition.

[FL observation]

For issue 1 (Channel to use early CSI acquisition), we can conclude that PUCCH is not used as nobody proposes to use it for LTM CSI acquisition, which looks reasonable.

For issue 2 (timing of PUSCH), there are mainly two proposals as summarized above, i.e. universal solution or RACH-dependent solution. Even though option 2 may enables earlier CSI reporting than option 1, some companies showed a concern to use Msg.3 because the transmitted UCI is suffered from contention. Based on this reason, FL preference is to go with option 1.

For issue 3, FL thinks this is a reasonable proposal, and further discuss online to finalize the RRC parameter discussion.

[FL proposal 5-2-v1]

For LTM CSI acquisition,

  - Only aperiodic is supported for reportConfigType

  - The PUSCH carrying RRC reconfiguration complete message via either the configured grant (CG) or dynamic grant (DG) is used to convey the CSI report

    - FL note: the description of “The PUSCH carrying RRC reconfiguration complete message” may not be appropriate for L1 specs because L1 may not be aware of the contents of PUSCH. Also, FL believes this is identical to the proposal to use the first PUSCH. The wording improvement may be needed. The focus point is whether and/or how to exclude Msg.3 for CBRA to convey CSI report.

  - RRC parameter beta_offset is introduced for each candidate cell

  - Table 6.3.1.1.2-7 in TS 38.212 is used as a UCI report format

[To Chair: link to the list of online proposals]

[Comments to 5-2-v1]

| Company | Comment |
| --- | --- |
| Fujitsu | Support the FL proposal. One concerning point is that we need to avoid using the term of “The PUSCH carrying RRCReconfigurationComplete message”. For the case that the target cell is included in the MCG, the RRCReconfigurationComplete message is sent to the target cell. On the other hand, if the target cell is included in the SCG and SRB3 is not configured in SCG, then the RRCReconfigurationComplete message is firstly sent to MCG rather than SCG. We think this is not the case what we intended, and what we agreed that “UE reports the CSI to the target cell”. Therefore, our suggestion is to use “the PUSCH” scheduled by UL grant in RAR for RACH-based LTM, and “the earliest PUSCH” after finishing the cell switch procedure for RACH-less LTM without the description of the RRCReconfigurationComplete message. |
| vivo | Fine with the FL proposal. |
| InterDigital | We prefer Option 2. For RACH-based, Option 1 results in additional delay. Collision for msg3 is a corner case that is only possible with CB-RACH. |
| Nokia | Support the first two bullet points in principle. The other two we need to check further. |
| Qualcomm | Proposal 5-2-v1: For the RACH-less scenario, Option 1 and Option 2 are identical. In the case of RACH-based scenarios, the physical layer may be unaware of which PUSCH will carry the RRC reconfiguration complete message, as the FL has already noted; it could be Msg3/MsgA, or even segmented across multiple PUSCHs. To avoid ambiguity, it is preferred to piggyback the UCI onto the earliest PUSCH to the target cell, whether it is Msg3/MsgA or CG/DG-PUSCH, regardless of the message carried in the PUSCH. |
| Ericsson | Support. In our understanding, with RACH-based LTM, the RRCReconfiguration-Complete is sent in the PUSCH scheduled by the RAR grant, so the there is no difference between the two timings in option 2. |
| NEC | We are OK with FL proposal. |
| ITRI | Fine with the FL proposal. |
| Samsung | We are fine with the second bullet of the proposal (without the FL note), which we think is the most essential component of this proposal. The rest of the bullets are unclear to us, which we cannot agree for now and prefer to have further discussions. For example, for the first bullet, it is unclear to us whether we can set reportQuantity to aperiodic – which has other implications to various design aspects – for the one-shot reporting. |
| Spreadtrum | We think the Msg3 for 4-step RACH based LTM cell switch can be used for report transmission to help gNB acquire CSI information immediately. |
| Lenovo | We support this proposal in principle. And we agree with FL that we need to further discuss whether Msg.3 for CBRA can be used for this purpose. Since only CSI report with fixed size is reported in the UL resource, the parameter beta_offset seems not needed. |
| OPPO | We do not need to specify any reportingType for this particular scenario. |
| ETRI | We support the unified solution proposed by FL, where the CSI report is transmitted together with the RRCReconfigurationComplete. While we initially considered Option 2 to reduce CSI reporting delay, we now agree that in CBRA-based LTM, the UCI sent in Msg 3 may face contention issues, which can affect reliability. For this reason, it seems clearer and more robust to send the CSI report together with the RRCReconfigurationComplete in both RACH-less and RACH-based procedures. Still, it should be clarified whether the CSI report is meant to indicate valid CSI only, or whether it may also include invalid CSI, such as a report with CQI=0. |
| CATT | We are fine with the proposal. |
| Xiaomi | We are fine with a unified design for both RACH-less and RACH based access. |
| LG | Support. |
| NTT DOCOMO | We are fine with FL proposal in principle. |
| Huawei, HiSilicon | Generally fine with the FL proposal. For the RRC parameter beta offset, we are thinking the beta offset configured in the RRCReconfiguration for the target cell can be reused. No need to have a separate beta offset. |
| ZTE | Generally, we are fine with this proposal. But for the bullet “RRC parameter beta_offset is introduced for each candidate cell”, we noticed that Betaoffset is introduced for CSI part 1 and CSI part 2. So we think that we should first clarify CSI reporting contents, such as for valid CSI, CSI report consists of CSI part 1 and CSI part 2, while for invalid CSI, a CSI report only includes CSI part 1 with fixed size. |
| Sharp | We support option 2 to acquire CSI report immediately for RACH-based case. For beta offset parameter, we think this is a separate discussion, and we can focus on first two bullets in this proposal. |

[FL proposal 5-2-v2]

For PUSCH to convey the early CSI report,

  - For RACH-less LTM, the first CG or DG PUSCH after CSC is used

  - For RACH-based LTM with CFRA, PUSCH scheduled by RAR or Msg.A

  - For RACH-based LTM with CBRA The first CG or DG PUSCH after HARQ-ACK transmission for Msg.4 or Msg.B

For the reporting LTM early CSI reporting

  - Table 6.3.1.1.2-7 in TS 38.212 is used as a UCI report format

[To Chair: link to the list of online proposals]

[Conclusion]

The following agreement was made during the 1st online discussion on Thursday, May 22, 2025

Agreement

For PUSCH to convey the early CSI report,

- For RACH-less LTM, the first CG or DG PUSCH after CSC is used

- For RACH-based LTM with CFRA, PUSCH scheduled by RAR or Msg.A

- For RACH-based LTM with CBRA, the first CG or DG PUSCH after HARQ-ACK transmission for Msg.4 or Msg.B

For the reporting LTM early CSI reporting, Table 6.3.1.1.2-7 in TS 38.212 is used as a UCI report format

With this, the discussion of this section is closed

[Closed] Support of intra- and/or inter- frequency for CSI acquisition

[Summary of contributions]

Option 1: Both intra- and inter- frequency can be supported from RAN1 perspective

  - Supported by CATT, CMCC, Fujitsu, Google, KDDI, ETRI, Ericsson, DOCOMO, MediaTek

  - From RAN1 perspective, there is no restriction with regards to the frequency location of CSI-RS used for CSI acquisition.

Option 2: Only intra-frequency can be supported

  - Supported by Vivo, Nokia (ask RAN4)

  - Vivo: Do not support CSI measurement on candidate cells performed outside the active BWP of the serving cell.

  - Nokia: Before cell switch: CSI-RS measurements should be performed only using CSI-RSs within the serving cell’s active BWP. During cell switch: CSI-RS measurements should be performed using CSI-RSs within the source cell’s active BWP or within the target cell’s initial BWP.

[FL observation]

Even though FL understands the motivation of option 2 (i.e. precluding inter frequency to simplify the system design), FL sees no strong necessity to limit the mechanism to intra-frequency from RAN1 perspective as the support of inter-frequency has no spec impact in RAN1. If dropped in Rel-19, we need to comeback to WID to support this feature in the future release, which is not good. Therefore, FL would like to propose option 1 and the final decision is up to RAN4.

[FL proposal 5-3-v1]

From RAN1 perspective, there is no restriction with regards to the frequency location of CSI-RS used for CSI acquisition.

Send an LS to RAN4 (CC RAN2) and ask them to take this information into their further work.

FL note: check if Nokia and ZTE is OK with this direction

[To Chair: link to the list of online proposals]

[Comments to 5-3-v1]

| Company | Comment |
| --- | --- |
| Fujitsu | Support the FL proposal. |
| vivo | Support the FL proposal. |
| InterDigital | Support the FL proposal. |
| Nokia | Support to check with RAN4 as it seems even for L1-RSRP, they may only support intra-frequency scenario. |
| Qualcomm | Support the FL proposal. |
| Ericsson | Support the FL proposal |
| Google | FL proposal 5-2-v1: Just to clarify, even for CFRA RACH-based LTM, the PUSCH carrying RRC reconfiguration complete message seems to be Msg3 as well. It is still scheduled by RAR grant with limited UCI multiplexing capability, we are unsure whether it can carry UCIs on it. |
| ITRI | Support the FL proposal. |
| Samsung | Support. We do not think any restriction is needed. |
| Spreadtrum | Support the FL proposal. |
| Lenovo | Support. Both intra- and inter-frequency should be supported. |
| ETRI | Support the FL proposal |
| CATT | Support the FL proposal. |
| Xiaomi | Support the FL proposal. |
| LG | Support the FL proposal. |
| NTT DOCOMO | Support the FL proposal. |
| KDDI | Support the FL proposal |
| Huawei, HiSilicon | support |
| ZTE | We have the same view with Nokia, whether to differentiate intra- and inter-frequency for CSI-RS measurement should be decided in RAN4 first. |
| Sharp | Support the FL proposal. |

[Conclusion]

The following conclusion was made during Monday online session

Conclusion

- From RAN1 perspective, there is no restriction with regards to the frequency location of CSI-RS used for CSI acquisition for candidate cells

Send an LS to RAN4 (CC RAN2) and ask them to take the above conclusion in their further work. LS in R1-250XXXS.

With this conclusion, the discussion of this section is closed.

[Closed] Time domain property of CSI-RS resource

[Agreements in previous meetings]

Agreement(RAN1#120)

For target cell CSI acquisition,

- A UE is provided with RRC configurations for periodic CSI-RS resource(s) and CSI report(s) for one or more candidate cell

  - For a candidate cell,

    - down-select from the following alternatives:

      - Alt 1: A single CSI report configuration is configured

      - Alt 2: Multiple CSI report configurations can be configured

    - down-select from the following alternatives:

      - Alt X: A single CSI-RS resource for CMR is associated with a CSI report configuration

      - Alt Y: Multiple CSI-RS resources for CMR can be associated with a CSI report configuration

  - FFS : Semi-persistent CSI-RS resource

- After the RRC configuration and before the reception of CSC, the UE may measure CSI based on the configured CSI-RS resource(s), which is subject to UE capability

  - FFS: whether or how to select a subset of CSI-RS resources to measure

  - FFS: when the UE may start measuring the configured CSI-RS resources

- UE determines the CSI report configuration based on the CSC

- After the reception of cell switch command, the UE may measure (depending on the timeline) CSI-RS resource(s) associated with determined CSI report configuration

- The latest available measured CSI on target cell resource(s) is conveyed at least by a single report, and the report is sent to the target cell

  - Option 1: to use UCI

  - Option 2: to use MAC CE

Note: with this agreement, the working assumption made in RAN1#119 is automatically confirmed.

Agreement (RAN1#120bis)

For candidate cell CSI acquisition

  - In addition to periodic CSI-RS resource, semi-persistent CSI-RS resource is supported

    - Support of semi-persistent CSI-RS resource is subject to separate UE capability.

    - MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2

  - Send an LS to RAN2 and RAN3 to inform this agreement

[Summary of contributions]

The following 2 issues are identified in companies’ contributions.

Issue 1: Remaining issues on SP CSI-RS resource

  - Autonomous deactivation of SP CSI-RS resource after CSC is proposed by Huawei and vivo

  - Autonomous activation of SP CSI-RS resource by CSC is proposed by Huawei, but a concern by ZTE i.e. the SP CSI-RS resource should be activated before CSC.

Issue 2: Support of AP CSI-RS resource

  - Support: ZTE, TCL, Spreadtrum

  - Concern: MediaTek

[FL observation]

For issue 1, in Rel-18, there was a discussion on autonomous TCI state deactivation after cell switch, but no agreement was made in RAN1. Thus, FL doesn’t see any urgency to make decision on this issue in this meeting.

For issue 2, the introduction of AP CSI-RS resource can be discussed if time available. FL believes the most important thing is to complete the design using P/SP CSI-RS first.

With this situation, no FL proposal is made under this section at this moment, The necessity of FL proposal in this meeting will be decided based on the comments.

[Comments if any]

| Company | Comment |
| --- | --- |
| vivo | Issue 1: Support autonomous deactivation of SP CSI-RS resource after CSC other than the SP CSI-RS resources associated with the target cell, as the SP CSI-RS resources associated with the target cell are still used to acquire CSI. Issue 2: Open to discuss. In our view, the only use case for AP CSI-RS resource is that CSC MAC CE triggers AP CSI-RS resource to acquire CSI of the target cell. |
| Nokia | For L1-RSRP measurements, RAN2 is currently discussing whether all activated SP CSI-RSs should be deactivated upon cell switch. However, for early CSI, it would be beneficial for RAN1 to agree on the expected behavior, as it may differ from L1-RSRP. Specifically, for early CSI, the UE needs to measure CSI-RSs after the cell switch. Therefore, the activated SP CSI-RSs for the target cell should be assumed to remain activated after the switch, while all other activated SP CSI-RSs can be assumed deactivated. |
| Samsung | We do not think any of the two issues are essential |
| Spreadtrum | For UE only supporting CSI measurement after the reception of LTM CSC, AP-CSI-RS associated the CSI report configuration is more reasonable and also can be supported. |

[Conclusion]

The discussion of this section is closed without any FL proposal.

[Closed] Configuration details for CSI report and CSI-RS resource - IMR

[Agreements in previous meetings]

Agreement(RAN1#120)

For target cell CSI acquisition,

- A UE is provided with RRC configurations for periodic CSI-RS resource(s) and CSI report(s) for one or more candidate cell

  - For a candidate cell,

    - down-select from the following alternatives:

      - Alt 1: A single CSI report configuration is configured

      - Alt 2: Multiple CSI report configurations can be configured

    - down-select from the following alternatives:

      - Alt X: A single CSI-RS resource for CMR is associated with a CSI report configuration

      - Alt Y: Multiple CSI-RS resources for CMR can be associated with a CSI report configuration

  - FFS : Semi-persistent CSI-RS resource

- After the RRC configuration and before the reception of CSC, the UE may measure CSI based on the configured CSI-RS resource(s), which is subject to UE capability

  - FFS: whether or how to select a subset of CSI-RS resources to measure

  - FFS: when the UE may start measuring the configured CSI-RS resources

- UE determines the CSI report configuration based on the CSC

- After the reception of cell switch command, the UE may measure (depending on the timeline) CSI-RS resource(s) associated with determined CSI report configuration

- The latest available measured CSI on target cell resource(s) is conveyed at least by a single report, and the report is sent to the target cell

  - Option 1: to use UCI

  - Option 2: to use MAC CE

Note: with this agreement, the working assumption made in RAN1#119 is automatically confirmed.

Agreement(RAN1#120bis)

Regarding CSI acquisition, for a candidate cell,

  - A single CSI report configuration is configured

  - Multiple CSI-RS resources for CMR can be associated with the CSI report configuration

  - The number of CSI-RS resources for CMR is subject to UE capability

[Summary of contributions]

Issue 1: Support of IMR

  - Ericsson

    - A list of interference measurement resources should also be introduced in LTM-TCI-Info:

  - OPPO

    - The NZP CSI-RS resources configured for CMR is used as IMR.

  - Qualcomm

    - For CSI acquisition on candidate cells, configuration of CSI-IM resources for interference measurement should be supported.

      - Each CSI-RS resource for CMR is resource-wise associated with a CSI-IM resource.

Issue 2: Structure of CSI-RS resource and report configurations

  - Nokia:

    - For early CSI acquisition for a candidate cell, the report configuration is provided in the candidate cell’s LTM-Candidate IE (but outside of the ltm-CandidateConfig).

    - For early CSI acquisition of a candidate cell, the CSI-RS resource information can be provided using an NZP-CSI-RS-ResourceSet given under LTM-Candidate IE

  - ZTE

    - CSI-RS resource/set configuration

      - A list of NZP CSI-RS resources (e.g., ltm-nzp-CSI-RS-ResourceToAddModList-r18) and a list of NZP CSI-RS resource sets (e.g., ltm-nzp-CSI-RS-ResourceSetToAddModList-r18) are provided by LTM-TCI-Info-r18 in LTM-Candidate-r18, where NZP-CSI-RS-ResourceSet is configured without the higher layer parameters ‘repetition’ and ‘trs-Info’.

    - CSI-RS measurement configuration

      - A list of CSI resource configurations (e.g., ltm-csi-ResourceConfigToAddModList-r19) is configured in LTM-Candidate-r18, where one CSI resource configuration (e.g., ltm-CSI-ResourceConfig-r19) provides one or more NZP CSI-RS resource sets and each NZP CSI-RS resource set includes multiple CSI-RS resources for CSI measurement.

    - CSI reporting configuration

      - One LTM CSI Reporting configuration (i.e., LTM-CSI-ReportConfig-r19) is configured in LTM-Candidate-r18.

  - Fujitsu

    - CSI report configuration for each candidate cell is provided under LTM-Config.

    - For CSI-RS resource set configuration, CSI-RS resource(s) in a single resource set can be associated with only one candidate cell.

  - Ericsson

    - The report configuration for CSI acquisition is the same in the source cell, the target cell and the UE. The UE keeps it over cell switch execution.

    - Adapt LTM-CSI-ReportConfig to support relevant report quantities, including cri-RI-PMI-CQI and cqi-Table for CQI reporting.

[FL observation]

For issue 1, RAN1 has already discussed on CMR but the discussion on IMR has been postponed in order to complete the basic design of CSI acquisition. Now it is time to conclude this issue as RRC parameter discussion should be concluded in this meeting. Nevertheless, the input from companies is not enough to make a concrete FL proposal at this moment. Therefore, we can start our discussion by gathering the companies’ view.

For issue 2 (RRC structure), FL recommendation is to directly discuss under RRC parameter discussion by the rapporteur to save time for main session.

[FL proposal 5-5-v1]

Companies are encouraged to provide their answer (yes or no) to the following proposal.

A list of interference measurement resources for candidate cells is introduced for LTM CSI acquisition

  - Yes: Ericsson, Qualcomm

  - No (CMR is used for interference measurement) : OPPO

[To Chair: link to the list of online proposals]

[Comments to 5-5-v1]

| Company | Comment |
| --- | --- |
| Fujitsu | Yes |
| vivo | Considering large specification impact of the introduction of dedicated IMR, including RAN1, RAN2 and RAN4, it should not be discussed considering late stage of Rel-19. |
| InterDigital | Yes |
| Nokia | No – we share the same concern, which is also raised by vivo. |
| Ericsson | Yes |
| Google | Yes |
| Samsung | No need for this proposal |
| Lenovo | We support both CMR and IMR based interference measurement for CSI acquisition. |
| OPPO | No need to configure dedicated IMR |
| ETRI | Yes |
| CATT | No. We share the same opinion as OPPO, where CSI-RS would be used as IMR. |
| Xiaomi | No need to configure dedicated IMR. Otherwise, many resource need to be configured. |
| LG | Yes |
| NTT DOCOMO | Yes |
| Huawei, HiSilicon | No. |
| ZTE | Yes, CSI-IM for interference measurement is very important for CSI acquisition, so it should be supported when discussing related RRC parameters. |
| Sharp | No. We don’t think IMR is needed considering complexity. |

[FL proposal 5-5-v2]

A list of interference measurement resources for candidate cells is introduced for LTM CSI acquisition

  - Yes: Ericsson, Qualcomm, Fujitsu, IDC, Google, Lenovo, ETRI, ZTE, LG

  - No: OPPO, Nokia, Samsung, CATT, Xiaomi, Huawei, Sharp

[To Chair: link to the list of online proposals]

[Conclusion]

The following agreement was made during the online discussion on Monday.

Agreement

A list of interference measurement resources for candidate cells is supported for LTM CSI acquisition

- If this list is not configured, CMR is used for interference measurement

With this agreement, the discussion of this section is closed.

[Closed] Subset of CSI-RS resources for CSI measurement before CSC

[Agreements in the previous meetings]

Agreement(RAN1#120)

For target cell CSI acquisition,

- A UE is provided with RRC configurations for periodic CSI-RS resource(s) and CSI report(s) for one or more candidate cell

  - For a candidate cell,

    - down-select from the following alternatives:

      - Alt 1: A single CSI report configuration is configured

      - Alt 2: Multiple CSI report configurations can be configured

    - down-select from the following alternatives:

      - Alt X: A single CSI-RS resource for CMR is associated with a CSI report configuration

      - Alt Y: Multiple CSI-RS resources for CMR can be associated with a CSI report configuration

  - FFS : Semi-persistent CSI-RS resource

- After the RRC configuration and before the reception of CSC, the UE may measure CSI based on the configured CSI-RS resource(s), which is subject to UE capability

  - FFS: whether or how to select a subset of CSI-RS resources to measure

  - FFS: when the UE may start measuring the configured CSI-RS resources

- UE determines the CSI report configuration based on the CSC

- After the reception of cell switch command, the UE may measure (depending on the timeline) CSI-RS resource(s) associated with determined CSI report configuration

- The latest available measured CSI on target cell resource(s) is conveyed at least by a single report, and the report is sent to the target cell

  - Option 1: to use UCI

  - Option 2: to use MAC CE

Note: with this agreement, the working assumption made in RAN1#119 is automatically confirmed.

Final FL proposal in RAN1#120bis

For the subset selection of the measurement CSI-RS resource(s) from the configured CSI-RS resources before CSC, further study a mechanism to indicate subset of CSI-RS resources for CSI measurement, and if introduced down-select one from the following options in RAN1#121

  - Option 1: The subset is indicated by a new MAC CE

  - Option 2: The subset is indicated by a DCI (e.g. cell indicator field for PDCCH order, CSI request field)

  - Option 3: The subset is implicitly indicated by the Candidate cell TCI state activation/deactivation MAC CE

  - Support: Fujitsu, Ericsson, IDC, Nokia, Samsung, vivo, ZTE, Lenovo, CATT, Xiaomi, Qualcomm, ETRI,

  - Concern: OPPO, Huawei, MediaTek

Note: the following consequence is expected, if introduced

  - Alt .1: UE shall measure the subset before CSC

  - Alt .2: UE is not expected to measure/report the CSI-RS resource(s) outside of the subset before CSC

[Summary of contributions]

The proposals on subset indication of CSI-RS resources before CSC is summarized as follows:

Option 1: Subset indication of CSI-RS resources before CSC is NOT needed

  - Supported by Huawei, Spreadtrum, Google, MediaTek

  - Rationale

    - Redundant functionality, can be discussed in Rel-20

    - For UE supporting CSI measurement before the reception of LTM CSC, before the reception of LTM CSC, the UE shall perform CSI measurements on each candidate cells

    - It is up to UE implementation to determine which CSI-RS resources are measured before CSC

Option 2: Subset indication of CSI-RS resources before is needed

  - Supported by Qualcomm ZTE, Fujitsu, KDDI, Sharp, vivo, Nokia, LGE, ZTE, Samsung, Xiaomi, IDC, ETRI, Ericsson, DOCOMO

  - Rationale

    - The CSI-RS measurement starts only after the TCI state activation/indication for the candidate cell.

    - Only a subset of CSI-RS resource(s) that are the same or QCLed with the QCL RS(s) in the activated/indicated TCI state(s) should be measured.

    - To reduce the measurement overhead by a UE

  - Detailed mechanism for indication

    - Alt .1 Introduce a new MAC CE: (ZTE, Fujitsu, KDDI, Sharp)

      - Indicating the CSI-RS resources plus candidate TCI-state activation/deactivation

    - Alt. 2 Reuse the existing field in DCI format, e.g. CSI request field Sharp

    - Alt. 3 Implicitly indicated by candidate cell TCI state activation/deactivation MAC CE: (vivo, Nokia, LGE, ZTE, Samsung, Xiaomi, IDC, ETRI, Ericsson, DOCOMO)

      - SP CSI-RS resource(s) are also implicitly activated

      - The UE only needs to keep/store CSI for a limited number of candidate cell(s) before cell switching command

      - After a candidate TCI state activation, the UE continues/initiates the measurements for early CSI acquisition only for cells/RSs associated with active TCI states.

    - Alt.4: Candidate cells which L1 measurement is performed (TCL)

    - Alt.5: Implicitly indicated by the cell indicator field in PDCCH order (Nokia-FFS, CATT)

In addition, the UE behaviour related to this subset indication needs to be further clarified, if introduced.

After reception of the indication

  - UE shall measure the CSI after reception of the indication of the subset selection for CSI acquisition before CSC, and/or

  - UE is not expected to measure the configured CSI-RS resources not indicated by the indication.

Before reception of the indication

  - As agreed, UE “may” measure the configured CSI-RS resources, thus it is up to UE implementation, or

  - UE is basically required to measure the configured CSI-RS resources with some exception. The exception needs to be defined.

[FL observation]

Considering the number of supporting companies and the technical reason to associate TCI state activation and the reception of CSI-RS, FL suggestion is to focus on an indication mechanism using MAC CE associated with candidate cell TCI state activation.

The next question is that whether the existing signaling (i.e. MAC CE) is used or not. FL view is that a new MAC CE would be a cleaner solution for this purpose to decouple the Rel-18 UE feature and capability for candidate cell TCI state activation/deactivation. If so, the new MAC CE should also be used to activate/deactivate candidate cell TCI states based on the UE capability.

Finally, it is also needs to be clarified what is the expected UE behaviour when the UE receives the indication, i.e. the measurement is up to UE or measurement is a must. Furthermore, it should also be clarified if the UE is required to measure the CSI-RS resources not indicated.

[FL proposal 5-6-v1]

FL note: the following proposal is discussed including the down-selection of the alternatives.

A UE is provided with an indication of a subset of CSI-RS resources for CSI measurement before CSC

  - Alt 1-1: A new MAC CE is used for the explicit indication of the subset

    - Candidate cell TCI state(s) associated with the indicated CSI-RS resource(s) are activated by the MAC CE

  - Alt 1-2: Candidate Cell TCI States Activation/Deactivation MAC CE in Rel-18 is used for the implicit indication of the subset

    - qcl-InfoPeriodicCSI-RS in NZP-CSI-RS-Resource is used to identify the associated activated TCI states

    - Note: This means that the UE capability for candidate cell TCI state activation in Rel-18 will be the prerequisite for CSI measurement before CSC.

The UE which the subset is indicated:

  - Alt 2-1: shall measure the indicated CSI-RS resources before CSC

  - Alt 2-2: may measure the indicated CSI-RS resources before CSC, i.e. up to UE implementation

The UE that the subset is indicated is not expected to measure the configured CSI-RS resources not included in the subset.

FL note: FL wonders if the opponents of the introduction of subset can compromise with Alt 1-2 + Alt 2-2.

[Comments to 5-6-v1]

| Company | Comment |
| --- | --- |
| Fujitsu | Support Alt 1-1 and Alt 2-1. In Rel-18 LTM, the candidate cell TCI state activation / deactivation is optional feature. We would like to decouple the feature from the subset indication. For the UE behaviour after reception of the indication, we think the UE shall measure the indicated CSI-RS resources before CSC so that the NW can easily manage the status whether the UE measured the CSI or not. |
| vivo | Support Alt 1-1 and Alt 2-1. |
| InterDigital | Support Alt 1-2 and Alt 2-1. If Alt 2-2 is adopted, it is unclear why a capability is needed. |
| Nokia | Support Alt 1-2 and Alt 2-1. If early TCI state activation is not performed, a capable UE is expected to measure all the configured CSI-RSs, provided the configuration complies with its capabilities. This reduction in measurement scope helps release CPUs on the UE side, allowing resources to be used for other measurements. Determining which RSs to measure should not be left to UE implementation, as it would impact the definition of valid CSI, CPU occupancy, and the counting of active RSs, and would not align with legacy behavior. |
| Ericsson | Support Alt 1-2 and 2-1: Rel. 18 UE capability for candidate cell TCI state activation is a reasonable prerequisite capability. Alt 1-2 means reduced specification impact compared to Alt 1-1. If subset selection is up to UE implementation, then active resource/port counting may not be tracked since the NW doesn’t have knowledge of what the UE measures. For periodic CSI-RS for CSI acquisition, RRC reconfiguration on a per-UE basis is not dynamic enough as a tool to manage UE measurement operations. |
| Samsung | We are fine with the direction of Alt 1-2, i.e., leveraging candidate cells TCI states activation to determine the subset for measurement. However, it is unclear to us how the UE would utilize the activated TCI states for the corresponding candidate cells – to our understanding, there are two simple ways: - Alt 1-2a: The UE only measures CSI-RSs on candidate cells with TCI state(s) activation; whether the UE would measure all the CSI-RSs configured for the corresponding candidate cells or only the CSI-RSs specific to the activated TCI states for the candidate cells is up to UE - Alt 1-2b: The UE only measurements CSI-RS specific to activated TCI state(s) for the corresponding candidate cells; for candidate cells that are not with activated TCI state(s), the UE would measure all the corresponding CSI-RSs - Without clarifying the specific UE behaviors, we do not think simply saying based on candidate cell TCI state(s) activation is enough. Given that, the last bullet in the FL proposal is not needed for now. - Support Alt 2-2. Whether or not to measure before CSC is still subject to UE’s capability regardless of whether or not subset is indicated. |
| Spreadtrum | For the UE supporting CSI measurement before the reception of LTM CSC, it is up to UE implementation to determine which CSI-RS resources are measured before CSC. And after the reception of LTM CSC, the UE only performs CSI measurement on the target cell indicated by LTM CSC. |
| Lenovo | Support Alt 1-2. There is no need to introduce a new MAC CE for the CSI-RS subset selection. The UE only needs to measurement the CSI-RS resources associated with the activated TCI state for a candidate cell. The second part is not clear. How can the UE know the indicated TCI state before CSC for Alt 2-1? |
| OPPO | We do not see the justification to support this feature. If gNB wants the UE to only measure a subset of the CSI-RS resources, the gNB just need to configure those CSI-RS resources instead of configuring the so-called full set. Furthermore, we have serious technical concern on Alt 1-2. It does not make sense. The results of Alt 1-2 can not ensure the UE to measure the future target cell CSI, but only increase the UE power consumption. |
| ETRI | Support Alt 1-2 and Alt 2-1. |
| CATT | Support Alt 1-2 and Alt 2-1. Only those candidate cells with DL sync established prior to CSC would likely be the target cell. Therefore, candidate Cell TCI States Activation/Deactivation MAC CE in Rel-18 should be reused. |
| Xiaomi | Support Alt 1-2 and Alt 2-1 |
| LG | Support Alt 1-2 and Alt 2-1. And gNB should control the number of CSI-RS ports/resources to be measured. |
| NTT DOCOMO | Support Alt 1-2 and Alt 2-1. |
| KDDI | Support Alt 1-1 and Alt 2-1. |
| Huawei, HiSilicon | We do not think it essential to specify subset selection in R19. We hope a unified solution in R20 for all measurement types. |
| ZTE | Support Alt 1-2 and Alt 2-1. |

[FL proposal 5-6-v2]

A UE is provided with an indication of a subset of CSI-RS resources for CSI measurement before CSC

  - Alt 1-1: Fujitsu, KDDI

    - A new MAC CE is used for the explicit indication of the subset

      - Candidate cell TCI state(s) associated with the indicated CSI-RS resource(s) are activated by the MAC CE

  - Alt 1-2: IDC, Nokia, Ericsson, [Samsung], Lenovo, ETRI, CATT, Xiaomi, LG, DOCOMO, ZTE, vivo

    - Candidate Cell TCI States Activation/Deactivation MAC CE in Rel-18 is used for the implicit indication of the subset

      - UE measures the CSI-RS resource(s) associated with the activated TCI states(s)

        - FL note: For association between CSI-RS and TCI state, the CSI-RS (for CSI acquisition) is QCLed with SSB or CSI-RS (for BM/TRS) given as QCL RS of the activated TCI state(s)

      - Note: This means that the UE capability for candidate cell TCI state activation in Rel-18 will be the prerequisite for CSI measurement before CSC.

  - Alt 1-3: Samsung

    - Candidate Cell TCI States Activation/Deactivation MAC CE in Rel-18 is used for the implicit indication of the subset

      - UE measures the CSI-RS resource(s) for the candidate cells associated with activated TCI states(s)

        - FL note: For association between CSI-RS and TCI state, the CSI-RS (for CSI acquisition) is QCLed with SSB or CSI-RS (for BM/TRS) given as QCL RS of the activated TCI state(s)

      - Note: This means that the UE capability for candidate cell TCI state activation in Rel-18 will be the prerequisite for CSI measurement before CSC.

The UE which the subset is indicated:

  - Alt 2-1: Fujitsu, vivo, IDC, Nokia, Ericsson, ETRI, CATT, Xiaomi, LG, DOCOMO, KDDI, ZTE

    - The UE shall measure the indicated subset of CSI-RS resources before CSC

  - Alt 2-2:

    - The may measure the indicated subset of CSI-RS resources before CSC, i.e. up to UE implementation

The UE that the subset is indicated is not expected to measure the configured CSI-RS resources not included in the subset.

Concern on the proposals above: Huawei, Spreadtrum, OPPO, MediaTek

[FL proposal 5-6-v3]

A UE is provided with an indication of a subset of CSI-RS resources for CSI measurement before CSC

  - Candidate Cell TCI States Activation/Deactivation MAC CE in Rel-18 is used for the implicit indication of the subset

  - Alt 1-2: Upon reception of TCI state activation/deactivation MAC CE, the UE is not required to measure the CSI-RS resource(s) not associated with the activated TCI states(s)

  - Alt 1-2’: Upon reception of TCI state activation/deactivation MAC CE, the UE should measure the CSI-RS resource(s) associated with the activated TCI states(s)

    - (alt 1-2 and 1-2’) Supported by IDC, Nokia, Ericsson, [Samsung], Lenovo, ETRI, CATT, Xiaomi, LG, DOCOMO, ZTE, vivo

    - FL note: For association between CSI-RS and TCI state, the CSI-RS (for CSI acquisition) is QCLed with SSB or CSI-RS (for BM/TRS) given as QCL RS of the activated TCI state(s)

    - FL note: It was pointed out that Alt 1-2’ (UE shall/should measure) would be appropriate if the timeline for valid CSI is defined as “CSI-RS occasion before CSC and the report timing”

  - Alt 1-3: Upon reception of TCI state activation/deactivation MAC CE, the UE should measure the CSI-RS resource(s) for the candidate cell(s) associated with activated TCI states(s)

    - Supported by Samsung

  - Note: The UE capability for candidate cell TCI state activation in Rel-18 will be the prerequisite for CSI measurement before CSC.

Concern on the proposals above, i.e. no subset indication for CSI-RS resources before CSC is introduced

  - Huawei, Spreadtrum, OPPO, MediaTek

[To Chair: link to the list of online proposals]

[FL proposal 5-6-v4]

No consensus on the following proposal

A UE is provided with an indication of a subset of CSI-RS resources for CSI measurement before CSC

- Candidate Cell TCI States Activation/Deactivation MAC CE in Rel-18 is used for the implicit indication of the subset

  - Alt 1-2: Upon reception of TCI state activation/deactivation MAC CE, the UE is not required to measure the CSI-RS resource(s) not associated with the activated TCI states(s)

  - Alt 1-2’: Upon reception of TCI state activation/deactivation MAC CE, the UE should measure the CSI-RS resource(s) associated with the activated TCI states(s)

    - (alt 1-2 and 1-2’) Supported by IDC, Nokia, Ericsson, [Samsung], Lenovo, ETRI, CATT, Xiaomi, LG, DOCOMO, ZTE, vivo

    - FL note: For association between CSI-RS and TCI state, the CSI-RS (for CSI acquisition) is QCLed with SSB or CSI-RS (for BM/TRS) given as QCL RS of the activated TCI state(s)

    - FL note: It was pointed out that Alt 1-2’ (UE shall/should measure) would be appropriate if the timeline for valid CSI is defined as “CSI-RS occasion before CSC and the report timing”

  - Alt 1-3: Upon reception of TCI state activation/deactivation MAC CE, the UE should measure the CSI-RS resource(s) for the candidate cell(s) associated with activated TCI states(s)

    - Supported by Samsung

  - Note: The UE capability for candidate cell TCI state activation in Rel-18 will be the prerequisite for CSI measurement before CSC.

[To Chair: link to the list of online proposals]

[Conclusion]

The following conclusion was made during the 1st online discussion on Thursday.

Conclusion

There is no RAN1 consensus in Rel-19 to provide the UE with an indication of a subset of CSI-RS resources for CSI measurement before CSC

With this, the discussion of this section is losed

[Closed] Subset of report and CSI-RS resources for CSI measurement after CSC

[Agreements in the previous meetings]

Agreement(RAN1#120)

For target cell CSI acquisition,

- A UE is provided with RRC configurations for periodic CSI-RS resource(s) and CSI report(s) for one or more candidate cell

  - For a candidate cell,

    - down-select from the following alternatives:

      - Alt 1: A single CSI report configuration is configured

      - Alt 2: Multiple CSI report configurations can be configured

    - down-select from the following alternatives:

      - Alt X: A single CSI-RS resource for CMR is associated with a CSI report configuration

      - Alt Y: Multiple CSI-RS resources for CMR can be associated with a CSI report configuration

  - FFS : Semi-persistent CSI-RS resource

- After the RRC configuration and before the reception of CSC, the UE may measure CSI based on the configured CSI-RS resource(s), which is subject to UE capability

  - FFS: whether or how to select a subset of CSI-RS resources to measure

  - FFS: when the UE may start measuring the configured CSI-RS resources

- UE determines the CSI report configuration based on the CSC

- After the reception of cell switch command, the UE may measure (depending on the timeline) CSI-RS resource(s) associated with determined CSI report configuration

- The latest available measured CSI on target cell resource(s) is conveyed at least by a single report, and the report is sent to the target cell

  - Option 1: to use UCI

  - Option 2: to use MAC CE

Note: with this agreement, the working assumption made in RAN1#119 is automatically confirmed.

Agreement(RAN1#120bis)

Regarding CSI acquisition, for a candidate cell,

  - A single CSI report configuration is configured

  - Multiple CSI-RS resources for CMR can be associated with the CSI report configuration

  - The number of CSI-RS resources for CMR is subject to UE capability

[Summary of contributions]

Issue 1: details of report configuration selection using CSC

  - 5 companies (Huawei, NEC, ZTE, CMCC, DOCOMO) propose that CSI report configuration for CSI acquisition is implicitly determined from the field of Target Configuration ID in CSC.

Issue 2: selection of subset of the CSI-RS resources using CSC

  - Option 1: subset of CSI-RS resources is implicitly determined by the indicated TCI state in CSC

    - Supported by Huawei, vivo, Nokia, Spreadtrum, CATT, Lenovo, (KDDI?), DOCOMO, IDC

    - UE only measures the CSI-RS resource(s) associated with the TCI state indicated in the cell switch command

  - Option 2: subset is not introduced, i.e. UE measures all CSI-RS resources associated with the selected report configuration.

    - Supported by Samsung, Ericsson

  - Option 3: a CRI is included in CSC, and the CSI-RS resource to measure is explicitly indicated by the CRI

    - Supported by Fujitsu

Issue 3: Disabling of CSI acquisition

  - Two companies proposed to introduce a mechanism to disable CSI acquisition

    - Nokia: Send an LS to RAN4 to ask whether CSI measurements during cell switch impact cell switch delay.

      - In case the CSI measurements during the cell switch impact the cell switch delay, RAN1 to discuss whether the cell switch command may include an indication to specify whether the UE can perform CSI measurements (and reporting) as part of the cell switch operation.

    - Samsung: a UE can be provided by the network whether a candidate cell is enabled or not for CSI acquisition

[FL observation]

For the first issue (selection of report configuration), FL believes the companies view is aligned.

For the second issue (subset indication of CSI-RS resources by CSC), majority of the companies support to introduce an implicit subset indication using joint or DL TCI state. Meanwhile, explicit indication is also proposed to limit the measurement CSI-RS to one to reduce the measurement overhead by a UE. Meanwhile, FL thinks that no such subset indication is not always as the CRI report has already been specified and measurement of multiple CSI-RS resources is not a new functionality. Thus, the introduction of subset indication should be well justified. In this sense, FL preference is option 2 not to complicate the online discussion at this last moment.

For the third issue (disabling of CSI acquistion), it is not clear for FL which signaling the proponent want to use (RRC signaling, MAC CE) to disable the CSI acquisition. This needs further discussion, but FL assumes that the intention is (CSC) MAC CE.

[FL proposal 5-7-1-v1]

FL note: FL preference is option 2 not to complicate the online discussion at this last moment.

For LTM CSI acquisition:

  - CSI report configuration for CSI acquisition is implicitly determined from the field of Target Configuration ID in CSC MAC CE

  - For the selection of subset of the CSI-RS resources using CSC,

    - Option 1: subset of CSI-RS resources is implicitly determined by the indicated DL or joint TCI state in CSC

      - Supported by Huawei, vivo, Nokia, Spreadtrum, CATT, Lenovo, (KDDI?), DOCOMO, IDC

      - UE only measures the CSI-RS resource(s) associated with the TCI state indicated in the cell switch command

    - Option 2: Either explicit or implicit subset indication is not introduced, i.e. UE measures all CSI-RS resources associated with the selected report configuration.

      - Supported by Samsung, Ericsson

    - Option 3: a CRI field is included in CSC, and the CSI-RS resource to measure is explicitly indicated by the CRI

      - Supported by Fujitsu, (KDDI?)

[FL proposal 5-7-2-v1]

For LTM CSI acquisition:

  - LTM CSI report to the target cell can be disabled by an explicit field in CSC MAC CE

    - FL note: Nokia proposed to send an LS to RAN4 to ask the cell switch time with CSI acquisition to make the decision, but FL wonders if their reply will come before ASN.1 freeze.

[Comments to 5-7-1-v1 and 5-7-2-v1]

| Company | Comment |
| --- | --- |
| Fujitsu | FL proposal 5-7-1-v1: Support Option 3. When the several narrow beams are included in the wide beam indicated by the TCI state, it takes much time for the CSI-RS measurement for all narrow beams, and thus the invalid CSI will frequently happen. In order to avoid this situation, the NW needs to indicate the CSI-RS for the CSI so that the UE has enough time for the measurement. FL proposal 5-7-2-v1: We don’t think the disabling report scheme is needed. If UE determines that the measurement requires much time, the UE can send ‘invalid CSI’ indication to the target cell. |
| vivo | FL proposal 5-7-1-v1: Support Option-1. FL proposal 5-7-2-v1: Do not support this proposal. The motivation for the disabling of the LTM CSI report is not clear. |
| InterDigital | FL proposal 5-7-1-v1: Support Option 1 to reduce measurement effort without requiring impact to CSC MAC CE. FL proposal 5-7-2-v1: This seems already possible by not configuring CSI report configuration for the cell. |
| Nokia | Issue 5-7-1: We support Option 1, as it makes perfect sense to acquire CSI associated with a beam that will be applied in the target cell after the cell switch. If we select the subset based on the activated TCI states before the cell switch, it would not make sense to apply a different subset after the switch. We cannot envision a scenario where the UE is first instructed to measure only CSI-RSs associated with the activated TCI state, and then after the CSC, is allowed to measure additional CSI-RSs not associated with the activated TCI state (which is now an indicated TCI state in the CSC). Issue 5-7-2: Yes, it makes sense to allow network control, especially if CSI measurement impacts the cell switch delay. The network can disable this feature in scenarios where a very low cell switch delay is required (e.g., RACH-less Cell Switch). |
| Ericsson | FL proposal 5-7-1-v1: We support Option 2. Why is a subset needed? It is anyway limited to CSI-RS resources of one cell. FL proposal 5-7-2-v1: We agree with Fujitsu, Vivo, InterDigital, we don’t see that this is needed. |
| NEC | FL proposal 5-7-1-v1: We support Option 1, as mentioned in our contribution. FL proposal 5-7-2-v1: There doesn’t seem to be a strong motivation. |
| ITRI | FL proposal 5-7-1-v1: Support Option-1. FL proposal 5-7-2-v1: We do not see sufficient motivation to support this proposal. |
| Samsung | Proposal 5-7-1-v1: we support Option 2; we do not support “subset” indication for RS measurement, which is not needed after CSC. There is no motivation to introduce the “subset” indication for this case. Proposal 5-7-2-v1: we do not support this, which is not needed. |
| Spreadtrum | FL proposal 5-7-1-v1: Support Option 1. FL proposal 5-7-2-v1: Not support. |
| Lenovo | FL proposal 5-7-1-v1: Support option 1. For the case that multiple CSI-RS resources associated with the indicated TCI state, the UE can selecte one of them for CSI report and indicated it by CRI. FL proposal 5-7-2-v1: The motivation of such dynamic enabling/disabling CSI acquisition is not clear. If the UE cannot obtain the CSI before sending the first UL transmission, it can indicate a invalid CSI. |
| OPPO | 5-7-1: we support Option 2. The CSI report shall be measured based on the corresponding configured one or more CSI-RS resources, and that is also why the CSI report includes a CRI report. The issues with Option 1 is (1) cause difficulty to system on provide proper CSI report configuration (2) introduce unnecessary association between non-related system behavior (3) break the normal CSI measurement procedure. 5-7-2: not needed. If the system does not need CSI for one particular cell, the system just does not configure the corresponding CSI report configuration. There is no need to introduce extra indication in MAC CE. |
| ETRI | FL proposal 5-7-1-v1: We support Option 1. The TCI state included in the Cell Switch Command (CSC) refers specifically to the target cell, and therefore it is appropriate that the UE only measures the CSI-RS resource(s) associated with the TCI state indicated in the CSC. Measuring any other CSI-RS resources beyond those associated with the indicated TCI state would unnecessarily increase the UE complexity. FL proposal 5-7-2-v1: Support. For UEs that perform CSI measurement after receiving the CSC, it may be necessary for the NW to control whether LTM CSI acquisition is performed, especially when the network determines that the delay introduced by the measurement could be critical at the time of sending the CSC. On the other hand, for UEs that have already performed CSI measurement before CSC, no additional delay occurs, so this control is not required. |
| CATT | For FL proposal 5-7-1-v1: Support Option 1. For FL proposal 5-7-2-v1: Not support. The intention of dynamically disabling the CSI acquisition is not clear. If disabling is needed for a candidate cell, it is possible not to configure CSI report for that candidate cell. |
| Xiaomi | FL proposal 5-7-1-v1: regarding option 1, we would like to clarify what does “the CSI-RS resource(s) associated with the TCI state” mean? Does it mean UE need to measures all CSI-RSs QCLed with the QCL Type D RS in the TCI state and choose one to report? FL proposal 5-7-2-v1: what is the motivation of this proposal? |
| NTT DOCOMO | FL proposal 5-7-1-v1: Support Option-1. FL proposal 5-7-2-v1: We’re not sure why this proposal is needed. |
| KDDI | FL proposal 5-7-1-v1: Support Option 1. FL proposal 5-7-2-v1: support. |
| Huawei, HiSilicon | FL proposal 5-7-1-v1: we support option 1. FL proposal 5-7-2-v1: not essential |
| ZTE | FL proposal 5-7-1-v1: we tend to support Option 2 since we don’t see the need to further reduce measurement overhead after CSC. In our understanding, the number of CSI-RS resources associated with the determined LTM CSI reporting configuration by CSC is limited and not extremely large and it also be determined based on UE capability. FL proposal 5-7-2-v1: it is not clear to introduce the motivation and benefit of such operation. More clarifications or discussion are needed. |
| Sharp | FL proposal 5-7-1-v1: We support option 2 since the CSI-RS resource associated with the report configuration in CSC is specified and is limited, no more subset selection is required after CSC. In addition, we think ‘after CSC’ should be clarified in the proposal; otherwise it is not clear enough. FL proposal 5-7-2-v1: We don’t think this is needed. |

[FL proposal 5-7-1-v2]

For LTM CSI acquisition:

  - CSI report configuration for CSI acquisition is implicitly determined from the field of Target Configuration ID in CSC MAC CE

  - For the selection of subset of the CSI-RS resources using CSC,

    - Option 1: subset of CSI-RS resources is implicitly determined by the indicated DL or joint TCI state in CSC

      - Supported by Huawei, vivo, Nokia, Spreadtrum, CATT, Lenovo, KDDI, DOCOMO, IDC, Xiaomi, ETRI, ITRI, NEC

      - UE only measures the CSI-RS resource(s) associated with the TCI state indicated in the cell switch command

    - Option 2: Either explicit or implicit subset indication is not introduced, i.e. UE measures all CSI-RS resources associated with the selected report configuration

      - Supported by Samsung, Ericsson, OPPO, ZTE, Sharp

[FL proposal 5-7-1-v3]

For LTM CSI acquisition:

  - CSI report configuration for CSI acquisition is determined from the field of Target Configuration ID in CSC MAC CE

  - UE is expected to report the measurement result(s) for CSI-RS resource(s) associated with the indicated TCI state in CSC

    - FL note: For association between CSI-RS and TCI state, the CSI-RS (for CSI acquisition) is QCLed with SSB or CSI-RS (for BM/TRS) given as QCL RS of the indicated TCI state

[To Chair: link to the list of online proposals]

[Conclusion]

The following agreement was agreed during Wed online session

Agreement

CSI report configuration for CSI acquisition is determined from the field of Target Configuration ID in CSC MAC CE

With this, the discussion of this section is closed

[No issue] Contents of report and CSI-RS configurations

[Agreements in the previous meetings]

Agreement (RAN1#120bis)

Following restrictions are introduced

  - For the codebook configurations in report configuration, typeI-SinglePanel is supported for LTM CSI acquisition

  - For report frequency configuration in report configuration, wideband CQI and wideband PMI are supported for LTM CSI acquisition

  - For the report quantity in report configuration, cri-RI-PMI-CQI is supported for LTM CSI acquisition

    - The supported max rank is up to separate UE capability

  - For the number of CSI-RS ports of CSI-RS resource(s) associated with a CSI report configuration for a candidate cell for LTM CSI acquisition

    - Up to 128 ports is supported

  - The supported max number of CSI-RS ports is up to separate UE capability

[Summary of contributions]

ZTE

  - Report quantity of 'cri-RI-CQI' should be introduced to accommodate the case that a valid CSI is not available.

[FL observation]

The proposal by ZTE is discussed under 5.5.2 together with format issue. With this, no new issue is identified in this section, and not FL proposal is made in this section.

[Conclusion]

The discussion of this section is closed without any FL proposals.

[Closed] CSI-RS processing capability before CSC

[Agreements in the previous meetings]

The following figure is provided in the FLS for reference. The issues in this section are equivalent to Open issue 7 below.

[Summary of contributions]

Number of CPUs

  - For measurement performed before receiving CSC MAC-CE, ‘OCPU=N1’ where ‘N1’ is the total number of CSI-RS resources in the CSI-RS resource sets across candidate cells.

CPU occupancy time

  - Start point:

    - Alt. 1-1: one or more symbols starting from the last symbol of the uplink slot carrying the acknowledgment for the LTM configuration

    - Alt. 1-2: SP CSI-RS: the first symbol after 3 msec of the last symbol of HARQ-ACK information for the semi-persistent CSI-RS activation MAC CE

    - Alt. 1-3: P CSI-RS: the first symbol of earliest CSI-RS resource after RRC configuration

  - End point

    - Alt. 2-1: the last symbol of the downlink slot carrying the cell switch command

    - Alt. 2-2:  the last symbol of the uplink slot carrying the acknowledgment for the cell switch command

    - Alt. 2-3: if the CSI-RS is associated with target cell: the last symbol of the UL resource carrying the report

    - Alt. 2-4: if the CSI-RS is not associated with target cell: the last symbol of HARQ-ACK corresponding to LTM CSC MAC CE.

    - Note: start and end point may be affected by the introduction of subset indication of CSI-RS resources

UE Capability

  - Early CSI measurements should count toward the UE’s existing simultaneous CSI processing limit, or if a separate limit should apply.

[FL observation]

Even though no concern on the introduction of CPU involvement for CSI acquisition before CSC is provided, FL wonders the necessity depends on the interpretation of “UE may measure”. If this means “fully up to UE implementation”, then the definition of CPU is not so meaningful as it is not testable and gNB cannot manage it.

Therefore, we can start from the question whether or CPU for CSI acquisition before CSC should be defined, and then go to the detailed definition on CPU and occupancy time.

[FL proposal 5-9-v1]

FL note: RAN1 to further discuss the following proposal, and FL proposal will be updated based on the comment from the companies.

FL note: The problem to define CPU here is that the agreement is “UE may measure” and hence gNB cannot accurately estimate the CPU usage in a UE. This issue is related to [FL proposal 5-6-v1].

For the LTM CSI measurement before CSC

  - Option 1: CPU is not defined

    - CPU management for the CSI measurement before CSC is up to UE implementation

  - Option 2: CPU is defined

    - For measurement performed before receiving CSC MAC-CE, ‘OCPU=N1’ where ‘N1’ is the total number of CSI-RS resources in the CSI-RS resource sets across candidate cells.

    - CPU occupancy time is defined as

      - Start point:

        - Alt. 1-1: the last symbol of the uplink slot carrying the acknowledgment for the PDSCH carrying LTM configuration

        - Alt. 1-2: the first symbol after 3 msec of the last symbol of HARQ-ACK information for the semi-persistent CSI-RS activation MAC CE for periodic CSI-RS, and the first symbol of earliest CSI-RS resource after RRC configuration for semi-persistent CSI-RS

      - End point

        - Alt. 2-1: the last symbol of the downlink slot of PDSCH carrying the cell switch command MAC CE

        - Alt. 2-2: the last symbol of the uplink slot carrying the acknowledgment for the PDSCH carrying the cell switch command MAC CE

        - Alt. 2-3: if the CSI-RS is associated with target cell: the last symbol of the UL resource carrying the report. and if the CSI-RS is not associated with target cell: the last symbol of HARQ-ACK corresponding to LTM CSC MAC CE.

      - Note: start and end point may be affected by the introduction of subset indication of CSI-RS resources

    - FFS: whether or how to share the UE capability for CPU with legacy operation

      - This can be discussed under UE capability session

[Comments to 5-9-v1]

| Company | Comment |  |
| --- | --- | --- |
| Fujitsu | Support Option 2. We think the CSI measurement is quite large burden for UE, so the CPU should be defined. On the other hand, the legacy rule is somewhat inefficient because the UE ‘MAY’ measure the CSI before CSC. Therefore, the CPU should be defined only when the UE measures the CSI. Hence we prefer to defer the discussion after decision of the subset indication. |  |
| vivo | Prefer Option 2 and support Alt.1-2 as the starting point and Alt.2- 1 as the ending point. |  |
| InterDigital | Prefer Option 2. Alt. 1-2 and Alt. 2-1 seem fine. |  |
| Nokia | Support Option 2 in principle CSI acquisition should clearly consume CSI processing resources at the UE side. It would be surprising if it didn’t; otherwise, why would legacy CSI acquisition consume processing resources? Therefore, if this processing is to be shared with serving cell CSI measurements, it should be clearly clarified how many resources are involved and what the corresponding timeline is. Alternatives can be discussed further as some companies seems to be defining this from only the UE perspective. We need to define CPU occupancy from both UE and its serving cell perspective (before and after CSC). |  |
| Qualcomm | Proposal 5-9-v1: We support Option 1. There is a notable similarity between (the legacy aperiodic CSI reporting + periodic/semi-persistent CSI-RS) and (the target cell CSI reporting + CSI-RS measurement before CSC). Therefore, the same CSI processing criteria could be applied: Section 5.2.1.6, TS 38.214: - An aperiodic CSI report occupies CPU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the purpose of determining the CPU occupation duration, the PDCCH candidate that ends later in time is used.. That is, measuring candidate cell CSI-RS alone, without calculating CSI, is relevant to the “active CSI-RS” definition and does not occupy CPUs. The CPU occupancy duration starts only after the actual report is triggered, e.g., by DCI in legacy aperiodic CSI reporting or CSC MAC CE in target cell CSI reporting. |
| Ericsson | Support Option 1. Agree with Qualcomm: Since it is an AP CSI-RS report, CPU are occupied from the report trigger (i.e. LTM CSC MAC CE) to the report is sent in the first UL resource, not before CSC. |  |
| Google | Support Option 2. |  |
| NEC | We prefer Option 2. |  |
| Samsung | We prefer Option 1. As mentioned by Qualcomm, this is similar to legacy AP CSI report + P/SP CSI-RS for measurement. In this case, the OCPU would only start from the actual report is triggered by CSC. |  |
| Spreadtrum | Support Option 2. Slightly prefer Alt 1-2 and Alt 2-3. |  |
| Lenovo | Support Option 2. This CSI report is configured by CSI-ReportConfig, the CPU occupation mechanism should be specified. Firstly, the number of occupied CPUs should not be the total number of CSI-RS resources in the CSI-RS resource sets across candidate cells, because the UE only need to perform the CSI calculation and report on the CSI-RS resources of the target cell. Secondly, the CPU for the CSI report for early CSI acquisition should be occupied from the CSC to the end of the UL transmission carrying the CSI. |  |
| ETRI | Support Option 2. |  |
| CATT | In our opinion, CPU should be defined. Option 2 is OK for us. |
| Xiaomi | Prefer Option 2, Alt 1-2 and Alt 2-1 since it is before CSC. |  |
| NTT DOCOMO | Support Option 1. |  |
| Huawei, HiSilicon | Prefer option 2, Alt 1-2 and Alt 2-3. For Alt 1-2, I think there may be a typo that the start 3ms after ACK should be for SP CSI-RS and 1st symbol after configuration is for P-CSI-RS. |  |
| ZTE | We tend to introduce CPU for CSI acquisition and detailed definition of CPU can be discussed further. From our perspective, if CPU is not introduced for CSI acquisition, UE will keep using these resource continuously. To dynamically adapt to the computing resource of the CPU, it is very important and necessary to introduce CPU. |  |

[FL proposal 5-9-v2]

For the LTM CSI measurement before CSC

  - Option 1: CPU is not defined, Qualcomm, Ericsson, Samsung, Nokia, Apple, DOCOMO, Fujitsu, MediaTek,

    - CPU management for the CSI measurement before CSC is up to UE implementation

    - FL note: The CPU occupancy duration starts only after the actual report is triggered, e.g., by DCI in legacy aperiodic CSI reporting or CSC MAC CE in target cell CSI reporting

  - Option 2: CPU is defined, , vivo, IDC, , Google, NEC, ETRI, CATT, Xiaomi, Huawei, ZTE

    - For measurement performed before receiving CSC MAC-CE, ‘OCPU=N1’ where ‘N1’ is the total number of CSI-RS resources in the CSI-RS resource sets across candidate cells.

    - CPU occupancy time is defined as

      - Start point:

        - Alt. 1-1: the last symbol of the uplink slot carrying the acknowledgment for the PDSCH carrying LTM configuration

        - Alt. 1-2: the first symbol after 3 msec of the last symbol of HARQ-ACK information for the semi-persistent CSI-RS activation MAC CE for semi-persistent CSI-RS, and the first symbol of earliest CSI-RS resource after RRC configuration for periodic CSI-RS

      - End point

        - Alt. 2-1: the last symbol of the downlink slot of PDSCH carrying the cell switch command MAC CE

        - Alt. 2-2: the last symbol of the uplink slot carrying the acknowledgment for the PDSCH carrying the cell switch command MAC CE

        - Alt. 2-3: if the CSI-RS is associated with target cell: the last symbol of the UL resource carrying the report. and if the CSI-RS is not associated with target cell: the last symbol of HARQ-ACK corresponding to LTM CSC MAC CE.

      - Note: start and end point may be affected by the introduction of subset indication of CSI-RS resources

    - UE capability of CPU for candidate cells does not consume the legacy (i.e. serving cell) CPU occupancy

      - FL note: This can be discussed under UE capability session

[To Chair: link to the list of online proposals]

[Conclusion]

The following conclusion was made during the 2nd online discussion on Thursday.

Conclusion

For the LTM CSI measurement before CSC, CPU is not defined

With this, the discussion of this section is closed.

[Closed] CSI-RS processing capability after CSC

[Agreements in the previous meetings]

The following figure is provided in the FLS for reference. The issues in this section are equivalent to Open issue 7 below.

[Summary of contributions]

Number of CPUs

  - no CPU occupancy rule is required (LG, Qualcomm)

    - Triggered by a CSC MAC CE, UE processes only a single CSI without sharing CPUs with other CSI processes

  - For measurement performed after receiving CSC MAC-CE, ‘OCPU=N2’ where ‘N2’ is the total number of CSI-RS resources in the CSI-RS resource set of the candidate cell in the CSC MAC-CE.(Apple)

CPU occupancy time

  - Start point:

    - Alt. 1-1: the first symbol after the PDSCH carrying the LTM CSC MAC CE

    - Alt. 1-2: the end of the PUCCH with HARQ-ACK corresponding to the PDSCH carrying the LTM CSC

    - Alt. 1-3: the first symbol of earliest CSI-RS resource after 3 msec of the last symbol of HARQ-ACK information for the LTM CSC MAC CE

  - End point:

    - Alt 2: the last symbol of the uplink resource carrying the CSI report

  - Start and end point:

    - Alt 3: having two CPU counting are applied:

      - CPU counter #1: Counting for each CSI measurement occasion starting from the first symbol of the earliest one of each CSI-RS/SSB resource for channel measurement and ends at the symbol ‘C1+∆1’, where C1 is the last symbol of last CSI-RS resource.

      - CPU counter #2: countering for a first PUSCH occasion. Assuming the last symbol ‘L’ of first PUSCH, the CPU counter #2 starts from ‘L-∆2’ symbol and ends at the last PUSCH symbol ‘L’.

UE Capability

  - RAN1 to clarify whether the number of active CSI-RS ports or active CSI-RS resources, in both the serving and target cells, for early CSI acquisition measurements should be counted towards the current maximum UE limit of active CSI-RS ports/resources or as a separate limit.

[FL observation]

Similar to the discussion of section 5.5.9, we can start from the necessity to define CSI-RS processing as no other CSI-RS process other than CSI-acquisition will be conducted during the LTM procedure. After concluding that, we can go to the detailed definition.

[FL proposal 5-10-v1]

FL note: RAN1 to further discuss the following, and FL proposal will be updated based on the comment from the companies.

For the LTM CSI measurement after CSC

  - Option 1: CPU is not defined

    - The CSI measurement is up to UE implementation as no other CSI process is conducted during LTM procedure

  - Option 2: CPU is defined

    - For measurement performed before receiving CSC MAC-CE, ‘OCPU=N2’ where ‘N2’ is the total number of CSI-RS resources in the CSI-RS resource set of the candidate cell in the CSC MAC-CE

    - CPU occupancy time is defined as

      - Start point:

        - Alt. 1-1: the first symbol after the PDSCH carrying the LTM CSC MAC CE

        - Alt. 1-2: the end of the PUCCH with HARQ-ACK corresponding to the PDSCH carrying the LTM CSC MAC CE

        - Alt. 1-3: the first symbol of earliest CSI-RS resource after 3 msec of the last symbol of HARQ-ACK information for the LTM CSC MAC CE

      - End point:

        - Alt. 2: the last symbol of the uplink resource carrying the CSI report

      - Start and end point:

        - Alt. 3: having two CPU counting are applied:

          - CPU counter #1: Counting for each CSI measurement occasion starting from the first symbol of the earliest one of each CSI-RS/SSB resource for channel measurement and ends at the symbol ‘C1+∆1’, where C1 is the last symbol of last CSI-RS resource.

          - CPU counter #2: countering for a first PUSCH occasion. Assuming the last symbol ‘L’ of first PUSCH, the CPU counter #2 starts from ‘L-∆2’ symbol and ends at the last PUSCH symbol ‘L’.

    - FFS: whether or how to share the UE capability for CPU with legacy operation

      - This can be discussed under UE capability session

      - FL note: FL thinks there is no other CSI process during LTM. This FFS may be unnecessary

[Comments to 5-10-v1]

| Company | Comment |  |
| --- | --- | --- |
| Fujitsu | Support Option 1. CPU is defined for the number of simultaneous CSI processing based on the UE capability. Besides, the UE focuses on only the cell switch procedure after reception of CSC. Hence, we think that defining CPU after CSC is not so meaningful. |  |
| vivo | Support Option-1. |  |
| InterDigital | Support Option 1. |  |
| Nokia | We prefer to have consistent specification across all scenarios, which is why we support Option 2. Even if Option 1 is chosen, it should be clarified that the UE does not consume any CPUs for early CSI after the measurement report is sent, effectively defining an end point, which is more or less Option 2. |  |
| Qualcomm | Proposal 5-9-v1: We support Option-1. Once the LTM CSC is triggered, the UE will stop CSI processing for the serving cell and will concentrate its resources for target cell CSI processing. The purpose of the CPU occupancy rule is to facilitate the sharing of limited resources (i.e., CPUs) across parallel CSI processes. Therefore, for target cell CSI processing, since only one CSI processing is involved, a CPU occupancy rule does not need to be defined. |  |
| Ericsson | Support Option 2: It is an aperiodic CSI report triggered by LTM CSC MAC CE and reported in the first UL resource. CPU occupancy should be defined for this duration. (Similar to legacy AP CSI-RS where CPU occupancy is defined from the trigger to report.) We recognize that it may not be very useful to manage CPUs for the reasons given by e.g. Fujitsu, but we still think that we should keep the principle from legacy AP CSI-RS, it is a clean design. |  |
| Google | Support Option 2 with similar views as Nokia and Ericsson. |  |
| NEC | We have the same view as Ericsson, and support Option 2. |  |
| Samsung | We support Option 2. As mentioned in our earlier comments, the OCPU would start from when the LTM CSI report is triggered by the CSC – this is similar to legacy AP CSI reporting the only difference is the triggering medium/mechanism. |  |
| Spreadtrum | Support Option 2 and prefer Alt 1-1 and Alt 2. In legacy spec, for an aperiodic CSI report with CSI-ReportConfig with higher layer parameter reportQuantity not set to 'none', the aperiodic CSI report occupies CPU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report. In Rel-19 LTM, the CSI report only can be sent to target cell after the reception of LTM CSC MAC CE. Considering that the LTM CSI report is similar to a gNB controlled aperiodic CSI report with CSI-ReportConfig with higher layer parameter reportQuantity not set to 'none', the similar principle can be used for CSI computation time. That is to say, the LTM CSI report occupies CPU(s) from the first symbol after the trigger signaling of the LTM CSI report, e.g. LTM CSC MAC CE, until the last symbol of the uplink channel resource carrying the report in target cell. |  |
| Lenovo | As we discussed that early CSI acquisition is similar with the legacy aperiodic CSI reporting, thus the CPU mechanism for aperiodic CSI reporting can be used for early CSI acquisition for LTM by taking CSC as the ‘DCI triggering the aperiodic CSI reporting’. Thus, we support Alt. 1-2+Alt.2. |  |
| ETRI | Support Option 1. After the reception of the CSC, the UE is required to perform CSI measurement only for the CSI-RS resources associated with the target cell. Since the computational burden on the UE is significantly reduced, there is no strong need to explicitly define CPU after CSC. |  |
| CATT | Option 1 is preferred. As mentioned by FL, there is no other CSI process being conducted during LTM procedure. |
| Xiaomi | Prefer Alt 1-1 and Alt 2 in Option 2. Refer to the legacy definition of aperiodic CSI report, LTM CSC MAC CE can be considered as the DCI triggering the CSI report. |  |
| LG | Support Option 1. Because it is after receiving CSC command, there is no other CSI occupying procedure. Whole CPU could be assigned to CSI acquisition and no need to be controlled. |  |
| NTT DOCOMO | Support Option-2. |  |
| Huawei, HiSilicion | Support option 2. And Alt 1-3 and Alt 2. |  |
| ZTE | Support Option 2 and the same reason as in FL proposal 5-9-v1. |  |

[FL proposal 5-10-v2]

For the LTM CSI acquisition after CSC until the completion of LTM procedure

  - Option 1: CPU is not defined, Fujitsu, vivo, IDC, Qualcomm, ETRI, CATT, LG, Nokia, ZTE, Ericsson, MediaTek

    - FL note: no other CSI process than LTM CSI acquisition is conducted during LTM procedure and neither of serving cell and candidate cell can manage the CPU during this transient period

  - Option 2: CPU is defined, Google, NEC, Samsung, Spreadtrum, Lenovo, Xiaomi, DOCOMO, Huawei

    - For measurement performed before receiving CSC MAC-CE, ‘OCPU=N2’ where ‘N2’ is the total number of CSI-RS resources in the CSI-RS resource set of the candidate cell in the CSC MAC-CE

    - CPU occupancy time is defined as

      - Start point:

        - Alt. 1-1: the first symbol after the PDSCH carrying the LTM CSC MAC CE

        - Alt. 1-2: the end of the PUCCH with HARQ-ACK corresponding to the PDSCH carrying the LTM CSC MAC CE

        - Alt. 1-3: the first symbol of earliest CSI-RS resource after 3 msec of the last symbol of HARQ-ACK information for the LTM CSC MAC CE

      - End point:

        - Alt. 2: the last symbol of the uplink resource carrying the CSI report

      - Start and end point:

        - Alt. 3: having two CPU counting are applied:

          - CPU counter #1: Counting for each CSI measurement occasion starting from the first symbol of the earliest one of each CSI-RS/SSB resource for channel measurement and ends at the symbol ‘C1+∆1’, where C1 is the last symbol of last CSI-RS resource.

          - CPU counter #2: countering for a first PUSCH occasion. Assuming the last symbol ‘L’ of first PUSCH, the CPU counter #2 starts from ‘L-∆2’ symbol and ends at the last PUSCH symbol ‘L’.

    - FFS: whether or how to share the UE capability for CPU with legacy operation

      - This can be discussed under UE capability session

      - FL note: FL thinks there is no other CSI process during LTM. This FFS may be unnecessary

[To Chair: link to the list of online proposals]

[Conclusion]

The following conclusion was made during the 2nd online session on Thursday.

Conclusion

For the LTM CSI acquisition after CSC until the completion of LTM procedure, CPU is not defined

With this agreement, the discussion of this section is closed

[Closed] Active CSI-RS ports/resources before CSC

[Summary of contributions]

The proposals from companies are summarized as follows:

Definition of Active CSI-RS ports/resources

  - Alt .1: CSI-RS resources for candidate cells provided by LTM-CSI-SSB-ResourceSet are counted as active.

  - Alt. 2: Periodic CSI-RS resources for candidate cells provided by LTM-CSI-SSB-ResourceSet should be counted as active when

    - there is an activated TCI state in the cell and,

    - they are associated with an active CSI report configuration.

  - Alt.3: Semi-persistent CSI-RS resources for candidate cells provided by LTM-CSI-SSB-ResourceSet should be counted as active when

    - there is an activated TCI state in the cell,

    - the semi-persistent CSI-RS resources is activated, and

    - they are associated with an active CSI report configuration.

  - Alt.4: Active CSI-RS ports/resources for candidate cells are not counted before CSC

  - Note alternatives 1, 2, and 3 are applicable to UEs capable of CSI measurement before CSC

UE Capability

  - For CSI measurement before/after cell switch command, active CSI-RS resource for LTM should be separately counted from serving cells.

[FL observation]

Similar to CPU, the management of active CSI-RS ports/resources may not be meaningful if “UE may measure” means that the CSI measurement before CSC is up to UE implementation.

Since the input from companies are not enough, FL would like to start the discussion from collecting the comment from companies. Note that no RRC impact is foreseen for this issue.

[FL proposal 5-11-v1]

Companies are encouraged to provide their views on the following FL proposal.

Definition of Active CSI-RS ports/resources before CSC

  - Alt .1: CSI-RS resources for candidate cells provided by LTM-CSI-SSB-ResourceSet are counted as active based on the legacy rule.

    - Periodic CSI-RS

      - between RRC(re)configuration and RRC Release

    - Semi-persistent CSI-RS

      - between SP CSI-RS activation and deactivation

  - Alt. 2: CSI-RS resources for candidate cells provided by LTM-CSI-SSB-ResourceSet should be counted as active based on the new rule below

    - Periodic CSI-RS

      - there is an activated TCI state in the cell and,

      - they are associated with an active CSI report configuration.

    - Semi-persistent CSI-RS

      - there is an activated TCI state in the cell,

      - the semi-persistent CSI-RS resources is activated, and

      - they are associated with an active CSI report configuration.

  - Alt.3: Active CSI-RS ports/resources for candidate cells are not counted before CSC

  - Note alternatives 1, 2, and 3 are applicable to UEs capable of CSI measurement before CSC

UE Capability

  - For CSI measurement before cell switch command, active CSI-RS resource for LTM should be separately counted from serving cells.

[To Chair: link to the list of online proposals]

[Comments to 5-11-v1]

| Company | Comment |  |
| --- | --- | --- |
| Fujitsu | Support Alt.2. For ‘before CSC’, the UE ‘may’ measure the CSI, and thus the definition of the active CSI-RS resources/ports should be aligned with that of CPU as described in section 5.5.9. For example, the active CSI-RS resources/ports can be defined as below. - Periodic CSI-RS - Start point: after reception of the subset indication - End point: after reception of the CSC MAC CE - Semi persistent CSI-RS - Start point: after reception of the SP CSI-RS activation MAC CE - End point: earlier one among the SP CSI-RS deactivation MAC CE or CSC MAC CE We also support that the separate UE capability for the number of active CSI-RS resources/ports apart from that of the legacy UE. |  |
| vivo | Support Alt.2. |  |
| InterDigital | Support Alt.2. |  |
| Nokia | As mentioned above regarding CPU usage, it is important to define the timeline for which/when CSI-RSs are considered active for CSI acquisition. Whether this timeline should be linked to the activated TCI states depends on the agreement reached in Section 5.5.6. Otherwise, the behavior remains unclear and could lead to inconsistent UE implementations. We can support the Alt2 variation provided by Fujitsu, but this detail can be discussed further. |  |
| Ericsson | Support Alt.2. Whether Alt.1 or Alt 2 should be adapted should reflect on what is agreed for subset indication. If UE shall measure on indicated subset (Proposal 5-6-v1, Alt 2-1), then it should be Alt 2. If UE may measure on indicated subset (Proposal 5-6-v1, Alt 2-2), then Alt 3. |  |
| Samsung | Postpone the corresponding discussions after “subset” indication before CSC is clear. As discussed in our earlier comments related to “subset” indication before CSC, we still need to specify/clarify UE behaviors of CSI-RS measurement when some candidate cells are with activated TCI state(s) while some candidate cells for CSI acquisition do not have activated TCI state(s): - In one case, the UE only measures CSI-RSs on candidate cells with TCI state(s) activation; whether the UE would measure all the CSI-RSs configured for the corresponding candidate cells or only the CSI-RSs specific to the activated TCI states for the candidate cells is up to UE - In another case, the UE only measurements CSI-RS specific to activated TCI state(s) for the corresponding candidate cells; for candidate cells that are not with activated TCI state(s), the UE would measure all the corresponding CSI-RSs |  |
| Spreadtrum | Support reusing the starting time as legacy definition for P-/SP-CSI-RS. The ending time also can be modified as follows - the ending time for target cell is: at the end of the uplink channel resource containing the CSI report; the ending time for the candidate cell other than target cell is: when the PDSCH carrying the LTM CSC is received or the PUCCH with HARQ-ACK corresponding to the PDSCH carrying the LTM CSC is transmitted. |  |
| Lenovo | Before CSC, we understand only the subset of selected CSI-RS resources for CSI measurement are considered as activated. Thus, before discuss this issue, we should first fix the CSI-RS resource subset selection issue. |  |
| ETRI | Support Alt.2. |  |
| CATT |  | Support Alt .2. |
| NTT DOCOMO | Support Alt.2.in principle. |  |
| ZTE | Support Alt.2. |  |

[FL proposal 5-11-v2]

For the counting of active CSI-RS ports/resources before CSC,

  - Option 1: Counting of active CSI-RS ports/resources in a given slot for candidate cells CSI acquisition are introduced

    - The UE capability for number of active CSI-RS ports/resources in a given slot for candidate cells CSI acquisition is separately defined from legacy (i.e. serving cells)

  - Option 2: Active CSI-RS ports/resources in a given slot for candidate cells CSI acquisition are excluded from counting toward the UE capability

If Option 1 is introduced,

  - Alt .1: CSI-RS resources for candidate cells are counted as active

    - Periodic CSI-RS

      - between RRC(re)configuration and RRC Release or until CSC

    - Semi-persistent CSI-RS

      - between SP CSI-RS activation and deactivation or until CSC

  - Alt. 2: CSI-RS resources for candidate cells should be counted as active

    - Periodic CSI-RS

      - there is an activated TCI state in the cell and,

      - they are associated with an active CSI report configuration.

    - Semi-persistent CSI-RS

      - there is an activated TCI state in the cell,

      - the semi-persistent CSI-RS resources is activated, and

      - they are associated with an active CSI report configuration.

[To Chair: link to the list of online proposals]

[Conclusion]

The following conclusion was made during the 2nd online discussion on Thursday.

Conclusion

For the counting of active CSI-RS ports/resources before CSC, no new UE capability is introduced. The total number of active CSI-RS ports/resources is limited by legacy capability report. The total number of active CSI-RS ports/resources is shared by serving cell and candidate cell(s).

With this, the discussion of this section is closed.

[Closed] Active CSI-RS ports/resources after CSC

[Summary of contributions]

Definition of active CSI-RS resources

  - Alt .1: Both periodic and semi-persistent CSI-RS resources for the CSI-RS resources associated with the target cell are counted as active

  - Alt. 2: A periodic CSI-RS resource for target cell provided by LTM-CSI-SSB-ResourceSet should be counted as active when

    - it is associated with the indicated TCI state in cell switch command and,

    - it is associated with an active CSI report configuration.

  - Alt 3: A semi-persistent CSI-RS resource for target cell provided by LTM-CSI-SSB-ResourceSet should be counted as active when

    - it is activated,

    - it is associated with the indicated TCI state in cell switch command and,

    - it is associated with an active CSI report configuration.

  - Alt.4: Active CSI-RS ports/resources for candidate cells are not counted after CSC

UE Capability

  - For CSI measurement after cell switch command, active CSI-RS resource for LTM should be separately counted from serving cell.

[FL observation]

Similar to CPU, the management of active CSI-RS ports/resources may not be meaningful as no other CSI process other than early CSI acquisition is conducted during the LTM procedure.

Since the input from companies are not enough, FL would like to start the discussion from collecting the comment from companies. Note that no RRC impact is foreseen for this issue.

[FL proposal 5-12-v1]

Companies are encouraged to provide their views on the following FL proposal.

Definition of active CSI-RS resources after CSC

  - Alt .1: Both periodic and semi-persistent CSI-RS resource(s) for the CSI-RS resource(s) associated with the target cell are counted as active

  - Alt. 2: CSI-RS resource(s) for target cell provided by LTM-CSI-SSB-ResourceSet should be counted as active based on the new rule below

    - Periodic CSI-RS

      - it is associated with the indicated TCI state in cell switch command and,

      - it is associated with an active CSI report configuration.

    - Semi-persistent CSI-RS

      - it is activated,

      - it is associated with the indicated TCI state in cell switch command and,

      - it is associated with an active CSI report configuration.

  - Alt.3: Active CSI-RS ports/resources for candidate cells are not counted after CSC

UE Capability

  - For CSI measurement after cell switch command, active CSI-RS resource for LTM should be separately counted from serving cell.

    - FL note: FL thinks there is no other CSI process during LTM. This FFS may be unnecessary

[To Chair: link to the list of online proposals]

[Comments to 5-12-v1]

| Company | Comment |  |
| --- | --- | --- |
| Fujitsu | Support Alt 3. Similar reason to CPU definition after CSC, we don’t need to define the number of active CSI-RS resources/ports after CSC. |  |
| vivo | Support Alt.3. |  |
| InterDigital | Support Alt.3. |  |
| Nokia | As commented above for the CPU discussion, for this scenario the important aspect is the end point, i.e., from which point the CSI-RSs for early CSI can be assumed not active – therefore we should discuss the ending point. Similar to the previous discussion, whether this timeline should be linked to the indicated TCI states depends on the agreement reached in Section 5.5.7. |  |
| Ericsson | Support Alt 2. Even if it’s just in the target cell where there are active resources/ports, so the UE capability for maximum number of resources/ports may not be an issue, it is a clean design to count them. |  |
| Samsung | Do not support the proposal in its current form. As also commented by companies, some of the alternatives depends on outcome of whether to support subset indication after CSC, which is hypothetical. We prefer to further discuss this issue after some prerequisite issues have been addressed. |  |
| Spreadtrum | For UE only supporting CSI measurement after the reception of LTM CSC, we think the legacy definition can be modified as follows - the starting time is: the end of the PDSCH carrying the LTM CSC or the end of the PUCCH with HARQ-ACK corresponding to the PDSCH carrying the LTM CSC, the ending time is: at the end of the uplink channel resource containing the CSI report. |  |
| Lenovo | Support Alt.2. |  |
| ETRI | Support Alt.3. |  |
| CATT |  | Support Alt.3. There is no other CSI process being conducted during LTM procedure. The definition of active CSI-RS resources are not necessary. |
| NTT DOCOMO | Support Alt.2. |  |
| ZTE | Support Alt1, it can be further discussed till there is further conclusion on determination of CSI-RS resource before or after CSC |  |

[FL proposal 5-12-v2]

Definition of active CSI-RS resources after CSC until the completion of LTM procedure

  - Alt .1: Both periodic and semi-persistent CSI-RS resource(s) for the CSI-RS resource(s) associated with the target cell are counted as active

  - Alt. 2: CSI-RS resource(s) for target cell provided by LTM-CSI-SSB-ResourceSet should be counted as active based on the new rule below

    - Periodic CSI-RS

      - it is associated with the indicated TCI state in cell switch command and,

      - it is associated with an active CSI report configuration.

    - Semi-persistent CSI-RS

      - it is activated,

      - it is associated with the indicated TCI state in cell switch command and,

      - it is associated with an active CSI report configuration.

  - Alt.3: Active CSI-RS ports/resources for the candidate cells including target cell are not defined after CSC until the completion of LTM procedure

    - FL note: no other CSI process than is conducted during LTM procedure and neither of serving cell and candidate cell can manage the CPU during this transient period

UE Capability

  - For CSI measurement after cell switch command, active CSI-RS resource for LTM should be separately counted from serving cell.

    - FL note: FL thinks there is no other CSI process during LTM. This FFS may be unnecessary

[To Chair: link to the list of online proposals]

[Conclusion]

The following conclusion was made during the 2nd online discussion on Thursday.

Conclusion

Definition of active CSI-RS resources after CSC until the completion of LTM procedure:

- Active CSI-RS ports/resources for the candidate cells including target cell are not defined after CSC until the completion of LTM procedure

With this, the discussion of this section is closed.

[Closed] Other issues

[Summary of contributions]

LG:

  - Discuss CSI reference resource and UE assumption for CQI/PMI/RI calculation to enable CSI acquisition for LTM.

  - FL view: Even though this issue has been proposed for some meeting, no other companies discuss this issue in their contributions. FL suggestion is put on hold until more input is available.

[Conclusion]

The discussion of this section is closed without FL proposal.

Conditional intra-CU LTM

[Closed] RAN1 spec impact of Conditional intra-CU LTM

[Agreements in the previous meetings]

None

[Summary of contributions]

Vivo

  - Activated Candidate TCI state(s), other than the TCI state associated with the triggered beam, should be deactivated upon CLTM procedure being triggered

CATT

  - No need to define beam application time for NR Rel-19 RACH-less C-LTM

Sharp

  - How to determine the subcarrier spacing for an amount of time alignment, NTA=TA∙16∙642μ, may require further discussion in the case that the timing advance command is received in  the LTM Candidate Timing Advance Command MAC CE for conditional LTM.

    - For example, NTA is relative to the SCS of the first uplink transmission from the UE after the UE switches to the target cell during CLTM.

[FL observation]

Based on the contribution from companies, FL identifies no issue to immediately resolve in this meeting. With this, the discussion of this section is closed without any FL proposal.

[Comments if any]

| Company | Comment |
| --- | --- |
| Sharp | We think this issue should be resolved as LTM Candidate Timing Advance Command MAC CE for conditional LTM is already introduced, how to determine the subcarrier spacing for an amount of time alignment for this Timing Advance Command needs to be defined. |