3GPP TSG-RAN WG1 Meeting #121	R1- 2503994

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	9.8.2

Source:	NVIDIA

Title:	Channel model adaptation of TR 38.901 for 7-24 GHz

Document for:	Discussion

# 1	Introduction

The approval of the Rel-19 work package marks the next wave of 5G Advanced evolution [1]. The package includes a study item on channel modelling enhancements for 7-24GHz for NR, and the study item description can be found in [2].

The study item on channel modelling enhancements for 7-24 GHz for NR consists of two main objectives:

- Validate using measurements the channel model of TR38.901 at least for 7-24 GHz

  - Note: Only stochastic channel model is considered for the validation.

  - Note: The validation may consider all existing scenarios: UMi-street canyon, UMa, Indoor-Office, RMa and Indoor-Factory.

- Adapt/extend as necessary the channel model of TR38.901 at least for 7-24 GHz, including at least the following aspects for applicable scenarios:

  - Near-field propagation (with consideration being given to consistency between near-field and far-field)

  - Spatial non-stationarity

At the previous RAN1 meeting, several agreements on channel model adaptation were made (see Appendix).

In this contribution, we discuss the remaining issues of channel model adaptation of TR 38.901 for 7-24 GHz.

# 2	Channel modeling for near-field propagation and spatial non-stationarity

Deterministic ray tracing is an essential method to simulate the wireless channel between transmitter and receiver locations with high precision. Specifically, it uses the ray-optic approximation to simulate the wireless channel between Tx and Rx locations. This method offers high precision in channel modeling by considering the propagation characteristics of individual electromagnetic rays.

Traditionally, ray tracing performs a simple point-to-point simulation to estimate the channel between the transmitter and receiver. While effective for small-scale scenarios, this approach becomes unrealistic for large-scale channel realizations, particularly in the context of massive MIMO systems.

In massive MIMO systems, antenna arrays may consist of a large number of elements, resulting in large array apertures. The inconsistent responses across array elements pose challenges for traditional channel modeling approaches, as they may not accurately capture the spatial variations in channel characteristics.

To accurately represent the near-field and spatial non-stationary effects in the massive MIMO channel, one approach involves generating path responses for each array element. This method simulates the ray tracing channel individually for every element within the array. By accounting for various element locations and real-world environmental factors, this technique can effectively capture the near-field and spatial non-stationary effects characteristics inherent in the massive MIMO channel.

Furthermore, performing ray tracing for each transmit-receive element pair of the arrays can be efficiently implemented by exploiting parallelization and hardware accelerators such as GPUs [3].

In particular, for near-field channel, it is necessary to model the antenna element-wise channel parameters of direct path between TRP and UE, determined by the antenna element locations of both TRP and UE. Similarly, it is necessary to model the antenna element-wise channel parameters of indirect path between TRP and UE by the cluster location-based approach, where cluster location is directly dropped and generated in a deterministic manner.

Proposal 1: The antenna element-wise channel parameters of the indirect path between TRP and UE should be determined by the cluster location-based approach, where cluster location is directly dropped and generated in a deterministic manner.

For the modelling of spatial non-stationarity, the physical blocker-based approach should be adopted, as it is a physics-based, deterministic modeling approach.

Proposal 2: For the modelling of spatial non-stationarity, the physical blocker-based approach should be adopted, as it is a physics-based, deterministic modeling approach.

For the modelling of spatial non-stationarity with physical blocker-based approach, additional blocker types need to be introduced, which can be generated with deterministic locations, shapes and sizes in an environment and used to model spatial non-stationarity across BSs and UEs in the environment.

Proposal 3: For the modelling of spatial non-stationarity with the physical blocker-based approach, blockers are generated with deterministic locations, shapes and sizes in an environment and used to model spatial non-stationarity across BSs and UEs in the environment.

# Conclusion

In the previous sections, we discuss general aspects of channel model adaptation of TR 38.901 for 7-24 GHz. Based on the discussion in the previous sections we propose the following:

Proposal 1: The antenna element-wise channel parameters of the indirect path between TRP and UE should be determined by the cluster location-based approach, where cluster location is directly dropped and generated in a deterministic manner.

Proposal 2: For the modelling of spatial non-stationarity, the physical blocker-based approach should be adopted, as it is a physics-based, deterministic modeling approach.

Proposal 3: For the modelling of spatial non-stationarity with the physical blocker-based approach, blockers are generated with deterministic locations, shapes and sizes in an environment and used to model spatial non-stationarity across BSs and UEs in the environment.

# Reference

X. Lin, “The Bridge Toward 6G: 5G-Advanced Evolution in 3GPP Release 19,” arXiv preprint arXiv:2312.15174, December 2023.

RP-234018, “Study on channel modelling enhancements for 7-24GHz for NR,” 3GPP TSG RAN #102, December 2023.

S. Kasdorf, B. Troksa, C. Key, J. Harmon, S. Pasricha and B. M. Notaroš, “Parallel GPU Optimization of the Shooting and Bouncing Ray Tracing Methodology for Propagation Modeling,” IEEE Transactions on Antennas and Propagation, vol. 72, no. 1, pp. 174-182, Jan. 2024.

# Appendix

## A.1	RAN1#120bis agreements

Agreement

For the modelling of spatial non-stationarity, if the unified visible probability and visibility region based approach is adopted, for the Step 1 (whether a cluster is impacted by SNS) is determined as:

- For the nth UE, the Prn is generated using a truncated normal distribution within [0,1]

Prn=Nμ, σ2;     where 0≤Prn≤1.

  - FFS: the value of μ, σ.

    - Note: It should be fixed in RAN1#120-bis;

- For kth cluster of the nth user, if the xk,n is smaller than Prn, the cluster is impacted by the SNS, where the xk,n is defined as xk,n~U(0,1))

Agreement

For the modelling of spatial non-stationarity, if the unified visible probability and visibility region based approach is adopted, generate the visibility region as a rectangle randomly located at a corner of the antenna array with dimension a*b, where a~U(Vn*W,W), b=Vn*H*W/a, H is the antenna array height in vertical dimension and W is the antenna array width in horizontal dimension, Vn is calculated by

Vn=Α∙e-maxPn-Pnλ+Β+δ.

where A=0.15, B=0.45, λ=33, δ~N(0,0.0015) for UMa scenario, and n is a cluster index

- FFS: The value for other scenarios.

  - Note: It should be fixed in RAN1#120-bis;

- FFS: the details of randomly located VR

Agreement

For the modelling of spatial non-stationarity, if the unified visible probability and visibility region based approach is adopted,

- For clusters without SNS, the power attenuation factor αs,n is set to 1.

- For clusters with SNS, the power attenuation factor αs,n is generated by

αs,n=1                             ,xs-x0,n≤a,ys-y0,n≤be-xs-xa,nDn∙C        ,xs-x0,n>a,ys-y0,n≤b  e-ys-yb,nDn∙C        ,xs-x0,n≤a,ys-y0,n>b e-xs-xa,n2+ys-yb,n2Dn∙C                          ,others

where, Dn=xA,n-xa,n2+yB,n-yb,n2 , xA,n, yB,n is the coordinate of the other corner of the antenna array on the diagonal with reference corner x0,n, y0,n, and xa,n, yb,n is the coordinate of the other corner of the rectangular VR on the diagonal with reference corner x0,n, y0,n. The xs, ys denotes the coordinate of the antenna element out of VR region. C is the roll-off factor between the visible and invisible regions.

- FFS: The value of C.

  - Note: It should be fixed in RAN1#120-bis;

Agreement

For the modelling of spatial non-stationarity at UE side, introduce 3 sets of fixed attenuation values, each set corresponding to a range of frequency,

| Antenna index | Power attenuation (dB) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | One hand grip |  |  | Dual hand grip |  |  | Head and one hand grip |  |  |
|  | Below 1 GHz | (1-8.4) GHz | 14.5-15.5 GHz | Below 1 GHz | (1-8.4) GHz | 14.5-15.5 GHz | Below 1 GHz | (1-8.4) GHz | 14.5-15.5 GHz |
| 1 | - | 0.7 | 1.0 | - | 11.0 | 3.8 | - | 3.7 | 4.0 |
| 2 | - | 4.1 | 2.4 | - | 1.1 | 3.8 | - | 4.1 | 3.7 |
| 3 | - | 3.3 | 3.8 | - | 10.5 | 3.8 | - | 4.3 | 3.3 |
| 4 | 13.6 | 7.2 | 3.6 | 5.8 | 5.6 | 2.6 | 15.1 | 7.8 | 3.3 |
| 5 | - | 10.8 | 3.8 | - | 1.5 | 1.0 | - | 11.7 | 3.3 |
| 6 | - | 9.1 | 2.4 | - | 1.4 | 1.0 | - | 10.1 | 3.7 |
| 7 | - | 0.7 | 1.0 | - | 1.3 | 1.0 | - | 2.9 | 4.0 |
| 8 | 2.4 | 0.6 | 1.0 | 5.6 | 6.2 | 2.6 | 4.9 | 4.2 | 4.0 |

Note: For below 1GHz, the model is only applicable for 2-antenna system with antenna location index 4 and 8

Note: The selection of number of antenna and antenna index will be determined as part of the assumption for simulation.

Agreement

For the modelling of spatial non-stationarity, if the unified visible probability and visibility region based approach is adopted, for the Step 1 (whether a cluster is impacted by SNS):

- In UMa, the distribution of Prn is determined as: μ=0.56, σ=0.20;

Agreement

For near-field channel, the scaling factor (sUE,n) used to generate the d2 is:

- For number of k1 non-direct paths, sUE,n=1.

- For number of k2 non-direct paths:

  - Option-1: sUE,n=1-sTRP,n;

    - Note 1: It’s assumed that only single non-specular interaction along with specular reflection(s) is considered in the propagation channel

    - Note 2: At least most of the sources of non-specular interaction are assumed to be a point scatterer or diffractions.

Agreement

For near-field channel, to generate the d1 for the k2 non-direct paths, the distribution of scaling factor sTRP,n is:

- For the UMi scenarios, a Beta distribution with α=1.53, β=1.42

- For the UMa scenarios, a Beta distribution with α=1.93, β=1.33

- For the InH scenarios, a Beta distribution with α=1.25, β=1.27

Conclusion

For near-field channel, to generate the d1 for k2 non-direct paths:

- No additional lower bound (i.e., dmin) of d1 is defined.

Agreement

In the Indoor office scenario for near-field channel, k1= 4.

Agreement

In the Indoor factory scenarios, to generate the d1 non-direct path for the near-field channel model:

- k1= 4;

- For the k2 non-direct paths, the distribution of scaling factor sTRP,n is a Beta distribution with α=1.38, β=1.26

Agreement

For the modelling of spatial non-stationarity, if the unified visible probability and visibility region based approach is adopted, to generate the power attenuation factor αs,n for clusters with SNS, the value of C (i.e., the roll-off factor between the visible and invisible regions) is:

- C= 13 for UMa.

Agreement

For the modelling of spatial non-stationarity, the visibility region, i.e., a rectangle, is randomly located at a corner of the antenna array by following approach:

- The location of the VR is jointly determined by following conditions:

  - The VR is located at either left or right part in horizontal domain of antenna array with equal probability of 1/2.

  - The probability of VR at the upper part of antenna array is [0.8] and the probability of VR at the lower part of antenna array is [0.2];

Agreement

For the calibration parameters for the near-field channel,

| Parameter |  | Values |
| --- | --- | --- |
| Scenarios |  | UMi-street Canyon, Indoor-office (open office) |
| Carrier Frequency |  | 7 GHz (Optional) 15 GHz |
| Bandwidth |  | 20 MHz |
| BS antenna height |  | UMi-street Canyon: 10m Indoor-office: 3m |
| BS antenna downtilt |  | 102 degrees for UMi-street Canyon 180 degrees for indoor (i.e., array panel downward facing) |
| BS Tx power |  | 44 dBm for UMi-Street Canyon 24 dBm for Indoor |
| BS antenna configurations and port mapping |  | For UMi scenario: Config 1 for 7GHz - (M, N, P, Mg, Ng; MP, NP) = (24, 32, 2, 1, 1; 8, 32), dH = 0.5λ, dV = 0.7λ; - (Optional) (M, N, P, Mg, Ng; MP, NP) = (64, 16, 2, 1, 1; 16, 16), dH = dV = 0.5λ Config 2 for 15GHz - (M, N, P, Mg, Ng; MP, NP) = (64, 16, 2, 1, 1; 8, 16), dH = dV = 0.5λ For Indoor-office scenario for 7GHz: - (M, N, P, Mg, Ng; MP, NP) = (8, 24, 2, 1, 1; 8, 8), dH = dV = 0.5λ - (optional) (M, N, P, Mg, Ng; MP, NP) = (64, 16, 2, 1, 1; 16, 16), dH = dV = 0.5λ Mp and Np are the number of vertical, horizontal TXRUs within a panel and polarization |
| BS Polarized antenna modeling |  | Model-2 in Clause 7.3.2 of TR38.901 |
| UE Location | Outdoor/indoor | UMi-street Canyon: 100% outdoor Indoor-office: 100% indoor |
|  | LOS/NLOS | 100% LOS |
|  | UE antenna height | UMi-street Canyon: 1.5m Indoor-office: 1m |
| UT antenna configurations |  | Config A: Mg = Ng = 1, M = 2, N = 1, P = 2, dH = dV = 0.5λ Config B: 4 antenna port with single/linear polarization for calibration based on handheld device antenna model using candidate antenna locations (1,7,3,5) as described in Clause 7.3 |
| UT antenna pattern |  | Isotropic |
| UT Polarized antenna modelling |  | For UT Config A, Model-2 in Clause 7.3.2 of TR38.901 For UT Config B, following the Clause 7.3.2 of TR38.901 |
| Applicability of Nearfield Modeling |  | Component of NF channel to be considered: - Phase of Direct path - Phase of Non-direct path, applied only at the BS side (optionally) |
| Calibration method |  | Drop multiple users in the multiple cells at the fixed horizontal distance Z from the BS, and collect the metric for a each distance Z For UMi-street Canyon: Z = 10, 30, 50, 100 m (optional) Z = 20, 40, 80 m For Indoor: Z = 0, 2, 6, 10 m (optional) Z = 4, 8 m |
| Metric |  | CDF of the ratio between the 2nd, 3rd ,4th, ..., xth (smallest) PRB singular value and the 1st PRB (largest) singular value (serving cell) at t=0 plotted in 10*log10 scale. - Note-1: The PRB singular values of a PRB are the eigenvalues of the mean covariance matrix in the PRB. - Note-2: The value of x is the minimum value of (number of BS antenna ports, number of UT antenna ports) |

Agreement

The following assumption is used for UE side SNS calibration:

Agreement

The following assumption is used for BS side SNS calibration: