3GPP TSG-RAN WG1 Meeting #121	R1- 2503995

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	9.1.4.1

Source:	NVIDIA

Title:	Additional study on AI-enabled CSI compression and other aspects of AI model       and data

Document for:	Discussion

# 1	Introduction

The approval of the Rel-19 work package marks the next wave of 5G Advanced evolution [1]. The package includes a work item on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface and the latest version of the work item description can be found in [2].

The work item consists of multiple objectives, including further study on AI/ML-enabled CSI compression:

  - For CSI compression (two-sided model), further study ways to:

    - Improve trade-off between performance and complexity/overhead

      - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc.

    - Alleviate/resolve issues related to inter-vendor training collaboration.

while addressing necessary specification impact analysis, as well as other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843.

In addition, the work item consists of multiple objectives, including further study on other aspects of AI model and data:

- Necessity and details of model Identification concept and procedure in the context of LCM for two-sided models [RAN2/RAN1]

- CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]:

  - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection

  - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods

- Model transfer/delivery [RAN2/RAN1]:

  - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study

At the previous RAN1 meeting, several agreements were made on additional study for AI-enabled CSI compression as well as other aspects of AI model and data, for example “Processing Unit” (see Appendix).

In this contribution, we discuss the remaining issues of additional study on AI-enabled CSI compression and other aspects of AI model and data.

# 2	Background on CSI compression

AI/ML techniques, such as autoencoders, can be used in CSI compression and feedback. The basic principle of an autoencoder is to take an input, perform non-linear compression by an encoder to a lower-dimensional latent representation, and decompress the latent representation by a decoder to a defined target. In the context of CSI compression, the UE’s estimated CSI is taken as the input to the encoder. The output of the UE’s encoder (i.e., the latent representation of the channel in a quantized form) is transmitted over the air interface (Uu) to the BS. The BS uses a matching decoder to reconstruct the channel state. Hence, the autoencoder model is divided between the UE and BS, creating an inter-node dependency for model training and model management.

Using autoencoders for CSI compression is in line with the agreement that for the evaluation of the AI/ML based CSI compression sub use cases, a two-sided model is considered as a starting point, including an AI/ML-based CSI generation part to generate the CSI feedback information and an AI/ML-based CSI reconstruction part which is used to reconstruct the CSI from the received CSI feedback information. At least for inference, the CSI generation part is located at the UE side, and the CSI reconstruction part is located at the gNB side.

The autoencoders can be used to compress either the raw channel matrix estimated by UE or the eigenvector(s) of the raw channel matrix estimated by UE, etc. Raw channel matrix-based CSI feedback provides gNB with the most flexibility in determining downlink transmission, though it deviates from the existing codebook-based CSI feedback. In contrast, the channel eigenvector-based CSI feedback is more in line with the conventional codebook-based CSI feedback methods, but it does not provide as much flexibility as the raw channel matrix-based CSI feedback. Both types of CSI feedback schemes are of interest and deserve study.

# 3	Performance study for CSI compression

To evaluate AI/ML based algorithms for CSI compression, datasets are needed. Both real data and synthetic data can be used to develop and evaluate AI/ML based algorithms. 3GPP has well established simulation methodology, which can be used to generate synthetic data. CSI enhancement would be most valuable in the scenarios where there is high-capacity demand. Therefore, the evaluation could focus on UMi-street canyon and UMa scenarios.

In general, the statistical models in TR 38.901 can be used as baseline for link and system level evaluations of AI/ML based algorithms for CSI feedback enhancement. However, the Rel-18 study shows that using data generated from stochastic channel models is not sufficient to demonstrate the performance gain of CSI compression compared to non-AI/ML based algorithms.

Observation 1: The Rel-18 study shows that using data generated from stochastic channel models is not sufficient to demonstrate the performance gain of CSI compression compared to non-AI/ML based algorithms.

To improve the performance gain, site-specific AI/ML models for CSI compression should be considered. Site-specific optimization refers to the continuous process of fine-tuning network configurations on a per-site basis. The goal is to ensure optimal performance for individual cell-sites in terms of key performance indicators (KPIs) like coverage, capacity, quality-of-service (QoS), quality of experience (QoE) and energy efficiency. Which attributes are tuneable and important for site-specific optimization depends on the RAN deployment architecture, as well as the capabilities of cell-site specific network elements. As cellular networks continue to evolve with the emergence and deployment of innovative technologies beyond 5G, site-specific RAN optimization is poised to become more sophisticated due to the increasing number of frequency bands, demanding KPIs (e.g., higher data rates, ultra-low latency, wider coverage, lower energy consumption, and high-accuracy positioning) and innovation of more advanced antenna technologies.

Proposal 1: Site-specific AI/ML models for CSI compression should be considered to improve performance gain.

It was agreed that for the evaluation of AI/ML-based CSI compression using localized models in Release 19, consider the following options as a starting point to model the spatial correlation in the dataset for a local region:

- Option 1: The dataset is derived from UEs dropped within the local region, with spatial consistency modelling as per TR 38.901.

  - E.g., Dropped in a specific cell or within a specific boundary.

- Option 2: By using a scenario/configuration specific to the local region.

  - E.g., Indoor-outdoor ratio, LOS-NLOS ratio, TXRU mapping, etc.

It was emphasized that while modelling the spatial correlation, companies should ensure that the dataset distribution also correctly captures the decorrelation due to temporal variations in the channel.

Nonetheless, to enable the development and evaluation of site-specific AI/ML models, it is beneficial to define a common reference scenario with site specificity.

There are two options for defining a common reference scenario with site specificity:

- Option 1: Real-scenario map that is a virtual representation of a real area on earth.

- Option 2: Synthetic-scenario map that is artificially constructed to mimic a certain environment such as urban macro, rural macro, indoor office, and indoor factory.

As a starting point of the discussion, 3GPP may consider the existing scenarios, e.g., those defined by METIS [3]. Specifically, for urban scenarios, the Madrid grid developed by the METIS project can be considered.

Figure 1: Madrid grid for Urban Micro and Urban Macro. Source: METIS D6.1 [3].

Once a scenario is defined, ray tracing can be used to generate data for the development and evaluation of site-specific AI/ML models for CSI compression. Ray tracing is a rendering and simulation technique used in computer graphics, optics, and other fields to simulate the way rays of light or other radiation travel through a virtual environment. In the context of wireless communication and radio wave propagation, ray tracing is often employed to model and simulate the paths that EM waves take as they propagate through various materials and interact with surfaces and obstacles. It provides a deterministic and physics-based modelling approach that simulates the paths of individual rays of EM waves, considering reflections, refractions, diffractions, and other interactions with objects and surfaces. It offers high-resolution simulations, capturing the specific paths of rays and the effects of the surrounding environment, making it valuable for site-specific planning, antenna design, and network optimization in the field of wireless communication.

Proposal 2: With a common reference scenario with site specificity, ray tracing is used to generate channel data for the development and evaluation of site-specific AI/ML models for CSI compression.

# 4	Inter-vendor training collaboration

To alleviate / resolve the issues related to inter-vendor training collaboration of AI/ML-based CSI compression using two-sided model, several options deserve further study.

Option 1: Fully standardized reference model (structure + parameters)

Option 2: Standardized dataset

Option 3: Standardized reference model structure + Parameter exchange between NW-side and UE-side

Option 4: Standardized data / dataset format + Dataset exchange between NW-side and UE-side

Option 5: Standardized model format + Reference model exchange between NW-side and UE-side

Option 1 eliminates the inter-vendor collaboration complexity as it does not require bilateral collaboration between vendors. However, Option 1 may have limited performance in the field compared to Options 3, 4, and 5. Also, Option 1 may require high specification effort.

Similar to Option 1, Option 2 may have limited performance in the field compared to Options 3, 4, and 5, as well as requiring high specification effort. Option 2 may be deprioritized for inter-vendor training collaboration.

For Option 3, two variants may be considered. In Option 3a, parameters received at the UE or UE-side go through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., potential re-training, re-development of a different model, and/or offline testing. The parameter exchange originates from the NW-side and ends at the UE-side. Parameters exchanged from the NW-side to UE-side is either CSI generation (Option 3a-1) or reconstruction part (Option 3a-2) or both (Option 3a-3). In Option 3b, parameters received at the UE are directly used for inference at the UE without offline engineering, potentially with on-device operations. The method of exchanging is over the air-interface via model transfer/delivery Case z4. The parameter exchange is from NW to UE. Parameters exchanged from the NW-side to UE-side is CSI generation part.

For Option 4, dataset received at the UE or UE-side goes through offline engineering at the UE- side (e.g., UE-side OTT server), e.g., model training or offline testing. The dataset exchange originates from the NW-side and ends at the UE-side. The dataset exchanged from the NW-side to UE-side may consist of (target CSI, CSI feedback – Option 4-1), or (CSI feedback, reconstructed target CSI – Option 4-2), or (target CSI, CSI feedback, reconstructed target CSI – Option 4-3).

For Option 5, two variants may be considered. In Option 5a, model received at the UE or UE-side goes through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., potential re-training, re-development of a different model, and/or offline testing. The model exchange originates from the NW-side and ends at the UE-side. Model exchanged from the NW-side to UE-side is either CSI generation (Option 5a-1) or reconstruction part (Option 5a-2) or both (Option 5a-3). In Option 5b, model received at the UE are directly used for inference at the UE without offline engineering, potentially with on-device operations. The method of exchanging is over the air-interface via model transfer/delivery Case z4, assuming that the model structure is aligned based on offline inter-vendor collaboration. The model exchange is from NW to UE. Model exchanged from the NW-side to UE-side is CSI generation part.

For Options 3, 4, and 5, standardized signaling can be used for exchange of parameter, dataset, and model to alleviate the inter-vendor training collaboration complexity. Such exchanges have potential specification impacts, including exchange methods, format, contents, and additional information such as performance target and dataset information. Other potential specification impact areas include model pairing, UE capability, model scalability (e.g., payload sizes, antenna ports, bandwidth), rank and layer handling, quantization of feedback, and model structure details.

In the last RAN1 meeting (see Appendix), RAN1 concluded that both Direction A and Direction A are feasible.

- Direction A: Sharing parameters/reference model/dataset that enables UE-side offline engineering (Inter vendor collaboration option 3a/5a/4)

- Direction C: Fully standardized reference model(s) and parameters with specified CSI generation part and/or CSI reconstruction part (Inter vendor collaboration option 1)

Moreover, RAN1 also concluded that both sub-options 3a-1 (with and without target CSI sharing) and 4-1 mentioned above are feasible.

Further, RAN1 concluded based on the evaluation results that for both Direction C and Direction A Option 3a-1, scalable model structure specification is feasible, while for Direction C, scalable model parameters specification is feasible too.

Finally, RAN1 also concluded on ‘Issue 8’ in the last RAN1 meeting. Recall that in RAN1#118 meeting, it was agreed to consider whether to use 3GPP’s statistical channel model or field data for reference model(s) training (labelled as ‘Issue 8’) in the context of Direction C, fully standardized reference model(s) and parameters with specified CSI generation part and/or CSI reconstruction part (inter-vendor collaboration Option 1). In case of field data usage, the remaining issue would be to reach consensus related to the details of field data collection. In this context, RAN1 reached the following agreements in the last RAN1 meeting:

- For Direction C, the specified model should be trained using synthetic data.

- For Direction A inter-vendor collaboration option 3a-1, the specified model structure should be determined using synthetic data.

Even though RAN1 made some progress with Options 1, 3a-1 and 4-1, there has been no conclusion regarding the standardization feasibility of Options 2 and 5. As mentioned above, Option 2 may have limited performance gain in the field. Option 5, on the other hand, requires model structure alignment based on offline inter-vendor collaboration. For the sake of time, RAN 1 should focus on resolving the reaming issues for inter-vendor collaboration while deprioritizing Options 2 & 5 in Rel-19.

Proposal 3: RAN1 to deprioritize Options 2 & 5 in Rel-19 related to inter-vendor training collaboration for AI/ML-based CSI compression.

# 5	Performance monitoring

Traditionally, 3GPP defines requirements for testing equipment in controlled laboratory conditions prior to its field deployment. Generalization verification is an intricate task. In particular, the training of an AI/ML model can be tailored to encompass the entirety of conditions outlined in the standard, thus demonstrating superior performance during testing. However, the AI/ML model may be overfitted to the standardized setups, possibly compromising its robustness in real-world environments that are not aligned with the controlled conditions. Moreover, AI/ML models might need periodic updates even after they are deployed in live networks. These new considerations call for the implementation of performance monitoring mechanisms to detect non-compliance, as well as the formulation of new testing procedures to effectively validate the functionality of AI/ML-based features operating in the field.

For network-side performance monitoring, it may be based on the target CSI reported by the UE via enhanced Type-2 or alike codebook, or leverage SRS-based monitoring.

For UE-side performance monitoring, it may be based on the output of the CSI reconstruction model at the UE, or estimation of monitoring output such as intermediate KPI without reconstructing a target CSI. It may also leverage precoded RS (e.g., CSI-RS, DMRS) transmitted from network based on the output of the CSI reconstruction model. Additionally, the network may indicate to the UE the output of the CSI reconstruction model using enhanced Type-2 or alike codebook.

Proposal 4: RAN1 to study post-deployment performance monitoring mechanisms to detect performance degradation and non-compliance to guarantee satisfactory performance of AI/ML-based CSI compression in the field.

# 6	Physics-based propagation modeling for AI/ML data generation

Wireless networks are becoming increasingly heterogeneous, encompassing different inter-site distances, antenna array dimensions and makeup, radiated power per site, frequency bands, to name a few. Correspondingly, the wireless channel modeling needs to provide consistency and, above all, a correct representation of the frequency, spatial, and temporal correlations across base stations and devices. Achieving this without a propagation model grounded on the underlying physics of the scattering phenomena is simply unnatural, prone to modelling error and possibly a huge waste of effort for the industry. These considerations call for deterministic, physics-based modeling for wireless propagation, especially ray tracing.

Ray tracing is a rendering and simulation technique used in computer graphics, optics, and other fields to simulate the way rays of light or other radiation travel through a virtual or mathematical environment. In the context of wireless communication and radio wave propagation, ray tracing is often employed to model and simulate the paths that EM waves take as they propagate through various materials and interact with surfaces and obstacles. It provides a deterministic and physics-based modeling approach that simulates the paths of individual rays of EM waves, considering reflections, refractions, diffractions, and other interactions with objects and surfaces. It offers highly-detailed simulations, capturing the specific paths of rays and the effects of the surrounding environment, making it valuable for site-specific planning, antenna design, and network optimization in the field of wireless communication.

One can utilize the physics-based modelling for wireless propagation to generate a diverse dataset that simulates a wide range of radio wave propagation scenarios within the targeted environment. This dataset can include variations in terrain, building structures, material properties, and other relevant factors, providing accurate reference data for training the AI/ML models, including the ground truth against which predictions are evaluated.

Next, we consider a concrete use case study, where the data is channel impulse response and used as input to an AI/ML model for object localization. An indoor office environment was selected for evaluation.

- A first set of channel impulse response data was collected based on 3GPP stochastic channel model, and then a first AI/ML model was trained using the channel impulse response training data from stochastic channel model.

- A second set of channel impulse response data was collected based on ray tracing, and then a second AI/ML model was trained using the channel impulse response training data from ray tracing.

After training, both AI/ML models were tested using channel impulse response test data from ray tracing, assuming the physics-based ray tracing data represents the ground truth of the specific indoor environment.

The figure below shows the evaluation results. We can see that the test accuracy is poor for the AI/ML model trained on stochastic channel model data, compared to the test accuracy of the AI/ML model trained on ray tracing data. This implies that using data generated from stochastic channel models to train AI/ML models will lead to failure of the AI/ML models when deployed in real networks that are site-specific.

Figure 2: Comparison of stochastic channel model and ray tracing for AI/ML with sensing.

The key takeaway from this case study is that a propagation model, particularly ray tracing, grounded on the underlying physics of the scattering phenomena is indispensable for 5G-Advanced toward 6G.

Observation 2: Deterministic, physics-based modelling for wireless propagation, especially ray tracing, are essential for studying, evaluating, and developing AI/ML models in 5G-Advanced toward 6G.

# 7	Model identification and transfer/delivery

To enable and facilitate AI/ML for NR air interface, an AI/ML model needs to be created and then be managed during its entire lifecycle. For example, the AI/ML model needs to be validated, tested, deployed, etc. The AI/ML-enabled function also needs to be managed (e.g., deployment, configuration, and evaluation).

In RAN1#120 meeting, it was concluded that the study progress for model identification and model delivery /transfer are sufficient and new work/discussion can be triggered by LS from other working group(s)/TSG(s). The remaining scopes for discussion within agenda item 9.1.4.1 are two -fold, around: 1) Whether or not model identification is needed for CSI compression, and 2) Whether standardized solution(s) for model transfer/delivery are needed for CSI compression.

For model identification, the following options have been considered so far:

- MI-Option 1: Model identification with data collection related configuration(s) and/or indication(s)

- MI-Option 2: Model identification with dataset transfer

- MI-Option 3: Model identification in model transfer from NW to UE

- MI-Option 4. Model identification via standardization of reference models. (for CSI compression)

- MI-Option 5. Model identification via model monitoring

Examples have been given for MI-Option 1 and MI-Option 2, but not yet for MI-Option 3, MI-Option 4, and MI-Option 5.

Proposal 5: Describe examples for the following options to study their feasibility/necessity:

- MI-Option 3: Model identification in model transfer from NW to UE

- MI-Option 4: Model identification via standardization of reference models. (for CSI compression)

- MI-Option 5: Model identification via model monitoring

RAN1 further agreed to consider the following cases where different combinations of model delivery/transfer to UE, training location, and model delivery/transfer format are considered for UE-side models and UE-part of two-sided models.

- Case y, where model delivery (if needed) is over-the-top, model storage location is outside 3GPP network, and training location is UE-side / network-side / neutral site.

- Case z1, where model transfer is in proprietary format, model storage location is within 3GPP network, and training location is UE-side / neutral site.

- Case z2, where model transfer is in proprietary format, model storage location is within 3GPP network, and training location is network-side.

- Case z3, where model transfer is in open format, model storage location is within 3GPP network, and training location is UE-side / neutral site.

- Case z4, where model transfer is in open format of a known model structure at UE, model storage location is within 3GPP network, and training location is network-side.

- Case z5, where model transfer is in open format of an unknown model structure at UE, model storage location is within 3GPP network, and training location is network-side.

In Case z4, the “known model structure” includes known information about model type (e.g., Transformer, CNN, RNN), neural network parameters including number of layers, layer structure, layer size, connection between different layers, and model input/output related information. The network can transmit a first indication about the known model structure and a second indication about the transmitted parameters.

For various reasons, RAN1 agreed to deprioritize Cases z2, z3, and z5. But Cases y, z1, and z4 require further study.

Proposal 6: Conclude that there is a need to consider standardised solutions for transferring/delivering AI/ML models.

Proposal 7: Continue to study Cases y, z1 and z4 for transferring/delivering AI/ML models, including consideration of open format defined within 3GPP for Case z4.

# Conclusion

In the previous sections, we discuss general aspects of AI-enabled CSI compression as well as other aspects of AI model and data, and make the following observations:

Observation 1: The Rel-18 study shows that using data generated from stochastic channel models is not sufficient to demonstrate the performance gain of CSI compression compared to non-AI/ML based algorithms.

Observation 2: Deterministic, physics-based modelling for wireless propagation, especially ray tracing, are essential for studying, evaluating, and developing AI/ML models in 5G-Advanced toward 6G.

Based on the discussion in the previous sections we propose the following:

Proposal 1: Site-specific AI/ML models for CSI compression should be considered to improve performance gain.

Proposal 2: With a common reference scenario with site specificity, ray tracing is used to generate channel data for the development and evaluation of site-specific AI/ML models for CSI compression.

Proposal 3: RAN1 to deprioritize Options 2 & 5 in Rel-19 related to inter-vendor training collaboration for AI/ML-based CSI compression.

Proposal 4: RAN1 to study post-deployment performance monitoring mechanisms to detect performance degradation and non-compliance to guarantee satisfactory performance of AI/ML-based CSI compression in the field.

Proposal 5: Describe examples for the following options to study their feasibility/necessity:

- MI-Option 3: Model identification in model transfer from NW to UE

- MI-Option 4: Model identification via standardization of reference models. (for CSI compression)

- MI-Option 5: Model identification via model monitoring

Proposal 6: Conclude that there is a need to consider standardised solutions for transferring/delivering AI/ML models.

Proposal 7: Continue to study Cases y, z1 and z4 for transferring/delivering AI/ML models, including consideration of open format defined within 3GPP for Case z4.

# Reference

X. Lin, “The Bridge Toward 6G: 5G-Advanced Evolution in 3GPP Release 19,” arXiv preprint arXiv:2312.15174, December 2023.

RP-243245, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2,” 3GPP TSG RAN #106, December 2024.

METIS, “Deliverable D6.1 - Simulation guidelines,” April 2014.

# Appendix

## A.1	RAN1#120bis agreements on CSI Compression

Agreement

Introduce a dedicated AI/ML PU for AI/ML features for UE,

- The AI/ML PU is used at least for quantifying the simultaneous processing of multiple CSI reports subject to CSI-related AI/ML use case(s), e.g., CSI compression (if supported), CSI prediction, BM spatial prediction, BM temporal prediction.

Agreement

For inter-vendor-collaboration Options 3a-1 and 4-1 in Direction A, confirm SGCS and NMSE as the type of performance metric that may be used for the performance target shared as additional information along with the exchanged dataset or the model parameters.

- FFS: when to use SGCS, NMSE, and which one to use or both, and relationship with the inter-vender collaboration sub-options.

- FFS: details of the format of the performance target

  - Option 1: Average performance target, e.g. average SGCS and/or average NMSE

  - Option 2: distribution of the performance target, e.g., SGCS / NMSE for 5, 10, 20, 30 percentiles, etc.

- FFS: whether multiple performance targets should be exchanged for different configurations, such as antenna ports configuration, subband configuration and payload configuration, etc., along with each exchanged dataset or model parameters

Agreement

In Options 3a-1 and 4-1, the exchanged dataset or the model parameters can be associated with an ID for pairing related discussion, then

- The same ID can be used for UE to collect UE-side target CSI for UE-side training

- The same ID can be used for applicability inquiry and reporting

- The same ID can be used for inference configuration

- The same ID can be used for NW-side data collection

- FFS: whether ID/even same ID is needed for monitoring configuration

- FFS: where the ID is assigned

- Note: whether the purpose for pair will be specified will be discussed separately.

Agreement

In Direction C, the fully standardized reference model is associated with an ID for pairing related discussion, then

- The same ID can be used for UE to collect UE-side target CSI for UE-side training

- The same ID can be used for applicability inquiry and reporting

- The same ID can be used for inference configuration

- The same ID can be used for NW-side data collection

- FFS: whether ID/even same ID is needed for monitoring configuration

- FFS: where the ID is assigned or how the ID is specified

- Note: whether the purpose for pair will be specified will be discussed separately.

Agreement

For inter-vendor collaboration Direction C,

- Use standardized quantization codebook

For inter-vendor collaboration Direction A Options 4-1, 3a-1 (with or without NW-side target CSI sharing),

- Standardize configuration(s) of quantization codebook, e.g., scalar or vector quantization, segment size of VQ, codebook size.

  - FFS: applicability of the above for Case 2

  - Exchange quantization codebook of (the selected) standardized configuration(s) from NW-side to UE-side along with each exchanged dataset or model parameters.

- FFS: whether quantization codebook may be different across different payload size configurations

Agreement

It is clarified that, for previous agreement on standardize configuration(s) of quantization for inter-vendor collaboration Direction A Options 4-1, 3a-1 (with or without NW-side target CSI sharing), resolve the first FFS with the following clarifications:

- The standardized quantization configuration(s) refers to the configuration(s) related to the dimensionality related to the quantization operation (e.g., e.g., scalar or vector quantization, segment size of VQ, codebook size) and does not preclude the use of more advanced quantization algorithms (e.g., using past samples).

- Exchanged quantization codebook does not preclude the use of codebook parameterized by parameters, e.g. quantization range and step size determined by some parameterized formula.

Conclusion

- For Direction C, confirm that the specified model should be trained using synthetic data (answer to issue 8).

- For inter-vendor collaboration option 3a-1 of Direction A, confirm that the specified model structure should be determined using synthetic data.

Conclusion

- For Direction A Option 3a-1, based on evaluation results, RAN1 concludes that scalable model structure specification is feasible.

- For Direction C, based on evaluation results, RAN1 concludes that scalable model structure specification and model parameters specification is feasible.

  - Note: RAN4 feasibility is a separate study to be confirmed by RAN4.

Conclusion

- RAN1 concludes that both Direction A and Direction C are feasible.

- RAN1 concludes that both sub-option 4-1 and sub-option 3a-1 are feasible.

  - This includes both 3a-1 with and without target CSI sharing.

Conclusion

For NW-side monitoring with target CSI reporting

- Target CSI reporting via legacy CSI codebooks can be used for NW-side monitoring

- Target CSI reporting with CSI codebook enhancement via higher-resolution parameter combination may be beneficial for improving NW-side monitoring with additional cost of complexity and overhead at UE side.

Observation

For choice of token/feature dimension in the scalable model design, companies observe that there is small loss in SGCS for Case1 over Case2 under tokenization (Alt1, Alt3)

For layer1, under tokenization (Alt1),

- 8 sources [Qualcomm, CATT, Xiaomi, vivo, ZTE, Fujitsu, Samsung, Huawei] observe that average gain in SGCS of Case 1 over Case 2 is in the range of -3.34% to 0.33% with median value of -1.10%

- 2 sources [CATT, Xiaomi] observe average gain in SGCS of Case 1 (scalable structure) over benchmark [G1] is in the range of 2.04% to 7.84% with median value of 6.61%

- 3 sources [CATT,  Xiaomi, Ericsson] observe average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is in the range of 5.17% to 10.2% with median value of 10.1%

For layer1, under tokenization (Alt3),

- 1 source [vivo] observes average gain (%) in SGCS of Case 1 over Case 2 is in the range of -0.7% to 0.4% with median value of -0.3%

- 1 source [Ericsson] observes average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is in the range of 5.4% to 6%

For scalability over the feature dimension, companies observe that there is small loss in SGCS for Case1 over Case2 assuming (Alt1, Alt2, Alt1+Alt2)

For layer1, under (Alt1),

- 4 sources [Qualcomm, CATT, Xiaomi, Fujitsu ] observes average gain (%) in SGCS of Case 1 over Case 2 is in the range of -3.34% to -0.4% with median value of -2.08%

- 2 sources [CATT, Xiaomi] observe average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is in the range of 3% to 7.84% with median value of 6.71%

- 2 sources [CATT, Xiaomi] observe average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is in the range of 5.17% to 10.2% with median value of 10.1%

For layer1, under (Alt2),

- 7 sources [Qualcomm, Xiaomi, vivo, ZTE, Fujitsu, Samsung, Huawei] observe average gain in SGCS of Case 1 over Case 2 is in the range of -3.2% to 0.1% with median value of -0.85%

- 1 source [Xiaomi] observes gain in SGCS of Case 1 (scalable structure) over benchmark [G1] is 2.04%

- 1 source [Xiaomi] observes gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is 5.17%

For layer1, under (Alt1+Alt2),

- 1 source [Qualcomm] observes average gain (%) in SGCS of Case 1 over Case 2 is in the range of -3.3% to -2.9% with median value of -3.1%.

For scalability over the token dimension, companies observe that there is small loss in SGCS for Case1 over Case2 assuming (Alt1, Alt2)

For layer1, under (Alt1),

- 2 sources [Qualcomm, vivo] observe average gain in SGCS of Case 1 over Case 2 is in the range of -3.2% to 0.9% with median value of -1.5%.

For layer1, under (Alt2),

- 3 sources [CATT, vivo, ZTE ] observe average gain in SGCS of Case 1 over Case 2 is in the range of -3.34% to 0.4% with median value of -0.75%.

- 1 source [CATT] observes average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is in the range of 6.51% to 7.84% with median value of 6.87%

- 1 source [CATT] observes average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is in the range of 10.12% to 10.2% with median value of 10.1%

For scalability over the payload configurations, companies observe that there is small loss in SGCS for Case1 over Case2 assuming (Alt1, Alt2)

For layer1, under (Alt1),

- 4 sources [Qualcomm, CATT, Xiaomi, Huawei] observe average gain (%) in SGCS of Case 1 over Case 2 is in the range of -0.73% to -3.34% with median value of  -2.08%

- 2 sources [CATT, Xiaomi] observe average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is in the range of 3% to 7.84% with median value of 6.71%

- 2 sources [CATT, Xiaomi] observe average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is in the range of 5.17% to 10.2% with median value of 10.1%

For layer1, under (Alt2),

- 3 sources [Xiaomi, vivo, ZTE] observe average gain in SGCS of Case 1 over Case 2 is in the range of -3.2% to 0.4% with median value of  -0.85%

- 1 source [Xiaomi] observes average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is 2.04%

- 1 source [Xiaomi] observes average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is 5.17%

For scalability over feature dimension, token dimension and payload configuration jointly,

For tokenization Alt1, feature scalability Alt1, token scalability Alt2 and payload scalability Alt1,

- 1 source [CATT] observes average gain in SGCS of Case 1 over case 2 is in the range of -3.34%~2.03% with median value of -2.93%

- 1 source [CATT] observes average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is in the range of 6.51%~7.84%

- 1 source [CATT] observes average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is in the range of 10.12%~10.23%

For tokenization Alt1, feature scalability Alt2, token scalability Alt2, payload configuration Alt2,

- 2 sources [ZTE, vivo] observe average gain in SGCS of Case 1 over case 2 is in the range of -2.8%~1.2% with median value of -1.2%, under same parameter set.

- 1 source [vivo] observes average gain in SGCS of Case 1 over Case 2 is in the range of -0.1% ~ 0.8% with median value of 0.1%, under different parameter set

For tokenization Alt1, feature scalability Alt2, token scalability Alt1, payload configuration Alt2,

- 1 source [vivo] observes average gain in SGCS of Case 1 over Case 2 is in the range of -1.7% ~ 5.4% with median value of -2.6%, under same parameter set

- 1 source [vivo] observes average gain in SGCS of Case 1 over Case 2 is in the range of -2.6% ~ 0% with median value of -0.9%, under different parameter set

For tokenization Alt1, feature scalability Alt1, payload configuration Alt1,

- 1 source [Xiaomi] observes average loss in SGCS of Case 1 over case 2 is in the range of -4.76%~-0.82% with median value of -2.08%.

- 1 source [Xiaomi] observes average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is 3%

- 1 source [Xiaomi] observes average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is 5.17%

For tokenization Alt1, feature scalability Alt2, payload configuration Alt2,

- 1 source [Xiaomi] observes average loss in SGCS of Case 1 over case 2 is in the range of -5.52%~-1.59% with median value of -2.96%.

- 1 source [Xiaomi] observes average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is 2.04%

- 1 source [Xiaomi] observes average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is 5.17%

For tokenization Alt1, feature scalability Alt2, token scalability Alt2,

- 1 source [ZTE] observes average gain in SGCS of Case 1 over case 2 is in the range of -0.4%~0.9% with median value of -0.23%.

For tokenization Alt3, token scalability Alt1, payload configuration Alt2,

- 1 source [vivo] observes average gain in SGCS of Case 1 over Case 2 is in the range of -2.1% ~ 1.6% with median value of -0.5%, under same parameter set

- 1 source [vivo] observes average gain in SGCS of Case 1 over Case 2 is in the range of -0.8% ~ 0% with median value of -0.4%, under different parameter set

Conclusion

UE-side monitoring is feasible.

Observation

Some companies think that, at least in some UE-side monitoring options, NW-side monitoring with target CSI reporting is needed to check the reliability of UE-side monitoring reports.