3GPP TSG RAN WG1 #121		R1-2504072

St Julian’s, Malta, 19th – 23rd May 2025

Agenda Item 	:	9.11.4

Source 	:	Moderator (Sony)

Title 	:	FL Summary #1 for IoT-NTN

Document for	: Discussion

1	Introduction	2

2	WID objectives	3

3	Previous agreements	4

4	Incoming LS	11

5	NPUSCH	12

5.1	Overall summary of issues raised in Tdocs	12

5.2	3.75kHz single-tone OCC scheme	20

5.2.1	Working assumption on TDM DMRS scheme	22

5.2.2	3.75kHz TDM DMRS sequence	23

5.2.3	Signalling of TDM DMRS pattern applied by UE	24

5.3	15kHz single-tone OCC scheme	26

5.4	Multi-tone OCC scheme	26

5.5	UL gaps	29

5.6	Other features that should work with OCC	31

5.7	Signalling	33

5.7.1	Common design for 3.75kHz SCS and 15kHz SCS	36

5.7.2	Choice of DCI fields for repurposing	36

5.7.3	New or existing fields for DCI signalling	38

5.8	Physical Channel Aspects	38

5.8.1	Repetitions	39

5.8.2	RU size	40

5.8.3	RV size	40

5.8.4	TBS determination	41

5.8.5	Spreading	41

5.9	Downlink Issues and alignment	41

6	Monday: Proposals for offline session	42

7	Conclusions	42

8	References	42

# Introduction

This is the initial FLS for IoT NTN.

Main Introduction

This document is the Feature Lead Summary document for the Rel-19 IoT-NTN work item [1]. The final FLS for RAN1#120bis Wuhan is [2].

Items to be considered in this version of the feature lead summary are tagged with [FL3].

This FLS contains a set of proposals, which can hopefully be addressed in online meeting time at some stage. The document also contains a set of questions. These questions are intended for the purpose of sharing company views. If there is enough agreement, it might be possible to generate proposals.

NPUSCH

The following issues are discussed for NPUSCH:

- 3.75kHz single-tone OCC scheme. Details of TDM DMRS multiplexing scheme.

- 15kHz single tone OCC scheme. Whether any more agreements are needed.

- Multi-tone OCC scheme. Relative priority of this feature.

- UL gaps. How UL transmission gaps (of various types) affect OCC.

- Features that operate with OCC. Which features should be supported with OCC.

- Signalling. DCI signalling.

- Physical channel mapping. Meaning of the repetition DCI field. RU size, RV side, TBS determination and specification of spreading.

- Alignment and downlink issues. Consider how to align OCC between UEs, with gaps and with NPDCCH transmission.

When uploading updated documents with comments, please use the following convention for filenames:

Follow the naming convention in this example:

- IoTNTNFLS1-v000.docx

- IoTNTNFLS1-v001-CompanyA.docx

- IoTNTNFLS1-v002-CompanyA-CompanyB.docx

- IoTNTNFLS1-v003-CompanyB-CompanyC.docx

If needed, you may “lock” a discussion document for 30 minutes by creating a checkout file, as in this example:

- Assume CompanyC wants to update IoTNTNFLS1-v002-CompanyA-CompanyB.docx.

- CompanyC uploads an empty file named IoTNTNFLS1-v003-CompanyB-CompanyC.checkout.

- CompanyC checks that no one else has created a checkout file simultaneously, and if there is a collision, CompanyC tries to coordinate with the company who made the other checkout (see, e.g., contact list below).

- CompanyC then has 30 minutes to upload IoTNTNFLS1-v003-CompanyB-CompanyC.docx.

- If no update is uploaded in 30 minutes, other companies can ignore the checkout file.

In file names, please use the hyphen character (not the underline character) and include ‘v’ in front of the version number, as in the examples above and in line with the general recommendation, otherwise the sorting of the files will be messed up (which can only be fixed by the RAN1 secretary).

To avoid excessive email load on the RAN1 email reflector, please note that there is NO need to send an info email to the reflector just to inform that you have uploaded a new version of this document. Companies are invited to enter the contact info in the table below.

The table below provides a list of points of contact within companies for this WI. Contact details from RAN1#120bis Wuhan use a blue font. Please feel free to update your contact details and convert into a black font.

[FL1] Please consider entering contact info below for the points of contact for this email discussion.

| Company | Point(s) of contact | Email address(es) |
| --- | --- | --- |
| SONY | Martin Beale | martin.beale@sony.com |
| Ericsson | Gerardo Agni Medina Acosta | gerardo.agni.medina.acosta@ericsson.com |
| Lenovo | Zhi Yan | yanzhi1@lenovo.com |
| LGE | Daesung Hwang Seungmin Lee Hanjun Park | daesung.hwang@lge.com edison.lee@lge.com hanjun0128.park@lge.com |
| Gatehouse Satcom | René Brandborg Sørensen | rbs@gatehouse.com |
| Nokia, NSB | Jingyuan Sun | Jingyuan.sun@nokia-sbell.com |
| InterDigital, Inc | Umer Salim | umer.salim@interdigital.com |
| ETRI | Pansoo Kim | pskim@etri.re.kr |
| vivo | Zichao Ji Siqi Liu | jizichao@vivo.com liusiqi@vivo.com |
| Sharp | Zhanping Yin | zyin@sharplabs.com |
| Xiaomi | Xuemei Qiao | qiaoxuemei@xiaomi.com |
| Spreadtrum | Lei Gu | Lei.gu@unisoc.com |
| TCL | Yu Ding Yiwei Deng | yu10.ding@tcl.com yiwei1.deng@tcl.com |
| ZTE | Nan Zhang Fangyu Cui Ziyang Li | zhang.nan152@zte.com.cn cui.fangyu@zte.com.cn Li.ziyang1@zte.com.cn |
| Nordic | Mauri Nissila | Mauri.nissila@nordicsemi.no |
| CMCC | YI ZHENG Yongchang LIU | Zhengyi@chinamobile.com Liuyongchang@chinamobile.com |
| OPPO | Nande ZHAO Hao LIN Zuomin WU | zhaonande@oppo.com v-linhao1@oppo.com wuzuomin@oppo.com |

# WID objectives

The IoT-NTN WID [1] was updated in RANP#107 and has the following objectives:

- Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4] - Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2] Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI. Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome. Note: PWS notifications in paging, direct indication, and system information in RRC connected mode is not supported.

# Previous agreements

The following agreements were made in RAN1#116 Athens:

Agreement#116-IoT-NTN #1 For single-tone NPUSCH format 1 transmissions with both 3.75kHz and 15kHz SCS, the following OCC schemes are considered by RAN1 for further study: - Time domain OCC where OCC spreads across: - Symbol-level - Slot-level - Repetition-level - RV-level For multi-tone NPUSCH format 1 transmissions, the following OCC schemes are considered by RAN1 for further study: - Time domain OCC where OCC spreads across: - Symbol-level - Slot-level - Repetition-level - RV-level - Intra-symbol pre-DFT spreading OCC Agreement#116-IoT-2 The following evaluation assumptions are used for the study of OCC for NPUSCH format 1: Parameter; value. scenario; orbit; GEO; LEO600. Elevation angle; 12.5 degree; 30degree. Channel and impairments; carrier frequency; 2GHz. Channel model; NTN-TDL-C The channels from different UE are independent.. Frequency error; Uniform random selection from [-0.1 ppm, +0.1 ppm] for all UEs Variation of frequency error is negligible.. Timing error; Uniform random selection from [-97Ts, +97Ts] for all UEs Timing drift 80us/s for LEO600 and 0 for GEO.. Power imbalance; Uniformly distributed between +Pimb and -Pimb for all UEs Proponent to report the value of Pimb (can be zero) and justification for the chosen value. transmitter; SCS; 3.75KHz and 15KHz; 15kHz. Number of tones; Single tone; Single tone and multi tone up to 12 tones. Waveform; DFT-s-OFDM. Frequency hopping; w/o frequency hopping. MIMO scheme; SISO. DMRS configuration; For baseline evaluations: OS#3 per slot for 3.75kHz OS#4 per slot for 15kHz For OCC evaluations: Up to proponent; For baseline evaluations: OS#4 per slot for 15kHz For OCC evaluations: Up to proponent. Number of resource unit (NRU); Up to proponent; Up to proponent. Modulation order (Qm); Up to proponent; Up to proponent. TBS (ITBS); Up to proponent; Up to proponent. Number of repetitions (Nrep); Up to proponent. OCC length; Up to 4. OCC sequence; Up to proponent. Number of UE; Up to 4. Velocity of UE; 3km/h. receiver; Receiver algorithm; MMSE. Channel estimation; Real channel estimation. KPI; SNR at 10% BLER; Report for baseline and OCC schemes. Aggregated throughput; Total throughput of up to 4 UEs multiplexed.

The following agreements were made in RAN1#116bis Changsha:

Agreement For the NPUSCH evaluation assumptions, update the DMRS configuration, as follows: DMRS configuration; For baseline evaluations: OS#4 per slot for 3.75kHz OS#3 per slot for 15kHz For OCC evaluations: Up to proponent; For baseline evaluations: OS#3 per slot for 15kHz For OCC evaluations: Up to proponent. Agreement At least the following NPRACH OCC schemes are considered by RAN1 for study: - Intra-symbol group OCC - Inter-symbol group(s) OCC - Inter-repetition OCC Agreement The study of OCC for NPRACH does not consider NPRACH format 2. Agreement The following evaluation assumptions are used for the study of OCC for NPRACH: Parameter; value. Scenario; Orbit and elevation angle; GEO at 12.5 degrees; LEO600 at 30 degrees. Channel and impairments; carrier frequency; 2GHz. Channel model; NTN-TDL-C The channels from different UE are independent.. Frequency error; Uniform random selection from [-0.1 ppm, +0.1 ppm] for all UEs Variation of frequency error is negligible.. Timing error; Uniform random selection from [-97Ts, +97Ts] for all UEs Timing drift 80us/s for LEO600 and 0 for GEO.. Power imbalance; Uniformly distributed between +Pimb and -Pimb for all UEs Proponent to report the value of Pimb (can be zero) and justification for the chosen value. Transmitter; NPRACH format; 1 or 0. MIMO scheme; SISO. Number of repetitions (Nrep); Up to proponent. OCC length; Up to proponent. OCC sequence; Up to proponent. Number of UE; Up to proponent. Velocity of UE; 3km/h. Total NPRACH time / frequency resource utilisation; To be reported by proponent.. KPI; Target detection probability; 99%. Target false alarm probability; 0.1%. SNR operating point; Report SNR where target detection probability and false alarm probability are reached for baseline and OCC schemes. Agreement OCC multiplexing is not supported between a UE using NPUSCH format 1 with 3.75kHz SCS and another UE using NPUSCH format 1 with 15kHz SCS. Agreement For OCC of NPUSCH format 1, RAN1 will not consider multiplexing more than 4 UEs. Agreement For single-tone DMRS when OCC is applied to NPUSCH format 1, RAN1 considers at least the following for further study: - TDM of DMRS. The time domain locations of DMRS for different UEs are different. No OCC is applied for the DMRS of different UEs. - FFS: Detailed mapping - CDM of DMRS. The time domain locations of DMRS for different UEs are the same. Different OCCs are applied for the DMRS of different UEs. - FFS: Detailed mapping - Other schemes are not precluded, including combinations of the above Agreement For the NPUSCH evaluation assumptions, update the frequency error assumption, as follows. Frequency error; Uniform random selection from [-0.1 ppm, +0.1 ppm] for all UEs Variation of frequency error is negligible. For GEO, the same frequency error is applied to each subframe of a transport block. For LEO, the same frequency error is applied to each subframe of a segment (if applied in the evaluation). Companies to report their assumption on frequency error across segments..

The following agreements were made in RAN1#117 Fukuoka:

Agreement For 3.75kHz single-tone OCC for NPUSCH format 1, RAN1 supports either symbol-level OCC or slot-level OCC. Other OCC schemes are not pursued. For 15kHz single-tone OCC for NPUSCH format 1, RAN1 supports either symbol-level OCC or slot-level OCC. Other OCC schemes are not pursued. Agreement Inter-repetition OCC for NPRACH is not studied further in RAN1. Agreement - For the time-domain DMRS pattern (including blanked DMRS, if any): - For 15kHz single-tone, RAN1 strives to reuse the Rel-17 DMRS pattern - For 3.75kHz single-tone - RAN1 studies - Rel-17 DMRS pattern - A new DMRS pattern - The DMRS overhead (including blanked DMRS, if any) for OCC is the same as for Rel-17 Agreement The Rel-17 guard period locations and length for NB-IoT 3.75kHz UL slot are preserved when OCC is applied to NPUSCH format 1.

The following agreements were made in RAN#118bis Maastricht:

Agreement RAN1 studies whether the following types of UL transmission gap will impact the design of OCC for IoT-NTN when considering e.g. phase continuity - UL gaps for synchronization (from Rel-13) - Gaps around NPRACH occasions - UL timing adjustment gaps and segmentation for IoT-NTN (from Rel-17) - TDM DMRS that are muted - Guard periods for 3.75kHz UL transmissions Agreement The following combinations are considered for further simulation in RAN1 for 3.75kHz SCS OCC for NPUSCH format 1: - Option 1: OCC2, Symbol-level, TDM DMRS - Option 2: OCC2, Symbol-level, CDM DMRS with new pattern - Option 3: OCC2, Slot-level, TDM DMRS - Option 4: OCC2, Slot-level, CDM DMRS with legacy pattern - Option 6: OCC4, Symbol-level, CDM DMRS with new pattern The following combinations are considered for further simulation in RAN1 for 15kHz SCS OCC for NPUSCH format 1: - Option 1: OCC2, Symbol-level, TDM DMRS - Option 3: OCC2, Slot-level, TDM DMRS - Option 4: OCC2, Slot-level, CDM DMRS with legacy pattern - Option 5: OCC4, Symbol-level, TDM DMRS - Option 7: OCC4, Slot -level, TDM DMRS - Option 8: OCC4, Slot-level, CDM DMRS with legacy pattern Note 1: For TDM, the legacy DMRS pattern, with DMRS symbols appropriately muted/blanked is used. Companies to report their assumption on whether spreading is applied to the legacy DMRS pattern for 15 kHz SCS. Note 2: Companies to report DMRS sequence applied. Agreement For 3.75kHz SCS, NPUSCH format 1 simulations are performed using an appropriate MCS with SNR at least in the range of -8dB to 0dB.

The following agreements were made in RAN1#118bis Hefei:

Agreement At least the following schemes are supported for single-tone: - For 3.75kHz SCS OCC for NPUSCH format 1: - OCC length 2, Symbol-level - FFS: DMRS pattern(s) - For 15kHz SCS OCC for NPUSCH format 1: - OCC length 2, Slot-level, CDM DMRS with legacy pattern - FFS: CDM details, e.g. with or without spreading Agreement For NPRACH transmission, inter-symbol group OCC is not further studied. Agreement For support of single-tone OCC for NPUSCH format 1, RAN1 studies: - The parameters that need to be signalled, considering the following: - OCC codeword - Enabling of OCC feature - FFS: other parameters - For dynamic grant in RRC CONNECTED, study which and whether any parameters are signalled via DCI and which and whether any parameters are signalled by RRC - FFS: whether/how to support cases other than dynamic grant in RRC CONNECTED, e.g. Msg3, PUR, CB Msg3 EDT

The following agreements were made in RAN1#119 Orlando:

Agreement For 3.75kHz SCS OCC for NPUSCH format 1, the maximum OCC length is 2 for connected mode. Agreement For single tone 15kHz SCS Slot-level OCC for NPUSCH format 1, OCC length larger than 2 is not supported. Agreement For NPUSCH Format 1 single-tone 15kHz SCS, RAN1 studies at least the following options for CDM DMRS with legacy pattern for down-selection: - Option 1: DMRS symbols are spread before the OCC is applied, e.g. according to the formula: ru,OCCn=ru(nM)q(n mod M) Where: M is the OCC length, q is the assigned OCC codeword for the UE and run is the reference signal sequence defined in TS36.211 section 10.1.4.1.1 - Option 2: DMRS symbols are not spread before the OCC is applied. - Option 2_1: OCC is applied to the legacy complex-valued DMRS symbol used in slot 1 and slot 2, e.g. according to the formula: ru,OCCn=ru(n)q(n mod M) - Option 2_2: OCC is applied to the complex-valued DMRS symbol used in slot 1 and slot 2. Depending on the OCC codeword, different DMRS sequence is used. - Option 3: DMRS symbols are not spread and OCC is not applied. - Legacy complex-valued DMRS symbol is used in slots corresponding to an OCC codeword of NPUSCH. Different DMRS sequences are used for multiplexed UEs. Agreement For NPUSCH Format 1 single-tone 15kHz SCS, the slot-level scheme for non-DMRS symbols is that spreading is performed in the unit of one slot. - Note: whether RU length is extended or not after applying OCC is a separate discussion. Agreement For support of single-tone OCC for NPUSCH format 1 for connected mode, the parameters that need to be signalled are: - OCC sequence index - Enabling of OCC feature - FFS: whether signaling is explicit or implicit Agreement The following response is sent in reply to RAN2 LS R1-2409343: Response to RAN2: The OCC solution that RAN1 is working on may be applied to CB-Msg3 NPUSCH format 1 single tone at least for OCC length 2, for the subcarrier spacing(s) for which CB-Msg3 is supported, when the power imbalance is small. The design details for the OCC solution are ongoing. Action to RAN2: RAN1 respectfully asks RAN2 to take the above information into account. R1-2410894 Agreement The draft LS reply to RAN2 is endorsed in R1-2410894. Final LS in R1-2410895.

The following agreements were made in RAN1#120 Athens:

Agreement For the support of OCC length 2 for NPUSCH Format 1 single-tone with 3.75 kHz SCS and 15 kHz SCS, the orthogonal sequences are [1 1; 1 -1]. Working assumption For 3.75kHz SCS OCC for NPUSCH format 1, support TDM DMRS over 4 slots where DMRS are transmitted in the first 2 slots and DMRS REs are blanked in the next 2 slots, or vice-versa, where the DMRS REs are as in legacy NB-IoT and the guard period within the slot is as in legacy NB-IoT. Send LS to RAN4 asking whether there would be any issue (e.g. phase continuity) for supporting such TDM DMRS for IoT NTN. Agreement Send the following LS to RAN4: Overall description RAN1 has made the following working assumption on the DMRS pattern for OCC for 3.75kHz SCS OCC for NPUSCH format 1: For 3.75kHz SCS OCC for NPUSCH format 1, support TDM DMRS over 4 slots where DMRS are transmitted in the first 2 slots and DMRS REs are blanked in the next 2 slots, or vice-versa, where the DMRS REs are as in legacy NB-IoT and the guard period within the slot is as in legacy NB-IoT. Question: 1. From a RAN4 perspective, is it feasible to introduce support of TDM DMRS, as per the description above, in Rel-19? Agreement The draft LS to RAN4 in R1-2501621 is endorsed. Final LS in R1-2501622. Agreement For CONNECTED mode, UE-specific RRC signalling is used for enabling of the OCC feature. Conclusion RAN1 did not reach consensus on whether OCC for NPRACH is beneficial or not. RAN1 will not specify support for OCC for NPRACH in Rel-19 IoT NTN.

The following agreements were made in RAN1#120bis Wuhan:

Agreement For the 3.75kHz SCS symbol-based OCC scheme, the granularity of spreading for data is one symbol. Agreement Dynamic activation / deactivation of OCC is supported by DCI. - FFS: details of signalling by DCI Conclusion RAN1 assumes no specification change to support pairing UEs with different modulation orders. Agreement For 3.75kHz SCS OCC for NPUSCH format 1, RAN1 down selects between the following mappings between DMRS sequence samples and active TDM DMRS slots: - Option 1: Sequential mapping of samples of the original DMRS sequence to active DMRS slots - Option 2: Dropping of samples of the original DMRS sequence in blanked slots R1-2502332 FL Summary #2 for IoT-NTN Moderator (Sony) Agreement For NPUSCH Format 1 single-tone 15kHz SCS, for CDM DMRS with legacy pattern: - DMRS symbols are spread before the OCC is applied - Option 1_1: according to the formula: ru,OCCMn+m=runqm, 0≤n<X/M, m=0, …, M-1 Where: M is the OCC length, q is the assigned OCC codeword for the UE, run is the reference signal sequence defined in TS36.211 section 10.1.4.1.1 and X is the total number of slots in the NPUSCH transmission after OCC is applied Agreement The OCC sequence index is signalled using DCI format N0. Agreement The RRC parameters for IoT-NTN UL capacity enhancements are: RAN2 Parent IE; Parameter name in the spec; New or existing?; Description; Value range; UE-specific or Cell-specific. NPUSCH-ConfigDedicated-NB; npusch-OCC-Enabled; new; The parameter is used to enable OCC for NPUSCH format 1 single tone.; ENUMERATED {‘true’}; UE-specific. Agreement For indicating OCC sequence index and activation / deactivation, the following fields may be repurposed or constrained to be not present in the DCI: - Modulation and coding scheme - Repetition number - Redundancy version - Number of scheduled TB for Unicast - Subcarrier indication - Resource reservation

# Incoming LS

In RAN1#121 Malta, RAN1 received the following LS reply from RAN4 [4]:

1. Overall Description: RAN4 would like to thank RAN1 on the LS on TDM DMRS for 3.75kHz SCS OCC [1], and RAN4 has studied the feasibility of TDM DMRS. • RAN4 has concluded that it is feasible for the UE to maintain the phase continuity across the blank DMRS symbol in TDM DMRS scheme. • RAN4 will further discuss potential need for requirements if TDM DMRS would be specified (e.g. the measurement time for frequency error requirement). Therefore, from RAN4 side, no feasibility issue is identified and RAN4 respectfully asks RAN1 to take into account the information above when determining the final feasibility of TDM DMRS. 2. Actions: To RAN1: RAN4 respectfully asks RAN1 to take into account the information above when determining the final design on TDM DMRS. 3. Reference: [1] R1-2501622 LS on TDM DMRS for 3.75kHz SCS OCC, RAN1

# NPUSCH

## Overall summary of issues raised in Tdocs

The following is an overall summary of issues raised by companies in input contributions.

Priorities for completion of design: [Ericsson]

- 3.75kHz SCS: TDM DMRS

- 15kHz SCS: CDM DMRS

- DCI design

3.75kHz OCC scheme

- TDM DMRS

  - Sequential mapping vs dropping DMRS sequence symbols

    - Option 1: sequential mapping

      - Intercell interference issue if UEs have staggered start times [Samsung][Spreadtrum]

      - Intercell interference is problematic [TCL][Interdigital][HW]

      - Intercell interference when one cell operates with OCC and the other uses legacy [CATT][ETRI]

      - May preserve orthogonality between UEs [Samsung]

      - Specify functionality via equation: [Ericsson]

    - Option 2: dropping

      - Reduces potential interference with neighbour cells (fewer DMRS are transmitted) [Samsung][HW][CMCC]

      - Less specification impact [Samsung][HW][Xiaomi]

      - Implementation simpler than option 1 [Samsung][ETRI][MTK][Apple]

      - DMRS reception at eNB similar to legacy [Interdigital]

      - Simulated performance from previous meetings OK [CATT][HW]

      - Can lead to identical sequence in different cells [LGE]

      - Specify functionality via equation: [Ericsson]

      - Leads to the same DMRS sequences in neighbour cells for some pairs of UEs due to the dropping [QC]

        - FL: this seems contradictory to the view of other companies for option 1, where those other companies say that it is option 1 that has the intercell interference issue

        - FL: Maybe we need to clarify when the DMRS cause an intercell interference issue (with legacy UEs in neighbour cell; with other OCC UE in neighbour cell)

    - Preference

      - Option 1: sequential mapping

        - LGE, QC

      - Option 2: Dropping

        - Samsung, Spreadtrum, TCL, CATT, Interdigital, NEC, ETRI, MTK, Apple, HW, vivo, CMCC, Xiaomi, Nok, Sony

  - Potential equations for option 1 and option 2: [Ericsson]

    - Option 1: rmu,OCCn=run' if modnM,M=m 0 otherwise ,  0≤n<(MrepNPUSCHNslotsULNRU)M

      - Where n'=n -nM -m, M = 2 (i.e., OCC length), m = 0, 1.

    - Option 2: rmu,OCCn=run(modnM+(1-m),M) if modnM,M=m 0 otherwise ,  (0≤n<MrepNPUSCHNslotsULNRU)M

      - Where M = 2 (i.e., OCC length), m = 0, 1.

- Signalling of TDM DMRS pattern

  - Associate with OCC sequence index [TCL][CMCC][Xiaomi][ZTE]

  - Signalled separately to OCC sequence index [TCL][LGE]

    - Additional signalling overhead [TCL][CMCC][LGE]

  - Choose between the following options: [Lenovo][LGE][Sony]

For 3.75kHz SCS OCC for NPUSCH format 1, the DMRS position in the TDM DMRS pattern is:

- Option 1: DMRS position follows fixed TDM DMRS pattern from the first scheduled slot for each UE [Lenovo][LGE][Sony]

- Option 2: associated with the OCC sequence index

      - Also associated with the absolute slot index [Lenovo][LGE][Sony]

- Option 3: signalled separately to the OCC sequence index

  - FFS: signalling (e.g. DCI signals DMRS pattern)

- Option 4: TDM DMRS pattern semi-statically associated with UE [Sony]

  - Scheduler schedules UEs with compatible DMRS patterns

- Working assumption on TDM DMRS:

  - Confirm: MTK, CMCC, vivo, Nok, Ericsson, Sony

  - Not confirm:

15kHz OCC scheme

- Clarification on CDM DMRS equation: [Ericsson]

  - Since length of NPUSCH transmission is X=MrepNPUSCHNslotsULNRUM  , update the equation from RAN1#120bis as:

    - ru,OCCMn+m=runqm, 0≤n<MrepNPUSCHNslotsULNRU, m=0, …, M-1

    - Where: M is the OCC length, q is the assigned OCC codeword for the UE, run is the reference signal sequence defined in TS36.211 section 10.1.4.1.1.

  - FL: do we need to clarify the equation or can the spec editor decide what to do?

- Spec impact of slot level scheme

  - Specify as Nslots = 1, Midentical_NPUSCH = OCC length [OPPO][ZTE]

  - FL: Can this be up to the spec editor?

Multi-tone support:

- No: Ericsson

  - Multi-tone would only be applicable in high SNR conditions [Ericsson]

    - High SNR conditions are not an issue since they do not use many resources [Ericsson]

  - Many different numbers of subcarriers leads to complicated specification updates [Ericsson]

- Yes: ZTE, OPPO, Nokia, Spreadtrum, Xiaomi, Lenovo

- Scheme:

  - Slot level [OPPO]

    - Commonality with single-tone[Spreadtrum]

    - Common solution with single-tone [Nok]

  - NSlot level: [Spreadtrum][ZTE][Xiaomi][Lenovo]

    - Minimal spec changes [HW][ZTE][Spreadtrum]

    - Support Nslot = 2 [Xiaomi]

    - OCC lengths:

      - 2: Spreadtrum

      - 4:

- DMRS

  - Different cyclic shifts [Spreadtrum]

  - CDM DMRS [Spreadtrum][Xiaomi]

    - Reuse CDM DMRS scheme from single-tone 15kHz [Spreadtrum]

    - CDM DMRS in the frequency domain [Xiaomi]

- Signalling

  - Use single-tone design for:

    - Indication of OCC index [Spreadtrum]

    - Activation / deactivation [Spreadtrum]

- Multitone performance

OCC2:

|  | Loss to baseline |
| --- | --- |
| Slot-level |  |
| Nslot level | Xiaomi: 1.6dB |

OCC4:

|  | Loss to baseline |
| --- | --- |
| Slot-level |  |
| Nslot level | Xiaomi: 2.6dB |

Features that NPUSCH should work with:

- EDT

  - Yes: [Spreadtrum]

  - No: Xiaomi

    - Specification impact [Xiaomi]

- PUR:

  - Yes: [Spreadtrum]

    - Easier than for TN since the UE knows the timing from GNSS [TCL]

  - No: [Ericsson][Xiaomi]

    - Consider relationship of OCC to “shared PUR” in Rel-16 [Ericsson]

    - Specification impact [Xiaomi]

    - Shared-PUR in Rel-16 already supports reuse of time / frequency resources through use of orthogonal DMRS sequences [Ericsson]

    - Not clear that OCC UEs can be persistently paired [Ericsson]

      - i.e. over time the suitability of a PUR pairing might change / erode

- CB-Msg3-EDT (R19)

  - RAN2 are already discussing this wrt OCC, so RAN1 should support it

  - Yes: [QC]

  - Power imbalance is not a problem.

    - Simulations show that a 10dB power imbalance leads to only a 0.2dB loss for the lower power UE [QC]

      - Send an LS to RAN2 to inform them of this [QC]

      - FL: Related LS from RAN2 in R1-2503613

  - Need further study: [CMCC]

    - Pairing not possible in IDLE mode [CMCC]

- Msg3:

  - Study performance impact of:

    - Uplink timing alignment [CMCC]

    - Power imbalance [CMCC]

    - Frequency shifting [CMCC]

  - Need to identify the UE capability for OCC for Msg3 [CMCC]

  - Support or not:

    - Yes:

    - No: Xiaomi

      - Do not know OCC capability of UE at initial access, unless PRACH partitioning is applied [CMCC][Xiaomi]

        - Early indication of UE capability [Xiaomi]

      - Specification impact [Xiaomi]

        - OCC index assignment / determination mechanism [Xiaomi]

- Multi-TB:

  - No: Ericsson

    - Too much spec impact (clauses 16.4.1, 16.4.2, 16.5, 16.5.1, 16.5.1.2, and 16.6 of TS36.213)

  - Yes: QC

    - Both TBs used the same OCC index

    - Unit if interleaving multiplied by M for interleaved multi-TB: g=M NRUNslotsUL

UL gaps:

- RAN1 assumes that issues are resolved by scheduler. If this is not true, solutions can be discussed in the maintenance phase [Ericsson]

- Need to align OCC around transmission gaps [QC]

- Need to accurately align UEs if their NPUSCH starting times cause gaps to occur at different times [QC]

- Align OCC DMRS such that they don’t straddle a gap [QC]

- NPUSCH transmissions can only start in slots that are a multiple of NOCC_slots [QC]

  - Proposal 4 in R1-2504413.

    - FL: This could also be relevant to the TDM DMRS alignment

  - If gap occurs within the NOCC_slots, whole set of NOCC_slots is postponed to next set slot that is a multiple of NOCC_slots [QC]

- UL gaps for synchronization (from Rel-13):

  - Phase continuity broken across gap [CMCC][vivo][HW]

  - UE resynchronizes in the gap, so phase continuity will be lost [vivo]

  - HD-FDD UE swaps from UL to DL in gap, which further erodes phase continuity [CMCC]

  - Scheduler solutions: [ZTE]

    - eNB does not schedule UE with OCC if the NPUSCH would span a gap [vivo]

    - Network implementation can resolve issue by deactivating OCC if there is a gap [Spreadtrum]

  - OCC never straddles a gap since gap length is 40ms after 256ms transmission (and pairing must happen at a multiple of 2 slots) [Apple]

  - Postpone NPUSCH with granularity of OCC span [HW]

- Gaps around NPRACH occasions

  - FL: note that “Postponing” is the current rule for NPRACH gaps (see Apple R1-2504345)

  - Drop OCC segment that spans gap[Nordic]

  - Phase continuity not maintained across NPRACH occasion gap [HW]

    - Phase continuity may not be maintained for long NPRACH occasions [CMCC]

  - Postpone segment that would traverse gap [Apple][LGE][HW][CMCC]

    - Postpone only for 3.75kHz SCS, not a problem at 15kHz SCS [Apple]

    - Postpone to the first slot after the gap that is a multiple of 2 [LGE]

    - Postpone NPUSCH with granularity of OCC span [HW][ZTE]

      - Ensures that OCC patterns of both UEs are still aligned after a gap, assuming they were aligned before the gap.

      - FL: note, while the formulation in the ZTE Tdoc is different, I think the outcome is the same.

    - See figure in CMCC R1-2503848

  - For 15kHz, no issue since NPRACH occasion always occurs on even slot and max OCC = 2 [Apple][LGE]

    - Network implementation can resolve issue by deactivating OCC if there is a gap [Spreadtrum]

    - eNB can often avoid collision with NPRACH gap, but we should specify functionality (dropping) if a collision does happen [Nordic]

- UL timing adjustment gaps and segmentation for IoT-NTN (from Rel-17)

  - UE resynchronizes during a gap so phase continuity is not maintained [vivo][CMCC]

  - Drop OCC segment that spans gap [Apple][Nordic][LGE][HW][Nok]

    - Specify as “transmission gap of N_occ time units shall be counted for the NPUSCH resource mapping but not used for transmission of the NPUSCH” [LGE]

  - Scheduler solutions:

    - Scheduler should be able to avoid OCC colliding with this gap [Nordic]

    - Network implementation can resolve issue by deactivating OCC if there is a gap [Spreadtrum]

  - UL TA gaps are not an issue if segment length is a multiple of OCC length (i.e. when mod(Nsegmentprecompensation , OCC-length) = 0) and eNB can decide this [MTK]

  - Consider in design of OCC scheme [CMCC]

  - OCC application should be restricted to within segment [vivo]

  - Pre-segmented compensation segment length should be at least 8ms for 3.75Hz with OCC2 [QC]

    - Avoids the DMRS being excluded from a segment according to new DMRS schemes [QC]

- TDM DMRS that are muted

  - No issue [Spreadtrum][HW][CMCC]

  - Phase rotation impacts on channel estimation need to be considered [Apple]

- Guard periods for 3.75kHz UL transmissions

  - Do not affect phase continuity [ZTE][Apple][Spreadtrum][HW]

- Collision with reserved subframes / slots

  - These are subframes / slots that are reserved by resource reservation

  - Reserved subframe

    - 15kHz: overlap with reserved subframe leads => postponement of subframe [OPPO]

    - 3.75kHz: overlap with reserved subframe => postponement of 4 slots (to maintain TDM DMRS pattern) [OPPO]

  - Reserved symbol

    - Either:

      - SC-FDMA symbols within any part of OCC group that align with reserved symbol are punctured [OPPO]

      - OCC is only enabled if reserved uplink symbols are not configured [OPPO]

Segmented pre-compensation length

- Pre-compensation length needs to be longer than the OCC DMRS pattern to allow for channel estimation [QC]

- Pre-compensation length at least 8ms for 3.75kHz SCS with OCC2

Physical channel mapping

- Resource

  - Increase RU size

    - Super-RU = M RUs [QC]

    - Avoids a reduction of coding rate [QC]

  - RU size same as R18 [vivo][HW][Xiaomi][ZTE]

- RV issues

  - Increase RV size

    - Super-RV = NRU super-RUs [QC]

      - FL: Huawei phrase this differently, but I understand that they have the same unerstanding

    - Avoids a reduction of coding rate [QC]

    - Circular buffer manipulation done at a super-RC basis [QC]

      - FL: does this require changes to transport channel processing?

    - RV length in time increased by OCC factor [Lenovo][HW]

      - HW proposal (proposal 2 in R1-2503283) is:

        - “For Symbol-level OCC applied to 3.75kHz SCS single tone and Slot-level OCC applied to 15kHz SCS single-tone, the same RV value is used within an OCC group for NoccNRUNslotsUL slots. RV cycling can be performed across the OCC groups.”

    - Changes to RV should minimise spec impact [Nok]

- TBS determination

  - Based on existing ITBS and IRU procedure [HW][Lenovo][Xiaomi]

    - FL: i.e. TBS is based on the ITBS and IRU that are signalled in the DCI

  - Consider whether this is based on the allocated number of RUs or the number of OFDM symbols in a slot are scaled by the OC length [OPPO]

- Spreading

  - Define spreading function in physical channel mapping of [TS36.211]: [Oppo]]

    - “after mapping to Nsymbs=1 SC-FDMA symbols, the Nsymbs SC-FDMA symbols shall be repeated NSF-1 additional times, before continuing the mapping of z(⋅) to the following SC-FDMA symbols.”

  - Define spreading function and then OCC function [ZTE]

    - FL: understand that this is compatible with OPPO view.

- Repetitions

- understanding 1:  MrepNPUSCH=M×Mrepdata

    - Scheduled number of repetitions = OCC length * data repetitions [TCL]

      - This avoids the total transmission length for OCC being longer than for legacy [TCL][ETRI][Lenovo]

        - Means we cannot schedule REP128 and OCC2

          - i..e. coverage is not increased by this option

      - Actual number of repetitions should be scaled down by a factor of 2 [Lenovo]

      - Some codepoints cannot be indicated (e.g. REP1 is not useful) [vivo]

- understanding 2:  MrepNPUSCH =Mrepdata

    - Scheduled number of repetitions relates to the number of data repetitions

      - Leads to coverage enhancement, which is not an objective of WID (max rep = 128 and OCC2 can be applied leading to an overall repetition of 256) [Lenovo]

  - Preference:

    - Understanding 1 MrepNPUSCH=M×Mrepdata: TCL, ETRI, Lenovo, Nordic, HW(??), Xiaomi

    - Understanding 2  MrepNPUSCH =Mrepdata: vivo

Signalling

- DCI signalling

  - Number of bits required:

    - 2 bits or 3 states [Apple][HW]

      - 3 code points are needed [1 1], [1 -1] and deactivated. [Apple]

  - #OCC sequence index

    - New field [Samsung]

      - When new field is applied, another field is not configured [Samsung]

    - Repurpose existing fields [ETRI]

  - DCI size:

    - Don’t change [Nok]

    - Alignment between 3.75kHz and 15kHz DCI sizes [QC]

  - Extra fields to signal:

    - OCC factor (M) [QC]

      - FL: the choice seems to be between M=1 (legacy) and M=2. Isn’t this the same as activation / deactivation, which has already been agreed?

  - Views on fields:

    - SIF:

      - Spare code points at 15kHz SCS [Sharp][LGE][CMCC]

      - 3 code points are needed [1 1], [1 -1] and deactivated. These can be taken from SIF field [Apple][ZTE]

      - 3.75kHz SCS limitations:

        - 16 subcarriers can be indicated at 3.75kHz for OCC if indication is taken from SIF field [Apple][CMCC]

        - Not enough code points to indicate 1 bit (at 3.75kHz SCS) [Spreadtrum][LGE]

        - 3.75kHz SCS: use SIF field along with MCS field [HW][QC]

          - Split UEs into two groups of available 3.75kHz subcarriers via RRC signalling [HW]

        - Not all 48 possible subcarriers are allocated to NPUSCH anyway – some are allocated to NPRACH. Hence we do not need to signal all 48 possible legacy allocations [Ericsson]

      - 6 states used for legacy allocations, 48 states used for OCC allocations [Nordic]

      - All subcarriers can be indicated at 15kHz ST and MT [Apple][Nordic][HW]

        - If REP1 then use legacy SIF table, some codepoints considered for legacy, some for MT, some of OCC[1 1] and some for OCC [1 -1]

      - SIF re-purposing would lead to different designs for 3.75kHz SCS and 15kHz SCS [Xiaomi]

      - Different designs: [ZTE]

        - Use SIF field at 15kHz field (spare codepoints can be repurposed)

        - Use MCS and RV fields at 3.75kHz

      - FL: Should we have an agreement that says that we should strive for the same design for both 15kHz and 3.75kHz?

    - RV:

      - Can’t be used as this would disable RV signalling feature [Spreadtrum][Nok]

        - FFS whether it can be reused, before design is completed [Ericsson]

      - Can reuse. [Lenovo][Nordic][LGE]

        - With repetitions it doesn’t matter whether RV0 is first or RV2 is first [Lenovo]

        - Fix RV as RV0 [vivo][OPPO]

        - Joint signalling of OCC with RV [Nordic][Xiaomi]

          - OCC [1 1]: starting RV = 0

          - OCC [1 -1]: starting RV = 2

        - FL: note that there seem to be two options:

          - RV field is replaced with OCC field

          - Joint coding of RV and OCC field

    - MTB:

      - Can’t be used as this would disable multi-TB feature [Spreadtrum][vivo][Nok]

        - Multi-TB feature is feasible for OCC and should be supported [vivo]

      - No need to combine OCC feature with MTB feature [MTK]

      - Can’t be repurposed as field is not always present [CMCC]

      - MTB is useful when there are capacity limitations and that is what this WI is targeting, hence do not repurpose MTB field [Xiaomi][Nok]

    - MCS

      - Doesn’t change much in NTN [Spreadtrum]

      - High MCS not used in NTN [TCL][Sharp][TCL][Nok]

        - OCC may degrade performance, meaning high MCS are not applicable [Xiaomi]

      - High MCS not used in OCC when repetitions are applied [MTK][Xiaomi]

      - Using an MCS bit would lead to scheduling restrictions, but acceptable [vivo]

      - All current MCS values are needed (no 16QAM in baseline table) [Ericsson]

    - Number of NPUSCH Repetitions

      - Doesn’t change much in NTN [Spreadtrum][Sharp]

      - Using this field would introduce scheduling restrictions [vivo]

      - Which REP codepoints to reuse:

        - Low REP values are not used in NTN [Xiaomi][Nok]

          - FL: Implies low REP values are deleted from table

        - Only low REP values would be supported and we might need to support higher REP values [Ericsson]

          - FL: Implies high REP values are deleted from table

    - Resource reservation

      - RR needs to be supported to allow for support of IoT-NTN in NR carrier [vivo][Nok]

        - See R2-2503064 “Reply LS on NR NB-IoT in-band operation”, where RAN2 say that IoT-NTN in NR carrier is feasible [vivo]

        - Hence RR field cannot be repurposed [vivo]

      - Use of this field would preclude resource reservation between OCC and NR

      - Can’t be repurposed as field is not always present [CMCC][Xiaomi]

      - RR is not compatible with OCC [OPPO]

      - Reuse resource reservation field [QC]

    - DCI Repetition number

      - UE assumes that the NPDCCH is always transmitted with the maximum REP factor in order to free up 1 bit [LGE]

| Company | MCS | REP | RV | MTB | SIF | RR | DCI REP | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Samsung |  |  |  | yes |  | yes |  |  |
| Spreadtrum | yes | yes | no | no | no |  |  | MCS and repetition number do not change significantly for NTN |
| TCL | yes |  |  |  |  |  |  | High MCS not used in NTN |
| CATT | yes | yes |  |  |  |  |  | High MCS not used in NTN. Low REP not used in NTN |
| Sharp |  | yes |  |  | Yes |  |  | SIF: considered 15kHz and MT, not 3.75kHz. Small variation in required number of repetitions |
| Mediatek | yes |  |  | yes |  |  |  | High MCS not used in OCC |
| Apple |  |  |  | yes |  |  |  | 2 bits from SIF field. |
| Lenovo | yes |  | yes |  | yes |  |  | RV: doesn’t matter whether RV0 is first or RV1 Consider joint encoding of fields |
| Nordic |  |  | yes |  | yes |  |  | SIF: can indicate all allocations for 15kHz ST and MT for OCC and legacy RV index and OCC index are indicated jointly |
| LGE |  |  |  |  | yes |  | yes | DCI REP: UE assumes max REP always applied to DCI |
| HW | yes |  |  |  | yes |  |  | 15kHz: just use SIF 3.75kHz: SIF + MCS; interpretation of SIF is UE-specific and signalled by RRC |
| vivo | yes |  | yes | no |  |  |  | RV: fix RV field as e.g. RV0 |
| CMCC | yes | no | yes | no | yes | no |  | REP: need to provide for a large number of repetitions |
| Xiaomi | yes | yes | yes | no | no | no |  | Low REP values are not useful in NTN RV: linkage between RV and OCC index |
| Nokia | yes | yes | no | no | yes | yes |  |  |
| OPPO |  |  | yes |  |  | yes |  | RV: always start with RV0 |
| ZTE | yes |  | yes |  | yes |  |  |  |
| Ericsson | no | maybe |  | yes | yes | no |  | Includes TP |
| QC | yes |  |  |  | yes | yes |  | Joint encoding of SIF and MCS |

- RRC signalling

  - OCC sequence can be based on RRC configuration and DCI-based configuration [CMCC]

  - Implicit activation

    - Based on DCI field being in a particular range configured by RRC => OCC is activated / certain OCC codeword index is used

  - RRC signalling for PUR, CB-Msg3-EDT (or other semi-statically configured transmission)

Pairing

- Factors to be considered for pairing:

  - Power imbalance [CMCC]

  - Timing accuracy [CMCC]

  - Frequency offset [CMCC]

Compatibility with legacy UEs

- Should be supported [Nokia]

  - FL: seems like claimed issues are the amount of resource used for OCC and scheduling of both OCC and non-OCC UEs

Downlink issues

- Alignment of NPUSCH requires staggered NPDCCH, requiring new k0 values (subframes between NPDCCH and NPUSCH) [Ericsson]

  - Additional offset: k0 + {1,2,4,8,16} to allow for NPPDCH repetitions [Ericsson]

Alignment

- Alignment of NPUSCH requires staggered NPDCCH, requiring new k0 values (subframes between NPDCCH and NPUSCH) [Ericsson]

- Reference point in time introduced to align OCC from different UEs [TCL]

  - Helps with UL gaps [TCL]

  - OCC transmissions only start in slots that are multiples of NOCC_slots [QC]

    - 3.75kHz: NOCC_slots = 4

    - 15kHz: NOCC_slots = 2

- RAN1 to discuss how to align OCC transmissions from different UEs [Nok]

- Alignment of adding new UEs with OCC to existing group of UEs already doing OCC [QC][Nok]

- Cannot just rely on scheduling to get alignment [Nok]

LS on CB-Msg3-EDT

- Discussion included in the following Tdocs:

  - LGE: R1-2504564

## 3.75kHz single-tone OCC scheme

The following views were expressed about the type of OCC scheme that should be supported for 3.75kHz single-tone:

3.75kHz OCC scheme

- TDM DMRS

  - Sequential mapping vs dropping DMRS sequence symbols

    - Option 1: sequential mapping

      - Intercell interference issue if UEs have staggered start times [Samsung][Spreadtrum]

      - Intercell interference is problematic [TCL][Interdigital][HW]

      - Intercell interference when one cell operates with OCC and the other uses legacy [CATT][ETRI]

      - May preserve orthogonality between UEs [Samsung]

      - Specify functionality via equation: [Ericsson]

    - Option 2: dropping

      - Reduces potential interference with neighbour cells (fewer DMRS are transmitted) [Samsung][HW][CMCC]

      - Less specification impact [Samsung][HW][Xiaomi]

      - Implementation simpler than option 1 [Samsung][ETRI][MTK][Apple]

      - DMRS reception at eNB similar to legacy [Interdigital]

      - Simulated performance from previous meetings OK [CATT][HW]

      - Can lead to identical sequence in different cells [LGE]

      - Specify functionality via equation: [Ericsson]

      - Leads to the same DMRS sequences in neighbour cells for some pairs of UEs due to the dropping [QC]

        - FL: this seems contradictory to the view of other companies for option 1, where those other companies say that it is option 1 that has the intercell interference issue

        - FL: Maybe we need to clarify when the DMRS causes an intercell interference issue (with legacy UEs in neighbour cell; with other OCC UE in neighbour cell)

    - Preference

      - Option 1: sequential mapping

        - LGE, QC

      - Option 2: Dropping

        - Samsung, Spreadtrum, TCL, CATT, Interdigital, NEC, ETRI, MTK, Apple, HW, vivo, CMCC, Xiaomi, Nok, Sony

  - Potential equations for option 1 and option 2: [Ericsson]

    - Option 1: rmu,OCCn=run' if modnM,M=m 0 otherwise ,  0≤n<(MrepNPUSCHNslotsULNRU)M

      - Where n'=n -nM -m, M = 2 (i.e., OCC length), m = 0, 1.

    - Option 2: rmu,OCCn=run(modnM+(1-m),M) if modnM,M=m 0 otherwise ,  (0≤n<MrepNPUSCHNslotsULNRU)M

      - Where M = 2 (i.e., OCC length), m = 0, 1.

- Signalling of TDM DMRS pattern

  - Associate with OCC sequence index [TCL][CMCC][Xiaomi][ZTE]

  - Signalled separately to OCC sequence index [TCL][LGE]

    - Additional signalling overhead [TCL][CMCC][LGE]

  - Choose between the following options: [Lenovo][LGE][Sony]

For 3.75kHz SCS OCC for NPUSCH format 1, the DMRS position in the TDM DMRS pattern is:

- Option 1: DMRS position follows fixed TDM DMRS pattern from the first scheduled slot for each UE [Lenovo][LGE][Sony]

- Option 2: associated with the OCC sequence index

      - Also associated with the absolute slot index [Lenovo][LGE][Sony]

- Option 3: signalled separately to the OCC sequence index

  - FFS: signalling (e.g. DCI signals DMRS pattern)

- Option 4: TDM DMRS pattern semi-statically associated with UE [Sony]

  - Scheduler schedules UEs with compatible DMRS patterns

- Working assumption on TDM DMRS:

  - Confirm: MTK, CMCC, vivo, Nok, Ericsson, Sony

  - Not confirm:

The following agreements were made related to the support of NPUSCH format 1 with 3.75kHz SCS:

RAN1#118bis

Agreement At least the following schemes are supported for single-tone: - For 3.75kHz SCS OCC for NPUSCH format 1: - OCC length 2, Symbol-level - FFS: DMRS pattern(s) - For 15kHz SCS OCC for NPUSCH format 1: - OCC length 2, Slot-level, CDM DMRS with legacy pattern - FFS: CDM details, e.g. with or without spreading

RAN1#119

Agreement For 3.75kHz SCS OCC for NPUSCH format 1, the maximum OCC length is 2 for connected mode.

RAN1#120:

Working assumption For 3.75kHz SCS OCC for NPUSCH format 1, support TDM DMRS over 4 slots where DMRS are transmitted in the first 2 slots and DMRS REs are blanked in the next 2 slots, or vice-versa, where the DMRS REs are as in legacy NB-IoT and the guard period within the slot is as in legacy NB-IoT. Send LS to RAN4 asking whether there would be any issue (e.g. phase continuity) for supporting such TDM DMRS for IoT NTN.

RAN1#120bis

Agreement For 3.75kHz SCS OCC for NPUSCH format 1, RAN1 down selects between the following mappings between DMRS sequence samples and active TDM DMRS slots: - Option 1: Sequential mapping of samples of the original DMRS sequence to active DMRS slots - Option 2: Dropping of samples of the original DMRS sequence in blanked slots

### Working assumption on TDM DMRS scheme

RAN4 have sent a reply LS to RAN1 (R1-2503211), stating the following:

1. Overall Description: RAN4 would like to thank RAN1 on the LS on TDM DMRS for 3.75kHz SCS OCC [1], and RAN4 has studied the feasibility of TDM DMRS. • RAN4 has concluded that it is feasible for the UE to maintain the phase continuity across the blank DMRS symbol in TDM DMRS scheme. • RAN4 will further discuss potential need for requirements if TDM DMRS would be specified (e.g. the measurement time for frequency error requirement). Therefore, from RAN4 side, no feasibility issue is identified and RAN4 respectfully asks RAN1 to take into account the information above when determining the final feasibility of TDM DMRS. 2. Actions: To RAN1: RAN4 respectfully asks RAN1 to take into account the information above when determining the final design on TDM DMRS. 3. Reference: [1] R1-2501622 LS on TDM DMRS for 3.75kHz SCS OCC, RAN1

In input Tdocs, companies in RAN1 hence consider that TDM DMRS for 3.75kHz SCS is feasible and propose to confirm the working assumption.

Proposal 5_2_1v1: Confirm the following working assumption from RAN1#120 Athens:

“For 3.75kHz SCS OCC for NPUSCH format 1, support TDM DMRS over 4 slots where DMRS are transmitted in the first 2 slots and DMRS REs are blanked in the next 2 slots, or vice-versa, where the DMRS REs are as in legacy NB-IoT and the guard period within the slot is as in legacy NB-IoT.”

Companies are invited to comment on proposal 5_2_1v1 below:

| Company | Comment |
| --- | --- |
| Ericsson | Ok with confirming the Working Assumption. |
| LGE | OK. |
| Nokia, NSB | OK. |
| Xiaomi1 | OK |
| OPPO | OK |
| Lenovo | OK |
| Spreadtrum | Support |
| SONY1 | OK |
| FL3 | This proposal seemed agreeable in the offline session on Monday and will be an online proposal for Tuesday. |

### 3.75kHz TDM DMRS sequence

At RAN1#120bis Wuhan, RAN1 agreed the following:

RAN1#120bis

Agreement For 3.75kHz SCS OCC for NPUSCH format 1, RAN1 down selects between the following mappings between DMRS sequence samples and active TDM DMRS slots: - Option 1: Sequential mapping of samples of the original DMRS sequence to active DMRS slots - Option 2: Dropping of samples of the original DMRS sequence in blanked slots

These options are described below:

Option 1: Sequential mapping of samples of the original DMRS sequence to active DMRS slots

The following timeline for transmission of DMRS for the TDM DMRS scheme is proposed by Qualcomm in R1-2504413:

|  | rmu,OCCn |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE 0, m=0 Cell u | ru0 | ru1 | 0 | 0 | ru2 | ru3 | 0 | 0 |
| UE 1, m=1 Cell u | 0 | 0 | ru0 | ru1 | 0 | 0 | ru2 | ru3 |
| Slot number (n) | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |

This functionality is described by the following equation (proposed by Qualcomm in R1-2504413 and Ericsson in R1-2503338):

rmu,OCCn=run' if modnM,M=m 0 otherwise ,  0≤n<(MrepNPUSCHNslotsULNRU)M

where n'=n -nM -m, M = 2 (i.e., OCC length), m = 0, 1

Option 2: Dropping / puncturing samples of the original DMRS sequence in blanked slots

In this option, symbols from the legacy DMRS sequence are just dropped / punctured in blanked DMRS slots, leading to the following timeline for the transmission of DMRS:

|  | rmu,OCCn |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE 0, m=0 Cell u | ru0 | ru1 | 0 | 0 | ru4 | ru5 | 0 | 0 |
| UE 1, m=1 Cell u | 0 | 0 | ru2 | ru3 | 0 | 0 | ru6 | ru7 |
| Slot number (n) | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |

This functionality is described by the following equation (proposed by Ericsson in R1-2503338):

rmu,OCCn=run(modnM+(1-m),M) if modnM,M=m 0 otherwise ,  (0≤n<MrepNPUSCHNslotsULNRU)M

where M = 2 (i.e., OCC length), m = 0, 1.

The choice between option 1 and option 2 according to input Tdocs depends on the following aspects:

- Specification impact

- Intercell interference. There seem to be at least two scenarios:

  - Scenario 1: UE with TDM pattern in serving cell interferes with other UE with TDM DMRS pattern in neighbour cell

    - Sequential mapping (option 1) seems preferable in this case since neighbour cells have unique DMRS sequences. In contrast, the dropping mapping can lead to the same DMRS sequence in neighbour cells, depending on cell ID.

  - Scenario 2: UE with TDM DMRS pattern in serving cell interferes with legacy UE in neighbour cell

    - Dropping mapping (option 2) seems preferable in this case since the interference characteristics are similar to legacy.

[FL1] Question 5_2_2v1: Which DMRS sequence sample mapping (sequential mapping or dropping mapping) has preferable characteristics with regards to the following scenarios:

- Scenario 1: UE with TDM pattern in serving cell interferes with other UE with TDM DMRS pattern in neighbour cell

- Scenario 2: UE with TDM DMRS pattern in serving cell interferes with legacy UE in neighbour cell

| Company | Preferable mapping scheme in scenario |  | Overall comment |
| --- | --- | --- | --- |
|  | Scenario 1. Serving cell = TDM DMRS UE; neighbour cell = TDM DMRS UE | Scenario 2. Serving cell = TDM DMRS UE Neighbour cell = legacy UE |  |
| Example | Sequential mapping better since sequences unique in both cells | Dropping mapping better since interference similar to legacy | Prefer dropping mapping |
| Ericsson |  |  | Beyond inter-cell interference there are other aspects that need to be pondered e.g., specification impact. At this point we do not have any strong preference for Opt1 or Opt2, but it is important to clearly define the length of the OCC reference signal sequence (i.e., rmu,OCCn) of both Opt1 and Opt2. |
| LGE | Sequential mapping can avoid the case when the different cells use the same DMRS sequence. It is important to avoid the ambiguity on which cell the NPUSCH transmission is associated with. Moreover, it is the principle of adopting Option 1 for 15kHz SCS case. Situation is almost the same. | Some proponents supporting dropping mapping will make the legacy sequence. But it is not true. First of all, when two NPUSCH transmissions can have the different starting slot. In this case, the combined sequence is different from the legacy sequence. Next, even with the same starting position, these two UEs will have different channel coefficients and different initial phase setting. In this case, the combined sequence never be the legacy sequence. For both options, the final sequence will be different from the legacy sequence, and this kind of situation is already seen in 15kHs SCS case. | Support sequential mapping |
| Nokia, NSB | Dropping mapping better since no additional sequence mapping for both cells | Dropping mapping better since similar as legacy interference. | Prefer dropping mapping. |
| Qualcomm | Dropping of samples may lead to 2 cells having the exact same DMRS sequence. Sequential mapping will keep the same properties as in legacy specs. | For sequential mapping, the interference will be similar to transmissions starting at different points in time (due to the Gold sequence). For dropping, the Gold sequence will be exactly the same, in half the samples (the other half will be dropped) | Sequential |
| Xiaomi1 | Both sequential mapping and dropping mapping could work. We can’t see any difference between these two methods. | Dropping mapping better as present by FL. | Prefer dropping mapping |
| OPPO |  |  | Prefer sequential mapping. We agree with the FL that option 1 and option 2 has its own preferable scenario respectively. However, the inter-cell interference randomization can be still realized through the appropriate selection of cell u among the cells. In this case, the sequential mapping mechanism allows more flexible scheduling than dropping mapping. For example, for option 2, the NPUSCH transmissions between the OCC multiplexed UEs have to be fully aligned to guarantee the received DMRS sequences are ru0, ru1, ru2, ru3…, corresponding to slot 0,1,2,3…, from the gNB side. |
| Vivo1 | Dropping mapping better For NW perspective, the DMRS sequences from two OCCed UEs could be regarded as a DMRS sequence of one legacy UE, while the sequential mapping would generate two different new DMRS sequence due to the spreading of legacy DMRS sequence. | Dropping mapping better | Dropping mapping better. |
| Lenovo |  |  | Both options have some interference issues, we slightly prefer dropping method since it is more simple. |
| Spreadtrum | Dropping mapping is better. Sequential mapping may generate new DMRS sequence. | In current spec, DMRS sequence is different between serving cell and neighbor cell. Dropping mapping is better since it follow legacy DMRS sequence. | Prefer dropping mapping. |
| ZTE | Similar performance between two mapping solutions. | Dropping mapping will bring smaller interference since the sequence orthogonality with legacy DMRS sequence can be maintained. | Dropping mapping is preferred |
| SONY1 | Agree with QC that dropping mapping can lead to two cells have identical DMRS sequences, depending on cell ID. Sequential mapping does not suffer from this problem. | Agree with LGE that dropping mapping can lead and OPPO that the “combined legacy DMRS” will only be received by the gNB if the UE transmissions are aligned. Hence the “dropping mapping” still lead to intercell interference to legacy UEs. | Sequential mapping is preferred. It shares some characteristics with the 15kHz SCS scheme. The scheme we choose should be compatible with the motivations we had for choosing option 1 for CDM DMRS for 15kHz SCS. |

#### Proposal for 3.75kHz TDM DMRS sequence

The choice between the following two options was discussed in the offline session on Monday:

- Option 1: Sequential mapping of samples of the original DMRS sequence to active DMRS slots

- Option 2: Dropping of samples of the original DMRS sequence in blanked slots

Based on the above discussion and the comments in the offline session, the following table provides a summary of the functionality of the two schemes:

| aspect | Option 1: sequential | Option 2: dropping |
| --- | --- | --- |
| Interference when serving cell and neighbour cell are both TDM DMRS | Avoids situation where both serving and neighbour cells have the same sequence. This was the principle for adoption of “Option 1” for the 15kHz SCS case. Interference in neighbour cells is pseudorandom | For some cell IDs, both serving and neighbour cells have the same TDM DMRS sequence There are cases when the interference in neighbour cells is not pseudorandom |
| Interference when serving cell is TDM DMRS, neighbour cell is legacy | Interference in neighbour cells is similar to transmissions starting at different times due to properties of Gold sequence. Interference in neighbour cells is pseudorandom. | The sequence received in the neighbour cells does not look like a legacy sequence in the following cases: - When starting times of UEs in serving cell are staggered - When OTA channels for OCC pair UEs are different Interference in neighbour cells is pseudorandom if the starting times are staggered When aligned, there are cases (depending on OTA channel) where the DMRS are completely correlated between serving and neighbour cells |
| Need for new DRMS sequence | No. This is just a method of mapping the existing sequences. | No. Parts of the existing sequences are used. |
| Supporting companies | QC, SONY, OPPO, Nordic, LGE | MTK, IDC, HW, ZTE, Apple, Nok, Samsung, Xiaomi, CATT, Lenovo, vivo |

In the offline discussions, technical disadvantages of the “dropping” scheme were identified, but technical merits of the “dropping” scheme over the “sequential” scheme were not identified.

In summary, the “dropping” mapping has some drawbacks (that the DMRS sequence can be the same in both serving and neighbour cells for some cell IDs) that the “sequential” mapping does not have.

Hence, the following proposal is made:

[FL3] Proposal 5_2_2v2:

For 3.75kHz SCS OCC for NPUSCH format 1, RAN1 the following mappings between DMRS sequence samples and active TDM DMRS slots is applied:

- Option 1: Sequential mapping of samples of the original DMRS sequence to active DMRS slots

Companies are invited to comment on proposal 5_2_2v2:

| Company | Comment |
| --- | --- |

If this sequential mapping is applied, a proposal for how to implement this mapping as an equation is (updated from Qualcomm in R1-2504413 and Ericsson in R1-2503338 and assuming that the length of an NPUSCH transmissions with OCC is agreed in proposal 5_8_1v2 as MrepNPUSCHNslotsULNRU:

rmu,OCCn=run' if modnM,M=m 0 otherwise ,  0≤n<MrepNPUSCHNslotsULNRU

where n'=n -nM -m, M = 2 (i.e., OCC length), m = 0, 1

FL thinks that this proposed equation makes assumptions on the starting position of the DMRS pattern, which is discussed in section 5.2.3. We hence need to agree on a proposal from section 5.2.3 before defining an equation for the DMRS.

### Signalling of TDM DMRS pattern applied by UE

While RAN1 has agreed to use a TDM DMRS pattern to provide DMRS orthogonality between UEs at 3.75kHz SCS, it is still undecided how the UE determines which TDM DMRS pattern (phasing) to apply (either ON ON OFF OFF; or OFF OFF ON ON). The following options have been identified:

- Option 1: DMRS position follows fixed TDM DMRS pattern from the first scheduled slot for each UE [Lenovo][LGE][Sony]

- Option 2: associated with the OCC sequence index

  - Option 2_1: Also associated with the absolute slot index [Lenovo][LGE][Sony]

  - Option 2_2: NPUSCH transmissions always start at a slot n where n mod 4 = 0

- Option 3: signalled separately to the OCC sequence index

  - FFS: signalling (e.g. DCI signals DMRS pattern)

- Option 4: TDM DMRS pattern semi-statically associated with UE [Sony]

  - Scheduler schedules UEs with compatible DMRS patterns

Option 1 is illustrated below:

Option 2_1 is illustrated below:

[FL1] Question 5_2_3v1: How does the UE determine which TDM DMRS positions to activate within the TDM DMRS pattern for 3.75kHz SCS:

- Option 1: DMRS position follows fixed TDM DMRS pattern from the first scheduled slot for each UE

- Option 2: DMRS position associated with the OCC sequence index

  - Option 2_1: Also associated with the absolute slot index

  - Option 2_2: NPUSCH transmissions always start at a slot n where n mod 4 = 0

- Option 3: signalled separately to the OCC sequence index

  - FFS: signalling (e.g. DCI signals DMRS positions)

- Option 4: DMRS positions semi-statically associated with UE

Companies are invited to answer question 5_2_3 below.

| Company | Comment |
| --- | --- |
| Ericsson | We think it can be associated to the OCC sequence index. We are not sure a down-selection between Option 2_1 and Option 2_2 is needed. For both TDM DMRS schemes under discussions (Opt1 and Opt2), the variable “m” referring UE number (from 0 to M-1) which is used to generate the OCC reference signal sequence rmu,OCCn, can be incorporated into the “OCC sequence index” field in TS 36.212 as follows: “OCC sequence index – 1 bit, where value 0 indicates OCC sequence [1 1] and UE number m = 0, whereas value 1 indicates OCC sequence [1 -1] and UE number m = 1. This field is only present if higher layer parameter npusch-OCC-Enabled is configured” |
| LGE | We are supportive of Option 2_1. We do not need to have restriction on the NPUSCH starting position. |
| Nokia, NSB | Option 2 but with modification We do not think there should be fixed DMRS position for UE or start fixed for any slot, considering the UE’s data arrival will be dynamic and scheduling should be flexible up to eNB. To make it simple, the DMRS position can be associated with OCC index but up to eNB scheduling for the position. Option 2-3: DMRS position associated with the OCC sequence index, with NPUSCH starting position up to eNB scheduling. |
| Qualcomm | We would be supportive of Option 2_2. This issue may need to be discussed at the same time as the postponement: for instance, we should not have a case in which due to postponement we have only 2 slots (both of which may lack DMRS). Our proposal is that both the initial transmission and the transmission after postponement should start at a slot that is a multiple of 4. |
| Xiaomi | Generally fine with the proposal. Considering the partial overlapping case, slightly prefer Nokia’s version. |
| OPPO | Option 2 is enough for the UE to determine the TDM DMRS positions. Further restrictions proposed in Option 2_1 and Option 2_2 are not needed. |
| Vivo1 | Option1 is not clear, does it mean that UE always apply [1 1 0 0 ] from the starting? Or UE always apply a pattern ([1 1 0 0 ] or [0 0 1 1]) configured by RRC? Support Option 2_1, which is straightforward. DMRS position can be associated with the absolute slot index, and the paired UE are scheduled by NW implementation. |
| Lenovo | We support Option 2, either Option 2-1 or 2-2 is OK for us. |
| ZTE | We think DMRS positions should be associated with OCC index but neither option 2_1 or option 2_2 is preferred. From our point of view, the TDM DMRS pattern should be counted from first scheduled slot for each UE similar as option 1 instead of based on absolute slot index. That is, following option 2_3 is preferred: Option 2-3: DMRS position associated with the OCC sequence index, and the pattern of DMRS position is from the first scheduled slot for each UE |
| SONY1 | We think that the starting position of the NPUSCH should be flexible, allowing: - NPUSCH to start “as soon as possible” - Provide flexibility to the scheduler to apply a start slot for NPSUCH, also considering that there will be limitations in where the NPDCCH can be sent (see also issues in section 5.9) For these reasons, we prefer option 2_1. Option 2_2 is less flexible in terms of where the NPUSCH is scheduled (and where the NPDCCH can be transmitted). Option 1 works, but has restrictions on where the NPUSCH can be scheduled (e.g. if two NPUSCH arrived in buffers at the same time eNB would have to delay the start of one .NPUSCH to start 2 slots later than the other one). Option 3 increases the DCI signalling (DCI additionally needs to signal DMRS start phase). Option 4 increases RRC signalling and makes pairing difficult for the scheduler. It is equivalent to vivo’s “UE always apply a pattern ([1 1 0 0 ] or [0 0 1 1]) configured by RRC”, where UE1 is configured with [1 1 0 0] and UE2 is configured with [0 0 1 1]). Summary: support option 2_1. |
| FL3 | Companies prefer the option 2 proposals. Hence, the following proposal is made: [FL3] Proposal 5_2_3v13: For 3.75kHz SCS, the TDM DMRS positions that are activated within the TDM DMRS pattern are associated with the OCC sequence index and - Option 2_1: is associated with the absolute slot index - Option 2_2: NPUSCH transmissions always start at a slot n where n mod 4 = 0 - Option 2-3: the DMRS pattern starts from the first scheduled slot for each UE |

## 15kHz single-tone OCC scheme

The following views were expressed with regards to the 15kHz single-tone OCC scheme:

- Clarification on CDM DMRS equation: [Ericsson]

  - Since length of NPUSCH transmission is X=MrepNPUSCHNslotsULNRUM  , update the equation from RAN1#120bis as:

    - ru,OCCMn+m=runqm, 0≤n<MrepNPUSCHNslotsULNRU, m=0, …, M-1

    - Where: M is the OCC length, q is the assigned OCC codeword for the UE, run is the reference signal sequence defined in TS36.211 section 10.1.4.1.1.

  - FL: do we need to clarify the equation or can the spec editor decide what to do?

- Spec impact of slot level scheme

  - Specify as Nslots = 1, Midentical_NPUSCH = OCC length [OPPO][ZTE]

  - FL: Can this be up to the spec editor?

FL’s understanding is that the above issues can be resolved by the spec editor and RAN1 does not need to make further agreements on the 15kHz SCS single tone OCC scheme.

[FL1] Question 5_3v1: Does RAN1 need to make further agreements on the following (or can details be left to the spec editor):

- Clarification of CDM DMRS equation

- Specifying the slot level scheme based on Nslots = 1, Midentical_NPUSCH = OCC length

Companies are invited to comment on question 5_3v1 in the table below.

| Company | Comment |
| --- | --- |
| Ericsson | On the first bullet, we need to provide a clarification on the upper-limit of the OCC reference signal sequence rmu,OCCn. The OCC reference signal sequence ru,OCCMn+m of the CDM DMRS formula agreed in RAN1# 120bis, invokes the legacy reference signal sequence run which is multiplied by qm which is the OCC codeword [1 1] or [1 -1], meaning that for every run sample we get two as a result [run run] or [run-run], which implicitly accounts for the OCC spreading without having to scale the length of the new ru,OCCMn+m sequence length by M. Thus, in our view the upper limit (i.e., X/M in 0≤n<X/M) of the OCC reference signal sequence length for the CDM DMRS option 1_1 agreed in RAN1# 120bis is as follows: 0≤n<MrepNPUSCHNslotsULNRU On the second bullet, since it involves the “OCC length,” we believe it touches upon what needs to be clarified in the first bullet. |
| LGE | It would be up to editor, or it can be handed during the maintenance phase if necessary. |
| Nokia, NSB | Generally it should be fine at least for the first bullet. While we think it is also ok no impact to spec for second bullet. |
| Xiaomi | Further clarification is necessary for better understanding. In our mind, regarding the equalization X=MrepNPUSCHNslotsULNRUM, where MrepNPUSCH is the actual repetition number. However, the number of repetitions indicated by the eNB is MrepNPUSCH*Midentical_NPUSCH, which is just similar with multi-tone NPUSCH format 1. |
| OPPO | Regarding the clarification on CDM DMRS equation, we already have the note that “X is the total number of slots in the NPUSCH transmission after OCC is applied”, which is clear. If we change it to X=MrepNPUSCHNslotsULNRUM, the meaning of MrepNPUSCH should be firstly discussed (understanding 1 or 1 in proposal 5_8_1). Regarding the clarification on slot-level OCC scheme, we think the editor is waiting for RAN1’s further agreement based on the email discussion in the last meeting. |
| Vivo1 | We think this discussion is related to resource mapping discussion in 5.8, we may need to postpone the discussion until the resource mapping is finalized. |
| Lenovo | We share the same view as OPPO, the editor is waiting for RAN1 agreement of detail value X, so we should make the decision this meeting. We can’t rely on the editor. |
| ZTE | Regarding first bullet, we are fine for further clarification. But we think MrepNPUSCH need to be clarified first before updating the equation. From our point of view, it should refer to the repetition number indicated by DCI, which is targeting same coverage performance as in legacy. Regarding second bullet, we think it can be beneficial to assist editor for the specification as mentioned by OPPO. |
| FL3 | Assuming that the proposal on “understandings” / length of NPUSCH is agreed in section 5.8.1, i.e. that the length of the NPUSCH transmission with OCC is MrepNPUSCHNslotsULNRU , we can define that X = MrepNPUSCHNslotsULNRU in the following previous agreement: For NPUSCH Format 1 single-tone 15kHz SCS, for CDM DMRS with legacy pattern: - DMRS symbols are spread before the OCC is applied - Option 1_1: according to the formula: ru,OCCMn+m=runqm, 0≤n<X/M, m=0, …, M-1 Where: M is the OCC length, q is the assigned OCC codeword for the UE, run is the reference signal sequence defined in TS36.211 section 10.1.4.1.1 and X is the total number of slots in the NPUSCH transmission after OCC is applied FL thinks that we don’t need a separate agreement on the value of X since this would be covered by the agreement from proposal 5_8_1. |

## Multi-tone OCC scheme

The following views were expressed about the type of OCC scheme that should be supported for multi-tone:

- No: Ericsson

  - Multi-tone would only be applicable in high SNR conditions [Ericsson]

    - High SNR conditions are not an issue since they do not use many resources [Ericsson]

  - Many different numbers of subcarriers leads to complicated specification updates [Ericsson]

- Yes: ZTE, OPPO, Nokia, Spreadtrum, Xiaomi, Lenovo

- Scheme:

  - Slot level [OPPO]

    - Commonality with single-tone[Spreadtrum]

    - Common solution with single-tone [Nok]

  - NSlot level: [Spreadtrum][ZTE][Xiaomi][Lenovo]

    - Minimal spec changes [HW][ZTE][Spreadtrum]

    - Support Nslot = 2 [Xiaomi]

    - OCC lengths:

      - 2: Spreadtrum

      - 4:

- DMRS

  - Different cyclic shifts [Spreadtrum]

  - CDM DMRS [Spreadtrum][Xiaomi]

    - Reuse CDM DMRS scheme from single-tone 15kHz [Spreadtrum]

    - CDM DMRS in the frequency domain [Xiaomi]

- Signalling

  - Use single-tone design for:

    - Indication of OCC index [Spreadtrum]

    - Activation / deactivation [Spreadtrum]

- Multitone performance

OCC2:

|  | Loss to baseline |
| --- | --- |
| Slot-level |  |
| Nslot level | Xiaomi: 1.6dB |

OCC4:

|  | Loss to baseline |
| --- | --- |
| Slot-level |  |
| Nslot level | Xiaomi: 2.6dB |

If multi-tone were to be supported for OCC, there seems to be consensus that a slot-based or Nslot-based approach would be suitable. There are proposals that a multi-tone scheme should maintain commonality with the single-tone scheme, which would favour a slot-based approach. A DMRS scheme would also need to be selected, where the two options are the use of cyclic shifts of the DMRS sequence and use of the single-tone CDM DMRS scheme.

However, there are concerns that multi-tone transmissions aren’t suitable for NTN since they are more suitable high for SNR conditions (that are not really achieved in NTN). In lower SNR conditions, multiplexing more users can be achieved by scheduling single-tone transmissions.

In previous meetings, a lot of time has been spent on choice of the single-tone scheme. This has meant that there has not been time to discuss the multi-tone scheme. In order to agree multi-tone support, there would need to be agreement on the scheme to be applied. FL considers that the following aspects would need to be decided on:

- Basic scheme:

  - Option S1: Slot-based scheme (i.e. the same scheme as for single-tone)

  - Option S2: Nslot-based scheme (Nslot = 2 and the OCC codeword [w1 w2] is applied as w1 * {slot1,slot2} and w2*{slot3,slot4}

- DMRS

  - Option D1: CDM DMRS pattern for single-tone scheme is reused.

  - Option D2: Cyclic shifts of DMRS pattern. Is there spec impact?

- Signalling

  - Use single-tone signalling design

In order to see whether there is common understanding on the multi-tone scheme, FL asks the following questions:

[FL1] Question 5.4-1v1: If multi-tone OCC were supported, which basic scheme would be preferable:

- Option S1: Slot-based scheme (i.e. the same scheme as for single-tone)

- Option S2: Nslot-based scheme (Nslot = 2 and the OCC codeword [w1 w2] is applied as w1 * {slot1,slot2} and w2*{slot3,slot4}

| Company | Comment |
| --- | --- |
| Ericsson | We think that RAN1 should prioritize single-tone since there are still several design aspects for both 15 kHz and 3.75 kHz that need to be concluded (TDM DMRS, CDM DRMS and DCI design). Thus, OCC for NPUSCH Format 1 for multi-tone is not supported in Rel-19. |
| LGE | We prefer Option S2 to minimize specification change. |
| Nokia, NSB | Option S1 for common solution between ST and MT. |
| Xiaomi | We prefer Option S2 with minimum spec impact. |
| OPPO | We prefer Option S1 due to the better decoding performance than Option S2, and a unified solution for single-tone and multi-tone OCC should be supported given the limited time. |
| Lenovo | We prefer Option S2 |
| Spreadtrum | Support option S2. |
| ZTE | Option S2 is preferred with minimum spec impact and implementation impact. |

[FL1] Question 5.4-2v1: If multi-tone OCC were supported, which DMRS scheme would be preferable:

- Option D1: CDM DMRS pattern for single-tone scheme is reused.

- Option D2: Cyclic shifts of DMRS pattern. Is there spec impact?

| Company | Comment |
| --- | --- |
| Ericsson | We need to finalize first the TDM DMRS and CDM DMRS discussions for single-tone, which encompass several proposals to be resolved in this FLS. |
| LGE | If we go with Option S1, we can adopt Option D1. If we go with Option S2, we can adopt Option D2. |
| Nokia, NSB | Option D2 for good orthogonality based on legacy MT design. |
| Xiaomi | Both D1 and D2 could be considered. |
| OPPO | We prefer Option D1 for a unified design between single-tone OCC and multi-tone OCC |
| Spreadtrum | Both D1 and D2 can work. We prefer D1 for unified design. |
| ZTE | Option D2 can have minimized spec impact. |

[FL1] Question 5.4-3v1: If multi-tone OCC were supported, which numbers of tones (3, 6, 12?) would be supported (in addition to single tone)?

| Company | Comment |
| --- | --- |
| Ericsson | None, supporting multi-tone requires more discussions and a deeper analysis. We have started with the support of OCC for single-tone from the beginning of the release and we are not done with it yet, so it feels unfeasible aiming to support multi-tone in the very last meeting of the release. Supporting it will trigger more spec impacts, it will also impact UE Feature list discussions and likely RRC parameter list. |
| LGE | It seems that we cannot use the SIF for indicating OCC activation and OCC sequence index. It would be focusing on single-tone and/or 12 tones. |
| Nokia, NSB | At least for 3 tone case, while better also support 6 tone and 12 tone cases. |
| Xiaomi | At least 6 and 12 tones could be considered. FFS: 3 tones for the DMRS design |
| OPPO | All the number of tones should be supported. |
| Lenovo | All the number of tones can be supported. |
| Spreadtrum | All the number of tones can be support for OCC2. |
| ZTE | All the number can be supported. |

[FL1] Question 5.4-4v1: If multi-tone OCC were supported, can the single-tone signalling mechanism be reused?

| Company | Comment |
| --- | --- |
| Ericsson | Supporting multi-tone requires more discussions and a deeper analysis. It feels unfeasible aiming to support multi-tone in the very last meeting of the release. Supporting it will trigger more spec impacts, it will also impact UE Feature list discussions and likely RRC parameter list. |
| LGE | Depending on how many tones can be considered, some DCI indication may not be applicable. |
| Nokia, NSB | Yes. Good to reuse. |
| Xiaomi | The mechanism could be reused, but considering OCC for single-tone and for multi-tone may be two separate UE features, the RRC parameters could be separately configured. |
| OPPO | It may depend on the signaling design discussed in section 5.7. |
| Spreadtrum | Support to use unified design. |
| ZTE | Depend on the discussion of signaling design for single-tone. |

## UL gaps

The following issues were identified in Tdocs related to UL gaps:

- RAN1 assumes that issues are resolved by scheduler. If this is not true, solutions can be discussed in the maintenance phase [Ericsson]

- Need to align OCC around transmission gaps [QC]

- Need to accurately align UEs if their NPUSCH starting times cause gaps to occur at different times [QC]

- Align OCC DMRS such that they don’t straddle a gap [QC]

- NPUSCH transmissions can only start in slots that are a multiple of NOCC_slots [QC]

  - Proposal 4 in R1-2504413.

    - FL: This could also be relevant to the TDM DMRS alignment

  - If gap occurs within the NOCC_slots, whole set of NOCC_slots is postponed to next set slot that is a multiple of NOCC_slots [QC]

- UL gaps for synchronization (from Rel-13):

  - Phase continuity broken across gap [CMCC][vivo][HW]

  - UE resynchronizes in the gap, so phase continuity will be lost [vivo]

  - HD-FDD UE swaps from UL to DL in gap, which further erodes phase continuity [CMCC]

  - Scheduler solutions: [ZTE]

    - eNB does not schedule UE with OCC if the NPUSCH would span a gap [vivo]

    - Network implementation can resolve issue by deactivating OCC if there is a gap [Spreadtrum]

  - OCC never straddles a gap since gap length is 40ms after 256ms transmission (and pairing must happen at a multiple of 2 slots) [Apple]

  - Postpone NPUSCH with granularity of OCC span [HW]

- Gaps around NPRACH occasions

  - FL: note that “Postponing” is the current rule for NPRACH gaps (see Apple R1-2504345)

  - Drop OCC segment that spans gap[Nordic]

  - Phase continuity not maintained across NPRACH occasion gap [HW]

    - Phase continuity may not be maintained for long NPRACH occasions [CMCC]

  - Postpone segment that would traverse gap [Apple][LGE][HW][CMCC]

    - Postpone only for 3.75kHz SCS, not a problem at 15kHz SCS [Apple]

    - Postpone to the first slot after the gap that is a multiple of 2 [LGE]

    - Postpone NPUSCH with granularity of OCC span [HW][ZTE]

      - Ensures that OCC patterns of both UEs are still aligned after a gap, assuming they were aligned before the gap.

      - FL: note, while the formulation in the ZTE Tdoc is different, I think the outcome is the same.

    - See figure in CMCC R1-2503848

  - For 15kHz, no issue since NPRACH occasion always occurs on even slot and max OCC = 2 [Apple][LGE]

    - Network implementation can resolve issue by deactivating OCC if there is a gap [Spreadtrum]

    - eNB can often avoid collision with NPRACH gap, but we should specify functionality (dropping) if a collision does happen [Nordic]

- UL timing adjustment gaps and segmentation for IoT-NTN (from Rel-17)

  - UE resynchronizes during a gap so phase continuity is not maintained [vivo][CMCC]

  - Drop OCC segment that spans gap [Apple][Nordic][LGE][HW][Nok]

    - Specify as “transmission gap of N_occ time units shall be counted for the NPUSCH resource mapping but not used for transmission of the NPUSCH” [LGE]

  - Scheduler solutions:

    - Scheduler should be able to avoid OCC colliding with this gap [Nordic]

    - Network implementation can resolve issue by deactivating OCC if there is a gap [Spreadtrum]

  - UL TA gaps are not an issue if segment length is a multiple of OCC length (i.e. when mod(Nsegmentprecompensation , OCC-length) = 0) and eNB can decide this [MTK]

  - Consider in design of OCC scheme [CMCC]

  - OCC application should be restricted to within segment [vivo]

  - Pre-segmented compensation segment length should be at least 8ms for 3.75Hz with OCC2 [QC]

    - Avoids the DMRS being excluded from a segment according to new DMRS schemes [QC]

- TDM DMRS that are muted

  - No issue [Spreadtrum][HW][CMCC]

  - Phase rotation impacts on channel estimation need to be considered [Apple]

- Guard periods for 3.75kHz UL transmissions

  - Do not affect phase continuity [ZTE][Apple][Spreadtrum][HW]

- Collision with reserved subframes / slots

  - These are subframes / slots that are reserved by resource reservation

  - Reserved subframe

    - 15kHz: overlap with reserved subframe leads => postponement of subframe [OPPO]

    - 3.75kHz: overlap with reserved subframe => postponement of 4 slots (to maintain TDM DMRS pattern) [OPPO]

  - Reserved symbol

    - Either:

      - SC-FDMA symbols within any part of OCC group that align with reserved symbol are punctured [OPPO]

      - OCC is only enabled if reserved uplink symbols are not configured [OPPO]

The following types of gap have been identified in RAN1#121. FL summary comments on the issues related to these gap types are included.

- UL gaps for synchronization (from Rel-13)

  - Issues identified. Scheduler may be able to resolve issues either by scheduling the start time of the NPUSCH or by constraining the length of the NPSUCH transmission to be shorter than 256ms.

- Gaps around NPRACH occasions

  - Issues identified. Scheduler could potentially avoid these gaps with a restriction on NPRACH occasions. A “postponing” rule could be applied with a granularity of the OCC span.

- UL timing adjustment gaps for NTN (from Rel-17)

  - Issues identified. The network might be able to configure the system such that these gaps do not cause an issue. A “dropping” rule is preferred.

- TDM DMRS that are muted

  - No issue

- Guard periods for 3.75kHz UL transmissions

  - No issue

There are several (potentially related) problems with UL gaps:

- Firstly, it is necessary to ensure that an OCC transmission does not span a gap as this will leave part of the OCC codeword on one side of the gap and the other part of the codeword on the other side of the gap.

- Secondly, UL gaps may occur at different times for different UEs – there may need to be alignment such that both UEs in an OCC pair are transmitting consistently.

- Thirdly, there is likely to be phase discontinuity on either side of an UL transmission gap – this will introduce loss of orthogonality between UEs.

- Fourthly, the DMRS that are associated with OCC codewords should not fall into an UL gap.

It is fairly clear that muted TDM DMRS and guard periods for 3.75kHz UL transmissions do not cause an issue.

For the other UL gap types, RAN1 needs to come to a consensus on whether the scheduler / network can resolve potential issues with the UL gap type and, if not, whether a dropping or postponing rule should be applied.

[FL1] Question 5.5-1v1: Can the network resolve issues related to UL gaps for synchronisation (Rel-13) without specification impact? If not, should a dropping rule or a postponing rule be applied?

| Company | Can network resolve without specification impact | Dropping or postponing rule preferred? |
| --- | --- | --- |
| Example | [Spec changes required to avoid scheduler restrictions] | [Postponing at granularity of OCC span] |
| Ericsson | We believe that at least in some cases, the network should be able to avoid or at least alleviate the issues through a proper OCC scheduling that mind about the presence of gaps. |  |
| LGE | If there is no postponement due to NPRACH, we do not need to have specific enhancement. To be specific, UL gap of 40msec will be added every 256msec transmission. So, it is already multiples of 2 slots or 4 slots for both SCSs. So, if we handles postponement due to NPRACH, the OCC broken issue will be resolved. |  |
| Nokia, NSB | Minimum impact is preferred. | Postpone or drop at granularity of OCC span from the scheduled starting position of the NPUSCH. |
| ZTE | UL gap for synchronization can be overlapped with NPRACH, where NPRACH overlapping with UL gap is counted as UL gap. This will lead to variant gap between two NPUSCH transmissions. Spec impact may be needed to handle such case. | Postpone can be considered |
| SONY1 | Understanding is that this can be handled by the scheduler. If we later find that that is not the case, it can be resolved in the maintenance phase. | Postponing |

[FL1] Question 5.5-2v1: Can the network resolve issues related to Gaps around NPRACH occasions without specification impact? If not, should a dropping rule or a postponing rule be applied?

| Company | Can network resolve without specification impact | Dropping or postponing rule preferred? |
| --- | --- | --- |
| Example | [Spec changes required to avoid scheduler restrictions] | [Postponing at granularity of OCC span] |
| Ericsson | We believe that at least in some cases (NPRACH occasions are configurable and deterministic), the network should be able to avoid or at least alleviate the issues through a proper OCC scheduling that mind about the presence of gaps. |  |
| LGE | It will limit the usage of the sufficiently long NPUSCH transmission. | For 15kHz SCS, the granularity of the postponement is already 2 slots which is the same as the OCC length. So, it does not need to any enhancement. Meanwhile, for 3.75kHz SCS, the granularity of postponement needs to be further increased into 2 slots. Since the NPRACH resource will be the UE common. It does not need to be 4 slots. 2 slots are sufficient. Moreover, depending on the decision on the DMRS TDM pattern, NPUSCH DRMS may not be actually postponed. |
| Nokia, NSB | Minimum impact is preferred. | Postpone or drop at granularity of OCC span from the scheduled starting position of the NPUSCH. |
| ZTE | UL gap for synchronization can be overlapped with NPRACH, where NPRACH overlapping with UL gap is counted as UL gap. This will lead to variant gap between two NPUSCH transmissions. Spec impact may be needed to handle such case. | Postpone can be considered |
| SONY1 | Understanding is that this can be handled by the scheduler. If we later find that that is not the case, it can be resolved in the maintenance phase. | Postponing |

[FL1] Question 5.5-3v1: Can the network resolve issues related to UL timing adjustment gaps and segmentation for IoT-NTN (from Rel-17) without specification impact? If not, should a dropping rule or a postponing rule be applied?

| Company | Can network resolve without specification impact | Dropping or postponing rule preferred? |
| --- | --- | --- |
| Example | [Spec changes required to avoid scheduler restrictions] | [Dropping at granularity of OCC span] |
| LGE | It will limit the usage of the sufficiently long NPUSCH transmission. | Dropping is preferred. Segment gap can be 1 symbol. In this case, for 3.75kHz SCS, dropping 2 symbols would be sufficient. Postponing 2 or 4 slots would be too long. |
| Nokia, NSB | Spec change is needed. RAN1 to check how to make minimum changing. | Drop at granularity of OCC span from the scheduled starting position of the NPUSCH. |
| ZTE | Network can configure segment gap whose length is aligned with time duration for same OCC. Then, no specification impact is needed. |  |
| SONY1 | Spec change is needed since these types of gap occur too often and the gaps can be small (down to 1 symbol). It is hence not possible to schedule OCC solely within a segment – OCC transmissions will traverse gaps. The gaps themselves will “break” the OCC orthogonality for any OCC codeword that spans a gap. | Drop at granularity of OCC span. |

## Other features that should work with OCC

The following issues were raised in relation to other features that OCC should work with:

- EDT

  - Yes: [Spreadtrum]

  - No: Xiaomi

    - Specification impact [Xiaomi]

- PUR:

  - Yes: [Spreadtrum]

    - Easier than for TN since the UE knows the timing from GNSS [TCL]

  - No: [Ericsson][Xiaomi]

    - Consider relationship of OCC to “shared PUR” in Rel-16 [Ericsson]

    - Specification impact [Xiaomi]

    - Shared-PUR in Rel-16 already supports reuse of time / frequency resources through use of orthogonal DMRS sequences [Ericsson]

    - Not clear that OCC UEs can be persistently paired [Ericsson]

      - i.e. over time the suitability of a PUR pairing might change / erode

- CB-Msg3-EDT (R19)

  - RAN2 are already discussing this wrt OCC, so RAN1 should support it

  - Yes: [QC]

  - Power imbalance is not a problem.

    - Simulations show that a 10dB power imbalance leads to only a 0.2dB loss for the lower power UE [QC]

      - Send an LS to RAN2 to inform them of this [QC]

      - FL: Related LS from RAN2 in R1-2503613

  - Need further study: [CMCC]

    - Pairing not possible in IDLE mode [CMCC]

- Msg3:

  - Study performance impact of:

    - Uplink timing alignment [CMCC]

    - Power imbalance [CMCC]

    - Frequency shifting [CMCC]

  - Need to identify the UE capability for OCC for Msg3 [CMCC]

  - Support or not:

    - Yes:

    - No: Xiaomi

      - Do not know OCC capability of UE at initial access, unless PRACH partitioning is applied [CMCC][Xiaomi]

        - Early indication of UE capability [Xiaomi]

      - Specification impact [Xiaomi]

        - OCC index assignment / determination mechanism [Xiaomi]

- Multi-TB:

  - No: Ericsson

    - Too much spec impact (clauses 16.4.1, 16.4.2, 16.5, 16.5.1, 16.5.1.2, and 16.6 of TS36.213)

  - Yes: QC

    - Both TBs used the same OCC index

    - Unit if interleaving multiplied by M for interleaved multi-TB: g=M NRUNslotsUL

In order to get an overall impression of company views on which features should be supported with OCC, companies are invited to indicate which are the priority features that should be supported with OCC.

[FL1] Question 5.6-1v1: From a RAN1 perspective, which are the priority features to support with OCC?

| Company | EDT | PUR | CB-Msg3-EDT | Msg3 | Multi-TB |
| --- | --- | --- | --- | --- | --- |
| Example | low | low | high | Not possible | high |
| Ericsson | low | low | high | low | low |
| LGE |  |  | It would be better to understand RAN2’s assumption. | Not preferred. It will change RAR message design a lot. |  |
| Nokia, NSB | Low | low | low | low | low |
| Xiaomi | low | low | Same view as LGE | low | Not possible |
| OPPO | low | low | Already agreed | low | High |
| SONY1 | low | low | Depends on RAN2 | low | high |

Companies are invited to make more general comments on features that should be supported with OCC in the following table:

| Company | Comment |
| --- | --- |
| Ericsson | At this point of the release, we can consider “CB-Msg3-EDT” as high given the LS we got from RAN2. |
| OPPO | At this stage, we should focus on the features already agreed to support OCC, including dynamic grant in RRC CONNECTED and CB-Msg3-EDT. Other features to support OCC should not be pursued in R19 IoT NTN. |
| NEC | Agree with Ericsson and OPPO. |
| ZTE | For multi-TB, there is limited spec impact to be supported together with OCC. Hence, multi-TB with OCC seems to be supported by default. |

## Signalling

The following issues related to signalling were identified in RAN1#121:

Signalling

- DCI signalling

  - Number of bits required:

    - 2 bits or 3 states [Apple][HW]

      - 3 code points are needed [1 1], [1 -1] and deactivated. [Apple]

  - #OCC sequence index

    - New field [Samsung]

      - When new field is applied, another field is not configured [Samsung]

    - Repurpose existing fields [ETRI]

  - DCI size:

    - Don’t change [Nok]

    - Alignment between 3.75kHz and 15kHz DCI sizes [QC]

  - Extra fields to signal:

    - OCC factor (M) [QC]

      - FL: the choice seems to be between M=1 (legacy) and M=2. Isn’t this the same as activation / deactivation, which has already been agreed?

  - Views on fields:

    - SIF:

      - Spare code points at 15kHz SCS [Sharp][LGE][CMCC]

      - 3 code points are needed [1 1], [1 -1] and deactivated. These can be taken from SIF field [Apple][ZTE]

      - 3.75kHz SCS limitations:

        - 16 subcarriers can be indicated at 3.75kHz for OCC if indication is taken from SIF field [Apple][CMCC]

        - Not enough code points to indicate 1 bit (at 3.75kHz SCS) [Spreadtrum][LGE]

        - 3.75kHz SCS: use SIF field along with MCS field [HW][QC]

          - Split UEs into two groups of available 3.75kHz subcarriers via RRC signalling [HW]

        - Not all 48 possible subcarriers are allocated to NPUSCH anyway – some are allocated to NPRACH. Hence we do not need to signal all 48 possible legacy allocations [Ericsson]

      - 6 states used for legacy allocations, 48 states used for OCC allocations [Nordic]

      - All subcarriers can be indicated at 15kHz ST and MT [Apple][Nordic][HW]

        - If REP1 then use legacy SIF table, some codepoints considered for legacy, some for MT, some of OCC[1 1] and some for OCC [1 -1]

      - SIF re-purposing would lead to different designs for 3.75kHz SCS and 15kHz SCS [Xiaomi]

      - Different designs: [ZTE]

        - Use SIF field at 15kHz field (spare codepoints can be repurposed)

        - Use MCS and RV fields at 3.75kHz

      - FL: Should we have an agreement that says that we should strive for the same design for both 15kHz and 3.75kHz?

    - RV:

      - Can’t be used as this would disable RV signalling feature [Spreadtrum][Nok]

        - FFS whether it can be reused, before design is completed [Ericsson]

      - Can reuse. [Lenovo][Nordic][LGE]

        - With repetitions it doesn’t matter whether RV0 is first or RV2 is first [Lenovo]

        - Fix RV as RV0 [vivo][OPPO]

        - Joint signalling of OCC with RV [Nordic][Xiaomi]

          - OCC [1 1]: starting RV = 0

          - OCC [1 -1]: starting RV = 2

        - FL: note that there seem to be two options:

          - RV field is replaced with OCC field

          - Joint coding of RV and OCC field

    - MTB:

      - Can’t be used as this would disable multi-TB feature [Spreadtrum][vivo][Nok]

        - Multi-TB feature is feasible for OCC and should be supported [vivo]

      - No need to combine OCC feature with MTB feature [MTK]

      - Can’t be repurposed as field is not always present [CMCC]

      - MTB is useful when there are capacity limitations and that is what this WI is targeting, hence do not repurpose MTB field [Xiaomi][Nok]

    - MCS

      - Doesn’t change much in NTN [Spreadtrum]

      - High MCS not used in NTN [TCL][Sharp][TCL][Nok]

        - OCC may degrade performance, meaning high MCS are not applicable [Xiaomi]

      - High MCS not used in OCC when repetitions are applied [MTK][Xiaomi]

      - Using an MCS bit would lead to scheduling restrictions, but acceptable [vivo]

      - All current MCS values are needed (no 16QAM in baseline table) [Ericsson]

    - Number of NPUSCH Repetitions

      - Doesn’t change much in NTN [Spreadtrum][Sharp]

      - Using this field would introduce scheduling restrictions [vivo]

      - Which REP codepoints to reuse:

        - Low REP values are not used in NTN [Xiaomi][Nok]

          - FL: Implies low REP values are deleted from table

        - Only low REP values would be supported and we might need to support higher REP values [Ericsson]

          - FL: Implies high REP values are deleted from table

    - Resource reservation

      - RR needs to be supported to allow for support of IoT-NTN in NR carrier [vivo][Nok]

        - See R2-2503064 “Reply LS on NR NB-IoT in-band operation”, where RAN2 say that IoT-NTN in NR carrier is feasible [vivo]

        - Hence RR field cannot be repurposed [vivo]

      - Use of this field would preclude resource reservation between OCC and NR

      - Can’t be repurposed as field is not always present [CMCC][Xiaomi]

      - RR is not compatible with OCC [OPPO]

      - Reuse resource reservation field [QC]

    - DCI Repetition number

      - UE assumes that the NPDCCH is always transmitted with the maximum REP factor in order to free up 1 bit [LGE]

| Company | MCS | REP | RV | MTB | SIF | RR | DCI REP | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Samsung |  |  |  | yes |  | yes |  |  |
| Spreadtrum | yes | yes | no | no | no |  |  | MCS and repetition number do not change significantly for NTN |
| TCL | yes |  |  |  |  |  |  | High MCS not used in NTN |
| CATT | yes | yes |  |  |  |  |  | High MCS not used in NTN. Low REP not used in NTN |
| Sharp |  | yes |  |  | Yes |  |  | SIF: considered 15kHz and MT, not 3.75kHz. Small variation in required number of repetitions |
| Mediatek | yes |  |  | yes |  |  |  | High MCS not used in OCC |
| Apple |  |  |  | yes |  |  |  | 2 bits from SIF field. |
| Lenovo | yes |  | yes |  | yes |  |  | RV: doesn’t matter whether RV0 is first or RV1 Consider joint encoding of fields |
| Nordic |  |  | yes |  | yes |  |  | SIF: can indicate all allocations for 15kHz ST and MT for OCC and legacy RV index and OCC index are indicated jointly |
| LGE |  |  |  |  | yes |  | yes | DCI REP: UE assumes max REP always applied to DCI |
| HW | yes |  |  |  | yes |  |  | 15kHz: just use SIF 3.75kHz: SIF + MCS; interpretation of SIF is UE-specific and signalled by RRC |
| vivo | yes |  | yes | no |  |  |  | RV: fix RV field as e.g. RV0 |
| CMCC | yes | no | yes | no | yes | no |  | REP: need to provide for a large number of repetitions |
| Xiaomi | yes | yes | yes | no | no | no |  | Low REP values are not useful in NTN RV: linkage between RV and OCC index |
| Nokia | yes | yes | no | no | yes | yes |  |  |
| OPPO |  |  | yes |  |  | yes |  | RV: always start with RV0 |
| ZTE | yes |  | yes |  | yes |  |  |  |
| Ericsson | no | maybe |  | yes | yes | no |  | Includes TP |
| QC | yes |  |  |  | yes | yes |  | Joint encoding of SIF and MCS |

- RRC signalling

  - OCC sequence can be based on RRC configuration and DCI-based configuration [CMCC]

  - Implicit activation

    - Based on DCI field being in a particular range configured by RRC => OCC is activated / certain OCC codeword index is used

  - RRC signalling for PUR, CB-Msg3-EDT (or other semi-statically configured transmission)

The following agreement was made at RAN1#118bis Hefei:

Agreement For support of single-tone OCC for NPUSCH format 1, RAN1 studies: - The parameters that need to be signalled, considering the following: - OCC codeword - Enabling of OCC feature - FFS: other parameters - For dynamic grant in RRC CONNECTED, study which and whether any parameters are signalled via DCI and which and whether any parameters are signalled by RRC - FFS: whether/how to support cases other than dynamic grant in RRC CONNECTED, e.g. Msg3, PUR, CB Msg3 EDT

The following agreement was made at RAN1#119 Orlando:

Agreement For support of single-tone OCC for NPUSCH format 1 for connected mode, the parameters that need to be signalled are: - OCC sequence index - Enabling of OCC feature - FFS: whether signaling is explicit or implicit

The following agreement was made at RAN1#120 Athens. The final FLS for RAN1#120 [2] includes the following note from FL: “FL’s understanding of the discussion among delegates and the session chair is that the above agreement does not preclude future discussion of whether OCC can be dynamically activated / deactivated. This activation / deactivation can be considered as a part of a 2-step enabling / disabling.”

Agreement For CONNECTED mode, UE-specific RRC signalling is used for enabling of the OCC feature.

The following agreement was made at RAN1#120bis Wuhan:

Agreement For indicating OCC sequence index and activation / deactivation, the following fields may be repurposed or constrained to be not present in the DCI: - Modulation and coding scheme - Repetition number - Redundancy version - Number of scheduled TB for Unicast - Subcarrier indication - Resource reservation

### Common design for 3.75kHz SCS and 15kHz SCS

An initial issue to resolve is whether there should be a common signalling design for 3.75kHz SCS and 15kHz SCS (should the same fields be repurposed for both SCS?).

[FL1] Proposal 5.7.1-1v1: RAN1 strives for a common signalling design for 3.75kHz SCS and 15kHz SCS NPUSCH format 1.

Companies are invited to comment on proposal 5.7.1-1.

| Company | Comment |
| --- | --- |
| Ericsson | Ok (if possible) |
| LGE | If multi-tone NPUSCH with OCC is not supported, we can simply reuse the SIF for 15kHz SCS. |
| Nokia, NSB | Good to have common design. |
| Xiaomi | Support. |
| OPPO | We support this proposal. |
| Spreadtrum | Support |
| ZTE | Not needed. For 15kHz SCS, SIF have larger number of reserved bits and can indicate the OCC without necessity to reuse other bit fields. We should not preclude such solution to achieve common solution with 3.75kHz SCS. |
| SONY1 | Support. DCI design will be a mess if we have different designs for 3.75kHz and 15kHz – and we don’t see a technical benefit for having different designs. |

### Choice of DCI fields for repurposing

In order to keep the DCI size the same as for non-OCC operation, 2 bits (or more specifically, 3 states [no OCC, OCC with [1 1], OCC with [1 -1]) need to be made available for OCC operation. From input Tdocs, the following preferences for fields that can be repurposed were expressed (the xtable includes only those parameters that were agreed for consideration in RAN1#120bis Wuhan).

| Company | MCS | REP | RV | MTB | SIF | RR | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Samsung |  |  |  | yes |  | yes |  |
| Spreadtrum | yes | yes | no | no | no |  | MCS and repetition number do not change significantly for NTN |
| TCL | yes |  |  |  |  |  | High MCS not used in NTN |
| CATT | yes | yes |  |  |  |  | High MCS not used in NTN. Low REP not used in NTN |
| Sharp |  | yes |  |  | Yes |  | SIF: considered 15kHz and MT, not 3.75kHz. Small variation in required number of repetitions |
| Mediatek | yes |  |  | yes |  |  | High MCS not used in OCC |
| Apple |  |  |  | yes |  |  | 2 bits from SIF field. |
| Lenovo | yes |  | yes |  | yes |  | RV: doesn’t matter whether RV0 is first or RV1 Consider joint encoding of fields |
| Nordic |  |  | yes |  | yes |  | SIF: can indicate all allocations for 15kHz ST and MT for OCC and legacy RV index and OCC index are indicated jointly |
| LGE |  |  |  |  | yes |  | DCI REP: UE assumes max REP always applied to DCI |
| HW | yes |  |  |  | yes |  | 15kHz: just use SIF 3.75kHz: SIF + MCS; interpretation of SIF is UE-specific and signalled by RRC |
| vivo | yes |  | yes | no |  |  | RV: fix RV field as e.g. RV0 |
| CMCC | yes | no | yes | no | yes | no | REP: need to provide for a large number of repetitions |
| Xiaomi | yes | yes | yes | no | no | no | Low REP values are not useful in NTN RV: linkage between RV and OCC index |
| Nokia | yes | yes | no | no | yes | yes |  |
| OPPO |  |  | yes |  |  | yes | RV: always start with RV0 |
| ZTE | yes |  | yes |  | yes |  |  |
| Ericsson | no | maybe |  | yes | yes | no | Includes TP |
| QC | yes |  |  |  | yes | yes | Joint encoding of SIF and MCS |

FL’s summary / opinion on the various potential fields that could be repurposed are:

| Field | Field title | # supporting companies | Issues |
| --- | --- | --- | --- |
| MCS | Modulation and coding scheme | 12 | High MCS not required when repetition is applied (repetition is a prerequisite for OCC). One bit could be repurposed if high MCS are not allowed. |
| REP | Repetition number | 5 | Range of coverage (min to max) is limited in NTN, hence fewer bits needed for REP field in NTN. Need to decide whether low REP or high REP would be precluded. |
| RV | Redundancy version | 6 | For repetitions, it doesn’t really matter whether RV0 is first or RV1 is first. Decision would need to be made on whether RV0 is always first RV or whether first RV is related to OCC index. |
| MTB | Number of scheduled TB for Unicast | 4 | Many companies think that it should be possible to support both OCC and multiple transport blocks at the same time. MTB helps increases capacity by reducing the DL control signalling. |
| SIF | Subcarrier indication | 10 | Spare code points at 15kHz. Fewer spare code points at 3.75kHz, which may lead to a complicated design. |
| RR | Resource reservation | 4 | Several companies want to allow for resource reservation, allowing IoT-NTN to operate within an NR NTN carrier. |

While SIF has more support than RV, FL thinks that repurposing SIF will lead to more specification work in the limited time available, hence FL proposes that the MCS and RV fields are repurposed for OCC signalling.

[FL1] Proposal 5.7.2-1v1: The modulation and coding scheme DCI field is repurposed for indicating OCC parameters.

- MCS indices 8, 9 and 10 in Table 16.5.1.2-1 of TS36.213 are not used when OCC is applied

Companies are invited to comment on proposal 5.7.2-1.

| Company | Comment |
| --- | --- |
| Ericsson | We are open to discuss. From the table summarizing views, we can see other DCI fields with similar number of supporters (e.g., subcarrier indication fields), those fields should be considered for discussion too. It would be good to discuss how Modulation and coding scheme and the subcarrier indication is intended to be done in the case of OCC for 3.75 kHz SCS and 15 kHz SCS. |
| LGE | In our understanding, if the OCC feature is deactivated, the NPUSCH transmission would be the similar or the same as the NPUSCH transmission with OCC feature disabled. In our understanding, if the OCC feature is enabled, but not activated, all the MCS needs to be used. So, “OCC is applied” would need to be clarified as “the OCC feature is activated”. |
| Nokia, NSB | OK |
| Xiaomi | Fine. |
| Spreadtrum | Fine with proposal |
| ZTE | Fine with proposal for 3.75kHz SCS. For 15kHz SCS, SIF should be considered. |
| SONY1 | Agree with LGE that this would preclude use of MCS 8,9,10 even when OCC is deactivated (assuming OCC is enabled). However, we think that this is not a big issue since company assumptions are that high MCS is not applied in NTN (and that is not specific to whether OCC is applied in NTN or not). |

[FL1] Proposal 5.7.2-2v1: The redundancy version DCI field is configured to be not present in DCI when the OCC feature is enabled.

Companies are invited to comment on proposal 5.7.2-2.

| Company | Comment |
| --- | --- |
| Ericsson | It has not been decided yet how the redundancy version is going to be used for OCC-based transmissions. Thus, unless until the usage of the redundancy version for OCC gets clarified it is better to keep this legacy field unmodified. |
| LGE | OK |
| Nokia, NSB | Not support. We prefer to reuse the field that does not impact the performance. We suggest to discuss SIF firstly as most companies support. |
| Xiaomi | Fine with this proposal in general. But one issue needs to be considered: how to understand “when OCC feature is enabled”? Whether the OCC feature is enabled by RRC signalling, or it enabled by other field in DCI? For example, it is enabled by the 1MSB field of MCS, and then using RV field to indicate the OCC sequence index. |
| OPPO | Support, and we can also say, when the OCC feature is enabled, the RV filed is repurposed for indicating OCC parameters. In this case, the NPUSCH transmission is initiated with RV0, similar as PUR NPUSCH transmission. |
| Spreadtrum | Not support. It not clear whether RV field is needed for OCC-based transmission. |
| ZTE | Fine with proposal for 3.75kHz SCS. For 15kHz SCS, SIF should be considered. |
| SONY1 | OK We think that the RV field is not important with repetition. Starting a transmission with RV0 will lead to the same performance as starting with RV1. |

### Proposal for DCI design

Based on discussions above and during the offline session, it is proposed that the following approach is adopted:

- If there are no repetitions, the interpretation of DCI is interpreted as per legacy

  - Rationale: OCC only works when there are repetitions. When there are no repetitions, the full functionality of the DCI can be used, such as scheduling higher MCS and varying the redundancy version: these are functions that are required when there are no repetitions (good link budget), but are not required when there are repetitions

- For 15kHz SCS, the reserved states in the subcarrier indication field are used to indicate location of subcarriers, OCC sequence index and OCC activation / deactivation

- For 3.75kHz SCS:

  - A redundancy version field is repurposed to indicate OCC activation / deactivation

    - Rationale: redundancy version is not useful when repetitions are applied. RV is not a configurable field, so it is repurposed

  - The fields “subcarrier indication” and “modulation and coding scheme” are jointly encoded to indicate the location of the subcarriers, the value of ITBS and the OCC sequence index

    - Rationale: Use of SIF field aligns with 15kHz SCS, higher MCS values are not used with repetitions. MCS and SIF provide 10 bits = 1024 states, allowing for signalling 48 subcarrier locations, 10 values of ITBS and 2 OCC codewords (48*10*2 = 960; 960 < 1024)

[FL3] Proposal 5.7.3-1v1:

- If the number of repetitions of NPUSCH is equal to 1, the DCI is interpreted as per legacy

- For 15kHz SCS:

  - the reserved states in the subcarrier indication field are used to indicate the location of subcarriers, OCC sequence index and OCC activation / deactivation

- For 3.75kHz SCS:

  - The redundancy version field is repurposed to indicate OCC activation / deactivation

  - The fields “subcarrier indication” and “modulation and coding scheme” are jointly encoded to indicate the location of the subcarriers, the value of ITBS and the OCC sequence index

Companies are invited to comment on proposal 5.7.3-1.

| Company | Comment |
| --- | --- |

v

## Physical Channel Aspects

The following issues were raised with respect to physical channel mapping aspects:

Physical channel mapping

- Resource

  - Increase RU size

    - Super-RU = M RUs [QC]

    - Avoids a reduction of coding rate [QC]

  - RU size same as R18 [vivo][HW][Xiaomi][ZTE]

- RV issues

  - Increase RV size

    - Super-RV = NRU super-RUs [QC]

      - FL: Huawei phrase this differently, but I understand that they have the same unerstanding

    - Avoids a reduction of coding rate [QC]

    - Circular buffer manipulation done at a super-RC basis [QC]

      - FL: does this require changes to transport channel processing?

    - RV length in time increased by OCC factor [Lenovo][HW]

      - HW proposal (proposal 2 in R1-2503283) is:

        - “For Symbol-level OCC applied to 3.75kHz SCS single tone and Slot-level OCC applied to 15kHz SCS single-tone, the same RV value is used within an OCC group for NoccNRUNslotsUL slots. RV cycling can be performed across the OCC groups.”

    - Changes to RV should minimise spec impact [Nok]

- TBS determination

  - Based on existing ITBS and IRU procedure [HW][Lenovo][Xiaomi]

    - FL: i.e. TBS is based on the ITBS and IRU that are signalled in the DCI

  - Consider whether this is based on the allocated number of RUs or the number of OFDM symbols in a slot are scaled by the OC length [OPPO]

- Spreading

  - Define spreading function in physical channel mapping of [TS36.211]: [Oppo]]

    - “after mapping to Nsymbs=1 SC-FDMA symbols, the Nsymbs SC-FDMA symbols shall be repeated NSF-1 additional times, before continuing the mapping of z(⋅) to the following SC-FDMA symbols.”

  - Define spreading function and then OCC function [ZTE]

    - FL: understand that this is compatible with OPPO view.

- Repetitions

- understanding 1:  MrepNPUSCH=M×Mrepdata

    - Scheduled number of repetitions = OCC length * data repetitions [TCL]

      - This avoids the total transmission length for OCC being longer than for legacy [TCL][ETRI][Lenovo]

        - Means we cannot schedule REP128 and OCC2

          - i..e. coverage is not increased by this option

      - Actual number of repetitions should be scaled down by a factor of 2 [Lenovo]

      - Some codepoints cannot be indicated (e.g. REP1 is not useful) [vivo]

- understanding 2:  MrepNPUSCH =Mrepdata

    - Scheduled number of repetitions relates to the number of data repetitions

      - Leads to coverage enhancement, which is not an objective of WID (max rep = 128 and OCC2 can be applied leading to an overall repetition of 256) [Lenovo]

  - Preference:

    - Understanding 1 MrepNPUSCH=M×Mrepdata: TCL, ETRI, Lenovo, Nordic, HW(??), Xiaomi

    - Understanding 2  MrepNPUSCH =Mrepdata: vivo

### Repetitions

In the final FLS [2] for RAN1#120bis, the following proposal was made:

RAN1_120bis proposal

When [Nrep  in 213 or] MrepNPUSCH [in 211] is the value from the repetition field of the DCI, M is the OCC factor and  Mrepdata is the number of repetitions of the data before OCC itself, which of the following understandings applies:

- understanding 1:  MrepNPUSCH=M×Mrepdata

- understanding 2:  MrepNPUSCH =Mrepdata

Most companies agree with understanding 1 for the meaning of the number of repetitions. This understanding does not create unintended coverage enhancement for the IoT-NTN feature. Hence, the following proposal is made:

[FL1] Proposal 5_8_1v1:

When [Nrep  in 213 or] MrepNPUSCH [in 211] is the value from the repetition field of the DCI, M is the OCC factor and  Mrepdata is the number of repetitions of the data before OCC itself, the following understanding applies:

- understanding 1:  MrepNPUSCH=M×Mrepdata

Companies are invited to comment on proposal 5_8_1.

| Company | Comment |
| --- | --- |
| Ericsson | This can be discussed together with the OCC reference signal sequence length for both TDM DMRS and CDM DMRS, since for both the variable “MrepNPUSCH” is present in 0≤n<MrepNPUSCHNslotsULNRU. |
| LGE | OK |
| Nokia, NSB | We prefer the case with less impact to spec. From this point, understanding 2 may be better. RAN1 should discuss the impact to spec before making decision. |
| Qualcomm | Our understanding is that signaling OCC length = 2 would make the transmission be twice as long as if the OCC length was equal to 1. However, we are not sure of the equation above: in our understanding, the length of each repetition itself will be 2x as long. |
| Xiaomi | Support. These factors should be aligned in Section 5.3 and in this section. In section 5.3, there is: X=MrepNPUSCHNslotsULNRUM. I think where MrepNPUSCH should be updated as Mrepdata here, and M equals to the OCC length. |
| OPPO | We support this proposal. The OCC factor should be counted into the number of repetitions Nrep (in TS36.213) indicated in the DCI field. That is, the OCC is enabled when Nrep≥2. |
| Vivo1 | For understanding2, if the number of repetitions indicated in DCI is halved, and more RU are scheduled for a codeword, it can maintain the same code rate and time spanning as the case without OCC. In this case, there would be no coverage enhancement as the number of repetitions after OCC does not increase. For example, assuming , MrepNPUSCH=4 for PUSCH without OCC Figure 1. example of the legacy pattern (without OCC) for NPUSCH format 1 single-tone with 15kHz SCS To maintain the same code rate with legacy pattern in Figure 1, more RUs could be scheduled for the PUSCH codeword as example in Figure 2, with assuming , MrepNPUSCH=2. In the end, the same TB is repeated 4 times after OCC. Figure 2. example of expected slot-level OCC for NPUSCH format 1 single-tone with 15kHz SCS We believe that with understanding2, the definition of MrepNPUSCH remains unchanged, and therefore M_identical is not affected by OCC, resulting in fewer specification changes. Understanding 1 can also work, but it would require additional specification modifications, e.g., changes on M_identical as proposed by some companies. Having said that, we can live with the majority’s preference |
| Lenovo | We share the same value as OPPO, and We can directly determine the value of X instead, and there is no need to discuss this proposal. |
| NEC | Understanding 2 is better. |
| ZTE | It seems not necessary to introduce new concept of M_rep^data. What matters is that M_rep^NPUSCH should refer to the value indicated by DCI, and is equal to all the repetitions including the OCC spread. |
| SONY1 | OK. We are actually OK with either understanding 1 or understanding 2. Once we have a common understanding, equations can be made to be consistent with that understanding. |
| FL3 | The offline discussion showed that companies do not have a consistent understanding of the proposal. Understanding 1 leads to the total length of the NPSUCH being the same for OCC and non-OCC for a given value of the repetition field in the DCI (which is the parameter MrepNPUSCH). A consensus was that RAN1 just tries to agree on the length of the NPUSCH transmission: The length of the NPUSCH transmission is MrepNPUSCHNslotsULNRU, where the parameters are as defined in TS36.211 section 3.1. Note that the definitions are: Scheduled number of repetitions of a NPUSCH transmission Number of scheduled UL resource units for NB-IoT Number of consecutive slots in an UL resource unit for NB-IoT Hence, the following proposal is made: total number of slots in the NPUSCH transmission after OCC is applied [FL3] Proposal 5_8_1v2: The total number of slots in the NPUSCH transmission after OCC is applied is MrepNPUSCHNslotsULNRU , where the parameters have the legacy definitions in TS36.211. |

### RU size

A majority of companies think that the RU size should be the same as R18. The following proposal is made:

[FL1] Proposal 5_8_2v1:

- For OCC, the RU size is the same as for Release-18.

Companies are invited to comment on proposal 5_8_2.

| Company | Comment |
| --- | --- |
| Ericsson | We think that any decision around it also depends on the out of the previous proposal (and the proposals touching upon the OCC reference signal sequence length). |
| LGE | Some clarification would be needed. I think the intention would be the RU size for determining the TBS. Mapping resource itself would be increased. |
| Nokia, NSB | Support. |
| Qualcomm | Agree with LGE |
| Xiaomi | Support. |
| OPPO | Support. |
| Vivo1 | yes |
| Lenovo | Support |
| NEC | Support |
| Spreadtrum | Support |
| ZTE | Support. |
| SONY1 | Decide on “understandings” in section 5.8.1 first. RU size before spreading is the same |

### RV size

A majority of companies think that the RV length in time should be increased by the OCC factor relative to R18. The following proposal is made (from R1-2503283 – Huawei):

[FL1] Proposal 5_8_3v1:

For Symbol-level OCC applied to 3.75kHz SCS single tone and Slot-level OCC applied to 15kHz SCS single-tone, the same RV value is used within an OCC group for NoccNRUNslotsUL slots. RV cycling can be performed across the OCC groups.

Companies are invited to comment on proposal 5_8_3.

| Company | Comment |
| --- | --- |
| Ericsson | Is the intention of the following sentence “RV cycling can be performed across the OCC groups” to say “The same RV cycling can be performed across the OCC groups”? |
| LGE | OK |
| Nokia, NSB | We support to use same RV within one OCC group. For RV cycling, it should be discussed together with other issues, and better have less impact to spec. |
| Qualcomm | OK |
| Xiaomi | Generally fine with this proposal. But it can also be modified as follows to keep the factors alignment between these several proposals: For Symbol-level OCC applied to 3.75kHz SCS single tone and Slot-level OCC applied to 15kHz SCS single-tone, the same RV value is used within an OCC group for M*NRU*NslotsUL slots. RV cycling can be performed across the OCC groups. |
| OPPO | Support. |
| Lenovo | Support |
| ZTE | OK |
| SONY1 | Agree with the update from Xiaomi. |

### TBS determination

The majority of companies propose that the TBS is determined based on the existing ITBS and IRU procedure. Hence the following proposal is made:

[FL1] Proposal 5_8_4v1:

The TBS is determined based on the existing ITBS and IRU procedure.

Companies are invited to comment on proposal 5_8_4.

| Company | Comment |
| --- | --- |
| Ericsson | Ok |
| LGE | OK |
| Nokia, NSB | Support. |
| Qualcomm | OK |
| Xiaomi | Support. |
| OPPO | Support. |
| Lenovo | Support |
| NEC | Support |
| Spreadtrum | Support |
| ZTE | Support |
| SONY1 | OK |

### Spreading

Some companies would prefer an explicit agreement that a spreading function is explicitly applied in the physical channel mapping procedure:

[FL1] Proposal 5_8_5v1:

A spreading function (by a factor of the OCC length) is applied before the OCC codeword is applied.

Companies are invited to comment on proposal 5_8_5.

| Company | Comment |
| --- | --- |
| Ericsson | It is unclear to us what is meant by the statement in this proposal. |
| LGE | It would be up to editor. |
| Nokia, NSB | Generally it should be fine while we think it is better to mention the spreading function is applied based on OCC codeword, as CDMA. |
| Xiaomi | Fine. |
| OPPO | We support this proposal in principle, but do we need to provide more details on the spreading function? For example, for 3.75kHz SCS OCC, after mapping to Nsymbs=1 SC-FDMA symbols, the Nsymbs SC-FDMA symbols shall be repeated NSF-1 additional times, before continuing the mapping of z(⋅) to the following SC-FDMA symbols. |
| ZTE | The intention is not clear. In our view, OCC is applied based on repetition. Hence, the repetition mapping can be performed first, and then OCC is used to cover the repetitions. |
| SONY1 | We are ambivalent about this. We are OK with this proposal if it is considered to provide clarification. |

## Downlink Issues and alignment

The following issues related to the downlink have been identified:

- Alignment of NPUSCH requires staggered NPDCCH, requiring new k0 values (subframes between NPDCCH and NPUSCH) [Ericsson]

  - Additional offset: k0 + {1,2,4,8,16} to allow for NPPDCH repetitions [Ericsson]

A related issue is that of alignment of the OCC transmissions in general, where the following issues were identified:

- Alignment of NPUSCH requires staggered NPDCCH, requiring new k0 values (subframes between NPDCCH and NPUSCH) [Ericsson]

- Reference point in time introduced to align OCC from different UEs [TCL]

  - Helps with UL gaps [TCL]

  - OCC transmissions only start in slots that are multiples of NOCC_slots [QC]

    - 3.75kHz: NOCC_slots = 4

    - 15kHz: NOCC_slots = 2

- RAN1 to discuss how to align OCC transmissions from different UEs [Nok]

- Alignment of adding new UEs with OCC to existing group of UEs already doing OCC [QC][Nok]

- Cannot just rely on scheduling to get alignment [Nok]

The following proposal was not considered in previous RAN1 meetings and FL proposes that we further consider this proposal in order to progress alignment and DL signalling issues. The proposal covers both the “downlink” issues and the “alignment” issues:

[FL1] Proposal 5.9-1: For NPUSCH format 1 single-tone for both 15kHz and 3.75kHz, RAN1 supports for following for alignment of OCC transmissions:

- Modification to scheduling delays (k0) between NPDCCH and NPUSCH, considering appropriate repetition values for NPDCCH

- Definition of a reference point in the slot structure to align OCC transmissions

Companies are invited to comment on proposal 5.9-1.

| Company | Comment |
| --- | --- |
| Ericsson | We think that this is an important proposal to be discussed given that DL is not orthogonal and that currently in legacy there are only 4 scheduling delays values available (this leads to a very restrictive allocation and no efficient use of time-frequency resources). |
| LGE | Regarding the 1st sub-bullet, we only support OCC capacity of 2. Considering the typical scenario on the NPDCCH repetition and allowing NPUSCH transmission with different starting position can be CDMed, this kind of modification need to be deprioritized. Regarding the 2nd sub-bullet, if it allows a single OCC sequence is broken, then it would not be helpful for multiplexing with OCC. |
| Nokia, NSB | We think flexible scheduling in time domain for NPUSCH with OCC is important. Additional k0 (0 or 1) for scheduling of NPUSCH is needed to align the NPUSCH with OCC pairing, especially when UE’s NPUSCH have different starting time and repetition number. We think the second bullet should be removed as it will have big restriction on scheduling in different cases especially when a cell serves both UE with OCC and UE without OCC. |
| Xiaomi | We can’t see the need to introduce the new value of k0 since the position of DCI and the value k0 (with several candidate values in the current specification) could be properly determined by the gNB. |
| SONY1 | We are OK with the first sub-bullet, since we think that it is important to allow flexibility in the start time of NPUSCH. Another (additional approach) to improving DL scheduling flexibility is option 2_1 in section 5.2.3: option 2_1 avoids added restrictions on potential start times of NPSUCH. |

## Others

Are there any other issues that RAN1 needs to cover with priority in RAN1#121 Malta?

[FL1] Question 5.10-1: Are there any other issues that should be covered in RAN1#121 Malta that are not so far addressed in this FLS?

| Company | Comment |
| --- | --- |
| OPPO | Regarding the collision issue between NPUSCH and reserved subframes / slots, it is captured in section 5.5, but there is not related proposal in the FL summary. Therefore, some discussion may be needed. |

# Monday: Proposals for offline session

Proposal 5_2_1v1: Confirm the following working assumption from RAN1#120 Athens:

“For 3.75kHz SCS OCC for NPUSCH format 1, support TDM DMRS over 4 slots where DMRS are transmitted in the first 2 slots and DMRS REs are blanked in the next 2 slots, or vice-versa, where the DMRS REs are as in legacy NB-IoT and the guard period within the slot is as in legacy NB-IoT.”

Agreement

For 3.75kHz SCS OCC for NPUSCH format 1, RAN1 down selects between the following mappings between DMRS sequence samples and active TDM DMRS slots:

- Option 1: Sequential mapping of samples of the original DMRS sequence to active DMRS slots

  - QC, SONY, OPPO, Nordic, LGE,

- Option 2: Dropping of samples of the original DMRS sequence in blanked slots

  - MTK, IDC, HW, ZTE, Apple, Nok, Samsung, Xiaomi, CATT, Lenovo, vivo

QC: drop: 2 UEs in different cells have same DMRS sequence. Sequence at eNB is not legacy when staggering accounted for

Sequential is pseudorandom in NC. This is equivalent functionality to what we have done for 15kHz SCS DMRS design

LGE: agree with QC

OPPO: option 2 can lead to alignment (bad)

Technical merit:

- Option 1: it works

- Option 2: [silence]

Ericsson: add note in proposal that we will agree an equation including length of sequence

For further discussion in FLS

Discussion 5_2_2

Discuss the following down-selection from RAN1#120bis Wuhan:
For 3.75kHz SCS OCC for NPUSCH format 1, RAN1 down selects between the following mappings between DMRS sequence samples and active TDM DMRS slots:

- Option 1: Sequential mapping of samples of the original DMRS sequence to active DMRS slots

- Option 2: Dropping of samples of the original DMRS sequence in blanked slots

Proposal 5_2_3: RAN1 down-selects between the following options for determining the TDM positions to activate within the TDM DMRS pattern for 3.75kHz SCS:

- Option 1: DMRS position follows fixed TDM DMRS pattern from the first scheduled slot for each UE

- Option 2: DMRS position associated with the OCC sequence index

  - Option 2_1: Also associated with the absolute slot index

  - Option 2_2: NPUSCH transmissions always start at a slot n where n mod 4 = 0

- Option 3: signalled separately to the OCC sequence index

  - FFS: signalling (e.g. DCI signals DMRS positions)

- Option 4: DMRS positions semi-statically associated with UE

[FL1] Proposal 5_8_1v1:

When [Nrep  in 213 or] MrepNPUSCH [in 211] is the value from the repetition field of the DCI, M is the OCC factor and  Mrepdata is the number of repetitions of the data before OCC itself, the following understanding applies:

- understanding 1:  MrepNPUSCH=M×Mrepdata

>> note that the other possibility would be:

- understanding 2:  MrepNPUSCH =Mrepdata

vivo: U2 means CE only with certain sets of parameters. Happy with either way.

MTK: like U1, but wording not clear

ZTE: cov should not be enhanced. Hence U1. Wording unclear

Nok: TX time should be kept the same (which would mean they prefer U2). Don’t want to introduce new concept. Maybe we have a note that says Mdata_rep doesn’t appear in specs

QC: what we are actually trying to define Mdata_rep and the equation should be the other way round

LGE: don’t increase the length in terms of time resources. Means U1

Nordic: prefer U1. But both understandings work

Lenovo: note maximum NPUSCH transmission time is same as legacy, based on same Mrep_NPUSCH param

QC: just state that the total number of slots after OCC is MrepNPUSCHNslotsULNRU, as per legacy. We then probably don’t need to talk about understandings. << formulate proposal around this

[FL1] Proposal 5.7.1-1v1: RAN1 strives for a common signalling design for 3.75kHz SCS and 15kHz SCS NPUSCH format 1.

[FL1] Proposal 5.7.2-1v1: The modulation and coding scheme DCI field is repurposed for indicating OCC parameters.

- MCS indices 8, 9 and 10 in Table 16.5.1.2-1 of TS36.213 are not used when OCC is applied

QC: would like to use SIF for 15kHz and separate design for 3.75kHz SCS

Note: 3.75 vs 15 is fixed via the RACH process. This facilitates SIF interpretation

QC: 15kHz is simple. Do that (reserved states of SIF). Do something different for 3.75kHz

Nok: agree with QC

Lenovo: what about MT

QC: 15kHz SIF would handle MT OK

[FL1] Proposal 5.7.2-2v1: The redundancy version DCI field is configured to be not present in DCI when the OCC feature is enabled.

# Tuesday: Proposals for online session

Proposal 5_2_1v1: Confirm the following working assumption from RAN1#120 Athens:

“For 3.75kHz SCS OCC for NPUSCH format 1, support TDM DMRS over 4 slots where DMRS are transmitted in the first 2 slots and DMRS REs are blanked in the next 2 slots, or vice-versa, where the DMRS REs are as in legacy NB-IoT and the guard period within the slot is as in legacy NB-IoT.”

[FL3] Proposal 5_8_1v2:

The total number of slots in the NPUSCH transmission after OCC is applied is MrepNPUSCHNslotsULNRU , where the parameters have the legacy definitions in TS36.211.

[FL3] Proposal 5_2_2v2:

For 3.75kHz SCS OCC for NPUSCH format 1, RAN1 the following mappings between DMRS sequence samples and active TDM DMRS slots is applied:

- Option 1: Sequential mapping of samples of the original DMRS sequence to active DMRS slots

[FL3] Proposal 5_2_3v13:

For 3.75kHz SCS, the TDM DMRS positions that are activated within the TDM DMRS pattern are associated with the OCC sequence index and

- Option 2_1: is associated with the absolute slot index

- Option 2_2: NPUSCH transmissions always start at a slot n where n mod 4 = 0

- Option 2-3: the DMRS pattern starts from the first scheduled slot for each UE

[FL3] Proposal 5.7.3-1v1:

- If the number of repetitions of NPUSCH is equal to 1, the DCI is interpreted as per legacy

- For 15kHz SCS:

  - the reserved states in the subcarrier indication field are used to indicate the location of subcarriers, OCC sequence index and OCC activation / deactivation

- For 3.75kHz SCS:

  - The redundancy version field is repurposed to indicate OCC activation / deactivation

  - The fields “subcarrier indication” and “modulation and coding scheme” are jointly encoded to indicate the location of the subcarriers, the value of ITBS and the OCC sequence index

# Conclusions

This document is the feature lead summary for IoT-NTN in RAN1#121. It contains the FLS discussion and lists the proposals that were considered in online and offline sessions.

# References

[1]     RP-250472, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, MediaTek Inc. (Rapporteur), RAN#107, March 2025.

[2]	R1-2502333 “Final FL Summary for IoT-NTN”. Moderator (Sony). RAN1#120bis. Wuhan, China. May 2025.

[3] 	R1-2401298 “Work Plan for Rel-19 IoT NTN”. Mediatek (rapporteur)

[4]	R1-2503211. “LS reply on TDM DMRS”. RAN4 (Sony). RAN1#121. Malta. May 2025.

R1-2503283	Discussion on UL capacity enhancements for IoT NTN	Huawei, HiSilicon

R1-2503338	On uplink capacity enhancements for IoT-NTN	Ericsson

R1-2503381	Remaining issues on IoT-NTN uplink capacity enhancement	vivo

R1-2503530	Discussion on IoT-NTN uplink capacity/throughput enhancement	Spreadtrum, UNISOC

R1-2503585	Discussion on uplink capacity/throughput enhancement for IoT-NTN	Samsung

R1-2503638	Discussion on UL capacity enhancement for IoT NTN	ZTE Corporation, Sanechips

R1-2503764	Discussion on the IoT-NTN uplink capacity/throughput enhancements	TCL

R1-2503777	Discussion on UL capacity enhancement for IoT NTN	CATT

R1-2503815	IoT-NTN uplink capacity/throughput enhancement	InterDigital, Inc.

R1-2503848	Discussion on the IoT-NTN uplink capacity/throughput enhancements	CMCC

R1-2503898	Discussion on IoT-NTN uplink capacity enhancement	Xiaomi

R1-2503932	IoT-NTN uplink capacity/throughput enhancement	NEC

R1-2503960	IoT NTN OCC activation and sequence index indication for NPUSCH	Sharp

R1-2504034	IoT-NTN uplink capacity enhancement	Nokia, Nokia Shanghai Bell

R1-2504071	On IoT-NTN uplink capacity enhancements	Sony

R1-2504152	Discussion on uplink capacity/throughput enhancement for IoT NTN	ETRI

R1-2504202	Discussion on IoT-NTN uplink capacity/throughput enhancement	OPPO

R1-2504273	IoT-NTN uplink capacity and throughput enhancement	MediaTek Inc.

R1-2504345	Discussion on IoT-NTN Uplink Capacity Enhancement	Apple

R1-2504413	IOT-NTN uplink capacity/throughput enhancement	Qualcomm Incorporated

R1-2504460	Discussion on uplink capacity enhancement for IoT NTN	Lenovo

R1-2504543	IoT-NTN uplink capacity/throughput enhancement	Nordic Semiconductor ASA

R1-2504564	Discussion on IoT-NTN uplink capacity/throughput enhancement	LG Electronics