3GPP TSG RAN WG1 #121                              	R1-2504075

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:	9.5.1

Source: 	Sharp

Title: 	Discussion on remaining details of on-demand SSB operation on SCell

Document for:	Discussion

Introduction

The Rel-19 WID on enhancements to NES for NR was approved in [1], which includes the following objective:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In this contribution, we will provide our views on remaining details on on-demand SSB (OD-SSB) operation for SCell.

Discussions

Collision between OD-SSB and PDCCH

In legacy [1], it is specified that if a PDCCH candidate and a (AO-)SSB are overlapped, the UE is not required to monitor the PDCCH candidate.

TS 38.213 Section 10 UE procedure for receiving control information For monitoring of a PDCCH candidate by a UE, if the UE - has received ssb-PositionsInBurst in SIB1 and has not received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell, and - does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block, after puncturing if applicable, corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in SIB1, the UE is not required to monitor the PDCCH candidate. For monitoring of a PDCCH candidate by a UE, if the UE - has received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell, and - does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block, after puncturing if applicable, corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in ServingCellConfigCommon, the UE is not required to monitor the PDCCH candidate. For monitoring of a PDCCH candidate by a UE, if the UE - has received ssb-PositionsInBurst in SSB-MTCAdditionalPCI for a serving cell, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block, after puncturing if applicable, corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in SSB-MTCAdditionalPCI with same physical cell identity as the one associated with a RS having same quasi-collocation properties as a CORESET for the PDCCH candidate, the UE is not required to monitor the PDCCH candidate.

We think the same rules should apply to OD-SSB.

If a PDCCH candidate and a transmitted OD-SSB are overlapped in at least one RE, the UE is not required to monitor the PDCCH candidate.

Collision between OD-SSB and PDSCH

In legacy [1], the rate matching rules and restrictions for SSB and PDSCH resource mapping are shown as follows:

TS 38.214 Section 5.1.4 PDSCH resource mapping When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 0, the UE shall assume that no SS/PBCH block, after puncturing if applicable, is transmitted in REs used by the UE for a reception of the PDSCH. When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 1, RA-RNTI, MSGB-RNTI, P-RNTI or TC-RNTI, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst, and if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources the UE shall assume that the PRBs containing SS/PBCH block transmission resources, after puncturing if applicable, are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI, MCCH-RNTI, Multicast MCCH-RNTI or PDSCHs with SPS, the REs corresponding to the configured or dynamically indicated resources in Clauses 5.1.4.1, 5.1.4.2 are not available for PDSCH. Furthermore, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, after puncturing if applicable, and the UE shall assume that the PRBs containing SS/PBCH block transmission resources, after puncturing if applicable, are not available for PDSCH in the OFDM symbols where SS/PBCH block associated with the same PCI is transmitted.

We think the legacy rules should also apply to the OD-SSB.

If a PDSCH resource allocation overlaps with PRBs containing OD-SSB transmission resources, the UE shall assume that the PRBs containing the OD-SSB are not available for the PDSCH reception.

Other usages of the OD-SSB

If an SCell is configured with OD-SSB, it means (1) this SCell is operated with a co-located PCell in a manner of inter-band CA, or (2) this SCell is not co-located with a SpCell and it may be the reference cell of other co-located SCells. In this sense, at least the TRS in this SCell shall be QCLed with SSB, which is not other but OD-SSB in Case #1. Therefore, it should be supported that OD-SSB can be used as a QCL source associated with a TCI state.

Support that OD-SSB can be used as a QCL source of a TCI state at least for Case #1.

Conclusions

Based on the discussion, we have the following observation and proposals:

- If a PDCCH candidate and a transmitted OD-SSB are overlapped in at least one RE, the UE is not required to monitor the PDCCH candidate.

If a PDSCH resource allocation overlaps with PRBs containing OD-SSB transmission resources, the UE shall assume that the PRBs containing the OD-SSB are not available for the PDSCH reception.

Support that OD-SSB can be used as a QCL source of a TCI state at least for Case #1.

References

3GPP TS 38.214 V18.6.0 (2025-03)

RP-242354, Revised WID: Enhancements of network energy savings for NR.

R1-2503119, Summary #5 of on-demand SSB for NES, RAN1#120bis.

Chair’s notes RAN1#120, Frebruary 17th – Frebruary 21st, 2025.