3GPP TSG-RAN WG1 Meeting #121															R1-2504076

Malta, MT, 19 – 23 May 2025

Agenda item:		9.2.1

Source:	Nokia

Title:	Enhancements to facilitate UE-initiated/event-driven beam management

Document for:		Discussion and Decision

# Introduction

The UE-initiated beam management (UEIBM) was agreed upon for the Rel. 19 MIMO WI in RP-234007 [1], and the agreed work item description was as follows:

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

This paper discusses the possible enhancements that would facilitate UE-initiated/event-driven beam management in the context of the unified TCI framework, i.e., for UL and DL beam management.

Such enhancements should result in reducing the overhead and/or latency of FR2 beam management. The preferred approach is leveraging the legacy CSI measurement and reporting configuration framework. The use case focuses on sTRP while including intra-cell and inter-cell beam management.

The discussion is structured in five parts. First, the UL signaling medium containers are discussed, including left-over for first PUCCH. Second, trigger event detection is discussed, including extra condition(s) of resetting the counting. Third, the UL signaling contents are described and compared for all the agreed events. Fourth, cross-CC measurements and reports are discussed. Fifth, solutions for beam application latency reduction are compared.

The paper closes with the conclusions on this objective and Nokia’s observations and proposals.

# Discussion

## UL signaling medium/container

There are few issues still open for the topic of UL signaling medium/container:

#1 - Whether/how to (re-)transmit the first PUCCH channel.

#2 - Design for the scenario with 1-bit first PUCCH associated with multiple CSI report configurations.

#3 - Multiplexing/dropping rule(s) of 1-bit first PUCCH.

#4 - Left-over on Step-2 in Mode-B, e.g., whether/how to introduce UE capability of ‘X’, and clarify the definition of ‘available’ transmission occasion of the second UL channel.

#5 - Counting of occupied CPU(s)

#1 - Whether/how to (re-)transmit the first PUCCH channel

Few candidates are on the table regarding first PUCCH retransmissions controlled by a prohibit timer as agreed in RAN1#120-bis:

Agreement [120-bis] On beam report transmission procedure for UE-initiated/event-driven beam reporting for a CSI report configuration, down-select at least one of the following candidates in RAN1#121: - Candidate#1: To introduce a prohibit timer for mode-A and/or mode-B - Option-1A: In the case that triggering-event associated with the CSI report configuration is determined [by the same triggering beams(s) as the last transmitted PUCCH], if the prohibit timer is NOT running, the UE can transmit first PUCCH; - At the first symbol after the end of the PUSCH transmission, the UE starts the prohibit timer - Option-1B: In the case that triggering-event associated with the CSI report configuration is determined, if the prohibit timer is NOT running, the UE can transmit first PUCCH; - At the first symbol after the end of the first PUCCH transmission, the UE starts the prohibit timer - If the prohibit timer is running, the first PUCCH is not allowed to be transmitted. - Candidate#2: To introduce a time interval for mode-A and/or mode-B - Option-2A: For a first PUCCH transmission occasion, if there is a transmission of another second PUSCH [corresponding to the CSI report determined by the same triggering beam(s) as the first PUCCH] within a configurable time interval before the first PUCCH transmission occasion, the UE should not transmit the first PUCCH. - Option-2B: For a first PUCCH transmission occasion, if there is a transmission of another first PUCCH(s) within a configurable time interval before the first PUCCH transmission occasion, the UE should not transmit the first PUCCH. - Candidate#3: No further enhancement.

In our understanding, Candidate#1 and Candidate#2 have technically the same effect, i.e., Option-1A is equivalent to Option-2A and Option-1B is equivalent to Option-2B, with the only difference that Candidate#1 uses the term “prohibit timer”, while Candidate#2 uses the term “time interval”, so here we comment only Candidate#1 options.

In our view, Option-1B is about retransmission of first channel/ PUCCH (carrying request for UL resources for beam/CSI reporting) for the situation when the UE does not receive response to the first PUCCH from the gNB, where this response is nothing but UL DCI triggering beam/CSI reporting corresponding to an event. Specifically, the UE would need to monitor for a reception of the beam/CSI report triggering indication from the gNB, and if the UE doesn’t receive or correctly decode the indication from the gNB within a configured period of time, the UE can be configured to retransmit the first PUCCH. That could be controlled by introducing such prohibit timer that starts after the first PUCCH has been sent. On the other hand, a prohibit timer is simply gaining time though a time window, is not a solution as such to a potential problem of PUCCH transmission failure, the solution lying in fact in the PUCCH re-transmissions. The utilization of the prohibit timer may be avoided by implementation where reasonable amount of time is allowed before a PUCCH re-transmission.

There is another issue related to first PUCCH (re-)transmission that deserves attention, and that relates to Option-1A. In Mode A and in Mode B, after the UE has successfully sent the beam report (in step 3 for Mode A and in step 2 for Mode B), it may happen that the network does not take any action, e.g., no beam switching, but the UE cannot know if that happened because the beam report was lost or just because of a network decision. Anyhow, it may happen that the conditions triggering the same event and beam report may persist over a period of time (the same Event-2, i.e., for same new beam(s) and for same conditions, e.g., same RSRP value(s) of the new beam(s) and of the current beam, keep repeating over time). In the current framework this may cause the transmission of multiple consecutive beam reports with same content, i.e., same top-N beams with same RSRP values, which bring no additional information to the network and just cause additional overhead. That could be controlled by a prohibit timer, such that the UE may send a first PUCCH for same event (with same conditions) only after such timer has expired. On the other hand, the utilization of such timer may not be really needed, as UEIBM already strongly reduced the overhead when compared for example to periodic reporting, and solution without any prohibit timer would be the leanest.

For both Mode A and Mode B, do not support introducing a prohibit timer for the first PUCCH (Candidate#3).

#2 - Design for the scenario with 1-bit first PUCCH associated with multiple CSI report configurations

The following agreements and conclusions were made on this topic up to RAN1#120-bis:

Conclusion [120] There is no RAN1 consensus on the following proposal: - On beam report transmission procedure for UE-initiated/event-driven beam reporting, multi-bit indication in the first PUCCH is supported. Agreement [120] On beam report transmission procedure for UE-initiated/event-driven beam reporting, one PUCCH resource of first PUCCH can be associated with one or multiple CSI report configurations, regarding Event-2. - Only a single UEI beam report is carried in the second PUSCH. - Additional indication of one ‘CSI report configuration’ is provided in the report format. - The CSI report configurations associated with the same PUCCH resource are ordered in ascending order of corresponding CSI-ReportConfigId, the number of bits of the additional indication field is ceil(log2(N_CSIconfig)), where the N_CSIconfig denotes the number of CSI report configurations associated with the same PUCCH resource. - The payload size of the single UEI beam report is determined according to the max payload size among the associated CSI report configurations. - Zero-padding can be appended if the payload size of the UEI beam report is less than the max report payload. - The reported UEI beam report should satisfy the triggering condition. - If multiple UE initiated beam report procedures occur, down-select one of the following options: - Option-1: It is up to UE implementation to select one of configuration. - Option-2: The UEI beam report with highest priority is reported - Option-3: The report triggered in the latest measurement is reported in PUSCH - The multiple CSI report configurations associated with the one first PUCCH resource should be configured in the same CC. - Working Assumption: For Mode-A, the multiple CSI report configurations associated with the same PUCCH resource should be associated with a same CSI-AperiodicTriggerState. - For Mode-B, the multiple CSI report configurations associated with the same PUCCH resource should be associated with the same second configured PUSCH FFS: Alt-2 Multiple UEI beam reports associated with the same PUCCH resource for first PUCCH can be transmitted in the second PUSCH. - If RAN1 cannot converge on the support of Alt-2 in RAN1#120bis, this alternative will be dropped from Rel-19 Conclusion [120-bis] On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the case that one PUCCH resource of first PUCCH can be associated with one or multiple CSI report configurations, there is NO RAN1 consensus on supporting multiple UEI beam reports carrying in a second PUSCH. Agreement [120-bis] On beam report transmission procedure for UE-initiated/event-driven beam reporting, confirming the following working assumption with the following modification. - For Mode-A, the multiple CSI report configurations associated with the same PUCCH resource should be associated with a same CSI-AperiodicTriggerState. - FFS: For Mode-A, the multiple CSI report configurations associated with the same CSI-AperiodicTriggerState should be associated with a same PUCCH resource. Conclusion [120-bis] There is no RAN1 concensus on the following. No spec change needed. For event 2, when one PUCCH resource of first PUCCH can be associated with one or multiple CSI report configurations, and if multiple UE initiated beam report procedures occur, down-select one of the following options: - Option-1: It is up to UE implementation to select one of configuration. - Option-2: The UEI beam report with highest priority is reported - FFS: priority, e.g., lowest CSI-ReportConfigId, event-type or based on legacy CSI report priority rule - Option-3: The report triggered in the latest measurement is reported in PUSCH

RAN1#120 eventually excluded any multi-bit first PUCCH and defined a procedure as in Figure 1 below, where:

- A UE may be configured with multiple CSI report configurations all associated to a same first PUCCH and all for Event-2.

- Once the conditions of at least one of the configured CSI report configurations associated to Event-2 are met, the UE sends a single-bit first PUCCH to the gNB.

  - As the first PUCCH is single-bit and common for all the configured CSI report configurations, the gNB does not know which conditions associated to which CSI report configurations were met.

- The PUSCH size for carrying the beam report (either scheduled by the gNB in Mode A or pre-configured in Mode B) is decided based on the maximum payload size among the configured CSI report configurations.

- The UE sends on the PUSCH the UEI beam report plus an indication of the CSI report configuration whose conditions were met.

  - If the payload size of the sent UEI beam report is less than the maximum payload size among the configured CSI report configurations, zero padding is used.

- In case the conditions of multiple CSI report configurations are met, only one UEI beam report is sent, and which is sent is up to UE implementation.

Figure 1. Visual representation of an example of RAN1#120 agreement on multiple Event-2 CSI report configurations associated to a same first PUCCH.

For Mode A, it has been agreed that the multiple CSI report configurations associated with the same PUCCH resource should also be associated with a same CSI-AperiodicTriggerState. On the RAN1#120-bis FFS about associating with the same PUCCH resource multiple CSI report configurations associated with a same CSI-AperiodicTriggerState, we think that there is no need to specify that, and such optimization can be left up to the network implementation. For example, we can have two CSI report configurations associated to PUCCH resource X, and a third CSI report configuration associated to another PUCCH resource Y, with all three sharing the same CSI-AperiodicTriggerState without ambiguity: if the UE uses the PUCCH resource X, then the UE will transmit a CSI report associated to one of the first two CSI report configurations, else if the UE uses PUCCH resource Y, then the UE will transmit a CSI report associated to the third CSI report configuration.

For a UE configured with one first PUCCH associated with multiple CSI report configurations regarding Event-2, do not support that the multiple CSI report configurations associated with the same CSI-AperiodicTriggerState should also be associated with a same PUCCH resource.

Moreover, although in RAN1#120 we agreed that a UE can be configured with multiple Event-2 CSI report configurations associated to the same 1-bit first PUCCH, we think that such restriction of having only Event-2, i.e., only one event type, is not necessary. Because the agreed event types target different use cases, e.g., Event-2 to facilitate beam switching, Event-1 to avoid Beam Failure Recovery and Event-7 to update the active TCI state list, we think that we should support that multiple event types can be configured simultaneously.

For a UE configured with one first PUCCH associated with multiple CSI report configurations, support that different event types can be configured simultaneously.

In RAN1#120-bis, we also discussed which CSI report the UE must transmit in case the conditions of multiple Event-2 CSI report configurations associated to a same first PUCCH were simultaneously met. Although different options were on the table (based on ID, based on triggering time), eventually it was decided to leave that selection up to the UE.

In our perspective, such solution makes sense in case indeed all these CSI report configurations are associated with the same event type, e.g., Event-2. On the other hand, it is beneficial that the network can introduce some priority in case different event types are configured and the simplest way to do that is by using either the lowest CSI-ReportConfigId or legacy CSI report priority rules as defined in clause 5.2.5 of TS 38.214 (and as already discussed for the same Event-2 scenario). With such solution, the network can configure for example a UE with a CSI report configuration associated to Event-2 and a CSI report configuration associated to Event-1 and, by assigning a lower ID to the Event-2 configuration, ensures that, in case the conditions of both CSI report configurations are simultaneously met, the CSI report associated to Event-2 is transmitted by the UE.

For a UE configured with one first PUCCH associated with multiple CSI report configurations associated with different event types, in case the conditions of more than one CSI report configurations are met, the UE reports the UEI beam report with highest priority, for example based on the lowest CSI-ReportConfigId or legacy CSI report priority rules.

#3 - Multiplexing/dropping rule(s) of 1-bit first PUCCH.

In RAN1#119, the following cases were identified for further studies on this topic:

Agreement [119] On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, further study at least the following cases: - Case-1: The 1-bit first PUCCH is collided/overlapped with a PUCCH carrying normal SR and/or a PUCCH with normal LRR - Case-2: The 1-bit first PUCCH is collided/overlapped with a PUSCH - Case-3: The 1-bit first PUCCHs corresponding to different CSI configuration for UE-initiated/event-driven beam reporting are overlapping in the time domain.

In RAN1#120, the following agreements were then made on this topic:

Agreement [120] On beam report transmission procedure for UE-initiated/event-driven beam reporting, support the following option of dropping rule for the Case-1: the 1-bit first PUCCH is collided/overlapped with a PUCCH carrying normal SR and/or a PUCCH with normal LRR - Option-1: LRR > first PUCCH > normal SR Note: When the 1-bit first PUCCH is collided/overlapped with a PUCCH carrying normal SR and/or a PUCCH with normal LRR, only one of them is transmitted based on the above priority rule Agreement [120] On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, down-select one of the following rule for the Case-2: the 1-bit first PUCCH is collided/overlapped with a PUSCH - Option-1: Prioritize first PUCCH over PUSCH, i.e., PUSCH is dropped. - Option-3: Piggyback 1-bit indication of first PUCCH into the PUSCH. - FFS: If the PUSCH should be with UL-SCH or not for UEI beam report - Option-4: Reuse the SR dropping rules, i.e., the first PUCCH is dropped. - FFS: whether/how to handle the case of different PHY priorities.

In RAN1#120-bis, the following agreements were further made on this topic:

Agreement [120-bis] On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, support the following rule for the Case-4: the 1-bit first PUCCH is collided/overlapped with a PUCCH format 0/1 carrying HARQ. - Option-1: Follow legacy SR multiplexing rule. - Note: There is no further enhancement on additional phase offset/constellation point(s). Agreement [120-bis] On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, down-select one of the following rules for the Case-2: the 1-bit first PUCCH is collided/overlapped with a PUSCH, in RAN1#121 - Option-1: Prioritize first PUCCH over PUSCH, i.e., PUSCH is dropped. - FFS: If the PUSCH should be with UL-SCH or not for UEI beam report - Option-3: Piggyback 1-bit indication of first PUCCH into the PUSCH. - FFS: The 1-bit indication is always multiplexed in the PUSCH, regardless that UEI beam report procedure is triggered. - FFS: If the PUSCH should be with UL-SCH or not for UEI beam report - Option-4: Reuse the SR dropping rules. - FFS: whether/how to handle the case of different PHY priorities. Conclusion [120-bis] There is no RAN1 concensus on the following. No spec change needed. On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, support one of the following rule for the Case-3: the 1-bit first PUCCHs corresponding to different CSI configuration for UE-initiated/event-driven beam reporting are overlapping in the time domain - Option-1: Per CSI report configuration. - Option-2: Up to implementation. - Option-3: UE transmits the first PUCCH with higher priority (e.g., PUCCH resource with lower PUCCH resource ID, or event-type) - Option-4: Multiplexing all 1-bit indications

For Case 1, where the 1-bit first PUCCH is collided/overlapped with a PUCCH carrying normal SR and/or a PUCCH with normal LRR, it has been agreed in RAN1#120 to define a dropping rule with the following order: LRR > first PUCCH > normal SR.

For Case 2, where the 1-bit first PUCCH is collided/overlapped with a PUSCH, we have three options for down-selection. Among those we support Option-4 where SR dropping rules are reused.

Regarding Case 2, where the 1-bit first PUCCH overlap with a PUSCH, support Option-4, where SR dropping rule is reused by dropping the first PUCCH.

For Case 3, where multiple 1-bit first PUCCHs corresponding to different CSI report configurations for UE-initiated/event-driven beam reporting are overlapping in the time domain, it has been concluded in RAN1#120-bis to not capture such case in the spec, and leave that up to UE implementation.

For Case 4, where the 1-bit first PUCCH is collided/overlapped with a PUCCH format 0/1 carrying HARQ, it has been agreed in RAN1#120-bis to follow legacy SR multiplexing rules.

In R1-2501556 during RAN1#120 the following additional case was also discussed:

- Case 5: The 1-bit first PUCCH is collided/overlapped with a PUCCH format 2/3/4 carrying HARQ/CSI.

In our view, although a new UCI type is used to represent the information carried by the first channel/ PUCCH, for multiplexing involving the first PUCCH, the new UCI type can be essentially treated as SR, meaning that same/similar multiplexing and prioritization rules as for SR could be used for the new UCI type.

Regarding Case 5, where the 1-bit first PUCCH is collided/overlapped with a PUCCH format 2/3/4 carrying HARQ/CSI, the 1-bit first PUCCH is a new UCI type that can be treated as SR, i.e., similar multiplexing/dropping rules as for SR can be used for the 1-bit first PUCCH.

#4 - Left-over on Step-2 in Mode-B, e.g., whether/how to introduce UE capability of ‘X’, and clarify the definition of ‘available’ transmission occasion of the second UL channel

In previous meetings it was agreed that for Mode B there is one-to-one mapping between first PUCCH and pre-configured resource for second UL channel. That means that only one periodic PUCCH resource for the first channel and only one pre-configured resource in the form of a CG-PUSCH for second UL channel can be associated with a CSI report configuration for UEIBM. The first PUCCH and the CG-PUSCH have the same periodicity. After sending the first PUCCH, it is uniquely defined on which specific resource occasion the UE will transmit the UEI report, i.e., it will transmit it on the first available transmission occasion of the second UL channel X symbols after the first PUCCH. The following has been agreed up to RAN1#120-bis on this topic:

Agreement [118] On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-B, UEI beam report is carried on a first available transmission occasion of the second UL channel X symbols after sending the last symbol of report notification on the first PUCCH channel. Agreement [118-bis] On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding resource mapping/configuration between first and second UL channel associated with a same CSI report configuration in Mode-B, - The UE expects that there is the same periodicity (in ms) between first PUCCH resource and pre-configured resource for second UL channel. - FFS: Whether first PUCCH resource and pre-configured resource for second UL channel can have different periodicities (in ms) Agreement [120] On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-B, the value of X symbols for determining available transmission occasion of the second UL channel is configured by RRC (FFS: subject to a corresponding UE capability) Conclusion [120-bis] Regarding resource mapping/configuration between first and second UL channel associated with a same CSI report configuration in Mode-B, there is NO consensus on further supporting the case that first PUCCH resource and pre-configured resource for second UL channel can have different periodicities (in ms).

We do not see the need to define a UE capability for the value of X configured by the network via RRC. The network just configures the UE with a certain value of X. In our view, we could have even X=0, because, when the UE sends the first PUCCH, the UE has already computed the RSRP and all other parameters needed for the UEI beam report, i.e., it is expected that the UE has the UEI beam report ready to be sent in the first available PUSCH immediately after the first PUCCH.

For Mode B, do not support that the value of X symbols for determining the available transmission occasion of the second UL channel is subject to a UE capability.

#5 - Counting of occupied CPU(s)

One of the aspects that needs to be clarified for UE-initiated reporting is the CPU occupancy time. CPU occupancy time is defined in clause 5.1.2.6 of TS 38.214 for the legacy CSI reporting mechanisms as follows (considering only the configurations related to UE-initiated reporting):

- For periodic and semi-persistent (excluding the initial semi-persistent CSI report after the network trigger) L1-RSRP reporting, 1 CPU is occupied from the first symbol of the associated earliest beam/RS until the last symbol of the PUCCH/PUSCH carrying the report;

- For aperiodic L1-RSRP reporting, 1 CPU is occupied from the first symbol after the PDCCH triggering the CSI report until the last symbol of the PUSCH carrying the report.

UE-initiated reporting is, from a certain perspective, in-between aperiodic and periodic/semi-persistent reporting because:

- similarly to aperiodic reporting, actual CSI reports are sent only occasionally;

- similarly to periodic/semi-persistent reporting, the beams/RSs are measured continuously.

Therefore, also the CPU occupancy time should be defined similarly, taking into account aspects of both legacy CSI reporting mechanisms.

Considering that the UE indicates with both Mode A and Mode B if an event has been triggered by sending a 1-bit first PUCCH before the actual CSI report on PUSCH, the CPU occupancy should last

- until the last symbol of the first PUCCH resource, if an event has not been triggered, i.e., the UE has not transmitted the 1-bit first PUCCH, else

- until the last symbol of the PUSCH resource carrying the CSI report, if an event has been triggered, i.e., the UE has transmitted the 1-bit first PUCCH.

For UE-initiated reporting, 1 CPU is occupied until

- the last symbol of the first PUCCH resource, if an event has not been triggered, i.e., the UE has not transmitted the 1-bit first PUCCH, else

- the last symbol of the PUSCH resource carrying the CSI report, if an event has been triggered, i.e., the UE has transmitted the 1-bit first PUCCH.

Regarding the starting time since when such CPU is occupied, as the UE continuously monitors beams/RSs to check if an event has been triggered, we think it should be similar to periodic/semi-persistent reporting where the reporting slot is used to determine the CSI reference resource. In this case the uplink reporting slot (or the usage of the UL slot) is not known prior to the first PUCCH thus the CSI reference resource should be calculated based on the first PUCCH. For UEIBM reporting configuration, 1 CPU should be occupied starting from the first symbol of the earliest beam/RS associated with the CSI reference resource for the CSI report configuration. The beam/RS may be may be either one of the configured new beams or the current beam.

For UE-initiated reporting, 1 CPU is occupied from the first symbol of the earliest beam/RS associated with the CSI reference resource, where the CSI reference resource is determined based on the first PUCCH of the CSI report configuration. The beam/RS may be either one of the configured new beams or the current beam.

## Trigger-event detection

Regarding trigger-event detection, we have the following relevant agreements and conclusions up to RAN1#120-bis:

Agreement [120-bis] On UE-initiated/event-driven beam reporting, the procedure of triggering event determination is captured in RAN1 spec. Agreement [120-bis] Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, down-select one of the following options in RAN1#120bis. - Option-1: The measurement window is from T_PUCCH – T_proc – T_window to T_PUCCH – T_proc, where T_PUCCH is a transmission occasion of a first PUCCH, and T_proc is RRC configured. - Option-3: The length, slot offset and periodicity of a measurement window are configured per CSI report configuration by NW. - Option-4: If an Event-2 instance for a new beam is obtained at the time t [and the time for the new beam is not running], UE (re)starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window) - Note: T_window is the agreed time window parameter for measurement. Agreement [120-bis] Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, support Option-4. - Option-4: If an Event-2 instance for a new beam is obtained at the time t and the timer for the new beam is not running, UE starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window) - Note: Timer is new beam specific. - Note: T_window is the agreed time window parameter for measurement. - Introduce separate UE capability to limit the number of timers. There is only 1 timer per new beam. Above agreement is captured in RAN1 specifications. Agreement [120] Regarding triggering event determination for Event 2, at least Candidate #2 is supported for resetting the counting. - Candidate#1: RS reconfiguration/update or MAC-CE signaling (if supported) for new beam is received; - FFS: whether to reset the counting for all new beams. - FFS: whether to maintain the counting whose new beam is NOT updated. - Candidate#2: [The measured current beam based on] indicated TCI state is updated; - In such case, the UE need to reset the counting for all new beams. - Candidate#3: UEI beam report is transmitted; - FFS: Only reset the counting of new beams fulfilling triggering condition and reported by the UEI beam report - Candidate#4: NW response (e.g., DCI in step-2 of Mode-A) is detected. - Candidate#5: The time window expires - Candidate#6: The threshold for event evaluation is re-configured by RRC signaling - (FFS) Candidate#7: The RRC parameter(s) associated with the CSI report configuration for UEI beam report is reconfigured. - FFS: RRC parameter(s) - FFS: Other candidates Note: Whether this proposal is captured in RAN1 or RAN2 is a separate discussion point. Agreement [120-bis] Regarding triggering event determination for Event 2, on resetting the counting, the following modification on the agreed Candidate #2 in RAN1#120 is supported. - Candidate#2: [The measured current beam RS is updated based on] indicated TCI state is updated; - In such case, the UE needs to reset the counting for all new beams. - FFS: Further details on the update (if necessary)

In RAN1#120-bis we finally reached an important agreement on how the time window is designed: events will be triggered by counting event instances on a sliding window / timer that starts any time there is a new event instance (in case the time window / timer has not started yet or has ended/expired), with one time window / timer per new beam (except for Event-1 where timer and counter exist only for the current beam).

An open issue that still needs some clarification regards the condition(s) for resetting/restarting a counter.

Several proposals have been discussed so far, while currently only Candidate#2 has been agreed, such that the counter for any new beam is reset when the current beam is updated. Such Candidate#2 makes sure that when the current beam, i.e., either the RS in the indicated TCI state when the set of new beams include CSI-RSs or the SSB QCLed with the RS in the indicated TCI state when the set of new beams include SSBs, is updated, then the counter is reset for all the new beams. That is in our view the best solution: in fact, when the current beam based on the indicated TCI state is updated, it makes technically sense that the counter for each new beam is just reset.

Regarding trigger-event detection for Event-7, there was an open issue for the case where the UE is configured with the time window and counter, as discussed for example in R1-2501556 during RAN1#120:

Proposal 1.2.2: Regarding the triggering event determination for Event-7, when the time window and counter with M are configured, down-select at least one of the following for determining event instance(s) counting: - Option-1: The event instance(s) counting is per new beam (as Event-2) - Option-2: The event instance(s) counting is per a pair of ‘new beam’ and ‘activated TCI state with the Q-th best quality’

The discussion was if the counting should be just done “per new beam” (as in Event-2) or per pair of “new beam and activated TCI state with the Q-th best quality”. The issue raised from the fact that, while the current beam for Event-2 and Event-1 is associated to the indicated TCI state, that essentially does not change unless the network directly update the indicated TCI state, the current beam for Event-7 is associated to the activated TCI state with the Q-th best quality, and clearly that can change because of radio channel variations within the time window, as depicted in Figure 2. The issue has been partially solved by the following Conclusion in RAN1#120-bis, which essentially states that counting should be done per new beam (as in Event-2):

Conclusion [120-bis] There is no RAN1 consensus on the following proposal: On UE-initiated/event-driven beam reporting, support at least Option-1 of following for Event 7 as an extension on report format for Event-2 - Option-1: Additional field to indicate the codepoint of the activated TCI state with Q-th best quality. - FFS: Further report codepoints of other activated TCI state(s) [, e.g., from {Q+1}-th best quality to the worst one]; - FFS: Details of the additional indication. - Option-2: Extending the maximum number of reported RS(s) to 8. - [Note: If Option 2 is agreed, RAN1 can revisit the agreed maximum value of N for event-2.] - Above is applicable only if time window and a configurable value M for event counting on Event-7 are not configured.

Figure 2. Variation of the current beam for Event-7, i.e., the beam associated to the activated TCI state with the Q-th best quality, with Q=2 in this example.

On the other hand, this in our view leads to another issue that RAN1 needs to discuss about how to reset the counter for Event-7. The agreed Candidate#2 solution for resetting the counter for Event-2 when the indicated TCI state is updated is applicable to Event-1 but it is not applicable/helpful for Event-7. The reason is simply, as already stated, that the current beam in Event-7 is associated to the activated TCI state with the Q-th best quality (and not to the indicated TCI state as in Event-2 and Event-1).

The agreed Candidate#2 solution for resetting the counter for Event-2 when the indicated TCI state is updated is not applicable to Event-7 where the current beam is associated to the activated TCI state with the Q-th best quality (and not to the indicated TCI state as in Event-2).

The issue is that, while for Event-2 the current beam is a) uniquely identified by the indicated TCI state and b) can be fully and directly controlled by the network for example updating the indicated TCI state using a DCI command, for Event-7 there is not such unique identification for the current beam, which then cannot be directly and uniquely updated/controlled by the network. In case a time window and counter are configured, the Event-7 current beam may even change during the time window just because of varying RSRP, as depicted in Figure 2.

The straightforward extension of Event-2 Candidate#2 to Event-7 is to simply consider resetting the counter when the activated TCI state list is updated, e.g., via MAC-CE.

Regarding triggering event determination for Event-7, when the time window and counter are configured, support resetting the counter for all new beams when the active TCI state list is updated.

## UL signaling content

Regarding the UL signaling content, we have the following relevant agreements and conclusions up to RAN1#120-bis:

Agreement [118] On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, the following differential L1-RSRP report format is supported. CRI or SSBRI #1. CRI or SSBRI #2. …. CRI or SSBRI #N. L1-RSRP #1. Differential L1-RSRP #2. …. Differential L1-RSRP #N. Differential L1-RSRP for current beam, if report mode that current beam is always reported is enabled by RRC. Note: Other contents are not precluded. . - Differential L1-RSRP #2~#N/current beam is determined based on the difference between measured L1-RSRP corresponding to the CRI/SSBRI #2~#N/current beam and the measured L1-RSRP corresponding to CRI/SSBRI #1 - L1-RSRP#1 is the largest measured RSRP among reported ones, and an absolute L1-RSRP. - FFS: range and step size of differential L1-RSRP - FFS: Whether/how to report additional indication of which CRI/SSBRI(s) satisfy the condition of Event-2. - FFS: Additional report content(s) (e.g., reporting configuration ID, indication for synchronization state, event ID, or cell ID). Agreement [120] On UE-initiated/event-driven beam reporting, Option-1 is supported as an extension on L1-RSRP report format depending on Event-2. - Option-1: For each of reported CRI/SSBRI, to introduce additional indication of whether the CRI/SSBRI satisfies the condition of Event-2. - The presence of this field is enabled by RRC with subjective to UE capability. - Note: The presence of this field is only for the case that N > 1 and the time window and M are configured Above is NOT applicable for event 1. Agreement [120-bis] On UE-initiated/event-driven beam reporting, support the following interpretation on each of the codepoints of the ‘1-bit’ condition met indicator as follows: - ‘0’ – indicating that the Event-2 instances for corresponding CRI/SSBRI within the time window doesn’t reach the configured number M. - ‘1’ – indicating that the Event-2 instances for corresponding CRI/SSBRI within the time window reach the configured number M. FFS: whether/how to introduce the ‘1-bit’ condition met indicator for Event 7. Agreement [120-bis] On UE-initiated/event-driven beam reporting, support the following for Event 1 - Regarding report format of L1-RSRP value of current beam, - Option-A2: L1-RSRP of current beam is absolute L1-RSRP (7-bit quantized, rather than 4-bit as in Event-2). Agreement [120-bis] On UE-initiated/event-driven beam reporting, regarding current beam report on Event 1, reuse RRC parameter enabledCurrentBeamReport-r19 to enable/disable the RSRP report of current beam.

In RAN1#120, we finally closed the pending issue on indicating which CRI/SSBRI(s) satisfy the condition of Event-2, and such indication is included when time window and counter are configured for Event-2.

This additional indication has been introduced because, in particular with (but in our view also without) the time window, the UE may report RSRP of N new beams and the current beam, but the gNB may not know from the actual report which specific new beam triggered the report itself because of RSRP variation within the time window. In our view, such indication was not needed, because eventually what really matters are the actual RSRP of the reported beams.

On the other hand, this leads to another related issue, which is the format of the RSRP of the current beam that is reported. We agreed in RAN1#118 to just report the differential RSRP of the current beam for Event-2, and in RAN1#120-bis to report the absolute RSRP of the current beam for Event-1. On the other hand, for similar motivation that led to the RAN1#120 agreement about this ‘1-bit’ condition met indicator, it may happen that a UE measures certain new beam(s) that trigger Event-2, but, because of some delay between the triggering determination of the Event at the UE and the transmission of the beam report in the second UL channel, the RSRP of the current beam has changed, and has potentially become the best. That would then not easily fit the current report format, and it would be best to report the absolute L1-RSRP of the current beam. To be homogenous we should have absolute L1-RSRP of the current beam also for the other events beyond Event-1.

On beam report format for all events, support reporting absolute RSRP value of the current beam.

Regarding the FFS on if this ‘1-bit’ condition met indicator agreed for Event-2 should be introduced also for Event-7, we think that other events design should reuse the same design of Event-2, so we support that.

On beam report format, support introducing the ‘1-bit’ condition met indicator also for Event-7, reusing the same design of Event-2.

There is a further discussion point on the assumption for the current beam. In the current beam report format there is no indication of current beam assumption/index provided by the UE. Therefore, there could be ambiguity at the gNB regarding which current beam/ TCI state the UE has reported for example when the indicated TCI state is updated after the first PUCCH transmission but before the actual transmission of the beam report in the second uplink channel. Further, ambiguity may also occur when the DCI indicating a (new) TCI state is mis-detected. In fact, in RAN1#118, there was an FFS point on the indication of the “current beam assumption”.

This then justifies the need for either reporting indication regarding the current beam assumption (e.g., corresponding TCI state identifier, RS index, or simply “current beam” change, etc.) or some predefined rule that would help clearing out such ambiguity, for example assuming that the reported current beam is the one associated to the indicated TCI state at the time of the UE transmission of the first PUCCH.

On beam report format for Event-2, support the network to configure the UE to indicate/report the current beam index in the UE-initiated beam report.

## Cross-CC measurement/report

In RAN1#120-bis, the following has been agreed on cross-CC measurement/report:

Agreement [120-bis] On cross-CC beam report measurement for UE-initiated/event-driven beam reporting, regarding Event-2, introduce an RRC parameter to indicate ServCellIndex on which the indicated TCI state used to determine the current beam RS is applied - FFS: Indication of the cell that corresponds to the configured first PUCCH resource in CSI report config including whether it is necessary

On this topic, it is also worth recalling the following previous agreements:

Agreement [118-bis] On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, for both Mode-A and Mode-B, the first PUCCH and the second PUSCH can be from the same or different CC(s) - FFS: whether the first PUCCH and the second PUSCH should be from the same PUCCH group - The first PUCCH and the second PUSCH should be in the same CG - FFS: Different CGs Agreement [118-bis] On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, the following is supported - For new beam measurement, in a CSI report configuration, configure legacy RRC parameter carrier that indicates the CC that the RS resource set associated with the CSI reporting configuration can be found - FFS: Whether the current beam RS and new beam RS(s) can be in the same CC or in different CCs, regarding cross-CC beam measurement. - FFS: Whether the indicated TCI state and new beam RS(s) can be in the same CC or in different CCs, regarding cross-CC beam measurement. Agreement [120] On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, the following working assumption is confirmed with the following modification. - The first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from different PUCCH groups - Subject to separate UE capability - Note: From RAN1 perspective, the above does NOT introduce any spec impact except for UE capability signaling Agreement [120] On cross-CC beam report measurement for UE-initiated/event-driven beam reporting, regarding Event-2, the following is supported - The current beam RS and new beam RS(s) are in the same CC - The above does NOT imply to preclude the cross-CC TCI indication. - The CC on which the indicated TCI state is applied, and the CC in which new beam RS(s) are can be the same or different. - FFS: Whether/how to introduce an RRC parameter to indicate the CC on which the indicated TCI state is applied.

Regarding the FFS in RAN1#120-bis, in our view, there is no need to introduce an RRC parameter to indicate the CC corresponding to the configured first PUCCH resource. Note that up to now, for cross-CC beam reporting we have agreed a) to use the legacy parameter carrier to indicate the CC of the new beam RSs and b) to introduce a new parameter ServCellIndex to indicate the CC on which the indicated TCI state used to determine the current beam RS is.

The first PUCCH resource is configured by the parameter firstPUCCHResourceConfig-UEIBR-r19, and should be assumed to be in the same CC of CSI-ReportConfig.

On cross-CC beam report transmission procedure, regarding Event-2, do not support introducing an RRC parameter to indicate the CC corresponding to the configured first PUCCH resource.

## Beam application latency reduction

Regarding beam application latency reduction, we have the following open issue:

Agreement [119] Study the following to reduce beam application latency after a UEI/ED beam report is sent - Alt-1: After confirmation/acknowledgement from NW, apply new beam without RRC configuration signaling or MAC-CE signaling - after sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report - update TCI state(s) with the reported new beam(s) - activate new beam(s) without additional SSB reception - Alt-2: After receiving a TCI state activation command to activate a TCI state(s), if the new beam(s) associated with the TCI state(s) is reported as synchronized in the UEI/ED beam report, the TCI state(s) becomes applicable for DL reception without additional SSB reception. - Note: A reported new beam is determined as synchronized by UE, if the UE stores the QCL properties associated with the reported new beam(s) after the UEI/ED beam report is sent - FFS: How to inform a reported new beam in a UEI/ED beam report (i.e., introducing one-bit indicator for each reported new beam or all the reported new beam are assumed to be synchronized) - Alt-3: After sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report. - In such case, at the reception of a subsequent reception of Unified TCI States Activation/Deactivation MAC CE, the UE activates new beam(s) without additional SSB reception - Other alternatives are not precluded

The three alternatives which have been proposed for further discussion in RAN1#119 target any event, i.e., Event-1/2/7, and have the objective to reduce the TCI state activation delay.

-	Alt-1 proposes to automatically activate reported new beams. This alternative can be supported, although some further discussion is needed on some aspects. For example, in case a UE supports just one active TCI state, that is also the indicated TCI state, updating the active TCI state list in such context would mean that a UE decides a network beam switching, and of course that cannot be supported.

-	Alt-3 proposes that, after the MAC CE, a very fast activation delay is required by the UE without additional SSB reception for T/F sync for the reported new beams.

-	Alt-2 is in our view similar to Alt-3, and just restricts this very fast activation delay after the MAC CE to a subset of the reported new beams, with this subset indicated for example by the UE.

These three alternatives seem more RAN4 solutions, and we think should be discussed within that WG.

On beam application latency reduction, send an LS to RAN4 to further study Alt-1, Alt-2 and Alt-3.

In the context of the enhancements that can be introduced after the UE has sent the beam report on PUSCH, besides beam application latency reduction solutions discussed above, we think some further aspects could be considered. The UE expectation after reporting certain beams is that these beams are for example activated via MAC-CE, to allow a potential fast beam switching. However, there may be a mismatch between beams reported by the UE on PUSCH and the MAC-CE command to update the active TCI state list, for example because a new beam reported by the UE may temporarily not be available for network scheduling for that UE. In such scenario, it makes sense that the UE stops monitoring, even just temporarily such reported but not activated beams for UE-initiated reporting.

After sending a UE-initiated beam report, the UE stops monitoring the new beams included in the UE-initiated beam report but not included in the subsequent TCI states Activation/Deactivation MAC-CE.

# 3.	Conclusions

Based on the discussion above we made the following observations and proposals:

Proposal 1:	For both Mode A and Mode B, do not support introducing a prohibit timer for the first PUCCH (Candidate#3).

Proposal 2:	For a UE configured with one first PUCCH associated with multiple CSI report configurations regarding Event-2, do not support that the multiple CSI report configurations associated with the same CSI-AperiodicTriggerState should also be associated with a same PUCCH resource.

Proposal 3:	For a UE configured with one first PUCCH associated with multiple CSI report configurations, support that different event types can be configured simultaneously.

Proposal 4:	For a UE configured with one first PUCCH associated with multiple CSI report configurations associated with different event types, in case the conditions of more than one CSI report configurations are met, the UE reports the UEI beam report with highest priority, for example based on the lowest CSI-ReportConfigId or legacy CSI report priority rules.

Proposal 5:	Regarding Case 2, where the 1-bit first PUCCH overlap with a PUSCH, support Option-4, where SR dropping rule is reused by dropping the first PUCCH.

Proposal 6:	Regarding Case 5, where the 1-bit first PUCCH is collided/overlapped with a PUCCH format 2/3/4 carrying HARQ/CSI, the 1-bit first PUCCH is a new UCI type that can be treated as SR, i.e., similar multiplexing/dropping rules as for SR can be used for the 1-bit first PUCCH.

Proposal 7:	For Mode B, do not support that the value of X symbols for determining the available transmission occasion of the second UL channel is subject to a UE capability.

Proposal 8:	For UE-initiated reporting, 1 CPU is occupied until

-	the last symbol of the first PUCCH resource, if an event has not been triggered, i.e., the UE has not transmitted the 1-bit first PUCCH, else

-	the last symbol of the PUSCH resource carrying the CSI report, if an event has been triggered, i.e., the UE has transmitted the 1-bit first PUCCH.

Proposal 9:	For UE-initiated reporting, 1 CPU is occupied from the first symbol of the earliest beam/RS associated with the CSI reference resource, where the CSI reference resource is determined based on the first PUCCH of the CSI report configuration. The beam/RS may be either one of the configured new beams or the current beam.

Observation 1:	The agreed Candidate#2 solution for resetting the counter for Event-2 when the indicated TCI state is updated is not applicable to Event-7 where the current beam is associated to the activated TCI state with the Q-th best quality (and not to the indicated TCI state as in Event-2).

Proposal 10:	Regarding triggering event determination for Event-7, when the time window and counter are configured, support resetting the counter for all new beams when the active TCI state list is updated.

Proposal 11:	On beam report format for all events, support reporting absolute RSRP value of the current beam.

Proposal 12:	On beam report format, support introducing the ‘1-bit’ condition met indicator also for Event-7, reusing the same design of Event-2.

Proposal 13:	On beam report format for Event-2, support the network to configure the UE to indicate/report the current beam index in the UE-initiated beam report.

Proposal 14:	On cross-CC beam report transmission procedure, regarding Event-2, do not support introducing an RRC parameter to indicate the CC corresponding to the configured first PUCCH resource.

Proposal 15:	On beam application latency reduction, send an LS to RAN4 to further study Alt-1, Alt-2 and Alt-3.

Proposal 16:	After sending a UE-initiated beam report, the UE stops monitoring the new beams included in the UE-initiated beam report but not included in the subsequent TCI states Activation/Deactivation MAC-CE.

# References

[1] RP-234007 New WID: NR MIMO Phase 5