3GPP TSG RAN WG1 #121			R1-2504131

Saint Julian’s, Malta, May 19th – 23rd, 2025

Source:	ETRI

Title:	Discussion on specification support for CSI prediction

Agenda item:	9.1.3

Document for:	Discussion

- Introduction

At the RAN1#120-bis meeting, followings were concluded for specification support for CSI prediction [1].

Agreement in RAN1#120-bis Introduce a dedicated AI/ML PU for AI/ML features for UE, - The AI/ML PU is used at least for quantifying the simultaneous processing of multiple CSI reports subject to CSI-related AI/ML use case(s), e.g., CSI compression (if supported), CSI prediction, BM spatial prediction, BM temporal prediction. Conclusion For CSI prediction using UE-side model, for data collection for training, aperiodic CSI-RS resource for CMR is not supported. Agreement in RAN1#120-bis For CSI prediction using UE-side model, for training data collection, support - CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report - FFS on how to indicate without CSI report in CSI-ReportConfig Agreement in RAN1#120-bis For CSI prediction using UE-side model, for performance monitoring, support UE assisted performance monitoring subject to an additional UE capability, and UE assisted performance monitoring is based on Type 3 performance monitoring Agreement in RAN1#120-bis For CSI prediction using UE-side model, for performance monitoring type 3, support SGCS as a performance metric. Agreement in RAN1#120-bis For the definition of SGCS, Note: How to handle layer mapping mismatch, if any, is up to UE implementation Agreement in RAN1#120-bis For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, down-select one alternative by RAN1#121. - Alt 1: one SGCS is calculated based on predicted CSI for one inference reporting, ground truth CSI - Alt 2: one SGCS is calculated based on predicted CSI for one inference reporting, and ground truth CSI, another SGCS is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance. - Alt 3: statistic of reporting contents (e.g., mean, x% CDF of SGCS values) of inference reporting instances within configured monitoring window - Monitoring window can be configured by NW - FFS on signalling details - FFS on whether to report per prediction instance, selected prediction instance(s), averaged over prediction instances - FFS on report frequency granularity (e.g., per wideband or per subband or averaged over subband or selected subband) - FFS on whether to report per layer, the first layer, averaged over layer - FFS on how to quantize and report mechanism Agreement in RAN1#120-bis For CSI prediction using UE side model, for inference, consider following options for potential down selection - Option 1: only dedicated AI/ML PU (OAPU) is occupied - Option 2: only legacy CPU (OCPU) is occupied - Option 3: both dedicated AI/ML PU (OAPU) and legacy CPU (OCPU) are occupied - FFS whether OAPU and OCPU are based on defined rule and/or reported by UE - Note: The supported option(s) by UE is reported by UE capability, if multiple options are supported. The total number of dedicated AI/ML PU for AI/ML is reported by UE capability. - Note: The total number of Use case specific dedicated AI/ML PU could be discussed separately. Agreement in RAN1#120-bis For CSI prediction using UE-side model, at least for inference, introduce new RRC parameter for CSI report configuration to distinguish CSI report of AI-CSI prediction and non-AI CSI prediction. - Note: terminology of “AI-CSI prediction” and “non-AI CSI prediction” is separate discussion - Detailed parameter name is upto RAN2

In this contribution, we discuss issues on specification support for CSI prediction and provide our views.

Discussion on specification support for CSI prediction

In the RAN#106 meeting, it was concluded that from Q1 of 2025, normative work in RAN1 for CSI prediction is started. Hence, in this section, we discuss issues on specification support for CSI prediction.

Issue 1: Consistency of training/inference

Ensuring consistency between training and inference in an AI/ML model is critical for achieving optimal performance. If consistency is not maintained, the AI/ML model may yield inappropriate results, as the inference environment can differ from the training environment. For example, in CSI prediction, consider a scenario where the UE-side AI/ML model is trained under "Network Side Additional Condition – Setting A." If this condition changes to "Setting B," where factors like tilt angle or TXRU mapping differ from those in Setting A, the model's inference results may become unreliable. Because training and inference occur under distinct network conditions, the model inputs for each may have different characteristics. This discrepancy means the trained model might not align well with the input used during inference, potentially leading to mismatched results. Therefore, in our view, ensuring consistency between training and inference is essential for preventing significant performance degradation in CSI prediction for UE-side AI/ML models.

- Observation 1: Ensuring the consistency between training and inference of UE-sided AI/ML model is essential for preventing non-negligible performance degradation.

In the RAN1 #118 meeting, it was agreed to support associated ID to ensure consistency between training and inference for AI/ML based beam management [2].

Agreement at RAN1#118 For UE sided model in beam management, introduce support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

From our perspective, since the support for an associated ID has already been agreed upon, there is no reason not to extend this support to CSI prediction, as it would reduce the specification workload. Therefore, we propose supporting an associated ID for consistency in the UE-side model for CSI prediction. In CSI prediction, UE may assume the same NW-side additional conditions of DL RS associated with the same associated ID.

- Proposal 1: For consistency of training/inference of UE sided model in CSI prediction, support associated ID.

- Proposal 2: In CSI prediction, UE may assume the same NW-side additional conditions of DL RS associated with the same associated ID.

If an associated ID is supported, the UE should identify the associated ID of received CSI-RS during the data collection process. The UE (or UE server) may categorize collected data based on the associated ID, allowing its use in model training and inference. Therefore, specifying the configuration of the associated ID is essential. In our view, the associated ID can be configured through either the CSI report configuration or the CSI resource configuration. If the associated ID is configured via the CSI report configuration, the UE can assume that the CSI-RS configured within the CSI report configuration shares the same associated ID. Similarly, if the associated ID is configured through the CSI resource configuration, the UE can assume that the CSI-RS configured within the CSI resource configuration shares the same associated ID.

- Proposal 3: The associated ID can be configured through the CSI report configuration or CSI resource configuration from a higher layer, with the UE assuming that the configured CSI-RS within the configuration shares the same associated ID.

Issue 2: Performance monitoring for UE-sided AI/ML model

In RAN1 #120-bis meeting, it was agreed to support Type 3 performance monitoring using SGCS as the performance metric, along with the definition of SGCS. In the upcoming meeting, RAN1 should determine the specific reporting contents for the performance metric based on the previous agreements.

Agreement in RAN1#120-bis For CSI prediction using UE-side model, for performance monitoring, support UE assisted performance monitoring subject to an additional UE capability, and UE assisted performance monitoring is based on Type 3 performance monitoring Agreement in RAN1#120-bis For CSI prediction using UE-side model, for performance monitoring type 3, support SGCS as a performance metric. Agreement in RAN1#120-bis For the definition of SGCS, Note: How to handle layer mapping mismatch, if any, is up to UE implementation Agreement in RAN1#120-bis For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, down-select one alternative by RAN1#121. - Alt 1: one SGCS is calculated based on predicted CSI for one inference reporting, ground truth CSI - Alt 2: one SGCS is calculated based on predicted CSI for one inference reporting, and ground truth CSI, another SGCS is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance. - Alt 3: statistic of reporting contents (e.g., mean, x% CDF of SGCS values) of inference reporting instances within configured monitoring window - Monitoring window can be configured by NW - FFS on signalling details - FFS on whether to report per prediction instance, selected prediction instance(s), averaged over prediction instances - FFS on report frequency granularity (e.g., per wideband or per subband or averaged over subband or selected subband) - FFS on whether to report per layer, the first layer, averaged over layer - FFS on how to quantize and report mechanism

At the previous meeting, three options were discussed regarding the reporting contents for UE-assisted performance monitoring. Fundamentally, SGCS is a metric that quantifies the similarity between two vectors. According to the agreed-upon definition, SGCS - calculated based on the predicted CSI for one inference reporting instance and the corresponding ground truth CSI – provides a reliable indication of the similarity between the inference results and the actual values. This metric also offers meaningful insights into the status of the AI/ML model.

In particular, such SGCS reporting enables the network to determine appropriate operational decisions such as switching, fallback, or maintenance of the AI/ML model. Conversely, calculating SGCS using the ground truth CSI and the CSI associated with the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance may fail to reflect scenarios where the channel changes rapidly.

From our perspective, including an additional SGCS based on the ground truth CSI and baseline CSI does not offer significant benefits and instead introduces unnecessary reporting overhead. Accordingly, we propose supporting Alt 1 for the reporting contents.

- Proposal 4: For CSI prediction using UE-sided model, for reporting contents of UE assisted performance monitoring, support Alt 1, that UE only reports one SGCS calculated based on predicted CSI for one inference reporting, ground truth CSI.

For the SGCS reporting operation, the UE may report a single SGCS value that is averaged across all layers, prediction instances, and subbands in order to minimize signalling overhead. If SGCS values are reported separately for each layer, instance, or subband, this would significantly increase the amount of data to be reported. In other words, such detailed reporting would require additional signalling and consume substantial uplink resources.

From our perspective, in order to mitigate uplink resource consumption, the UE should report and SGCS value that is averaged over all relevant dimensions, including layers, prediction instances, and subbands.

- Proposal 5: For reporting contents of UE assisted performance monitoring, UE reports the SGCS value averaged across all layers, prediction instances, and subbands, in order to reduce uplink reporting overhead.

In the context of SGCS reporting operations, quantization of the SGCS value constitutes an additional consideration for UE-assisted performance monitoring. Specifically, the network should configure a set of range intervals for SGCS, enabling the UE to report the corresponding range to which the calculated SGCS belongs.

Given that the SGCS value lies within the range [0, 1], the network can configure a number N of uniformly divided intervals over this range. The UE can then report the index of the interval that contains the calculated SGCS value.

For example, if the calculated SGCS is 0.94 and the network configures N=10, the interval set is: {0,0.1,0.1,0.2,0.2,0.3,0.3,0.4,0.4,0.5,0.5,0.6,0.6,0.7,0.7,0.8,0.8,0.9,0.9,1}. In this case, since 0.94∈(0.9, 1], the UE would report ‘9’, which is the index of the corresponding interval.

- Proposal 6: For the quantization of the reported SGCS, the network can configure the number of uniformly divided range intervals over [0, 1] – ‘N’.

- Proposal 7: For the reporting contents of UE assisted performance monitoring, UE reports the index of the configured interval that corresponds to the calculated SGCS value.

For SGCS reporting, L1 signalling can be leveraged to enable prompt processing and ensure high reliability, similar to conventional CSI reporting mechanisms. From our perspective, SGCS reporting can be introduced as a new quantity within the CSI report to facilitate performance monitoring of CSI prediction.

In particular, when the parameter N is configured in the CSI-ReportConfig, representing the number of uniformly divided range intervals over [0, 1], the UE can report the calculated SGCS using log2N bits. This allows efficient and compact quantization of SGCS while aligning with existing CSI reporting frameworks.

- Proposal 8: Introduce a new quantity for SGCS in the CSI report to support performance monitoring of CSI prediction.

- Proposal 9: For the reporting contents of UE assisted performance monitoring, the size of the CSI field associated with SGCS reporting is log2N, where N is the number of uniformly divided range intervals over [0, 1], as configured in ‘CSI-ReportConfig’

Issue 3: CSI processing criteria and timeline

At the RAN1 #120-bis meeting, three options for CSI prediction for inference were agreed upon to be down-selected in the subsequent meeting, as follows.

Agreement in RAN1#120-bis For CSI prediction using UE side model, for inference, consider following options for potential down selection - Option 1: only dedicated AI/ML PU (OAPU) is occupied - Option 2: only legacy CPU (OCPU) is occupied - Option 3: both dedicated AI/ML PU (OAPU) and legacy CPU (OCPU) are occupied - FFS whether OAPU and OCPU are based on defined rule and/or reported by UE - Note: The supported option(s) by UE is reported by UE capability, if multiple options are supported. The total number of dedicated AI/ML PU for AI/ML is reported by UE capability. - Note: The total number of Use case specific dedicated AI/ML PU could be discussed separately.

Given that the inference operation involves AI/ML processing units, Option 2 is deemed unsuitable for inference purposes. From our perspective, inference operations encompass not only the AI/ML inference itself, but also associated tasks such as CSI processing. In other words, the inference procedure relies on both AI/ML processing units and conventional CSI processing units.

From this standpoint, Option 3, which involves the utilization of both dedicated AI/ML processing units and legacy CPUs, appears to be the most appropriate among the proposed options.

- Proposal 10: For CSI prediction using UE side model, for inference, support Option 3, that both dedicated AI/ML PU and legacy CPU are occupied.

Issue 4: Data collection related aspects

At the previous meeting, the CSI configuration method for data collection for training purposes was discussed. It was agreed that the CSI-ReportConfig can be utilized to configure resources for data collection, without requiring the transmission of CSI report by the UE.

Given that the UE does not transmit a CSI report in this case, the network should provide an appropriate indication. As the current specification already supports the value ‘none’ for the ‘reportQuantity’ field, it appears reasonable to reuse this existing information element for such indication.

Therefore, we propose that the absence of CSI reporting within the CSI-ReportConfig be indicated by setting the ‘reportQuantity’ field to ‘none’.

- Proposal 11: To indicate no CSI report in ‘CSI-ReportConfig’, reuse the existing parameter by setting the ‘reportQuantity’ field to ‘none’

The choice between a single or separate configuration of model input and ground truth CSI remains an open issue for data collection. From our perspective, separate configuration offers no clear advantage. Furthermore, it would incur additional downlink resource consumption compared to a single configuration approach. Therefore, we support the use of a single configuration for both model input and ground truth CSI to ensure efficiency and minimize resource overhead.

- Proposal 12: Support the single configuration of model input and ground truth CSI.

- Conclusion

In this contribution, we have discussed issues on specification support for CSI prediction. As a conclusion, we summarize our views as follows:

- Observation 1: Ensuring the consistency between training and inference of UE-sided AI/ML model is essential for preventing non-negligible performance degradation.

- Proposal 1: For consistency of training/inference of UE sided model in CSI prediction, support associated ID.

- Proposal 2: In CSI prediction, UE may assume the same NW-side additional conditions of DL RS associated with the same associated ID.

- Proposal 3: The associated ID can be configured through the CSI report configuration or CSI resource configuration from a higher layer, with the UE assuming that the configured CSI-RS within the configuration shares the same associated ID.

- Proposal 4: For CSI prediction using UE-sided model, for reporting contents of UE assisted performance monitoring, support Alt 1, that UE only reports one SGCS calculated based on predicted CSI for one inference reporting, ground truth CSI.

- Proposal 5: For reporting contents of UE assisted performance monitoring, UE reports the SGCS value averaged across all layers, prediction instances, and subbands, in order to reduce uplink reporting overhead.

- Proposal 6: For the quantization of the reported SGCS, the network can configure the number of uniformly divided range intervals over [0, 1] – ‘N’.

- Proposal 7: For the reporting contents of UE assisted performance monitoring, UE reports the index of the configured interval that corresponds to the calculated SGCS value.

- Proposal 8: Introduce a new quantity for SGCS in the CSI report to support performance monitoring of CSI prediction.

- Proposal 9: For the reporting contents of UE assisted performance monitoring, the size of the CSI field associated with SGCS reporting is log2N, where N is the number of uniformly divided range intervals over [0, 1], as configured in ‘CSI-ReportConfig’

- Proposal 10: For CSI prediction using UE side model, for inference, support Option 3, that both dedicated AI/ML PU and legacy CPU are occupied.

- Proposal 11: To indicate no CSI report in ‘CSI-ReportConfig’, reuse the existing parameter by setting the ‘reportQuantity’ field to ‘none’

- Proposal 12: Support the single configuration of model input and ground truth CSI.

References

- Draft Report of 3GPP TSG RAN WG1 #120bis v0.1.0

- Draft Report of 3GPP TSG RAN WG1 #118b v0.2.0