3GPP TSG RAN WG1 #121		                                                        R1- 2504149

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:	9.11.1

Source: 	ETRI

Title: 	Discussion on NR-NTN downlink coverage enhancement

Document for:	Discussion/Decision

Introduction

In RAN#106, the work item on Non-Terrestrial Networks (NTN) for NR Phase 3 was revised [1]. The objectives of the work item on NR-NTN downlink coverage enhancement are the following:

Specify downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Specify solutions, including link level enhancements for FR1-NTN and system level enhancements for FR1-NTN and FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - Link level enhancements are to be specified for the following channels: - PDCCH at least for CSS (except for Type-3) via PDCCH repetition - PDSCH with Msg4 via PDSCH repetition - PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration - System-level enhancements are to be specified for the following: - Support of extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is 160 ms - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km

In this contribution, the link level enhancements for NR NTN downlink coverage enhancements will be discussed.

Link-level enhancements

PDSCH with Msg4

The following agreement for PDSCH with Msg4 was made in RAN1#120bis [2].

Agreement For Msg4 PDSCH repetition scheme, the Msg4 PDSCH is repeated in N consecutive slots: - The same resource allocation is assumed for all repetitions - The supported repetition factors are: 2 and 4 - The network configures a single value between 2 and 4 at a given time - The RV cycling is used for each repetition - If N=4: Starting RV; RV applicable to the nth repetition/transmission. n mod 4 = 0; n mod 4 = 1; n mod 4 = 2; n mod 4 = 3. 0; 0; 2; 3; 1. 2; 2; 3; 1; 0. 3; 3; 1; 0; 2. 1; 1; 0; 2; 3. Agreement For enabling/disabling Msg4 PDSCH repetition, RAN1 to down-select among the following options: - Option 1: UE specific PDSCH with Msg4 repetition activation indicated via PDCCH- DCI Format 1_0 - FFS: indication details. - FFS: whether/how network is informed by the UE that certain conditions are met to trigger Msg4 PDSCH repetition (e.g. RSRP detected at UE is less than x dB) - Option 2: The enabling/disabling of Msg4 PDSCH repetition is implicitly indicated by the enabling/disabling of SIB1 PDSCH repetition. - Option 3: The enabling/disabling is indicated by SIB1 configuration - FFS: whether/how network is informed by the UE that certain conditions are met to trigger Msg4 PDSCH repetition (e.g. RSRP detected at UE is less than x dB) FFS: Whether UE reports its capability

Signaling for enabling/disabling Msg4 PDSCH repetition

When all UEs within a cell experience similar channel conditions, a cell-specific indication (Option 3) for Msg4 PDSCH repetition by SIB1 can be effective, with small signaling overhead and simplified design. When UEs within a cell experience different channel conditions, the UE-specific indication (Option 1) becomes beneficial. However, Option 2 can result in unnecessary repetitions. For example, when SIB1 with a payload size of 800 bits is transmitted at a CNR of –5.6 dB, unnecessary repetition of the SIB1 should be performed under Option 2.

Observation 1: Implicitly indicating the enabling/disabling of Msg4 PDSCH repetition by the enabling/disabling of SIB1 PDSCH repetition can lead to unnecessary repetitions.

Proposal 1: RAN1 to support Option 1 and/or Option3 for enabling/disabling Msg4 PDSCH repetition.

With limited time left for discussion, it can be a suitable approach to reuse the signaling mechanism from Rel-18 NR NTN for PUCCH repetitions for Msg4 HARQ-ACK. In this case, both Option 1 and Option 3 can be applied simultaneously without conflict: The repetition factor or factors are configured in SIB1. When single repetition factor is configured in SIB1, PDSCH with Msg4 repetition is configured as a cell-common operation. When multiple repetition factors are configured in SIB1, UE specific PDSCH with Msg4 repetition activation indicated via PDCCH- DCI Format 1_0.

Observation 2: By reusing the signaling mechanism from Rel-18 NR NTN for PUCCH repetitions for Msg4 HARQ-ACK, both Option 1 and Option 3 for enabling/disabling Msg4 PDSCH repetition can be applied simultaneously without conflict.

Proposal 2: RAN1 to reuse the signaling mechanism from Rel-18 NR NTN for PUCCH repetitions for Msg4 HARQ-ACK to also support the signaling of Msg4 PDSCH repetition.

For UE specific PDSCH with Msg4 repetition activation indicated via PDCCH- DCI Format 1_0, a specific field can be reused to indicate the number of Msg4 repetition transmissions where the DCI format for Msg4 scheduling is structured as follows:

Table 1: The fields in DCI scheduling Msg4 transmission

DCI_1_0 scrambled by TC_RNTI can be utilized to signal Msg4 resource allocation information. Fields such as MCS, HARQ, and DAI bits can be repurposed to indicate the number of Msg4 repetitions. Additionally, frequency hopping can be applied for Msg4 repetition transmission, and this information can also be conveyed through DCI field reuse. After transmitting Msg3, the UE reads the DCI associated with Msg4 and determines the number of times Msg4 will be repeated. For instance, if a 2-bit field is used to represent the number of repetitions, different values can correspond to different repetition levels as follows:

00 → 1 no repetition

01 → 2 repetitions

10 → 4 repetitions

11 → reserved

or

00 → 1 no repetition

01 → 2 repetitions

10 → 3 repetitions

11 → 4 repetitions

or

00 → 1 no repetition

01 → 2 repetitions

10 → 4 repetitions

11 → 8 repetitions

.

Proposal 3: RAN1 to use a field of PDCCH- DCI Format 1_0 to indicate the number of UE specific PDSCH with Msg4 repetition.

Method and signaling for triggering condition and UE capability report

For signaling how to initiate the repetitions of PDSCH with Msg4, RSRP based scheme can be considered. UE can assess whether Msg4 repetition transmission is required by measuring the received signal strength and informing the gNB accordingly: Before transmitting Msg1, the UE can measure the RSRP of the received SSB signals and compare the measured RSRP with a predefined or dynamically configured RSRP threshold. If the measured RSRP falls below the threshold, the UE can implicitly or explicitly request Msg4 repetition transmission from the gNB. UE can additionally utilize SIB1/SIB19 and/or Msg2 RAR to assess the RSRP of the received signal.

Proposal 4: RAN1 to consider the RSRP measuring and threshold-based reporting for UE request/capability reporting for Msg4 PDSCH repetition transmission.

Proposal 5: RAN1 to introduce new filed in SIB1 for RSRP threshold used to initiate Msg4 PDSCH repetition.

For signaling of UE capability report, UE can directly notify the gNB of the necessity for Msg4 repetition by embedding an indication in Msg3, such as using MAC CE (Control Elements) or upper-layer signaling fields within the message.

Proposal 6: RAN1 to consider UE request/capability report for Msg4 PDSCH repetition by embedding the indication in Msg3 such as using MAC CE.

PDSCH repetition of SIB1

RAN1#120bis made the following agreements [2].

Agreement When PDCCH CSS type-0 repetition is performed, for SIB1 link level enhancement, support PDSCH repetition of SIB1 transmitted within the same slot as the type0-CSS PDCCH repetition. - UE supporting SIB1 PDSCH coverage enhancement assumes that the PDCCH and associated PDSCH to be repeated in both slots where the corresponding PDCCHs are transmitted. - Each PDSCH SIB1 repetition is within the same slot of each PDCCH candidate for scheduling DCI - The two associated PDSCHs have the same RV FFS: Whether it is supported that type-0 PDCCH repetition is not performed while the PDSCH-SIB1 repetition is performed, and if so whether/how to handle the slot determination. Agreement For enabling/disabling SIB1 PDSCH repetition, RAN1 to consider the following options: - Option 1: Using reserved bit(s) in PBCH payload. - Option 2: Using scheduling PDCCH. - Option 3: The enabling/disabling of SIB1 PDSCH repetition is implicitly indicating by the enabling/disabling of Type-0 CSS PDCCH repetition.

Supporting PDSCH-SIB1 repetition when type-0 PDCCH repetition is not performed can avoid unnecessary repetition of type-0 PDCCH CSS. For example, when SIB1 with a payload size of 1280 bits is transmitted within a CNR range of –6 dB ≤ CNR ≤ –3.4 dB, unnecessary repetition of the Type-0 PDCCH CSS can be avoided. When SIB1 with a payload size of 800 bits is transmitted within a CNR range of –6 dB ≤ CNR ≤ –5.8 dB, unnecessary repetition of the Type-0 PDCCH CSS can be avoided; however, the benefit is marginal.

Observation 3: Supporting PDSCH-SIB1 repetition when type-0 PDCCH repetition is not performed can avoid unnecessary repetition of type-0 PDCCH CSS, particularly when considering SIB1 with a payload size of 1280 bits.

Proposal 7: RAN1 to supporting PDSCH-SIB1 repetition without type-0 PDCCH repetition.

For enabling/disabling SIB1 PDSCH repetition, we can use scheduling PDCCH for PDSCH-SIB1. The DCI format used to schedule SIB1 is DCI format 1_0 with CRC scrambled by SI-RNTI. The content of this DCI includes the following elements:

Table 2: Fields and contents in DCI format 1_0 with CRC scrambled by SI-RNTI

Among the fields, redundancy version field or MCS field can be reused to indicate SIB1 repetition. Alternatively, the reserved field in the DCI can be used to indicate retransmission. To indicate the time and frequency resource information of the retransmitted SIB1, the reserved bits in the DCI can be utilized. Since the reserved field consists of 15 bits, a portion of these bits can be allocated for this purpose.

Proposal 8: RAN1 to use scheduling PDCCH for enabling/disabling SIB1 PDSCH repetition.

To handle the slot determination of repeated PDSCH-SIB1 when supporting PDSCH-SIB1 repetition without type-0 PDCCH repetition, we can also use scheduling PDCCH for PDSCH-SIB1;

    - Step1>

Some field(s) such as redundancy version field, or MCS field, or reserved bits can be reused/used to indicate SIB1 repetition without type-0 PDCCH repetition.

    - Step2>

Option A (when supporting PDSCH-SIB1 repetition in non-consecutive slots): Some field(s) such as redundancy version field, or MCS field, or reserved bits can be reused/used to handle the slot determination and timing offset of the repeated PDSCH-SIB1 when supporting PDSCH-SIB1 repetition in non-consecutive slots.

Option B (when supporting PDSCH-SIB1 repetition only in consecutive slots): Some field(s) such as redundancy version field, or MCS field, or reserved bits can be reused/used to indicating the timing offset of repeated PDSCH-SIB1 from the consecutive slots

Proposal 9: RAN1 to use scheduling PDCCH for SIB1 PDSCH to support/handle PDSCH-SIB1 repetition without type-0 PDCCH repetition.

When PDSCH-SIB1 repetition without type-0 PDCCH repetition is not supported, implicit indicating of SIB1 PDSCH repetition by the enabling/disabling of Type-0 CSS PDCCH repetition can be a simple solution.  In this case, Option 1 and Option 3 become equivalent because enabling/disabling for PDCCH repetition for Type0 PDCCH CSS use reserved bit(s) in PBCH payload.

Observation 4:  When PDSCH-SIB1 repetition without type-0 PDCCH repetition is not supported, the Option 1 and Option 3 become equivalent.

Proposal 10: RAN1 to consider implicit indicating of SIB1 PDSCH repetition by the enabling/disabling of Type-0 CSS PDCCH repetition only when PDSCH-SIB1 repetition without type-0 PDCCH repetition is not supported.

Type0 PDCCH CSS

The following agreement regarding link-level enhancements for PDCCH repetition for Type0 PDCCH CSS was made in RAN1#120bis [2].

Agreement For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1: - Enabling/disabling using reserved bit(s) in PBCH payload - No UE behavior is defined for UE in connected mode specifically for the case where the network changes its signaling between enabling and disabling PDCCH repetition for Type0 PDCCH CSS. Agreement For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1, support repeated PDCCH candidates in the two consecutive slots n0 and n0+1 associated with the same SSB index ( n0 as defined in section 13 of TS 38.213) at least for M=2. - Repeated PDCCH candidates share the same aggregation level (AL), coded bits and same candidate index - Note: if the network repeats the Type 0 PDCCH across two consecutive slots, a legacy UE might decode the PDCCH and associated PDSCH in one slot and skip PDCCH monitoring in the other slot. Note: further discuss the potential solution for M=1 and M=1/2.

It is possible for supporting repeated PDCCH candidates in the two consecutive slots n0 and n0+1 associated with the same SSB index for M=1 and M=1/2.

Proposal 11: RAN1 to support repeated PDCCH candidates in the two consecutive slots n0 and n0+1 associated with the same SSB index.

Conclusions

Observation 1: Implicitly indicating the enabling/disabling of Msg4 PDSCH repetition by the enabling/disabling of SIB1 PDSCH repetition can lead to unnecessary repetitions.

Observation 2: By reusing the signaling mechanism from Rel-18 NR NTN for PUCCH repetitions for Msg4 HARQ-ACK, both Option 1 and Option 3 for enabling/disabling Msg4 PDSCH repetition can be applied simultaneously without conflict.

Observation 3: Supporting PDSCH-SIB1 repetition when type-0 PDCCH repetition is not performed can avoid unnecessary repetition of type-0 PDCCH CSS, particularly when considering SIB1 with a payload size of 1280 bits.

Observation 4:  When PDSCH-SIB1 repetition without type-0 PDCCH repetition is not supported, the Option 1 and Option 3 become equivalent.

Proposal 1: RAN1 to support Option 1 and/or Option3 for enabling/disabling Msg4 PDSCH repetition.

Proposal 2: RAN1 to reuse the signaling mechanism from Rel-18 NR NTN for PUCCH repetitions for Msg4 HARQ-ACK to also support the signaling of Msg4 PDSCH repetition.

Proposal 3: RAN1 to use a field of PDCCH- DCI Format 1_0 to indicate the number of UE specific PDSCH with Msg4 repetition.

Proposal 4: RAN1 to consider the RSRP measuring and threshold-based reporting for UE request/capability reporting for Msg4 PDSCH repetition transmission.

Proposal 5: RAN1 to introduce new filed in SIB1 for RSRP threshold used to initiate Msg4 PDSCH repetition.

Proposal 6: RAN1 to consider UE request/capability report for Msg4 PDSCH repetition by embedding the indication in Msg3 such as using MAC CE.

Proposal 7: RAN1 to supporting PDSCH-SIB1 repetition without type-0 PDCCH repetition.

Proposal 8: RAN1 to use scheduling PDCCH for enabling/disabling SIB1 PDSCH repetition.

Proposal 9: RAN1 to use scheduling PDCCH for SIB1 PDSCH to support/handle PDSCH-SIB1 repetition without type-0 PDCCH repetition.

Proposal 10: RAN1 to consider implicit indicating of SIB1 PDSCH repetition by the enabling/disabling of Type-0 CSS PDCCH repetition only when PDSCH-SIB1 repetition without type-0 PDCCH repetition is not supported.

Proposal 11: RAN1 to support repeated PDCCH candidates in the two consecutive slots n0 and n0+1 associated with the same SSB index.

References

- RP-243300 Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3

- Chair's notes RAN1#120bis, 9.11.1 NR-NTN downlink coverage enhancement