3GPP TSG RAN WG1 #121		R1-2504201

St Julian’s, Malta, May 19th - 23th, 2025

Source:	OPPO

Title:	Discussion on NR-NTN uplink capacity/throughput enhancement

Agenda Item:	9.11.3

Document for:	Discussion and Decision

Introduction

In RAN1 meeting #120b, the following agreements regarding NR-NTN uplink capacity/ throughput enhancement were achieved.

Agreement RAN1 assumes that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH. - FFS: under which conditions the above applies, e.g. under the same conditions that phase continuity applies for DMRS bundling Agreement Send LS to RAN4 on requirements for the phase continuity and power consistency: RAN1 has agreed the following: - RAN1 assumes that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH. - FFS: under which conditions the above applies, e.g. under the same conditions that phase continuity applies for DMRS bundling RAN1 ask RAN4 whether the same phase continuity requirements as for DMRS bundling can be applied for OCC length 2 and/or OCC length 4 under the same conditions as for DMRS bundling, or if new requirements are needed. Agreement The draft LS in R1-2503070 is endorsed. Final LS is in R1-2503071. Agreement OCC length and OCC sequence for OCC with CG PUSCH Type 1 is configured by RRC higher-layers. - Up to RAN2 whether to signal this with one or two RRC parameters - Note: OCC lengths and sequences to be provided in the table of RRC parameters to be prepared by the WI rapporteur Agreement For OCC with CG-PUSCH, the RV sequence applied across OCC groups is RRC configured among the RV sequences defined for legacy CG PUSCH, i.e., [0,2,3,1], [0,3,0,3] or [0,0,0,0]. - Note: no new RRC parameter is needed for the above - FFS: if OCC group dropping is later agreed, how to count RVs may need to be discussed for that case Agreement For resolving the overlapping PUSCH repetitions with inter-slot OCC and PUCCH with/without repetitions, the legacy timeline conditions for UCI multiplexing or prioritization for dropping PUSCH [/ PUCCH] applies according with the following update: - “the first PUSCH repetition of an OCC group” is used instead of “PUSCH repetition” for the legacy timeline condition, where the legacy PUSCH repetition that overlaps with a PUCCH belongs to the OCC group Note: how to capture the above agreement is up to the spec editor. Agreement For the OCC length applied for OCC DG PUSCH and CG PUSCH Type 2, consider the following options: - Option 1: Configured by RRC higher-layer parameter. - Note: this does not preclude jointly configuring OCC sequence and OCC length with the same RRC configuration - Option 2: Max value is configured by RRC higher-layer parameter, and the applied value is implicitly determined from the configured repetition number. - Option 3: Candidates values are configured by RRC higher-layer parameter, and the applied value is indicated by scheduling DCI for DG PUSCH or by activation DCI for CG PUSCH Type 2. - Note: this does not preclude jointly configuring OCC sequence and OCC length with the same RRC configuration - Option 4: Single value is indicated by scheduling DCI for DG PUSCH and by activation DCI for CG PUSCH Type 2 (no RRC configuration of candidate values). - Option 5: Max value is configured by RRC higher-layer parameter, and the applied value is indicated by scheduling DCI for DG PUSCH and by activation DCI for CG PUSCH Type 2. Note: it is not precluded to select a different option for DG PUSCH and CG PUSCH Type 2. Agreement If PUCCH without repetitions overlaps with inter-slot OCC with any PUSCH repetitions in an OCC group with a same priority index for PUCCH/PUSCH, the legacy conditions and rules for UCI multiplexing or prioritization for dropping applies with the following updates: - If the UCI is multiplexed on the PUSCH repetition according to legacy rules and the updated timeline conditions for UCI multiplexing are satisfied, UCI is multiplexed on all PUSCH repetitions without A-CSI reports within an OCC group with inter-slot OCC overlaps with the PUCCH. (Option 3-a) - FFS: PUSCH repetition with A-CSI reports - If the PUSCH repetition is dropped according to legacy rules and the updated timeline conditions for PUSCH dropping are satisfied, UCI is transmitted on PUCCH and all PUSCH repetitions within the OCC group that overlaps with the PUCCH are dropped (Option 2) - FFS: if PUCCH is overlap with PUSCH repetition in both time and frequency domain. - UE does not expect there are multiple PUCCHs without repetitions in different PUCCH slots with a same or different UCI types other than SR overlapping with multiple PUSCH repetitions in the same OCC group. - FFS: whether the above applies only when at least one of the overlapping PUCCHs result in a UCI being multiplexed on the PUSCH Note 1: If the UCI on the PUCCH is dropped according to legacy rules and [updated] timeline conditions for UCI dropping are satisfied, there is no [additional] spec impact. (Option 1) Note 2: There can be multiple PUCCHs with same or different UCI types in the same slot (i.e. CSI report and HARQ-ACK) as in the legacy specifications Working assumption 1: The above agreement applies to different priority indexes for PUCCH/PUSCH if no additional specification impact is identified. Working assumption 2: The above agreement applies to PUCCH with repetitions if no additional specification impact is identified. Agreement For the OCC sequence applied for OCC DG PUSCH and CG PUSCH Type 2, the sequence is indicated dynamically in DCI - FFS: with a new field or reusing an existing field

In this contribution, we further discuss the potential specification impacts to support OCC for PUSCH transmission in NR-NTN.

Discussion

It is agreed to support OCC length 2 and 4 with inter-slot OCC for PUSCH repetition Type A to multiplex up to 4 UEs in previous meetings. In this section, we discuss potential specification impacts, including OCC configuration, UCI multiplexing, and PUSCH frequency hopping for PUSCH transmission with OCC.

## 2.1 OCC configuration

In RAN1 meeting #120b, five candidate options for determining the OCC length for OCC DG PUSCH and CG PUSCH Type 2 are discussed. UE may support OCC length 2 or 4 based on its capability. One UE with OCC length-4 capability may be scheduled to multiplex with another UE with OCC length-2 capability dynamically. Therefore, from scheduling flexibility point of view, it is preferred to support dynamically indicating OCC length in the scheduling DCI for DG PUSCH or the activation DCI for CG PUSCH Type 2 (i.e., Option 4). For OCC sequence indication, the sequence associated OCC index can be indicated directly to UE or can be associated with DMRS port to reduce signaling overhead.

Proposal 1: For OCC DG PUSCH and CG PUSCH Type 2, support indicating OCC length and OCC index in the corresponding DCI.

## 2.2 UCI multiplexing

In RAN1 meeting #120b, it was agreed to support multiplexing UCI on all PUSCH repetitions within an OCC group with inter-slot OCC if conditions and rules for UCI multiplexing or prioritization are fulfilled. In this section, we discuss remaining issues on UCI multiplexing on PUSCH with OCC.

According to current specification, in the procedure of UCI and data multiplexing, if the number of HARQ-ACK information bits to be transmitted on PUSCH is 0, 1 or 2 bits and without CG-UCI, then there will be reserved REs for potential HARQ-ACK transmission, and the number of the reserved REs is calculated by assuming the number of HARQ-ACK information bits is 2. For UCI multiplexing on PUSCH with inter-slot OCC, the same rule should be reused and the resources for potential HARQ-ACK transmission should be reserved on all PUSCH repetitions within an OCC group. Figure 1 gives an example of resources reservation for potential HARQ-ACK transmission for PUSCH with and without inter-slot OCC.

Figure 1. Example of resources reservation for potential HARQ-ACK transmission for PUSCH with and without inter-slot OCC

Proposal 2: For UCI multiplexing on PUSCH with inter-slot OCC, if the number of HARQ-ACK information bits to be transmitted on PUSCH is 0, 1 or 2 bits and without CG-UCI, the resources for potential HARQ-ACK transmission should be reserved on all PUSCH repetitions within an OCC group.

For PUCCH without repetitions overlaps with PUSCH repetition Type A, UCI is mapped on one of the PUSCH repetitions, and the number of REs for UCI transmission on that PUSCH is calculated according to β factor, α factor, and the coding rate of TB on that PUSCH. For PUCCH without repetitions overlaps with PUSCH repetition Type A with inter-slot OCC, one UCI is expected to be transmitted on all PUSCH repetitions within one OCC group, which requires much more allocated UCI resources. To reduce UCI overhead in this case, one method is to configure separate β factors for PUSCH without OCC and PUSCH with OCC. However, this method may not solve the issue completely since the legacy range and candidate values of the β factor is reused. Besides, signaling overhead will be increased to configure the additional β factors. In our view, there is no need to introduce a separate β factor. A simple solution is to reduce the β factor in proportional to the OCC length.

Proposal 3: Do not introduce a separate β factor for UCI multiplexing on PUSCH with inter-slot OCC. The actual β factor is proportionally reduced based on the OCC length for calculating the number of REs allocated for UCI on each PUSCH.

## 2.3 PUSCH frequency hopping

For PUSCH repetition Type A, in case of inter-slot frequency hopping, PUSCH frequency hopping is performed in unit of one slot, i.e., the starting RB during even slot number within a radio frame is different from the starting RB during odd slot number. For PUSCH repetition Type A with inter-slot OCC, PUSCH frequency hopping should be performed in unit of OCC group to guarantee orthogonality of PUSCH transmissions between UEs.

Figure 2 gives an example. In Figure 2, the number of slots in each hop is determined according to OCC length.

Proposal 4: For PUSCH repetition Type A with inter-slot OCC, PUSCH frequency hopping should be performed in unit of OCC group if inter-slot frequency hopping is configured.

Figure 2. Example of PUSCH frequency hopping for PUSCH with inter-slot OCC

Conclusion

In this contribution, we discuss the potential specification impacts to support PUSCH repetition Type A with inter-slot OCC in NR-NTN. The following proposals are made.

Proposal 1: For OCC DG PUSCH and CG PUSCH Type 2, support indicating OCC length and OCC index in the corresponding DCI.

Proposal 2: For UCI multiplexing on PUSCH with inter-slot OCC, if the number of HARQ-ACK information bits to be transmitted on PUSCH is 0, 1 or 2 bits and without CG-UCI, the resources for potential HARQ-ACK transmission should be reserved on all PUSCH repetitions within an OCC group.

Proposal 3: Do not introduce a separate β factor for UCI multiplexing on PUSCH with inter-slot OCC. The actual β factor is proportionally reduced based on the OCC length for calculating the number of REs allocated for UCI on each PUSCH.

Proposal 4: For PUSCH repetition Type A with inter-slot OCC, PUSCH frequency hopping should be performed in unit of OCC group if inter-slot frequency hopping is configured.

Reference

- R1-2502267, Discussion on NR-NTN uplink capacity/throughput enhancement, OPPO.

- R1-2503094, Feature lead summary #4 of AI 9.11.3 on NR-NTN uplink capacity and throughput, Moderator (MediaTek).