3GPP TSG RAN WG1 #121           	                                   R1-2504263

St Julian’s, Malta, May 19th – 23th, 2025

Agenda Item: 9.5.2

Source: MediaTek Inc.

Title: On-demand SIB1 for idle or inactive mode UEs

Document for: Discussion and decision

- Introduction

In the WID of Rel-19 network energy savings (NES) [1], the objective of on-demand SIB1 for idle or inactive mode UEs are listed as below:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3]

  - Specify procedures and signaling method(s) for Case 2 [RAN1/2]

    - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell

    - Triggering method by UL WUS using PRACH

  - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3]

  - Note 1: No modification of SSB will be discussed under this objective

  - Note 2:

    - UL WUS: Uplink wake-up signal

    - Cell A: A cell that is periodically transmitting at least its own SIB1

    - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE

  - Note 3:

    - RAN1 strives to minimize impact to legacy UE

    - RAN1 specification impact to support this feature should be minimized

In RAN1 #120b [2], the following is agreed:

Conclusion

There is no RAN1 consensus to support SSB with on-demand SIB1 not on sync raster.

Agreement

The following agreement is updated as follows:

The UE assumes that, in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in PDCCH monitoring occasions corresponding only to at least the SSB associated with the transmitted PRACH for UL-WUS if this is indicated via UL WUS configuration by a 1-bit indication.

- If the 1-bit indication is included in UL WUS config and indicate as TRUE, the UE assumes that, in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in PDCCH monitoring occasions corresponding only to the SSB associated with the PRACH for UL-WUS.

- If the 1-bit indication is not included in UL WUS config or the 1-bit indication is FALSE, the UE assumes the same as legacy, i.e., in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB.

- Further study possible down selection from the following options to indicate additional information related to SSBs associated with PDCCH monitoring occasions.

  - Alt1: The information is provided in UL-WUS config

  - Alt2: The information is provided in RAR in response to UL-WUS transmission

  - Alt3: No additional information is provided

Agreement

Indication of K_SSB value in the UL-WUS configuration should be 5 bits for FR1 and 4 bits for FR2.

- Note: there is no change to current PBCH structure.

- UE does not expect K_SSB to be larger than 23 for FR1 and larger than 11 for FR2.

Agreement

The value of the time offset between the reference time point and the starting time for the time window of OD-SIB1 can be either 0 or positive value.

Agreement

After transmitting a UL-WUS, UE is not required to monitor Type0-PDCCH occasion (for OD-SIB1) before UE successfully received its RAR in response to the UL WUS.

Agreement

If a UE has SIB1 request configuration of a cell and before transmitting UL WUS,

- If the UE detects a SSB where K_SSB>=24 for FR1 or K_SSB>=123 for FR2, select the following:

  - Alt. 3: It is up to UE implementation on whether to monitor Type 0 PDCCH for SIB1 transmission

Agreement

In the UL WUS configuration, how to support multiple NES-CellId(s) is up to RAN2 discussion.

Agreement

From RAN1 perspective, for agreed UL WUS parameters, regarding their mandatory or optional presence and applicability to TDD and/or FDD, adopt the followings:

- PhysCellId and ARFCN-ValueNR are mandatory

- frequencyBandList and absoluteFrequencyPointA are present in IE FrequencyInfoUL for FDD (as in the legacy specification)

- K_SSB is mandatory

- searchSpaceZero and controlResourceSetZero are mandatory

- ra-PreambleStartIndex, od-sib1-duration, offsetToTimeWindow are mandatory

Agreement

The parameter ra-SearchSpace in the UL-WUS configuration is of type SearchSpace.

Agreement

Replace the current parameter name offsetToTimeWindow with od-sib1-windowStartOffset (to make the naming more comprehensive).

Agreement

Include CarrierBandwidth and locationAndBandwidth in the UL-WUS configuration.

- Previous RAN1 #120 conclusion is reverted

Agreement

The following previous RAN1 #118-bis agreement is updated as:

- Msg1-FrequencyStart is with respect to the first RB of the UE carrier InitialUplinkBWP determined by locationAndBandwidth offsetToCarrier and absoluteFrequencyPointA (same as legacy)

Agreement

RAN1 to discuss the value range for od-sib1-duration. For example,

- Option 1: Use the value-range for si-windowLength (from 5 slots to 5120 slot) as starting point.

- Option 2: The duration of the time window can be a multiple of 160ms (i.e. legacy SIB1 periodicity).

- Option 3: The duration of the time window can be a multiple of the on-demand SIB1 repetition periodicity (if supported)

- Other options are not precluded.

Agreement

RAN1 clarifies frequencyBandList in UL WUS configuration refers to the IE within FrequencyInfoUL-SIB.

Also, RAN2 has discussed on-demand SIB1 in RAN2 #125-bis to #129-bis and the corresponding agreements are collected in Appendix A.

This contribution provides our views of this study (on-demand SIB1 for idle or inactive mode UEs).

- Discussion of detailed design of on-demand SIB1 and UL WUS

2.1 Transmission for on-demand SIB1 with the association of SSB(s) based on a received UL-WUS

It is agreed in RAN1 #120b that

Agreement

The following agreement is updated as follows:

The UE assumes that, in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in PDCCH monitoring occasions corresponding only to at least the SSB associated with the transmitted PRACH for UL-WUS if this is indicated via UL WUS configuration by a 1-bit indication.

- If the 1-bit indication is included in UL WUS config and indicate as TRUE, the UE assumes that, in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in PDCCH monitoring occasions corresponding only to the SSB associated with the PRACH for UL-WUS.

- If the 1-bit indication is not included in UL WUS config or the 1-bit indication is FALSE, the UE assumes the same as legacy, i.e., in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB.

- Further study possible down selection from the following options to indicate additional information related to SSBs associated with PDCCH monitoring occasions.

  - Alt1: The information is provided in UL-WUS config

  - Alt2: The information is provided in RAR in response to UL-WUS transmission

  - Alt3: No additional information is provided

For Alt. 1, providing additional information of SSB in UL-WUS can not adapt with the time-variant channel conditions for each beam. For Alt. 2, changing RAR contents would have large RAN1/RAN2 spec impact. Hence, we think Alt 3 should be adopted.

Observation 1: On indication of additional information related to SSBs

- For Alt. 1, providing additional information of SSB in UL-WUS can not adapt with the time-variant channel conditions for each beam.

- For Alt. 2, changing RAR contents would have large RAN1/RAN2 spec impact.

Observation 2: The following RAN2 agreement is achieved in RAN2 #129b

- “Align with legacy RAR for OSI for OD-SIB1 operation. Legacy RAR MAC PDU subheader with RAPID only to be used as NW acknowledgement for OD-SIB1 request.”

Proposal 1: On indication of additional information related to SSBs, RAN1 to adopt

- Alt. 3: No additional information is provided.

2.2 Issue of UL-WUS on NES cell overlapped with paging occasion on Cell A

A potential issue of paging dropping may arise if UL-WUS on NES cell overlapped with paging occasion on Cell A as shown in Figure 1. RAN1 can further discuss how to solve this issue.

Figure 1. An Example of UL-WUS on NES cell overlapped with paging occasion on Cell A

Proposal 2: A potential issue of paging dropping may arise if UL-WUS on NES cell overlapped with paging occasion on Cell A as shown in Figure 1. UE should prioritize paging if the overlapping happens.

2.3 Value range for od-sib1-duration

It is agreed in RAN1 #120b that

Agreement

RAN1 to discuss the value range for od-sib1-duration. For example,

- Option 1: Use the value-range for si-windowLength (from 5 slots to 5120 slot) as starting point.

- Option 2: The duration of the time window can be a multiple of 160ms (i.e. legacy SIB1 periodicity).

- Option 3: The duration of the time window can be a multiple of the on-demand SIB1 repetition periodicity (if supported)

- Other options are not precluded.

As SIB1 contents would be repeated within 160ms, it seems reasonable to adopt Option 2 above.

Proposal 3: For value range for od-sib1-duration, as SIB1 contents would be repeated within 160ms, adopt

- Option 2: The duration of the time window can be a multiple of 160ms (i.e. legacy SIB1 periodicity).

2.4 UE behavior when OD-SIB1 window starting time falls into the middle of SSB burst

During RAN1 #120-bis, it was discussed (Issue 10 of the FL summary [3]) how to define the UE behavior when OD-SIB1 window starting time falls into the middle of SSB burst. If the value range for od-sib1-duration can be set to multiple of 160ms, we think it can be up to UE implementation to handle this scenario.

Proposal 4: When OD-SIB1 window starting time falls into the middle of SSB burst, it is up to UE implementation to handle this scenario.

- Assuming the value range for od-sib1-duration is set to multiple of 160ms.

- Conclusion

In this contribution, we focus on the discussions of on-demand SIB1 for idle or inactive mode UEs and have the following proposals:

Observation 1: On indication of additional information related to SSBs

- For Alt. 1, providing additional information of SSB in UL-WUS can not adapt with the time-variant channel conditions for each beam.

- For Alt. 2, changing RAR contents would have large RAN1/RAN2 spec impact.

Observation 2: The following RAN2 agreement is achieved in RAN2 #129b

- “Align with legacy RAR for OSI for OD-SIB1 operation. Legacy RAR MAC PDU subheader with RAPID only to be used as NW acknowledgement for OD-SIB1 request.”

Proposal 1: On indication of additional information related to SSBs, RAN1 to adopt

- Alt. 3: No additional information is provided.

Proposal 2: A potential issue of paging dropping may arise if UL-WUS on NES cell overlapped with paging occasion on Cell A as shown in Figure 1. UE should prioritize paging if the overlapping happens.

Proposal 3: For value range for od-sib1-duration, as SIB1 contents would be repeated within 160ms, adopt

- Option 2: The duration of the time window can be a multiple of 160ms (i.e. legacy SIB1 periodicity).

Proposal 4: When OD-SIB1 window starting time falls into the middle of SSB burst, it is up to UE implementation to handle this scenario.

- Assuming the value range for od-sib1-duration is set to multiple of 160ms.

- Reference

RP-242354, “Revised WID: Enhancements of network energy savings for NR”, Ericsson, RAN #105

Chairman’s Notes (Younsun’s Session), RAN1 #120b (Rel-19 9.5.2 On-demand SIB1)

R1-2503008, “FL summary 5 for on-demand SIB1 in idle/inactive mode”, Moderator (MediaTek), RAN1 #120-bis

Appendix A: RAN2 #125b/#126/#127/#127b/#128/#129 agreements for on-demand SIB1

RAN2 #125-bis

Agreement

At least RAN2 starts scenario 1a. Other scenarios are not excluded.

- Scenario 1a: Cell A SIB assisted intra-cell WUS. And WUS and SIB1 is sent to/from NES cell. with below potential RAN2 impacts:

  - Add WUS configuration in SIB of cell A.

  - Cell reselection from cell A to NES cell, including trigger condition and cell barring changes.

  - Whether allow camping, paging and SIB update in NES cell.

  - Cell reselection from NES cell to cell A or normal cell.

Agreement

RAN2 assume that RACH procedure is reused for UE to request on-demand SIB1.

Agreement

UL WUS configuration includes at least below information:

- RACH configuration

Agreement

A UE needs to know a UL WUS configuration to request SIB1 of which cell.

Agreement

Existing Msg 1 based on-demand procedure is reused for on-demand SIB1 acquisition procedure.

- FFS on Msg 3.

- FFS if / when the UE monitors the OD-SIB1 upon reception of RAR.

- FFS: whether introduce specified UE behavior if RACH failure of OD-SIB1 request.

Agreement

The UE first should acquire valid SIB1 (e.g. via SIB1 request) for camping to NES cell (if the UE knows the cell doesn’t broadcast SIB1 and supports on-demand SIB1).

RAN2 #126

Agreement

Study on-demand SIB1 provisioning for NES Cell(s) in versions of Scenario 1a with multiple Cells A and/or NES Cells:

a.	More than one Cell A may provide configuration for the same NES cell.

b.	The same Cell A may assist more than one NES Cells.

Agreement

RRC release message assisted intra-cell WUS can be discussed as option of signaling details in stage 3.

Agreement

Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with a NES Cell.

Agreement

For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline.

Agreement

Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB.

Agreement

If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

Agreement

After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell.

Agreement

RAN2 not to support on-demand SIB1 request that is combined with an initial access to perform RRC connection establishment/resume on the NES cell.

Agreement

NW need to bar the legacy UE from accessing the on-demand SIB1 cell (e.g. based on the existing barring mechanism).

Agreement

Study how to avoid/deprioritize the legacy UE camping at the Cell A attempting to switch to the NES Cell (but allowing the R19 NES UE to do that).

Agreement

If the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment. R19 NES UE bars the cell if the UE fails to acquire SIB1 via on-demand SIB1 for NES cell.

Agreement

After Rel-19 NES UEs camp in NES cell, the UE behaviour is same as the one defined as legacy normal camped state, e.g. paging reception, SIB1 update, etc.

Agreement

RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

Agreement

As baseline, upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation.

Agreement

Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.

Agreement

RAN2 to wait for RAN1’s progress whether to support scenario 3 (case 1 in RAN1).

RAN2 #127

Agreements

Scenarios:

1. No consensus for RAN1 case 3 in RAN2.

OD-SIB1 Validity:

2. We can rely on legacy UE behaviour to ensure UE has valid SIB1 for the cell (i.e. UE reacquire SIB1 whenever (re)selecting cell).

3. Further UE keeps SIB1 updated while on cell via regular SI modification procedure (confirmation of earlier RAN2 agreement).

WUS configuration of NES cell from NES cell:

4. Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

MSG3 based OD-SIB1 REQ:

5. Msg 3 based OD-SI procedure is not supported for on-demand SIB1 request in case 2 (for requesting to the NES Cell).

Access restriction for legacy UEs:

6. Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded.

- Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB.

- Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

- Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

Impacts to RRC connected UEs:

7. RAN2 understands the NW can avoid impact on legacy RRC connected UE and R19 RRC connected UE due to on-demand SIB1 (e.g. NES cell with OD-SIB1 is measured by legacy RRC_CONNECTED UE and can be configured as its PSCell/SCells/target cell).

OD-SIB1 SI conclusion:

8. RAN2 conclude that on-demand SIB1 is feasible from RAN2 perspective and recommend normative work of case 2 for on-demand SIB1.

RAN2 #127-bis

Agreements

- We will inherit all agreements made during SI phase to WI phase.

- A NES cell can include neighbouring NES cell’s WUS configuration.

- NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration).

- In on-demand SIB1 procedure, the UE considers RACH failure when PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1.

- The MAC layer will indicate the RACH failure for SI request to upper layers. FFS: after that the UE upper layer will consider the cell as barred.

- The legacy UE behaviour can be reused upon on-demand SIB1 acquisition failure, i.e., the NES UE should follow the intraFreqReselection in MIB of NES cell.

- A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

- If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

- Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced.

- NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).

- RAN2 will not start the discussion on the issue when the UL WUS configuration update in Cell A is not synchronised with the UL WUS configuration update in the NES cell, unless we’re asked to do that by other WG, e.g. RAN3.

RAN2 #128

Agreements

- NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing K_SSB < 24 for FR1 and K_SSB < 12 for FR2, will acquire SIB1 as in legacy.

- New NES-specific reselection priority parameters for NES UEs are defined for the purpose of prioritizing/deprioritizing a NES frequency.

- Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs.

- Reuse legacy cell reselection criterion as trigger condition of OD-SIB1 acquisition. No need to specify other conditions (e.g. a new RSRP threshold).

- The existing 1-second rule in the cell reselection criteria is still applied to the triggering condition of UL WUS transmission.

- The UE considers the cell as barred after MAC indicates max number of preamble transmission for the OD-SIB1 request.

- If MSG2 (ACK) is received, but UE fails to receive SIB1 then the UE may consider this cell as barred, no spec impact.

- A UE bars the NES/SIB1 less cell and/or excludes it as a candidate for reselection since the UE had no corresponding UL WUS configuration, the UE would treat this cell as if cell status is “not barred” and consider it as candidate for cell reselection once it has received a UL-WUS configuration to request SIB1 for this cell.

- UE considers WUS configuration only valid in directly succeeding cell reselection from cell where UE acquired WUS configuration. FFS on SI validity area rather than cell level.

- Working assumption: UL-WUS configuration in RRC release is not supported.

RAN2 #129

Agreements

- There is no need for additional barring mechanisms (in addition to the k_ssb signaling “no SIB1” indication in MIB) to handle legacy to be able to bar cell using OD-SIB1.

- Specify the following UE behavior to allow the UEs in RRC_CONNECTED state to acquire OD-SIB1 when T311 is running:

- When T311 is running, the UE can trigger the OD-SIB1 acquisition procedure with stored UL WUS configuration in SIB-X, if it is still valid.

- The legacy cell selection criteria are reused as the trigger condition of OD-SIB1 acquisition.

- The OD-SIB1 acquisition behavior is same as that of RRC_IDLE/IANCTIV UEs.

- The UE follows the legacy validity principle of stored SIB.

- For Rel-19 NES UE in RRC_CONNECTED, rely on the NW dedicated RRC for SIB1 delivery if searchSpaceSIB1 is not configured. It is legacy UE behavior and no spec change is expected.

- SIB-x can be cell specific or area specific, as legacy.

- Upon reception of RAR, the UE monitors OD-SIB1 in the window agreed by RAN1.

RAN2 #129-bis

Agreements

- NW ensures that the RRC connected UE has the latest SIB1 (e.g. dedicated RRC message to deliver SIB1 or not configure searchSpaceSIB1), as baseline. UE understands that the stored SIB1 is the latest SIB1.

- When the cell supporting on demand SIB1 is broadcasting SIB1 (e.g. upon SIB1 request), legacy UE can camp on the cell if the legacy UE is able to acquire the broadcasted SIB1. No specification impact is foreseen.

- Confirm the working assumption that “UL-WUS configuration in RRC release is not supported”.

- SIBX that was acquired during RRC connected state can be used for OD-SIB1 request in RLF.

- Align with legacy RAR for OSI for OD-SIB1 operation. Legacy RAR MAC PDU subheader with RAPID only to be used as NW acknowledgement for OD-SIB1 request.

- The UL WUS configuration includes the IE totalNumberOfRA-Preambles.

- Backoff is not applied to OD-SIB1 request when backoff is included in RAR.

- Leave this issue (how to handle paging interruption during OD-SIB1 acquisition) to RAN4.

- If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE may consider the cell as being barred.

- WUS configuration can be associated with a list of cells if the whole WUS configuration is same.

- Area id will be used as legacy.

- We do not need a separate new triggering condition of OD-SIB1 acquisition.

- When timer T311 is running, SIB1 acquisition triggering condition is same as legacy. No additional spec impact is foreseen.