3GPP TSG RAN WG1 #121		             R1-2504303

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	5

Source:	Apple

Title:	Discussion on the LS on UL Tx switching for TEI19

Document for:	Discussion/Decision

# Introduction

In this contribution, we discuss our views on the RAN4 LS on the newly approved TEI19 for enhancements to UL Tx switching.

# Discussion

In [1], RAN4 provided following information:

RAN4 would like to inform RAN1 that RAN4 discussed the following scenarios of Tx switching enhancement for 2 configured UL bands in Rel-19 TEI.

- Scenario 1. Switching between the following 2 cases for 3Tx UEs configured with uplinkTxSwitchingOption set to 'dualUL’

  - Case 1-1: Two antenna ports on Band A and one antenna port on Band B

  - Case 1-2: One antenna port on Band A and two antenna ports on Band B

- Scenario 1a. Switching between the following 2 cases for 3Tx UEs configured with uplinkTxSwitchingOption set to ‘switchedUL’

  - Case 2-1: Three antenna ports on Band A and zero antenna port on Band B

  - Case 2-2: Zero antenna port on Band A and three antenna ports on Band B

- Scenario 2. Switching between the following 2 cases for 4Tx UEs configured with uplinkTxSwitchingOption set to ‘switchedUL’

  - Case 3-1: Four antenna ports on Band A and zero antenna port on Band B

  - Case 3-2: Zero antenna port on Band A and Four antenna ports on Band B

RAN4 agreed to support at least scenario 1 in Rel-19 TEI. Whether RAN4 will support scenario 1a and scenario 2 in Rel-19 TEI is pending on RAN1 decision in RAN1#121 meeting.

The related UE capabilities will be addressed later together with Rel-19 feature list.

RAN4 respectfully asks RAN1 to:

- Introduce UL Tx switching functionalities for scenarios 1 in Rel-19 RAN1 specifications as part of TEI-19.

- Decide at RAN1#121 whether to support scenario 1a and/or scenario 2 in Rel-19 as part of TEI-19, and feedback to RAN4 the decision.

Based on above LS, scenario 1 is already agreed in RAN4 and RAN1 is required to identify the corresponding specification enhancements. From RAN1 perspective, following aspects need to be discussed and agreed up on to determine the specification enhancements:

- Ports combinations corresponding to case 1-1 and case 1-2 for scenario 1

- Whether/what ambiguity may exist (similar to ambiguity issue in Rel-17 dualUL) and if any, how to handle it in RAN1

- Identify the cases that require change in the Tx states corresponding to the 2 bands

Tables 1 shows all possible ports’ combinations for case 1-1 and case 1-2 as per the legacy dualUL. However, in our view, supporting same ports’ combinations for the two cases could result in ambiguity at the UE side due to potential misalignment between UE and network. Therefore, we should avoid supporting same ports’ combinations for both case 1-1 and case 1-2. To resolve this issue, (0P+1P) is defined to be associated with 1T+2T only, and (1P+0P) is defined to be associated with 2T+1T only. This procedure not only resolves potential ambiguity but also utilizes both Tx chains for 1P transmission. In Table 1, we strike through invalid combinations for each Tx state. For (1P+1P), either RRC indication or fix association to one of Tx states (1T+2T or 2T+1T) could be considered or UE capability could be introduced on whether UE can support 1P+1P for both cases or only one of the cases.

Table 1: Ports combinations for scenario 1

| Cases | Tx Antennas | Antenna Ports Combinations |
| --- | --- | --- |
| Case 1-1 | 2T+1T | (1P+0P), (2P+0P), (2P+1P), (1P+1P), (0P+1P) |
| Case 1-2 | 1T+2T | (1P+0P), (1P+1P), (1P+2P), (0P+2P), (0P+1P) |

Proposal 1: For scenario 1, avoid supporting same port combinations for both case 1-1 and case 1-2.

Based on proposal 1 and above considerations, for any port indication which is not associated with the current Tx state, an uplink Tx switching is triggered, and NW shall meet the switching gap accordingly. Switching is needed for the following scenarios in which UE is not expected to transmit on any of the bands during the switching gap:

- When the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission was a 2-port transmission on another uplink carrier

- When the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission was a 1-port transmission on another uplink carrier and the UE is under the operation state in which 2-port transmission can be supported on the same uplink carrier

- When the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission was a 1-port transmission on the same uplink carrier and 1-port transmission on another uplink carrier and the UE is under the operation state in which 2-port transmission cannot be supported in the same uplink carrier

Based on above, RAN1 CR for TS 38.214 can be introduced as an additional section under clause 6.1.6

Proposal 2: Switching is needed for the following scenarios in which UE is not expected to transmit on any of the bands during the switching gap:

- When the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission was a 2-port transmission on another uplink carrier

- When the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission was a 1-port transmission on another uplink carrier and the UE is under the operation state in which 2-port transmission can be supported on the same uplink carrier

- When the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission was a 1-port transmission on the same uplink carrier and 1-port transmission on another uplink carrier and the UE is under the operation state in which 2-port transmission cannot be supported in the same uplink carrier

For other two scenarios, i.e. scenario 1a and scenario 2, RAN4 has asked RAN1 to discuss them and corresponding cases and decide by RAN1#121 on whether any of these two scenarios will be supported or not. In our view, we don’t identify any current deployments or benefit when 3 or more Tx are associated with either band for the 2 band switching cases. Especially, for FDD bands, we don’t see this happening Therefore, from lack of motivation from practical deployments itself, we should not consider support scenario 1a and scenario 2. Additionally, typically we don’t expect a TEI from 1 working group to have any major impact on another working group, unless just a CR discussion could handle that. For scenario 1a and scenario 2, it might require more discussion beyond that. From this perspective as well, we should not go beyond the already agreed scenario 1 in RAN4.

Observation 1: Currently, we don’t identify deployments with UL Tx switching for band combinations of TDD+TDD and TDD+FDD bands, where 3 or 4 Tx are associated with one band, particularly not with FDD

- Therefore, we don’t see the benefit to support scenario 1a and scenario 2

Proposal 3: Scenario 1a and scenario 2 are not supported for TEI19 on enhancements to UL Tx switching

# Conclusion

In this contribution, we made the following observations/proposals related to the RAN4 LS on the TEI for enhancements to UL Tx switching:

Observation 1: Currently, we don’t identify deployments with UL Tx switching for band combinations of TDD+TDD and TDD+FDD bands, where 3 or 4 Tx are associated with one band, particularly not with FDD

- Therefore, we don’t see the benefit to support scenario 1a and scenario 2

Proposal 1: For scenario 1, avoid supporting same port combinations for both case 1-1 and case 1-2.

Proposal 2: Switching is needed for the following scenarios in which UE is not expected to transmit on any of the bands during the switching gap:

- When the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission was a 2-port transmission on another uplink carrier

- When the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission was a 1-port transmission on another uplink carrier and the UE is under the operation state in which 2-port transmission can be supported on the same uplink carrier

- When the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission was a 1-port transmission on the same uplink carrier and the UE is under the operation state in which 2-port transmission cannot be supported in the same uplink carrier

Proposal 3: Scenario 1a and scenario 2 are not supported for TEI19 on enhancements to UL Tx switching

# References

- R1-2503217, “LS on UL Tx switching for TEI19”