3GPP TSG RAN WG1 #120bis			R1-2504375

St Julian’s, Malta, May 19th – 23rd, 2025

Source:	Sharp

Title:	Cross-link interference (CLI) handling

Agenda Item:	9.3.3

Document for:	Discussion and Decision

# Introduction

Cross-link interference (CLI) management was discussed in the previous meetings, and the following was agreed on uplink resource muting for gNB-to-gNB CLI measurement [1-3]:

RAN1#118: Agreement For the time location configuration of UL resource muting for a PUSCH, one of the following options is selected - Option 1: Semi-statically configure the position for each of the up to two UL muting symbols within a slot - Option 2: Semi-statically configure X (X>=1) possible positions for each of the up to two UL muting symbols within a slot Note: Above has no implication on how UL muting symbol indication/determination is done. Note: For both options, there will at most up to 2 UL muting symbols for the PUSCH. Agreement For the reference point of time location of UL resource muting for PUSCH, Option 1 is supported - Option 1: Starting symbol of a slot for both PUSCH mapping type A and type B RAN1#119: Agreement To determine the time location of UL muting symbol(s) in a slot for a PUSCH, the following option is selected for DG PUSCH and Type 2 CG PUSCH - Option 2: The time location of each of one or two UL muting symbols is semi-statically configured, and muting all of the semi-statically configured time location of UL muting symbol(s) can be dynamically turned ON/OFF by TDRA field in DCI RAN1#120: Agreement For the semi-statically configured time location and frequency location of UL muting symbol(s) for PUSCH, - Separate configurations of up to two UL muting symbols can be provided per UE for DG PUSCH/Type 2 CG PUSCH and Type 1 CG PUSCH - Separate configuration for each of multiple Type 1 CG PUSCH configurations is supported Agreement Confirm the following working assumption Working assumption For the frequency location configuration of UL resource muting for a PUSCH, for DFT-S-OFDM - A comb offset {0, 1} is configured for the up to two UL muting symbols Agreement For the configuration of UL resource muting - Alt.1: A new IE PUSCH-MutingResources is introduced to define the time location and frequency location of UL muting resources. The UL muting pattern configures up to two symbols within a slot and a configurable comb offset. Agreement A new parameter ul-MutingIndicator is introduced to PUSCH-Allocation-r16 to indicate whether the semi-statically configured UL muting symbol(s) is enabled. Agreement For the PUSCH symbol containing PT-RS with UL resource muting when transform precoding is not enabled, 3 dB power boosting for both PUSCH and PTRS REs if present, regardless of the PTRS density K and number of PUSCH layers L.

In this paper, we provide views on uplink resource muting for gNB-to-gNB CLI measurement.

# Discussion

It was agreed in RAN1#120 to introduce a new IE PUSCH-MutingResources to indicate the time location and frequency location of UL muting resources. The configuration is to indicate up to two symbols within a slot and a configurable comb offset for a comb-2 pattern. It was further agreed for the comb offset to take the values 0 or 1, which can be used to indicate UL resource muting on even or odd REs, respectively.

For the configuration of uplink resource muting, a ZP SRS can be configured for one or two symbols within a slot that indicates a comb pattern on even or odd REs of all the allocated PRBs.

In summary, PUSCH-MutingResources is to include at least the following parameters:

- Comb offset = {0, 1}

- Time-domain position for the first muting symbols and optionally for the second muting symbol

With respect to the frequency-domain position, it was agreed in RAN1#118-bis that the UL muting pattern is to be applied to the all the allocated PRBs of the PUSCH on the muting symbols.

With respect to whether the UE is to apply the semi-statically configured UL resource muting, it was agreed in RAN1#120 to introduce a new parameter ul-MutingIndicator in PUSCH-Allocation-r16 to indicate whether the UL resource muting is enabled. Since this is an RRC parameter, the latency of the indication is not sufficient for dynamic indication.

Therefore, it was agreed in RAN1#119 that muting the one or two UL muting symbol(s) can be dynamically turned ON/OFF by a TDRA field in the DCI. For CG PUSCH Type 2, the TDRA field can be included in the DCI message that activates the transmission of the CG PUSCH.

For DG PUSCH, the TDRA field can be included in the DCI format 0_1, 0_2, or 0_3 message that schedules the PUSCH.

Proposal 1: Support including the TDRA field for turning the UL muting ON/OFF in the DCI format 0_x message that provides CG PUSCH Type 2 grant or schedules DG PUSCH.

The gNB may or may not turn the UL resource muting ON/OFF based on the periodicity of the CLI measurements intended by the gNB. If the gNB does not intend to turn the UL resource muting ON/OFF, the TDRA field in the DCI imposes an unnecessary overhead. Therefore, RRC should be able to indicate to the UE whether to expect the TDRA field in the DCI.

Proposal 2: Support RRC indicating to the UE whether to expect the TDRA field for turning the UL muting ON/OFF.

It should be noted that a DCI message is absent for granting CG PUSCH Type 1. Therefore, a DCI message can be dedicated to communicating the TDRA field for turning the UL muting ON/OFF.

Proposal 3: Consider dedicating a DCI message for the case of CG PUSCH Type 1 to communicate the TDRA field for turning the UL resource muting ON/OFF.

With respect to the following agreement from RAN1#120:

Agreement The following agreement replaces the earlier agreement from RAN1#119 If transform precoding is enabled for a PUSCH, the following options are provided for the purpose of specification drafting (how UE implementation is done is left to each company). - Option 1: The DFT size is modified as MscPUSCH2 for symbol(s) with UL resource muting - The MscPUSCH2 modulated symbols are transformed to the frequency domain with a DFT of size MscPUSCH2, and the result is mapped to the MscPUSCH2 unmuted scheduled subcarriers. - Option 2: The DFT size is MscPUSCH for symbol(s) with UL resource muting - Option 2A: The block of MscPUSCH2 modulated symbols is repeated before DFT. After DFT, the MscPUSCH2 non-zero even-indexed subcarriers are extracted and then mapped to the unmuted scheduled subcarriers. - Option 2B: The block of MscPUSCH2 modulated symbols is extended to MscPUSCH modulated symbols by inserting ‘0’ after every modulated symbol. After DFT, the first MscPUSCH2 subcarriers are extracted and then mapped to the unmuted scheduled subcarriers. Note: MscPUSCH is the total number of subcarriers of the PUSCH. Note: Above options are mathematically equivalent. Note: It is up to the spec editor how to capture the above as long as the specification captures the above in a mathematically identical manner.

It was agreed in the RAN1 meeting to include both options and leave it to the editor to decide how to capture the agreement with a note that the two options are equivalent. We do not consider this the best practice. The two options are not articulated similarly, and the task of converting multiple “mathematically identical” options to the specification text should not be left to the editor.

One may realize from the agreement that the mention of DFT size is unnecessary. What the specification should capture is to zero the MscPUSCH2 even or odd subcarriers on the symbol based on the indicated comb offset. How this is implemented, whether it is by reducing the DFT size or otherwise, should be left to implementation and does not need to be captured in the specification.

We suggest that to revoke the agreement to follow best practices and replace the agreement with a simple version that can be captured by the editor in a straight-forward manner.

Proposal 4: Revoke the following agreement and replace it with a simple version that specifies to zero the MscPUSCH2 even or odd subcarriers on the symbol based on the indicated comb offset:

Agreement The following agreement replaces the earlier agreement from RAN1#119 If transform precoding is enabled for a PUSCH, the following options are provided for the purpose of specification drafting (how UE implementation is done is left to each company). […]

# Conclusion

In this paper, we discussed cross-link interference (CLI) management and proposed the following:

Proposal 1: Support including the TDRA field for turning the UL muting ON/OFF in the DCI format 0_x message that provides CG PUSCH Type 2 grant or schedules DG PUSCH.

Proposal 2: Support RRC indicating to the UE whether to expect the TDRA field for turning the UL muting ON/OFF.

Proposal 3: Consider dedicating a DCI message for the case of CG PUSCH Type 1 to communicate the TDRA field for turning the UL resource muting ON/OFF.

Proposal 4: Revoke the following agreement and replace it with a simple version that specifies to zero the MscPUSCH2 even or odd subcarriers on the symbol based on the indicated comb offset:

Agreement The following agreement replaces the earlier agreement from RAN1#119 If transform precoding is enabled for a PUSCH, the following options are provided for the purpose of specification drafting (how UE implementation is done is left to each company). […]

# References

- Chairman’s notes, RAN1#118, August 2024.

- Chairman’s notes, RAN1#119, November 2024.

- Chairman’s notes, RAN1#120, February 2025.