3GPP TSG-RAN WG1 Meeting #121	Tdoc R1-2504455

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	9.7.2

Source:	Ericsson

Title:	Discussion on ISAC Channel Modelling

Document for:	Discussion, Decision

# Introduction

The objective of the study on channel modelling for Integrated Sensing And Communication (ISAC) for NR [1] includes the following:

The focus of the study is to define channel modelling aspects to support object detection and/or tracking (as per the SA1 meaning in TS 22.137). The study should aim at a common modelling framework capable of detecting and/or tracking the following example objects and to enable them to be distinguished from unintended objects:

- UAVs

- Humans indoors and outdoors

- Automotive vehicles (at least outdoors)

- Automated guided vehicles (e.g. in indoor factories)

- Objects creating hazards on roads/railways, with a minimum size dependent on frequency

All six sensing modes should be considered (i.e. TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, UE monostatic).

For the above use cases, sensing modes and frequencies:

- Identify details of the deployment scenarios corresponding to the above use cases.

The contribution addresses several themes, each in a different section. The themes describe target modelling, spatial consistency, background channel, environment objects, and link-level channel model.

# Target Modelling

  - Shadowing

Shadows behind targets is a very important aspect for sensing. In some sensing scenarios, these shadows are significant, and their existence may be a strong indicator of target presence. Examples are intruder detection or vehicular sensing of other vehicles and obstacles. In Figure 1, a human is in the line of sight between Tx and Rx1 and between Tx and Rx2. Rx1 receives very low power and by monitoring the received power the intruder can be detected. The target’s impact on the overall received power at Rx2 is much smaller. Therefore, in some cases, these shadows are insignificant and can be disregarded. Another example is a small UAV at a large distance.

Figure 1	Shadows behind a human

There is a shadow behind any sufficiently large target, i.e. less RF energy will reach a receiver behind the target compared to if the target had not been there. The amount of RF energy that reaches the receiver is proportional to HISAC2=Hbackground+Htarget2, so obviously the combined target and background channel needs to have lower energy than the background channel in the absence of any target.

In some sensing scenarios, the shadows are significant, and their existence may be a strong indicator of target presence. Examples are intruder detection or vehicular sensing of other vehicles and obstacles.

Shadow behind a sufficiently large target means the combined target and background channel needs to have lower energy than the background channel in the absence of any target.

Shadow is a distance-dependent phenomenon. It was agreed that sensing targets are in the far field of sensing Tx/Rx. But it doesn’t mean that sensing Rx is in the far field of sensing target. Instead, sensing Rx is likely to locate in the near field of a large target, where shadow effect needs to be taken into account. For instance, a car has a size of several meters, typically 5 m. As is known, the far field boundary can be considered to occur at the Fraunhofer distance dF=2D2λ. For a car of size D=5 and a frequency of 6 GHz, the far field from the car occurs at a distance of 1 km. A sensing Rx with its distance to the vehicle smaller than 1km is in the car’s near field. So clearly, all sensing scenarios involving cars are likely to occur at distances corresponding to the near field of the car.

Agreement - In the target channel between Tx and Rx, scattering of a sensing target can be modelled as single scattering point or multiple scattering points - FFS one or multiple incoming/output rays corresponding to a scattering point - FFS how to select single or multiple scattering points for the target, e.g. depending on the distance between target and Tx/Rx, size/shape of target, etc. - Note: the sensing target can be assumed in far field of sensing Tx/Rx. - FFS details to model the single or multiple scattering points

It was agreed that sensing target is in the far field of sensing Tx/Rx. However, sensing Rx is likely to locate in the near field of a large target, where shadow effect needs to be taken into account.

For a car of size D=5 and a frequency of 6 GHz, the far field from the car occurs at a distance of 1 km. A sensing Rx with its distance to the vehicle smaller than 1km is in the car’s near field.

How to derive bi-static RCS based on mono-static RCS was agreed as a working assumption in RAN1#120bis, meanwhile angle-dependent mono-static RCS models reuse BS radiation pattern with different parameters for different target types. The RCS modelling follows the principle that it is distance independent. Therefore, shadows behind targets are not modelled by RCS and have to be separately addressed.

Working assumption For vehicle with single/multiple scattering points, the bistatic RCS is generated by - The values/pattern of A*B1 of bistatic RCS is given by: rcsdBθi,ϕi,θs,ϕs=maxGmax-min-σVdBθ+σHdB φ,σmax-Attenuatefactor,RCSdB,fsθi,ϕi,θs,ϕs where σVdBθ=-min12θ-θcenterθ3dB2, σmax σHdB φ=-min12φ-φcenterφ3dB2, σmax - Attenuatefactor=k1×sin(k2×β2) is applied to the β within 0~180 degrees. k1= 6 and k2=1.65. β is the bistatic angle between the incident ray and scattering ray within the plane of incident direction (θi,ϕi,) and scattering direction (θs,ϕs). - The angles of (θ,φ) are the projections of the bisector angle on the vertical plane and the horizontal plane, respectively. - FFS: RCS value when β is 180 degrees - The effect of forward scattering RCSdB,fsθi,ϕi,θs,ϕs is -Inf in Rel-19 - 5 sets of parameters φcenter, φ3dB, n, θcenter, θ3dB,n, Gmax, σmax, Applicable Range of θ and Applicable Range of φ are applicable as defined for the monostatic RCS of vehicle with single/multiple SPSTs - Continue study on a new formula for Attenuatefactor to resolve the issue of angular discontinuity. - The new formula should retain following property: the linear bistatic RCS for a vehicle with single scattering point is the sum of the bistatic RCS of the multiple scattering points of the vehicle - the following formula can be a reference for the study Attenuatefactor=k1sink2β2(1-k3β-k4*(Gmax-min-σVdBθ+σHdB φ,σmax)) Note: the working assumption agreed on Thursday was updated on Friday as follows: k1= 6 and k2=1.65 [H][FL1] Proposal 6.7-1 blockage model The existing blockage model B is reused as an optional feature to model the blocking effect due to a target - Applicable to the LOS/NLOS rays in the background channel of the target - Applicable to the LOS/NLOS rays in the Tx-target and target-Rx link of the other target

A method of modelling the shadows is to apply the existing blockage model to background channel where a target casts a shadow for sensing Rx, so that Hbackground is reduced in the shadow region.

A method of modelling the shadows is to apply the existing blockage model to background channel, where a target casts a shadow for sensing Rx, so that Hbackground is reduced in the shadow region.

Interaction between targets is also critical, when one target’s shadows have significant impacts on another target, i.e., when incident or scattered rays of the second target are severely attenuated. Figure 2 shows a sensing scenario where a pedestrian needs to be sensed, but the pedestrian is affected by the shadow of a passing car and hence the Tx-target channel becomes very weak. In other words, the car blocks the LOS ray and some of the NLOS rays of Tx-human link. With blockage option, the car is modelled as a blocker in the pedestrian’s target channel and affects the LOS and NLOS rays between Tx and the pedestrian. The blocker may appear in target-Rx link as well and casts a shadow on Rx. This may happen to both mono-static and bi-static sensing modes and more serious for mono-static sensing mode because of double blockage in Tx-target and target-Rx links. In short, the blockage of one target can be considered in the Tx-target and/or target-Rx links of another target channel depending on sensing mode.

Figure 2	Shadow created by a vehicle affects a human’s Tx-target and/or target-Rx link for both mono-static and bi-static sensing modes

Interaction between targets is critical, when one target’s shadows have significant impacts on another target, i.e., when incident or scattered rays of the second target are severely attenuated.

The blockage of one target can be considered in the Tx-target and/or target-Rx links of another target channel depending on sensing mode.

We propose to use blockage model B in 38.901 to account for shadows. There are two existing blockage models in 38.901. Blockage model A is a blocker that only block/attenuate an interval of angles not determined by size of the blocking object, i.e, the distances between the blocker and the Tx/Rx must be predefined. Blockage model B is a geometric model, where attenuation is determined based on the physical sizes of blockers and the geometry of blocker, Tx, and Rx. Similarly, the significance of shadows depends on the size of the blocker, which can be a sensing target, and its distance to the affected node, which can be sensing Rx or another target. Therefore, Blockage model B is preferred.

Support blockage model B in 39.901 to model shadows, including

- modelling blockage in the background channel of the target

- modelling interactions between the targets when needed by introducing blockage in the Tx-target and target-Rx link of another target

  - Radar Cross-Section

### Object types

Based on the following agreements, unintended objects are called EO Type-1 and have comparable physical characteristics as sensing targets and modelled in the same way as targets.

Agreement EO is a non-target object with known location. - FFS other known parameters of the EO - FFS details on EO modeling The following options for EO modeling are considered for further study - Option 2: EO is modeled same/similar as a sensing target - Applicable for an EO having comparable physical characteristics as a sensing target, (referred as EO type-1 for discussion purpose) - FFS Applicable for EO type-2 - FFS EO modeling impacts the target channel and/or the background channel Agreement - EO type-1 (when modelled) is modelled in the same way as a sensing target in the ISAC channel model.

There is no discussion on types of unintended targets and their modelling, including RCS, CPM, and velocity. But some birds do have similar sizes, flying heights, and speed as UAVs. Algorithms for UAV detection based on 3GPP ISAC channel model will suffer from false detection of birds. Moreover, ground clutter, e.g. trees moving in the wind, will challenge sensing algorithms in distinguishing an intended target from the unintended targets in frequency domain due to their similar Doppler shifts. SID states unintended objects and clutter/scattering patterns [1]. Without Type-1 EO modelling, the following objectives in SID are not fulfilled and the study is incomplete. Note there is no discussion on vscatt and p of moving stochastic clusters, and therefore they cannot serve as unintended objects or generate comparable physical characteristics as sensing targets.

The study should aim at a common modelling framework capable of detecting and/or tracking the following example objects and to enable them to be distinguished from unintended objects: - modelling of sensing targets and background environment, including, for example (if needed by the above use cases), radar cross-section (RCS), mobility and clutter/scattering patterns;

There is no discussion on types of unintended targets and their modelling, including RCS, CPM, and velocity.

Trees moving in the wind will challenge sensing algorithms in distinguishing an intended target from the unintended targets in frequency domain due to their similar Doppler shifts.

Without modelling of unintended objects, the SID’s objective of distinguishing targets from unintended objects and clutter/scattering patterns cannot be fulfilled, and the study is incomplete.

There is no discussion on vscatt and p of moving stochastic clusters, and therefore they cannot serve as unintended objects or generate comparable physical characteristics as sensing targets.

A measurement at 28 GHz has been performed to qualitatively assess the existence of scattering from trees. The measurement setup is shown in Figure 3. The Tx used a 13 dBi horn antenna pointing at a group of trees, while the Rx used a very directional 38 dBi antenna that could be scanned in different azimuth and elevation angles, thereby resolving multipath directions. One example is shown in Figure 4. Here it is clearly seen that there are many multipath coming from the direction of the trees. This gives insight into the needs of using multiple scattering points or multiple trees in the ISAC channel model.

Figure 3	Measurement setup for measuring bistatic scattering from several trees at 28 GHz.

Figure 4	Angular scan of multipath at the Rx superimposed of a panoramic photo

Another recent measurement by KAIST and Samsung at 28 GHz [6] also points to the importance of tree scattering even for communication scenarios: “It was found that tree clutters are the strongest cluster and the walls of buildings are the second valid cluster”.

Measurements show that there are many multipath coming from the direction of the trees and trees are strong clusters.

Study modelling of birds and trees.

Birds are a type of objects that affects sensing of UAVs in particular. The RCS of birds is a well-studied field, see for instance [5]. The summary of [5] contains the following sentence: “Single birds typically have mean σ from 100 to 102 cm2, while migrant flocks, which often occur during the day, can have mean σ greater than 104 cm2 “. Translated to dBsm units, this would be equivalent of saying that single birds have an A value in the range from -40 to -20 dBsm while a migrant flock can have an A value greater than 0 dBsm.

Considering the small sizes of birds we propose to model them as isotropic scatterers.

For the bistatic RCS of a bird, A is uniformly distributed between -40 to -20 dBsm, B=0dB, standard deviation of B2=3dB.

The bistatic RCS of a tree at 1.9 GHz was measured in [2], see Figure 5 and Figure 6. The results show that the mean RCS of a tree is typically in the 10 dBm2 range but it increases to more than 20 dBm2 in the forward scattering direction. This can be used to quantify A*B1 for a tree. The bars in Figure 6 show the difference between the 10th and the 90th percentile of the RCS, which quantifies the rapid variations of the RCS, i.e. B2. This can be represented by the following RCS model.

- The values/pattern of A*B1 of the bistatic RCS is given by:

rcsdBθi,ϕi,θs,ϕs=Gmax-min12β-πβ3dB2, σmax

  - β is the bistatic angle between the incident ray and scattering ray within the plane of incident direction (θi,ϕi,) and scattering direction (θs,ϕs).

  - β3dB is 20 degree

  - Gmax is 23 dBsm

  - σmax is 13 dBsm

- Component B2: standard deviation of B2 is 3 dB

Figure 5	Tree for bistatic RCS measurements, from [2].

Figure 6	Measured and theoretical bistatic RCS of a tree, from [2].

Use the following bistatic RCS model of a single scattering point for a tree: A*B1 with a constant level of 10 dBm2 and a single lobe in the forward direction with A*B1 increasing to 23 dBm2. The B2 standard deviation is approximately 3 dB.

For tree RCS model with two scattering points, consider a (larger) fraction of the RCS to correspond to a fixed scattering point representing static parts of the tree such as the trunk, and another (smaller) fraction of the RCS to correspond to moving scattering points representing leaves and branches.

Truck/bus, child and animal were agreed as sensing targets with the following sizes in AI 9.7.1, while their RCS models have not been discussed yet. Two sizes of AGV were agreed with only one RCS model. To fix the inconsistency, we suggest studying RCS of these targets.

Type 3 (truck/bus): 13m x 2.6m x 3m

Child: 0.2m x 0.3m x 1m

AGV Option 1: 0.5m x 1.0m x 0.5m

AGV Option 2: 1.5 m x 3.0m x 1.5 m

Animal: 1.5m x 0.5m x 1 m

Study RCS of truck/bus, child, and animal based on agreed sizes in AI 9.7.1 and RCS of AGV of both sizes

Doppler processing is a very fundamental aspect of clutter suppression in sensing and radar, hence including non-zero Doppler of certain false targets is a crucial component in an ISAC channel model. Sensing algorithms need to be able to distinguish UAVs from birds, for instance using micro-Doppler as discussed in [4]. The Doppler of trees moving in the wind has been studied in other areas of research, for instance in [3].

Since micro-Doppler is essential for distinguishing wanted and unwanted targets, methods to model micro-Doppler are needed for the completion of the Study Item.

To model the Doppler signature (micro-Doppler) of various targets and Type-1 EOs, the target is represented by one (or more) fixed scattering points and one (or more) scattering points that are moving in relation to the fixed scattering point

- For a UAV, the moving scattering point moves in a horizontal circle with radius r and rotation speed v_rot around the fixed scattering point. The moving scattering point has a value A that is 20-40 dB lower than the value A for the fixed scattering point, B is isotropic, and B2 has standard deviation 3 dB

  - Small UAV: r = 10 cm, v_rot = 40 m/s

  - Large UAV: r = 40 cm, v_rot = 10 m/s

- For a flapping bird, the moving scattering point moves in a vertical sinusoidal motion with frequency 2-6 Hz and speed v = 2-12 m/s. The moving scattering point has a value A that is 0-10 dB lower than the value A for the fixed scattering point, B is isotropic, and B2 has standard deviation 3 dB

- For a gliding bird, the scattering point becomes stationary

- For a human walking a speed v, the torso is represented by a moving rather than fixed scattering point, where the horizontal speed varies sinusoidally between v-v’ and v+v’. The arms and legs are represented by a second scattering point that moves horizontally in the direction of travel in a sinusoidal manner with speed between 0 and 2*v. The arms/legs scattering point has a value A that is 10 dB lower than the value A for the torso scattering point, B is isotropic, and B2 has standard deviation 3 dB

- FFS: other targets and Type-1 EOs

### Bi-static RCS model

One issue of the bi-static RCS model in the working assumption is angular discontinuity. To illustrate the consequences of a discontinuity, we have computed the target channel for a case of a vehicle driving in a straight line in between the transmitter and receiver, see Figure 7 for the setup.

Working assumption For vehicle with single/multiple scattering points, the bistatic RCS is generated by Option D: - The values/pattern of A*B1 of bistatic RCS is given by: rcsdBθi,ϕi,θs,ϕs=maxGmax-min-σVdBθ+σHdB φ,σmax-Attenuatefactor,RCSdB,fsθi,ϕi,θs,ϕs where σVdBθ=-min12θ-θcenterθ3dB2, σmax σHdB φ=-min12φ-φcenterφ3dB2, σmax - Attenuatefactor=k1×sin(k2×β2) is applied to the β within 0~180 degrees. k1= 6 and k2=1.65. β is the absolute bistatic angle between the incident ray and scattering ray within the plane of incident direction (θi,ϕi,) and scattering direction (θs,ϕs). - The angles of (θ,φ) are the projections of the bisector angle on the vertical plane and the horizontal plane, respectively. - FFS: RCS value when β is 180 degrees - The effect of forward scattering RCSdB,fsθi,ϕi,θs,ϕs is -Inf in Rel-19 - 5 sets of parameters φcenter, φ3dB, n, θcenter, θ3dB,n, Gmax, σmax, Applicable Range of θ and Applicable Range of φ are applicable as defined for the monostatic RCS of vehicle with single/multiple SPSTs - FFS: how to avoid angular discontinuity - Continue study on a new formula for Attenuatefactor to resolve the issue of angular discontinuity. - The new formula should retain following property: the linear bistatic RCS for a vehicle with single scattering point is the sum of the bistatic RCS of the multiple scattering points of the vehicle - the following formula can be a reference for the study Attenuatefactor=k1sink2β2(1-k3β-k4*(Gmax-min-σVdBθ+σHdB φ,σmax)) Note: the working assumption agreed on Thursday was updated on Friday as follows: k1= 6 and k2=1.65

The radar cross-section of the direct path during the simulation is shown in Figure 8. B2 was not considered in the simulation. In the zoom-in a discontinuity is seen at the time instant, when the vehicle crosses the line-of-sight between the transmitter and receiver. The reason for this discontinuity is fact that before this point the bisector is pointing to the front of the vehicle and after this point the bisector points to the back of the vehicle. Right at the point where the vehicle crosses the line-of-sight, the bisector is not defined as illustrated in Figure 9, which corresponds to the FFS: RCS value when β is 180 degrees. To still obtain a RCS of β = 180 degrees, the bisector was chosen at this point such that its zenith angle was 180 degrees and its azimuth 0 degrees, which makes the bisector orthogonal to the incidence direction.

Figure 7	Geometry of simulation setup.  The object moves along the blue trajectory from the point at (25, -150) to the point at (25, 150) at 10 m/s.

Figure 8	RCS during simulation.  Left: Whole duration of simulation.  Right: Zoom-in around t=15s when the vehicle is passing the line-of-sight between the transmitter and receiver.

Figure 9	The bisector is not defined when the bistatic angle is 180 degrees.

As the vehicle approaches the line-of-sight line in between the transmitter and receiver, the path length shrinks, and the Doppler shift of the channel is expected to be positive. When the vehicle has passed the line-of-sight and as it distances itself from the transmitter and receiver, the path length increases, and the Doppler shift of the channel is expected to be negative.

The Doppler spectrogram of the channel is shown in Figure 10. It can be seen that the Doppler shift follows the expected shape – positive the first half of the simulation and negative the second half. The effect of the discontinuity of the RCS is also seen at time t=15 s in the zoomed in Doppler spectrogram as a vertical purple line that covers all Doppler frequencies. The fact that the artifact is not bandlimited is a typical artifact of a discontinuity. A Hamming window was used to compute the spectrogram in order to reduce the sidelobes.  The strongest side lobes are still visible in the figure though and explain the dark purple curves that go in parallel to the main curve. The channel was sampled at 1ms periodicity and a carrier frequency of 3GHz was assumed.

Figure 10	Doppler spectrogram of target channel.

Discontinuities in the RCS cause signals that are not bandlimited and non-physical artifacts in the Doppler spectrum.

If angular discontinuity issue in bi-static RCS model is not solved, a note should be added to remind users of the possible unintended consequence of band-unlimited Doppler artifacts.

If angular discontinuity issue in bi-static RCS model is not solved, add a note: angular discontinuity of bi-static RCS happens when bi-sector changes direction and the incurred band-unlimited Doppler artifacts may lead to unintended sensing performance loss or other unexpected simulation behavior.

### Rapid angular fading and angular correlation of B2

There are diverse views regarding angular correlation of B2. The first two options below do not consider angular correlation, while the latter three do by different means. The drawback with option 1 is captured in the first sub-bullet. Both movement and rotation of the target may result in a large change of incident/scattered angles. To illustrate the consequence of no angular correlation of B2, we have made the following simulations for Option 1, Option 2 and Option 5.

Some observations on solutions and pros/cons of each solution from early discussions are as follows - Option 1: Same value of B2 always applies to a path per simulation - Option 1 is not accurate if the movement of Tx/target/Rx results in large change of incident/scattered angles - Option 1 doesn’t mean no change of RCS value since A*B1 is still angular dependent even if B2 is unchanged in mobility. - Option 2: Same value of B2 applies to a path before the value of B2 is updated. The periodicity to update B2 can be discussed in evaluation phase or up to company choice - The large/small scale parameters in existing 38.901, once generated, are used for a while (multiple frames/slots) before update. Option 2 keeps the similar principle. - Option 2 doesn’t mean no change of RCS value since A*B1 is still angular dependent even if B2 is unchanged in mobility. - Option 3: angular correlation of B2 modelled by the existing correlation distance based spatial consistency defined in TR 38.901 - In reality, the correlation of B2 is related to the change of angle, not the distance of target - Option 4: angular correlation of B2 modelled by the ‘correlation angle’. - This option may be more accurate than Option 3, but it needs a completely new parameter - No validation results available on ‘correlation angle’ - Option 5: The proposal from Ericsson: Model the stochastic B2 with C random scattering centres as B2dinc, dsca=c=1Cβexp⁡(-j2πdinc+dsca,pc/λ)2 - It is questionable whether Option 5 still follows log-normal distribution - Need details on how to model the C scattering centres

We used the same set up as in Figure 7. To highlight the effect of B2, A*B1 part of the radar cross-section of the target is modelled as isotropic. Upper bound of B2 is considered. For Option 2, the periodicity to update B2 is 1 ms. In Option 5, C=5 scattering centers are randomly selected from a grid with a uniform spacing of 0.7 wavelengths and their locations remain the same in a simulation.

Equation and parameters of Option 5 are as follows. More details can be found in [7] and [8].

B2dinc, dsca=c=1Cβexp⁡(-j2πdinc+dsca,pc/λ)2,

where dinc and dsca are vectors of incident and scattering directions, the β is a power coefficient, and the 3D position vectors pc are, e.g, randomly selected with a sufficiently large minimum spacing. x,y denotes the inner product. Note that the scattering centres that these position vectors model only exist as part of the B2 modelling – the target is still modelled as a single scattering point.

The value of B2 over time along the trajectory of the object is shown in Figure 11. For Option 5, as expected, the variations in B2 are slow when the object is far away from the transmitter and receiver and get rapid when the object is close to the transmitter and receiver in the region where the incidence and scattering directions change fast with time. This behaviour is seen in the scatter centre model of Option 5, but not in Options 1 and 2.

Figure 11	B2 values of the target along the trajectory.  Top: Option 1.  Middle: Option 2, B2 is modelled using samples from a lognormal distribution without correlation.  Bottom: Option 5, B2 is modelled using the scatter centre model.

When plotting the Doppler spectrogram of the target channel for these B2 models, the plots in Figure 12 are obtained. We can observe how the Doppler spectrogram varies. We expect to see trenches where deep fades occur in the channel and a continuously varying intensity over time. This is seen in the Doppler spectrogram corresponding to the scatter centre model Option 5, but not in that of Option 1 or 2. Instead, Option 1 and 2 showcase a steady intensity without deep fades or strong peaks. Furthermore, the Doppler spectrogram of Option 2 shows strong Doppler artifacts outside the main peak of the Doppler frequency of the target.

Figure 12	Doppler spectrogram of target channel.  Top: Option 1, middle: Option 2, bottom: Option 5

The SID objectives list that the channel model should support “detection and/or tracking” with equal emphasis on these two properties. To track a target means to follow it while it is moving or rotating. As a target moves or rotates with a change of incident and scattering angles, it is important to ensure that radar cross-section varies continuously over time. Multiple observations of a target over time may help to provide more robust estimates of the type and position of the target since the rapid RCS variations can be averaged. With Option 1 above there is no such averaging since the B2 values are constant. With Option 2 there is instead a risk of negatively affecting the accuracy of detection due to the introduction of Doppler artifacts. Having said this, the finding that Doppler artifacts only occur in the target channel but not in the background channel may be used by a sensing algorithm to artificially inflate the target detection probability and reduce the false alarm rate. Hence it is important to not introduce such artifacts in the channel model.

With Option 1, there are no rapid random variations of RCS, which are observed in measured channels. It obviates the need of combining observations over time to ensure a good probability of detection, which is necessary in reality.

With Option 2, discontinuous RCS leads to non-physical artifacts in the channel, including very high Doppler frequencies and spurious out-of-band artifacts, which may negatively affect the accuracy of detection.

On the other hand, the finding that Doppler artifacts only occur in the target channel but not in the background channel may be used by a sensing algorithm to artificially inflate the target detection probability and reduce the false alarm rate. Hence, it is very important to not introduce such artifacts in the channel model.

Support angular correlation of B2 and model the stochastic B2 with C random scattering centres as B2dinc, dsca=c=1Cβexp⁡(-j2πdinc+dsca,pc/λ)2.

The offset pc can be taken from a uniform grid with spacing 0.7 wavelengths and the number of scattering centres C can be chosen as 5.  Other methods to select these parameters that result in equivalent properties exist.

### RCS component A to calculate power scaling factor

Mono-static RCS models and bi-static RCS models are separately discussed in RAN1. Value of RCS component A was given for isotropic mono-static RCS of small UAV and human model 1. With the working assumption of bi-static RCS model, mono-static RCS is a subset of bi-static RCS. Regarding value A of the whole RCS, it was proposed that RCS component A is the mean value of mono-static RCS. In our view, bi-static RCS is far more useful than mono-static RCS.

With 1-degree angular granularity, there are (360*180)^2 values of bi-static RCS, among which only 360*180 values are mono-static RCS, where incident azimuth and zenith angles are the same as scattering angles. Therefore, mono-static RCS is a very small subset of bi-static RCS.

For mono-static sensing mode and NLOS condition, due to the lack of direct path, none of mono-static RCS is used for target channel. With the existing channel model, NLOS clusters’ power are randomly generated with cluster shadowing, and the NLOS cluster with shortest delay is not necessary to be the strongest. For mono-static sensing, it is inappropriate to assume that the strongest indirect paths are generated in NLOS clusters with sufficiently small delay and their power can be represented with mono-static RCS of the absent direct path. For mono-static sensing mode, in the case of LOS condition, with 1:1 random ray pairing, one mono-static RCS value is used to generate the direct path, while nearly 1200 bi-static RCS values are used to generate the indirect paths. Thus bi-static RCS is more widely used than mono-static RCS.

Mono-static RCS component of A*B1 is in a large value range for angle-dependent RCS. The maximum mono-static RCS value and the maximum value can be calculated with Gmax and σmax. The value range for vehicle is [7.06, 20.75] dBsm. Such a large value range indicates that the mean value doesn’t reflect RCS of the direct path with a particular incident angle. Therefore, from the aspects of both direct paths and indirect paths, RCS component A as mean of mono-static RCS may not reflect the actual path loss in the target channel.

With bi-static RCS WA, mono-static RCS is a very small subset of bi-static RCS, with the ratio of 1 : 360*180, with 1-degree angular granularity.

Bi-static RCS values are more widely used than mono-static RCS in target channel.

Mono-static RCS component of A*B1 is in a large value range for angle-dependent RCS. Mean of mono-static RCS doesn’t reflect RCS of the direct path with a particular incident angle.

If RCS Component A is the mean value of mono-static RCS, it should be noted that such RCS component A may not reflect the actual path loss in the target channel.

  - Impact of Object’s Orientation on CPM

The matrix CPMsp should characterize how waves of different polarizations incident on the target are scattered into the same or other polarizations by the target. When objects move and rotate, their scattering properties stay the same in the local coordinate system of the object, similar to the field patterns of BS and UE. This is why scattering (RCS) and cross-polarization (CPM) is modelled in the coordinate system of the object (not in the global coordinate system). Therefore, the incidence and scattering directions of the propagation path have to be converted to the object coordinate system when evaluating the RCS and cross-polarization matrix (CPM) for a propagation path. Transformation between LCS and GCS is necessary to make sure that Tx, target, and Rx can relate one spherical unit vector in GCS with the same definition of θ and φ.

One open issue is whether CPMsp,i is modelled in the Global Coordinate System (GCS) or a Local Coordinate System (LCS). If CPMsp,i is defined in a Global Coordinate System (GCS), it does not allow arbitrary orientation of the target---the axes of the LCS of the target should be parallel to the axes of the GCS. With the following definition of LCS from the draft CR, vehicle, large UAV, AGV, and human can only move in the direction of positive x-axis. But if CPMsp,i is defined in a LCS, the channel model can support rotation of targets by reusing the legacy transformation procedure in 38.901, which is used to support arbitrary orientation of BS and UE antenna panel and Type-2 EO as well.

The orientation of a sensing target in LCS is provided as follows. - The front of a vehicle, a UAV with large size or an AGV has azimuth angle ϕ=00 and zenith angle θ=900 in LCS. - For a human, it faces the direction of azimuth angle ϕ=00 and zenith angle θ=900 in LCS.

When objects move and rotate, their scattering properties stay the same in the local coordinate system of the object, similar to the field patterns of BS and UE. This is why scattering (RCS) and cross-polarisation (CPM) is modelled in the coordinate system of the object.

Transformation between LCS and GCS is necessary to make sure that Tx, target, and Rx can relate one spherical unit vector in GCS with the same definition of θ and φ.

If CPMsp,i is defined in the Global Coordinate System (GCS), rotation of the target will not be supported by the ISAC channel model. Vehicle, large UAV, AGV, and human can only move in the direction of the positive x-axis.

If CPMsp,i is defined in a Local Coordinate System (LCS), the channel model can support rotation of the targets by reusing the legacy transformation procedure in 38.901, which is used to support arbitrary orientation of BS and UE antenna panel and Type-2 EO as well.

We discuss in the following how to use the legacy transformation procedure for CPM and the reasons why the procedure can be skipped if the rotation of the target is in x-y plane only and why it cannot be skipped for other rotations.

We first review the BS and UE rotation procedure, before digging into the matrices needed to support a rotation of the target. The existing communication channel in 38.901 supports arbitrary orientations of the BS and UE antenna arrays in their LCS. Though field patterns of transmitter and receiver used in the equations of the fast-fading model in section 7.5 are given in GCS, section 7.1.3 provides the transformation from LCS and GCS. As a special case, section 7.1.4 discusses downtilted BS antenna with rotation along y axis only.

Existing communication channel in 38.901 supports arbitrary orientation of BS and UE antenna array in their LCS. If the orientation of the LCS is different from the GCS orientation, a transformation between LCS and GCS is needed.

## 38.901 ## 7.5 Fast fading model Step 1: Set environment, network layout, and antenna array parameters c) Give 3D locations of BS and UT, and determine LOS AOD (ϕLOS,AOD), LOS ZOD (θLOS,ZOD), LOS AOA (ϕLOS,AOA), and LOS ZOA (θLOS,ZOA) of each BS and UT in the global coordinate system d) Give BS and UT antenna field patterns Frx and Ftx in the global coordinate system and array geometries e) Give BS and UT array orientations with respect to the global coordinate system. BS array orientation is defined by three angles ΩBS,α (BS bearing angle), ΩBS,β (BS downtilt angle) and ΩBS,γ (BS slant angle). UT array orientation is defined by three angles ΩUT,α (UT bearing angle), ΩUT,β (UT downtilt angle) and ΩUT,γ (UT slant angle). f) Give speed and direction of motion of UT in the global coordinate system Step 11: Generate channel coefficients for each cluster n and each receiver and transmitter element pair u, s. (7.5-22) where Frx,u,θ and Frx,u,ϕ are the field patterns of receive antenna element u according to (7.1-11) and in the direction of the spherical basis vectors, and respectively, Ftx,s,θ and Ftx,s,ϕ are the field patterns of transmit antenna element s in the direction of the spherical basis vectors, and respectively. Note that the patterns are given in the GCS and therefore include transformations with respect to antenna orientation as described in Clause 7.1. ### 7.1.2 Local and global coordinate systems An array antenna for a BS or a UT can be defined in a Local Coordinate System (LCS). An LCS is used as a reference to define the vector far-field that is pattern and polarization, of each antenna element in an array. It is assumed that the far-field is known in the LCS by formulae. The placement of an array within the GCS is defined by the translation between the GCS and a LCS. The orientation of the array with respect to the GCS is defined in general by a sequence of rotations (described in clause 7.1.3). Since this orientation is in general different from the GCS orientation, it is necessary to map the vector fields of the array elements from the LCS to the GCS. This mapping depends only on the orientation of the array and is given by the equations in clause 7.1.3. Note that any arbitrary mechanical orientation of the array can be achieved by rotating the LCS with respect to the GCS. draft CR [12] Section 7.9.4.1 Step 14: Generate channel coefficients for paths in set R for each receiver and transmitter element pair u, s. The channel coefficient for a path (k,p,n',m',n,m) in set R is generated by Hu,s,n',m',n,mk,pt=Pn',m',n,mk,pFrx,u,θθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pFrx,u,ϕθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pTCPMrx,n',m'k,pCPMn',m',n,mk,pCPMtx,n, mk,pdθθ2+dϕϕ22 ∙Ftx,s,θθtx,n,m,ZODk,p,ϕtx,n,m,AODk,pFtx,s,ϕθtx,n,m,ZODk,p,ϕtx,n,m,AODk,pexpj2πrrx,k,p,n',m'Tt.drx,u+rtx,k,p,n,mTt.dtx,sλ0expj2πt0tfD,n',m',n,mk,ptdt (7.9-xx)

The procedure of transformation from LCS to GCS in 38.901 is copied below. Here +cosψ-sinψ+sinψ+cosψ in Equation (7.1-11) is generated based on α, β and γ, the rotation angles used to convert a point from the GCS to the LCS.

7.1.3 Transformation from a LCS to a GCS In order to establish the equations for transformation of the coordinate system and the polarized antenna field patterns between the GCS and the LCS, it is necessary to determine the composite rotation matrix that describes the transformation of point (x, y, z) in the GCS into point (x', y', z') in the LCS. This rotation matrix is computed as the product of three elemental rotation matrices. The matrix to describe rotations about the z,and axes by the angles a, b and g respectively and in that order is defined as [omitted] Let us denote the polarized field components in the LCS by , and in the GCS by , . Then they are related by equation (7.1-11). Fθθ,φFφθ,φ=+cosψ-sinψ+sinψ+cosψFθ''θ',φ'Fφ''θ',φ' (7.1-11) It can be shown thatandcan be expressed as: (7.1-16) (7.1-17)

The legacy procedure can be reused to model a rotation of the target, including the same definition of α, β and γ as in 38.901, Equation (7.1-11) used for the transformation from LCS to GCS, and +cosψ+sinψ-sinψ+cosψ used for the transformation from GCS to LCS. The procedure is to transform Tx-target link from GCS to LCS before it impinges on the target and then transform the intermediate result back to GCS before it impinges on the Rx, with the following steps.

Step 1: transform Tx-target link from GCS to LCS before it impinges on the target,

Step 2: apply the effect of the target, CPM of target,

Step 3: transform the intermediate result back to GCS,

Step 4: apply the effect of the target-Rx link.

Based on equation (7.9-xx), in step 1, CPMtx,n, mk,pFtx,s,θθtx,n,m,ZODk,p,ϕtx,n,m,AODk,pFtx,s,ϕθtx,n,m,ZODk,p,ϕtx,n,m,AODk,p is pre-multiplied by +cosψ+sinψ-sinψ+cosψ, leading to +cosψ+sinψ-sinψ+cosψCPMtx,n, mk,pFtx,s,θθtx,n,m,ZODk,p,ϕtx,n,m,AODk,pFtx,s,ϕθtx,n,m,ZODk,p,ϕtx,n,m,AODk,p.

After step 2, we get CPMn',m',n,mk,p+cosψ+sinψ-sinψ+cosψCPMtx,n, mk,pFtx,s,θθtx,n,m,ZODk,p,ϕtx,n,m,AODk,pFtx,s,ϕθtx,n,m,ZODk,p,ϕtx,n,m,AODk,p.

In step 3, the outcome of step 2 is pre-multiplied by  +cosψ-sinψ+sinψ+cosψ, generating +cosψ-sinψ+sinψ+cosψ CPMn',m',n,mk,p+cosψ+sinψ-sinψ+cosψCPMtx,n, mk,pFtx,s,θθtx,n,m,ZODk,p,ϕtx,n,m,AODk,pFtx,s,ϕθtx,n,m,ZODk,p,ϕtx,n,m,AODk,p.

In step 4, Frx,u,θθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pFrx,u,ϕθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pTCPMrx,n',m'k,p is applied. The final result is Frx,u,θθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pFrx,u,ϕθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pTCPMrx,n',m'k,p+cosψ-sinψ+sinψ+cosψCPMn',m',n,mk,p+cosψ+sinψ-sinψ+cosψCPMtx,n, mk,pFtx,s,θθtx,n,m,ZODk,p,ϕtx,n,m,AODk,pFtx,s,ϕθtx,n,m,ZODk,p,ϕtx,n,m,AODk,p.

If the CPM of the target is defined in the LCS, a rotation of the target can be supported by replacing CPMrx,n',m'k,pCPMn',m',n,mk,pCPMtx,n, mk,p with CPMrx,n',m'k,p+cosψ-sinψ+sinψ+cosψCPMn',m',n,mk,p+cosψ+sinψ-sinψ+cosψCPMtx,n, mk,p. Namely, CPMn',m',n,mk,p is replaced with +cosψ-sinψ+sinψ+cosψCPMn',m',n,mk,p+cosψ+sinψ-sinψ+cosψ.

If the CPM of the target is defined in the LCS, a rotation of the target can be supported by replacing CPMn',m',n,mk,p with +cosψ-sinψ+sinψ+cosψCPMn',m',n,mk,p+cosψ+sinψ-sinψ+cosψ.

Some simplification of the rotation procedure can be made depending on the target mobility. Let us first consider the case where the target moves in the x-y plane with a rotation of α around the z axis, such as turning left or right. In this case, α≠0, β=0, and γ=0. Both +cosψ-sinψ+sinψ+cosψ and +cosψ+sinψ-sinψ+cosψ are identity matrices, which means that the transformation procedure can be skipped.

If the target’s movement involves rotation around the x axis or the y axis, such as a vehicle driving on/off a slope, γ or β are non-zero. Then +cosψ-sinψ+sinψ+cosψ and +cosψ+sinψ-sinψ+cosψ are not identity matrix any more, and +cosψ-sinψ+sinψ+cosψCPMn',m',n,mk,p+cosψ+sinψ-sinψ+cosψ will generate two different scaling factors on the main diagonal and also two different factors on the antidiagonal in the matrix, and the transformation procedure cannot be skipped.

If the target’s mobility is in the x-y plane only, both +cosψ-sinψ+sinψ+cosψ and +cosψ+sinψ-sinψ+cosψ are identity matrices.

If the target’s movement involves rotation around the x axis or y axis, +cosψ-sinψ+sinψ+cosψCPMn',m',n,mk,p+cosψ+sinψ-sinψ+cosψ will generate two different scaling factors on the main diagonal and also two different factors on the antidiagonal in the matrix, and the transformation procedure cannot be skipped.

If the target’s mobility is in the x-y plane only, the transformation procedure can be skipped. Otherwise, it cannot.

Note the transformation procedure in 38.901 has been used to model Type-2 EO in the draft CR. The same procedure is also applicable to targets.

The effective polarization matrix of the type-2 EO reflection path is given by CPMEO=cosγ2sinγ2-sinγ2cosγ2R∥00-R⊥cosγ1-sinγ1sinγ1cosγ1 (7.9-xx)

Targets can be treated in the same way as Type-2 EO by reusing the transformation procedure in 38.901.

# Spatial consistency of target and vertical/3D spatial consistency

Spatial consistency procedure defined in section 7.6.3 of 38.901 is to make sure the random variables generated are correlated across multiple drops of a simulation, e.g. if two UEs are within the correlation distance of each other. The procedure can also be used for spatially consistent mobility simulations, such as when a single UE is moving a larger distance than the correlation distance. When targets are introduced in ISAC channel model, the random variables of targets, including RCS component of B2 and XPR, should be considered for spatial consistency to address target movements in a simulation. The legacy correlation distances for spatial consistency can be reused for target movements.

B2 and XPR of a target, which are the random variables, should be considered for spatial consistency to address target movements in a simulation by reusing the legacy correlation distances.

In communication channel, when a UE moves, the geometry of BS and UE changes, including departure and arrival angles. Accordingly, cluster-specific random variables of AOD/AOA/ZOD/ZOA are included in the spatial consistency procedure. For sensing, both RCS component B2 and XPR of target are generated for each path, namely they are angle specific. A target’s movement causes a change of arrival and departure angles, and therefore, it is important that angle-specific random variables, B2 and XPM of the target, are spatially consistent, i.e. that they very smoothly for a smooth change in the arrival and departure angles. Since spatial consistency of B2 and XPR is related to angles, it is also called angular correlation.

Both RCS component B2 and XPR of target are generated for each path, namely they are angle specific.

Angle-specific random variables, B2 and XPM of the target, should be spatially consistent between adjacent angles, i.e. the random variables should not change abruptly for small changes in the angles.

# Background Channel

  - Mono-static background channel

One remaining issue is how to scale d3D for absolute delay.

Agreement On background channel for mono-static sensing, the following details are provided: - [omitted] - The absolute delay model d3D and Δτ as agreed for bistatic sensing for the same sensing scenario applies. Down-select one option from the following: - Option 0: no scaling factor is applied to d3D - Option 1: An offset is applied to d3D, i.e., d3D-c1 - Option 2: A scaling factor d_s is multiplied to d3D, i.e., d3D*d_s. d_s is a value within range [0, 1]. - Note: The adjustment of absolute delay doesn’t impact the generation of NLOS clusters between the Tx and each reference point Agreement The ISAC background channel can be generated between a sensing Tx and a sensing Rx or RP (relevant for monostatic case) via the following steps: - Step 1: generate a first set of clusters/rays according to TR 38.901(or other related TRs) - Step 2: generate a second set of NLOS clusters/rays according to TR 38.901 (or other related TRs), where the power of the second set of clusters/rays should be scaled down such that PnS2=PnS2nPnS2Pdrop=PnS2nPnS2P1S110G10 - P1S1 is the power of the NLOS cluster with the strongest power from the first set. - PnS2 is the power of the n-th cluster from the second set. - Where, N is the number of clusters, M is the number of rays within each cluster, value of G relates to power - N=360, M=1, G = -25dB, no further change from 38.901, 36.777, 38.858 (i.e., utilizing the same DS, ASA, ASD, ZSA, ZSD, Cθ, Cϕ as used for the first step) - The step 2 is an additional modeling component

A measurement of the monostatic background channel for an UMa base station has been performed. The TRP was deployed on top of an office building at a height of 25 m over ground. In the closest surroundings there are commercial buildings of similar height as the office building, while further away there are residential buildings of varying height and even some farmlands. The scenario can thus be considered to be mainly urban but changing to suburban or rural at longer distances. A 3.5 GHz frequency with 100 MHz bandwidth was used in the measurements. The Tx and Rx were collocated but used separate antenna panels that covered partially overlapping sectors. Therefore, the illumination of the environment is uneven in azimuth angle which is expected to underestimate the amount of multipath in the monostatic channel compared to if isotropic Tx and Rx antennas had been used.

An impulse response from the measurements is shown in Figure 13. The multipath dispersion is very large, covering the range from close to zero absolute delay up to many microseconds. This is due to the backscattering from buildings and other objects in the environment, both from very close objects in the vicinity of the TRP as well as from distant buildings.

Figure 13	Measured impulse response of a monostatic UMa background channel at 3.5 GHz.

Based on parameters agreed for TRP mono-static background channel, the minimum delay, 20%, 50%, 80-tile delay and maximum delay are summarized in Table 1. CDF curves of absolute delay of BS mono-static channel for different scenarios are in Annex. For Uma scenario, the first path arrives with an absolute delay of 0.36us and a path length of 108m, meaning targets with distance to BS smaller than 54m would be sensed without the interference of background channel. Multiple proposals were discussed to reduce the minimum delay.

Meanwhile, the maximum delay of RMa scenario is very small, 2.26us with a propagation path length of 678m. It was explained that -25dB threshold is used to remove paths and there are few and small scatterers in RMa scenario in RT model. Nevertheless, the silence period after the last path in mono-static background channel will make detection of distant targets, 339m away from BS, very easy. To reduce the silence period for distant targets, either the maximum delay of mono-static background channel model in RMa scenario is increased or low-power clusters is used with some changes.

Table 1	Delay of mono-static background channel (unit: us)

|  | minimum delay | 20% | 50% | 80% | maximum delay |
| --- | --- | --- | --- | --- | --- |
| UMa | 0.36 | 0.66 | 1.03 | 2.12 | 30.15 |
| UMi | 0.08 | 0.25 | 0.35 | 0.7 | 16.02 |
| Rma | 0.14 | 0.46 | 0.62 | 0.8 | 2.26 |
| Indoor Office | 0.01 | 0.2 | 0.31 | 0.43 | 2.11 |
| Indoor Factory | 0.03 | 0.29 | 0.46 | 0.8 | 5.84 |

UMa measurements show that the multipath dispersion is very large, covering the range from close to zero absolute delay up to many microseconds.

Absolute delay of the first path of mono-static background channel model for UMa scenario is 0.36us, corresponding to a path length of 108m. Targets with distance to BS smaller than 54m would be sensed without the interference of background channel.

The maximum delay of RMa scenario is only 2.26us with a propagation path length of 678m.

For low-power clusters, it was agreed that the DS in 38.901, 36.777, 38.858 is used in step2. The purpose is to make sure the last path has a path length comparable to that of target channel for the furthest target in the cell for some possibility. For mono-static sensing mode, delay spread of low-power cluster can be increased for example by 5~10 times for RMa scenario.

If low-power clusters is used for mono-static sensing mode, delay spread of low-power clusters can be increased by for example 5~10 times for RMa scenario.

Another issue is whether the same Δτ should be used in the three BS-reference point channels. In our view, the agreement that existing 38.901 procedures are used for each of the three channels means that existing spatial consistency procedures for correlating Δτ between reference points takes precedence.

Agreement When absolute delay model Δτ is configured, it applies to all NLOS clusters in each of Tx-target and target-Rx links and background channel. - For bistatic sensing: Different values of Δτ are separately generated for the Tx-target link, target-Rx link and the background channel - For monostatic sensing: the same value of Δτ is used for Tx-target link and target-Rx link, and a different value of Δτ is separately generated for the background channel Agreement The following working assumption is confirmed Working assumption Absolute delay model (referring to 7.6.9 in TR 38.901 as starting point) is a mandatory feature for both target channel and background channel for ISAC for UMi, UMa, InH, InF - Related model referring to Δτ values from 7-24GHz study item.

The existing absolute delay model in 38.901 defines correlation distance in horizontal plane.

Table 7.6.9-1: Parameters for the absolute time of arrival model

| Scenarios |  | InF-SL, InF-DL | InF-SH, InF-DH |
| --- | --- | --- | --- |
| lgΔτ=log10Δτ1s | μlgΔτ | -7.5 | -7.5 |
|  | σlgΔτ | 0.4 | 0.4 |
| Correlation distance in the horizontal plane [m] |  | 6 | 11 |

Note that the correlation distance (i.e. spatial consistency) is an integral part of the absolute time of arrival model. So when we agreed to use absolute delay for the background channel we already agreed to use the correlation distance to find the similarity of the absolute delay of different locations.

The agreement from 120bis on the background channel states that:

The mono-static background channel for the Tx would be sum of channels of the links between the Tx and all related reference points, which is

So, the channel for each reference point is generated with existing absolute delay procedures including partially correlated Δτ depending on the separation between the reference points. This is analogous to Δτ generated for each BS-UE channel. If correlation distance in Table 7.6.9-1 is conformed for mono-static background channel, the same Δτ used for the three reference points means they have the same level of correlation, which instead puts a restriction on their positions. Note the three reference points are very likely to have different heights, how to use the ‘correlation distance in the horizontal plane [m]’ needs further study.

If correlation distance in Table 7.6.9-1 is conformed, the same Δτ used for the three reference points means they have the same level of correlation, which instead puts a restriction on their positions.

In order to reuse the legacy procedure of absolute delay including correlation distance, Δτ for each of the three reference points is independently generated considering their relative positions and correlation distance.

Further study how to use the ‘correlation distance in the horizontal plane [m]’ for reference points of different heights.

  - Doppler of moving scatterers

In most communication scenarios, the UE is moving while the environment is mostly stationary. Hence the Doppler in the channel is mainly caused by the UE movement and the additional Doppler variations due to movement of scatterers in the environment has only a minor impact on communication performance. Nevertheless, TR 38.901 clause 7.6.10 contains provisions for modeling Doppler variations due to scatterer mobility. The Doppler variations due to scatterer mobility can be parameterized using two variables, vscatt which is the maximum speed of the scatterers, and p which is the proportion of the scatterers (clusters) that are mobile. Due to the limited impact on communication, no parameterization via measurements of vscatt and p in different scenarios have been made. TR 38.901 just contains some very high-level guidance on the subject.

Doppler due to moving scatterers (clusters) in the environment is modeled in TR 38.901 clause 7.6.10 using a maximum speed vscatt of the scatterers and a proportion p of the scatterers (clusters) that are mobile.

The existing guidance on vscatt and p in TR 38.901 does not cover sensing scenarios.

It is known from radar processing that the Doppler in the target channel can be very important for distinguishing the target from the background. Doppler processing can identify targets in channels even if the mostly stationary background channel is much stronger than the target channel. Movement in the environment may introduce additional Doppler variations both in the background channel and in the target channel. The latter can occur if the Tx-target link or the target-Rx link includes multipath due to reflections or scattering from a mobile object in the environment. One example of this with relevance to sensing is the case when a roadside hazard in the form of an animal should be detected. If the road goes through a forest, then the Tx-target or target-Rx channels may include multipath due to scattering from the trees. On a windy day this scattering may contribute significant Doppler to the target channel. However, this in itself may not be of particular concern for sensing performance. The problem occurs when the Tx-Rx background channel also includes scattering from the same trees. The combined energy from scattering from many trees may become much larger than the energy from scattering from the animal. Since both the background channel and the target channel will have similar Doppler signatures, the ability to detect and distinguish the target from the background becomes much more challenging.

For sensing, Doppler due to moving scatterers (clusters) in the environment can affect both the target channel and the background channel.

Moving scatterers in the background channel can have a profound impact on sensing performance.

It is a necessity to model Doppler variations from other objects than the target in the ISAC channel, otherwise results on target detection reliability and false alarm rates will be wildly inaccurate.

To gain further insight into the non-stationarity of the background channel, a measurement of a SMa/RMa BS-BS background channel was performed [11] and a delay-Doppler plot produced from these measurements is shown in Figure 14. These results highlight that even between stationary Tx and Rx there can be Doppler shifts in the background channel which may complicate sensing of desired targets. The relative power in the non-zero Doppler peaks highlighted with the red circles are some 40-45 dB lower than the LOS path. However, the peak(s) in a hypothetical target channel may also be much weaker than the LOS path.

In a measurement of a SMa/RMa BS-BS background channel, several paths (clusters) show non-zero Doppler shifts.

Figure 14	Delay-Doppler plot based on a time series of impulse responses between a pair of SMa/RMa base stations in LOS. Paths with non-zero Doppler frequencies have been highlighted.

A second set of measurements of the background BS-BS channel in an urban macro setting has been performed using 20 MHz bandwidth at 3.5 GHz [11]. In Figure 15, the impulse responses over the one hour of measurement time are superimposed on top of each other. The first arriving path, the LOS path, is extremely stable in amplitude. More importantly, some of the later arriving multipath show clear power variations over time. From this qualitative analysis it is obvious that the BS-BS background channel may include non-stationarity due to movement in the environment. Such non-stationarity can have a profound impact on the sensing performance. Therefore, further measurements are needed to quantify and model the non-stationarity.

Figure 15	Measured impulse responses between two UMa base stations during approximately one hour. Non-stationary paths, i.e. paths where the amplitude changes over time, are indicated.

In a measurement of a UMa BS-BS background channel, several paths (clusters) show power variations over time. The power variations could tentatively be connected to moving objects in the environment that had LOS to both Tx and Rx.

Movement in the environment as seen in these two different measurements can be modelled in two different ways:

- Option 1: Use the existing procedure in clause 7.6.10 in TR 38.901, which creates Doppler variations for a proportion p of the stochastic clusters in the background channel

- Option 2: Drop additional moving unintended targets, Type-1 EO, and use the target channel model for each of these.

Note that Option 1 may need very careful tuning of p and other parameters as the modeled movement could strongly affect the sensing performance. For instance, the measurement in Figure 14 and Figure 15 indicates a p value that is much smaller than the typical value of 0.2 listed in 38.901, section 7.6.10. As the measurements show that the multipath with Doppler can be weaker than the 25 dB cluster dropping threshold. This means low-power clusters are mandatory. Option 1 would require many new measurement campaigns which seems unlikely considering the remaining time for the SI.

In contrast, the second option only requires parameterization of RCS of certain Type-1 EOs, see section 2.2.1, and procedures for dropping these Type-1 EOs and generating their mobility. The second option is also attractive since it may create weaker clusters than the -25 dB threshold used in TR 38.901. If option 2 is to be used, this may require RCS modeling of additional types of objects such as trees. These objects can then be modeled the same as unintended targets, though maybe necessarily not with similar size as the intended target. Nevertheless, by specifying the RCS and movement patterns for these objects, see section 2.2.1, then their contribution to the ISAC channel can be readily determined.

The measurements show that the multipath with Doppler can be weaker than the 25 dB cluster dropping threshold. This requires low-power clusters to be mandatory.

Model movement in the environment using unintended targets modelled by Type-1 EOs.

# EO Type 2

  - In target channel only and independently from sensing scenarios

One issue about Type-2 EO is whether it should be modelled in background channel. If Type2-EO is modelled in the background channel, it can form the path Tx-Type2 EO-Rx. Since stochastic clusters have been used to generate multipath propagation in UE-BS communication channel, the need for a deterministic Type-2 EO in the background channel is not clear. One consequence of modelling Type-2 EO in the background channel is that the received power of the sensing Rx’s in the ISAC model with the absence of sensing targets would be different from that in the legacy UE-BS communication channel.

Since stochastic clusters have been used to generate multipath propagation in UE-BS communication channel, the need to model Type-2 EO in background channel is not clear.

Type-2 EO if modelled in background channel would lead to the consequence that the received power of the sensing Rx’s in the ISAC channel model in the absence of sensing targets is different from that in the legacy UE-BS communication channel.

Type-2 EO is modelled in the target channel only.

Type-2 EO is now only supported for automotives and objects creating hazards in urban grid scenario and FFS for other targets as discussed in AI 9.7.1. One argument was that there are buildings and walls modelled in urban grid. However, walls are also implicitly modelled by room size in indoor scenarios. Note that random orientation and random direction was agreed for human. Therefore, in addition to the walls modelled in urban grid, a Type-2 EO can be modelled with a random orientation and a physical size in other outdoor scenarios and modelled reusing the room size in indoor scenario.

In addition to the walls modelled in urban grid, a Type-2 EO can be modelled with a random orientation and a physical size in other outdoor scenarios and modelled reusing the room size in indoor scenario.

  - LOS/NLOS determination

One open issue about Type-2 EO is whether and how Type-2 EO may impact LOS state, with two options in the agreement.

Agreement When the EO type-2 is modelled in the target channel, down select between the following options to determine the LOS condition of the Tx-target link and target-Rx link - Option A: If type-2 EO is in the LOS ray of one link, the link is determined as NLOS condition, and otherwise use the LOS probability equation to determine the LOS/NLOS condition - FFS changes to the LOS probability defined in existing TRs - FFS details on blockage by EO type-2 - Option B: Use the LOS probability equation to determine the LOS/NLOS condition of one link, and then the impacts of type-2 EO is modeled by a blockage model

According to 38.901, propagation condition (LOS/NLOS) is determined according to LOS probability equations at Step 2, the beginning of channel generation. Note that the impact of a blocker in the existing model is restricted to adjusting the amplitudes of paths that have previously been generated for the certain LOS/NLOS state. If spatial consistency procedure is applied, when UE position changes with regard to the blocker, the presence of the blocker does not trigger a re-evaluation of the LOS state nor regeneration of paths. Therefore, it does not change the procedures of LOS/NLOS determination and spatial consistency.

The impact of a blocker in existing channel models is restricted to adjusting the amplitudes of paths that have previously been generated assuming a certain LOS state.

The presence of the blocker does not trigger a re-evaluation of the LOS state nor regeneration of paths in existing channel models.

Option B in the agreement is in line with the existing procedure for blocking in TR 38.901, where the blocking does not change the LOS state and only attenuates the direct path. In other words, paths in the target channel are first generated without the EO type 2, then it is subsequently used to generate additional reflected paths or block some of the paths.

Option B is in line with existing procedure for blocking in TR 38.901, where the blocking does not change the LOS state.

Option A, on the contrary, allows the blocker to determine NLOS condition, which causes a hard blockage that fully removes the direct path, and which is against the existing procedure in 38.901 and the following agreement. Assume a UE is close to a target. With Option A, a Type-2 EO in the LOS ray between Tx and target will set Tx-target link as NLOS, but BS-UE channel has a LOS condition, breaking the spatial consistency.

Agreement In order to generate each of the Tx-target link and target-Rx link in the target channel, the large scale and small scale parameters defined in existing 3GPP TRs, e.g., TR 38.901. TR 36.777, TR 37.885, TR 38.858, TR 38.859, TR 38.802, TR 38.854, etc. are used as starting point

Option A allows the blocker to determine NLOS condition. Not only does it go against the existing blockage procedure, but also the spatial consistency between BS-target link and BS-UE channel is broken, when the target and UE are close to each other.

Hard transition of LOS/NLOS condition in the channel response due to UT mobility is stated in clause 7.6.3.3 of 38.901 (quoted below). With spatial consistency procedure, if a UE moves a distance larger than correlation distance, LOS/NLOS state, pathloss and other large-scale parameters may change. Consequently, the channel needs to be regenerated with the new large-scale parameters and the corresponding small-scale parameters. Soft LOS state is used instead to avoid the trouble caused by the hard LOS/NLOS state transition.

In case there is a transition from LOS to NLOS due to UT mobility, there will be a hard transition in the channel response. This is because pathloss and LS parameters are different for these states, leading automatically to very different channel realizations. To circumvent such hard transitions the optional soft LOS state can be considered to determine the PL and the channel impulse responses containing characteristics of both LOS and NLOS.

With Option A, the LOS and NLOS condition of Tx-target and target-Rx links may change as the target moves, similar to the hard transition in the channel response due to UT mobility stated in clause 7.6.3.3 of 38.901, which legacy communication channel tries to avoid. With Option A, simulation stops with the change of LOS/NLOS state. It puts a constraint on targets’ movement and hinders and disables target tracking. With Option B, the legacy soft LOS state is also applicable to ISAC channel model by avoiding the hard transition.

Legacy communication channel avoids a hard transition between LOS and NLOS states due to UT mobility by using soft LOS state.

Option A may cause a hard transition in the channel response as the targets moves, similar to UT movement, which legacy communication channel tries to avoid.

With Option A, simulation stops with the change of LOS/NLOS state. It puts a constraint on targets’ movement and hinders and disables target tracking.

Type-2 EO has no impact on LOS/NLOS condition.

The legacy soft LOS state is applicable to ISAC channel model.

A different LOS/NLOS determination method is provided for map-based hybrid channel model specified in section 8.4 of 38.901. For the hybrid channel model, a digitized map is imported as the environment first, BS and UT are dropped with their 3D locations, and then LOS/NLOS flag is determined with geometric calculations in ray-tracing. Option A relies on Type-2 EO to geometrically determines NLOS state, which is similar to the map-based hybrid channel model. Option A is more suitable for the map-based hybrid channel model or ray-tracing than the stochastic channel stated in section 7.5 of 38.901.

Option A is more suitable for the map-based hybrid channel model or ray-tracing than the stochastic channel stated in section 7.5 of 38.901.

Figure 8.4-1: Channel coefficient generation procedure

  - Pre-requisite of specular reflection

Specular reflection is a distance-dependent phenomenon, and it occurs when Type-2 EO is of the same size as or larger than the first Fresnel zone. The radius of the first Fresnel zone is approximately given by:

r1≈d1d2λd1+d2,

for tx—target distance d₁ and target—rx distance d₂, and wavelength λ.

For mono-static sensing mode, d₁ = d₂. If the prerequisite doesn’t meet, scattering is the dominant phenomenon. (There exist some middle area/distances where quasi-specular reflection and scattering may happen.) Since Type-2 EO is modelled for specular reflection only, the prerequisite of specular reflection should be taken into account. Namely, specular reflection is modelled, if Type-2 EO is of the same size as or larger than the first Fresnel zone. Otherwise, it is not modelled.

Agreement If EO type-2 is modelled in an indirect path, only specular reflection is modeled for EO type-2 - polarization of the indirect path is product of polarization matrix of the target-Rx link, the target, and the Tx-target link, i.e., CPMtx,sp,rx= CPMsp,rx . CPMsp . CPMtx,sp - For the specular reflected ray generated by a EO type-2 in the Tx-target link or the target-Rx link (i.e., Tx-EO type-2-target, or target-EO type-2-Rx), CPMtx,sp or CPMsp,rx is the polarization matrix of EO type-2 - To generate polarization matrix of EO type-2, the procedure in [R1-2409394, R1-2410648] is taken as starting point - FFS applicability if the surface of EO type-2 is tilted

Specular reflection is a distance-dependent phenomenon, and it occurs when the object is of the same size as or larger than the first Fresnel zone.

Specular reflection is modelled, if Type-2 EO is of the same size as or larger than the first Fresnel zone. Otherwise, it is not modelled.

# Reference TR

The combinations of multiple sensing modes and sensing scenarios require many new links and new channels to be modelled.

Agreement In order to generate Tx-target link, target-Rx link and the background channel, the above table on reference TRs (excluding the already agreed part) is adopted for the mapping between reference TRs and a pair of nodes (STX, SRX, target) - Note: continue discussion for updating the table with RSU type UE - FFS: the generation of background channel based on reference TRs is subject to the addition of low-energy clusters 7; normal UE; aerial UE; UMi-AV, UMa-AV, and RMa-AV - TRP-aerial UE link of UMi-AV in Annex A and B of TR 36.777 by setting hBS =1.5m for FR1 - LOS probability is not reused, FFS new LOS probability - FFS pathloss model, shadowing fading - Working assumption: Reuse the channel model of scenario UMa-AV, UMi-AV, and RMa-AV of FR1 for FR2 - The corresponding parameter values in FR2 are used.

For terrestrial UE-UAV link and terrestrial UE-aerial UE channel in UMi, UMa, RMa, UMi-AV, UMa-AV, and RMa-AV scenarios, it is problematic to reuse LOS probability of BS-aerial UE in 36.777 by changing BS height to 1.5m, the height of a terrestrial UE. Note that in 36.777 different LOS probabilities equations are provided for different UAV height ranges, e.g., 100% LOS probability between TRP and aerial UE, , in UMa-AV scenario. Such 100% LOS probability doesn’t hold between a terrestrial UE and an aerial UE with the height between 100m and 300m.

It needs evaluation whether BS-aerial UE in 36.777 can be reused for terrestrial UE-UAV link and terrestrial UE-aerial UE channel, especially for indoor UEs.

Similar LOS probabilities have been studied extensively in [9] and [10]. The methodology used in [9] and [10] is to use digital maps of different scenarios to assess the LOS probability between a ground station and an airborne station. The results are presented as a closed-form expression for the LOS probability as a function of the ground station height and the elevation angle to the airborne station.

Figure 16	Digital maps of different scenarios

For the purposes of case 7, we can equate the ground station with a normal UE and the airborne station with an aerial UE. Furthermore, the elevation angle θ can be determined from the 2D distance and the height difference between the two UEs as:

θ=180πarctanhaerial UE-hnormal UEd2D

In [9] and [10], three different urban scenarios are studied and parameterized: a low-rise suburban scenario, a mid-rise urban scenario and a high-rise urban scenario. We can then equate the mid-rise scenario with UMa and Umi and the low-rise scenario with RMa. In addition, LOS probability is proposed as

pLoS=Vmax(1-exp⁡(kx+Ce90)1-exp⁡(k90+Ce90))

where x is the elevation angle in degrees and

k=h+abc

Ce=ah+b

Vmax=ah+b,   Vmax=min⁡(Vmax,  100)

The two references differ slightly about the parameter values. [10] is based on the earlier work but contains additional investigations, therefore we present the values from [10] below:

|  | k: [a, b, c] | Ce: [a, b] | Vmax: [a, b] |
| --- | --- | --- | --- |
| Low-rise | [4.9, 6.7, 2.6] | [0.19, 0] | [1.6 -17] |
| Mid-rise | [-2.6, 6.6, 2.0] | [0.42, -6.7] | [1 -21] |
| High-rise | [2.4, 7, 1] | [0.19, -2.7] | [1 -18] |

Based on this, we would like to make the following proposal.

The LOS probability between an aerial UE and a normal UE is the following:

pLoS=Vmax(1-expkx+Ce901-expk90+Ce90)

With

x=180πarctanabs(haerial UE-hnormal UE)d2D

k=hnormal UE+akbkck

Ce=achnormal UE+bc

Vmax=aVhnormal UE+bV,   Vmax=min⁡(Vmax,  100)

with parameter values

|  | k: [ak, bk,ck] | Ce: [ac, bc] | Vmax: [aV, bV] |
| --- | --- | --- | --- |
| RMa | [4.9, 6.7, 2.6] | [0.19, 0] | [1.6 -17] |
| UMa, UMi | [-2.6, 6.6, 2.0] | [0.42, -6.7] | [1 -21] |

An example is provided in Figure 17.

Figure 17	LOS probability vs elevation angle for UMa/Umi. Each curve represents the probability for a specific hnormal UE between 1 (lowest curve) and 24 m (uppermost curve).

The agreement also mentions the path loss and shadow fading as FFS. To address this, we can consider a link between a normal (outdoor or indoor) UE and an aerial UE at exactly the same height as a UMa BS. In this case, it is easy to see that we can represent this channel between the normal UE and aerial UE using the regular UMa channel model, where the BS assumes the identity of the aerial UE. The same applies to an RMa BS that can assume the position of an aerial UE in RMa. For path loss and shadow fading of case 7, for aerial UE heights of 22.5m-35m, which is similar to the corresponding macro BS height (above roof of 22.5m, 25 m in UMa, 35 m in RMa), use terrestrial UE-BS link of UMa or RMa model of TR 38.901 with the BS representing the aerial UE. If aerial UE is lower than 22.5m, pathloss and shadow fading of terrestrial UE-aerial UE link in urban areas can refer to terrestrial UE-BS link in UMi scenario of 38.901, where micro BS is also lower than 22.5m.

For path loss and shadow fading of terrestrial UE-aerial UE link, for aerial UE height of 22.5m-35m, which is similar to the corresponding BS height, use terrestrial UE-BS link of UMa scenario in TR 38.901 for urban areas and that of RMa scenario for rural areas with the BS representing the aerial UE.

For path loss and shadow fading of terrestrial UE-aerial UE link, for aerial UE heights that are lower than 22.5m, use terrestrial UE-BS link of UMi scenario in TR 38.901 for urban areas with the BS representing the aerial UE.

For lack of better approaches and measurements, we suggest to reuse the UMa or RMa models also for higher aerial UE heights, i.e. above 35m.

For path loss and shadow fading of terrestrial UE-aerial UE link, for aerial UE heights that are higher than 35m, use terrestrial UE-BS link of UMa in TR 38.901 for urban areas and that of RMa scenario for rural areas with the BS representing the aerial UE.

# Link-level ISAC channel models

Compared with system-level channel models, link-level channel models in 38.901 generate the channel between one BS and one UE and focus on small-scale parameters only. Specific hard-coded parameters of cluster delay and cluster power are used for TDL and CDL channel models, and cluster angles are additionally used for CDL channel models.

Based on the characteristics of link-level channel models, link-level ISAC channel models should follow the same design principles of system-level ISAC channel models. For example, the agreement that HISAC=Htarget+Hbackground applies also to link level channel models, where the target channel is the concatenation of Tx-target and target-Rx links. Legacy communication channel models in existing TRs are used for Tx-target, target-Rx links and Tx-Rx channel.

Accordingly, legacy link-level channel models can be reused to model Tx-target and target-Rx links of target channel and Tx-Rx channel of background channel. Since targets are modelled deterministically in a certain geometry of target, Tx, and Rx, CDL channel models are more suitable than TDL models. When target channel is generated by concatenating Tx-target and target-Rx link, angles of two concatenated rays are used to determine RCS of the path.

For link-level ISAC channel models,

- HISAC=Htarget+Hbackground still holds.

- legacy CDL models are reused to model Tx-target and target-Rx links of target channel and Tx-Rx channel of background channel.

- target channel is generated by concatenating Tx-target and target-Rx link

# Conclusion

In the previous sections, we made the following observations:

Observation 1	In some sensing scenarios, the shadows are significant, and their existence may be a strong indicator of target presence. Examples are intruder detection or vehicular sensing of other vehicles and obstacles.

Observation 2	Shadow behind a sufficiently large target means the combined target and background channel needs to have lower energy than the background channel in the absence of any target.

Observation 3	It was agreed that sensing target is in the far field of sensing Tx/Rx. However, sensing Rx is likely to locate in the near field of a large target, where shadow effect needs to be taken into account.

Observation 4	For a car of size D=5 and a frequency of 6 GHz, the far field from the car occurs at a distance of 1 km. A sensing Rx with its distance to the vehicle smaller than 1km is in the car’s near field.

Observation 5	A method of modelling the shadows is to apply the existing blockage model to background channel, where a target casts a shadow for sensing Rx, so that Hbackground is reduced in the shadow region.

Observation 6	Interaction between targets is critical, when one target’s shadows have significant impacts on another target, i.e., when incident or scattered rays of the second target are severely attenuated.

Observation 7	The blockage of one target can be considered in the Tx-target and/or target-Rx links of another target channel depending on sensing mode.

Observation 8	There is no discussion on types of unintended targets and their modelling, including RCS, CPM, and velocity.

Observation 9	Trees moving in the wind will challenge sensing algorithms in distinguishing an intended target from the unintended targets in frequency domain due to their similar Doppler shifts.

Observation 10	Without modelling of unintended objects, the SID’s objective of distinguishing targets from unintended objects and clutter/scattering patterns cannot be fulfilled, and the study is incomplete.

Observation 11	There is no discussion on vscatt and p of moving stochastic clusters, and therefore they cannot serve as unintended objects or generate comparable physical characteristics as sensing targets.

Observation 12	Measurements show that there are many multipath coming from the direction of the trees and trees are strong clusters.

Observation 13	Discontinuities in the RCS cause signals that are not bandlimited and non-physical artifacts in the Doppler spectrum.

Observation 14	With Option 1, there are no rapid random variations of RCS, which are observed in measured channels. It obviates the need of combining observations over time to ensure a good probability of detection, which is necessary in reality.

Observation 15	With Option 2, discontinuous RCS leads to non-physical artifacts in the channel, including very high Doppler frequencies and spurious out-of-band artifacts, which may negatively affect the accuracy of detection.

Observation 16	On the other hand, the finding that Doppler artifacts only occur in the target channel but not in the background channel may be used by a sensing algorithm to artificially inflate the target detection probability and reduce the false alarm rate. Hence, it is very important to not introduce such artifacts in the channel model.

Observation 17	With bi-static RCS WA, mono-static RCS is a very small subset of bi-static RCS, with the ratio of 1 : 360*180, with 1-degree angular granularity.

Observation 18	Bi-static RCS values are more widely used than mono-static RCS in target channel.

Observation 19	Mono-static RCS component of A*B1 is in a large value range for angle-dependent RCS. Mean of mono-static RCS doesn’t reflect RCS of the direct path with a particular incident angle.

Observation 20	When objects move and rotate, their scattering properties stay the same in the local coordinate system of the object, similar to the field patterns of BS and UE. This is why scattering (RCS) and cross-polarisation (CPM) is modelled in the coordinate system of the object.

Observation 21	Transformation between LCS and GCS is necessary to make sure that Tx, target, and Rx can relate one spherical unit vector in GCS with the same definition of θ and φ.

Observation 22	If CPMsp,i is defined in the Global Coordinate System (GCS), rotation of the target will not be supported by the ISAC channel model. Vehicle, large UAV, AGV, and human can only move in the direction of the positive x-axis.

Observation 23	If CPMsp,i is defined in a Local Coordinate System (LCS), the channel model can support rotation of the targets by reusing the legacy transformation procedure in 38.901, which is used to support arbitrary orientation of BS and UE antenna panel and Type-2 EO as well.

Observation 24	Existing communication channel in 38.901 supports arbitrary orientation of BS and UE antenna array in their LCS. If the orientation of the LCS is different from the GCS orientation, a transformation between LCS and GCS is needed.

Observation 25	If the target’s mobility is in the x-y plane only, both +cosψ-sinψ+sinψ+cosψ and +cosψ+sinψ-sinψ+cosψ are identity matrices.

Observation 26	If the target’s movement involves rotation around the x axis or y axis, +cosψ-sinψ+sinψ+cosψCPMn',m',n,mk,p+cosψ+sinψ-sinψ+cosψ will generate two different scaling factors on the main diagonal and also two different factors on the antidiagonal in the matrix, and the transformation procedure cannot be skipped.

Observation 27	Both RCS component B2 and XPR of target are generated for each path, namely they are angle specific.

Observation 28	UMa measurements show that the multipath dispersion is very large, covering the range from close to zero absolute delay up to many microseconds.

Observation 29	Absolute delay of the first path of mono-static background channel model for UMa scenario is 0.36us, corresponding to a path length of 108m. Targets with distance to BS smaller than 54m would be sensed without the interference of background channel.

Observation 30	The maximum delay of RMa scenario is only 2.26us with a propagation path length of 678m.

Observation 31	If correlation distance in Table 7.6.9-1 is conformed, the same Δτ used for the three reference points means they have the same level of correlation, which instead puts a restriction on their positions.

Observation 32	Doppler due to moving scatterers (clusters) in the environment is modeled in TR 38.901 clause 7.6.10 using a maximum speed vscatt of the scatterers and a proportion p of the scatterers (clusters) that are mobile.

Observation 33	The existing guidance on vscatt and p in TR 38.901 does not cover sensing scenarios.

Observation 34	For sensing, Doppler due to moving scatterers (clusters) in the environment can affect both the target channel and the background channel.

Observation 35	Moving scatterers in the background channel can have a profound impact on sensing performance.

Observation 36	It is a necessity to model Doppler variations from other objects than the target in the ISAC channel, otherwise results on target detection reliability and false alarm rates will be wildly inaccurate.

Observation 37	In a measurement of a SMa/RMa BS-BS background channel, several paths (clusters) show non-zero Doppler shifts.

Observation 38	In a measurement of a UMa BS-BS background channel, several paths (clusters) show power variations over time. The power variations could tentatively be connected to moving objects in the environment that had LOS to both Tx and Rx.

Observation 39	Since stochastic clusters have been used to generate multipath propagation in UE-BS communication channel, the need to model Type-2 EO in background channel is not clear.

Observation 40	Type-2 EO if modelled in background channel would lead to the consequence that the received power of the sensing Rx’s in the ISAC channel model in the absence of sensing targets is different from that in the legacy UE-BS communication channel.

Observation 41	The impact of a blocker in existing channel models is restricted to adjusting the amplitudes of paths that have previously been generated assuming a certain LOS state.

Observation 42	The presence of the blocker does not trigger a re-evaluation of the LOS state nor regeneration of paths in existing channel models.

Observation 43	Option B is in line with existing procedure for blocking in TR 38.901, where the blocking does not change the LOS state.

Observation 44	Option A allows the blocker to determine NLOS condition. Not only does it go against the existing blockage procedure, but also the spatial consistency between BS-target link and BS-UE channel is broken, when the target and UE are close to each other.

Observation 45	Legacy communication channel avoids a hard transition between LOS and NLOS states due to UT mobility by using soft LOS state.

Observation 46	Option A may cause a hard transition in the channel response as the targets moves, similar to UT movement, which legacy communication channel tries to avoid.

Observation 47	With Option A, simulation stops with the change of LOS/NLOS state. It puts a constraint on targets’ movement and hinders and disables target tracking.

Observation 48	Option A is more suitable for the map-based hybrid channel model or ray-tracing than the stochastic channel stated in section 7.5 of 38.901.

Observation 49	Specular reflection is a distance-dependent phenomenon, and it occurs when the object is of the same size as or larger than the first Fresnel zone.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Support blockage model B in 39.901 to model shadows, including

	modelling blockage in the background channel of the target

	modelling interactions between the targets when needed by introducing blockage in the Tx-target and target-Rx link of another target

Proposal 2	Study modelling of birds and trees.

Proposal 3	For the bistatic RCS of a bird, A is uniformly distributed between -40 to -20 dBsm, B=0dB, standard deviation of B2=3dB.

Proposal 4	Use the following bistatic RCS model of a single scattering point for a tree: A*B1 with a constant level of 10 dBm2 and a single lobe in the forward direction with A*B1 increasing to 23 dBm2. The B2 standard deviation is approximately 3 dB.

Proposal 5	For tree RCS model with two scattering points, consider a (larger) fraction of the RCS to correspond to a fixed scattering point representing static parts of the tree such as the trunk, and another (smaller) fraction of the RCS to correspond to moving scattering points representing leaves and branches.

Proposal 6	Study RCS of truck/bus, child, and animal based on agreed sizes in AI 9.7.1 and RCS of AGV of both sizes

Proposal 7	Since micro-Doppler is essential for distinguishing wanted and unwanted targets, methods to model micro-Doppler are needed for the completion of the Study Item.

Proposal 8	To model the Doppler signature (micro-Doppler) of various targets and Type-1 EOs, the target is represented by one (or more) fixed scattering points and one (or more) scattering points that are moving in relation to the fixed scattering point

Proposal 9	If angular discontinuity issue in bi-static RCS model is not solved, add a note: angular discontinuity of bi-static RCS happens when bi-sector changes direction and the incurred band-unlimited Doppler artifacts may lead to unintended sensing performance loss or other unexpected simulation behavior.

Proposal 10	Support angular correlation of B2 and model the stochastic B2 with C random scattering centres as B2dinc, dsca=c=1Cβexp⁡(-j2πdinc+dsca,pc/λ)2.

Proposal 11	The offset pc can be taken from a uniform grid with spacing 0.7 wavelengths and the number of scattering centres C can be chosen as 5.  Other methods to select these parameters that result in equivalent properties exist.

Proposal 12	If RCS Component A is the mean value of mono-static RCS, it should be noted that such RCS component A may not reflect the actual path loss in the target channel.

Proposal 13	If the CPM of the target is defined in the LCS, a rotation of the target can be supported by replacing CPMn',m',n,mk,p with +cosψ-sinψ+sinψ+cosψCPMn',m',n,mk,p+cosψ+sinψ-sinψ+cosψ.

Proposal 14	If the target’s mobility is in the x-y plane only, the transformation procedure can be skipped. Otherwise, it cannot.

Proposal 15	Targets can be treated in the same way as Type-2 EO by reusing the transformation procedure in 38.901.

Proposal 16	B2 and XPR of a target, which are the random variables, should be considered for spatial consistency to address target movements in a simulation by reusing the legacy correlation distances.

Proposal 17	Angle-specific random variables, B2 and XPM of the target, should be spatially consistent between adjacent angles, i.e. the random variables should not change abruptly for small changes in the angles.

Proposal 18	If low-power clusters is used for mono-static sensing mode, delay spread of low-power clusters can be increased by for example 5~10 times for RMa scenario.

Proposal 19	In order to reuse the legacy procedure of absolute delay including correlation distance, Δτ for each of the three reference points is independently generated considering their relative positions and correlation distance.

Proposal 20	Further study how to use the ‘correlation distance in the horizontal plane [m]’ for reference points of different heights.

Proposal 21	The measurements show that the multipath with Doppler can be weaker than the 25 dB cluster dropping threshold. This requires low-power clusters to be mandatory.

Proposal 22	Model movement in the environment using unintended targets modelled by Type-1 EOs.

Proposal 23	Type-2 EO is modelled in the target channel only.

Proposal 24	In addition to the walls modelled in urban grid, a Type-2 EO can be modelled with a random orientation and a physical size in other outdoor scenarios and modelled reusing the room size in indoor scenario.

Proposal 25	Type-2 EO has no impact on LOS/NLOS condition.

Proposal 26	The legacy soft LOS state is applicable to ISAC channel model.

Proposal 27	Specular reflection is modelled, if Type-2 EO is of the same size as or larger than the first Fresnel zone. Otherwise, it is not modelled.

Proposal 28	It needs evaluation whether BS-aerial UE in 36.777 can be reused for terrestrial UE-UAV link and terrestrial UE-aerial UE channel, especially for indoor UEs.

Proposal 29	The LOS probability between an aerial UE and a normal UE is the following:

Proposal 30	For path loss and shadow fading of terrestrial UE-aerial UE link, for aerial UE height of 22.5m-35m, which is similar to the corresponding BS height, use terrestrial UE-BS link of UMa scenario in TR 38.901 for urban areas and that of RMa scenario for rural areas with the BS representing the aerial UE.

Proposal 31	For path loss and shadow fading of terrestrial UE-aerial UE link, for aerial UE heights that are lower than 22.5m, use terrestrial UE-BS link of UMi scenario in TR 38.901 for urban areas with the BS representing the aerial UE.

Proposal 32	For path loss and shadow fading of terrestrial UE-aerial UE link, for aerial UE heights that are higher than 35m, use terrestrial UE-BS link of UMa in TR 38.901 for urban areas and that of RMa scenario for rural areas with the BS representing the aerial UE.

Proposal 33	For link-level ISAC channel models,

	HISAC=Htarget+Hbackground still holds.

	legacy CDL models are reused to model Tx-target and target-Rx links of target channel and Tx-Rx channel of background channel.

	target channel is generated by concatenating Tx-target and target-Rx link

# Annex

CDF curves of absolute delay of BS mono-static channel for different scenarios

# References

- RP-242348, Revised SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR, Xiaomi, AT&T, 3GPP TSG RAN Meeting #105, September 2024

Y. L. C. de Jong and M. H. A. J. Herben, "A tree-scattering model for improved propagation prediction in urban microcells," in IEEE Transactions on Vehicular Technology, vol. 53, no. 2, pp. 503-513, March 2004.

Jackson et al, “The motion of trees in the wind: a data synthesis”, Biogeosciences, vol. 18, no. 13, pp. 4059-4072, 2021.

Rahman, S., Robertson, D.A. “Radar micro-Doppler signatures of drones and birds at K-band and W-band”, Sci Rep 8, 17396 (2018), https://doi.org/10.1038/s41598-018-35880-9.

C. R. Vaughn, "Birds and insects as radar targets: A review," in Proceedings of the IEEE, vol. 73, no. 2, pp. 205-227, Feb. 1985.

J. Ko et al., "Measurements and Analysis of Radio Propagation at 28 GHz in Vegetated Areas of Typical Residential Environments," in IEEE Transactions on Antennas and Propagation, vol. 68, no. 5, pp. 4149-4154, May 2020.

- R1-2500756, Discussion on ISAC Channel Modelling, Ericsson, RAN1#120, February 2025

- Eric Huang, et al., Spherical Harmonic-Based Anisotropic Point Scatterer Models for Radar Applications Using Inverse Optimization, 2022 IEEE

People’s Republic of China, “Slant path clutter loss model update proposals based on latest working document 3K/335”, Document 3K/38-E (also available as Document 3M/65-E), ITU-R WP3 meetings, Denver, Colorado, USA, 29 May - 7 June 2024.

Telefon AB – LM Ericsson, “Simulations and model proposal of LOS probabilities”, Document 3K/118-E (also available as Document 3J/107-E and Document 3M/151-E), ITU-R WP3 meetings, Geneva, 17-21 February 2025.

- R1-2502726, Discussion on ISAC Channel Modelling, Ericsson, RAN1#120bis, April 2025

- R1-2503157, Draft CR for TR 38.901 to introduce channel model for ISAC, Xiaomi, AT&T, RAN1 #120bis, April, 2025