3GPP TSG RAN WG1#121			    R1-2504471

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:	9.2.1

Source: 	Moderator (ZTE)

Title: 	Moderator Summary #1 on UE-initiated/event-driven beam management

Document for:	Discussion and Decision

- Introduction

In RAN#102, the Rel-19 WID on NR MIMO phase 5 is approved. In the approved WID, UE-initiated/event-driven beam management is a part of the RAN1 objectives as follows:

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

- Plan

Per RAN1#120-bis outcome, the following issues are prioritized for this meeting:

|  | Issue | Topics |
| --- | --- | --- |
| 1 | Trigger-event detection | Left-over for trigger detection on Event-1, Event-2 and Event-7, e.g., extra condition(s) of resetting the counting, etc. |
| 2 | UL signaling content(s) | Left-over for L1-RSRP report format for Event-1, Event-2, Event-7 |
| 3 | UL signaling medium/container | Left-over on first PUCCH for both Mode-A and Mode-B. - #1 Whether/how to (re-)transmit the first PUCCH channel - #2 Multiplexing/dropping rule(s) of 1-bit first PUCCH, e.g., Case-2, (collided with PUSCH). and Case-5 (collided with a PUCCH format 2/3/4 carrying HARQ/CSI). |
| 4 |  | Left-over on Step-2&3 in Mode-A - #1 Whether/how to handle the case if UE has not sent any first PUCCH associated for UE-initiated/event-driven beam reporting. - #2 Whether/how to introduce timeline (e.g., from the DCI to second PUSCH), CSI reference resource, the counting of occupied CPU(s) and active resource(s); - #3 Left-over on UCI-multiplexing/dropping rule for second PUSCH/UEI-CSI. |
| 5 |  | Left-over on Step-2 in Mode-B, - #1 Whether/how to introduce CSI reference resource, the counting rule of occupied CPU(s) and active resource(s). - #2 Whether/how to introduce UE capability of ‘X’, and clarify the definition of ‘available’ transmission occasion of the second UL channel. - #3 Left-over on UCI-multiplexing/dropping rule for second PUSCH/UEI-CSI. |
| 6 | Cross-CC measurement/report | Left-over on cross-CC measurement/report in UEI beam report |

Then, based on the outcome of the contributions from companies [1]-[36], the followings are provided in this document:

- Summary of companies’ views on each of open issues raised by interested companies, where the open issues are categorized as follow:

  - Issue 1 – Trigger-event detection

  - Issue 2 – UL signaling content(s)

  - Issue 3 – UL signaling medium/container

  - Issue 4 – Cross-CC measurement/report

- Observations and recommended proposals based on the summary of companies’ views

Note: While considering the progress of each of above issues, we optionally handle related TPs for current spec draft, in order to identifying the issue(s) on details and avoid unnecessary clarification on the background.

- Contact Person

For potential offline discussion, companies/delegates are encouraged to enter the contact information in the table below:

Table 0 Contact Information

| Company | Point(s) of contact | Email address(es) |
| --- | --- | --- |
| Apple | Hong He | hhe5@apple.com |
| CATT | Jiayi Yang | yangjiayi@catt.cn |
| CEWiT | Pardh | pardhasarathy.j@cewit.org.in |
| CMCC | Yan LI | liyanwx@chinamobile.com |
| Ericsson | Claes Tidestav | claes.tidestav@ericsson.com |
| ETRI | Cheulsoon Kim | cs.kim@etri.re.kr |
| Fujitsu | David | wangguotong@fujitsu.com |
| FUTUREWEI | Weimin Xiao | weimin.xiao@futurewei.com |
| FUTUREWEI | Zhigang Rong | zrong@futurewei.com |
| Google | Alex Liou | alexliou@google.com |
| Huawei/HiSilicon | Keyvan Zarifi | Keyvan.zarifi@huawei.com |
| Huawei/HiSilicon | Fanbo | asen.fanbo@huawei.com |
| HONOR | Guozeng Zheng | zhengguozeng@honor.com |
| Intel | Gary | gang.xiong@intel.com |
| InterDigital | Jonghyun Park | jonghyun.park@interdigital.com |
| KDDI | Shunsuke Kamiwatari | sh-kamiwatari@kddi.com |
| LG | Hyungtae Kim | ht.kim@lge.com |
| MediaTek | Rebecca Chen | rebecca.chen@mediatek.com |
| Meta | Avik Sengupta | aviksg@meta.com |
| NEC | Peng GUAN | guan_peng@nec.cn |
| NEC | Yukai GAO | gao_yukai@nec.cn |
| NICT | Kenichi Takizawa | takizawa@nict.go.jp |
| Nokia | Mihai Enescu | mihai.enescu@nokia.com |
| Nokia | Youngsoo Yuk | youngsoo.yuk@nokia.com |
| NTT DOCOMO | Mamoru Okumura | Mamoru.okumura.nz@nttdocomo.com |
| OPPO | Li Guo | guoli@oppo.com |
| Panasonic | Khalid Zeineddine | khalid.zeineddine@eu.panasonic.com |
| Qualcomm | Wooseok Nam | wnam@qti.qualcomm.com |
| Ruijie Networks | Ke Zhong | zhongke@ruijie.com.cn |
| Samsung | Dalin Zhu | dalin.zhu@samsung.com |
| Samsung | Sa Zhang | sa.zhang@samsung.com |
| Sharp | Taka | (New)fukui.takahisa@mail.sharp/(Old) fukui.takahisa@sharp.co.jp |
| Sony | Naoki Kusashima | Naoki.Kusashima@sony.com |
| Sony | Jose Flordelis | Jose.Flordelis@sony.com |
| Spreadtrum | Yu Yang | yu.yang2@unisoc.com |
| vivo | Rakesh Tamrakar | rakesh@vivo.com |
| Xiaomi | Mingju LI | limingju@xiaomi.com |
| ZTE | Yang Zhang | zhang.yang220@zte.com.cn |
| Lenovo | Bingchao Liu | Liubc2@lenovo.com |
| Ofinno | Jae-Nam Shim | jshim@ofinno.com |

- Discussion

## Issue 1 – Trigger-event detection

Table 1-1 Summary for Issue 1

| # | Issue | Companies’ view and Recommended Proposal |
| --- | --- | --- |
| 1.1 | Left-over for timer/window/ counting | [120b] Agreement Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, support Option-4. - Option-4: If an Event-2 instance for a new beam is obtained at the time t and the timer for the new beam is not running, UE starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window) - Note: Timer is new beam specific. - Note: T_window is the agreed time window parameter for measurement. - Introduce separate UE capability to limit the number of timers. There is only 1 timer per new beam. Above agreement is captured in RAN1 specifications. Endorsed CR for R19-MIMO UEIBM in TS 38.214 is provided as following, and then we have the following bracket(s) related to trigger-event detection: 5.2.1.5.4.1 UE Initiated CSI reporting For a UE configured with a CSI-ReportConfig with eventType-r19 for periodic reference signals [with the same periodicity configured by the newBeamResourceSet-r19] and with dl-OrJointTCI-StateList, when an event instance is determined (as described in the following clauses), - for a reference signal configured by the newBeamResourceSet-r19, if eventType-r19 is set to ‘event2’ or ‘event7’, or - for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI, if eventType-r19 is set to ‘event1’, [if eventDetectionTimeWindowLength-r19 is provided, the UE starts a time window for such reference signal, wherein the time window does not overlap with other time windows for such reference signal.] When the number of event instances determined for such reference signal within the time window, if eventDetectionTimeWindowLength-r19 is provided, is greater than or equal to 1 or to eventInstanceCount-r19, if provided, the UE transmits PUCCH format 0 or format 1 in the PUCCH resource configured by PUCCHResource-r19 in the CSI-ReportConfig. [The counting of the event instances for such reference signal is restarted: - if the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state is updated.] The UE does not expect that a CSI trigger state associated with CSI report configuration(s) configured with the higher layer parameter eventType-r19 is further associated with other CSI report configurations that are not configured with the higher layer parameter eventType-r19.. FL Note: No.1 goal of this Malta meeting is indeed to make sure that the basic functionality of UEIBR involving timer/time-window/counting can be well specified in the first version of Rel-19! FL Observation: - Option-1 (clarify event-detection by “time-window/counting(?)”): - Supported by: Samsung, vivo, Sharp, - Option-2 (clarify event-detection by “timer/counting” to align agreement) - Supported by: Huawei/HiSi, MediaTek, Ericsson, ZTE, Google, Panasonic, OPPO, Ofinno, Google, xiaomi, Spreadtrum, MediaTek, Futurewei, NEC, Fujitsu, Lenovo, NTT DOCOMO, Qualcomm, HONOR, Transsion, ETRI FL Note: Per companies input, the proposals for time-window/counting (option-1) are controversial even, and then the following three highlighted part should be solved. - Then, while considering the misalignment between periodicity of first PUCCH and the length of (sliding) windows, Panasonic and ZTE, etc. prefer to introduce the trigger-state declaration (as MAC) for initiating PUCCH transmission and clarifying 1-bit condition-met-indicator. Proposal 1.1A (time-window/counting): Adopt the following changes in Clause 5.2.1.5.4.1 in TS38.214. 5.2.1.5.4.1 UE Initiated CSI reporting For a UE configured with a CSI-ReportConfig with eventType-r19 for periodic reference signals [with the same periodicity configured by the newBeamResourceSet-r19] and with dl-OrJointTCI-StateList, when an event instance is determined (as described in the following clauses), - for a reference signal configured by the newBeamResourceSet-r19, if eventType-r19 is set to ‘event2’ or ‘event7’, or - for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI, if eventType-r19 is set to ‘event1’, if eventDetectionTimeWindowLength-r19 is provided, and the time window for such reference signal has not started yet or has ended, the UE starts a time window a timer of a length eventDetectionTimeWindowLength-r19, if provided, at the beginning of a transmission occasion of for such the reference signal for which an event instance is determined, if the timer is not running; the time window includes the time while the timer is running wherein the time window does not overlap with other time windows for such reference signal. When the number of event instances determined for such reference signal within the time window, if eventDetectionTimeWindowLength-r19 is provided, is greater than or equal to 1 or to eventInstanceCount-r19, if provided, the UE transmits PUCCH format 0 or format 1 in the earlist transmission occasion of in the PUCCH resource configured by PUCCHResource-r19 in the CSI-ReportConfig, T_proc after the end of the time window. [The counting time window of the event instances for such reference signal is stopped restarted]: - if the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state is updated.] The UE does not expect that a CSI trigger state associated with CSI report configuration(s) configured with the higher layer parameter eventType-r19 is further associated with other CSI report configurations that are not configured with the higher layer parameter eventType-r19.. Supported by Samsung, Sharp, Proposal 1.1B (timer/counting): Adopt the following changes in Clause 5.2.1.5.4.1 in TS38.214. 5.2.1.5.4.1 UE Initiated CSI reporting For a UE configured with a CSI-ReportConfig with eventType-r19 for periodic reference signals [with the same periodicity configured by the newBeamResourceSet-r19] and with dl-OrJointTCI-StateList and without eventDetectionTimeWindowLength-r19, when an event instance is determined (as described in the following clauses), - for a reference signal configured by the newBeamResourceSet-r19, if eventType-r19 is set to ‘event2’ or ‘event7’, or - for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI, if eventType-r19 is set to ‘event1’, the UE transmits PUCCH format 0 or format 1 in the PUCCH resource configured by PUCCHResource in the CSI-ReportConfig. For a UE configured with a CSI-ReportConfig with eventType-r19 for periodic reference signals [with the same periodicity configured by the newBeamResourceSet-r19] and with dl-OrJointTCI-StateList and with eventDetectionTimeWindowLength-r19, when an event instance is determined (as described in the following clauses), - for a reference signal configured by the newBeamResourceSet-r19, if eventType-r19 is set to ‘event2’ or ‘event7’, or - for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI, if eventType-r19 is set to ‘event1’, [if eventDetectionTimeWindowLength-r19 is provided, the UE - starts a timer for such reference signal from the value provided by eventDetectionTimeWindowLength-r19 and sets the counter to 1, if the timer is not running; or - increments the counter for such reference signal by 1 if the timer is already running. wherein the time window does not overlap with other time windows for such reference signal.] When the number of event instances determined for such reference signal within the time window, if eventDetectionTimeWindowLength-r19 is provided, is greater than or equal to 1 or to eventInstanceCount-r19, if provided, If the timer is running and the counter for such reference signal is greater than or equal to eventInstanceCount-r19, the UE transmits PUCCH format 0 or format 1 in the PUCCH resource configured by PUCCHResource-r19 in the CSI-ReportConfig. [The counting counter of the event instances for such reference signal is reset to zero restarted and the timer for such reference signal is stopped: - if the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state is updated.] The UE does not expect that a CSI trigger state associated with CSI report configuration(s) configured with the higher layer parameter eventType-r19 is further associated with other CSI report configurations that are not configured with the higher layer parameter eventType-r19.. Supported by: OPPO, Ofinno, Google, xiaomi, ZTE, Spreadtrum, MediaTek, Futurewei, Huawei/HiSi, NEC, Fujitsu, Lenovo, NTT DOCOMO, Qualcomm, HONOR, Ericsson, IDC, Transsion, ETRI, China Telecom, |
| 1.2 | Left-over for current beam for Event-2 —— Extra condition(s) of resetting the counting | [120] Agreement Regarding triggering event determination for Event 2, at least Candidate #2 is supported for resetting the counting. - Candidate#1: RS reconfiguration/update or MAC-CE signaling (if supported) for new beam is received; - FFS: whether to reset the counting for all new beams. - FFS: whether to maintain the counting whose new beam is NOT updated. - Candidate#2: [The measured current beam based on] indicated TCI state is updated; - In such case, the UE need to reset the counting for all new beams. - Candidate#3: UEI beam report is transmitted; - FFS: Only reset the counting of new beams fulfilling triggering condition and reported by the UEI beam report - Candidate#4: NW response (e.g., DCI in step-2 of Mode-A) is detected. - Candidate#5: The time window expires - Candidate#6: The threshold for event evaluation is re-configured by RRC signaling - (FFS) Candidate#7: The RRC parameter(s) associated with the CSI report configuration for UEI beam report is reconfigured. - FFS: RRC parameter(s) - FFS: Other candidates Note: Whether this proposal is captured in RAN1 or RAN2 is a separate discussion point. [120b] Agreement Regarding triggering event determination for Event 2, on resetting the counting, the following modification on the agreed Candidate #2 in RAN1#120 is supported. - Candidate#2: [The measured current beam RS is updated based on] indicated TCI state is updated; - In such case, the UE needs to reset the counting for all new beams. - FFS: Further details on the update (if necessary) FL observation on other conditions of reseting the counting: - Candidate #1: ZTE, vivo, Qualcomm, ETRI, CATT, Google, Futurewei, Panasonic, Huawei/HiSi, Sharp, Spreadtrum, Fujitsu, CMCC, IDC, Lenovo, Apple, Ofinno, ITRI, - Candidate #3: ETRI, CATT, NEC, Google, Huawei/HiSi, Apple, Ofinno, ZTE - Candidate #4: - Candidate #5: NTT DOCOMO, Futurewei, Huawei/HiSi, IDC, Ericsson, Ofinno, CATT - Candidate #6: ZTE, vivo, Qualcomm, ETRI, CATT, Google, Xiaomi (only for larger threshold), Huawei/HiSi, Sharp, CMCC, Apple, Ofinno, - Candidate #7: MediaTek (to capture #1 and #6), Qualcomm, Google, NTT DOCOMO, Panasonic, Nokia, Ericsson, Ofinno, ZTE (to capture #1 and #6) - No additional conditions: OPPO, Samsung, FL Note: While considering the pending issue as mentioned in Issue 1.1, wrapping up the discussion/the following proposal may depend on the final decision on Issue 1.1. Proposal 1.2: Regarding triggering event determination for Event 2, besides for Candidate#2, at least Candidate #1 and Candidate#7 are additionally supported for resetting the counting. - Candidate#1: RS reconfiguration/update or MAC-CE signaling (if supported) for new beam is received; - FFS: whether to reset the counting for all new beams. - FFS: whether to maintain the counting whose new beam is NOT updated. - Candidate#7: The RRC parameter(s) associated with the CSI report configuration for UEI beam report is reconfigured. - The RRC parameter(s) at least comprises the threshold for event evaluation - In such case, the UE need to reset the counting for all new beams. - FFS: other parameters, e.g., the length of the time window or maximum counting number M. Supported by: ZTE, vivo, Qualcomm, ETRI, CATT, Google, Futurewei, Panasonic, Huawei/HiSi, Sharp, Spreadtrum, Fujitsu, CMCC, IDC, Lenovo, Xiaomi (only for larger threshold, MediaTek (to capture #1 and #6), NTT DOCOMO, Panasonic, Nokia, Apple, Ericsson, Ofinno, ITRI, Qualcomm, NEC, LGE, Ericsson, IDC (Open), Not supported by: OPPO (okay for Candidate#7 w.o FFS), Samsung, Nokia (Candidate#7), Apple, |
| 1.3 | RRC candidate values on event-detection | FL Note: Per companies input, we need to handle the following pending candidate values for RRC parameter on event detection. Proposal 1.3: Regarding triggering event determination, on candidate values of supported RRC parameters, - Down-select one of the following as RRC candidate values for a threshold value eventThreshold-r19 for trigger event detection regarding Event-2 or Event-7. - Option-1: 0, 1, 2, 3, …, 15 - Option-2: 0, 1, …, 30 - … - Down-select one of the following as RRC candidate values for a threshold value eventThreshold-Event1-r19 for trigger event detection regarding Event-1. - Option-1: reusing RSRP-Range in RRC - Note: only values 14, …,113 in RSRP-Range are valid - … - Down-select one of the following as RRC candidate values for the time window length for triggering event determination eventDetectionTimeWindowLength-r19 - Option-1: 4ms, 5ms, 8ms, 10ms, 16ms, 20ms, 40ms, 80ms, 160ms, 320ms, 640 ms, 1280ms, - … - Down-select one of the following as RRC candidate values for the counting threshold eventInstanceCount-r19 - Option-1: 1, 2, …, 16 - … Support by Nokia (except of 1 in eventInstanceCount-r19), FYI, the following is the definition of RSRP-Range The IE RSRP-Range specifies the value range used in RSRP measurements and thresholds. For measurements, integer value for RSRP measurements is according to Table 10.1.6.1-1 in TS 38.133 [14]. For thresholds, the actual value is (IE value – 156) dBm, except for the IE value 127, in which case the actual value is infinity. RSRP-Range information element -- ASN1START -- TAG-RSRP-RANGE-START RSRP-Range ::= INTEGER(0..127) -- TAG-RSRP-RANGE-STOP -- ASN1STOP. |

Table 1-2 Company input for Issue 1

| Company | Input |
| --- | --- |
| Mod V00 | - Please input your comment/preference to those proposals and questions on the issue 1.1~1.3, if needed. |
| OPPO | Re 1.1A vs 1.1B: we support the proposal 1.1B because the wording is clearer and more aligned with the agreement. Re proposal 1.2: Candidate #1 does not need to be supported. RRC reconfiguration for new beam set generally includes two action: removing one from the set or adding one to the set. If the action is removing one, the timer for that removed one would be deleted naturally. If the action is adding one, the timer for that newly added RS should be started naturally. Therefore, supporting Candidate#1 is not needed and no spec impact. For Candidate#7: we can be ok with it if there is not any FFS. We are in the last meeting of normative work. How can we still have FFS. [Mod]: Captured! |
| Ofinno | Support Proposal 1.1B According to the agreement in the previous meeting, triggering event determination is described as “UE starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window”. Therefore, the timer should be used as in proposal 1.1B. Support Proposal 1.2 in general For candidate #1, we do not see technical motivation to reset counting for all new beams, including unchanged new beams by RS reconfiguration/update. For those of beams not updated by RS reconfiguration/update, the counting should be maintained to reduce reporting delay which is motivation of agenda. [Mod]: As OPPO mentioned, after updating the CR draft, the candidate#1 may be handled automatically! For candidate #7, it is our understanding that the motivation is, RRC parameter reconfiguration may impact on the change of triggering condition, therefore resetting the counting is required. Accounting for that, especially the length of the time window and maximum counting number should be included. Those parameters have a direct impact on the triggering condition and when they are not considered, ambiguity in UE behavior can occur. For example, when the length of the time window is shrunk and/or maximum counting number is reduced by reconfiguration, compared to the current time window and the counting number, whether UE should transmit first PUCCH or not is ambiguous. To prevent such, time window and the maximum counting number M should be accounted for. And still, we think candidate #3, #5 should be supported to capture good quality of the beam accurately. [Mod]: Captured! |
| Google | Proposal 1.1A and Proposal 1.1B: We support Proposal 1.1B in general. Proposal 1.1A does not align with the agreement we made in last meeting to adopt Option-4 (timer based). In addition, if we would like to mention counter explicitly in Proposal 1.1B, we should also have similar wordings for Proposal 1.1A, shouldn’t we? For both proposals, UE would set counter to 1, increment counter by 1 and reset counter. [Mod]: Captured! |
| Samsung | Issue 1.1: We provide our TP for this issue as follows: “ if eventDetectionTimeWindowLength-r19 is provided, and the time window for such reference signal has not started yet or has ended, the UE starts determines a time window of a length eventDetectionTimeWindowLength-r19, if provided, where the time window starts at the beginning of a transmission occasion of for such reference signal for which an event instance is determined, wherein the time window does not overlap with other time windows for such reference signal. ” Our proposed TP clearly clarifies/captures the following three points: - Point #1 (starting of a time window): starting of a time window (with a fixed length T_window) specific to a new beam RS of a CSI report configuration should correspond to start of a transmission occasion of the new beam RS that satisfies the event. Without the clarification, it is unclear what would be the exact condition/position the UE can start a time window – saying “time window starts or is started” is unclear/not enough. Besides, our proposed TP exactly follows RAN1’s previous agreement on event evaluation(s)/determination(s) for Event-2. - Point #2 (multiple non-overlapping time windows for a same new beam RS): based on the agreed Option-4, multiple time windows for the same new beam RS can be determined, and they are not allowed to be overlapping in time domain – this is according to the restriction of “the timer for the new beam is not running” in the agreement. We provide conceptual examples illustrating determination of the time window(s) for a given new beam RS (i.e., RS #1) in the above figure, which shows that, there can be multiple time windows before a PUCCH transmission occasion – we highlight in green a TO of RS #1 that determines an event instance, otherwise a TO of RS #1 does not determine an event instance. As it is possible that the number of determined event instances within a time window can be smaller than M, for the case (a) with M=3, the UE cannot determine the event based on the first time window (i.e., Window #1) – the number of the determined event instances within Window #1 is 2, which is less than M=3. On the other hand, the second time window (i.e., Window #2) has 3 determined event instances, and the UE can determine the event based on Window #2. - Point #3 (condition(s) for sending UEIRI via first PUCCH): For another case (b) depicted in the above figure, following those described in the case (a), the UE can determine the event for the first time window (i.e., Window #1) but not the second time window (i.e., Window #2) – in this case, the UE would store the event evaluation result(s) before determining whether a PUCCH can be transmitted to initiate beam reporting. We think case (b) is valid: though the UE can determine the event based on one time window, the UE is still allowed to start another subsequent time window to (i) assess whether the event condition(s) is still satisfied or not, and/or (ii) obtain the latest measurement results that satisfy the event condition(s). The UE can store the event evaluation/determination results specific to different time windows for a same new beam RS, upon/on which the UE can determine whether or not to transmit UEIRI via the first PUCCH, which is beneficial for overhead reduction to say the least. The above three critical points are not captured by the TP in Proposal 1.1B or not fully revealed by the TP in Proposal 1.1A (regardless of whether a timer or time window is used in the corresponding descriptions). We also need to keep in mind that time windows for event evaluation can be determined for multiple new beam RSs – in this case, without clarifying Point #1, Point #2 and Point #3, subsequent UE’s operation/behaviors would become convoluted. We would also like to highlight that if Point #1, Point #2 and Point #3 are not clearly captured/clarified, RAN4’s corresponding discussions on requirements for Option-4 based event evaluation will also be affected. [Mod]: Captured! Proposal 1.2: This issue is not essential – other candidates do not have immediate impact on event evaluation. Some details of Candidate #2 are still missing, which need to be addressed first. Proposal 1.3: Prefer to take care of this in RRC discussions. [Mod]: RRC candidate value, if not being straightforward, should be handled in RAN1 normal discussion. |
| Xiaomi | Issue 1.1: We prefer to go with the direction of proposal 1.1B. One point to be clarified is that if the counter is equal to eventInstanceCount-r19 before the timer expired, UE will transmit PUCCH format 0 or format 1 before or after the time expired. If UE can transmit PUCCH format 0 or format 1 before the time expired, when to stop the timer? And regarding the PUCCH resource, we suggest to add some restriction that the PUCCH resource is the PUCCH resource no later than Tgap after the last determined event instance before the timer expired. Otherwise, it means the beam report was triggered long time before and it is not necessary to report it because of outdated. 5.2.1.5.4.1 UE Initiated CSI reporting For a UE configured with a CSI-ReportConfig with eventType-r19 for periodic reference signals [with the same periodicity configured by the newBeamResourceSet-r19] and with dl-OrJointTCI-StateList, when an event instance is determined (as described in the following clauses), - for a reference signal configured by the newBeamResourceSet-r19, if eventType-r19 is set to ‘event2’ or ‘event7’, or - for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI, if eventType-r19 is set to ‘event1’, [if eventDetectionTimeWindowLength-r19 is provided, the UE starts a timer, provided by eventDetectionTimeWindowLength-r19, window for such reference signal, and sets the corresponding counter to 1, if the timer is not running; otherwise, if the timer is already running, the UE increments the counter for such reference signal by 1. wherein the time window does not overlap with other time windows for such reference signal.] When the number of event instances determined counter for such reference signal within the time window, if before the corresponding timer, if applicable, is expired eventDetectionTimeWindowLength-r19 is provided, is greater than or equal to 1 or to eventInstanceCount-r19, if provided, the UE transmits PUCCH format 0 or format 1 in the PUCCH resource no later than Tgap after the last determined event instance before the timer expired and configured by PUCCHResource-r19 in the CSI-ReportConfig. [The counting counter of the event instances for such reference signal is reset restarted and the timer for such reference signal is stopped: - if the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state is updated.] The UE does not expect that a CSI trigger state associated with CSI report configuration(s) configured with the higher layer parameter eventType-r19 is further associated with other CSI report configurations that are not configured with the higher layer parameter eventType-r19.. [Mod]: Clearly, we do not have such agreement for now. Let’s make the update per agreement, please. Issue 1.2 Regarding candidate#1, suggest to solve the FFS as below: If the RS type is updated, reset the counting for all new beams. Else, maintain the counting whose new beam is not updated. |
| ZTE | Issue 1.1: Support to clarify event-detection by “timer/counter”, e.g., Proposal 1.1B. As per the latest agreement reached in RAN1#120b meeting, the term of “timer” in Option-4 was endorsed to replace “time-window” in previous agreement for specifying event detection in RAN1 specifications. Note that the mechanism/criteria of “timer+counter” has already been typically and broadly used for event related features in RAN2 specifications, that is what RAN1 can refer to or learn from. In particular, it seems somehow agnostic/difficult to capture the behaviors of “running” and “expiry” for “time-window” in the specs. Proposal 1.2: Support Candidate#7 and Candidate#3. Basically, we think the UE needs to reset counter of all new beams as long as the corresponding CSI report configuration for UEI beam report is reconfigured, i.e., Candidate#7. In this sense, Candidate#1 and Candidate#6 are included accordingly. Furthermore, if counting is not reset after transmission of the UEI beam report as in Candidate#3, UE may redundantly trigger the UEI beam report again before expiry of the timer, which should be avoided. Thus, Candidate#3 should be supported as well. Consequently, we suggest the following changes of Proposal 1.2. Proposal 1.2: Regarding triggering event determination for Event 2, besides for Candidate#2, at least Candidate #1#3 and Candidate#7 are additionally supported for resetting the counting. - Candidate#1: RS reconfiguration/update or MAC-CE signaling (if supported) for new beam is received; - FFS: whether to reset the counting for all new beams. - FFS: whether to maintain the counting whose new beam is NOT updated. - Candidate#3: UEI beam report is transmitted; - Candidate#7: The RRC parameter(s) associated with the CSI report configuration for UEI beam report is reconfigured. - The RRC parameter(s) at least comprises the threshold for event evaluation - In such case, the UE need to reset the counting for all new beams. - FFS: other parameters, e.g., the length of the time window or maximum counting number M. [Mod]: Noted! As mentioned to OPPO, we may consider the update per outcome of Issue1.1. Proposal 1.3: Prefer to address such issues in RRC discussions. [Mod]: RRC candidate value, if not being straightforward, should be handled in RAN1 normal discussion. |
| Spreadtrum | Proposal 1.1A and Proposal 1.1B: Prefer Proposal 1.1B to align with the agreement. Proposal 1.2: Support. For Candidate #1, in case of reconfiguration/update for some new beam RS, there is no need to reset the counting whose new beam is not reconfigured/updated, because the Event-2 instance counting is per new beam. These new beam RSs need to continue to be measured for event evaluation based on the corresponding counters. In addition, the behavior of MAC-CE based new beam RS update should be removed from Candidate #1. [Mod]: Captured! |
| MediaTek | Proposal 1.1A/1.1B: Support Proposal 1.1B with updates We shared the same view with the above companies that counter/timer-based operation more align with the agreement. Meanwhile, we’d like to suggest update the text proposal according to the following comments: - According to R1#117 agreement, counter/timer will be inapplicable for UE basic feature which should not have counter/timer-based UE behavior. - For better readability, we prefer to specific UE basic feature and counter/timer-based feature in two individual paragraphs. Then, those if condition of whether eventDetectionTimeWindowLength -r19 is provided are not needed. - The wording of “the timer provided by eventDetectionTimeWindowLength -r19” is unclear. It will be better to say that “UE starts the timer from the value according to eventDetectionTimeWindowLength -r19”. Note that the timer here is interpreted as a count-down operation (i.e., starting from eventDetectionTimeWindowLength -r19 and expiring at zero). - We prefer to clearly specify the counter will be reset to zero for avoiding ambiguity. [Mod]: Good comments! Let’s fix this issue pre existing agreement. Hence, we suggest the updated Proposal 1.1B as follows: 5.2.1.5.4.1 UE Initiated CSI reporting (UE basic feature) For a UE configured with a CSI-ReportConfig with eventType-r19 for periodic reference signals [with the same periodicity configured by the newBeamResourceSet-r19] and with dl-OrJointTCI-StateList and without eventDetectionTimeWindowLength -r19, when an event instance is determined (as described in the following clauses), - for a reference signal configured by the newBeamResourceSet-r19, if eventType-r19 is set to ‘event2’ or ‘event7’, or - for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI, if eventType-r19 is set to ‘event1’, the UE transmits PUCCH format 0 or format 1 in the PUCCH resource configured by PUCCHResource-r19 in the CSI-ReportConfig. (Couter/timer-based feature) For a UE configured with a CSI-ReportConfig with eventType-r19 for periodic reference signals [with the same periodicity configured by the newBeamResourceSet-r19] and with dl-OrJointTCI-StateList and with eventDetectionTimeWindowLength -r19, when an event instance is determined (as described in the following clauses), - for a reference signal configured by the newBeamResourceSet-r19, if eventType-r19 is set to ‘event2’ or ‘event7’, or - for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI, if eventType-r19 is set to ‘event1’, the UE - starts a timer for such referenece signal from the value according to eventDetectionTimeWindowLength-r19 and sets the counter for such reference signal to 1, if the timer is not running; - increments the counter for such reference signal by 1 if the timer is running. If the timer is running, and the counter for such reference signal is greater than or equal to eventInstanceCount-r19, if provided, the UE transmits PUCCH format 0 or format 1 in the PUCCH resource configured by PUCCHResource-r19 in the CSI-ReportConfig. The counter for such reference signal is reset to zero and the timer for such reference signal is stopped: - if the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state is updated. The UE does not expect that a CSI trigger state associated with CSI report configuration(s) configured with the higher layer parameter eventType-r19 is further associated with other CSI report configurations that are not configured with the higher layer parameter eventType-r19.. [Mod]: Done! Proposal 1.2: Support We think Candidate#1 is necessary to specify that UE needs to not only reset the counter but also stop the timer for a removed new beam RS according to RRC reconfiguration. Otherwise, the timer will keep running and the counter will remain non-zero value which may trigger reporting. On whether to keep counting for the unchanged new beam, we prefer to reset the counting and stop the timers for all the new beams regardless that the corresponding new beam RS(s) is newly removed or unchanged, since the RRC configuration structure of new beam RS resource set is not friendly to make UE identify which new beam RS(s) has been removed/unchanged. On Candidate#7, besides of event threshold, the number of M and the window length for counting evaluation if enabled also triggering event determination for any new beam RS or the current beam RS. Form our view, RRC parameter in Candidate#7 compromises event threshold and the number of M and the window length if provided and UE needs to reset the counters and stop the timers for all the new beams when Candidate#7 is met. Proposal 1.3: We also prefer to handle this in RRC discussions. [Mod]: RRC candidate value, if not being straightforward, should be handled in RAN1 normal discussion. |
| Mod_V09 | Updated, especially for Proposal 1.1A/B based on Samsung, MediaTek and other companies’ input. For Issue 1.2, we may fully review that after Proposal 1.1A/B is agreed. @all, let’s focus on the candidate value for the RRC parameter. While considering that RRC candidate value, if not being straightforward, should be handled in RAN1 normal discussion. |
| Nokia | Issue 1.1 We do not think this is procedurally OK as we are not at the stage of discussing TPs! We are fine to discuss some terminology issues on the other hand. TPs would be discussed once indicated by Mr chairman, very likely sometime in maintenance stage! [Mod]: While having a list of agreement reached in different phases, per my assessment, the proposal is necessary for clarifying the UE behavior. Proposal 1.2 We do not support the proposal in the current form. We have already Candidate#2 as a method to reset the counter, and that was in our view necessary for Event-2. Regarding Candidate#1, in its current form, it is not necessary: when the set of new beams is updated, for the beams that are removed the counter is clearly reset (as they are not in the set of new beams), for the beams that are added, the counter starts from the initial value, for example 0 (and it cannot be otherwise). Among all the other remaining Candidates, the only one that we could support is Candidate#7. In our view, updating the CSI report configuration is a form of implicit acknowledgement w.r.t. the fact that the gNB took a decision regarding the future operations. However, the sub-bullet points on which RRC parameters should be compared is not needed. Those would unnecessarily over-complicate the solution. It is a much leaner solution to simply reset the counter as soon as the CSI report configuration is updated. Furthermore, we want to raise an observation regarding Candidate#2. The agreed Candidate#2 solution for resetting the counter for Event-2 when the current beam associated to the indicated TCI state is updated is applicable to Event-1, but it is not applicable/helpful for Event-7. The reason is simply that the current beam in Event-7 is associated to the activated TCI state with the Q-th best quality (and not to the indicated TCI state as in Event-2 and Event-1). The issue is that, while for Event-2 the current beam is a) uniquely identified by the indicated TCI state and b) can be fully and directly controlled by the network for example updating the indicated TCI state using a DCI command, for Event-7 there is not such unique identification for the current beam, which then cannot be directly and uniquely updated/controlled by the network. The straightforward extension of Event-2 Candidate#2 to Event-7 is to simply consider resetting the counter when the activated TCI state list is updated, e.g., via MAC-CE. If we do not clarify such aspects, because of the logic that “The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise”, the agreed Candidate#2 in its current form is supposed to apply also to Event-7, which technically does not bring much. Therefore, we propose the following: - Candidate#2 applies only to Event-2 and Event-1; - Instead of Candidate#2, for Event-7 the counter for a new beam is reset when the activated TCI state list is updated, e.g., via MAC-CE. Proposal 1.3 Overall the proposed values are ok, but we think we could include larger values for eventDetectionTimeWindowLength-r19, and eventInstanceCount-r19 does not need to include the value “1”. Anyhow, we share the view of the other companies, this can be handled in the RRC discussion. [Mod]: Captured! |
| FUTUREWEI | Proposal 1.1B is preferred, as it aligns with the existing agreement to use a timer to limit the duration for counting event instances. Proposal 1.2: We are in general fine with the proposal. However, in addition to Candidate#1 and Candidate#7, Candidate#5 should also be supported. In Candidate#5, if the time window expires (or the timer corresponding to the time window expires), the UE resets the associated counting for new beams. That is, if Event 2 is not fulfilled when the timer expires, UE resets the associated counting for new beams. Otherwise, the evaluation outcome for Event 2 could be affected by the remaining counting value. [Mod]: Captured! |
| Huawei, HiSilicon | Proposal 1.1A/1.1B: We think Proposal 1.1B is a better starting point as the language closer to the agreement. However, this need further discussion. In particular, it is useful to have separate texts for the basic feature and non-basic feature (timer/counter-based). Proposal 1.2: Support. For candidate 1, our understanding is that RRC reconfiguration/update for new beam can only add or remove resources from the new beam resource set. If the new beam RS corresponding to the counter is removed from the new beam resource set, its corresponding counter should be reset and the counter whose corresponding new beam is not updated should keep counting. Also, we think both Candidate#3 and Candidate#5 should be supported: For Candidate#3, we think the counter of the beam that has triggered the report should be reset to zero to avoid the possibility of re-triggering the event immediately after UEI beam report is transmitted or, if supported, immediately after the prohibit timer expires. In turn, counters associated with other new beams should keep counting after UEI beam report since, if they trigger the event, the triggering beam in the corresponding report would be different from that of the earlier report. For Candidate#5, when a time window is expired, the counter corresponding to the associated new beam should be reset to zero since, according to the definition of Event-2, the report is triggered only when the number of event-2 instances within the time window reaches the threshold and, therefore, continue counting the event instances after the timer is expired is useless as, even if the counter reaches the RRC-configured threshold after the expiry of the timer, the report will not be triggered. Proposal 1.3: - eventThreshold-r19: suggest Option-2: 0,1,…,30. Value Range for RSRP offset values MeasTriggerQuantityOffset for offset-based L3 events A1 and A3 is -30,…,30. Since, for Event-2 and Event-7 the event is triggered only if the measured beam is threshold value better than the current beam, the negative values MeasTriggerQuantityOffset don’t seem necessary. - eventThreshold-Event1-r19: According to Table 10.1.6.1-1 of 38.133, only 14,…,113 in the RSRP-Range are valid for L1-RSRP and the whole RSRP range of 0,…,127 is valid only for L3 RSRP. It is OK to reuse RSRP-Range for value range of eventThreshold-Event1-r19 but it should be mentioned that only values 14,…,113 are valid. - eventDetectionTimeWindowLength-r19: Suggest to also add 640 ms as 640 slots is the maximum periodicity of CSI-RS. It may also be a good idea to modify the time window length unit from “ms” to “slots” since the CSI-Rs periodicity unit is slot. [Mod]: For ms vs slot, we may quick check that during online. - eventInstanceCount-r19: OK. |
| Panasonic | Issue 1.1: If there is a different understanding among companies, we prefer to cast this as a proposal and not as a text proposal. The proposal would be up to the editor to capture. For example, we support clarifying points 1 , 2, 3 in Samsung’s comment. Proposal 1.2: We support the proposal. We still think though that candidate 1 is already supported following the definition of window from previous agreements. The same thing with candidate 7. But it is still clearer to reach an agreement on this. |
| NEC | Proposal 1.1B is aligned with agreement. Proposal 1.2: generally OK, we also support Candidate#3. |
| Fujitsu | Issue 1.1: Option 2 is preferred, i.e., by using “timer and counting”. The timer-related procedures are clear and accurate. As for Proposal 1.1B which implements Option 2, it is suggested to change “the timer for such reference signal is stopped” to “the timer for such reference signal is considered as expired”. If using “timer is stopped”, when the timer is started again, it does not start from the beginning. If using “timer is expired”, when the timer is started again, it will start from the very beginning and thus it is aligned with the design intention. [Mod]: Let’s check companies input. Per my knowledge, ‘be stopped’ is per 321 description. Proposal 1.2: Support. |
| Lenovo | Proposal 1.1A/B, we support 1.1B since it’s more aligned with the agreement. Proposal 1.2: Support. Proposal 1.3: Support. [Mod]: Captured! |
| Sharp | Proposal 1.1A/1.1B: We prefer Proposal 1.1A, which is simpler than Proposal 1.1B. In our view, both proposals are technically same. Proposal 1.2: Support. In our understanding, since event instances are counted within each time window, Candidate #5 is automatically applied, right? [Mod]: Captured! |
| NTT DOCOMO | Issue 1.1: We think that Proposal 1.1B is more aligned with the previous agreement. If there are any unclear part in the previous agreement, separate discussion is preferred. Proposal 1.2: Support in principle. [Mod]: Captured! |
| Samsung2 | We would like to further elaborate our views on Point #2 (multiple non-overlapping time windows for a same new beam RS) in our earlier reply based on comments from companies. First of all, “There is only 1 timer per new beam” does not necessarily mean that the UE cannot start multiple time windows for event evaluation/determination. For example, in cell DTX, there is ONLY ONE timer, i.e., cellDTX-DRX-onDurationTimer, specified/defined in the MAC spec., but there can be multiple active periods (equivalent to time windows in the context of event evaluation here) specific to the timer when it is running. Below, we provide the corresponding descriptions (highlighted in red) in the TS 38.321; we also provide a corresponding conceptual figure to illustrate the above. ## 5.34 Cell-Level Energy Saving ### 5.34.1 General Each Serving Cell may be configured by RRC with a periodic cell DTX pattern (i.e., Active and Non-Active Periods). The cell DTX operation controls UE's monitoring activity of PDCCH and configured downlink assignments in RRC_CONNECTED. For all activated Serving Cells with cell DTX configured and activated, the MAC entity may monitor PDCCH and configured downlink assignments using the cell DTX operation specified in clause 5.34.2. … RRC controls cell DTX and cell DRX operation by configuring the following parameters in cellDTX-DRX-Config per Serving Cell: - cellDTX-DRX-ConfigType: defines whether only cell DTX is configured, only cell DRX is configured, or both are configured; - cellDTX-DRX-onDurationTimer: the active duration at the beginning of a cell DTX/DRX cycle; - cellDTX-DRX-StartOffset: defines the subframe where the cell DTX/DRX cycle starts; - cellDTX-DRX-SlotOffset: the delay before starting the cellDTX-DRX-onDurationTimer; - cellDTX-DRX-Cycle: the cell DTX/DRX cycle period. - cellDTX-DRX-ActivationStatus: the initial activation status of cell DTX and cell DRX operation. ### 5.34.2 Cell Discontinuous Transmission … When cell DTX is configured and activated for a Serving Cell, the cell DTX Active Period includes the time while: - cellDTX-DRX-onDurationTimer is running for the associated Serving Cell.. Back to our timer/time window determination, our understanding is that even if there is only one timer per new beam, when the timer is running, it still can have or determine multiple time windows – similar to the example of cell DTX as explained above. Again, using the conceptual example presented above (also in our earlier comments), the timer can start at the 1st TO and ends at the 3rd TO (corresponding to a time window); the (same) timer can restart at the 5th TO and ends at the 7th TO (corresponding to another time window). Again, we are open for discussions (terminology etc.) if Point #1, Point #2 and Point #3 in our previous comments are clearly captured, which is not the case even for the updated timer/counter TP. We would also like to reiterate that level of clarity of our discussions/outcome would also affect RAN4’s corresponding discussions on requirements for Option-4 based event evaluation. [Mod]: Thanks for being flexible. Per my assessment, we may firstly make a clear TP which is only based on the current agreement, and then, we can make further discussion or other detailed scenarios after that! That is not only for the above three points and also for other companies’ preference. |
| Qualcomm | Issue 1.1: We support Proposal 1.1B in principle, but want to raise a small concern regarding the phrase “If the timer is running”. Our understanding is that “If the timer is running” is intended to qualify the condition “the counter for such reference signal is greater than…”. However, the current wording could be misinterpreted to mean that “the UE transmits PUCCH format 0 or format 1 in the PUCCH” only when the timer is running. To avoid this potential confusion, our suggestion is moving the entire sentence to follow the description of the counter reset condition. Since it’s already clear that the counter only increments while the timer is running—and not when it’s reset—there may be no need to explicitly state “If the timer is running” again. 5.2.1.5.4.1 UE Initiated CSI reporting For a UE configured with a CSI-ReportConfig with eventType-r19 for periodic reference signals [with the same periodicity configured by the newBeamResourceSet-r19] and with dl-OrJointTCI-StateList and without eventDetectionTimeWindowLength-r19, when an event instance is determined (as described in the following clauses), - for a reference signal configured by the newBeamResourceSet-r19, if eventType-r19 is set to ‘event2’ or ‘event7’, or - for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI, if eventType-r19 is set to ‘event1’, the UE transmits PUCCH format 0 or format 1 in the PUCCH resource configured by PUCCHResource in the CSI-ReportConfig. For a UE configured with a CSI-ReportConfig with eventType-r19 for periodic reference signals [with the same periodicity configured by the newBeamResourceSet-r19] and with dl-OrJointTCI-StateList and with eventDetectionTimeWindowLength-r19, when an event instance is determined (as described in the following clauses), - for a reference signal configured by the newBeamResourceSet-r19, if eventType-r19 is set to ‘event2’ or ‘event7’, or - for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI, if eventType-r19 is set to ‘event1’, [if eventDetectionTimeWindowLength-r19 is provided, the UE - starts a timer for such reference signal from the value provided by eventDetectionTimeWindowLength-r19 and sets the counter to 1, if the timer is not running; or - increments the counter for such reference signal by 1 if the timer is already running. wherein the time window does not overlap with other time windows for such reference signal.] When the number of event instances determined for such reference signal within the time window, if eventDetectionTimeWindowLength-r19 is provided, is greater than or equal to 1 or to eventInstanceCount-r19, if provided, If the timer is running and the counter for such reference signal is greater than or equal to eventInstanceCount-r19, the UE transmits PUCCH format 0 or format 1 in the PUCCH resource configured by PUCCHResource-r19 in the CSI-ReportConfig. [The counting counter of the event instances for such reference signal is reset to zero restarted and the timer for such reference signal is stopped: - if the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state is updated.] If the counter for such reference signal is greater than or equal to eventInstanceCount-r19, the UE transmits PUCCH format 0 or format 1 in the PUCCH resource configured by PUCCHResource-r19 in the CSI-ReportConfig. The UE does not expect that a CSI trigger state associated with CSI report configuration(s) configured with the higher layer parameter eventType-r19 is further associated with other CSI report configurations that are not configured with the higher layer parameter eventType-r19.. [Mod]: I can understand your intention of this update, but which is also relevant to the subsequent discussion of whether/how to introduce a state/pending for keeping this memory. Even if having this update, we still have left-over issue(s), e.g., initially, the counter was great than a threshold, but, for a first PUCCH transmission occasion, the counter is set to 1 while timer is running again. So, per the following agreement, I think the current version seems okay. Then the rest can be left to further discussion as I replied to Samsung. [117] Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. · Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS : Whether the above is captured in RAN1 or RAN2 specification. Proposal 1.2: We support the proposal. |
| HONOR | Proposal 1.1B is more aligned with previous agreements. This is not very essential issue to be discussed in this meeting. TP can be discussed in maintenance phase. Proposal 1.2: Support in principle to better clarify the UE behaviors. |
| vivo | Issue 1.1: As mentioned Samsung and Nokia, some details about the event evaluation procedure are not clear and more discussion is required to align understanding. Before discussing exact wording of the TP, it is important to clarify the unclear points first. For the Point 3 mentioned by Samsung, to reflect beam situation in time, whether the first PUCCH indication corresponds to a report is transmitted or not should depend on the event evaluation results within the latest time window for a new beam RS before the PUCCH resource, if multiple time windows for the new beam RS exist between PUCCH transmission occasion. As shown in the figure (a), even if new beam RS#2 meets the reporting trigger condition within the first time window (i.e., the number of event instance is equal to M (i.e.,3), the 1-bit PUCCH indication will not be transmitted as the beam quality of the new beam RS becomes poor and only one event instance occurs within the second time window. For the case of multiple new beam RSs, as shown in Figure (b), the 1-bit PUCCH indication corresponds to the CSI report associated with new beam RS#1 and new beam RS#2 will be transmitted because new beam RS#2 meets the reporting trigger condition within the second window (i.e., the number of event instance is equal to M (i.e.,3). Beside time-window based event evaluation procedure, the above mechanism should be also applied for the basic feature (i.e. without time window configured). Thus, whether the reporting trigger condition is met or not is determined based on the measurement results obtained by the latest RS occasion of the current beam and new beam before PUCCH. Figure (a) Figure (b) In addition, we also think the basic feature and time-window based event evaluation should be captured separately, as the definition of time window is not needed for the basic feature. Therefore, the wording in Proposal 1.1A should be revised as Proposal 1.1B. [Mod]: Highly appreciated if having your proposed TP. But, as I mentioned to Samsung and QC, we only focus the current agreement firstly. Proposal 1.2: For Candidate#1, we think the following case should be clarified. As shown in the following figure, RRC reconfiguration of new beam RS set is applied after the time window, in which the new beam RS#1 meets the reporting trigger condition, before the first symbol of the PUCCH resource. After the RRC configuration, the new beam RS#1 is absent in the new beam RS set. In this case, if counting value of new beam RS#1 is not reset, whether the 1-bit PUCCH indication should be transmitted? Proposal 1.3: For the candidate value of the threshold for event, we think reusing RSRP-range without the negative values is fine. [Mod]: Captured! |
| Ericsson | Support proposal 1.1B, which is aligned with the agreement. Proposal 1.2: Support. We think that the RS configuration is part of the RRC parameter(s) associated with the CSI report configuration, so candidate #1 is a subset of candidate #7. What is the motivation not to reset? If the NW reconfigures parameters that controls the UEIBM, why should the UE continue the evaluation using the old parameters? Proposal 1.3: Support for the thresholds. For timer, we need larger values, due to the Rx beam sweeping in the UE. Propose to add {640ms, 1280ms} [Mod]: Captured! |
| IDC | Issue1.1: Support Proposal 1.1B in principle, which is aligned with the agreement. Proposal 1.2: Support Candidate#1 and Candidate#5. We think Candidate#7 needs more discussions, as it’s not clear which RRC parameter(s) exactly refers to. Since Proposal 1.2 captures more details on Candidate#7, we’re open for further discussions based on Proposal 1.2. [Mod]: Captured! |
| Apple | Proposal 1.1 and 1.1B Prefer P1.1B as timer is widely used to capture this window-based procedure in 3GPP spec and also used in implimentation. However, for the following part in P1.1B: [The counting time window of the event instances for such reference signal is stopped restarted]: - if the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state is updated.]. It does not precisely capture the following agreement in our view. Agreement Regarding triggering event determination for Event 2, on resetting the counting, the following modification on the agreed Candidate #2 in RAN1#120 is supported. - Candidate#2: [The measured current beam RS is updated based on] indicated TCI state is updated; - In such case, the UE needs to reset the counting for all new beams. - FFS: Further details on the update (if necessary). For instance, consider a scenario where the UE is configured with three TCI-states: - TCI-state #1: QCL Type-D with SSB#1 - TCI-state #2: QCL Type-D with CSI-RS #1 - TCI-state #3: QCL Type-D with CSI-RS #2 Both CSI-RS#1 and CSI-RS#2 are assigned ‘TCI-state#1’, i.e., QCLed with SSB#1. The indicated TCI-state is TCI-state #2. If SSB#1 serves as measurement RS based on NW configuration and indicated TCI-state is updated from TCI#2 to TCI#3, the counter and window assocaited with ‘SSB#1’ should NOT be reset, as per the RAN1 agreement, since SSB#1 remains measurement resource, even the indicated TCI-state is updated from #2 to #3. However, according to the wording in P1.1B, the event instance for ‘SSB#1’ is stopped, because it meets the condition: ‘if the reference signal in the indicated TCI state is updated’ and the word ‘or’ is used. However, the resetting should not occur based on the RAN1 agreement, since‘the measured current beam RS’ (i.e., SSB#1) remains unchanged. Therefore, the following is proposed to capture RAN1 agreement: [The counting time window of the event instances for such reference signal is stopped restarted]: - if the reference signal is the CSI-RS resource configured in the indicated TCI state and the indicated TCI-state is updated, or - if the reference signal is a SS/PBCH block which is QCLed with the CSI-RS resource configured in the indicated TCI state and SS/PBCH block is updated due to the indicated TCI-state is updated.]. [Mod]: Per my understanding, the original description is to provide the same description as you proposed. Then, let’s check other’s input. With this wording, UE will continue the time window for SSB, even if the TCI-state is updated from TCI#2 to #3, ensuring alignment with the RAN1 agreement. Proposal 1.2: Not support. We support Candidate #1. In addition, we think Candidate #3 and #6 should be supported. However, Candidate #7 should not be supported. as it is essentially a ‘blank check’. The ‘CQI-reportConfig’ RRC message contains many IEs, some of which are irrelevant to the UEIBR report. Using ‘any RRC parameter assocaited with CSI report configuraton for UEIBR’ is not feasible and inaccurate. Instead, the discussion should focus on specific IEs of the UEIBR report e.g., threshold, window size or M value, etc. to propertly justify the need for resetting. Proposal 1.3: Ok as starting point. |
| Mod_V30 | Updated in red per companies input. |
| Transsion | Proposal 1.1B: Support Proposal 1.1B as it aligns with the agreement. Proposal 1.2: Support. Proposal 1.3: Support. |
| Mod_V32 | Capture companies’ views. |
| ETRI | Proposal 1.1: We think 1.1B should be more aligned with the agreement. Proposal 1.2: Our view is properly captured. |
| Mod_V34 | Capture companies’ views. |
| Samsung3 | Thanks for further discussions and FL’s efforts. To us, timer in Option-4 is just one representation of the time window in the corresponding agreements – highlighted below: The most important issue should be to clarify when the UE would start the timer/time window and the corresponding assumptions, which is what we are always targeting in previous discussions; we cannot agree with companies’ comments that timer is a replacement of the time window operation (as see in the agreement of Option-4, we still have time/measurement window in the main sentence of descriptions). Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification. Agreement (120b) On UE-initiated/event-driven beam reporting, support the following interpretation on each of the codepoints of the ‘1-bit’ condition met indicator as follows: - ‘0’ – indicating that the Event-2 instances for corresponding CRI/SSBRI within the time window doesn’t reach the configured number M. - ‘1’ – indicating that the Event-2 instances for corresponding CRI/SSBRI within the time window reach the configured number M. FFS: whether/how to introduce the ‘1-bit’ condition met indicator for Event 7. Agreement (120b) Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, support Option-4. - Option-4: If an Event-2 instance for a new beam is obtained at the time t and the timer for the new beam is not running, UE starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window) - Note: Timer is new beam specific. - Note: T_window is the agreed time window parameter for measurement. - Introduce separate UE capability to limit the number of timers. There is only 1 timer per new beam. Above agreement is captured in RAN1 specifications. Even if a timer needs to be defined, effectively the timer would still have time window(s) in which the corresponding UE’s operations are specified – as commented before, this is similar to cell DTX where not only a (single) timer is defined but also the corresponding activate period(s) is used/applied to characterize the corresponding operation(s) – provided below: NOTE 1: A timer is running once it is started, until it is stopped or until it expires; otherwise it is not running. A timer can be started if it is not running or restarted if it is running. A Timer is always started or restarted from its initial value. The duration of a timer is not updated until it is stopped or expires (e.g. due to BWP switching). When the MAC entity applies zero value for a timer, the timer shall be started and immediately expire unless explicitly stated otherwise. When cell DTX is configured and activated for a Serving Cell, the cell DTX Active Period includes the time while: - cellDTX-DRX-onDurationTimer is running for the associated Serving Cell.. If companies really would like to capture the timer operation, e.g., start of the timer or timer is (not) running, at least we should follow similar ways to CDTX. Based on the above, we can compromise to have the following TP. Further note that Option-4 does not say anything about incrementing a counter – to our understanding, how to increment counting of the event instance(s) should follow the corresponding mechanism(s) in earlier agreements, and exact means of the counting (e.g., start from 1 etc.) can be up to UE’s implementations: “ if eventDetectionTimeWindowLength-r19 is provided, and the time window for such reference signal has not started yet or has ended, the UE starts a time window a timer of a length eventDetectionTimeWindowLength-r19, if provided, at the beginning of a transmission occasion of for such the reference signal for which an event instance is determined, if the timer is not running; the time window includes the time while the timer is running wherein the time window does not overlap with other time windows for such reference signal.” ” |
| China Telecom | Issue 1.1: We prefer Proposal 1.1B. Proposal 1.2: Support. |
| Mod_V38 | Capture companies’ views. |

## Issue 2 – UL signaling content(s)

Table 2-1 Summary for Issue 2

| # | Issue | Companies’ view and Recommended Proposal |
| --- | --- | --- |
| 2.1 | L1-RSRP report format —— any critical update for supporting Event-7 | [120b] Agreement On UE-initiated/event-driven beam reporting, support the following interpretation on each of the codepoints of the ‘1-bit’ condition met indicator as follows: - ‘0’ – indicating that the Event-2 instances for corresponding CRI/SSBRI within the time window doesn’t reach the configured number M. - ‘1’ – indicating that the Event-2 instances for corresponding CRI/SSBRI within the time window reach the configured number M. FFS: whether/how to introduce the ‘1-bit’ condition met indicator for Event 7. Q2.1: Regarding Event-7, please share your views on the following aspects: - #1 Whether to introduce a new field of indicating the Q-th best activated TCI state for interpreting ‘current beam RSRP’ in the report format of Event-7. - #2 Whether/how to introduce the ‘1-bit’ condition met indicator for Event-7? FL observation: - #1 On introducing the new field of indicating the Q-th best activated TCI state for interpreting ‘current beam RSRP’ - Support: NTT DOCOMO, NEC, Ofinno, ZTE, MediaTek, Futurewei, NEC, CATT (open), Lenovo, KDDI, vivo, Ericsson, IDC, Apple, ETRI, - Not support: Spreadtrum, Nokia, Panasonic, LG, Qualcomm, HONOR, Transsion, China Telecom, - #2 On introducing the ‘1-bit’ condition met indicator for Event-7 - Support: Huawei/HiSi’, Nokia, Qualcomm, vivo, Spreadtrum, CMCC, Google, xiaomi, ZTE, MediaTek, Futurewei, NEC, Fujitsu, Lenovo, NTT DOCOMO, HONOR (open), KDDI, Ericsson, IDC, Apple, Transsion, China Telecom, - Not support: Samsung, OPPO, Panasonic, CATT, ETRI |

Table 2-2 Company input for Issue 2

| Company | Input |
| --- | --- |
| Mod V00 | - Please input your comment/preference to those proposals and questions on the issue 2.1, if needed. |
| OPPO | #1 is needed but #1 is not enough. In addition to reporting Q-th best activated TCI state, the UE also need to report all the TCI states that has lower RSRP than the Q-th best activated TCI state. Only doing that can meet the motivation of Event 7. Without this reporting content, the event 7 would be totally useless and we can remove the supporting of event-7. [Mod]: Do sympathize you, but, unfortunately, it is precluded per last meeting agreement. Let’s focus interpret the meaning of current beam RSRP. #2: not needed if #1 is not supported. #1 is essential to event 7 but #2 is just something nice to have. |
| Ofinno | Q2.1: #1. The gNB may not have the information of the activated TCI state with the Q-th best quality as the activated TCI state with Q-th best quality is determined by the UE based on its own measurements, therefore it should be reported. #2. Support to introduce the ‘1-bit’ condition for event- 7, for consistency of the design. |
| Google | Q2.1: - For the #1 question, although we supported it in last meeting, it seems it is already concluded no consensus to support it. We do not see why this is brought out again. - [Mod]: To interpret the meaning of current beam RSRP. - For the #2 question, we support introducing the ‘1-bit’ condition met indicator for Event-7. Event-2 and Event-7 are almost twins in view of procedure. It is reasonable to have the condition met indicator to let NW know which reported beams actually satisfy the triggering event, especially when timer and counting are configured. |
| Samsung | #1 When the quality order of the activated TCI states varies within the time window for event evaluation, the UE may need to indicate, in a same report, more than one codepoint of the corresponding activated TCI state with the Q-th best quality that satisfies the event condition(s). As the number of indicated codepoint(s) could vary from one report to another, it is unclear how to construct the report format to avoid ambiguities of mapping(s)/correspondence(s) between different CSI fields, unless the UE only determines only one activated TCI state with the the Q-th best quality – for this case, M>1 for Event 7 should not be supported #2 As aforementioned, we don’t support M>1 for Event 7 because it could (1) increase the measurement latency, which is not aligned with the objective of the WID for latency reduction, and (2) complicate UE’s implementations. To elaborate: if counting more than one event instance is supported for Event-7, we need to first clarify whether the RS derived from the activated TCI state with the Q-th best quality is the same for each instance with respect to a same new beam considering that the channel conditions can vary across different evaluation instances. If we restrict a same RS derived from the activated TCI state with the Q-th best quality for event evaluation(s), it will inevitably lead to longer latency and thus against the objective of the WID for latency reduction; otherwise, the event evaluation procedure(s) would become very complicated, which is not friendly to UE’s implementations. |
| Xiaomi | #1: it was concluded in last RAN1 meeting that no consensus to support it. [Mod]: Please review my reply to OPPO and Google. #2: we are fine to support ‘1-bit’ condition met indicator for Event-7 to align the design of Event-2. |
| ZTE | Q2.1: For #1, as mentioned by FL, the indication of current beam in Event-7 seems needed to be reported. Otherwise, how to utilize the solely reported “current beam RSRP” is quite agnostic to NW. For #2, we think the ‘1-bit’ condition met indicator for Event-7 is needed due to (1) it is to align with the report format of Event-2 as per the previous agreement that “The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise”, and (2) this indication is necessary for NW to determine which of reported new beam(s) for update of activated TCI states can be used for beam application eventually, e.g., one indicated TCI state from the update of activated TCI states based on Event-7. [Mod]: Captured! |
| Spreadtrum | Q2.1: #1: Not support. In RAN#120bis, it was not agreed that the report format is extended, e.g. additional field to indicate the codepoint of the activated TCI state with Q-th best quality, or extending the maximum number of reported RS(s) to 8. When the quality order of the activated TCI states varies within the time window, it is up to UE implementation to choose the one corresponding to the Q-th best quality across the Event-7 instances. If Candidate #2 on resetting the counting for Event-2 is reused, the measured current beam RS should be based on the activated TCI state with Q-th best quality, and which activated TCI state is the Q-th best is determined by the UE. #2: Support introducing the 1-bit condition met indicator for Event-7 for unified design. [Mod]: Captured! |
| MediaTek | Q2.1: yes The codepoint is unnecessary information to be reported since the quality ranking of activated TCI state(s) is NW-transparent. We understand that the quality ranking of activated TCI states may vary during the timer is running, but we think that it can be up to UE implementation to determine which one activated TCI state should correspond to the Q-th best activated TCI state based on comprehensive quality evaluation during the timer is running. However, UE should ensure the same selected TCI state is used to determine the event instance(s) for new beam(s) during the running timer is running. Q2.2: yes if counter/timer-based evaluation is configured. [Mod]: Captured! |
| Mod_V09 | Capture companies’ views @all, for question 2.1#1, let’s focus on whether we need one more field of interpreting the ‘current beam RSRP’ in the report format. Indeed, we do not want to touch the last meeting conclusion! |
| Nokia | Q2.1 On aspect #1, although in RAN1#120bis we supported indicating the activated TCI state with Q-th best quality to the network in Event-7 report, as some other companies observed, we also reached a conclusion there stating that there was no consensus to support that, so not sure if this should be discussed again. On aspect #2, we support introducing the ‘1-bit’ condition met indicator also for Event-7, reusing the same design of Event-2. [Mod]: Captured! |
| FUTUREWEI | Q2.1: #1: Support to introduce a new field of indicating the Q-th best activated TCI state. Otherwise, NW lacks the information needed to determine which TCI state the reported current RSRP value corresponds to. #2: Support to introduce the ‘1-bit’ condition met indicator for Event-7, which is consistent with the agreement for Event-2. [Mod]: Captured! |
| Huawei, HiSilicon | Q 2.1: #1: Support. If the Q-th best activated TCI state is not indicated, gNB has no way to know the current beam for Event-7 and, consequently, it is not clear which activated TCI state(s) need to be updated. #2: Support. Similar to Event-2, this indicator needs to be included in the report to let the gNB know which reported beams satisfy the event condition and, therefore, are prime candidate to replace the current beam and which beams are just additionally reported. [Mod]: Captured! |
| Panasonic | Q.2.1 - #1 Whether to introduce a new field of indicating the Q-th best activated TCI state for interpreting ‘current beam RSRP’ in the report format of Event-7. Not support. We agree with what OPPO described. In this sense, we support the older proposal of “Extending the maximum number of reported RS(s) to 8”. - #2 Whether/how to introduce the ‘1-bit’ condition met indicator for Event-7? Not support. We think even for event 1 and 2, it is not clear how this bit is determined, in particular what time window is used to decide whether a beam satisfies the criterion or not. We explain this in our tdoc. We cannot support this before clarifying this issue first. [Mod]: Captured! |
| NEC | Q2.1 #1: Support, if companies have common understanding as FL that it is not excluded by conclusion from last meeting. #2: Support. [Mod]: Captured! |
| Fujitsu | Q2.1: #1: Not sure what’s the relationship with the conclusion in RAN1#120b. #2: Support. The reason is the same as that of Event 2 where such condition met indicator is supported. [Mod]: Captured! |
| LG | #1 We should not revisit this issue since this issue has been discussed and conclusion has been made. [Mod]: Captured! |
| CATT | Q2.1: #1: Open to discuss. #2: We prefer to not introduce the indicator for Event-7. [Mod]: Captured! |
| Lenovo | #1: Not needed. RAN1 has concluded that no consensus on this issue. On the other hand the followed NW and UE behavior is not clear at least for the case that Q=8 while only 4 new beams are reported in the beam report. #2: Fine with it to have a unified beam report format with event-2. [Mod]: Captured! |
| NTT DOCOMO | Q2.1 #1: Support the direction in principle to achieve usecase of Event-7. On the other hand, we think that it would be useful to report X RSs which is derived from the worst X current activated TCI states rather than current activated TCI state with Q-the best quality. The following figure is one example when Q=2 and X = 2 in case 4 new beams are reported. #2: Support. [Mod]: Captured! |
| Qualcomm | Q2.1: For Issue #1, although the discussion is focused on the interpretation of “current beam RSRP”, it appears to be similar in principle to the last meeting’s discussion, which concluded without consensus. Therefore, there is concern that reopening this discussion may lead to a repeated situation. As noted by a couple of companies (e.g., NTT Docomo and QC) in their contributions, there could be a method for the network to infer the TCI states to be updated (i.e., those with Q-th best quality and onward) from the reported CRI/SSBRIs, if they are included as one of the N reported beams. Therefore, we do not consider this a critical issue. For Issue #2, we support the additional indication due to the same reason that we support it for Event-2. [Mod]: Captured! |
| HONOR | Q2.1: it’s precluded in last meeting. As mentioned by other companies, network may get the channel information according to the reported N beams. Q2.2: Open to discuss this issue. However, we should first discuss whether the time window is applicable to Event 7. [Mod]: Captured! |
| KDDI | Q2.1 #1 We support. The order of quality among activated TCI state is vary in each measurement occasions, and the NW could not identify it from beam report. #2 We support. Similar to #1, the NW could not know which beams meet the event condition. Moreover, to align with the format of Event-2, the indicator is preferable. [Mod]: Captured! |
| vivo | For #1: We share the same view as OPPO that #1 is needed but this does not provide meaningful information to gNB. Only based on #1, the network cannot determine which TCI state/activated TCI state should be activated/deactivated respectively. The essence Event-7 is to activate/deactivate TCI states, UE needs to report which activated TCI state(s) can be replaced by reported new beams that meet the corresponding trigger condition. For #2: If additional information mentioned in #1 is supported, #2 is useful. Otherwise, #2 is not needed. [Mod]: Captured! [Mod]: Captured! |
| Ericsson | #1: support #2: support [Mod]: Captured! |
| IDC | We share similar views as ZTE, and support both #1 and #2. [Mod]: Captured! |
| Apple | Q2.1: #1: Yes, support. Without this, the event-7 has very limited applicablity. #2: The radionale of ‘1-bit’ condition met indicator is same as Event-2. Since purly based on the report L1-RSRP, NW can not determine whether a beam with with an L1-RSRP exceeding the threshold meets the Event-‘M’ criteria. To simiplify and unify the report content design, it is logical to adopt the same approach as Event-2 when the window size for Event-7 is greater than one. [Mod]: Captured! |
| Mod_V30 | Update per companies input |
| Transsion | Q2.1 #1: Not needed as RAN1 has concluded that no consensus on this issue. #2: We support the additional indication due to the same reason that we support it for Event-2. |
| Mod_V32 | Update per companies input |
| ETRI | #1 is beneficial because it tells which one is the current beam. However, we think if fading state rapidly changes, then it would be most effective to report as many beams as possible in this case. #2: It may be effective if #1 is supported. |
| Mod_V32 | Update per companies input |
| China Telecom | Q2.1 #1: Not Support. We think that there is no need for #1 and #2 to exist simultaneously. On the basis, continuing to use the new beam indication method of Event - 2 can have better consistency in specifications. Besides, regarding the beam reporting for beam switching, it has lower priority to indicate the Qth best beam for the current beam of event-7. #2: Support. |
| Mod_V32 | Update per companies input |

## Issue 3 – UL signaling medium/container

Table 3-1 Summary for Issue 3

| # | Issue | Companies’ view and Recommended Proposal |
| --- | --- | --- |
| 3.1.1 | First PUCCH for Mode-A and Mode-B —— the first PUCCH (re-)transmission | [120b] Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting for a CSI report configuration, down-select at least one of the following candidates in RAN1#121: - Candidate#1: To introduce a prohibit timer for mode-A and/or mode-B - Option-1A: In the case that triggering-event associated with the CSI report configuration is determined [by the same triggering beams(s) as the last transmitted PUCCH], if the prohibit timer is NOT running, the UE can transmit first PUCCH; - At the first symbol after the end of the PUSCH transmission, the UE starts the prohibit timer - Option-1B: In the case that triggering-event associated with the CSI report configuration is determined, if the prohibit timer is NOT running, the UE can transmit first PUCCH; - At the first symbol after the end of the first PUCCH transmission, the UE starts the prohibit timer - If the prohibit timer is running, the first PUCCH is not allowed to be transmitted. - Candidate#2: To introduce a time interval for mode-A and/or mode-B - Option-2A: For a first PUCCH transmission occasion, if there is a transmission of another second PUSCH [corresponding to the CSI report determined by the same triggering beam(s) as the first PUCCH] within a configurable time interval before the first PUCCH transmission occasion, the UE should not transmit the first PUCCH. - Option-2B: For a first PUCCH transmission occasion, if there is a transmission of another first PUCCH(s) within a configurable time interval before the first PUCCH transmission occasion, the UE should not transmit the first PUCCH. - Candidate#3: No further enhancement. FL observations: - Candidate #1: - Option-1A: ZTE, MediaTek, CATT, Google, Huawei/HiSi, xiaomi, CMCC, LG, Apple, Ericsson, China Telecom, NEC, Ericsson, NTT DOCOMO, HONR, KDDI, - Option-1B: Qualcomm (both), ETRI, Futurewei, Sharp, xiaomi, Fujitsu, LG, IDC, Apple, China Telecom, NEC, Ericsson, Ofinno, KDDI, Transsion, - Candidate #2: - Option-2A: OPPO, Samsung (2nd priority), Lenovo, LG, vivo, - Option-2B: Samsung (2nd priority), Lenovo - No enhancement: Samsung, Nokia, Q3.1.1: Please share your views/analysis on the above candidates. Once we have progress on Issue 1.1, we will try to wrap up this discussion including the possibility of no further enhancement. |
| 3.1.2A | First PUCCH for Mode-A and Mode-B —— Extension of scope(s) | [120] Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, one PUCCH resource of first PUCCH can be associated with one or multiple CSI report configurations, regarding Event-2. - Only a single UEI beam report is carried in the second PUSCH. - Additional indication of one ‘CSI report configuration’ is provided in the report format. - The CSI report configurations associated with the same PUCCH resource are ordered in ascending order of corresponding CSI-ReportConfigId, the number of bits of the additional indication field is ceil(log2(N_CSIconfig)), where the N_CSIconfig denotes the number of CSI report configurations associated with the same PUCCH resource. - The payload size of the single UEI beam report is determined according to the max payload size among the associated CSI report configurations. - Zero-padding can be appended if the payload size of the UEI beam report is less than the max report payload. - The reported UEI beam report should satisfy the triggering condition. - If multiple UE initiated beam report procedures occur, down-select one of the following options: - Option-1: It is up to UE implementation to select one of configuration. - Option-2: The UEI beam report with highest priority is reported - Option-3: The report triggered in the latest measurement is reported in PUSCH - The multiple CSI report configurations associated with the one first PUCCH resource should be configured in the same CC. - Working Assumption: For Mode-A, the multiple CSI report configurations associated with the same PUCCH resource should be associated with a same CSI-AperiodicTriggerState. - For Mode-B, the multiple CSI report configurations associated with the same PUCCH resource should be associated with the same second configured PUSCH FFS: Alt-2 Multiple UEI beam reports associated with the same PUCCH resource for first PUCCH can be transmitted in the second PUSCH. - If RAN1 cannot converge on the support of Alt-2 in RAN1#120bis, this alternative will be dropped from Rel-19 Q3.1.2A: On beam report transmission procedure for UE-initiated/event-driven beam reporting, please share your views on whether the above can be applied to multi-Event(s) (e.g., involving Event-1/7)? Proposal 3.1.2A: On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-1 and Event-7, one PUCCH resource of first PUCCH can be associated with one or multiple CSI report configurations. - In such case, the agreed report format for Event-2 is reused, i.e., only a single UEI beam report is carried in the second PUSCH - Down-select one of the following in RAN1#121 - Option-1: The event type(s) of multiple CSI report configurations can be the same or different, i.e., no restriction. - Option-2: The event type(s) of multiple CSI report configurations should be the same. - Note: One CSI report configuration for UEI is associated with one event only Option-1: Nokia, Spreadtrum, CATT, Fujitsu, Ofinno, xiaomi, Spreadtrum, MediaTek, NEC, LG, CATT, Lenovo, Sharp, NTT DOCOMO, NTT DOCOMO, HONOR, vivo, Ericsson, IDC, Apple, Transsion, ETRI, Option-2: OPPO, Futurewei, Not supported: Samsung, Huawei/HiSi’ (?), KDDI, |
| 3.1.2B | First PUCCH for Mode-A and Mode-B —— Additional rule | [120b] Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, confirming the following working assumption with the following modification. - For Mode-A, the multiple CSI report configurations associated with the same PUCCH resource should be associated with a same CSI-AperiodicTriggerState. - FFS: For Mode-A, the multiple CSI report configurations associated with the same CSI-AperiodicTriggerState should be associated with a same PUCCH resource. Q3.1.2B: On one PUCCH resource of first PUCCH can be associated with one or multiple CSI report configurations, please share your views/analysis on whether to support the following rule additionally. - For Mode-A, the multiple CSI report configurations associated with the same CSI-AperiodicTriggerState should be associated with a same PUCCH resource. Proposal 3.1.2B: On one PUCCH resource of first PUCCH can be associated with one or multiple CSI report configurations, - For Mode-A, the multiple CSI report configurations associated with the same CSI-AperiodicTriggerState should be associated with a same PUCCH resource. Supported by Huawei/HiSi’, OPPO, ofinno, Fujitsu, xiaomi, ZTE, Spreadtrum, MediaTek, Futurewei, NEC, Fujitsu, LG, Sharp, NTT DOCOMO(okay), Qualcomm, HONOR, Apple, Transsion, Not supported by Nokia, vivo, Samsung, Lenovo, KDDI, Ericsson, |
| 3.1.3A | First PUCCH for Mode-A and Mode-B —— multiplexing and/or dropping rule on Case-2: the 1-bit first PUCCH is collided/overlapped with a PUSCH | [120b] Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, down-select one of the following rules for the Case-2: the 1-bit first PUCCH is collided/overlapped with a PUSCH, in RAN1#121 - Option-1: Prioritize first PUCCH over PUSCH, i.e., PUSCH is dropped. - FFS: If the PUSCH should be with UL-SCH or not for UEI beam report - Option-3: Piggyback 1-bit indication of first PUCCH into the PUSCH. - FFS: The 1-bit indication is always multiplexed in the PUSCH, regardless that UEI beam report procedure is triggered. - FFS: If the PUSCH should be with UL-SCH or not for UEI beam report - Option-4: Reuse the SR dropping rules. - FFS: whether/how to handle the case of different PHY priorities. FL observations: - Option-1: OPPO, NTT DOCOMO (2nd priority), Panasonic, Huawei/HiSi, HONOR, - Option-3: ZTE, MediaTek, vivo (similar to HARQ-ACK), Qualcomm, ETRI, NEC, CATT (1st priority), Google, Futurewei, Sharp, xioami, Samsung, Fujitsu, CMCC, Lenovo, Apple, NTT DOCOMO (1st priority), Ofinno, China Telecom (?), Ruijie, Transsion, TCL, ITRI, Sony, NICT, Ofinno, - Option-4: CATT (2nd priority), Spreadtrum, Nokia, Ericsson, FL Note: Per companies input, we have the following stabilized proposal for further down-selection, regarding the case-2: the 1-bit first PUCCH is collided/overlapped with a PUSCH. Since this is a last meeting for Rel-19 MIMO, we should solve the FFS as well. - Note: Per previous agreement(s), if no consensus, reusing the same rule as SR should be assumed as a baseline. Proposal 3.1.3A: On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, support Option-3 of the following rules for the Case-2: the 1-bit first PUCCH is collided/overlapped with a PUSCH - Option-3: Piggyback 1-bit indication of first PUCCH into the PUSCH. - FFS: The 1-bit indication is always multiplexed in the PUSCH, regardless that UEI beam report procedure is triggered. - FFS: If the PUSCH should be with UL-SCH or not for UEI beam report Supported by: ZTE, MediaTek, vivo, Qualcomm, ETRI, NEC, CATT, Google, Futurewei, Sharp, xioami, Samsung, Fujitsu, CMCC, Lenovo, Apple, NTT DOCOMO, Ofinno, China Telecom, Ruijie, Transsion, TCL, ITRI, Sony, NICT, Ofinno, LG, KDDI, Ericsson, ETRI, Not supported by: Spreadtrum, Nokia, Huawei/HiSi’, |
| 3.1.3B | First PUCCH for Mode-A and Mode-B ——multiplexing and/or dropping rule on Case-4: the 1-bit first PUCCH is collided/overlapped with a PUCCH format 2/3/4 carrying HARQ/CSI/SR | FL Note: Per RAN1#120-bis offline, we have the following proposal for further discussion. If needed, we may draft the corresponding text proposal for accelerating the discussion. - FYI, there is one example for clarifying this issue from RAN1#120b offline. Proposal 3.1.3B-1: On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing a number of L (L>=1) first PUCCH(s) with UEIRIs are collided/overlapped with a PUCCH format 2/3/4 carrying HARQ/CSI, reuse the legacy SR multiplexing rule. - The value of log2L+1 bits is based on an ascending order of the values of PUCCH resource ID associated with the first PUCCHs. Supported by: Huawei/HiSi’, vivo, Nokia, Ericsson, Apple, Qualcomm, ZTE, NTT DOCOMO, ofinno, Fujitsu, HONOR, Transsion, OPPO, Ofinno, xiaomi, Spreadtrum, MediaTek, Futurewei, NEC, LGE, CATT, Lenovo, Sharp, HONOR, Apple, ETRI, China Telecom, Samsung, Proposal 3.1.3B-2: On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) the 1-bit a number of L (L>=1) first PUCCH is collided/overlapped with a PUCCH format 2/3/4 carrying HARQ/CSI/SR. - Option-1: Extend the SR field of log2K+1 bits to a field of log2K+L+1 bits of representing at most one of positive SR or positive UEIRI - The codepoint in the field is generated based on a first ascending order of the values of SR ID associated with the SRs and a second ascending order of the values of PUCCH resource ID associated with the first PUCCHs. - If one of the SRs is a positive LRR, the value of the log2K+L+1 bits indicates the positive LRR, else, if one of the UEIBRs is a positive UEIBR, the value of the log2K+L+1 bits indicates the positive UEIBR. - An all-zero value for the log2K+L+1 bits represents a negative SR or UEIRI value across all K LRR/SRs and/or L UEIRIs. - Option-2: Append the UEIRI to HARQ-ACK, if any, with joint coding. - Option-3: The field of log2L+1 bits for UEIRI is appended to the field of log2K+1 bits for SR/LRRs. Option-1: Fujitsu, Ericsson, ZTE, NTT DOCOMO, Transsion, Qualcomm, Apple, HONOR, MediaTek, Futurewei, OPPO, Fujitsu, CMCC, Spreadtrum, NEC, Lenovo, Nokia, Ofinno, vivo, Transsion, xiaomi (1st priority), Samsung, MediaTek, Nokia, NEC, Fujitsu, Lenovo, Sharp, Apple, Transsion, ETRI, China Telecom, Option-2: Samsung, Option-3: Huawei/HiSi’, xiaomi(2nd), Spreatrum, |
| 3.2 | Details on Step-2 in Mode-B —— value of X | [118] Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-B, UEI beam report is carried on a first available transmission occasion of the second UL channel X symbols after sending the last symbol of report notification on the first PUCCH channel. [120] Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-B, the value of X symbols for determining available transmission occasion of the second UL channel is configured by RRC (FFS: subject to a corresponding UE capability) FL Note: Per companies input and outcome of RAN1#120-bis offline, we have the following proposals for the further progress. Then, per Samsung and Ofinno’s input, one more update on SCS for X is clarified. Proposal 3.2: On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the value of X symbols for determining available transmission occasion of the second UL channel on Mode-B - Down-select one of the following as RRC candidate values for X symbols - Option-1: [0,] 1, 2, 4, 8, 16, 32, 64 - Option-2: [0,] 1 - Option-3: 0 - Option-4: [0,] 1, 2, 4, 8 - Note: X is based on the SCS of the first PUCCH. - The ‘available’ transmission occasion of the second UL channel is clarified as ‘an available transmission occasion corresponds to a configured PUSCH resource for UEIBR is within uplink symbols, and does not collide with a channel with a higher priority (e.g., PRACH)’. - Note: Whether/how to introduce additional UE capability signaling is up to Rel-19 UE feature discussion. Option-1: Samsung, OPPO, Ofinno, Google (w.o. 0), Nokia, Futurewei, NEC, Lenovo, KDDI, vivo, Ericsson, Apple (w.o. 0), Transsion, ETRI Option-2: ZTE, Lenovo, KDDI, CATT, Option-3: China Telecom, KDDI, Google (w.o. 0), NEC, Lenovo, CATT, Option-4: Ofinno, Google (w.o. 0), ZTE, Huawei, NEC, Lenovo, KDDI, Ericsson, ETRI, CATT, Postpone the discussion on value of X: QC, HONOR, |
| 3.3 | Details on dropping/priority rule for UEI beam report | [120b] Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case the pre-configured Type-1 CG PUSCH carry the beam report, for the second UL channel in Mode-B, reuse the intra-UE multiplexing/prioritization rules of PUSCH with SP-CSI for Type-1 CG PUSCH with UEI-BR for Mode B FL Note: If two CSI reports collide, they are multiplexed or one of them is dropped based on the priority values. Per companies input, we have the following proposals for clarifying the priority function for UEI-BR. Proposal 3.3: On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding priority rules for CSI report multiplexing/dropping, UEI beam report for both mode-A and mode-B is prioritized over Semi-persistent CSI reports on PUCCH and Periodic CSI reports on PUCCH - Down-select one of the following in RAN1#121-bis: - Option-1: UEI beam report for mode-A > Aperiodic CSI report > UEI beam report (for mode-B) > Semi-persistent CSI reports on PUSCH - Option-2: UEI beam report for mode-A = Aperiodic CSI report > UEI beam report for mode-B = Semi-persistent CSI reports on PUSCH - Option-3: UEI beam report for mode-A > UEI beam report for mode-B > Aperiodic CSI report > Semi-persistent CSI reports on PUSCH Note-1: The intra-UE multiplexing/prioritization rules of PUSCH with A-CSI for PUSCH is reused for UEI-BR for Mode A. Note-2: How to capture the above is up to Editor. Option-1: MediaTek, Huawei/HiSi’, ZTE, Nokia, NTT DOCOMO, vivo, Apple, Option-2: Samsung, OPPO, Ofinno, CATT, Lenovo, Sharp, Qualcomm, Ericsson, ETRI, Option-3: ZTE, Spreadtrum, xiaomi, Google, NEC (no different between mode-A and mode-B), Fujitsu, LGE, HONOR, Transsion, CATT, |
| 3.4 | Details on CSI reference resource | FL Note: Per companies input, we have the following proposals for clarifying CSI-reference resource for UEI beam report procedure. Proposal 3.4: On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding CSI reference resource definition for a UEI beam report, down-select at least one of the following: - Option-1: For both mode-A and mode-B, reuse legacy CSI reference definition of P/SP-CSI reporting, except that, in TS 38.214 Clause 5.2.2.5, slot n is based on a downlink slot with reference to the uplink slot in which the first PUCCH is transmitted. - Option-2: - For mode A, support to reuse legacy CSI reference definition of aperiodic CSI reporting. - For mode B, support to reuse legacy CSI reference definition of semi-persistent CSI reporting. - Option-3: For both mode-A and mode-B, support to reuse legacy CSI reference definition of aperiodic CSI reporting for UEI/ED beam reporting. Option-1: Nokia, ZTE, vivo, Lenovo, Samsung, Huawei/HiSi, NEC, Fujitsu, LG, Lenovo, Sharp, Transsion, Option-2: MediaTek, OPPO, Ofinno, Google, Nokia (2nd priority), Futurewei, Apple, Option-3: NTT DOCOMO, Qualcomm, xiaomi, Ericsson, ETRI, |
| 3.5 | Details on the counting of occupied CPU(s) | FL Note: Per companies input, we have the following proposals for clarifying CPU occupancy in UEIBR. Except for one company, majority companies prefer to use OCPU = 1 as usual, and then as a first step, we may try to make progress on that firstly. Proposal 3.5A: On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding occupied CPU(s), OCPU = 1 is occupied. Proposal 3.5B: On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding occupied CPU(s), down-select one of the following options for occupancy time of UEI beam reporting: - Option-1: The start of CPU occupancy time is: the first symbol of the earliest one of current beam RS and new beam RS, no later than a CSI reference resource: - Option-1A: The end of CPU occupancy time is: the last symbol of the first PUCCH transmission occasion, regardless of whether the UEIRI is transmitted on such PUCCH transmission occasion. - Option-1B: - If the UEIRI is transmitted on the first PUCCH resource, the end point of CPU occupancy time is the last symbol of the scheduled or pre-configured PUSCH carrying the report. - If the UEIRI is not transmitted on the first PUCCH resource, the end of CPU occupancy time can be the last symbol of the first PUCCH resource. - Option-1C: The end of CPU occupancy time is: the last symbol of the second PUSCH. - Option-2: UE periodically occupies one CSI processing unit for Z'' symbols every evaluation occasion, starting from the first symbol of the earliest one of each measurement RS resource respective latest measurement RS occasion no later than an evaluation occasion. - FFS: The value of Z'' is subject to UE capability or specified as a certain value. - FFS: Whether to apply different value of Z'' depending on counting evaluation within the window is enabled - Option-3: two CPU counting is applied: - CPU counting #1: from the first symbol of the earliest one of each transmission occasions of periodic CSI-RS/SSB resource for channel measurement for L1-RSRP computation, until X symbols after the last symbol of the latest one of the CSI-RS/SSB resource for channel measurement for L1-RSRP computation in each transmission occasion, - FFS: X, e.g., reusing Z3' - CPU counting #2: - Option-3A: from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report for Mode-A, or from the first symbol after the first PUCCH transmission until the last symbol of the PUSCH carrying the report for Mode-B. - Option-3B: countering for each PUSCH occasion. Assuming the last symbol ‘L’ of PUSCH, the CPU counter #2 starts from ‘L-∆_2’ symbol and ends at the last PUSCH symbol ‘L’. - Opition-3C: - In Mode A, from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report for Mode-A, - In Mode B, from the first symbol of the earliest one of periodic CSI-RS/SSB resource for channel measurement for L1-RSRP computation, respective latest CSI-RS/CSI-IM/SSB occasion no later than the corresponding CSI reference resource, until the last symbol of the PUSCH carrying the report for Mode-B - Option-4: Occupation time starts when the UEI beam report is configured and ends when it is released. - Option-5: For Mode-A and Mode-B, occupation time starts from the first symbol after the first PUCCH transmission until the last symbol of the PUSCH carrying the report - Option-6: from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report for Mode-A, or from the first symbol after the first PUCCH transmission until the last symbol of the PUSCH carrying the report for Mode-B. Option-1: ZTE, Huawei, Nokia, NTT DOCOMO, Futurewei, - Option-1A: ZTE, Sharp, Transsion, - Option-1B: Huawei, Nokia, NEC, - Option-1C: NTT DOCOMO, ETRI, Option-2: MediaTek, OPPO, Lenovo, Ericsson, Option-3: Qualcomm, Apple, xiaomi, HONOR(?) - Option-3A: Qualcomm, NTT DOCOMO, - Option-3B: Apple - Option-3C: MediaTek, Option-4: LG, Ofinno, Spreadtrum,vivo, Option-5: Samsung, Option-6: Fujitsu, Not support: vivo (OCPU being based on the number of time windows) |
| 3.6 | Details on active resource(s) | FL Note: Since only periodic CSI-RS/SSB is configured herein for L1-RSRP measurement/report, we may not need any additional efforts on the active resource counting. If something is missing, please correct that or leave that for UE feature discussion (e.g., being based on 16-1g/-1). |

Table 3-2 Company input for Issue 3

| Company | Input |
| --- | --- |
| Mod V00 | Please input your comment/preference to those proposals and questions on the issues 3.1~3.6, if needed. |
| OPPO | Re 3.1: Candidate #1 and #2 are actually same just with different wording. We are ok with either Option-1B or Option -2B for mode-A. For mode-A, such a prohibit time duration is needed. After the UE sends 1st PUCCH, the UE shall wait for a reasonable time duration for the gNB to process and request and schedule the PUSCH. But for mode-B, we do not support to introduce such function. Q3.1.2A: Before we can conclude this question, we shall first clarify that the gNB can not configure different events simultaneously in the same CC [Mod]: Initially, I think the different events can be simultaneously configured in the same CC. Please review the update proposal. Q3.1.2B: this restriction is necessary to avoid the potential operation issue. 3.1.3A: we can be ok with the proposal of Option-3 if the FFS can be resolved in the proposal. 3.1.3B-1: ok 3.1.3B-2: support Option-1. 3.2: support Option 1. 3.3: we support Option 2. Mode-A is actually just one AP report and it should have the same priority as AP CSI report. The same reason for mode B. Option-1 and Option-3 complicates the priority rule, which is not preferred. 3.4: support Option 2. 3.5: support Option 2. [Mod]: Captured! |
| Ofinno | Q3.1.1. Support option-1B. In our view, candidate 1 provides a simpler and more efficient approach compared to candidate 2, which requires the UE to look backward over a configured time interval for every potential PUCCH transmission occasion to determine whether a recent transmission occurred. Between candidate 1, option-1B provides a good balance between implementation simplicity and robust control of redundant reporting. Starting the prohibit timer immediately after the PUCCH transmission avoids the complexity of having to monitor the completion of the associated PUSCH transmission, which can vary in length and timing depending on scheduling especially for Mode-A. Q3.1.2A. According to the current agreements, the inclusion of an event ID in the report contents is still under FFS. Therefore, from the gNB’s perspective, there may be no reliable way to distinguish between reports triggered by different events explicitly, but if each CSI report configuration is associated with only one event, it can be done implicitly. We can support this. [Mod]: Please review the latest proposal. One more note is added for clarification. Q3.1.2B. Support proposed rule. In Mode-A, when multiple CSI report configurations share the same CSI-AperiodicTriggerState and local ID, assigning different PUCCH resources to them can cause ambiguity. If the UE sends first PUCCHs on multiple resources and later transmits a report on the second PUSCH identified only by the local ID, the gNB cannot determine which configuration the report corresponds to. This uncertainty may lead to incorrect interpretation of the report and affect scheduling or beam management. Aligning these configurations to the same PUCCH resource avoids such issues and ensures consistent report handling. Proposal 3.1.3A. Support the proposal. For the FFS points, we do not think piggyback 1-bit indication into gNB indicated CSI report and UEI beam report is needed. Proposal 3.1.3B-1. Support the proposal. Proposal 3.1.3B-2. Support option 1. Proposal 3.2. Support the proposal. Considering the first PUCCH and the second PUSCH can be located in different CCs, we think at least option-2 and option-3 seem too restrictive. Proposal 3.3. Support option-2. We note that Proposal 3.3, particularly Options 1 and 3, conflicts with the existing agreement above the proposal, which specifies that Mode-B UEI beam reports should follow the same intra-UE prioritization rules as SP-CSI on PUSCH. This implies equal prioritization between Mode-B UEI-BR and SP-CSI. Therefore, unless the agreement is revised, Option-2 is the only consistent choice. Proposal 3.4. Support option-2. From our understanding, the CSI report in Mode-A is essentially an aperiodic CSI report scheduled by DCI, and therefore, it should naturally follow the CSI reference resource definition of aperiodic CSI reporting without requiring a separate agreement. It would be more appropriate to limit the discussion to Mode-B, and considering that the report is transmitted via CG-PUSCH, it makes sense to define the reference resource based on that of P/SP-CSI reporting. Proposal 3.5. It is good to define CPU occupancy in detail, but from the gNB’s perspective, the important point is that if the UE’s CPU occupancy exceeds the value it reported as its capability, the UE may skip reporting CSI report(s) with the lowest priority. Therefore, a simple approach is desirable, and option-4 is slightly preferred. [Mod]: Captured! |
| Google | Q3.1.2A: We want to clarify the question first. We assume it is about whether the above can be applied to other Event(s) (e.g., involving Event-1/7)? Otherwise, it seems against below conclusion. If our assumption is correct, we can support the applicability of Event-1/7. Conclusion On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the case that one PUCCH resource of first PUCCH can be associated with one or multiple CSI report configurations, there is NO RAN1 consensus on supporting multiple UEI beam reports carrying in a second PUSCH. [Mod]: Please check the new proposal. Proposal 3.1.3A: Support this proposal. Proposal 3.2: We have a concern that value of X can be 0. If value of X is 0, it means UE needs to prepare UEI beam report and is ready to transmit it before transmitting the first PUCCH. However, the first PUCCH may be dropped due to collision with other UL, which makes the prepared UEI beam report useless. Therefore, it is not reasonable to set X equals to 0, which makes UE prepares UEI beam report before transmitting the first PUCCH. With that said, we are OK with Option-1/2/4 with 0 value removed. Proposal 3.3: We support Option-3. UEI-BR should be anyway prioritized than AP CSI report. Proposal 3.4: We support Option-2 to minimize SPEC impact. |
| Samsung | Q3.1.1: The issue of prohibit timer is not essential and not supporting the feature can also be beneficial for overhead reduction. For example, after transmitting the first PUCCH and the channel conditions have become better, the UE can stop transmitting the next first PUCCH so that the network can avoid indicating a PUSCH resource for UEIR or indicating TCI state update (unnecessary in this case). For another example, if a UE keeps sending UEIRI via first PUCCH, the network can, more promptly, indicate a PUSCH resource to carry the corresponding UEIR. Based on the above, we support candidate #3 i.e. no further enhancement needed. Q3.1.2A: We don’t think the issue is essential and suggest not to extend to other events. Q3.1.2B: We think that it can be up to network’s implementation. Open for further discussions/clarifications. Proposal 3.1.3A: Support. Regarding the concerns on the blind detection, the first bullet can be supported. Please note that negative indication of UEIRI is supported based on the agreement below. For multiplexing SR and HARQ-ACK/CSI, UCI can include negative SR; similarly, negative UEIRI can be supported. [Mod]: Good comments! Captured! [119] Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding first PUCCH channel configuration, Alt-2 is supported for both mode-A and mode-B: the first PUCCH channel is a new UCI type - Alt-2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig- UEIBR, for the periodic PUCCH resource configuration. - It is RAN1’s understanding that the RRC parameter is NOT associated with SchedulingRequestId. - 1-bit to PUCCH resource is encoded by reusing the encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above applies at least for the single CC case. - Reuse multiplexing/dropping rule(s) of SR as baseline - FFS: overlapping with SR/LRR or PUSCH Note: Further details on first PUCCH retransmission (if supported) for mode A and mode B will be continue to be separately discussed in RAN1. TS 38.213 If a UE would transmit a PUCCH with OACK HARQ-ACK information bits in a resource using PUCCH format 2 or PUCCH format 3 or PUCCH format 4 in a slot, as described in clauses 9.2.1 and 9.2.3, log2K+1 bits representing a negative or positive SR, in ascending order of the values of schedulingRequestResourceId, a schedulingRequestResourceId associated with schedulingRequestID-BFR-SCell, a schedulingRequestResourceId associated with schedulingRequestID-BFR, a schedulingRequestResourceId associated with schedulingRequestID-BFR2 if the UE provides twoLRRcapability, and a schedulingRequestResourceId associated with schedulingRequestID-LBT-SCell, are appended to the HARQ-ACK information bits and the UE transmits the combined OUCI=OACK+log2K+1 UCI bits in a PUCCH using a resource with PUCCH format 2 or PUCCH format 3 or PUCCH format 4 that the UE determines as described in clauses 9.2.1 and 9.2.3. If one of the SRs is a positive LRR, the value of the log2K+1 bits indicates the positive LRR. An all-zero value for the log2K+1 bits represents a negative SR value across all K SRs. Proposal 3.1.3B-1: Not support. This proposal is not aligned with reusing the multiplexing rule for SR. Proposal 3.1.3B-2: We are fine with the intention of Option 1 and suggest the following update to minimize spec change. We suggest to follow the same rule as in the agreement below as a unified design. In addition, based on the highlighted yellow text above (If one of the SRs is a positive LRR, the value of the log2K+1 bits indicates the positive LRR), we should define a similar rule accordingly. [120] Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, support the following option of dropping rule for the Case-1: the 1-bit first PUCCH is collided/overlapped with a PUCCH carrying normal SR and/or a PUCCH with normal LRR - Option-1: LRR > first PUCCH > normal SR Note: When the 1-bit first PUCCH is collided/overlapped with a PUCCH carrying normal SR and/or a PUCCH with normal LRR, only one of them is transmitted based on the above priority rule (Updated) Proposal 3.1.3B-2: On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) the 1-bit a number of L (L>=1) first PUCCH is collided/overlapped with a PUCCH format 2/3/4 carrying HARQ/CSI/SR, reuse the legacy SR multiplexing rule with the following update. - Option-1a: K SRs are replaced with K SRs and UEIRIs where there are K-L SRs and L UEIRIs and Extend the SR field of log2K+1 bits to a field of log2K+L+1 bits of representing at most one of positive SR or positive UEIRI - The value of log2K+1 is ordered in ascending order of SRs and then UEIBRs where for UEIBRs the value is ascending order of the associated PUCCH resource ID. - If one of the SRs is a positive LRR, the value of the log2K+1 bits indicates the positive LRR, else, if one of the UEIBRs is a positive UEIBR, the value of the log2K+1 bits indicates the positive UEIBR. - An all-zero value for the log2K+1 bits represents a negative SR or UEIRI value across all K SRs and/or UEIRI [Mod]: Captured except for being the original format. In my views, the update is the same and how to capture that in the spec is up to Editor. Proposal 3.2: Support Option-1 Proposal 3.3: Support Option-2 which follows legacy design principle for CSI reporting. However, the proposal is not complete since it does not address the case that multiple CSI report configurations are associated with the same PUCCH resource. In this case, if the priority value of a UEI beam report is based on the its corresponding CSI report configuration, NW does not know the UEI beam report is initiated by which CSI report configuration until the content of the report is decoded and cannot know the result for prioritization if the UEI beam report overlaps with another CSI report. As a consequence, NW needs to perform blind detection. To avoid blind detection for the network, it is preferred to use a fixed value regardless of which CSI report configuration is reported. For example, the priority value is determined by the CSI report configuration with highest priority. Hence, we propose the following updates to Proposal 3.3. (Updated) Proposal 3.3: On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding priority rules for CSI report multiplexing/dropping, UEI beam report for both mode-A and mode-B is prioritized over Semi-persistent CSI reports on PUCCH and Periodic CSI reports on PUCCH - Down-select one of the following in RAN1#121-bis: - Option-1: UEI beam report for mode-A > Aperiodic CSI report > UEI beam report (for mode-B) > Semi-persistent CSI reports on PUSCH - Option-2: UEI beam report for mode-A = Aperiodic CSI report > UEI beam report for mode-B = Semi-persistent CSI reports on PUSCH - Option-3: UEI beam report for mode-A > UEI beam report for mode-B > Aperiodic CSI report > Semi-persistent CSI reports on PUSCH - In case of the same PUCCH resource of a UEI beam report is associated with multiple CSI report configurations, the priority of the UEI beam report is determined by the CSI report configuration with highest priority from the multiple CSI report configurations. Note-1: The intra-UE multiplexing/prioritization rules of PUSCH with A-CSI for PUSCH is reused for UEI-BR for Mode A. Note-2: How to capture the above is up to Editor. [Mod]: Let’s check other views. In addition, we think that Note-1 in the above proposal can be separately formulated for Mode B. Proposal 3.4: We prefer Option-1 – the UEI beam report is derived based on the measurement results before the transmission of the first PUCCH. Then, it makes sense to use the slot associated with the first PUCCH for CSI reference resource. With Option-1, the determined slot of CSI reference resource is used for both deriving CSI report and determining the event(s), which is streamlined. Proposal 3.5: A simple and unified solution is preferred. To our understanding, for both mode-A and mode-B, the calculation of CSI can start after the transmission of UEIRI via first PUCCH – basically after all associated measurements for event evaluation have completed and the event is triggered. For the ending of the CSI calculation, the last symbol of UEI beam report can be used which follows legacy principle. Hence, Option-5 is provided below, which is much simpler than all of the options meanwhile capturing only the components that are really essential/needed. We are also open to Option-4. (Updated) Proposal 3.5: On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding occupied CPU(s), OCPU = 1 is occupied - Down-select one of the following options for occupancy time of UEI beam reporting: - Option-1: The start of CPU occupancy time is: the first symbol of the earliest one of current beam RS and new beam RS, no later than a CSI reference resource: - Option-1A: The end of CPU occupancy time is: the last symbol of the first PUCCH transmission occasion, regardless of whether the UEIRI is transmitted on such PUCCH transmission occasion. - Option-1B: - If the UEIRI is transmitted on the first PUCCH resource, the end point of CPU occupancy time is the last symbol of the scheduled or pre-configured PUSCH carrying the report. - If the UEIRI is not transmitted on the first PUCCH resource, the end of CPU occupancy time can be the last symbol of the first PUCCH resource. - Option-1C: The end of CPU occupancy time is: the last symbol of the second PUSCH. - Option-2: UE periodically occupies one CSI processing unit for Z'' symbols every evaluation occasion, starting from the first symbol of the earliest one of each measurement RS resource respective latest measurement RS occasion no later than an evaluation occasion. - FFS: The value of Z'' is subject to UE capability or specified as a certain value. - FFS: Whether to apply different value of Z'' depending on counting evaluation within the window is enabled - Option-3: two CPU counting is applied: - CPU counting #1: from the first symbol of the earliest one of each transmission occasions of periodic CSI-RS/SSB resource for channel measurement for L1-RSRP computation, until X symbols after the last symbol of the latest one of the CSI-RS/SSB resource for channel measurement for L1-RSRP computation in each transmission occasion, - FFS: X, e.g., reusing Z3' - CPU counting #2: - Option-3A: from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report for Mode-A, or from the first symbol after the first PUCCH transmission until the last symbol of the PUSCH carrying the report for Mode-B. - Option-3B: countering for each PUSCH occasion. Assuming the last symbol ‘L’ of PUSCH, the CPU counter #2 starts from ‘L-∆_2’ symbol and ends at the last PUSCH symbol ‘L’. - Option-4: Occupation time starts when the UEI beam report is configured and ends when it is released. - Option-5: For Mode-A and Mode-B, occupation time starts from the first symbol after the first PUCCH transmission until the last symbol of the PUSCH carrying the report [Mod]: Captured! |
| Xiaomi | Q3.1.1 Support Option 1A for both modes and support option 1B for mode A. Q3.1.2A We think the above can be applied to multi-Events which means some of multiple CSI report configurations are associated with Event 2, and others are associated with Event1/7. But it is necessary to discuss whether the CSI report configuration associated with Event 2 has higher priority than others. Q3.1.2B Support. Otherwise, if PUCCH format 0 or format 1 was transmitted on multiple PUCCH resources, UE and NW doesn’t know the beam report associated with which PUCCH resource will be transmitted on the DG PUSCH since the additional indication of one ‘CSI report configuration’ in the report format refer to the configurations associated with the same PUCCH resource. Proposal 3.1.3A Support the proposal and the first FFS need to be supported, which is beneficial for NW reception. [Mod]: Already done! Proposal 3.1.3B-1 Support Proposal 3.1.3B-2 Support option-1 with the first priority and support option-3 with the second priority since the motivation of 1st PUCCH and SR/LRRs is different especially for mode B. Proposal 3.3 Support Option-3, it means UEI BR should have high priority than Aperiodic CSI report. Since UEI BR is reported aperiodically but with satisfying triggering condition which means it is more essential. Proposal 3.4 Support Option-3. Since both mode A and mode B are aperiodic CSI report. Proposal 3.5 Support Option 3. Regarding CPU counting#2, we support Option-3A for mode A. while for mode B, we prefer the starting time is the first symbol of the earliest one of the latest transmission occasions of periodic CSI-RS/SSB resource for channel measurement for L1-RSRP computation before the PUSCH carrying the report. [Mod]: Captured! |
| ZTE | Q3.1.1: Support Option-1A and the addition condition in the bracket should be removed so that the prohibit timer is resource specific rather than new beam specific. Q3.1.2A: Support that one PUCCH resource of first PUCCH can be associated with one or multiple CSI report configurations depending on Event-1, Event-2 or Event-7. Consequently, the event type of the multiple CSI report configurations can be same or different based on NW configuration. Q3.1.2B: Support. We think the issue caused by the agreed local ID of CSI report configurations (a.k.a in ascending order of corresponding CSI-ReportConfigId) is valid and needs to be addressed. In the meanwhile, if the max payload size among the associated CSI report configurations corresponding to different first PUCCH resources are different, NW cannot determine second PUSCH resource to carry UEI beam report. Proposal 3.1.3A: Support Option-3. The first FFS should be supported to have a simple multiplexing design, e.g., the 1-bit indication is multiplexed refer to CSI part 1. The second FFS should be removed since issues of multiplexing 1-bit indication on PUSSCH with UL-SCH/UEIBR have not been identified so far. Consequently, we suggest the following updates. Proposal 3.1.3A: On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, support Option-3 of the following rules for the Case-2: the 1-bit first PUCCH is collided/overlapped with a PUSCH - Option-3: Piggyback 1-bit indication of first PUCCH into the PUSCH. - The 1-bit indication is encoded jointly with CSI part 1 if CSI part 1 is present in the PUSCH, or - The 1-bit indication is multiplexed in the PUSCH as CSI part 1 if CSI part 1 is absent in the PUSCH. - REs for multiplexing the 1-bit indication is mapped from the REs starting from the first OFDM symbol for UCI. - FFS: The 1-bit indication is always multiplexed in the PUSCH, regardless that UEI beam report procedure is triggered. - FFS: If the PUSCH should be with UL-SCH or not for UEI beam report [Mod]: That can be assumed as a baseline. Let’s check other’s views. Proposal 3.1.3B-1: Support. Proposal 3.1.3B-2: Support Option-1. Proposal 3.2: Fine with Option 2 or 4 for the sake of report latency reduction. Proposal 3.3: Prefer Option-3 and can be fine with Option-1. For Option-2, whether to drop a UEI CSI report with Mode-A or an AP CSI report, or whether to drop a UEI CSI report with Mode-B or an SP CSI report depends on priority values of the transmitted UEI CSI report and the AP/SP CSI report. In such case, network may suffer blind detection to identify the above cases. Proposal 3.4: Support Option-1 to guarantee the measurement RS used to trigger the first PUCCH and that used to determine the report contents can be aligned. Proposal 3.5: Prefer Option-1A. Given that CSI reference resource should be determined based on the first PUCCH transmission, the CSI computation can end up after the first PUCCH transmission occasion, and the computed CSI report contents are restored to be transmitted later in the second PUSCH. [Mod]: Captured! |
| Spreadtrum | Q 3.1.2A: Support application to Event-1 and Event-7 for unified design. The report content should be discussed for the case that multiple UEI beam report procedures associated with different events occur. The priority needs to be introduced to determine which report should be transmitted to enable the gNB to obtain more useful reported content for beam update, e.g. the priority is based on the event type, i.e. Event-2>Event-1>Event-7. To reduce the overhead of RS resource and configuration signaling, one CSI report can be configured to associate with a single RS resource set for multiple events, where the RS resource set is used to measure the new beam L1-RSRP for Event-1/2/7. When multiple events are associated with the same CSI report configuration, if an event occurs, the event ID corresponding to the occurred event is carried in UEI beam report; if multiple events occur, the event ID corresponding to the event with highest priority is carried in the UEI beam report. Q 3.1.2B: Fine with this restriction to avoid the ambiguity when the multiple occurred CSI report procedures are associated with different PUCCH resources and have the same local report config ID. Proposal 3.1.3A: Option-4 is our first preference and Option-1 is second preference to avoid the spec complexity. Proposal 3.1.3B-1: Fine. Proposal 3.1.3B-2: Support Option-3. It should be noted that it may be necessary to indicate both positive SR and first PUCCH simultaneously, which case is different from Case-1(when the 1-bit first PUCCH is collided/overlapped with a PUCCH carrying normal SR and/or a PUCCH with normal LRR, only one is transmitted), and Option-1 cannot achieve. A direct way is to introduce a new field to indicate whether there is the first PUCCH in the PUCCH format 2/3/4 carrying HARQ/CSI/SR. As for Option-3, the current description does not seem to support multiple first PUCCH indication. Regarding the number of first PUCCH configured in a slot, considering the overhead reduction of periodic first PUCCH resource, it is recommended to configure at most one first PUCCH in a slot. So this restriction can be added in Option-3 as sub-bullet. Proposal 3.3: Support Option-3. Since the UE initiated beam report is intended to update current beam with deteriorating quality as soon as possible, the UE initiated beam report should have a higher priority than the legacy CSI report. Proposal 3.5: Support to discuss the number of CPU and computation time for the UE initiated beam reporting. For the number of CPU, in legacy spec, OCPU=1 for the gNB controlled beam reporting. The same number of CPUs to process the UE initiated beam reporting can be reused, i.e. OCPU=1. Considering that the UE initiated beam report is similar to a gNB controlled aperiodic CSI report with CSI-ReportConfig with higher layer parameter reportQuantity not set to 'none', the similar principle can be used for CSI computation time. That is to say, the UE initiated beam report occupies CPU(s) from the first symbol after the RRC configuring the UE initiated beam report until the last symbol of the second PUSCH carrying the report. [Mod]: Captured! |
| MediaTek | Q3.1.1: Support Option-1A We support transmission prohibition after transmitting a report because NW may not have beam indication/update immediately. Then duplicated reporting in the short time may happens, which wastes UE power. Q3.1.2A: Support We don’t see any issue if supporting associating with multiple CSI report configurations according to different events. We tend to agree with Ofinno that it can be achieved already without need of further agreement if a CSI report configuration is associated with one event only. Then, event ID is unnecessary. Q3.1.2B: Support A reasonable NW configuration should be that the CSI report configuration associated with a single trigger state must associate with the same PUCCH resource. Otherwise, there may have the problem on reporting of “Additional indication of one ‘CSI report configuration’”, since the additional indication of the CSI report configuration is a local ID determined according to multiple report configurations associated with the one PUCCH resource. It is possible two CSI report configurations associated with different PUCCH resources have the same local ID. Then NW cannot distinguish that. We are fine with adding the restriction. Proposal 3.1.3A: Support in principle. Analogous to SR, we prefer to limit the PUSCH to be a PUSCH not carrying the UEI/ED beam reporting. If UE already get a PUSCH for transmitting the report, there is no need to send the first PUCCH in this case. [Mod]: Per my assessment, we may focus on the basic issue. That can be left for maintenance. Proposal 3.1.3B-1: Support Proposal 3.1.3B-2: Support Option-1. Proposal 3.2: Prefer to remove X=0 from all the options. We don’t think X=0 is feasible. X symbol is reserved for not only UE report preparation but only NW processing on the first PUCCH. The case of X=0 enforces NW preform blind decoding on CG-PUSCH in Mode B no matter the first PUCCH is received. According to the following agreement, the CG-PUSCH is transmitted only if there is a UEI/ED beam report to be transmitted; that is, UE has transmitted the first PUCCH. In that case, NW receives/decodes CG-PUSCH only if received the first PUCCH. If X=0, NW doesn’t have time to process the first PUCCH first. Moreover, from UE’s perspective, no time gap between the first PUCCH and the second PUSCH may enforce UE prepare the report before the first PUCCH. [119] Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case the pre-configured Type-1 CG PUSCH does NOT carry the beam report, for the second UL channel in Mode-B, Option-2 is supported: - Option-2: Type-1 CG PUSCH can NOT be transmitted. [Mod]: That may be relevant to CSI reference resource discussion. Proposal 3.3: Support Option-1 Option-1 is preferred because the dynamic scheduled PUSCH (e.g., AP beam report or UEI beam report in Mode A) should have higher priority than semi-static configured PUSCH (e.g., SP beam report or UEI beam report in Mode B). In the concept, NW knows the existing time-domain allocation of those semi-static configured PUSCH. If NW dynamically schedule a PUSCH overlapping with the existing time-domain allocation of those semi-static configured PUSCH, then that means the dynamic scheduled PUSCH is more important. Otherwise, NW shall avoid scheduling any PUSCH resource overlapping with semi-static configured PUSCH resource. Proposal 3.4: Support Option-2 which leverages legacy rules. In legacy, the CSI reference resource in time domain for a CSI report is determined according to the reporting resource for the CSI reporting. For Rel-19 UEI/ED beam reporting, the CSI reference resource in time domain should be determined according to the second PUSCH. Note that it is possible that a RS measurement(s) is received after the CSI reference resource of first PUCCH transmission but before CSI reference resource of the second PUSCH for UEI/ED beam reporting, the RS measurement is still available for second PUSCH. Considering the second PUSCH is dynamically scheduled in Mode A and semi-static configured in Mode B, the CSI reference resource in mode A can follow the legacy CSI reference resource definition for aperiodic CSI reporting, while the CSI reference resource in mode B can follow the legacy CSI reference resource definition for semi-persistent CSI reporting. Proposal 3.5: Support one CPU is occupied with two CPU occupancy timelines (Option-2 for evaluation and Option-3C for PUSCH preparation) Besides of UEI/ED report generation to be transmitted in PUSCH, the event evaluation will occupy the CPU for L1-RSRP computation regardless of whether the UEI/ED beam reporting is triggered, as Apple/QC commented in their contribution. So, two CPU occupancy timelines are necessary: one occupancy timeline is for L1-RSRP computation during periodic event evaluation and the other one is for L1-RSRP computation for UEI/ED report preparation if triggered. On the occupancy timeline for event evaluation, the CPU occupy time should be a certain duration with the configured/specified length, starting after the first symbol of the earliest one of each measurement RS resource respective latest measurement RS occasion no later than an evaluation occasion, like Option-2 and CPU counting #1 in Option-3. On the occupancy timeline for UEI/ED report preparation in second PUSCH, we can leverage timeline of aperiodic CSI report for the PUSCH carrying the UEIED beam report in mode A and leverage timeline of SP CSI report for the PUSCH carrying the UEIED beam report in mode B. Hence, we are supportive with Option-2 and Option-3 with new proposed option (Option-3C): - Option-3: two CPU counting is applied: - CPU counting #1: from the first symbol of the earliest one of each transmission occasions of periodic CSI-RS/SSB resource for channel measurement for L1-RSRP computation, until X symbols after the last symbol of the latest one of the CSI-RS/SSB resource for channel measurement for L1-RSRP computation in each transmission occasion, - FFS: X, e.g., reusing Z3' - CPU counting #2: - Option-3A: from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report for Mode-A, or from the first symbol after the first PUCCH transmission until the last symbol of the PUSCH carrying the report for Mode-B. - Option-3B: countering for each PUSCH occasion. Assuming the last symbol ‘L’ of PUSCH, the CPU counter #2 starts from ‘L-∆_2’ symbol and ends at the last PUSCH symbol ‘L’. - Opition-3C: - In Mode A, from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report for Mode-A, - In Mode B, from the first symbol of the earliest one of periodic CSI-RS/SSB resource for channel measurement for L1-RSRP computation, respective latest CSI-RS/CSI-IM/SSB occasion no later than the corresponding CSI reference resource, until the last symbol of the PUSCH carrying the report for Mode-B. Illustration for Mode A: Illustration for Mode B: |
| Nokia | Q3.1.1 In our view, the utilization of the prohibit timer may be avoided by UE implementation, so our preference is to introduce no prohibit timer, i.e., Candidate#3. Q3.1.2A Although in RAN1#120-bis we agreed that a UE can be configured with multiple CSI report configurations associated to the same first PUCCH explicitly “regarding Event-2”, we think that such restriction of having only Event-2, i.e., only one event type, is not necessary. Because the agreed event types target different use cases, e.g., Event-2 to facilitate beam switching, Event-1 to avoid beam failure recovery and Event-7 to update the active TCI state list, we think that we should support that multiple CSI report configurations corresponding to different event types can be associated to the same first PUCCH. Furthermore, in RAN1#120-bis, we also discussed which CSI report the UE must transmit in case the conditions of multiple Event-2 CSI report configurations associated to a same first PUCCH were simultaneously met. Although different options were on the table (based on ID, based on triggering time), eventually it was decided to leave that selection up to the UE. In our perspective, such solution in case different event types are configured is too restrictive, and it is beneficial to allow the network to introduce some priority among the event types / CSI report configurations. The simplest way to do that is for example by using either the lowest CSI-ReportConfigId or legacy CSI report priority rules as defined in clause 5.2.5 of TS 38.214 (and as already discussed but dropped for the “just Event-2” scenario). With such solution, the network can configure for example a UE with a CSI report configuration associated to Event-2 and a CSI report configuration associated to Event-1 and, by assigning a lower ID to the Event-2 configuration, ensures that, in case the conditions of both CSI report configurations are simultaneously met, the CSI report associated to Event-2 is transmitted by the UE. Q3.2.1B We do not support such additional rule. For Mode A, it has already been agreed that the multiple CSI report configurations associated with the same PUCCH resource should also be associated with a same CSI-AperiodicTriggerState. We think on the other hand that there is no need to further specify that multiple CSI report configurations associated with a same CSI-AperiodicTriggerState should also be associated with the same PUCCH resource, and such optimization can be left up to the network implementation. For example, we can have two CSI report configurations associated to PUCCH resource X, and a third CSI report configuration associated to another PUCCH resource Y, with all three sharing the same CSI-AperiodicTriggerState without real ambiguity: if the UE uses the PUCCH resource X, then the UE will transmit a CSI report associated to one of the first two CSI report configurations, else if the UE uses PUCCH resource Y, then the UE will transmit a CSI report associated to the third CSI report configuration. Proposal 3.1.3A We do not support. For Case 2, where the 1-bit first PUCCH is collided/overlapped with a PUSCH, we prefer Option-4, where SR dropping rules are reused. Proposal 3.1.3B-1 We support the proposal. Proposal 3.1.3B-2 We support Option-1. Proposal 3.2 Among the options for the RRC candidate values, we support Option-1: 0, 1, 2, 4, 8, 16, 32, 64. Also, we do not fully understand the “Note” in the proposal: essentially, the value of X is already defined in symbols, so for a given value of X, the absolute time in ms already depends on the SCS, and in our perspective that note is not really needed. [Mod]: Let’s check other input. The intention is to clarify the ‘symbols’ corresponding to X ‘between first PUCCH and second PUSCH’. Proposal 3.3 We support the proposal. Among the three options, we prefer Option-1. Proposal 3.4 We support the proposal. We support Option-1 and can be fine with Option-2. Proposal 3.5 We support the proposal. Among the options, we prefer Option-1B. |
| FUTUREWEI | Q3.1.1: Support Candidate#1 and Option-1B, but only for mode-A. If retransmission support is required, Candidate #3 is clearly not a suitable option. Functionally, Candidate #1 and Candidate #2 can offer similar behavior, even with different mechanisms (i.e., a prohibit timer vs. a time interval for restriction). In our view, introducing the new concept of a "time interval" as in Candidate #2 is neither necessary nor convenient, and it would require additional standardization efforts without clear benefits. In that sense, Candidate#1 based on a legacy prohibit timer is preferred. Since prohibit timer has been defined for SR in legacy system, similar concept can also be applied to the first PUCCH in mode-A to avoid unnecessary resource requests. For mode-B, the second UL channel carrying the beam report is based on pre-configured resource(s). Even though the concept of legacy SR like signal can be reused for the first PUCCH in mode-B, the prohibit timer for the first PUCCH may not be needed. Note that it has been agreed to have a one-to-one mapping between the first PUCCH and the second UL channel. Since the second UL channel is pre-configured, its transmission timing is known at UE. Therefore, once UE transmits the first PUCCH, it knows when to send the second UL channel. Before transmitting the second UL channel, there is no need for UE to retransmit the first PUCCH. This rule can be applied without introducing a prohibit timer. After sending the second UL channel, if UE receives a negative acknowledgement, UE should be allowed to retransmit the first PUCCH without further delay. In this case, a prohibit timer is also not needed. Q3.1.2A: Support. The agreement can be applied to multiple events. Q3.1.2B: Support. Proposal 3.1.3A: Support. - Option-1: Prioritize the first PUCCH over the PUSCH, meaning the PUSCH is dropped. This approach may significantly impact PUSCH transmission performance, particularly when the PUSCH utilizes a large size of uplink resources and in URLLC-type services. - Option-4: Reuse the SR dropping rules, where the first PUCCH is dropped. This option does not guarantee timely and reliable transmission of the UEI/ED beam report, as the first PUCCH may be discarded. As a result, it could negatively affect beam management performance, especially in fast beam management scenarios at high frequencies. - Option-3: A compromise between Option-1 and Option-4, and this approach piggybacks a 1-bit indication of the first PUCCH into the PUSCH. This option can ensure the transmission performance of both the UEI/ED beam report and the PUSCH. Proposal 3.1.3B-1: Support. Proposal 3.1.3B-2: Support and prefer Option-1. Proposal 3.2: Support and prefer Option-1. Proposal 3.3: Support. Proposal 3.4: Support and prefer Option-2. Proposal 3.5: Support and prefer Option-1. [Mod]: Captured! |
| Huawei, HiSilicon | Q 3.1.1: We support Option-1A. A problem with Option-1B for Mode-A is that, if the first PUCCH is not received successfully and/or UE does not receive DCI scheduling UL resource for the beam report, UE cannot re-transmit the PUCCH since the prohibit timer has already started. As a result, the beam report cannot be transmitted in time. In addition, for Mode-B, PUCCH repetition is useful to compensate for the lack of NW feedback and improve the reliability. However, it is not clear whether/how PUCCH repetition is supported if Option-1B is agreed. On the other hand, in Option-1A, the prohibit timer starts only after the whole beam reporting procedure is completed. This facilitates a seamless support of PUCCH repetition and, if necessary, PUCCH re-transmission. In addition, we think the prohibit timer should be associated with the triggering beam so the first PUCCH cannot be transmitted only when the same beam that has triggered the last transmitted PUCCH re-triggers the event. For Candidate#2, Option-2A has similar design principles as Option-1A. However, we think Option-1A is clearer which has similar procedure as legacy SR. For Option-2B, it has the same problems as Option-1B as discussed above. Q3.1.2A: If the question is whether or not the same CSI-ReportConfig IE can configure multiple events of the same or different types (eg, one Event-2 and one Event-1 or two Event-2), our answer is “yes”. Considering the maximum number of CSI report configurations per cell is limited, we think one CSI report configuration associated with multiple events should be supported. If a CSI report configuration is associated with multiple events, an event information such as event id should be included in the beam report to enable gNB to determine which event has occurred and to correctly interpret each field of the beam report. [Mod]: Guess not. Please review the new proposal. Q3.1.2B: Support. we think multiple CSI report configurations associated with a same CSI-AperiodicTriggerState should also be associated with a same PUCCH resource. Let us explain through an example what happens if the FFS in the Agreement 120-bis in NOT supported: Assume that CSI-ReportConfig#1 and CSI-ReportConfig#2 are associated with PUCCH#1 while CSI-ReportConfig#3 and CSI-ReportConfig#4 are associated with PUCCH#2 and, at the same time, all four CSI-ReportConfigs are associated with the same trigger state#1 (see left sub-figure of the following figure). If UE sends PUCCH#1 for the report associated with CSI-ReportConfig#1 and, before receiving the corresponding scheduling DCI, sends PUCCH#2 for the report associated with CSI-ReportConfig#3, once gNB sends a scheduling DCI with the trigger state#1, UE will send its report associated with either CSI-ReportConfig#1 or CSI-ReportConfig#3 which include a CSI report configuration indicator to indicate the CSI report configuration corresponding to the report (see right sub-figure of the following figure). However, based on RAN1#120 agreement, the CSI report configuration indicator is a local ID of a CSI-ReportConfig among all CSI-ReportConfigs that are associated with the same PUCCH. Therefore, based on the CSI report configuration indicator, gNB cannot pinpoint the CSI report configuration associated with the received report. For example, if the CSI report configuration indicator is set to zero, gNB can only infer that either the report of CSI-ReportConfig#1 or the report of CSI-ReportConfig#3 is sent and, if the local ID is one, it can only infer that either the report of CSI-ReportConfig#2 or the report of CSI-ReportConfig#4 is sent. Proposal 3.1.3A: Not support. For Option-3, considering that gNB does not know whether the 1-bit first PUCCH is transmitted, piggybacking the 1-bit first PUCCH into PUSCH would require blind detection based on two different hypotheses and is computationally complex. Between Option-1 and Option-4 in RAN1 120-bis agreement, we think dropping the PUSCH is more reasonable. When the first PUCCH is transmitted to report based on Event-1 or Event-2, from UE respective, the candidate beam for the UL transmission is not reliable anymore. Therefore, when the 1-bit first PUCCH is overlapped with a PUSCH, PUSCH can be dropped since its performance is affected by the UL transmission beam. Proposal 3.1.3B-1: Support. Proposal 3.1.3B-2: Support Option-3. Supporting Option-1 means that one of SR or PUCCH for UEIBR will always be dropped. We don’t think such a design is acceptable since SR is configured for requesting data transmission while 1-bit PUCCH is configured for requesting/notifying a UEI beam report. Proposal 3.2: Support and prefer Option-4. Proposal 3.3: Support the proposal and prefer Option-1. For Mode A based report, if the beam report is dropped, a 3-step procedure of UE request, gNB scheduling, and UE reporting, is required for a retransmission. However, for Mode B based report and legacy aperiodic CSI reports, only a 2-step procedure is required for a report retransmission. Therefore, we think Mode A based beam report should have a higher priority than both Mode B based beam report and legacy aperiodic CSI report considering the complexity of its retransmission procedure. Compared to the legacy aperiodic CSI report, the UL resource carrying Mode B based beam report is preconfigured. Hence, if a Mode B based beam report is dropped, UE can retransmit the measurement result in another preconfigured UL resource after re-sending the notification in a first channel. This means that the reporting opportunity is more accessible for Mode B based beam reports than for legacy aperiodic CSI reports. Therefore, legacy aperiodic CSI reports should be prioritized over Mode B based report. Proposal 3.4: Support the proposal and Option-1. All the measurement results are already available before transmitting the first PUCCH. Further, PUCCH is transmitted only if at least one of the measurement RSs determine the event prior to PUCCH transmission. If, similar to legacy CSI, the CSI reference resource timeline is based on the reporting slot and not the PUCCH slot, it is possible that none of the new RS occasions after PUCCH but before CSI reference resource determine the event anymore. In such a case, UE would send the report while none of the reported beams determine the event which would be against earlier RAN1 agreement. Based on above discussion, we don’t think Option-2 or Option-3 is acceptable. Proposal 3.5: Support Option-1B. For both Mode A and Mode B based beam reporting, if the report notification/request is sent on the pre-configured first PUCCH resource, similar to the legacy CSI report, the end of the CPU occupancy time can be the last symbol of the scheduled or pre-configured PUSCH resource carrying the report. In turn, if the report notification/request is not sent on the pre-configured first PUCCH resource, CPU will not be occupied after the last symbol of this resource. Therefore, in this case, we think the end of CPU occupancy time can be the last symbol of the pre-configured PUCCH resource. [Mod]: Captured! |
| Panasonic | Q3.1.1: We do not support this proposal. First of all, we think the structure of this proposal is what is hindering the progress. It is the third or fourth meeting with this proposal structure and the discussion is going no where. So we think how we approach this design problem should change. First instead of focusing on a prohibit timer, it is important to discuss the expiration time of an event trigger. So far, the discussion in RAN1 stresses prohibit timer but an expiration timer is important because after a certain time period, channel conditions will change, and the event is not relevant anymore. And before the event trigger expires, the UE should keep trying to notify the NW about the event. After it expires, the trigger should be considered obsolete. This applies to both modes A and B. Regarding the prohibit timer for mode B, let’s discuss if the UE should start a timer after the transmission of the request bit on PUCCH. If no retransmission of the report is possible, then no timer is needed. We suggest discussing the need of any retransmission schemes for Mode B first. At this stage, we only support Candidate 1 for mode A only. Both options are fine. Q3.1.2A: Does this proposal include a mix of events? For example, events 1 and 7 are configured and both are triggered? I think in this case, some priority of the events should be discussed. [Mod]: Please review the updated proposal. Proposal 3.1.3A: We can only support Option 3 if the first FFS is removed so that blind decoding is not on the table. It is the last meeting, so why do we need FFS? [Mod]: Done! |
| NEC | Q3.1.1: thanks for capture our position, there should be a method to prevent frequent UEI report. Q3.1.2A: we would like to understand whether ‘multi-Events’ means multi-SAME-events or multi-different-events. Q3.1.2B: the additional rule is aligned with our understanding. Proposal 3.1.3A: support. For the FFS, if UEI is not triggered, there shall be no collision. Proposal 3.1.3B-1: OK Proposal 3.1.3B-2: support Option-1. Proposal 3.2: support. We are open to the values except Option-3 which basically says not supporting X. Proposal 3.3: we don’t understand why we need to set priority between UEI beam report for mode-A and UEI beam report for mode-B. They shall not be configured simultaneously. Other than that, we support UEI-BR>AP CSI >SP CSI on PUSCH Proposal 3.4: support Option 1. Proposal 3.5: support Option 1B. In addition, Z/Z’ can be 0 as there is no further processing after first PUCCH channel transmission. [Mod]: Captured! |
| Fujitsu | Q3.1.1: Option-1B is preferred. It is reasonable to start the prohibit timer after the first PUCCH especially when there are multiple PUCCH occasions before the beam report transmission. This can happen in both Mode A and Mode B. In Mode A, due to the scheduling uncertainty, there can be other PUCCH occasions between the first PUCCH transmission and the DCI reception. In Mode B, a PUCCH occasion can be only associated with a valid PUSCH occasion. If a PUSCH occasion is not valid, the PUCCH occasion will skip this PUSCH occasion and be associated with the next valid PUSCH occasion. Due to validation, there can be an additional PUCCH occasion between a PUCCH occasion and its associated PUSCH occasion. If the prohibit timer is not started after the first PUCCH transmission, the UE will transmit PUCCH multiple times before transmitting the beam report. For Option-1A, since the beam report has been transmitted on the PUSCH, the UE does not need to retransmit the beam report. There is no need to support retransmission for the CSI report. Therefore, it is not necessary to transmit PUCCH again for the same triggering. Instead, after transmitting PUSCH, the UE should stop transmitting PUCCH for the same triggering, i.e., the triggering should be released. For Option-2A and Option-2B under Candidate#2, it seems that the intentions are similar to those of Option-1A and Option-1B respectively. Compared with that, the prohibit timer is preferred, which is more aligned with the legacy design. Q3.1.2A: Support. Q3.1.2B: Support. Q3.1.3A: Support. Q3.1.3B-1: Support. Q3.1.3B-2: Option 1 is preferred. Proposal 3.3: Option-3 is preferred. In our view, the UEI beam report should be always prioritized over the legacy CSI report. This is to have a timely response to the event. Proposal 3.4: Option-1 is preferred. Proposal 3.5: Option-1B with modifications or a newly added Option-5 is preferred. The suggested modifications are highlighted in the red colour. For Option-1B (or Option-1) in general, it is suggested to add “the latest CSI-RS/SSB no later than the CSI reference resource”. This is more accurate to describe the CPU starting time. The following Option-5 is a newly suggested option. In legacy, aperiodic CSI report can be also based on the measurement of periodic CSI-RS. In this case, the CPU is still started after the PDCCH triggering the CSI report. This is similar to Mode-A. Therefore, the CPU starting time can be just defined with reference to PDCCH for Mode-A. Based on the PDCCH, both the UE and the gNB know that a UEI beam report will be transmitted. Following the same logic, for Mode-B, the CPU starting time can be just defined with reference to the first PUCCH. Based on the first PUCCH, both the UE and the gNB know that a UEI beam report will be transmitted. - Option-1: The start of CPU occupancy time is: the first symbol of the earliest one of current beam RS and new beam RS, each CSI-RS/SSB resource respective latest CSI-RS/SSB occasion no later than a CSI reference resource: - Option-1A: The end of CPU occupancy time is: the last symbol of the first PUCCH transmission occasion, regardless of whether the UEIRI is transmitted on such PUCCH transmission occasion. - Option-1B: - If the UEIRI is transmitted on the first PUCCH resource, the end point of CPU occupancy time is the last symbol of the scheduled or pre-configured PUSCH carrying the report. - If the UEIRI is not transmitted on the first PUCCH resource, the end of CPU occupancy time can be the last symbol of the first PUCCH resource. - Option-1C: The end of CPU occupancy time is: the last symbol of the second PUSCH. - Option-5: from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report for Mode-A, or from the first symbol after the first PUCCH transmission until the last symbol of the PUSCH carrying the report for Mode-B. [Mod]: From proposal perspective, Option-1 description seems fine. Option-5 is added as Option-6. |
| LG | Q3.1.1: Support Option A and B for Candidate#1. Especially for Option A, it is beneficial to start the prohibit timer after second PUSCH transmission. This is because, depending on network situation, gNB can updates beams, not right after UEI beam is reported. Therefore, UE should wait for gNB’s beam updates, for certain amount of time after reporting UEI beams. If UE transmit first PUCCH again without waiting gNB’s beam updates, it just results in the waste of UL resource and UE Tx power. Q3.1.2A: We support to apply the agreement for Event-1/7, as well, in order to achieve the same benefit of saving PUCCH resource for multiple CSI configurations. Also, considering the following agreement, the baseline is to apply the same design as Event-2 for Event-1/7. - The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise Q3.1.3A: Support Q3.1.3B-1: Support Proposal 3.3: Support Option-3. Since UEI beam report is initiated based on event evaluation from UE side measurement, it makes sense that UEI beam report has higher priority than legacy beam/CSI report. Proposal 3.4: Support Option-1. Proposal 3.5: Support Option 4 for UE to simply use one CPU exclusively for UEI BM. Specifically, with Option 4, UE uses a CPU to calculate and compare L1-RSRP for every event evaluation occasions, without occupying and returning it in each event evaluation occasion, which seems complicated. For other Options, we are open for further discussion but Option 4 is our first preference up to now. [Mod]: Captured! |
| CATT | 3.1.1: Support option-1A. 3.1.2A: Support that one PUCCH resource of first PUCCH can be associated with one or multiple CSI report configurations and the event type of the multiple CSI report configurations can be same or different based on NW configuration. 3.1.2B: open to discuss. 3.1.3A: Support. We do not support option-4 as the dropping rule for SR is not clarified for now. The companies’ preferences are updated accordingly. 3.1.3B: Open to disucss. 3.2: Support. 3.3: Not support. Not necessary to specify between the first PUCCH and AP/SP CSI. [Mod]: So, Option-2? 3.4: Support. 3.5: We failed to see the necessity of supporting this proposal. The legacy value (OCPU = 1) is ok for us. |
| Lenovo | Q3.1.1: Basically, we don’t think such prohibit behavior is necessary because it may cause more complicated UE behaviors. For example, whether the UE needs to perform the measurement on certain beams when the prohibit timer is running? However, if majority companies want to this feature, we can accept Candidate#2, which can avoid maintaining a timer. Q3.1.2A: There is no reason to restrict that only one type of event can be configured for a serving cell. Given that, the agreed principle should be applied for this case as well. However, for the multiple event case, it’s better to specify a priority among different events. Q3.1.2B: We don’t think the restriction is needed. Proposal 3.1.3A: Support. To avoid the gNB’s blind decoding, the 1-bit indication is always multiplexed in the PUSCH, regardless that UEI beam report procedure is triggered. Proposal 3.1.3B-1: Support. Proposal 3.1.3B-2: Support Option 1. Proposal 3.2: We are fine with either option. Proposal 3.3: Support Option 2. Mode A is like an aperiodic CSI while Mode B is like SP CSI on PUSCH. Considering the potential spec effort and impact, Option 2 is enough. Proposal 3.4: Support Option 1 to have an unified solution for both Mode A and Mode B. Proposal 3.5: Support Option-2. Actually, the UE needs to perform measurement for each current beam and new beams transmission occasion to determine whether the condition for the configured event is satisfied. Even a beam report corresponding to a new beam is triggered, the UE cannot stop the followed beam measurement, thus the CPU should be periodically occupied. [Mod]: Done! |
| Sharp | Q3.1.1: We support Option-1B to prevent from frequent first-PUCCH retransmission after the gNB successfully receives it. Q3.1.2A: We support that one first-PUCCH resource is associated with multiple CSI report configurations for different events, but we don’t support that one CSI report configuration is associated multiple events because an additional bit for event indication is needed. Q3.1.2B: Support. If different CSI trigger states are associated with multiple CSI report configurations corresponding to one first-PUCCH resource, the NW cannot identify which trigger state should be indicated in response to the request on the first-PUCCH and cannot identify which UEIBR satisfies the event condition. Proposal 3.1.3A: Support. Proposal 3.1.3B-1: Support. Proposal 3.1.3B-2: Support Option-1. Proposal 3.3: Support Option-2. According to the agreement, the prioritization rule of SP-CSI on PUSCH is reused for UEIBR for Mode-B. Hence, UEIBR for Mode-B should have the same priority value as SP-CSI on PUSCH. Furthermore, in the current spec, AP-CSI has highest priority value (i.e., 0). Hence, UEIBR for Mode-A should have the same priority value as AP-CSI. In Option-2, the legacy time domain behavior can be reused. Proposal 3.4: Support Option-1. There was no consensus on supporting of UEIBR without condition-satisfied beam, and at least one of reported new beams needs to meet the event condition. However, in Option-2/3, all the reported new beams potentially do not meet the event condition due to ranking by RSRP value at the CSI reference resource. Proposal 3.5: Support Option-1A. This is because we support that CSI reference resource is associated with a slot of first-PUCCH. In Option-2, the evaluation occasion is not defined in our understanding, while the evaluation periodicity is defined. [Mod]: Good point! |
| NTT DOCOMO | Q3.1.1: We support Option-1A. Q3.1.2A: Support to apply for Event-1/7. Q3.1.2B: We are OK to introduce additional restriction, but it may not be essential since the error case could be avoided by NW. Proposal 3.1.3A: Support Proposal 3.1.3B-1: Support Proposal 3.1.3B-2: Support Option-1. Proposal 3.2: Support. Proposal 3.3: We support Option-1. Proposal 3.4: We support Option 3. Proposal 3.5: We support either Option-1C or Option-3A. [Mod]: Captured! |
| Qualcomm | Proposal 3.1.1: We still support the proposal with Candidate#1; Option-1B, to prevent misuse/abuse of the first PUCCH resources. But, this time, we are okay to capture it only for Mode-A. As we commented in the offline discussion, aside from mis-detecting the first PUCCH, which has a very low probability, there may be many reasons for the network not sending the grant for the second PUSCH. For instance, the network may decide not to update the UE’s current beam because it is deemed unnecessary, despite an event being triggered and the first PUCCH being received. For this, there was an offline argument on how the network can determine that the beam update is unnecessary without receiving the beam report (i.e., the PUSCH). One possibility is that, with joint TCI state config, the NW may anticipate the DL beam quality from recent UL reception from the UE. Additionally, if there has already been another beam update for the same UE recently, the network may not want to update the beam again. Additionally, if there is no immediate downlink traffic for the UE, the network may decide to postpone the beam update. Q3.1.2A: Yes, the report transmission procedure should be supported for Event-1 and Event-7. Currently, there does not appear to be any need for special handling for other Event types besides Event-2. Q3.1.2B: We support the additional rule as it provides further clarity on expected behavior. While we generally believe that the network can handle this independently, providing additional clarification causes no harm. Proposal 3.1.3A: We support the proposal. Proposal 3.1.3B-1: We support the proposal Proposal 3.1.3B-2: We support the proposal, with Option 1. Proposal 3.2: It is suggested that the discussion be postponed or merged into the CSI processing timeline/criteria discussion in Issue 3.5. The value of X symbol appears to be related to the CSI processing (i.e., CPU occupation duration), associated with the legacy UE capability "beamReportTiming", which could serve as a baseline for X. The value X might be smaller than "beamReportTiming", since "beamReportTiming" includes both RS measurement/processing and CSI calculation, while X may only include the CSI calculation. However, it remains unclear how much smaller it could be and whether value "0" is feasible from the legacy CSI processing framework. Proposal 3.3: We support Option-2. Prioritization between Mode-A and aperiodic beam reporting is unnecessary, as it is up to the network. For the prioritization between Mode-B and SP CSI reporting on PUSCH, we also believe that the network can avoid the collision by configuration, making it less critical. It is also noted that there is no prioritization between multiple colliding SP CSI reports on PUSCH in the legacy specification, because it is up to network. Proposal 3.4: We support Option-3 for a simple, consistent design. Proposal 3.5: We support Option-3A. From the perspective of the PHY layer, CSI processing involves: (1) calculating and storing CSI metrics (e.g., L1-RSRP) and (2) compiling the stored CSI metrics into a CSI report of the configured format. In the legacy CSI reporting framework, such as periodic, semi-persistent, and aperiodic CSI reporting, (1) and (2) are indistinguishable since they typically happen together. However, for UE-initiated beam reporting, (1) and (2) are decoupled, and CPU durations for (1) and (2) should be separately accounted. For Process (1) in UE-initiated beam reporting, there is a similar process in legacy CSI reporting, i.e., Rx beam refinement procedure, and the same CPU occupancy duration can be applied, as captured in “CPU counting #1” under Option-3. For Process (2) in UE-initiated beam reporting, there is also similarity with legacy aperiodic CSI reporting, and the same CPU occupancy duration can be applied, as captured in “CPU counting #2, Option-3A” under Option-3. |
| HONOR | Proposal 3.1.1: We support Option-1A to align with legacy SR. In addition, the granularity of the prohibit timer should be discussed. It may be per CSI report configuration. Per new beam for each CSI report configuration is not preferred as UE has to maintain too many prohibit timers. Q3.1.2A: Support. It’s better to clarify first whether one CSI report configuration can include more than one events or not. Q3.1.2B: Support. Proposal 3.1.3B-1: Support. Proposal 3.1.3B-2: Support and prefer Option-1. Proposal 3.2: Agree with Qualcomm, X is somehow related to the CSI report timeline. We can postpone this proposal. Proposal 3.3. Support option-3. We’re also fine that both mode A and mode B share the same priority level as both are urgent to reflect the link quality. In addition, we agree with Samsung that when first PUCCH is associated with multiple CSI report configurations, the priority of the UEI beam report is determined by the CSI report configuration with highest priority from the multiple CSI report configurations. Otherwise, UE behavior is not clear. Proposal 3.5: There are too many options on the Table. To move forward, it’s better to clarify the following three questions： Q1：whether the CPU occupancy time should be always assumed by UE regardless of whether the first PUCCH is transmitted or not, especially for Mode B? Q2: whether RS measurement during the event evaluation should occupy CPU or not? Q3: when the first PUCCH is associated with multiple CSI report configurations, whether UE only assumes CPU is occupied only for a single CSI report configuration? Q1: In legacy, the CPU occupancy time is determined according to DL activation/triggering command so that UE and network have a consistent understanding on the CPU occupancy. However, if CPU occupancy time depends on whether the first PUCCH is transmitted or not, it means CPU occupancy time is determined according to UL signal initiated by UE. However, network may miss-detect the first PUCCH so that the CPU occupancy time is not synchronized, which may impact other CSI reports that are processed simultaneously. With that being said, we prefer that the CPU occupancy time should be always assumed regardless of whether the first PUCCH is transmitted or not especially for Mode B. Q2: RS measurement during the event evaluation should occupy CPU especially when the time window is configured. Because UE has to conduct the L1-RSRP measurement during the event evaluation. The CPU occupancy rule can follow the same mechanism as legacy Rx beam sweeping when report quantity set to ‘none’. Q3: There is only one UEI beam report in the PUSCH. Assuming all the CSI report configurations should occupy CPU may waste UE computation resource. Therefore, when the first PUCCH is associated with multiple CSI report configurations, UE only assumes CPU is occupied only for a single CSI report configuration during CSI calculation. |
| Mod_V24 | Update per companies input, especially for adding new proposal for Issue 3.1.2A and some update for 3.1.3A, 3.1.3B-2 and 3.5 @all, please be constructive for Issue 3.5. If failed to be converged, the only outcome is nothing!! Let’s identify an way-forward solution. |
| KDDI | Q3.1.1 We support both Option-1A and Option-1B. Q 3.1.2A/B: We don’t support. Proposal 3.1.3A: We support. Proposal 3.2: We support. We are also OK with all options, at least X=0 should be supported. Proposal 3.4: We prefer unified solution for both modes. [Mod]: Capture companies input. |
| vivo | Q3.1.1: In our view, Candidate#1 and Candidate#2 are same with different description. From the perspective of RAN1’s specification, Candidate#2 is more suitable. To avoid frequent report and ensure the transmission of the second UL channel meanwhile, Option -2A is more reasonable. Q3.1.2A: We agree with report configuration framework and report mechanism agreed for Event-2 are also applied for Event-1 and Event-7 if only one event is associated with a report configuration for UE-initiated/event-driven beam report. Q3.1.2B: In our view, proper configuration by gNB can such a corner case so it is unnecessary to be specified. Proposal 3.1.3A: Support Proposal 3.1.3B-1: Support Proposal 3.1.3B-2: Support Option-1 Proposal 3.2: Support and prefer Option-1. Proposal 3.3: Support and prefer Option -1. Proposal 3.4: Support Option-1. Proposal 3.5: Do not support this proposal. According to the agreement achieved before, two event evaluation mechanisms have been supported. Considering that the trigger conditions are different for the two event evaluation mechanisms, the CPU occupation rule should be different. For the basic feature of event evaluation, i.e., at least one event instance occurs, the measurement occasion used to evaluate the trigger condition should be the latest RS occasion before the CSI reference determined based on the first PUCCH. Thus, the corresponding CPU should be occupied from the first symbol of the earliest RS resource with the measurement occasion. For the ending of the occupied CPU is associated with the transmission of the first PUCCH and the report mode. For Mode A, as DCI scheduling the second UL channel is a feedback for the first PUCCH indication, if the DCI is received within a duration after the transmission of the first PUCCH, as shown in Figure 1, the ending is the last symbol of the second UL channel; Otherwise, the ending is the last symbol of the duration after the first PUCCH. For Mode B, due to the absence of the feedback for the first PUCCH indication, the ending is the last symbol of the pre-configured PUSCH. Figure 1 For the time-window-based event evaluation, UE needs to always perform measurement and caching within the time window. However, the time window used to event evaluation is not fixed. Therefore, it is difficult to specify the CPU occupation rule for this case. To simplify specification design, one most straightforward way is to occupy the corresponding CPU from the RRC configuration of UE-initiated/event-driven beam report is provided and to the RRC configuration is released, i.e., Option-4. [Mod]: Of course, we may not have this CPU occupation finally like BFR, but, for counting, hopefully you can live with OCPU = 1 as a minor step. |
| Ericsson | Q3.1.1: Preferably, we would have 1A+1B for Mode A and 1A for mode B Q3.1.1A: only one UEI beam report should be included in the PUSCH Q3.1.2B: Don’t support. This is up to NW configuration. P3.1.3A: We can support if the bits are always included. P3.1.3B-1: Support P3.1.3B-2: Support P3.2: option-1 or option-4. Option-3 sounds strange, if we only have one value, why do we need an RRC parameter. P3.3: Without spec change, mode A would be the same as any other A-CSI report, which is fine and preferable. Hence we prefer option-2. P3.4: support option-3. P3.5: As a general remark, the CPU occupancy cannot depend on the outcome of measurements, since the NW cannot reasonably compensate for varying output of the measurements. For example, option-1B would require that the NW adjusts its requests for CSI processing when it receives the first PUCCH, which is unreasonable. The same is true for option-3A, mode B: the NW cannot adjust its CSI requests based on the detection of the first PUCCH: in practice, the NW would have to reserve the CPUs between every PUCCH and PUSCH instance. Option-4 is the simplest and most conservative approach. In our view, option 1C is the same as option-4. Option 1-3 seem to agree that the CPU counting starts when any RS is received. In option 1, the CPU occupancy ends based on where the PUCCH or PUSCH is. For option-1A, (and also partly 1B) the time between the RS and the PUCCH can be extremely short, so it’s not clear to us if that works. In option 2, the CPU occupancy ends some time after the measurement RSs end. In option 3, the CPU occupancy ends some time after the measurement RSs end. There is an additional CPU occupancy associated with the reporting. Could we agree on option-2, and then discuss further if we need additional CPUs during the reporting? [Mod]: Captured! |
| IDC | Q3.1.1: Support Option-1B at least for Mode-A, similar to legacy SR, preventing from frequent retransmissions of the first PUCCH. In our view, Option-1A does not provide benefits for Mode-A, while it can be considered for Mode-B. Q3.1.2A: Support. The agreement made for Event-2 can be reused in principle for Event-1 and Event-7 as well. [Mod]: Captured! |
| Apple | Q3.1.1: Support both Opt.1A and Opt.1B. The selection between these two options can be left for NW configuration. The rationale behind is that: theset two mechnisms targets for different use cases. For Opt.1B, it is same as SR prohabit time and targets for the case that to control UL overhead and avoid any first PUCCH re-transmission during a timer interval. For Opt.1A, it ensures first PUCCH channel high reliabilty by allowing a first PUCCH channel re-transmission until reception of UL grant. A decision best left to NW scheduler. For example, it is more practical to apply scenario 1 for UEs located in the cell center or those with good link quality, while scenario #2 should be considered for cell-edge UE to ensure the reliabilty of the first channel. Q3.1.2A: Support different events. There is no reason to restrict this to a single event. The advantage of ‘multiple-to-one’ mapping between CSI reports and first PUCCH resource is its ability to reduce PUCCH overhead by taking advantage of the low probability of multiple reports being triggered simultaneously. The NW has the flexibility to configure either the same or different events for multiple CSI reports. Q3.1.2B: Support Without this restriction, RAN1 would need to discuss how to determine CSI report content when processing the corresponding CSI request codepoints. For example, one initial PUCCH is associated with CSI reports <#1, #2>, while another is associated with CSI reports <#3, #4>. Since both PUCCHs are linked to codepoint <101>, the question arises: What should the report content in PUSCH be when codepoint <101> appears in the scheduling DCI? Should it include CSI reports <#1, #2>, <#3, #4>, a selection from these sets, all four reports, or another combination? Additionally, if we consider the scenario of PUCCH miss detection, the network would need to perform blind detection on the CSI content—something we have deliberately avoided in all UEIBR designs. At this point, we do not see any strong use case to justify allowing this. Proposal 3.1.3A : Support In general, dropping UEIR-I bit should be avoided to gurantee the robust beam linkage. The UEIR-I bit is generated precisely when its corresponding event is triggered, meaning it carries critical operational information—its loss can lead to severe issues such as beam failure (as observed in Event-1) or to noticeable performance degradation (as in Event-2 or Event-7). By ensuring that these bits are preserved, network reliability and service quality remain intact, preventing cascading failures. Proposal 3.1.3B-1: Support. Proposal 3.1.3B-2: Support Opt.1. Proposal 3.2: We agree that the value ‘0’ should be removed, as the purpose of parameter ‘X’ is to enable NW to re-schedule the CG-PUSCH to other UEs and allow flexiblity for UE in CSI processing on PUSCH. Opt.1 is acceptable to us once value ‘0’ is removed. Proposal 3.3: Support Opt.1. Proposal 3.4: Support Opt.2. Proposal 3.5: Our preference is Opt.3 and particularly Opt.3B for CPU counting #2. At a high level, there are two key distinctions between a legacy NW-initiated CSI report and a UE-initiated beam report. First, in the case of a NW-initiated beam report, after measurement, the CSI report is always transmitted by the UE on either the PUCCH or PUSCH as configured. Conversely, for a UEIBR, the CSI report transmission is conditional-it is only sent if a specific triggering event occurs. Otherwise, the UE may perform the measurement without actually reporting the results. Secondly, for UEIBR, the UE is required to perform measurements at every configured measurement occasion, regardless of whether a report is ultimately transmitted. Therefore, for UEIBR procedure, counting the CPU usage based on transmission on first PUCCH or second PUSCH is not technically accurate. The fundamental principle of CPU counting is to prevent discrepancies between the gNB and UE in terms of occupied CPU at any given time. This should be followed for UEIBR report as well. It is unclear whether and how the Opt.1 effectively meets this design requirement. For example, if mutliple evaluation occasions (EOs) exists for a given first PUCCH occasion (such as four EOs as illustrated in MTK’s comment), what marks the starting point of CPU occupacy in Opt.1? is it the starting of first EO or the 4-th EO preceding the CSI reference resource? If it is the former, CPU occupancy is overestimated; If the latter, it is signficantly underestimated, since the UE is requried to perform L1-RSRP measurement/comparsion in the first three EOs as well. Opt.2 and Opt.3 account for this consideration by segmenting CPU counting for each EO, ensuring proper allocation since the UE is mandated to conduct measurements, thereby utilizing CPU resources. Opt.3A and 3B are nearly identical, except a smaller ∆2 is used in 3B but the T_process 2 is used in 3A. We are flexible to go with Opt.3A, balancing the tradeoff between spec impact and CPU counting accuracy aspects. Opt.4 is the most simplest method for CPU counting but is also the least efficient, leading to signifiantly waste UE’s CSI processing capability and negatively impacting spectrum efficiency for both the UE and NW. [Mod]: Captured! |
| Transsion | Q3.1.1: Support Option-1B to prevent frequent retransmissions of the first PUCCH as legacy SR. Q3.1.2A:Support that one PUCCH resource of first PUCCH can be associated with one or multiple CSI report configurations, where the event type of the multiple CSI report configurations can be same or different. Proposal 3.1.2B: Support the proposal so that the ambiguity of CSI report configuration can be avoided. Proposal 3.1.3A: Support the proposal and support that the 1-bit indication is always multiplexed in the PUSCH. Proposal 3.1.3B-2: Support Option-1. Proposal 3.2: Support Option-1 Proposal 3.3: Support Option-3. The UEI beam report for mode-A and mode-B should be always prioritized over the A-CSI report. Proposal 3.4: We prefer Option-1. Proposal 3.5: Prefer Option-1A. We share the similar view with ZTE. [Mod]: Captured! |
| Mod_V32 | Capture companies’ input. |
| ETRI | Q3.1.1: Our view is properly captured. Q3.1.2A: We support Option-1 in Proposal 3.1.2A. We see no Q3.1.2B: We do not support the Proposal 3.1.2B because it is too restrictive. Proposal 3.1.3A: We support the proposal. Proposal 3.1.3B-1: Support Proposal 3.1.3B-2: Support Option-1. Proposal 3.2: We are not sure of having 0 and can live with option-1 and option-4. Proposal 3.3: Option-2 is the most reasonable one in our understanding. Proposal 3.4: Support Option-3 because both modes are essentially aperiodic reporting in a way of UEI format. Proposal 3.5: We believe that the first report is similar to the periodic beam report whereas the second report is similar to the aperiodic beam report. We are fine option-1C and we are not fully fine to counting #1 but fine to counting#2 in Option-3. In short, we support Option-1C. [Mod]: Captured! |
| Mod_V32 | Capture companies’ input. |
| China Telecom | Q3.1.1: Support both Option A and B for Candidate#1. Consider that there may be more than one PUCCH escalation time in the measurement time window, two types of Prohibit timers is feasible for the UEIBR. The first prohibit timer is used to disable some PUCCH transmission occasions, thus to avoid the frequent transmissions of the first PUCCH for the same new beam. And on the other hand, if the UEIBR has been triggered within a measuring time window, The second prohibit timer is used to delay the transmission of the first PUCCH. Two types of prohibit timer is illustrated in the Figure 1 and 2, which are marked as Prohibit Timer#1 and Prohibit Timer#2, respectively. Figure 1 The PUCCH transmission with Prohibit Timer#1 when M=3 for the rigger-event evaluation. Figure 2 The PUCCH transmission with Prohibit Timer#2 when M=3 for the rigger-event evaluation. Proposal 3.1.3A: Support and prefer Option-3. Proposal 3.1.3B-1: Support. Proposal 3.1.3B-2: Prefer Option-1. Proposal 3.2: support. Prefer Option-2/3/4 to reduce the report latency. Proposal 3.3: we prefer Option-3. We think the UEIBR precedes over the CSI report. |
| Samsung2 | For Proposal 3.1.3B-1: The intention it to multiplex multiple UEIRI, but the wording “1 bit first PUCCH” seems not aligned with the intention. We suggest to replace “codepoint”with “value” to follow the spec language. Suggest the following update. On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) the 1-bit a number of L (L>=1) first PUCCH(s) with UEIRIs is(are) collided/overlapped with a PUCCH format 2/3/4 carrying HARQ/CSI, reuse the legacy SR multiplexing rule. - The codepoint in the field value of log2L+1 bits is generated based on an ascending order of the values of PUCCH resource ID associated with the first PUCCHs. [Mod]: Done! For Proposal 3.1.3B-2: we suggest the following update On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) the 1-bit a number of L (L>=1) first PUCCH(s) with UEIRIs is(are) is collided/overlapped with a PUCCH format 2/3/4 carrying HARQ/CSI/SR. - Option-1: Regarding log2K+1 bits, extend the legacy K SRs to K-L SRs and L UEIRIs field of log2K+1 bits to a field of log2K+L+1 bits of representing at most one of positive SR or positive UEIRI - The codepoint in the field is generated based on a first ascending order of the values of SR ID associated with the SRs and a second ascending order of the values of PUCCH resource ID associated with the first PUCCHs. - A value representing positive SR is smaller than a value representing positive UEIBR. - If one of the SRs is a positive LRR, the value of the log2K+L+1 bits indicates the positive LRR, else, if one of the UEIBRs is a positive UEIBR, the value of the log2K+L+1 bits indicates the positive UEIBR. - An all-zero value for the log2K+L+1 bits represents a negative SR or UEIRI value across all K LRR/SRs and/or L UEIRIs. - Option-2: Append the UEIRI to HARQ-ACK, if any, with joint coding. - Option-3: The field of log2L+1 bits for UEIRI is appended to the field of log2K+1 bits for SR/LRRs. [Mod]: Per my understanding, the both description is the same, and then the description in the current spec can be up to editor. |
| Mod_V38 | Capture companies input |

## Issue 4 – Cross-CC measurement/report

Table 4-1 Summary for Issue 4

| # | Issue | Companies’ view and Recommended Proposal |
| --- | --- | --- |
| 4.1 | Mechanism for configuration of current/new beam RS and indicated TCI state | [120] Agreement On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, the following working assumption is confirmed with the following modification. - The first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from different PUCCH groups - Subject to separate UE capability - Note: From RAN1 perspective, the above does NOT introduce any spec impact except for UE capability signaling [120bis] Agreement On cross-CC beam report measurement for UE-initiated/event-driven beam reporting, regarding Event-2, introduce an RRC parameter to indicate ServCellIndex on which the indicated TCI state used to determine the current beam RS is applied - FFS: Indication of the cell that corresponds to the configured first PUCCH resource in CSI report config including whether it is necessary FL Note: - Per online and offline discussion, we have the following proposal from proponent. Then, let’s have a quick check. Proposal 4.1: On cross-CC beam report measurement for UE-initiated/event-driven beam reporting, regarding Event-2, introduce an RRC parameter to indicate one from {SpCell, PUCCH-Scell} as the cell of the configured PUCCH. Supported by: Huawei/HiSi, TCL, NTT DOCOMO, vivo, Ofinno, CATT, OPPO, ZTE, MediaTek, Futurewei, Huawei/HiSi, NEC, Lenovo, Sharp, NTT DOCOMO, Qualcomm, vivo, Apple, Transsion, ETRI, China Telecom, Not supported by: Samsung, xiaomi, Nokia, Ericsson, |
| 4.2 | Cross-CC beam measurement | [119] Agreement Regarding for the evaluation periodicity for determining Event-2 instance [at least when DRX is not configured], at least Alt-1 is supported for the single-CC beam reporting (for case that the CSI report configuration and RS for the current beam and new beam(s) are in the same CC). - Alt-1: The periodicity of the current beam RS should be the same as that of the new beam RS(s). - The evaluation periodicity is the same as the periodicity of the current and new beam RS(s). - Alt-2: The periodicity of the current beam RS can be different from that of the new beam RS(s) - Alt-2_1: The evaluation periodicity is the same as the periodicity of the current beam RS. - Alt-2_2: The evaluation periodicity is the same as periodicity of the new beam RS. - Alt-2_3: The evaluation periodicity is the same as shortest periodicity of the current beam RS and new beam RS(s). - Alt-2_4: The evaluation periodicity is the maximum of {X ms, shortest periodicity of the current beam RS and new beam RS(s)}. - Alt-2_5: The evaluation periodicity is the same as largest periodicity of the current beam RS and new beam RS(s). Note: There is the same periodicity for the new beam RS(s). FFS: Down-selection among above Alt(s) for cross-CC beam reporting. Strong concerns were expressed by Huawei and CATT that if only Alt-1 is supported in the end, the feature will not be practical. [120] Agreement On cross-CC beam report measurement for UE-initiated/event-driven beam reporting, regarding Event-2, the following is supported - The current beam RS and new beam RS(s) are in the same CC - The above does NOT imply to preclude the cross-CC TCI indication. - The CC on which the indicated TCI state is applied, and the CC in which new beam RS(s) are can be the same or different. - FFS: Whether/how to introduce an RRC parameter to indicate the CC on which the indicated TCI state is applied. FL Note: Per companies input, we need to handle the left-over issue for cross-CC, while considering the support of cross-CC beam measurement/report. Proposal 4.2: On cross-CC beam report measurement for UE-initiated/event-driven beam reporting, regarding the evaluation periodicity for determining event instance, the evaluation periodicity of the current beam RS should be the same as that of the new beam RS(s). - Option-1: The evaluation periodicity is the same as the periodicity of the current and new beam RS(s). - Option-2: The evaluation periodicity is the maximum of {X ms, the periodicity of the current and new beam RS(s)} Option-1: NTT DOCOMO, Nokia, Futurewei, Huawei/HiSi, Fujitsu, LG, Lenovo, Sharp, Qualcomm, Ericsson, Transsion, ETRI, Option-2: Apple, CATT, OPPO, Ofinno, ZTE, NEC, vivo, China Telecom, |

Table 4-2 Company input for Issue 4

| Company | Input |
| --- | --- |
| Mod V00 | - Please input your comment/preference to Issue 4.1~4.2. |
| OPPO | 4.1: support. It is really unclear whether we need the cell indication for PUCCH resource or not per the current spec and it seems people have different understanding. But proposal 4.1 is the safe way we can take. 4.2: Option-2 is preferred, which at least gives a lower bound of UE evaluation interval. [Mod]: Captured! |
| Ofinno | Proposal 4.1. Support the proposal. When we look at the current PUCCH spatial relation activation/deactivation MAC-CE, both serving cell ID and BWP ID are included which is unnecessary if the PUCCH resource ID is global. Therefore, {SpCell, PUCCH-Scell} as the cell ID should be supported. On top of that, regardless of PUCCH resource ID is global or not as discussed in our tdoc[R1-2503554], further clarification is required regarding the expected UE behavior upon detecting the event if the configured PUCCH resource located in an uplink BWP that is currently inactive. In this scenario, the UE is unable to transmit because the necessary uplink resources are unavailable in the currently active BWP. Initiating a random-access procedure, similar to legacy SR procedure in the case active uplink BWP does not have PUCCH resource for SR, does not help, especially in mode-B of UE-initiated CSI reporting if the network is not aware of the CSI reporting triggered at the UE. Consequently, the UE should delay the CSI reporting process until the network activates an appropriate uplink BWP containing the required resources. One solution can be to perform BWP switching to the uplink BWP configured with the first PUCCH resource, or multiple PUCCH resource IDs, one per uplink BWP may be configured in the first PUCCH resource configuration. On the other hand, if this case is not to be addressed, the UE should either stop or not even start event evaluation when the configured resources are not in the active uplink BWP. In another words, what is the point of performing event evaluation if the UE won’t be able to transmit the first PUCCH. [Mod]: Captured! Proposal 4.2. Support option-2. We note that when X ms is configured by the periodicity of the current and new beam RS(s), option 2 is identical to option 1. Considering longer evaluation periodicity may provide UE power saving due to infrequent evaluation, which is configurable by gNB, we think option 2 seems better choice. [Mod]: Captured! |
| Samsung | Proposal 4.1: NOT support. As pointed out by FL, the issue was discussed in previous RAN1 meetings and not supported with the understanding that the PUCCH resource ID is a global parameter, and the PUCCH resource ID can be used to determine the corresponding serving cell. Even if some companies argue that PUCCH resource ID is a local ID, it can be up to network implementation to ensure that the IDs are different for PUCCH resources on different cells. We don’t see any issue without the proposal. Actually, the proposal contradicts with the Note from the agreement made in RAN1#120, this kind of discussion should be avoided especially for the last meeting to complete the WID. Proposal 4.2: The main bullet is already supported according to the agreement made in RAN1#119. If there is no consensus to support different periodicities, RAN1 can simply conclude the following. There is no consensus for supporting different periodicities of new beam and current beam for Event 2. [Mod]: Herein, we also fix the left-over issue for other events for the main bullet. Thanks. For the evaluation of new beam and current beam with the same periodicity, it should be a common understanding that the comparison is restricted to transmission occasions within one (same) periodicity, i.e., the time gap between a transmission occasion of the new beam and a transmission occasion of the current beam that are used for event evaluation should be no larger than their periodicity. For example, as shown in the conceptual figure below, the comparison should be based on TO#1 of new beam RS and TO#1 of current beam RS, comparing the RSRP of TO#1 of new beam RS and TO#2 of current beam RS should better be NOT allowed – otherwise, having the restriction of the same periodicity for both new and current beam RSs would become meaningless and the event evaluation/triggering may become less reliable. Based on the above discussions, we propose the following: For determining an Event-2 instance, the time offset between a transmission occasion of the new beam and a transmission occasion of the current beam should be less than the periodicity. [Mod]: Interesting! Let’s check other’s views. |
| ZTE | Proposal 4.1: Fine to support. Proposal 4.2: Prefer Option-2. [Mod]: Captured! |
| MediaTek | Proposal 4.1. Support the proposal. After specification checking, PUCCH resource configuration is BWP-specific, such that the PUCCH resource ID is not a global ID. So, we support to have this RRC parameter. [Mod]: Captured! |
| Nokia | Proposal 4.1 We do not support the proposal. In our view that RRC parameter to indicate the CC where the PUCCH is configured is not needed. Proposal 4.2 We support the proposal, and prefer Option-1. [Mod]: Captured! |
| FUTUREWEI | Proposal 4.1: Support. Proposal 4.2: Support and prefer Option-1. [Mod]: Captured! |
| Huawei, HiSilicon | Proposal 4.1: Support. If the proposal is not agreed, UE has no way to know whether the configured PUCCH resource is on the special cell (PCell, PsCell) or on a PUCCH-scell. In contrary to Samsung view, we have not found any evidence in any spec that PUCCH resource Id is global. In fact, there is a clear evidence on the contrary since pucch-Cell is already used in PDSCH-ServingCellConfig to indicate whether the PUCCH that carries HARQ-ACK is on the SpCell, if the parameter is absent, or on a specific PUCCH-SCell, if the parameter is configured (if the PUCCH resource ID was global, introducing such RRC parameter would have been useless): PDSCH-ServingCellConfig ::= SEQUENCE { codeBlockGroupTransmission SetupRelease { PDSCH-CodeBlockGroupTransmission } OPTIONAL, -- Need M xOverhead ENUMERATED { xOh6, xOh12, xOh18 } OPTIONAL, -- Need S nrofHARQ-ProcessesForPDSCH ENUMERATED {n2, n4, n6, n10, n12, n16} OPTIONAL, -- Need S pucch-Cell ServCellIndex OPTIONAL, -- Cond SCellAddOnly ..., [[ maxMIMO-Layers INTEGER (1..8) OPTIONAL, -- Need M processingType2Enabled BOOLEAN OPTIONAL -- Need M ]], [[ pdsch-CodeBlockGroupTransmissionList-r16 SetupRelease { PDSCH-CodeBlockGroupTransmissionList-r16 } OPTIONAL -- Need M ]], [[ downlinkHARQ-FeedbackDisabled-r17 SetupRelease { DownlinkHARQ-FeedbackDisabled-r17 } OPTIONAL, -- Need M nrofHARQ-ProcessesForPDSCH-v1700 ENUMERATED {n32} OPTIONAL -- Need R ]] } PDSCH-CodeBlockGroupTransmission ::= SEQUENCE { maxCodeBlockGroupsPerTransportBlock ENUMERATED {n2, n4, n6, n8}, codeBlockGroupFlushIndicator BOOLEAN, ... } PDSCH-CodeBlockGroupTransmissionList-r16 ::= SEQUENCE (SIZE (1..2)) OF PDSCH-CodeBlockGroupTransmission DownlinkHARQ-FeedbackDisabled-r17 ::= BIT STRING (SIZE (32)). pucch-Cell The ID of the serving cell (of the same cell group) to use for PUCCH. If the field is absent, the UE sends the HARQ feedback on the PUCCH of the SpCell of this cell group, or on this serving cell if it is a PUCCH SCell.. Proposal 4.2: Support and prefer Option-1. Since, regardless of cross-CC or single-CC reporting scheme, based on RAN1 120 agreement, current and new beam RSs are always on the same CC, it makes sense to use Alt-1 in RAN1 119 agreement also for the cross-CC reporting. [Mod]: Captured! |
| NEC | Proposal 4.1. OK. Proposal 4.2. Support Option-2. [Mod]: Captured! |
| Fujitsu | Proposal 4.2: Option-1 is preferred. [Mod]: Captured! |
| LG | Proposal 4.2: Support Option 1. If the first sentence of the proposal is supported, then why Option 2 is needed? For example, for Option 2, if evaluation periodicity becomes X, why does gNB configure measurement RS with shorter periodicity than needed, which just causes unnecessary RS overhead? [Mod]: Captured! |
| CATT | Proposal 4.1: Support. Proposal 4.2: Support with modifications and prefer to option-2. For cross-CC, the configured periodicity can be different. Otherwise, the scheduling from gNB is influenced. We are ok that the evaluation periodicities are the same. To clarify the issue, the following modifications are suggested: Proposal 4.2: On cross-CC beam report measurement for UE-initiated/event-driven beam reporting, regarding the evaluation periodicity for determining event instance, the evaluation periodicity of the current beam RS should be the same as that of the new beam RS(s). - Option-1: The evaluation periodicity is the same as the periodicity of the current and new beam RS(s). - Option-2: The evaluation periodicity is the maximum of {X ms, the periodicity of the current and new beam RS(s)} [Mod]: Good suggestion. Captured! |
| Lenovo | Proposal 4.1: Support. In addition, this parameter should also be applied to all the events and should be configured in the CSI-ReportConfig for UEIBM. Proposal 4.2: Option-1 is preferred. [Mod]: Captured! |
| Sharp | Proposal 4.1: Support. Proposal 4.2: We prefer Option-1 that is simpler. [Mod]: Captured! |
| NTT DOCOMO | Proposal 4.1: Support Proposal 4.2: Support Option-1. [Mod]: Captured! |
| Samsung2 | It seems we are repeating the same discussion for RAN1#120 and we would like to share the discussion history in RAN1#120. The following is the first round FL summary in v00 and Samsung’s comment in v01. FL update the proposal in v23 based on Samsung’s comment as below The following is the agreement made on Monday Agreement On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, the following working assumption is confirmed with the following modification. - The first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from different PUCCH groups - Subject to separate UE capability - Note: From RAN1 perspective, the above does NOT introduce any spec impact except for UE capability signaling Actually, our first position is not to confirm the WA because there is no clear use case in our view, but to be constructive and with the understanding of the global PUCCH resource ID we compromised with the above. We cannot accept to repeat the same discussion and revert previous agreement. Also, as clarified above, even if companies have a different view that the PUCCH resource ID is not global, it is still can be implemented by network that the PUCCH resource IDs are different for different cells – there is no issue. [Mod]: Captured! |
| Qualcomm | Proposal 4.1: We support the proposal. We share the same view as Huawei/HiSilicon and believe that the legacy parameter design, pucch-Cell, can be reused. Proposal 4.2: We support Option-1. [Mod]: Captured! |
| vivo | Proposal 4.1: Support Proposal 4.2: Form UE’s perspective, Option-2 is preferred. [Mod]: Captured! |
| Ericsson | P4.1: Do not support. We agree with Samsung: the PUCCH resource Id must be unique across all configured cells with PUCCH. Check how the scheduling request is configured. In the SR configuration, there is no pointer to SpCell or PUCCH-SCell, only a PUCCH-ResourceId. P4.2: We fail to see why the evaluation periodicity should be different: the current beam RS and the new RSs are still in the same CC, only the reporting is performed on another CC. The solution for the same carrier case should be adopted, i.e., option-1 [Mod]: Captured! |
| Apple | Proposal 4.1:Support. The PUCCH resources are indexed per-CC and are typically repeated across PCell and PUCCH SCell. Modifying this rule for UEIBR could introduce unnecssary complexity in the PUCCH resource planning on the NW side. In our view, using a-bit cell indictor is the simplest way as it is already used in other procedures, e.g., HARQ-ACK for PDSCH reception etc. If overhead is a concern, a default value can be defined without requiring additional signaling if defaul CC is used (e.g., PCell). Proposal 4.2:Opt.2. We believe this proposal should apply to both same-CC and cross-CC cases, rather than being limited to cross-CC only. For RLM, the minimum evaluation periodicity is set to 2ms when DRX is not configured. When DRX is configured, a table in TS 38.133 defines the minimum measurement value. The same approach should be adopted for the UEIBR procedure, as the UE is required to perform measurements on the periodic RS resource. It is imporant to balance between measurement latency and UE’s power consumption. [Mod]: Captured! |
| Mod_V30 | Updated per companies input. |
| Transsion | Proposal 4.1: Support. Proposal 4.2: Support and prefer Option-1. |
| Mod_V32 | Updated per companies input. |
| ETRI | Proposal 4.1: Support Proposal 4.2: Support the Option-1. |
| Mod_V34 | Updated per companies input. |
| China Telecom | Proposal 4.1. Support. Proposal 4.2. Support and prefer Option-2. |

- Proposals for Monday Online Discussion

Proposal 3.1.3B-1: On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing a number of L (L>=1) first PUCCH(s) with UEIRIs are collided/overlapped with a PUCCH format 2/3/4 carrying HARQ/CSI, reuse the legacy SR multiplexing rule.

- The value of log2L+1 bits is based on an ascending order of the values of PUCCH resource ID associated with the first PUCCHs.

Supported by: Huawei/HiSi’, vivo, Nokia, Ericsson, Apple, Qualcomm, ZTE, NTT DOCOMO, ofinno, Fujitsu, HONOR, Transsion, OPPO, Ofinno, xiaomi, Spreadtrum, MediaTek, Futurewei, NEC, LGE, CATT, Lenovo, Sharp, HONOR, Apple, ETRI, China Telecom, Samsung,

Proposal 3.1.3B-2: On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing a number of L (L>=1) first PUCCH is collided/overlapped with a PUCCH format 2/3/4 carrying HARQ/CSI/SR.

- Option-1: Extend the SR field of  log2K+1 bits to a field of log2K+L+1 bits of representing at most one of positive SR or positive UEIRI

  - The value in the field is based on a first ascending order of the values of SR ID associated with the SRs and a second ascending order of the values of PUCCH resource ID associated with the first PUCCHs.

  - If one of the SRs is a positive LRR, the value of the log2K+L+1 bits indicates the positive LRR, else, if one of the UEIBRs is a positive UEIBR, the value of the log2K+L+1 bits indicates the positive UEIBR.

  - An all-zero value for the log2K+L+1 bits represents a negative SR or UEIRI value across all K LRR/SRs and/or L UEIRIs.

- Option-3: The field of log2L+1 bits for UEIRI is appended to the field of log2K+1 bits for SR/LRRs.

Option-1: Fujitsu, Ericsson, ZTE, NTT DOCOMO, Transsion, Qualcomm, Apple, HONOR, MediaTek, Futurewei, OPPO, Fujitsu, CMCC, Spreadtrum, NEC, Lenovo, Nokia, Ofinno, vivo, Transsion, xiaomi (1st priority), Samsung, MediaTek, Nokia, NEC, Fujitsu, Lenovo, Sharp, Apple, Transsion, ETRI, China Telecom,

Option-3: Huawei/HiSi’, xiaomi(2nd), Spreatrum,

Proposal 3.2: On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the value of X symbols for determining available transmission occasion of the second UL channel on Mode-B

- Down-select one of the following as RRC candidate values for X symbols in RAN1#121

  - Option-1: [0,] 1, 2, 4, 8, 16, 32, 64

  - Option-2: [0,] 1

  - Option-3: 0

  - Option-4: [0,] 1, 2, 4, 8

  - Note: X is based on the SCS of the first PUCCH.

- The ‘available’ transmission occasion of the second UL channel is clarified as ‘an available transmission occasion corresponds to a configured PUSCH resource for UEIBR is within uplink symbols, and does not collide with a channel with a higher priority (e.g., PRACH)’.

- Note: Whether/how to introduce additional UE capability signaling is up to Rel-19 UE feature discussion.

Option-1: Samsung, OPPO, Ofinno, Google (w.o. 0), Nokia, Futurewei, NEC, Lenovo, KDDI, vivo, Ericsson, Apple (w.o. 0), Transsion, ETRI

Option-2: ZTE, Lenovo, KDDI, CATT,

Option-3: China Telecom, KDDI, Google (w.o. 0), NEC, Lenovo, CATT,

Option-4: Ofinno, Google (w.o. 0), ZTE, Huawei, NEC, Lenovo, KDDI, Ericsson, ETRI, CATT,

Postpone the discussion on value of X: QC, HONOR,

Proposal 3.4: On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding CSI reference resource definition for a UEI beam report, support Option-1 of the following:

- Option-1: For both mode-A and mode-B, reuse legacy CSI reference definition of P/SP-CSI reporting, except that, in TS 38.214 Clause 5.2.2.5, slot n is based on a downlink slot with reference to the uplink slot in which the first PUCCH is transmitted.

- Option-2:

  - For mode A, support to reuse legacy CSI reference definition of aperiodic CSI reporting.

  - For mode B, support to reuse legacy CSI reference definition of semi-persistent CSI reporting.

- Option-3: For both mode-A and mode-B, support to reuse legacy CSI reference definition of aperiodic CSI reporting for UEI/ED beam reporting.

Option-1: Nokia, ZTE, vivo, Lenovo, Samsung, Huawei/HiSi, NEC, Fujitsu, LG, Lenovo, Sharp, Transsion,

Option-2: MediaTek, OPPO, Ofinno, Google, Nokia (2nd priority), Futurewei, Apple,

Option-3: NTT DOCOMO, Qualcomm, xiaomi, Ericsson, ETRI,

Proposal 3.5A: On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding occupied CPU(s), OCPU = 1 is occupied.

Proposal 1.1A (time-window/counting):

Adopt the following changes in Clause 5.2.1.5.4.1 in TS38.214.

5.2.1.5.4.1 UE Initiated CSI reporting For a UE configured with a CSI-ReportConfig with eventType-r19 for periodic reference signals [with the same periodicity configured by the newBeamResourceSet-r19] and with dl-OrJointTCI-StateList, when an event instance is determined (as described in the following clauses), - for a reference signal configured by the newBeamResourceSet-r19, if eventType-r19 is set to ‘event2’ or ‘event7’, or - for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI, if eventType-r19 is set to ‘event1’, if eventDetectionTimeWindowLength-r19 is provided, and the time window for such reference signal has not started yet or has ended, the UE starts a time window a timer of a length eventDetectionTimeWindowLength-r19, if provided, at the beginning of a transmission occasion of for such the reference signal for which an event instance is determined, if the timer is not running; the time window includes the time while the timer is running wherein the time window does not overlap with other time windows for such reference signal. When the number of event instances determined for such reference signal within the time window, if eventDetectionTimeWindowLength-r19 is provided, is greater than or equal to 1 or to eventInstanceCount-r19, if provided, the UE transmits PUCCH format 0 or format 1 in the earlist transmission occasion of in the PUCCH resource configured by PUCCHResource-r19 in the CSI-ReportConfig, T_proc after the end of the time window. [The counting time window of the event instances for such reference signal is stopped restarted]: - if the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state is updated.] The UE does not expect that a CSI trigger state associated with CSI report configuration(s) configured with the higher layer parameter eventType-r19 is further associated with other CSI report configurations that are not configured with the higher layer parameter eventType-r19.

Supported by Samsung, Sharp,

Proposal 1.1B (timer/counting):

Adopt the following changes in Clause 5.2.1.5.4.1 in TS38.214.

5.2.1.5.4.1 UE Initiated CSI reporting For a UE configured with a CSI-ReportConfig with eventType-r19 for periodic reference signals [with the same periodicity configured by the newBeamResourceSet-r19] and with dl-OrJointTCI-StateList and without eventDetectionTimeWindowLength-r19, when an event instance is determined (as described in the following clauses), - for a reference signal configured by the newBeamResourceSet-r19, if eventType-r19 is set to ‘event2’ or ‘event7’, or - for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI, if eventType-r19 is set to ‘event1’, the UE transmits PUCCH format 0 or format 1 in the PUCCH resource configured by PUCCHResource in the CSI-ReportConfig. For a UE configured with a CSI-ReportConfig with eventType-r19 for periodic reference signals [with the same periodicity configured by the newBeamResourceSet-r19] and with dl-OrJointTCI-StateList and with eventDetectionTimeWindowLength-r19, when an event instance is determined (as described in the following clauses), - for a reference signal configured by the newBeamResourceSet-r19, if eventType-r19 is set to ‘event2’ or ‘event7’, or - for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI, if eventType-r19 is set to ‘event1’, [if eventDetectionTimeWindowLength-r19 is provided, the UE - starts a timer for such reference signal from the value provided by eventDetectionTimeWindowLength-r19 and sets the counter to 1, if the timer is not running; or - increments the counter for such reference signal by 1 if the timer is already running. wherein the time window does not overlap with other time windows for such reference signal.] When the number of event instances determined for such reference signal within the time window, if eventDetectionTimeWindowLength-r19 is provided, is greater than or equal to 1 or to eventInstanceCount-r19, if provided, If the timer is running and the counter for such reference signal is greater than or equal to eventInstanceCount-r19, the UE transmits PUCCH format 0 or format 1 in the PUCCH resource configured by PUCCHResource-r19 in the CSI-ReportConfig. [The counting counter of the event instances for such reference signal is reset to zero restarted and the timer for such reference signal is stopped: - if the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state is updated.] The UE does not expect that a CSI trigger state associated with CSI report configuration(s) configured with the higher layer parameter eventType-r19 is further associated with other CSI report configurations that are not configured with the higher layer parameter eventType-r19.

Supported by: OPPO, Ofinno, Google, xiaomi, ZTE, Spreadtrum, MediaTek, Futurewei, Huawei/HiSi, NEC, Fujitsu, Lenovo, NTT DOCOMO, Qualcomm, HONOR, Ericsson, IDC, Transsion, ETRI, China Telecom,

- Previous agreements

  - RAN1#120-bis

[120] Agreement

On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, the following working assumption is confirmed with the following modification.

- The first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from different PUCCH groups

  - Subject to separate UE capability

- Note: From RAN1 perspective, the above does NOT introduce any spec impact except for UE capability signaling

[120b] Agreement

Regarding triggering event determination for Event 2, on resetting the counting, the following modification on the agreed Candidate #2 in RAN1#120 is supported.

- Candidate#2: [The measured current beam RS is updated based on] indicated TCI state is updated;

  - In such case, the UE needs to reset the counting for all new beams.

  - FFS: Further details on the update (if necessary)

[120b] Agreement

On UE-initiated/event-driven beam reporting, support the following interpretation on each of the codepoints of the ‘1-bit’ condition met indicator as follows:

- ‘0’ – indicating that the Event-2 instances for corresponding CRI/SSBRI within the time window doesn’t reach the configured number M.

- ‘1’ – indicating that the Event-2 instances for corresponding CRI/SSBRI within the time window reach the configured number M.

FFS: whether/how to introduce the ‘1-bit’ condition met indicator for Event 7.

[120b] Agreement

On cross-CC beam report measurement for UE-initiated/event-driven beam reporting, regarding Event-2, introduce an RRC parameter to indicate ServCellIndex on which the indicated TCI state used to determine the current beam RS is applied

- FFS: Indication of the cell that corresponds to the configured first PUCCH resource in CSI report config including whether it is necessary

[120b] Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case the pre-configured Type-1 CG PUSCH carry the beam report, for the second UL channel in Mode-B, reuse the intra-UE multiplexing/prioritization rules of PUSCH with SP-CSI for Type-1 CG PUSCH with UEI-BR for Mode B

[120b] Agreement

Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, down-select one of the following options in RAN1#120bis.

- Option-1: The measurement window is from T_PUCCH – T_proc – T_window to T_PUCCH – T_proc, where T_PUCCH is a transmission occasion of a first PUCCH, and T_proc is RRC configured.

- Option-3: The length, slot offset and periodicity of a measurement window are configured per CSI report configuration by NW.

- Option-4: If an Event-2 instance for a new beam is obtained at the time t [and the time for the new beam is not running], UE (re)starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window)

- Note: T_window is the agreed time window parameter for measurement.

[120b] Agreement

On UE-initiated/event-driven beam reporting, support the following for Event 1

- Regarding report format of L1-RSRP value of current beam,

  - Option-A2: L1-RSRP of current beam is absolute L1-RSRP (7-bit quantized, rather than 4-bit as in Event-2).

[120b] Agreement

On UE-initiated/event-driven beam reporting, regarding current beam report on Event 1, reuse RRC parameter enabledCurrentBeamReport-r19 to enable/disable the RSRP report of current beam.

[120b] Conclusion

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the case that one PUCCH resource of first PUCCH can be associated with one or multiple CSI report configurations, there is NO RAN1 consensus on supporting multiple UEI beam reports carrying in a second PUSCH.

[120b] Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, support the following rule for the Case-4: the 1-bit first PUCCH is collided/overlapped with a PUCCH format 0/1 carrying HARQ.

- Option-1: Follow legacy SR multiplexing rule.

  - Note: There is no further enhancement on additional phase offset/constellation point(s).

[120b] Conclusion

Regarding resource mapping/configuration between first and second UL channel associated with a same CSI report configuration in Mode-B, there is NO consensus on further supporting the case that first PUCCH resource and pre-configured resource for second UL channel can have different periodicities (in ms).

[120b] Agreement

Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, support Option-4.

- Option-4: If an Event-2 instance for a new beam is obtained at the time t and the timer for the new beam is not running, UE starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window)

  - Note: Timer is new beam specific.

- Note: T_window is the agreed time window parameter for measurement.

- Introduce separate UE capability to limit the number of timers. There is only 1 timer per new beam.

Above agreement is captured in RAN1 specifications.

[120b] Agreement

On UE-initiated/event-driven beam reporting, the procedure of triggering event determination is captured in RAN1 spec.

[120b] Conclusion

There is no RAN1 consensus on the following proposal:

On UE-initiated/event-driven beam reporting, support at least Option-1 of following for Event 7 as an extension on report format for Event-2

- Option-1: Additional field to indicate the codepoint of the activated TCI state with Q-th best quality.

  - FFS: Further report codepoints of other activated TCI state(s) [, e.g., from {Q+1}-th best quality to the worst one];

  - FFS: Details of the additional indication.

- Option-2: Extending the maximum number of reported RS(s) to 8.

  - [Note: If Option 2 is agreed, RAN1 can revisit the agreed maximum value of N for event-2.]

- Above is applicable only if time window and a configurable value M for event counting on Event-7 are not configured.

[120b] Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting for a CSI report configuration, down-select at least one of the following candidates in RAN1#121:

- Candidate#1: To introduce a prohibit timer for mode-A and/or mode-B

  - Option-1A: In the case that triggering-event associated with the CSI report configuration is determined [by the same triggering beams(s) as the last transmitted PUCCH], if the prohibit timer is NOT running, the UE can transmit first PUCCH;

    - At the first symbol after the end of the PUSCH transmission, the UE starts the prohibit timer

  - Option-1B: In the case that triggering-event associated with the CSI report configuration is determined, if the prohibit timer is NOT running, the UE can transmit first PUCCH;

    - At the first symbol after the end of the first PUCCH transmission, the UE starts the prohibit timer

  - If the prohibit timer is running, the first PUCCH is not allowed to be transmitted.

- Candidate#2: To introduce a time interval for mode-A and/or mode-B

  - Option-2A: For a first PUCCH transmission occasion, if there is a transmission of another second PUSCH [corresponding to the CSI report determined by the same triggering beam(s) as the first PUCCH] within a configurable time interval before the first PUCCH transmission occasion, the UE should not transmit the first PUCCH.

  - Option-2B: For a first PUCCH transmission occasion, if there is a transmission of another first PUCCH(s) within a configurable time interval before the first PUCCH transmission occasion, the UE should not transmit the first PUCCH.

- Candidate#3: No further enhancement.

[120b] Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, down-select one of the following rules for the Case-2: the 1-bit first PUCCH is collided/overlapped with a PUSCH, in RAN1#121

- Option-1: Prioritize first PUCCH over PUSCH, i.e., PUSCH is dropped.

  - FFS: If the PUSCH should be with UL-SCH or not for UEI beam report

- Option-3: Piggyback 1-bit indication of first PUCCH into the PUSCH.

  - FFS: The 1-bit indication is always multiplexed in the PUSCH, regardless that UEI beam report procedure is triggered.

  - FFS: If the PUSCH should be with UL-SCH or not for UEI beam report

- Option-4: Reuse the SR dropping rules.

- FFS: whether/how to handle the case of different PHY priorities.

Conclusion

There is no RAN1 concensus on the following. No spec change needed.

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, support one of the following rule for the Case-3: the 1-bit first PUCCHs corresponding to different CSI configuration for UE-initiated/event-driven beam reporting are overlapping in the time domain

- Option-1: Per CSI report configuration.

- Option-2: Up to implementation.

- Option-3: UE transmits the first PUCCH with higher priority (e.g., PUCCH resource with lower PUCCH resource ID, or event-type)

- Option-4: Multiplexing all 1-bit indications

[120b] Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, confirming the following working assumption with the following modification.

- For Mode-A, the multiple CSI report configurations associated with the same PUCCH resource should be associated with a same CSI-AperiodicTriggerState.

- FFS: For Mode-A, the multiple CSI report configurations associated with the same CSI-AperiodicTriggerState should be associated with a same PUCCH resource.

[120b] Conclusion

There is no RAN1 concensus on the following. No spec change needed.

For event 2, when one PUCCH resource of first PUCCH can be associated with one or multiple CSI report configurations, and if multiple UE initiated beam report procedures occur, down-select one of the following options:

- Option-1: It is up to UE implementation to select one of configuration.

- Option-2: The UEI beam report with highest priority is reported

  - FFS: priority, e.g., lowest CSI-ReportConfigId, event-type or based on legacy CSI report priority rule

- Option-3: The report triggered in the latest measurement is reported in PUSCH

  - RAN1#120

[120] Agreement

On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, the following working assumption is confirmed with the following modification.

- The first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from different PUCCH groups

  - Subject to separate UE capability

- Note: From RAN1 perspective, the above does NOT introduce any spec impact except for UE capability signaling

[120] Agreement

On cross-CC beam report measurement for UE-initiated/event-driven beam reporting, regarding Event-2, the following is supported

- The current beam RS and new beam RS(s) are in the same CC

  - The above does NOT imply to preclude the cross-CC TCI indication.

- The CC on which the indicated TCI state is applied, and the CC in which new beam RS(s) are can be the same or different.

  - FFS: Whether/how to introduce an RRC parameter to indicate the CC on which the indicated TCI state is applied.

[120] Agreement

On UE-initiated/event-driven beam reporting, for Event 7, the candidate value of RRC parameter Q = {1, 2, 3, 4, 5, 6, 7, 8}

- Note: The UE does not expect that the configured Q is greater than the number of the activated DL/joint TCI state(s).

[120] Conclusion

There is no RAN1 consensus on the following proposal:

- On beam report transmission procedure for UE-initiated/event-driven beam reporting, multi-bit indication in the first PUCCH is supported.

[120] Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-B, the value of X symbols for determining available transmission occasion of the second UL channel is configured by RRC (FFS: subject to a corresponding UE capability)

[120] Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, support the following option of dropping rule for the Case-1: the 1-bit first PUCCH is collided/overlapped with a PUCCH carrying normal SR and/or a PUCCH with normal LRR

- Option-1: LRR > first PUCCH > normal SR

Note: When the 1-bit first PUCCH is collided/overlapped with a PUCCH carrying normal SR and/or a PUCCH with normal LRR, only one of them is transmitted based on the above priority rule

[120] Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the triggering procedure in Step-2 of Mode-A, the following Option-1 is supported.

- Option-1: A CSI trigger state corresponding to UE-initiated/event-driven beam reporting can NOT be associated with legacy AP-CSI report configuration.

[120] Agreement

Regarding triggering event determination for Event 2, at least Candidate #2 is supported for resetting the counting.

- Candidate#1: RS reconfiguration/update or MAC-CE signaling (if supported) for new beam is received;

  - FFS: whether to reset the counting for all new beams.

  - FFS: whether to maintain the counting whose new beam is NOT updated.

- Candidate#2: [The measured current beam based on] indicated TCI state is updated;

  - In such case, the UE need to reset the counting for all new beams.

- Candidate#3: UEI beam report is transmitted;

  - FFS: Only reset the counting of new beams fulfilling triggering condition and reported by the UEI beam report

- Candidate#4: NW response (e.g., DCI in step-2 of Mode-A) is detected.

- Candidate#5: The time window expires

- Candidate#6: The threshold for event evaluation is re-configured by RRC signaling

- (FFS) Candidate#7: The RRC parameter(s) associated with the CSI report configuration for UEI beam report is reconfigured.

  - FFS: RRC parameter(s)

- FFS: Other candidates

Note: Whether this proposal is captured in RAN1 or RAN2 is a separate discussion point.

[120] Agreement

On UE-initiated/event-driven beam reporting, for Event 1, support Option-2 for RS measurement:

- Option-2: RS resource set for new beam is configured in the CSI reporting configuration, and the following implicit manner for enabling one of either scheme-1 or scheme-2 is used:

  - If the RS(s) for new beam are CSI-RS configured in a CSI-RS resource set configured with repetition, Scheme-1 is enabled; otherwise, Scheme-2 is enabled.

  - In such case, the report format for Event-2 is reused unless critical technical issue is identified

    - FFS: How to report the RSRP value of the current beam

    - FFS: RSRP of the current beam is always reported

[120] Agreement

Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, further study the following options for possible down-selection

- Option-1: The measurement window is from T_PUCCH – T_proc – T_window to T_PUCCH – T_proc, where T_PUCCH is a transmission occasion of a first PUCCH, and T_proc is RRC configured.

- Option-2: The measurement window is from T_Instance – T_window to T_Instance, where T_Instance is an evaluation occasion of event instance, and T_proc is RRC configured.

  - The UEI beam report in the second PUSCH is based on the most recent measurement for new/current beam RS(s).

- Option-3: The length, slot offset and periodicity of a measurement window are configured per CSI report configuration by NW.

- Option-4: If an Event-2 instance for a new beam is obtained at the time t, UE (re)starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window)

- Note: T_window is the agreed time window parameter for measurement.

- Note: Other options are not precluded.

[120] Agreement

On UE-initiated/event-driven beam reporting, Option-1 is supported as an extension on L1-RSRP report format depending on Event-2.

- Option-1: For each of reported CRI/SSBRI, to introduce additional indication of whether the CRI/SSBRI satisfies the condition of Event-2.

  - The presence of this field is enabled by RRC with subjective to UE capability.

  - Note: The presence of this field is only for the case that N > 1 and the time window and M are configured

Above is NOT applicable for event 1.

[120] Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, one PUCCH resource of first PUCCH can be associated with one or multiple CSI report configurations, regarding Event-2.

- Only a single UEI beam report is carried in the second PUSCH.

  - Additional indication of one ‘CSI report configuration’ is provided in the report format.

    - The CSI report configurations associated with the same PUCCH resource are ordered in ascending order of corresponding CSI-ReportConfigId, the number of bits of the additional indication field is ceil(log2(N_CSIconfig)), where the N_CSIconfig denotes the number of CSI report configurations associated with the same PUCCH resource.

  - The payload size of the single UEI beam report is determined according to the max payload size among the associated CSI report configurations.

    - Zero-padding can be appended if the payload size of the UEI beam report is less than the max report payload.

  - The reported UEI beam report should satisfy the triggering condition.

  - If multiple UE initiated beam report procedures occur, down-select one of the following options:

    - Option-1: It is up to UE implementation to select one of configuration.

    - Option-2: The UEI beam report with highest priority is reported

    - Option-3: The report triggered in the latest measurement is reported in PUSCH

  - The multiple CSI report configurations associated with the one first PUCCH resource should be configured in the same CC.

  - Working Assumption: For Mode-A, the multiple CSI report configurations associated with the same PUCCH resource should be associated with a same CSI-AperiodicTriggerState.

  - For Mode-B, the multiple CSI report configurations associated with the same PUCCH resource should be associated with the same second configured PUSCH

FFS: Alt-2 Multiple UEI beam reports associated with the same PUCCH resource for first PUCCH can be transmitted in the second PUSCH.

- If RAN1 cannot converge on the support of Alt-2 in RAN1#120bis, this alternative will be dropped from Rel-19

[120] Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, down-select one of the following rule for the Case-2: the 1-bit first PUCCH is collided/overlapped with a PUSCH

- Option-1: Prioritize first PUCCH over PUSCH, i.e., PUSCH is dropped.

- Option-3: Piggyback 1-bit indication of first PUCCH into the PUSCH.

  - FFS: If the PUSCH should be with UL-SCH or not for UEI beam report

- Option-4: Reuse the SR dropping rules, i.e., the first PUCCH is dropped.

- FFS: whether/how to handle the case of different PHY priorities.

  - RAN1#119

[119] Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case the pre-configured Type-1 CG PUSCH does NOT carry the beam report, for the second UL channel in Mode-B, Option-2 is supported:

- Option-2: Type-1 CG PUSCH can NOT be transmitted

[119] Agreement

On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, the following is supported

- the first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from the same PUCCH group.

- (working assumption) the first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from different PUCCH groups

  - Subject to separate UE capability

[119] Conclusion

On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, the case that the first PUCCH and the second PUSCH are from the different CG is NOT supported.

[119] Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding first PUCCH channel configuration, down-select one of Alt-1 and Alt-2 for both mode-A and mode-B.

- Alt-1 (dedicated SR): Introduce RRC parameter, e.g., reportResourceRequest-UEIBR/reportNotification-UEIBR, corresponding to the one-bit indication in the first PUCCH channel

  - The RRC parameter is associated with the dedicated SchedulingRequestId.

    - Note: The detailed signaling is up to RAN2.

  - FFS: whether/how to enhance legacy SR to support notification of beam report transmission in Mode-B

- Alt-2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig- UEIBR, for the periodic PUCCH resource configuration.

  - Note: The RRC parameter is NOT associated with SchedulingRequestId.

  - 1-bit to PUCCH resource is encoded by reusing the encoding mechanism of positive/negative SR.

  - The dedicated RRC parameter at least comprises the following:

    - periodicityAndOffset

    - PUCCH-ResourceID

- Above applies at least for the single CC case.

- Reuse multiplexing/dropping rule(s) of SR as baseline

  - FFS: overlapping with SR/LRR or PUSCH

Note: Further details on first PUCCH retransmission for mode A and mode B will be separately discussed.

[119] Agreement

On UE-initiated/event-driven beam reporting, for Event 7, the scheme-1 and scheme-2 for deriving ‘RS for current beam’ on Event-2 is reused with the following further interpretation:

- Scheme-1: ‘RS for current beam’ is the QCL RS in the activated TCI state with the Q-th best quality.

- Scheme-2: ‘RS for current beam’ is the SSB which is QCLed with the QCL RS in the activated TCI state with the Q-th best quality.

- Basic feature of the triggering event determination for Event-7: Once quality of at least one new beam becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality, UE initiated beam report occurs

Note: For Event-2, we have the following definition for scheme-1 and scheme-2

- Scheme-1: RS for current beam is the QCL RS in the indicated TCI state

- Scheme-2: RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state.

[119] Agreement

On UE-initiated/event-driven beam reporting, for Event 1, down-select at least one among the following options for RS measurement

- Option-1: RS resource set for new beam is NOT configured in the CSI reporting configuration, and an explicit RRC selection for scheme-1 and scheme-2 is introduced.

- Option-2: RS resource set for new beam is configured in the CSI reporting configuration, and the following implicit manner for enabling one of either scheme-1 or scheme-2 is used:

  - If the RS(s) for new beam are CSI-RS configured in a CSI-RS resource set configured with repetition, Scheme-1 is enabled; otherwise, Scheme-2 is enabled.

[119] Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding first PUCCH channel configuration, Alt-2 is supported for both mode-A and mode-B: the first PUCCH channel is a new UCI type

- Alt-2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig- UEIBR, for the periodic PUCCH resource configuration.

  - It is RAN1’s understanding that the RRC parameter is NOT associated with SchedulingRequestId.

  - 1-bit to PUCCH resource is encoded by reusing the encoding mechanism of positive/negative SR.

  - The dedicated RRC parameter at least comprises the following:

    - periodicityAndOffset

    - PUCCH-ResourceID

- Above applies at least for the single CC case.

- Reuse multiplexing/dropping rule(s) of SR as baseline

  - FFS: overlapping with SR/LRR or PUSCH

Note: Further details on first PUCCH retransmission (if supported) for mode A and mode B will be continue to be separately discussed in RAN1.

[119] Agreement

Confirm the following working assumption in RAN1#117:

On beam report transmission procedure for UE-initiated/event-driven beam reporting

- For mode-A, at least support one-bit indication in the first PUCCH channel to request a resource for a second UL channel to carry beam report.

  - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling.

- For mode-B, at least support one-bit indication in the first PUCCH channel to notify a second UL channel to carry beam report.

  - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling.

- FFS: Whether/how to support multi-bit indication in the first PUCCH for mode-A and mode-B, e.g., when multi-event(s) are approved.

- FFS: details on the dedicated RRC signaling

- Above applies at least for the single CC case.

[119] Agreement

On UE-initiated/event-driven beam reporting, at least one of the following is supported as an extension on L1-RSRP report format depending on Event-2.

- Option-1: For each of reported CRI/SSBRI, to introduce additional indication of whether the CRI/SSBRI satisfies the condition of Event-2.

  - The presence of this field is enabled by RRC with subjective to UE capability.

  - The presence of this field is only for the case that N > 1 and the time window and M are configured

- Option-2: For each of reported CRI/SSBRI, to introduce additional indication of the number of Event-2 instances for the CRI/SSBRI(s) within a time window.

  - The presence of this field is enabled by RRC with subjective to UE capability.

  - The presence of this field is only for the case that N > 1 and the time window and M are configured.

- Option-3: No further enhancement.

Note: As agreed in RAN1#117, at least one of the reported CRI/SSBRI(s) should satisfy the condition of Event-2

FFS: Whether/how to handle the case if UE has not sent any first PUCCH associated for UE-initiated/event-driven beam reporting, for Mode-A.

[119] Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the triggering procedure in Step-2 of Mode-A, down-select one of the following options in RAN1#120

- Option-1: A CSI trigger state is dedicated to UE-initiated/event-driven beam reporting, i.e., not associated with legacy AP-CSI report configuration.

- Option-2: A CSI trigger state can be associated with

  - only UE-initiated/event-driven beam reporting

  - or only legacy AP-CSI configuration

  - or UE-initiated/event-driven beam reporting and legacy AP-CSI configuration

[119] Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, further study at least the following cases:

- Case-1: The 1-bit first PUCCH is collided/overlapped with a PUCCH carrying normal SR and/or a PUCCH with normal LRR

- Case-2: The 1-bit first PUCCH is collided/overlapped with a PUSCH

- Case-3: The 1-bit first PUCCHs corresponding to different CSI configuration for UE-initiated/event-driven beam reporting are overlapping in the time domain.

[119] Agreement

Study the following to reduce beam application latency after a UEI/ED beam report is sent

  - Alt-1: After confirmation/acknowledgement from NW, apply new beam without RRC configuration signaling or MAC-CE signaling

    - after sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report

    - update TCI state(s) with the reported new beam(s)

    - activate new beam(s) without additional SSB reception

  - Alt-2: After receiving a TCI state activation command to activate a TCI state(s), if the new beam(s) associated with the TCI state(s) is reported as synchronized in the UEI/ED beam report, the TCI state(s) becomes applicable for DL reception without additional SSB reception.

    - Note: A reported new beam is determined as synchronized by UE, if the UE stores the QCL properties associated with the reported new beam(s) after the UEI/ED beam report is sent

    - FFS: How to inform a reported new beam in a UEI/ED beam report (i.e., introducing one-bit indicator for each reported new beam or all the reported new beam are assumed to be synchronized)

  - Alt-3: After sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report.

    - In such case, at the reception of a subsequent reception of Unified TCI States Activation/Deactivation MAC CE, the UE activates new beam(s) without additional SSB reception

  - Other alternatives are not precluded

[119] Agreement

On UE-initiated/event-driven beam reporting, regarding trigger events, the following working assumption in RAN1#118bis is confirmed:

On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported.

- Event-1: Quality of the current beam is worse than a certain threshold.

- Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality.

  - Q is RRC configured with subjective to UE capability signalling

    - UE may only indicate a single candidate value or not support Event-7.

- The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise

- The additionally supported events will be lower priority compared to event 2.

[119] Agreement

Regarding for the evaluation periodicity for determining Event-2 instance [at least when DRX is not configured], at least Alt-1 is supported for the single-CC beam reporting (for case that the CSI report configuration and RS for the current beam and new beam(s) are in the same CC).

- Alt-1: The periodicity of the current beam RS should be the same as that of the new beam RS(s).

  - The evaluation periodicity is the same as the periodicity of the current and new beam RS(s).

- Alt-2: The periodicity of the current beam RS can be different from that of the new beam RS(s)

  - Alt-2_1: The evaluation periodicity is the same as the periodicity of the current beam RS.

  - Alt-2_2: The evaluation periodicity is the same as periodicity of the new beam RS.

  - Alt-2_3: The evaluation periodicity is the same as shortest periodicity of the current beam RS and new beam RS(s).

  - Alt-2_4: The evaluation periodicity is the maximum of {X ms, shortest periodicity of the current beam RS and new beam RS(s)}.

  - Alt-2_5: The evaluation periodicity is the same as largest periodicity of the current beam RS and new beam RS(s).

Note: There is the same periodicity for the new beam RS(s).

FFS: Down-selection among above Alt(s) for cross-CC beam reporting.

Strong concerns were expressed by Huawei and CATT that if only Alt-1 is supported in the end, the feature will not be practical.

Send an LS to RAN4. Final LS in R1-2410914.

  - RAN1#118-bis

[118b] Agreement

On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2,

- The differential L1-RSRP is quantized to a 4-bit value with 2 dB step size

[118b] Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the triggering procedure in Step-2 of Mode-A

- Reuse CSI request field in DCI format 0_1/0_2 to trigger the transmission of the UEI beam report

  - If a CSI trigger state associated with UEI beam report configuration(s) is indicated by the CSI request field in DCI format 0_1/0_2, the UE transmits the corresponding UEI beam report(s) in the second PUSCH scheduled by the DCI format 0_1/0_2

  - FFS: DCI format 0_3

- FFS: Whether a CSI trigger state should be dedicated to UE-initiated/event-driven beam reporting, i.e., not associated with legacy AP-CSI report configuration.

[118b] Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, resource mapping/configuration between first and second UL channel in Mode-B, at least Option-1 is supported

- Option-1 (one-to-one): Only one periodic PUCCH resource for the first channel and only one pre-configured resource for second UL channel can be associated with the CSI report configuration for UE-initiated/event-driven beam reporting.

  - Down-select one of the following in RAN1#118bis

    - Option-1A: Same periodicity between first PUCCH resource and pre-configured resource for second UL channel.

    - Option-1B: No restriction in terms of periodicity.

[118b] Agreement

On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, for both Mode-A and Mode-B, the first PUCCH and the second PUSCH can be from the same or different CC(s)

- FFS: whether the first PUCCH and the second PUSCH should be from the same PUCCH group

- The first PUCCH and the second PUSCH should be in the same CG

  - FFS: Different CGs

[118b] Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case the pre-configured Type-1 CG PUSCH carry the beam report, for the second UL channel in Mode-B, at least option3 is supported:

- Option-1: The same Type-1 CG PUSCH can carry UL-SCH, any other UCI, and the beam report.

- Option-2: The Type-1 CG PUSCH is a dedicated type-1 CG PUSCH for carrying the beam report

  - Note: This PUSCH can NOT carry UL-SCH. This PUSCH can NOT carry any other UCI.

- Option-3: The Type-1 CG PUSCH is a type-1 CG PUSCH for carrying the beam report

  - Note: This PUSCH can NOT carry UL-SCH. This PUSCH can carry any other UCI.

FFS: whether Type-1 CG PUSCH can be transmitted if the pre-configured Type-1 CG PUSCH does NOT carry the beam report

[118b]Working Assumption

The following working assumption in RAN1#118 is revised in red.

On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported.

- Event-1: Quality of the current beam is worse than a certain threshold.

- Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th Q-th best quality.

  - M Q is RRC configured with subjective to UE capability signalling

    - UE may only indicate a single candidate value or not support Event-7.

- The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise

- The additionally supported events will be lower priority compared to event 2.

[118b] Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding resource mapping/configuration between first and second UL channel associated with a same CSI report configuration in Mode-B,

- The UE expects that there is the same periodicity (in ms) between first PUCCH resource and pre-configured resource for second UL channel.

  - FFS: Whether first PUCCH resource and pre-configured resource for second UL channel can have different periodicities (in ms)

[118b] Agreement

Regarding RS measurement for the current beam for Event 2, for Option-2a, besides for scheme-1 and scheme-2, there is no RAN1 consensus on the following enhancement for handling the case that only one TRS is configured in the indicated TCI state in RAN1#118bis

- Option-1: Introducing additional scheme: the RS for current beam can be a CSI-RS for beam management derived from the QCL RS in the indicated TCI state;

- Option-2: Further support TRS as measurement RS of current beam for determining L1-RSRP

- Option-3: Introducing additional scheme: The RS for current beam is explicitly configured by RRC or MAC-CE (Option-2C in RAN1 116b agreement).

Note 3: When only one TRS is configured in the indicated TCI state, either Scheme-1(working assumption) or Scheme-2 is used where enabling one of either Scheme-1 or Scheme-2 is selected by NW.

When the Scheme-1 is used, the UE assumes that the CSI-RS resource in the indicated TCI state is configured in a CSI-RS resource set configured with repetition.

[118b] Agreement

Regarding RS measurement for the current beam for Event 2, for Option-2a, confirm the following working assumption

- Note 3: When only one TRS is configured in the indicated TCI state, either Scheme-1(working assumption) or Scheme-2 is used where enabling one of either Scheme-1 or Scheme-2 is selected by NW.

[118b] Agreement

Regarding RS measurement for the current beam for Event 2, for Option-2a, the following working assumption in RAN1#117 is confirmed with modification:

- (Working Assumption) Enabling of either Scheme-1 or Scheme-2 should ensure the same RS type for RS measurement for current beam and new beam.

  - Note: In such case, the RS type comprises SSB and CSI-RS configured in a CSI-RS resource set configured with repetition.

[118b] Agreement

Regarding RS measurement for the current beam for Event 2, for enabling one of either Scheme-1 or Scheme-2 by NW in Option-2a, the following implicit manner is supported:

- If the RS(s) for new beam are CSI-RS configured in a CSI-RS resource set configured with repetition, Scheme-1 is enabled; otherwise, Scheme-2 is enabled.

[118b] Agreement

Regarding the triggering event determination for Event 2, the event instance(s) counting is per new beam. Further study candidate condition(s) of resetting the counting including whether resetting is needed.

[118b] Agreement

Regarding the triggering event determination for Event 2, down-select among the following alternatives for the evaluation periodicity for determining Event-2 instance [at least when DRX is not configured]

- Alt-1: The periodicity of the current beam RS should be the same as that of the new beam RS(s).

  - The evaluation periodicity is the same as the periodicity of the current and new beam RS(s)

- Alt-2: The periodicity of the current beam RS can be different from that of the new beam RS(s)

  - Alt-2_1: The evaluation periodicity is the same as the periodicity of the current beam RS;

  - Alt-2_2: The evaluation periodicity is the same as periodicity of the new beam RS;

  - Alt-2_3: The evaluation periodicity is the same as shortest periodicity of the current beam RS and new beam RS(s):

  - Alt-2_4: The evaluation periodicity is the maximum of {X ms, shortest periodicity of the current beam RS and new beam RS(s)}:

  - Alt-2_5: The evaluation periodicity is the same as largest periodicity of the current beam RS and new beam RS(s):

Note: There is the same periodicity for the new beam RS(s).

[118b] Agreement

On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, the following is supported

- For new beam measurement, in a CSI report configuration, configure legacy RRC parameter carrier that indicates the CC that the RS resource set associated with the CSI reporting configuration can be found

- FFS: Whether the current beam RS and new beam RS(s) can be in the same CC or in different CCs, regarding cross-CC beam measurement.

- FFS: Whether the indicated TCI state and new beam RS(s) can be in the same CC or in different CCs, regarding cross-CC beam measurement.

[118b] Conclusion

There is no RAN1 consensus on the following proposal:

On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, the candidate value of ‘N’ can further comprise {5, 6, 7, 8}, besides for previously agreed candidate value of {1, 2, 3, 4}.

  - RAN1#118

[118] Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, for regarding Mode-B, the pre-configured resource(s) for the second channel in Step-2 is at least type 1 CG-PUSCH.

- FFS: PUCCH as the second channel

- FFS: Whether the PUSCH can be with UL data

[118] Agreement

Regarding explicit RS configuration for new beam measurement for Event 2, at least Option-1 is supported

  - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration

  - If legacy UE capability signaling cannot be reused, introduce a UE capability signaling of indicating the maximum number of the configured RS(s) in the RS resource set.

  - FFS: The RS in the RS resource set can be updated by MAC-CE

[118] Agreement

On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, the following differential L1-RSRP report format is supported.

CRI or SSBRI #1

CRI or SSBRI #2

…

CRI or SSBRI #N

L1-RSRP #1

Differential L1-RSRP #2

…

Differential L1-RSRP #N

Differential L1-RSRP for current beam, if report mode that current beam is always reported is enabled by RRC

Note: Other contents are not precluded

.

    - Differential L1-RSRP #2~#N/current beam is determined based on the difference between measured L1-RSRP corresponding to the CRI/SSBRI #2~#N/current beam and the measured L1-RSRP corresponding to CRI/SSBRI #1

      - L1-RSRP#1 is the largest measured RSRP among reported ones, and an absolute L1-RSRP.

      - FFS: range and step size of differential L1-RSRP

    - FFS: Whether/how to report additional indication of which CRI/SSBRI(s) satisfy the condition of Event-2.

    - FFS: Additional report content(s) (e.g., reporting configuration ID, indication for synchronization state, event ID, or cell ID).

[118] Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the dedicated RRC signaling for first PUCCH channel configuration for Mode-A, down-select one of the following

- Alt-1 (dedicated SR for mode-A): Introduce RRC parameter, e.g., reportResourceRequest-UEIBR, corresponding to the one-bit indication in the first PUCCH channel

  - The RRC parameter is associated with the dedicated SchedulingRequestId.

    - Note: The detailed signaling is up to RAN2.

- Alt 2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig-ModeA-UEIBR, for the periodic PUCCH resource configuration.

  - Note: The RRC parameter is NOT associated with SchedulingRequestId.

  - FFS: how to encode 1-bit to PUCCH resource, e.g., reuse encoding mechanism of positive/negative SR.

  - The dedicated RRC parameter at least comprises the following:

    - periodicityAndOffset

    - PUCCH-ResourceID

- Above applies at least for the single CC case.

[118] Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the dedicated RRC signaling for first PUCCH channel configuration for Mode-B, down-select one of the following

- Alt 1 (dedicated SR for mode-B): Introduce RRC parameter, e.g., reportNotification-UEIBR, corresponding to the one-bit indication in the first PUCCH channel

  - The RRC parameter is associated with the dedicated SchedulingRequestId.

    - Note: The detailed signaling is up to RAN2.

- Alt 2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig-ModeB-UEIBR, for the periodic PUCCH resource configuration.

  - Note: The RRC parameter is NOT associated with SchedulingRequestId.

  - FFS: how to encode 1-bit to PUCCH resource, e.g., reuse encoding mechanism of positive/negative SR.

  - The dedicated RRC parameter at least comprises the following:

    - periodicityAndOffset

    - PUCCH-ResourceID

- Above is at least applied to a single CC case.

[118] Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the triggering procedure in Step-2 of Mode-A, select one of the following options

- Option-1: Introduce a new 1-bit field in DCI format 0_1/0_2 to trigger the transmission of the UEI beam report

  - FFS: DCI format 0_3

- Option-2: Reuse CSI request field in DCI format 0_1/0_2 to trigger the transmission of the UEI beam report

  - FFS: DCI format 0_3

- FFS: Whether/how to handle the case that multiple CSI report configuration(s) for the UE-initiated/event-driven beam report are associated with the same first PUCCH resource and/or the same scheduled PUSCH

[118] Conclusion

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-A, there is no RAN1 consensus on additionally supporting that the DCI format in Step-2 comprises DL-grant DCI format, and the second channel in Step-3 is PUCCH.

[118] Agreement

On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, the candidate value of ‘N’ at least comprises {1, 2, 3, 4}

- FFS: additional candidate value(s) of {5, 6, 7, 8}

- FFS: If ‘N’ is not RRC configured, only one L1-RSRP and CRI/SSBRI are reported by default.

[118] Agreement

Regarding RS measurement for the current beam for Event 2, for Option-2a, besides for scheme-1 and scheme-2, further down-select one of the following for handling the case that only one TRS is configured in the indicated TCI state in RAN1#118bis

- Option-1: Introducing additional scheme: the RS for current beam can be a CSI-RS for beam management derived from the QCL RS in the indicated TCI state;

- Option-2: Further support TRS as measurement RS of current beam for determining L1-RSRP

- Option-3: Introducing additional scheme: The RS for current beam is explicitly configured by RRC or MAC-CE (Option-2C in RAN1 116b agreement).

- Option-4: No further enhancement

Note: If there is no consensus in RAN1 on one of Options 1/2/3, Option 4 will be taken.

[118] Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case the pre-configured Type-1 CG PUSCH carry the beam report, for the second UL channel in Mode-B, at least one or both of the following should be supported:

- Option-1: The same Type-1 CG PUSCH can carry UL-SCH and the beam report.

- Option-2: The Type-1 CG PUSCH is a dedicated type-1 CG PUSCH for carrying the beam report

  - Note: This PUSCH can NOT carry UL-SCH. This PUSCH can NOT carry any other UCI.

[118] Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-B, UEI beam report is carried on a first available transmission occasion of the second UL channel X symbols after sending the last symbol of report notification on the first PUCCH channel.

[118] Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding resource mapping/configuration between first and second channel in Mode-B, for a given CSI report configuration, the following is provided for down-selection.

- Option-1 (one-to-one): Only one first PUCCH resource and only one pre-configured resource for second UL channel can be associated with the CSI report configuration for UE-initiated/event-driven beam reporting.

  - Option-1A: Same periodicity between first PUCCH resource and pre-configured resource for second UL channel.

  - Option-1B: No restriction in terms of periodicity.

- Option-2 (one-to-M): Only one first PUCCH resource and one or more pre-configured resource(s) for second UL channel can be associated with CSI report configuration for UE-initiated/event-driven beam reporting.

[118] Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, for at least Mode-B, the beam report should be carried in the second UL channel in the CC where the corresponding CSI report configuration is configured.

- Above applies to both cross-CC and same-CC beam report.

- Note: Above is applied to the case that second UL channel is PUSCH.

- FFS: Whether the first and second channels can be from the same/different CC.

[118] Working Assumption

On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported.

- Event-1: Quality of the current beam is worse than a certain threshold.

- Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality.

  - M is RRC configured with subjective to UE capability signalling

    - UE may only indicate a single candidate value or not support Event-7.

- The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise

- The additionally supported events will be lower priority compared to event 2.

[118] Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-A and/or Mode-B, further study the following for first PUCCH transmission

- UCI multiplexing/dropping/prioritization rule

- Conditions for the transmission of the first PUCCH.

- Whether the PUCCH resource in the first PUCCH channel can be associated with multiple CSI report configurations for UE-initiated/event-driven beam reporting from one or multiple CC(s).

- Whether/how to re-transmit the first PUCCH channel.

Whether/how to apply prohibit-timer or maximum number of (re)transmission(s) for first PUCCH channel.

  - RAN1#117

[117] Agreement

On UE-initiated/event-driven beam reporting, regarding UL signaling content(s) of L1-RSRP report depending on Event-2, in a report instance, at least Option-3 is supported

- Option-3: N ≥ 1 beam(s) are reported in the report instance,

  - At least one of N reported beam(s) should satisfy the condition of Event-2

  - N is configured by gNB

    - FFS: candidate value of ‘N’.

  - FFS: RRC can enable or disable whether current beam is always reported in addition to the N beams

- FFS: Option-1/1a/1b/2.

- Above applies at least for the single CC case

[117] Working Assumption

On beam report transmission procedure for UE-initiated/event-driven beam reporting

- For mode-A, at least support one-bit indication in the first PUCCH channel to request a resource for a second UL channel to carry beam report.

  - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling.

- For mode-B, at least support one-bit indication in the first PUCCH channel to notify a second UL channel to carry beam report.

  - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling.

- FFS: Whether/how to support multi-bit indication in the first PUCCH for mode-A and mode-B, e.g., when multi-event(s) are approved.

- FFS: details on the dedicated RRC signaling

- Above applies at least for the single CC case.

[117] Agreement

Regarding RS measurement for the current beam for Event 2, for Option-2a, support the both schemes as follows.

- Scheme-1: RS for current beam is the QCL RS in the indicated TCI state

  - FFS: Whether/How to handle the case if only one TRS is configured in the indicated TCI state.

- Scheme-2: the RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state.

- Enabling one of either Scheme-1 or Scheme-2 is selected by NW.

  - FFS: The above selection is via an explicit RRC parameter or an implicit manner, e.g., if the RS(s) for new beam are CSI-RS, Scheme-1 is enabled; otherwise, Scheme-2 is enabled.

  - (Working Assumption) Enabling of either Scheme-1 or Scheme-2 should ensure the same RS type for RS measurement for current beam and new beam.

- The above QCL RS is the RS w.r.t. QCL-TypeD, if there are two QCL RSs in the indicated TCI state.

[117] Agreement

Regarding RS measurement for the new beam for Event 2, at least Option-3a is supported

- Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured

- FFS: Option-3b/3c

  - Option-3b: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s).

  - Option-3c: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of TCI state(s) in a configured subset of the legacy RRC-configured TCI state list

[117] Agreement

On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, for a report instance where N ≥ 1 beam(s) are reported, the following is supported.

- RRC can enable or disable whether current beam is always reported

    - When enabled by RRC, the current beam + N beams from the measurement RSs for new beam(s) are reported

      - Note: The reported current beam is NOT counted in the N reported beams.

    - When disabled by RRC, N beams are reported.

[117] Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-A, the DCI format in Step-2 comprises UL-grant DCI format, and the second channel in Step-3 is at least PUSCH.

- The UL-grant DCI format at least comprises DCI format 0_1/0_2.

  - FFS: DCI format 0_3

- FFS: How to trigger the UEI beam report by the UL-grant DCI format

- FFS: the DCI format in Step-2 comprises DL-grant DCI format, and the second channel in Step-3 is PUCCH.

    - 1-bit field in the DL-grant DCI format is introduced to indicate the transmission of the UEI beam report

      - The PUCCH resource for HARQ-ACK transmission can be reused to carry both the HARQ-ACK and UEI beam report.

    - The DL-grant DCI format at least comprises DCI format 1_1/1_2.

      - FFS: DCI format 1_3

[117] Agreement

Regarding the triggering event determination for Event 2:

- If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs.

  - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam

Above feature is subject to UE capability.

- Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs

FFS: Whether the above is captured in RAN1 or RAN2 specification.

[117] Agreement

On UE-initiated/event-driven beam reporting, regarding trigger events, the following Event-1 and 7a/7b, are provided for down-selection or combination in RAN1#118 (possible outcome is that no new event is supported)

- Event-1: Quality of the current beam is worse than a certain threshold.

- Event-7a: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the worst quality.

- Event-7b: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the best quality.

[117] Agreement

Regarding explicit RS configuration for new beam measurement for Event 2, down-select the following options in the RAN1#118:

- Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration;

  - FFS: The RS in the RS resource set can be updated by MAC-CE.

- Option-2: A list of RS(s) for new beam measurement can be configured by RRC, and a subset can be activated for new beam measurement by MAC-CE.

  - FFS: If a list size is small, MAC-CE activation is not needed

- Option-3: A list of RS resource (s) for new beam measurement can be configured by RRC, and a subset of RS resource(s) in the list can be provided for new beam measurement by indicated TCI state.

- Others are not precluded.

- FFS: Each RS for new beam measurement should be associated with a configured joint/DL TCI state which can be used as the indicated TCI state

[117] Agreement

Regarding RS measurement for the current beam for Event 2, for Option-2a, besides for scheme-1 and scheme-2, further study the following for handling the case that only one TRS is configured in the indicated TCI state.

- Option-1: Introducing additional scheme: the RS for current beam can be a CSI-RS for beam management derived from the QCL RS in the indicated TCI state;

- Option-2: Further support TRS as measurement RS of current beam for determining L1-RSRP

- Option-3: Introducing additional scheme: The RS for current beam is explicitly configured by RRC or MAC-CE (Option-2C in RAN1 116b agreement).

- Option-4: No further enhancement (i.e., in such case, Scheme-2 is used)

- Others are not precluded.

  - RAN1#116-bis

[116b] Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, following modes are supported:

- Mode A (dynamically scheduling UCI by gNB):

  - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) to request a resource for a second UL channel to carry beam report

    - FFS: Request format, e.g., SR or a new UCI type.

  - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry beam report.

  - Step 3: Beam report is transmitted in second UL channel.

    - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both

  - This mode is basic UE capability (i.e. all UE supporting UE-initiated/event-driven beam reporting should support this feature).

  - No new DCI format is introduced.

- Mode B (UCI in pre-configured resource(s) for second UL channel):

  - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry beam report

    - FFS: Notification format, e.g., SR or a new UCI type.

  - Step 2: UE transmits the beam report in the second UL channel.

    - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both

  - The notification in Step1 is in a separate reporting instance from the beam report in Step 2.

FFS: Whether UE receives acknowledge information with response to each step for all modes

For above procedures, cross-CC beam reporting is supported for both modes.

- FFS: Details.

[116b] Agreement

On UE-initiated/event-driven beam reporting, regarding trigger-event detection for beam reporting, at least support Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam.

- At least L1-RSRP is supported as quality metrics used for Event-2

  - FFS: How the L1-RSRP is used to determine the triggering event (e.g. timer, counter, filter coefficient)

  - FFS: Whether the network controls how the L1-RSRP is used to determine the triggering event

- Regarding RS measurement for the new beam for Event-2, down-select one or more of the following:

  - Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured by RRC (e.g., reusing legacy configuration of RS measurement or in TCI-State) or MAC-CE

  - Option-3b (implicit manner): The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s).

  - Option-3c (implicit manner): The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of configured TCI state(s).

- Note-1: ‘New/current beam’ is for discussion purpose.

- Note-2: Other trigger events/quality metrics (e.g., L1-SINR) are not precluded.

- Note-3: For above implicit manner(s), if there are two QCL RSs in a TCI state, the measurement RS is derived from RS w.r.t. QCL-TypeD, if applicable.

[116b] Agreement

On UE-initiated/event-driven beam reporting, regarding Event-2, the threshold value is RRC configured

[116b] Agreement

On UE-initiated/event-driven beam reporting, regarding Event-2, ‘current beam’ is a beam corresponding to the indicated TCI state.

- Regarding RS measurement for the current beam for Event-2, Option-2a is supported:

  - Option-2a (implicit manner): The RS for current beam is implicitly derived from a QCL RS of indicated TCI state.

    - FFS: The RS for current beam can be either the QCL RS in the indicated TCI state or the SSB which is QCLed with the QCL RS in the indicated TCI state.

  - FFS: Option-2c (explicit manner): The RS for current beam is explicitly configured by RRC or MAC-CE.

    - Note: SSB or CSI-RS can be configured

[116b] Agreement

On UE-initiated/event-driven beam reporting, further study the following trigger events:

- Event-1: Quality of the current beam is worse than a certain threshold.

- Event-3: Quality of a new beam is better than a certain threshold.

- Event-4: Quality of the current beam is worse than a threshold 1, and quality of at least one new beam is better than a threshold 2.

- Event-5: Absolute value of the difference between the quality of the current beam and the quality of at least one new beam is lower than a threshold.

- Event-6: When the current beam is not in the best K>1 beams (out of configured beams for measurement and reporting).

- Event-7a: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the worst quality.

- Event-7b: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the best quality.

- Event-8: Quality of M>1 new beams, such as L1-RSRP, become a threshold value better than the current beam.

- Event-9: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the configured reference RS (can be SSB or CSI-RS).

[116b] Agreement

On UE-initiated/event-driven beam reporting, regarding UL signaling content(s) of L1-RSRP report depending on Event-2, in a report instance, the following options are provided for down-selection (other options are not precluded) in RAN1#117

- Option-1 (variable size): N beam(s) are reported in the report instance, where N ∈ {1, 2, ..., Nmax}

  - The N beam(s) should satisfy the condition of Event-2

  - Nmax is configured by gNB

  - FFS: Whether the indication of payload size should be provided additionally.

- Option-1a (variable size): N beam(s) are reported in the report instance, where N ∈ {1, 2, ..., Nmax}

  - At least one of N reported beam(s) should satisfy the condition of Event-2

  - Nmax is configured by gNB

  - FFS: Whether the indication of payload size should be provided additionally.

  - FFS: Details on how value of N is determined by the UE

- Option-1b: N beam(s) are reported in the report instance, where N ∈ {1, 2, ..., Nmax}

  - The N beam(s) should satisfy the condition of Event-2

  - Nmax is configured by gNB

  - Payload size does not vary as a function of N

  - FFS: Zero-padding can be provided if N is less than Nmax.

- Option-2: Only N=1 beam is reported in the report instance

  - The reported beam should satisfy the condition of Event-2

- Option-3: N ≥ 1 beam(s) are reported in the report instance,

  - At least one of N reported beam(s) should satisfy the condition of Event-2

  - N is configured by gNB

- Other options are not precluded.

- FFS: Whether the measurement results for current beam is always reported or can be enabled by RRC.

- FFS: When current beam is reported, whether the current beam is counted in the N reported beams.

- The selected option shall satisfy Event-2.

  - RAN1#116

[116] Agreement

On UE-initiated/event-driven beam report, at least of following aspects should be included:

- Trigger-event detection for beam reporting by UE

  - UE monitors RS to assess if a beam-reporting trigger condition has been met

  - FFS: Trigger condition for declaring beam-reporting event

- Beam-report transmission by UE

  - Signaling contents in the beam report

  - Down-selection one or more options (strive for one) between the following options as signaling medium/container for beam report transmission

    - MAC-CE

    - UCI

    - Others are not precluded.

On UE-initiated/event-driven beam report, the following aspects may be included:

- UE requesting UL resource(s) for the beam report

- UE notifying transmission of beam report

- gNB preconfigured resources

Other procedure(s) as required

[116] Agreement

On UE-initiated/event-driven beam reporting, regarding trigger-event detection for beam reporting, RAN1 further study at least the following aspects: quality metrics, event-definition and threshold.

- Further study trigger events, including the following example as a starting point

  - Event-1: Quality of the current beam is worse than a certain threshold.

  - Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam.

  - Event-3: Quality of a new beam is better than a certain threshold.

  - Event-4: Quality of the current beam is worse than a threshold 1, and quality of at least one new beam is better than a threshold 2.

  - Others are not precluded.

- Note: Companies are encouraged to provide details on procedure (e.g. how it is used) related to their preferred event

[116] Agreement

On UE-initiated/event-driven beam reporting, at least support L1-RSRP as a measurement quantity on SSB for intra-cell and inter-cell, and periodic CSI-RS for beam management

- Notes: measurement results may be contained in the beam report and/or used as quality metric(s) to initiate/trigger the reporting.

- FFS: Semi-persistent CSI-RS and aperiodic CSI-RS.

- FFS: Whether/how to support L1-SINR measurement, assuming legacy RS or RS combination (e.g., CMR only, CMR+ZP/NZP-IMR) for Rel-16 SINR is reused.

- FFS: Whether/how to specify filtering operation for L1-RSRP.

[116] Agreement

On UE-initiated/event-driven beam reporting, regarding signaling content(s), at least support DL RS resource indicator and L1-RSRP

- FFS: Study and decide whether additional contents can be supported.

- FFS: L1-RSRP format, e.g., absolute and/or differential value.

- Note: Above does not imply to preclude discussion on L1-RSRP filtering.

- The actual reported content depends on the triggering event

  - Support of one or multiple events will be discussed separately

# References

| 1 | R1-2503237 | Remaining issues for UE-initiated/event-driven beam management | FUTUREWEI |
| --- | --- | --- | --- |
| 2 | R1-2503275 | On UE initiated/event-driven beam management | Huawei, HiSilicon |
| 3 | R1-2503303 | Enhancements for UE-initiated/event-driven beam management | MediaTek Inc. |
| 4 | R1-2503351 | Remaining issues on UE-initiated/event-driven beam management | vivo |
| 5 | R1-2503433 | Remaining Details of Rel-19 UE/Event-Driven Beam Management | InterDigital, Inc. |
| 6 | R1-2503509 | Discussion on UE-initiated/event-driven beam management | Spreadtrum, UNISOC |
| 7 | R1-2503554 | Views on Rel-19 UE-initiated/event-driven beam management | Samsung |
| 8 | R1-2503612 | Remaining issues for UE-initiated beam management | Ericsson |
| 9 | R1-2503668 | Discussion on enhancements for UE-initiated/event-driven beam management | ZTE Corporation, Sanechips |
| 10 | R1-2503729 | On UE-initiated/event-driven beam management | Ofinno |
| 11 | R1-2503786 | Remaining issues on UE-initiated/event-driven beam report | CATT |
| 12 | R1-2503816 | Enhancements for UE-initiated/Event-Driven Beam Management | Panasonic |
| 13 | R1-2503824 | Discussion on enhancements for UE-initiated/event-driven beam management | CMCC |
| 14 | R1-2503876 | Discussion on enhancements for UE-initiated/event-driven beam management | Xiaomi |
| 15 | R1-2503912 | Enhancements for UE-initiated/event-driven beam management | Lenovo |
| 16 | R1-2503928 | Discussion on enhancements for UE-initiated or event-driven beam management | NEC |
| 17 | R1-2503953 | Discussion on enhancements for UE-initiated/event-driven beam management | Ruijie Networks Co. Ltd |
| 18 | R1-2503985 | UE-initiated beam management | LG Electronics |
| 19 | R1-2504061 | Enhancements for UE-initiated/event-driven beam management | Sony |
| 20 | R1-2504076 | Enhancements to facilitate UE-initiated/event-driven beam management | Nokia |
| 21 | R1-2504084 | Discussion on enhancements for UE-initiated/event-driven beam management | Fujitsu |
| 22 | R1-2504095 | Discussion on the UE-initiated/event-driven beam management | HONOR |
| 23 | R1-2504117 | Discussion on enhancements for UE-initiated/event-driven beam management | Hyundai Motor Company |
| 24 | R1-2504133 | Enhancements for UE-initiated/event-driven beam management | ETRI |
| 25 | R1-2504172 | Enhancements for UE-initiated/event-driven beam management | Transsion Holdings |
| 26 | R1-2504228 | Discussions on UE-initiated/event-driven beam management | OPPO |
| 27 | R1-2504297 | Discussion on enhancements for UE-initiated or event-driven beam management | Google |
| 28 | R1-2504312 | Remaining issues for UE-initiated beam management | Apple |
| 29 | R1-2504388 | UE-initated/event-driven beam management | Qualcomm Incorporated |
| 30 | R1-2504444 | Enhancements for UE-initiated/event-driven beam management | Sharp |
| 31 | R1-2504453 | Discussions on UE-initiated/event-driven beam management | China Telecom |
| 32 | R1-2504476 | Remaining issues on enhancements for UE-initiated/event-driven beam management | KDDI Corporation (TTC) |
| 33 | R1-2504495 | Discussion on enhancements for UE-initiated/event-driven beam management | NTT DOCOMO, INC. |
| 34 | R1-2504535 | Discussion on UE-initiated/event-driven beam management | ITRI, Acer Incorporated |
| 35 | R1-2504572 | Discussion on UE initiated beam report | ASUSTeK |
| 36 | R1-2504579 | Discussion on Enhancements for UE-initiated/event-driven beam management | NICT |
| 37 | R1-2504607 | Discussion on UE-initiated/event-driven beam management | TCL |