3GPP TSG RAN WG1 #121							         R1-2504501

St Julian’s, Malta, May 19th – 23th, 2025

Source:	NTT DOCOMO, INC.

Title:	Discussion on modulation aspects of physical channel design for Ambient IoT

Agenda Item:	9.4.1

Document for: 	Discussion and Decision

- Introduction

At the RAN#107 meeting, a revised WID “Solutions for Ambient IoT (Internet of Things) in NR” was approved [1]. The objectives are as follows.

General Scope The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope: - The overall objective shall be to standardize the following Ambient IoT device: Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Deployment scenario 1 with Topology 1, according to D1T1-B. - FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum. - Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor. - Traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command). - Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769: - Case 1-4 for D1T1-B - Proximity determination via Solution 1 in TR 38.769 only. - Device (un)availability via Direction 1 in TR 38.769 only. WGs begin their discussions from the decisions already made in TR 38.769, with the following refinements for the scope: The following objectives are set, within the General Scope: - RAN1 scope: - PRDCH and PDRCH, which are the only physical channels in R2D and D2R, respectively. - R2D and D2R signal(s) - Multiplexing/multiple access in R2D is by only TDMA, and in D2R is by only TDMA and FDMA. - R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations. - R2D transmission supports only the Manchester line code in TR 38.769 - D2R transmission supports: - Either the Manchester line code in TR 38.769 or no line code (one to be down-selected); and - A corresponding small frequency shift method according to the options in TR 38.769. - R2D does not support FEC. D2R supports only convolutional code with generator polynomials as per TS 36.212. Applying or not applying the FEC to D2R is specified by ensuring it is under the reader control and applies to all devices targeted by the reader. - PRDCH and PDRCH both support transmission without CRC, and with CRC as per the generator polynomials in TS 38.212 for 6-bit CRC and 16-bit CRC. Cases to use which length of CRC, or no CRC, to be decided in RAN1. - D2R supports physical layer repetition transmission. R2D does not support physical-layer repetition transmission.

As captured in WID objective, PRDCH and PDRCH, which are the only physical channels in R2D and D2R are specified in Rel-19. In this contribution, we discuss the modulation aspects of these physical channels. We would like to note that chip duration for D2R is discussed in our companion tdoc for 9.4.2[2].

- Discussion

  - R2D (PRDCH)

According to the WID objective above, only OOK-4 modulation would be supported for R2D. In this section, we discuss the details on OOK-4 for Rel-19 A-IoT device.

    - CP handling

CP handling for PRDCH

At the RAN1#120bis meeting, following agreement was made for the further down-selection of CP handling method especially for M values>12.

Agreement For the below agreement, further update on the followings Agreement For further down-selection among CP handing which retains subcarrier orthogonality, at least for PRDCH, at least Method Type 1 is supported - For supported M values <= 12 - RAN1 will not further pursue additional CP handling design - For supported M values > 12 - RAN1 will further down-select one from the followings - Option 1: Candidate 3 of M2-1-1 (as per agreements from RAN1#120) - Insert padding chips only at the end OOK chips of OFDM symbol - The last 2 out of M OOK chips at the end of an OFDM symbol are always ‘ON’ - Option 3: RAN1 will not further pursue additional CP handling design

At the RAN1#120bis meeting, it was agreed to support M=2, 6, 12 and 24 for OOK-4 of R2D. Then, it should be further discussed whether to apply option 1 or option 3 for M=24.

For option 1, candidate 3 of M2-1-1 is based on the CP handling Method 2 which device does not remove CP. As in the agreement, the last 2 chips of 24 OOK chips are always ON and cannot be used to convey payload. This means that candidate 3 introduces 8.3% overhead for such parity chips.

Observation 1: For R2D, candidate 3 of CP handling Method Type 2-1-1 introduces 8.3% overhead for parity chips when M=24.

In addition, it has already been agreed to support only Method Type 1 for M=2, 6, and 12, and hence it introduces additional complexity at both reader and device side for CP handling of M=24 if candidate 3 of M2-1-1 is applied. In fact, it is captured in WID objective that “one solution for CP handling”. In our understanding, the intention is to down-select one Method type regardless of M value. In that sense, we believe only Method 1 is supported even for M=24 unless critical concern is identified.

Based on the above, the discussion point is whether performance gain by M2-1-1 candidate 3 is worth to accept the additional complexity at reader/device and 8.3% overhead by ON chips of parity chip.

For CP handling Method type 1, it can be concerned whether and how to handle the difference on long CP and short CP. According to the current NR specification below, long CP is inserted per 0.5ms. More specifically, CP length of normal CP with 15 kHz SCS is short as 144κ∙2-μ when symbol index l is other than 0 or 7 while it is long as 144κ∙2-μ+16κ when symbol index l is 0 or 7.

If we assume that reader uses only short CP symbols for R2D, this introduces scheduling restriction, e.g., only six symbols in row can be used. Thus, this is not practical and not preferable in terms of resource utilization. Otherwise, i.e., long CP symbol as well as short CP symbol are used for R2D while device would not differentiate the long/short CP, it is equivalent to the time drift of 16κ per 0.5ms, and hence, this may result in the performance degradation. However, according to the discussion at the RAN1#120bis meeting, it was observed that the difference on the number of sample can be only 1 between long CP and short CP, e.g., assuming 1.92MHz clock frequency. With this assumption, it was also observed that the performance impact due to the different length on long CP and short CP can be negligible.

For CP handling Method Type 1, it can be also concerned that the performance would be degraded due to the time drift especially for large M value such as 24 and for large TBS which takes long time to receive. According to the discussion at RAN1#120bis meeting, it was evaluated by companies that CP handling Method 1 can work at least for 96bits TBS without error floor while candidate 3 of M2-1-1 can provide a several dB gain. For the larger TBS, it is unclear whether Method type 1 only works well for M=24. However, considering that Manchester coding is supported for PRDCH and device can calibrate the time drift by detecting rising/falling edge at the middle of each codeword, the performance degradation by time drift for large payload can be alleviated in our view.

Based on the discussion above, we don’t see the critical concern even if only Method 1 is supported for M=24.

Proposal 1: For PRDCH, support only CP handling Method Type 1 for M=24, i.e., support option 3 in the agreement at the RAN1#120bis.

For CP handling Method 1, it was observed during SI that device needs to be aware of or determine the boundary of an OFDM symbol (i.e. the beginning of an OFDM symbol) to determine CP location[3]. To ensure that A-IoT device can identify the OFDM symbol boundary for CP handling, how to implicitly/explicitly indicate the OFDM symbol boundary should be discussed. In our understanding, as assumed in TR, it can be implicitly indicated that the start of R-TAS (especially SIP) is aligned with OFDM symbol boundary. In addition, it can be further discussed whether the start of R-SIP should be aligned with OFDM symbol index 0 or 7 in a slot, i.e., R2D transmission always starts from OFDM symbol index 0 or 7. With this restriction, device can differentiate long CP and short CP and handle differently. However, as discussed above, the performance impact on the difference of long CP and short CP can be marginal. In that sense, it seems not necessary to restrict the starting point of R2D transmission, i.e., R2D can start from any OFDM symbol.

It should be noted that this can be discussed in 9.4.3 as a part of R-TAS design.

Proposal 2: Confirm that start timing of R-SIP is aligned with the OFDM symbol boundary for CP handling of PRDCH reception.

  - R-SIP can start from any OFDM symbol.

CP handling for CAP

According to the above agreement at the RAN1#120bis meeting, it is “at least” for PRDCH and it should be further discussed whether the same CP handling as PRDCH can be applied to R-TAS. In our view, given that device may not be able to remove CP accurately before clock calibration, CP between SIP and CAP may need to be handled differently from that for PRDCH. Therefore, we suggest to discuss CP handling for R-TAS before concluding that RAN1 will not further pursue additional CP handling design even for R-TAS.

It would be related to the exact chip length for SIP and CAP, then the detailed discussion is in section 2.1.2 of our companion tdoc for AI9.4.3[4].

    - Chip duration

At the RAN1#120 meeting, it was agreed to support only 15 kHz SCS for R2D in Rel-19 and OOK-4 is defined as OOK modulation of M-chips per one OFDM symbol with 15kHz SCS. To define the exact chip duration, it should be clarified whether chip length is uniform with or without CP. In our understanding, it depends on which CP handling Method type to apply. More specifically, chip length should be uniform excluding CP part for CP handling Method 1 or Method 2 combined with Method 1 while chip length should be uniform including CP part for CP handling Method 2 without combination with Method 1. As discussed above, we believe that CP handling Method type 1 should be applied to all the supported M values, thus chip duration for PRDCH should be uniform excluding CP. It should be noted that the exact chip duration for CAP can be separately discussed from that for PRDCH.

Proposal 3: For PRDCH, one chip duration should be (1/SCS)/M.

    - Padding for the last OFDM symbol

It was captured in TR[3] that padding can be assumed when the last OFDM symbol of R2D is not fully occupied from transmitter perspective.

TR38.769: For R2D waveform generation using a DFT-s-OFDM based transmitter, from the transmitter perspective, when the generated number of chips for the R2D transmission does not fully occupy the last OFDM symbol, at least padding can be used.

Based on the TR description, it was discussed how to handle padding for the last OFDM symbol of PRDCH and the following agreement was made.

Agreement When the generated number of chips for the R2D transmission does not fully occupy the last OFDM symbol, padding is used. - Padding is to be down-selected among the following alternatives: - Alt 1a: The content of padding is up to reader implementation and transparent to device. - Note: the timeline determination of any timing relationship refers to the end of padding. - Note: it implies the device should be aware of the duration of padding or the last OFDM symbol boundary by implementation. FFS how accurately the device can be aware. - Alt 1b: The content of padding is up to reader implementation and transparent to device. - Note: the timeline determination of any timing relationship refers to the start of the padding. - Alt 2: Specify the detailed content of padding - Note: the timeline determination of any timing relationship refers to the end of padding. - Note: the end chip(s) of the padding content shall follow the CP handling solution determined in RAN1, and may be affected by other agreements.

Regarding whether the content of padding should be specified or not, we don’t see the strong need to specify. In our understanding, the end of PRDCH before padding is indicated by message size in MAC layer, TBS in PHY layer, R2D postamble or violation of Manchester coding rule. In addition, as discussed for CP handling, device can identify OFDM symbol boundary. In that sense, without specifying the content of padding, device can identify the end of padding in our understanding. Furthermore, the content of padding should consider various aspects such as SIP design, R2D postamble design if supported, Manchester coding violation rule and CP handling for PRDCH. It would complicate the discussion and given only one meeting remaining, the workload is concerned. Hence, we suggest supporting either Alt.1a or Alt.1b. it should be noted that reader would handle that padding chips are not misunderstood as SIP, R2D postamble if supported etc.

For Al.1a and Alt.1b, one difference is the reference point which is used for timing determination, i.e., end of padding is referred for Alt.1a and start of padding is referred for Alt.1b. Meanwhile a few chip-level latency reduction may not help system level latency, we slightly prefer Alt.1b which can start timing count bit earlier than Alt.1a.

Proposal 4: When the generated number of chips for the R2D transmission does not fully occupy the last OFDM symbol, padding is used while the content of padding it up to reader implementation and transparent to device.

  - The timeline determination of any timing relationship refers to the start of the padding.

Fig.1: reference point for timing determination for R2D.

- Conclusion

In this contribution, we discussed modulation aspects on physical channel design for A-IoT device. Based on the discussion, we made following observations and proposals.

Observation 1: For R2D, candidate 3 of CP handling Method Type 2-1-1 introduces 8.3% overhead for parity chips when M=24.

Proposal 1: For PRDCH, support only CP handling Method Type 1 for M=24, i.e., support option 3 in the agreement at the RAN1#120bis.

Proposal 2: Confirm that start timing of R-SIP is aligned with the OFDM symbol boundary for CP handling of PRDCH reception.

  - R-SIP can start from any OFDM symbol.

Proposal 3: For PRDCH, one chip duration should be (1/SCS)/M.

Proposal 4: When the generated number of chips for the R2D transmission does not fully occupy the last OFDM symbol, padding is used while the content of padding it up to reader implementation and transparent to device.

  - The timeline determination of any timing relationship refers to the start of the padding.

Reference

[1] RP-250796, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN#107, Mar. 2025.

[2] R1-254502, “Discussion on coding and CRC aspects of physical channel design for Ambient IoT”, NTT DOCOMO, INC., May 2025.

[3] TR 38.769 V19.0.0, “Study on solutions for Ambient IoT (Internet of Things) in NR”, Dec. 2024.

[4] R1-254503, “Discussion on timing acquisition and synchronization for Ambient IoT”, NTT DOCOMO, INC., May 2025.