3GPP TSG RAN WG1 #121 				                                                                       R1-2504589

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	9.4.4

Source:	TCL

Title:	Discussion on multiple access, scheduling information and timing relationships for Ambient IoT

Document for:	Discussion and Decision

Introduction

A new WID for Ambient IoT was approved in RAN#106 meeting [1]. Rel-19 work item focuses on Device 1 and deployment scenario 1 with topology 1 (D1T1-B). According to the WID, RAN1 related scope is provided as following.

- RAN1 scope: - PRDCH and PDRCH, which are the only physical channels in R2D and D2R, respectively. - R2D and D2R signal(s) - Multiplexing/multiple access in R2D is by only TDMA, and in D2R is by only TDMA and FDMA. - R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations. - R2D transmission supports only the Manchester line code in TR 38.769 - D2R transmission supports: - Either the Manchester line code in TR 38.769 or no line code (one to be down-selected); and - A corresponding small frequency shift method according to the options in TR 38.769. - R2D does not support FEC. D2R supports only convolutional code with generator polynomials as per TS 36.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis). - PRDCH and PDRCH both support transmission without CRC, and with CRC as per the generator polynomials in TS 38.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis) for 6-bit CRC and 16-bit CRC. Cases to use which length of CRC, or no CRC, to be decided in RAN1. - D2R supports physical layer repetition transmission. R2D does not support physical-layer repetition transmission.

In this contribution, we will discuss multiple access, scheduling related, timing relationships and any other L1 procedural aspects for Ambient IoT.

Discussion

Random access

In this section, we will further discuss AIoT random access procedure and potential impact of AIoT paging message.

MSG1

In previous meeting, FDMA based MSG1 has been supported. In RAN1#120bis meeting [2], the following agreements on MSG1 transmission were achieved. Based on the agreement, TDMA with X>1 was supported for MSG1 transmission and the maximum number of X is 2.

Agreement For Msg 1 transmission determined by one R2D transmission triggering random access, support X=1 and X=2 time domain resource(s) for D2R transmission(s) for Msg1 only, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s) in Rel-19. - All devices support the above - Note: the impact of specification support (at least including signalling overhead) for X=2 to a reader supporting only X=1 should be minimized Only support X=1 time domain resource for D2R transmission for Msg3 in response to a PRDCH for Msg2 transmission. Agreement Use 1 bit to indicate the value of X (X=1 or X=2) time domain resource(s) for Msg1 transmission(s). Agreement For X=1 or X=2, from device perspective, the starting time for the first Msg1 time domain resource is Toffset1 after the end of the corresponding R2D transmission triggering random access. - FFS Toffset1 is predefined in the spec or indicated implicitly or explicitly by the R2D transmission triggering random access. - FFS detailed value(s) of Toffset1, considering at least the device processing time including FEC impact. - FFS: how to define the end of the R2D transmission in 9.4.1. Working assumption For X=2, from device perspective, for determination of the starting time for the second Msg1 time domain resource: - Derived based on the starting time of the first Msg1 time domain resource plus Toffset2 - FFS Toffset2 is predefined in the spec or derived by a rule or indicated implicitly or explicitly by the R2D transmission triggering random access. Agreement At least for CBRA, for FDMA of multiple Msg1 transmissions in response to a R2D transmission triggering random access, support only the same data rate for the FDMed Msg1 transmissions.

It was agreed that the same data rate is used for the FDMed MSG1 transmissions. The agreement can also be applied for TDMAed MSG1 transmissions. We have the following proposal.

Proposal 1: At least for CBRA, for multiple Msg1 transmissions in response to a R2D transmission triggering random access, support only the same data rate for the Msg1 transmissions.

The remaining issue is how to determine the multiple MSG1 resources. Based on the agreement, the number of MSG1 resources can be indicated by a R2D signaling. For contention-based random access and X=2, a MSG1 resource set includes two resources, devices can randomly select a resource within the MSG1 resource set to transmit MSG1. The time interval between the R2D signaling and the corresponding MSG1 resource set (i.e., time interval Toffset1 between the R2D signaling and the first MSG1 resource) can also be explicitly indicated by the R2D signaling. For the resource location of the second MSG1 resource when X=2, it can be derived based on the starting time of the first MSG1 time resource plus Toffset2, i.e., the starting time of the second MSG1 resource can be derived by Toffset1+Toffset2. Toffset2 can be predefined or explicitly indicated by the R2D signaling. For example, the value of Toffset2 corresponds to the length of the first MSG1 time resource and the guard time between the adjacent resources, then the starting time for the second Msg1 time resource can be derived by Toffset1+Ndur+GP, in which Ndur denotes the length of the first MSG1 time resource and GP denotes the guard time between the adjacent resources. The guard time can also be included in Ndur.

Proposal 2: Clarify each Msg1 time domain resource should include the resource for D2R preamble.

Proposal 3: For TDMA based MSG1, Toffset1 is explicitly indicated by a R2D signaling.

Proposal 4: For X=2, from device perspective, for determination of the starting time for the second Msg1 time domain resource:

- Derived based on the starting time of the first Msg1 time domain resource plus Toffset2

- Toffset2 is predefined in the spec or indicated explicitly by the R2D transmission triggering random access.

As discussed as above, the R2D signaling will indicate multiple information for MSG1 time domain resources, which includes the value of X, Toffset1 and Toffset2. Another way to indicate the information is to indicate a combination of  {X, Toffset1, Toffset2} or {X, Toffset1} as shown in Table 1. According to {X, Toffset1, Toffset2} or {X, Toffset1}, the time location of one or two MSG1 resources can be determined.

Table 1 An example of MSG1 time domain resource set

| Index | X | Toffset1 | Toffset2 |
| --- | --- | --- | --- |
| 1 | X1 | Offset1 | Offset3 |
| 2 | X2 | Offset2 | Offset4 |

Proposal 5: Consider indicating the combination of {X, Toffset1, Toffset2} or {X, Toffset1} by a R2D signaling to determine MSG1 time domain resources.

If device does not detect a MSG2 or device receives a MSG2 and the random ID in the MSG2 does not match the device’s ID, MSG1 retransmission will happen. For the MSG1 retransmission, the same information of MSG1 resource set for initial transmission can be considered. The difference is that the reference point should be the subsequent AIoT paging triggering MSG1 retransmission.

Proposal 6: For the MSG1 retransmission, the same information of MSG1 resource set for initial transmission can be used.

MSG2

Option 1: A PRDCH for Msg2 transmission corresponds to a A-IoT Msg1 received from one device

As shown in Figure 1, regardless of MSG1 transmission manner (taking TDMA as an example), multiple MSG2 resources are needed to response to multiple MSG1 from multiple devices. Referring to RFID and NR random access, each MSG2 contains device ID liking RN16 in RFID or preamble in NR. Device determines whether the random access is successful or not by comparing its own ID with the ID in MSG2. One issue is whether it is necessary to define the Msg2 monitoring window for Msg2 reception. If TDMA with X>1 or FDMA is used for MSG1 transmission, the MSG2 resources for different devices can be within a common time duration which is defined based on the end of the MSG1 resource set. For further saving the device’s power consumption, how the device determines its own MSG2 resource location should be further study. Implicitly or explicitly way can be considered. For example, the MSG2 resource location for each device can be indicated by a R2D message or determined based on a predefined rule. If TDMA with X=1 is used for MSG1 transmission, the MSG2 resource location can be indicated by R2D message or determined based on a predefined rule directly.

Figure 1 Illustration of MSG2 for option 1

Proposal 7: For option 1 and TDMA with X>1 or FDMA based MSG1 transmission, a common time duration (i.e., Msg2 monitoring window) can be defined for MSG2 resources for different devices.

Proposal 8: For Option 1, consider how the device to determine the MSG2 resource location within the common time duration to further saving the device’s power consumption.

Option 2: A PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices

A PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices, the starting time for Msg2 monitoring is common for multiple Msg1 resources.

In this way, one MSG2 can contain responses for multiple devices. One MSG2 is divided into several parts and each part carries the feedback for one device. If the information of feedback is similar as RFID and NR, i.e., RN16 in RFID or preamble in NR, the resource overhead would also be large. Another solution is each part corresponds to one MSG1 resource, and only 1 bit is needed for each part. Device achieves MSG2 based on its own MSG1 resource and determines whether the random access is successful.

As similarly as Option 1, how the device determines its own MSG2 resource location within the MSG2 should be further study. It can be indicated by a R2D message or determined based on a predefined rule.

Proposal 9: Consider how the information carried by MSG2 to confirm whether the random access for a device is successful:

- Random ID as similar as in RFID and NR

- ACK/NACK feedback

Proposal 10: For Option 2, consider how the device to determine its own location within a MSG2.

Another issue is that MSG2 retransmission is possible since the device does not detect it or the reader device does not detect MSG3 because of the device’s poor channel quality or energy status. How to determine the retransmission resource of MSG2 should be further discussed.

Proposal 11: Further study the retransmission resource of MSG2.

MSG3

In the last meeting, whether the same or different data rate is used for MSG3 was discussed. To simplify the AIoT design in Rel-19, only the same data rate is considered for MSG3. For the D2R chip rate, the same chip rate can be used for MSG1 and MSG3, which is indicated by a R2D signaling.

Proposal 12: Support only the same data rate for MSG3. The same chip rate is used for MSG1 and MSG3.

In RAN1#120 meeting, the following agreement on MSG3 were achieved [3].

Agreement Support FDMA of D2R transmissions for Msg3 from multiple devices in response to a PRDCH for Msg2 transmission corresponding to multiple Msg1 during access procedure. Agreement Support the following option(s) for frequency domain resource for Msg3 transmission: - Option 2: The frequency domain resource for Msg3 can be determined based on explicit indication in the PRDCH for Msg2 transmission for one or multiples devices. - FFS: support option 1 as a default if PRDCH does not provide the indication or based on a rule - Option 1: the frequency domain resource for Msg3 reuses the frequency domain resource for Msg1 for the same device

According to the agreement, FDMA is used for Msg3 from multiple devices. It was discussed the frequency domain resource for Msg3 transmission can be determined by implicit rule, i.e., reuses the frequency domain resource for Msg1 for the same device. The method can be used in case of FDMA for Msg1 transmission. However, if TDMA is used for Msg1 is applied, how to determine the resource for Msg3 by implicit rule should be considered.

Observation 1: If TDMA is used for Msg1 is applied, option 1 (i.e., the frequency domain resource for Msg3 reuses the frequency domain resource for Msg1 for the same device) cannot be used directly.

Proposal 13: Support option 1 (i.e., the frequency domain resource for Msg3 reuses the frequency domain resource for Msg1 for the same device) at least for FDMA based Msg1 transmission.

As it is possible that the payload of MSG3 is larger than that of MSG1, the size of resources for MSG1 and MSG3 should be different. Hence, MSG2 can indicate the resource size of MSG3 and the other parameters can reuse that of MSG1.

Proposal 14: The resource size of MSG3 can be indicated by MSG2 and the frequency location of MSG3 can be reused that of MSG1.

Scheduling and Timing relationships

Processing timing

Several minimum and maximum processing time were defined for AIOT as shown in the following:

- TR2D_min: Minimum Time between a R2D transmission and the corresponding D2R transmission following it.

- TD2R_min: Minimum Time between a D2R transmission and the corresponding R2D transmission following it.

- TR2D_R2D_min: Minimum Time between two different consecutive R2D transmissions to the same A-IoT device.

- TR2D_max: Define a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it, so that the device transmits D2R transmission within [TR2D_min, TR2D_max].

- TD2R_max: Maximum time between the D2R transmission and the corresponding R2D transmission following it, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max], when a R2D transmission in response to a D2R transmission is expected for A-IoT Msg2 response to A-IoT Msg1 for the A-IoT device.

Regarding the minimum and maximum processing time, the following agreements were achieved in the last meeting [2].

Agreement A device is not required to monitor a corresponding R2D transmission earlier than TD2R_min after the end of a D2R transmission from the device. Conclusion For any timing requirement that needs to be specified from device perspective by RAN1, the timing(s) do not include the impacts of SFO from the RAN1 perspective. - Whether 3GPP needs to specify some requirement on device’s SFO is up to RAN4 discussion. - Note: RAN1 will not specify TR2D_min or TR2D_max in Rel-19. - Note: SFO may still be considered when discussing time-domain resources for Msg1 when X=2.

For TD2R_min, it includes the processing time of a D2R transmission and the preparation time of the corresponding R2D transmission by the reader. For TR2D_R2D_min and TD2R_max, it is beneficial to specify the values from the perspective of saving device’s power consumption. Based on TR2D_R2D_min and TD2R_max, it can avoid the device to continuous reception. The value of TD2R_min and TD2R_max can be defined based on D2R chip duration. And the value of TR2D_R2D_min can be defined based on R2D chip duration.

Observation 7: For TR2D_R2D_min and TD2R_max, it can avoid the device to continuous reception, which can save device’s power consumption.

Proposal 22: Specify the value of TD2R_min, TR2D_R2D_min, and TD2R_max based on chip duration:

- The value of TD2R_min and TD2R_max is defined based on D2R chip duration.

- The value of TR2D_R2D_min is defined based on R2D chip duration.

Scheduling related aspects

In previous meetings [2] [3], the following agreements on scheduling related aspects were achieved.

Agreement (RAN1#120) The following is supported for indicating the D2R chip duration: • The D2R chip duration is indicated in R2D control information from predefined a set of D2R chip duration values Agreement (RAN1#120) The payload size (i.e. TBS-like) for PDRCH transmission with variable size is explicitly indicated in the corresponding R2D control information. Agreement (RAN1#120bis) For scheduling D2R transmission, any scheduling information related to resource allocation that needs to be signaled is indicated by higher-layer signaling via the corresponding PRDCH. Working assumption (RAN1#120bis) For indicating the payload size (i.e. TBS-like) for PDRCH transmission with variable size: - 7 bits for byte-level D2R payload size indication.

For D2R transmission, the scheduling information related to time/frequency resource allocation will be indicated by higher-layer signaling according the agreement. The other information for D2R transmission should include D2R chip duration, payload size, coding, number of repetitions, midamble related information and target device ID(s). For the information, some of indicated information can be one-shot valid while some of them can be long-term valid, i.e., valid for multiple D2R transmission. Taking the chip duration as an example, it may not essential to change the chip duration for each D2R transmission, hence, the chip duration indicated by a PRDCH can be used for multiple D2R transmission. In the last meeting, separate or joint indication of FEC and number of repetitions was discussed. Separate indication is preferred.

Proposal 27: For the information for D2R transmission carried by PRDCH, some of the indicated information is one-shot valid while some of them is long-term valid, at least the chip duration indicated by a PRDCH can be used for multiple D2R transmission.

Proposal 27: Support separate indication of FEC and number of repetitions.

For R2D transmission, the clock-acquisition part of R2D preamble can be used to determine the R2D chip duration. A straightforward way is that PRDCH chip duration uses the same as the CAP immediately preceding the PRDCH.

Proposal 26: For R2D transmission, PRDCH chip duration uses the same as the CAP immediately preceding the PRDCH.

Conclusion

In this contribution, we discuss random access, scheduling and timing relationships for Ambient IoT. We have the following observations.

Observation 1: If TDMA is used for Msg1 is applied, option 1 (i.e., the frequency domain resource for Msg3 reuses the frequency domain resource for Msg1 for the same device) cannot be used directly.

It is proposed that

Proposal 1: At least for CBRA, for multiple Msg1 transmissions in response to a R2D transmission triggering random access, support only the same data rate for the Msg1 transmissions.

Proposal 2: Clarify each Msg1 time domain resource should include the resource for D2R preamble.

Proposal 3: For TDMA based MSG1, Toffset1 is explicitly indicated by a R2D signaling.

Proposal 4: For X=2, from device perspective, for determination of the starting time for the second Msg1 time domain resource:

- Derived based on the starting time of the first Msg1 time domain resource plus Toffset2

- Toffset2 is predefined in the spec or indicated explicitly by the R2D transmission triggering random access.

Proposal 5: Consider indicating the combination of {X, Toffset1, Toffset2} or {X, Toffset1} by a R2D signaling to determine MSG1 time domain resources.

Proposal 6: For the MSG1 retransmission, the same information of MSG1 resource set for initial transmission can be used.

Proposal 7: For option 1 and TDMA with X>1 or FDMA based MSG1 transmission, a common time duration (i.e., Msg2 monitoring window) can be defined for MSG2 resources for different devices.

Proposal 8: For Option 1, consider how the device to determine the MSG2 resource location within the common time duration to further saving the device’s power consumption.

Proposal 9: Consider how the information carried by MSG2 to confirm whether the random access for a device is successful:

- Random ID as similar as in RFID and NR

- ACK/NACK feedback

Proposal 10: For Option 2, consider how the device to determine its own location within a MSG2.

Proposal 11: Further study the retransmission resource of MSG2.

Proposal 12: Support only the same data rate for MSG3. The same chip rate is used for MSG1 and MSG3.

Proposal 13: Support option 1 (i.e., the frequency domain resource for Msg3 reuses the frequency domain resource for Msg1 for the same device) at least for FDMA based Msg1 transmission.

Proposal 14: The resource size of MSG3 can be indicated by MSG2 and the frequency location of MSG3 can be reused that of MSG1.

- References

RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”

Draft Report of 3GPP TSG RAN WG1 #120bis v0.1.0

Draft Report of 3GPP TSG RAN WG1 #120 v0.3.0