3GPP TSG RAN WG1 #121			 R1-2504945

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:	9.7.2

Source:	Moderator (Xiaomi)

Title:	Summary #6 on ISAC channel modelling

Document for:	Discussion/Decision

Introduction

In RAN #105, the updated SID on channel modelling for Integrated Sensing and Communication (ISAC) was agreed.

The focus of the study is to define channel modelling aspects to support object detection and/or tracking (as per the SA1 meaning in TS 22.137). The study should aim at a common modelling framework capable of detecting and/or tracking the following example objects and to enable them to be distinguished from unintended objects: - UAVs - Humans indoors and outdoors - Automotive vehicles (at least outdoors) - Automated guided vehicles (e.g. in indoor factories) - Objects creating hazards on roads/railways, with a minimum size dependent on frequency All six sensing modes should be considered (i.e. TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, UE monostatic). Frequencies from 0.5 to 52.6 GHz are the primary focus, with the assumption that the modelling approach should scale to 100 GHz. (If significant problems are identified with scaling above 52.6 GHz, the range above 52.6 GHz can be deprioritized.) For the above use cases, sensing modes and frequencies: - Identify details of the deployment scenarios corresponding to the above use cases. - Define channel modelling details for sensing using 38.901 as a starting point, and taking into account relevant measurements, including: - modelling of sensing targets and background environment, including, for example (if needed by the above use cases), radar cross-section (RCS), mobility and clutter/scattering patterns; - spatial consistency.

This document summarizes the contributions and discussions on ISAC channel modelling in RAN1 #121 meeting.

- We target resolving all necessary issues for ISAC channel model. To proceed the discussion, the proposals in this document are tagged and color coded respectively

  - [H] Proposal: High Priority, to be handled firstly.

  - [M] Proposal: We will also try to handle all such proposals.

  - [L] Proposal: We may treat it if there is a consensus.

- The new/revised proposals in current round for discussion is further tagged with [FL5]. Note: Some proposals without update are still marked with early round number. If any company didn’t provide a comment yet, or prefer to update the comments, feel free to continue comment on them.

The following email thread is assigned for discussion of the study item (agenda 9.7):

[121-R19-ISAC] Email discussion on Rel-19 ISAC channel model – Yingyang (xiaomi)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

Proposed online proposals

Tuesday (5/20)

After Tuesday online session

Agreement

Confirm the following working assumption with updates in red.

Working assumption

For vehicle with single/multiple scattering points, the bistatic RCS is generated by

- The values/pattern of A*B1 of bistatic RCS is given by:

rcsdBθi,ϕi,θs,ϕs=maxGmax-min-σVdBθ+σHdB φ,σmax-Attenuatefactor,  Gmax-σmax,RCSdB,fsθi,ϕi,θs,ϕs

where

σVdBθ=-min12θ-θcenterθ3dB2, σmax

σHdB φ=-min12φ-φcenterφ3dB2, σmax

  - Attenuatefactor=k1×sin(k2×β2)-5log10cosβ2 is applied to the  β within 0~180 degrees. k1= 6 and k2=1.65. β is the bistatic angle between the incident ray and scattering ray within the plane of incident direction (θi,ϕi,) and scattering direction (θs,ϕs).

  - The angles of (θ,φ) are the projections of the bisector angle on the vertical plane and the horizontal plane, respectively.

    - FFS: RCS value when β is 180 degrees

  - The effect of forward scattering RCSdB,fsθi,ϕi,θs,ϕs is -Inf in Rel-19

  - 5 sets of parameters φcenter, φ3dB, n, θcenter, θ3dB,n, Gmax, σmax, Applicable Range of θ and Applicable Range of φ are applicable as defined for the monostatic RCS of vehicle with single/multiple SPSTs

  - Continue study on a new formula for Attenuatefactor to resolve the issue of angular discontinuity.

    - The new formula should retain following property: the linear bistatic RCS for a vehicle with single scattering point is the sum of the bistatic RCS of the multiple scattering points of the vehicle

    - the following formula can be a reference for the study

Attenuatefactor=k1sink2β2(1-k3β-k4*(Gmax-min-σVdBθ+σHdB φ,σmax))

Agreement

The agreement on bistatic RCS for vehicle is reused for large size UAV and AGV.

  - For large size UAV, k₁=6.05 and k₂=1.33

  - For AGV, k₁=12 and k₂=1.45

Agreement

AGV can be modelled with multiple scattering points.

- The values/pattern of component A*B1 are generated by the following parameters

|  | ϕcenter in [°] | ϕ3dB, n in [°] | θcenter in [°] | θ3dB,n in [°] | Gmax | σmax | Range of θ in [°] | Range of ϕ in [°] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Front | 0° | 13.68° | 90° | 13.68° | 13.00 | 30.26 | [0,180] | [0,360] |
| Left | 90° | 15.53° | 75° | 20.03° | 7.27 | 24.53 | [0,180] | [0,360] |
| Back | 180° | 12.49° | 90° | 11.89° | 10.98 | 28.24 | [0,180] | [0,360] |
| Right | 270° | 15.53° | 75° | 20.03° | 7.27 | 24.53 | [0,180] | [0,360] |
| Roof | / | / | 0° | 11.44° | 11.77 | 29.03 | [0,180] | [0,360] |

  - Note: For the scattering point associated with roof of the AGV, σHdBϕ=0.

  - Note: the measurements from companies are done by AGV option 1.

Agreement

- The bistatic RCS of UAV with small size is modelled as

  - The values/pattern of A*B1 is given by

rcsdBθi,ϕi,θs,ϕs=maxσRCS,A-Attenuatefactor,RCSdB,fsθi,ϕi,θs,ϕs

    - Component A, i.e., σRCS,A: same as component A of mono-static RCS for UAV of small size

    - Attenuatefactor=3sin(β2) dB, where β is the bi-static angle between incident ray and scattered ray, β is within 0 and 180 degree

    - The effect of forward scattering RCSdB,fsθi,ϕi,θs,ϕs is -Inf in Rel-19

  - Component B2: same as component B2 of mono-static RCS for UAV of small size

- The bistatic RCS of Human with RCS model 1 is modelled as

  - The values/pattern of A*B1 is given by

rcsdBθi,ϕi,θs,ϕs=maxσRCS,A-Attenuatefactor,RCSdB,fsθi,ϕi,θs,ϕs

    - Component A, i.e., σRCS,A: same as component A of mono-static RCS for Human with RCS model 1

    - Attenuatefactor=3sin(β2) dB, where β is the bi-static angle between incident ray and scattered ray, β is within 0 and 180 degree

    - The effect of forward scattering RCSdB,fsθi,ϕi,θs,ϕs is -Inf in Rel-19

  - Component B2: same as component B2 of mono-static RCS for Human with RCS model 1

Agreement

On the monostatic RCS of human with RCS model 2,

- The values/pattern of component A*B1 are generated by the following parameters

|  | ϕcenter in [°] | ϕ3dB, n in [°] | θcenter in [°] | θ3dB,n in [°] | Gmax | σmax | Range of θ in [°] | Range of ϕ in [°] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Front | 0 | 216.65 | 90 | 55.7 | 2.14 | 7.7 | [0,180] | [-90, 90] |
| Back | 180 | 216.65 | 90 | 55.7 | 2.14 | 7.7 | [0,180] | [90,270] |

- The standard deviation of component B2 is 3.94 dB

Agreement

The agreement on bistatic RCS for vehicle with single scattering point is reused to model bistatic RCS of human with RCS model 2

  - k1=0.5714 and k2=0.1

Agreement

The following values of the RCS component A are applied to both monostatic and bistatic RCS of the target.

  - UAV with large size: -5.85 dBsm

  - Human with RCS model 2: -1.37 dBsm

    - Note: measurement is based on adult

  - Vehicle: 11.25 dBsm

    - Note: measurement is based on vehicle type 1 and 2

  - AGV: -4.25 dBsm

    - Note: measurement is based on AGV option 1

Note: component A on its own may not fully reflect the RCS in the target channel. This note will not be captured in the TR.

Agreement

The mean and standard deviation values of XPR of sensing target AGV for monostatic sensing and bistatic sensing are (9.60, 6.85) dB.

Conclusion

The component B2 of two different targets are generated independently.

Conclusion

The component XPR/initial random phase of two different targets are generated independently.

Agreement

In order to generate Tx-target link, target-Rx link and the background channel between a RSU-type UE and another node (TRP, pedestrian UE, vehicle UE, RSU-type UE), the following reference TRs are adopted

| Case | Tx/Rx | Rx/Tx | Existing TRs as starting point |
| --- | --- | --- | --- |
|  | TRP | RSU-type UE | Highway and Urban grid - B2R link in section 6 of TR 37.885 |
|  | RSU-type UE | normal UE | Highway and Urban grid - V2V link in section 6 of TR 37.885, with antenna height at RSU is 5m |
|  | RSU-type UE | RSU-type UE | Highway and Urban grid - V2V link in section 6 of TR 37.885, with antenna height at RSU is 5m |
|  | RSU-type UE | vehicle UE | Highway and Urban grid - V2V link in section 6 of TR 37.885, with antenna height at RSU is 5m |

Agreement

The initial random phase (generated in Step 10, section 7.5, TR38.901) is the same for the same ray in Tx-target link and target-Rx link of a target for monostatic sensing.

Agreement

For UMi-AV and RMa-AV with aerial UE as sensing transmitter or receiver, the values of parameters to generate background channel for UT monostatic sensing are provided in the following table

| Scenario |  | UT monostatic sensing |  |
| --- | --- | --- | --- |
|  |  | UMi-AV | RMa-AV |
| Distribution of 2D distance between Tx and reference points | α1 | 0.0156h+5.5399 | 4.423+0.001926h |
|  | β1 | 40.4517/(h+254.6318) | 13.8467+0.6547h |
|  | c1 | 0.0140h+15.1184 | 3.864+0.1538h |
| Distribution of height of reference points | α2 | 0.0123h+11.9569 | 1.4231+0.00192h |
|  | β2 | 17.8047/(h-0.2202) | 11.7157-0.00538h |
|  | c2 | 0.0532h-0.0120 | 2.6541-0.003851h |

Note 1: Distributions of height and distance of reference point are not subject to geographical constraints on TRP for the corresponding deployment scenario.

Note 2: The reference points for generating the UT monostatic background channel have the same velocity as UT.

Note 3: In the UT monostatic sensing in UMa and UMi scenario, the ZOD offset in the background channel should be set as 0

Agreement

To generate the background channel for TRP monostatic sensing and UT monostatic sensing, ‘maxd3D-c12+(c2-height_Tx)2c,0 +Δτ’ is used to model the absolute delay between the Tx and each reference point.

Agreement

Power threshold for path dropping after concatenation is up to -40dB for target channel for option 3. Up to company to choose a value in the implementation.

Power threshold for path dropping after concatenation is up to -25dB for target channel for option 0. Up to company to choose a value in the implementation.

For calibrations for both option 0 and option 3, power threshold for path dropping after concatenation is -40dB for target channel.

Agreement

To generate the absolute delay model for sensing scenarios Urban grid, highway and HST, for both target channel and background channel

- For Urban grid, the values of parameters for Δτ of scenarios UMa are reused.

- For Highway, the values of parameters for Δτ of scenarios RMa and UMa are reused for FR1 and FR2 respectively.

- For HST, the values of parameters for Δτ of scenarios RMa and UMa are reused for FR1 and FR2 respectively.

Note: no measurements on Δτ of the 3 scenarios are submitted in Rel-19.

Agreement

Spatial consistency is not modelled for

- the links that are generated referring to channel models with parameter values of different communication scenarios

  - E.g., between TRP-target/UT link in one scenario and target/UT-UT link in another scenario

- the background channels for TRP monostatic sensing of different TRPs

Agreement

Spatial consistency is not modelled between TRP-target/UT link and target/UT-UT link for sensing scenario UMi, InH and InF.

Agreement

Spatial consistency is not modelled between TRP-TRP link and any other links for ISAC channel.

Agreement

Spatial consistency can be enabled for multiple scattering points of a target.

Spatial consistency, if enabled, for the links between BS/UT and multiple scattering points of a target are modelled as if multiple scattering points are multiple targets.

Agreement

The existing horizontal correlation distance in Table 7.6.3.1-2 in TR38.901 is used as the correlation distance for 3D spatial consistency for ISAC channel at least for UAV scenario, within same ‘Applicability range in terms of aerial UE height (defined in 36.777)’.

[FL1][M] Proposal 6.3-1-rev1

The large scale parameter of the ray specular reflected by an EO type 2 in the STX-SPST link or SPST-SRX link, is calculated by PEO2=PLLOS+10log10dEO2d3D, where

  - PLLOS is the pathloss assuming a LOS condition for the STX-SPST link or SPST-SRX link

  - dEO2 is the total propagation distance due to EO type-2

  - Note: impact of polarization matrix of EO type-2 is not included in the above

Agreement

EO type-2 can be modelled in NLOS condition.

Agreement

In sensing scenario UMi, UMa, if the height of a scattering point of target is less than 1.5m, for pathloss calculation,

  - use hUT 1.5 m for breakpoint distance (dBP) calculation

  - Note: hUT 1.5 m is only used for dBP calculation. The exact h_UT of the scattering point is still used to determine all other parameters of ISAC channel, e.g., delay, AOD/ZOD/AOA/ZOA, etc.

Agreement

On background channel modelling,

- Spatial consistency is not supported for TRP monostatic sensing across different TRPs

- Spatial consistency is not supported for UE monostatic sensing across different UEs

- Spatial consistency is not supported across different Reference Points for same TRP for TRP monostatic sensing

- Spatial consistency is not supported across different Reference Points for same UE for UE monostatic sensing

Agreement

- RCS component B2 of different direct/indirect paths of a target in the target channel are generated independently.

- On the RCS component B2 of a direct/indirect path of a target in the target channel, the same value of B2 applies to a path before the value of B2 is updated.

  - Note: whether/how/when to update B2 can be discussed in evaluation phase or up to companies’ choices

Agreement

- XPR of different direct/indirect paths of a target in the target channel are generated independently.

- On the XPR of a direct/indirect path of a target in the target channel, the same value of XPR applies to a path before the value of XPR is updated.

  - Note: whether/how/when to update XPR can be discussed in evaluation phase or up to companies’ choices

Agreement

- Initial random phase of different direct/indirect paths of a target in the target channel are generated independently.

- On the initial random phase of a direct/indirect path of a target in the target channel, the same value of initial random phase applies to a path before the value of initial random phase is updated.

  - Note: whether/how/when to update initial random phase can be discussed in evaluation phase or up to companies’ choices

[FL1][H] Proposal 6.2-1

To determine the LOS condition of the Tx-target link and target-Rx link:

- Option A: If type-2 EO is in the LOS ray of one link, the link is determined as NLOS condition, and otherwise use the LOS probability equation defined in existing TRs to determine the LOS/NLOS condition

Supported by (12): HW, ZTE, Sony, QC, LG, Xiaomi, MTK, Nokia, CMCC, Spreadtrum, OPPO, vivo

- Option B: Use the LOS probability equation to determine the LOS/NLOS condition of one link, and then the impacts of type-2 EO is modelled by a blockage model

Supported by (6): Ericsson, CATT (without blockage model), Lenovo, IDCC, DOCOMO, BUPT

- Option C: Use the LOS probability equation to determine the LOS/NLOS condition of one link,

Supported by (7): CATT, Ericsson, Nokia, Lenovo, DOCOMO, IDCC,

Note: If EO type-2 is agreed to be modelled in background channel, the agreed option is extended to LOS condition determination for background channel.

Proposal B

To determine the LOS condition of the Tx-target link and target-Rx link:

- Option A: If type-2 EO is in the LOS ray of one link, the link is determined as NLOS condition with probability p1, and otherwise use the LOS probability equation defined in existing TRs to determine the LOS/NLOS condition

Proposal C

To determine the LOS condition of the Tx-target link and target-Rx link:

- Option C: Use the LOS probability equation to determine the LOS/NLOS condition of one link,

Proposal

EO type-2 is not modelled in background channel.

Wednesday (5/21)

After Wednesday online session

Agreement

The follow TP is used generate the power (except for the impact of polarization matrix of EO type-2) of the ray specular reflected by an EO type 2 in the STX-SPST link or SPST-SRX link.

- 7.9.5.2 Type-2 environment object < Unchanged text omitted > - In Step 10 in Clause 7.9.4.1, Prx,0,m'k,p,Ptx,0,mk,p for a NLOS ray specularly reflected by a type-2 EO, if present, in the SPST-SRX link and the STX-SPST link is determined as follows. - If the STX-SPST link is in LOS condition, Ptx,0,mk,p=dtx,3Dk,pdtx,EO,mk,p2 - If the STX-SPST link is not in LOS condition, Ptx,0,mk,p=10-PLtx,k,pLOS+PLtx,k,p/10∙dtx,3Dk,pdtx,EO,mk,p2, where PLtx,k,pLOS is the pathloss of STX-SPST link assuming LOS condition. - If the SPST-SRX link is in LOS condition, Prx,0,m'k,p=drx,3Dk,pdrx,EO,m'k,p2 - If the SPST-SRX link is not in LOS condition, Prx,0,m'k,p=10-PLrx,k,pLOS+PLrx,k,p/10∙drx,3Dk,pdrx,EO,m'k,p2, where PLrx,k,pLOS is the pathloss of SPST-SRX link assuming LOS condition. < Unchanged text omitted >

Agreement

To generate the absolute delay model for sensing scenarios UMi-AV, UMa-AV and RMa-AV, for both target channel and background channel,

- For the TRP-TRP link and TRP- terrestrial UE link, the values of parameters for Δτ of scenarios UMi, UMa and RMa are respectively reused.

- For the terrestrial UE- terrestrial UE link, the values of parameters for Δτ of scenarios UMi are reused.

- For the TRP- aerial UE link, the values of parameters for Δτ of scenarios UMi, UMa and RMa are respectively reused.

- For the terrestrial UE- aerial UE link, the values of parameters for Δτ of scenarios UMi are reused.

- For the aerial UE- aerial UE link, the values of parameters for Δτ of scenarios UMi are reused.

Note: no measurements on Δτ of the scenarios UMi-AV, UMa-AV and RMa-AV are submitted in Rel-19.

Agreement

- Remove the brackets for first sub-bullet under Step 4 for Clause 7.9.4.2 in the CR to TR 38.901.

- On the absolute delay of the background channel for both TRP and UE monostatic sensing, three ∆τ are independently generated and respectively applied to the 3 channels between the STX/SRX and the 3 RPs.

Agreement

To generate the channel between an aerial UE and a normal UE,

- The LOS probability is generated by:

| Low-UAV | Mid-UAV | High-UAV |
| --- | --- | --- |
| UMi in Table 7.4.2-1 in TR 38.901 for UMi-AV/UMa-AV/RMa-AV | UMi-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region | UMi-AV in Table B-1 in TR 36.777 for BS to [high] UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region |

- The pathloss and shadow fading are generated using TRP-aerial UE link of UMi-AV in Annex A and B of TR 36.777 by setting hBS =1.5m for FR1

Note:

- The height ranges of low-UAV, Mid-UAV and High-UAV are defined following the applicability range in terms of aerial UE height in Table B-1: LOS probability in TR 36.777

- The second height range for UMi-AV is further divided into 2 regions, i.e., [22.5, 100] and [100, 300] for mid-UAV and high-UAV, respectively.

Conclusion

No further study on power normalization of target channel and background channel of ISAC channel in Rel-19

  - Note: sub-section “7.9.5.3 Power normalization across target channel and background channel” in the TR remains as a placeholder with the following text.

    - To combine the target channel and the background channel, an alternative scheme of power normalization can be applied to keep the same/similar channel power as the background channel without sensing target.

Agreement

The polarization matrix CPMsp,i of a direct/indirect path i of a scattering point of a target is defined in LCS.

Agreement

To generate the channel between a first aerial UE with height h1 and a second aerial UE with height h2, abs(h1-hBS) <= abs(h2-hBS),

- The LOS probability between the two aerial UEs is generated by:

|  | Low-UAV | Mid-UAV | High-UAV |
| --- | --- | --- | --- |
| Low-UAV | UMi in Table 7.4.2-1 in TR 38.901 for UMi-AV/UMa-AV/RMa-AV | UMi-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region | UMi-AV in Table B-1 in TR 36.777 for BS to [high] UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region |
| Mid-UAV | UMi-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region | UMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region | 1 |
| High-UAV | UMi-AV in Table B-1 in TR 36.777 for BS to [high] UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region | 1 | 1 |

- The pathloss and shadow fading between two aerial UEs are generated using TRP-aerial UE link of UMi-AV in Annex A and B of TR 36.777 by setting height of TRP equal to the height of the first aerial UE.

Note:

- The height ranges of low-UAV, Mid-UAV and High-UAV are defined following the applicability range in terms of aerial UE height in Table B-1: LOS probability in TR 36.777

- The second height range for UMi-AV is further divided into 2 regions, i.e., [22.5, 100] and [100, 300] for mid-UAV and high-UAV, respectively.

[FL2][H] Proposal 6.1-1

EO type-2 can be modelled in background channel if modelled in target channel

[FL2][H] Proposal 6.2-1

To determine the LOS condition of the Tx-target link and target-Rx link,

- Option A: If type-2 EO is in the LOS ray of one link, the link is determined as NLOS condition with a probability p, and otherwise use the LOS probability equation defined in existing TRs to determine the LOS/NLOS condition

  - p equals to the percentage of the area of vehicle that is blocked by the building

Thursday (5/22)

After Thursday online session

Agreement

Update the agreements on LOS probability calculation for channel between an aerial UE and a normal UE as follows.

| Low-UAV | Mid-UAV | High-UAV |
| --- | --- | --- |
| UMi in Table 7.4.2-1 in TR 38.901 for UMi-AV/UMa-AV/RMa-AV | UMi-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region for RMa-AV | UMi-AV in Table B-1 in TR 36.777 for BS to high UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to high UAV region for RMa-AV |

Agreement

Update the agreements on LOS probability calculation for channel between two aerial UE as follows.

|  | Low-UAV | Mid-UAV | High-UAV |
| --- | --- | --- | --- |
| Low-UAV | UMi in Table 7.4.2-1 in TR 38.901 for UMi-AV/UMa-AV/RMa-AV | UMi-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region for RMa-AV | UMi-AV in Table B-1 in TR 36.777 for BS to high UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to high UAV region for RMa-AV |
| Mid-UAV | UMi-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region for RMa-AV | UMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region | 1 |
| High-UAV | UMi-AV in Table B-1 in TR 36.777 for BS to high UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to high UAV region for RMa-AV | 1 | 1 |

Agreement

To determine the LOS condition of any link in ISAC channel model, when EO type-2 is modelled, the following two options are agreed as solutions:

- Option A: If type-2 EO is in the LOS ray of the link, the LOS probability is p, p=0, and otherwise use the LOS probability equation defined in existing TRs to determine the LOS/NLOS condition

- Option C: Use the LOS probability equation to determine the LOS/NLOS condition of the link.

Note1: in which conditions/scenarios to use option A or option C can be determined in future evaluations.

Note2: as already agreed, monostatic background channel is always NLOS

Agreement

EO type-2 can be optionally modelled in background channel when EO type-2 is modelled in target channel.

[FL4][M] Proposal 8-4-rev5

Down select one from the following 2 options in RAN1 #121

Option 3: LLS channel model is not supported in Rel-19.

Option 4:

- add a general sentence as follows in subsection 7.9.6 on LLS channel model for ISAC

  - The target channel of link-level channel for ISAC can be generated by adding and/or reusing one or more cluster(s) representing target(s) in the existing LLS channel model.

- No further study on LLS channel model for ISAC in Rel-19.

Supported: ZTE, Huawei, SKT, Apple, Xiaomi, QC, CATT, CMCC, LGE, MTK, NIST, DOCOMO, Sony

Conclusion

Delete subsection 7.9.6 from the draft CR. For ISAC, no enhancement to LLS channel model is introduced in Rel-19.

[FL4][M] Proposal 4.8-3

RCS of EO type-1 is not further studied in Rel-19.

Conclusion

Other than RCS for human, vehicle, AGV, UAV, no other RCS for other objects is introduced in Rel-19.

- Future studies are not precluded for adding RCS of other objects/sizes for modelling target or EO type-1, based on validation results from companies.

Friday (5/23)

After Friday online session

Conclusion

Delete subsection 7.9.6 from the draft CR. For ISAC, no enhancement to existing TR38.901 LLS channel model is introduced in Rel-19.

Agreement

- The existing blockage model A/B procedures can be reused to model the blocking effect due to a target as an optional feature

  - Applicable to the LOS/NLOS rays in the background channel of the target

  - Applicable to the LOS/NLOS rays in the Tx-target and target-Rx link of another target

- The location, orientation and size of the target as a blocker is known before applying the blockage model A/B.

Agreement

- The square brackets on formula 7.9.5-10 (copied below) in the draft CR are removed

The effective polarization matrix of the type-2 EO reflection path is given by

[CPMEO=cosγ2sinγ2-sinγ2cosγ2R∥00-R⊥cosγ1-sinγ1sinγ1cosγ1	(7.9.5-10)]

- Encourage companies to check and compare with the results that can be obtained with Alt2 and Alt3 below. If problem is found, RAN1 will revise TR 38.901 by new CR.

Alt2

[CPMEO=cosγ2-sinγ2sinγ2cosγ2R∥00-R⊥cosγ1sinγ1-sinγ1cosγ1	(7.9.5-10)]

With reusing the legacy transformation method for deriving γ1 and γ2.

Alt3

[CPMEO=cosγ2-sinγ2sinγ2cosγ2R∥00-R⊥cosγ1sinγ1-sinγ1cosγ1	(7.9.5-10)]

Where,

-	γ1=atan2-eθEO, ZODnplane,eϕEO, AODnplane. nplane represents the normal vector of the incident plane. nplane=vtx→w×vw→rx, in which vtx→w=xw-xtx,yw-ytx,zw-ztx and vw→rx=xrx-xw,yrx-yw,zrx-zw. eθEO, ZOD represents the spherical basis vector of incident ray in vertical direction. eθEO,ZOD=cosθEO, ZODcosϕEO, AOD,cosθEO, ZODsinϕEO, AOD,-sinθEO, ZODT. eϕEO,AOD represents the spherical basis vector of incident ray in horizontal direction. eϕEO,AOD=-sinφEO, AOD,cosφEO, AOD,0T.

-	γ2=atan2-eθEO, ZOAnplane,eϕEO, AOAnplane. eθEO, ZOA represents the polar basis vector of scattering ray in vertical direction. eϕEO, AOA represents the polar basis vector of scattering ray in horizontal direction. eθEO, ZOA=cosθEO, ZOAcosϕEO, AOA,cosθEO, ZOAsinϕEO, AOA,-sinθEO, ZOAT. eϕEO, AOA=-sinϕEO, AOA,cosϕEO, AOA,0T.

Conclusion

There is no consensus to introduce an exact formula for micro-Doppler in Rel-19. The placeholder in the channel impulse response is kept in the draft CR.

Proposed offline proposals

Monday (5/19)

After Monday offline session

[FL1][H] Proposal 4.0-1

Confirm the following working assumption with updates in red.

Working assumption

For vehicle with single/multiple scattering points, the bistatic RCS is generated by

- The values/pattern of A*B1 of bistatic RCS is given by:

rcsdBθi,ϕi,θs,ϕs=maxGmax-min-σVdBθ+σHdB φ,σmax-Attenuatefactor,  Gmax-σmax,RCSdB,fsθi,ϕi,θs,ϕs

where

σVdBθ=-min12θ-θcenterθ3dB2, σmax

σHdB φ=-min12φ-φcenterφ3dB2, σmax

  - Attenuatefactor=k1×sin(k2×β2)-5log10cosβ2 is applied to the  β within 0~180 degrees. k1= 6 and k2=1.65. β is the bistatic angle between the incident ray and scattering ray within the plane of incident direction (θi,ϕi,) and scattering direction (θs,ϕs).

  - The angles of (θ,φ) are the projections of the bisector angle on the vertical plane and the horizontal plane, respectively.

    - FFS: RCS value when β is 180 degrees

  - The effect of forward scattering RCSdB,fsθi,ϕi,θs,ϕs is -Inf in Rel-19

  - 5 sets of parameters φcenter, φ3dB, n, θcenter, θ3dB,n, Gmax, σmax, Applicable Range of θ and Applicable Range of φ are applicable as defined for the monostatic RCS of vehicle with single/multiple SPSTs

  - Continue study on a new formula for Attenuatefactor to resolve the issue of angular discontinuity.

    - The new formula should retain following property: the linear bistatic RCS for a vehicle with single scattering point is the sum of the bistatic RCS of the multiple scattering points of the vehicle

    - the following formula can be a reference for the study

Attenuatefactor=k1sink2β2(1-k3β-k4*(Gmax-min-σVdBθ+σHdB φ,σmax))

[FL1][H] Proposal 4.1-1-rev3

- The WA on bistatic RCS for vehicle is reused for large size UAV and AGV.

  - For large size UAV, k₁=6.05 and k₂=1.33

  - For AGV, k₁=12 and k₂=1.45

[FL1][M] Proposal 4.1-2-rev1

AGV can be modelled with multiple scattering points.

- The values/pattern of component A*B1 are generated by the following parameters

|  | ϕcenter in [°] | ϕ3dB, n in [°] | θcenter in [°] | θ3dB,n in [°] | Gmax | σmax | Range of θ in [°] | Range of ϕ in [°] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Front | 0° | 13.68° | 90° | 13.68° | 13.00 | 30.26 | [0,180] | [0,360] |
| Left | 90° | 15.53° | 75° | 20.03° | 7.27 | 24.53 | [0,180] | [0,360] |
| Back | 180° | 12.49° | 90° | 11.89° | 10.98 | 28.24 | [0,180] | [0,360] |
| Right | 270° | 15.53° | 75° | 20.03° | 7.27 | 24.53 | [0,180] | [0,360] |
| Roof | / | / | 0° | 11.44° | 11.77 | 29.03 | [0,180] | [0,360] |

  - Note: For the scattering point associated with roof of the AGV, σHdBϕ=0.

  - Note: the measurements from companies are done by AGV option 1.

[FL1][H] Proposal 4.2-1

- The bistatic RCS of UAV with small size is modelled as

  - The values/pattern of A*B1 is given by

rcsdBθi,ϕi,θs,ϕs=maxσRCS,A-Attenuatefactor,RCSdB,fsθi,ϕi,θs,ϕs

    - Component A, i.e., σRCS,A: same as component A of mono-static RCS for UAV of small size

    - Attenuatefactor=3sin(β2) dB, where β is the bi-static angle between incident ray and scattered ray, β is within 0 and 180 degree

    - The effect of forward scattering RCSdB,fsθi,ϕi,θs,ϕs is -Inf in Rel-19

  - Component B2: same as component B2 of mono-static RCS for UAV of small size

- The bistatic RCS of Human with RCS model 1 is modelled as

  - The values/pattern of A*B1 is given by

rcsdBθi,ϕi,θs,ϕs=maxσRCS,A-Attenuatefactor,RCSdB,fsθi,ϕi,θs,ϕs

    - Component A, i.e., σRCS,A: same as component A of mono-static RCS for Human with RCS model 1

    - Attenuatefactor=3sin(β2) dB, where β is the bi-static angle between incident ray and scattered ray, β is within 0 and 180 degree

    - The effect of forward scattering RCSdB,fsθi,ϕi,θs,ϕs is -Inf in Rel-19

  - Component B2: same as component B2 of mono-static RCS for Human with RCS model 1

[FL1][M] Proposal 4.3-1

On the monostatic RCS of human with RCS model 2,

- The values/pattern of component A*B1 are generated by the following parameters

|  | ϕcenter in [°] | ϕ3dB, n in [°] | θcenter in [°] | θ3dB,n in [°] | Gmax | σmax | Range of θ in [°] | Range of ϕ in [°] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Front | 0 | 216.65 | 90 | 55.7 | 2.14 | 7.7 | [0,180] | [-90, 90] |
| Back | 180 | 216.65 | 90 | 55.7 | 2.14 | 7.7 | [0,180] | [90,270] |

- The standard deviation of component B2 is 3.94 dB

[FL1][M] Proposal 4.3-2

- The working assumption on bistatic RCS for vehicle with single scattering point is reused to model bistatic RCS of human with RCS model 2

  - k1=0.5714 and k2=0.1

[FL1] [H] Proposal 4.4-1-rev3

- The following values of the RCS component A is the mean of monostatic RCS values in all 3D incident/scattered angles of a target, and is are applied to both monostatic and bistatic RCS of the target.

  - UAV with large size: -5.85 dBsm

  - Human with RCS model 2: -1.37 dBsm

    - Note: measurement is based on adult

  - Vehicle: 11.25 dBsm

    - Note: measurement is based on vehicle type 1 and 2

  - AGV: -4.25 dBsm

    - Note: measurement is based on AGV option 1

[FL1][H] Proposal 4.5-1

The mean and standard deviation values of XPR of sensing target AGV for monostatic sensing and bistatic sensing are (9.60, 6.85) dB.

[FL1][M] Proposal 4.6-1 for conclusion

The component B2 of two different targets are generated independently.

[FL1][M] Proposal 4.7-1 for conclusion

The component XPR/initial random phase of two different targets are generated independently.

[FL1][H] Proposal 5.1-3

- In order to generate Tx-target link, target-Rx link and the background channel between a RSU-type UE and another node (TRP, pedestrian UE, vehicle UE, RSU-type UE), the following reference TRs are adopted

| Case | Tx/Rx | Rx/Tx | Existing TRs as starting point |
| --- | --- | --- | --- |
|  | TRP | RSU-type UE | Highway and Urban grid - B2R link in section 6 of TR 37.885 |
|  | RSU-type UE | normal UE | Highway and Urban grid - V2V link in section 6 of TR 37.885, with antenna height at RSU is 5m |
|  | RSU-type UE | RSU-type UE | Highway and Urban grid - V2V link in section 6 of TR 37.885, with antenna height at RSU is 5m |
|  | RSU-type UE | vehicle UE | Highway and Urban grid - V2V link in section 6 of TR 37.885, with antenna height at RSU is 5m |

[FL1][H] Proposal 5.2-1

- The initial random phase (generated in Step 10, section 7.5, TR38.901) is the same for the same ray in Tx-target link and target-Rx link of a target for monostatic sensing.

[FL1][H] Proposal 5.2-2

For UMi-AV and RMa-AV with aerial UE as sensing transmitter or receiver, the values of parameters to generate background channel for UT monostatic sensing are provided in the following table

| Scenario |  | UT monostatic sensing |  |
| --- | --- | --- | --- |
|  |  | UMi-AV | RMa-AV |
| Distribution of 2D distance between Tx and reference points | α1 | 0.0156h+5.5399 | 4.423+0.001926h |
|  | β1 | 40.4517/(h+254.6318) | 13.8467+0.6547h |
|  | c1 | 0.0140h+15.1184 | 3.864+0.1538h |
| Distribution of height of reference points | α2 | 0.0123h+11.9569 | 1.4231+0.00192h |
|  | β2 | 17.8047/(h-0.2202) | 11.7157-0.00538h |
|  | c2 | 0.0532h-0.0120 | 2.6541-0.003851h |

Note 1: Distributions of height and distance of reference point are not subject to geographical constraints on TRP for the corresponding deployment scenario.

Note 2: The reference points for generating the UT monostatic background channel have the same velocity as UT.

Note 3: In the UT monostatic sensing in UMa and UMi scenario, the ZOD offset should be set as 0

[FL1][H] Proposal 5.2-3-rev2

- To generate the background channel for TRP monostatic sensing and UT monostatic sensing, ‘maxd3D-c12+(c2-height_Tx)2c,0 +Δτ’ is used to model the absolute delay between the Tx and each reference point.

[FL1][M] Proposal 5.3-1-rev1

- Power threshold for path dropping after concatenation is up to -40dB for target channel. Up to company to choose a value in the implementation

[FL1][H] Proposal 5.5-1-rev1

To generate the absolute delay model for sensing scenarios Urban grid, highway and HST, for both target channel and background channel

- For Urban grid, the values of parameters for Δτ of scenarios UMa are reused.

- For Highway, the values of parameters for Δτ of scenarios RMa and UMa are reused for FR1 and FR2 respectively.

- For HST, the values of parameters for Δτ of scenarios RMa and UMa are reused for FR1 and FR2 respectively.

Note: no measurements on Δτ of the 3 scenarios are submitted in Rel-19.

[FL1][M] Proposal 5.7-1-rev1

- Dual mobility model in 7.6.10, TR 38.901 is reused as start point to model Doppler effect ft due to movement of stochastic clusters, i.e., 2αn,mDn,mλ0

  - Necessary changes on maximum speed and ratio of moving scatters can be discussed in evaluation phase.

[FL1][H] Proposal 7.2-1

Spatial consistency is not modelled for

- the links that are generated referring to channel models with parameter values of different communication scenarios

  - E.g., between TRP-target/UT link in one scenario and target/UT-UT link in another scenario

- the background channels for TRP monostatic sensing of different TRPs

Supported by: ZTE, CATT, HW, LGE, vivo, QC, BUPT, Xiaomi, OPPO, Ericsson (bullet 1)

[FL1][H] Proposal 7.2-2

Spatial consistency is not modelled between TRP-target/UT link and target/UT-UT link for sensing scenario UMi, InH and InF

Supported by: CATT, LGE, vivo, QC, BUPT, Xiaomi, DOCOMO, OPPO, Huawei

[FL1][H] Proposal 7.2-3

Spatial consistency is not modelled between TRP-TRP link and any other links for ISAC channel.

[FL1][H] Proposal 7.4-2

- Spatial consistency, if enabled, for the links between BS/UT and multiple scattering points of a target are modelled as if multiple targets.

[FL1] [H] Proposal 7.5-1-rev1

- The existing horizontal correlation distance in Table 7.6.3.1-2 in TR38.901 is used as the correlation distance for 3D spatial consistency for ISAC channel at least for UAV scenario, within same ‘Applicability range in terms of aerial UE height (defined in 36.777)’

[FL1][M] Proposal 6.3-1

- The pathloss of the ray specular reflected by an EO type 2 in the STX-SPST link or SPST-SRX link is calculated by PLEO2=PLLOS+10log10dEO2d3D, where

  - PLLOS is a pathloss assuming a LOS condition for the STX-SPST link or SPST-SRX link

  - dEO2 is the total propagation distance due to EO type-2

[FL1][H] Proposal 5.4-1

- In sensing scenario UMi, UMa, if the height of a scattering point of target is less than 1.5m, for pathloss calculation,

  - use hUT 1.5 m for breakpoint distance (dBP) calculation

  - Note: hUT 1.5 m is only used for dBP calculation. The exact h_UT of the scattering point is still used to determine all other parameters of ISAC channel, e.g., delay, AOD/ZOD/AOA/ZOA, etc.

[FL1][H] Proposal 7.3-1-rev1

- On background channel for UE monostatic sensing, spatial consistency, if enabled, is supported for the following additional parameters.

| Parameters | Correlation type |
| --- | --- |
| Distance between reference point and Tx | All-correlated |
| Height of reference point | All-correlated |
| LOS AOD between Tx and reference point | All-correlated |

[FL1][H] Proposal 7.3-1-rev2 for conclusion

On background channel modelling,

- Spatial consistency is not supported for TRP monostatic sensing across different TRPs

- Spatial consistency is not supported for UE monostatic sensing across different UEs

- Additionally spatial consistency is not supported for different RPs for same UE for UE monostatic sensing

- Additionally spatial consistency is not supported for different RPs for same TRP for TRP monostatic sensing

[FL1][M] Proposal 8-4-rev3

- The link-level channel for ISAC is generated by adding one or more cluster(s) representing target(s) to the existing LLS channel model

  - A new parameter of velocity is added for the cluster(s) of target(s), and if necessary, for at least some of the existing cluster(s) in existing LLS channel model.

  - Values of parameters of the added cluster(s) of target(s), and if necessary, the velocity of at least some of the existing cluster(s), can be discussed in performance evaluation stage

  - Note: the effect of RCS of target is reflected by the power of the cluster(s) of target(s)

Supported by: ZTE, Nokia, Samsung, OPPO, MTK, CMCC,

No LLS channel in Rel-19

Supported by: QC, Ericsson, Lenovo, Sony

Tuesday (5/20)

After Tuesday offline session

[FL2][M] Proposal 6.3-2

The follow TP is used generate the power (except for the impact of polarization matrix of EO type-2) of the ray specular reflected by an EO type 2 in the STX-SPST link or SPST-SRX link.

- 7.9.5.2 Type-2 environment object < Unchanged text omitted > - In Step 10 in Clause 7.9.4.1, Prx,0,m'k,p,Ptx,0,mk,p for a NLOS ray specularly reflected by a type-2 EO, if present, in the SPST-SRX link and the STX-SPST link is determined as follows. - If the STX-SPST link is in LOS condition, Ptx,0,mk,p=dtx,3Dk,pdtx,EO,mk,p2 - If the STX-SPST link is not in LOS condition, Ptx,0,mk,p=10-PLtx,k,pLOS+PLtx,k,p/10∙dtx,3Dk,pdtx,EO,mk,p2, where PLtx,k,pLOS is the pathloss of STX-SPST link assuming LOS condition. - If the SPST-SRX link is in LOS condition, Prx,0,m'k,p=drx,3Dk,pdrx,EO,m'k,p2 - If the SPST-SRX link is not in LOS condition, Prx,0,m'k,p=10-PLrx,k,pLOS+PLrx,k,p/10∙drx,3Dk,pdrx,EO,m'k,p2, where PLrx,k,pLOS is the pathloss of SPST-SRX link assuming LOS condition. < Unchanged text omitted >

[FL2][H] Proposal 5.5-2

To generate the absolute delay model for sensing scenarios UMi-AV, UMa-AV and RMa-AV, for both target channel and background channel,

- For the TRP-TRP link and TRP- terrestrial UE link, the values of parameters for Δτ of scenarios UMi, UMa and RMa are respectively reused.

- For the terrestrial UE- terrestrial UE link, the values of parameters for Δτ of scenarios UMi are reused.

- For the TRP- aerial UE link, the values of parameters for Δτ of scenarios UMi, UMa and RMa are respectively reused.

- For the terrestrial UE- aerial UE link, the values of parameters for Δτ of scenarios UMi are reused.

- For the aerial UE- aerial UE link, the values of parameters for Δτ of scenarios UMi are reused.

Note: no measurements on Δτ of the scenarios UMi-AV, UMa-AV and RMa-AV are submitted in Rel-19.

[FL2][H] Proposal 5.2-4-rev1

- Remove the brackets for first sub-bullet under Step 4 for Clause 7.9.4.2 in the CR to TR 38.901.

- On the absolute delay of the background channel for monostatic sensing, the same three ∆τ are independently generated and respectively applied to the 3 channels between the STX/SRX and the 3 RPs.

[FL2][H] Proposal 5.1-1-rev3

To generate the channel between an aerial UE and a normal UE,

- The LOS probability is generated by:

| Low-UAV | Mid-UAV | High-UAV |
| --- | --- | --- |
| UMi in Table 7.4.2-1 in TR 38.901 for UMi-AV/UMa-AV/RMa-AV | UMi-AV, UMa-AV and RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region | UMi-AV, UMa-AV and RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region |

- The pathloss and shadow fading are generated using TRP-aerial UE link of UMi-AV in Annex A and B of TR 36.777 by setting hBS =1.5m for FR1

Note:

- The height ranges of low-UAV, Mid-UAV and High-UAV are defined following the applicability range in terms of aerial UE height in Table B-1: LOS probability in TR 36.777

- The second height range for UMi-AV is further divided into 2 regions, i.e., [22.5, 100] and [100, 300]

[FL2][H] Proposal 5.1-2-rev2

To generate the channel between a first aerial UE with height h1 and a second aerial UE with height h2, abs(h1-hBS) <= abs(h2-hBS),

- The LOS probability between the two aerial Ues is generated by:

|  | Low-UAV | Mid-UAV | High-UAV |
| --- | --- | --- | --- |
| Low-UAV | UMi in Table 7.4.2-1 in TR 38.901 for UMi-AV/UMa-AV/RMa-AV | UMi-AV, UMa-AV and RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region | UMi-AV, UMa-AV and RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region |
| Mid-UAV | UMi-AV, UMa-AV and RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region | UMi-AV, UMa-AV and RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region | 1 |
| High-UAV | UMi-AV, UMa-AV and RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region | 1 | 1 |

- The pathloss and shadow fading between two aerial Ues are generated using TRP-aerial UE link of UMi-AV in Annex A and B of TR 36.777 by setting height of TRP equal to the height of the first aerial UE.

Note:

- The height ranges of low-UAV, Mid-UAV and High-UAV are defined following the applicability range in terms of aerial UE height in Table B-1: LOS probability in TR 36.777

- The second height range for UMi-AV is further divided into 2 regions, i.e., [22.5, 100] and [100, 300]

[FL2][M] Proposal 5.6-1-rev3

To do power normalization of target channel and background channel of ISAC channel

- A power scaling factor α is applied on both target channel and background channel, such as the following holds:

If  k=0K-110Ptarget,k1010Pbackground10<1 , then

k=0K-110Ptarget,k10+α∙10Pbackground10=10Pbackground10

Otherwise,

α∙k=0K-110Ptarget,k10+10Pbackground10=10Pbackground10

Where,

  - Ptarget,k equals to the power scaling factor of target channel, i.e.,

PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2.

  - Pbackground is the power of the NLOS clusters calculated by the large scale parameters of background channel, i.e., PLdBd+SFdB.

Support: OPPO, QC

[FL2][M] Proposal 5.6-1-rev2

To do power normalization of target channel and background channel of ISAC channel

- A power scaling factor α is applied on both target channel and background channel, subjected to

α∙k=0K-110Ptarget,k10+10Pbackground10=10Pbackground10

Where,

  - Ptarget,k equals to the power scaling factor of target channel, i.e.,

PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2.

  - Pbackground is the nlos power  calculated by the large scale parameters of background channel and the k factor, i.e., PLdBd+SFdB.

Supported:

Alt 3: the target channel of a target will replace one cluster in the background channel

Supported: BUPT (indoor/outdoor scenarios), Panasonic, vivo

[FL2][M] Proposal 5.6-1-rev4

- Power normalization of target channel and background channel of ISAC channel is not supported in Rel-19

  - Note: this means the early agreement on option 2 for power normalization of target channel and background channel is reverted

Supported: vivo, LGE, Apple, Sony, Xiaomi, CATT, Nokia, ZTE, MTK, BUPT,

[FL2][M] Proposal 4.5-2 for conclusion

- The transformation between LCS and GCS is not considered to model the polarization matrix CPMsp,i of a direct/indirect path i of a scattering point of a target

Wednesday (5/21)

After Wednesday offline session

[FL4][H] Proposal 5.1-1-rev5

Update the agreements on LOS probability calculation for channel between an aerial UE and a normal UE as follows.

| Low-UAV | Mid-UAV | High-UAV |
| --- | --- | --- |
| UMi in Table 7.4.2-1 in TR 38.901 for UMi-AV/UMa-AV/RMa-AV | UMi-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region for RMa-AV | UMi-AV in Table B-1 in TR 36.777 for BS to high UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to high UAV region for RMa-AV |

[FL4][H] Proposal 5.1-2-rev4

Update the agreements on LOS probability calculation for channel between two aerial UE as follows.

|  | Low-UAV | Mid-UAV | High-UAV |
| --- | --- | --- | --- |
| Low-UAV | UMi in Table 7.4.2-1 in TR 38.901 for UMi-AV/UMa-AV/RMa-AV | UMi-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region for RMa-AV | UMi-AV in Table B-1 in TR 36.777 for BS to high UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to high UAV region for RMa-AV |
| Mid-UAV | UMi-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region for RMa-AV | UMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region | 1 |
| High-UAV | UMi-AV in Table B-1 in TR 36.777 for BS to high UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to high UAV region for RMa-AV | 1 | 1 |

[FL3][H] Proposal 6.2-1-rev1

Down selection among one from the following 3 options in RAN1 #121

Option 1:

To determine the LOS condition of the Tx-target link and target-Rx link,

- Option A: If type-2 EO is in the LOS ray of one link, the NLOS probability is p, and otherwise use the LOS probability equation defined in existing TRs to determine the LOS/NLOS condition

  - p = 1 for all scenarios

Supported by QC, HW

Option 2:

To determine the LOS condition of the Tx-target link and target-Rx link,

- For urban grid scenario modelling EO type-2: If type-2 EO is in the LOS ray of one link, the link is determined as NLOS condition, and otherwise use the LOS probability equation defined in existing TRs to determine the LOS/NLOS condition

- For other scenarios:  The LOS probability equation defined in existing TRs to determine the LOS/NLOS condition.

  - This can be revisited and updated when necessary in the evaluation phase when the details of EO type-2 deployment is settled.

Supported by HW

Option 3:

To determine the LOS condition of the Tx-target link and target-Rx link, the following two options are agreed as candidate solutions:

- Option A: If type-2 EO is in the LOS ray of one link, the link is determined as NLOS condition, and otherwise use the LOS probability equation defined in existing TRs to determine the LOS/NLOS condition

- Option C: Use the LOS probability equation to determine the LOS/NLOS condition of one link.

Supported by CATT

Option 4:

- Support legacy stochastic LOS/NLOS condition of Type-2 EO in section 7.9.

- Option A, deterministic NLOS of Type-2 EO, in the section of map-based hybrid channel of 38.901.

Supported by Ericsson

[FL3][H] Proposal 6.1-1

- EO type-2 can be modelled in background channel if modelled in target channel

[FL4][M] Proposal 8-4-rev5

Down select one from the following 2 options in RAN1 #121

Option 3: LLS channel model is not supported in Rel-19.

Option 4:

- add a general sentence as follows in subsection 7.9.6 on LLS channel model for ISAC

  - The target channel of link-level channel for ISAC can be generated by adding and/or reusing one or more cluster(s) representing target(s) in the existing LLS channel model.

- No further study on LLS channel model for ISAC in Rel-19.

Supported: ZTE, Huawei, SKT, Apple, Xiaomi, QC, CATT, CMCC, LGE, MTK, NIST, DOCOMO, Sony

[FL4][M] Proposal 4.8-1-rev2

RCS of EO type-1 is not further studied in Rel-19.

[FL4][M] Proposal 4.8-2-rev1

Exact formula for micro-Doppler is not further studied in Rel-19.

Polarization matrix generation for EO type-2

Thursday (5/22)

After Thursday offline session

[FL5][M] Proposal 8-4-rev6 for conclusion

Delete subsection 7.9.6 from the draft CR. For ISAC, no enhancement to existing LLS channel model is introduced in Rel-19.

Proposed conclusion

- There is no consensus that the current CR has problem on polarization matrix generation. The brackets on formula 7.9.5.2-10 are removed

- Encourage more companies to check the draft CR. If problem is found, RAN1 will revised TR 38.901 by CR.

[FL5][M] Proposal 5.8-1-rev3

- The existing blockage model A/B procedures can be reused to model the blocking effect due to a target as an optional feature

  - Applicable to the LOS/NLOS rays in the background channel of the target

  - Applicable to the LOS/NLOS rays in the Tx-target and target-Rx link of another target

- The location, orientation and size of the target as a blocker is known before applying the blockage model A/B.

Physical object model

  - Bistatic RCS for vehicle

Working assumption For vehicle with single/multiple scattering points, the bistatic RCS is generated by - The values/pattern of A*B1 of bistatic RCS is given by: rcsdBθi,ϕi,θs,ϕs=maxGmax-min-σVdBθ+σHdB φ,σmax-Attenuatefactor,RCSdB,fsθi,ϕi,θs,ϕs where σVdBθ=-min12θ-θcenterθ3dB2, σmax σHdB φ=-min12φ-φcenterφ3dB2, σmax - Attenuatefactor=k1×sin(k2×β2) is applied to the β within 0~180 degrees. K1= 6 and k2=1.65. β is the absolute bistatic angle between the incident ray and scattering ray within the plane of incident direction (θi,ϕi,) and scattering direction (θs,ϕs). - The angles of (θ,φ) are the projections of the bisector angle on the vertical plane and the horizontal plane, respectively. - FFS: RCS value when β is 180 degrees - The effect of forward scattering RCSdB,fsθi,ϕi,θs,ϕs is -Inf in Rel-19 - 5 sets of parameters φcenter, φ3dB, n, θcenter, θ3dB,n, Gmax, σmax, Applicable Range of θ and Applicable Range of φ are applicable as defined for the monostatic RCS of vehicle with single/multiple SPSTs - FFS: how to avoid angular discontinuity - Continue study on a new formula for Attenuatefactor to resolve the issue of angular discontinuity. - The new formula should retain following property: the linear bistatic RCS for a vehicle with single scattering point is the sum of the bistatic RCS of the multiple scattering points of the vehicle - the following formula can be a reference for the study Attenuatefactor=k1sink2β2(1-k3β-k4*(Gmax-min-σVdBθ+σHdB φ,σmax)) Note: the working assumption agreed on Thursday was updated on Friday as follows: k1= 6 and k2=1.65

[Moderator’s note] In the WA from RAN1 #120bis, it requires study on new formula satisfying two merits: 1) RCS continuity; 2) the linear bistatic RCS for a vehicle with single scattering point is the sum of the bistatic RCS of the multiple scattering points of the vehicle (second one is already satisfied by the original WA).

According to the offline discussion by proponent companies, the following formula is proposed with both two merits.

RCSdBθi,ϕi,θs,ϕs=maxGmax-min-σVdBθ+σHdB φ,σmax-AF,Gmin,RCSdB,fsθi,ϕi,θs,ϕs

AF=k1×sink2×β2-5log10cosβ2, k1=6, k2=1.65

Gmin=Gmax-σmax

The function of new formula can be explained as follows.

- The linear value for rcsdB=Gmax-min-σVdBθ+σHdB φ,σmax-AF is

rcslinear=10Gmax-min-σVdBθ+σHdB φ,σmax-AF10

=10Gmax-min-σVdBθ+σHdB φ,σmax-k1×sin(k2×β2)10∙cosβ2

10Gmax-min-σVdBθ+σHdB φ,σmax-k1×sin(k2×β2)10 is right the linear RCS value of the original WA. cosβ2 is a scaling factor which has following properties,

  - equals to 1 for β=0°. Therefore, the bistatic RCS with β=0° is still equal to the monostatic RCS value (as the original WA)

  - equals to 0 for β=180°, i.e., rcslinear=0. By this way, no matter how to reach  β=180° (e.g., from the front, the back, or any other 3D angles), rcslinear is always a fixed value of 0.

The curves for scaling factor cosβ2 are shown in the following figures (both linear scale and dB scale), i.e., blue curves. The reason to use cosβ2 instead of cosβ2 is to reduce the impact compared to the original WA. BUPT colleagues already did the validation that cosβ2 provides good match with measurement results.

- The minimum value of bistatic RCS is set the same as monostaic RCS, i.e., Gmin=Gmax-σmax in dB scale.

- Finally, RCS continuity is achieved by taking the maximum operation

maxGmax-min-σVdBθ+σHdB φ,σmax-AF,Gmin

Since a linear value 0 will result in dB value of -inf (always <Gmin ), the bistatic RCS values rcsdB around β=180° are forced to Gmin dB, which guarantees continuity of RCS values around β=180° in all 3D angles.

[FL1][H] Proposal 4.0-1

Confirm the following working assumption with updates in red.

Working assumption

For vehicle with single/multiple scattering points, the bistatic RCS is generated by

- The values/pattern of A*B1 of bistatic RCS is given by:

rcsdBθi,ϕi,θs,ϕs=maxGmax-min-σVdBθ+σHdB φ,σmax-Attenuatefactor,  Gmax-σmax,RCSdB,fsθi,ϕi,θs,ϕs

where

σVdBθ=-min12θ-θcenterθ3dB2, σmax

σHdB φ=-min12φ-φcenterφ3dB2, σmax

  - Attenuatefactor=k1×sin(k2×β2)-5log10cosβ2 is applied to the  β within 0~180 degrees. K1= 6 and k2=1.65. β is the bistatic angle between the incident ray and scattering ray within the plane of incident direction (θi,ϕi,) and scattering direction (θs,ϕs).

  - The angles of (θ,φ) are the projections of the bisector angle on the vertical plane and the horizontal plane, respectively.

    - FFS: RCS value when β is 180 degrees

  - The effect of forward scattering RCSdB,fsθi,ϕi,θs,ϕs is -Inf in Rel-19

  - 5 sets of parameters φcenter, φ3dB, n, θcenter, θ3dB,n, Gmax, σmax, Applicable Range of θ and Applicable Range of φ are applicable as defined for the monostatic RCS of vehicle with single/multiple SPSTs

  - Continue study on a new formula for Attenuatefactor to resolve the issue of angular discontinuity.

    - The new formula should retain following property: the linear bistatic RCS for a vehicle with single scattering point is the sum of the bistatic RCS of the multiple scattering points of the vehicle

    - the following formula can be a reference for the study

Attenuatefactor=k1sink2β2(1-k3β-k4*(Gmax-min-σVdBθ+σHdB φ,σmax))

| Company | Yes/No | Comments |
| --- | --- | --- |
| CATT, CICTCI |  | Generally OK with this proposal |
| Samsung | Yes |  |
| Lenovo | Suggesting an addition | Currently, when a sensing target (e.g., vehicle) blocks a path towards an Rx node, the blockage model of 38.901 predicts the total received power at the Rx capturing the interaction of the said path with the blocker. From the above proposal, also, when a target is blocking a path, the received power at the Rx is given via the RCS effect. Hence, both predicting the Rx power due to the interaction of the path and the target. Nevertheless, the above two are not consistent. Since blockage models of 38.901 has been already validated, we need to clarify via a note that when a sensing target blocks a path terminated at an Rx node, then the blockage model in 38.901 shall be used to predict the Rx power. |
| BUPT3 | Yes | We agree to introduce parameter -5log10cosβ2, as it only takes effect when β is close to 180° and ensures the continuity of bi-static RCS. Meanwhile, it can fit well with our measured results. |
| Ericsson |  | OK with the proposal and Lenovo’s comment. |
| QC | Yes |  |
| Apple |  | We think that the parameter could be set to have minimal effect at lower angles and larger effect near beta = 180 degrees e.g. with a larger exponent factor e.g. 1/8 resulting in -1.25log10cosβ2, |
| NIST |  | Need some clarification on how this bistatic works together with the blockage model |
| Moderator4 |  | Agreed in Tuesday online session |

- Agreement

Confirm the following working assumption with updates in red.

Working assumption

For vehicle with single/multiple scattering points, the bistatic RCS is generated by

- The values/pattern of A*B1 of bistatic RCS is given by:

rcsdBθi,ϕi,θs,ϕs=maxGmax-min-σVdBθ+σHdB φ,σmax-Attenuatefactor,  Gmax-σmax,RCSdB,fsθi,ϕi,θs,ϕs

where

σVdBθ=-min12θ-θcenterθ3dB2, σmax

σHdB φ=-min12φ-φcenterφ3dB2, σmax

  - Attenuatefactor=k1×sin(k2×β2)-5log10cosβ2 is applied to the  β within 0~180 degrees. K1= 6 and k2=1.65. β is the bistatic angle between the incident ray and scattering ray within the plane of incident direction (θi,ϕi,) and scattering direction (θs,ϕs).

  - The angles of (θ,φ) are the projections of the bisector angle on the vertical plane and the horizontal plane, respectively.

    - FFS: RCS value when β is 180 degrees

  - The effect of forward scattering RCSdB,fsθi,ϕi,θs,ϕs is -Inf in Rel-19

  - 5 sets of parameters φcenter, φ3dB, n, θcenter, θ3dB,n, Gmax, σmax, Applicable Range of θ and Applicable Range of φ are applicable as defined for the monostatic RCS of vehicle with single/multiple SPSTs

  - Continue study on a new formula for Attenuatefactor to resolve the issue of angular discontinuity.

    - The new formula should retain following property: the linear bistatic RCS for a vehicle with single scattering point is the sum of the bistatic RCS of the multiple scattering points of the vehicle

    - the following formula can be a reference for the study

Attenuatefactor=k1sink2β2(1-k3β-k4*(Gmax-min-σVdBθ+σHdB φ,σmax))

UAV with large size and AGV

[Moderator’s note] Similar to monostatic RCS (reuse as framework as vehicle), can we extend the WA on bistatic RCS for vehicle to large size UAV and AGV?

[H] Proposal 4.1-1

- The WA on bistatic RCS for vehicle is reused for large size UAV and AGV.

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| LGE | Yes |  |
| BUPT | Yes | We agree that the WA for vehicle bistatic RCS can be reused for large UAVs and AGVs. For large UAVs, we suggest k₁=8 and k₂=1.85. For AGVs, we suggest k₁=8 and k₂=1.45. |
| Moderator |  | Thanks for inputs from BUPT, appreciated. So, let’s try the following proposal with values k1, k2. |
| Huawei, HiSilicon | Yes |  |
| NIST | Yes | We are okay to extend the WA on vehicle bistatic RCS to large size UAVs, and we propose k1 = 4.09 and k2=0.8 |
| SPREADTRUM | Yes |  |

[H] Proposal 4.1-1-rev1

- The WA on bistatic RCS for vehicle is reused for large size UAV and AGV.

  - For large size UAV, k₁=8 and k₂=1.85

  - For AGV, k₁=8 and k₂=1.45

| Company | Yes/No | Comments |
| --- | --- | --- |
| vivo | Yes |  |
| Apple | Yes |  |
| Moderator |  | Thanks for inputs from NIST on large size UAV. I think it should be acceptable to all to average the inputs from BUPT and NIST. Two decimals are kept. |

[H] Proposal 4.1-1-rev2

- The WA on bistatic RCS for vehicle is reused for large size UAV and AGV.

  - For large size UAV, k₁=6.05 and k₂=1.33

  - For AGV, k₁=8 and k₂=1.45

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes |  |
| LGE | Yes |  |
| Moderator |  | Thanks for the support from LGE and Samsung. There is an update on the value (8 -> 12) for AGV based on some offline discussion from proponent companies. |

[FL1][H] Proposal 4.1-1-rev3

- The WA on bistatic RCS for vehicle is reused for large size UAV and AGV.

  - For large size UAV, k₁=6.05 and k₂=1.33

  - For AGV, k₁=12 and k₂=1.45

| Company | Yes/No | Comments |
| --- | --- | --- |
| DOCOMO |  | Generally OK with this proposal. We are not sure about the value k1 for AGV. But the majority opinion is OK with k1=12 for AGV, we are fine. |
| Lenovo |  | the same comment as in proposal 4.0-1. |
| BUPT3 | Yes |  |
| NIST | Yes |  |
| Moderator4 |  | Agreed in Tuesday online session |

- Agreement

The agreement on bistatic RCS for vehicle is reused for large size UAV and AGV.

  - For large size UAV, k₁=6.05 and k₂=1.33

  - For AGV, k₁=12 and k₂=1.45

[Moderator’s note] The monostatic RCS for AGV with single scattering point was agreed by email discussion [Post-120bis-ISAC-02].

Agreement ([Post-120bis-ISAC-02]) On the monostatic RCS of AGV with single scattering point, - The values/pattern of component A*B1 are generated by the following parameters φcenter; φ3dB, n; θcenter; θ3dB,n; Gmax; σmax; Applicable Range of θ; Applicable Range of φ. Front; 0°; 13.68°; 90°; 13.68°; 13.02; 23.29; [30°,180°]; [-45°,45°]. Left; 90°; 15.53°; 75°; 20.03°; 7.33; 17.60; [30°,180°]; [45°,135°]. Back; 180°; 12.49°; 90°; 11.89°; 11.01; 21.28; [30°,180°]; [135°,225°]. Right; 270°; 15.53°; 75°; 20.03°; 7.33; 17.60; [30°,180°]; [225°,315°]. Roof; /; /; 0°; 11.44°; 11.79; 22.06; [0°,30°]; [0°,360°]. - When θ is in the range [0°,30° ], σHdB φ=0 - The standard deviation of component B2 is 2.51 dB

Based on the same method handling vehicle, the RCS of multiple scattering points of AGV can be derived. Please check if the following proposal is agreeable.

Note: there is no agreement to support AGV with multiple scattering points yet. So this proposal is to agree on this feature together with parameter values.

[FL1][M] Proposal 4.1-2

AGV can be modelled with multiple scattering points.

- The values/pattern of component A*B1 are generated by the following parameters

|  | ϕcenter in [°] | ϕ3dB, n in [°] | θcenter in [°] | θ3dB,n in [°] | Gmax | σmax | Range of θ in [°] | Range of ϕ in [°] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Front | 0° | 13.68° | 90° | 13.68° | 13.00 | 30.26 | [0,180] | [0,360] |
| Left | 90° | 15.53° | 75° | 20.03° | 7.27 | 24.53 | [0,180] | [0,360] |
| Back | 180° | 12.49° | 90° | 11.89° | 10.98 | 28.24 | [0,180] | [0,360] |
| Right | 270° | 15.53° | 75° | 20.03° | 7.27 | 24.53 | [0,180] | [0,360] |
| Roof | / | / | 0° | 11.44° | 11.77 | 29.03 | [0,180] | [0,360] |

  - Note: For the scattering point associated with roof of the AGV, σHdBϕ=0.

| Company | Yes/No | Comments |
| --- | --- | --- |
| CATT, CICTCI |  | Maybe one typo from the Note?: vehicle -> AGV |
| LGE |  | We think that it’s premature to support this proposal since the shape of AGV may be quite different from a vehicle. |
| Moderator |  | Thanks CATT, revised |
| Lenovo |  | We need to define the positions of the scattering points in the LCS of the target, and then fit a model for the RCS pattern. |
| BUPT3 | Yes |  |
| Ericsson |  | Two options were given for AGV’s size. Which Option is the RCS model for? AGV Option 1: 0.5m x 1.0m x 0.5m AGV Option 2: 1.5 m x 3.0m x 1.5 m |
| Moderator3 |  | Adding a note in the Monday offline session |

[FL1][M] Proposal 4.1-2-rev1

AGV can be modelled with multiple scattering points.

- The values/pattern of component A*B1 are generated by the following parameters

|  | ϕcenter in [°] | ϕ3dB, n in [°] | θcenter in [°] | θ3dB,n in [°] | Gmax | σmax | Range of θ in [°] | Range of ϕ in [°] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Front | 0° | 13.68° | 90° | 13.68° | 13.00 | 30.26 | [0,180] | [0,360] |
| Left | 90° | 15.53° | 75° | 20.03° | 7.27 | 24.53 | [0,180] | [0,360] |
| Back | 180° | 12.49° | 90° | 11.89° | 10.98 | 28.24 | [0,180] | [0,360] |
| Right | 270° | 15.53° | 75° | 20.03° | 7.27 | 24.53 | [0,180] | [0,360] |
| Roof | / | / | 0° | 11.44° | 11.77 | 29.03 | [0,180] | [0,360] |

  - Note: For the scattering point associated with roof of the AGV, σHdBϕ=0.

  - Note: the measurements from companies are done by AGV option 1.

[Moderator’s note] Agreed in Tuesday online session

- Agreement

AGV can be modelled with multiple scattering points.

- The values/pattern of component A*B1 are generated by the following parameters

|  | ϕcenter in [°] | ϕ3dB, n in [°] | θcenter in [°] | θ3dB,n in [°] | Gmax | σmax | Range of θ in [°] | Range of ϕ in [°] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Front | 0° | 13.68° | 90° | 13.68° | 13.00 | 30.26 | [0,180] | [0,360] |
| Left | 90° | 15.53° | 75° | 20.03° | 7.27 | 24.53 | [0,180] | [0,360] |
| Back | 180° | 12.49° | 90° | 11.89° | 10.98 | 28.24 | [0,180] | [0,360] |
| Right | 270° | 15.53° | 75° | 20.03° | 7.27 | 24.53 | [0,180] | [0,360] |
| Roof | / | / | 0° | 11.44° | 11.77 | 29.03 | [0,180] | [0,360] |

  - Note: For the scattering point associated with roof of the AGV, σHdBϕ=0.

  - Note: the measurements from companies are done by AGV option 1.

Bistatic RCS for UAV with small size and Human with RCS model 1

[Moderator’s note] for these two target types, the monostatic RCS is angular independent. The following proposal is proposed by ZTE and other companies in last RAN1 meeting.

The bistatic RCS of UAV with small size is modelled as

- Component A: same as component A of mono-static RCS for UAV of small size

- Component B1:

  - -3sin(β2) dB, where β is the bi-static angle between incident ray and scattered ray, β is within 0 and 180 degree

- Component B2: same as component B2 of mono-static RCS for UAV of small size

Supported by: HW, ZTE, LGE, CATT, Xiaomi, OPPO, SS,

One thing to note is the above proposal is essentially aligned with the WA on bistatic RCS for vehicle. In details, Gmax-min-σVdBθ+σHdB φ,σmax is simplified to a given value of component A due to the nature of angular independent. Attenunatefactor is 3sin(β2).

Further, considering the placeholder for forward scattering, the following proposal is made.

[FL1][H] Proposal 4.2-1

- The bistatic RCS of UAV with small size is modelled as

  - The values/pattern of A*B1 is given by

rcsdBθi,ϕi,θs,ϕs=maxσRCS,A-Attenuatefactor,RCSdB,fsθi,ϕi,θs,ϕs

    - Component A, i.e., σRCS,A: same as component A of mono-static RCS for UAV of small size

    - Attenuatefactor=3sin(β2) dB, where β is the bi-static angle between incident ray and scattered ray, β is within 0 and 180 degree

    - The effect of forward scattering RCSdB,fsθi,ϕi,θs,ϕs is -Inf in Rel-19

  - Component B2: same as component B2 of mono-static RCS for UAV of small size

- The bistatic RCS of Human with RCS model 1 is modelled as

  - The values/pattern of A*B1 is given by

rcsdBθi,ϕi,θs,ϕs=maxσRCS,A-Attenuatefactor,RCSdB,fsθi,ϕi,θs,ϕs

    - Component A, i.e., σRCS,A: same as component A of mono-static RCS for Human with RCS model 1

    - Attenuatefactor=3sin(β2) dB, where β is the bi-static angle between incident ray and scattered ray, β is within 0 and 180 degree

    - The effect of forward scattering RCSdB,fsθi,ϕi,θs,ϕs is -Inf in Rel-19

  - Component B2: same as component B2 of mono-static RCS for Human with RCS model 1

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| LGE |  | We’re generally ok with the attenuation factor in the form of sin(β2). Can you clarify how the scale factor 3 is determined? |
| BUPT |  | We agree that the bi-static RCS decreases as the β angle increases and that this modeling approach is rational. But we want to confirm whether a 3dB attenuation at β-180° is reasonable. |
| Moderator |  | @LGE, @BUPT: Scale factor is proposed by ZTE and other companies based on the validation results from ZTE. |
| ZTE2 |  | The scaling factor 3 is based on the evaluation results. Actually, the CDF curves below show scaling factor = 4 and 7 matched the RT results better. The value of 3 is a compromise outcome with vivo. - UAV (b)Human |
| vivo | Yes |  |
| Huawei, HiSilicon | Yes | Minor suggestion: It would be better to use the same term for the two equations for small UAV and human respectively in terms of A or σRCS,A : rcsdBθi,ϕi,θs,ϕs=maxσRCS,A-Attenuatefactor,RCSdB,fsθi,ϕi,θs,ϕs rcsdBθi,ϕi,θs,ϕs=maxA-Attenuatefactor,RCSdB,fsθi,ϕi,θs,ϕs |
| Moderator |  | @Huawei: thanks and directly updated the proposal 4.2-1 |
| Apple | Yes | Ideally, from our measurements, the attenuation factor should be linear but we can accept the consensus. |
| Samsung | Yes |  |
| Lenovo |  | When the target moves to the “blocker” region of a path towards an Rx point, the sudden change in the received energy due to the interaction of the path and the target is not captured by the proposed smooth loss function. Hence, we suggest to add the same note, as commented in 4.0-1. |
| Moderator4 |  | Agreed in Tuesday online session |

- Agreement

- The bistatic RCS of UAV with small size is modelled as

  - The values/pattern of A*B1 is given by

rcsdBθi,ϕi,θs,ϕs=maxσRCS,A-Attenuatefactor,RCSdB,fsθi,ϕi,θs,ϕs

    - Component A, i.e., σRCS,A: same as component A of mono-static RCS for UAV of small size

    - Attenuatefactor=3sin(β2) dB, where β is the bi-static angle between incident ray and scattered ray, β is within 0 and 180 degree

    - The effect of forward scattering RCSdB,fsθi,ϕi,θs,ϕs is -Inf in Rel-19

  - Component B2: same as component B2 of mono-static RCS for UAV of small size

- The bistatic RCS of Human with RCS model 1 is modelled as

  - The values/pattern of A*B1 is given by

rcsdBθi,ϕi,θs,ϕs=maxσRCS,A-Attenuatefactor,RCSdB,fsθi,ϕi,θs,ϕs

    - Component A, i.e., σRCS,A: same as component A of mono-static RCS for Human with RCS model 1

    - Attenuatefactor=3sin(β2) dB, where β is the bi-static angle between incident ray and scattered ray, β is within 0 and 180 degree

    - The effect of forward scattering RCSdB,fsθi,ϕi,θs,ϕs is -Inf in Rel-19

  - Component B2: same as component B2 of mono-static RCS for Human with RCS model 1

Bistatic/monostatic RCS for human with RCS model 2

Agreement On the monostatic RCS for human with RCS model 2 - The monostatic RCS for a scattering point of the target is generated by - The values/pattern A*B1, i.e., rcsdB(θ,φ) is deterministic based on incident/scattered angles rcsdB(θ,φ)=Gmax-min-σVdBθ+σHdB φ,σmax Where, σVdBθ=-min12θ-θcenterθ3dB2, σmax σHdB φ=-min12φ-φcenterφ3dB2, σmax - FFS how many rows of the values/pattern A*B1 are defined for the target - Note: each row has a defined applicable range of θ and φ - Note: whether the RCS is elevation angle dependent or dependent on both elevation and horizontal angles can be separately discussed

[Moderator’s note] it is agreed that monostatic RCS for human RCS model 2 uses same framework as other targets. The missing part is number of peaks and related parameter values.  So far, different companies have different proposal regarding the details.

Based on all inputs from companies (Apple, OPPO, LGE, Huawei, BUPT), the following joint proposal is proposed by the companies.

[FL1][M] Proposal 4.3-1

On the monostatic RCS of human with RCS model 2,

- The values/pattern of component A*B1 are generated by the following parameters

|  | ϕcenter in [°] | ϕ3dB, n in [°] | θcenter in [°] | θ3dB,n in [°] | Gmax | σmax | Range of θ in [°] | Range of ϕ in [°] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Front | 0 | 216.65 | 90 | 55.7 | 2.14 | 7.7 | [0,180] | [-90, 90] |
| Back | 180 | 216.65 | 90 | 55.7 | 2.14 | 7.7 | [0,180] | [90,270] |

- The standard deviation of component B2 is 3.94 dB

| Company | Yes/No | Comments |
| --- | --- | --- |
| LGE | Yes |  |
| BUPT | Yes |  |
| Huawei, HiSilicon |  | Per the proposal, the beam width for azimuth angle is 216.65 degrees which is almost omni-directional as the model-1. We can be ok with this proposal if the average RCS value align with the human model-1. |
| OPPO | Yes |  |
| Moderator |  | Huawei and OPPO reply at about same time 😊 Just one information, the value for Gmax and σmax are selected to have same average RCS between the two RCS models for human. Thanks Wenfeng |
| Apple | Yes |  |
| Moderator4 |  | Agreed in Tuesday online session |

- Agreement

On the monostatic RCS of human with RCS model 2,

- The values/pattern of component A*B1 are generated by the following parameters

|  | ϕcenter in [°] | ϕ3dB, n in [°] | θcenter in [°] | θ3dB,n in [°] | Gmax | σmax | Range of θ in [°] | Range of ϕ in [°] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Front | 0 | 216.65 | 90 | 55.7 | 2.14 | 7.7 | [0,180] | [-90, 90] |
| Back | 180 | 216.65 | 90 | 55.7 | 2.14 | 7.7 | [0,180] | [90,270] |

- The standard deviation of component B2 is 3.94 dB

[Moderator’s note] Further, assuming monostatic RCS for human RCS mode 2 can be agreed, can we extend the WA on bistatic RCS for vehicle to human with RCS model 2?

[FL1][M] Proposal 4.3-2

- The working assumption on bistatic RCS for vehicle with single scattering point is reused to model bistatic RCS of human with RCS model 2

  - k1=0.5714 and k2=0.1

| Company | Yes/No | Comments |
| --- | --- | --- |
| BUPT |  | We believe a uniform modeling method for the bi – static RCS of different sensing is targets needed. After simulations with k₁ = 0.5714 and k₂ = 0.1, this method shows no significant specular peak. So we don’t object to this method, but also want to other see companies’ views. |
| Apple | Yes | From our measurements, the attenuation factor shows no specular peak and is linear. Based on earlier discussions, the values of k1 and k2 are fine. |
| DOCOMO |  | We are Ok with main bullet. For the values k1 and k2, we want to see companies’ views. |
| Moderator4 |  | Agreed in Tuesday online session |

- Agreement

The agreement on bistatic RCS for vehicle with single scattering point is reused to model bistatic RCS of human with RCS model 2

  - k1=0.5714 and k2=0.1

Value of component A for bistatic and monostatic sensing modes

[Moderator’s note] From inputs in RAN1 #120bis, there is good consensus that value of component A is common to bistatic and monostatic RCS and needs to be defined per target.

[H] Proposal 4.4-1

- The value of the RCS component A is the mean of monostatic RCS values in all 3D incident/scattered angles of a target, and is applied to both monostatic and bistatic RCS of the target.

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| LGE | Yes |  |
| Nokia, NSB | Yes |  |
| BUPT | Yes |  |
| Ericsson |  | Component A and power-scaling factor are intermediate parameters used for channel modelling. Whether and how power-scaling factor will be used for other purposes in Rel-20 evaluation phase needs further discussion. For mono-static sensing mode and LOS condition, with 1:1 random ray pairing, 1 mono-static RCS value is used to generate the direct path, while nearly 1200 bi-static RCS values are used to generate the indirect paths. For mono-static sensing mode and NLOS condition, none of mono-static RCS is used for target channel. Therefore, we propose to add Note: RCS component A may not reflect the actual path loss in the target channel. |
| Vivo | Yes |  |
| CATT, CICTCI | Yes |  |
| OPPO | Yes |  |
| Apple | Yes |  |
| MediaTek | Yes |  |
| Moderator |  | Thanks and it seems the views are converged. Value of A are added by spherical integral of the monostatic RCS |
| Lenovo |  | “mean” is a statistical average, and depends on the angle-distribution. Suggest the following addition: The value of the RCS component A is the mean of monostatic RCS values for a uniform distribution of all 3D incident/scattered angles of a target, and is applied to both monostatic and bistatic RCS of the target. |

[H] Proposal 4.4-1-rev1

- The value of the RCS component A is the mean of monostatic RCS values in all 3D incident/scattered angles of a target, and is applied to both monostatic and bistatic RCS of the target.

  - UAV with large size: -5.85 dBsm

  - Human: -1.37 dBsm

  - Vehicle: 11.25 dBsm

  - AGV: -4.25 dBsm

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE |  | It is better to clarify the UAV above is large size UAV |
| CATT, CICTCI | Yes, but | Maybe we need to clarify the component A for UAV in this proposal is for small size or large size UAV? |
| Samsung | Yes |  |
| LGE |  | - Can you clarify how the values are determined. For example, are they the average of the companies’ input? - From the deployment scenario agenda, multiple types are defined for each target object. Which type per target does each value represent? UAV size: - Option 1 : 1.6m x 1.5m x 0.7m - Option 2 : 0.3m x 0.4m x 0.2m Human size: - Child: 0.2m x 0.3m x 1m - Adult Pedestrian: 0.5m x 0.5m x 1.75m Vehicle: - Type 1/2 (passenger vehicle) - Type 3 (truck/bus) AGV size - Option 1 : 0.5m x 1.0m x 0.5m - Option 2 : 1.5 m x 3.0m x 1.5 m |
| Moderator |  | Thanks ZTE and CATT, revised in Proposal 4.4-1-rev1 directly @LGE: the proposal on component A is based on all agreed values of monostatic RCS (it is still a proposal for human RCS model 2). |
| Moderator |  | The proposal is further revised since the important thing is the value of component A. How to derive the component A is not necessarily in the agreement. |

[FL1] [H] Proposal 4.4-1-rev2

- The following values of the RCS component A is the mean of monostatic RCS values in all 3D incident/scattered angles of a target, and is are applied to both monostatic and bistatic RCS of the target.

  - UAV with large size: -5.85 dBsm

  - Human with RCS model 2: -1.37 dBsm

  - Vehicle: 11.25 dBsm

  - AGV: -4.25 dBsm

| Company | Comments |
| --- | --- |
| BUPT3 | Yes |
| Ericsson | Same comment to [M] Proposal 4.1-2 about AGV size. |
| QC | Yes |
| Apple | OK |
| Moderator3 | Adding 3 notes in the Monday offline session |

[FL1] [H] Proposal 4.4-1-rev3

- The following values of the RCS component A is the mean of monostatic RCS values in all 3D incident/scattered angles of a target, and is are applied to both monostatic and bistatic RCS of the target.

  - UAV with large size: -5.85 dBsm

  - Human with RCS model 2: -1.37 dBsm

    - Note: measurement is based on adult

  - Vehicle: 11.25 dBsm

    - Note: measurement is based on vehicle type 1 and 2

  - AGV: -4.25 dBsm

    - Note: measurement is based on AGV option 1

[Moderator’s note] Agreed in Tuesday online session

- Agreement

The following values of the RCS component A are applied to both monostatic and bistatic RCS of the target.

  - UAV with large size: -5.85 dBsm

  - Human with RCS model 2: -1.37 dBsm

    - Note: measurement is based on adult

  - Vehicle: 11.25 dBsm

    - Note: measurement is based on vehicle type 1 and 2

  - AGV: -4.25 dBsm

    - Note: measurement is based on AGV option 1

Note: component A on its own may not fully reflect the RCS in the target channel. This note will not be captured in the TR.

XPR to generate polarization matrix

[Moderator’s note] XPR values for AGV are still missing, which can be handled in same way as other targets to merge the values from interested companies.

[H] Question 4.5-1

- Please input XPR values for AGV

| Company | Comments |
| --- | --- |
| BUPT2 | To address the missing XPR values of the AGV, we conducted RCS measurements under different polarizations. The results are shown in Fig.1. Fig.1: The RCS of a AGV at different polarization Based on the measured data, we calculate the kapa using the formula κ =RCSVVRCSVH. The results are fitted to a normal distribution, yielding: κAGVdB~N(9.60, 6.852) Fig.2: The CDF of the [dB] κ of AGV |
| Moderator | Thanks for inputs from BUPT. |

[FL1][H] Proposal 4.5-1

The mean and standard deviation values of XPR of sensing target AGV for monostatic sensing and bistatic sensing are (9.60, 6.85) dB.

| Company | Comments |
| --- | --- |
| BUPT3 | Yes |
| Moderator4 | Agreed in Tuesday online session |

- Agreement

The mean and standard deviation values of XPR of sensing target AGV for monostatic sensing and bistatic sensing are (9.60, 6.85) dB.

[Moderator’s note] There is a FFS in the early agreement on XPR. The proponent proposed to assume XPR is generated in LCS, then it needs to be transformed to GCS to multiply with the polarization matrix for Tx-target link and target-Rx link. On the other hand, majority companies (at least in RAN1 #120) don’t think such operation is necessary since all parameters in CPM, i.e., XPR and initial phase are randomly generated. It is anyway random matrix no matter transformation between LCS/GCS is done or not. Further, RAN1 agrees on power normalization on the product of CPMtx,sp,rx= CPMsp,rx . CPMsp . CPMtx,sp, which effective introduce a random change on the polarization matrix of the target since both CPMsp,rx and CPMtx,sp are random matrix too.

Agreement To model polarization matrix CPMsp,i=αi,1expjΦsp,iθθβi,1expjΦsp,iθϕβi,2expjΦsp,iϕθαi,2expjΦsp,iϕϕ of a direct/indirect path i of a scattering point of a target - in Rel-19 study item (e.g., UAV, human, vehicle, AGV), αi,1=αi,2=1, βi,1=βi,2=κsp,i-1, i.e., CPMsp,i=expjΦsp,iθθκsp,i-1expjΦsp,iθϕκsp,i-1expjΦsp,iϕθexpjΦsp,iϕϕ Where, - κsp,i is XPR ratio is randomly generated by log-normal distribution per target type - The initial random phase Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ is uniformly distributed within -π,π - FFS: spatial consistency of random phase when a scattering point moves - FFS: whether the base station rotation procedure in 38.901 can be reused to support rotation of the target

With the above observations, the moderator makes the following proposal to close this FFS.

[FL2][M] Proposal 4.5-2 for conclusion

- The transformation between LCS and GCS is not considered to model the polarization matrix CPMsp,i of a direct/indirect path i of a scattering point of a target

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| CATT, CICTCI |  | Generally OK with this proposal. |
| Samsung | Yes |  |
| LGE | Yes |  |
| BUPT3 | Yes |  |
| Ericsson | No | Even with randomly generated kappa, phase and power normalization for CPMrx,n',m'k,pCPMn',m',n,mk,pCPMtx,n, mk,p, HH and VV always have the same power 1 for the direct path. This is because without the transformation procedure, CPM of direct path =100-1* ejΦ1kejΦ2kejΦ3ejΦ4 *100-1=ejΦ1-kejΦ2-kejΦ3ejΦ4. Power normalization doesn’t change the ratio between HH and VV power. However, the same HH and VV power is not true when the target changes its z axis during the movement. The legacy transformation procedure in 38.901 has been used to model Type-2 EO. Now we apply it to support rotation of a target. We provided the steps on obtaining the transformation matrix in our updated Tdoc R1-2504707. If CPM of target is defined in LCS, a rotation of the target can be supported by replacing CPMn',m',n,mk,p with +cosψ2-sinψ2+sinψ2+cosψ2CPMn',m',n,mk,p+cosψ1+sinψ1-sinψ1+cosψ1. 1 is derived using equation 7.1-15 in 38.901 with α, β and γ, and spherical position (θtx,n,m,ZOAk,p,ϕtx,n,m,AOAk,p) in equation 7.9.4-12 in draft CR and 2 is derived in the same way with spherical position (θrx,n',m',ZODk,p,ϕrx,n',m',AODk,p) in equation 7.9.4-11. If z axis of the LCS of the target is always the same as z axis of the GCS during the movement, e.g., a vehicle turning left or right on x-y plane, +cosψ2-sinψ2+sinψ2+cosψ2 and +cosψ1+sinψ1-sinψ1+cosψ1 are identity matrix. The additional procedure can be skipped and the conclusion can work. But if z axis of LCS changes in the movement, such as a vehicle driving on/off a slope, it changes the power distribution among polarizations, and HH and VV of direct path are not equal any more. In such case, the transformation procedure cannot be skipped. We have the following two proposals, which have the same results. - The transformation between LCS and GCS is not considered to model the polarization matrix CPMsp,i of a direct/indirect path i of a scattering point of a target, if the target’s z axis of LCS is always the same as that of GCS during the movement. Otherwise, transformation between LCS and GCS is considered. - Or alternatively, CPM of target is modelled in LCS. |
| Moderator5 |  | Agreement from Wednesday online session. |

- Agreement

The polarization matrix CPMsp,i of a direct/indirect path i of a scattering point of a target is defined in LCS.

Angular correlation of RCS component B2

[Moderator’s note] In general, we need separately consider three aspects: 1) correlation of B2/XPR/initial random phase for different targets; 2) angular correction of B2/XPR/initial random phase in different direct/indirect paths of same target. 3) correlation of B2/XPR/initial random phase for a direct/indirect path for same target.

The proponent argues that if angular correlation is not satisfied, there will strong Doppler artifacts outside the main peak of the Doppler frequency of the target.

However, the other companies comment that the correlation between angle is already modelled by angular dependent B1. The motivation for B2 is the random glint observed in the validation of RCS.

From inputs in RAN1 #120bits - Companies are encouraged to comment on whether/how to model correlation of RCS of a scattering point in adjacent incident/scattered angles? Yes: Lenovo, Ericsson, NIST, Sony No: ZTE, LGE, HW, CATT, QC, SS, Xiaomi, DOCOMO, Spreadtrum, Nokia, MTK Low priority: Apple Evaluation needed: IDCC

Some observations on solutions and pros/cons of each solution from early discussions are as follows

- Option 1: Same value of B2 always applies to a path per simulation

  - Option 2 is not accurate if the movement of Tx/target/Rx results in large change of incident/scattered angles

  - Option 2 doesn’t mean no change of RCS value since A*B1 is still angular dependent even if B2 is unchanged in mobility.

- Option 2: Same value of B2 applies to a path before the value of B2 is updated. The periodicity to update B2 can be discussed in evaluation phase or up to company choice

  - The large/small scale parameters in existing 38.901, once generated, are used for a while (multiple frames/slots) before update. Option 2 keeps the similar principle.

  - Option 2 doesn’t mean no change of RCS value since A*B1 is still angular dependent even if B2 is unchanged in mobility.

- Option 3: angular correlation of B2 modelled by the existing correlation distance based spatial consistency defined in TR 38.901

  - In reality, the correlation of B2 is related to the change of angle, not the distance of target

- Option 4: angular correlation of B2 modelled by the ‘correlation angle’.

  - This option may be more accurate than Option 3, but it needs a completely new parameter

  - No validation results available on ‘correlation angle’

- Option 5: The proposal from Ericsson: Model the stochastic B2 with C random scattering centres as B2dinc, dsca=c=1Cβexp⁡(-j2πdinc+dsca,pc/λ)2

  - It is questionable whether Option 5 still follows log-normal distribution

  - Need details on how to model the C scattering centres

[Moderator’s note] A first question is whether B2 for different targets are independent or correlated. From offline discussions, it seems straightforward to assume it is independent.

[FL1][M] Proposal 4.6-1 for conclusion

The component B2 of two different targets are generated independently.

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes | We support the above option 1 or option 2 |
| LGE | Yes |  |
| Nokia, NSB | Yes | Option 1 |
| BUPT | Yes | The spatial consistency between two distinct reception points with the same transmitter arises from the similarity of their propagation environments, which leads to correlated channel statistical properties. It should be acknowledged that for the same STx-SRx link pair, the B2 values of two different targets theoretically require consideration of their correlation. However, to simplify the model and due to time constraints for the conference, we suggest independently generating the B2 values for the two targets. |
| Ericsson | Yes | Some clarification of Option 5 - Option 5: The proposal from Ericsson: Model the stochastic B2 with C random scattering centres as B2dinc, dsca=c=1Cβexp⁡(-j2πdinc+dsca,pc/λ)2 B2 of each scattering center is log-normal distribution. As suggested in the reference, the C scattering centers are randomly distributed over the surface of a unit sphere. Locations of the scattering centers remain the same in a simulation. β is the scaling factor. Reference: Spherical Harmonic-Based Anisotropic Point Scatterer Models for Radar Applications Using Inverse Optimization, 2022 IEEE |
| vivo | Yes | We prefer Option 1. The reason is as follows: First of all, we have to understand which mobility phenomenon is modelled by the parameter B2. To this end, we differentiate the mobility phenomena of the channel changed by the Tx, Target, Rx. For simplicity, we assume Tx is stationary, e.g., as gNB, and thus, the mobility phenomena are only impacted due to the motions of Target and Rx. Bistatic RCS, principally, is quantitatively depends on the bistatic angle, and the relation between the type of polarization of the incident electromagnetic field and the type of polarization of the scattering electromagnetic field. The RCS impact of bistatic angle is modelled by the component of B1, while the CPM is used to model the polarization properties. Please note that mono-static RCS is a special case where the bistatic angle is 0o. In our understanding, the component B2 is only a supplemental parameter, used to model the small variation, e.g., the same type of target but with different exterior. The temporal/spatial impact on RCS due to the mobility of target and/or Rx is captured in B1, meaning that B2 needs to be kept unchanged after B2 is initiated in the beginning of the simulation. This is because we do not want to doubly model the mobility phenomenon on RCS. |
| CATT, CICTCI | Yes | Prefer Option 1 or 2. Minor comment on typo: current sub-bullet of Option 1 still says’ Option 2’. Better change it to ‘Option 1’ to avoid misleading. |
| OPPO | Yes | Option 1 or 2. |
| Huawei, HiSilicon | Yes |  |
| NIST | Yes | Prefer Option 3 and 4. |
| Apple | Yes | Option 1 or Option 2 |
| MediaTek | Yes |  |
| DOCOMO | Yes |  |
| Lenovo | Yes |  |
| Moderator4 |  | Agreed in Tuesday online session |

- Conclusion

The component B2 of two different targets are generated independently.

[Moderator’s note] Since the main concern is the impacts to Doppler of each direct/indirect path due to mobility of Tx/target/Rx, we may follow the majority view that no angular correlation of B2 among different direct/indirect paths of a target.

[FL1][M] Proposal 4.6-2

RCS component B2 of different direct/indirect paths of a target in the target channel are generated independently

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes | This is based on the RCS validation results. |
| LGE | Yes |  |
| Nokia, NSB | Yes |  |
| BUPT | Yes | For the same target, different incident/scattering paths can be treated as two independent observations generated from a stochastic process. Therefore, we recommend modeling them as independently generated. |
| Vivo | Yes |  |
| CATT, CICTCI | Yes |  |
| OPPO | Yes |  |
| Huawei, HiSilicon | Yes |  |
| NIST |  | Prefer to model B2 correlation for direct paths. |
| Apple | Yes |  |
| MediaTek | Yes |  |
| DOCOMO | Yes |  |
| Lenovo |  | Agree to the comment by NIST (correlation for direct path) |
| SPREADTRUM | Yes |  |
| QC | Yes |  |
| Moderator4 |  | Merged to next agreement |

[Moderator’s note] Due to mobility of Tx/target/Rx, the incident/scattered angle of a direct/indirect path changes, which impacts the RCS values.

- Option 1 is simpler, but it also has clear drawback.

- Option 2 maximizes reusing existing principles to handle large/small scale parameters in 38.901

- If Option 3 or 4 is selected, we open more discussions and cannot be completed in one meeting

- Option 5 is quite different from existing solutions from 38.901.

The moderator would like to check whether Option 2 can be agreeable, which seems the only solution which we could complete the issue in the limited remaining time.

Note: Option 2 seems also the default behaviour since it keeps the similar principle as large/small scale parameters generation in existing 38.901. In a simulation with existing 38.901, once generated, such parameters are used for a while (multiple frames/slots) before update. Option 2 keeps the similar principle.

[M] Proposal 4.6-3

On the RCS component B2 of a direct/indirect path of a target in the target channel, the same value of B2 applies to a path before the value of B2 is updated.

- The periodicity to update B2 can be discussed in evaluation phase or up to company choice

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Nokia, NSB | Yes |  |
| BUPT | Yes | We endorse maintaining consistency with the established principles outlined in 38.901. |
| CMCC |  | Since option-4 and option-5 have parameters to be further studied, we prefer to keep B2 unchanged before updated (if update is needed). And whether/how/when to update B2 can be up to companies’ choices. It is recommended to change the wording of the bullet as: - “Whether/how/when to update B2 can be discussed in evaluation phase or up to companies’ choices” |
| Ericsson | No | A channel model has to be continuous in time. Everything else is unphysical and can have unforeseen consequences, some of which are Doppler artifacts. It is important to realize that artifacts that only occur in the target channel but not in the background channel may be used to artificially inflate the target detection probability and reduce the false alarm rate. Hence it is very important to not introduce such artifacts in the channel model. Recall that one of the objectives in the SID is to support target tracking, which works on longer time scales than typical communication simulations. Therefore, it is not at all obvious that previously used approaches for drop-based communication simulations can be reused. Indeed, in the time scales considered for tracking, the target may move or rotate such that the incidence and outgoing angles are significantly changed over time. |
| Vivo | Yes |  |
| CATT, CICTCI | Yes | Option 1 can be considered as a special case that the periodicity of B2 update is infinite. If spatial consistency is enabled, B2 might be updated according to the correlation distance. |
| OPPO | Yes | Although we agree with the proposal’s intention, we think the proposal itself sounds redundant. It offers nothing meaningful by saying “the same value applies (is maintained) before it is updated”. With this wording, the key question is still when/how it is updated. It seems the proposal would leave a footprint in the TR with a question which the TR refuses to answer. 38.901 contains quite some random number generations in small scale, and may or may not mention when/how each of these random numbers should be updated. RCS B2 is just not an exception among those. |
| Huawei, HiSilicon | Yes | The B2 is modelled as a glint for the RCS per angle and in our view it is generated once at the initialization of the simulation drops. i.e. the entire RCS(A*B1*B2) of the sensing target is initialized at the beginning of the simulation. |
| Moderator |  | Based on all inputs, there is a clear majority view to have B2 unchanged before update, which is also a case of continuous B2. Hope Ericsson could be OK. Updated the proposal as suggested by CMCC |
| Lenovo | Yes, with comment | However, would like to point out that this step-wise structure for update of B2 does not solve the problem of “how fast (with which slope) shall B2 be updated” which is the main question. |

[FL1][M] Proposal 4.6-3-rev1

On the RCS component B2 of a direct/indirect path of a target in the target channel, the same value of B2 applies to a path before the value of B2 is updated.

- The periodicity to update B2 can be discussed in evaluation phase or up to company choice

- Whether/how/when to update B2 can be discussed in evaluation phase or up to companies’ choices

| Company | Yes/No | Comments |
| --- | --- | --- |
| Apple | Yes |  |
| MediaTek | Yes |  |
| ZTE | Yes |  |
| CATT, CICTCI | Yes |  |
| LGE |  | Even for the same target, we support that the component B2 values are generated independently as long as the incident or scattered angle changes. |
| Lenovo | Yes, with comment | However, would like to point out that this step-wise structure for update of B2 does not solve the problem of “how fast (with which slope) shall B2 be updated” which is the main question. |
| BUPT3 | Yes |  |
| InterDigital |  | We propose to treat both [M] Proposal 4.6-3-rev1 and [M] Proposal 4.6-2 jointly as both proposals discuss treatment of B2 for direct and indirect paths for a target. |
| Ericsson |  | If whether to support angular correlation of B2 is up to companies’ choice, we would like to point out the consequences of Option 1 and Option 2. With Option 1, there are no rapid random variations of RCS, which are observed in measured channels. It obviates the need of combining observations over time to ensure a good probability of detection, which is necessary in reality. With Option 2, discontinuous RCS leads to non-physical artifacts in the channel, including very high Doppler frequencies and spurious out-of-band artifacts (the purple vertical lines in the figure below), which may negatively affect the accuracy of detection and even cause unexpected impacts on communication. The following figures are for Option 5. As expected, we can see trenches where deep fades occur in the channel and a continuously varying intensity over time. |
| QC | Yes |  |
| Moderator4 |  | Agreed in Tuesday online session |

- Agreement

- RCS component B2 of different direct/indirect paths of a target in the target channel are generated independently.

- On the RCS component B2 of a direct/indirect path of a target in the target channel, the same value of B2 applies to a path before the value of B2 is updated.

  - Note: whether/how/when to update B2 can be discussed in evaluation phase or up to companies’ choices

Angular correlation of polarization matrix of a target

[Moderator’s note] In general, we need separately consider three aspects: 1) correlation of B2/XPR/initial random phase for different targets; 2) angular correction of B2/XPR/initial random phase in different direct/indirect paths of same target. 3) correlation of B2/XPR/initial random phase for a direct/indirect path for same target.

Based on company inputs on the following questions

- Companies are encouraged to comment on whether/how to model correlation of XPR/initial random phase of a scattering point in adjacent incident/scattered angles?

Yes: Ericsson, CATT

No: ZTE, LGE, QC, Apple, SS, Xiaomi, BUPT, DOCOMO

The proponent argues that the discontinues of XPR/initial random phase is similar to B2, i.e., causing strong Doppler artifacts outside the main peak of the Doppler frequency of the target.

The other companies comment XPR/initial random phase may have different values even for the same scattering point with small gap in incident/scattered angle

IDCC

- The XPR and phase of two different targets should not be correlated.

- The XPR and phase of the same target should be correlated if the location of the target at two different time instances are within the correlation distance.

Ericsson

- Keep the initial phases Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕunchanged for the duration of the simulation

Huawei

- The XPR correlation between SPs can be modelled as the spatial consistency.

The 5 options as discussed in section 4.6 can apply to XPR handling too.

[Moderator’s note] As proposed by IDCC, XPR and initial random phase of two different targets would be independent since the different target may have different orientation/shape/size/material/etc. From offline discussions, it seems straightforward to assume it is independent.

[FL1][M] Proposal 4.7-1 for conclusion

The component XPR/initial random phase of two different targets are generated independently.

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| LGE | Yes |  |
| Nokia, NSB | Yes |  |
| InterDigital |  | We are ok with the proposal. As a separate proposal, we would like to propose to model spatial consistency when a target is moving. We had the following proposal in our contribution for RAN1#120b. From offline discussion, we believe there’s sufficient interest to discuss/support the proposal below. Proposal 4: Initial random phase and XPR of CPMtx,sp and CPMsp,rx are link-correlated for spatial consistency odelling and spatially consistent mobility odelling of the same target. |
| BUPT | Yes | As mentioned in Section 4.6, we suggest that the component XPR and initial random phase for two different targets be generated independently. |
| Ericsson | Yes |  |
| vivo | Yes |  |
| CATT, CICTCI | Yes |  |
| Huawei, HiSilicon | Yes | To be clear, spatial consistency can still be applied when necessary which includes XPR and initial random phase as agreed. |
| Moderator |  | @Huawei: I think you refer to the following agreement, but it is for the links, i.e., UT-target link or target to UT link. I think it doesn’t include target itself when we make the agreement. This is the reason we discuss the issue again using proposal 4.7-1/2/3/4/5. Hope this clarification works. |
| NIST | Yes |  |
| Apple | Yes |  |
| MediaTek | Yes |  |
| Samsung | Yes |  |
| DOCOMO | Yes |  |
| Lenovo | Yes |  |
| QC | Yes |  |
| Moderator4 |  | Agreed in Tuesday online session |

- Conclusion

The component XPR/initial random phase of two different targets are generated independently.

[Moderator’s note] Since the main concern is the impacts to Doppler of each direct/indirect path due to mobility of Tx/target/Rx, we may follow the majority view that no angular correlation of XPR among different direct/indirect paths of a target.

[FL1][M] Proposal 4.7-2

The XPR of a target for the different direct/indirect paths in the target channel are generated independently

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| LGE | Yes |  |
| Nokia, NSB | Yes |  |
| BUPT | Yes | Although strong Doppler artifacts exist outside the main peak of the target’s Doppler frequency spectrum, their amplitudes are observed to be relatively small. Considering the limited remaining time, we recommend independently generating the XPR values for different paths of the same target. |
| Vivo | No | First of all, according to the current agreement, XPR is randomly generated by log-normal distribution per target type. The CPM is modelled how the polarization of the incident electromagnetic field and the polarization of the scattering electromagnetic field are changed by the target. This means that the CPM depends on the incident and scattering angles. If XPR of different incident/scattered angles for a target is generated independently, the overall ray-based channel after combining will eliminate the information interacted by the target. The mechanism of the spatial consistency can be utilized to model the XPR of different incident/scattered angles for a target. |
| CATT, CICTCI | Yes |  |
| Huawei, HiSilicon | Yes |  |
| NIST | No | XPR depends on the target material, surface shape, roughness, and orientation. Spatial consistency should be considered. |
| MediaTek | Yes |  |
| Samsung | Yes |  |
| DOCOMO | Yes |  |
| Moderator4 |  | Merged to next agreement |

[Moderator’s note] Due to mobility of Tx/target/Rx, the incident/scattered angle of a direct/indirect path changes, which impacts the XPR values.

- Option 1 is simpler, but it also has clear drawback.

- Option 2 maximizes reusing existing principles to handle large/small scale parameters in 38.901

- If Option 3 or 4 is selected, we open more discussions and cannot be completed in one meeting

- Option 5 is quite different from existing solutions from 38.901.

The moderator would like to check whether Option 2 can be agreeable, which seems the only solution which we could complete the issue in the limited remaining time.

Note: Option 2 seems also the default behaviour since it keeps the similar principle as large/small scale parameters generation in existing 38.901. In a simulation with existing 38.901, once generated, such parameters are used for a while (multiple frames/slots) before update. Option 2 keeps the similar principle.

[M] Proposal 4.7-3

On XPR of a target for a direct/indirect path in the target channel, the same value of XPR applies to a path before the value of XPR is updated.

- The periodicity to update XPR can be discussed in evaluation phase or up to company choice

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Nokia, NSB | Yes |  |
| BUPT | Yes | We suggest maintaining alignment with the 38.901 specification. |
| CMCC |  | Uncorrelation in XPR causes similar effects as B2. We also prefer to keep XPR unchanged before updated. And whether/how/when to update XPR can be up to company choice. It is recommended to change the wording of the bullet as: “Whether/how/when to update XPR can be discussed in evaluation phase or up to companies’ choices” |
| Ericsson | No | Same comment as for 4.6-3 |
| vivo | Yes |  |
| CATT, CICTCI | Yes | Same comment as for B2 update. |
| Huawei, HiSilicon | Yes |  |
| Moderator |  | Thanks CMCC and update the proposal |
| QC | Yes |  |

[FL1][M] Proposal 4.7-3-rev1

On XPR of a target for a direct/indirect path in the target channel, the same value of XPR applies to a path before the value of XPR is updated.

- The periodicity to update XPR can be discussed in evaluation phase or up to company choice

- Whether/how/when to update XPR can be discussed in evaluation phase or up to companies’ choices

| Company | Yes/No | Comments |
| --- | --- | --- |
| Apple | Yes | Same behavior as B2 |
| MediaTek | Yes |  |
| ZTE | Yes |  |
| CATT, CICTCI | Yes |  |
| Samsung | Yes |  |
| LGE |  | Even for the same target, we support that the XPR values are generated independently as long as the incident or scattered angle changes. |
| BUPT3 | Yes |  |
| InterDigital |  | We propose to treat both [M] Proposal 4.7-3-rev1 and [M] Proposal 4.7-2 jointly as both proposals discuss treatment of XPR for direct and indirect paths for a target. |
| Ericsson |  | Same comments to [M] Proposal 4.6-3-rev1 |
| QC | Yes |  |
| Moderator4 |  | Agreed in Tuesday online session |

- Agreement

- XPR of different direct/indirect paths of a target in the target channel are generated independently.

- On the XPR of a direct/indirect path of a target in the target channel, the same value of XPR applies to a path before the value of XPR is updated.

  - Note: whether/how/when to update XPR can be discussed in evaluation phase or up to companies’ choices

[Moderator’s note] Since the main concern is the impacts to Doppler of each direct/indirect path due to mobility of Tx/target/Rx, we may follow the majority view that no angular correlation of initial random phasamong different direct/indirect paths of a target.

[FL1][M] Proposal 4.7-4

The initial random phase of a target for the different direct/indirect paths in the target channel are generated independently

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| LGE | Yes |  |
| Nokia, NSB | Yes |  |
| BUPT | Yes |  |
| vivo | No | Similar to what we commented for XPR in Proposal 4.7-2, the initial random phase is one of parameter to model CPM. The mechanism of the spatial consistency can be utilized to model it. |
| CATT, CICTCI | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Apple | Yes |  |
| MediaTek | Yes |  |
| Samsung | Yes |  |
| DOCOMO | Yes |  |
| Lenovo | Yes |  |
| Moderator4 |  | Merged to next agreemeent |

[Moderator’s note] Due to mobility of Tx/target/Rx, the incident/scattered angle of a direct/indirect path changes, which impacts the initial random phase at target.

- Option 1 is simpler, but it also has clear drawback.

- Option 2 maximizes reusing existing principles to handle large/small scale parameters in 38.901

- If Option 3 or 4 is selected, we open more discussions and cannot be completed in one meeting

- Option 5 is quite different from existing solutions from 38.901.

The moderator would like to check whether Option 2 can be agreeable, which seems the only solution which we could complete the issue in the limited remaining time.

Note: Option 2 seems also the default behaviour since it keeps the similar principle as large/small scale parameters generation in existing 38.901. In a simulation with existing 38.901, once generated, such parameters are used for a while (multiple frames/slots) before update. Option 2 keeps the similar principle.

[M] Proposal 4.7-5

On the initial random phase of a target for a direct/indirect path in the target channel, the same value of initial random phase applies to a path before the value of the initial random phase is updated.

- The periodicity to update initial random phase can be discussed in evaluation phase or up to company choice

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Nokia, NSB | Yes |  |
| BUPT | Yes |  |
| CMCC |  | Random variation in initial random phase matters more than that in B2 or XPR. For simplicity, we prefer to keep initial phase unchanged for the duration of the simulation, considering that there still exists spatial consistent random phase in TX-target link and target-RX link. |
| Ericsson | No | Same comment as for 4.6-3 |
| vivo | Yes |  |
| CATT, CICTCI | Yes | Same comment as for B2 update. |
| Huawei, HiSilicon | Yes |  |
| Moderator |  | CMCC proposal on 4.6-3 and 4.7-3 can be applicable to this proposal too |
| Apple | Yes |  |

[FL1][M] Proposal 4.7-5-rev1

On the initial random phase of a target for a direct/indirect path in the target channel, the same value of initial random phase applies to a path before the value of the initial random phase is updated.

- The periodicity to update initial random phase can be discussed in evaluation phase or up to company choice

- Whether/how/when to update initial random phase can be discussed in evaluation phase or up to companies’ choices

| Company | Yes/No | Comments |
| --- | --- | --- |
| MediaTek | Yes |  |
| ZTE | Yes |  |
| CATT, CICTCI | Yes |  |
| Samsung | Yes |  |
| LGE |  | Even for the same target, we support that the initial random phase values are generated independently as long as the incident or scattered angle changes. |
| BUPT3 | Yes |  |
| InterDigital |  | We propose to treat both [M] Proposal 4.7-5-rev1 and [M] Proposal 4.7-4 jointly as both proposals discuss treatment of the initial random phase for direct and indirect paths for a target. |
| Ericsson |  | In the section of spatial consistency, random phase in Step 10 is one of the random variables which consistency should be kept. Similarly, how to update initial random phase should take spatial consistency into account. |
| QC | Yes |  |
| Moderator4 |  | Agreed in Tuesday online session |

- Agreement

- Initial random phase of different direct/indirect paths of a target in the target channel are generated independently.

- On the initial random phase of a direct/indirect path of a target in the target channel, the same value of initial random phase applies to a path before the value of initial random phase is updated.

  - Note: whether/how/when to update initial random phase can be discussed in evaluation phase or up to companies’ choices

RCS for other targets/EO type-1

[Moderator’s note] We agreed several meetings ago that EO type-1 is modelled as if target in the channel generation. If possible, it is good to define RCS model for other targets or EO type-1 in Rel-19. Otherwise, the additional targets/EO type-1 can be discussed in evaluation phase.

The following two proposals are made based on inputs from Ericsson tdoc.

[M] Proposal 4.8-1

The bistatic RCS of a bird is modelled by,

- Component A: uniformly distributed within range [-40, -20] dBsm

- Component B1: 0 dB

- Component B2: standard deviation of B2 is 0 dB

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE |  | If we have time to discuss this, we suggest reuse the RCS model for small UAV with the following modification: Smaller component A value, and may be smaller B2 value (but should not be 0dB). |
| Nokia, NSB |  | Not sure why we specifically discuss RCS of a “bird” as one typical EO type-1. |
| Ericsson |  | Type-1 EO is defined as follows. So far we have not discussed types of Type-EO other than sensing targets, but SID states ‘a common modelling framework capable of detecting and/or tracking the following example objects and to enable them to be distinguished from unintended objects’. This is the motivation for birds and trees as Type-1 EO. - Applicable for an EO having comparable physical characteristics as a sensing target, (referred as EO type-1 for discussion purpose) Trees can generate Doppler shift as observed in our measurement and also literature. Bi-static RCS models of a bird and a tree are based on our measurements and the following references. - Y. L. C. de Jong and M. H. A. J. Herben, “A tree-scattering model for improved propagation prediction in urban microcells,” in IEEE Transactions on Vehicular Technology, vol. 53, no. 2, pp. 503-513, March 2004. - Jackson et al, “The motion of trees in the wind: a data synthesis”, Biogeosciences, vol. 18, no. 13, pp. 4059-4072, 2021. - Rahman, S., Robertson, D.A. “Radar micro-Doppler signatures of drones and birds at K-band and W-band”, Sci Rep 8, 17396 (2018), https://doi.org/10.1038/s41598-018-35880-9. - C. R. Vaughn, “Birds and insects as radar targets: A review,” in Proceedings of the IEEE, vol. 73, no. 2, pp. 205-227, Feb. 1985. - J. Ko et al., “Measurements and Analysis of Radio Propagation at 28 GHz in Vegetated Areas of Typical Residential Environments,” in IEEE Transactions on Antennas and Propagation, vol. 68, no. 5, pp. 4149-4154, May 2020. |
| Vivo | No | We cannot make such an agreement. If we take such a simple RCS model, it will be optimistic to detect the bird. If we take the same model as small UAV, it will be so hard to differentiate the targets once both UAV and bird are in the same area. |
| Huawei, HiSilicon |  | It is debatable whether modelling birds or even trees are in the scope. Whatsoever, to save time, we can reuse the RCS model of the angular independency for the small size UAV without specifying the values to avoid the potential debate on the large or tiny birds. The values for the birds or other animals can be discussed in the evaluation stage if necessary. |
| CATT, CICTCI |  | Can be discussed in evaluation phase. Anyway it is proposed to reuse already existed model as small size UAV. |
| LGE |  | We think that it’s premature to support this proposal since we don’t have enough discussion on this issue. |
| Moderator |  | Change std to 3dB based on Ericsson’s tdoc |

[FL3][M] Proposal 4.8-1-rev1

The bistatic RCS of a bird is modelled by,

- Component A: uniformly distributed within range [-40, -20] dBsm

- Component B1: 0 dB

- Component B2: standard deviation of B2 is 3 dB

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson |  | Without Type-1 EO modelled in a simulation, how to fulfill the objective in SID ‘a common modelling framework capable of detecting and/or tracking the following example objects and to enable them to be distinguished from unintended objects’? Moreover, there would be nearly no type identification error and low mis-detection rate, which doesn’t reflect the reality which sensing products have to face. |
| Ericsson2 |  | In addition to unintended object, the SID also states clutter pattern. SID: The focus of the study is to define channel modelling aspects to support object detection and/or tracking (as per the SA1 meaning in TS 22.137). The study should aim at a common modelling framework capable of detecting and/or tracking the following example objects and to enable them to be distinguished from unintended objects: [omitted] - Define channel modelling details for sensing using 38.901 as a starting point, and taking into account relevant measurements, including: - modelling of sensing targets and background environment, including, for example (if needed by the above use cases), radar cross-section (RCS), mobility and clutter/scattering patterns; - spatial consistency. Clutter can be stochastic clutter (moving scatterers) or deterministic clutter (Type-1 EO). There is no discussion on vscatt and p of moving stochastic clusters, and therefore they cannot serve as unintended objects or generate comparable physical characteristics as sensing targets. Without modelling of Type-1 EO, the SID’s objectives of distinguishing targets from unintended objects and clutter/scattering patterns cannot be fulfilled, and the study is incomplete. |
| Moderator6 |  | From the Wednesday offline session, it is clear majority view is not to discuss EO type-2 in Rel-19. See Proposal 4.8-3 |

[Moderator’s note] RCS for tree

[FL3][M] Proposal 4.8-2

The bistatic RCS of a tree is modelled by,

- The values/pattern of A*B1 of the bistatic RCS is given by:

rcsdBθi,ϕi,θs,ϕs=Gmax-min12β-πβ3dB2, σmax

  - β is the bistatic angle between the incident ray and scattering ray within the plane of incident direction (θi,ϕi,) and scattering direction (θs,ϕs).

  - β3dB is 20 degree

  - Gmax is 23 dBsm

  - σmax is 13 dBsm

- Component B2: standard deviation of B2 is 3 dB

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE |  | More study is needed. |
| Ericsson |  | See comments to Proposal 4.8-1. |
| Vivo | No | In consideration of one meeting left, we postpone this study in next releases. |
| Huawei, HiSilicon |  | At every end of the study phase, we are hesitant to open more discussions on something even debatable whether it is in the scope. |
| LGE |  | We think that it’s premature to support this proposal since we don’t have enough discussion on this issue. |
| Ericsson |  | See comments to Proposal 4.8-1-rev1. The RCS model is given based on measurements from the following references. Y. L. C. de Jong and M. H. A. J. Herben, “A tree-scattering model for improved propagation prediction in urban microcells,” in IEEE Transactions on Vehicular Technology, vol. 53, no. 2, pp. 503-513, March 2004. Jackson et al, “The motion of trees in the wind: a data synthesis”, Biogeosciences, vol. 18, no. 13, pp. 4059-4072, 2021. J. Ko et al., “Measurements and Analysis of Radio Propagation at 28 GHz in Vegetated Areas of Typical Residential Environments,” in IEEE Transactions on Antennas and Propagation, vol. 68, no. 5, pp. 4149-4154, May 2020. |
| Moderator6 |  | From the Wednesday offline session, it is clear majority view is not to discuss EO type-2 in Rel-19. See Proposal 4.8-3 |

[FL4][M] Proposal 4.8-3 for conclusion

RCS of EO type-1 is not further studied in Rel-19.

[Moderator’s note] Agreed in Thursday online session

- Conclusion

Other than RCS for human, vehicle, AGV, UAV, no other RCS for other objects is introduced in Rel-19.

- Future studies are not precluded for adding RCS of other objects/sizes for modelling target or EO type-1, based on validation results from companies.

Collection on the setup for RCS measurement/simulation from companies

[Moderator’s note] Based the email discussion [Post-120-ISAC-01] and confirmation by Chair, we would create a document to collect details on the setup for RCS measurement/simulation (i.e. vehicle size, frequency, distance, etc). The file includes separate sheets for different target types. Please consider adding your information for measurement/simulation.

/Inbox/drafts/9.7(FS_Sensing_NR)/9.7.2 Channel Modelling/ Setup for RCS validation/

So far, ZTE and OPPO already input the related information.

If any additional comments, please provide it in following table

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | One question is whether and how to reflect this file in the CR. |
| Moderator | Welcome more views |
|  | No more comment here |

- [Closed][FL1][L] Proposal 4.9-1

The excel file which collects the setup for RCS measurement/simulation from all companies will be submitted with a tdoc number, and will be used as a reference for the CR to TR 38.901 for ISAC channel model.

| Company | Yes/No | Comments |
| --- | --- | --- |
| BUPT3 | Yes |  |
| Ericsson |  | Thanks for the initiative. If possible, we would like to add our background channel measurements. |
| Ericsson2 |  | Please consider improving the template file so that it can capture all measurements that have been contributed to the SI, including measurements of background and target channels, RCS of targets, RCS of type-1 and type-2 EO, Doppler signatures, etc. |

ISAC channel model

Reference TRs

Agreement In order to generate Tx-target link, target-Rx link and the background channel, the above table on reference TRs (excluding the already agreed part) is adopted for the mapping between reference TRs and a pair of nodes (STX, SRX, target) - Note: continue discussion for updating the table with RSU type UE - FFS: the generation of background channel based on reference TRs is subject to the addition of low-energy clusters 7; normal UE; aerial UE; Umi-AV, Uma-AV, and Rma-AV - TRP-aerial UE link of UMi-AV in Annex A and B of TR 36.777 by setting hBS =1.5m for FR1 - LOS probability is not reused, FFS new LOS probability - FFS pathloss model, shadowing fading - Working assumption: Reuse the channel model of scenario UMa-AV, UMi-AV, and RMa-AV of FR1 for FR2 - The corresponding parameter values in FR2 are used. 9; aerial UE; aerial UE; Umi-AV, Uma-AV, Rma-AV - TRP-aerial UE link of UMi-AV in Annex A and B of TR 36.777 by setting height of TRP equal to the height of the first aerial UE for FR1 - LOS probability is not reused, FFS new LOS probability - FFS pathloss model, shadowing fading, angular spread - Working assumption: Reuse the channel model of scenario UMa-AV, UMi-AV, and RMa-AV of FR1 for FR2 The corresponding parameter values in FR2 are used.

Summary on company views

Ericsson

- The LOS probability between an aerial UE and a normal UE is the following:

pLoS=Vmax(1-expkx+Ce901-expk90+Ce90)

With

x=180πarctanabs(haerial UE-hnormal UE)d2D

k=hnormal UE+akbkck

Ce=achnormal UE+bc

Vmax=aVhnormal UE+bV,   Vmax=min⁡(Vmax,  100)

with parameter values

|  | k: [ak, bk,ck] | Ce: [ac, bc] | Vmax: [aV, bV] |
| --- | --- | --- | --- |
| RMa | [4.9, 6.7, 2.6] | [0.19, 0] | [1.6 -17] |
| UMa, UMi | [-2.6, 6.6, 2.0] | [0.42, -6.7] | [1 -21] |

Ericsson

- For path loss and shadow fading of terrestrial UE-aerial UE link, for aerial UE height of 22.5m-35m, which is similar to the corresponding BS height, use terrestrial UE-BS link of UMa scenario in TR 38.901 for urban areas and that of RMa scenario for rural areas with the BS representing the aerial UE.

- For path loss and shadow fading of terrestrial UE-aerial UE link, for aerial UE heights that are lower than 22.5m, use terrestrial UE-BS link of UMi scenario in TR 38.901 for urban areas with the BS representing the aerial UE.

- For path loss and shadow fading of terrestrial UE-aerial UE link, for aerial UE heights that are higher than 35m, use terrestrial UE-BS link of UMa in TR 38.901 for urban areas and that of RMa scenario for rural areas with the BS representing the aerial UE.

[Moderator’s note] In RAN1 #120bis, we discussed the above Case 7 but cannot conclude detailed model on LOS probability, pathloss and shadowing. Ericsson suggests the following proposal to model UE-aerial UE link. Let’s check if it is agreeable.

[H] Proposal 5.1-1

- The LOS probability between an aerial UE and a normal UE is the following:

pLoS=Vmax(1-exp⁡(kx+Ce90)1-exp⁡(k90+Ce90)

With

x=180πarctanhaerial UE-hnormal UEd2D

k=hnormal UE+akbkck

Ce=achnormal UE+bc

Vmax=aVh+bV,   Vmax=min⁡(Vmax,  100)

with parameter values

|  | k: [ak, bk,ck] | Ce: [ac, bc] | Vmax: [aV, bV] |
| --- | --- | --- | --- |
| RMa | [4.9, 6.7, 2.6] | [0.19, 0] | [1.6 -17] |
| UMa, UMi | [-2.6, 6.6, 2.0] | [0.42, -6.7] | [1 -21] |

- For aerial UE heights similar to the corresponding macro BS height (25 m in Uma, 35 m in Rma), use the Uma or Rma model in TR 38.901 with the BS representing the aerial UE.

- For aerial UE heights that are higher than the corresponding macro BS height (25 m in Uma, 35 m in Rma), use the Uma or Rma model in TR 38.901 with the BS representing the aerial UE.

| Company | Yes/No | Comments |
| --- | --- | --- |
| InterDigital |  | We have some clarification questions. What is the value for “h”? Could the proponent indicate the reference documents used for derivation of pLoS? Do the last two bullets correspond to both pathloss and shadowing? |
| CMCC |  | We are ok with the proposed LOS probability model for case-7. In addition, according to the agreement in RAN1#120bis meeting, case-9 for aerial UE to aerial UE also has three FFS about LOS probability, shadow fading and angular spread. Here we present our modeling methodology for case-9 angular spread, with minor modifications on ZOA/ZOD generation based on TR 36.777 and TR 38.901. Either Option 1, Option 2, or Option 3 is fine for us. However, at least one option should be agreed to solve the angular spread issue. - Option 1 Reuse the procedure of TRP-aerial UE link generation defined in TR 36.777 by modifying the procedure of ZOA/ZOD generation defined in Step 7 of section 7.5 of TR 38.901 by the following procedure. For ZOA generation, equation (7.5-16) in TR 38.901, which is θn,ZOA=Xnθn,ZOA'+Yn+θZOA, is replaced by the following equation θn,ZOA=Xnθn,ZOA'+Yn+θZOA+μoffset,ZOA .. And equation (7.5-17) in TR 38.901 is not adopted in the LOS case. ZOD generation follows the same procedure as ZOA. But reuse the distribution of ZSA for ZSD, considering symmetry. μoffset,ZOA and μoffset,ZOD is defined as in Figure 2 and Figure 3 for aerial-UE-to-aerial-UE link of Rma and Uma/Umi respectively. And h is the average height of rooftop. Once the RX aerial Ues is below the rooftop, μoffset,ZOA=0 . Once the TX aerial Ues is below the rooftop, μoffset,ZOD=0. Definition of μoffset,ZOA and μoffset,ZOD for RMa aerial-UE-to-aerial-UE link Definition of μoffset,ZOA and μoffset,ZOD for UMa/UMi aerial-UE-to-aerial-UE link - Option 2 Reuse the procedure of TRP-aerial UE link generation defined in TR 36.777 by setting the lower aerial UE as a TRP of same height, with an additional procedure that all rays which depart or arrive from above are removed if LOS. - Option 3 Reuse the procedure of TRP-aerial UE link generation defined in TR 36.777 by setting the lower aerial UE as a TRP of the same height, with an additional procedure that all rays which depart or arrive from above are removed with a given probability. The probability is defined as equal to the LOS probability (either LOS probability defined for TRP-aerial UE or possible new formula given for aerial-UE-to-aerial-UE LOS probability) between these two Ues. |
| OPPO |  | We have the same question as InterDigital on definition/range of “h”. |
| Ericsson |  | Thanks for the comments. Please find the updates in red. pLoS=Vmax(1-expkx+Ce901-expk90+Ce90) Vmax=aVhnormal UE+bV The issue has been studied extensively in the following ITU contributions. The common methodology is to use digital maps of different scenarios to assess the LOS probability between a ground station and an airborne station. The results are presented as a closed-form expression for the LOS probability as a function of the ground station height and the elevation angle to the airborne station. For the purposes of case 7, we can equate the ground station with a normal UE and the airborne station with an aerial UE. People’s Republic of China, “Slant path clutter loss model update proposals based on latest working document 3K/335”, Document 3K/38-E (also available as Document 3M/65-E), ITU-R WP3 meetings, Denver, Colorado, USA, 29 May – 7 June 2024. Telefon AB – LM Ericsson, “Simulations and model proposal of LOS probabilities”, Document 3K/118-E (also available as Document 3J/107-E and Document 3M/151-E), ITU-R WP3 meetings, Geneva, 17-21 February 2025. For terrestrial UE height range, we suggest using the same applicability range of UE antenna height in 38.901, i.e., 1.5m~22.5m for Uma and Umi scenario and 1m~10m for Rma. The last two bullets address both pathloss and shadowing. |
| Moderator |  | Thanks for clarification from Ericsson and also the contributions. - The pathloss and shadow fading between an aerial UE and a normal UE are generated by - For sensing scenario Uma-AV, Umi-AV - For aerial UE height <= 22.5m, use terrestrial UE-BS link of UMi scenario in TR 38.901 with the BS representing the aerial UE - For aerial UE height > 22.5m, use terrestrial UE-BS link of UMa scenario in TR 38.901 with the BS representing the aerial UE - For sensing scenario Rma-AV - For aerial UE height <= 22.5m, FFS - For aerial UE height > 22.5m, use terrestrial UE-BS link of RMa scenario in TR 38.901 with the BS representing the aerial UE Alternatively, since the fast fading parameters are agreed to follow “TRP-aerial UE link of UMi-AV in Annex A and B of TR 36.777 by setting hBS =1.5m for FR1”, the same logic can be applied to large scale parameters too. |

[FL1][H] Proposal 5.1-1-rev1

- The LOS probability between an aerial UE and a normal UE is generated by:

pLoS=Vmax(1-expkx+Ce901-expk90+Ce90)

With

x=180πarctanabs(haerial UE-hnormal UE)d2D

k=hnormal UE+akbkck

Ce=achnormal UE+bc

Vmax=aVhnormal UE+bV,   Vmax=min⁡(Vmax,  100)

with parameter values

|  | k: [ak, bk,ck] | Ce: [ac, bc] | Vmax: [aV, bV] |
| --- | --- | --- | --- |
| RMa | [4.9, 6.7, 2.6] | [0.19, 0] | [1.6 -17] |
| UMa, UMi | [-2.6, 6.6, 2.0] | [0.42, -6.7] | [1 -21] |

- The pathloss and shadow fading between an aerial UE and a normal UE are generated using TRP-aerial UE link of UMi-AV in Annex A and B of TR 36.777 by setting hBS =1.5m for FR1

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE |  | It seems x is the elevation angle based on the proponent input. From the results, if aerial UE is also 1.5m, is the LOS probability always 0 without considering distance between two Ues ? even the two Ues are very closed to each other, it is still NLOS? From our view, the function should not only depend on elevation angle, but also depend on the d2D. It is noted that the LOS probability formula in current TR 38.901 is highly relying on the d2D. |
| Samsung |  | We would like to confirm whether the normal UE’s height is assumed to be 22.5 m only for Uma and Umi. P_LOS seems not workable when the normal UE’ height is 1.5 m especially (or less than 22.5 m) |
| InterDigital |  | We agree with ZTE that d2D should be considered for LOS probability. In our contribution (R1-2503753), In Proposal 8 and 10 we proposed to segment UAV heights into 3 segments and reuse equations in TR 36.777 which account for d2D distance. |
| Ericsson |  | For the LOS probability, please find below the updated proposal, with changes in red. pLoS=Vmax(1-exp-kx+Ce901-exp-k90+Ce90) With x=180πarctanmax(haerial UE-hnormal UE,0)d2D k=hnormal UE+akbkck Ce=achnormal UE+bc Vmax=aVhnormal UE+bV, Vmax=min⁡(Vmax, 100) with parameter values k: [ak, bk,ck]; Ce: [ac, bc]; Vmax: [aV, bV]. RMa; [4.9, 6.7, 2.6]; [0.19, 0]; [1.4, 74]. UMa, UMi; [-2.6, 6.6, 2.0]; [0.42, -6.7]; [0.15, 97]. pLoS is bounded within 0%, 100%. The second bullet is problematic. Neither 36.777 nor 38.901 supports indoor TRPs in the Umi or Umi-AV scenarios. However, the majority of the normal Ues are indoors in the Umi communication scenario, and will experience a building penetration loss towards any outdoor aerial UAV. This would not be captured by the second bullet. Additionally, the consequences of “setting hBS =1.5m for FR1” are unclear. The path loss models are not applicable for such heights. A more workable solution is to acknowledge that 38.901 already provides path loss and shadow fading model between all indoor and outdoor normal Ues and TRPs at different heights: - Uma between a normal UE and an outdoor Tx (TRP) at 25 m height, i.e. above rooftop - Umi between a normal UE and an outdoor Tx (TRP) at 10 m height, i.e. below rooftop - Rma between a normal UE and an outdoor Tx (TRP) at 35 m height, i.e. above rooftop We can let the aerial UE be represented by a Tx (TRP) in a similar height condition, i.e. above or below rooftops: - The pathloss and shadow fading between an aerial UE and a normal UE are generated by - For sensing scenario Uma-AV, Umi-AV - For aerial UE height <= 22.5m, use terrestrial UE-BS link of UMi scenario in TR 38.901 with the BS representing the aerial UE - For aerial UE height > 22.5m, use terrestrial UE-BS link of UMa scenario in TR 38.901 with the BS representing the aerial UE - For sensing scenario Rma-AV - For aerial UE height <= 22.5m, FFS - For aerial UE height > 22.5m, use terrestrial UE-BS link of RMa scenario in TR 38.901 with the BS representing the aerial UE |
| ZTE |  | For the first bullet, we did the simulation with assumption of d2D = 100m for Uma based on Ericsson’s update formula, the results are as follows. It is observed that when aerial UE is not high enough, the LOS probability is even a negative value. Also, LOS probability can never be 100% even aerial UE is very high. - d_2D = 100m - d_2D = 5m For the case when normal UE is close to aerial UE (5m is assumed), it can seen the LOS probability is still very low if the aerial UE is not high enough. The result seems not aligned with the reality in which LOS should exist if two Ues are closed enough. We suggest discussing FL proposal 5.1-2 first. Case 7 is just one extreme case of case 9. |
| Moderator3 |  | The proponent companies clarified the issue together using offline. Case 7 can be considered as special case of Case 9. So far two options are possible for Case 9. Low-UAV; Mid-UAV; High-UAV. Terrestrial UE; Option 1: Table 7.4.2-1 in TR 38.901 Option 2: agreement for Proposal 5.1-1-rev1; Option 1: Table B-1 in TR 36.777 for BS to mid UAV region Option 2: agreement for Proposal 5.1-1-rev1; Option 1: Table 6.6.1-1 in TR 38.811 (NTN model) Option 2: agreement for Proposal 5.1-1-rev1. From the offline discussion, Proposal 5.1-1-rev1 has certain problem, e.g., the LOS probability will be 0 if low UE height, e.g., 1.5m which doesn’t match with the reality. The impact of d2D is not as expectation too. Therefore Option 1 is proposed. |

[FL2][H] Proposal 5.1-1-rev2

To generate the channel between an aerial UE and a normal UE,

- The LOS probability is generated by:

| Low-UAV | Mid-UAV | High-UAV |
| --- | --- | --- |
| Table 7.4.2-1 in TR 38.901 | Table B-1 in TR 36.777 for BS to mid UAV region | Table 6.6.1-1 in TR 38.811 (NTN model) |

- The pathloss and shadow fading are generated using TRP-aerial UE link of UMi-AV in Annex A and B of TR 36.777 by setting hBS =1.5m for FR1

Note:

- The height ranges of low-UAV, Mid-UAV and High-UAV are defined as in Table B-1: LOS probability in TR 36.777

- The second height range for UMi-AV is further divided into 2 regions, i.e., [22.5, 100] and [100, 300]

[Moderator’s note] revised in Tuesday offline session

[FL2][H] Proposal 5.1-1-rev3

To generate the channel between an aerial UE and a normal UE,

- The LOS probability is generated by:

| Low-UAV | Mid-UAV | High-UAV |
| --- | --- | --- |
| UMi in Table 7.4.2-1 in TR 38.901 for UMi-AV/UMa-AV/RMa-AV | UMi-AV, UMa-AV and RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region | UMi-AV, UMa-AV and RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region |

- The pathloss and shadow fading are generated using TRP-aerial UE link of UMi-AV in Annex A and B of TR 36.777 by setting hBS =1.5m for FR1

Note:

- The height ranges of low-UAV, Mid-UAV and High-UAV are defined following the applicability range in terms of aerial UE height in Table B-1: LOS probability in TR 36.777

- The second height range for UMi-AV is further divided into 2 regions, i.e., [22.5, 100] and [100, 300]

[Moderator’s note] Agreed in Wednesday online session. Resolving bracket is still necessary.

- Agreement

To generate the channel between an aerial UE and a normal UE,

- The LOS probability is generated by:

| Low-UAV | Mid-UAV | High-UAV |
| --- | --- | --- |
| UMi in Table 7.4.2-1 in TR 38.901 for UMi-AV/UMa-AV/RMa-AV | UMi-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region | UMi-AV in Table B-1 in TR 36.777 for BS to [high] UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region |

- The pathloss and shadow fading are generated using TRP-aerial UE link of UMi-AV in Annex A and B of TR 36.777 by setting hBS =1.5m for FR1

Note:

- The height ranges of low-UAV, Mid-UAV and High-UAV are defined following the applicability range in terms of aerial UE height in Table B-1: LOS probability in TR 36.777

- The second height range for UMi-AV is further divided into 2 regions, i.e., [22.5, 100] and [100, 300] for mid-UAV and high-UAV, respectively.

[FL3][H] Proposal 5.1-1-rev4

Remove the brackets of [RMa-AV] in the agreements for channel generation between an aerial UE and a normal UE.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson2 |  | In the table of LOS probability in 36.777, for RMa-AV scenario, mid-UAV's height is 10~40m, which corresponds to both low UAV height 1.5~22.5m and mid UAV heigh 22.5~300m in UMi-AV scenario. If [] are removed, a low UAV region should be added to address mid UAV height in the range of 10~22.5 as follows. UMi-AV in Table B-1 in TR 36.777 for BS to low and mid UAV region for RMa-AV The following agreement also needs some updates, since mid UAV height range [10, 40m] in RMa-AV is divided into [10m, 22.5m] and [22.5m, 40m] regions. Agreement The existing horizontal correlation distance in Table 7.6.3.1-2 in TR38.901 is used as the correlation distance for 3D spatial consistency for ISAC channel at least for UAV scenario, within same ‘Applicability range in terms of aerial UE height (defined in 36.777)’. Therefore, the crossed-out sentence is preferred, i.e., using RMa BS-mid UAV for normal UE-mid UAV, as follows Mid-UAV; High-UAV. UMi-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region; UMi-AV in Table B-1 in TR 36.777 for BS to [high] mid UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region. |
| Moderator6 |  | Updated in Wednesday offline session |

[FL4][H] Proposal 5.1-1-rev5

Update the agreements on LOS probability calculation for channel between an aerial UE and a normal UE as follows.

| Low-UAV | Mid-UAV | High-UAV |
| --- | --- | --- |
| UMi in Table 7.4.2-1 in TR 38.901 for UMi-AV/UMa-AV/RMa-AV | UMi-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region for RMa-AV | UMi-AV in Table B-1 in TR 36.777 for BS to high UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to high UAV region for RMa-AV |

[Moderator’s note] Agreed in Thursday online session

- Agreement

Update the agreements on LOS probability calculation for channel between an aerial UE and a normal UE as follows.

| Low-UAV | Mid-UAV | High-UAV |
| --- | --- | --- |
| UMi in Table 7.4.2-1 in TR 38.901 for UMi-AV/UMa-AV/RMa-AV | UMi-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region for RMa-AV | UMi-AV in Table B-1 in TR 36.777 for BS to high UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to high UAV region for RMa-AV |

Agreement

Update the agreements on LOS probability calculation for channel between two aerial UE as follows.

|  | Low-UAV | Mid-UAV | High-UAV |
| --- | --- | --- | --- |
| Low-UAV | UMi in Table 7.4.2-1 in TR 38.901 for UMi-AV/UMa-AV/RMa-AV | UMi-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region for RMa-AV | UMi-AV in Table B-1 in TR 36.777 for BS to high UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to high UAV region for RMa-AV |
| Mid-UAV | UMi-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region for RMa-AV | UMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region | 1 |
| High-UAV | UMi-AV in Table B-1 in TR 36.777 for BS to high UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to high UAV region for RMa-AV | 1 | 1 |

[Moderator’s note] Another open issue on the large scale parameters for link between two aerial Ues (Case 9). To be honest, there is no validation at all. The only possible approach for us is to reuse certain existing proposals/solutions from other cases. The following proposal is made the by moderator.

- The LOS probability between two aerials Ues are generated by updating Table B-1: LOS probability in TR 36.777, with following updates (marked as red in the Table)

  - Condition is revised to be based max(h1, h2), where h1, h2 are the height of the two aerial Ues

  - The formula for the first range of height is changed to that agreed for Proposal 5.1-1

    - In the third height range of each scenario, LOS probability is 100%

    - In the second height range, the proposal is to reuse 36.777, which is the only reference we have

    - In the first height range, we may use the agreement for Proposal 5.1-1 since it targets low Tx/Rx

Please check if the proposal can be agreeable or any suggested update are welcome? However, please being constructive since we are short of any validations.

Note: using UAV to sense another UAV is not a typical case, at least in the quite early stage that we even never study using TRP to sense UAV yet. If a solution cannot be agreeable, the moderator would suggest we remove this case from Rel-19.

[FL1][H] Proposal 5.1-2

To generate the channel between a first aerial UE with height h1 and a second aerial UE with height h2, h1<=h2,

- The LOS probability between the two aerial Ues is generated by:

| Scenario | LOS probability (distance is in meters) | Applicability range in terms of aerial UE height |
| --- | --- | --- |
| RMa-AV | According to Table 7.4.2-1 of [4] using the formula of RMa Use same formula as what can be agreed for Proposal 5.1-1 | 1.5m≤max(h1, h2)≤10m |
|  | PLOS=1,d2D≤d1d1d2D+exp-d2Dp11-d1d2D,d2D>d1 p1=max15021log10(hUT)-16053,1000 d1=max1350.8log10(hUT)-1602,18, see Note 2. | 10m<max(h1, h2)≤40m |
|  | 100% | 40m<max(h1, h2)≤300m |
| UMa-AV | According to Table 7.4.2-1 of [4] using the formula of UMa Use same formula as what can be agreed for Proposal 5.1-1 | 1.5m≤max(h1, h2)≤22.5m |
|  | PLOS=1,d2D≤d1d1d2D+exp-d2Dp11-d1d2D,d2D>d1 p1=4300log10(hUT)-3800 d1=max460log10(hUT)-700,18, see Note 2. | 22.5m<max(h1, h2)≤100m |
|  | 100% | 100m<max(h1, h2)≤300m |
| UMi-AV | According to Table 7.4.2-1 of [4] using the formula of UMi Use same formula as what can be agreed for Proposal 5.1-1 | 1.5m≤max(h1, h2)≤22.5m |
|  | PLOS=1,d2D≤d1d1d2D+exp-d2Dp11-d1d2D,d2D>d1 p1=233.98log10(hUT)-0.95 d1=max294.05log10(hUT)-432.94,18, see Note 2. | 22.5m<max(h1, h2)≤100m |
|  | 100% | 100m<max(h1, h2)≤300m |
| Note 1: The LOS probability is derived assuming antenna heights of 35m for Rma-AV, 25m for Uma-AV, and 10m for Umi-AV. Note 2: d1 is given in units of meters. |  |  |

- The pathloss and shadow fading between two aerial Ues are generated using TRP-aerial UE link of UMi-AV in Annex A and B of TR 36.777 by setting height of TRP equal to the height of the first aerial UE.

| Company | Yes/No | Comments |
| --- | --- | --- |
| BUPT3 | Yes |  |
| InterDigital |  | We agree with the principle of the proposed approach based on height segments for LOS probability between two aerial Ues. However, we propose to use the equations proposed in our contribution (R1-2503753), in Proposal 9 and 11 we proposed to model the LoS probability and the corresponding pathloss and shadow fading model also based on the height segments. The difference between the moderator proposal and our proposal is in the height function to choose the equations corresponding to a height region, i.e., our model is a function of (h1 and h2) and moderator proposal is a function of max(h1, h2). The implication of the difference is that, for a high UAV region to a low UAV region (e.g., h2>>h1), based on the moderator proposal, the LoS probability is always 1, whereas based on our model, it is angle based, similar to the NTN model in TR 38.811. |
| Ericsson |  | What is the reasoning for the second bullet? The first aerial UE’s height may not be similar to Umi BS’s height. |
| ZTE |  | For the second column of the table, we suggest completely reuse 36.777 |
| Moderator3 |  | The proponent companies clarified the issue together using offline. So far two options are possible for Case 9. Low-UAV; Mid-UAV; High-UAV. Low-UAV; Option 1: Table 7.4.2-1 in TR 38.901 Option 2: agreement for Proposal 5.1-1-rev1; Option 1: Table B-1 in TR 36.777 for BS to mid UAV region Option 2: agreement for Proposal 5.1-1-rev1; Option 1: Table 6.6.1-1 in TR 38.811 (NTN model) Option 2: agreement for Proposal 5.1-1-rev1. Mid-UAV; Option 1: Table B-1 in TR 36.777 for BS to mid UAV region Option 2: agreement for Proposal 5.1-1-rev1; Option 1: Table B-1 in TR 36.777 for BS to mid UAV region Option 2: agreement for Proposal 5.1-1-rev1; 1. High-UAV; Option 1: Table 6.6.1-1 in TR 38.811 (NTN model) Option 2: agreement for Proposal 5.1-1-rev1; 1; 1. From the offline discussion, Proposal 5.1-1-rev1 has certain problem, e.g., the LOS probability will be 0 if low UE height, e.g., 1.5m which doesn’t match with the reality. The impact of d2D is not as expectation too. Therefore Option 1 is proposed. I add a differentiation on the two aerial Ues. The first aerial UE is defined as the aerial UE which has smaller different from BS height. |

[FL2][H] Proposal 5.1-2-rev1

To generate the channel between a first aerial UE with height h1 and a second aerial UE with height h2, abs(h1-hBS) <= abs(h2-hBS),

- The LOS probability between the two aerial Ues is generated by:

|  | Low-UAV | Mid-UAV | High-UAV |
| --- | --- | --- | --- |
| Low-UAV | Table 7.4.2-1 in TR 38.901 | Table B-1 in TR 36.777 for BS to mid UAV region | Table 6.6.1-1 in TR 38.811 (NTN model) |
| Mid-UAV | Table B-1 in TR 36.777 for BS to mid UAV region | Table B-1 in TR 36.777 for BS to mid UAV region | 1 |
| High-UAV | Table 6.6.1-1 in TR 38.811 (NTN model) | 1 | 1 |

- The pathloss and shadow fading between two aerial Ues are generated using TRP-aerial UE link of UMi-AV in Annex A and B of TR 36.777 by setting height of TRP equal to the height of the first aerial UE.

Note:

- The height ranges of low-UAV, Mid-UAV and High-UAV are defined as in Table B-1: LOS probability in TR 36.777

- The second height range for UMi-AV is further divided into 2 regions, i.e., [22.5, 100] and [100, 300]

[Moderator’s note] revised in Tuesday offline session

[FL2][H] Proposal 5.1-2-rev2

To generate the channel between a first aerial UE with height h1 and a second aerial UE with height h2, abs(h1-hBS) <= abs(h2-hBS),

- The LOS probability between the two aerial Ues is generated by:

|  | Low-UAV | Mid-UAV | High-UAV |
| --- | --- | --- | --- |
| Low-UAV | UMi in Table 7.4.2-1 in TR 38.901 for UMi-AV/UMa-AV/RMa-AV | UMi-AV, UMa-AV and RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region | UMi-AV, UMa-AV and RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region |
| Mid-UAV | UMi-AV, UMa-AV and RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region | UMi-AV, UMa-AV and RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region | 1 |
| High-UAV | UMi-AV, UMa-AV and RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region | 1 | 1 |

- The pathloss and shadow fading between two aerial Ues are generated using TRP-aerial UE link of UMi-AV in Annex A and B of TR 36.777 by setting height of TRP equal to the height of the first aerial UE.

Note:

- The height ranges of low-UAV, Mid-UAV and High-UAV are defined following the applicability range in terms of aerial UE height in Table B-1: LOS probability in TR 36.777

- The second height range for UMi-AV is further divided into 2 regions, i.e., [22.5, 100] and [100, 300]

[Moderator’s note] Agreed in Wednesday online session. Resolving bracket is still necessary.

- Agreement

To generate the channel between a first aerial UE with height h1 and a second aerial UE with height h2, abs(h1-hBS) <= abs(h2-hBS),

- The LOS probability between the two aerial Ues is generated by:

|  | Low-UAV | Mid-UAV | High-UAV |
| --- | --- | --- | --- |
| Low-UAV | UMi in Table 7.4.2-1 in TR 38.901 for UMi-AV/UMa-AV/RMa-AV | UMi-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region | UMi-AV in Table B-1 in TR 36.777 for BS to [high] UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region |
| Mid-UAV | UMi-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region | UMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region | 1 |
| High-UAV | UMi-AV in Table B-1 in TR 36.777 for BS to [high] UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region | 1 | 1 |

- The pathloss and shadow fading between two aerial Ues are generated using TRP-aerial UE link of UMi-AV in Annex A and B of TR 36.777 by setting height of TRP equal to the height of the first aerial UE.

Note:

- The height ranges of low-UAV, Mid-UAV and High-UAV are defined following the applicability range in terms of aerial UE height in Table B-1: LOS probability in TR 36.777

- The second height range for UMi-AV is further divided into 2 regions, i.e., [22.5, 100] and [100, 300] for mid-UAV and high-UAV, respectively.

[FL3][H] Proposal 5.1-2-rev3

Remove the brackets of [RMa-AV] in the agreements for channel generation between two aerial Ues.

[Moderator’s note] revised in Wednesday offline session

[FL4][H] Proposal 5.1-2-rev4

Update the agreements on LOS probability calculation for channel between two aerial UE as follows.

|  | Low-UAV | Mid-UAV | High-UAV |
| --- | --- | --- | --- |
| Low-UAV | UMi in Table 7.4.2-1 in TR 38.901 for UMi-AV/UMa-AV/RMa-AV | UMi-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region for RMa-AV | UMi-AV in Table B-1 in TR 36.777 for BS to high UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to high UAV region for RMa-AV |
| Mid-UAV | UMi-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region for RMa-AV | UMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region | 1 |
| High-UAV | UMi-AV in Table B-1 in TR 36.777 for BS to high UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to high UAV region for RMa-AV | 1 | 1 |

[Moderator’s note] Agreed in Thursday online session

- Agreement

Update the agreements on LOS probability calculation for channel between two aerial UE as follows.

|  | Low-UAV | Mid-UAV | High-UAV |
| --- | --- | --- | --- |
| Low-UAV | UMi in Table 7.4.2-1 in TR 38.901 for UMi-AV/UMa-AV/RMa-AV | UMi-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region for RMa-AV | UMi-AV in Table B-1 in TR 36.777 for BS to high UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to high UAV region for RMa-AV |
| Mid-UAV | UMi-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region for RMa-AV | UMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region | 1 |
| High-UAV | UMi-AV in Table B-1 in TR 36.777 for BS to high UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to high UAV region for RMa-AV | 1 | 1 |

[Moderator’s note] RSU type UE is agreed in 9.7.1. Please check the following proposal on reference TRs for RSU type UE.

[FL1][H] Proposal 5.1-3

- In order to generate Tx-target link, target-Rx link and the background channel between a RSU-type UE and another node (TRP, pedestrian UE, vehicle UE, RSU-type UE), the following reference TRs are adopted

| Case | Tx/Rx | Rx/Tx | Existing TRs as starting point |
| --- | --- | --- | --- |
|  | TRP | RSU-type UE | Highway and Urban grid - B2R link in section 6 of TR 37.885 |
|  | RSU-type UE | normal UE | Highway and Urban grid - V2V link in section 6 of TR 37.885, with antenna height at RSU is 5m |
|  | RSU-type UE | RSU-type UE | Highway and Urban grid - V2V link in section 6 of TR 37.885, with antenna height at RSU is 5m |
|  | RSU-type UE | vehicle UE | Highway and Urban grid - V2V link in section 6 of TR 37.885, with antenna height at RSU is 5m |

| Company | Yes/No | Comments |
| --- | --- | --- |
| vivo | Yes |  |
| CATT, CICTCI | Yes |  |
| OPPO | Yes |  |
| Samsung | Yes |  |
| DOCOMO | Yes |  |
| Lenovo | Yes |  |
| Spreadtrum | Yes |  |
| BUPT3 | Yes |  |
| PCL | Yes |  |
| Moderator4 |  | Agreed in Tuesday online session |

- Agreement

In order to generate Tx-target link, target-Rx link and the background channel between a RSU-type UE and another node (TRP, pedestrian UE, vehicle UE, RSU-type UE), the following reference TRs are adopted

| Case | Tx/Rx | Rx/Tx | Existing TRs as starting point |
| --- | --- | --- | --- |
|  | TRP | RSU-type UE | Highway and Urban grid - B2R link in section 6 of TR 37.885 |
|  | RSU-type UE | normal UE | Highway and Urban grid - V2V link in section 6 of TR 37.885, with antenna height at RSU is 5m |
|  | RSU-type UE | RSU-type UE | Highway and Urban grid - V2V link in section 6 of TR 37.885, with antenna height at RSU is 5m |
|  | RSU-type UE | vehicle UE | Highway and Urban grid - V2V link in section 6 of TR 37.885, with antenna height at RSU is 5m |

Background channel for monostatic

Agreement To generate the parameters (in the steps before concatenation), the large-scale parameters and the small-scale parameters used to generate the Tx-target link are respectively the same as that of the target-Rx link for monostatic sensing, where departure angle on one link and arrival angle on the other link are reciprocal. - FFS: whether this applies to initial phase

[Moderator’s note] In RAN1 #120bis, the above agreement is made with basic intention to have reciprocity for the Tx-target link and target-Rx link. Initial random phase for the two links should have same property otherwise it breaks the reciprocity. Please check the following proposal.

[FL1][H] Proposal 5.2-1

- The initial random phase (generated in Step 10, section 7.5, TR38.901) is the same for the same ray in Tx-target link and target-Rx link of a target for monostatic sensing.

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| LGE |  | We wonder if the reciprocity can be applied also to the initial random phase. |
| Nokia, NSB | Yes |  |
| BUPT | Yes |  |
| CMCC | Yes | Modeling two different initial phase with uniform distribution is equivalent to modeling a single uniform distributed initial phase for both Tx-target link and target-Rx link. |
| CATT, CICTCI |  | Maybe not strong concern, but we have the same view/question as LGE |
| OPPO | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Apple | Yes |  |
| MediaTek | Yes |  |
| Samsung |  | We have similar view with LGE and CATT. It’s difficult to find reciprocity relationship between random phases |
| Spreadtrum | Yes |  |
| Ericsson | Yes |  |
| QC | Yes |  |
| PCL | Yes |  |
| Moderator4 |  | Agreed in Tuesday online session |

- Agreement

The initial random phase (generated in Step 10, section 7.5, TR38.901) is the same for the same ray in Tx-target link and target-Rx link of a target for monostatic sensing.

[Moderator’s note] In [Post-120bis-ISAC-02], the parameters for UT monostatic sensing for Uma-AV is agreed. We still need values for parameters for UT monostatic sensing for Umi-AV and Rma-AV.

[H] Question 5.2-2

Please inputs on the remaining values of parameters for UT monostatic sensing for Umi-AV and Rma-AV with aerial UE as sensing transmitter or receiver.

| Scenario |  | UT monostatic sensing |  |  |
| --- | --- | --- | --- | --- |
|  |  | Uma-AV | UMi-AV | RMa-AV |
| Distribution of 2D distance between Tx and reference points | α1 | value agreed | [] | [] |
|  | β1 | value agreed | [] | [] |
|  | c1 | value agreed | [] | [] |
| Distribution of height of reference points | α2 | value agreed | [] | [] |
|  | β2 | value agreed | [] | [] |
|  | c2 | value agreed | [] | [] |

| Company | Comments |
| --- | --- |
| ZTE | For Rma-AV scenario, we have following fitting results for the parameters of UAV and RSU-type UE mono static: Parameters; Fitting models. α1; α1=4.423+0.001926h. β1; β1=13.8467+0.6547h. c1; c1=3.864+0.1538h. α2; α2=1.4231+0.00192h. β2; β2=11.7157-0.00538h. c2; c2=2.6541-0.003851h. The verification results are shown as following: - Verification: TX/RX height = 6 m Table 3.1-x Parameters for reference points in RMa scenario (TX/RX height=6m) Number of reference points; 3. Distribution of 2D distance between Tx and reference points drp_2D_n~Γα1, β1+c1; 4.4346. 0.1286. 4.7688. Distribution of height of reference points hrp_n~Γα2, β2+c2; 1.4346. 0.5940. 2.6756. - TX/RX height = 20 m Table 3.1-x Parameters for reference points in RMa scenario (TX/RX height=20m) Number of reference points; 3. Distribution of 2D distance between Tx and reference points drp_2D_n~Γα1, β1+c1; 4.4615. 0.0590. 6.9220. Distribution of height of reference points hrp_n~Γα2, β2+c2; 1.4615. 0.6219. 2.7257. - TX/RX height = 40 m Table 3.1-x Parameters for reference points in RMa scenario (TX/RX height=40m) Number of reference points; 3. Distribution of 2D distance between Tx and reference points drp_2D_n~Γα1, β1+c1; 4.5000. 0.0333. 9.9980. Distribution of height of reference points hrp_n~Γα2, β2+c2; 1.4999. 0.6664. 2.7973. - TX/RX height = 60 m Table 3.1-x Parameters for reference points in RMa scenario (TX/RX height=60m) Number of reference points; 3. Distribution of 2D distance between Tx and reference points drp_2D_n~Γα1, β1+c1; 4.5386. 0.0232. 13.0740. Distribution of height of reference points hrp_n~Γα2, β2+c2; 1.5383. 0.7179. 2.8690. - TX/RX height = 80 m Table 3.1-x Parameters for reference points in RMa scenario (TX/RX height=80m) Number of reference points; 3. Distribution of 2D distance between Tx and reference points drp_2D_n~Γα1, β1+c1; 4.5771. 0.0178. 16.1500. Distribution of height of reference points hrp_n~Γα2, β2+c2; 1.5767. 0.7780. 2.9406. - TX/RX height = 100 m Table 3.1-x Parameters for reference points in RMa scenario (TX/RX height=100m) Number of reference points; 3. Distribution of 2D distance between Tx and reference points drp_2D_n~Γα1, β1+c1; 4.6156. 0.0144. 19.2260. Distribution of height of reference points hrp_n~Γα2, β2+c2; 1.6151. 0.8491. 3.0122. - TX/RX height = 140 m Table 3.1-x Parameters for reference points in RMa scenario (TX/RX height=140m) Number of reference points; 3. Distribution of 2D distance between Tx and reference points drp_2D_n~Γα1, β1+c1; 4.6926. 0.0105. 25.3780. Distribution of height of reference points hrp_n~Γα2, β2+c2; 1.6919. 1.0390. 3.1554. - TX/RX height = 180 m Table 3.1-x Parameters for reference points in RMa scenario (TX/RX height=180m) Number of reference points; 3. Distribution of 2D distance between Tx and reference points drp_2D_n~Γα1, β1+c1; 4.7697. 0.0082. 31.5300. Distribution of height of reference points hrp_n~Γα2, β2+c2; 1.7687. 1.3382. 3.2987. - TX/RX height = 220 m Table 3.1-x Parameters for reference points in RMa scenario (TX/RX height=220m) Number of reference points; 3. Distribution of 2D distance between Tx and reference points drp_2D_n~Γα1, β1+c1; 4.8467. 0.0068. 37.6820. Distribution of height of reference points hrp_n~Γα2, β2+c2; 1.8455. 1.8793. 3.4419. - TX/RX height = 260 m Table 3.1-x Parameters for reference points in RMa scenario (TX/RX height=260m) Number of reference points; 3. Distribution of 2D distance between Tx and reference points drp_2D_n~Γα1, β1+c1; 4.9238. 0.0057. 43.8340. Distribution of height of reference points hrp_n~Γα2, β2+c2; 1.9223. 3.1556. 3.5852. - TX/RX height = 300 m Table 3.1-x Parameters for reference points in RMa scenario (TX/RX height=300m) Number of reference points; 3. Distribution of 2D distance between Tx and reference points drp_2D_n~Γα1, β1+c1; 5.0008. 0.0050. 49.9860. Distribution of height of reference points hrp_n~Γα2, β2+c2; 1.9991. 9.8328. 3.7284. |
| Moderator | Thanks for inputs on parameter values from ZTE and BUPT, the following proposal is made accordingly. Same notes are copied from early agreement on Uma-AV |

[FL1][H] Proposal 5.2-2

For Umi-AV and Rma-AV with aerial UE as sensing transmitter or receiver, the values of parameters to generate background channel for UT monostatic sensing are provided in the following table

| Scenario |  | UT monostatic sensing |  |
| --- | --- | --- | --- |
|  |  | UMi-AV | RMa-AV |
| Distribution of 2D distance between Tx and reference points | α1 | 0.0156h+5.5399 | 4.423+0.001926h |
|  | β1 | 40.4517/(h+254.6318) | 13.8467+0.6547h |
|  | c1 | 0.0140h+15.1184 | 3.864+0.1538h |
| Distribution of height of reference points | α2 | 0.0123h+11.9569 | 1.4231+0.00192h |
|  | β2 | 17.8047/(h-0.2202) | 11.7157-0.00538h |
|  | c2 | 0.0532h-0.0120 | 2.6541-0.003851h |

Note 1: Distributions of height and distance of reference point are not subject to geographical constraints on TRP for the corresponding deployment scenario.

Note 2: The reference points for generating the UT monostatic background channel have the same velocity as UT.

Note 3: In the UT monostatic sensing in Uma and Umi scenario, the ZOD offset should be set as 0

| Company | Yes/No | Comments |
| --- | --- | --- |
| BUPT3 | Yes | The details of the Umi-AV scenario are presented in BUPT’s proposal [R1-2503859]. |
| Moderator4 |  | Agreed in Tuesday online session |

- Agreement

For Umi-AV and Rma-AV with aerial UE as sensing transmitter or receiver, the values of parameters to generate background channel for UT monostatic sensing are provided in the following table

| Scenario |  | UT monostatic sensing |  |
| --- | --- | --- | --- |
|  |  | UMi-AV | RMa-AV |
| Distribution of 2D distance between Tx and reference points | α1 | 0.0156h+5.5399 | 4.423+0.001926h |
|  | β1 | 40.4517/(h+254.6318) | 13.8467+0.6547h |
|  | c1 | 0.0140h+15.1184 | 3.864+0.1538h |
| Distribution of height of reference points | α2 | 0.0123h+11.9569 | 1.4231+0.00192h |
|  | β2 | 17.8047/(h-0.2202) | 11.7157-0.00538h |
|  | c2 | 0.0532h-0.0120 | 2.6541-0.003851h |

Note 1: Distributions of height and distance of reference point are not subject to geographical constraints on TRP for the corresponding deployment scenario.

Note 2: The reference points for generating the UT monostatic background channel have the same velocity as UT.

Note 3: In the UT monostatic sensing in Uma and Umi scenario, the ZOD offset in the background channel should be set as 0

[Moderator’s note] The issue handling d3D is delayed as remaining issue from the email discussion [Post-120bis-ISAC-02]. The following options are available.

- Option 0: no scaling factor is applied to d3D

- Option 1: An offset is applied to d3D, i.e., d3D-c1

- Option 2: A scaling factor d_s is multiplied to d3D, i.e., d3D*d_s. d_s is a value within range [0, 1].

- Option 3: An offset is applied to d3D, i.e., d3D-c12+(c2-height_Tx)2

From the email discussions, all interested companies except one are OK with Option 0 or 1. The moderator would like to check if Option 3 can be possible compromise solution.

[H] Proposal 5.2-3

- To generate the background channel for monostatic sensing, ‘d3D-c12+(c2-height_Tx)2+Δτ’ is used to model the absolute delay between the Tx and each reference point.

| Company | Yes/No | Comments |
| --- | --- | --- |
| CATT, CICTCI |  | There is insufficient data to support that the method of correcting the values for d3D is universal. More measurement results in different scenarios need to be proposed. |
| OPPO |  | Ok in principle. But ‘d3D-c12+(c2-height_Tx)2+Δτ’ is mistakenly a summation over a distance and a delay, which has light of speed missing somewhere. |
| Huawei, HiSilicon |  | We prefer the option0, we are fine with this proposal as well. One clarification question is whether this proposal applies to UE mono-static sensing mode as well. It would be better to clarify it in the proposal. |
| Moderator |  | @CATT: there are some discussions based on the validation results regarding how to resolve the comments from Ericsson. We are trying to find some middle point for compromise. Thanks for always being flexible for progress from OPPO and Huawei. I understood the proposal is not your first preference. Updated as suggested by OPPO and huawei |

[H] Proposal 5.2-3-rev1

- To generate the background channel for TRP monostatic sensing and UT monostatic sensing, ‘d3D-c12+(c2-height_Tx)2c +Δτ’ is used to model the absolute delay between the Tx and each reference point.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson |  | We are generally fine with the new proposal. With some random values, it is possible that abs(BS height – height of reference point) is smaller than (BS height – c2), and d3D-c12+(c2-height_Tx)2 is a negative value. We suggest that the following. ‘max(0, d3D-c12+(c2-height_Tx)2c) +Δτ’ is used to model the absolute delay between the Tx and each reference point. |
| Moderator |  | @Ericsson: revised as suggested |

[FL1][H] Proposal 5.2-3-rev2

- To generate the background channel for TRP monostatic sensing and UT monostatic sensing, ‘maxd3D-c12+(c2-height_Tx)2c,0 +Δτ’ is used to model the absolute delay between the Tx and each reference point.

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| CATT, CICTCI |  | Thanks for explanation. We can live with it. |
| Ericsson | Yes |  |
| Moderator4 |  | Agreed in Tuesday online session |

- Agreement

To generate the background channel for TRP monostatic sensing and UT monostatic sensing, ‘maxd3D-c12+(c2-height_Tx)2c,0 +Δτ’ is used to model the absolute delay between the Tx and each reference point.

[Moderator’s note] In the email discussion [Post-120bis-ISAC-03], two sub-bullets under Step 4 for Clause 7.9.4.2 in the CR to Tr 38.901 are put in brackets.

- There should be no problem for the first sub-bullet

- For the second sub-bullet, there was a comment that ∆τ should be per RP. However, based on the early agreement that a separate ∆τ is generated for the background channel, the same ∆τ should be applied to the background channel

Agreement When absolute delay model Δτ is configured, it applies to all NLOS clusters in each of Tx-target and target-Rx links and background channel. - For bistatic sensing: Different values of Δτ are separately generated for the Tx-target link, target-Rx link and the background channel - For monostatic sensing: the same value of Δτ is used for Tx-target link and target-Rx link, and a different value of Δτ is separately generated for the background channel

[FL1][H] Proposal 5.2-4

- Remove the brackets for first sub-bullet under Step 4 for Clause 7.9.4.2 in the CR to TR 38.901.

- On the absolute delay of the background channel for monostatic sensing, the same ∆τ is generated and applied to the 3 channels between the STX/SRX and the 3 RPs.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | No for the second bullet | The existing absolute delay model in 38.901 defines correlation distance in horizontal plane. The channel for each reference point is generated with existing absolute delay procedures including partially correlated Δτ depending on the separation between the reference points. This is analogous to Δτ generated for each BS-UE channel. If correlation distance in Table 7.6.9-1 is conformed for mono-static background channel, the same Δτ used for the three reference points means they have the same level of correlation, which instead puts a restriction on their positions. Thus, ∆τ should be independently generated for the three reference points. Note the three reference points are very likely to have different heights. A new question is how to use the ‘correlation distance in the horizontal plane [m]’. |
| ZTE |  | For the second bullet, we are flexible to have the same or purely independent ∆τ for 3 RPs. |
| Moderator3 |  | Based on the Monday offline session, the three RP for monostatic background channel are independently generated. The same principle can be extended to ∆τ |

[FL2][H] Proposal 5.2-4-rev1

- Remove the brackets for first sub-bullet under Step 4 for Clause 7.9.4.2 in the CR to TR 38.901.

- On the absolute delay of the background channel for monostatic sensing, the same three ∆τ are independently generated and respectively applied to the 3 channels between the STX/SRX and the 3 RPs.

[Moderator’s note] Agreed in Wednesday online session.

- Agreement

- Remove the brackets for first sub-bullet under Step 4 for Clause 7.9.4.2 in the CR to TR 38.901.

- On the absolute delay of the background channel for both TRP and UE monostatic sensing, three ∆τ are independently generated and respectively applied to the 3 channels between the STX/SRX and the 3 RPs.

Power threshold for path dropping in target channel

[Moderator’s note] There were discussions on the motivation to have -40dB as threshold for path dropping in target in RAN1 #120bis, please check tdoc from Xiaomi and Apple. According to Xiaomi’s results,

For concatenation Option 0, with a probability of about 90% in LOS condition Case 2 and LOS condition Case 3

- with X = -25dB, the power drop ratio of indirect paths is up to 30%

- with X = -40dB, the power drop ratio of indirect paths is up to 5%.

| Path drop ratio in LOS condition Case 2 with X = -25dB | Path drop ratio in LOS condition Case 2 with X = -40dB |
| --- | --- |

[FL1][M] Proposal 5.3-1

- Power threshold for path dropping after concatenation is -40dB for target channel

| Company | Yes/No | Comments |
| --- | --- | --- |
| LGE |  | We think that the metric to decide the power threshold is not the ratio of dropping, but the impact to the sensing performance. Maybe one way is to reuse -25dB as default, and give freedom to companies to report the results with -40dB in simulation phase. |
| Moderator |  | @LGE: Thanks for comments. It would great if we agree on single value, either -40 (with bracket in early agreement) or -25. The reason for value -40 is to minimize the difference between the cases with or without path dropping. |
| Huawei, HiSilicon | Yes | We are OK for the -40dB or even without the path dropping. |
| Apple | Conditional | From a study of the huma outdoor scenario, we plot the CDFs of the delay spread, the RSRP, and the channel odelling complexity captured by the average number of channel taps for different thresholds compared with the full convolution option. -40 dB does show no difference from full convolution and results in a drop in complexity. -25dB does show a difference for the delay spread. Ideal compromise may be -30 dB but we can live with -40 dB. |
| Samsung | Yes |  |
| EURECOM | Yes |  |
| Ericsson |  | Power threshold for path dropping after concatenation can be up to implementation. |
| Moderator2 |  | To ease the comparison of evaluation results from companies, it is good to leave path dropping implementation. |
| QC | No | Do not specify any path dropping threshold |
| MediaTek |  | Ok with the latest comments from Moderator. |
| Moderator3 |  | Revised in Monday offline session |

[FL1][M] Proposal 5.3-1-rev1

- Power threshold for path dropping after concatenation is up to -40dB for target channel. Up to company to choose a value in the implementation

[Moderator’s note] Agreed in Tuesday online session

- Agreement

Power threshold for path dropping after concatenation is up to -40dB for target channel for option 3. Up to company to choose a value in the implementation.

Power threshold for path dropping after concatenation is up to -25dB for target channel for option 0. Up to company to choose a value in the implementation.

For calibrations for both option 0 and option 3, power threshold for path dropping after concatenation is -40dB for target channel.

Impact of height of target on LOS probability or pathloss

Agreement In sensing scenario UMi, UMa, RMa, if the height of a scattering point of target is less than 1.5m, for pathloss calculation, down-selection one of the options below: - Option 4: use PL1 in Table 7.4.1-1: Pathloss models in TR 38.901 - Option 5: use hUT 1.5 m for pathloss calculation

[Moderator’s note] In reality, the height of scattering point of a target may be less than 1.5m. However, some concern raised if we directly use the height of scattering point to calculate pathloss by extension of applicable range of existing pathloss formula.

  - Some companies have concern on the large calculated pathloss since hUT<1m results in negative breakpoint distance, PL2 in Table 7.4.1-1 Pathloss models in TR 38.901 will apply. Then, the proposal is to enforce the use of PL1

  - Some other companies have concern on extension of application range of pathloss formula without validation results.

  - Some companies comment that there is no much difference for pathloss around 1.5m (the assumed breakpoint distance should be positive in the comment)

Which option (4 vs. 5) is preferred?

[H] Question 5.4-1

Which option (4 vs. 5) is preferred?

  - Option 4: use PL1 in Table 7.4.1-1: Pathloss models in TR 38.901

  - Option 5: use hUT 1.5 m for pathloss calculation

| Company | Option 4/5 | Comments |
| --- | --- | --- |
| ZTE | 5 |  |
| LGE | 5 | We prefer option 5, as a single solution same as human case. But if option 4 is agreed, one question is whether we still need to fix the human height as 1.5m (previous agreement), because we have a solution for the height less than 1.5m with this proposal. |
| Nokia, NSB | Option 5 |  |
| BUPT | Option 5 | We recommend against modifying the application rules of TR 38.901 without proper validation. |
| Vivo | Option 5 |  |
| CATT, CICTCI | Option 5 | Although the probability of d3D being greater than dBP is relatively low, to ensure consistency in channel modeling for both communication and sensing, it is proposed to keep the pathloss calculation aligned. BTW, based on our calculation, dBP is usually larger than 1.5*ISD. Maybe no need to worry too much. |
| OPPO | Option 5 |  |
| Huawei, HiSilicon |  | When the path loss model of Umi, Uma and Rma scenario of TR38.901 is used for the target channel, our view is that only the PL1 is applied in the target channel irrespective of the breakpoint distance, or use hUT 1.5 m for the breakpoint distance calculation instead of pathloss calculation as in option 5. Similar issues arose for the V2B channel model in urban grid scenario before. The 37.885 provides a solution with the hE = 0.25m for calculate the breakpoint distance. We also would like to reuse this solution for the UMi, UMa, RMa scenario. Refer to the 37.885: Pathloss in V2B, P2B, B2R link is given as follows: - LOS propagation type is used for V2B and B2R links in the highway scenario. - LOS/NLOS propagation types are used for V2B, P2B and B2R links in the urban scenario and spatial consistency is maintained following procedure in Subclause 7.6.3.3 in [15]. - Derive propagation type based on probability formula - The effective environment height hE to 0.25m. |
| Moderator |  | After further offline checking, it seems Huawei proposal on using hUT 1.5 m for breakpoint distance (dBP) calculation can be agreeable. Therefore, please check the following proposal. I also add a note to clarify hUT 1.5 m is only for breakpoint calculation, while other parameters are still generated by the real height of the scattering point, e.g., d3D is calculated based on the real location/height of the scattering point. |

[FL1][H] Proposal 5.4-1

- In sensing scenario UMi, UMa, RMa, if the height of a scattering point of target is less than 1.5m, for pathloss calculation,

  - use hUT 1.5 m for breakpoint distance (dBP) calculation

  - Note: hUT 1.5 m is only used for dBP calculation. The exact h_UT of the scattering point is still used to determine all other parameters of ISAC channel, e.g., delay, AOD/ZOD/AOA/ZOA, etc.

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Samsung | Yes |  |
| LGE | Yes |  |
| DOCOMO | Yes |  |
| EURECOM | Yes |  |
| QC | Yes |  |
| MediaTek | Yes |  |
| Moderator4 |  | Agreed in Tuesday online session |

- Agreement

In sensing scenario UMi, UMa, if the height of a scattering point of target is less than 1.5m, for pathloss calculation,

  - use hUT 1.5 m for breakpoint distance (dBP) calculation

  - Note: hUT 1.5 m is only used for dBP calculation. The exact h_UT of the scattering point is still used to determine all other parameters of ISAC channel, e.g., delay, AOD/ZOD/AOA/ZOA, etc.

Absolute delay

[Moderator’s note] In RAN1 #120bis, we confirm the WA on absolute delay as agreement. However, we don’t have the values of parameters for Δτ for Urban grid, Highway, HST. Since the channel model for Urban grid, Highway, HST in the existing TRs are derived based on UMa and RMa, it is straightforward we could reuse the values of parameters for Δτ of UMa and RMa in such scenarios.

[FL1][H] Proposal 5.5-1

To generate the absolute delay model for sensing scenarios Urban grid, highway and HST, for both target channel and background channel

- For Urban grid, the values of parameters for Δτ of scenarios UMa are reused.

- For Highway, the values of parameters for Δτ of scenarios RMa and UMa are reused for FR1 and FR2 respectively.

- For HST, the values of parameters for Δτ of scenarios RMa and UMa are reused for FR1 and FR2 respectively.

Note: If significant problem is found by validation, we can change the distribution of Δτ

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| BUPT | Yes | This is a reasonable solution that aligns perfectly with the TR’s application rules. |
| Vivo | Yes |  |
| CATT, CICTCI | Yes |  |
| OPPO | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Apple | yes |  |
| MediaTek | Yes |  |
| Samsung | Yes |  |
| LGE | Yes |  |
| DOCOMO | Yes |  |
| Lenovo | Yes |  |
| Spreadtrum | Yes |  |
| Ericsson |  | Values of parameters for Δτ in 38.901 or 7~24GHz agreements are for BS-UE channel. There is no validation whether the values can be used for other links. If a company does measurements in future, can we update the values even after the SI is complete? |
| QC | Yes |  |
| Moderator3 |  | Adding one note in Monday offline session |

[FL1][H] Proposal 5.5-1-rev1

To generate the absolute delay model for sensing scenarios Urban grid, highway and HST, for both target channel and background channel

- For Urban grid, the values of parameters for Δτ of scenarios UMa are reused.

- For Highway, the values of parameters for Δτ of scenarios RMa and UMa are reused for FR1 and FR2 respectively.

- For HST, the values of parameters for Δτ of scenarios RMa and UMa are reused for FR1 and FR2 respectively.

Note: no measurements on Δτ of the 3 scenarios are submitted in Rel-19.

[Moderator’s note] Agreed in Tuesday online session

- Agreement

To generate the absolute delay model for sensing scenarios Urban grid, highway and HST, for both target channel and background channel

- For Urban grid, the values of parameters for Δτ of scenarios UMa are reused.

- For Highway, the values of parameters for Δτ of scenarios RMa and UMa are reused for FR1 and FR2 respectively.

- For HST, the values of parameters for Δτ of scenarios RMa and UMa are reused for FR1 and FR2 respectively.

Note: no measurements on Δτ of the 3 scenarios are submitted in Rel-19.

[Moderator’s note] As commented during the offline discussion, it needs clarification what is the absolute delay model for scenario Umi-AV, Uma-AV and Rma-AV?

[FL2][H] Proposal 5.5-2

To generate the absolute delay model for sensing scenarios Umi-AV, Uma-AV and Rma-AV, for both target channel and background channel,

- For the TRP-TRP link and TRP-UE link, the values of parameters for Δτ of scenarios Umi, Uma and RMa are respectively reused.

- For the UE-UE link, the values of parameters for Δτ of scenarios UMi are reused.

- For the TRP-UAV link, the values of parameters for Δτ of scenarios Umi, Uma and RMa are respectively reused.

- For the UE-UAV link, the values of parameters for Δτ of scenarios UMi are reused.

- For the UAV-UAV link, the values of parameters for Δτ of scenarios UMi are reused.

Note: no measurements on Δτ of the scenarios Umi-AV, Uma-AV and Rma-AV are submitted in Rel-19.

[Moderator’s note] Agreed in Wednesday online session

- Agreement

To generate the absolute delay model for sensing scenarios Umi-AV, Uma-AV and Rma-AV, for both target channel and background channel,

- For the TRP-TRP link and TRP- terrestrial UE link, the values of parameters for Δτ of scenarios Umi, Uma and RMa are respectively reused.

- For the terrestrial UE- terrestrial UE link, the values of parameters for Δτ of scenarios UMi are reused.

- For the TRP- aerial UE link, the values of parameters for Δτ of scenarios Umi, Uma and RMa are respectively reused.

- For the terrestrial UE- aerial UE link, the values of parameters for Δτ of scenarios UMi are reused.

- For the aerial UE- aerial UE link, the values of parameters for Δτ of scenarios UMi are reused.

Note: no measurements on Δτ of the scenarios Umi-AV, Uma-AV and Rma-AV are submitted in Rel-19.

Power normalization between target channel and background channel

[Moderator’s note] The original proposal is per majority view, however, no much supports are observed. Please note that we already have Option 1, i.e., no power normalization for the combined channel. If an agreement cannot be made for Option 2, we have to drop Option 2 from Rel-19.

For reference, this is the list of supporting companies for each alternative

- Alt 1: Power normalization on both target channel and background channel

  - Apple, Nokia, CATT, LGE, OPPO, QC, CT, Spreadtrum

- Alt 2: Only the power of the background channel is scaled down to make total power normalized

  - EURECOM, ZTE, IDC, OPPO

- Alt 3: the target channel of a target will replace one cluster in the background channel

  - BUPT (indoor/outdoor scenarios), Panasonic, vivo

[M] Proposal 5.6-1

- Power normalization applied on both target channel and background channel, i.e., HISAC=α⋅(Htarget+Hbackground), where α is such that E|HISAC​2=E|​Hbackground2

| Company | Yes/No | Comments |
| --- | --- | --- |
| LGE | Yes | Can you clarify the intention of the last equation as follows? Where α is such that E¦HISAC​2=E¦​Hbackground2 of the communication channel |
| Moderator |  | @LGE: The equation is proposed by Qualcomm. The intention is to normalize power of the ISAC channel to be equal to the power of background channel. In other words, power of ISAC channel equals to communication only channel. |
| Vivo |  | Two options can be considered. Option 1 is, no power normalization applied on both channels, and Option 2 is, one cluster is selected to replace the target path. Which one can be taken depends on the deployment scenarios. For instance, if UAV is a deployment target, we can choose Option 1, while if indoor room is a deployment scenario, we can choose Option 2. |
| CATT, CICTCI |  | We are OK with this proposal. |
| OPPO | No | First of all, E¦HISAC​2=E¦​Hbackground2 is not analytical for derivation of α. In fact, the generation of HISAC is just one sample in the sample space, against which E¦HISAC​2 is calculated. The proposal would not intend to force the simulation to generate all samples in the sample space for HISAC, right? Secondly, the proposal lets the model always modify the target channel power (relating to its sensing performance) as number of targets changes. It is preferable to maintain target channel power unchanged unless it has to be changed in Option 2 (e.g., to maintain a reasonably small channel energy on background channel). We would like to raise our proposal in our contribution for the last several meetings: For a power normalization in , where P0 is power of background channel, and Pi is i-th target channel linked to the same pair of Tx/Rx. The following normalization coefficients reaches a compromise between Option 1 and Option 2. - α0=max1-1≤i≤KPiP0, ε - αi=1-α0P01≤j≤KPj for 1≤i≤K - ε is a positive fractional number that guards the lowest power for P0 to be scaled down to. We can set ε=10% or any other number agreeable can work. If 1-1≤i≤KPiP0≥ε, the procedure behaves as Option 2; otherwise, it behaves as Option 1. |
| Moderator |  | @OPPO: your solution requires a new parameter ε. It seems hard to get a common value on proper value for ε. Therefore, I still think my early Proposal 5.6-1 is the only possible outcome considering the majority view of the 3 alternatives. Since “E¦HISAC​2=E¦​Hbackground2” causes confusions, I elaborate the proposal with 3 possible options. Hope it is helpful. |

[M] Proposal 5.6-1-rev1

Please provide your preference

Alt 1A

- A power scaling factor α is applied on both target channel and background channel, subjected to

α∙i=0K-1Ptarget,k+Pbackground=Pbackground

Where,

  - Ptarget,k,Pbackground are the powers of target channel and background channel of one implementation/sample of the ISAC channel.

Alt 1B

- A power scaling factor α is applied on both target channel and background channel,

α=Eα=EPbackgroundPISAC=EPbackgroundi=0K-1Ptarget,k+Pbackground

Alt 1C

- A power scaling factor α is applied on both target channel and background channel,

α=EPbackgroundEPISAC=EPbackgroundi=0K-1EPtarget,k+EPbackground

[Moderator’s note] If Alt 1B/1C is agreeable, we need to run simulation to come up with value α. For simplicity, we may only consider the large scale parameters, e.g., the power of the target channel can be estimated using power scaling factor

LTX-SPST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2

On the other hand, if majority view of Alt 1 is not agreeable, we may end up with conclusion that power normal over target/background channel are not supported in Rel-19.

| Company | 1A/1B/1C or others | Comments |
| --- | --- | --- |
| ZTE |  | Such per slot normalization is not needed. As Moderator suggested, alpha based on large scale parameters are quite enough. Here is my suggestion: - A power scaling factor α is applied on both target channel and background channel, subjected to α∙i=0K-1Ptarget,k+Pbackground=Pbackground Where, - Ptarget,k,Pbackground are the large scale parameter of target channel and background channel. |
| CATT, CICTCI | 1A | Slightly prefer Alt 1A for simplicity. |
| LGE | 1A |  |
| Moderator |  | Thanks for the comments. To accelerate the progress, some further updates are done based on suggestion from ZTE |

[FL2][M] Proposal 5.6-1-rev2

To do power normalization of target channel and background channel of ISAC channel

- A power scaling factor α is applied on both target channel and background channel, subjected to

α∙k=0K-110Ptarget,k10+10Pbackground10=10Pbackground10

Where,

  - Ptarget,k equals to the power scaling factor of target channel, i.e.,

PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2.

  - Pbackground is the nlos power  calculated by the large scale parameters of background channel and the k factor, i.e., PLdBd+SFdB.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Lenovo | Comments | For any power normalization for the ISAC channel, we need to clarify first if the ISAC channel is the background channel with target as an add-on, or the ISAC channel has intended to be the background channel, with target being part of the background. i.e., do we have a Uma scenario and add a vehicle (as sensing target) as an add-on to the initial Uma scenario to define the ISAC channel, or, the vehicle is considered as an integral part of the Uma scenario. All of our agreements so far, has been based on the first assumption, that the target is considered as an add-on (which means option 1, no power normalization). If we want to introduce power normalization, fist we need to clarify first what ISAC channel intends to model (is it Uma + a vehicle or Uma). In the above text, the index “k” is not defined. |
| EURECOM | Yes |  |
| vivo | No | Based on our simulation result, the power normalization of target channel and background channel is not necessary. The power summation of target channel and background channel is the most directly method for all the scenarios. |
| QC | Yes |  |
| MediaTek | No | We share the similar view with vivo that the power normalization of target channel and background channel is not needed. |
| QC2 |  | Revised Proposal: To do power normalization of target channel and background channel of ISAC channel - A power scaling factor α is applied on both target channel and background channel, such as the following holds: If k=0K-110Ptarget,k1010Pbackground10<1 , then k=0K-110Ptarget,k10+α∙10Pbackground10=10Pbackground10 Otherwise, α∙k=0K-110Ptarget,k10+10Pbackground10=10Pbackground10 Where, - Ptarget,k equals to the power scaling factor of target channel, i.e., PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2. - Pbackground is calculated by the large scale parameters of background channel, i.e., PLdBd+SFdB. |
| QC3 |  | Revised Proposal: To do power normalization of target channel and background channel of ISAC channel - A power scaling factor α is applied on both target channel and background channel, such as the following holds: If k=0K-110Ptarget,k1010Pbackground10<1 , then k=0K-110Ptarget,k10+α∙10Pbackground10=10Pbackground10 Otherwise, α∙k=0K-110Ptarget,k10+10Pbackground10=10Pbackground10 Where, - Ptarget,k equals to the power scaling factor of target channel, i.e., PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2. - Pbackground is the power of the NLOS clusters calculated by the large scale parameters of background channel, i.e., PLdBd+SFdB. |

[FL2][M] Proposal 5.6-1-rev3

To do power normalization of target channel and background channel of ISAC channel

- A power scaling factor α is applied on both target channel and background channel, such as the following holds:

If  k=0K-110Ptarget,k1010Pbackground10<1 , then

k=0K-110Ptarget,k10+α∙10Pbackground10=10Pbackground10

Otherwise,

α∙k=0K-110Ptarget,k10+10Pbackground10=10Pbackground10

Where,

  - Ptarget,k equals to the power scaling factor of target channel, i.e.,

PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2.

  - Pbackground is the power of the NLOS clusters calculated by the large scale parameters of background channel, i.e., PLdBd+SFdB.

[Moderator’s note] Agreed in Wednesday online session.

- Conclusion

No further study on power normalization of target channel and background channel of ISAC channel in Rel-19

  - Note: sub-section “7.9.5.3 Power normalization across target channel and background channel” in the TR remains as a placeholder with the following text.

    - To combine the target channel and the background channel, an alternative scheme of power normalization can be applied to keep the same/similar channel power as the background channel without sensing target.

[Moderator’s note] Regarding ‘FFS condition to select option, e.g. depending on scenario, sensing mode, number of target/EO type-2’, there is a good consensus it is not necessary based on companies’ inputs in RAN1#120bis.

Yes:

No: ZTE, CATT, Apple, vivo, QC, SS, Ericsson, Xiaomi

- [Closed][FL2][L] Proposal 5.6-2

A condition is not specified to select between Option 1 ‘no power normalization’ and Option 2 ‘with power normalization’ in the combination of target channel and background channel.

| Company | Yes/No | Comments |
| --- | --- | --- |
| LGE |  | Yes |
| vivo |  | We agree to have a condition, whereby Options can be selected. As we described for Proposal 5.6-1, however, Option 2 should be a replacement solution. |
| CATT, CICTCI | Yes |  |
| OPPO |  | If there is no condition, it should be made clear that the selection between Option-1 and Option-2 is simulation-wise, not Tx-Rx pair wise, i.e., it is not allowed to leave one pair of Tx-Rx using Option-1 and another pair of Tx-Rx using Option-2 in the same simulation drop. |
| EURECOM |  | We should specify when an option is chosen, it is used in the whole simulation from the beginning to the end |
| Ericsson | Yes | Otherwise the two options become conditionally mandatory, against the agreement. Agreement - The following options are supported to generate the combined ISAC channel - Option 1: The ISAC channel of a pair of sensing Tx/Rx is obtained by summing the target channel(s) and background channel, i.e., power normalization is not performed. - Option 2: As an additional modelling component, power normalization is performed when summing the target channel(s) and background channel, to keep the same/similar channel power as the background channel without target. Down select between - Alt 1: Power normalization on both target channel and background channel - Alt 2: Power normalization on background channel only - Alt 3: the target channel of a target will replace one cluster in the background channel - FFS Blockage is modelled for the background channel due to sensing target and/or EO type-2 - FFS condition to select option, e.g. depending on scenario, sensing mode, number of target/EO type-2 |
| QC | Yes |  |
| Moderator5 |  | Based on the agreement of no further study on power normalization between target and background channel in Rel-19, we don’t need to discuss this issue again. |

Doppler of moving scatters

[Moderator’s note] based on inputs from RAN1 #120bis, the maximum speed and ratio of moving scatters are scenario dependent and may be modelled by EO type-1.

[M] Proposal 5.7-1

- Dual mobility model in 7.6.10, TR 38.901 is reused as start point to model Doppler effect ft due to movement of stochastic clusters, i.e., 2αn,mDn,mλ0

  - Necessary changes on maximum speed and ratio of moving scatters can be discussed in evaluation phase.

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| LGE |  | The existing dual mobility holds for the monostatic sensing. We wonder if it also holds for the bistatic sensing case. |
| Nokia, NSB | ok |  |
| BUPT | Yes |  |
| vivo |  | The Doppler effect due to movement of stochastic clusters can be studied in the future release. This is because the stochastic clusters affecting sensing will be much complex than that affecting communication. We need time to investigate the details before making decision. |
| OPPO | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Apple | Ok |  |
| Spreadtrum | Yes |  |
| Ericsson |  | The scope of this SI is given in the SID and it includes modeling both intended and unintended targets where the latter can be represented by Type-1 Eos and/or moving stochastic scatterers. This SI does not include an evaluation phase, therefore we have no authority to decide on what work a potential future SI/WI should include, that is for the RAN plenary to decide. |
| Moderator2 |  | Per Ericsson’s comments, let’s focus on what we can do in Rel-19. We can enforce what we will do in the future. Revision by deleting the sub-bullet. |

- [Closed][FL1][M] Proposal 5.7-1-rev1

- Dual mobility model in 7.6.10, TR 38.901 is reused as start point to model Doppler effect ft due to movement of stochastic clusters, i.e., 2αn,mDn,mλ0

  - Necessary changes on maximum speed and ratio of moving scatters can be discussed in evaluation phase.

| Company | Yes/No | Comments |
| --- | --- | --- |
| PCL | No | In practice, the background channel is mainly contributed by stationary reference points, with a limited number of stochastic clusters (e.g., insects, fallen leaves, etc.), whose contribution is usually limited. Therefore, we believe it is unnecessary to model the influence of stochastic clusters in the background channel, as this would introduce additional complexity with only marginal improvements in accuracy. In fact, in monostatic sensing, the Doppler effect in the background channel mainly comes from the movement of the UT/TX/RX, and the Doppler caused by a few low-speed scatterers can be neglected. We propose that: Proposal: The reference points to generate the UT monostatic background channel are stationary, and their Doppler frequencies are fD,n,m=2rtx,n,mT∙vtxλ0 by reusing Eq. (7.6-45) in Section 7.6.10 of TR38.901. |
| Moderator3 |  | Based on the offline session discussions, we don’t need to continue discussion on this issue. The rapporteur will delete the sentence “A typical value of p is 0.2” from the draft CR. So this issue is closed now. |

Blockage model A/B

[Moderator’s note] It is generally OK that an existing additional feature in existing R 38.901 can be reused for ISAC. One new point for this issue is to clarify the target can be considered as a blocker, which happens in the reality. Therefore, the moderator makes the following proposal. However, if it is not agreeable, the moderator suggest we could discuss such details of blockage model A/B in evaluation phase.

[M] Proposal 5.8-1

- The existing blockage model A/B is reused to model the blocking effect due to a target as an optional feature

  - Applicable to the LOS/NLOS rays in the background channel of the target

  - Applicable to the LOS/NLOS rays in the Tx-target and target-Rx link of the other target

| Company | Yes/No | Comments |
| --- | --- | --- |
| BUPT | Yes |  |
| Moderator |  | Revised by offline |

[FL2][M] Proposal 5.8-1-rev1

- The existing blockage model A/B is reused to model the blocking effect due to a target as an optional feature

  - Applicable to the LOS/NLOS rays in the background channel of the target

  - Applicable to the LOS/NLOS rays in the Tx-target and target-Rx link of the other target

- Note: The location of the target as a blocker is known before applying the blockage model A/B.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Lenovo | Yes, with comment | Blockage is an essential (and not optional) feature of a sensing channel. When target blocks a path, it is a useful way to obtain sensing information (e.g., as we can see via observing a shadow/blocked area). Also, due to the inter-dependency with the bistatic RCS model, they have to be discussed together. |
| BUPT3 | Yes |  |
| Ericsson | Yes with comment | In the note, not only location, but also orientation and size. |
| NIST | Yes with comments | When a target partially or completely blocks the LOS path, it can be modelled as a blocker using 38.901 blockage model. We validated this model based on our measurement data (R1-2504013). The target as a blocker is also closely related to the target bistatic RCS model, particularly when the bistatic angle is close to 180 degrees. Some clarification is required to make sure these two models are work seamlessly together. In our contribution, we also proposed an enhancement to the existing blockage model in TR38.901 to model the blockage effect of a 3D object. |
| Moderator5 |  | @Ericsson: revised |

[FL5][M] Proposal 5.8-1-rev2

- The existing blockage model A/B is reused to model the blocking effect due to a target as an optional feature

  - Applicable to the LOS/NLOS rays in the background channel of the target

  - Applicable to the LOS/NLOS rays in the Tx-target and target-Rx link of the other target

- Note: The location, orientation and size of the target as a blocker is known before applying the blockage model A/B.

[Moderator’s note] Agreed in Thursday online session

[FL5][M] Proposal 5.8-1-rev3

- The existing blockage model A/B procedures can be reused to model the blocking effect due to a target as an optional feature

  - Applicable to the LOS/NLOS rays in the background channel of the target

  - Applicable to the LOS/NLOS rays in the Tx-target and target-Rx link of another target

- The location, orientation and size of the target as a blocker is known before applying the blockage model A/B.

[Moderator’s note] Agreed in Friday online session

- Agreement

- The existing blockage model A/B procedures can be reused to model the blocking effect due to a target as an optional feature

  - Applicable to the LOS/NLOS rays in the background channel of the target

  - Applicable to the LOS/NLOS rays in the Tx-target and target-Rx link of another target

- The location, orientation and size of the target as a blocker is known before applying the blockage model A/B.

Micro-Doppler

[Moderator’s note] A placeholder for micro-Doppler was agreed in RAN1 #119. If there is time, it is nice to discuss certain detailed functions to handle micro-Doppler. However, it is the moderator’s understanding that the discussion is mostly informative since we don’t know the exact interested use case/target in future evaluation. Decision on micro-Doppler in evaluation phase sounds better.

[FL3][M] Question 5.9-1

- Companies are encouraged to input on the proper functions/models for micro-Doppler

| Company | Comments |
| --- | --- |
| vivo | According to our theoretical analysis and experiment measurement (R1-2500361), at lease, the formula of the micro-Doppler frequency shift induced by the vibration of scatterer point is mature. The proposed formula can be expressed as fDt=2λωvDvsinωvtcosϕ'cosθ where θ is the elevation angle of the point O at the origin of the coordinate system x,y,z, ϕ' is the azimuth angle of the point O' at the coordinate system x',y',z', ωv is the angular frequency of scatterer point P due to its vibration, and Dv is the maximum amplitude. As an example, the vibration scenario for human arm and leg is illustrated in the following figure. |
| OPPO | In general, we think functions/models for micro-Doppler should be discussed in the evaluation stage. If there are functions/models to be studied and endorsed in next meeting, those should be made as just examples and not intended to prevent using of other micro-Doppler functions in the evaluation phase. |
| ZTE | For micro-Doppler odelled, we can reuse the patterns defined in report from ETSI ISAC ISG in LS R1-2501370 as follows - The micro-Doppler motion displacement φmicro,pt induces phase variations on the scattering signals from the sensing target expressed as 2πφmicro,ptλ, where λ is the carrier wavelength. The micro-Doppler motion displacement, φmicro,pt, can be expressed through the micro-Doppler velocity function, vmicro,pτ, using the following expression: φmicro,p(t)=0tvmicro,p(τ)dτ. Type of target; Formula of micro-Doppler function φmicro,p(t). Human respiration/heartbeat; φmicro,p(t)=R0+rpDsin2πfrtmax is the initial location offset. is the location vector of human. is the maximum amplitude of respiration/heartbeat. is the frequency of respiration/heartbeat.. Human walking/running; φmicro,p(t)=R0+rpp=1NpDpsin2πfpt is the initial location offset. is the location vector of human. is the number of human segments. is the maximum amplitude of motion of pth segment. is the frequency of motion of pth segment.. UAV propeller rotation; φmicro,k,p(t)=Rj+ruL2cosΩjt+ϕj+k2πNB j=1,⋯,NR;k=0,⋯,NB-1 For kth rotation blade on the jth rotor of UAV is the initial location offset of jth rotor and is the initial phase of jth rotor. is the location vector of UAV is the the distance between the center of blade and the center of the rotation and is the frequency of jth blade rotation.. |
| Ericsson | In our contribution R1-2504707 (revision of R1-2504455) we discuss modelling of Doppler signatures with the following proposal: Proposal To model the Doppler signature (micro-Doppler) of various targets and Type-1 EOs, the target is represented by one (or more) fixed scattering points and one (or more) scattering points that are moving in relation to the fixed scattering point - For a UAV, the moving scattering point moves in a horizontal circle with radius r and rotation speed v_rot around the fixed scattering point. The moving scattering point has a value A that is 20-40 dB lower than the value A for the fixed scattering point, B is isotropic, and B2 has standard deviation 3 dB - Small UAV: r = 10 cm, v_rot = 40 m/s - Large UAV: r = 40 cm, v_rot = 10 m/s - For a flapping bird, the moving scattering point moves in a vertical sinusoidal motion with frequency 2-6 Hz and speed v = 2-12 m/s. The moving scattering point has a value A that is 0-10 dB lower than the value A for the fixed scattering point, B is isotropic, and B2 has standard deviation 3 dB - For a gliding bird, the scattering point becomes stationary - For a human walking a speed v, the torso is represented by a moving rather than fixed scattering point, where the horizontal speed varies sinusoidally between v-v’ and v+v’. The arms and legs are represented by a second scattering point that moves horizontally in the direction of travel in a sinusoidal manner with speed between 0 and 2*v. The arms/legs scattering point has a value A that is 10 dB lower than the value A for the torso scattering point, B is isotropic, and B2 has standard deviation 3 dB - FFS: other targets and Type-1 EOs |
| NIST | - Micro-Doppler model for human respiration motion and heartbeat motion ### Respiration Motion The breathing displacement function is modelled as a composite of a parabolic inhalation and an exponential exhalation profile: rbreatht=-rmaxTiTet2+rmaxTTiTet, &t≤Tirmax1-e-Teτ e-t-Teτ-e-Teτ, &Ti<t≤T Equation 4-1 Where rmax is the maximum chest displacement due to breathing, Ti is the inspiratory time Te is the expiratory time, T= Ti+ Te is the full respiration cycle period and τis the exhalation decay constant. ### Heartbeat Motion The heartbeat-induced displacement is modelled using a sinusoidal function based on the subject’s heart rate: rheartt=rh,maxsin2πtHR60 Equation 4-2 Where rh,max is the maximum displacement due to cardiac motion and HR is the heart rate in beats per minute. The total displacement of the chest is thus: rtotalt=rbreatht+rheart(t). Equation 4-3 The corresponding micro-Doppler shift can be computed as follows: fmicro=1 λ [-drtotaltdt r1(t)-drtotaltdt r2(t) ], where λ is the wavelength, rtotalt is the total displacement of the chest, and r1(t) and r2(t) are the unit radial vector from TX to the human subject and from RX to the human subject, respectively. |
| Modeartor6 | From the Wednesday offline session, it is clear majority view is not to discuss further on micro-Doppler in Rel-19. |

[FL5][M] Proposal 5.9-1

Exact formula for micro-Doppler is not further studied in Rel-19.

[Moderator’s note] Agreed in Friday online session

- Conclusion

There is no consensus to introduce an exact formula for micro-Doppler in Rel-19. The placeholder in the channel impulse response is kept in the draft CR.

EO type-2

EO type-2 in background channel

[Moderator’s note] It is a long discussion whether EO type-2 can be modeled in background channel, but no conclusion yet. The following arguments are observed

From proponent

- In the real propagation with target and wall (EO type-2) present, it is possible that the signal reflected by EO type-2 can be observed in both target channel and background channel.

- For bistatic sensing, if there is no LOS ray, specular reflected ray of EO type-2 in background channel may serve as reference for measurement of time difference

From opponent

- GBSM channel in existing 38.901 is validated assuming possible presence of wall (i.e., EO type-2). If further model EO type-2 in background channel, it just increases the power and change the distribution of background channel comparing with existing channel for communication.

Based on discussion from RAN1 #120bis,

- Option 1: EO type-2 is modelled in background channel if modelled in target channel

  - Supported by (7): HW, LG, Lenovo, SS, ZTE, Sony, vivo(InH)

- Option 2: EO type-2 is not modelled in background channel

  - Supported by (10): CATT, QC, vivo(others), SPRD, Ericsson, IDCC, DOCOMO, OPPO, Xiaomi, Nokia

I soft the wording of Option 1, let’s check if following proposal is agreeable.

[FL4][H] Proposal 6.1-1

- EO type-2 can be modelled in background channel if modelled in target channel

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes | We are also flexible not to model EO type-2 in background channel for reducing simulation complexity |
| LGE | Yes |  |
| Nokia, NSB | No | No need to model in background channel (Option 2). |
| BUPT | No | We consider this approach would contradict the fundamental definitions of both the target channel and background channel. |
| Vivo | Yes | We prefer to model EO type-2 in indoor office. According to our experiment measurement, the phenomenon of EO type-2 reflection highly occurs in indoor office. This may assist to improve the accuracy for sensing, especially for range detection. |
| CATT, CICTCI | No | The background channel is no longer required to account for the influence of EO type-2. |
| Huawei, HiSilicon | Yes |  |
| Samsung | Yes |  |
| DOCOMO | No | Option 2 |
| Lenovo | Yes |  |
| SPREADTRUM | Yes |  |
| EURECOM | Yes |  |
| Ericsson | No | No such need. |
| QC | No |  |
| PCL | Yes | Whether EO type-2 is modeled in the background channel is related to whether it is modeled in the target channel: If it is modeled in the target channel, we must also model it in the background channel, because the gain of paths containing EO type-2 in the target channel is far smaller than those in the background channel. Additionally, we emphasize that EO type-2’s effect on signals is reflection, which differs from the scattering behavior of stochastic clusters in existing UE-BS communication channels, and thus it is necessary to model the EO type-2 in the background channel if it is modelled in the target channel. |
| Moderator7 |  | Agreed in Thursday online session |

- Agreement

EO type-2 can be optionally modelled in background channel when EO type-2 is modelled in target channel.

LOS condition considering EO type-2

Agreement The following cases of radio propagation in the target channel are considered for the study Case; Tx-target; Target-Rx. 1; LOS condition; LOS condition. 2; LOS condition; NLOS condition. 3; NLOS condition; LOS condition. 4; NLOS condition; NLOS condition. - Case 1/2/3/4 can be considered for bistatic sensing mode - At least Case 1/4 can be considered for monostatic sensing mode - Note: It doesn’t imply the channel response for each link is separately generated then concatenated - FFS how to determine LOS condition and NLOS condition, e.g., based on LOS probability, or determined based on geometrical locations of environment object (EO). - In LOS condition, line of sight ray(s) are present between Tx/Rx and target, and there may or may not exist non-line of sight ray(s) between Tx/Rx and target too - In NLOS condition, there only exist non-line of sight ray(s) between Tx/Rx and target

[Moderator’s note] This is a key assumption to model EO type-2. If we cannot agree on a solution, we may end up with removing the feature of EO type-2.

- Option A reflect the real channel propagation. If a LOS ray is blocked, it is NLOS condition. However, it changes the behavior in current TR

- Option B binds EO type-2 with blockage model B, which is not desired by majority of companies.

- Option C is likely kind of compromise, which reuses existing solution for LOS condition determination by LOS probability and doesn’t tight EO type-2 with blockage mode.

If the majority view of Option A cannot be agreeable, Option C may be the outcome. Without an agreed solution, EO type-2 may need to be removed from the Rel-19 the study item.

Comments are welcome, especially whether or not a proponent of Option A can be fine with Option C.

[FL2][H] Proposal 6.2-1

Down-select in RAN1#121 one option from the following options to determine the LOS condition of the Tx-target link and target-Rx link?

- Option A: If type-2 EO is in the LOS ray of one link, the link is determined as NLOS condition, and otherwise use the LOS probability equation defined in existing TRs to determine the LOS/NLOS condition

Supported by (12): HW, ZTE, Sony, QC, LG, Xiaomi, MTK, Nokia, CMCC, Spreadtrum, OPPO, vivo

- Option B: Use the LOS probability equation to determine the LOS/NLOS condition of one link, and then the impacts of type-2 EO is modelled by a blockage model

Supported by (6): Ericsson, CATT (without blockage model), Lenovo, IDCC, DOCOMO, BUPT

- Option C: Use the LOS probability equation to determine the LOS/NLOS condition of one link,

Supported by (7): CATT, Ericsson, Nokia, Lenovo, DOCOMO, IDCC,

Note: If EO type-2 is agreed to be modelled in background channel, the agreed option is extended to LOS condition determination for background channel.

| Company | Option A/B/C | Comments |
| --- | --- | --- |
| LGE | A |  |
| Nokia, NSB | A | For Option B, it might be not necessary to redefine a blockage model for EO type-2 |
| BUPT | Option B | We consider this scenario to fall within the scope of blockage. |
| Vivo | Option A |  |
| CATT, CICTCI | Option B (without blockage model) or Option C | Note that the Rx could be far from EO type-2, where the distance is considerably larger than the wavelength, leading to negligible diffraction effects. |
| Huawei, HiSilicon | A | - Necessary supplement needed for modelling EO type-2 EO type-2 typically as wall is now deterministically modelled by a specular reflection ray as one more component ray on top of the stochastic rays as generated per the current TR38.901, some necessary supplement is needed, which was not considered in the past was because the wall was not explicitly modelled then. The supplement includes the LOS/NLOS condition determination and pathloss for the ray under NLOS condition. It is worth to note that the supplement is only applied to the ST/Uts which is affected by the EO type-2, and for the ST/Uts not affected by the EO type-2, the channel generation procedure is kept the same as the stochastic model as shown in the Figure 1. - LOS/NLOS condition determination In the current TR 37.885 as excerpted, for V2P and V2V link, when the LOS path is blocked by buildings, the status of this link is defined as NLOS condition whenever the two vehicles are located in different streets since the vehicle height is always lower than the building. Otherwise, the condition of the link will be determined by the LOS probability equation. The underlying reason for this modelling is reflecting the realistic situation that it should be NLOS condition whenever the building blocks the LoS ray between the Tx and Rx and the evaluation performance will be overestimated under the propagation channel otherwise. TR37.885 The V2V sidelink channel is modeled according to the following three states: - LOS: - A V2V link is in LOS state if the two vehicles are in the same street and the LOS path is not blocked by vehicles. - NLOS: LOS path blocked by buildings - A V2V link is in NLOS state if the two vehicles are in different streets. - NLOSv: LOS path blocked by vehicles - A V2V link is in NLOSv state if the two vehicles are in the same street and the LOS path is blocked by vehicles.. Likewise, for the TRP-ST/UT link, the LOS/NLOS condition should not be based on the LOS probability equation and the supplement is needed given the building with height is modelled now, i.e., the LOS/NLOS condition should be determined by the 3D geometry location of the TRP and the ST/UT but not always 100% NLOS condition when the TRP and ST/UT are located in different streets because of different heights for TRP and ST/UT. Note that the evaluation performance would be overestimated if it is LOS condition based on the probability equation while it should be NLOS condition instead because the building blocks the LOS ray between the TRP and ST/UT. Figure 1: ST affected by the EO type-2. With that modelled for the LOS/NLOS condition, some companies view that the LOS probability model for the stochastic component rays needs to be updated accordingly since the EO type-2 is already part of the existence when the LOS probability model is made. However, in our view the update is not needed, because the EO type-2 only affects the ST/UT blocked by the EO type-2, for which the condition should be based on the geometry. For other ST/UT not affected by the EO type-2, e.g., when located at the same street as the TRP with distance of d as illustrated in Figure 2, the LOS probability model is the same and should be the same as the legacy even when the 3D EO type-2 is modelled. Figure 2: The Los probability keeps the same for ST/UE not affected by EO type-2, e.g., when located at the same street as TRP. Observation 1: - The evaluation performance would be overestimated if it is LOS condition based on the probability equation while it should be NLOS condition instead because the building blocks the LOS ray between the TRP and ST/UT. - For ST/UE not affected by the EO type-2, the LOS probability model is the same and should be the same as the legacy even when the 3D EO type-2 is modelled, e.g., when located at the same street as TRP. Option B is to use the blockage model to generate the propagation channel for the EO type-2. However, the blockage is an additional feature on top of the basic propagation path without changing the LOS/NLOS status. The hard transition from LOS to NLOS or vice versa is the same for the case when the EO type-2 is not modelled and can also be avoided as the legacy as well by modelling spatial consistency. Observation 2: - The blockage model is an additional feature on top of the basic propagation path without changing the LOS/NLOS status. - The hard transition from LOS to NLOS or vice versa is the same for the case when the EO type-2 is not modelled and can also be avoided as the legacy procedure as well by modelling spatial consistency. Overall, we believe the LOS/NLOS condition being determined by the 3D geometric location of the TRP and the ST/UT makes more sense and should be adopted. |
| Moderator |  | Though some companies do not show opinion yet, it seems we face the same situation as early meetings. Suggestion from moderator is please each company being more constructive on this issue. Since LOS condition determination is quite basic step for channel generation, if we cannot reach an agreeable solution, we may end up dropping this feature. |
| CATT, CICTCI |  | We think a consistent view should be adopted in Proposal 6.1-1 and 6.2-1. If we believe there is no interaction between EO type-2 and background channel, why do we still have to consider the EO type-2 impact on LOS/NLOS probability? We think it is not meaningful to discuss this change, since we already get what we want: a mirror reflection from EO type-2 in target channel, to improve the sensing detection performance. |
| Lenovo |  | Option B |
| EURECOM |  | Option A |
| Ericsson | Option B, C | We have different view from the highlighted FL’s comment below. [Moderator’s note] This is a key assumption to model EO type-2. If we cannot agree on a solution, we may end up with removing the feature of EO type-2. - Option A reflect the real channel propagation. If a LOS ray is blocked, it is NLOS condition. However, it changes the behavior in current TR - Option B binds EO type-2 with blockage model B, which is not desired by majority of companies. - Option C is likely kind of compromise, which reuses existing solution for LOS condition determination by LOS probability and doesn’t tight EO type-2 with blockage mode. If the majority view of Option A cannot be agreeable, Option C may be the outcome. With such agreement, EO type-2 cannot survive in the study item. Comments are welcome, especially whether or not a proponent of Option A can be fine with Option C.. Option A is a new solution different from the legacy one, while Option B and C follows the legacy stochastic LOS/NLOS probability. If there is no consensus, the legacy method can be reused for Type-2 EO. We don’t see any problem. Option A may cause a hard transition in the channel response as the targets moves, similar to UT movement, which legacy communication channel tries to avoid. With Option A, simulation stops when the LOS/NLOS state changes based on the latest geometry of target, Tx, and Rx. Thus Option A puts a constraint on targets’ movement and hinders and disables target tracking. As part of spatial consistency feature, in case there is a transition from LOS to NLOS due to UT mobility, there will be a hard transition in the channel response, which should be avoided. Soft LOS state (in section 7.6.3.3 of 38.901) addresses the hard transition. It means a UE is in both LOS and NLOS condition, which is not compatible with Option A, a deterministic NLOS condition. Soft LOS state LOSsoft is generated by floating numbers between 0 (NLOS) and 1 (LOS) in the spatial consistency modelling. The value of LOSsoft is determined by , (7.6-18). |
| QC | Option A |  |
| PCL | Option A | the signal is always being reflected, not being blocked randomly. If no EO is detected, we can still use the standard LOS probability equation defined in existing TRs. This avoids redundant calculations when EO is present and keeps things simple. Option B and C introduce unnecessary steps, as EO already confirms the NLOS condition. |
| Moderator5 |  | The following proposals are provided by interested companies. Please share your view which one is preferrable/acceptable. Thanks. |

[FL2][H] Proposal 6.2-1-rev1

Down selection among one from the following 3 options in RAN1 #121

Option 1:

To determine the LOS condition of the Tx-target link and target-Rx link,

- Option A: If type-2 EO is in the LOS ray of one link, the NLOS probability is 100%, and otherwise use the LOS probability equation defined in existing TRs to determine the LOS/NLOS condition

Supported by QC, HW

Option 2:

To determine the LOS condition of the Tx-target link and target-Rx link,

- For urban grid scenario modelling EO type-2: If type-2 EO is in the LOS ray of one link, the link is determined as NLOS condition, and otherwise use the LOS probability equation defined in existing TRs to determine the LOS/NLOS condition

- For other scenarios:  The LOS probability equation defined in existing TRs to determine the LOS/NLOS condition.

  - This can be revisited and updated when necessary in the evaluation phase when the details of EO type-2 deployment is settled.

Supported by HW

Option 3:

To determine the LOS condition of the Tx-target link and target-Rx link, the following two options are agreed as candidate solutions:

- Option A: If type-2 EO is in the LOS ray of one link, the link is determined as NLOS condition, and otherwise use the LOS probability equation defined in existing TRs to determine the LOS/NLOS condition

- Option C: Use the LOS probability equation to determine the LOS/NLOS condition of one link.

Supported by CATT

Note: Common to all 3 options, if EO type-2 is agreed to be modelled in background channel, the agreed option is extended to LOS condition determination for background channel when EO type-2 is present.

| Company | Option 1/2/3 | Comments |
| --- | --- | --- |
| Ericsson2 |  | The compromise we suggest is as follows. Option 4: Support legacy stochastic LOS/NLOS condition of Type-2 EO in section 7.9 and Option A, deterministic NLOS of Type-2 EO, in the section of map-based hybrid channel of 38.901 |
| Moderator6 |  | Updated proposal based on some further offline discussions |

[FL4][H] Proposal 6.2-1-rev2

Down selection among one from the following options in RAN1 #121

Option 2:

To determine the LOS condition of the Tx-target link and target-Rx link, when EO type-2 is modelled,

- For urban grid scenario: If type-2 EO is in the LOS ray of one link, the LOS probability is p, p=0, and otherwise use the LOS probability equation defined in existing TRs to determine the LOS/NLOS condition

- For other scenarios: The LOS probability equation defined in existing TRs to determine the LOS/NLOS condition.

  - This can be revisited and updated when necessary in the evaluation phase when the details of EO type-2 deployment is settled.

Supported by HW, QC, ZTE

Option 3:

To determine the LOS condition of the Tx-target link and target-Rx link, when EO type-2 is modelled, the following two options are agreed as candidate solutions:

- Option A: If type-2 EO is in the LOS ray of one link, the link is determined as NLOS condition, and otherwise use the LOS probability equation defined in existing TRs to determine the LOS/NLOS condition

- Option C: Use the LOS probability equation to determine the LOS/NLOS condition of one link.

Supported by CATT

Option 4:

- Support legacy stochastic LOS/NLOS condition of Type-2 EO in section 7.9.

- Support deterministic NLOS condition of Type-2 EO, in the section of map-based hybrid channel of 38.901.

Supported by Ericsson

[Moderator’s note] Agreed in Thursday online session

- Agreement

To determine the LOS condition of any link in ISAC channel model, when EO type-2 is modelled, the following two options are agreed as solutions:

- Option A: If type-2 EO is in the LOS ray of the link, the LOS probability is p, p=0, and otherwise use the LOS probability equation defined in existing TRs to determine the LOS/NLOS condition

- Option C: Use the LOS probability equation to determine the LOS/NLOS condition of the link.

Note1: in which conditions/scenarios to use option A or option C can be determined in future evaluations.

Note2: as already agreed, monostatic background channel is always NLOS

Agreement

EO type-2 can be optionally modelled in background channel when EO type-2 is modelled in target channel.

EO type-2 for a link in NLOS condition

[Moderator’s note] The existing ground reflection model in section 7.6.8, TR 38.901 is defined assuming LOS condition between Tx and Rx. This is the default behavior since we agreed to use section 7.6.8 of TR 38.901 as reference. Therefore, EO type-2 is at least supported in LOS condition.

In last meeting, Huawei and ZTE discussed the issue and propose to solve it under NLOS condition. The method is to derive power of EO type-2 based on power of LOS ray temporally generated assuming LOS condition. If this is agreeable, we allow such enhancement.

[FL1][M] Proposal 6.3-1

- The pathloss of the ray specular reflected by an EO type 2 in the STX-SPST link or SPST-SRX link is calculated by PLEO2=PLLOS+10log10dEO2d3D, where

  - PLLOS is a pathloss assuming a LOS condition for the STX-SPST link or SPST-SRX link

  - dEO2 is the total propagation distance due to EO type-2

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| LGE | Yes | If we make agreement for Proposal 6.1-1, we can also apply the same for the background channel. |
| BUPT | Yes | It is reasonable. |
| CATT, CICTCI | Yes |  |
| Huawei, HiSilicon | Yes | The wall specular reflection ray can exist irrespective of the existence of the LOS ray between the Tx and Rx. Therefore, the PLEO2=PLLOS+10log10dEO2d3D for specular reflection ray is applicable for both LOS and NLOS condition. Given the draft CR has already reflected the proposal for the LOS condition, we suppose this proposal is for NLOS condition. It would be better to clarify it in the proposal. |
| EURECOM | Yes |  |
| Ericsson | No |  |
| Moderator4 |  | Revised based on offline comments |

[FL1][M] Proposal 6.3-1-rev1

- The pathloss of the ray specular reflected by an EO type 2 in the STX-SPST link or SPST-SRX link is calculated by PLEO2=PLLOS+10log10dEO2d3D, where

  - PLLOS is a pathloss assuming a LOS condition for the STX-SPST link or SPST-SRX link

  - dEO2 is the total propagation distance due to EO type-2

[Moderator’s note] Proposal revised in the online session and a related agreement is made

[FL1][M] Proposal 6.3-1-rev1

The large scale parameter of the ray specular reflected by an EO type 2 in the STX-SPST link or SPST-SRX link, is calculated by PEO2=PLLOS+10log10dEO2d3D, where

  - PLLOS is the pathloss assuming a LOS condition for the STX-SPST link or SPST-SRX link

  - dEO2 is the total propagation distance due to EO type-2

  - Note: impact of polarization matrix of EO type-2 is not included in the above

- Agreement

EO type-2 can be modelled in NLOS condition.

[Moderator’s note] Based on the discussion online, the moderator prepares a TP to capture power metric for EO type-2

[FL2][M] Proposal 6.3-2

The follow TP is used generate the power (except for the impact of polarization matrix of EO type-2) of the ray specular reflected by an EO type 2 in the STX-SPST link or SPST-SRX link.

- 7.9.5.2 Type-2 environment object < Unchanged text omitted > - In Step 10 in Clause 7.9.4.1, Prx,0,m'k,p,Ptx,0,mk,p for a NLOS ray specularly reflected by a type-2 EO, if present, in the SPST-SRX link and the STX-SPST link is determined as follows. - If the STX-SPST link is in LOS condition, Ptx,0,mk,p=dtx,3Dk,pdtx,EO,mk,p2 - If the STX-SPST link is not in LOS condition, Ptx,0,mk,p=10-PLtx,k,pLOS+PLtx,k,p/10∙dtx,3Dk,pdtx,EO,mk,p2, where PLtx,k,pLOS is the pathloss of STX-SPST link assuming LOS condition. - If the SPST-SRX link is in LOS condition, Prx,0,m'k,p=drx,3Dk,pdrx,EO,m'k,p2 - If the SPST-SRX link is not in LOS condition, Prx,0,m'k,p=10-PLrx,k,pLOS+PLrx,k,p/10∙drx,3Dk,pdrx,EO,m'k,p2, where PLrx,k,pLOS is the pathloss of SPST-SRX link assuming LOS condition. < Unchanged text omitted >

[Moderator’s note] Agreed in Wednesday online session

- Agreement

The follow TP is used generate the power (except for the impact of polarization matrix of EO type-2) of the ray specular reflected by an EO type 2 in the STX-SPST link or SPST-SRX link.

- 7.9.5.2 Type-2 environment object < Unchanged text omitted > - In Step 10 in Clause 7.9.4.1, Prx,0,m'k,p,Ptx,0,mk,p for a NLOS ray specularly reflected by a type-2 EO, if present, in the SPST-SRX link and the STX-SPST link is determined as follows. - If the STX-SPST link is in LOS condition, Ptx,0,mk,p=dtx,3Dk,pdtx,EO,mk,p2 - If the STX-SPST link is not in LOS condition, Ptx,0,mk,p=10-PLtx,k,pLOS+PLtx,k,p/10∙dtx,3Dk,pdtx,EO,mk,p2, where PLtx,k,pLOS is the pathloss of STX-SPST link assuming LOS condition. - If the SPST-SRX link is in LOS condition, Prx,0,m'k,p=drx,3Dk,pdrx,EO,m'k,p2 - If the SPST-SRX link is not in LOS condition, Prx,0,m'k,p=10-PLrx,k,pLOS+PLrx,k,p/10∙drx,3Dk,pdrx,EO,m'k,p2, where PLrx,k,pLOS is the pathloss of SPST-SRX link assuming LOS condition. < Unchanged text omitted >

Whether/how to normalize the power of target channel, background channel or the combined channel if EO type-2 is modelled?

Agreement When EO type-2 is modelled, specular reflection is considered to model EO type-2 using section 7.6.8 of TR 38.901 as reference - As starting point, the effect of type-2 EO (i.e., in the path node1-EO-node2) is modelled as bR∥specular00-R⊥specular, b is a scaling factor (e.g., c equals to d3DdGR relative to LOS ray in section 7.6.8 in TR 38.901) - FFS any update to b,R∥specular,R⊥specular - FFS any update taking EO orientation into account - FFS any changes based on section 7.6.8 of TR 38.901 if EO type-2 has finite size - FFS whether diffraction and scattering can be considered in addition to specular reflection - EO type-2 is an optional modelling component if supported in a sensing scenario - FFS which deployment scenario(s) EO type-2 will apply

[Moderator’s note] In existing ground reflection model in section 7.6.8, TR 38.901, the power of the ground reflected ray is added to the generated fast fading channel without further power normalization. This is the default behavior since we agreed to use section 7.6.8 of TR 38.901 as reference.

Based on discussion from RAN1 #120bis, all interested companies agree to reuse existing rule from ground reflection model in 7.6.8, TR 38.901.

- Supported: ZTE, LGE, HW, CATT, vivo, QC, BUPT, Ericsson, Xiaomi

- Not supported:

Therefore, the moderator suggest we don’t spend more time on this issue.

Polarization matrix for EO type-2

Agreement -RAN1 #118 When EO type-2 is modelled, specular reflection is considered to model EO type-2 using section 7.6.8 of TR 38.901 as reference - As starting point, the effect of type-2 EO (i.e., in the path node1-EO-node2) is modelled as bR∥specular00-R⊥specular, b is a scaling factor (e.g., c equals to d3DdGR relative to LOS ray in section 7.6.8 in TR 38.901) - FFS any update to b,R∥specular,R⊥specular - FFS any update taking EO orientation into account - FFS any changes based on section 7.6.8 of TR 38.901 if EO type-2 has finite size - FFS whether diffraction and scattering can be considered in addition to specular reflection - EO type-2 is an optional modelling component if supported in a sensing scenario - FFS which deployment scenario(s) EO type-2 will apply Agreement -RAN1 #120 If EO type-2 is modelled in an indirect path, only specular reflection is modeled for EO type-2 - polarization of the indirect path is product of polarization matrix of the target-Rx link, the target, and the Tx-target link, i.e., CPMtx,sp,rx= CPMsp,rx . CPMsp . CPMtx,sp - For the specular reflected ray generated by a EO type-2 in the Tx-target link or the target-Rx link (i.e., Tx-EO type-2-target, or target-EO type-2-Rx), CPMtx,sp or CPMsp,rx is the polarization matrix of EO type-2 - To generate polarization matrix of EO type-2, the procedure in [R1-2409394, R1-2410648] is taken as starting point - FFS applicability if the surface of EO type-2 is tilted

[Moderator’s note] The draft CR capturing the polarization matrix generation for EO type-2 is based on [R1-2409394, R1-2410648] and is endorsed in email discussion [Post-120bis-ISAC-03]. One open issue is on formula 7.9.5-10 and related descriptions.

Besides the current CR, one more procedure is proposed by proponent company to generate the polarization matrix. Based on the offline discussions, both are correct, i.e., the same polarization matrix for EO type-2 is generated. Given it is quite controversial for down selection, it may be up to each company to take a preferred way for source coding.

[FL5] Proposal for offline discussion

- Capture both procedures generating the polarization matrix of EO type-2 in the draft CR for ISAC channel model.

- Down-select one TP from the following two TPs.

TP 1:

The effective polarization matrix of the type-2 EO reflection path is given by [CPMEO=cosγ2sinγ2-sinγ2cosγ2R∥00-R⊥cosγ1-sinγ1sinγ1cosγ1 (7.9.5-10)] Where, - γ1=atan2eθEO, ZODnplane,eϕEO, AODnplane. nplane represents the normal vector of the incident plane. nplane=vtx→w×vw→rx, in which vtx→w=xw-xtx,yw-ytx,zw-ztx and vw→rx=xrx-xw,yrx-yw,zrx-zw. eθEO, ZOD represents the spherical basis vector of incident ray in vertical direction. eθEO,ZOD=cosθEO, ZODcosϕEO, AOD,cosθEO, ZODsinϕEO, AOD,-sinθEO, ZODT. eϕEO,AOD represents the spherical basis vector of incident ray in horizontal direction. eϕEO,AOD=-sinφEO, AOD,cosφEO, AOD,0T. - γ2=atan2eθEO, ZOAnplane,eϕEO, AOAnplane. eθEO, ZOA represents the polar basis vector of scattering ray in vertical direction. eϕEO, AOA represents the spherical basis vector of scattering ray in horizontal direction. eθEO, ZOA=cosθEO, ZOAcosϕEO, AOA,cosθEO, ZOAsinϕEO, AOA,-sinθEO, ZOAT. eϕEO, AOA=-sinϕEO, AOA,cosϕEO, AOA,0T. Alternatively, the polarization matrix R∥00-R⊥ is defined in LCS. γ1 and γ2 are the angles to perform the transformation between LCS to GCS for the type-2 EO as defined in Clause 7.1.3 and 7.1.4.

TP 2:

The effective polarization matrix of the type-2 EO reflection path is given by [CPMEO=cosγ2sinγ2-sinγ2cosγ2R∥00-R⊥cosγ1-sinγ1sinγ1cosγ1 (7.9.5-10)] Two alternatives are provided to generate γ1 and γ2. Alternative 1: - γ1=atan2eθEO, ZODnplane,eϕEO, AODnplane. nplane represents the normal vector of the incident plane. nplane=vtx→w×vw→rx, in which vtx→w=xw-xtx,yw-ytx,zw-ztx and vw→rx=xrx-xw,yrx-yw,zrx-zw. eθEO, ZOD represents the spherical basis vector of incident ray in vertical direction. eθEO,ZOD=cosθEO, ZODcosϕEO, AOD,cosθEO, ZODsinϕEO, AOD,-sinθEO, ZODT. eϕEO,AOD represents the spherical basis vector of incident ray in horizontal direction. eϕEO,AOD=-sinφEO, AOD,cosφEO, AOD,0T. - γ2=atan2eθEO, ZOAnplane,eϕEO, AOAnplane. eθEO, ZOA represents the polar basis vector of scattering ray in vertical direction. eϕEO, AOA represents the spherical basis vector of scattering ray in horizontal direction. eθEO, ZOA=cosθEO, ZOAcosϕEO, AOA,cosθEO, ZOAsinϕEO, AOA,-sinθEO, ZOAT. eϕEO, AOA=-sinϕEO, AOA,cosϕEO, AOA,0T. Alternative 2: - The polarization matrix R∥00-R⊥ is defined in LCS. γ1 and γ2 are the angles to perform the transformation between LCS to GCS for the type-2 EO as defined in Clause 7.1.3 and 7.1.4.

[Moderator’s note] Agreed in Thursday offline session

Proposal for conclusion

- There is no consensus that the endorsed draft CR has problem on polarization matrix generation. The brackets on formula 7.9.5-10 in the draft CR are removed

- Encourage more companies to check the draft CR. If problem is found, RAN1 will revise TR 38.901 by new CR.

[Moderator’s note] Agreed in Friday online session

- Agreement

- The square brackets on formula 7.9.5-10 (copied below) in the draft CR are removed

The effective polarization matrix of the type-2 EO reflection path is given by

[CPMEO=cosγ2sinγ2-sinγ2cosγ2R∥00-R⊥cosγ1-sinγ1sinγ1cosγ1	(7.9.5-10)]

- Encourage companies to check and compare with the results that can be obtained with Alt2 and Alt3 below. If problem is found, RAN1 will revise TR 38.901 by new CR.

Alt2

[CPMEO=cosγ2-sinγ2sinγ2cosγ2R∥00-R⊥cosγ1sinγ1-sinγ1cosγ1	(7.9.5-10)]

With reusing the legacy transformation method for deriving γ1 and γ2.

Alt3

[CPMEO=cosγ2-sinγ2sinγ2cosγ2R∥00-R⊥cosγ1sinγ1-sinγ1cosγ1	(7.9.5-10)]

Where,

-	γ1=atan2-eθEO, ZODnplane,eϕEO, AODnplane. nplane represents the normal vector of the incident plane. nplane=vtx→w×vw→rx, in which vtx→w=xw-xtx,yw-ytx,zw-ztx and vw→rx=xrx-xw,yrx-yw,zrx-zw. eθEO, ZOD represents the spherical basis vector of incident ray in vertical direction. eθEO,ZOD=cosθEO, ZODcosϕEO, AOD,cosθEO, ZODsinϕEO, AOD,-sinθEO, ZODT. eϕEO,AOD represents the spherical basis vector of incident ray in horizontal direction. eϕEO,AOD=-sinφEO, AOD,cosφEO, AOD,0T.

-	γ2=atan2-eθEO, ZOAnplane,eϕEO, AOAnplane. eθEO, ZOA represents the polar basis vector of scattering ray in vertical direction. eϕEO, AOA represents the polar basis vector of scattering ray in horizontal direction. eθEO, ZOA=cosθEO, ZOAcosϕEO, AOA,cosθEO, ZOAsinϕEO, AOA,-sinθEO, ZOAT. eϕEO, AOA=-sinϕEO, AOA,cosϕEO, AOA,0T.

Spatial consistency

General question

[Moderator’s note] We agreed that path dropping after concatenation based on a power threshold. During movement of Tx/target/Rx, the power of each direct/indirect path will change. The set of paths with power higher than threshold change accordingly. However, it is time consuming if path dropping is redo every time when Tx/target/Rx moves, especially for Option 0 for concatenation.

Just echo one comments from OPPO, the intention of proposal is to do path dropping only in the beginning of simulation, while the parameter values, e.g. absolute delay, for each path can be updated based on location of Tx, target and/or Rx

The following are summary on views from companies. Not sure

- Supported by: ZTE, CATT, LGE, vivo, QC, Xiaomi, OPPO

- Not Supported by: Ericsson

- Neutral: BUPT (neglectable impact)

Note: Cluster dropping with power -25 dB lower than maximum cluster power is supported in Step 6 in TR 38.901. however, whether to update the cluster dropping is not explicitly captured TR. The similar handling may be applicable for path dropping, if we don’t have time to clearly make an agreement.

- [Closed][FL1][L] Proposal 7.1-1 for conclusion

When spatial consistency is enabled, the path dropping after concatenation of Tx-target and target-Rx link is not reperformed even if Tx, target, Rx positions change during simulation.

- Note: the large scale/small scale parameter values can still be updated according to the procedure of spatial consistency.

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| LGE | Yes |  |
| Nokia, NSB | Yes |  |
| BUPT |  | We cannot justify this approach solely based on time consumption concerns. Taking the fully convolutional case as an example: Do you have empirical evidence showing that path dropping after concatenation is more computationally expensive than dropping multi-path components within a single snapshot simulation? |
| Vivo | Yes |  |
| CATT, CICTCI | Yes |  |
| OPPO | Yes |  |
| EURECOM | Yes |  |
| Ericsson | No | As mentioned above by the FL, Step 6 in TR 38.901 doesn’t specify whether updates to cluster dropping should be performed or not, instead leaving this for companies’ implementation. Clearly, the same principle should apply in the target channel. Hence there is no need for this proposal. |
| Moderator2 |  | Considering the comments from BUPT and Ericsson, it seems the better way is leave it to implementation. The handling of cluster dropping in Step 5 in section 7.5 in 38.901 can be a reference. So, let’s close this issue. |
| QC | Yes |  |

Cases that spatial consistency are supported or not supported

[Moderator’s note] Based on the discussion in last meeting, it should be quite agreeable if the two links are generated using different parameter value sets of channel model. For background channel for TRP monostatic sensing, it is straightforward there is no spatial consistency for the virtual receivers/reference points of different TRPs.

Based on the inputs from RAN1 #120bis, all received comments are supporting the proposal.

[FL1][H] Proposal 7.2-1

Spatial consistency is not modelled for

- the links that are generated referring to channel models with parameter values of different communication scenarios

  - E.g., between TRP-target/UT link in one scenario and target/UT-UT link in another scenario

- the background channels for TRP monostatic sensing of different TRPs

Supported by: ZTE, CATT, HW, LGE, vivo, QC, BUPT, Xiaomi, OPPO, Ericsson (bullet 1)

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| LGE | Yes |  |
| Nokia, NSB | Yes |  |
| InterDigital |  | For the second bullet (spatial consistency for background channels), does the distance between TRPs affect spatial consistency? |
| BUPT | Yes |  |
| Moderator |  | @IDC: seems we could follow existing rule in 38.901, i.e., for two TRPs (normally with a large distance), we don’t need to consider spatial consistency. Otherwise, we open a discussion without any validation results submitted till now. |
| vivo | Yes |  |
| CATT, CICTCI | Yes |  |
| Huawei, HiSilicon | Yes |  |
| DOCOMO | Yes |  |
| Lenovo | Yes |  |
| EURECOM | Yes |  |
| Ericsson | Yes |  |
| QC | Yes |  |
| Moderator4 |  | Agreed in Tuesday online session |

- Agreement

Spatial consistency is not modelled for

- the links that are generated referring to channel models with parameter values of different communication scenarios

  - E.g., between TRP-target/UT link in one scenario and target/UT-UT link in another scenario

- the background channels for TRP monostatic sensing of different TRPs

[Moderator’s note] For sensing scenario UMi, InH and InF, it is likely we use the channel model referring the same communication scenario to generate TRP-UT/ST link and UT-ST/UT link. However, as discussed by BUPT, the ASD/ZSD may still be different for the two links. Therefore, it is proposed to not model spatial consistency for the links.

Based on the inputs from RAN1 #120bis, all received comments are supporting the proposal.

[FL1][H] Proposal 7.2-2

Spatial consistency is not modelled between TRP-target/UT link and target/UT-UT link for sensing scenario UMi, InH and InF

Supported by: CATT, LGE, vivo, QC, BUPT, Xiaomi, DOCOMO, OPPO, Huawei

| Company | Yes/No | Comments |
| --- | --- | --- |
| LGE | Yes |  |
| InterDigital |  | In office scenario, TRP height is 3m (antenna at ceiling, Table 7.2-2 in TR 38.901) and UT height is 1m. Difference in height between TRP and UT is relatively small compared to other scenarios mentioned in the proposal. Even in this case spatial consistency does not hold between the two links? |
| BUPT | Yes |  |
| Moderator |  | @IDC: please refer to comments/contribution from BUPT in last meeting. For e.g., InH, though BS-target link and target-UE link both follow InH, there is some change on AOD/ZOD. I mean, when we reuse InH to target-UE link, we change the AOD/ZOD according to TR 38.858. Finally, the two links are not exactly same, so it is difficult to do spatial consistency. |
| Vivo | Yes |  |
| CATT, CICTCI | Yes |  |
| Huawei, HiSilicon | Yes | Combine with the [H] Proposal 7.2-1, is it a common understanding that the spatial consistency is not modelled between TRP-target/UT link and target/UT-UT link for all scenario? |
| Moderator |  | @Huawei: if Proposal 7.2-1/2 are agreed, it seems we finally don’t support spatial consistency between TRP-target/UT link and target/UT-UT link for all scenarios. I don’t know an exception by now. |
| DOCOMO | Yes |  |
| QC | Yes |  |
| Moderator4 |  | Agreed in Tuesday online session |

- Agreement

Spatial consistency is not modelled between TRP-target/UT link and target/UT-UT link for sensing scenario UMi, InH and InF.

[Moderator’s note] Assuming Proposal 7.2-1/2 are agreeable, one remaining issue is the spatial consistency between TRP-TRP link and other links with UT/ST in at least one end. With similar reason as Proposal 7.2-2, ASD/ZSD/ASA/ZSA for TRP-TRP link are set according to TRP statistics, while for other links with UT/ST, ASD/ZSD/ASA/ZSA of at least one end (i.e., the ST/UT) doesn’t follow TRP statistics. Therefore, the moderator makes the following proposal to not model spatial consistency for the links.

[FL1][H] Proposal 7.2-3

Spatial consistency is not modelled between TRP-TRP link and any other links for ISAC channel.

| Company | Yes/No | Comments |
| --- | --- | --- |
| CATT, CICTCI |  | OK |
| LGE |  | Can we clarify the applicable scenario for this proposal? In 7.2.2, Umi, InH and InF were specified. The reason is that if UAV with same height as TRP is considered, we may have spatial consistency. |
| Moderator |  | @LGE: for UAV scenario, TRP-TRP link is generated following 38.858, which TRP-UAV link or UE-UAV link is modeled using 36.777. In short, they are different channel model and we cannot model spatial consistency |
| BUPT3 | Yes |  |
| EURECOM | Yes |  |
| QC | Yes |  |
| Moderator4 |  | Agreed in Tuesday online session |

- Agreement

Spatial consistency is not modelled between TRP-TRP link and any other links for ISAC channel.

Additional parameters for spatial consistency

[Moderator’s note] On background channel for UE monostatic sensing, we need a way to maintain spatial consistency of generated background channel during the movement of UE. Please check following proposal from ZTE.

[H] Proposal 7.3-1

- On background channel for UE monostatic sensing, spatial consistency is supported for the following parameters.

| Parameters | Correlation type |
| --- | --- |
| Distance between reference point and Tx | All-correlated |
| Height of reference point | All-correlated |
| LOS AOD between Tx and reference point | All-correlated |

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes | This is for the spatial consistency in the scenarios that two Ues are closed such that their background channel relying on reference points are also correlated. |
| Nokia, NSB | No |  |
| BUPT | Yes |  |
| ZTE | Yes | This is for the spatial consistency in the scenarios that two Ues are closed such that their background channel relying on reference points are also correlated. |
| OPPO | Partially No | The above modelling of spatial consistency is performed only when the spatial consistency is enabled. We prefer to have an “switch” condition to turn on/off the above modeling. |
| Moderator |  | @Nokia, OPPO: maybe we could say “spatial consistency, if enabled, is …”? |

[FL1][H] Proposal 7.3-1-rev1

- On background channel for UE monostatic sensing, spatial consistency, if enabled, is supported for the following additional parameters.

| Parameters | Correlation type |
| --- | --- |
| Distance between reference point and Tx | All-correlated |
| Height of reference point | All-correlated |
| LOS AOD between Tx and reference point | All-correlated |

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| BUPT3 | Yes |  |
| Ericsson | No | To support spatial consistency of mono-static background channel, d3D should be correlated. Even if the 2D distance of Tx-RP1 is correlated with Tx-RP2 2d distance, the same to the heights of two RF, there is no guarantee that the 3D distance between Tx and RP1 is related to that between Tx and RP2. |
| Moderator3 |  | Revised in the Monday offline session |

[FL1][H] Proposal 7.3-1-rev2 for conclusion

On background channel modelling,

- Spatial consistency is not supported for TRP monostatic sensing across different TRPs

- Spatial consistency is not supported for UE monostatic sensing across different Ues

- Additionally spatial consistency is not supported for different RPs for same TRP for TRP monostatic sensing

- Additionally spatial consistency is not supported for different RPs for same UE for UE monostatic sensing

[Moderator’s note] Agreed in Tuesday online session

- Agreement

On background channel modelling,

- Spatial consistency is not supported for TRP monostatic sensing across different TRPs

- Spatial consistency is not supported for UE monostatic sensing across different UEs

- Spatial consistency is not supported across different Reference Points for same TRP for TRP monostatic sensing

- Spatial consistency is not supported across different Reference Points for same UE for UE monostatic sensing

Spatial consistency between multiple scattering points

Huawei

- The XPR correlation between SPs can be modelled as the spatial consistency.

[Moderator’s note] different views are observed on the relation between multiple scattering point for a target and spatial consistency.

- Option 1: When spatial consistency is NOT implemented, channels of the multiple scattering points of a target is independently generated

  - Supported by: OPPO, QC, CATT, Spreadtrum, SS, CMCC, Nokia

- Option 2: When multiple scattering points of target is modelled, spatial consistency is used

  - Supported by: Huawei, ZTE, vivo, BUPT, DOCOMO, Ericsson, LG, IDCC,

It is quite controversial. However, either option doesn’t require new feature for ISAC channel model. I mean, the Rel-19 SI anyway supports both components, multiple scattering points and spatial consistency. The two options are essentially second level work to combine the two components.

Since RAN1 already makes the two components and it seems hard to get a consensus on either option, the moderator suggests RAN1 to clarify the selected option in evaluation phase.

[H] Proposal 7.4-1

- Delete the following sentence from the Step 2 in section 7.9.4.1 in the draft CR on ISAC channel model to TR 38.901.

“The propagation conditions for different STX-SPST links and SPST-SRX links are uncorrelated.”

- Whether to enable spatial consistency when multiple scattering points for a target is used can be discussed in evaluation phase

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes | We prefer above Option 2, at least the LOS/NLOS spatial consistency should be enabled for multiple scattering points. |
| LGE |  | For the 2nd bullet, we also support to model the spatial consistency among multiple scattering points. |
| Nokia, NSB |  | Option 1. Spatial consistency is an additional feature. Option 2 is okay only when the spatial consistency is enabled. |
| BUPT | Yes | The modeling of spatial consistency for multi-point targets is target-size-dependent, with implementation left to individual companies’ discretion. |
| Vivo |  | The electromagnetic wave behaviors reflected and/or scattered by a target can be modelled by multiple points if the size of target is large. Such multiple points are the important components, offering the reflected and/or scattered information together for sensing. If a target is modeled by the multiple points, we believe that the more the correlated points, the more the accurate information collection for sensing. If we model multiple points by an independent way, coherent sensing information will be eliminated, resulting in a poor sensing performance. |
| CATT, CICTCI |  | What we worry about is that Option 2 leads to: when multi-scattering point target is dropped, spatial consistency (for all targets, all channels…) is enforced. This is creating a ‘condition’ on spatial consistency. That’s why we can only go with Option 1 for now. |
| OPPO | Yes | We support proposal to discuss whether to enable spatial consistency in evaluation phase. |
| Huawei, HiSilicon |  | Firstly, the spatial consistency procedure defined in current TR38.901 is used to model the spatial correlation of the propagation channel when UT is moving or when different Uts are located very close. However, it is the different case for the multiple scattering points within one ST, the distance between any two of points is usually in significant proximity, e.g., minimum distance is 0.8m for vehicle, which leads to the correlation coefficient close to 1, even when the UT is static. If the spatial consistency is not modelled to the multiple scattering point within one ST, e.g., some scattering points are LOS condition while others are NLOS condition even when it is in a LOS scenario, and the generated clusters will totally independent for the two sets of scattering points even when they are less than 1m apart, such phenomenon is unreasonable because it does not reflect the realistic situation. Therefore, we still believe the spatial consistency between the SPs within the same ST is still needed. We suggest to update the proposal as: Proposal: When multiple scattering points of target is modelled, spatial consistency is always applied to the scattering points within the sensing target. Even with that, the procedure of spatial consistency can still be added in the section of ‘additional feature’, and all the additional CR needed is as follows: Alternative Proposal: agreeing on the following TP. ----------------------- Start of Text Proposal-------------------- Large scale parameters: Step 2: Assign propagation condition (LOS/NLOS) according to Table 7.4.2-1. The propagation conditions for different BS-UT links are uncorrelated. LOS/NLOS condition of the SPs within the same ST are correlated. ……. These LSPs for different BS-UT links are uncorrelated, but the LSPs for links from co-sited sectors to a UT are the same. The LSPs of the SPs within the same ST are correlated. In addition, these LSPs for the links of Uts on different floors are uncorrelated. ….. The outcome of Steps 1-12 shall be identical for all the links from co-sited sectors to a STX/ST/SRX. The outcome of Steps 1-12 shall be correlated for all the links for the SPs within the same sensing target. -----------------------------End of Text Proposal----------------------------- |
| Moderator |  | Thanks for all inputs. We face similar situation as last meeting. So, the moderator thinks Proposal 7.4-1 should be a good way forward. I add more remarks above the proposal, so that new reader can easily find the logic. Note: the following sentence is not included in the CR, i.e., the CR for now doesn’t restrict the relation between multiple scattering points of target and spatial consistency. |

[Closed][FL1][M] Proposal 7.4-1-rev1 for conclusion

- Delete the following sentence from the Step 2 in section 7.9.4.1 in the draft CR on ISAC channel model to TR 38.901.

“The propagation conditions for different STX-SPST links and SPST-SRX links are uncorrelated.”

- Whether to enable spatial consistency when multiple scattering points for a target is used can be discussed in evaluation phase

| Company | Yes/No | Comments |
| --- | --- | --- |
| CATT, CICTCI |  | We can live with this proposal. |
| Spreadtrum | Yes |  |
| BUPT3 | Yes | The modeling of spatial consistency for multi-point targets is target-size-dependent, with implementation left to individual companies’ discretion. |
| Ericsson |  | Spatial consistency is mandatory for multiple scattering points. RCS model of single-scattering point is used to generate RCS model of multiple scattering points under the condition that the same angles of incident/scattered ray are observed by all scattering points, i.e., all the points are co-located. This may be a little different depending on the relative distance between the points. But they are still correlated. Without spatial consistency, multi-point RCS model doesn’t work. |
| QC | Yes |  |
| Moderator |  | This issue is solved by the agreement made in Tuesday online when we discussion Proposal 7.4-2 |

[Moderator’s note] The agreement from RAN1 #120 is to separately generated LOS condition/pathloss and LOS ray/NLOS clusters for the links of multiple scattering points. From inputs in RAN1 #120bits, all companies are OK with model spatial consistency of the links between STX/SRX and the multiple scattering points.

Our existing agreement on spatial consistency is applicable to links between STX/SRX and targets. Therefore, the following proposal is made.

[FL1][H] Proposal 7.4-2

- Spatial consistency, if enabled, for the links between BS/UT and multiple scattering points of a target are modelled as if multiple targets.

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Nokia, NSB | Yes |  |
| BUPT | Yes | No more comments. |
| Vivo | Yes |  |
| CATT, CICTCI |  | We hesitate to support this proposal. To us, it is reasonable to consider the correlation of multiple scattering points of one target, but this should better be decoupled with spatial consistency (which will enforce spatial consistency to different targets of target channels, background channels…) |
| OPPO | Yes |  |
| MediaTek | Yes |  |
| EURECOM | Yes |  |
| QC | Yes |  |
| PCL | Yes |  |
| Moderator4 |  | Agreed in Tuesday online session |

- Agreement

Spatial consistency can be enabled for multiple scattering points of a target.

Spatial consistency, if enabled, for the links between BS/UT and multiple scattering points of a target are modelled as if multiple scattering points are multiple targets.

3D spatial consistency

[Moderator’s note] This is an essential feature to simulate 3D correlation when UAV can have a vertical speed. On the other hand, if there is not conclusion, we end up with limitation on UAV trajectory evaluation in evaluation phase.

Based on discussion in last meeting, there is a good support to reuse existing correlation distance. I avoid using either vertical or 3D correlation but only capture the basic intention.

[H] Proposal 7.5-1

- Option 1: The existing horizontal correlation distance in Table 7.6.3.1-2 in TR38.901 is reused as vertical correlation distance for ISAC channel at least for UAV scenario, within same ‘Applicability range in terms of aerial UE height (defined in 36.777)’

- Option 2: 3GPP assumes UAV height cannot change in a simulation drop, if spatial consistency is enabled.

- Option 3: The existing horizontal correlation distance in Table 7.6.3.1-2 in TR38.901 is reused to model 3D spatial consistency for ISAC channel at least for UAV scenario, within same ‘Applicability range in terms of aerial UE height (defined in 36.777)’

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes | One question for the sentence below, does it mean there is no vertical spatial consistency for different applicability ranges, e.g. between 9m UE height and 11m ? within same ‘Applicability range in terms of aerial UE height (defined in 36.777)’ Applicability range in terms of aerial UE height. |
| LGE |  | In UAV case, we think that the vertical correlation distance may be different from the horizontal correlation distance. For the different height, the LOS probability will be quite different. But for the same height, the LOS probability will not change so much. As a result, we think that the vertical correlation distance will be much shorter than the horizontal correlation distance. |
| BUPT | Yes |  |
| Moderator |  | @LGE: technical speaking, your comments are correct. On the other hand, the channel model will consider the balance between complexity and accuracy. Given the limited time (last RAN1 meeting), it sounds reasonable we try certain simple proposal otherwise, we may not complete the study especially when there is no validation results for new proposals. |
| Vivo | Yes |  |
| CATT, CICTCI | Yes | Same question as ZTE. Maybe ‘Applicability range’ needs to be modified. |
| OPPO |  | The proposal is not clear enough. Which one of following does it refer to? Opt-1: The existing horizontal correlation distance in Table 7.6.3.1-2 in TR38.901 is reused as 3D correlation distance in modelling of to model 3D spatial consistency for ISAC channel at least for UAV scenario, ….. Opt-2: The existing horizontal correlation distance in Table 7.6.3.1-2 in TR38.901 is reused [still as horizontal correlation distance] to model 3D spatial consistency for ISAC channel at least for UAV scenario, [and another same or different distance is used as correlation distance on vertical direction, which will be a separate proposal], ….. |
| Huawei, HiSilicon |  | Clarification question for the Applicability range in terms of aerial UE height (defined in 36.777): the applicability range for the aerial UE height is different for different parameters, which range is applied? E.g. |
| Moderator |  | @Huawei: thanks for raising this comment. Fortunately, the division of height range of the two parameters are still overlap. I mean, that for LOS probability is further split of range of pathloss. We may leave detailed handling to implementation since anyway in our CR for spatial consistency, we only describe some high level principle or correlation types. |
| Moderator |  | @OPPO: Thanks for pointing out the confusion. Revised in 7.5-1-rev1 now. To @all: From early meeting, companies are generally ok with reuse the horizontal correlation distance (no other way, right?), however, there are different view to use it as vertical correlation distance or 3D correlation distance. One possible way to move forward is we agree on the correlation distance for 3D spatial consistency, then leave other details as implementation choice. |

[FL1] [H] Proposal 7.5-1-rev1

- The existing horizontal correlation distance in Table 7.6.3.1-2 in TR38.901 is used as the correlation distance for 3D spatial consistency for ISAC channel at least for UAV scenario, within same ‘Applicability range in terms of aerial UE height (defined in 36.777)’

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| BUPT3 | Yes |  |
| LGE |  | One question is that what will be the correlation distance if UAVs are not within same applicability range, especially near boundary of each range? |
| EURECOM | Yes |  |
| Ericsson |  | Fine. |
| QC | Yes |  |
| Moderator4 |  | Agreed in Tuesday online session |

- Agreement

The existing horizontal correlation distance in Table 7.6.3.1-2 in TR38.901 is used as the correlation distance for 3D spatial consistency for ISAC channel at least for UAV scenario, within same ‘Applicability range in terms of aerial UE height (defined in 36.777)’.

Solutions for new spatial consistency

[Moderator’s note] We don’t have time to align the understanding on exact solution to model spatial consistency for ST/UT links. However, it is always possible two or many may align their views offline.

Procedure A vs. B for spatial consistency

[Moderator’s note] Based on inputs in RAN1 #120bis, all companies agree that procedure A/B are both inherited, with one company commented it is the default behaviour and no need for an agreement. With such understanding, we don’t need to spend time on this issue again.

Link level channel model

Summary on company views

- Option 1: Based on existing CDL channel model: ZTE, CATT, vivo, DOCOMO

  - Alt 1: add one or more clusters representing target(s): HW, Apple, SK Telecom, BUPT, Xiaomi, QC

  - Alt 2: Update the parameters of one or more cluster to represent target(s)

- Option 2: Reducing the system level channel model for ISAC to single Tx and single Rx: CMCC, MTK

- Option 3: The Tx-target link and target-Rx link are modelled by separate CDL channels, concatenation is then performed to get the target channel; the background channel is modelled by another CDL channel.

Ericsson

- Option 4: Not supported: QC

Parameter values for the target cluster to be decided in evaluation stage: Huawei

certain modification, e.g., the delay, angle, velocity or doppler parameter to the target cluster: vivo

CDL only: Ericsson

CDL & TDL:

Ericsson: For link-level ISAC channel models,

- H_ISAC=H_target+H_background still holds.

- legacy CDL models are reused to model Tx-target and target-Rx links of target channel and Tx-Rx channel of background channel.

- target channel is generated by concatenating Tx-target and target-Rx link

[Moderator’s note] For better understanding, please proponent companies clarify the 3 options.

- [Closed][FL1][M] Question 8-1

For Option 1, please clarify whether/how to apply RCS of a target. Comments are other details are welcome too.

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes | Option 1 with Alt 1 may be more aligned with the logic of the current ISAC SLS design. RCS is reflected in the power of the new cluster(s) which represent target(s) as marked in yellow below. The specific values for normalized delay, power, angles and velocity depend on the type of target, and can be discussed in evaluation phase if time budget is not sufficient in this release. From our view, the angle spread should be much smaller than existing cluster values if a direct path in a new cluster is not introduced. 2° is suggested by the LS R1-2501370 sent by ETSI ISAC ISG. Alternatively, one direct path for sensing target may be enough without intra-cluster spread. |
| BUPT | Yes | Here, we present a feasible solution based on the “add-on” concept from Section 9.7.2’s channel combination approach: - The original TR38.901 CDL model serves as the background channel. For newly added sensing target components, we propose deterministic odelling while considering only LOS+LOS ray. Based on deterministic positioning, we generate path loss for two sub-links and RCS values (A, B1, and B2) before concatenation. For sensing algorithm performance evaluation, the critical factor remains signal-to-noise ratio (SNR). As long as the SNR matches that of the SLS cascaded link, there will be no impact on the assessment of key performance metrics such as sensing accuracy and detection probability. |
| Huawei, HiSilicon | Yes | The purpose for the LLS is to evaluate the physical layer technologies such as the waveform. The physical layer evaluation mainly focusses on the SNR/SINR with the interested sensing KPIs e.g. accuracy. The SNR/SINR is simulated from the SLS that including the RCS and the deployment effect. For the CDL model, the section 7.7.1 in TR38.901 can be reused for the channel generation with the velocity in step11 in Clause 7.5 should be updated as the relative velocity to the ST from the UT perspective. For the TDL model, The Doppler spectrum for each tap is characterized by a classical (Jakes) spectrum shape and a maximum Doppler shift fD where , in which the velocity is the relative value between ST and UT. E.g. if the ST and UT move with the same direction, the v = vut + vst. Since the sensing target may appear in anywhere in a scenario with arbitrary delay and angular. Therefore, the parameters of the sensing target path can be set as a range within the current CDL/TDL and the values can be designed per the evaluation requirements. Table 1 The ISAC link level simulation for CDL-A Cluster #; Normalized delay; Power in [dB]; AOD in [°]; AOA in [°]; ZOD in [°]; ZOA in [°]. 1; 0.0000; -13.4; -178.1; 51.3; 50.2; 125.4. 2; 0.3819; 0; -4.2; -152.7; 93.2; 91.3. 3; 0.4025; -2.2; -4.2; -152.7; 93.2; 91.3. 4; 0.5868; -4; -4.2; -152.7; 93.2; 91.3. 5; 0.4610; -6; 90.2; 76.6; 122; 94. 6; 0.5375; -8.2; 90.2; 76.6; 122; 94. 7; 0.6708; -9.9; 90.2; 76.6; 122; 94. 8; 0.5750; -10.5; 121.5; -1.8; 150.2; 47.1. 9; 0.7618; -7.5; -81.7; -41.9; 55.2; 56. 10; 1.5375; -15.9; 158.4; 94.2; 26.4; 30.1. 11; 1.8978; -6.6; -83; 51.9; 126.4; 58.8. 12; 2.2242; -16.7; 134.8; -115.9; 171.6; 26. 13; 2.1718; -12.4; -153; 26.6; 151.4; 49.2. 14; 2.4942; -15.2; -172; 76.6; 157.2; 143.1. 15; 2.5119; -10.8; -129.9; -7; 47.2; 117.4. 16; 3.0582; -11.3; -136; -23; 40.4; 122.7. 17; 4.0810; -12.7; 165.4; -47.2; 43.3; 123.2. 18; 4.4579; -16.2; 148.4; 110.4; 161.8; 32.6. 19; 4.5695; -18.3; 132.7; 144.5; 10.8; 27.2. 20; 4.7966; -18.9; -118.6; 155.3; 16.7; 15.2. 21; 5.0066; -16.6; -154.1; 102; 171.7; 146. 22; 5.3043; -19.9; 126.5; -151.8; 22.7; 150.7. 23; 9.6586; -29.7; -56.2; 55.2; 144.9; 156.1. ST; [-0.5,10]; [-30,10]; [-180,180]; [-180,180]; [0,180]; [0,180]. Per-Cluster Parameters. Parameter; cASD in [°]; cASA in [°]; cZSD in [°]; cZSA in [°]; XPR in [dB]. Value; 5; 11; 3; 3; 10. Table 8 The ISAC link level simulation for TDL-A Tap #; Normalized delay; Power in [dB]; Fading distribution. 1; 0.0000; -13.4; Rayleigh. 2; 0.3819; 0; Rayleigh. 3; 0.4025; -2.2; Rayleigh. 4; 0.5868; -4; Rayleigh. 5; 0.4610; -6; Rayleigh. 6; 0.5375; -8.2; Rayleigh. 7; 0.6708; -9.9; Rayleigh. 8; 0.5750; -10.5; Rayleigh. 9; 0.7618; -7.5; Rayleigh. 10; 1.5375; -15.9; Rayleigh. 11; 1.8978; -6.6; Rayleigh. 12; 2.2242; -16.7; Rayleigh. 13; 2.1718; -12.4; Rayleigh. 14; 2.4942; -15.2; Rayleigh. 15; 2.5119; -10.8; Rayleigh. 16; 3.0582; -11.3; Rayleigh. 17; 4.0810; -12.7; Rayleigh. 18; 4.4579; -16.2; Rayleigh. 19; 4.5695; -18.3; Rayleigh. 20; 4.7966; -18.9; Rayleigh. 21; 5.0066; -16.6; Rayleigh. 22; 5.3043; -19.9; Rayleigh. 23; 9.6586; -29.7; Rayleigh. ST; [-0.5, 10]; [-30,10]; Rayleigh. |
| MediaTek |  | Our understanding is that the option 1 is not algin with our current SLS common framework. The newly added path is only the H-direct path of target channel, and there is not any reflection on H-indirect path of target channel. Instead, option 2 with simplified SLS design includes all the consensus in previous discussion, so, it is a better choice that option 1. |
| CATT, CICTCI |  | We are fine with Option 1 Alt 1. We think RCS impact can at least be reflected in power of the cluster. |
| DOCOMO | Yes | We have similar views to ZTE. However, we suggest that RCS should be reflected in the powers of two types of sensing clusters. One with short delay and strong power for the cases of sensing targets close to the TRP/UE, and one with long delay and weak power for the cases of sensing targets far from the TRP/UE |
| Moderator |  | @DOCOMO: power related to RCS is already clarified in last note in the Proposal 8-4-rev2/3. Thanks. |
| Ericsson |  | In the tables above, we don’t know why the ST cannot be modelled by any of the existing clusters, given similar parameter values. No velocity is introduced for the ST. Also, the ST power values proposed in the above tables are several orders of magnitude too large. This power should reflect the received/transmitted power which is not very different from the value obtained with the radar equation. Below is our measurement of power of target channel(red) and background channel(blue). |

- [Closed][FL1][M] Question 8-2

For Option 2 if preferred by any company, please clarify

- How to generate the location/distance of the Tx and Rx in link level simulation?

- Any limitation on the relative location target and Tx/Rx?

Comments are other details are welcome too.

| Company | Yes/No | Comments |
| --- | --- | --- |
| CMCC | Yes | To our knowledge, no detail about Doppler frequency generation is provided in current CDL channel in TR 38.901, which writes: “CDL models can be implemented by e.g., coefficient generation Step 10 and Step 11 in Clause 7.5 or generating TDL model using spatial filter from Clause 7.7.4.” However, procedure defined in Step 11 in Clause 7.5 considers only the mobility of UE. And Doppler spectrum defined in TDL model for each tap is characterized by a classical (Jakes) spectrum shape, which also considering only the mobility of UE. Thus, if the ISAC link-level channel model is based on existing CDL-model, we have to discuss the procedure for deterministic Doppler generation for multiple targets (e.g. assign a velocity for each target-cluster in option-1 and modify Step 11 with dual mobility feature in Clause 7.6.10 of TR 38.901). Considering absolute delay modelling (probably lead to major modification to existing CDL-model), target’s Doppler and target’s RCS, we prefer option-2, which needs no additional work. For generation of the location of Tx/Rx, we suggest using current parameters defined in Clause 7.2 (Scenarios) in TR 38.901. Distance between Tx and Rx is set as ISD. And omni antenna is adopted for both Tx and Rx. We prefer that the number and locations of targets are not modelled and are defined as inputs. A shortest distance between targets and Tx/Rx is restricted by Clause 7.2 also. |
| MediaTek | Yes | Regarding the position of BS and UE, it can be regarded as an input parameters for the simulation, and up to compony report or which value will be used can be discussed in evaluation phase. |
| Lenovo | Yes |  |

- [Closed][FL1][M] Question 8-3

For Option 3, please clarify

- how to control the power of target channel by concatenating two CDL channels?

- Do you prefer to allow both LOS condition and NLOS condition for each CDL channel?

Comments are other details are welcome too.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson |  | For the first question: Tx-target channel is generated with one CDL channel, including cluster power, delay and angles of rays. The same to target-Rx channel. When concatenating rays of the two links, RCS of a target is applied. For the second question: it can be up to companies’ choice on which CDL model is used, including LOS ray or not. |

[Moderator’s note] Hope the early questions are helpful to align companies’ understandings.

Due to limited time, suggest to agree on the principle in Rel-19. Given the diverse purpose/scenario for simulation, suggest to keep exact parameter values open and discuss them in evaluation phase.

With them, can we agree on the following proposal from RAN1 #120bis?

[M] Proposal 3-4

- The link-level channel for ISAC is generated by adding one or more cluster(s) representing target(s) to the existing CDL channel model

  - Values of parameters of the added cluster(s) can be discussed in performance evaluation stage

- TDL channel model from exiting 38.901 is not enhanced for ISAC channel model

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE |  | RAN4 usually use TDL channel model. The same solution for ISAC CDL can be reused for TDL. We suggest: - The link-level channel for ISAC is generated by adding one or more cluster(s) representing target(s) to the existing LLS channel model - Values of parameters of the added cluster(s) can be discussed in performance evaluation stage |
| BUPT | Yes | We emphasize that newly added target clusters should not be randomly generated (fully uncontrolled). Deterministic generation based on user-prespecified locations is technically justified. Two implementation approaches are reasonable: - Target cluster contains only LOS + LOS ray generated deterministically. - Cluster parameters are fully determined by target geometry - LOS angles calculated from geometric positions. Angular/delay spreads (recommended smaller than CDL values in 38.901) to be finalized during evaluation. |
| Moderator |  | @ZTE: revised as proposed for RAN1 usage |

[M] Proposal 3-4-rev1

- The link-level channel for ISAC is generated by adding one or more cluster(s) representing target(s) to the existing CDL LLS channel model

  - A new parameter of velocity of the cluster(s) of target(s) is added

  - Values of parameters of the added cluster(s) of target(s) can be discussed in performance evaluation stage

- TDL channel model from exiting 38.901 is not enhanced for ISAC channel model

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| CATT, CICTCI |  | OK |
| Moderator |  | Based on the offline discussions, one note is added. Further, speed for existing clusters are clarified. |

[M] Proposal 3-4-rev2

- The link-level channel for ISAC is generated by adding one or more cluster(s) representing target(s) to the existing CDL LLS channel model

  - A new parameter of velocity is added for the cluster(s) of target(s), and if necessary, for at least some of the existing clusters in existing LLS channel model.

  - Values of parameters of the added cluster(s) of target(s) can be discussed in performance evaluation stage

  - Note: the effect of RCS of target is reflected by the power of the cluster(s) of target(s)

- TDL channel model from exiting 38.901 is not enhanced for ISAC channel model

| Company | Yes/No | Comments |
| --- | --- | --- |
| DOCOMO |  | We think that CDL channel models are appropriate for the reason of providing delay and angle information for detecting sensing targets. In addition, absolute delay Δτ should be added to the top of the table of CDL channel model. |
| Moderator |  | @DOCOMO: after checking with RAN2 colleagues, they didn’t considerΔτ in their evaluation for NR positioning. It seems we don’t need to consider Δτ in LLS |

[FL1][M] Proposal 8-4-rev3

- The link-level channel for ISAC is generated by adding one or more cluster(s) representing target(s) to the existing LLS channel model

  - A new parameter of velocity is added for the cluster(s) of target(s), and if necessary, for at least some of the existing cluster(s) in existing LLS channel model.

  - Values of parameters of the added cluster(s) of target(s), and if necessary, the velocity of at least some of the existing cluster(s), can be discussed in performance evaluation stage

  - Note: the effect of RCS of target is reflected by the power of the cluster(s) of target(s)

| Company | Yes/No | Comments |
| --- | --- | --- |
| BUPT3 | Yes | We think that the newly added cluster(s) of target(s) should not be generated in a completely random manner. It is necessary to introduce some deterministic components, such as AoA, ZoA, AoD, ZoD, delay, etc., which are determined based on the geometric position. |
| Ericsson |  | The note is unclear. The power in the target channel depends on the RCS and respective Tx-target and target-Rx distances among other things. This does NOT mean “that power = RCS”, far from it. Also, without absolute delay modeling the LLS will be useless for determining sensing accuracy. |
| MediaTek |  | Our first preference for sensing LLS is option 2 as commented in previous round. If majority views are ok with this proposal, we also can compromise to live with it. |
| Moderator3 |  | Based on the Monday offline discussions, the proposal is split to multiple options, please check which one is your preference. |

[FL3][M] Proposal 8-4-rev4

Down select one from the following 3 options in RAN1 #121

Option 1:

- The link-level channel for ISAC is generated by adding one or more cluster(s) representing target(s) to the existing LLS channel model

  - A parameter of velocity is added for the cluster(s) of target(s), and if necessary, for at least some of the existing cluster(s) in existing LLS channel model.

  - Values of parameters of the added cluster(s) of target(s), and if necessary, the velocity of at least some of the existing cluster(s), can be discussed in performance evaluation stage

  - Note: the effect of RCS of target is reflected by the power of the cluster(s) of target(s)

Option 2:

- The link-level channel for ISAC is generated by adding one or more cluster(s) representing target(s) to the existing LLS channel model

  - A parameter of velocity is added for the cluster(s) of target(s), and if necessary, use dual mobility model in section 7.6.10 in TR 38.901 to model Doppler of the moving scatters.

  - Values of parameters of the added cluster(s) of target(s), and if necessary, the parameters for the moving scatters, can be discussed in performance evaluation stage

  - Note: the effect of RCS of target is reflected by the power of the cluster(s) of target(s)

Option 3: LLS channel model is not supported in Rel-19.

Option 4: add a general sentence as follows in subsection 7.9.6 on LLS channel model for ISAC

- The link-level channel for ISAC can be generated by adding one or more cluster(s) representing target(s) to the existing LLS channel model.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Moderator5 |  | One more options, i.e., Option 4 is added based on comments received in Wednesday online session |
| Ericsson2 |  | We don’t believe the new cluster can represent a target without geometry. If this is the only candidate solution for LLS channel model, we support Option 3. |
| Moderator6 |  | Revised in offline session Wednesday |

[FL4][M] Proposal 8-4-rev5

Down select one from the following 2 options in RAN1 #121

Option 3: LLS channel model is not supported in Rel-19.

Option 4:

- add a general sentence as follows in subsection 7.9.6 on LLS channel model for ISAC

  - The target channel of link-level channel for ISAC can be generated by adding and/or reusing one or more cluster(s) representing target(s) in the existing LLS channel model.

- No further study on LLS channel model for ISAC in Rel-19.

Supported: ZTE, Huawei, SKT, Apple, Xiaomi, QC, CATT, CMCC, LGE, MTK, NIST, DOCOMO, Sony

[Moderator’s note] the following conclusion is almost agreed in the Thursday online session. Some further discussion is needed

Conclusion (proposed online)

Delete subsection 7.9.6 from the draft CR. For ISAC, no enhancement to LLS channel model is introduced in Rel-19.

Based on further offline discussion, the following wording may be acceptable.

[FL5][M] Proposal 8-4-rev6 for conclusion

Delete subsection 7.9.6 from the draft CR. For ISAC, no enhancement to existing LLS channel model is introduced in Rel-19.

[Moderator’s note] Agreed in Friday online session

- Conclusion

Delete subsection 7.9.6 from the draft CR. For ISAC, no enhancement to existing TR38.901 LLS channel model is introduced in Rel-19.

Hybrid channel model

- [Closed][FL5][L] Question 9.1-1

Please provide your view whether/how ISAC channel based on map-based hybrid channel model should be supported I Rel-19. If so, how to achieve it?

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes | The map based channel model has been supported in the current 38.901, it is easy to make it compatible for sensing. The motivation is to make the existing procedure of hybrid channel modeling workable for ISAC. We don’t need to specify any details of map. The TR effort is quite limited. A TP is suggested below where step 14 for ISAC is newly added in the end of section 8 of TR 38.901: Step 14: Generated deterministic propagation paths in sensing targets channel for ISAC BS and UT can be replaced by ISAC Tx and Rx in the above step 2. The first arrival absolute delay are added for all deterministic paths in the above step 3. The deterministic paths and random paths generated in above step 3 and 4 can include low power paths whose power similar as or larger than that of sensing objects. a) Import sensing object(s) in the digitized map. The sensing object(s) should at least contain the following information: - The 3D geometric information. - The material and shape as well as the corresponding electromagnetic properties including permittivity and conductivity; - The velocity b) Generate power, absolute delay, angle, XPR, Doppler, etc. based on ray tracing for deterministic propagation paths which interact with sensing object channel. - To support for true motion, i.e. the case when a trajectory is specified for a sensing object, a sensing path ID is associated for each deterministic path. The same ID is associated for a path across a number of sensing object locations as far as 1) it has same interaction types in the same order and 2) its interactions occur in same walls or other surfaces. c) Combine the sensing channel in step 14(b) and channel step13. |
| CMCC | No | A map-based hybrid channel model or even ray-tracing-only-based ISAC channel may be needed due to further explorations on clutters in background channel (e.g. clutter suppression enabled by long-time observation). However, it cannot be achieved in a short time. Thus, we suggest not to support map-based hybrid channel model in Rel-19. |
| ZTE2 |  | @CMCC The above TP is quite simple, the motivation is only to update the procedure in section 8.1 rather than to define new map, or new ray tracing methodology. The effort is very limited. |
| OPPO |  | To our best understanding for above new step-14, this add-on of sensing modeling within hybrid channel would be fully deterministic based on ray-tracing, without any involvement of stochastic paths getting around the target. Is this understanding correct? In Rel-14 when first version of 38.901 (called 38.900 at that time) was finalized, hybrid channel model was agreed after passing at least two criteria: - Criteria #1 (complexity comparing to stochastic model): The complexity introduced to evaluation simulation by this hybrid channel model should be acceptable, where such complexity (when comparing to full stochastic model complexity) is controlled by the total number of deterministic paths decided in Step-3. - Criteria #2 (statistics similarity to stochastic model): The channel statistics (mainly in large scale coupling loss) obtained from hybrid channel model is similar to the ones obtained from full-stochastic channel model. It would be interesting to see how the new full-RT based target channel model (w/o background channel) in section 8 stands on above two criteria when comparing to the stochastic version (w/o background channel) in section 7.9. One more question: could steps before step-14 (as in their current shapes) be safely extended to mono-static background channel so that the mono-static ISAC channel model is automatically supported by section 8 with above new step-14? |
| BUPT2 | No | We agree with CMCC. We are concerned that there are too many details to be discussed and finalized. |
| ZTE3 |  | @OPPO Yes, the target channel is fully deterministic based on ray-tracing. As for the criteria #2, since the target channel is usually low power, and does not impact the statistics (mainly in large scale coupling loss), so it should be fine. As for the criteria #1, the original hybrid model now become background, the new thing is deterministic paths for target, i.e. the extra complexity is only caused by target channel based on RT. Compared with ISAC stochastic model with two link concatenation, the complexity is even lower based on our observation. For mono-static background, since the current random clusters in the hybrid mode is anyway generated without a UE location. The procedures is still applicable for mono-static in which the random clusters are added between the real Tx and Rx. |

- Reference

- RP-242348, “Revised SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR”, Xiaomi, AT&T

- TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz, V18.0.0 (2024-03)”

- R1-2503248	Channel modelling for ISAC	Huawei, HiSilicon

- R1-2503373	Views on Rel-19 ISAC channel modelling	vivo, BUPT

- R1-2503446	Discussion on ISAC channel modeling	EURECOM

- R1-2503525	Discussion on ISAC channel modeling	Spreadtrum, UNISOC

- R1-2503577	Discussion on ISAC channel modelling	Samsung

- R1-2503646	Discussion on ISAC channel modelling	Pengcheng Laboratory

- R1-2503698	Discussion on channel modelling for ISAC	ZTE Corporation, Sanechips, CAICT

- R1-2503720	Discussion on ISAC channel modelling	Tejas Network Limited

- R1-2503726	Discussion on ISAC channel modelling	TOYOTA InfoTechnology Center

- R1-2503753	Discussion on ISAC channel modeling	InterDigital, Inc.

- R1-2503761	Discussion on ISAC Channel Modeling	SK Telecom

- R1-2503804	Discussion on ISAC channel modelling	CATT, CICTCI

- R1-2503842	Discussion on ISAC channel modeling	CMCC

- R1-2503859	ISAC Channel Modeling and Measurement Validation	BUPT, CMCC, VIVO, X-Net

- R1-2503893	Discussion on ISAC channel model	Xiaomi, BJTU, BUPT

- R1-2503955	Discussion on ISAC channel modeling	Nokia, Nokia Shanghai Bell

- R1-2503969	Discussion on ISAC Channel Modeling	Tiami Networks

- R1-2503991	Channel modelling for integrated sensing and communication with NR	NVIDIA

- R1-2504013	Discussion on ISAC Channel Modeling     	NIST

- R1-2504054	Discussion on ISAC channel modelling	China Telecom

- R1-2504069	Remaining issues on ISAC Channel Modeling	Sony

- R1-2504110	Discussion on ISAC Channel Modelling 	Panasonic

- R1-2504119	Discussion on channel modelling for ISAC	CALTTA

- R1-2504159	Discussion on ISAC channel modelling	Pengcheng Laboratory

- R1-2504168	Discussion on ISAC channel modelling	Pengcheng Laboratory

- R1-2504221	Study on ISAC channel modelling	OPPO

- R1-2504240	Discussion on Channel Modelling for ISAC	Lenovo

- R1-2504269	Discussion on ISAC channel modelling	MediaTek Inc.

- R1-2504337	Discussion on ISAC channel modelling	Apple

- R1-2504405	Discussion on ISAC channel modelling	Qualcomm Incorporated

- R1-2504455	Discussion on ISAC Channel Modelling	Ericsson

- R1-2504511	Discussion on ISAC Channel Modelling	NTT DOCOMO, INC.

- R1-2504567	Discussion on ISAC channel modelling	LG Electronics

- R1-2504161	Summary #1 on ISAC channel modelling	Moderator (Xiaomi)

- R1-2504162	Summary #2 on ISAC channel modelling	Moderator (Xiaomi)

- R1-2504163	Summary #3 on ISAC channel modelling	Moderator (Xiaomi)

- R1-2504164	Summary #4 on ISAC channel modelling	Moderator (Xiaomi)

- R1-2504165	Summary #5 on ISAC channel modelling	Moderator (Xiaomi)

- ANNEX: All agreements

- RAN1 #116

Agreement

The common framework for ISAC channel model is composed of a component of target channel and a component of background channel,

HISAC=Htarget+Hbackground

- Target channel Htarget includes all [multipath] components impacted by the sensing target(s).

  - FFS details of the target channel

- Background channel Hbackground includes other [multipath] components not belonging to target channel

  - FFS details of the background channel

- FFS whether/how to model environment object(s), i.e., object(s) with known location, other than sensing target(s)

  - FFS whether/how to model propagation path(s) between the target(s) and the environment object(s)

- FFS whether/how to model propagation path(s) between the target(s) and the stochastic clutter(s)

- Note: the notation HISAC can be revised later if needed

- RAN1 #116bis

Agreement

The following cases of radio propagation in the target channel are considered for the study

| Case | Tx-target | Target-Rx |
| --- | --- | --- |
| 1 | LOS condition | LOS condition |
| 2 | LOS condition | NLOS condition |
| 3 | NLOS condition | LOS condition |
| 4 | NLOS condition | NLOS condition |

- Case 1/2/3/4 can be considered for bistatic sensing mode

- At least Case 1/4 can be considered for monostatic sensing mode

- Note: It doesn’t imply the channel response for each link is separately generated then concatenated

- FFS how to determine LOS condition and NLOS condition, e.g., based on LOS probability, or determined based on geometrical locations of environment object (EO).

- In LOS condition, line of sight ray(s) are present between Tx/Rx and target, and there may or may not exist non-line of sight ray(s) between Tx/Rx and target too

- In NLOS condition, there only exist non-line of sight ray(s) between Tx/Rx and target

Agreement

- In the target channel between Tx and Rx, scattering of a sensing target can be modelled as single scattering point or multiple scattering points

- FFS one or multiple incoming/output rays corresponding to a scattering point

- FFS how to select single or multiple scattering points for the target, e.g. depending on the distance between target and Tx/Rx, size/shape of target, etc.

- Note: the sensing target can be assumed in far field of sensing Tx/Rx.

- FFS details to model the single or multiple scattering points

Agreement

RCS of a physical object shows dependency to at least the following factors:

- Type of the object

  - The size of the object

  - The material of the object

  - The shape of the object

- Orientation of the object

- FFS: Distance between Tx/Rx and the object

- The incident angle and scatter angle

- The carrier frequency

- polarization of the transmitter and receiver

- FFS Temporal or spatial consistency

- FFS antenna pattern

- FFS whether/how to model the above factors in the CR, e.g. with an RCS model with a scattering point

Agreement

EO is a non-target object with known location.

- FFS other known parameters of the EO

- FFS details on EO modelling

The following options for EO modelling are considered for further study

  - Option 1: EO is modelled different from a sensing target

    - Applicable at least for an EO having extremely large size (referred as EO type-2 for discussion purpose)

    - FFS modeled similar to section 7.6.8 ground reflection in TR 38.901

    - FFS EO modelling impacts the target channel and/or the background channel

  - Option 2: EO is modeled same/similar as a sensing target

    - Applicable for an EO having comparable physical characteristics as a sensing target, (referred as EO type-1 for discussion purpose)

    - FFS Applicable for EO type-2

    - FFS EO modelling impacts the target channel and/or the background channel

  - Option 3: EO is modeled and its location is determined from a stochastic clutter generated following the cluster generation in TR 38.901

    - FFS details

  - Option 4: EO is not modelled

  - Other options are not precluded

  - Note: it is not precluded that multiple options can be supported in the channel modelling

Agreement

The following options are considered for further study to model the target channel for a target

- Option 1: modelled by concatenation of path(s) from Tx to target and from target to Rx

- Option 2: modelled by Tx-to-Rx path(s) satisfying Tx-target-Rx geometry

- Option 3: combination of Option 1 and Option 2

Agreement

If a target is modelled with single scattering point, the following options to model RCS of the target are considered for further study.

  - Option 1: Random RCS value generated by a statistical distribution, depending on the factor(s) having impacts on the RCS modelling.

    - FFS the distribution.

    - FFS the factor(s)

  - Option 2: Deterministic RCS value is defined by a function and/or a table, depending on the factor(s) having impacts on the RCS modelling

    - Note: Constant RCS for a target type can be a special case of Option 2

    - FFS the factor(s)

    - FFS details of function and/or table

  - Option 3: combination of Option 1 & 2, e.g., RCS value is generated by combining a deterministic component and a randomly generated component.

  - FFS application of each option to large scale fading and/or small scale fading

  - FFS target with multiple scattering points

Agreement

- Interested companies are encouraged to submit validation results together with their proposal for ISAC channel modelling

- Up to each company to select the way for validation

  - Option 1: Experimental results

  - Option 2: Experimental results to validate a ray-tracing model, then the ray-tracing based results to validate the ISAC channel model

    - Note: the layout of the scenario used for validation is up to company choice

Agreement

ISAC channel model for link level simulation is to be discussed after the system level channel model is sufficiently stable with basic functionalities.

- RAN1 #117

Agreement

- Multiple sensing targets can be modelled in the ISAC channel of a pair of sensing Tx and sensing Rx

  - FFS whether to model a propagation path from Tx to Rx interacting with more than one sensing target

- The same sensing target can be modelled in the ISAC channels of multiple pairs of sensing Tx and Rx

Agreement

- For discussion purpose, the propagation paths in the target channel are classified

  - The direct path, i.e., LOS ray from Tx to target + LOS ray from target to Rx

  - The indirect paths, i.e., any propagation path other than the direct path, including

    - LOS ray from Tx to target + NLOS ray from target to Rx

    - NLOS ray from Tx to target + LOS ray from target to Rx

    - NLOS ray from Tx to target + NLOS ray from target to Rx

- For radio propagation Case 1,

  - For a direct path, the following parameters are [deterministically] generated at least based on the geometry location of Tx, target and Rx

    - AoA/ZoA at Rx

    - AoD/ZoD at Tx

    - AoA/ZoA/AoD/ZoD at target

    - delay

    - FFS initial phase

    - Doppler

    - FFS power/polarization including the impact of RCS

    - FFS the number of direct path(s) for a target

  - FFS on detailed modelling of indirect path(s)

- FFS on details of modelling of indirect paths in  radio propagation Case 2/3/4

- To generate the channel coefficients of direct/indirect path(s) in the target channel, the channel coefficient generation function in step 11 in section 7.5 of TR 38.901 (e.g., formula 7.5-22) is used as the start point

  - Note: modification to step 11 is deemed necessary

  - FFS adding impact of small scale RCS

  - FFS Doppler

Agreement

- Spatial consistency should be supported for ISAC channel

- Spatial consistency should be supported based on movement of sensing Tx, sensing target and/or sensing Rx

  - FFS EO handling

Agreement

When the stochastic cluster is used to generate the indirect paths in the target channel of a target

  - The stochastic cluster generation in section 7, TR 38.901 is used as starting point.

    - FFS a stochastic cluster is generated between Tx and Rx satisfying Tx-target-Rx geometry, or between Tx/Rx and target

      - FFS modification to stochastic cluster generation in section 7, TR 38.901

      - FFS use of sub-cluster to model the indirect paths

Note: RAN1 continues studying using EO to generate the indirect paths in the target channel of a target

Agreement

When the stochastic cluster is used to model indirect path in the target channel

- For bistatic, the LOS condition from Tx to target and from target to Rx is determined separately for a target

  - FFS: The correlation of LOS condition of Tx-target and Rx-target links of a target

- For monostatic, a same LOS condition is determined for Tx to target and target to Rx

- The LOS condition from Tx to target and/or from target to Rx is determined with the LOS probability

  - The probability schemes in existing 3GPP TRs, e.g., TR 38.901. TR 36.777, TR 37.885, etc. are considered as start point

  - FFS: How to consider the impacts of target height on LOS probability.

Agreement

When stochastic cluster is used to model indirect path in the target channel, down-select between the following options

- Option 1: modelled by concatenation of path(s) from Tx to target and from target to Rx

  - For each of the Tx-target link and target-Rx link,

    - The parameters delay, power, angle, [initial phase], [Doppler] of NLOS ray(s) in the link Tx-to-Target or Target to RX are generated

      - FFS following cluster generation in section 7, 38.901

    - The target channel is generated by concatenating the parameters of the Tx-target link and target-Rx link.

      - FFS on Convolutional or 1-by-1 coupling or 1-to-many coupling

  - FFS how to combine the clusters in target channel and the clusters in background channel

- Option 2: modelled by Tx-to-Rx path(s) satisfying Tx-target-Rx geometry of the direct path

  - The parameters delay, power, angle, initial phase of a stochastic (sub-)cluster between Tx and Rx are generated following cluster generation in section 7, 38.901

    - The parameters [delay], [power], [angle], [Doppler] of the (sub-)cluster are updated by the target property

  - FFS how to combine the clusters in target channel and the clusters in background channel

- RAN1 #118

Agreement

if RCS related coefficient of a scattering point is included in small scale, the RCS related coefficients are separately determined for different pairs of incident/scattered ray(s) at the scattering point.

Agreement

For radio propagation Case 1, for modelling the target channel of a target with single scattering point,

- To model a direct path, a single LOS ray from Tx to target and a single LOS ray from target to Rx are generated

  - AoA/ZoA of the direct path at Rx, AoD/ZoD of the direct path at target are generated at least based on the 3D location of target and Rx in the global coordinate system

  - AoD/ZoD of the direct path at Tx, AoA/ZoA of the direct path at target are generated at least based on the 3D location of Tx and target in the global coordinate system

  - The Delay of the direct path = (d3D_tx_target + d3D_target_rx)/c

  - The Doppler of the direct path is generated by spherical unit vectors by AoD/ZoD at Tx, by spherical unit vectors by AoA/ZoA at Rx, and velocity of Tx, target and Rx

  - The power of the direct path is generated as the product of the power of the LOS ray from Tx to target, the power of the LOS ray from target to Rx, and the effect of RCS

  - FFS initial phase

  - FFS how to model RCS, polarization of target

- FFS number of direct paths

- FFS on detailed modelling of indirect path(s)

- FFS applicability of direct path generation to each scattering point when the target is modelled as multiple scattering points

Agreement

For the target channel of a target with single scattering point, when stochastic cluster is used to model an indirect path in the target channel,

- An indirect path in small scale is modelled by concatenation of path(s) from Tx to target and from target to Rx, i.e., Option 1 in the agreement of RAN1 #117

  - AoD/ZoD/AoA/ZoA from Tx to target or from target to Rx are

    - generated for a LOS ray at least based on the 3D location of Tx/Rx and target in the global coordinate system

    - stochastically generated for a NLOS ray using section 7, 38.901 as starting point

  - Delay is sum of delay of LOS/NLOS ray from Tx to target and the LOS/NLOS ray from target to Rx

  - Doppler is generated by spherical unit vector by AoD/ZoD at Tx and velocity of Tx, by spherical unit vector by AoA/ZoA at Rx and velocity of Rx, and by spherical unit vectors by AoA/ZoA/AoD/ZoD at target and velocity of target

    - FFS The mobility of stochastic clutter

  - The power of the indirect path is generated as the product of the power of the LOS/NLOS ray from Tx to target, the power of the LOS/NLOS ray from target to Rx and the effect of RCS

  - FFS initial phase

  - FFS how to model effect of RCS at target

  - FFS whether/how to model polarization at target

  - FFS How to reduce complexity

  - FFS applicability of the indirect path generation to each/a single scattering point when the target is modelled as multiple scattering points

Working assumption

The RCS related coefficient of a scattering point can be modelled with two components, i.e., linear value RCS = A*B

- A first RCS component A (m2) is included in large scale

  - FFS the first RCS component is deterministic or stochastic

  - FFS The first component is dependent on incident and scattered directions at target

- A second RCS component B (unit ratio) is included in small scale

  - FFS The second component is dependent on incident and scattered directions at target

  - FFS the second RCS component is deterministic or stochastic or combination

- Note: RCS component A or B can be disabled by setting its linear value to 1

  - Whether to disable a component can be discussed per target type

- FFS how to determine A and B for each target

- FFS whether/how to model polarization impact at target

- FFS whether/how to normalize power accounting for target channel and background channel

Agreement

The impact of a scattering point of the target in the target channel is modelled by a scalar RCS value RCS times a complex-valued 2x2 polarization matrix CPMsp, i.e., RCS∙CPMsp

  - FFS whether CPMsp is angular/ray-dependent or independent

  - FFS whether polarization matrix CPMsp is modelled assuming specular reflection or random coefficient for diffraction or scattering

  - FFS whether polarization matrix CPMsp is explicitly modelled or merged with other polarization matrixes from Tx to target and/or from target to Rx

Agreement

For modeling stochastic cluster in background channel, in order to define the background channel for TRP-UE and UE-TRP bistatic sensing mode,

- The large scale and small scale parameters defined in TR 38.901, TR 37.885, TR 36.777 are used as start point

In order to define the background channel for TRP-TRP and UE-UE bistatic sensing mode,

- RAN1 to study how to model background channel

  - Option 1: The large scale and small scale parameters defined in TR 38.901, TR 38.858, 37.885, 38.859 are used as start point

  - Option 2: New channel model based on measurement results or ray-tracing model validated by experimental results

FFS whether/how to do power normalization between target channel and background channel

Agreement

In order to define the background channel for TRP and UE mono-static sensing mode,

- RAN1 to study how to model the background channel

  - Option 1: randomly drop at least one virtual Rx, and then the background channel is generated based on the channel generated as TR 38.901 between the real Tx and each virtual Rx for a scenario

    - FFS EO is modelled in the background channel

  - Option 2: New channel model based on measurement results or ray-tracing model validated by experimental results or radar literatures

    - FFS EO is modelled in the background channel

  - Option 3: the locations of clusters in the target channel are deterministically generated, then the background channel is generated using the clusters with the determined locations.

  - Other options are not precluded

Agreement

When EO type-2 is modelled, specular reflection is considered to model EO type-2 using section 7.6.8 of TR 38.901 as reference

- As starting point, the effect of type-2 EO (i.e., in the path node1-EO-node2) is modelled as bR∥specular00-R⊥specular, b is a scaling factor (e.g., c equals to d3DdGR relative to LOS ray in section 7.6.8 in TR 38.901)

  - FFS any update to b,R∥specular,R⊥specular

  - FFS any update taking EO orientation into account

- FFS any changes based on section 7.6.8 of TR 38.901 if EO type-2 has finite size

- FFS whether diffraction and scattering can be considered in addition to specular reflection

- EO type-2 is an optional modelling component if supported in a sensing scenario

  - FFS which deployment scenario(s) EO type-2 will apply

- RAN1 #118bis

Agreement

RAN1 strives to define a single option per target per monostatic/bistatic sensing mode from the following two options to generate RCS values/patterns for a scattering point of a target.

- Option 2: The RCS=A*B of a scattering point can be generated by

  - The component A is commonly applied to any incident/scattered angles at the scattering point

    - A is [mean] RCS value. FFS value(s) A

      - Note: Mean RCS value is defined as the mean value of the distribution of RCS

  - The component B

    - B is generated by [log-normal] distribution, the related [log-normal] distribution has mean μ=1 and variance V, FFS σ2

      - B is separately generated for each direct/indirect path at the scattering point. FFS correlation dependent on the incident/scattered angles of the direct/indirect paths

  - FFS whether/how power of all generated direct/indirect paths need to be normalized considering impact of RCS

- Option 3: The RCS=A*B=A*B1*B2 of a scattering point can be generated by

  - The component A is commonly applied to any incident/scattered angles at the scattering point

    - FFS: A = 1 m2 or [mean] RCS value

      - Note: Mean RCS value is defined as the mean value of the distribution of RCS

  - The component B is further split into B1, B2, i.e., B=B1*B2

    - B1 is deterministic based on incident/scattered angles

      - FFS: B1 is defined by a function or by a table

    - B2 is generated by [log-normal] distribution, the related [log-normal] distribution has mean μ=1 and variance V, FFS σ2

      - B2 is separately generated for each direct/indirect path at the scattering point. FFS correlation dependent on the incident/scattered angles of the direct/indirect paths

  - FFS whether/how power of all generated direct/indirect paths need to be normalized considering impact of RCS

Agreement

RCS Option 3 is selected to model RCS of UAV with single scattering point for monostatic

  - B2 of UAV is modelled using log-normal distribution for monostatic

  - Different mean RCS values can be supported for UAV due to different size, shape, frequency, etc.

  - For UAV of small size (option 2 for UAV size in UAV parameters table)

    - B1=1

    - A is mean RCS value

  - For UAV of large size (option 1 for UAV size in UAV parameters table)

    - B1 have dependency on incident/scattered angles

    - A is mean RCS value

Agreement

To model the effect of polarization for each direct/indirect path:

- Polarization of a direct/indirect path is product of polarization matrix of Tx-target link, the target, and the target-Rx link

  - Total polarization of a direct/indirect path is CPMtx,sp,rx= CPMsp,rx . CPMsp . CPMtx,sp

    - For a LOS ray from Tx to target or from target to Rx, CPM=100-1 for CPMtx,sp or CPMsp,rx

    - For a NLOS ray generated by a stochastic cluster from Tx to target or from target to Rx, CPMtx,sp or CPMsp,rx is generated by XPR ratio  and initial random phases referring to TR 38.901 as start point

    - FFS how to normalize on CPMtx,sp,rx

    - FFS CPMsp of a scattering point of the target

- FFS: how to model the effect of polarization when EO type-2 is present

Agreement

A single direct path is modeled for a scattering point of target

- In each of the Tx-target and target RX links, the first NLOS cluster is generated with same delay as the LOS ray (when the absolute delay modelling of Δτ as in section 7.6.9, TR 38.901 is not applied) and with the same direction as the LOS ray.

  - FFS how to generate NLOS cluster when Δτ is applied

Agreement

In order to generate each of the Tx-target link and target-Rx link in the target channel, the large scale and small scale parameters defined in existing 3GPP TRs, e.g., TR 38.901. TR 36.777, TR 37.885, TR 38.858, TR 38.859, TR 38.802, TR 38.854, etc. are used as starting point

Agreement

On the background channel for TRP-TRP and UE-UE bistatic sensing mode, the large scale and small scale parameters defined in TR 38.901, TR 38.858, 37.885, 38.859 are used as starting point

- Update on values of the LSP/SSP parameters can be discussed based on validation data acquired by measurement or ray-tracing model

  - FFS The power threshold for removing clusters in step 6, i.e., -25 dB is revised to X<-25 dB. FFS X

- FFS whether/how to resolve the inconsistency between TRP-TRP channel according to TR 38.858 and the TRP-target (UAV) channel according to TR 36.777 when UAV and TRP are set to same height

Agreement

3D spatial consistency needs to be studied for at least UAV scenario

Agreement

In LOS condition between sensing Tx/Rx and target, the power of LOS ray is generated following power of LOS ray in TR 38.901.

Agreement

The following options are to be studied for the concatenation of Tx-target and target-Rx link in the target channel

- Direct path (if present) is always kept

- Indirect paths of LOS+NLOS, NLOS+LOS (if present) are generated

- On other indirect paths of NLOS + NLOS

  - Option 0: ray level full convolution between Tx-target link and target-Rx link for radio propagation Case 1/2/3/4

  - Option 0A: ray level full convolution between Tx-target link and target-Rx link only for radio propagation Case 4

  - Option 1: cluster level full convolution between Tx-target link and target-Rx link, then 1-by-1 coupling rays within each pair of clusters for radio propagation Case 1/2/3/4

  - Option 1A: cluster level full convolution between Tx-target link and target-Rx link, then 1-by-1 coupling rays within each pair of clusters only for radio propagation Case 4

  - Option 2: cluster level 1-by-1 coupling between Tx-target link and target-Rx link, then 1-by-1 coupling rays within each pair of clusters for radio propagation Case 1/2/3/4

  - Option 2A: cluster level 1-by-1 coupling between Tx-target link and target-Rx link, then 1-by-1 coupling rays within each pair of clusters only for radio propagation Case 4

  - Option 3: ray level 1-by-1 coupling between Tx-target link and target-Rx link for radio propagation Case 1/2/3/4

  - Option 3A: ray level 1-by-1 coupling between Tx-target link and target-Rx link only for radio propagation Case 4

- Note: reducing the number of rays per cluster and/or reducing the number of clusters can be considered for the options above

- Any indirect path with power metric less than [threshold] is dropped

  - the power metric of a path is the product of power of a ray in Tx-target link, power of a ray in target-Rx link and RCS of the pair of rays

  - FFS power normalization of target channel after path dropping

  - FFS the set of remaining indirect paths can be updated during movement of Tx, target or Rx

- RAN1 #119

Agreement

Bistatic RCS values for a scattering point of a target are obtained by fixing an incident direction in LCS of target and varying the scattered directions in LCS of target; then changing to other incident direction.

Agreement

- To generate indirect paths of NLOS ray + NLOS ray in the target channel

  - Option 0 is recommended, i.e., ray level full convolution between Tx-target link and target-Rx link for radio propagation Case 1/2/3/4

  - Option 3 to generate a reduced number of indirect paths of NLOS ray + NLOS ray is recommended, i.e., ray level 1-by-1 random coupling between Tx-target link and target-Rx link is supported for radio propagation Case 1/2/3/4

    - If number of rays in the two links are different, e.g., M1, M2 respectively for link 1 and link 2,

      - If M1<M2, randomly M1 rays are selected in link 2, otherwise randomly M2 rays are selected in link 1

  - Other methods are up to company choice for complexity reduction

  - Both option 0 and 3 will be calibrated independently. Company should report which option is used in calibration

- The power threshold for path dropping is X=[-25] dB

  - X is relative to the strongest indirect path in the target channel

- FFS: further power normalization of target channel is performed after path dropping,

- Note: power normalization when target channel and background channel are combined can be discussed separately

- FFS The set of remaining indirect paths can be updated during movement of Tx, target or Rx

Agreement

The following RCS models are supported when human is modelled with single scattering point for monostatic, where different RCS values and/or models can be supported for human due to different size, shape, frequency, etc.

- Model 1

  - B1=0 dB

  - A is mean RCS value

  - B2 is modelled using log-normal distribution

- Model 2

  - B1 have dependency on incident/scattered angles, with further down-selection among the alternatives below:

    - Alt 1: formulated similar as the antenna radiation power pattern in 38.901

    - Alt 2: a function

    - Alt 3: Lookup table

  - B2 is modelled using log-normal distribution

  - FFS RCS component A

- FFS: conditions for using which model

Agreement

The following RCS model is supported when vehicle is modelled with single scattering point for monostatic, where different RCS values can be supported for vehicle due to different size, shape, frequency, etc.

  - B1 have dependency on incident/scattered angles, with further down-selection among the alternatives below:

    - Alt 1: formulated similar as the antenna radiation power pattern in 38.901

    - Alt 2: a function

    - Alt 3: Lookup table

  - B2 is modelled using log-normal distribution

  - FFS RCS component A

Agreement

When vehicle is modelled with multiple scattering points for monostatic, where different RCS values can be supported for vehicle due to different size, shape, frequency, etc.

- the recommended five scattering points are located in front, left, back, right and roof side of the vehicle

- the following RCS model is supported for each scattering point

  - B1 have dependency on incident/scattered angles, with further down-selection among the alternatives below:

    - Alt 1: formulated similar as the antenna radiation power pattern in 38.901

    - Alt 2: a function

    - Alt 3: Lookup table

  - B2 is modelled using log-normal distribution

  - FFS RCS component A

Agreement

EO type-1 (when modelled) is modelled in the same way as a sensing target in the ISAC channel model.

Agreement

- If blockage/forward scattering between sensing targets is not considered, a propagation path from Tx to Rx interacting with more than one sensing targets is not modelled

- FFS whether/how blockage/forward scattering can be modelled in the target channel.

Agreement

- Doppler for a target including both macro-Doppler and micro-Doppler can be modeled using a unified formula,

rrx,n',m'Tt.vrxt+rp,n',m'Tt∙vspt+rtx,n,mTt.vtxt+rp,n,mTt∙vsptλ0+ft

Where,

    - rrx,n',m'T is the spherical unit vector at receiver for the link from Rx to the scattering point

    - rtx,n,mT  is the spherical unit vector at transmitter for the link from Tx to the scattering point

    - rp,n',m'T is the spherical unit vector at the scattering point for the link from the scattering point to Rx

    - rp,n,mT is the spherical unit vector at the scattering point for the link from the scattering point to Tx

  - Dual mobility model in 7.6.10, TR 38.901 is used as start point to model Doppler effect ft due to movement of stochastic clusters, i.e., 2αn,mDn,mλ0

    - ft is only applicable for indirect path

    - Support one term of 2αn,mDn,mλ0 for indirect path of LOS ray+NLOS ray, NLOS ray+LOS ray

    - Support two terms of  2αn,mDn,mλ0 for indirect path of NLOS ray+NLOS ray

  - Doppler is separately determined for each of the multiple scattering points of a target

  - vspt can include macro-Doppler and/or micro-Doppler motion, 
vspt=vmacrot+vmicro,pt

  - FFS: maximum speed of moving scatterers

  - FFS: ratio of moving scatterers among all scatterers

Agreement

- The following options are supported to generate the combined ISAC channel

  - Option 1: The ISAC channel of a pair of sensing Tx/Rx is obtained by summing the target channel(s) and background channel, i.e., power normalization is not performed.

  - Option 2: As an additional modelling component, power normalization is performed when summing the target channel(s) and background channel, to keep the same/similar channel power as the background channel without target. Down select between

    - Alt 1: Power normalization on both target channel and background channel

    - Alt 2: Power normalization on background channel only

    - Alt 3: the target channel of a target will replace one cluster in the background channel

- FFS Blockage is modelled for the background channel due to sensing target and/or EO type-2

- FFS condition to select option, e.g. depending on scenario, sensing mode, number of target/EO type-2

Agreement

To model the polarization matrix of a direct/indirect path at a scattering point of an object other than EO type-2, the polarization matrix of the scattering point, i.e., CPMsp,i is modelled by αi,1,αi,2,βi,1,βi,2, and initial random phases Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ, i.e., CPMsp,i=αi,1expjΦsp,iθθβi,1expjΦsp,iθϕβi,2expjΦsp,iϕθαi,2expjΦsp,iϕϕ

  - The initial random phase Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ is [uniformly distributed within -π,π]

  - FFS correlation between αi,1,αi,2,βi,1,βi,2

  - FFS specular reflection

  - FFS: CPM normalization

The following options are considered for further study, down select one option from the following

- Option 1: αi,1=αi,2=1, βi,1=βi,2=κsp,i-1 is generated for path i, where κsp,i is XPR ratio

  - κsp,i is randomly generated by log-normal distribution. FFS mean/variance of the distribution

- Option 2: αi,1=1, αi,2,βi,1,βi,2 are variables generated for path i

- Option 3: αi,1,αi,2,βi,1,βi,2 are variables generated for path i

  - CPMsp,i defined in LCS

- Option 4: αi,1=αi,2=1, βi,1=βi,2=0 is generated for path i

Agreement

The finite size of the EO type-2 affects identification of specular reflection point. In the target channel, EO type-2 is modelled only if the specular reflection point is in the area of the EO type-2.

Agreement

Component B2 of RCS is upper bounded by kσ dB for the log-normal distribution, where σ is the standard deviation of B2 in dB. FFS the value of k.

Agreement

When the EO type-2 is modelled in the target channel, down select between the following options to determine the LOS condition of the Tx-target link and target-Rx link

- Option A: If type-2 EO is in the LOS ray of one link, the link is determined as NLOS condition, and otherwise use the LOS probability equation to determine the LOS/NLOS condition

  - FFS changes to the LOS probability defined in existing TRs

  - FFS details on blockage by EO type-2

- Option B: Use the LOS probability equation to determine the LOS/NLOS condition of one link, and then the impacts of type-2 EO is modeled by a blockage model

- RAN1 #120

Agreement

For bistatic/monostatic RCS

- RCS values/pattern for a scattering point of a target for bistatic sensing is generated by A*B1*B2 (i.e., Option 3 from the agreement in RAN1 #118bis)

- RCS values/pattern obtained by setting the same incident/scattered angle in the RCS model for bistatic sensing should be aligned with RCS for monostatic sensing

Agreement

RCS model and application in ISAC channel generation

- To define the RCS model (RCS=A*B1*B2) for a scattering point of a target, when the target type is vehicle, large size UAV, human with RCS model 2, AGV

  - The values/pattern of the product of component A and B1, i.e., A*B1 is given per target type, expressed in dBsm scale

  - Component A is expressed in dBsm scale. B1 is dependent on A*B1 and value of component A.

    - A is equal to a single value per target type

      - FFS: this allows different values for the same target type with different size, if needed

      - FFS: this allows different values for monostatic and bistatic sensing, if needed

  - Component B2 follows log-normal distribution. The mean μB2_dB and variance σB2_dB2 used to characterize 10lgB2 satisfied a fixed relation μB2_dB=-ln1020σB2_dB2.

- In the procedure of generating ISAC target channel, B1*B2 is applied after coupling of rays for a STX-SPST link and the corresponding SPST-SRX link before path dropping

- In the procedure of generating ISAC target channel, the following power scaling factor is applied in the last step in target channel generation (i.e., step 14 in the running CR).

LTX-SPST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2

Where,

    - PLdBd1 is pathloss between Tx and SPST, where d1 is the distance between Tx and SPST

    - PLdBd2 is pathloss between Rx and SPST, where d2 is the distance between SPST and Rx

    - σRCS,A is the value of RCS component A

    - SFdB,1,SFdB,2 are shadow fading respectively generated for the Tx- SPST link and SPST -Rx link referring to step 4 in section 7.5, TR 38.901

    - Note: for monostatic sensing, PLdBd1=PLdBd2,SFdB,1=SFdB,2

Agreement

RCS upper bound: k equals to 3 is adopted to derive the upper bound of RCS component B2, kσ, where σ is the standard deviation of B2 in dB

Agreement

For vehicle with single/multiple scattering points:

- For mono-static, the RCS=A*B=A*B1*B2 for a scattering point of a vehicle is generated by

  - The values/pattern A*B1, i.e., rcsdB(θ,φ) is deterministic based on incident/scattered angles

rcsdB(θ,φ)=Gmax-min-σVdBθ+σHdB φ,σmax

Where,

σVdBθ=-min12θ-θcenterθ3dB2, σmax,

σHdB φ=-min12φ-φcenterφ3dB2, σmax,

For example, in case of vehicle with multiple scattering points:

|  | φcenter | φ3dB, n | θcenter | θ3dB,n | Gmax | σmax | Applicable Range of θ | Applicable Range of φ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Left | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |
| Back | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |
| Right | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |
| Front | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |
| Roof | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |

  - Note: the applicable angular range is 360 degrees per row in horizontal domain in case of vehicle with multiple scattering points, and the applicable angular range is < 360 degrees per row in horizontal domain in case of vehicle with a single scattering point.

    - FFS: angular continuity

Agreement

For mono-static, the following values of component A, B2 are agreed for UAV of small size

  - Component A: -12.81 dBsm

  - Component B1: 0 dB (already agreed in RAN1#118bis)

  - Component B2:

    - standard deviation: 3.74 dB

Agreement

For mono-static, the following values of component A, B2 are agreed for RCS model 1 of human

  - Component A: -1.37 dBsm

  - Component B1: 0 dB (already agreed in RAN1#118bis)

  - Component B2:

    - standard deviation: 3.94 dB

Agreement

To model polarization matrix CPMsp,i=αi,1expjΦsp,iθθβi,1expjΦsp,iθϕβi,2expjΦsp,iϕθαi,2expjΦsp,iϕϕ of a direct/indirect path i of a scattering point of a target

  - in Rel-19 study item (e.g., UAV, human, vehicle, AGV), αi,1=αi,2=1, βi,1=βi,2=κsp,i-1, i.e.,

CPMsp,i=expjΦsp,iθθκsp,i-1expjΦsp,iθϕκsp,i-1expjΦsp,iϕθexpjΦsp,iϕϕ

Where,

    - κsp,i is XPR ratio is randomly generated by log-normal distribution per target type

    - The initial random phase Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ is uniformly distributed within -π,π

  - FFS: spatial consistency of random phase when a scattering point moves

  - FFS: whether the base station rotation procedure in 38.901 can be reused to support rotation of the target

Agreement

For reducing options for reference TRs: for sensing scenario UMi, UMa, RMa, InH, InF, UMi-AV, UMa-AV, and RMa-AV, the reference TR to generate a TRP-TRP channel is:

- TRP-TRP link of scenario UMi, UMa, InH, and InF following the option based on TR 38.901 defined in section A.3 of TR 38.858

  - For InF, hUE is changed to the same height as the BS

- TRP-UE link of scenario RMa defined in section 7 of TR 38.901 by setting hUE=35m

- FFS: whether to add very low power clusters

Agreement

For sensing scenario UMi, UMa, RMa, InH, InF, UMi-AV, UMa-AV, and RMa-AV, the reference TR to generate a UE-UE channel is

- UE-UE link of scenario UMi, UMa, InH, and InF following the option based on TR 38.901 defined in section A.3 of TR 38.858

- TRP-UE link of scenario RMa defined in section 7 of TR 38.901 by setting hBS =1.5m

- FFS: whether to add very low power clusters

Agreement

The reference TR to generate a TRP-UE channel is

| TRP | normal UE | UMi, UMa, RMa, InH, InF, UMi-AV, UMa-AV, and RMa-AV - Option 1: TRP-UE link of scenario UMi, UMa, RMa, InH, and InF in section 7 of TR 38.901 Highway and Urban grid - Option 1: P2B link of scenario Highway and Urban grid in section 6 of TR 37.885 HST - Option 1: TRP-UE link of scenario RMa in section 7 of TR 38.901 for FR1 and TRP-UE link of scenario UMa in section 7 of TR 38.901 for FR2 |
| --- | --- | --- |
| TRP | vehicle UE | Highway and Urban grid - Option 1: V2B link of scenario Highway and Urban grid in section 6 of TR 37.885 UMi, UMa, and RMa - Option 1: TRP-UE link of scenario UMi, UMa, and RMa in section 7 of TR 38.901 |
| TRP | aerial UE | UMa-AV, UMi-AV, and RMa-AV - Option 1: - TRP-aerial UE link of scenario UMa-AV, UMi-AV, and RMa-AV in section Annex A and B of TR 36.777 for FR1 - FFS reuse the channel model of scenario UMa-AV, UMi-AV, and RMa-AV of FR1 for FR2 |

Working assumption

For modelling background channel for monostatic sensing:

Solution A: (previous Option 1)

- drop N reference point(s) following certain distribution, and then the background channel is generated based on the channel generated as in existing TR between the real Tx and the reference point for a scenario.

  - The distance between Tx and the reference point, and the height of the reference point follow Gamma distribution,

drp_2D_n~Γα1, β1+c1

hrp_n~Γα2, β2+c2

  - FFS: value of N (N<4)

- FFS: whether to add additional very low power clusters

- FFS: any update to parameters e.g. angular distribution and delay spread

Working assumption

Absolute delay model (referring to 7.6.9 in TR 38.901 as starting point) is a mandatory feature for both target channel and background channel for ISAC for UMi, UMa, InH, InF

- Related model referring to Δτ values from 7-24GHz study item

Agreement

When absolute delay model Δτ is configured, it applies to all NLOS clusters in each of Tx-target and target-Rx links and background channel.

- For bistatic sensing: Different values of Δτ are separately generated for the Tx-target link, target-Rx link and the background channel

- For monostatic sensing: the same value of Δτ is used for Tx-target link and target-Rx link, and a different value of Δτ is separately generated for the background channel

Agreement

The existing spatial consistency model in TR 38.901 is reused to model correlation of links between one TRP and different STs/UEs.

Agreement

Spatial consistency is not modelled at least for the following links

- Case 1: the links from/to two non-co-located TRPs

  - Link TRP1-X and link TRP2-X, where node X can be a target, a UE or another TRP

  - Link TRP1-X and link TRP2-Y, where node X, Y can be a target, a UE or another TRP

- Case 2: the two links with different LOS/NLOS condition

- Case 3: background channel for monostatic vs. any link (Tx-ST, ST-Rx, background channel) for bistatic

Agreement

Spatial consistency is needed to model correlation of the following links from ST-UT links and UT-UT links

- Case 5: links between same UT and two nodes X/Y, subjected to correlation distance, i.e., link UT1-X and link UT1-Y, where nodes X/Y can be target or UT

- Case 6: links between same target and two nodes X/Y, subjected to correlation distance, i.e., link target1-X and link target1-Y, where nodes X, Y are different UTs

- Case 7: link X1-Y1 and link X2-Y2, subjected to correlation distance, where X1, X2, Y1, Y2 are 4 different nodes

- FFS: Spatial consistency between multiple scattering points of the same target

Agreement

Correlation type is introduced for large scale parameter, cluster specific parameter and ray specific parameter of ST-UT links and UT-UT links

- Definition of link Correlated: parameters for any two links between STs/UTs are correlated, subjected to correlation distance.

Table 4: Correlation type for links between STs/UTs

| Parameters | Correlation type |
| --- | --- |
| Delays | link Correlated |
| Cluster powers | link Correlated |
| AOA/ZOA/AOD/ZOD offset | link Correlated |
| AOA/ZOA/AOD/ZOD sign | link Correlated |
| Random coupling | link Correlated |
| XPR | link Correlated |
| Initial random phase | link Correlated |
| LOS/NLOS states | link Correlated |
| Blockage (Model A) | All-correlated |
| O2I penetration loss | All-correlated |
| Indoor distance | All-correlated |
| Indoor states | All-correlated |

- Note: it is not precluded more parameters for spatial consistency can be discussed and added in the table

Agreement

- For target channel

  - The power threshold for removing clusters in step 6 in section 7.5, TR 38.901, i.e., -25 dB is reused to generate Tx-target link and target-Rx link

  - The power threshold for path dropping after concatenation is relaxed to [X=-40] dB

- For background channel

  - The power threshold for removing clusters in step 6 in section 7.5, TR 38.901, i.e., [-25 dB] is reused to generated the background channel

    - FFS: whether to add additional very low power clusters

    - FFS: The reference power for removing cluster is the min (max. Tx-target link cluster power, max. target-Rx link cluster power)

Agreement

If EO type-2 is modelled in an indirect path, only specular reflection is modeled for EO type-2

- polarization of the indirect path is product of polarization matrix of the target-Rx link, the target, and the Tx-target link, i.e., CPMtx,sp,rx= CPMsp,rx . CPMsp . CPMtx,sp

  - For the specular reflected ray generated by a EO type-2 in the Tx-target link or the target-Rx link (i.e., Tx-EO type-2-target, or target-EO type-2-Rx), CPMtx,sp or CPMsp,rx is the polarization matrix of EO type-2

  - To generate polarization matrix of EO type-2, the procedure in [R1-2409394, R1-2410648] is taken as starting point

    - FFS applicability if the surface of EO type-2 is tilted

Agreement ([Post-120-ISAC-01])

On the monostatic RCS of vehicle with single scattering point,

- The values/pattern of component A*B1 are generated by the following parameters

|  | φcenter | φ3dB, n | θcenter | θ3dB,n | Gmax | σmax | Applicable Range of θ | Applicable Range of φ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 90 | 26.90 | 79.70 | 44.42 | 20.75 | 13.68 | [30°,180° ] | (45°,135°] |
| Back | 180 | 36.32 | 79.65 | 36.73 | 14.56 | 7.50 | [30°,180° ] | (135°,225°] |
| Right | 270 | 26.90 | 79.70 | 44.42 | 20.75 | 13.68 | [30°,180° ] | (225°,315°] |
| Front | 0 | 40.54 | 71.75 | 29.13 | 15.52 | 8.45 | [30°,180° ] | (-45°, 45°] |
| Roof | - | - | 0.00 | 18.13 | 21.26 | 14.19 | [0°,30° ) | [0°,360°) |

  - When θ is in the range [0°,30° ), σHdB φ=0

- The standard deviation of component B2 is 3.41 dB

Agreement ([Post-120-ISAC-01])

On the monostatic RCS for each scattering point of vehicle with multiple scattering points,

- The values/pattern of component A*B1 are generated by the following parameters

|  | φcenter | φ3dB, n | θcenter | θ3dB,n | Gmax | σmax | Applicable Range of θ | Applicable Range of φ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 90 | 26.90 | 79.70 | 44.42 | 20.60 | 20.52 | [0°,180° ] | [0°,360°] |
| Back | 180 | 36.32 | 79.65 | 36.73 | 13.90 | 13.82 | [0°,180° ] | [0°,360°] |
| Right | 270 | 26.90 | 79.70 | 44.42 | 20.60 | 20.52 | [0°,180°] | [0°,360°] |
| Front | 0 | 40.54 | 71.75 | 29.13 | 14.99 | 14.91 | [0°,180° ] | [0°,360°] |
| Roof | - | - | 0.00 | 18.13 | 21.12 | 21.05 | [0°,180° ] | [0°,360°] |

  - For the scattering point associated with roof of the vehicle, σHdB φ=0

- The standard deviation of component B2 is 3.41 dB

Agreement ([Post-120-ISAC-01])

On the monostatic RCS for UAV with large size and AGV

- The monostatic RCS for a scattering point of the target is generated by

  - The values/pattern A*B1, i.e., rcsdB(θ,φ) is deterministic based on incident/scattered angles

rcsdB(θ,φ)=Gmax-min-σVdBθ+σHdB φ,σmax

Where,

σVdBθ=-min12θ-θcenterθ3dB2, σmax,

σHdB φ=-min12φ-φcenterφ3dB2, σmax,

  - FFS how many rows of the values/pattern A*B1 are defined for the target

    - Note: each row has a defined applicable range of θ and φ

- FFS human RCS model 2

- RAN1 #120bis

Agreement

In order to generate Tx-target link, target-Rx link and the background channel, the above table on reference TRs (excluding the already agreed part) is adopted for the mapping between reference TRs and a pair of nodes (STX, SRX, target)

  - Note: continue discussion for updating the table with RSU type UE

  - FFS: the generation of background channel based on reference TRs is subject to the addition of low-energy clusters

| Case | Node 1 | Node 2 | Existing TRs as starting point |
| --- | --- | --- | --- |
| 1 | TRP | TRP | Highway - TRP-UE link of scenario RMa in section 7 of TR 38.901 by setting hUE=35m for FR1 - TRP-TRP link of scenario UMa following the option based on TR 38.901 defined in section A.3 of TR 38.858 Urban grid - TRP-TRP link of scenario UMa following the option based on TR 38.901 defined in section A.3 of TR 38.858 HST - TRP-UE link of scenario RMa in section 7 of TR 38.901 by setting hUE=35m for FR1 - TRP-TRP link of scenario UMa in section A.3 of TR 38.858 for FR2 |
| 4 | TRP | aerial UE | UMa-AV, UMi-AV, and RMa-AV - Reuse the channel model of scenario UMa-AV, UMi-AV, and RMa-AV of FR1 for FR2 |
| 5 | normal UE | normal UE | For pedestrian type UE: Highway and Urban grid - P2P link in section 6 of TR 37.885 HST - TRP-UE link of scenario RMa in section 7 of TR 38.901 for FR1, e.g., hBS=1.5m, UE-UE link of scenario UMa following the option based on TR 38.901 defined in section A.3 of TR 38.858 for FR2 |
| 6 | normal UE | vehicle UE | UMi, UMa, RMa - UE-UE link of scenario UMi, UMa following the option based on TR 38.901 defined in section A.3 of TR 38.858 - TRP-UE link of scenario RMa defined in section 7 of TR 38.901 by setting hBS =1.5m For pedestrian type UE: Highway and Urban grid - V2P link in section 6 of TR 37.885 |
| 7 | normal UE | aerial UE | UMi-AV, UMa-AV, and RMa-AV - TRP-aerial UE link of UMi-AV in Annex A and B of TR 36.777 by setting hBS =1.5m for FR1 - LOS probability is not reused, FFS new LOS probability - FFS pathloss model, shadowing fading - Working assumption: Reuse the channel model of scenario UMa-AV, UMi-AV, and RMa-AV of FR1 for FR2 - The corresponding parameter values in FR2 are used |
| 8 | vehicle UE | vehicle UE | Highway and Urban grid - V2V link of scenario Highway and Urban grid in section 6 of TR 37.885 UMi, UMa, and RMa - UE-UE link of scenario UMi, UMa following the option based on TR 38.901 defined in section A.3 of TR 38.858 - TRP-UE link of scenario RMa defined in section 7 of TR 38.901 by setting hBS =1.5m |
| 9 | aerial UE | aerial UE | UMi-AV, UMa-AV, RMa-AV - TRP-aerial UE link of UMi-AV in Annex A and B of TR 36.777 by setting height of TRP equal to the height of the first aerial UE for FR1 - LOS probability is not reused, FFS new LOS probability - FFS pathloss model, shadowing fading, angular spread - Working assumption: Reuse the channel model of scenario UMa-AV, UMi-AV, and RMa-AV of FR1 for FR2 - The corresponding parameter values in FR2 are used |

Agreement

To generate the parameters (in the steps before concatenation), the large-scale parameters and the small-scale parameters used to generate the Tx-target link are respectively the same as that of the target-Rx link for monostatic sensing, where departure angle on one link and arrival angle on the other link are reciprocal.

  - FFS: whether this applies to initial phase

Agreement

Normalization on the product of three polarization matrixes of a direct/indirect path generated by stochastic cluster, i.e., CPMtx,sp,rx= CPMsp,rx . CPMsp . CPMtx,sp is supported

  - The scaling factor is 2d112+d222

Agreement

Power normalization of target channel after path dropping of the target channel is not supported.

Agreement

On the monostatic RCS for human with RCS model 2

- The monostatic RCS for a scattering point of the target is generated by

  - The values/pattern A*B1, i.e., rcsdB(θ,φ) is deterministic based on incident/scattered angles

rcsdB(θ,φ)=Gmax-min-σVdBθ+σHdB φ,σmax

Where,

σVdBθ=-min12θ-θcenterθ3dB2, σmax

σHdB φ=-min12φ-φcenterφ3dB2, σmax

  - FFS how many rows of the values/pattern A*B1 are defined for the target

    - Note: each row has a defined applicable range of θ and φ

  - Note: whether the RCS is elevation angle dependent or dependent on both elevation and horizontal angles can be separately discussed

Agreement

The following mean and standard deviation values of XPR of targets are agreed for monostatic sensing and bistatic sensing as follows:

- UAV: (13.75, 7.07) dB

- Human: (19.81, 4.25) dB

- Vehicle: (21.12, 6.88) dB

Agreement

When spatial consistency is enabled, the 1-by-1 random coupling generated by concatenation Option 3 is not updated per simulation drop even if Tx, target, Rx positions change during simulation.

Agreement

The following working assumption is confirmed

Working assumption Absolute delay model (referring to 7.6.9 in TR 38.901 as starting point) is a mandatory feature for both target channel and background channel for ISAC for UMi, UMa, InH, InF - Related model referring to Δτ values from 7-24GHz study item

Working assumption

For vehicle with single/multiple scattering points, the bistatic RCS is generated by

- The values/pattern of A*B1 of bistatic RCS is given by:

rcsdBθi,ϕi,θs,ϕs=maxGmax-min-σVdBθ+σHdB φ,σmax-Attenuatefactor,RCSdB,fsθi,ϕi,θs,ϕs

where

σVdBθ=-min12θ-θcenterθ3dB2, σmax

σHdB φ=-min12φ-φcenterφ3dB2, σmax

  - Attenuatefactor=k1×sin(k2×β2) is applied to the  β within 0~180 degrees. k1= 6 and k2=1.65. β is the absolute bistatic angle between the incident ray and scattering ray within the plane of incident direction (θi,ϕi,) and scattering direction (θs,ϕs).

  - The angles of (θ,φ) are the projections of the bisector angle on the vertical plane and the horizontal plane, respectively.

    - FFS: RCS value when β is 180 degrees

  - The effect of forward scattering RCSdB,fsθi,ϕi,θs,ϕs is -Inf in Rel-19

  - 5 sets of parameters φcenter, φ3dB, n, θcenter, θ3dB,n, Gmax, σmax, Applicable Range of θ and Applicable Range of φ are applicable as defined for the monostatic RCS of vehicle with single/multiple SPSTs

  - FFS: how to avoid angular discontinuity

  - Continue study on a new formula for Attenuatefactor to resolve the issue of angular discontinuity.

    - The new formula should retain following property: the linear bistatic RCS for a vehicle with single scattering point is the sum of the bistatic RCS of the multiple scattering points of the vehicle

    - the following formula can be a reference for the study

Attenuatefactor=k1sink2β2(1-k3β-k4*(Gmax-min-σVdBθ+σHdB φ,σmax))

Note: the working assumption agreed on Thursday was updated on Friday as follows: k1= 6 and k2=1.65

Agreement

On background channel for mono-static sensing, the following details are provided:

- Nrp=3 reference points are dropped for one Tx, based on the Gamma distribution for distance and height of reference point.

- The LOS AOD between Tx and the first reference point, which is denoted as AOD1, is generated based on uniform distribution unif-π,π.

- The LOS AOD between Tx and the second reference point is AOD1 + 2Nrpπ

- The LOS AOD between Tx and the third reference point is AOD1 + 4Nrpπ

- The background channel is generated based on the channel generated as in existing TR between the real Tx and the reference point assuming NLOS condition.

- The antenna field pattern and array orientation of reference point are set same as Tx.

- Arrival angles for both azimuth and elevation φn,m,AOA and θn,m,ZOA are set equal to departure angles

- The absolute delay model d3D and Δτ as agreed for bistatic sensing for the same sensing scenario applies. Down-select one option from the following:

- Option 0: no scaling factor is applied to d3D

- Option 1: An offset is applied to d3D, i.e., d3D-c1

- Option 2: A scaling factor d_s is multiplied to d3D, i.e., d3D*d_s. d_s is a value within range [0, 1].

- Note: The adjustment of absolute delay doesn’t impact the generation of NLOS clusters between the Tx and each reference point

- The mono-static background channel for the Tx would be sum of channels of the links between the Tx and all related reference points, which is

HsNLOS(τ,t)=u=1NrpPLu,sNLOSHu,sNLOS(τ,t)

- FFS: Doppler frequency in background channel for monostatic sensing

- The rays in a stochastic cluster with ZOA at BS less than D degree are dropped

- D=[90] for RMa,

- D=[60] for UMa

- D=[50] for UMi

- Note: this threshold for ZOA is not applicable to other sensing scenarios

Agreement

To generate the background channel, the power threshold (-25 dB) for removing clusters in step 6 in section 7.5, TR 38.901 is reused.

Agreement

The ISAC background channel can be generated between a sensing Tx and a sensing Rx or RP (relevant for monostatic case) via the following steps:

- Step 1: generate a first set of clusters/rays according to TR 38.901(or other related TRs)

- Step 2: generate a second set of NLOS clusters/rays according to TR 38.901 (or other related TRs), where the power of the second set of clusters/rays should be scaled down such that

PnS2=PnS2nPnS2Pdrop=PnS2nPnS2P1S110G10

- P1S1 is the power of the NLOS cluster with the strongest power from the first set.

- PnS2 is the power of the n-th cluster from the second set.

- Where, N is the number of clusters, M is the number of rays within each cluster, value of G relates to power

  - N=360, M=1, G = -25dB, no further change from 38.901, 36.777, 38.858 (i.e., utilizing the same DS, ASA, ASD, ZSA, ZSD, Cθ, Cϕ as used for the first step)

- The step 2 is an additional modeling component

Agreement

For human as a sensing target with a single scattering point, the height of the scattering point is 1.5 m.

Agreement

In sensing scenario UMi, UMa, RMa, if the height of a scattering point of target is less than 1.5m, for pathloss calculation, down-selection one of the options below:

  - Option 4: use PL1 in Table 7.4.1-1: Pathloss models in TR 38.901

  - Option 5: use hUT 1.5 m for pathloss calculation

Agreement

For sensing scenario UMi, UMa, RMa, UMi-AV, UMa-AV and RMa-AV, the height of a scattering point of a target is used to calculate the LOS probability and pathloss, regardless of the lower bound in the existing TRs that are referred to generate ISAC channel.

  - FFS for the case where the height of a scattering point of target is less than 1.5m in sensing scenario UMi, UMa, RMa

Agreement ([Post-120bis-ISAC-02])

On the monostatic RCS of UAV of large size,

- The values/pattern of component A*B1 are generated by the following parameters

|  | φcenter | φ3dB, n | θcenter | θ3dB,n | Gmax | σmax | Applicable Range of θ | Applicable Range of φ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 90° | 7.13° | 90° | 8.68° | 7.43 | 14.30 | [45°,135°] | [45°,135°] |
| Back | 180° | 10.09° | 90° | 11.43° | 3.99 | 10.86 | [45°,135°] | [135°,225°] |
| Right | 270° | 7.13° | 90° | 8.68° | 7.43 | 14.30 | [45°,135°] | [225°,315°] |
| Front | 0° | 14.19° | 90° | 16.53° | 1.02 | 7.89 | [45°,135°] | [-45°,45°] |
| Bottom | / | / | 180° | 4.93° | 13.55 | 20.42 | [135°,180°] | [0°,360°] |
| Roof | / | / | 0° | 4.93° | 13.55 | 20.42 | [0°,45°] | [0°,360°] |

  - When θ is in the range [0°,45° ] or [135°,180°], σHdB φ=0

- The standard deviation of component B2 is 2.50 dB

Agreement ([Post-120bis-ISAC-02])

On the monostatic RCS of AGV with single scattering point,

- The values/pattern of component A*B1 are generated by the following parameters

|  | φcenter | φ3dB, n | θcenter | θ3dB,n | Gmax | σmax | Applicable Range of θ | Applicable Range of φ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Front | 0° | 13.68° | 90° | 13.68° | 13.02 | 23.29 | [30°,180°] | [-45°,45°] |
| Left | 90° | 15.53° | 75° | 20.03° | 7.33 | 17.60 | [30°,180°] | [45°,135°] |
| Back | 180° | 12.49° | 90° | 11.89° | 11.01 | 21.28 | [30°,180°] | [135°,225°] |
| Right | 270° | 15.53° | 75° | 20.03° | 7.33 | 17.60 | [30°,180°] | [225°,315°] |
| Roof | / | / | 0° | 11.44° | 11.79 | 22.06 | [0°,30°] | [0°,360°] |

  - When θ is in the range [0°,30° ), σHdB φ=0

- The standard deviation of component B2 is 2.51 dB

Agreement ([Post-120bis-ISAC-02])

The values of the parameters to generate background channel for TRP monostatic sensing for each sensing scenario are provided in the following table

| Scenario |  | TRP monostatic sensing |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Uma / Urban grid / Highway(FR2) / HST(FR2) | UMi | Rma / Highway(FR1) / HST(FR1) | Indoor office | Indoor Factory |
| Distribution of 2D distance between Tx and reference points | α1 | 10.3370 | 6.1996 | 6.2025 | 4.236 | 0.039836 |
|  | β1 | 0.1317 | 0.1558 | 0.0391 | 0.19255 | 0.179783 |
|  | c1 | 68.7778 | 15.2697 | 1.2940 | 4.99 | 1.130020 |
| Distribution of height of reference points | α2 | 16.2253 | 12.0487 | 0.0007 | 1.3293 | 0.283447 |
|  | β2 | 1.9218 | 2.3261 | 5.0146 | 0.1442 | 0.435965 |
|  | c2 | 2.6142 | 0.0157 | 0.0522 | -13.19 | -17.043530 |
| scaling factor d_s to d3D (if validated) |  |  |  |  |  |  |
| Threshold D for ZOA |  | 80 | 50 | 90 | N/A | N/A |

Note 1: Distributions of height and distance of reference point are not subject to geographical constraints on UT given in TR 38.901 for the corresponding deployment scenario.

Note 2: The reference points for generating the TRP monostatic background channel have no mobility, i.e. 0 km/h.

Agreement ([Post-120bis-ISAC-02])

The values of the parameters to generate background channel for UT monostatic sensing for the following sensing scenarios are provided in the following table

| Scenario |  | UT monostatic sensing |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Uma / Urban grid / Highway(FR2) / HST(FR2) | UMi | Rma / Highway(FR1) / HST(FR1) | Indoor office | Indoor Factory |
| Distribution of 2D distance between Tx and reference points | α1 | 2.9072 | 10.0220 | 10.2421 | 4.3733 | 0.231418 |
|  | β1 | 0.1031 | 1.2522 | 0.0526 | 0.4457 | 0.128133 |
|  | c1 | 3.8471 | 11.0040 | 3.3131 | 4.6302 | 2.004903 |
| Distribution of height of reference points | α2 | 1.6640 | 3.0487 | 0.3175 | 0.2974 | 0.462968 |
|  | β2 | 1.6215 | 1.9128 | 1.4150 | 0.4103 | 0.281526 |
|  | c2 | -1.4205 | 0.1785 | 1.5906 | 2.9711 | -16.921515 |
| scaling factor d_s to d3D (if validated) |  |  |  |  |  |  |
| Threshold D for ZOA |  | 80 | 50 | 90 | N/A | N/A |

Note 1: Distributions of height and distance of reference point are not subject to geographical constraints on TRP given in TR 38.901 for the corresponding deployment scenario.

Note 2: The reference points for generating the UT monostatic background channel has the same velocity as UT.

Note 3: In the UT monostatic sensing in UMa and UMi scenario, the ZOD offset should be set as 0

Agreement ([Post-120bis-ISAC-02])

The values of the parameters to generate background channel for UT monostatic sensing for the following sensing scenarios are provided in the following table

| Scenario |  | UT monostatic sensing |  |  |
| --- | --- | --- | --- | --- |
|  |  | UMa-AV | UMi-AV | RMa-AV |
| Distribution of 2D distance between Tx and reference points | α1 | α1=0.83+0.00015h |  |  |
|  | β1 | β1=1536.305+1.0279h |  |  |
|  | c1 | c1=13.824+0.03085h |  |  |
| Distribution of height of reference points | α2 | α2=0.9054-0.0001117h |  |  |
|  | β2 | β2=138.672-0.04658h |  |  |
|  | c2 | c2=25.4898-0.02398h |  |  |
| scaling factor d_s to d3D (if validated) |  |  |  |  |
| Threshold D for ZOA |  |  |  |  |

Note 1: Distributions of height and distance of reference point are not subject to geographical constraints on TRP given in TR 38.901 for the corresponding deployment scenario.

Note 2: The reference points for generating the UT monostatic background channel has the same velocity as UT.

Note 3: In the UT monostatic sensing in UMa and UMi scenario, the ZOD offset should be set as 0

- RAN1 #121

Agreement

Confirm the following working assumption with updates in red.

Working assumption

For vehicle with single/multiple scattering points, the bistatic RCS is generated by

- The values/pattern of A*B1 of bistatic RCS is given by:

rcsdBθi,ϕi,θs,ϕs=maxGmax-min-σVdBθ+σHdB φ,σmax-Attenuatefactor,  Gmax-σmax,RCSdB,fsθi,ϕi,θs,ϕs

where

σVdBθ=-min12θ-θcenterθ3dB2, σmax

σHdB φ=-min12φ-φcenterφ3dB2, σmax

  - Attenuatefactor=k1×sin(k2×β2)-5log10cosβ2 is applied to the  β within 0~180 degrees. k1= 6 and k2=1.65. β is the bistatic angle between the incident ray and scattering ray within the plane of incident direction (θi,ϕi,) and scattering direction (θs,ϕs).

  - The angles of (θ,φ) are the projections of the bisector angle on the vertical plane and the horizontal plane, respectively.

    - FFS: RCS value when β is 180 degrees

  - The effect of forward scattering RCSdB,fsθi,ϕi,θs,ϕs is -Inf in Rel-19

  - 5 sets of parameters φcenter, φ3dB, n, θcenter, θ3dB,n, Gmax, σmax, Applicable Range of θ and Applicable Range of φ are applicable as defined for the monostatic RCS of vehicle with single/multiple SPSTs

  - Continue study on a new formula for Attenuatefactor to resolve the issue of angular discontinuity.

    - The new formula should retain following property: the linear bistatic RCS for a vehicle with single scattering point is the sum of the bistatic RCS of the multiple scattering points of the vehicle

    - the following formula can be a reference for the study

Attenuatefactor=k1sink2β2(1-k3β-k4*(Gmax-min-σVdBθ+σHdB φ,σmax))

Agreement

The agreement on bistatic RCS for vehicle is reused for large size UAV and AGV.

  - For large size UAV, k₁=6.05 and k₂=1.33

  - For AGV, k₁=12 and k₂=1.45

Agreement

AGV can be modelled with multiple scattering points.

- The values/pattern of component A*B1 are generated by the following parameters

|  | ϕcenter in [°] | ϕ3dB, n in [°] | θcenter in [°] | θ3dB,n in [°] | Gmax | σmax | Range of θ in [°] | Range of ϕ in [°] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Front | 0° | 13.68° | 90° | 13.68° | 13.00 | 30.26 | [0,180] | [0,360] |
| Left | 90° | 15.53° | 75° | 20.03° | 7.27 | 24.53 | [0,180] | [0,360] |
| Back | 180° | 12.49° | 90° | 11.89° | 10.98 | 28.24 | [0,180] | [0,360] |
| Right | 270° | 15.53° | 75° | 20.03° | 7.27 | 24.53 | [0,180] | [0,360] |
| Roof | / | / | 0° | 11.44° | 11.77 | 29.03 | [0,180] | [0,360] |

  - Note: For the scattering point associated with roof of the AGV, σHdBϕ=0.

  - Note: the measurements from companies are done by AGV option 1.

Agreement

- The bistatic RCS of UAV with small size is modelled as

  - The values/pattern of A*B1 is given by

rcsdBθi,ϕi,θs,ϕs=maxσRCS,A-Attenuatefactor,RCSdB,fsθi,ϕi,θs,ϕs

    - Component A, i.e., σRCS,A: same as component A of mono-static RCS for UAV of small size

    - Attenuatefactor=3sin(β2) dB, where β is the bi-static angle between incident ray and scattered ray, β is within 0 and 180 degree

    - The effect of forward scattering RCSdB,fsθi,ϕi,θs,ϕs is -Inf in Rel-19

  - Component B2: same as component B2 of mono-static RCS for UAV of small size

- The bistatic RCS of Human with RCS model 1 is modelled as

  - The values/pattern of A*B1 is given by

rcsdBθi,ϕi,θs,ϕs=maxσRCS,A-Attenuatefactor,RCSdB,fsθi,ϕi,θs,ϕs

    - Component A, i.e., σRCS,A: same as component A of mono-static RCS for Human with RCS model 1

    - Attenuatefactor=3sin(β2) dB, where β is the bi-static angle between incident ray and scattered ray, β is within 0 and 180 degree

    - The effect of forward scattering RCSdB,fsθi,ϕi,θs,ϕs is -Inf in Rel-19

  - Component B2: same as component B2 of mono-static RCS for Human with RCS model 1

Agreement

On the monostatic RCS of human with RCS model 2,

- The values/pattern of component A*B1 are generated by the following parameters

|  | ϕcenter in [°] | ϕ3dB, n in [°] | θcenter in [°] | θ3dB,n in [°] | Gmax | σmax | Range of θ in [°] | Range of ϕ in [°] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Front | 0 | 216.65 | 90 | 55.7 | 2.14 | 7.7 | [0,180] | [-90, 90] |
| Back | 180 | 216.65 | 90 | 55.7 | 2.14 | 7.7 | [0,180] | [90,270] |

- The standard deviation of component B2 is 3.94 dB

Agreement

The agreement on bistatic RCS for vehicle with single scattering point is reused to model bistatic RCS of human with RCS model 2

  - k1=0.5714 and k2=0.1

Agreement

The following values of the RCS component A are applied to both monostatic and bistatic RCS of the target.

  - UAV with large size: -5.85 dBsm

  - Human with RCS model 2: -1.37 dBsm

    - Note: measurement is based on adult

  - Vehicle: 11.25 dBsm

    - Note: measurement is based on vehicle type 1 and 2

  - AGV: -4.25 dBsm

    - Note: measurement is based on AGV option 1

Note: component A on its own may not fully reflect the RCS in the target channel. This note will not be captured in the TR.

Agreement

The mean and standard deviation values of XPR of sensing target AGV for monostatic sensing and bistatic sensing are (9.60, 6.85) dB.

Conclusion

The component B2 of two different targets are generated independently.

Conclusion

The component XPR/initial random phase of two different targets are generated independently.

Agreement

In order to generate Tx-target link, target-Rx link and the background channel between a RSU-type UE and another node (TRP, pedestrian UE, vehicle UE, RSU-type UE), the following reference TRs are adopted

| Case | Tx/Rx | Rx/Tx | Existing TRs as starting point |
| --- | --- | --- | --- |
|  | TRP | RSU-type UE | Highway and Urban grid - B2R link in section 6 of TR 37.885 |
|  | RSU-type UE | normal UE | Highway and Urban grid - V2V link in section 6 of TR 37.885, with antenna height at RSU is 5m |
|  | RSU-type UE | RSU-type UE | Highway and Urban grid - V2V link in section 6 of TR 37.885, with antenna height at RSU is 5m |
|  | RSU-type UE | vehicle UE | Highway and Urban grid - V2V link in section 6 of TR 37.885, with antenna height at RSU is 5m |

Agreement

The initial random phase (generated in Step 10, section 7.5, TR38.901) is the same for the same ray in Tx-target link and target-Rx link of a target for monostatic sensing.

Agreement

For UMi-AV and RMa-AV with aerial UE as sensing transmitter or receiver, the values of parameters to generate background channel for UT monostatic sensing are provided in the following table

| Scenario |  | UT monostatic sensing |  |
| --- | --- | --- | --- |
|  |  | UMi-AV | RMa-AV |
| Distribution of 2D distance between Tx and reference points | α1 | 0.0156h+5.5399 | 4.423+0.001926h |
|  | β1 | 40.4517/(h+254.6318) | 13.8467+0.6547h |
|  | c1 | 0.0140h+15.1184 | 3.864+0.1538h |
| Distribution of height of reference points | α2 | 0.0123h+11.9569 | 1.4231+0.00192h |
|  | β2 | 17.8047/(h-0.2202) | 11.7157-0.00538h |
|  | c2 | 0.0532h-0.0120 | 2.6541-0.003851h |

Note 1: Distributions of height and distance of reference point are not subject to geographical constraints on TRP for the corresponding deployment scenario.

Note 2: The reference points for generating the UT monostatic background channel have the same velocity as UT.

Note 3: In the UT monostatic sensing in UMa and UMi scenario, the ZOD offset in the background channel should be set as 0

Agreement

To generate the background channel for TRP monostatic sensing and UT monostatic sensing, ‘maxd3D-c12+(c2-height_Tx)2c,0 +Δτ’ is used to model the absolute delay between the Tx and each reference point.

Agreement

Power threshold for path dropping after concatenation is up to -40dB for target channel for option 3. Up to company to choose a value in the implementation.

Power threshold for path dropping after concatenation is up to -25dB for target channel for option 0. Up to company to choose a value in the implementation.

For calibrations for both option 0 and option 3, power threshold for path dropping after concatenation is -40dB for target channel.

Agreement

To generate the absolute delay model for sensing scenarios Urban grid, highway and HST, for both target channel and background channel

- For Urban grid, the values of parameters for Δτ of scenarios UMa are reused.

- For Highway, the values of parameters for Δτ of scenarios RMa and UMa are reused for FR1 and FR2 respectively.

- For HST, the values of parameters for Δτ of scenarios RMa and UMa are reused for FR1 and FR2 respectively.

Note: no measurements on Δτ of the 3 scenarios are submitted in Rel-19.

Agreement

Spatial consistency is not modelled for

- the links that are generated referring to channel models with parameter values of different communication scenarios

  - E.g., between TRP-target/UT link in one scenario and target/UT-UT link in another scenario

- the background channels for TRP monostatic sensing of different TRPs

Agreement

Spatial consistency is not modelled between TRP-target/UT link and target/UT-UT link for sensing scenario UMi, InH and InF.

Agreement

Spatial consistency is not modelled between TRP-TRP link and any other links for ISAC channel.

Agreement

Spatial consistency can be enabled for multiple scattering points of a target.

Spatial consistency, if enabled, for the links between BS/UT and multiple scattering points of a target are modelled as if multiple scattering points are multiple targets.

Agreement

The existing horizontal correlation distance in Table 7.6.3.1-2 in TR38.901 is used as the correlation distance for 3D spatial consistency for ISAC channel at least for UAV scenario, within same ‘Applicability range in terms of aerial UE height (defined in 36.777)’.

Agreement

EO type-2 can be modelled in NLOS condition.

Agreement

In sensing scenario UMi, UMa, if the height of a scattering point of target is less than 1.5m, for pathloss calculation,

  - use hUT 1.5 m for breakpoint distance (dBP) calculation

  - Note: hUT 1.5 m is only used for dBP calculation. The exact h_UT of the scattering point is still used to determine all other parameters of ISAC channel, e.g., delay, AOD/ZOD/AOA/ZOA, etc.

Agreement

On background channel modelling,

- Spatial consistency is not supported for TRP monostatic sensing across different TRPs

- Spatial consistency is not supported for UE monostatic sensing across different UEs

- Spatial consistency is not supported across different Reference Points for same TRP for TRP monostatic sensing

- Spatial consistency is not supported across different Reference Points for same UE for UE monostatic sensing

Agreement

- RCS component B2 of different direct/indirect paths of a target in the target channel are generated independently.

- On the RCS component B2 of a direct/indirect path of a target in the target channel, the same value of B2 applies to a path before the value of B2 is updated.

  - Note: whether/how/when to update B2 can be discussed in evaluation phase or up to companies’ choices

Agreement

- XPR of different direct/indirect paths of a target in the target channel are generated independently.

- On the XPR of a direct/indirect path of a target in the target channel, the same value of XPR applies to a path before the value of XPR is updated.

  - Note: whether/how/when to update XPR can be discussed in evaluation phase or up to companies’ choices

Agreement

- Initial random phase of different direct/indirect paths of a target in the target channel are generated independently.

- On the initial random phase of a direct/indirect path of a target in the target channel, the same value of initial random phase applies to a path before the value of initial random phase is updated.

  - Note: whether/how/when to update initial random phase can be discussed in evaluation phase or up to companies’ choices

Agreement

The follow TP is used generate the power (except for the impact of polarization matrix of EO type-2) of the ray specular reflected by an EO type 2 in the STX-SPST link or SPST-SRX link.

- 7.9.5.2 Type-2 environment object < Unchanged text omitted > - In Step 10 in Clause 7.9.4.1, Prx,0,m'k,p,Ptx,0,mk,p for a NLOS ray specularly reflected by a type-2 EO, if present, in the SPST-SRX link and the STX-SPST link is determined as follows. - If the STX-SPST link is in LOS condition, Ptx,0,mk,p=dtx,3Dk,pdtx,EO,mk,p2 - If the STX-SPST link is not in LOS condition, Ptx,0,mk,p=10-PLtx,k,pLOS+PLtx,k,p/10∙dtx,3Dk,pdtx,EO,mk,p2, where PLtx,k,pLOS is the pathloss of STX-SPST link assuming LOS condition. - If the SPST-SRX link is in LOS condition, Prx,0,m'k,p=drx,3Dk,pdrx,EO,m'k,p2 - If the SPST-SRX link is not in LOS condition, Prx,0,m'k,p=10-PLrx,k,pLOS+PLrx,k,p/10∙drx,3Dk,pdrx,EO,m'k,p2, where PLrx,k,pLOS is the pathloss of SPST-SRX link assuming LOS condition. < Unchanged text omitted >

Agreement

To generate the absolute delay model for sensing scenarios UMi-AV, UMa-AV and RMa-AV, for both target channel and background channel,

- For the TRP-TRP link and TRP- terrestrial UE link, the values of parameters for Δτ of scenarios UMi, UMa and RMa are respectively reused.

- For the terrestrial UE- terrestrial UE link, the values of parameters for Δτ of scenarios UMi are reused.

- For the TRP- aerial UE link, the values of parameters for Δτ of scenarios UMi, UMa and RMa are respectively reused.

- For the terrestrial UE- aerial UE link, the values of parameters for Δτ of scenarios UMi are reused.

- For the aerial UE- aerial UE link, the values of parameters for Δτ of scenarios UMi are reused.

Note: no measurements on Δτ of the scenarios UMi-AV, UMa-AV and RMa-AV are submitted in Rel-19.

Agreement

- Remove the brackets for first sub-bullet under Step 4 for Clause 7.9.4.2 in the CR to TR 38.901.

- On the absolute delay of the background channel for both TRP and UE monostatic sensing, three ∆τ are independently generated and respectively applied to the 3 channels between the STX/SRX and the 3 RPs.

Agreement

To generate the channel between an aerial UE and a normal UE,

- The LOS probability is generated by:

| Low-UAV | Mid-UAV | High-UAV |
| --- | --- | --- |
| UMi in Table 7.4.2-1 in TR 38.901 for UMi-AV/UMa-AV/RMa-AV | UMi-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region | UMi-AV in Table B-1 in TR 36.777 for BS to [high] UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region |

- The pathloss and shadow fading are generated using TRP-aerial UE link of UMi-AV in Annex A and B of TR 36.777 by setting hBS =1.5m for FR1

Note:

- The height ranges of low-UAV, Mid-UAV and High-UAV are defined following the applicability range in terms of aerial UE height in Table B-1: LOS probability in TR 36.777

- The second height range for UMi-AV is further divided into 2 regions, i.e., [22.5, 100] and [100, 300] for mid-UAV and high-UAV, respectively.

Conclusion

No further study on power normalization of target channel and background channel of ISAC channel in Rel-19

  - Note: sub-section “7.9.5.3 Power normalization across target channel and background channel” in the TR remains as a placeholder with the following text.

    - To combine the target channel and the background channel, power normalization can be applied to keep the same/similar channel power as the background channel without sensing target.

Agreement

The polarization matrix CPMsp,i of a direct/indirect path i of a scattering point of a target is defined in LCS.

Agreement

To generate the channel between a first aerial UE with height h1 and a second aerial UE with height h2, abs(h1-hBS) <= abs(h2-hBS),

- The LOS probability between the two aerial UEs is generated by:

|  | Low-UAV | Mid-UAV | High-UAV |
| --- | --- | --- | --- |
| Low-UAV | UMi in Table 7.4.2-1 in TR 38.901 for UMi-AV/UMa-AV/RMa-AV | UMi-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region | UMi-AV in Table B-1 in TR 36.777 for BS to [high] UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region |
| Mid-UAV | UMi-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region | UMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region | 1 |
| High-UAV | UMi-AV in Table B-1 in TR 36.777 for BS to [high] UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region | 1 | 1 |

- The pathloss and shadow fading between two aerial UEs are generated using TRP-aerial UE link of UMi-AV in Annex A and B of TR 36.777 by setting height of TRP equal to the height of the first aerial UE.

Note:

- The height ranges of low-UAV, Mid-UAV and High-UAV are defined following the applicability range in terms of aerial UE height in Table B-1: LOS probability in TR 36.777

- The second height range for UMi-AV is further divided into 2 regions, i.e., [22.5, 100] and [100, 300] for mid-UAV and high-UAV, respectively.

Agreement

Update the agreements on LOS probability calculation for channel between an aerial UE and a normal UE as follows.

| Low-UAV | Mid-UAV | High-UAV |
| --- | --- | --- |
| UMi in Table 7.4.2-1 in TR 38.901 for UMi-AV/UMa-AV/RMa-AV | UMi-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region for RMa-AV | UMi-AV in Table B-1 in TR 36.777 for BS to high UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to high UAV region for RMa-AV |

Agreement

Update the agreements on LOS probability calculation for channel between two aerial UE as follows.

|  | Low-UAV | Mid-UAV | High-UAV |
| --- | --- | --- | --- |
| Low-UAV | UMi in Table 7.4.2-1 in TR 38.901 for UMi-AV/UMa-AV/RMa-AV | UMi-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region for RMa-AV | UMi-AV in Table B-1 in TR 36.777 for BS to high UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to high UAV region for RMa-AV |
| Mid-UAV | UMi-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region for RMa-AV | UMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region | 1 |
| High-UAV | UMi-AV in Table B-1 in TR 36.777 for BS to high UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to high UAV region for RMa-AV | 1 | 1 |

Agreement

To determine the LOS condition of any link in ISAC channel model, when EO type-2 is modelled, the following two options are agreed as solutions:

- Option A: If type-2 EO is in the LOS ray of the link, the LOS probability is p, p=0, and otherwise use the LOS probability equation defined in existing TRs to determine the LOS/NLOS condition

- Option C: Use the LOS probability equation to determine the LOS/NLOS condition of the link.

Note1: in which conditions/scenarios to use option A or option C can be determined in future evaluations.

Note2: as already agreed, monostatic background channel is always NLOS

Agreement

EO type-2 can be optionally modelled in background channel when EO type-2 is modelled in target channel.

Conclusion

Delete subsection 7.9.6 from the draft CR. For ISAC, no enhancement to LLS channel model is introduced in Rel-19.

Conclusion

Other than RCS for human, vehicle, AGV, UAV, no other RCS for other objects is introduced in Rel-19.

- Future studies are not precluded for adding RCS of other objects/sizes for modelling target or EO type-1, based on validation results from companies.

Conclusion

Delete subsection 7.9.6 from the draft CR. For ISAC, no enhancement to existing TR38.901 LLS channel model is introduced in Rel-19.

Agreement

- The existing blockage model A/B procedures can be reused to model the blocking effect due to a target as an optional feature

  - Applicable to the LOS/NLOS rays in the background channel of the target

  - Applicable to the LOS/NLOS rays in the Tx-target and target-Rx link of another target

- The location, orientation and size of the target as a blocker is known before applying the blockage model A/B.

Agreement

- The square brackets on formula 7.9.5-10 (copied below) in the draft CR are removed

The effective polarization matrix of the type-2 EO reflection path is given by

[CPMEO=cosγ2sinγ2-sinγ2cosγ2R∥00-R⊥cosγ1-sinγ1sinγ1cosγ1	(7.9.5-10)]

- Encourage companies to check and compare with the results that can be obtained with Alt2 and Alt3 below. If problem is found, RAN1 will revise TR 38.901 by new CR.

Alt2

[CPMEO=cosγ2-sinγ2sinγ2cosγ2R∥00-R⊥cosγ1sinγ1-sinγ1cosγ1	(7.9.5-10)]

With reusing the legacy transformation method for deriving γ1 and γ2.

Alt3

[CPMEO=cosγ2-sinγ2sinγ2cosγ2R∥00-R⊥cosγ1sinγ1-sinγ1cosγ1	(7.9.5-10)]

Where,

-	γ1=atan2-eθEO, ZODnplane,eϕEO, AODnplane. nplane represents the normal vector of the incident plane. nplane=vtx→w×vw→rx, in which vtx→w=xw-xtx,yw-ytx,zw-ztx and vw→rx=xrx-xw,yrx-yw,zrx-zw. eθEO, ZOD represents the spherical basis vector of incident ray in vertical direction. eθEO,ZOD=cosθEO, ZODcosϕEO, AOD,cosθEO, ZODsinϕEO, AOD,-sinθEO, ZODT. eϕEO,AOD represents the spherical basis vector of incident ray in horizontal direction. eϕEO,AOD=-sinφEO, AOD,cosφEO, AOD,0T.

-	γ2=atan2-eθEO, ZOAnplane,eϕEO, AOAnplane. eθEO, ZOA represents the polar basis vector of scattering ray in vertical direction. eϕEO, AOA represents the polar basis vector of scattering ray in horizontal direction. eθEO, ZOA=cosθEO, ZOAcosϕEO, AOA,cosθEO, ZOAsinϕEO, AOA,-sinθEO, ZOAT. eϕEO, AOA=-sinϕEO, AOA,cosϕEO, AOA,0T.

Conclusion

There is no consensus to introduce an exact formula for micro-Doppler in Rel-19. The placeholder in the channel impulse response is kept in the draft CR.