3GPP TSG RAN WG1 Meeting #121	R1-2503237

St. Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	9.2.1

Source:	Futurewei

Title:	Remaining Issues for UE-initiated/Event-driven Beam Management

Document for:	Discussion and decision

# Introduction

During the RAN1#120bis meeting, many agreements were reached [1], including resetting the counting based on current beam RS update for Event-2, interpretations of codepoints for additional indications of  whether CRI/SSBRI satisfying event condition is met, measurement window determination for initiating the UE initiated/event driven (UEI/ED) beam reporting, report format of L1-RSRP value of current beam for Event 1, the procedure of triggering event determination to be captured in RAN1 specification, etc. However, some important issues and details of UEI/ED beam management remain unclear or are still under discussion (FFS/TBD). In this paper, we will discuss the remaining issues related to UEI/ED beam management, focusing on down selection of different candidate options for resetting the counting of Event-2, down selection of multiple alternatives for Case-2: the 1-bit first PUCCH is collided/overlapped with a PUSCH, whether/how to handle the case if UE has not sent any first PUCCH associated for UE-initiated/event-driven beam reporting for Mode A, as well as whether/how to introduce a prohibit timer or a time interval for retransmissions of Mode A and/or Mode B.

# Discussion

## UL signaling content(s) for UEI/ED beam report

Regarding UL signaling content(s) for UEI/ED beam report, the below agreements were reached during RAN1#119 and RAN1#120 meetings [2] [3]:

Agreement On UE-initiated/event-driven beam reporting, at least one of the following is supported as an extension on L1-RSRP report format depending on Event-2. - Option-1: For each of reported CRI/SSBRI, to introduce additional indication of whether the CRI/SSBRI satisfies the condition of Event-2. - The presence of this field is enabled by RRC with subjective to UE capability. - The presence of this field is only for the case that N > 1 and the time window and M are configured - Option-2: For each of reported CRI/SSBRI, to introduce additional indication of the number of Event-2 instances for the CRI/SSBRI(s) within a time window. - The presence of this field is enabled by RRC with subjective to UE capability. - The presence of this field is only for the case that N > 1 and the time window and M are configured. - Option-3: No further enhancement. Note: As agreed in RAN1#117, at least one of the reported CRI/SSBRI(s) should satisfy the condition of Event-2 FFS: Whether/how to handle the case if UE has not sent any first PUCCH associated for UE-initiated/event-driven beam reporting, for Mode-A. Agreement On UE-initiated/event-driven beam reporting, Option-1 is supported as an extension on L1-RSRP report format depending on Event-2. - Option-1: For each of reported CRI/SSBRI, to introduce additional indication of whether the CRI/SSBRI satisfies the condition of Event-2. - The presence of this field is enabled by RRC with subjective to UE capability. - Note: The presence of this field is only for the case that N > 1 and the time window and M are configured Above is NOT applicable for event 1.

During RAN1#120 meeting, Option-1 was agreed to extend the L1-RSRP report format for UEI/ED beam report, specifically addressing Event 2.  However, as highlighted in yellow above, the issue regarding how to handle the case where UE has not sent any initial PUCCH associated with UEI/ED beam reporting in Mode-A remains open. We understand that if condition of Event 2 is met, the UE should have no reason not to send the first PUCCH for UEI/ED beam reporting. Therefore, a more reasonable scenario is that condition of Event 2 is not met, and UE is not required to send the first PUCCH for UEI/ED beam reporting. However, the UE may still receive an uplink grant from the network, triggering the UEI/ED beam reporting.

To address this issue, two potential solutions were discussed during RAN1#120 meeting [4]. The first solution involves revising the RAN1#117 agreement to state that “at least one of the reported CRI/SSBRI(s) should satisfy the condition of Event 2 only for Mode B” in a UEI/ED beam report. The second solution suggests relying on UE/NW implementation, such as network blind detection.

Regarding the first solution, we believe it should be ensured that “at least one of the reported CRI/SSBRI(s) should satisfy the condition of Event 2” applies to both Mode A and Mode B. Otherwise, UEI/ED beam reporting may lose its relevance and value for the network. Therefore, revising such a critical agreement for this case is not reasonable and should not be supported.

As for the second solution, which depends on UE/NW implementation, network blind detection may not be necessary or useful here since it was already agreed during RAN1#120 meeting that a CSI trigger state related to UEI/ED beam reporting cannot be associated with a legacy AP-CSI report configuration [3]. In that regard, the NW can clearly differentiate whether UEI/ED beam reporting is received or not based on the uplink grant scheduling. However, a new rule can be considered and defined for UE to address this issue. For example, the UE does not expect to receive an uplink grant triggering the UEI/ED beam reporting if it has not sent any first PUCCH associated for UEI/ED beam reporting.

Therefore, based on the above discussions and analysis, we have the following proposal:

Proposal 1: If UE has not sent any first PUCCH associated for UEI/ED beam reporting, UE does not expect to receive an uplink grant triggering the UEI/ED beam reporting.

## UL signaling medium/container considering the UE-initiated/event-driven UL transmission for beam report

During RAN1#120bis meeting, the below agreements on beam report transmission procedure for UEI/ED beam reporting were reached [1]:

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting for a CSI report configuration, down-select at least one of the following candidates in RAN1#121: - Candidate#1: To introduce a prohibit timer for mode-A and/or mode-B - Option-1A: In the case that triggering-event associated with the CSI report configuration is determined [by the same triggering beams(s) as the last transmitted PUCCH], if the prohibit timer is NOT running, the UE can transmit first PUCCH; - At the first symbol after the end of the PUSCH transmission, the UE starts the prohibit timer - Option-1B: In the case that triggering-event associated with the CSI report configuration is determined, if the prohibit timer is NOT running, the UE can transmit first PUCCH; - At the first symbol after the end of the first PUCCH transmission, the UE starts the prohibit timer - If the prohibit timer is running, the first PUCCH is not allowed to be transmitted. - Candidate#2: To introduce a time interval for mode-A and/or mode-B - Option-2A: For a first PUCCH transmission occasion, if there is a transmission of another second PUSCH [corresponding to the CSI report determined by the same triggering beam(s) as the first PUCCH] within a configurable time interval before the first PUCCH transmission occasion, the UE should not transmit the first PUCCH. - Option-2B: For a first PUCCH transmission occasion, if there is a transmission of another first PUCCH(s) within a configurable time interval before the first PUCCH transmission occasion, the UE should not transmit the first PUCCH. - Candidate#3: No further enhancement. Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, down-select one of the following rules for the Case-2: the 1-bit first PUCCH is collided/overlapped with a PUSCH, in RAN1#121 - Option-1: Prioritize first PUCCH over PUSCH, i.e., PUSCH is dropped. - FFS: If the PUSCH should be with UL-SCH or not for UEI beam report - Option-3: Piggyback 1-bit indication of first PUCCH into the PUSCH. - FFS: The 1-bit indication is always multiplexed in the PUSCH, regardless that UEI beam report procedure is triggered. - FFS: If the PUSCH should be with UL-SCH or not for UEI beam report - Option-4: Reuse the SR dropping rules. - FFS: whether/how to handle the case of different PHY priorities.

Regarding retransmission for the first PUCCH, three candidates (i.e., Candidate#1, Candidate#2, and Candidate#3) were proposed and discussed during RAN1#120bis meeting. If retransmission support is required, especially for Mode-A, Candidate #3 is clearly not a suitable option. Functionally, Candidate #1 and Candidate #2 can offer similar behavior, even with different mechanisms (i.e., a prohibit timer vs. a time interval for restriction). In our view, introducing the new concept of a "time interval" as in Candidate #2 is neither necessary nor convenient, and it would require additional standardization efforts without clear benefits. In that sense, Candidate#1 based on a legacy prohibit timer is preferred.

In terms of applying a prohibit timer for Mode-A and/or Mode-B in Candidate#1, two options (i.e., Option-1A and Option-1B) were proposed and discussed during RAN1#120bis meeting. In Option-1A, UE starts the prohibit timer at the first symbol after the end of the second PUSCH transmission for both Mode-A and Mode-B. In Option-1B, UE starts the prohibit timer at the first symbol after the end of the first PUCCH transmission for both Mode-A and Mode-B. However, in our understanding, whether the prohibit timer is applied or not could be completely different between Mode-A and Mode-B.

For Mode-A, UE transmits a first PUCCH to request resources for a second UL channel to carry beam report and then dynamically scheduling UL grant is used by NW to schedule uplink resources for the second UL channel. As agreed in previous RAN1 meetings, the same concept of Scheduling Request (SR) like signal can be reused for the first PUCCH in Mode-A. Since prohibit timer (e.g., used to reduce unnecessary SR requests [5]) has been defined for SR in legacy system, similar concept can also be applied to the first PUCCH in Mode-A to avoid unnecessary resource requests. Furthermore, UE could stop the prohibit timer if it received a UL grant for the second UL channel to carry beam report.

For Mode-B, the second UL channel carrying the beam report is based on pre-configured resource(s). Even though the concept of legacy SR like signal can be reused for the first PUCCH in Mode-B, the prohibit timer for the first PUCCH may not be needed. Note that it has been agreed to have a one-to-one mapping between the first PUCCH and the second UL channel. Since the second UL channel is pre-configured, its transmission timing is known at UE. Therefore, once UE transmits the first PUCCH, it knows when to send the second UL channel. Before transmitting the second UL channel, there is no need for UE to retransmit the first PUCCH. This rule can be applied without introducing a prohibit timer. After sending the second UL channel, if UE receives a negative acknowledgement, UE should be allowed to retransmit the first PUCCH without further delay. In this case, a prohibit timer is also not needed. Therefore, based on the above discussions and analysis, we have the following proposal:

Proposal 2: Support Candidate#1 and Option-1B, but only for mode-A.

- Candidate#1: To introduce a prohibit timer for mode-A and/or mode-B

  - Option-1A: In the case that triggering-event associated with the CSI report configuration is determined [by the same triggering beams(s) as the last transmitted PUCCH], if the prohibit timer is NOT running, the UE can transmit first PUCCH;

    - At the first symbol after the end of the PUSCH transmission, the UE starts the prohibit timer

  - Option-1B: In the case that triggering-event associated with the CSI report configuration is determined, if the prohibit timer is NOT running, the UE can transmit first PUCCH;

    - At the first symbol after the end of the first PUCCH transmission, the UE starts the prohibit timer

  - If the prohibit timer is running, the first PUCCH is not allowed to be transmitted.

Regarding the multiplexing/dropping rules for a 1-bit first PUCCH that collides or overlaps with a PUSCH, three options (Option-1, Option-3, and Option-4) were proposed and discussed during RAN1#120bis meeting for further down selection.

- Option-1: Prioritize the first PUCCH over the PUSCH, meaning the PUSCH is dropped. This approach may significantly impact PUSCH transmission performance, particularly when the PUSCH utilizes a large size of uplink resources and in URLLC-type services.

- Option-4: Reuse the SR dropping rules, where the first PUCCH is dropped. This option does not guarantee timely and reliable transmission of the UEI/ED beam report, as the first PUCCH may be discarded. As a result, it could negatively affect beam management performance, especially in fast beam management scenarios at high frequencies.

- Option-3: A compromise between Option-1 and Option-4, and this approach piggybacks a 1-bit indication of the first PUCCH into the PUSCH. This option can ensure the transmission performance of both the UEI/ED beam report and the PUSCH.

Therefore, based on the above discussions and analysis, we have the following proposal:

Proposal 3: Support Option-3.

  - Option-3: Piggyback 1-bit indication of first PUCCH into the PUSCH.

    - FFS: If the PUSCH should be with UL-SCH or not for UEI beam report

## Trigger-event detection for UEI/ED beam report

On the definition of events to drive UEI/ED beam report, the below agreements were reached during RAN1 #120bis and RAN1 #120 meetings [1][3]:

Agreement Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, support Option-4. - Option-4: If an Event-2 instance for a new beam is obtained at the time t and the timer for the new beam is not running, UE starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window) - Note: Timer is new beam specific. - Note: T_window is the agreed time window parameter for measurement. - Introduce separate UE capability to limit the number of timers. There is only 1 timer per new beam. Above agreement is captured in RAN1 specifications. Agreement Regarding triggering event determination for Event 2, on resetting the counting, the following modification on the agreed Candidate #2 in RAN1#120 is supported. - Candidate#2: [The measured current beam RS is updated based on] indicated TCI state is updated; - In such case, the UE needs to reset the counting for all new beams. - FFS: Further details on the update (if necessary). Agreement Regarding triggering event determination for Event 2, at least Candidate #2 is supported for resetting the counting. - Candidate#1: RS reconfiguration/update or MAC-CE signaling (if supported) for new beam is received; - FFS: whether to reset the counting for all new beams. - FFS: whether to maintain the counting whose new beam is NOT updated. - Candidate#2: [The measured current beam based on] indicated TCI state is updated; - In such case, the UE need to reset the counting for all new beams. - Candidate#3: UEI beam report is transmitted; - FFS: Only reset the counting of new beams fulfilling triggering condition and reported by the UEI beam report - Candidate#4: NW response (e.g., DCI in step-2 of Mode-A) is detected. - Candidate#5: The time window expires - Candidate#6: The threshold for event evaluation is re-configured by RRC signaling - (FFS) Candidate#7: The RRC parameter(s) associated with the CSI report configuration for UEI beam report is reconfigured. - FFS: RRC parameter(s) - FFS: Other candidates Note: Whether this proposal is captured in RAN1 or RAN2 is a separate discussion point.

Regarding the determination of the triggering condition for Event 2, seven candidates (Candidate#1 through Candidate#7) were proposed and discussed as potential methods to reset the counting in RAN1#120 and RAN1#120bis meetings. Among them, only Candidate#2 has been agreed so far. According to the definition of Event 2, the counting of Event 2 instances specifically relates to occurrences involving the same new beam. Therefore, if the new beam is changed due to RS reconfiguration/update, or MAC-CE signaling, the associated counting clearly needs to be reset. In this regard, Candidate#1 should be supported.

In Candidate#3, if a UEI beam report is transmitted, UE resets the associated counting for new beams. However, if the UEI beam report requires retransmission, Candidate#3 could introduce additional latency or even cause persistent retransmission failures, as the counting is reset and the event condition may no longer be satisfied, which is not preferred. Similarly, in Candidate#4, if a network response (e.g., DCI in step 2 of Mode-A) is detected, UE resets the associated counting for new beams. In our view, Candidate#4 shares the same drawbacks as Candidate#3, potentially to an even greater extent, and is therefore also not preferred.

In Candidate#5, if the time window expires or the timer corresponding time window expires (as agreement above), the UE resets the associated counting for new beams. That is, if Event 2 is not fulfilled when the timer expires, UE resets the associated counting for new beams. Otherwise, the evaluation outcome for Event 2 could be affected by the remaining counting value if it is not reset. Therefore, Candidate#5 should also be supported. It seems that Candidate#6 is just a special case of Candidate#7, and as such, these two candidates could potentially be merged into a single candidate for further discussion.

Therefore, based on the above discussions and analysis, we have the following proposal:

Proposal 4: In addition to Candidate #2, support at least Candidate#1 and Candidate#5.

- Candidate#1: RS reconfiguration/update or MAC-CE signaling (if supported) for new beam is received

    - FFS: whether to reset the counting for all new beams.

    - FFS: whether to maintain the counting whose new beam is NOT updated.

- Candidate#5: The time window expires

# Conclusions

Based on above discussions, we have the following observations and proposals:

Proposal 1: If UE has not sent any first PUCCH associated for UEI/ED beam reporting, UE does not expect to receive an uplink grant triggering the UEI/ED beam reporting.

Proposal 2: Support Candidate#1 and Option-1B, but only for mode-A.

- Candidate#1: To introduce a prohibit timer for mode-A and/or mode-B

  - Option-1A: In the case that triggering-event associated with the CSI report configuration is determined [by the same triggering beams(s) as the last transmitted PUCCH], if the prohibit timer is NOT running, the UE can transmit first PUCCH;

    - At the first symbol after the end of the PUSCH transmission, the UE starts the prohibit timer

  - Option-1B: In the case that triggering-event associated with the CSI report configuration is determined, if the prohibit timer is NOT running, the UE can transmit first PUCCH;

    - At the first symbol after the end of the first PUCCH transmission, the UE starts the prohibit timer

  - If the prohibit timer is running, the first PUCCH is not allowed to be transmitted.

Proposal 3: Support Option-3.

  - Option-3: Piggyback 1-bit indication of first PUCCH into the PUSCH.

    - FFS: If the PUSCH should be with UL-SCH or not for UEI beam report

Proposal 4: In addition to Candidate #2, support at least Candidate#1 and Candidate#5.

- Candidate#1: RS reconfiguration/update or MAC-CE signaling (if supported) for new beam is received

    - FFS: whether to reset the counting for all new beams.

    - FFS: whether to maintain the counting whose new beam is NOT updated.

- Candidate#5: The time window expires

# References

Draft_Minutes_report_RAN1#120bis_v010.

Draft_Minutes_report_RAN1#119_v030.

Final_Minutes_report_RAN1#120_v100.

R1-2501556-Moderator Summary #4 on UE-initiated/event-driven beam management-ZTE.

TS 38.321, NR: Medium Access Control (MAC) protocol specification (Release 18).