3GPP TSG-RAN WG1 Meeting #121	R1-2503262

St Julian’s, Malta, May 19 – 23, 2025

Agenda Item:	9.3.2

Source:	Huawei, HiSilicon

Title:	On subband full duplex random access operation

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss the remaining issues related to SBFD random access operation.

Discussion

SBFD-aware UE behaviour

In RAN1#120bis, the behavior of SBFD-aware UE transmitting in legacy ROs were agreed as follows.

Agreement For CBRA, if a SBFD-aware UE transmits Msg1 in legacy-RO, the UE follows the legacy behavior during the random access procedure. - FFS: Whether specification change is necessary

The intention of the agreement is that gNB has no idea whether the UE has the capability to support random access operation in SBFD symbols if the UE initiates random access procedure by transmitting Msg1 in legacy-ROs. However, the agreement does not apply considering the potential switch for SBFD-aware UEs. If a SBFD-aware UE transmits Msg1 in legacy-RO for several times and fails, the UE can transmit PRACH in additional-ROs. Thus, the above agreement should be clarified such that for CBRA, if a SBFD-aware UE transmits Msg1 in legacy-RO without RO type switching, the UE should follow the legacy behavior during the random access procedure.

Proposal 1: Update the RAN1#120bis agreement as follows (red)

Agreement

For CBRA, if a SBFD-aware UE transmits Msg1 in legacy-RO and does not switch from legacy-ROs to additional-ROs, the UE follows the legacy behavior during the random access procedure.

- FFS: Whether specification change is necessary

In RAN1#120bis, the following was agreed on power ramping when a SBFD-aware UE switches from additional-ROs to legacy-ROs in PRACH transmission re-attempt in one RACH procedure

Agreement For RACH configuration Option 1 and Option 2, when a SBFD-aware UE switches from additional-ROs to legacy-ROs in PRACH transmission re-attempt in one RACH procedure, support Alt-1: Increase the power ramping counter. Conclusion For RACH configuration Option 2, when a SBFD-aware UE switches from additional-ROs to legacy-ROs in PRACH transmission re-attempt in one RACH procedure, there is no RAN1 consensus to support a power offset to compensate the power ramping difference

Given that RAN2 has agreed that RO type switch from legacy-RO to additional-RO is also supported, the above agreement and conclusion can be extended to ensure a simple and unified solution

On RO type fallback - RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold (we assume it is the same threshold with the fallback from additional RO to legacy RO). If fallback from legacy RO to additional RO occurs, no further fallback to legacy RO is supported.

Proposal 2: Update the RAN1#120bis agreement and conclusion as follows (red)

Agreement

For RACH configuration Option 1 and Option 2, when a SBFD-aware UE switches from additional-ROs to legacy-ROs or from legacy-ROs to additional-ROs in PRACH transmission re-attempt in one RACH procedure, support Alt-1: Increase the power ramping counter.

Conclusion

For RACH configuration Option 2, when a SBFD-aware UE switches from additional-ROs to legacy-ROs or from legacy-ROs to additional-ROs in PRACH transmission re-attempt in one RACH procedure, there is no RAN1 consensus to support a power offset to compensate the power ramping difference

Msg3 Power control

The potential Msg3 enhancements were discussed and the following was agreed in RAN1#120bis [1].

Agreement For determination of the Msg3 PUSCH transmission power for RACH configuration Option 2: - preambleReceivedTargetPower configured for legacy-RO is used if Msg3 PUSCH is transmitted in non-SBFD symbols; - preambleReceivedTargetPower configured for additional-RO is used if Msg3 PUSCH is transmitted in SBFD symbols; - FFS whether/how to support separate msg3-DeltaPreamble for non-SBFD symbols and SBFD-symbols. Agreement - For determination of Msg3 PUSCH transmission power, ∆Prampup_requested,b,f,c is provided by MAC layer and is the amount of power ramping applied to the latest Random Access Preamble transmission, regardless of the used RO types and symbol type of Msg3 PUSCH. - FFS: For RACH configuration Option 2, interpretation of ∆Prampup_requested,b,f,c when separate PREAMBLE_POWER_RAMPING_STEP configurations for different RO types when UE switches from additional-ROs to legacy-ROs. - This does not imply that separate PREAMBLE_POWER_RAMPING_STEP configurations are supported

Delta Preamble Power

Similar to PRACH transmissions, it is also beneficial to support separate power control for Msg3 PUSCH in SBFD symbols and non-SBFD symbols by configuring separate preambleReceivedTargetPower and msg3-DeltaPreamble for RACH configuration Option 2. Specifically, preambleReceivedTargetPower for legacy-ROs and msg3-DeltaPreamble for non-SBFD symbols can be configured together, while preambleReceivedTargetPower for additional-ROs and msg3-DeltaPreamble for SBFD symbols can be configured together.

Proposal 3: For determination of Msg3 PUSCH transmission power, support separate configuration of msg3-DeltaPreamble for non-SBFD symbols and SBFD-symbols for RACH configuration Option 2

- msg3-DeltaPreamble configured for legacy-ROs is used if Msg3 PUSCH is transmitted in non-SBFD symbols;

- msg3-DeltaPreamble configured for additional-ROs is used if Msg3 PUSCH is transmitted in SBFD symbols.

Accumulated Power

In case of no RO type switch, the actual accumulated power of the latest PRACH transmission is used as the accumulated power to compute Msg3 PUSCH transmission power.

In case of RO type switch, the accumulated power of Msg3 PUSCH transmission is dependent on the power ramping steps of PRACH transmission, the power ramping counter value of PRACH transmission, and the RO type switch (from legacy-ROs to additional-ROs or the other way around). However, considering the following agreement that the power ramping counter increases when a SBFD-aware UE switches from additional-ROs to legacy-ROs in PRACH transmission re-attempt in one RACH procedure, it seems reasonable to use the accumulated power of latest PRACH transmission to determine the Msg3 PUSCH transmission power. This applies regardless of whether separate PREAMBLE_POWER_RAMPING_STEP configurations are supported or not.

Agreement For RACH configuration Option 1 and Option 2, when a SBFD-aware UE switches from additional-ROs to legacy-ROs in PRACH transmission re-attempt in one RACH procedure, support Alt-1: Increase the power ramping counter.

Proposal 4: For determination of Msg3 PUSCH transmission power, ∆Prampup_requested,b,f,c is provided by MAC layer and is the amount of power ramping applied to the latest Random Access Preamble transmission, regardless of the used RO types, the used symbol type of Msg3 PUSCH and RO type switch.

Conclusion

With the discussion above, we have the following proposals

Proposal 1: Update the RAN1#120bis agreement as follows (red)

Agreement

For CBRA, if a SBFD-aware UE transmits Msg1 in legacy-RO and does not switch from legacy-ROs to additional-ROs, the UE follows the legacy behavior during the random access procedure.

- FFS: Whether specification change is necessary

Proposal 2: Update the RAN1#120bis agreement and conclusion as follows (red)

Agreement

For RACH configuration Option 1 and Option 2, when a SBFD-aware UE switches from additional-ROs to legacy-ROs or from legacy-ROs to additional-ROs in PRACH transmission re-attempt in one RACH procedure, support Alt-1: Increase the power ramping counter.

Conclusion

For RACH configuration Option 2, when a SBFD-aware UE switches from additional-ROs to legacy-ROs or from legacy-ROs to additional-ROs in PRACH transmission re-attempt in one RACH procedure, there is no RAN1 consensus to support a power offset to compensate the power ramping difference

Proposal 3: For determination of Msg3 PUSCH transmission power, support separate configuration of msg3-DeltaPreamble for non-SBFD symbols and SBFD-symbols for RACH configuration Option 2

- msg3-DeltaPreamble configured for legacy-ROs is used if Msg3 PUSCH is transmitted in non-SBFD symbols;

- msg3-DeltaPreamble configured for additional-ROs is used if Msg3 PUSCH is transmitted in SBFD symbols.

Proposal 4: For determination of Msg3 PUSCH transmission power, ∆Prampup_requested,b,f,c is provided by MAC layer and is the amount of power ramping applied to the latest Random Access Preamble transmission, regardless of the used RO types, the used symbol type of Msg3 PUSCH and RO type switch.

- References

- R1-2503017, Summary#2 on SBFD random access operation Moderator (CMCC)