3GPP TSG-RAN WG1 Meeting #121	R1-2503263

St Julian’s, Malta, May 19 – 23, 2025

Agenda Item:	9.3.3

Source:	Huawei, HiSilicon

Title:	On cross-link interference handling for subband full duplex

Document for:	Discussion and Decision

Introduction

The WID on SBFD was approved in RAN#104 [1] and the objectives related to CLI handling were updated based on the agreements in RAN1#117. In this contribution, we discuss the remaining details of gNB-to-gNB CLI handling and UE-to-UE CLI handling.

UL resource muting for PUSCH

The objectives related to UL resource muting are listed below.

- UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - Notes: Following are assumed for UL resource muting - No new DCI field / MAC CE is introduced for indication/determination of UL resource muting - No impact on data and control multiplexing as defined in section 6.2.7, TS 38.212 - UL resource muting does not apply for Msg A PUSCH and Msg 3 PUSCH - UL resource muting applies only for UEs in RRC_CONNECTED mode - UE assumes that the UL resource muting pattern does not overlap UL DMRS or PT-RS in the same symbol - Power boosting is assumed for REs in the symbol with UL resource muting. PUSCH transmit power does not change across symbols - No changes on TBS determination for PUSCH - Check impact on transmit signal quality/MPR requirement, if any, with RAN4 - This feature is subject to UE capability

UL resource muting in non-SBFD symbols

In RAN1#120bis, whether UL resource muting can be applied for PUSCH transmissions in non-SBFD symbols was discussed but no consensus can be reached. Our preference is that UL resource muting can be used as a generic tool to facilitate interference suppression. Of course, the potential specification impact needs to be taken into account.

The following two use cases were identified by companies:

- Case 1: Dynamic/flexible TDD

- Case 2: gNB with different SBFD subband configurations

In Case 1, the UL reception in UL symbols for one gNB is interfered by DL transmissions from another gNB. In Case 2, the DL transmission from one gNB (configured with SBFD operation) in SBFD symbols may interfere the UL reception of another gNB (configured with either SBFD operation or TDD operation) in UL symbols. In both cases, if UL resource muting can be applied in non-SBFD symbols, the gNB-to-gNB CLI covariance matrix can be estimated on the muted REs in UL symbols which is beneficial for PUSCH performance.

It is possible to support UL resource muting in non-SBFD symbols without additional specification impact. For DG PUSCH and Type 2 CG PUSCH, as long as the UL resource muting symbols is configured and turned ON by TDRA field in DCI, UL resource muting is applied regardless of whether PUSCH is transmitted in SBFD symbols or non-SBFD symbols. For Type 1 CG PUSCH, as long as the UL resource muting symbols is configured, UL resource muting is applied regardless of whether PUSCH is transmitted in SBFD symbols or non-SBFD symbols. However, one may need to make an update (highlighted in red) to the following agreement.

Agreement For PUSCH repetition Type A in SBFD symbols with Configuration 2, the same time location of none, one or two UL muting symbol(s) is applied for all PUSCH repetitions in SBFD symbols. For PUSCH repetition Type B in SBFD symbols with Configuration 2, the same time location of none, one or two UL muting symbol(s) per slot is applied for all the actual PUSCH repetitions in SBFD symbols. Note: whether UL resource muting is also applied in non-SBFD symbols is discussed separately

If one would like to have some controllability at the gNB, an indicator can be introduced to disable UL resource muting in non-SBFD symbols for SBFD-aware UEs. If this indicator is provided, UL resource muting can only be applied in SBFD symbols. Otherwise, UL resource muting can be applied in both SBFD symbols and non-SBFD symbols. Overall, the specification effort to support UL resource muting in non-SBFD symbols is marginal.

Proposal 1: An indicator is provided to disable UL resource muting in non-SBFD symbols for SBFD-aware UEs

- If this indicator is provided, UL resource muting cannot be applied in non-SBFD symbols

- Otherwise, UL resource muting is applied in both SBFD symbols and non-SBFD symbols

UL resource muting for PUSCH with PT-RS for DFT-S-OFDM

For DFT-S-OFDM, when an UL muting symbol overlaps a symbol containing PT-RS, two alternatives were listed for down-selection

Agreement When transform precoding is enabled and an UL muting symbol overlaps a symbol containing PT-RS, down-select from the following alternatives in RAN1#121 - Alt-3 - For PT-RS insertion in a block of MscPUSCH2 samples prior to transform precoding - NgroupPT-RS and Nsampgroup are determined as legacy from 38.214 Table 6.2.3.2-1 - PT-RS sample positions are determined as legacy, but only for the first NgroupPT-RS2 groups - Note: After PT-RS insertion, all of the mathematically equivalent DFT Options 1, 2A, and 2B from the RAN1#120 agreement are applicable, where the choice is up to UE implementation - Alt-4 - UL resource muting is not applied in the PUSCH symbol containing PT-RS

When transform precoding is enabled for PUSCH, PT-RS are inserted into a set of PUSCH complex-valued symbols within an OFDM symbol, prior to transform precoding, as described Section 6.3.1.4, TS 38.211 [2]. If UL resource muting is applied on this symbol, the same procedure can be applied. The output of transform precoding can then be mapped to unmuted subcarriers, ensuring there is no collision between PT-RS and UL muting resources. In other words, UL resource muting on a symbol containing PT-RS for PUSCH is feasible when transform precoding is enabled.

When transform precoding is enabled for PUSCH, the time density of PT-RS can be 1 or 2, i.e., PT-RS has to be present in all PUSCH symbols or every other symbol. If UL resource muting is not allowed on symbols containing PT-RS, UL resource muting cannot be supported if the time density of PT-RS is set to 1. Therefore, from a performance perspective, it is preferable to allow UL resource muting on symbols containing PT-RS when transform precoding is enabled for PUSCH.

#### 6.3.1.4 Transform precoding If transform precoding is not enabled according to 6.1.3 of [6, TS38.214], for each layer . If transform precoding is enabled according to 6.1.3 of [6, TS38.214], υ=1 and depends on the configuration of phase-tracking reference signals. If the procedure in [6, TS 38.214] indicates that phase-tracking reference signals are not being used, the block of complex-valued symbols x00,…,x0Msymblayer-1 for the single layer λ=0 shall be divided into MsymblayerMscPUSCHsets, each corresponding to one OFDM symbol and where set l contains Msc,lPUSCH symbols and is mapped to the complex-valued symbols xl0(i'), corresponding to OFDM symbol l prior to transform precoding, with i'∈0,1,…, Msc,lPUSCH-1. If the procedure in [6, TS 38.214] indicates that phase-tracking reference signals are being used, the block of complex-valued symbols x00,…,x0Msymblayer-1 shall be divided into sets, each set corresponding to one OFDM symbol, and where set l contains Msc,lPUSCH-εlNsampgroupNgroupPTRS symbols and is mapped to the complex-valued symbols xl0(i')x0(lMscPUSCH+i') corresponding to OFDM symbol l prior to transform precoding, with i'∈0,1,…, Msc,lPUSCH-1i'∈0,1,…,MscPUSCH-1 and i'≠m. The index m of PT-RS samples in setl, the number of samples per PT-RS group Nsampgroup, and the number of PT-RS groups NgroupPT-RS are defined in clause 6.4.1.2.2.2. The quantity when OFDM symbol contains one or more PT-RS samples, otherwise .

Observation 1: If an UL resource muting symbol overlaps with a symbol containing PT-RS for a PUSCH and when transform precoding is enabled for the PUSCH, there is no collision between PT-RS and UL muting resources since PT-RS is inserted prior to DFT, and the output of DFT is mapped only to unmuted subcarriers.

When transform precoding is enabled for a PUSCH, the UE determines the PT-RS group pattern based on the number of scheduled RBs for the PUSCH, as in Table 6.2.3.2-1, Section 6.2.3.2, TS 38.214.

Table 6.2.3.2-1: PT-RS group pattern as a function of scheduled bandwidth

| Scheduled bandwidth | Number of PT-RS groups | Number of samples per PT-RS group |
| --- | --- | --- |
| NRB0 NRB < NRB1 | 2 | 2 |
| NRB1 NRB < NRB2 | 2 | 4 |
| NRB2 NRB < NRB3 | 4 | 2 |
| NRB3 NRB < NRB4 | 4 | 4 |
| NRB4 NRB | 8 | 4 |

The indexes of PT-RS samples in OFDM symbol l prior to transform precoding are determined according to Table 6.4.1.2.2.2-1, Section 6.4.1.2.2.2, TS 38.213. The index m of PT-RS samples depends on the number of PT-RS groups NgroupPT-RS, the number of samples per PT-RS group Nsampgroup, and the number of subcarriers in the scheduled bandwidth MscPUSCH .

Table 6.4.1.2.2.2-1: PT-RS symbol mapping.

| Number of PT-RS groups | Number of samples per PT-RS group | Index m of PT-RS samples in OFDM symbol prior to transform precoding |
| --- | --- | --- |
| 2 | 2 | sMscPUSCH4+k-1 where s=1,3 and k=0,1 |
| 2 | 4 | sMscPUSCH+k where s=0andk=0,1,2,3s=1andk=-4,-3,-2,-1 |
| 4 | 2 | sMscPUSCH8+k-1 where s=1,3,5,7 and k=0,1 |
| 4 | 4 | sMscPUSCH4+n+k where s=0andk=0,1,2,3n=0s=1,2andk=-2,-1,0,1n=MscPUSCH8s=4andk=-4,-3,-2,-1n=0 |
| 8 | 4 | sMscPUSCH8+n+k where s=0andk=0,1,2,3n=0s=1,2,3,4,5,6andk=-2,-1,0,1n=MscPUSCH16s=8andk=-4,-3,-2,-1n=0 |

The above procedure can be mostly reused to determine the PT-RS samples in a symbol with UL resource muting. Alt-3 in the agreement is illustrated in Figure 1. With Alt-3, the time location of PT-RS is not impacted even though this is not an essential issue in practice.

Figure 1. PT-RS samples in time domain with UL resource muting for Alt-3

The specification impact of Alt-3 is minor. Since the first NgroupPT-RS2 groups are kept for PT-RS insertion, one can simply change εl=1 to εl=1/2 as provided below

The variable Msc,lPUSCH equals MscPUSCH/2 and εl=1/2 when OFDM symbol l is occupied by a muting resource, otherwise Msc,lPUSCH=MscPUSCH.

Proposal 2: When transform precoding is enabled and an UL muting symbol overlaps a symbol containing PT-RS, support Alt-3.

- Alt-3

  - For PT-RS insertion in a block of MscPUSCH2 samples prior to transform precoding

    - NgroupPT-RS and Nsampgroup are determined as legacy from 38.214 Table 6.2.3.2-1

    - PT-RS sample positions are determined as legacy, but only for the first NgroupPT-RS2 groups

    - Note: After PT-RS insertion, all of the mathematically equivalent DFT Options 1, 2A, and 2B from the RAN1#120 agreement are applicable, where the choice is up to UE implementation

L1 based UE-to-UE CLI measurement and reporting

The objectives related to L1 based UE-to-UE CLI measurement and reporting are listed below.

- L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Notes: Following are assumed for L1 based UE-to-UE CLI measurement and reporting - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs - Periodic and semi-persistent reporting can be considered in addition to aperiodic reporting - For the new reporting quantities, at least wideband reporting is supported - The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point

Measurement resource

Number of CLI measurement resources

For CLI measurement resource, one remaining issue is the number of CLI measurement resources that can be configured for the UE. In current specification, for L1-RSRP computation, UE may be configured with CSI-RS resource setting up to 16 CSI-RS resource sets having up to 64 resources within each set. The total number of different CSI-RS resources over all resource sets is no more than 128. For L3 based CLI measurement, a UE reports the maximum number of SRS resources for SRS-RSRP measurement from {n4, n8, n16, n32} and the maximum number of CLI-RSSI resources for CLI-RSSI measurement from {n8, n16, n32, n64}. In addition, UE also reports the maximum number of SRS-RSRP measurement resources per slot for SRS-RSRP measurement from {n2, n4, n8}.

In general, either the maximum number of CSI-RS resources for L1-RSRP reporting or the maximum number of SRS-RSRP and CLI-RSSI resources for L3 based CLI measurement can be taken as the starting point. For each CSI Reporting Setting, UE can be configured with up to 32 SRS-RSRP resources and 64 CLI-RSSI resources for L1 based CLI measurement.

In addition, for CLI-RSSI measurement resource, there are two measurement resource types. For measurement resource type#2, although the CLI-RSSI measurement resource is configured across two DL subbands, however, considering CLI-RSSI measurement is more like energy detection, the measurement complexity is limited and only wideband reporting is supported, so there is no need to differentiate it from measurement resource type#1.

Proposal 3: For each CSI Reporting Setting, UE can be configured with up to 32 SRS-RSRP resources and 64 CLI-RSSI resources regardless of measurement resource type for L1 based CLI measurement depending on UE capability.

Whether to configure the symbol type for SRS-RSRP/CLI-RSSI measurement resources

In RAN1#120bis, it was proposed to configure the symbol type for CLI measurement resources similar to CSI measurement.  To some extent, this issue is similar to apply UL resource muting in non-SBFD symbols. The potential use cases also include dynamic/flexible TDD and gNB with different SBFD subband configurations. It was observed that L1 UE-to-UE CLI measurement in non-SBFD symbols can be used to capture the inter-cell UE-to-UE CLI.

However, RAN1 has already informed RAN4 that from RAN1 perspective, intra-cell UE-to-UE L1 CLI measurement scenario can be prioritized for the requirements of L1 SRS-RSRP measurement. Even if L1 SRS-RSRP or CLI-RSSI measurement is supported in non-SBFD symbols, there is no such performance requirement in RAN4 in Rel-19. In addition, the timing assumed for UE-to-UE CLI measurement in SBFD symbols may not be suitable for UE-to-UE CLI measurement in non-SBFD symbols. Therefore, it does not seem to be critical to extend the support of UE-to-UE CLI measurement in non-SBFD symbols.

Proposal 4: The symbol type configuration does not apply for SRS-RSRP/CLI-RSSI measurement resources, i.e., L1 UE-to-UE CLI measurement are restricted in SBFD symbols for SBFD-aware UEs.

Default beam for aperiodic SRS-RSRP/CLI-RSSI measurement resources

For aperiodic CLI reporting, the following was agreed in RAN1#120

Agreement If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI for aperiodic CLI reporting and the first symbol of the aperiodic SRS-RSRP measurement resource or CLI-RSSI measurement resources is smaller than the UE reported threshold beamSwitchTiming, down-select from the following alternatives: - Alt.1: UE does not expect to be configured with a scheduling offset between the last symbol of the PDCCH carrying the triggering DCI for AP CLI and the first symbol of the aperiodic CLI SRS-RSRP or CLI-RSSI resources is smaller than the UE reported threshold beamSwitchTiming. - Alt.2: Same default beam rules as AP CSI-RS resource defined in 38.214 can be applied to AP CLI SRS-RSRP or CLI-RSSI resource. FFS: whether Alt.1 or Alt.2 has any specification impact.

With Alt.1, this is treated as an error case which does not need to be capture in the specification. With Alt.2, same default beam rules as AP CSI-RS resource defined in 38.214 are applied to AP CLI SRS-RSRP or CLI-RSSI resource. Both alternative work while Alt-2 the UE behavior is much more complex. Considering that the motivation to configure the ‘type-D’ QCL assumption for CLI measurement resources is to have beam-based UE-to-UE CLI measurement. Alt-2 seems like an overoptimization of a corner case.

Proposal 5: A UE does not expect to be configured with a scheduling offset between the last symbol of the PDCCH carrying the triggering DCI for AP CLI and the first symbol of the aperiodic CLI SRS-RSRP or CLI-RSSI resources is smaller than the UE reported threshold beamSwitchTiming.

Simultaneous configuration of L1 and L3 UE-to-UE CLI measurement resources

There was a discussion whether a UE can be simultaneously configured with L1 and L3 UE-to-UE CLI measurement resources in RAN1#120bis. Generally, it is not preferred to pose any configuration restriction. However, this may result in potential overlap for the two kinds of measurement resources. To resolve this issue, one way is to define the UE behavior in specification, i.e., whether to prioritize L1 or L3 UE-to-UE CLI measurement. Another way is leave this to UE implementation. Given that it is no obvious one should prioritize one over the other, we suggest to leave this to UE implementation.

Proposal 6: A UE can be configured with L1 and L3 UE-to-UE CLI measurement resources. It is up to UE implementation whether to prioritize L1 or L3 UE-to-UE CLI measurement when two measurement resources overlap in time.

Measurement reporting

CPU occupation

The following was agreed in RAN1#120 for the number of occupied CPUs for L1 CLI measurement

Agreement For L1 CLI measurement and reporting, the number of occupied CPU(s) is determined as follows - OCPU=1 for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to ‘cli-RSSI’, ‘cli-SRS-RSRP’.

According to the WID, the existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point.

In current specification, a UE indicates the number of supported simultaneous CSI calculations NCPU with parameter simultaneousCSI-ReportsPerCC in a component carrier, and simultaneousCSI-ReportsAllCC across all component carriers. If L1 based CLI reporting and the existing CSI reporting share the existing CPUs for processing CSI reports, UE complexity will not increase. If additional CPUs can be supported for processing CLI reports, UE can support more simultaneous CSI reports and CLI reports, the cost is UE complexity. It is suggested that the number of CPUs is not increased for L1 CLI measurement and reporting for simplicity.

Proposal 7: The number of CPUs in a component carrier and across all component carriers are not increased for L1 CLI measurement and reporting.

CSI processing timeline

For CSI timeline for the aperiodic CLI measurement and reporting, the following was agreed

Agreement The existing CSI timeline for L1 RSRP reporting is reused for the aperiodic CLI measurement and reporting in Rel-19 SBFD. - FFS: whether the slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured shall not be smaller than 1.

For aperiodic CSI report, the current specification defines the timing requirement to guarantee a UE can have enough time to generate the CSI report as shown in Figure 4. For the value of Z and Z’, current specification defines three classes: low latency class (Z1), high latency class (Z2), and latency for beam-reporting (Z3). In general, the processing complexity of an aperiodic CLI report and an existing aperiodic CSI is similar. So existing CSI computation delay requirement for beam-reporting class (Z3) can be used for L1 based UE-to-UE CLI measurement and reporting.

Figure 4.  Aperiodic CSI processing timeline

It was agreed that the existing CSI timeline for L1 RSRP reporting is reused for the aperiodic CLI measurement and reporting in Rel-19 SBFD.

For the FFS, the primary concern is that for SRS-RSRP measurement, the symbol time shall be based on a constant offset relative to the DL timing, the gap time between triggering PDCCH and the aperiodic CLI measurement resource will be reduced due to the advancement of the CLI measurement resource. In current specification, several UL related UE procedures have already taken UL the effect of the timing advance into account. One such example is PUSCH preparation time, i.e., to schedule a PUSCH transmission, the gNB should ensure that the PUSCH transmission should not start earlier than an interval after the last symbol of PDCCH taking into account that PUSCH transmission will be timing advanced at the UE. Therefore, there seems no need to pose any restriction on the scheduling timeline between DCI that triggers a set of aperiodic SRS-RSRP resources and the SRS-RSRP resource set is measured.

Proposal 8: For aperiodic CLI measurement and reporting, the slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured can be smaller than 1.

Conclusions

In this contribution, the following are proposed for gNB-to-gNB CLI handling and UE-to-UE CLI handling:

Observation 1: If an UL resource muting symbol overlaps with a symbol containing PT-RS for a PUSCH and when transform precoding is enabled for the PUSCH, there is no collision between PT-RS and UL muting resources since PT-RS is inserted prior to DFT, and the output of DFT is mapped only to unmuted subcarriers.

Proposal 1: An indicator is provided to disable UL resource muting in non-SBFD symbols for SBFD-aware UEs

- If this indicator is provided, UL resource muting cannot be applied in non-SBFD symbols

- Otherwise, UL resource muting is applied in both SBFD symbols and non-SBFD symbols

Proposal 2: When transform precoding is enabled and an UL muting symbol overlaps a symbol containing PT-RS, support Alt-3.

- Alt-3

  - For PT-RS insertion in a block of MscPUSCH2 samples prior to transform precoding

    - NgroupPT-RS and Nsampgroup are determined as legacy from 38.214 Table 6.2.3.2-1

    - PT-RS sample positions are determined as legacy, but only for the first NgroupPT-RS2 groups

    - Note: After PT-RS insertion, all of the mathematically equivalent DFT Options 1, 2A, and 2B from the RAN1#120 agreement are applicable, where the choice is up to UE implementation

Proposal 3: For each CSI Reporting Setting, UE can be configured with up to 32 SRS-RSRP resources and 64 CLI-RSSI resources regardless of measurement resource type for L1 based CLI measurement depending on UE capability.

Proposal 4: The symbol type configuration does not apply for SRS-RSRP/CLI-RSSI measurement resources, i.e., L1 UE-to-UE CLI measurement are restricted in SBFD symbols for SBFD-aware UEs.

Proposal 5: A UE does not expect to be configured with a scheduling offset between the last symbol of the PDCCH carrying the triggering DCI for AP CLI and the first symbol of the aperiodic CLI SRS-RSRP or CLI-RSSI resources is smaller than the UE reported threshold beamSwitchTiming.

Proposal 6: A UE can be configured with L1 and L3 UE-to-UE CLI measurement resources. It is up to UE implementation whether to prioritize L1 or L3 UE-to-UE CLI measurement when two measurement resources overlap in time.

Proposal 7: The number of CPUs in a component carrier and across all component carriers are not increased for L1 CLI measurement and reporting.

Proposal 8: For aperiodic CLI measurement and reporting, the slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured can be smaller than 1.

# References

- RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD).

- R1-2503455, Introduction of sub-band full duplex (SBFD), Ericsson

- R1-2502245, Higher layer parameters for Rel-19 SBFD, Rapporteur (Huawei)