3GPP TSG RAN WG1 #121	R1-2503351

St Julian’s, Malta, May 19th – 23th, 2025

Source:	vivo

Title:	Remaining issues on UE-initiated/event-driven beam management

Agenda Item:	9.2.1

Document for:	Discussion and Decision

Introduction

According to the discussion in the previous meetings, the whole procedure of UE-initiated/event-driven beam report, including measurement RS determination, event definition and evaluation, report contents and reporting procedure is almost clear. However, there are some remaining details to be resolved, such as the definition of the evaluation window, resetting counting value within the evaluation procedure, and the multiplexing/dropping rule for the first PUCCH indication and PUSCH. In this contribution, we provide our views on these details.

Discussion

Measurement and event evaluation

In this section, we analyze the remaining issues on the measurement and event evaluation for UE-initiated/event-driven L1/L2 beam reporting, including the measurement RS determination for the current beam and new beam, and the condition(s) of counting resetting.

Measurement RS

Agreement Regarding explicit RS configuration for new beam measurement for Event 2, at least Option-1 is supported - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration - If legacy UE capability signaling cannot be reused, introduce a UE capability signaling of indicating the maximum number of the configured RS(s) in the RS resource set. - FFS: The RS in the RS resource set can be updated by MAC-CE

In RAN1#118 meeting [1], the above agreement related to new beam was achieved and an FFS point is left for further discussion. For the FFS, the introduction of dynamic update of new beam RS(s) by a new MAC CE is to resolve the mismatch between new beam RS(s) and current beam RS when the current beam RS is dynamically changed. Meanwhile, the UE measurement overhead and computation complexity can be greatly reduced.  For example, as shown in Figure 1, the total beam RS is RS#1~RS#9. When the current beam RS is changed to RS#7 from RS#2, if the corresponding new beams are several beams around the current beam, the corresponding new beam RSs should be updated to {RS#6, RS#8, RS#9} from {RS#1, RS#3, RS#4}. And the update of new beam RS within the set can be achieved by the new MAC CE.

Update of new beam and current beam RSs

Besides dynamic new beam RS(s) update by a new MAC CE, some candidate solutions were proposed in the last meeting. One is the new beam RS set is a large set consisting of RS(s) corresponding to the current beam(s) applied for transmission and reception, all the RSs associated with activated TCI states (up to 8) other than the indicated one, and some RSs associated with inactive TCI states. The UE has to evaluate all the RSs in the new beam RS set and compare them with the current beam to determine whether the condition of the event is satisfied, which implies very high UE measurement complexity. Furthermore, as the counting rule for active CSI-RS resources and ports is based on the CSI-RS resources associated with report configuration, the large new beam RS set requires much more UE capabilities and introduces some restrictions on the CSI-RS configuration, especially when the CSI-RS resources within the new beam RS set are associated with other report configurations. From both gNB and UE perspectives, it is not desirable to double the number of active CSI-RS resources when the new beam RS set includes all CSI-RSs, hence it is reasonable to include in the new beam RS set a subset of the RRC configured RSs with MAC-CE update when necessary. If the new beam RS update is achieved by RRC reconfiguration, though it is feasible, it would result in a longer time than MAC CE update, the uncertainty of the new beam RS application time and potential frequent RRC reconfigurations after each update of indicated TCI state or activation/deactivation of TCI states. Since the new beam RS set is explicitly configured to the UE, using legacy TCI state activation MAC CE to update new beam RS set is meaningless as the UE is limited to monitoring the RSs that are associated with the activated TCI state(s). Thus, a new MAC CE is needed.

Support dynamic update of the measurement RS in the new beam measurement resource set by a new MAC CE.

In the previous meetings, the determination of measurement RSs corresponding to the current beam and new beam(s) was discussed for the multi-CC case respectively, and the following agreement was achieved.

RAN1#118bis meeting agreement On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, the following is supported - For new beam measurement, in a CSI report configuration, configure legacy RRC parameter carrier that indicates the CC that the RS resource set associated with the CSI reporting configuration can be found - FFS: Whether the current beam RS and new beam RS(s) can be in the same CC or in different CCs, regarding cross-CC beam measurement. - FFS: Whether the indicated TCI state and new beam RS(s) can be in the same CC or in different CCs, regarding cross-CC beam measurement. RAN#120 meeting agreement On cross-CC beam report measurement for UE-initiated/event-driven beam reporting, regarding Event-2, the following is supported - The current beam RS and new beam RS(s) are in the same CC - The above does NOT imply to preclude the cross-CC TCI indication. - The CC on which the indicated TCI state is applied, and the CC in which new beam RS(s) are can be the same or different. - FFS: Whether/how to introduce an RRC parameter to indicate the CC on which the indicated TCI state is applied. RAN#120bis meeting agreement On cross-CC beam report measurement for UE-initiated/event-driven beam reporting, regarding Event-2, introduce an RRC parameter to indicate ServCellIndex on which the indicated TCI state used to determine the current beam RS is applied - FFS: Indication of the cell that corresponds to the configured first PUCCH resource in CSI report config including whether it is necessary

Based on the agreements, the following TP is proposed.

| Reason for change | In endorsed CR for TS38.214 [2], the indicated TCI state is applied to the same CC of CSI-ReportConfig, or to the CC indicated by CCofIndicatedTCI-r19 in the CSI-ReportConfig. However, according to the above agreement, the CC on which the indicated TCI state is applied, and the CC in which new beam RS(s) can be the same or different. The point that needs clarification is that the CC of the CSI-ReportConfig may not be equal to the CC in which the new beam RS is configured, especially when the RRC parameter carrier is configured in the CSI-ReportConfig. To align with the agreement, the corresponding description should be revised. |
| --- | --- |
| Summary of change | In TS38.214 section 5.2.1.5.4.1, clarify the determination of CC on which the indicated TCI state is applied in the multi-CC case. |
| Consequences if not approved | It can cause misalignment between the agreement and the specification description. |
| TS38.214 5.2.1.5.4.1a UE Initiated CSI reporting for event 2 For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType-r19 set to ‘event2’, an event instance is determined for a reference signal configured by the newBeamResourceSet-r19 (in a same CC of CSI-ReportConfig, or in the CC provided by carrier in the CSI-ReportConfig), if the L1-RSRP value determined for a transmission occasion of such reference signal is an eventThreshold-r19 greater than the L1-RSRP value determined for a transmission occasion of - the reference signal in the indicated TCI state applied to the same CC of the reference signal configured by the newBeamResourceSet-r19 CSI-ReportConfig, or to the CC indicated by CCofIndicatedTCI-r19 in the CSI-ReportConfig), if newBeamResourceSet-r19 is a NZP-CSI-RS-ResourceSet configured with repetition, else - the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state (applied to the same CC of the reference signal configured by the newBeamResourceSet-r19 CSI-ReportConfig or to the CC indicated by CCofIndicatedTCI-r19 in the CSI-ReportConfig), if newBeamResourceSet-r19 is a CSI-SSB-ResourceSet. < Unchanged parts are omitted > |  |

Event-related issues

For the event evaluation procedure, related agreements were achieved as listed below.

RAN1#117 meeting agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification. RAN1#118bis meeting agreement Regarding the triggering event determination for Event 2, the event instance(s) counting is per new beam. Further study candidate condition(s) of resetting the counting including whether resetting is needed. RAN1#120 meeting agreement Regarding triggering event determination for Event 2, at least Candidate #2 is supported for resetting the counting. - Candidate#1: RS reconfiguration/update or MAC-CE signaling (if supported) for new beam is received; - FFS: whether to reset the counting for all new beams. - FFS: whether to maintain the counting whose new beam is NOT updated. - Candidate#2: [The measured current beam based on] indicated TCI state is updated; - In such case, the UE need to reset the counting for all new beams. - Candidate#3: UEI beam report is transmitted; - FFS: Only reset the counting of new beams fulfilling triggering condition and reported by the UEI beam report - Candidate#4: NW response (e.g., DCI in step-2 of Mode-A) is detected. - Candidate#5: The time window expires - Candidate#6: The threshold for event evaluation is re-configured by RRC signaling - (FFS) Candidate#7: The RRC parameter(s) associated with the CSI report configuration for UEI beam report is reconfigured. - FFS: RRC parameter(s) - FFS: Other candidates Note: Whether this proposal is captured in RAN1 or RAN2 is a separate discussion point. RAN1#120bis meeting agreement Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, support Option-4 of the following options in RAN1#120bis. - Option-4: If an Event-2 instance for a new beam is obtained at the time t and the timer for the new beam is not running, UE starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window) - Note: Timer is new beam specific. - Note: T_window is the agreed time window parameter for measurement. - Introduce separate UE capability to limit the number of timers. There is only 1 timer per new beam. Above agreement is captured in RAN1 specifications.

According to the agreement, the event instance(s) counting is performed per new beam and the counting value will be reset when Candidate#2 occurs. Besides Candidate#2, Candidate#1 and Candidate#6 should also be supported for resetting the counting value. In Candidate#1, when some of the new beam RS(s) are updated by RRC or MAC CE, the counting value of the new beam RS(s) that are not included in the new beam RS set should be reset, and the counting value(s) of the new beam RS(s) that is not updated should be maintained.

Support Canidate#1, i.e., when new beam RS is updated by RRC or MAC CE, the counting value of the new beam RS(s) that are not included in the new beam RS set should be reset, and the counting value of the new beam RS is not updated should be maintained.

Support Candidate#6, i.e., when the threshold for event evaluation is reconfigured by RRC signaling, the counting value of all new beam RS(s) should be reset.

Based on the above agreement, the following TP is proposed.

| Reason for change | According to the following agreement, only one time window is allowed for a new beam RS. However, in endorsed CR for TS38.214 [2], the condition for starting the time window corresponds to a new beam RS is the time window does not overlap with the other time windows for the new beam RS. It may introduce a misunderstanding that multiple time windows can be performed for a new beam RS. To avoid this ambiguity, the wording of the condition for starting the time window corresponds to a new beam RS should be revised. RAN1#120bis meeting agreement Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, support Option-4 of the following options in RAN1#120bis. - Option-4: If an Event-2 instance for a new beam is obtained at the time t and the timer for the new beam is not running, UE starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window) - Note: Timer is new beam specific. - Note: T_window is the agreed time window parameter for measurement. - Introduce separate UE capability to limit the number of timers. There is only 1 timer per new beam. Above agreement is captured in RAN1 specifications.. |
| --- | --- |
| Summary of change | In TS38.214 section 5.2.1.5.4.1, clarify the condition for starting the time window for the evaluation procedure of the UE-initiated/event-driven beam reporting. |
| Consequences if not approved | It can cause misalignment between the agreement and the specification description. |
| TS38.214 5.2.1.5.4.1 UE Initiated CSI reporting For a UE configured with a CSI-ReportConfig with eventType-r19 for periodic reference signals [with the same periodicity configured by the newBeamResourceSet-r19] and with dl-OrJointTCI-StateList, when an event instance is determined (as described in the following clauses), - for a reference signal configured by the newBeamResourceSet-r19, if eventType-r19 is set to ‘event2’ or ‘event7’, or - for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI, if eventType-r19 is set to ‘event1’, [if eventDetectionTimeWindowLength-r19 is provided, and the time window for such reference signal has not started yet or has ended, the UE starts a time window for such reference signal., wherein the time window does not overlap with other time windows for such reference signal.] When the number of event instances determined for such reference signal within the time window, if eventDetectionTimeWindowLength-r19 is provided, is greater than or equal to 1 or to eventInstanceCount-r19, if provided, the UE transmits PUCCH format 0 or format 1 in the PUCCH resource configured by PUCCHResource-r19 in the CSI-ReportConfig. < Unchanged parts are omitted > |  |

Report content

According to the agreements in previous meetings, report content part is completed for Event-1 and Event-2. At most 4 new beams would be reported in a report instance, where at least one of them needs to satisfy the condition of trigger event and the legacy L1-RSRP differential reporting is reused. Besides, whether the current beam is always reported or not is determined by RRC configuration. Except above, an additional indication of whether the CRI/SSBRI satisfies the condition of Event-2 and absolute L1-RSRP reporting for the current beam for Event-1 was agreed in the last meeting.

Agreement On UE-initiated/event-driven beam reporting, support the following interpretation on each of the codepoints of the ‘1-bit’ condition met indicator as follows: - ‘0’ – indicating that the Event-2 instances for corresponding CRI/SSBRI within the time window doesn’t reach the configured number M. - ‘1’ – indicating that the Event-2 instances for corresponding CRI/SSBRI within the time window reach the configured number M. FFS: whether/how to introduce the ‘1-bit’ condition met indicator for Event 7. Conclusion There is no RAN1 consensus on the following proposal: On UE-initiated/event-driven beam reporting, support at least Option-1 of following for Event 7 as an extension on report format for Event-2 - Option-1: Additional field to indicate the codepoint of the activated TCI state with Q-th best quality. - FFS: Further report codepoints of other activated TCI state(s) [, e.g., from {Q+1}-th best quality to the worst one]; - FFS: Details of the additional indication. - Option-2: Extending the maximum number of reported RS(s) to 8. - [Note: If Option 2 is agreed, RAN1 can revisit the agreed maximum value of N for event-2.] - Above is applicable only if time window and a configurable value M for event counting on Event-7 are not configured.

For Event-7, the corresponding report content still needs to be discussed. One remaining issue is whether to introduce the 1-bit condition met indicator for Event-7.  To drive a unified design of Event-2, we think the 1-bit condition met indicator is necessary for Event-7. However, even if the 1-bit condition met indicator is introduced, the network cannot decide on TCI state activation/deactivation operation accurately without the knowledge of the beam quality of the activated TCI state(s).  To resolve this issue, one feasible approach is to introduce a bitmap of active TCI states used to indicate which active TCI state(s) could be replaced by reported new beam(s) that meet the event trigger condition.

For Event-7, besides N reported new beams, support the introduction of the 1-bit condition met indicator and a bitmap of active TCI states used to indicate which active TCI state(s) could be replaced by reported new beam(s) that meets the event trigger condition.

For the case that UE receives a DCI scheduling PUSCH to carry UE-initiated/event-driven beam report but no first PUCCH associated for UE-initiated/event-driven beam reporting is transmitted before, we think it is a corner case. The false detection probability of the first PUCCH channel is generally lower than 1% and it can be further reduced by some enhanced operations, like PUCCH repetition and retransmission. In addition, the concept of UE-initiated/event-driven beam reporting is that UE reports beam(s) only when the trigger condition is satisfied. Therefore, if UE has not sent any first PUCCH associated with UE-initiated/event-driven beam reporting but receives a DCI scheduling a PUSCH to carry at least one UE-initiated/event-driven beam report, the UE may ignore the scheduling DCI if no HARQ-ACK or transport block is multiplexed on the PUSCH. However, when a transport block and/or HARQ-ACK is multiplexed on the PUSCH, whether/how to transmit the beam report may need further study.

If UE has not sent any first PUCCH associated with UE-initiated/event-driven beam reporting but receives a PDCCH scheduling a PUSCH to carry a UE-initiated/event-driven beam report, the scheduling DCI may be ignored by UE if no HARQ-ACK or transport block is multiplexed on the PUSCH.

Report modes

UCI-based UE-initiated/event-driven beam reporting was agreed and the following two modes were supported.

- Mode A (dynamically scheduling UCI by gNB, as shown in Figure 3): - Step 1: UE transmits a first PUCCH (one-bit) to request a resource for a second UL channel to carry beam report - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry beam report. - Step 3: Beam report is transmitted in second UL channel. - This mode is basic UE capability (i.e. all UE supporting UE-initiated/event-driven beam reporting should support this feature). - Mode B (UCI in pre-configured resource(s) for second UL channel, as shown in Figure 4): - Step 1: UE transmits a first PUCCH (one-bit) notifying a second UL channel to carry beam report - Step 2: UE transmits the beam report in the second UL channel. - The notification in Step1 is in a separate reporting instance from the beam report in Step 2.

Procedure of beam reporting for Mode A

Procedure of beam reporting for Mode B

According to the agreements in previous meetings, most of the details on the above report procedures are clear, such as UCI type of the first PUCCH (one-bit) and reporting resource. In this section, we provide our views on the remaining issues of the two modes, such as the overlapping handing rule for the first PUCCH indication and second uplink channel, and the condition of the first PUCCH transmission, etc.

- First PUCCH transmission

To avoid frequent transmission of the first PUCCH indication, some transmission restrictions should be considered for both Mode A and Mode B. As shown in the following agreement achieved in the last meeting [3], some candidate solutions were proposed that need to be down-selected in this meeting.

Agreement: On beam report transmission procedure for UE-initiated/event-driven beam reporting for a CSI report configuration, down-select at least one of the following candidates in RAN1#121: - Candidate#1: To introduce a prohibit timer for mode-A and/or mode-B - Option-1A: In the case that triggering-event associated with the CSI report configuration is determined [by the same triggering beams(s) as the last transmitted PUCCH], if the prohibit timer is NOT running, the UE can transmit first PUCCH; - At the first symbol after the end of the PUSCH transmission, the UE starts the prohibit timer - Option-1B: In the case that triggering-event associated with the CSI report configuration is determined, if the prohibit timer is NOT running, the UE can transmit first PUCCH; - At the first symbol after the end of the first PUCCH transmission, the UE starts the prohibit timer - If the prohibit timer is running, the first PUCCH is not allowed to be transmitted. - Candidate#2: To introduce a time interval for mode-A and/or mode-B - Option-2A: For a first PUCCH transmission occasion, if there is a transmission of another second PUSCH [corresponding to the CSI report determined by the same triggering beam(s) as the first PUCCH] within a configurable time interval before the first PUCCH transmission occasion, the UE should not transmit the first PUCCH. - Option-2B: For a first PUCCH transmission occasion, if there is a transmission of another first PUCCH(s) within a configurable time interval before the first PUCCH transmission occasion, the UE should not transmit the first PUCCH. - Candidate#3: No further enhancement.

As the event evaluation procedure and reporting procedure are performed on the physical layer, it is most straightforward that the whole procedure of UE-initiated/event-driven beam reporting is a total physical-layer operation. Hence, Candidate#1 is not suitable since the timer is a MAC-layer element, rather than the physical layer. In contrast, Candidate#2 is more reasonable.  Compared to Option-2B, Option-2A is preferred. In Option-2A, the prohibit time interval starts from the last symbol of the second UL channel, thus PUCCH transmission reliability would be improved a lot. Furthermore, we think the bracket within the Option-2A should be removed. The prohibit time interval of the first PUCCH transmission should be performed per new beam for Event-2 and Event-7, or per current beam for Event-1. If the triggered beam or triggered event is changed, the prohibit time interval should not work.

Support Option-2A, i.e., for a first PUCCH transmission occasion, if there is a transmission of another second PUSCH corresponding to the CSI report determined by the same triggering beam(s) as the first PUCCH within a configurable time interval before the first PUCCH transmission occasion, the UE should not transmit the first PUCCH.

Besides the prohibit time interval for the first PUCCH transmission, the additional condition of the first PUCCH indication transmission for Mode B is that the CG occasion corresponding to the PUCCH transmission occasion is available. In Mode B, the pre-configured resources for the second UL channel are not available if they overlap with semi-static DL symbols or SSBs or PRACH occasions. In this case, UE cannot transmit the beam report. Thus, the transmission of the first PUCCH channel may be unnecessary.

Only when the event trigger condition is satisfied, and the associated resource for the second UL channel is available, the corresponding first PUCCH channel can be transmitted in Mode B.

RAN#120bis meeting agreement On cross-CC beam report measurement for UE-initiated/event-driven beam reporting, regarding Event-2, introduce an RRC parameter to indicate ServCellIndex on which the indicated TCI state used to determine the current beam RS is applied FFS: Indication of the cell that corresponds to the configured first PUCCH resource in CSI report config including whether it is necessary

As shown in the above agreement, another remaining issue about the first PUCCH transmission is the determination of the CC that corresponds to the configured first PUCCH resource in the CSI report configuration if the first PUCCH resource and second UL channel are in different PUCCH groups. In our view, it is necessary to introduce a new RRC parameter to indicate the CC to which the first PUCCH resource belongs. Furthermore, similar to the existing parameter pucch-cell, the parameter can be a cell-level parameter, which means the first PUCCH resources in all UE-initiated report configurations belong to the same CC indicated by the parameter.

If UE supports the first PUCCH channel and the second UL channel in different PUCCH groups, a new RRC parameter should be introduced to indicate the CC that corresponds to the configured first PUCCH resource(s) in the CSI report configuration(s) in the CC where the RRC parameter is configured.

- Handling rule for overlapping cases

In the last meeting, we discussed the overlapping case of the first PUCCH channel and PUSCH, and the following agreement related to the overlap handling rule was achieved.

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, down-select one of the following rules for the Case-2: the 1-bit first PUCCH is collided/overlapped with a PUSCH, in RAN1#121 - Option-1: Prioritize first PUCCH over PUSCH, i.e., PUSCH is dropped. - FFS: If the PUSCH should be with UL-SCH or not for UEI beam report - Option-3: Piggyback 1-bit indication of first PUCCH into the PUSCH. - FFS: The 1-bit indication is always multiplexed in the PUSCH, regardless that UEI beam report procedure is triggered. - FFS: If the PUSCH should be with UL-SCH or not for UEI beam report - Option-4: Reuse the SR dropping rules. - FFS: whether/how to handle the case of different PHY priorities.

In our view, Option-3 is the most reasonable as it is a trade-off between the first PUCCH transmission and PUSCH transmission. In Option-1, the first PUCCH indication always has a higher priority over PUSCH, regardless that the UE-initiated/event-driven beam report is triggered. To avoid blind detection on the network side, even if the first PUCCH indication is not transmitted, the overlapped PUSCH would not be transmitted.  Thus, PUSCH transmission will be affected a lot. In Option-4, if the SR dropping rule is reused, when the first PUCCH transmission overlaps with PUSCH with UL-SCH, the first PUCCH transmission will be dropped. In this case, the first PUCCH indication has a lower priority than PUSCH with UL-SCH. However, data transmission reliability may be low under a beam pair with poor quality if beam correspondence is supported. Therefore, Option-3 is preferred. Furthermore, in Option-3, the 1-bit indication is always multiplexed in the PUSCH, regardless of whether the UE-initiated/event-driven beam report is triggered and the information type carried in the PUSCH. To avoid ambiguity on the network side, the 1-bit indication is set to “1” when the UE-initiated/event-driven beam report is triggered, otherwise, the 1-bit indication is set to “0”.

When the first PUCCH channel overlaps with the PUSCH in the time domain, the 1-bit PUCCH indication is multiplexed and transmitted on the PUSCH by puncturing regardless of whether the UE-initiated/event-driven beam report procedure is triggered and the information type carried in the PUSCH.

- The 1-bit indication is set to “1” when the UE-initiated/event-driven beam report procedure is triggered, otherwise, the 1-bit indication is set to “0”.

In the current specification, the PHY priority is introduced to prioritize the URLLC traffic over the eMBB traffic. However, UE-initiated/event-driven beam reporting is a common design for both traffic types. Therefore, it is unnecessary to introduce different priorities for the first PUCCH indication. By default, the priority index of the first PUCCH indication is 0. If the PUCCH carrying the first PUCCH indication overlaps with other UCI with different priority indexes, the dropping/multiplexing rule of SR can also be reused.

The priority index of the first PUCCH indication is 0 and the multiplexing/dropping rule of SR can be reused when the PUCCH carrying the first PUCCH indication overlaps with other UCI with different priority indexes.

Besides the overlapping case of first PUCCH and PUSCH, the handling rule for overlapping between the second UL channel of UE-initiated/event-driven beam reporting and other UL channels should also be discussed. In the last meeting, the second UL channel in Mode-B (i.e., the pre-configured Type-1 CG PUSCH carrying the beam report) was agreed to reuse the intra-UE multiplexing/prioritization rules of PUSCH with SP-CSI. However, the handling rule of overlapping between the Type-1 CG PUSCH carrying the beam report and PUSCH with SP CSI, and overlapping between the PUSCH carrying the beam report in Mode A and other UL channels has not been determined yet.

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case the pre-configured Type-1 CG PUSCH carry the beam report, for the second UL channel in Mode-B, reuse the intra-UE multiplexing/prioritization rules of PUSCH with SP-CSI for Type-1 CG PUSCH with UEI-BR for Mode B

When the Type-1 CG PUSCH carrying the beam report and PUSCH with SP CSI overlap in the time domain, the PUSCH with SP CSI should be dropped. The UE-initiated/event-driven beam report is a type of aperiodic reporting and it can reflect the beam change quickly, so that it should have a higher priority than SP CSI report on PUSCH.

When the Type-1 CG PUSCH carrying the beam report in Mode B overlaps with PUSCH with SP CSI in the time domain, the PUSCH with SP CSI should be dropped.

For the overlapping case of the PUSCH carrying the beam report in Mode A and other UL channel(s), we think the intra-UE multiplexing/prioritization rules of PUSCH with AP-CSI can be reused, except for the case of the PUSCH carrying the beam report in Mode A and the PUSCH with AP-CSI. As a CSI trigger state corresponding to UE-initiated/event-driven beam reporting cannot be associated with legacy AP-CSI report configuration, and both of the PUSCH resources are scheduled by the network, the overlapping case between the PUSCH carrying the beam report in Mode A and the PUSCH with AP CSI can be avoided by the network. Thus, the multiplexing/prioritization rule can be simpler.

Support reuse of the intra-UE multiplexing/prioritization rules of PUSCH with AP-CSI for the PUSCH with UEI-BR for Mode A.

UE doesn’t expect that the PUSCH with UEI-BR for Mode A overlaps with the PUSCH carrying AP CSI in the time domain.

Support the following TP.

| Reason for change | In the endorsed CR for TS38.213 [4], only the Type-1 CG PUSCH carrying the beam report in Mode B reuses the intra-UE multiplexing/prioritization rules of PUSCH with SP-CSI is captured. However, the handling rule of overlapping between the Type-1 CG PUSCH carrying the beam report and PUSCH with SP CSI, and overlapping between the PUSCH carrying the beam report in Mode B and other UL channels has not been determined yet. Considering UE-initiated/event-driven beam report is aperiodic and can imply the beam change quickly, it has a higher priority than SP CSI. When the Type-1 CG PUSCH carrying the beam report in Mode B overlaps with the PUSCH with SP-CSI in the time domain, the PUSCH with SP-CSI will be dropped. For the overlapping case of the PUSCH carrying the beam report in Mode A and other UL channel(s), we think the intra-UE multiplexing/prioritization rules of PUSCH with AP-CSI can be reused, except for the case of the PUSCH carrying the beam report in Mode A and the PUSCH with AP-CSI. Furthermore, the case of the PUSCH carrying the beam report in Mode A overlapping with the PUSCH with AP CSI in the time domain can be avoided by the network. |
| --- | --- |
| Summary of change | In TS38.213 section 9, clarify the multiplexing/prioritization rules of the PUSCH with AP CSI is reused for the second UL channel in Mode A and the scheduling restriction on PUSCH with AP-CSI and PUSCH with UE-initiated/event-driven beam report in Mode A. In TS38.214 section 5.2.5, clarify that the UE-initiated beam report in mode B has a higher priority over the SP CSI report on PUSCH. |
| Consequences if not approved | The multiplexing/prioritization rules of the second UL channel for Mode A and Mode B are unclear. |
| TS38.213 9 UE procedure for reporting control information < Unchanged parts are omitted > For the remaining of this clause, when a UE - is not provided coresetPoolIndex or is provided coresetPoolIndex with a value of 0 for first CORESETs, and is provided coresetPoolIndex with a value of 1 for second CORESETs, on active DL BWPs of serving cells, and - is provided sTx-2Panel the UE separately determines and resolves time overlapping among first PUSCH transmissions that use respective first spatial domain filters corresponding to first TCI-State or TCI-UL-State associated with the first CORESETs, and among second PUSCH transmissions that use respective second spatial domain filters corresponding to second TCI-State or TCI-UL-State associated with the second CORESETs. For the remaining of this clause, reference to PUSCH with semi-persistent CSI reports includes a PUSCH with UE initiated reports when reportTransmissionMode is configured as ‘ModeB’ [6, TS 38.214]. A UE does not expect that a PUSCH with UE initiated reports when reportTransmissionMode is configured as ‘ModeA’ overlaps with PUSCH with aperiodic CSI reports. For the remaining of this clause, reference to PUSCH with aperiodic CSI reports includes a PUSCH with UE initiated reports when reportTransmissionMode is configured as ‘ModeA’ [6, TS 38.214]. When a UE determines overlapping for PUCCH transmissions with SL HARQ-ACK reports and PUCCH of larger and/or smaller priority index, the UE resolves the overlapping for PUCCH transmissions with SL HARQ-ACK reports and PUCCH of each priority index as described in clause 9.2.5 and 9.2.6 before resolving the overlapping for PUCCH transmissions without SL HARQ-ACK or the overlapping for PUCCH transmissions and PUSCH transmissions. When a UE determines overlapping for PUCCH and/or PUSCH transmissions of the same priority index other than PUCCH transmissions with SL HARQ-ACK reports before considering limitations for UE transmission due to cell DRX operation [11, TS 38.321] or as described in clauses 11.1, 11.1.1, 11.2A, 15 and 17.2 including repetitions if any, < Unchanged parts are omitted > TS38.214 5.2.5 Priority rules for CSI reports For two overlapping PUSCHs, the priority rules in this clause are applied for physical channels with same priority index according to clause 9 in [6, TS 38.213] if a UE is not configured with sTx-2Panel or a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in different ControlResourceSet in the active DL BWP and the UE is configured with sTx-2Panel and the two overlapping PUSCHs are associated with same value of coresetPoolIndex. CSI reports are associated with a priority value PriiCSIy,k,c,s=2∙Ncells∙Ms∙y+Ncells∙Ms∙k+Ms∙c+s where - for aperiodic CSI reports to be carried on PUSCH for semi-persistent CSI reports to be carried on PUSCH, for semi-persistent CSI reports to be carried on PUCCH and for periodic CSI reports to be carried on PUCCH; - for CSI reports carrying L1-RSRP or L1-SINR and for CSI reports not carrying L1-RSRP or L1-SINR; - c is the serving cell index and Ncells is the value of the higher layer parameter maxNrofServingCells; - for a CSI report configured with ltm-CSI-ReportConfig, c is the serving cell index value where the report configuration is configured. - s is the reportConfigID and is the value of the higher layer parameter maxNrofCSI-ReportConfigurations. - for a CSI report configured with ltm-CSI-ReportConfig, s is the ltm-CSI-ReportConfigId and Ms is the value of the higher layer parameter maxNrofLTM-CSI-ReportConfigurations A first CSI report is said to have priority over second CSI report if the associated value is lower for the first report than for the second report. Two CSI reports are said to collide if the time occupancy of the physical channels scheduled to carry the CSI reports overlap in at least one OFDM symbol and are transmitted on the same carrier. When a UE is configured to transmit two colliding CSI reports, - if y values are different between the two CSI reports, the following rules apply except for the case when one of the y value is 2 and the other y value is 3 (for CSI reports transmitted on PUSCH, as described in Clause 5.2.3; for CSI reports transmitted on PUCCH, as described in Clause 5.2.4): - The CSI report with higher PriiCSIy,k,c,s value shall not be sent by the UE. - otherwise, the two CSI reports are multiplexed or either is dropped based on the priority values, as described in Clause 9.2.5.2 in [6, TS 38.213]. A CSI report configured with ltm-CSI-ReportConfig has a higher priority over all CSI report(s) configured with CSI-ReportConfig irrespective of PriiCSIy,k,c,s value in case of collision with CSI report(s) configured with CSI-ReportConfig. A CSI report configured with reportTransmissionMode-r19 set to ‘ModeB’ has a higher priority over all semi-persistent CSI report(s) to be carried on PUSCH in case of collision with the CSI report(s) configured with CSI-ReportConfig. If a semi-persistent CSI report to be carried on PUSCH overlaps in time with PUSCH data transmission in one or more symbols on the same carrier, and if the earliest symbol of these PUSCH channels starts no earlier than N2+d2,1 symbols after the last symbol of the DCI scheduling the PUSCH where d2,1 is the maximum of the d2,1 associated with the PUSCH carrying semi-persistent CSI report and the PUSCH with data transmission, the CSI report shall not be transmitted by the UE. Otherwise, if the timeline requirement is not satisfied this is an error case. If a UE would transmit a first PUSCH that includes semi-persistent CSI reports and a second PUSCH that includes an UL-SCH on the same carrier, and the first PUSCH transmission would overlap in time with the second PUSCH transmission, the UE does not transmit the first PUSCH and transmits the second PUSCH. The UE expects that the first and second PUSCH transmissions satisfy the above timing conditions for PUSCH transmissions that overlap in time when at least one of the first or second PUSCH transmissions is in response to a DCI format detection by the UE. |  |

- Association between the first PUCCH channel and report configuration

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, confirming the following working assumption with the following modification. - For Mode-A, the multiple CSI report configurations associated with the same PUCCH resource should be associated with a same CSI-AperiodicTriggerState. - FFS: For Mode-A, the multiple CSI report configurations associated with the same CSI-AperiodicTriggerState should be associated with a same PUCCH resource.

To ensure the correct scheduling of the second UL channel on the network side, the working assumption was confirmed in the last meeting. However, the other configuration restriction, i.e., the multiple CSI report configurations associated with the same CSI-AperiodicTriggerState should be associated with the same PUCCH resource for Mode A, is unnecessary. As shown in Figure 4, CSI report config#1 and CSI report config#2 associated with CSI trigger state#x are associated with PUCCH resource#1 and PUCCH resource#2 respectively. Once one or both of the PUCCH resources are received by the network, the CSI trigger state#x would be indicated in the PDCCH to schedule beam reporting. Therefore, the mechanism can work well without the configuration restriction. Though the configuration restriction may save PUCCH resource overhead, it can be achieved by network implementation. Therefore, it is unnecessary to specify.

Association relationship

The multiple CSI report configurations associated with the same CSI-AperiodicTriggerState don't have to be associated with the same PUCCH resource for Mode A.

- Timeline of the second UL channel for Mode A and Mode B

Similar to the legacy aperiodic CSI reporting, the second UL channel for UE-initiated/event-driven beam report is scheduled by DCI in Mode A, where a CSI trigger state is indicated and the CSI trigger state is only associated with UE-initiated/event-driven beam report(s). In the legacy aperiodic CSI reporting, when the scheduled PUSCH is used to transmit a transport block and CSI report(s), UE determines the transmission slot of the PUSCH by mapping the TDRI field value to a resource allocation table. If the scheduled PUSCH is only used to transmit CSI report(s), UE determines the transmission slot of the PUSCH by mapping the TDRI field value to a list of report slot offsets. However, according to the RRC parameter list [5], the list of report slot offsets for UE-initiated/event-driven beam report is absent. Thus, the determination of the transmission slot of the PUSCH carrying the UE-initiated/event-driven beam report is unclear when the scheduled PUSCH is only used to transmit UE-initiated/event-driven beam report(s). To resolve it, one potential solution is to introduce a new RRC parameter of the list of report slot offsets to the UE-initiated/event-driven beam report configuration. The other solution is to always determine the transmission slot of the PUSCH by mapping the TDRI field value to the resource allocation table, even if the PUSCH only carries UE-initiated/event-driven beam report(s) for Mode A.

To ensure the consistency of the report content and event triggering status, the report content included in the PUSCH, including the reported RS IDs and corresponding measurement results, should be acquired before the corresponding PUCCH indication. Under this assumption, the scheduling offset of the PUSCH carrying the UE-initiated/event-driven beam report in Mode A can be almost the same as the PUSCH that only carries a transport block. Therefore, it is feasible that the transmission slot of the PUSCH can always be determined by mapping the TDRI field value to the resource allocation table, even if the PUSCH only carries UE-initiated/event-driven beam report(s) for Mode A. In addition, the scheduling timeline in Mode A also needs clarification. For example, the CSI computation time of the UE-initiated\event-driven beam report should be optimized. In detail, the value of Z should be reduced, where only PDCCH decoding and PUSCH preparation time should be considered. Besides, Z’ is unnecessary for Mode A.

The report content included in the PUSCH, including the reported RS ID(s) and corresponding measurement results, should be acquired before the corresponding PUCCH indication, regardless of Mode A and Mode B.

In mode A, the transmission slot of the PUSCH should be determined by mapping the TDRI field value to the resource allocation table when the UE receives a DCI scheduling a PUSCH used to carry only UE-initiated/event-driven beam report(s).

In Mode A, the CSI computation time of UE-initiated/event-driven beam report should be optimized.

For Mode B, one remaining issue is the candidate value of X symbol(s) between the last symbol of the first PUCCH indication and the first symbol of the available transmission occasion of the pre-configured CG PUSCH. Based on the above assumption, the minimum value of X can be 0 for the UEs with advanced capability. But for the UEs with poor capability, the minimum value of X may need further study. In addition, the definition of the available transmission occasion of the pre-configured CG PUSCH is unclear. In our view, an available transmission occasion corresponds to a configured PUSCH resource for UEIBR that does not overlap with a DL semi-static configured symbol or flexible symbol and does not conflict with a channel of higher priority, such as PRACH.

An available transmission occasion corresponds to a configured PUSCH resource for UEIBR in Mode B that does not overlap with a DL semi-static configured symbol or flexible symbol and does not conflict with a channel of higher priority, such as PRACH.

The minimum value of X in Mode B, i.e., the number of symbol(s) between the last symbol of the first PUCCH indication and the first symbol of the available transmission occasion of the pre-configured CG PUSCH can be {0, 1, …, N}, where the value of N needs further discussion.

CSI processing unit occupation rule for UE-initiated/event-driven beam report

Within the procedure of UE-initiated/event-driven beam reporting, RS measurement is performed in the physical layer, and the first indication and beam report are carried as UCI to be transmitted. Therefore, it is a total L1 operation which is just another type of CSI report. In the current specification, CSI processing unit occupation rule is defined to determine the UE capacity for calculation and caching occupied by each CSI report, ensuring the orderly CSI calculation and reporting. Since beam measurement and caching of measurement results within the procedure of UE-initiated/event-driven beam reporting also occupy a certain amount of UE capacity, to ensure UE processing all kinds of CSI reports orderly, the CSI processing unit occupation rule should take UE-initiated/event-driven beam reporting into account.

To ensure UE can process NW-initiated CSI report and UE-initiated/event-driven beam report orderly, the occupied CPU number and occupation time for the UE-initiated/event-driven beam report should be defined.

In the event evaluation procedure, UE needs to perform measurement and data caching and so the corresponding CSI processing unit(s) need to be occupied.  According to previous discussion, two event evaluation mechanisms have been supported for UE-initiated/event-driven beam reporting and the related agreement is listed below.

Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification. Agreement Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, support Option-4 of the following options in RAN1#120bis. - Option-4: If an Event-2 instance for a new beam is obtained at the time t and the timer for the new beam is not running, UE starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window) - Note: Timer is new beam specific. - Note: T_window is the agreed time window parameter for measurement. - Introduce separate UE capability to limit the number of timers. There is only 1 timer per new beam. Above agreement is captured in RAN1 specifications.

Considering the CPU occupation rule may be different for different event evaluation mechanisms, we analyze them respectively in the next section.

For the time window-based event evaluation procedure, the start of the time window is determined by the UE itself and the network cannot know it in time. Besides, the time window starts when a new beam satisfies the trigger event condition and the time window corresponding to the new beam has not started or has ended, so the first PUCCH channel and the second UL channel may be included in the time window. As mentioned before, UE needs to always perform beam measurement and data caching within the time window. It means UE may always perform beam measurement and data caching upon the reception of the RRC configuration of UE-initiated/event-driven beam report configuration. Therefore, the most straightforward way is to specify that the CPU(s) corresponding to a UE-initiated/event-driven beam report are occupied from the RRC configuration of the UE-initiated/event-driven beam report is configured and to the RRC configuration of the UE-initiated/event-driven beam report is released. Considering the time window is running per new beam, and measurement and counting operations within different time windows are independent, the occupied CPU number of a UE-initiated/event-driven beam report should be associated with the number of the time window.

If a UE-initiated/event-driven beam report is determined based on the time window-based event evaluation procedure, regardless of Mode A and Mode B,

- The number of occupied CPUs is determined based on the number of time windows,

- The corresponding CPU(s) are occupied from the RRC configuration of the UE-initiated/event-driven beam report is configured and to the RRC configuration of the UE-initiated/event-driven beam report is released.

For one-shot event evaluation procedure, compared to at least one event instances within the time window, it is more reasonable for UE to evaluate whether the trigger condition is met or not based on the latest measurement RS occasion before the CSI reference resource, where the CSI reference resource is determined based on the first PUCCH channel. Therefore, the corresponding CPU(s) is occupied from the first symbol of the earliest one of each CSI-RS/SSB resource for UE-initiated/event-driven beam reporting, respectively latest CSI-RS/SSB occasion no later than the corresponding CSI reference resource determined based on the first PUCCH channel, regardless of Mode A and Mode B. Besides, as only one-shot measurement is required and no counting value is needed, the number of occupied CPU for a UE-initiated/event-driven beam report can be 1.

For the one-shot event evaluation procedure, regardless of Mode A and Mode B, whether the trigger condition is met or not should be determined based on the latest measurement RS occasion before the CSI reference resource, where the CSI reference resource is determined based on the first PUCCH channel.

Support the following TP.

| Reason for change | For the basic procedure of event evaluation, i.e., once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs, it is unreasonable to capture it by at least one event instance within a time window as in endorsed CR [2]. For example, there are 5 transmission occasions of a new beam RS within the time window corresponding to the new beam RS, and measurement result acquired by the first transmission occasion satisfies the trigger event condition but the other four don’t satisfy. In this case, the reporting is meaningless. To avoid it, the event instance for the basic procedure of event evaluation should be determined based on the latest RS occasion of the new beam RS before CSI reference resource determined based on the first PUCCH channel of the CSI report configuration. |
| --- | --- |
| Summary of change | In TS38.214 section 5.2.1.5.4.1, clarify the determination of event instance for UE-initiated/event-driven beam reporting is based on the latest RS occasion of the new beam RS before CSI reference resource determined based on the first PUCCH channel when time-window based event evaluation procedure is not supported by UE. |
| Consequences if not approved | The determination of event instance for UE-initiated/event-driven beam reporting is unclear when time-window based event evaluation procedure is. |
| TS38.214 5.2.1.5.4.1 UE Initiated CSI reporting For a UE configured with a CSI-ReportConfig with eventType-r19 for periodic reference signals [with the same periodicity configured by the newBeamResourceSet-r19] and with dl-OrJointTCI-StateList, when an event instance is determined (as described in the following clauses), - for a reference signal configured by the newBeamResourceSet-r19, if eventType-r19 is set to ‘event2’ or ‘event7’, or - for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI, if eventType-r19 is set to ‘event1’, [if eventDetectionTimeWindowLength-r19 is provided, the UE starts a time window for such reference signal, wherein the time window does not overlap with other time windows for such reference signal.] When the number of event instances determined for such reference signal within the time window, if eventDetectionTimeWindowLength-r19 is provided, is greater than or equal to 1 or to eventInstanceCount-r19, if provided, or an event instance is determined by the latest transmission occasion of such reference signal before CSI reference resource determined based on the PUCCH resource configured by PUCCHResource-r19 in the CSI-ReportConfig if eventDetectionTimeWindowLength-r19 is not provided, the UE transmits PUCCH format 0 or format 1 in the PUCCH resource configured by PUCCHResource-r19 in the CSI-ReportConfig. < Unchanged parts are omitted > |  |

If a UE-initiated/event-driven beam report is determined based on the one-shot event evaluation procedure, regardless of Mode A and Mode B,

- The number of occupied CPUs is 1;

- the first symbol of the earliest one of each CSI-RS/SSB resource for UE-initiated/event-driven beam reporting, respectively latest CSI-RS/SSB occasion no later than the corresponding CSI reference resource.

As the reporting procedures for Mode A and Mode B are different, the ending time of the occupied CPU for a UE-initiated/event-driven shall be different. For Mode A, as shown in Figure 2, UE performs beam measurement before the first PUCCH channel and only when the beam measurement result(s) satisfy the trigger condition, the first PUCCH channel can be transmitted by UE. Therefore, there are three cases in Mode A.

- Case 1-1: UE performs beam measurement before the first PUCCH channel and the trigger condition is not satisfied, so the first PUCCH channel is not transmitted and the first PUCCH channel is not detected by the network;

- Case 1-2: UE performs beam measurement before the first PUCCH channel and the trigger condition is satisfied, so the first PUCCH channel is transmitted, but the first PUCCH channel is not detected by the network;

- Case 1-3: UE performs beam measurement before the first PUCCH channel and the trigger condition is satisfied. Then the first PUCCH channel is transmitted by UE and is detected successfully by the network. A moment later, a PDCCH scheduling a PUSCH is detected by UE and the beam report is carried on the PUSCH.

CPU occupation time for Case 1-1 and 1-2 in Mode A

In Case 1-1, the occupied CPU for a UE-initiated/event-driven beam report seems to be released after the beam measurement. In Case 1-2, the occupied time seems to end with the last symbol of a duration after the first PUCCH channel. In Case 1-3, the occupied CPU(s) seem to be released after reporting. The network can only be aware whether the first PUCCH channel is detected successfully or not, but cannot differentiate Case 1-1 and Case 1-2 if the first PUCCH channel is not detected. To align the understanding between the network and UE, as shown in Figure 5, one approach is to specify the CPU for a UE-initiated/event-driven beam report in a way that the occupied CPU is released at the end of duration after the first PUCCH channel if the UE does not detect the PDCCH scheduling a PUSCH to carry the beam report within the duration, where the length of the duration can be configured by RRC. If the UE detects the PDCCH within the duration, as shown in Figure 6, the occupied time ends with the last symbol of the PUSCH carrying the beam report.

CPU occupation time for Case 1-3 in Mode A

If a UE-initiated/event-driven beam report is determined based on the one-shot event evaluation procedure and configured with Mode A,  the occupied time of the corresponding CPU ends at the last symbol of a duration after the first PUCCH channel if UE does not detect the PDCCH scheduling a PUSCH to carry the beam report within the duration, or end at the last symbol of the PUSCH carrying the beam report if the PDCCH is detected within the duration.

For Mode B, as shown in Figure 3, UE performs beam measurement before the first PUCCH channel and only when the beam measurement result(s) satisfy the trigger condition, the first PUCCH channel and corresponding CG-PUSCH occasion can be transmitted by UE. Therefore, there are three cases in Mode B.

- Case 2-1: UE performs beam measurement before the first PUCCH channel and the trigger condition is not satisfied, so the first PUCCH channel and corresponding CG-PUSCH occasion are not transmitted by UE;

- Case 2-2: UE performs beam measurement before the first PUCCH channel and the trigger condition is satisfied, so the first PUCCH channel and corresponding CG-PUSCH occasion are transmitted by UE, but the first PUCCH channel is not detected successfully by the network;

- Case 2-3: UE performs beam measurement before the first PUCCH channel and the trigger condition is satisfied, so the first PUCCH channel and corresponding CG-PUSCH occasion are transmitted by UE. The first PUCCH channel is detected successfully by the network.

CPU occupation time in Mode B

From the UE’s perspective, the occupied CPU for a UE-initiated/event-driven beam report seems to be released after the beam measurement in Case 2-1. In Case 2-2 and Case 2-3, it seems to be occupied during the whole measurement and reporting procedure. However, from the network’s perspective, it seems to be occupied starting from the beam measurement and ending up with the last symbol of the first PUCCH channel for Case 2-1 and Case 2-2. However, the network cannot identify Case 2-1 and Case 2-2 either. It is difficult to align the CPU occupation between UE and the network with the above assumptions. The most straightforward way to define the ending time of the occupied CPU for a UE-initiated/event-driven beam report is the end of the CG-PUSCH occasion, as shown in Figure 7.

If a UE-initiated/event-driven beam report is determined based on the non-time window-based event evaluation procedure and configured with Mode B, the occupied CPU for the UE-initiated/event-driven beam report is released at the last symbol of the CG-PUSCH occasion carrying the UE-initiated/event-driven beam report.

Conclusion

For UE-initiated/event-driven L1/L2 beam reporting, we have the following proposals:

- Support dynamic update of the measurement RS in the new beam measurement resource set by a new MAC CE.

Based on the agreements, the following TP is proposed.

| Reason for change | In endorsed CR for TS38.214 [2], the indicated TCI state is applied to the same CC of CSI-ReportConfig, or to the CC indicated by CCofIndicatedTCI-r19 in the CSI-ReportConfig. However, according to the above agreement, the CC on which the indicated TCI state is applied, and the CC in which new beam RS(s) can be the same or different. The point that needs clarification is that the CC of the CSI-ReportConfig may not be equal to the CC in which the new beam RS is configured, especially when the RRC parameter carrier is configured in the CSI-ReportConfig. To align with the agreement, the corresponding description should be revised. |
| --- | --- |
| Summary of change | In TS38.214 section 5.2.1.5.4.1, clarify the determination of CC on which the indicated TCI state is applied in the multi-CC case. |
| Consequences if not approved | It can cause misalignment between the agreement and the specification description. |
| TS38.214 5.2.1.5.4.1a UE Initiated CSI reporting for event 2 For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType-r19 set to ‘event2’, an event instance is determined for a reference signal configured by the newBeamResourceSet-r19 (in a same CC of CSI-ReportConfig, or in the CC provided by carrier in the CSI-ReportConfig), if the L1-RSRP value determined for a transmission occasion of such reference signal is an eventThreshold-r19 greater than the L1-RSRP value determined for a transmission occasion of - the reference signal in the indicated TCI state applied to the same CC of the reference signal configured by the newBeamResourceSet-r19 CSI-ReportConfig, or to the CC indicated by CCofIndicatedTCI-r19 in the CSI-ReportConfig), if newBeamResourceSet-r19 is a NZP-CSI-RS-ResourceSet configured with repetition, else - the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state (applied to the same CC of the reference signal configured by the newBeamResourceSet-r19 CSI-ReportConfig or to the CC indicated by CCofIndicatedTCI-r19 in the CSI-ReportConfig), if newBeamResourceSet-r19 is a CSI-SSB-ResourceSet. < Unchanged parts are omitted > |  |

Support Canidate#1, i.e., when new beam RS is updated by RRC or MAC CE, the counting value of the new beam RS(s) that are not included in the new beam RS set should be reset, and the counting value of the new beam RS is not updated should be maintained.

Support Candidate#6, i.e., when the threshold for event evaluation is reconfigured by RRC signaling, the counting value of all new beam RS(s) should be reset.

Based on the above agreement, the following TP is proposed.

| Reason for change | According to the following agreement, only one time window is allowed for a new beam RS. However, in endorsed CR for TS38.214 [2], the condition for starting the time window corresponds to a new beam RS is the time window does not overlap with the other time windows for the new beam RS. It may introduce a misunderstanding that multiple time windows can be performed for a new beam RS. To avoid this ambiguity, the wording of the condition for starting the time window corresponds to a new beam RS should be revised. RAN1#120bis meeting agreement Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, support Option-4 of the following options in RAN1#120bis. - Option-4: If an Event-2 instance for a new beam is obtained at the time t and the timer for the new beam is not running, UE starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window) - Note: Timer is new beam specific. - Note: T_window is the agreed time window parameter for measurement. - Introduce separate UE capability to limit the number of timers. There is only 1 timer per new beam. Above agreement is captured in RAN1 specifications.. |
| --- | --- |
| Summary of change | In TS38.214 section 5.2.1.5.4.1, clarify the condition for starting the time window for the evaluation procedure of the UE-initiated/event-driven beam reporting. |
| Consequences if not approved | It can cause misalignment between the agreement and the specification description. |
| TS38.214 5.2.1.5.4.1 UE Initiated CSI reporting For a UE configured with a CSI-ReportConfig with eventType-r19 for periodic reference signals [with the same periodicity configured by the newBeamResourceSet-r19] and with dl-OrJointTCI-StateList, when an event instance is determined (as described in the following clauses), - for a reference signal configured by the newBeamResourceSet-r19, if eventType-r19 is set to ‘event2’ or ‘event7’, or - for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI, if eventType-r19 is set to ‘event1’, [if eventDetectionTimeWindowLength-r19 is provided, and the time window for such reference signal has not started yet or has ended, the UE starts a time window for such reference signal., wherein the time window does not overlap with other time windows for such reference signal.] When the number of event instances determined for such reference signal within the time window, if eventDetectionTimeWindowLength-r19 is provided, is greater than or equal to 1 or to eventInstanceCount-r19, if provided, the UE transmits PUCCH format 0 or format 1 in the PUCCH resource configured by PUCCHResource-r19 in the CSI-ReportConfig. < Unchanged parts are omitted > |  |

For Event-7, besides N reported new beams, support the introduction of the 1-bit condition met indicator and a bitmap of active TCI states used to indicate which active TCI state(s) could be replaced by reported new beam(s) that meets the event trigger condition.

If UE has not sent any first PUCCH associated with UE-initiated/event-driven beam reporting but receives a PDCCH scheduling a PUSCH to carry a UE-initiated/event-driven beam report, the scheduling DCI may be ignored by UE if no HARQ-ACK or transport block is multiplexed on the PUSCH.

Support Option-2A, i.e., for a first PUCCH transmission occasion, if there is a transmission of another second PUSCH corresponding to the CSI report determined by the same triggering beam(s) as the first PUCCH within a configurable time interval before the first PUCCH transmission occasion, the UE should not transmit the first PUCCH.

Only when the event trigger condition is satisfied, and the associated resource for the second UL channel is available, the corresponding first PUCCH channel can be transmitted in Mode B.

If UE supports the first PUCCH channel and the second UL channel in different PUCCH groups, a new RRC parameter should be introduced to indicate the CC that corresponds to the configured first PUCCH resource(s) in the CSI report configuration(s) in the CC where the RRC parameter is configured.

When the first PUCCH channel overlaps with the PUSCH in the time domain, the 1-bit PUCCH indication is multiplexed and transmitted on the PUSCH by puncturing regardless of whether the UE-initiated/event-driven beam report procedure is triggered and the information type carried in the PUSCH.

- The 1-bit indication is set to “1” when the UE-initiated/event-driven beam report procedure is triggered, otherwise, the 1-bit indication is set to “0”.

The priority index of the first PUCCH indication is 0 and the multiplexing/dropping rule of SR can be reused when the PUCCH carrying the first PUCCH indication overlaps with other UCI with different priority indexes.

When the Type-1 CG PUSCH carrying the beam report in Mode B overlaps with PUSCH with SP CSI in the time domain, the PUSCH with SP CSI should be dropped.

Support reuse of the intra-UE multiplexing/prioritization rules of PUSCH with AP-CSI for the PUSCH with UEI-BR for Mode A.

UE doesn’t expect that the PUSCH with UEI-BR for Mode A overlaps with the PUSCH carrying AP CSI in the time domain.

Support the following TP.

| Reason for change | In the endorsed CR for TS38.213 [4], only the Type-1 CG PUSCH carrying the beam report in Mode B reuses the intra-UE multiplexing/prioritization rules of PUSCH with SP-CSI is captured. However, the handling rule of overlapping between the Type-1 CG PUSCH carrying the beam report and PUSCH with SP CSI, and overlapping between the PUSCH carrying the beam report in Mode B and other UL channels has not been determined yet. Considering UE-initiated/event-driven beam report is aperiodic and can imply the beam change quickly, it has a higher priority than SP CSI. When the Type-1 CG PUSCH carrying the beam report in Mode B overlaps with the PUSCH with SP-CSI in the time domain, the PUSCH with SP-CSI will be dropped. For the overlapping case of the PUSCH carrying the beam report in Mode A and other UL channel(s), we think the intra-UE multiplexing/prioritization rules of PUSCH with AP-CSI can be reused, except for the case of the PUSCH carrying the beam report in Mode A and the PUSCH with AP-CSI. Furthermore, the case of the PUSCH carrying the beam report in Mode A overlapping with the PUSCH with AP CSI in the time domain can be avoided by the network. |
| --- | --- |
| Summary of change | In TS38.213 section 9, clarify the multiplexing/prioritization rules of the PUSCH with AP CSI is reused for the second UL channel in Mode A and the scheduling restriction on PUSCH with AP-CSI and PUSCH with UE-initiated/event-driven beam report in Mode A. In TS38.214 section 5.2.5, clarify that the UE-initiated beam report in mode B has a higher priority over the SP CSI report on PUSCH. |
| Consequences if not approved | The multiplexing/prioritization rules of the second UL channel for Mode A and Mode B are unclear. |
| TS38.213 9 UE procedure for reporting control information < Unchanged parts are omitted > For the remaining of this clause, when a UE - is not provided coresetPoolIndex or is provided coresetPoolIndex with a value of 0 for first CORESETs, and is provided coresetPoolIndex with a value of 1 for second CORESETs, on active DL BWPs of serving cells, and - is provided sTx-2Panel the UE separately determines and resolves time overlapping among first PUSCH transmissions that use respective first spatial domain filters corresponding to first TCI-State or TCI-UL-State associated with the first CORESETs, and among second PUSCH transmissions that use respective second spatial domain filters corresponding to second TCI-State or TCI-UL-State associated with the second CORESETs. For the remaining of this clause, reference to PUSCH with semi-persistent CSI reports includes a PUSCH with UE initiated reports when reportTransmissionMode is configured as ‘ModeB’ [6, TS 38.214]. A UE does not expect that a PUSCH with UE initiated reports when reportTransmissionMode is configured as ‘ModeA’ overlaps with PUSCH with aperiodic CSI reports. For the remaining of this clause, reference to PUSCH with aperiodic CSI reports includes a PUSCH with UE initiated reports when reportTransmissionMode is configured as ‘ModeA’ [6, TS 38.214]. When a UE determines overlapping for PUCCH transmissions with SL HARQ-ACK reports and PUCCH of larger and/or smaller priority index, the UE resolves the overlapping for PUCCH transmissions with SL HARQ-ACK reports and PUCCH of each priority index as described in clause 9.2.5 and 9.2.6 before resolving the overlapping for PUCCH transmissions without SL HARQ-ACK or the overlapping for PUCCH transmissions and PUSCH transmissions. When a UE determines overlapping for PUCCH and/or PUSCH transmissions of the same priority index other than PUCCH transmissions with SL HARQ-ACK reports before considering limitations for UE transmission due to cell DRX operation [11, TS 38.321] or as described in clauses 11.1, 11.1.1, 11.2A, 15 and 17.2 including repetitions if any, < Unchanged parts are omitted > TS38.214 5.2.5 Priority rules for CSI reports For two overlapping PUSCHs, the priority rules in this clause are applied for physical channels with same priority index according to clause 9 in [6, TS 38.213] if a UE is not configured with sTx-2Panel or a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in different ControlResourceSet in the active DL BWP and the UE is configured with sTx-2Panel and the two overlapping PUSCHs are associated with same value of coresetPoolIndex. CSI reports are associated with a priority value PriiCSIy,k,c,s=2∙Ncells∙Ms∙y+Ncells∙Ms∙k+Ms∙c+s where - for aperiodic CSI reports to be carried on PUSCH for semi-persistent CSI reports to be carried on PUSCH, for semi-persistent CSI reports to be carried on PUCCH and for periodic CSI reports to be carried on PUCCH; - for CSI reports carrying L1-RSRP or L1-SINR and for CSI reports not carrying L1-RSRP or L1-SINR; - c is the serving cell index and Ncells is the value of the higher layer parameter maxNrofServingCells; - for a CSI report configured with ltm-CSI-ReportConfig, c is the serving cell index value where the report configuration is configured. - s is the reportConfigID and is the value of the higher layer parameter maxNrofCSI-ReportConfigurations. - for a CSI report configured with ltm-CSI-ReportConfig, s is the ltm-CSI-ReportConfigId and Ms is the value of the higher layer parameter maxNrofLTM-CSI-ReportConfigurations A first CSI report is said to have priority over second CSI report if the associated value is lower for the first report than for the second report. Two CSI reports are said to collide if the time occupancy of the physical channels scheduled to carry the CSI reports overlap in at least one OFDM symbol and are transmitted on the same carrier. When a UE is configured to transmit two colliding CSI reports, - if y values are different between the two CSI reports, the following rules apply except for the case when one of the y value is 2 and the other y value is 3 (for CSI reports transmitted on PUSCH, as described in Clause 5.2.3; for CSI reports transmitted on PUCCH, as described in Clause 5.2.4): - The CSI report with higher PriiCSIy,k,c,s value shall not be sent by the UE. - otherwise, the two CSI reports are multiplexed or either is dropped based on the priority values, as described in Clause 9.2.5.2 in [6, TS 38.213]. A CSI report configured with ltm-CSI-ReportConfig has a higher priority over all CSI report(s) configured with CSI-ReportConfig irrespective of PriiCSIy,k,c,s value in case of collision with CSI report(s) configured with CSI-ReportConfig. A CSI report configured with reportTransmissionMode-r19 set to ‘ModeB’ has a higher priority over all semi-persistent CSI report(s) to be carried on PUSCH in case of collision with the CSI report(s) configured with CSI-ReportConfig. If a semi-persistent CSI report to be carried on PUSCH overlaps in time with PUSCH data transmission in one or more symbols on the same carrier, and if the earliest symbol of these PUSCH channels starts no earlier than N2+d2,1 symbols after the last symbol of the DCI scheduling the PUSCH where d2,1 is the maximum of the d2,1 associated with the PUSCH carrying semi-persistent CSI report and the PUSCH with data transmission, the CSI report shall not be transmitted by the UE. Otherwise, if the timeline requirement is not satisfied this is an error case. If a UE would transmit a first PUSCH that includes semi-persistent CSI reports and a second PUSCH that includes an UL-SCH on the same carrier, and the first PUSCH transmission would overlap in time with the second PUSCH transmission, the UE does not transmit the first PUSCH and transmits the second PUSCH. The UE expects that the first and second PUSCH transmissions satisfy the above timing conditions for PUSCH transmissions that overlap in time when at least one of the first or second PUSCH transmissions is in response to a DCI format detection by the UE. |  |

The multiple CSI report configurations associated with the same CSI-AperiodicTriggerState don't have to be associated with the same PUCCH resource for Mode A.

The report content included in the PUSCH, including the reported RS ID(s) and corresponding measurement results, should be acquired before the corresponding PUCCH indication, regardless of Mode A and Mode B.

In mode A, the transmission slot of the PUSCH should be determined by mapping the TDRI field value to the resource allocation table when the UE receives a DCI scheduling a PUSCH used to carry only UE-initiated/event-driven beam report(s).

In Mode A, the CSI computation time of UE-initiated/event-driven beam report should be optimized.

An available transmission occasion corresponds to a configured PUSCH resource for UEIBR in Mode B that does not overlap with a DL semi-static configured symbol or flexible symbol and does not conflict with a channel of higher priority, such as PRACH.

The minimum value of X in Mode B, i.e., the number of symbol(s) between the last symbol of the first PUCCH indication and the first symbol of the available transmission occasion of the pre-configured CG PUSCH can be {0, 1, …, N}, where the value of N needs further discussion.

To ensure UE can process NW-initiated CSI report and UE-initiated/event-driven beam report orderly, the occupied CPU number and occupation time for the UE-initiated/event-driven beam report should be defined.

If a UE-initiated/event-driven beam report is determined based on the time window-based event evaluation procedure, regardless of Mode A and Mode B,

- The number of occupied CPUs is determined based on the number of time windows,

- The corresponding CPU(s) are occupied from the RRC configuration of the UE-initiated/event-driven beam report is configured and to the RRC configuration of the UE-initiated/event-driven beam report is released.

For the one-shot event evaluation procedure, regardless of Mode A and Mode B, whether the trigger condition is met or not should be determined based on the latest measurement RS occasion before the CSI reference resource, where the CSI reference resource is determined based on the first PUCCH channel.

Support the following TP.

| Reason for change | For the basic procedure of event evaluation, i.e., once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs, it is unreasonable to capture it by at least one event instance within a time window as in endorsed CR [2]. For example, there are 5 transmission occasions of a new beam RS within the time window corresponding to the new beam RS, and measurement result acquired by the first transmission occasion satisfies the trigger event condition but the other four don’t satisfy. In this case, the reporting is meaningless. To avoid it, the event instance for the basic procedure of event evaluation should be determined based on the latest RS occasion of the new beam RS before CSI reference resource determined based on the first PUCCH channel of the CSI report configuration. |
| --- | --- |
| Summary of change | In TS38.214 section 5.2.1.5.4.1, clarify the determination of event instance for UE-initiated/event-driven beam reporting is based on the latest RS occasion of the new beam RS before CSI reference resource determined based on the first PUCCH channel when time-window based event evaluation procedure is not supported by UE. |
| Consequences if not approved | The determination of event instance for UE-initiated/event-driven beam reporting is unclear when time-window based event evaluation procedure is. |
| TS38.214 5.2.1.5.4.1 UE Initiated CSI reporting For a UE configured with a CSI-ReportConfig with eventType-r19 for periodic reference signals [with the same periodicity configured by the newBeamResourceSet-r19] and with dl-OrJointTCI-StateList, when an event instance is determined (as described in the following clauses), - for a reference signal configured by the newBeamResourceSet-r19, if eventType-r19 is set to ‘event2’ or ‘event7’, or - for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI, if eventType-r19 is set to ‘event1’, [if eventDetectionTimeWindowLength-r19 is provided, the UE starts a time window for such reference signal, wherein the time window does not overlap with other time windows for such reference signal.] When the number of event instances determined for such reference signal within the time window, if eventDetectionTimeWindowLength-r19 is provided, is greater than or equal to 1 or to eventInstanceCount-r19, if provided, or an event instance is determined by the latest transmission occasion of such reference signal before CSI reference resource determined based on the PUCCH resource configured by PUCCHResource-r19 in the CSI-ReportConfig if eventDetectionTimeWindowLength-r19 is not provided, the UE transmits PUCCH format 0 or format 1 in the PUCCH resource configured by PUCCHResource-r19 in the CSI-ReportConfig. < Unchanged parts are omitted > |  |

If a UE-initiated/event-driven beam report is determined based on the one-shot event evaluation procedure, regardless of Mode A and Mode B,

- The number of occupied CPUs is 1;

- the first symbol of the earliest one of each CSI-RS/SSB resource for UE-initiated/event-driven beam reporting, respectively latest CSI-RS/SSB occasion no later than the corresponding CSI reference resource.

If a UE-initiated/event-driven beam report is determined based on the one-shot event evaluation procedure and configured with Mode A,  the occupied time of the corresponding CPU ends at the last symbol of a duration after the first PUCCH channel if UE does not detect the PDCCH scheduling a PUSCH to carry the beam report within the duration, or end at the last symbol of the PUSCH carrying the beam report if the PDCCH is detected within the duration.

If a UE-initiated/event-driven beam report is determined based on the non-time window-based event evaluation procedure and configured with Mode B, the occupied CPU for the UE-initiated/event-driven beam report is released at the last symbol of the CG-PUSCH occasion carrying the UE-initiated/event-driven beam report.

References

RAN1#118 Chiarman Notes.

R1-2503490

RAN1#120bis Chiarman Notes.

R1-2503175

R1-2503156